European digital cellular telecommunications system (Phase 2);

Technical realization of the Short Message Service (SMS)

Point-to-Point (PP)

(GSM 03.40)

Contents

.Begin Table C.

Foreword	7

0.1	Scope	9

0.2	Normative references	9

0.3	Definitions and abbreviations	10

0.3.1	Key definitions	11

0.3.2	Abbreviations	13

1	Introduction	13

2	Not used	13

3	Services and service elements	13

3.1	Basic services	13

3.2	Short Message Service elements	15

3.2.1	Validity-Period	15

3.2.2	Service-Centre-Time-Stamp	15

3.2.3	Protocol-Identifier	15

3.2.4	More-Messages-to-Send	15

3.2.5	Delivery of Priority and non-Priority Messages	16

3.2.6	Messages-Waiting	16

3.2.7	Alert-SC	18

3.2.8	Options concerning MNRF, MCEF and MWD	18

3.2.9	Status report capabilities	19

3.2.10	Reply Path	19

3.3	Unsuccessful short message TPDU transfer SC -> MS	19

3.3.1	Errors occurring during transfer of TPDU to MS	20

3.3.2	Errors occurring after TPDU arrives at MS	20

3.4	Unsuccessful short message TPDU transfer MS -> SC	22

3.4.1	Errors occurring during transfer of TPDU to SC	22

3.4.2	Errors occurring after TPDU arrives at SC	22

3.5	Use of Supplementary Services in combination with the Short Message Service	22

3.6	Applicability of Operator Determined Barring to the Short Message Service	22

3.7	Multiple short message transfer	22

4	Network architecture	23

4.1	Basic network structure	23

4.2	Transfer on link 3	24

5	Service Centre and PLMN interconnection	24

5.1	Service centre connection	24

5.2	Routing requirements	24

5.2.1	Mobile terminated short message	24

5.2.2	Mobile originated short message	24

6	Service Centre functionality	25

6.1	Service Centre capabilities	25

6.2	SC functional requirements	25

7	MS functionality	26

7.1	MS capabilities	26

7.2	MS configuration	26

8	MSC functionality	27

8.1	MSC functionality related to SM MT	27

8.1.1	Functionality of the SMS-GMSC	27

8.1.2	Functionality of the MSC	28

8.2	MSC functionality related to SM MO	28

8.2.1	Functionality of the MSC	28

8.2.2	Functionality of the SMS-IWMSC	29

8.3	SMS-IWMSC functionality related to alerting	29

9	Protocols and protocol architecture	30

9.1	Protocol element features	30

9.1.1	Octet and Bit transmission order	30

9.1.2	Numeric and alphanumeric representation	30

9.1.2.1	Integer representation	30

9.1.2.2	Octet representation	31

9.1.2.3	Semi-octet representation	31

9.1.2.4	Alphanumeric representation	32

9.1.2.5	Address fields	32

9.2	Service provided by the SM-TL	34

9.2.1	General	34

9.2.2	PDU Type repertoire at SM-TL	34

9.2.2.1	SMS-DELIVER type	35

9.2.2.1a	SMS-DELIVER-REPORT type	37

9.2.2.2	SMS-SUBMIT type	38

9.2.2.2a	SMS-SUBMIT-REPORT type	40

9.2.2.3	SMS-STATUS-REPORT type	41

9.2.2.4	SMS-COMMAND type	43

9.2.3	Definition of the TPDU parameters	44

9.2.3.1	TP-Message-Type-Indicator (TP-MTI)	44

9.2.3.2	TP-More-Messages-to-Send (TP-MMS)	44

9.2.3.3	TP-Validity-Period-Format (TP-VPF)	44

9.2.3.4	TP-Status-Report-Indication (TP-SRI)	44

9.2.3.5	TP-Status-Report-Request (TP-SRR)	45

9.2.3.6	TP-Message-Reference (TP-MR)	45

9.2.3.7	TP-Originating-Address (TP-OA)	45

9.2.3.8	TP-Destination-Address (TP-DA)	45

9.2.3.9	TP-Protocol-Identifier (TP-PID)	45

9.2.3.10	TP-Data-Coding-Scheme (TP-DCS)	47

9.2.3.11	TP-Service-Centre-Time-Stamp (TP-SCTS)	47

9.2.3.12	TP-Validity-Period	47

9.2.3.13	TP-Discharge-Time (TP-DT)	48

9.2.3.14	TP-Recipient-Address (TP-RA)	48

9.2.3.15	TP-Status (TP-ST)	48

9.2.3.16	TP-User-Data-Length (TP-UDL)	50

9.2.3.17	TP-Reply-Path (TP-RP)	50

9.2.3.18	TP-Message-Number (TP-MN)	50

9.2.3.19	TP-Command-Type (TP-CT)	51

9.2.3.20	TP-Command-Data-Length (TP-CDL)	51

9.2.3.21	TP-Command-Data (TP-CD)	51

9.2.3.22	TP-Failure-Cause (TP-FCS)	51

9.3	Service provided by the SM-RL	52

9.3.1	General	52

9.3.2	Protocol element repertoire at SM-RL	53

9.3.2.1	RP-MO-DATA	53

9.3.2.2	RP-MT-DATA	54

9.3.2.3	RP-ACK	54

9.3.2.4	RP-ERROR	54

9.3.2.5	RP-ALERT-SC	55

9.3.2.6	RP-SM-MEMORY-AVAILABLE	55

10	Fundamental procedures within the point-to-point SMS	55

10.1	Short message mobile terminated	56

10.2	Short message mobile originated	67

10.3	Alert transfer	73

11	Mapping of error causes between RP layers	77

11.1	Mobile Terminated short message transfer	77

11.2	Memory available notification	77

11.3	Mobile Originated short message transfer	78

ANNEX 1 (Informative)	79

ANNEX 2	80

ANNEX 3 (Informative)	81

ANNEX 4 (Informative)	99

1	Introduction	99

2	The scope of applicability	99

3	Terminology	99

4	The reply path requesting procedure	100

5	The reception of an original MT SM	100

6	The submission of the reply MO SM	100

7	Usage of SCs for replying	101

8	Replying possibilities for phase 1 mobile stations	101

9	The resulting service for originating SMEs	101

History	102

.End Table C.

Number of pages: 102

Blank page

.c.Foreword

This European Telecommunication Standard (ETS) has been produced by the Special Mobile Group (SMG) Technical Committee (TC) of the European Telecommunications Standards Institute (ETSI).

This ETS defines the technical realization of the Point-to-Point (PP) Short Message Service (SMS) for the European digital cellular telecommunications system (Phase 2). This ETS corresponds to GSM Technical Specification (GSM-TS) GSM 03.40 version 4.9.1.

The specification from which this ETS has been derived was originally based on CEPT documentation, hence the presentation of this ETS may not be entirely in accordance with the ETSI/PNE rules.

Reference is made within this ETS to GSM-TSs.

NOTE:	TC-SMG has produced documents which give the technical specifications for the implementation of the European digital cellular telecommunications system. Historically, these documents have been identified as  GSM Technical Specifications (GSM-TSs). These TSs may have subsequently become I-ETSs (Phase 1), or ETSs (Phase 2), whilst others may become ETSI Technical Reports (ETRs). GSM-TSs are, for editorial reasons, still referred to in current GSM ETSs.

Blank page

.c.0.1	Scope

This specification describes the point-to-point Short Message Service (SMS) of the GSM PLMN system. It defines:

-	the services and service elements,

-	the network architecture,

-	the Service Centre functionality,

-	the MSC functionality (with regard to the SMS),

-	the routing requirements,

-	the protocols and protocol layering,

for the Teleservices 21 and 22, as specified in the GSM 02.03.

The use of radio resources for the transfer of short messages between the MS and the MSC is described in GSM 04.11 'Point-to-Point Short Message Service Support on Mobile Radio Interface', and is dealt with in that specification.

The network aspects of Short Message Service provision are outside the scope of this specification (i.e. the provision of network connectivity between the PLMN subsystems). The required and assumed network service offered to the higher layers is defined in this specification.

The Cell Broadcast Short Message Service (Teleservice 23) is a separate service, and is described in GSM 03.41 'Technical Realization of the Short Message Service - Cell Broadcast'.

.c.0.2	Normative references

This ETS incorporates by dated and undated reference, provisions from other publications. These normative references are cited at the appropriate places in the text and the publications are listed hereafter. For dated references, subsequent amendments to or revisions of any of these publications apply to this ETS only when incorporated in it by amendment or revision. For undated references, the latest edition of the publication referred to applies.

[1]	GSM 01.04 (ETR 100): "European digital cellular telecommunication system (Phase 2); Definitions, abbreviations and acronyms".

[2]	GSM 02.03 (ETS 300 502): "European digital cellular telecommunication system (Phase 2); Teleservices supported by a GSM Public Land Mobile Network (PLMN)".

[3]	GSM 02.04 (ETS 300 503): "European digital cellular telecommunication system (Phase 2); General on supplementary services".

[4]	GSM 02.41 (ETS 300 513): "European digital cellular telecommunication system (Phase 2);  Operator determined barring".

[5]	GSM 03.02 (ETS 300 522): "European digital cellular telecommunication system (Phase 2); Network architecture".

[6]	GSM 03.08 (ETS 300 526): "European digital cellular telecommunication system (Phase 2); Organisation of subscriber data".

[7]	GSM 03.11 (ETS 300 529): "European digital cellular telecommunication system (Phase 2); Technical realization of supplementary services".

[8]	GSM 03.15 (ETS 300 533): "European digital cellular telecommunication system (Phase 2); Technical realization of operator determined barring".

[9]	GSM 03.38 (ETS 300 537): "European digital cellular telecommunication system (Phase 2); Alphabets and language-specific information".

[10]	GSM 03.41 (ETS 300 537): "European digital cellular telecommunication system (Phase 2); Technical realization of Short Message Service Cell Broadcast (SMSCB)".

[11]	GSM 03.47 (ETR 106): "European digital cellular telecommunication system (Phase 2); Example protocol stacks for interconnecting Service Centre(s) (SC) and Mobile-services Switching Centre(s) (MSC)".

[12]	GSM 04.08 (ETS 300 557): "European digital cellular telecommunication system (Phase 2); Mobile radio interface layer 3 specification".

[13]	GSM 04.11 (ETS 300 559): "European digital cellular telecommunication system (Phase 2); Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[14]	GSM 07.05 (ETS 300 585): "European digital cellular telecommunication system (Phase 2); Use of Data Terminal Equipment - Data Circuit terminating Equipment (DTE - DCE) interface for Short Message Service (SMS) and Cell Broadcast Service (CBS)".

[15]	GSM 09.02 (ETS 300 599): "European digital cellular telecommunication system (Phase 2); Mobile Application Part (MAP) specification".

[16]	GSM 11.11 (ETS 300 608): "European digital cellular telecommunication system (Phase 2); Specification of the Subscriber Identity Module - Mobile Equipment (SIM - ME) interface".

[17]	CCITT Recommendation E.164 (Blue Book): "Numbering plan for the ISDN era".

[18]	CCITT Recommendation E.163 (Blue Book): "Numbering plan for the international telephone service".

[19]	CCITT Recommendation Q.771: "Specifications of Signalling System No.7; Functional description of transaction capabilities".

[20]	CCITT Recommendation T.100 (Blue Book): "International information exchange for interactive videotex".

[21]	CCITT Recommendation T.101 (Blue Book): "International interworking for videotex services".

[22]	CCITT Recommendation X.121 (Blue Book): "International numbering plan for public data networks".

[23]	CCITT Recommendation X.400 (Blue Book): "Message handling system and service overview".

.c.0.3	Definitions and abbreviations

Note:	Use of hyphens and full stops:

Care is needed when reading this specification as names containing words separated by hyphens have different meaning than when separated with full stops. E.g. TS-Status-Report-Request is a parameter within a TS-Submit primitive, whilst TS-Status-Report.Request is a primitive in its own right.

.c.0.3.1	Key definitions

Active MS:  A switched-on mobile station with a SIM module attached.

Alert-SC:  Service element provided by a GSM PLMN to inform an SC which has previously initiated unsuccessful short message delivery attempt(s) to a specific MS, that the MS is now recognized by the PLMN to have recovered operation.

Status Report:  SC informing the originating MS of the outcome of a short message submitted to an SME.

Gateway MSC for Short Message Service (SMS-GMSC):  A function of an MSC capable of receiving a short message from an SC, interrogating an HLR for routing information and SMS info, and delivering the short message to the VMSC of the recipient MS.

Interworking MSC for Short Message Service (SMS-IWMSC):  A function of an MSC capable of receiving a short message from within the PLMN and submitting it to the recipient SC.

Messages-Waiting (MW):  Service element that makes a PLMN store information (Messages-Waiting-Indication), listing those SCs that have made unsuccessful short message delivery attempts to MSs in that PLMN.

Messages-Waiting-Indication (MWI): Data to be stored in the HLR and VLR with which an MS is associated, indicating that there is one or more messages waiting in a set of SCs to be delivered to the MS (due to unsuccessful delivery attempt(s)).

Messages-Waiting-Data (MWD):  A part of the MWI to be stored in the HLR. MWD consists of an address list of the SCs which have messages waiting to be delivered to the MS.

Mobile-Station-Memory-Capacity-Exceeded-Flag (MCEF): A part of the MWI to be stored in the HLR. MCEF is a boolean parameter indicating if the address list of MWD contains one or more entries because an attempt to deliver a short message to an MS has failed with a cause of MS Memory Capacity Exceeded.

Mobile-Station-Not-Reachable-Flag (MNRF):  The part of the MWI to be stored in the VLR and the HLR. MNRF is a boolean parameter indicating if the address list of MWD contains one or more entries because an attempt to deliver a short message to an MS has failed with a cause of Absent Subscriber.

More-Messages-to-Send (MMS):  Information element offering an MS receiving a short message from an SC the information whether there are still more messages waiting to be sent from that SC to the MS.

Priority:  Service element enabling the SC or SME to request a short message delivery attempt to an MS irrespective of whether or not the MS has been identified as temporarily absent.

Protocol-Identifier:  Information element by which the originator of a short message (either an SC or an MS) may refer to a higher layer protocol.

Reply Path Procedure:  A mechanism which allows an SME to request that an SC should be permitted to handle a reply sent in response to a message previously sent from that SME to another SME. This may happen even though the SC may be unknown to the SME which received the initial message.

Report:  Response from either the network or the recipient upon a short message being sent from either an SC or an MS. A report may be a delivery report, which confirms the delivery of the short message to the recipient, or it may be a failure report, which informs the originator that the short message was never delivered and the reason why.

When issued by the Service Centre, the delivery report confirms the reception of the Short Message by the SC, and not the delivery of the Short Message to the SME.

When issued by the Mobile Station, the delivery report confirms the reception of the Short Message by the Mobile Station, and not the delivery of the Short Message to the user.

Replace Short Message Type:

A range of values in the Protocol Identifier which allows an indication to be sent with a short message (MT or MO) that the short message is of a particular type allowing the receiving MS or the SC to replace an existing message of the same type held in the SC, the ME or on the SIM, provided it comes:

-	in MT cases:	from the same SC and originating address;

-	in MO cases:	from the same MS.

Service Centre (SC):  Function responsible for the relaying and store-and-forwarding of a short message between an SME and an MS. The SC is not a part of the GSM PLMN, however MSC and SC may be integrated.

Short Message:  Information that may be conveyed by means of the Short Message Service described in this specification.

Short Message Entity (SME):  An entity which may send or receive Short Messages. The SME may be located in a fixed network, an MS, or an SC.

SMS-STATUS-REPORT:  Short message transfer protocol data unit informing the receiving MS of the status of a mobile originated short message previously submitted by the MS, i.e. whether the SC was able to forward the message or not, or whether the message was stored in the SC for later delivery.

SMS-COMMAND:  Short message transfer protocol data unit which enables an MS to invoke an operation at the SC. An MS may then, for example, delete a short message, cancel a Status Report Request, enquire about the status of a short message or request another function to be performed by the SC.

The type of operation is indicated by the TS-Command-Type and the particular SM to operate on is indicated by the TS-Message-Number. The response to an SMS-COMMAND is an SMS-STATUS-REPORT, the outcome of the operation being passed in its TS-status field.

SMS-DELIVER:  Short message transfer protocol data unit containing user data (the short message), being sent from an SC to an MS.

SMS-SUBMIT:  Short message transfer protocol data unit containing user data (the short message), being sent from an MS to an SC.

Service-Centre-Time-Stamp (SCTS):  Information element offering the recipient of a short message the information of when the message arrived at the SM-TL entity of the SC. The time of arrival comprises the year, month, day, hour, minute, second and time zone.

Validity-Period (VP):  Information element enabling the originator MS to indicate the time period during which the originator considers the short message to be valid.

.c.0.3.2	Abbreviations

In addition to those below, definitions used in this specification are listed in GSM 01.04.

E.163	CCITT Rec. E.163 (Blue Book)

E.164	CCITT Rec. E.164 (Blue Book)

ACSE	Association Control Service Element

SM MT	Short Message Mobile Terminated Point-to-Point

SM MO	Short Message Mobile Originated Point-to-Point

SM-AL	Short Message Application Layer

SM-TL	Short Message Transfer Layer

SM-RL	Short Message Relay Layer

SM-LL	Short Message Lower Layers

SM-TP	Short Message Transfer Layer Protocol

SM-RP	Short Message Relay Layer Protocol

SM-TS	Short Message Transfer Service

SM-RS	Short Message Relay Service

T.100	CCITT Rec. T.100 (Blue Book)

T.101	CCITT Rec. T.101 (Blue Book)

TPDU	Transfer protocol data unit

X.121	CCITT Rec. X.121 (Blue Book)

X.400	CCITT Rec. X.400 (Blue Book)

.c.1	Introduction

The Point-to-Point Short Message Service (SMS) provides a means of sending messages of limited size to and from GSM mobiles. The provision of SMS makes use of a Service Centre, which acts as a store and forward centre for short messages. Thus a GSM PLMN needs to support the transfer of short messages between Service Centres and mobiles.

Two different point-to-point services have been defined: mobile originated and mobile terminated. Mobile originated messages will be transported from an MS to a Service Centre. These may be destined for other mobile users, or for subscribers on a fixed network. Mobile terminated messages will be transported from a Service Centre to an MS. These may be input to the Service Centre by other mobile users (via a mobile originated short message) or by a variety of other sources, e.g. speech, telex, or facsimile.

.c.2	Not used

.c.3	Services and service elements

The SMS provides a means to transfer short messages between a GSM MS and an SME via an SC. The SC serves as an interworking and relaying function of the message transfer between the MS and the SME.

This specification describes only the short message point-to-point services between the MS and SC. It may, however, refer to possible higher layer applications.

.c.3.1	Basic services

The short message point-to-point services comprise two basic services:

SM MT   (Short Message Mobile Terminated Point-to-Point),

SM MO   (Short Message Mobile Originated Point-to-Point).

SM MT denotes the capability of the GSM system to transfer a short message submitted from the SC to one MS, and to provide information about the delivery of the short message either by a delivery report or a failure report with a specific mechanism for later delivery; see Figure 03.40/1.

SM MO denotes the capability of the GSM system to transfer a short message submitted by the MS to one SME via an SC, and to provide information about the delivery of the short message either by a delivery report or a failure report. The message must include the address of that SME to which the SC shall eventually attempt to relay the short message; see Figure 03.40/2.

The text messages to be transferred by means of the SM MT or SM MO contain up to 140 octets.

Short message delivery

┌────┐                                ┌────┐

│    │  ───────────────────────────  │    │

│ SC │                                │ MS │

│    │       ────────────────        │    │

└────┘                                └────┘

Report

Figure 03.40/1	The Short Message Service mobile terminated, point-to-point.

Short message submission

┌────┐                                ┌────┐

│    │  ───────────────────────────  │    │

│ SC │                                │ MS │

│    │       ────────────────        │    │

└────┘                                └────┘

Report

Figure 03.40/2	The Short Message Service mobile originated, point-to-point.

An active MS shall be able to receive a short message TPDU (SMS-DELIVER) at any time, independently of whether or not there is a speech or data call in progress. A report will always be returned to the SC; either confirming that the MS has received the short message, or informing the SC that it was impossible to deliver the short message TPDU to the MS, including the reason why.

An active MS shall be able to submit a short message TPDU (SMS-SUBMIT) at any time, independently of whether or not there is a speech or data call in progress. A report will always be returned to the MS; either confirming that the SC has received the short message TPDU, or informing the MS that it was impossible to deliver the short message TPDU to the SC, including the reason why.

Note:	When the transmission or reception of a short message coincide with a change of state in the MS, i.e. from busy to idle or from idle to busy, or during a handover, the short message transfer might be aborted.

It is also possible for two short messages to be received in sequence having the same originating address and identification, i.e. message reference number (MO) or SC Timestamp (MT). Such a situation may be due to errors at the RP or CP layers (e.g. during inter MSC handover) where it may be a duplicated message or otherwise it may be a valid new message.

The receiving entity should therefore make provision to check other parameters contained in the short message to decide whether the second short message is to be discarded.

.c.3.2	Short Message Service elements

The SMS comprises 7 elements particular to the submission and reception of messages:

Validity-Period

Service-Centre-Time-Stamp

Protocol-Identifier

More-Messages-to-Send

Priority

Messages-Waiting

Alert-SC

.c.3.2.1	Validity-Period

The Validity-Period is the information element which gives an MS submitting an SMS-SUBMIT to the SC the possibility to include a specific time period value in the short message (TP-Validity-Period field, see section 9). The TP-Validity-Period parameter value indicates the time period for which the short message is valid, i.e. for how long the SC shall guarantee its existence in the SC memory before delivery to the recipient has been carried out.

.c.3.2.2	Service-Centre-Time-Stamp

The Service-Centre-Time-Stamp is the information element by which the SC informs the recipient MS about the time of arrival of the short message at the SM-TL entity of the SC. The time value is included in every SMS-DELIVER (TP-Service-Centre-Time-Stamp field, see section 9) being delivered to the MS.

.c.3.2.3	Protocol-Identifier

The Protocol-Identifier is the information element by which the SM-TL either refers to the higher layer protocol being used, or indicates interworking with a certain type of telematic device.

The Protocol-Identifier information element makes use of a particular field in the message types SMS-SUBMIT, SMS-DELIVER and SMS-COMMAND TP-Protocol-Identifier (TP-PID).

.c.3.2.4	More-Messages-to-Send

The More-Messages-to-Send is the information element by which the SC informs the MS that there is one or more messages waiting in that SC to be delivered to the MS. The More-Messages-to-Send information element makes use of a boolean parameter in the message SMS-DELIVER, TP-More-Messages-to-Send (TP-MMS).

.c.3.2.5	Delivery of Priority and non-Priority Messages

Priority is the information element provided by an SC or SME to indicate to the PLMN whether or not a message is a priority message.

Delivery of a non-priority message will not be attempted if the MS has been identified as temporarily absent (see section 3.2.6).

Delivery of a non-priority message will be attempted if the MS has not been identified as temporarily absent irrespective of whether the MS has been identified as having no free memory capacity (see section 3.2.6).

Delivery of a priority message will be attempted irrespective of whether or not the MS has been identified as temporarily absent, or having no free memory capacity.

.c.3.2.6	Messages-Waiting

The Messages-Waiting is the service element that enables the PLMN to provide the HLR and VLR with which the recipient MS is associated with the information that there is a message in the originating SC waiting to be delivered to the MS. The service element is only used in case of previous unsuccessful delivery attempt(s) due to temporarily absent mobile or MS memory capacity exceeded. This information, denoted the Messages-Waiting-Indication (MWI), consists of Messages-Waiting-Data (MWD), the Mobile-Station-Not-Reachable-Flag (MNRF) and the Mobile-Station-Memory-Capacity-Exceeded-Flag (MCEF) located in the HLR, and the Mobile-Station-Not-Reachable-Flag (MNRF) located in the VLR. Figure 03.40/3 shows an example.

HLR;

┌─────────────────────────────────────────────────────────────────┐

│ MWD:                                                            │

│ ┌──────────────┬─────────────┬─────────────┬....┬─────────────┐ │

│ │              │             │             │    │             │ │

│ │ MSIsdn-Alert │ SC address1 │ SC address2 │    │ SC addressn │ │

│ │              │             │             │    │             │ │

│ └──────────────┴─────────────┴─────────────┴....┴─────────────┘ │

│ ┌──────────┐          ┌───────────┐                             │

│ │          │          │           │                             │

│ │   MNRF   │          │   MCEF    │                             │

│ └──────────┘          └───────────┘                             │

│                                                                 │

└─────────────────────────────────────────────────────────────────┘

VLR;

┌──────────────────┐

│                  │

│ ┌─────────┐      │

│ │         │      │

│ │  MNRF   │      │

│ └─────────┘      │

└──────────────────┘

Figure 03.40/3:	Example of how information on one MS can be put in relation to SC(s) in order to fulfil the requirement of Alert-SC mechanism.

The MWD shall contain a list of addresses (SC-Addr) of SCs which have made previous unsuccessful delivery attempts of a message (see section 5). In order to be able to send alert messages to every SC which has made unsuccessful delivery attempts to an MS, the HLR shall store the MSIsdn-Alert (see section 3.2.7) together with references to the SC addresses. The requirements placed upon the HLR are specified in GSM 03.08. The description of how the HLR is provided with SC and MS address information is given in GSM 09.02.

The Mobile-Station-Memory-Capacity-Exceeded-Flag (MCEF) within the HLR is a boolean parameter with the value TRUE when the list MWD contains one or more list elements because an attempt to deliver a short message to an MS has failed with a cause of MS Memory Capacity Exceeded, and with the value FALSE otherwise.

The Mobile-Station-Not-Reachable-Flag (MNRF) within the HLR and the VLR is a boolean parameter with the value TRUE when the list MWD contains one or more list elements because an attempt to deliver a short message to an MS has failed with a cause of Absent Subscriber, and with the value FALSE otherwise.

The MWD, MCEF and MNRF are updated in the following way:

1a.	When a mobile terminated short message delivery fails due to the MS being temporarily absent (i.e. either IMSI DETACH flag is set or there is no response from the MS to a paging request), the SC address is inserted into the MWD list (if it is not already present) and the MNRF is set (if it is not already set), as described in section 10.

1b.	When a mobile terminated short message delivery fails due to the MS memory capacity being exceeded, the SC address is inserted into the MWD list (if it is not already present) and the MCEF is set (if it is not already set), as described in section 10.

1c.	If the MSIsdn used by the SC to address the recipient MS for alerting purposes is different from the MSIsdn-Alert of the MS (see section 3.2.7), the HLR returns the MSIsdn-Alert to the SC within the failure report, see "1c Failure report" in figures 03.40/15 and /16.

2a.	When either the HLR or VLR detects that the MS (with a non-empty MWD and the MCEF clear in the HLR and the MNRF set in the VLR) has recovered operation (e.g. has responded to a paging request), the HLR directly or on request of the VLR will invoke operations to alert the SCs within the MWD (see section 3.2.7 and section 10). Once the Alert SC operations have been invoked, the MNRF is cleared. After each SC is alerted by the HLR, the address for that SC is deleted from the MWD. If the MCEF is set in the HLR, the HLR clears the MNRF, but does not invoke operations to alert the SCs within the MWD and data are not cleared from the MWD.

2b.	When the HLR receives (via the MSC and the VLR) a notification that the MS (with a non-empty MWD and the MCEF set in the HLR) has memory capacity available to receive one or more short messages, the HLR will invoke operations to alert the SCs within the MWD (see section 3.2.7 and section 10). Once the Alert SC operations have been invoked, the MNRF is cleared in the VLR and the MCEF and MNRF are cleared in the HLR. After each SC is alerted by the HLR, the address for that SC is deleted from the MWD.

2c.	When the HLR receives from the SMS-GMSC a notification that a short message has been successfully delivered from an SC to an MS for which the MCEF is set and the MWD are not empty, the HLR will invoke operations to alert other SCs within the MWD (see section 3.2.7 and section 10). Once the Alert SC operations have been invoked, the MCEF and MNRF are cleared in the HLR. After each SC is alerted by the HLR, the address for that SC is deleted from the MWD. The SC which successfully delivered the message is also deleted from the MWD, if present.

2d.	When the HLR receives (via the MSC and the VLR) a notification that the MS has memory capacity available to receive one or more short messages but the MCEF is not set and the MWD are empty, the HLR acknowledges the notification but does not alert any service centre.

.c.3.2.7	Alert-SC

The Alert-SC is the service element, which may be provided by some GSM PLMNs, to inform the SC that an MS

1)	to which a delivery attempt has failed because the MS is not reachable or because the MS memory capacity was exceeded

and

2)	which is now recognized by the PLMN:

a)	to have resumed operation (e.g. to have responded to a paging request) or

b)	to have memory newly available (which implies that the mobile is reachable)

is again ready to receive one or more short messages. The SC may - on reception of an Alert-SC - initiate the delivery attempt procedure for the queued messages destined for this MS.

To each MS there may be allocated several MSIsdns. When the HLR is to alert an SC that an MS is again attainable it will use a specific MSIsdn value for this purpose; in this specification called MSIsdn-Alert.

Note:	Repeated delivery attempts from the SC may be of two types:

i)	A repeated delivery attempt because the SC has been informed that the MS is active and available to receive short messages.

ii)	An autonomous repeated delivery attempt by the SC.

The application of these two options is defined by the providers of the SC and the network.

.c.3.2.8	Options concerning MNRF, MCEF and MWD

Setting the Mobile-Station-Not-Reachable-Flag (MNRF) in the VLR is mandatory. It is also mandatory for the VLR to send the 'MS Present' message (see section 10) to the HLR when the MS has been detected as becoming active and then to clear MWF.

The Messages-Waiting-Data (MWD, the Mobile-Station-Not-Reachable-Flag (MNRF) and the Mobile-Station-Memory-Capacity-Exceeded-Flag (MCEF)) within the HLR are optional, but if one is implemented all must be implemented. This is linked to the transmission of the 'Alert SC' message.

The following describes what happens when a delivery fails.

Case 1: MWD, MNRF and MCEF are implemented in the HLR

In the case of a delivery failure (to an MS) with cause Absent Subscriber, the MSC requests the HLR to add, if needed, a new entry in the MWD with cause Absent Subscriber. This new entry contains the SC address. The HLR sets its copy of the MNRF. The SC is notified of the failure and also of the MWD setting in the HLR within the Report message (see section 10).

In the case of a delivery failure (to an MS) with cause Mobile Station Memory Capacity Exceeded, the MSC requests the HLR to add, if needed, a new entry in the MWD with cause Mobile Station Memory Capacity Exceeded. This new entry contains the SC address. The HLR sets the MCEF. The SC is notified of the failure and also of the MWD setting in the HLR within the Report message (see section 10).

If the HLR indicates that it is able to store the SC address, then the SC will receive an Alert SC message when the MS becomes active.

If the HLR indicates that it is unable to store the SC address (e.g. because MWD is full), then the only way to ensure delivery is for the SC to try to retransmit the message periodically.

When the HLR receives the MS Present message, if the MCEF is clear it sends an Alert SC message to the concerned SC and updates MWD.

When the HLR receives the MS Memory Capacity Available message, it sends an Alert SC message to the concerned SC, updates MWD and clears the MCEF.

Case 2: MWD, MNRF and MCEF are not implemented in the HLR

In the case of a delivery failure, the SC is notified that the HLR is unable to store its address in the MWD. The SC must retransmit the short message periodically in order to ensure delivery.

The HLR discards the MS Present message received from the VLR without any failure or error report.

The HLR discards the MS Memory Capacity Available message received from the MS via the MSC and the VLR without any failure or error report.

.c.3.2.9	Status report capabilities

The SMS also offers to the SC the capabilities of informing the MS of the status of a previously sent mobile originated short message.  The status of the message can be:

-	Successfully delivered to the SME

-	The SC was not able to forward the message to the SME.  The reason can be an error of permanent or temporary nature. Permanent errors can be e.g. validity period expired, invalid SME address.  Errors of temporary nature can be e.g. SC-SME connection being down, SME temporarily unavailable.

This is achieved by the SC returning a status report TPDU (SMS-STATUS-REPORT) to the originating MS when the SC has concluded the status of the short message.  The status report may be initiated by a status report request within the mobile originated short message.  The status report TPDU is treated as an SMS-DELIVER TPDU by the SC when it comes to delivery procedures e.g. the alerting mechanism.

The SC may also return to a non-MS SME the status of a mobile terminated short message.  This is however outside the scope of this specification.

The status report capabilities of the SMS are optional, i.e. the choice of whether to offer status report or not is left to the SC operator.

.c.3.2.10	Reply Path

Reply Path specified in this specification provides a way of both requesting and indicating a service centre's commitment to deliver a reply from the replying MS to the originating SME.

Annex 4 deals with MS procedures, which in general are outside the scope of GSM specifications.  However, for advanced use of the SMS, including both application level protocols and human responses, it is of vital importance to guarantee that a reply-supporting MS is able to reply on every SM, to every SME capable of receiving such reply short messages.

.c.3.3	Unsuccessful short message TPDU transfer SC -> MS

Unsuccessful message transfer SC -> MS may be caused by a variety of different errors. The description of the occurrence of the different errors and how to handle and transfer the error indications is given in GSM 04.08, GSM 04.11 and GSM 09.02.

The different error indications which the SMS-GMSC shall be capable of returning to the SC following an unsuccessful short message TPDU transfer SC-> MS, are given in Table 03.40/1. In some cases additional diagnostic information may be provided.

.c.3.3.1	Errors occurring during transfer of TPDU to MS

These errors are generally due to barring or unsupported service in the PLMN or MS.  An error indication is returned to the SC from the SMS-GMSC, but further diagnostic information from the MS will not be available.

.c.3.3.2	Errors occurring after TPDU arrives at MS

These errors may occur due to the MS not supporting optional short message service features, or in connection with a short message application.  An error indication shall be returned to the SC from the SMS-GMSC.  Additionally, a TPDU (SMS-DELIVER-REPORT) containing diagnostic information may be conveyed from the MS to the originating SC, transparently through the PLMN, by means defined in GSM 04.11 and GSM 09.02.  The sending of the diagnostic information is optional at the MS, but when it is sent, the PLMN shall convey the information to the SC, and the SC shall support reception of the information.

Error indication	S1)	Meaning

────────────────────────────────────────────────────────────────────

Unknown subscriber	P	The PLMN rejects the short message TPDU because there is not allocated an IMSI or a directory number for the mobile subscriber in the HLR (see GSM 09.02).

Teleservice not	P	The PLMN rejects the short message TPDU because the recipient

provisioned 		MS has no SMS subscription (see GSM 09.02).

Call barred	T	The PLMN rejects the short message TPDU due to barring of the MS (see GSM 09.02, description of the Barring supplementary service, GSM 02.04 and 03.11), and description of Operator Determined Barring, GSM 02.41 and 03.15).

Facility not	T	The VPLMN rejects the short message TPDU due to no provision

supported		of the SMS in the VPLMN (see GSM 09.02).

Absent subscriber	T	The PLMN rejects the short message TPDU due to

- no paging response, see GSM 04.08

- invocation of IMSI DETACH (see GSM 09.02), or

- roaming restrictions (see 'Roaming not allowed', GSM 09.02).

SMS lower layers	T	The PLMN rejects the short message TPDU due to MS not being

capabilities not		able to support the Short Message Service.

provisioned		The short message transfer attempt is rejected either due to information contained in the class-mark, or the MSC not being able to establish connection at SAPI = 3 (see GSM 04.08 and GSM 09.02).

Error in MS	T	The PLMN rejects the short message TPDU due to an error occurring within the MS at reception of a short message, e.g.lack of free memory capacity or protocol error.

Illegal Subscriber	P	The PLMN rejects the short message TPDU because the MS failed authentification

Illegal Equipment	P	The PLMN rejects the short message TPDU because the IMEI of the MS was black-listed in the EIR

System failure	T	The PLMN rejects the short message TPDU due to network or protocol failure others than those listed above (see GSM 09.02)

Memory Capacity	T	The MS rejects the short message since it has no memory

Exceeded		capacity available to store the message

1) : Status (Permanent or Temporary)

Table 03.40/1	Error indications related to mobile terminated short message transfer which may be transferred to the originating SC.

The relation between the two sets of error indications is given in the table 03.40/1. Each error is classified as either 'Temporary' or 'Permanent'. This classification gives an indication of whether or not it is probable that the MS becomes attainable within a reasonable period, and so provides the recommended action to be taken by the SC, i.e. either to store the message for later transfer, or to discard it.

.c.3.4	Unsuccessful short message TPDU transfer MS -> SC

The error indications related to mobile originated short message transfer which may be transferred to the originating MS are given in  GSM 04.11. In some cases additional diagnostic information may be provided.

.c.3.4.1	Errors occurring during transfer of TPDU to SC

These errors are generally due to barring or unsupported service in the PLMN.  An error indication is returned to the MS from the MSC, but further diagnostic information from the SC will not be available.

.c.3.4.2	Errors occurring after TPDU arrives at SC

These errors may occur due to the SC not supporting optional short message service features, or in connection with a short message application.  An error indication shall be returned to the MS from the MSC.  Additionally, a TPDU (SMS-SUBMIT-REPORT) containing diagnostic information may be conveyed from the SC to the originating MS, transparently through the PLMN, as defined in GSM 09.02 and GSM 04.11.  The sending of the diagnostic information is optional at the SC, but when it is sent, the PLMN shall convey the information to the MS, and the MS shall support reception of the information.

Note:	The SMS-SUBMIT-REPORT is part of the negative acknowledgement to the mobile originated short message, and is not part of the status report capabilities described in section 3.2.9.

.c.3.5	Use of Supplementary Services in combination with the Short Message Service

Only a sub-set of the Supplementary Services defined in GSM 02.04 and 03.11 may be used in combination with the Short Message Service. This sub-set comprises the following Supplementary Services:

All the 5 Barring services

.c.3.6	Applicability of Operator Determined Barring to the Short Message Service

The network feature Operator Determined Barring (see GSM 02.41) applies to the Short Message Service.

If a short message fails due to operator determined barring then an appropriate error cause is returned to the originator.

.c.3.7	Multiple short message transfer

To avoid the need for a mobile to be paged, authenticated etc. for each message waiting in the Service Centre, the SC may indicate to the GMSC that there are more messages to send. When this indication is given, MAP procedures are invoked such that this indication is passed to the VMSC, and the VMSC does not release the MS until all short messages waiting in the SC have been transferred.

.c.4	Network architecture

.c.4.1	Basic network structure

The exchange of messages between an MS and an SME involves the entities shown in Figure 03.40/4.

The basic network structure of the SMS is depicted in Figure 03.40/5.

│

│

SMS-GMSC /

SME              SC          SMS-IWMSC *      MSC           MS

┌────┐          ┌────┐          ┌────┐        ┌────┐       ┌────┐

│    │ ── .. ── │    │ ──────── │    │ ────── │    │ ───── │    │

└────┘          └────┘          └────┘        └────┘       └────┘

────     │    ────

│

Outside the scope     │    Inside the scope of the GSM

of the GSM            │    specifications

specifications        │

*) :	SMS-GMSC when the short message is transferred from the SC to the MS, SMS-IWMSC when the short message is transferred from the MS to the SC. The SC may be integrated with the SMS-GMSC/SMS-IWMSC.

Figure 03.40/4	Entities involved in the provision of SM MT and SM MO: SC, SMS-GMSC/SMS-IWMSC, MSC and MS.

The links of Figure 03.40/5 support the short message transfer in the following way:

-	message transfer on link 1 is described in section 5,

-	the operations performed on links 2 and 4 is described in  GSM 09.02,

-	message transfer on link 3 is described in section 4.2,

-	message transfer on link 5 is supported by protocol described in  GSM 04.11.

SC                 SMS-GMSC /             MSC             MS

SMS-IWMSC

┌────┐      1.        ┌────┐       3.      ┌────┐    5.    ┌────┐

│    │ ──────────── │    │ ─────────── │    │ ────── │    │

└────┘                └────┘               └────┘          └────┘

│                    │

2. │                    │ 4.

┌────┐               ┌────┐

│    │               │    │

└────┘               └────┘

HLR                  VLR

Figure 03.40/5	The main network structure serving as a basis for the short message transfer.

.c.4.2	Transfer on link 3

The link 3 is used to support communications between MSCs (MSC, SMS-GMSC, SMS-IWMSC). Two cases can be distinguished according to whether or not the MSCs are located in the same PLMN.

In the first case the link definition is left to the operators. For example this link may use:

-	PSPDN or

-	CCITT SS no 7 (according to GSM 09.02).

In the second case CCITT SS no 7 shall be used over link 3 according to  GSM 09.02, unless otherwise bilaterally agreed.

.c.5	Service Centre and PLMN interconnection

This specification deals with the SC only with regard to the interchange of messages between SC and MS. Only the requirements put upon the SC by the SMS functionality are specified in this specification.

.c.5.1	Service centre connection

One SC may be connected to several PLMNs, and may be connected to several MSCs (SMS-GMSCs or SMS-IWMSCs) within one and the same PLMN.

The SC is addressed from the mobile by an E.164 number in the numbering plan of the PLMN to which the SC is connected. This E.164 number shall uniquely identify the SC to that PLMN.

There may be an intermediate network between the PLMN and the SC; in this case the PLMN must autonomously make a connection to the SC using the SC address in this intermediate network.

No mandatory protocol between the SC and the MSC below the transfer layer is specified by GSM; this is a matter for agreement between SC and PLMN operators. However, Annex 1 provides an example protocol stack which could be used.

.c.5.2	Routing requirements

.c.5.2.1	Mobile terminated short message

The SC sends the short message to the SMS-GMSC. The SMS-GMSC interrogates the HLR to retrieve routing information necessary to forward the short message, and then sends the message to the relevant MSC, transiting other networks if necessary. The MSC then sends the short message to the MS.

.c.5.2.2	Mobile originated short message

The MS sends the short message to the MSC. The MS will always address the required SC by an E.164 address. The visited PLMN will route the message to the appropriate SMS-IWMSC in the SC's PLMN, transiting other networks if necessary.

.c.6	Service Centre functionality

In this specification, only the SC functionality related to the short message point-to-point service between the SC and the MS is specified.

.c.6.1	Service Centre capabilities

The SC should be capable of

-	submitting a short message to an MS, retaining the responsibility of the message until

1)	the report has been received, or

2)	the Validity-Period expires;

-	receiving a report from the PLMN;

-	receiving a short message from an MS;

-	returning a report to the PLMN for a previously received short message.

.c.6.2	SC functional requirements

The detailed functionality of the SC is outside the scope of this specification, and is for the SC operator to define. However, the following functional requirements are mandatory for all SCs in order to support the SM-TP (see section 9) towards the PLMN:

1)	To identify each SMS-DELIVER sent to an MS in a unique way, a time stamp value is included in the field TP-Service-Centre-Time-Stamp, TP-SCTS, of the SMS-DELIVER. The time stamp gives the time when the message arrived at the SC with the accuracy of a second. If two or more messages to the same MS arrive at the SC within one second, the SC shall modify the time stamp of those messages in such a way that

a)	all messages to the MS contain different time stamps

b)	the modification of the time stamps is kept to a minimum.

2)	The SC is only allowed to have one outstanding SMS-DELIVER (i.e. a message for which a report has not been received) to a specific MS at a given time.

3)	The SC shall be able to initiate overwriting of short messages previously received by the SC if requested by the same originating address (MS or any other source) by use of the same message type.

.c.7	MS functionality

In this specification, only the MS functionality related to the short message point-to-point service between the SC and the MS is specified.

.c.7.1	MS capabilities

The MS, when equipped for SMS, should be capable of

-	submitting a short message TPDU to an SC, retaining the responsibility of the message until

1) the report arrives from the network, or

2) a timer expires

-	receiving a short message TPDU from an SC

-	returning a delivery report to the network for a previously received short message

-	receiving a report from the network

-	notifying the network when it has memory capacity available to receive one or more short messages when it has previously rejected a short message because its memory capacity was exceeded

-	notifying the SC when a short message is intended to replace a short message the MS has previously submitted to the same destination address

It is recommended that an MS supporting both replying and automatic SC selection (as specified in section 2 of Annex 4) follows procedures specified in Annex 4 when replying to MT short messages with MO short messages.

It is recommended that an MS supporting a capability for requesting a reply path follows procedures specified in Annex 4.

.c.7.2	MS configuration

The reference configuration is assumed as in Figure 03.40/6, i.e. only the case where the terminal is integrated in the MS is considered.

┌─────┐

│ MT0 ├────┼

└─────┘

Um

Figure 03.40/6	Reference configuration of the MS which apply to the SMS.

Note:	It is foreseen that a terminal interface may be offered, e.g. for higher layer protocols, memory capacity reasons or to be able to type in mobile originated messages. This terminal interface is regarded as an implementation option, although, where offered, it must be based upon an R- or S-reference point. TS GSM 07.05 provides an example based on the R reference point.

.c.8	MSC functionality

The overall requirements to the MSC with respect to handling of the Short Message Service point-to-point is to cater for the routing and necessary intermediate buffering of the short messages.

.c.8.1	MSC functionality related to SM MT

.c.8.1.1	Functionality of the SMS-GMSC

When receiving a short message TPDU from the SC, the SMS-GMSC is responsible for the following operations:

-	reception of the short message TPDU,

-	inspection of the parameters,

(Note: The SMS-GMSC may be identical to the MSC),

if parameters are incorrect:

-	returning the appropriate error information to the SC in a failure report (see section 9 and 10)

if errors are not found within parameters:

-	interrogating the HLR ('sendRoutingInfoForShortMsg', see section 10); retrieving routing information or possible error information,

if HLR is returning error information:

-	returning the appropriate error information to the SC in a failure report (see section 9 and 10)

if no errors are indicated by the HLR:

-	transferring the short message TPDU to the MSC using the routing information obtained from the HLR ('forwardShortMessage', see section 10).

When receiving the report associated with the short message from the MSC (positive or negative outcome of 'forwardShortMessage', see section 10), the SMS-GMSC is responsible for the following operations:

if the report indicates successful delivery:

-	notifying the HLR of the successful delivery, which will cause the HLR to alert any service centres whose addresses are stored in the MWD for the MS.

if the report is a failure report indicating 'absent subscriber' (see section 3.3):

-	requesting the HLR to insert the address of the  originating SC into the MWD (if implemented) with cause Absent Subscriber ('setMessageWaitingData', see section 9 and 10),

-	establishing, where necessary, a link with the addressed SC (see section 5),

-	creating and sending the report to the SC (see section 9 and 10).

if the report is a failure report indicating 'MS memory capacity exceeded' (see section 3.3):

-	requesting the HLR to insert the address of the  originating SC into the MWD (if implemented) with cause MS Memory Capacity Exceeded ('setMessageWaitingData', see sections 9 and 10),

-	establishing, where necessary, a link with the addressed SC (see section 5),

-	creating and sending the report to the SC (see sections 9 and 10).

.c.8.1.2	Functionality of the MSC

When receiving a short message TPDU from the SMS-GMSC ('forwardShortMessage', see section 10), the MSC is responsible for the following operations:

-	reception of the short message TPDU,

-	retrieving information from the VLR ('sendInfoFor-MT-SMS', see section 10); location area address and, when appropriate, error information,

if errors are indicated by the VLR:

-	returning the appropriate error information to the SMS-GMSC in a failure report (negative outcome of 'forwardShortMessage' see section 10 and 11),

if no errors are indicated by the VLR:

-	transferring the short message to the MS (see  GSM 04.11).

When receiving a confirmation that the message is received by the MS (see GSM 04.11):

-	relaying the delivery confirmation to the SMS-GMSC in a delivery report (positive outcome of 'forwardShortMessage', see section 10 and 11).

When receiving a failure report of the short message transfer to the MS (see GSM 04.11):

-	returning the appropriate error information to the SMS-GMSC in a failure report (negative outcome of 'forwardShortMessage', see section 10).

When receiving a notification from the MS that it has memory available to receive one or more short messages (see  GSM 04.11):

-	relaying the notification to the VLR ('mSMemoryCapacityAvailable', see section 10).

if errors are indicated by the VLR:

-	returning the appropriate error information to the MS in a failure report (negative outcome of 'ReadyForSM', see section 10 and 11).

.c.8.2	MSC functionality related to SM MO

.c.8.2.1	Functionality of the MSC

When receiving a short message TPDU from the MS, the MSC is responsible for the following operations:

-	reception of the short message TPDU  (see  GSM 04.11)

-	retrieving information from the VLR ('sendInfoForMO-SMS', see section 10); the MSISDN of the MS and, when appropriate, error information. The retrieval of information from the VLR is followed by the VLR investigating the MWF (to be used in the alerting procedure, see section 10)

if errors are indicated by the VLR:

-	returning the appropriate error information to the MS in a failure report (negative outcome of 'sendInfoForMO-SMS' see section 10 and 11),

if no errors are indicated by the VLR:

-	inspection of the TPDU parameters,

if parameters are incorrect:

-	returning the appropriate error information to the MS in a failure report (see  GSM 04.11),

if no parameter errors are found:

-	examination of the destination address,

(NOTE: The SMS-IWMSC may be identical to the MSC),

-	transferring the short message TPDU to the SMS-IWMSC ('forwardShortMessage', see section 10).

When receiving the report of the short message from the SMS-IWMSC (positive or negative outcome of the 'forwardShortMessage', see section 10), the MSC is responsible for the following operations:

-	relaying the report to the MS (see  GSM 04.11).

.c.8.2.2	Functionality of the SMS-IWMSC

When receiving a short message TPDU from the MSC ('forwardShortMessage', see section 10), the SMS-IWMSC is responsible for the following operations:

-	reception of the short message TPDU,

-	establishing, where necessary, a link with the addressed SC (see section 5),

-	transferring the short message TPDU to the SC (if the address is valid).

If a report associated with the short message is received from the SC, the SMS-IWMSC is responsible for the following operations:

-	relaying of the report to the MSC (positive or negative outcome of 'forwardShortMessage', see section 10).

If a report associated with the short message is not received from the SC before a timer expires or if the SC address is invalid, the SMS-IWMSC is responsible for the following operations:

-	returning the appropriate error information to the MSC in a failure report (negative outcome of 'forwardShortMessage', see section 10).

The value of the timer is dependent on the protocol between the SC and the SMS-IWMSC.

.c.8.3	SMS-IWMSC functionality related to alerting

When receiving an alert from the HLR ('alertServiceCentre', see section 10), the SMS-IWMSC is responsible for the following operations:

-	inspect the SC address,

-	generate an RP-Alert-SC (see section 9),

-	transferring the RP-Alert-SC to the SC.

Note that if the SC address is not valid, then no further action will be taken.

.c.9	Protocols and protocol architecture

The protocol layers of the SMS are structured as shown in Figure 03.40/7.

SME           SC        SMS-GMSC /     MSC           MS

SMS-IWMSC

┌────┐       ┌────┐       ┌────┐       ┌────┐       ┌────┐

│    │       │    │       │    │       │    │       │    │

└────┘       └────┘       └────┘       └────┘       └────┘

┌────┐

- - - - - - - - - - - - - - - - - - -│    │

SM-AL               ┌────┐                                 │    │

│    │- - - - - - - - - - - - - - - -│    │

├────┤                                 ├────┤

SM-TL               │    │───────────────────────────────│    │

├────┤       ┌────┐       ┌────┐       ├────┤

SM-RL               │    │─────│    │─────│    │─────│    │

├────┤       ├────┤       ├────┤       ├────┤

SM-LL               │    │─────│    │─────│    │─────│    │

└────┘       └────┘       └────┘       └────┘

Figure 03.40/7	Protocol layer overview for the Short Message Service point-to-point.

This specification specifies the protocol at the SM-TL, the service offered by the SM-TL at the MS and the SC, and the service offered by the SM-RL at the SC.

.c.9.1	Protocol element features

.c.9.1.1	Octet and Bit transmission order

The octets are transmitted according to their individual numbering; the octet with the lowest number being transmitted first. The bits within each octet are transmitted according to their individual numbering also; the bits with the lowest internal number being transmitted first.

.c.9.1.2	Numeric and alphanumeric representation

For parameters within the TPDUs, there are four ways of numeric representation: Integer representation, octet, semi-octet and alphanumeric representation.

.c.9.1.2.1	Integer representation

Wherever the bits from a number of octets, complete or in fractions, are to represent an integer, the interpretation will be according to the following:

1)	Between octets: The octets with the lowest octet numbers will contain the most significant bits.

2)	Within an octet: The bits with the highest bit numbers will be the most significant.

Below is given an example of octet and bit representation and transmission order of an integer represented field.

Let the 2 rightmost bits of octet no 5, the complete octet no 6 and 7, and the 3 leftmost bits of octet no 8 represent an integer, as shown in Figure 03.40/8.

α)

7   6   5   4   3   2   1   0

Oct.no.

.           .   .   .   .   .   .   .

├───┼───┼───┼───┼───┼───┼───┼───┤

5       │                       │5b1│5b0│

├───┼───┼───┼───┼───┼───┼───┼───┤

6       │6b7│6b6│6b5│6b4│6b3│6b2│6b1│6b0│

├───┼───┼───┼───┼───┼───┼───┼───┤

7       │7b7│7b6│7b5│7b4│7b3│7b2│7b1│7b0│

├───┼───┼───┼───┼───┼───┼───┼───┤

8       │8b7│8b6│8b5│                   │

├───┼───┼───┼───┼───┼───┼───┼───┤

.           .   .   .   .   .   .   .

ß)

5b1 5b0 6b7 6b6 . . . .6b1 6b0 7b7 7b6 . . . .7b1 7b0 8b7 8b6 8b5

Γ)

┌─  *)

┌───────────┴───────────┐

│                       │

5b0 5b1│5b2 5b3 5b4 5b5 5b6 5b7│6b0 6b1 6b2 6b3 6b4 6b5 6b6 6b7

7b0 7b1 7b2 7b3 7b4 7b5 7b6 7b7│8b0 8b1 8b2 8b3 8b4│8b5 8b6 8b7

│                   │

└─────────┬─────────┘

│

└──  *)

*) :	Bits not representing the integer.

Figure 03.40/8.	21 bits from the octets 5, 6, 7, and 8 in a short message α) will represent an integer as shown in ß), and will be transmitted in an order as shown in Γ).

.c.9.1.2.2	Octet representation

A field which is octet represented, will always consist of a number of complete octets. Each octet within the field represents one decimal digit. The octets with the lowest octet numbers will contain the most significant decimal digits.

.c.9.1.2.3	Semi-octet representation

A field which is semi-octet represented, will consist of a number of complete octets and - possibly - one half octet. Each half octet within the field represents one decimal digit. The octets with the lowest octet numbers will contain the most significant decimal digits. Within one octet, the half octet containing the bits with bit numbers 0 to 3, will represent the most significant digit.

In the case where a semi-octet represented field comprises an odd number of digits, the bits with bit numbers 4 to 7 within the last octet are fill bits and shall always be set to '1111'.

Within each semi octet, the bits with the highest bit numbers will be the most significant.

Below is given an example:

Octet no:

├───┼───┼───┼───┼───┼───┼───┼───┤

│               │               │

n+1  │    Digit 2    │    Digit 1    │

│               │               │

├───┼───┼───┼───┼───┼───┼───┼───┤

│               │               │

n+2  │    Digit 4    │    Digit 3    │

│               │               │

├───┼───┼───┼───┼───┼───┼───┼───┤

│               │               │

n+3  │ 1   1   1   1 │    Digit 5    │

│               │               │

├───┼───┼───┼───┼───┼───┼───┼───┤

.c.9.1.2.4	Alphanumeric representation

A field which uses alphanumeric representation will consist of a number of 7-bit characters represented as the default alphabet defined in GSM 03.38.

.c.9.1.2.5	Address fields

Address fields used by SM-RL are specified in TS GSM 04.11 and 09.02.

Each address field of the SM-TL consists of the following sub-fields: An Address-Length field of one octet, a Type-of-Address field of one octet, and one Address-Value field of variable length; as shown below:

.

.

┌      ├───┼───┼───┼───┼───┼───┼───┼───┤

│   1  │                               │   Address-Length

│      ├───┼───┼───┼───┼───┼───┼───┼───┤

│   2  │                               │   Type-of-Address

│      ├───┼───┼───┼───┼───┼───┼───┼───┤   ┐

│   3  │                               │   │

│      ├───┼───┼───┼───┼───┼───┼───┼───┤   │

│   4  │                               │   │

│      ├───                         ───┤   │

│   5  │                               │   │

Addr.  ┤      ├───                         ───┤   ├ Address-Value

│                                          │

│       ...............................    │

│                                          │

│      ├───                         ───┤   │

│   µ  │                               │   │

└      ├───┼───┼───┼───┼───┼───┼───┼───┤   ┘

.

.

The Address-Length field is an integer representation of the number of useful semi-octets within the Address-Value field, i.e excludes any semi octet containing only fill bits.

The Type-of-Address field format is as follows:

├───────┼───────┼───────┼───────┼───────┼───────┼───────┼───────┤

│       │                       │                               │

│   1   │     Type-of-number    │ Numbering-plan-identification │

│       │                       │                               │

├───────┼───────┼───────┼───────┼───────┼───────┼───────┼───────┤

Type-of-number:

Bits		6 5 4

0 0 0		Unknown1)

0 0 1		International number2)

0 1 0		National number3)

0 1 1		Network specific number4)

1 0 0		Subscriber number5)

1 0 1		Alphanumeric, (coded according to Annex 2)

1 1 0		Abbreviated number

1 1 1		Reserved for extension

The MS will not interpret reserved values but will store them as received.

The SC may reject messages with a type of number containing a reserved value or one which is not supported.

1)	'Unknown' is used when the user or network has no a priori information about the numbering plan. In this case, the Address-Value field is organized according to the network dialling plan, e.g. prefix or escape digits might be present.

2)	The international format shall be accepted also when the message is destined to a recipient in the same country as the MSC.

3)	Prefix or escape digits shall not be included.

4)	'Network specific number' is used to indicate administration/service number specific to the serving network, e.g. used to access an operator.

5)	'Subscriber number' is used when a specific short number representation is stored in one or more SCs as part of a higher layer application. (Note that 'Subscriber number' shall only be used in connection with the proper PID referring to this application).

Numbering-plan-identification (applies for Type-of-number = 000,001,010)

Bits		3 2 1 0

0 0 0 0		Unknown

0 0 0 1		ISDN/telephone numbering plan (E.164/E.163)

0 0 1 1		Data numbering plan (X.121)

0 1 0 0		Telex numbering plan

1 0 0 0		National numbering plan

1 0 0 1		Private numbering plan

1 0 1 0		ERMES numbering plan (ETSI DE/PS 3 01-3)

1 1 1 1		Reserved for extension

All other values are reserved.

For Type-of-number = 101 bits 3,2,1,0 are reserved and shall be transmitted as 0000. Note that for addressing any of the entities SC, MSC or MS, Numbering-plan-identification = 0001 will always be used. However, for addressing the SME, any specified Numbering-plan-identification value may be used.

The MS will not interpret reserved values but will store them as received.

The SC may reject messages with a type of number containing a reserved value or one which is not supported.

Within the Address-Value field, either a semi-octet or an alphanumeric1) representation applies.

The maximum length of the full address field (Address-Length, Type-of-Address and Address-Value) is 12 octets.

1)	Applies only to addressing at the SM-TL.

.c.9.2	Service provided by the SM-TL

.c.9.2.1	General

The Short Message Transfer Layer (SM-TL) provides a service to the Short Message Application Layer (SM-AL).  This service enables the SM-AL to transfer short messages to its peer entity, receive short messages from its peer entity and receive reports about earlier requests for short messages to be transferred.

In order to keep track of messages and reports about those messages, primitives between the SM-AL and SM-TL contain a Short Message Identifier (SMI), which is a reference number for the message associated with the primitive.  This Short Message Identifier is mapped to and from the Short Message Identifier used between the SM-TL and the Short Message Relay Layer (SM-RL).  The Short Message Identifier is not carried between entities and therefore a given message may have different SMIs at the MS and SC sides (see section 9.3.1 below).

The SM-TL communicates with its peer entity by the protocol described in the following sections.

.c.9.2.2	PDU Type repertoire at SM-TL

The SM-TL comprises the following six PDUs:

SMS-DELIVER, conveying a short message from the SC to the MS

SMS-DELIVER-REPORT, conveying a failure cause (if necessary)

SMS-SUBMIT, conveying a short message from the MS to the SC.

SMS-SUBMIT-REPORT, conveying a failure cause (if necessary)

SMS-STATUS-REPORT, conveying a status report from the SC to the MS

SMS-COMMAND, conveying a command from the MS to the SC.

.c.9.2.2.1	SMS-DELIVER type

Basic elements of the SMS-DELIVER type:

Abbr.	Reference	P1)	R2)	Description

────────────────────────────────────────────────────────────────────

TP-MTI	TP-Message-Type-Indicator	M	2b	Parameter describing the message type.

TP-MMS	TP-More-Messages-to-Send	M	b	Parameter indicating whether or not there are more messages to send

TP-RP	TP-Reply-Path	M	b	Parameter indicating that Reply Path exists.

TP-SRI	TP-Status-Report-	O	b	Parameter indicating if the SME

Indication			has requested a status report.

TP-OA	TP-Originating-Address	M	2-12o	Address of the originating SME.

TP-PID	TP-Protocol-Identifier	M	o	Parameter identifying the above layer protocol, if any

TP-DCS	TP-Data-Coding-Scheme	M	o	Parameter identifying the coding scheme within the TP-User-Data.

TP-SCTS	TP-Service-Centre-Time-	M	7o	Parameter identifying time when

Stamp			the SC received the message.

TP-UDL	TP-User-Data-Length	M	I	Parameter indicating the length of the TP-User-Data field to follow.

TP-UD	TP-User-Data	M	3)

1)	Provision;	Mandatory (M) or Optional (O).

2)	Representation;	Integer (I), bit (b), 2 bits (2b), Octet (o), 7 octets (7o), 2-12 octets (2-12o)

3)	Dependent on the TP-DCS

Layout of SMS-DELIVER:

Bit no.        7   6   5   4   3   2   1   0

┌───┬───┬───┬───┬───┬───┬───┬───┐TP-MTI, TP-MMS,

Oct.no.   1  │                               │TP-SRI, TP-RP

├───┼───┼───┼───┼───┼───┼───┼───┤    ┐

1  │                               │    │

├───                         ───┤    │

2  │                               │    │

├───                         ───┤    │

├   TP-OA

max 12        ...............................     │

│

├───                         ───┤    │

│                               │    │

├───┼───┼───┼───┼───┼───┼───┼───┤    ┘

1  │                               │        TP-PID

├───┼───┼───┼───┼───┼───┼───┼───┤

1  │                               │        TP-DCS

├───┼───┼───┼───┼───┼───┼───┼───┤    ┐

1  │                               │    │

├───                         ───┤    │

2  │                               │    │

├───                         ───┤    │

3  │                               │    │

├───                         ───┤    │

4  │                               │    ├   TP-SCTS

├───                         ───┤    │

5  │                               │    │

├───                         ───┤    │

6  │                               │    │

├───                         ───┤    │

7  │                               │    │

├───┼───┼───┼───┼───┼───┼───┼───┤    ┘

1  │                               │        TP-UDL

├───┼───┼───┼───┼───┼───┼───┼───┤    ┐

1  │                               │    │

├───                         ───┤    │

│                               │    │

├   TP-UD

max 140           . . . . . . . . . . . .         │

│                               │    │

├───                         ───┤    │

│                               │    │

└───┴───┴───┴───┴───┴───┴───┴───┘    ┘

Note:	Any unused bits will be set to zero by the sending entity and will be ignored by the receiving entity.

.c.9.2.2.1a	SMS-DELIVER-REPORT type

Basic elements of the SMS-DELIVER-REPORT type:

Abbr.	Reference	P1)	R2)

────────────────────────────────────────────────────────────────────

TP-MTI	TP-Message-Type-Indicator	M	2b	Parameter describing the message 				type

TP-FCS	TP-Failure-Cause 	M	I	Parameter indicating the reason

for SMS-DELIVER failure

1)	Provision:	Mandatory (M) or Optional (O)

2)	Representation:	Integer (I), bit (b), 2bits (2b), octet (o)

Layout of SMS-DELIVER-REPORT:

Bit no.      7  6  5  4  3  2  1  0

Oct. No.    ┌──┬──┬──┬──┬──┬──┬──┬──┐

1 │  │  │  │  │  │  │  │  │ TP-MTI

├──┼──┼──┼──┼──┼──┼──┼──┤

1 │  │  │  │  │  │  │  │  │ TP-FCS

└──┴──┴──┴──┴──┴──┴──┴──┘

The SMS-DELIVER-REPORT TPDU is carried as a RP-User-Data element within a RP-ERROR PDU, and is part of a negative acknowledgement to a SMS-DELIVER or SMS-STATUS-REPORT.

Bits 7 - 2 in octet 1 are presently unused and the sender shall set them to zero.  If any of these bits is non-zero, the receiver shall not examine the other field and shall treat the TP-Failure-Cause as "Unspecified error cause".

.c.9.2.2.2	SMS-SUBMIT type

Basic elements of the SMS-SUBMIT type:

Abbr.	Reference	P1)	R2)	Description

────────────────────────────────────────────────────────────────────

TP-MTI	TP-Message-Type-Indicator	M	2b	Parameter describing the message type.

TP-VPF	TP-Validity-Period-Format	M	2b	Parameter indicating whether or not the TP-VP field is present.

TP-RP	TP-Reply-Path	M	I	Parameter indicating the request for Reply Path.

TP-SRR	TP-Status-Report-Request	O	b	Parameter indicating if the MS is requesting a status report.

TP-MR	TP-Message-Reference	M	I	Parameter identifying the SMS-SUBMIT.

TP-DA	TP-Destination-Address	M	2-12o	Address of the destination SME.

TP-PID	TP-Protocol-Identifier	M	o	Parameter identifying the above layer protocol, if any

TP-DCS	TP-Data-Coding-Scheme	M	I	Parameter identifying the coding scheme within the TP-User-Data.

TP-VP	TP-Validity-Period	O	o/7o	Parameter identifying the time from where the message is no longer valid.

TP-UDL	TP-User-Data-Length	M	I	Parameter indicating the length of the TP-User-Data field to follow.

TP-UD	TP-User-Data	M	3)

1)	Provision;	Mandatory (M) or Optional (O).

2)	Representation;	Integer (I),  bit (b), 2 bits (2b), Octet (o), 7 octets (7o), 2-12 octets (2-12o).

3)	Dependent on the TP-DCS

Layout of SMS-SUBMIT:

Bit no.        7   6   5   4   3   2   1   0

┌───┬───┬───┬───┬───┬───┬───┬───┐TP-MTI, TP-VPF,

Oct.no.   1  │                               │TP-SRR, TP-RP

├───┼───┼───┼───┼───┼───┼───┼───┤

1  │                               │     TP-MR

├───┼───┼───┼───┼───┼───┼───┼───┤    ┐

1  │                               │    │

├───                         ───┤    │

2  │                               │    │

├───                         ───┤    │

├   TP-DA

max 12       .................................    │

│

├───                         ───┤    │

│                               │    │

├───┼───┼───┼───┼───┼───┼───┼───┤    ┘

1  │                               │        TP-PID

├───┼───┼───┼───┼───┼───┼───┼───┤

1  │                               │        TP-DCS

├───┼───┼───┼───┼───┼───┼───┼───┤    ┐

1  │                               │    │

├───                         ───┤    │

│

1 or 7            . . . . . . . . . . . .         ├   TP-VP

│

├───                         ───┤    │

│                               │    │

├───┼───┼───┼───┼───┼───┼───┼───┤    ┘

1  │                               │        TP-UDL

├───┼───┼───┼───┼───┼───┼───┼───┤    ┐

1  │                               │    │

├───                         ───┤    │

│                               │    │

├   TP-UD

max 140           . . . . . . . . . . . .         │

│                               │    │

├───                         ───┤    │

│                               │    │

└───┴───┴───┴───┴───┴───┴───┴───┘    ┘

Note:	Any unused bits will be set to zero by the sending entity and will be ignored by the receiving entity.

.c.9.2.2.2a	SMS-SUBMIT-REPORT type

Basic elements of the SMS-SUBMIT-REPORT type:

Abbr.	Reference	P1)	R2)

────────────────────────────────────────────────────────────────────

TP-MTI	TP-Message-Type-Indicator	M	2b	Parameter describing the message 				type

TP-FCS	TP-Failure-Cause	M	I	Parameter indicating the reason

for SMS-SUBMIT failure

1)	Provision:	Mandatory (M) or Optional (O)

2)	Representation:	Integer (I), bit (b), 2bits (2b), octet (o)

Layout of SMS-SUBMIT-REPORT:

Bit no.      7  6  5  4  3  2  1  0

Oct. No.    ┌──┬──┬──┬──┬──┬──┬──┬──┐

1 │  │  │  │  │  │  │  │  │ TP-MTI

├──┼──┼──┼──┼──┼──┼──┼──┤

1 │  │  │  │  │  │  │  │  │ TP-FCS

└──┴──┴──┴──┴──┴──┴──┴──┘

The SMS-SUBMIT-REPORT TPDU is carried as a RP-User-Data element within a RP-ERROR PDU, and is part of a negative acknowledgement to a SMS-SUBMIT or SMS-COMMAND.

Bits 7 - 2 in octet 1 are presently unused and the sender shall set them to zero.  If any of these bits is non-zero, the receiver shall not examine the other field and shall treat the TP-Failure-Cause as "Unspecified error cause".

.c.9.2.2.3	SMS-STATUS-REPORT type

Basic elements of the SMS-STATUS-REPORT type:

Abbr.	Reference	P1)	R2)	Description

────────────────────────────────────────────────────────────────────

TP-MTI	TP-Message-Type-	M	2b	Parameter describing the message type

Indicator

TP-MR	TP-Message-Reference	M	I	Parameter identifying the previously submitted SMS-SUBMIT

TP-MMS	TP-More-Messages-	M	b	Parameter indicating whether or not

to-SEND			there are more messages to send

TP-RA	TP-Recipient-Address	M	2-12o	Address of the recipient of the previously submitted mobile originated short message

TP-SCTS	TP-Service-Centre-	M	7o	Parameter identifying time when the SC

Time-Stamp			received the previously sent SMS-SUBMIT

TP-DT	TP-Discharge-Time	M	7o	Parameter identifying the time associated with a particular TP-ST outcome

TP-ST	TP-Status	M	o	Parameter identifying the status of the previously sent mobile originated short message

1)	Provision: Mandatory (M) or Optional (O)

2)	Representation: Integer (I), bit (b), 2 bits (2b), Octet (o), 7 octets (7o), 2-12 octets (2-12o)

Layout of SMS-STATUS-REPORT:

Bit no.        7   6   5   4   3   2   1   0

┌───┬───┬───┬───┬───┬───┬───┬───┐

Oct.no.   1  │                               │   TP-MTI, TP-MMS

├───┼───┼───┼───┼───┼───┼───┼───┤

1  │                               │   TP-MR

├───┼───┼───┼───┼───┼───┼───┼───┤      ┐

1  │                               │      │

├───                         ───┤      │

2  │                               │      │

├───                         ───┤      ├─   TP-RA

│

max 12       .................................      │

│

├───                         ───┤      │

│                               │      │

├───┼───┼───┼───┼───┼───┼───┼───┤    ┐ ┘

1  │                               │    │

├───                         ───┤    │

2  │                               │    │

├───                         ───┤    │

3  │                               │    │

├───                         ───┤    │

4  │                               │    ├─   TP-SCTS

├───                         ───┤    │

5  │                               │    │

├───                         ───┤    │

6  │                               │    │

├───                         ───┤    │

7  │                               │    │

├───┼───┼───┼───┼───┼───┼───┼───┤    ┘

1  │                               │       ┐

├───                         ───┤       │

2  │                               │       │

├───                         ───┤       │

3  │                               │       │

├───                         ───┤       │

4  │                               │       ├─   TP-RCT

├───                         ───┤       │

5  │                               │       │

├───                         ───┤       │

6  │                               │       │

├───                         ───┤       │

7  │                               │       │

├───┼───┼───┼───┼───┼───┼───┼───┤       ┘

1  │                               │   TP-ST

└───┴───┴───┴───┴───┴───┴───┴───┘

Note:	Any unused bits will be set to zero by the sending entity and will be ignored by the receiving entity.

.c.9.2.2.4	SMS-COMMAND type

Basic elements of the SMS-COMMAND type:

Abbr.	Reference	P1)	R2)	Description

────────────────────────────────────────────────────────────────────

TP-MTI	TP-Message-Type-	M	2b	Parameter describing the

Indicator			message type

TP-MR	TP-Message	M	I	Parameter identifying

Reference			the SMS-COMMAND

TP-PID	TP-Protocol-	M	o	Parameter identifying the

Identifier			above layer protocol, if any

TP-CT	TP-Command-Type	M	o	Parameter specifying which operation is to be performed on a SM

TP-MN	TP-Message-Number	M3)	o	Parameter indicating which

SM in the SC to operate on

TP-CDL	TP-Command-Data	M	o	Parameter indicating the length

Length			of the TP-CD field in octets

TP-CD	TP-Command-Data	O	o	Parameter containing user data

1)	Provision:	Mandatory (M) or Optional (O)

2)	Representation:	Integer (I), bit (b), 2bits (2b), octet (o)

3)	For TS-commands which are not for a specific SM this field shall be ignored when received. It's value is of no concern but the field must be present to maintain the structure.

Layout of SMS-COMMAND:

Bit no.      7  6  5  4  3  2  1  0

Oct.No.     ┌──┬──┬──┬──┬──┬──┬──┬──┐

1 │  │  │  │  │  │  │  │  │ TP-MTI

├──┼──┼──┼──┼──┼──┼──┼──┤

1 │  │  │  │  │  │  │  │  │ TP-MR

├──┼──┼──┼──┼──┼──┼──┼──┤

1 │  │  │  │  │  │  │  │  │ TP-PID

├──┼──┼──┼──┼──┼──┼──┼──┤

1 │  │  │  │  │  │  │  │  │ TP-CT

├──┼──┼──┼──┼──┼──┼──┼──┤

1 │  │  │  │  │  │  │  │  │ TP-MN

├──┼──┼──┼──┼──┼──┼──┼──┤

1 │  │  │  │  │  │  │  │  │ TP-CDL

.......................

TP-CD

max 157     └──┴──┴──┴──┴──┴──┴──┴──┘

.c.9.2.3	Definition of the TPDU parameters

.c.9.2.3.1	TP-Message-Type-Indicator (TP-MTI)

The TP-Message-Type-Indicator is a 2-bit field, located within bits no 0 and 1 of the first octet of SMS-DELIVER, SMS-SUBMIT, SMS-STATUS-REPORT or SMS-COMMAND and to be given the following values:

bit0	bit1	Message type

Bit no 0,1:	0	0	SMS-DELIVER (in the direction SC to MS)

0	0	SMS-DELIVER REPORT (in the direction MS to SC)

0	1	SMS-STATUS-REPORT (in the direction SC to MS)

0	1	SMS-COMMAND (in the direction MS to SC)

1	0	SMS-SUBMIT (in the direction MS to SC)

1	0	SMS-SUBMIT-REPORT (in the direction SC to MS)

1	1	Reserved

.c.9.2.3.2	TP-More-Messages-to-Send (TP-MMS)

The TP-More-Messages-to-Send is a 1-bit field, located within bit no 2 of the first octet of SMS-DELIVER and SMS-STATUS-REPORT, and to be given the following values:

Bit no 2:		0		More messages are waiting for the MS in this SC

1		No more messages are waiting for the MS in this SC

Note:	In the case of SMS-STATUS-REPORT this parameter refers to messages waiting for the mobile to which the status report is sent. The term message in this context refers to SMS-messages or status reports.

.c.9.2.3.3	TP-Validity-Period-Format (TP-VPF)

The TP-Validity-Period-Format is a 2-bit field, located within bit no 3 and 4 of the first octet of SMS-SUBMIT, and to be given the following values:

bit3	bit4

Bit no 3,4:	0	0	TP-VP field not present

0	1	TP-VP field present and integer represented (relative)

1	0	Reserved

1	1	TP-VP field present and semi-octet represented (absolute)

Any reserved values may be rejected by the SC.

.c.9.2.3.4	TP-Status-Report-Indication (TP-SRI)

The TP-Status-Report-Indication is a 1-bit field, located within bit no. 5 of the first octet of SMS-DELIVER, and to be given the following values:

Bit no. 5:	0		A status report will not be returned to the SME

1		A status report will be returned to the SME

.c.9.2.3.5	TP-Status-Report-Request (TP-SRR)

The TP-Status-Report-Request is a 1-bit field, located within bit no. 5 of the first octet of SMS-SUBMIT, and to be given the following values:

Bit no. 5:	0		A status report is not requested

1		A status report is requested

.c.9.2.3.6	TP-Message-Reference (TP-MR)

The TP-Message-Reference field gives an integer representation of a reference number of the SMS-SUBMIT or SMS-COMMAND submitted to the SC by the MS. The MS increments TP-Message-Reference by 1 for each SMS-SUBMIT or SMS-COMMAND being submitted. The value to be used for each SMS-SUBMIT is obtained by reading the Last-Used-TP-MR value from the SMS Status data field in the SIM (see TS GSM 11.11) and incrementing this value by 1. After each SMS-SUBMIT has been submitted to the network, the Last-Used-TP-MR value in the SIM is updated with the TP-MR that was used in the SMS-SUBMIT operation. The reference number may possess values in the range 0 to 255. The value in the TP-MR assigned by the MS is the same value which is received at the SC.

A phase 2 or later ME using a phase 1 SIM cannot read or update the TP-Message-Reference from/to the SIM, and so the ME shall always retain the Last-Used-TP-MR value in its own memory, to be used only in the case of a phase 1 SIM.

The SMS-STATUS-REPORT also contains a TP-Message-Reference field. The value sent to the MS will be the same as the TP-Message-Reference value generated by the MS in the earlier SMS-SUBMIT or SMS-COMMAND to which the status report relates.

.c.9.2.3.7	TP-Originating-Address (TP-OA)

The TP-Originating-Address field is formatted according to the formatting rules of address fields.

.c.9.2.3.8	TP-Destination-Address (TP-DA)

The TP-Destination-Address field is formatted according to the formatting rules of address fields.

.c.9.2.3.9	TP-Protocol-Identifier (TP-PID)

The TP-Protocol-Identifier parameter serves the purposes indicated in section 3.2.3. It consists of one octet, and the bits in the octet are used as follows:

The MS will not interpret reserved or unsupported values but shall store them as received.

The SC may reject messages with a TP-Protocol-Identifier containing a reserved value or one which is not supported.

bits		usage

7   6

0   0	Assigns bits 0..5 as defined below

0   1	Assigns bits 0..5 as defined below

1   0	reserved

1   1	Assigns bits 0-5 for SC specific use

In the case where bit 7 = 0 and bit 6 = 0,

bit 5	indicates telematic interworking:

value = 0  :  no interworking, but SME-to-SME protocol

value = 1  :  telematic interworking

In the case of telematic interworking, the following five bit patterns in bits 4..0 are used to indicate different types of telematic devices:

4...0

00000	implicit - device type is specific to this SC, or can be concluded on the basis of the address

00001	telex (or teletex reduced to telex format)

00010	group 3 telefax

00011	group 4 telefax

00100	voice telephone (i.e. conversion to speech)

00101	ERMES (European Radio Messaging System)

00110	National Paging system (known to the SC)

00111	Videotex (T.100/T.101)

01000	teletex, carrier unspecified

01001	teletex, in PSPDN

01010	teletex, in CSPDN

01011	teletex, in analog PSTN

01100	teletex, in digital ISDN

01101	UCI (Universal Computer Interface, ETSI DE/PS 3 01-3)

01110..01111	(reserved, 2 combinations)

10000	a message handling facility (known to the SC)

10001	any public X.400-based message handling system

10010..10111	(reserved, 6 combinations)

11000..11110	values specific to each SC, usage based on mutual agreement between the SME and the SC (7 combinations available for each SC)

11111	A GSM mobile station. The SC converts the SM from the received TP-Data-Coding-Scheme to any data coding scheme supported by that MS (e.g. the default).

If bit 5 has value 1 in an SMS-SUBMIT PDU, it indicates that the SME is a telematic device of a type which is indicated in bits 4..0, and requests the SC to convert the SM into a form suited for that device type. If the destination network is ISDN, the SC must also select the proper service indicators for connecting to a device of that type.

If bit 5 has value 1 in an SMS-DELIVER PDU, it indicates that the SME is a telematic device of a type which is indicated in bits 4..0.

If bit 5 has value 0 in an SMS-DELIVER PDU, the value in bits 4..0 identifies the SM-AL protocol being used between the SME and the MS.

Note that for the straightforward case of simple MS-to-SC short message transfer the Protocol Identifier is set to the value 0.

In the case where bit 7 = 0, bit 6 = 1, bits 5..0 are used as defined below

5....0

000000	Short Message Type 0

000001	Replace Short Message Type 1

000010	Replace Short Message Type 2

000011	Replace Short Message Type 3

000100	Replace Short Message Type 4

000101	Replace Short Message Type 5

000110	Replace Short Message Type 6

000111	Replace Short Message Type 7

001000..011110	Reserved

011111	Return Call Message

100000..111111	Reserved

A short message type 0 indicates that the ME must acknowledge receipt of the short message but may discard its contents.

The Replace Short Message feature is optional for the ME and the SIM but if implemented it shall be performed as described here.

For MT short messages, on receipt of a short message from the SC, the MS shall check to see if the associated Protocol Identifier contains a Replace Short Message Type code.

If such a code is present, then the MS will check the associated SC address and originating address and replace any existing stored message having the same Protocol Identifier code, SC address and originating address with the new short message and other parameter values. If there is no message to be replaced, the MS shall store the message in the normal way.

If a Replace Short Message Type code is not present then the MS will store the message in the normal way.

In MO short messages the SC reacts similarly but only the address of the originating MS or any other source is checked.

A Return Call Message indicates to the MS to inform the user that a call (e.g. a telephone call) can be established to the address specified within the TP-OA. The RP-OA contains the address of the SC  as usual. The message content (if present) gives displayable information (e.g. the number of waiting voice messages). The message is handled in the same way as all other messages of the Replace Short Message Types.

.c.9.2.3.10	TP-Data-Coding-Scheme (TP-DCS)

The TP-Data-Coding-Scheme is defined in GSM 03.38.

.c.9.2.3.11	TP-Service-Centre-Time-Stamp (TP-SCTS)

The TP-Service-Centre-Time-Stamp field is given in semi-octet representation, and represents the local time in the following way:

Year:	Month:	Day:	Hour:	Minute:	Second:	Time Zone

Digits:	2	2	2	2	2	2	2

(Semi-octets)

The Time Zone indicates the difference, expressed in quarters of an hour, between the local time and GMT. In the first of the two semi-octets, the first bit (bit 3 of the seventh octet of the TP-Service-Centre-Time-Stamp field) represents the algebraic sign of this difference (0 : positive,  1 : negative).

The Service-Centre-Time-Stamp, and any other times coded in this format, represents the time local to the sending entity.  The Time Zone code enables the receiver to calculate the equivalent time in GMT from the other semi-octets in the Service-Centre-Time-Stamp, or indicate the time zone (GMT, GMT+1H etc.), or perform other similar calculations as required by the implementation.

.c.9.2.3.12	TP-Validity-Period

The TP-Validity-Period field is given in either integer or semi-octet representation. In the first case, the TP-Validity-Period comprises 1 octet, giving the length of the validity period, counted from when the SMS-SUBMIT is received by the SC. In the second case, the TP-Validity-Period comprises 7 octets, giving the absolute time of the validity period termination.

In the first case, the representation of time is as follows:

TP-VP value	Validity period value

────────────────────────────────────────────────────────────────────

0 to 143	(TP-VP + 1) x 5 minutes (i.e. 5 minutes intervals up to 12 hours)

144 to 167	12 hours + ((TP-VP -143) x 30 minutes)

168 to 196	(TP-VP - 166) x 1 day

197 to 255	(TP-VP - 192) x 1 week

In the second case, the representation of time is identical to the representation of the TP-Service-Centre-Time-Stamp.

.c.9.2.3.13	TP-Discharge-Time (TP-DT)

The TP-Discharge-Time field indicates the time at which a previously submitted SMS-SUBMIT was successfully delivered to or attempted to deliver to the recipient SME or disposed of by the SC.

In the case of "transaction completed" the time shall be the time of the completion of the transaction. In the case of "SC still trying to transfer SM" the time shall be the time of the last transfer attempt. In the case of "permanent or temporary error - SC not making any more transfer attempts" the time shall be the time of either the last transfer attempt or the time at which the SC disposed of the SM according to the Status outcome in TP-ST.

The TP-Discharge-Time is given in semi-octet representation in a format identical to the TP-SCTS.

.c.9.2.3.14	TP-Recipient-Address (TP-RA)

The TP-Recipient-Address field indicates the address of the SME that was the destination of the previously submitted mobile originated short message being subject to the status report. The field is formatted according to the formatting rules of address fields.

.c.9.2.3.15	TP-Status (TP-ST)

The TP-Status field indicates the status of a previously submitted SMS-SUBMIT. It consists of one octet and the bits in the octet are used as follows:

The MS will not interpret any reserved values but shall store them as received.

bits		value/usage

7		0	Bits 0..6 as defined below

6....0	Indicate whether the previously submitted short message was successfully forwarded to the SME, or whether an error condition has been encountered, as follows:

Short message transaction completed

0000000			Short message received by the SME

0000001			Short message forwarded by the SC to the SME

0000010			Short message replaced by the SC

Reserved values

0000011..0001111	Reserved

0010000..0011111	Values specific to each SC

Temporary error, SC still trying to transfer SM

0100000			Congestion

0100001			SME busy

0100010			No response from SME

0100011			Service rejected

0100100			Quality of service not available

0100101			Error in SME

0100110..0101111	Reserved

0110000..0111111	Values specific to each SC

Permanent error, SC is not making any more transfer attempts

1000000			Remote procedure error

1000001			Incompatible destination

1000010			Connection rejected by SME

1000011			Not obtainable

1000100			Quality of service not available

1000101			No interworking available

1000110			SM Validity Period Expired

1000111			SM Deleted by originating SME

1001000			SM Deleted by SC Administration

1001001			SM does not exist (The SM may have previously existed in the SC but the SC no longer has knowledge of it or the SM may never have previously existed in the SC)

1001010..1001111	Reserved

1010000..1011111	Values specific to each SC

Temporary error, SC is not making any more transfer attempts

1100000			Congestion

1100001			SME busy

1100010			No response from SME

1100011			Service rejected

1100100			Quality of service not available

1100101			Error in SME

1100110..1101001	Reserved

1101010..1101111	Reserved

1110000..1111111	Values specific to each SC

bits				value/usage

7				1		Bits 0..6 as defined below

6			0		Bits 5..0 as defined below

1		Reserved

5		0		SMS-COMMAND response result code in bits 0 to 4

1		Reserved

4...0			SMS-COMMAND response result code

00000		Positive acknowledge (command actioned)

00001		Negative acknowledge (command cannot be actionend)

00010..01111	Reserved

10000..11111	Values specific for each SC

Note:	In the case where the SMS command is an Enquiry the acknowledgement shall be a TP-Status-Report with bit 7 in TP-ST set to 0 and bits 0...6 indicate the result. If the Enquiry command cannot be actioned than the SMS command response code Negative acknowledge shall be sent. The Positive acknowledge response code does not apply to Enquiry.

.c.9.2.3.16	TP-User-Data-Length (TP-UDL)

If the TP-User-Data is coded using the default alphabet, the TP-User-Data-Length field gives an integer representation of the number of characters within the TP-User-Data field to follow.

If the TP-User-Data is coded using 8-bit data, the TP-User-Data-Length field gives an integer representation of the number of octets within the TP-User-Data field to follow.

For other Data Coding Schemes, see GSM 03.38.

.c.9.2.3.17	TP-Reply-Path (TP-RP)

The TP-Reply-Path is a 1-bit field, located within bit no 7 of the first octet of both SMS-DELIVER and SMS-SUBMIT, and to be given the following values:

Bit no 7:	0			TS-Reply-Path parameter is not set in this SMS-SUBMIT/DELIVER

1			TS-Reply-Path parameter is set in this SMS-SUBMIT/DELIVER

Please refer to Annex 4 for details about the Reply procedures.

.c.9.2.3.18	TP-Message-Number (TP-MN)

The TP-Message-Number is an 8-bit field allowing an MS to refer uniquely to an SM in the SC which that MS has previously submitted.

.c.9.2.3.19	TP-Command-Type (TP-CT)

The TP-Command-Type is an 8-bit field specifying the type of operation that the SC is to perform. It has the following values:

00000000			Enquiry relating to previously submitted short message

00000001			Cancel Status Report Request relating to previously submitted short message

00000010			Delete previously submitted Short Message

00000011			Enable Status Report Request relating to previously submitted short message

00000100..00011111 	Reserved

11100000..11111111 	Values specific for each SC

A negative acknowledgement may be returned by the SC for any  reserved or unsupported value.

.c.9.2.3.20	TP-Command-Data-Length (TP-CDL)

The TP-Command-Data-Length field is used to indicate the number of octets contained within the TP-Command-Data-field. If this field is set to zero, the TP-Command-Data field will not be present.

.c.9.2.3.21	TP-Command-Data (TP-CD)

The TP-Command-Data field contains data relating to the operation requested by the MS which is to be performed at the SC. The maximum length of this field is 157 octets. The usage and provision of the optional TP-Command-Data field will be determined by the function selected by the TP-Command-Type field.

.c.9.2.3.22	TP-Failure-Cause (TP-FCS)

The TP-Failure-Cause field is used to report the reason for failure to transfer or process a short message.  It consists of a single octet used as follows:

┌──────────┬───────────────────────────────────────────────────────┐

│ TP-FCS   │ Meaning                                      When used│

│  Value   │                                               ┌───┬───┤

│  (Hex)   │                                               │MO │MT │

├──────────┼───────────────────────────────────────────────┼───┼───┤

│00 - 7F   │ Reserved                                      │   │   │

│          │                                               │   │   │

│80 - 8F   │ TP-PID errors                                 │   │   │

│80        │ Telematic interworking not supported          │ x │   │

│81        │ Short message Type 0 not supported            │ x │ x │

│82        │ Cannot replace short message                  │ x │ x │

│83 - 8E   │ Reserved                                      │   │   │

│8F        │ Unspecified TP-PID error                      │ x │ x │

│          │                                               │   │   │

│90 - 9F   │ TP-DCS errors                                 │   │   │

│90        │ Data coding scheme (alphabet) not supported   │ x │   │

│91        │ Message class not supported                   │   │ x │

│92 - 9E   │ Reserved                                      │   │   │

│9F        │ Unspecified TP-DCS error                      │ x │ x │

│          │                                               │   │   │

│A0 - AF   │ TP-Command Errors                             │   │   │

│A0        │ Command cannot be actioned                    │ x │   │

│A1 - AE   │ Reserved                                      │   │   │

│AF        │ Unspecified TP-Command error                  │ x │   │

│          │                                               │   │   │

│B0        │ TPDU not supported                            │ x │ x │

│B1 - BF   │ Reserved                                      │   │   │

│          │                                               │   │   │

│C0        │ SC busy                                       │ x │   │

│C1        │ No SC subscription                            │ x │   │

│C2        │ SC system failure                             │ x │   │

│C3        │ Invalid SME address                           │ x │   │

│C4        │ Destination SME barred                        │ x │   │

│C5 - CF   │ Reserved                                      │   │   │

│          │                                               │   │   │

│D0        │ SIM SMS storage full                          │   │ x │

│D1        │ No SMS storage capability in SIM              │   │ x │

│D2        │ Error in MS                                   │   │ x │

│D3        │ Memory Capacity Exceeded                      │ x │ x │

│D4 - DF   │ Reserved                                      │   │   │

│          │                                               │   │   │

│E0 - FE   │ Values specific to an application             │ x │ x │

│          │                                               │   │   │

│FF        │ Unspecified error cause                       │ x │ x │

└──────────┴───────────────────────────────────────────────┴───┴───┘

Notes:	Any reserved codes which are received should be treated as an unspecified error cause.

MT and MO refer to the overall mobile terminated and mobile originated services; not the direction of transmission of TP-FCS.

.c.9.3	Service provided by the SM-RL

.c.9.3.1	General

The Short Message Relay Layer (SM-RL) provides a service to the Short Message Transfer Layer (SM-TL).  This service enables the SM-TL to send Transfer Protocol Data Units (TPDUs) to its peer entity, receive TPDUs from its peer entity and receive reports about earlier requests for TPDUs to be transferred.

In order to keep track of TPDUs and reports about those TPDUs, primitives between the SM-TL and SM-RL contain a Short Message Identifier (SMI), which is a reference number for the TPDU associated with the primitive.  This Short Message Identifier is not carried via the SM-RL protocol of section 9.3.2.  It is carried via the relay layer service between the SC and GMSC.  It is also carried by SM-RL of GSM 04.11, between the visited MSC and MS.  The parameter is not carried by MAP but is mapped to and from the TCAP dialogue Identifier (see CCITT recommendation Q.771, "Blue Book") at the GMSC and the visited MSC (therefore the Message Identifier at the SC/GMSC interface is not the same as at the visited MSC/MS interface).

The SM-RL communicates with its peer entity by the protocol described in the following sections.

.c.9.3.2	Protocol element repertoire at SM-RL

Different protocols are required between different pairs of SM-RL entities. Those are described in other GSM specifications. This section gives a survey of the different information elements which have to be conveyed between those entities. (Note that the notation of the protocol and information elements may vary between different GSM specifications).

The SM-RL comprises the following 6 protocol elements:

RP-MO-DATA	for transferring a TPDU from MS to SC

RP-MT-DATA	for transferring a TPDU from SC to MS

RP-ACK	for acknowledging an RP-MO-DATA, an RP-MT-DATA or an RP-SM-MEMORY-AVAILABLE

RP-ERROR	for informing of an unsuccessful RP-MO-DATA or an RP-MT-DATA transfer attempt

RP-ALERT-SC	for alerting the SC that the MS has recovered operation (information sent from the HLR to the SC)

RP-SM-MEMORY-AVAILABLE	for notifying the network that the MS has memory available to accept one or more short messages (information sent from the MS to the HLR)

.c.9.3.2.1	RP-MO-DATA

Basic elements of the RP-MO-DATA type.

Abbr.	Reference	P1)	Description

────────────────────────────────────────────────────────────────────

RP-OA	RP-Originating-Address	++-	Address of the originating MS.

RP-DA	RP-Destination-Address	-++	Address of the destination SC.

RP-UD	RP-User-Data	+++	Parameter containing the TPDU

1)	Provision on the links SC<->MSC, MSC<->MSC and MSC<->MS indicated by 'xxx', where x may be either '+' or '-', dependent on whether the parameter is mandatory or not on the respective link.

.c.9.3.2.2	RP-MT-DATA

Basic elements of the RP-MT-DATA type.

Abbr.	Reference	P1)	Description

────────────────────────────────────────────────────────────────────

RP-PRI	RP-Priority-Request	+--	Parameter indicating whether or not the short message transfer should be stopped if the originator SC address is already contained in the MWD.

RP-MMS	RP-More-Messages-To-Send	OO-	Parameter indicating that there are more messages waiting in the SC

RP-OA	RP-Originating-Address	+++	Address of the originating SC.

RP-DA	RP-Destination-Address	++-	Address of the destination MS.

RP-UD	RP-User-Data	+++	Parameter containing the TPDU

1)	Provision on the links SC<->MSC, MSC<->MSC and MSC<->MS indicated by 'xxx', where x may be '+', '-' or 'O', dependent on whether the parameter is mandatory, not present or optional on the respective link.

.c.9.3.2.3	RP-ACK

The RP-ACK does not contain any specific information elements.

.c.9.3.2.4	RP-ERROR

Basic elements of the RP-ERROR type.

Abbr.	Reference	P1)	Description

────────────────────────────────────────────────────────────────────

RP-MSI	RP-MW-Set-Indication	+--	Parameter indicating whether or not the MWI has been up-dated. 2)

RP-CS	RP-Cause	+++	Parameter identifying the error type. The RP-Cause parameter gives the reason why a short message transfer attempt fails. In practice three relay layer protocols are used - SC to GMSC/IWMSC (see GSM 03.47), MAP (see GSM 09.02) and via the radio interface (see GSM 04.11)

RP-	RP-international-	+--	MSIsdn-Alert of the MS, see section 3.2.7 2)

MSIsdn	-MS-ISDN-number

RP-UD	RP-User-Data	---	Parameter containing a TPDU

1)	Provision on the links SC<->MSC, MSC<->MSC and MSC<->MS indicated by 'xxx', where x may be either '+' or '-', dependent on whether the parameter is mandatory or not on the respective link.

2)	Only present when the RP-ERROR is transferred from the SMS-IWMSC to the SC.

.c.9.3.2.5	RP-ALERT-SC

Basic elements of the RP-ALERT-SC type:

Abbr.	Reference	P1)	Description

────────────────────────────────────────────────────────────────────

RP-	RP-International-MS-

MSIsdn	ISDN-Number	 M 	MSIsdn of the MS.

1)	Provision;	Mandatory (M).

.c.9.3.2.6	RP-SM-MEMORY-AVAILABLE

Basic elements of the RP-SM-MEMORY-AVAILABLE type:

Abbr.	Reference	P1)	Description

────────────────────────────────────────────────────────────────────

RP-IMSI	RP-International-Mobile-

Subscriber-Identity	++-	IMSI of the MS.

1)	Provision on the links HLR<->VLR, VLR<->MSC and MSC<->MS indicated by 'xxx', where x may be either '+' or '-', dependent on whether the parameter is mandatory or not present on the respective link.

.c.10	Fundamental procedures within the point-to-point SMS

The point-to-point SMS comprises 3 fundamental procedures:

1)	Short message mobile terminated. This procedure consists of all necessary operations to

a)	transfer a short message or status report from the SC to the MS

b)	return a report to the SC, containing the result of the message transfer attempt.

2)	Short message mobile originated. This procedure consists of all necessary operations to

a)	transfer a short message from the MS to the SC

b)	return a report to the MS, containing the result of the message transfer attempt.

3)	Transfer of an Alert. This procedure consists of all necessary operations for an HLR or a VLR to initiate a transfer of an Alert to a specific SC, informing the SC that the MS has recovered operation.

GSM 09.02 defines operations necessary for the provision of the Short Message Service point-to-point. The operations defined in section 10 describe the requirement that the Short Message Service puts upon the network functionality. If discrepancies exist in nomenclature, it is the GSM 09.02 that will be the reference.

Annex 3 indicates the flow of primitives and parameters during the short message transfer between the SC and the MS. Both the Mobile terminated and the Mobile originated cases are covered.

.c.10.1	Short message mobile terminated

The entities involved in this procedure are depicted in Figure 03.40/14.

┌────┐           ┌────────┐             ┌─────┐             ┌────┐

│ SC ├─────┤ ├───┤SMS-GMSC├────┤ ├──────┤ MSC ├─────┤ ├─────┤ MS │

└────┘           └────┬───┘             └──┬──┘             └────┘

X         │                    │

┴                    ┴

┬                    ┬

│                    │

┌──┴──┐              ┌──┴──┐

│ HLR │              │ VLR │

└─────┘              └─────┘

Figure 03.40/14	Interfaces involved in the Short message mobile terminated procedure. GSM 03.02. X is the interface between an MSC and an SC as defined in section 5.