GSM		GSM 03.49

TECHNICAL		April 1997

SPECIFICATION		Version 5.4.0

Source: ETSI TC-SMG		Reference: TS/SMG-040349QR3

ICS:	33.020

Key words:	Digital cellular telecommunications system, Global System for Mobile communications (GSM)

Digital cellular telecommunications system (Phase 2+);

Example protocol stacks for interconnecting

Cell Broadcast Centre (CBC) and

Base Station Controller (BSC)

(GSM 03.49)

ETSI

European Telecommunications Standards Institute

ETSI Secretariat

Postal address: F-06921 Sophia Antipolis CEDEX - FRANCE

Office address: 650 Route des Lucioles - Sophia Antipolis - Valbonne - FRANCE

X.400: c=fr, a=atlas, p=etsi, s=secretariat - Internet: secretariat@etsi.fr

Tel.: +33 4 92 94 42 00 - Fax: +33 4 93 65 47 16

Copyright Notification: No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media.

© European Telecommunications Standards Institute 1997. All rights reserved.

Whilst every care has been taken in the preparation and publication of this document, errors in content, typographical or otherwise, may occur. If you have comments concerning its accuracy, please write to "ETSI Editing and Committee Support Dept." at the address shown on the title page.

Contents

Foreword

1	Scope

1.1	References

1.2	Abbreviations

2	A protocol stack which utilises an application-network layer convergence function for interconnecting CBC and BSC

2.1	CBSE Definition

2.2	ASN1 Specification

2.3	Application Rules for Avoidance of Collision of CBSE Operations

2.4	Non Support of 128 bytes of NS-user-data in Network Connection and Network Connection Release phases

3	An OSI Protocol Stack For Interconnecting CBC and BSC

3.1	Service elements on the application layer

3.2	Detailed specification of the CBRSE services

3.3	Application rules

3.3.1	Application rule set 1 Semi-permanent symmetric connection

3.3.2	Application rule set 2 Transient asymmetric connection

4	An SS7 Protocol Stack For Interconnecting CBC And BSC

History

Blank page

# Foreword

This Global System for Mobile communications Technical Specification (GTS) has been produced by the Special Mobile Group (SMG) Technical Committee (TC) of the European Telecommunications Standards Institute (ETSI).

This GTS specifies three alternative approaches to the specification of protocol stacks of communication protocols for the purpose of fulfilling the service requirements of the primitives specified for the CBC - BSC interface in GSM 03.41 within the digital cellular telecommunications system (Phase 2/Phase 2+).

This GTS is a TC-SMG approved GSM technical specification version 5, which contains GSM Phase 2+ enhancements/features to the version 4 GSM technical specification. The ETR from which this Phase 2+ GTS has evolved is Phase 2 GSM ETR 107 edition 3 (GSM 03.49 version 4.6.0).

The contents of this GTS are subject to continuing work within TC-SMG and may change following formal TC-SMG approval. Should TC-SMG modify the contents of this GTS it will then be republished by ETSI with an identifying change of release date and an increase in version number as follows:

Version 5.x.y

where:

y	the third digit is incremented when editorial only changes have been incorporated in the specification;

x	the second digit is incremented for all other types of changes, i.e. technical enhancements, corrections, updates, etc.

Blank page

# 1	Scope

No mandatory protocol between the Cell Broadcast Centre (CBC) and the Base Station Controller (BSC) is specified by GSM; this is a matter of agreement between CBC and PLMN operators.

This Global System for Mobile communications Technical Specification (GTS) specifies three alternative approaches to the specification of protocol stacks of communication protocols for the purpose of fulfilling the service requirements of the primitives specified for the CBC - BSC interface in GSM 03.41.

One approach is based upon the use of the complete OSI reference model (see X.200), another approach is based upon the use of only the lower 3 OSI layers, and another approach is based upon the use of CCITT Signalling System No. 7 (see Q.700).

Specifications are based upon individual contributions. Any judgement concerning functionality, completeness and advantages/disadvantages of implementation is intentionally omitted.

## 1.1	References

This GTS incorporates by dated and undated reference, provisions from other publications. These references are cited at the appropriate places in the text and the publications are listed hereafter. For dated references, subsequent amendments to or revisions of any of these publications apply to this GTS only when incorporated in it by amendment or revision. For undated references, the latest edition of the publication referred to applies.

[1]	GSM 01.04 (ETR 350): "Digital cellular telecommunications system (Phase 2+); Abbreviations and acronyms".

[2]	GSM 03.41 (ETS 300 902): "Digital cellular telecommunications system (Phase 2+); Technical realization of Short Message Service Cell Broadcast (SMSCB)".

[3]	GSM 12.20 (ETS 300 622): "Digital cellular telecommunication system (Phase 2); Network Management (NM) procedures and messages".

[4]	CCITT Recommendation Q.700: "Introduction to CCITT Signalling System No.7".

[5]	CCITT Recommendation Q.931: Integrated services digital network.(ISDN) User-Network interface layer 3 specification for basic control".

[6]	CCITT Recommendation Q.932: "Generic procedures for the control of ISDN supplementary services".

[7]	CCITT Recommendation Q.941: "ISDN user-network interface protocol profile for management".

[8]	CCITT Recommendation Q.1400: "Architecture framework for the development of signalling and organisation, administration and maintenance protocols using OSI concepts".

[9]	CCITT Recommendation X.2 (1988): "International data transmission services and optional user facilities in public data networks and ISDNs".

[10]	CCITT Recommendation X.200: "Reference Model of Open Systems Interconnection for CCITT Applications".

[11]	CCITT Recommendation X.213: "Information technology - Network service definition for Open Systems Interconnection".

[12]	CCITT Recommendation X.215: "Session service definition for open systems interconnection for CCITT applications".

[13]	CCITT Recommendation X.217: "Association control service definition for open systems interconnection for CCITT applications".

[14]	CCITT Recommendation X.219: " Remote operations: model, notation and service definition".

[15]	CCITT Recommendation X.225: "Session protocol specification for Open Systems Interconnection for CCITT Applications".

[16]	CCITT Recommendation X.227: "Information technology - Open Systems Interconnection - protocol specification for the association".

[17]	CCITT Recommendation X.229: "Remote operations Protocol specification".

## 1.2	Abbreviations

Abbreviations used in this GTS are listed in GSM 01.04.

# 2	A protocol stack which utilises an application-network layer convergence function for interconnecting CBC and BSC

A convergence function (see Draft CCITT Recommendation Q.941 Report R 22 May 1990) which maps an application entity protocol directly to the Network Layer service defined by X.213 can provide a practical alternative to ACSE, ROSE and OSI layers 6, 5 and 4.

Figure 1

Draft CCITT Recommendation Q.941 proposes to map application layer protocols ACSE and ROSE via a convergence function to network layers defined by CCITT Recommendations Q.931 and Q.932.

The complexity of dealing with the many different network layer protocols is avoided by mapping the application protocols to the Network Layer Service defined by X.213. ACSE and ROSE are specifically defined in terms of the full OSI stack. The use of ACSE and ROSE is avoided by incorporating the functionality provided by ACSE and ROSE into the CBS protocol. The convergence function is embedded in the CBS protocol.

## 2.1	CBSE Definition

The Cell Broadcast Short Message Service Element (CBSE) is defined in terms of the following service:

CBSE-BIND

This operation must be invoked by the party which is responsible for establishing the application association; only after the application association has been established may the remaining CBSE services be used. This operation reports either success (via CBSE-Bind-Confirm) or failure (via CBSE-Bind-Failure).

CBSE-BIND will be mapped to/from N-CONNECT request/indication with CBSE-BIND parameters carried in NS-user-data (if the network layer does not support NS-user-data of 128 octets then CBSE-BIND parameters may be carried by the first N-DATA request/indication following establishment of the network layer connection - see section 2.4).

CBSE-BIND-CONFIRM

This operation must be invoked by a party to accept an application association.

CBSE-BIND-CONFIRM will be mapped to/from N-CONNECT confirm/response with CBSE-BIND-CONFIRM parameters carried in NS-user-data (if the network layer does not support NS-user-data of 128 octets then CBSE-BIND-CONFIRM may be carried as the second N-DATA request/indication following establishment of the network layer connection - see Section 2.4).

CBSE-BIND-FAILURE

This operation must be invoked by a party to reject an attempted application association.

CBSE-BIND-FAILURE will be mapped to/from N-DISCONNECT request/indication with CBSE-BIND-FAILURE parameters carried in NS-user-data (if the network layer does not support NS-user-data of 128 octets then CBSE-BIND-FAILURE parameters will not be carried by the network layer - i.e. NS-user-data will be discarded).

CBSE-UNBIND

This operation must be invoked by a party to release the application association.

CBSE-UNBIND will be mapped to/from N-DISCONNECT request/indication with CBSE-UNBIND parameters be carried in NS-user-data (if the network layer does not support NS-user-data of 128 octets then CBSE-UNBIND parameters may be carried by the N-DATA request/indication preceding N-DISCONNECT - see Section 2.4 ).

CBSE-WRITE-REPLACE, CBSE-KILL-MESSAGE, CBSE-REPORT-SUCCESS,

CBSE-STATUS-CBCH-QUERY, CBSE-STATUS-CBCH-QUERY-RESP., CBSE-STATUS-MESS.-QUERY, CBSE-STATUS-MESS.-QUERY-RESP., CBSE-REPORT-FAILURE, CBSE-BSC-RESTART, CBSE-SET-DRX. CBSE-SET-DRX-RESP

Application data units CBSE-WRITE-REPLACE, CBSE-KILL-MESSAGE, CBSE-REPORT-SUCCESS, CBSE-STATUS-CBCH-QUERY, CBSE-STATUS-CBCH-QUERY-RESP., CBSE-STATUS-MESSAGE-QUERY, CBSE-STATUS-MESS.-QUERY-RESP., CBSE-REPORT-FAILURE, CBSE-BSC-RESTART, CBSE-SET-DRX. CBSE-SET-DRX-RESP provide the services specified via primitives Write-Replace, Kill-Message, Report-Success, Status-CBCH, Status-CBCH-Response, Status-Message, Status-Message-Response, Report-Failure, BSC-Restart, Set-DRX. and Set-DRX-Response respectively in GSM 03.41.

These application data units will be mapped to/from N-DATA request/indication.

## 2.2	ASN1 Specification

The Abstract Syntax Notation of the Cell Broadcast Short Message Service Element


CBSE

1st module of 2:

CBS-UsefulDefinitions

CBS-UsefulDefinitions	{

ccitt identified-organization (4) etsi (0) mobile-domain (0)

gsm-messaging (4) gsm-sms3 (12) usefulDefinitions (10) }

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

ID	::= OBJECT IDENTIFIER

mobile-domain ID	::= {ccitt identified-organization (4) etsi (0) mobile-domain(0)}

--	root for all sms allocations

gsm-messaging ID	::= { mobile-domain gsm-messaging(4) }

--	categories

gsm-sms3 ID		::= { gsm-messaging 12 }

END

2nd module of 2:

Application Protocol

ApplicationProtocol {

ccitt identified-organization (4) etsi (0) mobile-domain(0)

gsm-messaging(4) gsm-sms3 (12) applicationProtocol(11) }

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

--	CBSE-BIND will be carried as N-CONNECT request/indication

--	CBSE-BIND-Parameters will carried in the User Data field of the N-CONNECT

--	request/indication message.

CBSE-BIND-Parameters ::=  SEQUENCE {

initiatorID	[0] Name,

password	[1] Password   OPTIONAL

}

--	Above and in CBSE-BIND-CONFIRM

--	initiatorID/respID: identify the initiating/responding telecommunication subsystem

--	password: may assist in authentication

Name ::=    SEQUENCE {

operator			[0] Operator		OPTIONAL,

bilateralAgreem		[1] BilateralAgreem	OPTIONAL,

dataNetworkAddress	[2] Xl2lAddress	OPTIONAL,

iSDNAddress		[3] CBS-Address	OPTIONAL

}

--	operator is a text string containing the name of the CBC/PLMN operator. bilateralagreem is a text

--	string identifying the bilateral agreement between the CBC and the PLMN operators which allows

--	for this association to be established.

--	dataNetworkAddress is the PSPDN X.121 address of the CBC/BSC issuing the BIND or

--	CONFIRM, occurring only if a PSPDN is used.

--	iSDNAddress is the PLMN address of the CBC (same datum in both BIND and CONFIRM).

--	Any pair of subsets of these parameters may be used to identify the CBC and the BSC to one

--	another.

--	upper bound settings

Operator ::= PrintableString (SIZE (0..20))

BilateralAgreem  ::=  PrintableString  (SIZE (0 .. 20))

Xl2lAddress ::= NumericString  (SIZE(0..15))

--	Definition of Cell Broadcast Short Message Service address

CBS-Address	::= [APPLICATION 0]  SEQUENCE {

address-type      INTEGER {  unknown-type(0),

international-number(1),

national-number(2),

network-specific-number(3),

short-number(4) },

numbering-plan  INTEGER   { unknown-numbering(0),

SDN-numbering(1),

data-network-numbering(3),

telex-numbering(4),

national-numbering(8),

private-numbering(9) },

address-value     CHOICE   {

octet-format

SemiOctetString

-- other formats are for further study

}

}

--	each octet contains two binary coded decimal digits

SemiOctetString ::=  OCTET STRING (SIZE(1..10))

Password	::=   PrintableString  (SIZE(0..20))

--	CBSE-BIND-CONFIRM will carried as N-CONNECT response/confirm

--	CBSE-BIND-CONFIRM parameters will be carried in User Data of the N-CONNECT

--	response/confirm message

CBSE-BIND-CONFIRM-Parameters ::=    SEQUENCE {

respId		[0] Name,

password	[1] Password     OPTIONAL

}

--	The following defines the choices and tags for the N-DISCONNECT.request/indication User Data.

Applic-protocol-discs	::= CHOICE {

bindfail [1] CBSE-BIND-FAILURE,

unbindreq [2] CBSE-UNBIND

}

CBSE-BIND-FAILURE	::= Connect-failure-reason

--	connect-failure-reason provides one of the error indications given in the following table.

Table 1

| Error indications | Reason |
| --- | --- |
| not-entitled | The responder is not entitled to accept a request for an association between itself and the initiator. |
| temporary-overload | The responder is not capable of establishing an association due to temporary overload. |
| temporary-failure | The responder is not capable of establishing an association due to a temporary failure. |
| incorrect-ID-or-password | The responder will not accept the request to establish an association between itself and the initiator due to incorrect identity or password. |

--

Connect-failure-reason	::=  INTEGER {

not-entitled (0),

temporary-overload (1),

temporary-failure (2),

incorrect-ID-or-password (3)

}

CBSE-UNBIND  ::= NULL

--	The following defines the choices and tags for the N-DATA.request/indication User Data

CBSMSEapdus		::=	CHOICE {

cbse-WRITE-REPLACE			[1] Write-Replace,

cbse-KILL-MESSAGE			[2] Kill-Message,

cbse-REPORT-SUCCESS		[3] Report-Success,

cbse-STATUS-CBCH-QUERY		[4] Status-CBCH,

cbse-STATUS-CBCH-QUERY-RESP	[5] Status-CBCH-Resp,

cbse-STATUS-MESSAGE-QUERY	[6] Status-Message,

cbse-STATUS-MESS-QUERY-RESP	[7] Status-Mess-Resp,

cbse-REPORT-FAILURE			[8] Report-Failure,

cbse-BSC-RESTART			[9] BSC-Restart,

cbse-RESET				[10] Reset,

cbse-FAILURE-IND			[11] Failure-Ind

}

Write-Replace	::= SEQUENCE {

message-Identifier			INTEGER (0 .. 65535),

new-Serial-Number		Serial-Number,

no-of-Pages				INTEGER (1 .. 15),

data-coding-scheme		INTEGER (0 .. 255),

cell-list				Cell-List,

repetition-Rate			INTEGER (1 .. 1024),

no-of-broadcast-req		INTEGER (0 ..65535),

cbs-Page-Inf			SEQUENCE OF Page-Inf,

old-Serial-Number			[3] Serial-Number 	OPTIONAL,

category				INTEGER (0..2)	OPTIONAL,

channel-indicator			[2] Channel		OPTIONAL

}

Channel	::= INTEGER {

basic channel (0),

extended channel (1),

}

Page-Inf	::= 	SEQUENCE {

message-info-useful-octets	INTEGER (0..82),

message-info-page		Message-Info-Page

}

Message-Info-Page	::=	OCTET STRING (SIZE(82))

Cell-Id-Disc	::= 	OCTET-STRING (SIZE(1))

-- values from the following table

lacAndCi	::=	'00000001' -- 2 Octet lac followed by 2 Octet Cell Id

ciOnly	::=	'00000010' -- Cell Id only

Cell-Id		::=	OCTET-STRING (SIZE(4))

--Note: If Cell-Id-Disc equals ciOnly then only the last 2 octets of Cell-ID are to be considered --and the first 2 octets are filler octets.

Cell			::=	SEQUENCE OF {

disc		Cell-Id-Disc,

id		Cell-Id

}

Cell-List	::= SEQUENCE {

length		INTEGER, -- number of cells in the list

disc		Cell-Id-Disc,

list		SEQUENCE OF Cell-Id

}

Serial-Number	::= INTEGER (0 .. 65535)

Kill-Message	::= SEQUENCE {

message-Identifier		INTEGER (0 .. 65535),

old-Serial-Number		Serial-Number,

cell-List			Cell-List,

channel-indicator		[4] Channel		OPTIONAL

}

Report-Success	::= SEQUENCE {

message-Identifier	INTEGER (0 .. 65535),

serial-Number		Serial-Number

[0] SEQUENCE OF SEQUENCE {

cell-id				Cell,

no-of-broadcasts-compl	INTEGER

}OPTIONAL,

[1] SEQUENCE OF SEQUENCE {

cell-id				Cell,

cause				Failure-Reason,

diagnostic			Diagnostic-Info	OPTIONAL

}OPTIONAL,

channel-indicator			[4] Channel		OPTIONAL

}

Status-CBCH		::= SEQUENCE{

cell-List			Cell-List,

channel-indicator		[4] Channel		OPTIONAL

}

Status-CBCH-Resp	::= SEQUENCE{

[0] SEQUENCE OF SEQUENCE{

cell-id			Cell,

cbch-loading		INTEGER (0 .. 1019)

-- indicates total number of messages broadcast

-- across the air interface within the last 32

-- minutes (min: 0, max: 1019)

}OPTIONAL,

[1] SEQUENCE OF SEQUENCE{

cell-id			Cell,

cause			Failure-Reason,

diagnostic		Diagnostic-Info	OPTIONAL

}OPTIONAL,

channel-indicator		[4] Channel		OPTIONAL

}

Status-Message	::= SEQUENCE {

message-Identifier	INTEGER (0..65535),

current-Serial-No	Serial-Number,

cell-List			Cell-List,

channel-indicator		[4] Channel		OPTIONAL

}

Status-Mess-Resp	::= SEQUENCE {

message-Identifier		INTEGER (0 .. 65535),

old-serial-number		Serial-Number,

[0] SEQUENCE OF SEQUENCE {

cell-id				Cell,

no-of-broadcasts-compl	INTEGER

}OPTIONAL,

[1] SEQUENCE OF SEQUENCE {

cell-id				Cell,

cause				Failure-Reason,

diagnostic			Diagnostic-Info	OPTIONAL

}OPTIONAL,

channel-indicator			[4] Channel		OPTIONAL

}

Report-Failure ::= SEQUENCE {

cause			Failure-Reason,

diagnostic		Diagnostic-Info	OPTIONAL,

message-Identifier	INTEGER (0 .. 65535)	OPTIONAL,

serial-Number		Serial-Number	OPTIONAL

}

Failure-Reason ::= INTEGER {

parameter-not-recognised (0),

(1), -- not used

parameter-value-invalid (2),

valid-CBS-message-not-identified (3),

cell-identity-not-valid (4),

unrecognised-message (5),

missing-mandatory-element (6),

bss-capacity-exceeded (7),

cell-memory-exceeded (8),

bss-memory-exceeded (9),

unspecifed-error (10),

incompatible-DRX-parameter (11)

}

Diagnostic-Info ::= OCTET STRING (SIZE (1..20))

BSC-Restart	::= SEQUENCE {

cell-list				Cell-List,

recovery-Indication		BOOLEAN		OPTIONAL

}

Reset			::= SEQUENCE {

cell-list		Cell-List

}

Set-DRX	::= SEQUENCE {

cell-list			Cell-List,

schedule-Period		INTEGER (0 .. 48)		OPTIONAL,

reserved-Slots		INTEGER (0 .. 48)		OPTIONAL,

channel-indicator		[4] Channel			OPTIONAL

}

Set-DRX-Resp	::= SEQUENCE {

cell-list					Cell-List,

SEQUENCE OF SEQUENCE {

cell-id					Cell,

cause					Failure-Reason}	OPTIONAL,

channel-indicator				[4] Channel		OPTIONAL

}

Failure-Ind	::= SEQUENCE{

cell-list		Cell-List

}

END

## 2.3	Application Rules for Avoidance of Collision of CBSE Operations

For the purpose of establishing the association between CBSEs in CBC and BSC then either the CBC or the BSC shall be designated as the entity responsible for initiating the association by the operation CBSE-BIND.

Following premature release of the association by N-DISCONNECT then either the CBC or the BSC shall be designated as the entity responsible for re-establishing the association.

Following receipt of N-RESET any command sent by the CBC, for which no corresponding response has been received by the CBC, will be re-sent to the BSC.

## 2.4	Non Support of 128 bytes of NS-user-data in Network Connection and Network Connection Release phases

It is generally intended to make the support of 128 bytes of NS user-data mandatory (see Sections 12.2.8 and 13.2.3 of X.213). CCITT Recommendation X.2 regards provision of Fast Select as essential, thereby ensuring support of 128 bytes of NS-user-data in network connection and network connection release phases. For an interim period support of 128 bytes of NS-user-data in network connection and network connection release phases will remain a provider option. The following figures are therefore provided in order to indicate how CBSE-BIND, CBSE-BIND-CONFIRM and CBSE-UNBIND should be mapped to/from an OSI Network Service definition which does not support 128 bytes of NS-user-data in network connection and network connection release phases.

Figure 2

# 3	An OSI Protocol Stack For Interconnecting CBC and BSC

This section specifies a stack of communication protocols in terms of the OSI Reference Model (see X.200) and therefore makes use of all seven layers for the purpose of fulfilling the service requirements of the primitives specified for the CBC - BSC interface in GSM 03.41. The CBS application layer (layer 7) is mapped to the Presentation Layer via ACSE (see X.217 and X.227) and ROSE (see X.219 and X.229). Only the Kernel functional unit of the Presentation Layer is used. Only the Kernel and Duplex functional units are used in the Session Layer (see X.215 and X.225).

## 3.1	Service elements on the application layer

An association (class 3) between CBRSEs is formed via ACSE and ROSE operations (class 2 and 5) are used to implement the service requirements specified for the CBC - BSC interface in GSM 03.41.

This results in an asynchronous asymmetric situation where the application entity in the CBC or BSC can invoke a CBRSE operation at any time.

The new CBRSE service element is first defined in the following section, and then specified in ASN.1 notation in section 3.2.

CBRSE definition

This service element defines the following services:

CBRSE-BIND	This operation will normally be invoked by the CBC to establish the application association, but in exceptional circumstances (e.g. following loss of data) the BSC may invoke the operation; only thereafter the remaining CBRSE services may be used. This operation reports either success or failure (result or error).

CBR-WRITE-REPLACE, CBR-KILL-MESSAGE, CBR-STATUS-CBCH-QUERY, CBR-STATUS-MESSAGE-QUERY, CBR-RESET, CBR-SET-DRX

These operations may be invoked by the application entity in the CBC; They are used to relay commands from the CBC to a given BSC. The operations report either success or failure.

CBR-RESTART, CBR-FAILURE

This operation may be invoked by the application entity in the BSC. The operation reports success or failure.

CBR-UNBIND	This operation must be invoked by the CBC as the last CBRSE operation before releasing the application association. This operation reports success only.

Of the services defined above, CBR-WRITE-REPLACE semantically means the relay of cell broadcast messages across the CBC-BSC-connection in order to add them to the message list in the BSC, whereas CBR-KILL-MESSAGE is used to delete messages from the message list. The CBR-STATUS-CBCH-QUERY command inquires after the current loading of a specific cell broadcast channel, while the CBR-STATUS-MESSAGE-QUERY command requests status information concerning a specific message. The CBR-SET-DRX  command sets the DRX related parameters. These five services combine the primitives defined in GSM 03.41, which can be invoked by the CBC.

The CBR-BIND service is used to exchange identifications, passwords, etc., and in order to negotiate the usage of the other services. The CBR-UNBIND service prepares for the release of the application association.

## 3.2	Detailed specification of the CBRSE services

On the following pages, the new CBRSE service element is specified with the ASN.1 notation, together with the entire protocol.

The Abstract Syntax Notation of
the Cell Broadcast Relay Service Element

CBRSE

1st module of 3:

CBS-UsefulDefinitions

CBS-UsefulDefinitions	{

ccitt identified-organization (4) etsi (0) mobile-domain(0)

gsm-messaging(4)  gsm-sms4(13)  usefulDefinitions(0) }

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

EXPORTS		id-cb-ot-CBC, id-cb-ot-BSC, id-cb-port,

id-cb-ac-so, id-cb-CBRSE, id-cb-as-CBRSE;

ID	::= OBJECT IDENTIFIER

mobile-domain ID	::= { ccitt identified-organization (4) etsi (0) mobile-domain (0)}

--	root for all sms allocations

gsm-messaging	ID ::= { mobile-domain gsm-messaging (4) }

gsm-sms4	ID	::= {gsm-messaging (13)}

--	categories

id-cb-mod	ID ::= { gsm-messaging 1 } -- modules

id-cb-ot		ID ::= { gsm-messaging 2 } -- object type

id-cb-pt		ID ::= { gsm-messaging 3 } -- port types

id-cb-ac		ID ::= { gsm-messaging 4 } -- appl. contexts

id-cb-ase		ID ::= { gsm-messaging 5 } -- ASEs

id-cb-as		ID ::= { gsm-messaging 6 } -- abstract syntaxes

--	modules

usefulDefinitions		ID ::= { gsm-sms4 0 }

relayProtocol		ID ::= { gsm-sms4 1 }

relayAbstractService	ID ::= { gsm-sms4 2 }

--	object types

id-cb-ot-CBC		ID ::= { id-cb-ot 0 }

id-cb-ot-BSC		ID ::= { id-cb-ot 1 }

--	port types

id-cb-port		ID ::= { id-cb-pt 0 }

--	application contexts

id-cb-ac-so		ID ::= { id-cb-ac 0 }

--	application service elements

id-cb-CBRSE		ID ::= { id-cb-ase 0 }

--	abstract syntaxes

id-cb-as-CBRSE		ID ::= { id-cb-as 0 }

END

2nd module of 3

RelayAbstractService

RelayAbstractService	{

ccitt identified-organization (4) etsi (0) mobile-domain(0)

gsm-messaging(4) gsm-sms4(13)  relayAbstractService(2) }

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

IMPORTS

BIND, UNBIND

FROM Remote-Operations-Notation {

joint-iso-ccitt remote-operations(4) notation(0) }

OBJECT, PORT, ABSTRACT-BIND, ABSTRACT-UNBIND,

ABSTRACT-OPERATION, ABSTRACT-ERROR

FROM AbstractServiceNotation {

joint-iso-ccitt mhs-motis(6) asdc(2) modules(0) notation(1) }

id-cb-ot-CBC, id-cb-ot-BSC, id-cb-port

FROM CBS-UsefulDefinitions{

ccitt identified-organization (4) etsi (0) mobile-domain(0)

gsm-messaging(4) gsm-sms4(13) usefulDefinitions(0) }

--	upper bound settings

ub-operator-name-length INTEGER	::=	20

ub-agreem-name-length INTEGER	::=	20

ub-X121Address-length INTEGER	::=	15

ub-password-length INTEGER		::=	20

--	Objects

--	The CBC and the BSC are modelled as atomic objects, cBC--Object and bSC-Object. Each

--	object has one port for the interconnection. ([S] and [C] indicate supply and consumption of

--	services, respectively).

cBC-Object	OBJECT

PORTS { cBR-port [S] }

::= id-cb-ot-CBC

bSC-Object	OBJECT

PORTS { cBR-port [C] }

::= id-cb-ot-BSC

--	Port

cBR-port	PORT

CONSUMER INVOKES {CBR-Restart

CBR-Failure

}
			SUPPLIER INVOKES  { CBR-Write-Replace

CBR-Kill-Message

CBR-Status-CBCH-Query

CBR-Status-Message-Query

CBR-Reset

CBR-Set-DRX
				 }

::= id-cb-port

--	The CBR-Bind operation

--	Both, BIND and UNBIND operations, are exclusively within the responsibility of the CBC. The

--	BIND operation is therefore always requested by the CBC

CBR-Bind ::=

ABSTRACT-BIND

TO { cBR-port }

BIND

ARGUMENT    CBR-Bind-Parameters

RESULT      CBR-Bind-confirm

BIND-ERROR  CBR-Bind-failure

--	The CBR-Unbind operation

--	The UNBIND is a harsh release of the association and all outstanding operations are aborted.

--	UNBIND is always requested by the CBC. The CBC and the BSC should negotiate (during

--	CBR-BIND) the use of services on the association (the operations parameter - list of 	operation

--	types for the association) in such a way that no harmful losses of operations occur.

CBR-Unbind ::=

ABSTRACT-UNBIND

FROM { cBR-port }

UNBIND

ARGUMENT     Time-when-connected

RESULT       Time-when-disconnected

-	Association control parameters

CBR-Bind-Parameters ::=	SEQUENCE {

initiatorID	[0] Name,

password	[1] Password OPTIONAL,

pswNeeded	[2] BOOLEAN,

iniType	[3] Telecom-System-Type,

operations	[4] List-of-Operations,

transient	[5] BOOLEAN

}

--	Above and in SMR-Bind-confirm

--	initiatorID/respID: identify the initiating/responding telecommunication subsystem

--	password: may assist in authentication

--	pswNeeded (BIND only):requests password into SMR-Bind, SMR-Bind-Confirm

--	iniType/respType: identify the system entity

--	operations: lists the SM relay operations requested and supported on the association:

--	operations listed in both the BIND and the CONFIRM may be used (i.e. this is a negotiation

--	between CBC and BSC)

--	transient:forces the association (and the underlying connections), transient: it must be

--	UNBouND as soon as there are no operations to be performed

Name	::= SEQUENCE {

operator		[0] Operator		OPTIONAL,

bilateralAgreem	[1] BilateralAgreem	OPTIONAL,

dataNetworkAddress	[2] X121Address	OPTIONAL,

iSDNAddress		[3] CBS-Address	OPTIONAL

}

--	operator is a text string containing the name of the CBC/PLMN operator. bilateralAgreem is a

--	text string identifying the bilateral agreement between the CBC and the PLMN operators

--	which allows for this association to be established.

--	dataNetworkAddress is the PSPDN X.121 address of the CBC/BSC issuing the BIND or

--	CONFIRM, occurring only if a PSPDN is used.

--	iSDNAddress is the PLMN address of the CBC as seen by the MSs (same datum in both BIND

and CONFIRM).

--	Any pair of subsets of these parameters may be used to identify the CBC and the BSC to one

--	another.

Operator ::=	PrintableString (SIZE(0..ub-operator-name-length))

BilateralAgreem ::=		PrintableString  (SIZE(0..ub-agreem-name-length))

X121Address	::=	NumericString  (SIZE(0..ub-X121Address-length))

--	CBS-Address is specified later in this module.

Password ::=	PrintableString  (SIZE(0..ub-password-length))

Telecom-System-Type	::= INTEGER {

cell-Broadcast-Service-Centre	(0),

public-Land-Mobile-Network	(1)

--	Extensions are possible: additional telecommunication subsystems 			--	might adopt this service element for their interconnection.

}

List-of-Operations	::= BIT STRING {

cBR-From-CBC-Write-Replace	(0),

cBR-From-CBC-Kill-Message	(1),

cBR-From-CBC-Status-CBCH-Query	(2),

cBR-From-CBC-Status-Message-Query (3),

cBR-From-BSC-Restart (4),

cBR-From-CBC-Reset (5),

cBR-From-BSC-Failure (6)

cBR-From- CBC-Set-DRX (7)

--	Extensions are possible: additional operations may be defined

--	within this service element. Existing systems should tolerate

--	unknown values, but negotiate not to perform unknown

--	operations.}

CBR-Bind-confirm	::= SEQUENCE {

respId	[0] Name,

password	[1] Password OPTIONAL,

respType	[3] Telecom-System-Type,

operations	[4] List-of-Operations,

transient	[5] BOOLEAN,

connectTime	[6] Time-when-connected

}

CBR-Bind-failure  ::= SEQUENCE {

connect-failure-reason

[0] Connect-failure

}

--	connect-failure-reason contains one of the error indications given in the following table.

Table 2

| Error indications | Reason |
| --- | --- |
| not-entitled | The responder is not entitled to accept a request for an association between itself and the initiator. |
| temporary-overload | The responder is not capable of establishing an association due to temporary overload. |
| temporary-failure | The responder is not capable of establishing an association due to a temporary failure (having impact on an entity at SM-RL or at layers above). |
| incorrect-ID-or-password | The responder will not accept the request to establish an association between itself and the initiator due to incorrect identity or password. |
| not-supported | The responder does not recognize the telecommunication subsystem type of the initiator, or cannot support any of the operations suggested on the association. |

--

Connect-failure     ::= INTEGER {

not-entitled (0),

temporary-overload (1),

temporary-failure (2),

incorrect-ID-or-password (3),

not-supported (4)

}

Time-when-disconnected	::= UTCTime

Time-when-connected		::= UTCTime

--	The CBR-Write-Replace operation

CBR-Write-Replace ::=

ABSTRACT-OPERATION

ARGUMENT	Write-Replace

RESULT	Report-Success

ERRORS	{Parameter-not-recognized,

Parameter-value-invalid,

Valid-CBS-message-not-identified,

Cell-identity-not-valid,

Unrecognized-message,

Missing-mandatory-element,

BSS-capacity-exceeded,

Cell-memory-exceeded,

BSS-memory-exceeded,

Unspecified-error

}

--	The CBR-Kill-Message operation

CBR-Kill-Message ::=

ABSTRACT-OPERATION

ARGUMENT	Kill-Message

RESULT	Report-Success

ERRORS	{Parameter-not-recognized,

Parameter-value-invalid,

Unrecognized-message,

Missing-mandatory-element,

Unspecified-error

}

--	The CBR-Status-CBCH-Query operation

CBR-Status-CBCH-Query ::=

ABSTRACT-OPERATION

ARGUMENT	Status-CBCH-Request

RESULT	Status-CBCH-Response

ERRORS	{Parameter-not-recognized,

Parameter-value-invalid,

Cell-identity-not-valid,

Unrecognized-message,

Missing-mandatory-element,

Unspecified-error

}

--	The CBR-Status-Message-Query operation

CBR-Status-Message-Query ::=

ABSTRACT-OPERATION

ARGUMENT	Status-Message-Request

RESULT	Status-Message-Response

ERRORS	{Parameter-not-recognized,

Parameter-value-invalid,

Cell-identity-not-valid,

Unrecognized-message,

Missing-mandatory-element,

Unspecified-error

}

--	The CBR-BSC-Restart operation

CBR-Restart ::=

ABSTRACT-OPERATION

ARGUMENT	Restart-Indication

RESULT

ERRORS	{Parameter-not-recognised,

Parameter-value-invalid,

Cell-identity-not-valid,

Unrecognized-message,

Missing-mandatory-element,

Unspecified-error
					}

--	The CBR-Reset operation

CBR-Reset ::=

ABSTRACT-OPERATION

ARGUMENT	Reset-Request

RESULT	Result-Request

ERRORS	{Parameter-not-recognized,

Parameter-value-invalid,

Cell-identity-not-valid,

Unrecognized-message,

Missing-mandatory-element

Unspecified-error

}

--	The CBR-Failure operation

CBR-Failure ::=

ABSTRACT-OPERATION

ARGUMENT	Failure-Indication

RESULT

ERRORS	{Parameter-not-recognized,

Parameter-value-invalid,

Cell-identity-not-valid,

Unrecognized-message,

Missing-mandatory-element,

Unspecified-error

}

--	The CBR-Set-DRX operation

CBR-Set-DRX ::=

ABSTRACT-OPERATION

ARGUMENT	Set-DRX

RESULT		Set-DRX-Resp

ERRORS		{Parameter-not-recognized,

Parameter-value-invalid,

Valid-CBS-message-not-identified,

Cell-identity-not-valid,

Unrecognized-message,

Missing-mandatory-element,

BSS-capacity-exceeded,

Cell-memory-exceeded,

BSS-memory-exceeded,

Unspecified-error,

Incompatible-DRX-parameter

}

--	CBR operation ARGUMENT lists

Write-Replace ::= SEQUENCE {

message-identifier			INTEGER (0 .. 65535),

new-serial-number			Serial-Number,

no-of-pages				INTEGER (1 .. 15),

data-coding-scheme		INTEGER (0 .. 255),

cell-list				Cell-List,

repetition-rate			INTEGER (1 .. 1024),

no-of-broadcast-req		INTEGER (0 ..65535),

cBS-page-info			SEQUENCE OF	Page-Inf,

old-serial-number 			[1] Serial-Number		OPTIONAL,

category				INTEGER (0..2)		OPTIONAL,

channel-indicator			[2] Channel			OPTIONAL

}

Kill-Message ::= SEQUENCE {

message-identifier	INTEGER (0 .. 65535),

old-serial-number	Serial-Number,

cell-List			Cell-List,

channel-indicator		[2] Channel		OPTIONAL

}

Status-CBCH-Request ::= SEQUENCE {

cell-List			Cell-List,

channel-indicator		 [2] Channel		OPTIONAL

}

Status-Message-Request ::= SEQUENCE {

message-Identifier	INTEGER (0 .. 65535),

old-serial-no		Serial-Number,

channel-indicator	[2] Channel		OPTIONAL

}

Restart-Indication ::= SEQUENCE {

cell-list				Cell-List,

recovery-Indication		BOOLEAN		OPTIONAL

}

Failure-Indication ::= ::= SEQUENCE{

cell-list	Cell-List

}

Reset-Indication ::= ::= SEQUENCE{

cell-list	Cell-List

}

Set-DRX ::= SEQUENCE {

cell-list				Cell-List			OPTIONAL,

schedule-Period			INTEGER (0 .. 48)		OPTIONAL,

reserved-Slots			INTEGER (0 .. 48)		OPTIONAL,

channel-indicator			[2] Channel			OPTIONAL

}

--	CBR operation RESULT list

Report-Success ::= SEQUENCE {

message-identifier	INTEGER (0 .. 65535),

serial-number		Serial-Number,

[0] SEQUENCE OF SEQUENCE{

cell-id				Cell,

no-of-broadcasts-compl	INTEGER

}OPTIONAL,

[1] SEQUENCE OF SEQUENCE{

cell-id				Cell,

cause				Failure-Reason,

diagnostic			Diagnostic-Info	OPTIONAL

}OPTIONAL,

channel-indicator			[2] Channel		OPTIONAL

}

Status-CBCH-Response ::= SEQUENCE{

[0] SEQUENCE OF SEQUENCE {

cell-id				Cell,

cbch-loading			INTEGER (0..1019)

-- indicates the total number of messages broadcast

-- across the air interface within last 32

-- minutes (min: 0, max: 1019)

}OPTIONAL

[1] SEQUENCE OF SEQUENCE{

cell-id				Cell,

cause				Failure-Reason,

diagnostic			Diagnostic-Info	OPTIONAL

}OPTIONAL,

channel-indicator			[2] Channel		OPTIONAL

}

Status-Message-Response ::= SEQUENCE {

message-identifier	INTEGER (0 .. 65535),

old-serial-number	Serial-Number,

[0] SEQUENCE OF SEQUENCE {

cell-id				Cell,

no-of-broadcasts-compl	INTEGER

}OPTIONAL

[1] SEQUENCE OF SEQUENCE{

cell-id				Cell,

cause				Failure-Reason,

diagnostic			Diagnostic-Info	OPTIONAL

}OPTIONAL,

channel-indicator			[2] Channel		OPTIONAL

}

Set-DRX-Response ::= SEQUENCE {

cell-list				Cell-List			OPTIONAL,

SEQUENCE OF SEQUENCE{

cell-id				Cell,

cause				Failure-Reason}		OPTIONAL,

channel-indicator		[2] Channel			OPTIONAL

}

--	CBR operation errors listed below

Parameter-not-recognized ::=

ABSTRACT-ERROR

PARAMETER Diagnostic-Info OPTIONAL

Parameter-value-invalid ::=

ABSTRACT-ERROR

PARAMETER Diagnostic-Info OPTIONAL

Valid-CBS-message-not-identified ::=

ABSTRACT-ERROR

PARAMETER Diagnostic-Info OPTIONAL

Cell-Identity-not-valid ::=

ABSTRACT-ERROR

PARAMETER Diagnostic-Info OPTIONAL

Unrecognized-message ::=

ABSTRACT-ERROR

PARAMETER Diagnostic-Info OPTIONAL

Missing-mandatory-element ::=

ABSTRACT-ERROR

PARAMETER Diagnostic-Info OPTIONAL

BSS-capacity-exceeded ::=

ABSTRACT-ERROR

PARAMETER Diagnostic-Info OPTIONAL

Cell-memory-exceeded ::=

ABSTRACT-ERROR

PARAMETER Diagnostic-Info OPTIONAL

BSS-memory-exceeded ::=

ABSTRACT-ERROR

PARAMETER Diagnostic-Info OPTIONAL

Unspecified-error ::=

ABSTRACT-ERROR

PARAMETER Diagnostic-Info OPTIONAL

incompatible-DRX-parameter ::=

ABSTRACT-ERROR

PARAMETER Diagnostic-Info OPTIONAL

Serial-Number ::= INTEGER (0 .. 65535)

Page-Inf	::=	SEQUENCE{

message-info-useful-octets	INTEGER (0..82),

message-info-page		Message-Info-Page

}

Message-Info-Page	::=	OCTET STRING (SIZE(82))

Cell-Id-Disc ::= OCTET-STRING (SIZE(1))

-- values from the following table

lacAndCi	::=	'00000001'	--2 Octet lac followed by 2 Octet Cell Id

ciOnly		::=	'00000010'	--Cell Id only

Cell-Id ::= OCTET-STRING (SIZE(4))

--Note: If Cell-Id-Disc equals ciOnly then only the last 2 octets of Cell-ID are to be considered --and the first 2 octets are filler octets

Cell ::= SEQUENCE OF {

disc		Cell-Id-Disc,

id		Cell-Id

}

Cell-List ::= SEQUENCE {

length		INTEGER,

disc		Cell-Id-Disc,

list		SEQUENCE OF Cell-Id

}

Diagnostic-Info ::=	OCTET STRING (SIZE (1 .. 20))

--	Definition of Cell Broadcast Relay Service address

CBS-Address ::= [APPLICATION 0] SEQUENCE {

address-type	INTEGER  { unknown-type		(0),

international-number		(1),

national-number		(2),

network-specific-number	(3),

short-number			(4) },

numbering-plan	INTEGER { unknown-numbering	(0),

iSDN-numbering		(1),

data-network-numbering	(3),

telex-numbering		(4),

national-numbering		(8),

private-numbering		(9)}

address-value	CHOICE { octet-format

SemiOctetString

--other formats are for further study}

}

SemiOctetString ::= OCTET STRING (SIZE(1..10))

-- each octet contains two binary coded decimal digits

END

3rd module of 3

RelayProtocol

RelayProtocol {

ccitt identified-organization (4) etsi (0) mobile-domain(0)

gsm-messaging (4) gsm-sms4 (13) relayProtocol(1) }

DEFINITIONS

IMPLICIT TAGS

::=

BEGIN

IMPORTS

--	application service elements and application contexts

aCSE, APPLICATION-SERVICE-ELEMENT, APPLICATION-CONTEXT

FROM Remote-Operations-Notation-extension {

joint-iso-ccitt  remote-operations(4) notation-extension(2) }

rOSE

FROM Remote-Operations-APDUs {

joint-iso-ccitt  remote-operations(4) apdus(1) }

--	object identifiers

id-cb-ac-so, id-cb-CBRSE, id-cb-as-CBRSE,

FROM CBS-UsefulDefinitions{

ccitt identified-organization (4) etsi (0) mobile-domain(0)

gsm-messaging(4) gsm-sms4 (13)  usefulDefinitions(0) } ;

aS-ACSE OBJECT IDENTIFIER ::=

{ joint-iso-ccitt association-control (2) abstractSyntax(1) apdus(0) version(1) }

--	abstract service parameters

CBR-Bind, CBR-Unbind, CBR-Write-Replace, CBR-Kill-Message,

CBR-Kill-Message, CBR-Status-CBCH-Query,

CBR-Status-Message-Query, CBR-Reset, CBR-Restart,

CBR-Failure, CBR-Set-DRX,

Parameter-not-recognized, Parameter-value-invalid,

Valid-CBS-message-not-identified, Cell-identity-not-valid,

Unrecognized-message,

Missing-mandatory-element, BSS-capacity-exceeded,

Cell-memory-exceeded, BSS-memory-exceeded, Unspecified-error, incompatible-DRX-parameter

FROM RelayAbstractService{

ccitt identified-organization (4) etsi (0) mobile-domain(0)

gsm-messaging(4) gsm-sms4(13) relayAbstractService(2) } ;

--	Application contexts

--	Only one application contexts is specified: the CBC is exclusively responsible for the BIND and

--	UNBIND operations.

cBC-BINDs-and-UNBINDs

APPLICATION-CONTEXT

APPLICATION-SERVICE-ELEMENTS { aCSE }

BIND    CBR-Bind

UNBIND  CBR-Unbind

REMOTE OPERATIONS { rOSE }

INITIATOR CONSUMER OF { cBRSE }

ABSTRACT SYNTAXES { id-cb-as-CBRSE , aS-ACSE }

::= id-cb-ac-so

--	Application service elements

cBRSE	APPLICATION-SERVICE-ELEMENT

CONSUMER INVOKES {	CBR-Restart

CBR-Failure

}

SUPPLIER INVOKES { CBR-Write-Replace

CBR-Kill-Message

CBR-Status-CBCH-Query

CBR-Status-Message-Query

CBR-Reset

CBR-Set-DRX

}

::= id-cb-SMRSE

--	Remote operations

cbr-write-replace			CBR-Write-Replace

::= 1

-- Note:	localValue - words are omitted, since they are

--	typically not used, and likely to be removed from

--	the OPERATION and ERROR macros in ROSE.

cbr-kill-message			CBR-Kill-Message

::= 2

cbr-status-CBCH-query			CBR-Status-CBCH-Query

::= 3

cbr-status-message-query		CBR-Status-Message-Query

::= 4

cbr-restart				CBR-Restart

::= 5

cbr-reset				CBR-Reset

::= 6

cbr-failure				CBR-Failure

::= 7

cbr-set-DRX				CBR-Set-DRX

::= 8

--	Remote errors, the localValues are provisional

parameter-not-recognized		Parameter-not-recognized

::= 1

parameter-value-invalid			Parameter-value-invalid

::= 3

valid-CBS-message-not-identified	Valid-CBS-message-not-identified

::= 4

cell-identity-not-valid			Cell-identity-not-valid

::= 5

unrecognized-message			Unrecognized-message

::= 6

missing-mandatory-element		Missing-mandatory-element

::= 7

bss-capacity-exceeded			BSS-capacity-exceeded

::= 8

cell-memory-exceeded			Cell-memory-exceeded

::= 9

bss-memory-exceeded			BSS-memory-exceeded

::= 10

unspecified-error				Unspecified-error

::= 11

incompatible-DRX-parameter		Incompatible-DRX-Parameter

::= 12

END

## 3.3	Application rules

The following application rules specify the invocation of different operations on the association. Two alternative sets of application rules are given in 3.3.1 (for semi-permanent connections) and in 3.3.2 (for transient connections); additional sets are possible.

### 3.3.1	Application rule set 1 Semi-permanent symmetric connection

This set of application rules is to be used in situations where the connection (on all the protocol layers) between the CBC and the BSC is maintained for ever.

Within the CBR-BIND service, all operations are allowed on the association; semi-permanent connection is accepted (by not forcing the connection transient). This is negotiated within the CBR-BIND service as follows:

name of parameter	value in request and report

operations		{cBR-From-CBC-Write-Replace,

cBR-From-CBC-Kill-Message,

cBR-From-CBC-Status-CBCH-Query,

cBR-From-CBC-Status-Message-Query,

cBR-From-BSC-Restart,

cBR-From-BSC-Reset,

cBR-From-BSC-Failure

cBR-From-CBC-Set-DRX

}

transient			FALSE

The CBC invokes cBR-From-CBC-Write-Replace, cBR-From-CBC-Kill-Message, cBR-From-CBC-Status-CBCH-Query, cBR-From-CBC-Status-Message-Query, cBR-From-CBC-Set-DRX operations as needed. The BSC invokes CBR-BSC-RESTART.

The CBR-UNBIND operation is not normally invoked on the association.

### 3.3.2	Application rule set 2 Transient asymmetric connection

This set of application rules is to be used e.g. in situations where a CBC has connections with many BSCs, and there is a switched data network connecting them. A data network connection (and the higher layer connections on top of it) is maintained for the duration of the relay or alert operations only.

Within the CBR-BIND service, only one type of operation is negotiated for use on the association. The operation of that type must be invoked by the CBC or by the BSC in exceptional circumstances (e.g. in order to invoke CBR-BSC-RESTART). The BSC or CBC accepts the one type of operation and forces the association transient.

The following is an example of a negotiation procedure within the CBR-BIND service, where the CBR-Write-Replace operation is initiated by the CBC.

name of parameter	value

iniType			cell-Broadcast-Service-Centre

respType			public-Land-Mobile-Network

operations		{ cBR-From-CBC-Write-Replace }

transient			TRUE

The association for cBR-From-CBC-Kill-Message, cBR-From-CBC-Status-CBCH-Query or cBR-From-CBC-Status-Message-Query, cBR-From-CBC-Set-DRX are negotiated according to the same principle, the CBC always being the initiator of the CBR-BIND.

The association may be used for invoking operations of the negotiated type(s) as long as there are such operations to be invoked (in other words, until all commands have been relayed).

# 4	An SS7 Protocol Stack For Interconnecting CBC And BSC

Concepts described in Q.1400 (see CCITT Study Group XI - Report R219) are used. These concepts enable, with minor modifications, the protocol specified in Section 3 of GSM 03.49 to be supported via an SS7 protocol stack.

Q.1400 specifies the use of OSI concepts via SS7 for the development of signalling and operations and management protocols. The protocol specified in Section 3 of this report can be carried via an SS7 protocol stack consisting of TCAP, SCCP and MTP (see Q.700 series) with minor adaptations:

-	ROSE operation classes 2 and 5 are replaced by TCAP operation classes 1 and 4 respectively.

-	TCAP provides a connectionless service. The services provided by CBRSE-BIND, CBR-UNBIND, CBR-Bind-confirm and CBR-Bind-failure are therefore not required and Sections 3.3 is not applicable.

PLMN networks may provide interworking between either of the protocols specified by Sections 2 or 3 and the SS7 protocol stack for the purpose of fulfilling the service requirements of the primitives specified for the CBC - BSC interface in GSM 03.41.

# History

| Document history |  |
| --- | --- |
| October 1995 | Creation of Version 5.0.0 (Version 4.6.0 + AR001) |
| January 1996 | Publication of Version 5.0.0 |
| May 1995 | Publication of Version 5.1.0 |
| July 1995 | Publication of Version 5.2.0 |
| November 1996 | Publication of Version 5.3.0 |
| April 1997 | Publication of Version 5.4.0 |