TD <>

TS 101 045 V5.0.1 (1997-04)

Technical Specification

Digital cellular telecommunications system (Phase 2+);

Support of Optimal Routeing (SOR);

Technical Realization

(GSM 03.79)

Reference

DTS/SMG-030379Q (9oc02i0r.PDF)

Keywords

Digital cellular telecommunications system, Global System for Mobile communications (GSM)

ETSI Secretariat

Postal address

F-06921 Sophia Antipolis Cedex - FRANCE

Office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° 7803/88

X.400

c= fr; a=atlas; p=etsi; s=secretariat

Internet

secretariat@etsi.fr

http://www.etsi.fr

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© European Telecommunications Standards Institute 1997.

All rights reserved.

Contents

Foreword

1	Scope	7

2	Normative references	7

3	Definitions and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	8

4	Architecture	9

4.1	Optimal routeing for basic mobile-to-mobile calls	9

4.2	Optimal routeing for conditional call forwarding	10

5	Optimal routeing for basic mobile-to-mobile calls: message flows	11

5.1	Successful outcome	12

5.2	Unsuccessful outcome	12

5.2.1	B subscriber address not recognized as belonging to a GSM PLMN	12

5.2.2	HLRB or VLRB does not support optimal routeing	12

5.2.3	OR charging requirements contravened	12

6	Optimal routeing for conditional call forwarding: message flows	13

6.1	Early call forwarding	13

6.1.1	Forwarding without interrogation of VLRB	13

6.1.1.1	Successful outcome	13

6.1.1.2	Unsuccessful outcome	14

6.1.2	Forwarding after interrogation of VLRB	14

6.1.2.1	Successful outcome	14

6.1.2.2	Unsuccessful outcome	15

6.2	Late call forwarding	16

6.2.1	Successful outcome	17

6.2.2	Unsuccessful outcome	17

6.2.2.1	GMSC does not support OR	17

6.2.2.2	HLRB does not support OR	17

6.2.2.3	VMSCB/VLRB does not support OR	17

6.2.2.4	OR charging requirements contravened	17

7	Interactions between optimal routeing and supplementary services	18

7.1	Call forwarding	18

7.2	Closed User Group (CUG)	18

7.3	Advice of Charge	18

7.4	Call barring	18

7.5	Other supplementary services	18

8	Interactions between optimal routeing and other network features	19

8.1	Operator determined barring	19

8.2	CAMEL	19

9	Functional requirements of entities performing optimal routeing	19

9.1	Charging requirements for optimal routeing	19

9.2	Functional behaviour of VMSCA	20

9.3	Functional behaviour of VLRA	20

9.4	Functional behaviour of GMSC	20

9.4.1	Basic OR: incoming IAM	20

9.4.2	Basic OR: successful outcome	21

9.4.3	Early call forwarding	21

9.4.4	Optimal Routeing of Late Call Forwarding (ORLCF)	21

9.4.4.1	ORLCF: successful outcome	22

9.4.4.2	ORLCF: unsuccessful outcome	22

9.4.5	Basic OR: unsuccessful outcome	22

9.4.6	Call release	22

9.4.7	Relay of other messages	23

9.4.8	Procedure Obtain_Routeing_Address	23

9.4.8.1	Send Routeing Info negative response	23

9.4.8.2	Send Routeing Info ack	24

9.4.8.2.1	Routeing information is MSRN	24

9.4.8.2.2	Routeing information is forwarded-to number	24

9.4.8.2.3	Routeing information is CAMEL modified number	24

9.4.8.2.4	No routeing information	24

9.4.9	Procedure Route_Permitted	24

9.4.10	Procedure Handle_COLP_Forwarding_Interaction	24

9.5	Functional behaviour of HLR	32

9.5.1	Receipt of a request for routeing information	32

9.5.2	Call forwarding before VLR interrogation	32

9.5.3	Basic call enquiry	32

9.5.4	Response to Provide Roaming Number request	33

9.5.4.1	Response is MSRN	33

9.5.4.2	Response is negative	33

9.5.5	Forwarding information enquiry	33

9.5.5.1	Forwarding on busy	33

9.5.5.2	Forwarding on mobile subscriber not reachable	33

9.5.5.3	Forwarding on no subscriber reply	34

9.5.5.4	Handling of a forwarded-to number	34

9.5.6	Unsuccessful outcome	34

9.5.6.1	Subscription check failure	34

9.5.6.2	Forwarding interrogation not supported	34

9.5.6.3	Optimal Routeing not allowed	34

9.5.6.4	First forwarding failure	34

9.5.6.5	Provide Roaming Number negative response - no forwarding	35

9.5.6.6	Forwarding violation	35

9.5.6.7	Mobile subscriber busy	35

9.5.6.8	Mobile subscriber not reachable	35

9.5.6.9	No subscriber reply	35

9.5.7	Procedure Subscription_Check_HLR	35

9.5.8	Procedure First_Forwarding_HLR	35

9.5.9	Procedure PRN_Error_HLR	35

9.5.10	Procedure Handle_CFB	35

9.5.11	Procedure Handle_CFNRc	35

9.5.12	Procedure Handle_CFNRy	35

9.5.13	Procedure Forward_CUG_Check	35

9.6	Functional behaviour of VLRB	41

9.6.1	Functional behaviour of VLRB for provision of subscriber information	41

9.6.2	Functional behaviour of VLRB for roaming number allocation	41

9.6.3	Functional behaviour of VLRB when handling an incoming call	41

9.7	Functional behaviour of VMSCB	41

9.7.1	Receipt of Initial Address Message	41

9.7.2	Successful outcome	41

9.7.2.1	Completion of call	41

9.7.2.2	Conditional call forwarding at the GMSC	41

9.7.2.3	Call release	42

9.7.3	Unsuccessful outcome	42

9.7.4	Procedure Complete_Call_In_MSC	42

10	Contents of messages	45

10.1	Messages on the B interface (MSC-VLR)	45

10.1.1	Send Info For Outgoing Call	45

10.1.2	Send Info For Outgoing Call negative response	45

10.1.3	Send Info For Incoming Call	45

10.1.4	Send Info For Incoming Call ack	45

10.1.5	Send Info For Incoming Call negative response	45

10.1.6	Complete Call	45

10.1.7	Process Call Waiting	46

10.2	Messages on the C interface (MSC-HLR)	46

10.2.1	Send Routeing Info	46

10.2.2	Send Routeing Info ack	46

10.2.3	Send Routeing Info negative response	47

10.3	Messages on the D interface (VLR-HLR)	47

10.3.1	Provide Roaming Number	47

10.3.2	Provide Roaming Number ack	47

10.3.3	Provide Roaming Number negative response	47

10.3.4	Provide Subscriber Information	47

10.3.5	Provide Subscriber Information ack	47

10.4	Messages on the E interface (MSC-MSC)	48

10.4.1	Resume Call Handling	48

10.4.2	Resume Call Handling ack	48

10.4.3	Resume Call Handling negative response	48

Annex A (informative):	Handling of an IAM at an MSC	49

History	50

# Foreword

This Technical Specification (TS) has been produced by the Special Mobile Group (SMG) Technical Committee (TC) of the European Telecommunications Standards Institute (ETSI).

This TS specifies the technical realization of the first phase of the network feature Support of Optimal Routeing (SOR) within the digital cellular telecommunications system (Phase 2/Phase 2+).

The contents of this TS are subject to continuing work within TC-SMG and may change following formal TC-SMG approval. Should TC-SMG modify the contents of this TS it will then be republished by ETSI with an identifying change of release date and an increase in version number as follows:

Version 5.x.y

where:

y	the third digit is incremented when editorial only changes have been incorporated in the specification;

x	the second digit is incremented for all other types of changes, i.e. technical enhancements, corrections, updates, etc.