ETSI/GSM

Recommendation:    GSM 04.11 version 3.3.0

Title:          POINT-TO-POINT SHORT MESSAGE SERVICE SUPPORT

ON MOBILE RADIO INTERFACE

Date: January 1993

List of contents:

1  General

2  Overview of Short Message Service Support

3  Service definition

4  Network Interfaces

5  CM-procedures

6  SM-RL-procedures

7  Message functional definitions and content

8  Message format and information elements coding

Language of original: English

Number of pages: 73 total

Main text: 39

Annex A: 4

Annex B: 12

Annex C: 3

Annex D: 8

Annex E: 7

1. General

This Recommendation specifies the procedures used across the mobile radio interface by the signalling layer 3 function SMC - Short Message Control and SM-RL function Short Message Relay.

2. Overview of Short Message Service Support

The purpose of the Short Message Service is to provide the means to transfer messages between a GSM PLMN Mobile Station and a Short Message Entity via a Service Centre, as described in Rec. GSM 03.40.  The terms "MO" - mobile originating - and "MT" - mobile terminating - are used to indicate the direction in which the short message is sent.

This Recommendation describes the procedures necessary to relay a message between the MS and the MSC (and v.v), across the radio path.

The procedures are based on services provided by the Mobility Management sublayer as described in Rec. GSM 04.07/04.08.

2.1 Protocols and protocol architecture

The hierarchical model shows the layer structure of the MSC and the MS.

┌─────┐                            ┌─────┐

│ MSC │                            │  MS │

└─────┘                            └─────┘

─────────────                                          ┌ ─ ─ ┐

SM-AL                                                 │     │

─────────────                                          ├ ─ ─ ┤

SM-TL                                                 │     │

─────────────     ..┌─────┐ ...........................├─────┤:

SM-RL            : │ SMR │<───── SM-RP protocol──────>│ SMR │:

─────────────     : ├─────┤                            ├─────┤:

CM-sublayer      : │ SMC │<───── SM-CP protocol──────>│ SMC │:

─────────────     :.├─────┤............................├─────┤:

MM-sublayer        │     │                            │     │

─────────────       ├ ─ ─ ┤                            ├ ─ ─ ┤

RR-sublayer        │     │                            │     │

─────────────       └ ─ ─ ┘                            └ ─ ─ ┘

Figure 2.1/GSM 04.11:  Protocol hierarchy

The CM-sublayer, in terms of the Short Message Service Support,

provides services to the Short Message Relay Layer.

On the MS-side the Short Message Relay Layer provides services to the Short Message Transfer Layer.  The Short Message Relay Layer is the upper layer on the network side (MSC), and the SM-user information elements are mapped to TCAP/MAP.

The peer protocol between two SMC entities is denoted SM-CP, and between two SMR entities, SM-RP.

Abbreviations:

SM-AL  :  Short Message Application Layer

SM-TL  :  Short Message Transfer Layer

SM-RL  :  Short Message Relay Layer

SM-RP  :  Short Message Relay Protocol

SMR    :  Short Message Relay entity

CM-sub :  Connection Management sublayer

SM-CP  :  Short Message Control Protocol

SMC    :  Short Message Control entity

MM-sub :  Mobility Management sublayer

RR-sub :  Radio Resource Management sublayer

2.2 Use of channels

The short message will be transferred on a SDCCH or SACCH, depending on the use of a TCH;

-  When a TCH is not allocated, the short message will be

transferred on a SDCCH.

- If a TCH is allocated during a short message transaction

on a SDCCH, the short message transfer will stop and continue on the SACCH associated with the TCH.

- If a TCH is allocated when the short message arrive, the short message will be transferred on the associated SACCH.

- When an entity using a TCH, finalize its transaction, the RR-

sub. may choose to continue an ongoing short message transfer on the SACCH, or optionally transfer it to a SDCCH.

Table 2.1/GSM 04.11 summarize the use of channels during a short message transfer.  Arrows indicate change of channel.

TABLE 2.1/GSM 04.11

Channels used for short message transfer

┌──────────────────────────────────────┬─────────────────────────────┐

│  Channel dependency                  │   Channel used              │

├──────────────────────────────────────┼─────────────────────────────┤

│  TCH not allocated                   │   SDCCH                     │

│  TCH not allocated -> TCH allocated  │   SDCCH -> SACCH            │

│  TCH allocated                       │   SACCH                     │

│  TCH allocated -> TCH not allocated  │   SACCH -> SACCH opt. SDCCH │

└──────────────────────────────────────┴─────────────────────────────┘

2.3       Layer 2 SAPI 3 handling

General rule:

The Radio Resource Management (RR ref Rec. GSM 04.08) in the Mobile Station and on the network side (i.e. in the BSC) shall establish the acknowledged mode of operation on SAPI 3 whenever needed, i.e. when a message requiring SAPI 3 transfer shall be transmitted.

RR shall control the layer 2 also for SAPI 3, and keep knowledge of the mode.

The network side may initiate releasse of the acknowledged mode for SAPI 3 either explicitly (by the use of DISC- and UA-frames, ref., Rec. GSM 04.06) or indirectly by channel release (ref. Rec. GSM 04.08).

This means:

- the Mobile Station side will initiate establishment of SAPI 3 acknowledged mode in the case of Mobile originated short message transfer

- the network side will initiate establishment of SAPI 3 acknowledged mode in the case of Mobile terminated short message transfer

- The network side may choose to keep the channel and the acknowledged mode of operation to facilitate transfer of several short messages for the same Mobile Station. The queuing and scheduling function for this should reside in the MSC.

3. Service definition

3.1 General

The layer service is described as a set of service primitives.  These service primitives are abstractions and attempt to capture only those details of the interaction between the entities that are aspects of the layer service itself.  A service primitive neither specifies nor cons- trains the implementation of entities or the interface between them.

The general syntax of a primitive is specified in Recommendation 04.01.

The initials of the primitives is in line with Recommendation 04.07.

3.2 Service provided by CM-sublayer

In order to support the Short Message Service, the CM-sublayer provide services to the Short Message Relay Layer.

The CM-sublayer services are provided using layer specific functions and lower layer services offered to the CM-sublayer, controlled by short message control entities called SMCs.

An SMC entity in the MS communicate with an SMC entity in the MSC by means of a peer protocol, SM-CP - Short Message Control Protocol. The arrow diagrams in annex A gives an overview of the messaging on the CM- sublayer during a short message transfer.

An MS supporting the short message service shall have a minimum of two SMC entities.  This enables the MS to receive MT messages during a MO message transfer.

To ensure that an MS having the minimum of two SMC entities is able to receive MT messages during an MO message transfer, and to send MO messa- ges during MT message transfer, parallell message transfer in the same direction is prohibited.  This means that the SMC entities shall not simultaneously perform messaging in the same direction.

The MSC shall have a minimum of two SMC entities available during a MT message transfer to a mobile station, one being reserved for MO message transfer.  In a MO message transfer, the MSC shall have one SMC entity reserved for handling of the incoming message.

3.2.1 Definition of primitives on MS side

This section defines the service primitives used on the MS side.  Table 3.1/GSM 04.11 gives an overview of the service primitives and linked main parameter to the primitives.  All necessary control parameters to be used in a short message transfer is defined in chapter 7.  All MNSMS service primitives defined in this section are passed on a CM-connection.

TABLE 3.1/GSM 04.11

MNSMS service primitives on MS-side

┌───────────────────────────┬─────────────────────┐

│    SERVICE PRIMITIVES     │                     │

├──────────────────┬────────┤      PARAMETER      │

│       NAME       │  TYPE  │                     │

├──────────────────┼────────┼─────────────────────┤

│   MNSMS-ABORT-   │  Req   │  Cause              │

├──────────────────┼────────┼─────────────────────┤

│                  │  Req   │  MO RP-ACK/RP-ERROR │

│   MNSMS-DATA-    ├────────┼─────────────────────┤

│                  │  Ind   │  MT RP-ACK/RP-ERROR │

├──────────────────┼────────┼─────────────────────┤

│                  │  Req   │  MO RP-DATA         │

│   MNSMS-EST-     ├────────┼─────────────────────┤

│                  │  Ind   │  MT RP-DATA         │

├──────────────────┼────────┼─────────────────────┤

│   MNSMS-ERROR-   │  Ind   │  Cause              │

├──────────────────┼────────┼─────────────────────┤

│   MNSMS-REL-     │  Req   │  Cause              │

└──────────────────┴────────┴─────────────────────┘

3.2.1.1 MNSMS-ABORT-REQuest

A request from an SMR entity to release a CM-connection in abnormal cases.

When CM-sublayer receives this request, it may form and send the CP-ERROR message to release the connection, or it may release the connection by use of lower layer services.

3.2.1.2 MNSMS-DATA-REQuest

A request from an SMR entity to return the acknowledgement RP-ACK, or an error indication, RP-ERROR, to the Service Centre, after reception of an incoming RP-DATA.

The SMC entity forms the CP-DATA message, the user information

element being the RP-ACK or RP-ERROR, and transfer the message by means of the lower layer services.

3.2.1.3 MNSMS-DATA-INDication

An indication used by the SMC entity to pass the user information element (RP-ACK or RP-ERROR) of a received CP-DATA message to SM-RL.

The purpose of this service is to report the outcome of a MO messaging attempt.

3.2.1.4 MNSMS-ESTablish-REQuest

A request from an SMR entity to transmit a RP-DATA message, containing the SM-user information elements, and implicates:

- establishment of a CM-connection for MO message transfer - passing of RP-DATA to the CM-sublayer

- transfer of the CP-DATA message containing the RP-DATA

3.2.1.5 MNSMS-ESTablish-INDication

An indication used by the SMC entity to pass the SM-user information (RP-DATA) of a received CP-DATA message to SM-RL, and implicates esta- blishment of the CM-connection for MT message transfer.

3.2.1.6 MNSMS-ERROR-INDication

An indication used by the SMC entity to pass error information to SM-RL.

The error information may be local or relayed by the CP-ERROR message.

Use of this service implicates release of both CM- and MM-connection.

3.2.1.7 MNSMS-RELease-REQuest

A request to release CM-connection, passed from SM-RL to CM-sublayer when receiving the response from a RP-DATA messaging attempt.

Use of this service implies release of associated MM-connection.

3.2.2 Definition of primitives on network side

This section defines the service primitives used on the network side.

Table 3.2/GSM 04.11 gives an overview of the service primitives and linked main parameter. All MNSMS service primitives defined in this section are passed on a CM-connection.

TABLE 3.2/GSM 04.11

MNSMS service primitives on network side

┌───────────────────────────┬─────────────────────┐

│    SERVICE PRIMITIVES     │                     │

├──────────────────┬────────┤      PARAMETER      │

│       NAME       │  TYPE  │                     │

├──────────────────┼────────┼─────────────────────┤

│   MNSMS-ABORT-   │  Req   │  Cause              │

├──────────────────┼────────┼─────────────────────┤

│                  │  Req   │  MT RP-ACK/RP-ERROR │

│   MNSMS-DATA-    ├────────┼─────────────────────┤

│                  │  Ind   │  MO RP-ACK/RP-ERROR │

├──────────────────┼────────┼─────────────────────┤

│                  │  Req   │  MT RP-DATA         │

│   MNSMS-EST-     ├────────┼─────────────────────┤

│                  │  Ind   │  MO RP-DATA         │

├──────────────────┼────────┼─────────────────────┤

│   MNSMS-ERROR-   │  Ind   │  Cause              │

├──────────────────┼────────┼─────────────────────┤

│   MNSMS-REL-     │  Req   │  Cause              │

├──────────────────┼────────┼─────────────────────┤

│   MNSMS-RES-     │  Req   │  MT RP-ACK/RP-ERROR │

├──────────────────┼────────┼─────────────────────┤

│   MNSMS-SSP-     │  Req   │        --           │

└──────────────────┴────────┴─────────────────────┘

3.2.2.1 MNSMS-ABORT-REQuest

A request from an SMR entity to release a CM-connection in abnormal cases.

When CM-sublayer receives this request, it may form and send the CP-ERROR message to release the connection, or it may release the connection by use of lower layer services.

3.2.2.2 MNSMS-DATA-REQuest

A request from an SMR entity to return the acknowledgement RP-ACK, or an error indication RP-ERROR, to the Mobile Station, after reception of an incoming RP-DATA.

The SMC entity forms the CP-DATA message, the user information element being the RP-ACK or RP-ERROR, and transfer the message by means of the lower layer services.

3.2.2.3 MNSMS-DATA-INDication

An indication used by the SMC entity to pass the user information element (RP-ACK or RP-ERROR) of a received CP-DATA message to SM-RL.

The purpose of the service element is to report the outcome of a MT messaging attempt.

3.2.2.4 MNSMS-ESTablish-REQuest

A request from an SMR entity to transmit a RP-DATA message, containing the SM-user information elements, and implicates:

- establishment of a CM-connection for MT transfer

- passing of RP-DATA to the CM-sublayer

- transfer of the CP-DATA message containing the RP-DATA

3.2.2.5 MNSMS-ESTablish-INDication

An indication used by the SMC entity to pass the SM-user information (RP-DATA) of a received CP-DATA message to SM-RL, and implicates esta- blishment of the CM-connection for the MO message transfer.

3.2.2.6 MNSMS-ERROR-INDication

An indication used by the SMC entity to pass error information to SM-RL.

The error information may be local or relayed by the CP-ERROR message.

Use of the service primitive implicates release of both CM- and MM- connection.

3.2.2.7 MNSMS-RELease-REQuest

A request to release CM-connection, passed from SM-RL to CM-sublayer when receiving the response from a RP-DATA messaging attempt.

Use of this service implies release of associated MM-connection.

3.2.2.8 MNSMS-RESume-REQuest

A request, including a response from the Service Centre (RP-ACK or RP-ERROR), being passed to CM-sublayer to resume operation on a suspended CM-connection.

Use of this service primitive implicates:

- establishment of a new MM-connection by the SMC entity

- relaying of RP-ACK or RP-ERROR in a CP-DATA message

3.2.2.9 MNSMS-SuSPend-REQuest

A suspend request passed from SM-RL to the CM-sublayer to indicate that:

- the SMC entity shall pass the MMSM-RELease-REQuest to

MM-sublayer, enabling the radio resources to be released - the CM-connection shall be maintained while radio resources

are released.

The purpose of the suspend/resume service is to enable the radio resources to be released during the time it takes to:

- relay the RP-DATA message from the MSC to the Service Centre, through various intermediate networks,

- process the RP-DATA message (if possible)

- returning an acknowledgement or error indication (RP-ACK or RP-ERROR) to the MSC.

Use of the suspend/resume mechanism is optional.

3.3 Service provided by SM-RL

In order to support the Short Message Service, the Short Message Relay Layer provide services to the Short Message Transfer Layer.

The Short Message Relay Layer services are provided using layer specific functions and lower layer services offered to the Short Message Relay Layer, controlled by short message control entities called SMRs.

An SMR entity in the MS communicate with an SMR entity in the MSC by means of a peer protocol, SM-RP - Short Message Relay Protocol. The arrow diagrams in annex C gives an overview of the messaging on the Short Message Relay Layer during a short message transfer.  The diagrams in annex C indicates a layer RL.  This is not a layer, but the functional interface to the fixed network. The SM-RL is the upper layer in the MSC. Consequently the service primitives passed between SM-RL and RL indicates the interworking function.

The demands to the SM-RL is as for the CM-sublayer.  This means one SMR entity for each SMC entity, operating as described in section 3.2.

3.3.1 Definition of primitives on MS side

This section defines the service primitives used on the MS side.

Table 3.3/GSM 04.11 gives an overview of the service primitives and linked main parameter. All SM-RL service primitives defined in this section are passed on a SM-RL-connection.

TABLE 3.3/GSM 04.11

SM-RL service primitives on mobile station side

┌───────────────────────────┬─────────────────────┐

│    SERVICE PRIMITIVES     │                     │

├──────────────────┬────────┤      PARAMETER      │

│       NAME       │  TYPE  │                     │

├──────────────────┼────────┼─────────────────────┤

│                  │  Req   │ see sec. 3.3.1.1    │

│   SM-RL-DATA-    ├────────┼─────────────────────┤

│                  │  Ind   │ see sec. 3.3.1.2    │

├──────────────────┼────────┼─────────────────────┤

│   SM-RL-REPORT-  │  Ind   │     Cause           │

└──────────────────┴────────┴─────────────────────┘

3.3.1.1 SM-RL-DATA-REQuest

A request from the SM-TL entity to pass the SMS-SUBMIT PDU and

necessary control information to SM-RL, and implicates:

- establishment of a SM-RL-connection for MO message transfer - transfer of the RP-DATA message, containing the SMS-SUBMIT

PDU, by means of services offered from lower layers

The purpose of this service is to relay the SMS-SUBMIT PDU from the mobile station to the Service Centre.

3.3.1.2 SM-RL-DATA-INDication

An indication used by the SMR entity to pass the SMS-DELIVER PDU and necessary control information of a received RP-DATA message to SM-TL.

3.3.1.3 SM-RL-REPORT-INDication

An indication used by the SMR entity to pass an acknowledgement (RP-ACK) or error information to SM-TL. The error information may be local or relayed by the RP-ERROR message.

3.3.2 Definition of primitives on network side

This section defines the service primitives used on the network side.

Table 3.4/GSM 04.11 gives an overview of the service primitives and linked main parameter. All SM-RL service primitives defined in this section are passed on a SM-RL-connection.

TABLE 3.4/GSM 04.11

SM-RL service primitives on network side

┌───────────────────────────┬─────────────────────┐

│    SERVICE PRIMITIVES     │                     │

├──────────────────┬────────┤      PARAMETER      │

│       NAME       │  TYPE  │                     │

├──────────────────┼────────┼─────────────────────┤

│                  │  Req   │    MT SMS-DELIVER   │

│   SM-RL-DATA-    ├────────┼─────────────────────┤

│                  │  Ind   │    MO SMS-SUBMIT    │

├──────────────────┼────────┼─────────────────────┤

│                  │  Req   │     Cause           │

│   SM-RL-REPORT-  ├────────┼─────────────────────┤

│                  │  Ind   │     Cause           │

└──────────────────┴────────┴─────────────────────┘

3.3.2.1 SM-RL-DATA-REQuest

A request from RL to pass the SMS-DELIVER PDU to SM-RL, and implicates:

- establishment of a SM-RL-connection for MT message transfer

- transfer of the RP-DATA message, containing the SMS-DELIVER

PDU, by means of services offered from lower layers

The purpose of this service is to relay the SMS-DELIVER PDU from the Service Centre to the mobile station.

3.3.2.2 SM-RL-DATA-INDication

An indication used by the SMR entity to pass the SMS-SUBMIT PDU

of a received RP-DATA message to RL.

3.3.2.3 SM-RL-REPORT-REQuest

A request used by RL (the network interworking function) to relay the RP-ACK or RP-ERROR message from the network to the mobile station.

3.3.2.4 SM-RL-REPORT-INDication

An indication used by the SMR entity to pass an acknowledgement (RP-ACK) or error information to RL. The error information may be local or relayed by the RP-ERROR message.

4. Network interfaces

The procedures on SM-RL operates in tandem with the MAP procedures, see Rec. GSM 09.02 and GSM 03.40.

5. CM-procedures

5.1 General

This section describes the procedures used by the SMC entity for short message control on the Connection Management sublayer.  An SMC entity communicates with a correspondant peer entity using a MM-connection.

Multiple MM-connection may be established at the same time, allowing parallell transactions. The description of the procedures are related to one single transaction.

The CM-procedures described in this section can only be performed if a MM-connection has been established between the mobile station and the network. Detailed SDL-diagrams for short message control are contained in Annex B.

5.2 Short Message Control states

The state transition diagram for the SMC entities on both MS-side and network side is contained in Annex B.

5.2.1 SMC-states at the MS-side of the radio interface

The states described in this section is for a SMC entity in a MS, handling both mobile originated- and mobile terminated short message transfer.

5.2.1.1 Idle (State 0)

This state exist when the SMC entity is in idle mode, or when a short message transfer end in a normal or abnormal way.

5.2.1.2 MM-connection pending (State 1)

This state exist when the SMC has requested an MM-connection for mobile originated short message transfer.

5.2.1.3 Wait for CP-ACK (State 2)

This state exist for mobile originating short message transfer when the SMC has received a confirmation that the MM-connection is established, and initiated the transfer of the CP-DATA message.

5.2.1.4 MM-connection established (State 3)

This state exist in both mobile originating- and mobile terminating short message transfer when the SMC has:

-  received the acknowledgement CP-ACK or

-  received the message CP-DATA (including sending of the

associated CP-ACK)

5.2.1.5 MM-connection released (State 4)

This state exist only when the SMC entity has received a release indica- tion from the MM-sublayer during a mobile originated short message transfer.

5.2.2 SMC-states at the network side of the radio interface

The states described in this section is for a SMC entity in a MSC, handling both  mobile originated- and mobile terminated short message transfer.

5.2.2.1 Idle (State 0)

This state exist when the SMC entity is in idle mode, or when a short message transfer end in a normal or abnormal way.

5.2.2.2 MM-connection pending (State 1)

This state exist when the SMC has requested an MM-connection for mobile terminating short message transfer.

5.2.2.3 Wait for CP-ACK (State 2)

This state exist for mobile terminating short message transfer when the SMC has received a confirmation that the MM-connection is established, and initiated the transfer of the CP-DATA message.

5.2.2.4 MM-connection established (State 3)

This state exist in both mobile originating- and mobile terminating short message transfer when the SMC has:

-  received the acknowledgement CP-ACK or

-  received the message CP-DATA (including sending of the

associated CP-ACK)

5.2.2.5 MM-connection released (State 4)

This state exist in a mobile originated short message transfer when the SMC entity has received a suspend request from the Short Message Relay Layer and a release indication is passed to the MM-sublayer.

5.3 Short message transfer control procedures

The procedures needed for short message control are:

- connection establishment procedures

- short message transfer procedures

- clearing procedures

- miscellaneous procedures.

5.3.1 MM-connection establishment

Before the short message transfer is initiated, the peer to peer connec- tion between the MM-sublayers in the MS and the network (MSC) has to be established.

The SMC entity requests the MM-sublayer to establish a MM-connection, and enter the MM-Connection Pending state.

After completion of the MM-connection establishing attempt, a con- firmation is given to indicate whether the MM is ready for short message transfer or not.

The MM-connection establishment is indicated to SMC entity at the network side when the short message has been received by the MM-sublayer (in line with Rec. GSM 04.08).

5.3.2 Initiating short message transfer

When a MM-connection is established, the SMC forwards the CP-DATA message containg the RP-DATA, sets the timer TC1* and enters the Wait for CP-ACK state.  If the timer TC1* expire, the CP-DATA message is retransmitted and the state Wait for CP-ACK is reentered.

The CP-DATA message is only retransmitted once.  If the timer TC1* expire after a retransmission attempt, an error indication is passed to SM-RL and a MM-connection release request is passed to MM-sublayer. The Idle state is then entered.

On receipt on the CP-ACK message, the SMC reset the timer TC1* and enters the MM-Connection Established state.

When receiving a CP-DATA message containing the RP-DATA, the SMC entity checks the parameters.  If the message is accepted, the CP-ACK message is sent and the state MM-Connection Established is entered.

If the message is not accepted, the SMC entity in the MS discard the message and enters the Idle state. If the SMC entity in the network cannot accept a CP-DATA, it sends a CP-ERROR message followed by a MM- connection release request and then enters the Idle state.

5.3.3 Terminating short message transfer

Upon correctly receiving the CP-DATA message carrying the RP-ACK, the CP-ACK message is returned and the state MM-Connection Established is entered.

When sending the CP-DATA message carrying the RP-ACK, the timer TC1* is set and the state Wait for CP-ACK is entered.  If the MM-connection has been released, the procedure MM-connection establishment precedes this procedure.

If the timer TC1* expire, the CP-DATA message is retransmitted and the state Wait for CP-ACK is reentered.

The CP-DATA message is only retransmitted once.  If the timer TC1* expire after a retransmission attempt, an error indication is passed to SM-RL and a MM-connection release request is passed to MM-sublayer. The Idle state is then entered.

5.3.4 Suspend CM-connection

When the SMC entity in the network receives a suspend request from the Short Message Relay Layer, an MM-connection release request is issued and the state MM-Connection Released is entered.

When the suspend mechanism is used, the SMC entity of the MS receives a MM-connection released indication and the state MM-Connection Released is entered.

On both sides, the Transaction Identifier value is kept until the CM- connection is released.

5.3.5 Release MM-connection

Release of the MM-connection is controlled by the SM-RL, except for erroneous conditions.

Upon receiving a release or abort request from SM-RL, the MM-connection release request is sent to MM-sublayer and the state Idle is entered.

In case of abort request, the timer TC1* is reset before sending the MM-connection release request.

5.3.6 Release CM-connection

This procedure is only used in erroneous conditions and implies release of both MM- and CM-connection.

Upon receiving the CP-ERROR message or a MM-error indication, the SMC entity pass an error indication to SM-RL and enter Idle state. In case the timer TC1* is running, it is reset. The CP-ERROR message may only be received by the MS.

5.3.7 Abnormal cases

Errors occuring in the SM-RL causes an abort request to be passed to the CM-sublayer.

When the SMC in the mobile station receives an abort request or detects an erroneous condition it may act upon, a MM-connection release request, without indication of release cause, is passed to MM-sublayer.  The Short Message Relay Layer on the network side will discover this errone- ous condition by expiring timers.

When the SMC on the network side receives an abort request, or detects erroneous conditions it may act upon, a CP-ERROR message indicating the cause is transferred to the MS and a MM-connection release request is passed to the MM-sublayer. The coding of the cause values in the CP- ERROR message is given in Table 8.2/GSM 04.11.

6. SM-RL-procedures

6.1 General

This section describes the procedures used by the SMR entity for short message support on the Short Message Relay Layer.  An SMR entity communicates with a correspondent peer entity using a CM-connection.

Multiple CM-connection may be established at the same time, allowing parallel transactions. It is a functional one to one relation between the SMR entity and the SMC entity of the CM-sublayer for the delivery/submission and acknowledgement/error indication of one short message. Once the short message is delivered/submitted, the acknowledgement/error indication is expected on the same CM-connection. All messages exchanged on this CM-connection belong to the same transaction (identified by one Transaction Identifier value and Flag as defined in 04.08). For each Short Message transfer a new CM-connection is used.

The description of the procedures are related to one single transaction.

The RL-procedures described in this section can only be performed if a CM-connection has been established between the mobile station and the network. Detailed SDL-diagrams for short message control on SM-RL are contained in Annex D.

6.2 Transition states of SMR entity

The state transition diagram for the SMR entities on both MS-side and network side is contained in Annex D.

6.2.1 SMR-states at the MS-side of the radio interface

The states described in this section is for a SMR entity in a MS, handling both mobile originated- and mobile terminated short message transfer.

6.2.1.1 Idle (State 0)

This state exist when the SMR entity is in idle mode, or when a short message transfer end in a normal or abnormal way.

6.2.1.2 Wait for RP-ACK (State 1)

This state exists for mobile originating short message transfer when the SMR has passed the RP-DATA to the SMC entity and set the timer TR1M.

6.2.2 SMR-states at the network side of the radio interface

The states described in this section is for a SMR entity in a MSC, handling both mobile originated- and mobile terminated short message transfer.

6.2.2.1 Idle (State 0)

This state exist when the SMR entity is in idle mode, or when a short message transfer end in a normal or abnormal way.

6.2.2.2 Wait for RP-ACK (State 1)

This state exists for a mobile terminating short message transfer when the SMR has passed the RP-DATA message to the SMC entity and set the timer TR1N.

6.2.2.3 CM-connection suspended (State 2)

When the suspend/resume mechanism is used, the SMR entity will enter this state after passing a received RP-DATA message to RL and setting the timer TR2N.

6.2.2.4 Wait to send RP-ACK (State 3)

The suspend/resume mechanism not being used, the SMR entity will enter this state after passing a received RP-DATA message to RL and setting the timer TR2N.

6.3 Short Message Relay procedures

The procedures needed for short message relaying are:

-  connection establishment procedures

-  short message relay procedures

-  clearing procedures

-  miscellaneous procedures

6.3.1 Initiate short message relaying

When the SMR entity receives a request from SM-TL, or RL, to relay a TPDU, it form and transfer the RP-DATA message, sets the timers TR1* and enters the state Wait for RP-ACK.

Retransmission of RP-DATA message is handled by the CM-sublayer, as described in chapter 5.

6.3.2 Terminate short message relaying

MS-side

The procedure is used to terminate the short message transfer when the SMR entity is in the "Wait for RP-ACK" state.

Different actions are taken in the following situations:

a)  reception of a RP-message

b)  reception of an error indication from CM-sublayer

c)  the timer TR1M expires

In case of a) or b), the timer TR1M is reset, a report indication is passed to SM-TL and a release CM-connection request is passed to CM- sublayer.

In case c), an abort CM-connection request is passed to CM-sublayer, and a report indication is passed to SM-TL.

After completing the procedure, the SMR entity enters the Idle-mode.

Network side

a)  Mobile originated short message transfer.

The procedure is used to terminate the short message transfer when the SMR entity is in state "Wait to send RP-ACK" or state "CM-conn- ection suspended".

When the SMR entity receives the SM-RL-Report-Request (carrying a RP- ACK or RP-ERROR message) the timer TR2N is reset, the RP-message is relayed to the MS and a CM-connection release request is passed to CM- sublayer.

Note:  If the CM-connection is suspended, the Resume CM- connection procedure is invoked before the RP-message may be relayed.

If the SMR entity receives an error indication from CM-sublayer, the timer TR2N is reset, a report indication is passed to RL and the Idle-mode is entered.

If the timer TR2N expires, the SMR entity pass a CM-connection abort request to CM-sublayer, a report indication to RL and the Idle-mode is entered.

b)  Mobile terminated short message transfer.

When the SMR entity receives a RP-ACK or RP-ERROR message, or an error indication from CM, it resets the timer TR1N, passes a report indication to RL and enters the Idle-mode.

If the timer TR1N expires, the SMR entity pass an abort request to CM, a report indication to RL and the Idle-mode is entered.

6.3.3 Suspend CM-connection

This procedure is used when the network allows release of the radio resources while waiting for the report from the Service Centre.

6.3.4 Resume CM-connection

This procedure is used to resume operation on the suspended CM-

connection when the SMR entity receives the report from the Service Centre.

6.3.5 Reception of short message

This procedure checks the parameters of a received RP-DATA message. If it is accepted, the acknowledgement RP-ACK is returned to the origina- tor, the RP-DATA message is passed to SM-TL/RL and a CM-connection release request is passed to CM.  On the network side, the SMR entity also sets the timer TR2N.

If errors are detected, which the SMR entity may act upon, an RP-ERROR message containing a proper error cause is returned to the originator and a CM-connection release request is passed to CM-sublayer. In case of major errors in the RP-DATA message, a CM-connection abort request is passed to CM-sublayer and the Idle-mode is entered.

6.3.6 Abnormal cases

Network side:

Timer expiry:

In the case of TR1N expiry, the SMR entity should release the CM-

connection, and give an appropriate indication to the interworking function.

Error indication from SMC entity:

The SMR entity should release the CM-connection, and give an

appropriate indication to the interworking function.

Format errors etc:

If the SMR entity upon receipt of a RP-DATA message detects an

erroneous condition,(e.g. format errors, invalid parameters etc.) it shall return a RP-ERROR message with an appropriate cause value, and release the CM-connection.

Mobile Station side:

Timer expiry:

In the case of TR1M expiry, the SMR entity should locally release the CM-connection, and give an appropriate indication to the upper SM-TL entity.

Error indication from SMC entity:

The SMR entity should locally release the CM-connection, and give an appropriate indication to the upper SM-TL entity.

Format errors etc:

If the SMR entity upon receipt of a RP-DATA message detects an

erroneous condition,(e.g. format errors, invalid parameters etc.) it shall, if possible, return a RP-ERROR message with an appropriate cause value. Else it should ignore the message and locally release the CM-connection.

7. Message functional definitions and content

7.1 General

The notation used is in line with Rec. GSM 04.08/Ch. 9, and each definition includes:

a) A brief description of the message direction and use.

b) A table listing the information elements in the order of their appearance in the message.  For each information element the table indicates:

1) A reference to the section/recommendation describing the information element.

2) The direction in which the information element may be sent;  MSC to MS, MS to MSC or both.

3) The type of information element, i.e. whether it is - mandatory with fixed length (MF),

- mandatory with variable length (MV),

- optional with fixed length (OF) or

- optional with variable length (OV).

4) The length of the information element, in octets, where ? denotes an undefined maximum length.

7.2 Messages for short message transfer on CM

This section describes the functional definition and content of

the messages sent between two SMC entities.

There are three messages defined, CP-DATA, CP-ACK and CP-ERROR.

7.2.1 CP-DATA

The CP-DATA message is sent between an MSC and an MS, in both

directions. The message contains the user-data, to be relayed between the CM-users, and associated parameters. See Table 7.1/ GSM 04.11.

TABLE 7.1/GSM 04.11

CP-DATA message content

┌────────────────────────┬────────────┬────────────┬───────┬─────────┐

│ Information element    │ Reference  │ Direction  │ Type  │ Length  │

├────────────────────────┼────────────┼────────────┼───────┼─────────┤

│ Protocol discriminator │ GSM 04.08  │    both    │  MF   │         │

├────────────────────────┼────────────┼────────────┼───────┤  1 oct. │

│ Transaction identifier │ GSM 04.08  │    both    │  MF   │         │

├────────────────────────┼────────────┼────────────┼───────┼─────────┤

│ Message type           │ Sec. 8.1.3 │    both    │  MF   │  1 oct. │

├────────────────────────┼────────────┼────────────┼───────┼─────────┤

│ CP-User data           │ Sec.8.1.4.1│    both    │  MV   │≤256 oct.│

└────────────────────────┴────────────┴────────────┴───────┴─────────┘

7.2.2 CP-ACK

The CP-ACK message is sent between an MSC and an MS, in both directions, and is used to acknowledge the reception of a CP-DATA message.

See Table 7.2/GSM 04.11.

TABLE 7.2/GSM 04.11

CP-ACK message content

┌────────────────────────┬────────────┬────────────┬───────┬─────────┐

│ Information element    │ Reference  │ Direction  │ Type  │ Length  │

├────────────────────────┼────────────┼────────────┼───────┼─────────┤

│ Protocol discriminator │ GSM 04.08  │    both    │  MF   │         │

├────────────────────────┼────────────┼────────────┼───────┤  1 oct. │

│ Transaction identifier │ GSM 04.08  │    both    │  MF   │         │

├────────────────────────┼────────────┼────────────┼───────┼─────────┤

│ Message type           │ Sec. 8.1.3 │    both    │  MF   │  1 oct. │

└────────────────────────┴────────────┴────────────┴───────┴─────────┘

7.2.3 CP-ERROR

The CP-ERROR message is sent between an MSC and an MS in one directions, MSC -> MS.  The message is used to convey error information from the network to the mobile station. See Table 7.3/GSM 04.11.

TABLE 7.3/GSM 04.11

CP-ERROR message content

┌────────────────────────┬────────────┬────────────┬───────┬─────────┐

│ Information element    │ Reference  │ Direction  │ Type  │ Length  │

├────────────────────────┼────────────┼────────────┼───────┼─────────┤

│ Protocol discriminator │ GSM 04.08  │   n -> ms  │   MF  │         │

├────────────────────────┼────────────┼────────────┼───────┤  1 oct. │

│ Transaction identifier │ GSM 04.08  │   n -> ms  │   MF  │         │

├────────────────────────┼────────────┼────────────┼───────┼─────────┤

│ Message type           │ Sec. 8.1.3 │   n -> ms  │   MF  │  1 oct. │

├────────────────────────┼────────────┼────────────┼───────┼─────────┤

│ CP-Cause               │ Sec.8.1.4.2│   n -> ms  │   MF  │  1 oct. │

└────────────────────────┴────────────┴────────────┴───────┴─────────┘

7.3 Messages for short message transfer on SM-RL

This section describes the functional definition and content of

the messages sent between two SMR entities.

There are 3 messages defined, RP-DATA, RP-ACK and RP-ERROR.

7.3.1 RP-DATA

The RP-DATA message is sent between an MSC and an MS in both direction.

The message is used to relay the TPDUs. The information elements is in line with Rec. GSM 03.40.  See Table 7.4/GSM 04.11.

TABLE 7.4/GSM 04.11

RP-DATA message content

┌───────────────────────────┬────────────┬───────────┬──────┬─────────┐

│    Information element    │ Reference  │ Direction │ Type │  Length │

├───────────────────────────┼────────────┼───────────┼──────┼─────────┤

│ RP-Message type indicator │ Sec 8.2.2  │  both     │  MF  │   3 bit │

├───────────────────────────┼────────────┼───────────┼──────┼─────────┤

│ RP-Priority indicator     │ Sec 8.2.4  │  n -> ms  │  MF  │   1 bit │

├───────────────────────────┼────────────┼───────────┼──────┼─────────┤

│ RP-Message reference      │ Sec 8.2.3  │  both     │  MF  │   1 oct.│

├───────────────────────────┼────────────┼───────────┼──────┼─────────┤

│ RP-Originator Address     │ Sec 8.2.5.1│  both     │  MV  │1-13 oct.│

├───────────────────────────┼────────────┼───────────┼──────┼─────────┤

│ RP-Destination Address    │ Sec 8.2.5.2│  both     │  MV  │1-12 oct.│

├───────────────────────────┼────────────┼───────────┼──────┼─────────┤

│ RP-User data              │ Sec 8.2.5.3│  both     │  MV  │≤239 oct.│

└───────────────────────────┴────────────┴───────────┴──────┴─────────┘

7.3.2 RP-ACK

This message is used to relay the acknowledgement of a RP-DATA message reception. The information elements is in line with Rec. GSM 03.40.

See Table 7.5/GSM 04.11.

TABLE 7.5/GSM 04.11

RP-ACK message content

┌───────────────────────────┬───────────┬───────────┬──────┬─────────┐

│    Information element    │ Reference │ Direction │ Type │  Length │

├───────────────────────────┼───────────┼───────────┼──────┼─────────┤

│ RP-Message type indicator │ Sec 8.2.2 │  both     │  MF  │   3 bit │

├───────────────────────────┼───────────┼───────────┼──────┼─────────┤

│ RP-Message reference      │ Sec 8.2.3 │  both     │  MF  │   1 oct.│

└───────────────────────────┴───────────┴───────────┴──────┴─────────┘

7.3.3 RP-ERROR

This message is used to relay an error cause from an erroneous short message transfer attempt. The information elements is in line with Rec.

GSM 03.40. See Table 7.6/GSM 04.11.

The contents of the cause field is given in Section 8.2.5.4.

TABLE 7.6/GSM 04.11

RP-ERROR message content

┌───────────────────────────┬────────────┬───────────┬──────┬─────────┐

│    Information element    │ Reference  │ Direction │ Type │  Length │

├───────────────────────────┼────────────┼───────────┼──────┼─────────┤

│ RP-Message type indicator │ Sec 8.2.2  │   both    │  MF  │   3 bit │

├───────────────────────────┼────────────┼───────────┼──────┼─────────┤

│ RP-Message reference      │ Sec 8.2.3  │   both    │  MF  │   1 oct.│

├───────────────────────────┼────────────┼───────────┼──────┼─────────┤

│ RP-Cause                  │ Sec.8.2.5.4│   both    │  MV  │ 2-3 oct.│

└───────────────────────────┴────────────┴───────────┴──────┴─────────┘

8. Message format and information elements coding

8.1 CP-messages

8.1.1 General

The message format and information elements coding is in line with Rec.

GSM 04.08.

The message shall consist of the following parts:

a)  protocol discriminator

b)  transaction identifier

c)  message type

d)  other required information elements

8.1.2 Protocol Discriminator and Transaction Identifier

The Protocol Discriminator (value 1 0 0) and Transaction Identifier is described in Rec. GSM 04.08.

8.1.3 Message type

The purpose of the message type, together with the protocol discrimina- tor, is to identify the function of the message being sent.  The coding of message types is shown in Table 8.1/GSM 04.11.

TABLE 8.1/GSM 04.11

Message types for short message transfer on CM

┌──────────────────────────────────────┐

│                                      │

│     7 6 5 4 3 2 1 0                  │

│                                      │

│     0 0 0 0 0 0 0 1      CP-DATA     │

│     0 0 0 0 0 1 0 0      CP-ACK      │

│     0 0 0 1 0 0 0 0      CP-ERROR    │

│                                      │

└──────────────────────────────────────┘

8.1.4 Other required information elements

As for Rec. GSM 04.08, the Information Element Identifier for a given element is not sent when the element is mandatory in a message.

8.1.4.1 CP-User data element

The CP-User data element is used to carry the RPDU.  It has an

information element identifier, a length indicator and a data field.

The data field will contain the RPDUs.  The maximum length of the data field is 255 octets.  The layout is indicated in figure 8.1/GSM 04.11.

7    6    5    4    3    2    1    0

┌───┬──────────────────────────────────┐

│   │  0    0    0    0    0    0    1 │

│ 0 │      CP-User Data IEI            │   1 oct.

├───┴──────────────────────────────────┤

│           Length indicator           │   1 oct.

├──────────────────────────────────────┤

│               RPDU                   │

│       Maximum length 255 octets      │   ? oct.

└──────────────────────────────────────┘

Figure 8.1/GSM 04.11. CP-User data element layout.

8.1.4.2 CP-Cause element

This element is included in the CP-ERROR message, the layout is given in figure 8.2/GSM 04.11. The error causes are listed in Table 8.2/GSM 04.11.

7    6    5    4    3    2    1    0

┌───┬──────────────────────────────────┐

│   │  0    0    0    0    0    1    0 │

│ 0 │         CP-Cause IEI             │   1 oct.

├───┼──────────────────────────────────┤

│ 0 │          Cause value             │   1 oct.

└───┴──────────────────────────────────┘

Figure 8.2/GSM 04.11.  CP-Cause element layout.

TABLE 8.2/GSM 04.11

Content and coding of CP-Cause

────────────────────────────────────────────────────────────────────────

Cause value   Cause nr.   Cause

6 5 4 3 2 1 0     #

0 0 1 0 0 0 1    17   Network failure

0 0 1 0 1 1 0    22   Congestion

1 0 1 1 1 1 1    95   Invalid message, unspecified

1 1 0 0 0 0 0    96   Mandatory information element error

1 1 0 0 0 0 1    97   Message type non-existent or not implemented

1 1 0 0 0 1 0    98   Message not compatible with the short message transfer

state or message type non-existent or not implemented

1 1 0 0 0 1 1    99   Information element non-existent or not implemented

1 1 0 0 1 0 0   100   Invalid information element contents

1 1 0 0 1 0 1   101   Message not compatible with the short message transfer state

1 1 0 1 1 1 1   111   Protocol error, unspecified

────────────────────────────────────────────────────────────────────────

8.2 RP-messages

8.2.1 General

The coding and layout of the RP-messages is in line with GSM Rec 03.40.

The message shall consist of the following parts:

a)  message type indicator

b)  message reference

c)  other required information elements

8.2.1.1 RP-DATA layout

7    6    5    4    3    2    1    0

┌───┬─────┬──────────────┬─────────────┐

│ * │ PRI │ *    *    *  │    M T I    │   1 oct. MF

├───┴─────┴──────────────┴─────────────┤

│         Message reference            │   1 oct. MF

├──────────────────────────────────────┤

│         Originator Address           │ 1-13 oct. MV

├──────────────────────────────────────┤

│         Destination Address          │ 1-12 oct. MV

├──────────────────────────────────────┤

│         RP-User data                 │   ? oct. MV

│         Maximum length 239 oct.      │

└──────────────────────────────────────┘

Figure 8.3/GSM 04.11.  RP-DATA layout.

8.2.1.2 RP-ACK layout

7    6    5    4    3    2    1    0

┌────────────────────────┬─────────────┐

│ *    *    *    *    *  │    M T I    │   1 oct. MF

├────────────────────────┴─────────────┤

│         Message reference            │   1 oct. MF

└──────────────────────────────────────┘

Figure 8.4/GSM 04.11.  RP-ACK layout.

8.2.1.3 RP-ERROR layout

7    6    5    4    3    2    1    0

┌────────────────────────┬─────────────┐

│ *    *    *    *    *  │    M T I    │   1 oct. MF

├────────────────────────┴─────────────┤

│         Message reference            │   1 oct. MF

├──────────────────────────────────────┤

│         RP-Cause field               │ 2-3 oct. MV

└──────────────────────────────────────┘

Figure 8.5/GSM 04.11.  RP-ERROR layout.

8.2.2 Message type indicator (MTI)

The message type indicator, MTI, is a 3-bit field, located in the first octet of all RP-messages.  The coding of the MTI is shown in Table 8.3/GSM 04.11.

TABLE 8.3/GSM 04.11

Coding of Message Type Indicator

┌─────────────┬───────────────┬──────────────┐

│  Bit value  │   RP-Message  │   Direction  │

│   2 1 0     │               │              │

├─────────────┼───────────────┼──────────────┤

│   0 0 0     │   RP-DATA     │    ms -> n   │

│   0 0 1     │   RP-DATA     │     n -> ms  │

│   0 1 0     │   RP-ACK      │    ms -> n   │

│   0 1 1     │   RP-ACK      │     n -> ms  │

│   1 0 0     │   RP-ERROR    │    ms -> n   │

│   1 0 1     │   RP-ERROR    │     n -> ms  │

│   1 1 0     │   Spare       │       --     │

│   1 1 1     │   Spare       │       --     │

└─────────────┴───────────────┴──────────────┘

8.2.3 Message reference

The message reference field contains a sequence number in the range 0 through 255, and is used to link a RP-ACK message or RP-ERROR message to the associa- ted (preceding) RP-DATA message transfer attempt.

8.2.4. Priority indicator (PRI)

The priority indicator PRI is a 1-bit field indicating whether the RP-Priority Request (ref. Rec. GSM 03.40) have been carried out or not.

The priority indicator is located in bit 6 in the first octet of the RP-DATA message, and has the following coding:

0        RP-Priority Request is FALSE

1        RP-Priority Request is TRUE

8.2.5 Other required information elements

As for Rec.GSM 04.08, the Information Element Identifier for a given element is not sent when the element is mandatory in a message.

8.2.5.1 Originator address element

This element contains the originating entity address, the entity being the SC. The element corresponds to the Calling party BCD number in Rec.

GSM 04.08, with identical coding.

When no originator address is to be sent the value of the length octet of the element is set to 0.

8.2.5.2 Destination address element

This element contains the destination entity address, the entity being the Service Centre. The element corresponds to the Called party BCD number in Rec. GSM 04.08, with identical coding.

When no destination address is to be sent the value of the length octet of the element is set to 0.

8.2.5.3 RP-User data element

The RP-User data field contains the TPDU and is mandatory in a RP-DATA message. The element has a variable length, up to 239 octets, the first octet sent being a length indicator.

7    6    5    4    3    2    1    0

┌───┬──────────────────────────────────┐

│   │  1    0    0    0    0    0    1 │

│ 0 │      RP-User Data IEI            │   1 oct.

├───┴──────────────────────────────────┤

│           Length indicator           │   1 oct.

├──────────────────────────────────────┤

│               TPDU                   │

│       Maximum length 238 octets      │   ? oct.

└──────────────────────────────────────┘

Figure 8.6/GSM 04.11. RP-User data element layout.

8.2.5.4. RP-Cause element

This element is a variable length element always included the RP-ERROR message, conveying a negative result of a RP-DATA message transfer attempt. The element contains a cause value and optionally a diagnostic field giving further details of the error cause.

The coding of the cause value is given in Table 8.4/GSM 04.11. The mapping between error causes in Rec. GSM 04.11 and Rec. GSM 09.02 (MAP) is shown in Table 8.5/GSM 04.11. Parameters included in the return error from MAP (i.e. System Failure) are mapped directly into the diagnostic field.

7    6    5    4    3    2    1    0

┌───┬──────────────────────────────────┐

│   │  1    0    0    0    0    1    0 │

│ 0 │         RP-Cause IEI             │   1 oct.

├───┴──────────────────────────────────┤

│          Length indicator            │   1 oct.

├───┬──────────────────────────────────┤

│ 0 │          Cause value             │   1 oct.

├───┴──────────────────────────────────┤

│        Diagnostic field              │   1 oct.

└──────────────────────────────────────┘

Figure 8.7/GSM 04.11. RP-Cause element layout.

TABLE 8.4/GSM 04.11

Cause value to be contained in RP-ERROR message

in a mobile originating SM-transfer attempt

──────────────────────────────────────────────────────────────────────────────

Cause value   Cause        Cause

Class value   number

6 5 4 3 2 1 0     #

0 0 0│0 0 0 1     1  Unassigned (unallocated) number

0 0 0│1 0 1 0    10  Call barred

0 0 0│1 0 1 1    11  CUG reject

0 0 1│0 1 0 1    21  Short message transfer rejected

0 0 1│1 0 1 1    27  Destination out of service

0 0 1│1 1 0 0    28  Unidentified subscriber

0 0 1│1 1 0 1    29  Facility rejected

0 0 1│1 1 1 0    30  Unknown subscriber

0 1 0│0 0 1 0    38  Network out of order

0 1 0│1 0 0 1    41  Temporary failure

0 1 0│1 0 1 0    42  Congestion

0 1 0│1 1 1 1    47  Resources unavailable, unspecified

1 0 0│0 1 0 1    69  Requested facility not implemented

1 0 1│0 0 0 1    81  Invalid short message transfer reference value

1 0 1│1 1 1 1    95  Invalid message, unspecified

1 1 0│0 0 0 0    96  Mandatory information element missing

1 1 0│0 0 0 1    97  Message type non-existent or not implemented

1 1 0│0 0 1 0    98  Message not compatible with short message transfer state

│                or message type non-existent or not implemented

1 1 0│0 0 1 1    99  Information element non-existent or not implemented

1 1 0│0 1 0 0   100  Invalid information element contents

1 1 0│0 1 0 1   101  Message not compatible with short message transfer state

1 1 0│1 1 1 1   111  Protocol error, unspecified

1 1 1│1 1 1 1   127  Interworking, unspecified

─────┴────────────────────────────────────────────────────────────────────────

TABLE 8.4/GSM 04.11

Cause value to be contained in RP-ERROR message

in a mobile terminating SM-transfer attempt

──────────────────────────────────────────────────────────────────────────────

Cause value   Cause        Cause

Class value   number

6 5 4 3 2 1 0     #

0 0 1│0 1 1 0    22  Memory capacity exceeded

1 1 0│0 0 0 0    96  Mandatory information element missing

1 1 0│0 0 1 1    99  Information element non-existent or not implemented

1 1 0│0 1 0 0   100  Invalid information element contents

1 1 0│0 1 0 1   101  Message not compatible with short message transfer state

1 1 0│1 1 1 1   111  Protocol error, unspecified

─────┴────────────────────────────────────────────────────────────────────────

8.2.6 Mapping between error causes in MAP and 04.11

TABLE 8.5/GSM 04.11

MAP-04.11 error cause mapping.

┌──────────────────────────┬────────────────────────────┐

│ Return error             │  Cause value in the        │

│ from the MAP-proc.       │   RP-ERROR message         │

├──────────────────────────┼────────────────────────────┤

│ CallBarred               │ 10 Call barred             │

│ CUG_Reject               │ 11 CUG reject              │

│ DataMissing              │ 38 Network out of order    │

│ FacilityNotSupported     │ 29 Facility reject         │

│ SystemFailure            │ 38 Network out of order    │

│ UnexpectedDataValue      │ 38 Network out of order    │

│ UnidentifiedSubscriber   │ 28 Unidentified subscriber │

│ UnknownServiceCentre     │  1 Unassigned number       │

│ UnknownSubscriber        │ 30 Unknown subscriber      │

├──────────────────────────┤                            │

│ MAP operation failure    │ 38 Network out of order    │

│ (e.g. reject condition,  │                            │

│  timer expired or        │                            │

│  transaction abort).     │                            │

│                          │                            │

└──────────────────────────┴────────────────────────────┘

┌──────────────────────────┬────────────────────────────┐

│ Cause value in the       │  Return error to be        │

│ RP-ERROR message:        │  included in the MAP-proc  │

├──────────────────────────┼────────────────────────────┤

│ 22 Memory capacity       │ MemoryCapacityExceeded     │

│    exceeded              │                            │

│                          │                            │

│ 96, 99, 100, 101, 111    │ SM_DeliveryFailure         │

│                          │                            │

├──────────────────────────┤                            │

│ (local errors)           │                            │

│ TR1N timeout             │ SM_DeliveryFailure         │

│ MNSMS-error-ind          │        " "                 │

│  (i e No SAPI 3)         │                            │

└──────────────────────────┴────────────────────────────┘