Annex A

ARROW DIAGRAMS

Arrow diagram A1/A2:

The diagram reflects MO-messaging by means of interlayer service primitives and the actual messages being transferred between the layer entities.

-  MNSMS-primitives indicates services provided by CM to SM-RL

-  MMSM-primitives indicates services provided by MM to CM

-  CP-DATA is the CM-message carrying SM-RP data units

-  CP-ACK acknowledge CP-DATA reception on CM.

The difference between Diagram A1 and A2 is that use of thesuspend mechanism is indicated in A1.

Arrow diagram A3:

The diagram reflects MT-messaging by means of interlayer service primitives and the actual messages being transferred between the layer entities.

- MNSMS-primitives indicates services provided by CM to SM-RL.

- MMSM-primitives indicates services provided by MM to CM.

- CP-DATA is the CM-message carrying SM-RP data units

- CP-ACK acknowledge CP-DATA reception on CM.

ANNEX B

SDL-DESCRIPTION OF THE CM-LAYER

B1. Introduction

This annex contains an SDL-description of the Connection Management Sublayer in terms of the Short Message Service Support.  The CM- sublayer provides services to Short Message Relay Layer.

The SDLs contain a mixture of peer to peer messages and conceptual primitives between the layers SM-RL, CM and MM, as viewed by the SMC entities.  SDL-1/4 shows the SMC entity on MS-side, and SDL-5/8 on the network side.

The lower layers (below MM) are transparent to an SMC entity.

ANNEX C

ARROW DIAGRAMS

Arrow diagram C1:

The diagram reflects MO-messaging by means of interlayer service primitives and the actual messages being transferred between the layer entities.

-  SM-RL-primitives indicates services provided by SM-RL to SM-TL and RL (*).

-  MNSMS-primitives indicates services provided by CM to SM-RL.

-  RP-DATA is the SM-RL message carrying SM-TP data units

-  RP-ACK acknowledge RP-DATA reception on SM-RL

Arrow diagram C2:

The diagram reflects MT-messaging by means of interlayer service primitives and the actual messages being transferred between the layer entities.

-  SM-RL-primitives indicates services provided by SM-RL to SM-TL and RL (*).

-  MNSMS-primitives indicates services provided by CM to SM-RL.

-  RP-DATA is the SM-RL message carrying SM-TP data units

-  RP-ACK acknowledge RP-DATA reception on SM-RL

(*) Note:

The SM-RL being the upper layer in the MSC, an interworking function between SM-RL-procedures and MAP-procedure is necessary. The term "RL" is used in the diagrams to indicate this function (see figure).

──────────────────────────────────

┌───┬─────────────┬───┐

│   │Interw. func.│   │

│   └──────┬──────┘   │

SM-RL	   │ SM-RL-   │  MAP-    │

│ proc.    │  proc.   │

└──────────┴──────────┘

──────────────────────────────────

ANNEX D

SDL-DESCRIPTION OF THE SHORT MESSAGE RELAY LAYER

D1. Introduction

This annex contains an SDL-description of the Short Message Relay Layer in terms of the Short Message Service Support.  The Short Message Relay Layer provides services to Short Message Transfer Layer.

The SDLs contain a mixture of peer to peer messages and conceptual primitives between the layers SM-TL, SM-RL and CM, as viewed by the SMR entities.  SDL-1/2 shows the SMR entity on MS-side, and SDL-3/4 on the network side.

The lower layers (below CM) are transparent to an SMR entity.

ANNEX E

E-1:	 CP-CAUSE DEFINITION

Cause no 17:  "Network failure"

This cause is sent to the MS if the MSC cannot service an MS generated request because of PLMN failures, e.g. problems in MAP.

Cause no 22:  "Congestion"

This cause is sent if the service request cannot be actioned because of congestion (e.g. no channel, facility busy/congested etc.).

Cause no 95:  "Invalid message, unspecified"

This cause is used to report an invalid message event only when no other cause in the invalid message class applies.

Cause no 96:  "Mandatory information element missing"

This cause indicates that the equipment sending this cause has received a message where a mandatory information element is missing and/or has a content error (the two cases are undistinguishable).

Cause no 97:  "Message type non-existent or not implemented"

This cause indicates that the equipment sending this cause has received a message with a message type it does not recognize either because this is a message not defined or defined but not implemented by the equipment sending this cause.

Cause no 98:  "Message not compatible with short message transfer state or message type non-existent or not implemented"

This cause indicates that the equipment sending this cause has received a message such that the procedures do not indicate that this is a permissible message to receive while in the short message transfer state, or a STATUS message was received indicating an incompatible short message transfer state.

Cause no 99:  "Information element non-existent or not implemented"

This cause indicates that the equipment sending this cause has received a message which includes information elements not recognized because the information element identifier is not defined or it is defined but not implemented by the equipment sending the cause.

However, the information element is not required to be present in the message in order for the equipment sending the cause to process the message.

Cause no 100: "Invalid information element contents"

This cause indicates that the equipment sending this cause has received an information element which it has implemented; however, one or more of the fields in the information element are coded in such a way which has not been implemented by the equipment sending this cause.

Cause no 101: "Message not compatible with short message transfer state"

This cause indicates that a message has been received which is incompatible with the short message transfer state.

Cause no 111: "Protocol error, unspecified"

This cause is used to report a protocol error event only when no other cause applies.

RP-CAUSE DEFINITION

E-2:	Mobile originated SM-transfer

Cause no 1:  "Unassigned (unallocated) number"

This cause indicates that the destination requested by the Mobile Station cannot be reached because, although the number is in a valid format, it is not currently assigned (allocated).

Cause no 10:  "Call barred"

This cause indicates that the outgoing call barred service applies to the short message service for the called destination.

Cause no 11:  "CUG reject"

This cause indicates that the short message transfer attempt does not pass the CUG check or that the CUG barring conditions are transgressed.

Cause no 21:  "Short message transfer rejected"

This cause indicates that the equipment sending this cause does not wish to accept this short message, although it could have accepted the short message since the equipment sending this cause is neither busy nor incompatible.

Cause no 27:  "Destination out of service"

This cause indicates that the destination indicated by the Mobile Station cannot be reached because the interface to the destination is not functioning correctly.  The term "not functioning correctly" indicates that a signalling message was unable to be delivered to the remote user;  e.g., a physical layer or data link layer failure at the remote user, user equipement off-line, etc.

Cause no 28:  "Unidentified subscriber"

This cause indicates that the subscriber is not registered in the PLMN (i.e. IMSI not known)

Cause no 29:  "Facility rejected"

This cause indicates that the facility requested by the Mobile Station is not supported by the PLMN.

Cause no 30:  "Unknown subscriber"

This cause indicates that the subscriber is not registered in the HLR (i.e. IMSI or directory number is not allocated to a subscriber).

Cause no 38:  "Network out of order"

This cause indicates that the network is not functioning correctly and that the condition is likely to last a relatively long period of time; e.g., immediately reattempting the short message transfer is not likely to be successful.

Cause no 41:  "Temporary failure"

This cause indicates that the network is not functioning correctly and that the condition is not likely to last a long period of time; e.g., the Mobile Station may wish to try another short message transfer attempt almost immediately.

Cause no 42:  "Congestion"

This cause indicates that the short message service cannot be serviced because of high traffic.

Cause no 47:  "Resources unavailable, unspecified"

This cause is used to report a resource unavailable event only when no other cause applies.

Cause no 69:  "Requested facility not implemented"

This cause indicates that the PLMN does not support the

requested short message service.

Cause no 81:  "Invalid short message transfer reference value"

This cause indicates that the equipment sending this cause has received a message with a short message reference which is not currently in use on the MS-network interface.

Cause no 95:  "Invalid message, unspecified"

This cause is used to report an invalid message event only when no other cause in the invalid message class applies.

Cause no 96:  "Mandatory information element missing"

This cause indicates that the equipment sending this cause has received a message where a mandatory information element is missing and/or has a content error (the two cases are undistinguishable).

Cause no 97:  "Message type non-existent or not implemented"

This cause indicates that the equipment sending this cause has received a message with a message type it does not recognize either because this is a message not defined or defined but not implemented by the equipment sending this cause.

Cause no 98:  "Message not compatible with short message transfer state or message type non-existent or not implemented"

This cause indicates that the equipment sending this cause has received a message such that the procedures do not indicate that this is a permissible message to receive while in the short message transfer state, or a STATUS message was received indicating an incompatible short message transfer state.

Cause no 99:  "Information element non-existent or not implemented"

This cause indicates that the equipment sending this cause has received a message which includes information elements not recognized because the information element identifier is not defined or it is defined but not implemented by the equipment sending the cause.

However, the information element is not required to be present in the message in order for the equipment sending the cause to process the message.

Cause no 100: "Invalid information element contents"

This cause indicates that the equipment sending this cause has received an information element which it has implemented; however, one or more of the fields in the information element are coded in such a way which has not been implemented by the equipment sending this cause.

Cause no 101: "Message not compatible with short message transfer state"

This cause indicates that a message has been received which is incompatible with the short message transfer state.

Cause no 111: "Protocol error, unspecified"

This cause is used to report a protocol error event only when no other cause applies.

Cause no 127: "Interworking, unspecified"

This cause indicates that there has been interworking with a network which does not provide causes for actions it takes; thus, the precise cause for a message which is being send cannot be ascertained.

E-3:	 Mobile terminating SM-transfer

Cause no 22:  "Memory capacity exceeded"

This cause indicates that the mobile station cannot store the incoming short message due to lack of storage capacity.

Cause no 96:  "Mandatory information element missing"

This cause indicates that the equipment sending this cause has received a message where a mandatory information element is missing and/or has a content error (the two cases are undistinguishable).

Cause no 99:  "Information element non-existent or not implemented"

This cause indicates that the equipment sending this cause has received a message which includes information elements not recognized because the information element identifier is not defined or it is defined but not implemented by the equipment sending the cause.

However, the information element is not required to be present in the message in order for the equipment sending the cause to process the message.

Cause no 100: "Invalid information element contents"

This cause indicates that the equipment sending this cause has received an information element which it has implemented; however, one or more of the fields in the information element are coded in such a way which has not been implemented by the equipment sending this cause.

Cause no 101: "Message not compatible with short message transfer state"

This cause indicates that a message has been received which is incompatible with the short message transfer state.

Cause no 111: "Protocol error, unspecified"

This cause is used to report a protocol error event only when no other cause applies.