European digital cellular telecommunications system (Phase 2);

General on Terminal Adaptation Functions (TAF)

for Mobile Stations (MS)

(GSM 07.01)

Whilst every care has been taken in the preparation and publication of this document, errors in content, typographical or otherwise, may occur. If you have comments concerning its accuracy, please write to "ETSI Editing and Standards Approval Dept." at the address shown on the title page.

Contents

.Begin Table C.

Foreword	7

1.1	SCOPE	9

1.2	Normative references	9

1.3	Definitions and abbreviations	11

2	ACCESS REFERENCE CONFIGURATION	12

3	not used	12

4	not used	12

5	FUNCTIONS TO SUPPORT DATA SERVICES	13

6	SUPPORT OF NON TRANSPARENT BEARER SERVICES	13

6.1	Functions of the Layer 2 Relay	13

6.2	Radio Link Services Used	15

6.3	Flow Control - General Description	15

6.3.1	End to End Flow Control	15

6.3.2	Back Pressure	15

6.3.3	Receive not Ready	15

7	STRUCTURE OF THE GSM 07-SERIES OF SPECIFICATIONS	16

8	FUNCTIONS COMMON TO ALL INTERFACES	16

8.1	Synchronization of the Traffic Channel	16

8.2	Filtering of Channel Control Information	17

8.2.1	General	17

8.2.2	Filtering Process to Be Applied	18

8.3	Terminal Compatibility Decision	19

8.3.1	Compatibility Check	19

8.3.2	Selection of Appropriate Terminal Function	20

8.3.3	Indication of Compatibility Requirements to the PLMN	20

8.3.3.1	Indication in case of Mobile Terminating Calls	20

8.3.3.2	Indication in case of Mobile originating calls	21

8.4	Test Loops	21

8.5	Alternate speech/data and speech/facsimile group 3	22

ANNEX 1 (Informative) List of Bearer Capability Elements	23

ANNEX 2 (Normative) 	27

Setting of Bearer Capability, Low Layer Compatibility and High Layer Compatibility Information Element for GSM Bearer Services and GSM TeleServices	27

0	Scope	27

1	Bearer Capability Information Element	27

1.1	Introduction	27

1.1.1	General Consideration	27

1.1.2	Interpretation of the Diagrams	28

Table II.1	BC-Parameters subject to negotiation procedure	29

Table II.2	General Structure of the BC-Information Element	30

Table II.3	Selection of flow control method (for CE:NT with SA:A only)	30

Table II.4a	Modem Type subject to negotiation procedure	32

Table II.4b	Intermediate Rate negotiation procedure	33

Table II.5	BC parameter setting (part 1)	34

Table II.5	BC parameter setting (part 2)	35

Table II.6	Channel combinations	36

Table II.7	TS61/TS62 Negotiation rules	37

1.2	Bearer Service 21 ... 26, Data Circuit Duplex Asynchronous	38

1.2.1	Unrestricted digital information transfer capability	38

1.2.2	3.1 kHz audio ex-PLMN information transfer capability	39

1.3.	Bearer Service 31 ... 34, Data Circuit Duplex Synchronous	40

1.3.1	Unrestricted digital information transfer capability	40

1.3.1.1	Non-X.32 Cases	40

1.3.1.2	X.32 Case (Packet Service)	41

1.3.2	3.1 kHz audio ex-PLMN information transfer capability	42

1.3.2.1	Non-X.32 Cases	42

1.3.2.2	X.32 Case ( Packet Service )	43

1.4	Bearer Service 41 ... 46, PAD Access Asynchronous	44

1.5	Bearer Service 51 ... 53 ,Data Packet Duplex Synchronous,	45

Unrestricted digital information transfer capability	45

1.6	Bearer Service 61, Alternate Speech/Data	46

1.6.1	Bearer Service 61,"Speech"	46

1.6.2	Bearer Service 61, 3.1 kHz audio ex-PLMN information transfer capability	47

1.6.2.1	Asynchronous	47

1.6.2.2	Synchronous	48

1.7	Bearer Service 81, Speech followed by Data	49

1.7.1 Bearer Service 81,"Speech"	49

1.7.2	Bearer Service 81, 3.1 kHz audio ex-PLMN information transfer capability	49

1.7.2.1	Asynchronous	49

1.7.2.2	Synchronous	49

1.8	Teleservice 11 ... 12, Speech	49

1.9	Teleservice 21 ... 23, Short Message	49

1.10	Teleservice 61, Alternate Speech and Facsimile group 3	50

1.10.1	Teleservice 61, "Speech"	50

1.10.2	Teleservice 61, "Facsimile group 3" 	50

1.11	Teleservice 62, Automatic Facsimile Group 3	51

2	Low Layer/High Layer Compatibility Information Element	51

2.1	Introduction	51

2.1.1	General Consideration	51

2.1.2	Interpretation of the Tables	52

2.2	LLC Bearer Service 21 ... 26	52

2.2.1	Unrestricted digital information transfer capability 	52

2.2.2	3,1 kHz audio ex-PLMN information transfer capability	53

2.3	LLC Bearer Service 31 ... 34	54

2.3.1	Unrestricted digital information transfer capability	54

2.3.2	3.1kHz audio ex-PLMN information transfer capability 	55

2.4	LLC Bearer Services 41 ... 46	55

2.5	LLC Bearer Services 51 ... 53	56

2.5.1	Unrestricted digital information transfer capability	56

2.6	LLC Bearer Service 61	56

2.6.1	3,1 kHz audio ex-PLMN information transfer capability, Asynchronous	56

2.6.2	3,1 kHz audio ex-PLMN information transfer capability, Synchronous	56

2.7	LLC Bearer Service 81	56

2.7.1	3,1 kHz audio ex-PLMN information transfer capability, Asynchronous	56

2.7.2	3,1 kHz audio ex-PLMN information transfer capability, Synchronous	56

2.8	HLC Teleservices 11 ... 12	57

2.9	HLC Teleservices 21 ... 23	57

2.10	HLC Teleservice 61	57

2.11	HLC Teleservice 62	57

History	58

.End Table C.

Blank page

Foreword

This European Telecommunication Standard (ETS) has been produced by the Special Mobile Group (SMG) Technical Committee (TC) of the European Telecommunications Standards Institute (ETSI).

This ETS describes the functions required for the terminal adaptation of terminal equipment to the Mobile Termination (MT) within the European digital cellular telecommunications system (Phase 2).

This ETS is based upon the principles of terminal adaptor functions presented in the CCITT I-series of recommendations (I.460 - I.463).

This ETS corresponds to GSM technical specification, GSM 07.01 version 4.8.0.

The specification from which this ETS has been derived was originally based on CEPT documentation, hence the presentation of this ETS may not be entirely in accordance with the ETSI/PNE rules.

Reference is made within this ETS to GSM Technical Specifications (GSM-TS) (NOTE).

Reference is also made within this ETS to GSM 0x.xx. series. The specifications in the series can be identified, with their full title, within the normative reference Clause of this ETS by the first two digits of their GSM reference number e.g. GSM 09.xx series, refers to GSM 09.01, GSM 09.02, etc.

NOTE:	TC-SMG has produced documents which give the technical specifications for the implementation of the European digital cellular telecommunications system. Historically, these documents have been identified as GSM Technical Specifications (GSM-TSs). These TSs may have subsequently become I-ETSs (Phase 1), or ETSs (Phase 2), whilst others may become ETSI Technical Reports (ETRs). GSM-TSs are, for editorial reasons, still referred to in current GSM ETSs.

Proposed transposition dates

Date of adoption of this ETS:	30 July 1995

Date of latest announcement of this ETS (doa):	31 October 1995

Date of latest publication of new National Standard

or endorsement of this ETS (dop/e):	30 April 1996

Date of withdrawal of any conflicting National Standard (dow):	30 April 1996

Blank page

.c.1.1	SCOPE

This specification is based on the principles of terminal adaptor functions presented in the CCITT I-series of recommendations (I.460 - I.463).

The GSM PLMN supports a wide range of voice and non-voice services in the same network. In order to enable non-voice traffic in the GSM PLMN there is a need to connect various kinds of terminal equipments to the Mobile Termination (MT). The target of this Specification is to outline the functions needed for the terminal adaptation.

In the GSM 02.02 the bearer services are described. The general network configuration is described in GSM 03.02 and the GSM PLMN access reference configuration is defined in GSM 04.02. The various connection types used in the GSM PLMN are presented in GSM 03.10. Terminology used in this Specification is presented in GSM 01.04. For support of data services between GSM PLMN and other networks see GSM 09-series of Specifications.

.c.1.2	Normative references

This ETS incorporates by dated and undated reference, provisions from other publications. These normative references are cited at the appropriate places in the text and the publications are listed hereafter. For dated references, subsequent amendments to or revisions of any of these publications apply to this ETS only when incorporated in it by amendment or revision. For undated references, the latest edition of the publication referred to applies.

[1]	GSM 01.04 (ETR 100): "European digital cellular telecommunication system (Phase 2); Definitions, abbreviations and acronyms".

[2]	GSM 02.02 (ETS 300 501): "European digital cellular telecommunication system (Phase 2); Bearer Services (BS) supported by a GSM Public Land Mobile Network (PLMN)".

[3]	GSM 02.03 (ETS 300 502): "European digital cellular telecommunication system (Phase 2); Teleservices supported by a GSM Public Land Mobile Network (PLMN)".

[4]	GSM 03.02 (ETS 300 522): "European digital cellular telecommunication system (Phase 2); Network architecture".

[5]	GSM 03.10 (ETS 300 528): "European digital cellular telecommunication system (Phase 2); GSM Public Land Mobile Network (PLMN) connection types".

[6]	GSM 04.02 (ETS 300 551): "European digital cellular telecommunication system (Phase 2); GSM Public Land Mobile Network (PLMN) access reference configuration".

[7]	GSM 04.08 (ETS 300 557): "European digital cellular telecommunication system (Phase 2); Mobile radio interface layer 3 specification".

[8]	GSM 04.21 (ETS 300 562): "European digital cellular telecommunication system (Phase 2); Rate adaption on the Mobile Station - Base Station System (MS - BSS) interface".

[9]	GSM 04.22 (ETS 300 563): "European digital cellular telecommunication system (Phase 2); Radio Link Protocol (RLP) for data and telematic services on the Mobile Station - Base Station System (MS - BSS) interface and the Base Station System - Mobile-services Switching Centre (BSS - MSC) interface".

[10]	GSM 05.05 (ETS 300 577): "European digital cellular telecommunication system (Phase 2); Radio transmission and reception".

[11]	GSM 07.02 (ETS 300 583): "European digital cellular telecommunication system (Phase 2); Terminal Adaptation Functions (TAF) for services using asynchronous bearer capabilities".

[12]	GSM 07.03 (ETS 300 584): "European digital cellular telecommunication system (Phase 2); Terminal Adaptation Functions (TAF) for services using synchronous bearer capabilities".

[13]	GSM 07.05 (ETS 300 585): "European digital cellular telecommunication system (Phase 2); Use of Data Terminal Equipment - Data Circuit terminating Equipment (DTE - DCE) interface for Short Message Service (SMS) and Cell Broadcast Service (CBS)".

[14]	GSM 07.06 (ETS 300 586): "European digital cellular telecommunication system (Phase 2); Use of the V series Data Terminal Equipment - Data Circuit terminating Equipment (DTE - DCE) interface at the Mobile Station (MS) for Mobile Termination (MT) configuration".

[15]	GSM 09.01 (ETR 109): "European digital cellular telecommunication system (Phase 2); General network interworking scenarios".

[16]	GSM 09.02 (ETS 300 599): "European digital cellular telecommunication system (Phase 2); Mobile Application Part (MAP) specification".

[17]	GSM 09.03 (ETS 300 600): "European digital cellular telecommunication system (Phase 2); Signalling requirements on interworking between the Integrated Services Digital Network (ISDN) or Public Switched Telephone Network (PSTN) and the Public Land Mobile Network (PLMN)".

[18]	GSM 09.04 (ETS 300 601): "European digital cellular telecommunication system (Phase 2); Interworking between the Public Land Mobile Network (PLMN) and the Circuit Switched Public Data Network (CSPDN)".

[19]	GSM 09.05 (ETS 300 602): "European digital cellular telecommunication system (Phase 2); Interworking between the Public Land Mobile Network (PLMN) and the Packet Switched Public Data Network (PSPDN) for Packet Assembly/Disassembly (PAD) facility access".

[20]	GSM 09.06 (ETS 300 603): "European digital cellular telecommunication system (Phase 2);  Interworking between a Public Land Mobile Network (PLMN) and a Packet Switched Public Data Network/Integrated Services Digital Network (PSPDN/ISDN) for the support of packet switched data transmission services".

[21]	GSM 09.07 (ETS 300 604): "European digital cellular telecommunication system (Phase 2); General requirements on interworking between the Public Land Mobile Network (PLMN) and the Integrated Services Digital Network (ISDN) or Public Switched Telephone Network (PSTN)".

[22]	GSM 09.08 (ETR 110): "European digital cellular telecommunication system (Phase 2); Application of the Base Station System management Application Part (BSSMAP) on the E-interface".

[23]	GSM 09.10 (ETS 300 605): "European digital cellular telecommunication system (Phase 2); Information element mapping between Mobile Station - Base Station System and BSS - Mobile-services Switching Centre (MS - BSS - MSC)  Signalling procedures and the Mobile Application Part (MAP)".

[24]	GSM 09.11 (ETS 300 606): "European digital cellular telecommunication system (Phase 2); Signalling interworking for supplementary services".

[25]	GSM 09.90 (ETR 111): "European digital cellular telecommunication system (Phase 2); Interworking between Phase 1 infrastructure and Phase 2 Mobile Stations (MS)".

[26]	CCITT Series V Recommendations: "Data communication over the Telephone network".

[27]	CCITT Series X Recommendations: "Data communication networks".

[28]	CCITT Recommendation X.25 "Interface between data terminal equipment (DTE) and data circuit - terminating equipment (DCE) for terminals operating in the packet mode and connected to public data networks by dedicated circuit".

[29]	CCITT Recommendation X.150: "Data Communication Networks:Transmission, Signalling and Switching, Network Aspects, Maintenance and Administrative Arrangements".

[30]	CCITT Recommendation V.25bis: "Automatic Calling and/or Answering Equipment on the General Switched Telephone Network (GSTN) using the 100-Series Interchange Circuits".

[31]	CCITT Recommendation V.54: "Loop Test Devices for Modems".

[32]	CCITT Recommendation V.110: "Support of data terminal equipments (DTEs) with V-Series interfaces by an integrated services digital network".

[33]	CCITT Recommendation I.460-I.464: "ISDN Overall Network Aspects and Functions, User Network Interfaces".

[34]	ETS 300 102-1: "Integrated Services Digital Network (ISDN); User-network interface layer 3 specifications for basic call control".

[35]	ETR 018: "Integrated Services Digital Network (ISDN), Application of the BC-,HLC-,LLC-Information elements by terminals supporting ISDN services".

[36]	ISO/IEC 6429: "Information technology - Control functions for coded character sets".

.c.1.3	Definitions and abbreviations

In addition to those below, abbreviations used in this specification are listed in GSM 01.04.

CALL PROC	CALL PROCEEDING

CALL CONF	CALL CONFIRMED

CONNACK	CONNECT ACKNOWLEDGEMENT

.c.2	ACCESS REFERENCE CONFIGURATION

Figure 1 presents the reference configuration for access to a GSM PLMN (see GSM 04.02).

MS

:<────────────────────────────────────>:

:                           ┌───────┐  :  ┌───────────┐

:                           │  MT0  ├──┼──┤  BSS/MSC  │

:                           └───────┘  :  └───────────┘

:             ┌───────┐     ┌───────┐  :  ┌───────────┐

:             │  TE1  ├──┼──┤  MT1  ├──┼──┤  BSS/MSC  │

:             └───────┘  :  └───────┘  :  └───────────┘

┌───────┐     ┌───────┐  :  ┌───────┐  :  ┌───────────┐

│  TE2  ├──┼──┤  TA   ├──┼──┤  MT1  ├──┼──┤  BSS/MSC  │

└───────┘  :  └───────┘  :  └───────┘  :  └───────────┘

┌───────┐  :             :  ┌───────┐  :  ┌───────────┐

│  TE2  ├──┼────────────────┤  MT2  ├──┼──┤  BSS/MSC  │

└───────┘  :             :  └───────┘  :  └───────────┘

:             :             :

R             S             Um

─┼─  = reference point

TE1 = ISDN terminal

TE2 = V- or X-type terminal

TA  = Terminal Adaptor

BSS = Base Station System

MSC = Mobile Switching Centre

Figure 1:	GSM PLMN Access Reference Configuration

Within the scope of this Specification the Mobile Termination MT0 means a fully integrated MS including data terminal and its adaptation functions. MT1 includes ISDN terminal adaptation functions and MT2 includes CCITT V- or X-series terminal adaptation functions among other MT functions.

.c.3	not used

.c.4	not used

.c.5	FUNCTIONS TO SUPPORT DATA SERVICES

The main functions of the MT to support data services are:

-	functions to ensure conformity of terminal service requests to network capability

-	physical connection of the reference points R and S

-	flow control of signalling and mapping of user signalling to/from GSM PLMN access signalling

-	rate adaptation of user data (see GSM 04.21)

-	flow control of non-transparent user data and mapping of flow control for asynchronous data services

-	support of data integrity between the MS and the interworking function in the GSM PLMN

-	end-to-end synchronization between terminals

-	filtering of status information

-	functions to support non-transparent bearer services e.g. termination of the Radio Link Protocol (RLP) and the Layer 2 Relay function (L2R) (where applicable)

-	terminal compatibility checking

-	optional support of local test loops

In addition, functions to support autocalling and autoanswering are optionally specified in accordance with CCITT Rec. V.25 bis (although the use of other autocalling/autoanswering procedures are not prohibited provided that mapping in a functionally equivalent way to GSM 04.08 call control is also provided).

Other functional entities can be envisaged apart from the TAF. One of the physical interface to all these functions is the DTE/DCE interface to the MT. Normally, this DTE/DCE interface is associated with the TAF, if available. Therefore the access to any of these other functional entities, if implemented, via the DCE/DTE interface must be triggered by appropriate command sequences which are described in the applicable specifications (although the use of other procedures is not prohibited provided that mapping in a functionally equivalent way is also provided). These command sequences can be issued by the DTE only when the MT is in the appropriate command status and there is no data connection pending. They are interpreted by an MT internal control function and result in an association of the DTE/DCE interface with the addressed function, if available.

.c.6	SUPPORT OF NON TRANSPARENT BEARER SERVICES

In order to support non transparent bearer services a Layer 2 Relay (L2R) function is included in the mobile termination. The details of the particular L2R function for the different non transparent bearer services are contained in the appropriate GSM 07-series Specification. This section describes the general aspects of the L2R function.

The Layer 2 Relay (L2R) function provides for the reliable transportation of known, i.e. non transparent, user protocols across the radio interface of a GSM PLMN. The L2R functions are located in the Mobile Termination (MT) and the Interworking Function (IWF) associated with a Mobile Switching Centre (MSC). The L2R uses the services provided by the Radio Link Protocol (RLP) to transport the non transparent protocol information between the MS and the IWF.

.c.6.1	Functions of the Layer 2 Relay

The complete protocol reference models for data and telematic services are described in GSM 03.10. The subset of those protocol reference models relating to the L2R function is reproduced in Figure 2.

MS                               IWF

───────────── Transparent higher layer protocol ─────────────

┌─────────┐                       ┌─────────┐

│   L2R   ├───── L2R Protocol ────┤   L2R   │

─ NTP ──┤   ┌─┬───┤                       ├───┬─┐   ├── NTP ──

│   │ │   ├─ Radio Link Protocol ─┤   │ │   │

├───┤ ├───┤                       ├───┤ ├───┤

─ IFP ──┤   │                                   │   ├── IFP ──

├───┤                                   ├───┤

NTP - Non Transparent Protocol

IFP - Interface Protocol

Figure 2

The Non Transparent Protocol (NTP) will normally be a layer 2 protocol for OSI conformant protocols or an equivalent in the case of non OSI protocols. The Interface Protocol (IFP) will normally be a layer 1 protocol for OSI conformant systems or equivalent for non OSI systems.

The L2R can be considered to consist of 3 sub-functions, see figure 3.

┌────────────────────────────┐

│                            │

│        Relay Entity        │

│                            ├───────────

│                            │

│                      L2RP  │

──────────┤   NTP               Entity │

│  Entity   ┌───┬────────────┤

│           │   │            │

│           │   │    RLP     │

│           │   │   Entity   │

├───────────┤

Figure 3

The 3 sub-functions are:

-	A Non Transparent Protocol Entity

-	A L2R Protocol Entity

-	A Relay Entity

The NTP-entity interfaces the L2R to the IFP-entity and provides an interface to the particular NTP.

The L2RP-entity interfaces the L2R to the RLP-entity and provides an interface to the appropriate L2R protocol.

The Relay-entity provides the mapping between the NTP-entity and the L2R-entity.

It should be noted that the inter-layer interfaces within the MS and the IWF and within the L2R will not be specified by GSM, any description given is for explanatory purposes only and is not intended to indicate a method of implementation. Therefore, the specification of the L2R is in terms of the peer-peer protocols. Generally, the non transparent and interface protocols will be specified elsewhere, e.g. CCITT Recommendation X.25 Layer 2 and 1. Thus the main specification for the L2R will consist of the L2R peer-peer protocols.

.c.6.2	Radio Link Services Used

The L2R function uses services defined in GSM Specification 04.22 (Radio Link Protocol).

.c.6.3	Flow Control - General Description

A flow control active condition can take place under a number of circumstances.

-	End to end flow control (DTE to DTE matter)

-	Backpressure (buffers filling)

-	Receive not ready (RLP condition)

It is possible that there will be an interaction between flow control active and inactive conditions in each circumstances.

.c.6.3.1	End to End Flow Control

A DTE may wish to send a flow control active condition to another DTE.

Provisions exists in the L2R entity to transfer a flow control active condition (sent by its associated DTE) to the other L2R entity as soon as possible. This mechanism in the L2R entities allows such a flow control condition to be put ahead of any queuing which exists in the L2R entities.

Such a mechanism avoids build up of data in buffers which can be undesirable.

The L2R entity, receiving a flow control active condition from its associated DTE, stops sending data to that associated DTE immediately.

.c.6.3.2	Back Pressure

The L2R and RLP entities have buffers which may become full to a predetermined threshold for a number of reasons, e.g.severe radio fading, failure or slowness of DTE to react to end to end flow control, certain RNR conditions. When this predetermined threshold is reached, a flow control active condition is sent to the associated DTE which is then prevented from sending any data, subsequently, the flow control inactive condition is sent to the associated DTE when the L2R or RLP entities have indicated that there is sufficient free capacity in their buffers for data flow from the associated DTE to proceed.

The corresponding peer-layer procedure to assess the respective buffer conditions are a layer management matter and are not dealt with here. It is also considered an implementation matter to ensure that such procedure do not result in loss of data or considerable reduction in throughput.

.c.6.3.3	Receive not Ready

When the RNR condition arises, an RLP indication is sent to the other RLP entity which in turn shall send a flow control active condition to its associated L2R entity. That L2R entity shall then send a flow control active condition to its associated DTE.

An RNR condition may result in the Execution of "back pressure" as mentioned under 6.3.2.

.c.7	STRUCTURE OF THE GSM 07-SERIES OF SPECIFICATIONS

The structure of the Specifications is as follows:

07.01	General on Terminal Adaptation Functions for Mobile Stations

07.02	Terminal Adaptation Functions for Services using Asynchronous Bearer Capabilities

This Specification defines the interfaces and terminal adaption functions integral to a MT which enable the attachment of Asynchronous Terminals to a MT.

07.03	Terminal Adaptation Functions for Services using Synchronous Bearer Capabilities

This Specification defines the interfaces and terminal adaptation functions integral to a MT which enable the attachment of Synchronous Terminals to a MT.

.c.8	FUNCTIONS COMMON TO ALL INTERFACES

.c.8.1	Synchronization of the Traffic Channel

As long as there is no connection between the traffic channel and the interface to the TE this interface must be terminated in the appropriate way.

Prior to exposing the traffic channel of a GSM PLMN connection to transmission of user data, the controlling entities of the connection have to assure of the availability of the traffic channel. This is done by the so called synchronization process

-	starting on the indication of "physical connection established" resulting from the PLMN inherent outband signalling procedure. This indication is given on reception of the message CONNECT in case of MOC, on reception of the message CONNACK in case of MTC and on reception of the message MODIFY COMPLETE in case of in-call modification.

-	ending by indicating the successful execution of this process to the controlling entity, which then takes care of the further use of the inband information (data, status).

With respect to the TAF for the transparent bearer capability support the synchronization procedure is as follows:

-	sending of synchronization pattern 1/OFF (all data bits "1" / all status bits "OFF", all E-bits "1") to the IWF

-	searching for detection of the synchronization pattern 1/OFF received from the IWF. The value of the bits E4-E7 shall not be checked

-	holding CT106, 107 and 109 in the OFF condition

When the 1/OFF from the IWF has been recognized as a steady state note), the TAF continues sending the synchronization pattern 1/OFF to the IWF unless a timer T (=500ms) expires. From this time the filtering process according to section 8.2 applies and the information on the receiving lines CT106 and CT109 from the IWF are directly mapped to the respective sending lines. The condition on CT107 will be changed from "OFF" to "ON", the data bits received from the IWF are mapped to CT104, and CT103 is mapped to the data bits sent towards the IWF.

During the synchronization process described above i.e. while the synchronization pattern is being sent by the MT, the MT will not send the V.110 frame structure to the ISDN terminal. Once the timer T expires the synchronization pattern will continue to be transmitted from the MT to the IWF, however, the MT will start sending the V.110 frames received from the IWF to the ISDN terminal. The MT will start looking for the ISDN frame alignment to be received from the ISDN terminal. On recognizing frame alignment the MT will cease sending its synchronization pattern to the IWF and connect the ISDN terminal through to the IWF.

With respect to the TAF for non-transparent bearer capability support the synchronization procedure is as follows:

-	initiating the RLP link establishment by sending a RLP-SABM across the radio interface.

-	holding CT106, 107, 109 in the OFF condition.

When the RLP link has been established, CT107 will be changed from "OFF" to "ON". From this time the information from/to the RLP, including status changes, will be mapped by the L2R entity applicable to the particular bearer capability. The MT will not send V.110 frame structure to the ISDN terminal and will not start looking for ISDN frame alignment to be received from the ISDN terminal unless the RLP link has been established. On recognizing frame alignment the information from/to the RLP will be mapped by the L2R entity.

It should be noted that in a GSM PLMN V.-series and X.-series interfaces are only supported in full duplex mode. Thus the call control phase can be mapped almost completely to the signalling procedure (the S-bits during the call control phase are irrelevant). However, the "ready for data" condition (i.e. CT 106/109, in the case of V.-series interface, and I-circuit, in the case of X.-series interface) is derived directly from the traffic channel (see also filtering of channel information).

.c.8.2	Filtering of Channel Control Information

.c.8.2.1	General

The DTEs taken into account for the PLMN at the MS side conform to CCITT's DTE/DCE interface specifications, which assume basically an error-free environment, i.e.

-	limited distance, point-to-point local interconnection of the interface circuits for data and status

-	steady state signalling.

The envisaged use of these DTEs in the PLMN environment leads to the exposure of these "interconnections" to the PLMN radio channel. To assure proper operation even under these conditions appropriate measures have to be taken. In the non transparent case the RLP satisfies the requirement for both data and status lines.

In the transparent case the

-	data line aspects have to be dealt with end-to-end between the users, while

-	status line aspects are of concern to the network, which are dealt with in the following.

.c.8.2.2	Filtering Process to Be Applied

Filtering of channel control information is only relevant at the MS side in the transparent mode of operation. By applying filtering measures the condition of a DTE/DCE control interchange circuit, for which the DTE constitutes the information sink, will be preserved until another condition is signalled for an "integration time" period by the channel control information (status bits) of the rate adaptation scheme.

The filtering mechanism is understood to reside between the rate adaptation function (information source) and the DTE (information sink). It receives the unfiltered condition of the respective control interchange circuit set according to the actual sequential appearance of the individual associated status bit and forwards the filtered condition to the DTE.

The filtering process starts when the traffic channel synchronisation ends with the expiry of timer T.

V.-series interface

CT 106

In the transparent mode the remote inband control of this circuit is needed to support a modem retrain procedure.

OFF-ON transition at the MS will authorize the DTE to send data; if wrongly set, loss of data may occur.

ON-OFF transition at the MS will cause the DTE to cease transmitting data; set wrongly may impair the performance in connection usage.

CT 109

In the transparent mode the remote inband control of this circuit is needed to

-	trigger the interpretation of received data

-	indicate to the DTE the state of the connection.

OFF-ON transition at the MS will authorize the DTE to rely on the condition of the received data interchange circuit, set wrongly may cause receipt of wrong data, while setting late may cause loss of data.

ON-OFF transition at the MS

-	will cause the DTE to cease receiving data

-	may initiate release of the connection during a data phase by the DTE giving an ON-OFF transition on circuit 108/2.

Setting this condition wrongly may cause loss of data and potentially release the connection.

X.-series interface

I-circuit

The OFF-ON transition of this circuit in connection with the appropriate conditions of the other interchange circuit will indicate the "ready for data" status of the connection. As received data may commence immediately following this status change, the delay in conveying this condition shall be kept as short as possible.

As a clear request/indication will be directly mapped to the PLMN outband signalling the ON-OFF integration time should be rather long.

Filtering mechanism

A filtering mechanism shall be provided by an integration process on the SB and X datastreams carried in the V.110 frame. The integration periods applied are:

V-series	Transition	Integration	Status

period	stream

CT 106	Off-On	1 s	X

CT 106	On-Off	1 s	X

CT 109	Off-On	200 ms	SB

CT 109	On-Off	5 s	SB

X-series	Transition	Integration	Status

period	stream

I-circuit	Off-On	40 ms	SB

I-circuit	On-Off	5 s	SB

The integration process shall ensure that the interchange circuits do not change state in response to spurious transitions of the status bits during the integration period.

The integration process shall operate reliably with error characteristics as specified in GSM 05.05.

.c.8.3	Terminal Compatibility Decision

The establishment of a mobile terminated connection depends on a positive decision on the terminal compatibility. The Mobile Station (MS) contributes to this process by performing (depending on the individual call set-up condition)

-	a compatibility check

-	the selection of the appropriate terminal function, and

-	the indication of compatibility requirements to the PLMN

initiated by a call set-up request from the PLMN. The afore mentioned functions shall be carried out as follows.

.c.8.3.1	Compatibility Check

Annex B of GSM 04.08 applies, particularly paragraphs B.3, B.3.1 and B.3.2. As regards the therein mentioned user-to-user compatibility checking the following applies:

When the calling user requests a service with user-to-user compatibility significance indicated by the presence of HLC and LLC information element in the call set-up request, the MS shall check that the service supported by the called user matches concerning the contents of the HLC/LLC information element. If a mismatch is detected, then the MS shall reject the offered call using the cause No.88 "Incompatible Destination".

.c.8.3.2	Selection of Appropriate Terminal Function

The MS shall select the appropriate terminal functions following a positive result of the compatibility check and/or forwarding the indication of compatibility requirements to the PLMN.

.c.8.3.3	Indication of Compatibility Requirements to the PLMN

.c.8.3.3.1	Indication in case of Mobile Terminating Calls

In support of

-	PSTN originated calls and

-	ISDN originated calls using 3.1 kHz audio Bearer Capability (BC) as well as

-	ISDN originated calls using unrestricted digital Bearer Capability but not specifying all parameters for deducing a Bearer Service

-	mobile specific requirements to be dealt with in the Bearer Capability information element the call confirmed message has been introduced in the call control protocol (GSM 04.08). This also allows for renegotiation of specific parameters at the beginning of the connection set-up process. The specific parameters are:

a)	mobile specific requirements

-	Radio channel requirement

-	Connection element (transparent/non transparent)

-	Structure 1)

-	User information layer 2 protocol 1)

-	Intermediate rate 2)

-	Modem Type 1)

b)	requirements with effects at the partner terminal

-	Number of data bits

-	Number of stop bits

-	Parity.

The value for the radio channel requirement in the set-up message is an arbitrary one. The MS requests the value in the call confirmed message. If the MS indicates the support of "dual" (HR and FR channels) the final decision, which radio channel is chosen, is done by the network in the assignment command message.

If the network proposes optional support of both transparent and non transparent connection elements but does not indicate a user information layer 2 protocol, the MS shall set the appropriate value, if choosing non transparent in the call confirmed message and out-band flow control is not requested.

Additionally the values of the parameters structure, modem type and intermediate rate have to be set in conformance with the values of the parameters radio channel requirements, negotiation of intermediate rate requested and connection element.

Section 1.1.2 and table II.1 in the annex II describe the negotiation procedure. Annex II table II.4 describes the selection of the modem type and the dependence on the value of the parameter connection element. Annex II table II.4 describes the selection of the intermediate rate and user rate and their dependence upon the value of the NIRR parameter and the equipments capabilities.

The following MTC cases can be deduced from the individual call set-up request conditions

a) If the set-up does not contain a BC information element, the MS in the call confirmed message shall include any BC information (single or multiple BC-IE). In case of multiple BC-IEs one BC-IE must indicate the information transfer capability "speech".

b) If the set-up message contains a single BC-IE, the MS in the call confirm message shall use either a single BC-IE, if it wants to negotiate mobile specific parameter values, or no BC-IE, if it agrees with the requested ones.

c) If the set-up contains a multiple BC-IE, the MS in the call confirmed message shall use either a multiple BC-IE, if it wants to negotiate mobile specific parameter values, or no BC-IE, if it agrees with the requested ones. Alternatively a single BC-IE containing fax group 3 only shall be used if a multiple BC-IE requesting speech alternate fax group 3 is received and the MS is not able to support the speech capability. Annex II, table II.7, describes the negotiation rules.

If the BC-IE contains 3.1 kHz ex PLMN, the MS is allowed to negotiate all mobile specific parameter values listed above. If the BC-IE contains facsimile group 3, the MS is allowed to negotiate the connection element (transparent/non transparent) only. In any case, if the set-up message requests a "single service", the MS must not answer in the call confirmed message requesting a "dual service" and vice versa.

However, for dual services with repeat indicator set to circular (alternate) the MS may change the sequence of dual BC-IEs within the call confirmed message (preceded by the same value of the repeat indicator), if it wants to start with a different Bearer Capability than proposed by the network as the initial one.

.c.8.3.3.2	Indication in case of Mobile originating calls

In support of mobile originating calls the values of BC-IE parameters are requested in the set-up message from the MS. If the MS indicates the support of both transparent and non transparent connection elements the network shall return its choice in the call proceeding message. The MS is not allowed to indicate support of both transparent and non transparent, if the MS also requests out-band flow control, i.e. it does not indicate a layer 2 protocol.

Additionally the value of the parameter modem type has to be set depending on the value of the parameter connection element as described in Annex II, table II.4a.

The set-up message contains a single or multiple BC-IE. In case of multiple BC-IEs one BC-IE must indicate the information transfer capability "speech".

If the set-up message requests a "single service", the network must not answer in the call proceeding message requesting a "dual service" and vice versa. Alternatively the network shall answer with a single BC-IE containing fax group 3 if a multiple BC-IE requesting speech alternate fax group 3 is received but the network doesn't allow the use of this alternate service. Annex II, table II.7, describes the negotiation rules. If the MS requests a "dual service" the network is not allowed to change the sequence of the service.

If the set-up message is indicates that negotiation of intermediate rate is requested then the network shall behave as described in Annex II, table II.4b.

If no BC-IE parameter needs negotiation it is up to the network if it sends a CALL PROC message (with or without a BC-IE) towards the MS or not.

.c.8.4	Test Loops

In principle, both V.-series and X.-series interfaces allow for an activation of local or remote test loops by the terminal (ref. CCITT V.54/X.150). A comprehensive solution of such test loops in a GSM system has to consider the special conditions of the interface between the terminal (part of the MS) and the transmission equipment (part of the modem pool of a particular IWF within the MSC). In addition, the impact of the radiolink is to be taken into account with respect to the test objectives. Due to those special conditions a GSM system is not capable to support remote test loops. It is an implementation choice to what extent the activation of local test loops by the terminal is supported in the MT.

.c.8.5	Alternate speech/data and speech/facsimile group 3

These alternate services may be initiated by either V.25 bis or manual procedures. In the former case, standard call establishment procedures will apply. In the latter case, CT106, CT107, CT108.2 and CT109 are in the OFF condition.

Selection of the data phase (from the speech phase) will be by manual intervention via the MS causing ICM by means of CT108.2 going to ON condition. In case of dual data services (alternate speech/data or speech followed by data) the "Reverse call setup direction" information element of the modify message (determined by MMI) together with information about the initial call setup direction may be used to control the IWF modem (working as calling or answering modem). In case of alternate speech/facsimile refer to GSM 03.45. The ensuing data phase shall follow all the operational procedures as described in 07-series.

Selection of the speech phase (from the data phase) will be by manual intervention via the MS causing ICM (phone off-hook condition at the MT and data call end condition at the TE).

During the ensuing speech phases, CT107, CT106 and CT109 will be maintained in the OFF condition.

Subsequent re-selection of the data phase will be by manual intervention via the MS causing CT108.2 going to ON condition initiating ICM. At this point, re-synchronisation will take place as described in section 8.1 above.

.c.ANNEX 1 (Informative) List of Bearer Capability Elements

This annex lists the GSM Bearer Capability Elements which need to be provided on the Dm channel to support Terminal adaptation function to Interworking control procedures.

Elements and their Values:

Information Transfer Capability:

This element is relevant between the IWF and the fixed network

Values:	- Speech

- Unrestricted Digital

- Group 3 Facsimile 1)

- 3.1 kHz Ex PLMN 2)

1)	used for facsimile transmission, unrestricted digital between MT and IWF and 3.1 kHz audio from IWF towards the fixed network.

2)	unrestricted digital between MT and IWF and 3.1 kHz audio from IWF towards the fixed network.

Transfer Mode:

This element is relevant between MT and IWF

Values:	- Circuit

- Packet

Structure:

This element is relevant between MT and IWF.

Values:	- Service Data Unit Integrity 3)

- Unstructured 4)

3)	applicable for connection element "non transparent"

4)	applicable for connection element "transparent"

Configuration:

This element is relevant for a PLMN connection.

Values:	- Point to point

Establishment:

This element is relevant for a PLMN connection.

Values:	- Demand

Sync/Async:

This element is relevant between TE/TA and	MT and	between IWF and the fixed network.

Values:	- Synchronous

- Asynchronous

Negotiation:

This element is relevant between	MT and IWF.

Values:	- In band negotiation not possible

User Rate:

This element is relevant between TE/TA and MT and between IWF and the fixed network.

Values:	- 0.3 kbit/s

- 1.2 kbit/s

- 1200/75 bit/s

- 2.4 kbit/s

- 4.8 kbit/s

- 9.6 kbit/s

Intermediate Rate:

This element is relevant between MT and BSS and BSS and IWF

Values:

- 8 kbit/s

- 16 kbit/s

Network Independent Clock on Tx:

This element is relevant between TE/TA and	MT in the transmit direction.

Values:	- Not required

- Required

Network Independent Clock on Rx:

This element is relevant between TE/TA and	MT in the receive direction.

Values:	- Not accepted

- accepted

Number of Stop Bits:

This element is relevant between the TE/TA and MT and between IWF and fixed network in case of asynchronous transmission.

Values:	- 1 bit

- 2 bit

Number of Data Bits Excluding Parity If Present:

This element is relevant between TE/TA and MT and between IWF and the fixed network in case of a character oriented mode of transmission.

Values:	- 7 bit

- 8 bit

Parity Information:

This element is relevant between TE/TA and MT and between IWF and the fixed network for a character oriented mode of transmission.

Values:	- Odd

- Even

- None

- Forced to 0

- Forced to 1

Duplex Mode:

This element is relevant between MT and IWF.

Values:	- Full Duplex

Modem Type:

This element is relevant between the IWF and the fixed network in case of 3.1 kHz audio ex-PLMN information transfer capability.

Values:	- V.21

- V.22

- V.22 bis

- V.23

- V.26 ter

- V.32

- autobauding type 1

- none

Radio Channel Requirement:

This element is relevant between MT and BSS

Values:	- Full Rate Channel (Bm)

- Half Rate Channel (Lm)

- Dual Rate/Half Rate preferred

- Dual Rate/Full Rate preferred

Connection Element:

This element is relevant between MT and IWF

Values:	- Transparent

- Non Transparent

- both, Transparent preferred

- both, Non transparent preferred

User Information Layer 2 Protocol:

This element is relevant between TE/TA and MT and between IWF and the fixed network.

Values:	- ISO 6429

- X.25

- Character oriented Protocol with no Flow Control mechanism

Signalling Access Protocol:

This element is relevant between TE/TA and MT.

Values:	- I.440/450

- X.21

- X.28, dedicated PAD, individual NUI

- X.28, dedicated PAD, universal NUI

- X.28, non dedicated PAD

- X.32

Rate Adaptation:

This element is relevant between IWF and the fixed network.

Values:	- V.110/X.30

- X.31 flagstuffing

- no rate adaptation

Coding Standard:

This element refers to the structure of the BC-IE defined in GSM 04.08.

Values:	- GSM

User Information Layer 1 Protocol:

This element characterize the layer 1 protocol to be used between MT and BSS (Um interface) according to GSM 05.01.

Values:	- default

Negotiation of Intermediate Rate requested:

This element is relevant between MT and BSS and BSS and IWF.

Values:	- no meaning associated

- 6kbit/s radio interface is requested for a full rate channel with a user rate up to and including 4.8kbit/s, non transparent service

.c.ANNEX 2 (Normative)

Setting of Bearer Capability, Low Layer Compatibility and High Layer Compatibility Information Element for GSM Bearer Services and GSM TeleServices

.c.0	Scope

This annex describes the relationship between the various parameters of the GSM Bearer Capability Information Element (BC-IE), their validity and the possible settings with reference to each GSM Bearer service/Teleservice defined in GSM 02.02 and GSM 02.03 as well as the various occurrences during the connection control (section 1). Furthermore, the contents of the Low Layer (LLC) and the High Layer (HLC) Compatibility Information Elements are described (section 2).

.c.1	Bearer Capability Information Element

.c.1.1	Introduction

.c.1.1.1	General Consideration

In general, the purpose of the bearer capability information element (BC-IE) is to request a particular bearer service to be provided by the network. This indication is carried by certain connection control messages which for the subject matter of this document may be categorized into those messages

-	related to the call set-up phase and those

-	used during the established connection.

During the call set-up phase the GSM BC-IE (single or multiple) is included in

-	the SETUP message generated by the requesting entity (either MS or MSC) to establish a mobile-originated or mobile-terminated call, respectively, and in

-	the CALL CONFIRMED or CALL PROCEEDING messages, respectively, generated by the responding entity (either MS or MSC) in order to negotiate certain parameter values. If no BC-IE is contained in the SETUP message (PSTN-originated call with single-numbering scheme) the CALL CONFIRMED message indicates the complete applicable BC-IE. In this case the value "unrestricted digital" for the information transfer capability must not be used.

During the established connection the GSM BC-IE is included in the MODIFY, MODIFY COMPLETE, and MODIFY REJECT messages in order to change the service (bearer capability).

The subsequent tables and subsections of section 1 deal with the representation of the individual contents of the GSM BC-IE during the call set-up phase. For the use during the established connection refer to GSM 04.08.

With respect to the individual parameter settings at the MS the following cases may be distinguished (ref. GSM 07.02 and GSM 07.03):

-	Mobile-originated call set up by a MS consisting of a MT with R interface:

The setting results from respective MMI actions and/or MT internal settings.

-	Mobile-originated call set up by a MS consisting of a MT with S interface:

The setting of the GSM BC is derived from the ISDN BC and LLC/HLC elements contained in the ISDN SETUP message received from the terminal. It is complemented by information resulting from respective MMI actions and/or MT internal settings.

-	Mobile-terminated call set up to a MS consisting of a MT with R interface:

The BC related part of the compatibility check is carried out according to the knowledge of the MT concerning its implemented functions (i.e. answering the call). The requested field values of the non-negotiable parameters and the selected field values of the negotiable parameters determine the selection of the terminal function to be used for the intended connection.

-	Mobile-terminated call set up to a MS consisting of a MT with S interface:

The GSM BC received from the MSC is mapped by the MT onto an applicable ISDN BC. In some cases a HLC may be generated, if it is not otherwise available (e.g. for group 3 facsimile). The BC related part of the compatibility check is up to the terminal connected to the S interface of the MT, as is the selection of the terminal function (i.e. answering the call) to be used for the intended connection.

.c.1.1.2	Interpretation of the Diagrams

The purpose of the subsequent diagrams is to achieve unambiguous representation of the individual contents of the GSM BC-IE for the various occurrences during the call set-up phase, covering all bearer services and teleservices according to GSM 02.02 and GSM 02.03.

The basic principle adopted is a graphic scheme, or mask, wherein the ordinate designates the individual parameters of the GSM BC-IE and the abscissa gives the possible field values of these parameters. The abbreviations used in these sections are defined in table II.3. The allowed content of any GSM BC-IE is represented by a number of graphs connecting parameter values (abscissa points) of all parameters (ordinate points). Each graphic scheme is subdivided into two independent parts:

-	"Layer/Protocol related" part and

-	"Radio Channel related" part.

The generation of all GSM BC-IEs in all call set-up messages shall be in accordance with these graphs. Sections 1.2 through 1.14 show individual sets of graphs for each service group (BS/TS) and for each type of applicable Information Transfer Capability.

In addition, the following rules apply:

-	Those parameters which have only one possible field value for all recognized services are shown in table II.3, where they are marked accordingly in the column "common setting of field values". They are not represented in the graphic scheme.

-	Not all parameters of the GSM BC-IE are relevant for each service (BS/TS). This is represented by specific abscissa points with a value of "NA" (Not Applicable) allocated to these parameters. The graphs pass through these points for each such parameter. The actual field value to be used in the GSM BC-IE is marked in the column "default setting of field values (NA)" of table II.3. An abscissa point with a value of "NAV" (Not AVailable) indicates that the entire octet carrying this parameter (ref. table II.2 "General Structure of the GSM BC-Information Element") shall be omitted.

-	There is a particular dependency of the parameters "User Information Layer 2 Protocol (UIL2P)" and "Connection Element (CE)":

-	If the MS sends a GSM BC-IE with a CE value other than "Transparent (T)", the parameter UIL2P is essential. Its field value must be set as indicated in the applicable graph.

-	If the MSC sends a GSM BC-IE in the SETUP message, the parameter UIL2P may also be absent in the case of the CE parameter value being other than "Transparent (T)".

-	Certain parameters of the GSM BC-IE may be negotiated during the connection establishment phase. Table II.1 shows these parameters and the relations of their values in the SETUP message and in the CALL CONFIRMED/CALL PROCEEDING message, respectively, both for the mobile-originated and mobile-terminated case. A parameter may indicate a field value of one of the following types:

-	"requested value" indicating a request which cannot be changed by the responding entity;

-	"offered value" indicating a proposal which may be changed by the responding entity;

-	a particular choice value leaving it up to the responding entity which value ultimately applies;

-	"as requested" indicating that the requested value applies and is confirmed (by returning it);

-	"selected value" indicating that a particular value applies either out of the offered set or as a free choice out of the defined set of values.

.c.Table II.1	BC-Parameters subject to negotiation procedure

Mobile Originated Call:

┌──────────────────────────────────────────────────────────┐

│           Message                                        │

┌──────────────┼───────────────────────────────┬──────────────────────────┤

│ BC-parameter │           SETUP               │         CALL PROC        │

├══════════════╪═══════════════════════════════╪══════════════════════════┤

│    NDB       │ requested value               │  as requested            │

├──────────────┼───────────────────────────────┼──────────────────────────┤

│    NPB       │ requested value               │  as requested            │

├──────────────┼───────────────────────────────┼──────────────────────────┤

│    NSB       │ requested value               │  as requested            │

├──────────────┼───────────────────────────────┼──────────────────────────┤

│    CE        │ requested value (T/NT)        │  as requested            │

│              ├───────────────────────────────┼──────────────────────────┤

│              │ "both" with the preferred     │  selected value          │

│              │ value indicated(e.g. bothNT)  │  (T/NT)                  │

├──────────────┼───────────────────────────────┼──────────────────────────┤

│    RCR       │ requested value (HR/FR)       │  as requested            │

│              ├───────────────────────────────┼──────────────────────────┤

│              │ "dual" with the preferred     │  as requested1)          │

│              │ value indicated(e.g. dualFR)  │                          │

├──────────────┼───────────────────────────────┼──────────────────────────┤

│   UIL2P      │ requested value3) or NAV2)    │  as requested or NAV5)   │

└──────────────┴───────────────────────────────┴──────────────────────────┘

Mobile Terminated Call:

┌──────────────────────────────────────────────────────────────┐

│                            Message                           │

┌──────────────┼───────────────────────────────┬──────────────────────────────┤

│ BC-parameter │           SETUP               │         CALL CONF            │

├══════════════╪═══════════════════════════════╪══════════════════════════════┤

│    NDB       │  offered value                │  selected value (free choice)│

├──────────────┼───────────────────────────────┼──────────────────────────────┤

│    NPB       │  offered value                │  selected value (free choice)│

├──────────────┼───────────────────────────────┼──────────────────────────────┤

│    NSB       │  offered value                │  selected value (free choice)│

├──────────────┼───────────────────────────────┼──────────────────────────────┤

│    CE        │  requested value (T/NT)       │  as requested or             │

│              │                               │  selected value (T/NT)       │

│              │                               │  (free choice)4)             │

│              ├───────────────────────────────┼──────────────────────────────┤

│              │ "both" with the preferred     │  selected value              │

│              │  value indicated(e.g. bothNT) │  (T/NT)                      │

├──────────────┼───────────────────────────────┼──────────────────────────────┤

│    RCR       │  arbitrary value              │  requested value (FR/HR) or  │

│              │ (HR/FR/"dual" with the        │  "dual" with the preferred   │

│              │ preferred value indicated     │  value indicated             │

│              │ (e.g. dualFR) )               │  (e.g. dualHR)       1)      │

├──────────────┼───────────────────────────────┼──────────────────────────────┤

│   UIL2P      │ offered value 3) or NAV5)     │  selected or NAV2)           │

└──────────────┴───────────────────────────────┴──────────────────────────────┘

1)	the final decision, which radio channel is chosen, is done by the ASSIGNMENT COMMAND message. This message doesn't contain a BC-IE, but other parameters which define the radio channel.

2)	for CE:T only or out-band flow control requested by the MS

3)	not for CE:T

4)	when the SETUP message contains no BC-IE (single numbering scheme)

5)	"NAV" shall not be interpreted as an out-band flow control request by the MS

.c.Table II.2	General Structure of the BC-Information Element

OCTET	INFORMATION ELEMENT FIELD

3	Radio channel requirements

Coding standard

Transfer mode

Information Transfer Capability

4	Structure	2)

Duplex mode

Configuration

Establishment

Negotiation of Intermediate Rate Requested

5	Rate adaptation	2)

Signalling access protocol

6	User information layer 1 protocol	2)

Synchronous / asynchronous

6a	Number of stop bits	2)

Negotiation

Number of data bits

User rate

6b	Intermediate rate	2)

NIC on transmission

NIC on reception

Parity information

6c	Connection element	2)

Modem type

7	User information layer 2 protocol	1)	2)

1)	octets optional

2)	octets only available if the parameter "Information Transfer Capability" does not indicate "Speech".

.c.Table II.3	Selection of flow control method (for CE:NT with SA:A only)

flow control method

information element	in-band	out-band3)	none

number of data bits	7 or 8	7 or 8	7 or 8

user information layer 2 protocol	ISO 64291)	NAV	 COPnoFlCt2)

1)	ISO6429 stands for "ISO 6429, codeset 0, DC1/DC3" and is applicable for 7 and 8 bit codes.

2)	COPnoFlCt stands for a character oriented protocol with no flow control mechanism (no reserved characters for flow control)

3)	"out-band" flow control requires V.42 in case of PSTN or V.110 in case of ISDN.

If the V.110 flow control mechanism is not supported, where required, the call pending shall be terminated.

If the V.42 functionality is not supported by the modem in the IWF or in the fixed network, the call will be supported with a fallback to the non-V.42 mode. In this case the IWF will release the call if due to temporary throughput problems on the radio interface or initiation of flow control by the MS and the inability to flow control the fixed network modem an overflow of the L2R buffers (16-32kbit) occurs.

Note that a phase 1 network may release the call, if the V.42 functionality is not provided by the IWF or the fixed network modem. As V.42 does not apply to V.21 and V.23 modems, outband flow control can not be supported for these modem types.

.c.Table II.4a	Modem Type subject to negotiation procedure

Mobile Originated Call:

┌──────────────────────────────────────────────────────────┐

│                      BC-parameter MT                     │ ┌──────────────┼───────────────────────────────┬──────────────────────────┤

│ BC-parameter │ Message SETUP                 │ Message CALL PROC        │

│     CE       │                               │                          │

╞══════════════╪═══════════════════════════════╪══════════════════════════┤

│    T         │ V-series                      │  V-series                │

├──────────────┼───────────────────────────────┼──────────────────────────┤

│    NT        │ V-series                      │  V-series                │

├──────────────┼───────────────────────────────┼──────────────────────────┤

│              │ autobauding type 1            │  autobauding type 1 or   │

│              │                               │  V-series 1)             │

├──────────────┼───────────────────────────────┼──────────────────────────┤

│    bothT or  │ V-series                      │  V-series                │

│    bothNT    ├───────────────────────────────┼──────────────────────────┤

│              │ autobauding type 1            │  autobauding type 1 or   │

│              │                               │  V-series 1)2)           │

└──────────────┴───────────────────────────────┴──────────────────────────┘

Mobile Terminated Call:

┌──────────────────────────────────────────────────────────┐

│                      BC-parameter MT                     │ ┌──────────────┼───────────────────────────────┬──────────────────────────┤

│ BC-parameter │ Message SETUP                 │ Message CALL CONF        │

│     CE       │                               │                          │

╞══════════════╪═══════════════════════════════╪══════════════════════════┤

│    T         │ V-series                      │  V-series                │

├──────────────┼───────────────────────────────┼──────────────────────────┤

│    NT        │ V-series                      │  V-series or autobauding │

│              │                               │  type 1 3)               │

├──────────────┼───────────────────────────────┼──────────────────────────┤

│              │ autobauding type 1            │  autobauding type 1 or   │

│              │                               │  V-series 4)             │

├──────────────┼───────────────────────────────┼──────────────────────────┤

│    bothT or  │ V-series                      │  V-series                │

│    bothNT    ├───────────────────────────────┼──────────────────────────┤

│              │ autobauding type 1            │  autobauding type 1 or   │

│              │                               │  V-series 4)5)           │

└──────────────┴───────────────────────────────┴──────────────────────────┘

1)	No autobauding capability in the IWF:MSC

2)	CE:T selected by IWF/MSC

3)	Free choice if the SETUP contains no BC-IE (single numbering scheme)

If the IWF/MSC has no autobauding capability, a V-series modem type is used

4)	When the MS does not allow the use of autobauding capability

5)	CE:T selected by the MS

.c.Table II.4b	Intermediate Rate negotiation procedure

If the user rate is 9.6 kbit/s the intermediate rate negotiation procedure is not applicable and NIRR shall be set to "no meaning".

Recipient of SETUP supports full rate, non transparent, 6kbit/s radio interface rate and the user rate is up to/equal 4.8kbit/s:

┌──────────────────────────────────────────────────────────┐

│                      BC-parameter MT                     │ ┌──────────────┼───────────────────────────────┬──────────────────────────┤

│ BC-parameter │ Message SETUP                 │ Message CALL CONF        │

│              │                               │      or CALL PROC1)      │

╞══════════════╪═══════════════════════════════╪══════════════════════════┤

│  NIRR        │ 6kbit/s                       │ 6kbit/s                  │

├──────────────┼───────────────────────────────┼──────────────────────────┤

│  IR          │ 16kbit/s                      │ 8kbit/s                  │

├──────────────┼───────────────────────────────┼──────────────────────────┤

│  User Rate   │ up to/equal 4.8kbit/s         │ as requested             │

└──────────────┴───────────────────────────────┴──────────────────────────┘

NOTE:	In case of a Mobile Terminated Call, if the SETUP message does not contain a BC-IE, the MS shall behave as if NIRR set to "no meaning".

In case of a MOC or a MTC where no BC-IE is included in the CALL PROCEEDING or CALL CONFIRMED message, respectively, the MS or the network shall behave as if the NIRR was set to "No meaning".

Recipient of SETUP does support full rate, non transparent, but not in connection with 6kbit/s radio interface rate:

┌──────────────────────────────────────────────────────────┐

│                      BC-parameter MT                     │ ┌──────────────┼───────────────────────────────┬──────────────────────────┤

│ BC-parameter │ Message SETUP                 │ Message CALL CONF        │

│              │                               │      or CALL PROC1)      │

╞══════════════╪═══════════════════════════════╪══════════════════════════┤

│  NIRR        │ 6kbit/s                       │ No Meaning               │

├──────────────┼───────────────────────────────┼──────────────────────────┤

│  IR          │ 16kbit/s                      │ 16kbit/s                 │

├──────────────┼───────────────────────────────┼──────────────────────────┤

│  User Rate   │ up to/equal 4.8kbit/s         │ as requested             │

└──────────────┴───────────────────────────────┴──────────────────────────┘

NOTE:	If no other parameter needs negotiation, the CALL CONF/PROC message need not contain any BC-IE.

In case of a MOC or a MTC where no BC-IE is included in the CALL PROCEEDING or CALL CONFIRMED message, respectively, the MS or the network shall behave as if the NIRR was set to "No meaning".

.c.Table II.5	BC parameter setting (part 1)

────────────────────────────────────┐

common setting of field values      ──────────────┐  │

Abbreviations for Parameters and Values:                         ──────────────────────────────┐  │  │

default setting of field values (NA) ───────┐  │  │  │

─────────────────────────────────────────────────────────────────────────────────────────┐  v  │  v  │

ITC...Information Transfer Capability: - Speech                                          │     │     │

- UDI..Unrestricted Digital                       │     │     │

- FAX3..Group 3 Facsimile                         │     │     │

- 3.1 kHz..3.1 kHz Ex PLMN                        │     │     │

│     │     │

TM....Transfer Mode:                   - ci..Circuit                                     │  X  │  X  │

│     │     │

│     │     │

S.....Structure:                       - SDU..Service Data Unit Integrity                │     │     │

- Unstructured                                    │  X  │     │

│     │     │

C.....Configuration:                   - pp..Point to point                              │  X  │  X  │

│     │     │

E.....Establishment:                   - de..Demand                                      │  X  │  X  │

│     │     │

SA....Sync/Async:                      - S..Synchronous                                  │     │     │

- A..Asynchronous                                 │     │     │

│     │     │

N.....Negotiation                     - ibn..in band negotiation not possible            │  X  │  X  │

│     │     │

UR....User Rate:                       - 0.3..0.3 kbit/s                                 │     │     │

- 1.2..1.2 kbit/s                                 │     │     │

- 1.2/0.075..1200/75 bit/s                        │     │     │

- 2.4..2.4 kbit/s                                 │     │     │

- 4.8..4.8 kbit/s                                 │     │     │

- 9.6..9.6 kbit/s                                 │     │     │

│     │     │

IR....Intermediate Rate:               - 4.. 4  kbit/s                                   │     │     │

- 8.. 8  kbit/s                                   │     │     │

- 16.. 16 kbit/s                                  │     │     │

- not_used..not used                              │  X  │     │

│     │     │

NICT..Network Independent Clock on Tx: - not_required.. Not required                     │  X  │  X  │

- required                                        │     │     │

│     │     │

NICR..Network Independent Clock on Rx: - not_accepted..not accepted                      │  X  │  X  │

- accepted                                        │     │     │

│     │     │

NSB...Number of Stop Bits:             - 1..1 bit                                        │  X  │     │

- 2..2 bit                                        │     │     │

│     │     │

NDB...Number of Data Bits Excluding                                                      │     │     │

Parity If Present:               - 7.. 7 bit                                       │     │     │

- 8.. 8 bit                                       │  X  │     │

│     │     │

│     │     │

NPB...Parity Information:              - Odd                                             │     │     │

- Even                                            │     │     │

- None                                            │  X  │     │

- 0.. Forced to 0                                 │     │     │

- 1.. Forced to 1                                 │     │     │

│     │     │

│     │     │

UIL1P.User Information Layer 1                                                           │     │     │

Protocol                         - def..default layer 1 protocol                   │  X  │  X  │

│     │     │

│     │     │

.c.Table II.5	BC parameter setting (part 2)

────────────────────────────────────┐

common setting of field values      ──────────────┐  │

Abbreviations for Parameters and Values                          ──────────────────────────────┐  │  │

default setting of field values (NA) ───────┐  │  │  │

─────────────────────────────────────────────────────────────────────────────────────────┐  v  │  v  │

│     │     │

DM....Duplex Mode:                     -                                                 │     │     │

- fd.. Full Duplex                                │  X  │  X  │

│     │     │

MT....Modem Type:                      - V.21                                            │     │     │

- V.22                                            │     │     │

- V.22 bis                                        │     │     │

- V.23                                            │     │     │

- V.26 ter                                        │     │     │

- V.32                                            │     │     │

- auto1.. autobauding type 1                      │     │     │

- none                                            │  X  │     │

│     │     │

RCR...Radio Channel Requirement:       - FR.. Full Rate Channel (Bm                      │     │     │

- HR..Half Rate Channel (Lm)                      │     │     │

- dualHR..Dual Rate/Half Rate preferred           │     │     │

- dualFR..Dual Rate/Full Rate preferred           │     │     │

│     │     │

CE....Connection Element:              - T.. Transparent                                 │     │     │

- NT.. Non Transparent                            │     │     │

- bothT both transparent preferred                │     │     │

- bothNT both non Transparent preferred           │     │     │

│     │     │

UIL2P.User Information Layer 2                                                           │     │     │

Protocol:                        - ISO6429..ISO6429,codeset 0,DC1/DC3              │     │     │

- X.25                                            │     │     │

- COPnoFlCt..Character oriented protocol with     │     │     │

no flow control mechanism            │     │     │

│     │     │

│     │     │

SAP...Signalling Access Protocol:      - I.440.. I.440/450                               │  X  │     │

- X.21                                            │     │     │

- X.28deIN.. X.28, dedicated PAD,individual NUI   │     │     │

- X.28deUN.. X.28, dedicated PAD,universal NUI    │     │     │

- X.28nond.. X.28, non dedicated PAD              │     │     │

- X.32                                            │     │     │

│     │     │

RA....Rate Adaptation:                 - V.110.. V.110/X.30                              │     │     │

- X.31Flag.. X.31 flagstuffing                    │     │     │

- NO.. no rate adaptation                         │  X  │     │

│     │     │

CS....Coding Standard:                 - GSM                                             │  X  │  X  │

│     │     │

NIRR..Negotiation of Intermediate                                                        │     │     │

Rate Requested:                   NM..No Meaning associated with this value        │  X  │     │

6kb/s..6kbit/s radio interface rate requested    │     │     │

.c.Table II.6	Channel combinations

Single Bearer and Teleservices

MS indication	Network selection

BC	CT

FR	FR

HR	HR

dual FR	FR or HR

dual HR	HR or FR

Alternate services

MS indication		Network selection

BC(1)	BC(2)	CT(1)	CT(2)	or	CT(1)	CT(2)

FR	FR	FR	FR

HR	HR	HR	HR

FR	dual FR	FR	FR

FR	dual HR	FR	FR

HR	dual FR	HR	HR

HR	dual HR	HR	HR

dual FR	dual FR	FR	FR	or	HR	HR

dual FR	dual HR	FR	FR	or	HR	HR

dual HR	dual FR	FR	FR	or	HR	HR

dual HR	dual HR	FR	FR	or	HR	HR

dual FR	FR	FR	FR

dual FR	HR	HR	HR

dual HR	FR	FR	FR

dual HR	HR	HR	HR

Followed-by services

MS indication		Network selection

BC(1)	BC(2)	CT(1)	CT(2)	or	CT(1)	CT(2)	or	CT(1)	CT(2)

FR	FR	FR	FR

FR	dual FR	FR	FR	or	FR	HR

FR	dual HR	FR	FR	or	FR	HR

HR	dual FR	HR	HR

HR	dual HR	HR	HR

HR	HR	HR	HR

dual FR	dual FR	FR	FR	or	HR	HR	or	FR	HR

dual FR	dual HR	FR	FR	or	HR	HR	or	FR	HR

dual HR	dual FR	FR	FR	or	HR	HR	or	FR	HR

dual HR	dual HR	FR	FR	or	HR	HR	or	FR	HR

dual FR	FR	FR	FR

dual FR	HR	FR	HR	or	HR	HR

dual HR	FR	FR	FR

dual HR	HR	HR	HR	or	FR	HR

CT...Channel Type

.c.Table II.7	TS61/TS62 Negotiation rules

Mobile Originating Call

Subscription

TS61

TS62

SETUP	CALL PROCEED

TS61 s/f	TS61 s/f or TS62

TS61 f/s	TS61 f/s or TS62

TS62	TS62

TS61 s/f	TS62

TS61 f/s	TS62

TS62	TS62

Mobile Terminating Call

Subscription

TS61

TS62

SETUP	CALL CONFIRMED

TS61 s/f	TS61 s/f or TS61 f/s or TS62

TS61 f/s	TS61 s/f or TS61 f/s or TS62

TS62	TS62

no BC	TS61 s/f or TS61 f/s or TS62

TS62	TS62

no BC	TS62 (Note1)

s/f = speech then fax

f/s = fax then speech

Note 1: TS61 is also accepted if the VMSC supports TS61 and does not perform subscription checking on a CALL CONFIRMED message (see GSM TS 02.01 and GSM TS 09.07)

.c.1.2	Bearer Service 21 ... 26, Data Circuit Duplex Asynchronous

.c.1.2.1	Unrestricted digital information transfer capability

UDI      3.1 kHz      FAX3       speech

ITC------│----------------------------------------------

┌─────┘

│

├─Layer/protocol related:─┬──────────────────────────────────────────────┐3)

│                         │                                              │

│              NA       I.440        X.28deIN        X.28deUN         X.28nond       X.32          X.21    res

│    SAP------------------│----------------------------------------------│-----------------------------------

│                         └─────┐┌───────────────────────────────────────┘

│                               ││

│              NA             V.110              X.31Flag               No

│    RA-------------------------│----------------------------------------------------------------------------

│                               │

│                               A                                S                          NA

│    SA-------------------------│----------------------------------------------------------------------------

│                               ├──────────────────┬──────────────────────┬───────────────────┐

│                               NT               bothNT                 bothT                 T        NA

│    CE-------------------------│------------------│----------------------│-------------------│--------------

│                               ├──────────────────┴──────────────────────┘                   │

│                               │                                                             │

│              NA               SDU                                                     unstructured

│    S---------------------------│------------------------------------------------------------│--------------

│                                │                                                            │

│                                ├──────────────────────────┬────────────────────────────────┐│

│                                │                        5)│                              4)││

│                   X.25      ISO6429                    COPnoFlCt                           NAV

│    UIL2P ----------------------│--------------------------│---------------------------------│--------------

│                                └────────────────────┬─────┴──────────────────────┬──────────┘

│                                                     │                            │

│                     NA                              7                            8

│    NDB ---------------------------------------------│----------------------------│-------------------------

│                               ┌────────────┬────────┴───┬────────────┬──────────┐│

│                               │            │            │            │          ││

│                     NA       odd          even          0            1          none

│    NPB -----------------------│------------│------------│------------│-----------│-------------------------

│                               └────────────┴────────┬───┴────────────┴───────────┤

│                                                     │                            │

│                     NA                              1                            2

│    NSB ----------------------------------------------------------------------------------------------------

│

│

└─── Radio channel related: ────┬──────────────────┬──────────────────────┬──────────────────┐

│                  │                      │                  │

HR               dualHR                 dualFR               FR

RCR ------------------------│------------------│----------------------│------------------│---------------

│                  │                      │                  │

│                  └──────────┬───────────┴──────────────────┘

│                             │

│+----------------------------┤

│|               2)           │

8                             16              not used        NA

IR-------------------------│-----------------------------│----------------------------------------------

┌────────┴─┬──────────┬──────────┬─────│─────────────────┐

│+---------│+---------│+---------│+----└────┐-----------+│

│|         │|         │|         │|1)       │           |│

0.3        1.2        2.4        4.8         9.6      1.2/0.075 6)                NA

UR----------------│----------│----------│----------│-----------│-----------│----------------------------

│          │          │          │           │           │

┌────────┴──────────┴──────────┴──────────┴───────────┴───────────┘

│

none     V.21       V.22      V.22bis     V.26ter     V.32       V.23            auto1       NA

MT------------------------------------------------------------------------------------------------------

1) for CE:NT or "both"

2) for CE:T only or CE:NT and NIRR:6kb/s (not for the SETUP message)

3) for MOC only

4) for MTC in the SETUP message or MOC/MTC with "out-band" flow control requested

5) for MOC/MTC with no flow control requested

6) MOC only, 75 bit/s in the uplink, 1200 bit/s in the downlink direction

.c.1.2.2	3.1 kHz audio ex-PLMN information transfer capability

UDI      3.1 kHz      FAX3          speech

ITC-----------------│------------------------------------

│

┌────────────────┘

│

├─Layer/protocol related:─┬─────────────────────────────────────────────┐3)

│                         │                                             │

│              NA       I.440        X.28deIN        X.28deUN         X.28nond       X.32          X.21    res

│    SAP------------------│---------------------------------------------│------------------------------------

│                         └─────────────────────────────────────────────┤

│                                                                       │

│              NA             V.110              X.31Flag               No

│    RA-----------------------------------------------------------------│------------------------------------

│                               ┌───────────────────────────────────────┘

│                               A                                S                          NA

│    SA-------------------------│----------------------------------------------------------------------------

│                               ├──────────────────┬──────────────────────┬───────────────────┐

│                               │                  │                      │                   │

│                               NT               bothNT                 bothT                 T      NA

│    CE-------------------------│------------------│----------------------│-------------------│--------------

│                               ├──────────────────┴──────────────────────┘                   │

│                               │                                                             │

│              NA               SDU                                                     unstructured

│    S---------------------------│------------------------------------------------------------│--------------

│                                │                                                            │

│                                ├─────────────────────────┬─────────────────────────────────┐│

│                                │                       5)│                               4)││

│                   X.25      ISO6429                   COPnoFlCt                            NAV

│    UIL2P-----------------------│-------------------------│----------------------------------│--------------

│                                └────────────────────┬────┴───────────────────────┬──────────┘

│                                                     │                            │

│                     NA                              7                            8

│    NDB ---------------------------------------------│----------------------------│-------------------------

│                               ┌────────────┬────────┴───┬────────────┬──────────┐│

│                               │            │            │            │          ││

│                     NA       odd          even          0            1          none

│    NPB -----------------------│------------│------------│------------│-----------│-------------------------

│                               └────────────┴────────┬───┴────────────┴───────────┤

│                                                     │                            │

│                     NA                              1                            2

│    NSB ----------------------------------------------------------------------------------------------------

│

│

└─── Radio channel related: ────┬──────────────────┬──────────────────────┬──────────────────┐

│                  │                      │                  │

HR               dualHR                 dualFR               FR

RCR ------------------------│------------------│----------------------│------------------│----------------

│                  │                      │                  │

│                  └──────────┬───────────┴──────────────────┘

│                             │

│+----------------------------┤

│|               2)           │

8                             16             not_used        NA

IR-------------------------│-----------------------------│----------------------------------------------

┌────────┴─┬──────────┬──────────┬─────│─────────────────┐

│+---------│+---------│+---------│+----└────┐-----------+│

│|         │|         │|         │| 1)      │           |│

0.3        1.2        2.4        4.8         9.6      1.2/0.075 6)                 NA

UR----------------│|---------│|---------│|---------│|----------│|----------│|---------------------------

│+---------│+---------│+---------│+----------│+----------│+--------------+

│          │          │          └──────────┐│           │               | 1)

│          │          ├───────────┐         ││           │               |

none     V.21       V.22      V.22bis     V.26ter     V.32       V.23            auto1       NA

MT------------------------------------------------------------------------------------------------------

1) for CE:NT or "both"

2) for CE:T only or CE:NT and NIRR:6kb/s (not for the SETUP message)

3) for MOC only

4) for MTC in the SETUP message or MOC/MTC with "out-band" flow control requested

(not for V.21 and V.23 modem types)

5) for MOC/MTC with no flow control requested

6) MOC only, 75 bit/s in the uplink, 1200 bit/s in the downlink direction

.c.1.3.	Bearer Service 31 ... 34, Data Circuit Duplex Synchronous

.c.1.3.1	Unrestricted digital information transfer capability

.c.1.3.1.1	Non-X.32 Cases

UDI      3.1 kHz      FAX3       speech

ITC------│----------------------------------------------

┌─────┘

│

├─Layer/protocol related:─┬─────────────────────────────────────────────────────────────────────────┐

│                         │                                                                         │

│              NA       I.440        X.28deIN        X.28deUN         X.28nond       X.32          X.21    res

│    SAP------------------│-------------------------------------------------------------------------│--------

│                         │                                                                         │

│                         └─────┐┌──────────────────────────────────────────────────────────────────┘

│                               ││

│              NA             V.110              X.31Flag               No

│    RA-------------------------│----------------------------------------------------------------------------

│                               │

│                               └────────────────────────────────┐

│                               A                                S                          NA

│    SA----------------------------------------------------------│-------------------------------------------

│                                                                └────────────────────────────┐

│                                                                                             │

│                               NT               bothNT                 bothT                 T        NA

│    CE---------------------------------------------------------------------------------------│--------------

│                                                                                             │

│              NA               SDU                                                     unstructured

│    S----------------------------------------------------------------------------------------│--------------

│                                                                                             │

│                                                                                             │

│                                                                                             │

│                   X.25      ISO6429                      COPnoFlCt                         NAV

│    UIL2P -----------------------------------------------------------------------------------│--------------

│                     ┌───────────────────────────────────────────────────────────────────────┘

│                     NA                              7                            8

│    NDB -------------│--------------------------------------------------------------------------------------

│                     │

│                     │

│                     NA       odd          even          0            1          none

│    NPB -------------│--------------------------------------------------------------------------------------

│                     │

│                     │

│                     NA                              1                            2

│    NSB -----------------------------------------------------------------------------------------------------

│

│

└─── Radio channel related: ────┬──────────────────┬──────────────────────┬──────────────────┐

│                  │                      │                  │

HR               dualHR                 dualFR               FR

RCR ------------------------│------------------│----------------------│------------------│---------------

│                  └──────────┬───────────┴──────────────────┘

│                             │

│┌────────────────────────────┤

││                            │

8                             16             not_used        NA

IR-------------------------│-----------------------------│----------------------------------------------

└─┬──────────┬──────────┐     │

│          │          │     │

│          │          │     └────┐

0.3        1.2        2.4        4.8         9.6    1.2/0.075                    NA

UR---------------------------│----------│----------│-----------│----------- ----------------------------

│          │          │           │

┌───────────────────┴──────────┴──────────┴───────────┘

│

none     V.21       V.22      V.22bis     V.26ter     V.32       V.23            auto1      NA

MT------------------------------------------------------------------------------------------------------

.c.1.3.1.2	X.32 Case (Packet Service)

UDI      3.1 kHz      FAX3       speech

ITC:-----│----------------------------------------------

│

┌─────┘

│

├─Layer/protocol related:──────────────────────────────────────────────────────────────┐

│                                                                                      │

│              NA       I.440        X.28deIN        X.28deUN         X.28nond       X.32          X.21    res

│    SAP-------------------------------------------------------------------------------│---------------------

│                                                                                      │

│                                                  ┌───────────────────────────────────┘

│                                                  │

│              NA             V.110              X.31Flag               No

│    RA--------------------------------------------│---------------------------------------------------------

│                                                  └─────────────┐

│                                                                │

│                               A                                S                          NA

│    SA----------------------------------------------------------│-------------------------------------------

│                               ┌────────────────────────────────┘

│                               │

│                               NT               bothNT                 bothT                 T        NA

│    CE-------------------------│----------------------------------------------------------------------------

│                               │

│                               │

│              NA               SDU                                                     unstructured

│    S--------------------------│----------------------------------------------------------------------------

│                               │

│                     ┌─────────┘

│                     │

│                   x.25        ISO6429                    COPnoFlCt                         NAV

│    UIL2P -----------│--------------------------------------------------------------------------------------

│                     │

│                     NA                              7                            8

│    NDB -------------│--------------------------------------------------------------------------------------

│                     │

│                     │

│                     NA       odd          even          0            1          none

│    NPB -------------│--------------------------------------------------------------------------------------

│                     │

│                     │

│                     NA                              1                            2

│    NSB -----------------------------------------------------------------------------------------------------

│

│

└─── Radio channel related: ────┬──────────────────┬──────────────────────┬──────────────────┐

│                  │                      │                  │

HR               dualHR                 dualFR               FR

RCR ------------------------│------------------│----------------------│------------------│---------------

│                  └──────────┬───────────┴──────────────────┘

│                             │

│                             │

│                             │

8            1)               16             not_used        NA

IR-------------------------│-----------------------------│----------------------------------------------

└────────────┬──────────┐     │

│          │     │

├──────────┼─────┴────┐

0.3        1.2        2.4        4.8         9.6     1.2/0.075                    NA

UR--------------------------------------│----------│-----------│----------- ----------------------------

┌──────────────────────────────┴──────────┴───────────┘

│

none     V.21       V.22      V.22bis     V.26ter     V.32       V.23            auto1      NA

MT------------------------------------------------------------------------------------------------------

1) for NIRR:6kb/s (not for the SETUP message)

.c.1.3.2	3.1 kHz audio ex-PLMN information transfer capability

.c.1.3.2.1	Non-X.32 Cases

UDI      3.1 kHz      FAX3       speech

ITC:----------------│-----------------------------------

│

┌────────────────┘

│

├─Layer/protocol related:─┐

│                         │

│              NA       I.440        X.28deIN        X.28deUN         X.28nond       X.32          X.21    res

│    SAP------------------│----------------------------------------------------------------------------------

│                         │

│                         └─────────────────────────────────────────────┐

│                                                                       │

│              NA             V.110              X.31Flag               No

│    RA-----------------------------------------------------------------│------------------------------------

│                                                                ┌──────┘

│                                                                │

│                               A                                S                          NA

│    SA----------------------------------------------------------│-------------------------------------------

│                                                                └────────────────────────────┐

│                                                                                             │

│                               NT               bothNT                 bothT                 T        NA

│    CE---------------------------------------------------------------------------------------│--------------

│                                                                                             │

│                                                                                             │

│              NA               SDU                                                     unstructured

│    S----------------------------------------------------------------------------------------│--------------

│                                                                                             │

│                                                                                             │

│                                                                                             │

│                   X.25      ISO6429                       COPnoFlCt                        NAV

│    UIL2P -----------------------------------------------------------------------------------│--------------

│                     ┌───────────────────────────────────────────────────────────────────────┘

│                     NA                              7                            8

│    NDB -------------│--------------------------------------------------------------------------------------

│                     │

│                     │

│                     NA       odd          even          0            1          none

│    NPB -------------│--------------------------------------------------------------------------------------

│                     │

│                     │

│                     NA                              1                            2

│    NSB -----------------------------------------------------------------------------------------------------

│

│

└─── Radio channel related: ────┬──────────────────┬──────────────────────┬──────────────────┐

│                  │                      │                  │

HR               dualHR                 dualFR               FR

RCR ------------------------│------------------│----------------------│------------------│---------------

│                  └──────────┬───────────┴──────────────────┘

│                             │

│┌────────────────────────────┤

││                            │

8                             16             not_used        NA

IR-------------------------│-----------------------------│----------------------------------------------

└─┬──────────┬──────────┐     │

│          │          │     │

│          │          │     └────┐

0.3        1.2        2.4        4.8         9.6     1.2/0.075                    NA

UR---------------------------│----------│----------│-----------│----------- ----------------------------

│          │          └──────────┐│

│          ├────────────┐        ││

none     V.21       V.22      V.22bis     V.26ter     V.32       V.23            auto1      NA

MT------------------------------------------------------------------------------------------------------

.c.1.3.2.2	X.32 Case ( Packet Service )

UDI      3.1 kHz      FAX3       speech

ITC:---------------│------------------------------------

│

┌───────────────┘

│

├─Layer/protocol related:──────────────────────────────────────────────────────────────┐

│                                                                                      │

│              NA       I.440        X.28deIN        X.28deUN         X.28nond       X.32          X.21    res

│    SAP-------------------------------------------------------------------------------│---------------------

│                                                                                      │

│                                                                       ┌──────────────┘

│                                                                       │

│              NA             V.110              X.31Flag               No

│    RA-----------------------------------------------------------------│------------------------------------

│                                                                ┌──────┘

│                                                                │

│                               A                                S                          NA

│    SA----------------------------------------------------------│-------------------------------------------

│                               ┌──────────────────┬─────────────┴────────┬───────────────────┐

│                               │                  │                      │                   │

│                               NT               bothNT                 bothT                 T        NA

│    CE-------------------------│------------------│----------------------│-------------------│--------------

│                               │┌─────────────────┴──────────────────────┘                   │

│                               ││                                                            │

│              NA               SDU                                                     unstructured

│    S--------------------------│-------------------------------------------------------------│--------------

│                               │                                                             │

│                     ┌─────────┘                                                             │

│                     │                                                                       │

│                   x.25        ISO6429                       COPnoFlCT                      NAV

│    UIL2P -----------│-----------------------------------------------------------------------│--------------

│                     ├───────────────────────────────────────────────────────────────────────┘

│                     NA                              7                            8

│    NDB -------------│--------------------------------------------------------------------------------------

│                     │

│                     │

│                     NA       odd          even          0            1          none

│    NPB -------------│--------------------------------------------------------------------------------------

│                     │

│                     │

│                     NA                              1                            2

│    NSB -----------------------------------------------------------------------------------------------------

│

│

└─── Radio channel related: ────┬──────────────────┬──────────────────────┬──────────────────┐

│                  │                      │                  │

HR               dualHR                 dualFR               FR

RCR ------------------------│------------------│----------------------│------------------│---------------

│                  └──────────┬───────────┴──────────────────┘

│                             │

│┌────────────────────────────┤

││           2)               │

8                             16             not_used        NA

IR-------------------------│-----------------------------│----------------------------------------------

└────────────┬──────────┐     │

│+---------│+----│

││         ││ 1) └────┐

0.3        1.2        2.4        4.8         9.6     1.2/0.075                    NA

UR--------------------------------------│----------│-----------│----------- ----------------------------

│          └──────────┐│

├────────────┐        ││

none     V.21       V.22      V.22bis     V.26ter     V.32       V.23            auto1      NA

MT------------------------------------------------------------------------------------------------------

1) for CE:NT or "both"

2) for CE:T or CE:NT and NIRR:6kb/s (not for the SETUP message)

.c.1.4	Bearer Service 41 ... 46, PAD Access Asynchronous

UDI      3.1 kHz      FAX3       speech

ITC------│----------------------------------------------

┌─────┘

│

├─Layer/protocol related:────────────────────────────────┐

│                                                        │

│              NA       I.440        X.28deIN        X.28deUN         X.28nond       X.32          X.21    res

│    SAP-------------------------------- ----------------│---------------------------------------------------

│                               ┌────────────────────────┘

│                               │

│              NA             V.110              X.31Flag               No

│    RA-------------------------│----------------------------------------------------------------------------

│                               │

│                               A                                S                          NA

│    SA-------------------------│----------------------------------------------------------------------------

│                               ├──────────────────┬──────────────────────┬───────────────────┐

│                               NT               bothNT                 bothT                 T        NA

│    CE-------------------------│------------------│----------------------│-------------------│--------------

│                               │┌─────────────────┴──────────────────────┘                   │

│                               ││                                                            │

│              NA               SDU                                                     unstructured

│    S---------------------------│------------------------------------------------------------│--------------

│                                ├─────────────────────────────┬─────────────────────────────┐│

│                                │                           4)│                           3)││

│                   x.25      ISO6429                       COPnoFlCT                        NAV

│    UIL2P-----------------------│-----------------------------│------------------------------│--------------

│                                └────────────────────┬────────┴───────────────────┬──────────┘

│                                                     │                            │

│                     NA                              7                            8

│    NDB ---------------------------------------------│----------------------------│-------------------------

│                               ┌────────────┬────────┴───┬────────────┐           │

│                               │            │            │            │           │

│                     NA       odd          even          0            1          none

│    NPB -----------------------│------------│------------│------------│-----------│-------------------------

│                               └────────────┴────────┬───┴────────────┴───────────┤

│                                                     │                            │

│                     NA                              1                            2

│    NSB ----------------------------------------------------------------------------------------------------

│

│

└─── Radio channel related: ────┬──────────────────┬──────────────────────┬──────────────────┐

│                  │                      │                  │

HR               dualHR                 dualFR               FR

RCR ------------------------│------------------│----------------------│------------------│---------------

│                  └──────────┬───────────┴──────────────────┘

│                             │

│+----------------------------┤

│|               2)           │

8                             16               not_used       NA

IR-------------------------│-----------------------------│----------------------------------------------

┌────────┴─┬──────────┬──────────┬─────│─────────────────┐

│+---------│+---------│+---------│+----└────┐-----------+│

│|         │|         │|         │| 1)      │           |│

0.3        1.2        2.4        4.8         9.6      1.2/0.075  5)               NA

UR----------------│----------│----------│----------│-----------│-----------│----------------------------

│          │          │          │           │           │

┌────────┴──────────┴──────────┴──────────┴───────────┴───────────┘

│

none     V.21       V.22      V.22bis     V.26ter     V.32       V.23            auto1      NA

MT------------------------------------------------------------------------------------------------------

1) for CE:NT or "both"

2) for CE:T only or CE:NT and NIRR:6kb/s (not for the SETUP message)

3) for MOC with "outband" flow control requested

4) for MOC with no flow control requested

5) MOC only, 75 bit/s in the uplink, 1200 bit/s in the downlink direction

.c.1.5	Bearer Service 51 ... 53 ,Data Packet Duplex Synchronous,

Unrestricted digital information transfer capability

UDI      3.1 kHz      FAX3       speech

ITC:-----│----------------------------------------------

│

┌─────┘

│

├─Layer/protocol related:──────────────────────────────────────────────────────────────┐

│                                                                                      │

│              NA       I.440        X.28deIN        X.28deUN         X.28nond       X.32          X.21    res

│    SAP-------------------------------------------------------------------------------│---------------------

│                                                                                      │

│                                                  ┌───────────────────────────────────┘

│                                                  │

│              NA             V.110              X.31Flag               No

│    RA--------------------------------------------│---------------------------------------------------------

│                                                  └─────────────┐

│                                                                │

│                               A                                S                          NA

│    SA----------------------------------------------------------│-------------------------------------------

│                               ┌────────────────────────────────┘

│                               │

│                               NT               bothNT                 bothT                 T        NA

│    CE-------------------------│----------------------------------------------------------------------------

│                               │

│                               │

│              NA               SDU                                                     unstructured

│    S--------------------------│----------------------------------------------------------------------------

│                               │

│                     ┌─────────┘

│                     │

│                   x.25        ISO6429                    COPnoFlCt                         NAV

│    UIL2P -----------│--------------------------------------------------------------------------------------

│                     │

│                     NA                              7                            8

│    NDB -------------│--------------------------------------------------------------------------------------

│                     │

│                     │

│                     NA       odd          even          0            1          none

│    NPB -------------│--------------------------------------------------------------------------------------

│                     │

│                     │

│                     NA                              1                            2

│    NSB -----------------------------------------------------------------------------------------------------

│

│

└─── Radio channel related: ────┬──────────────────┬──────────────────────┬──────────────────┐

│                  │                      │                  │

HR               dualHR                 dualFR               FR

RCR ------------------------│------------------│----------------------│------------------│---------------

│                  └──────────┬───────────┴──────────────────┘

│                             │

│                             │

│             1)              │

8                             16             not_used        NA

IR-------------------------│-----------------------------│----------------------------------------------

└────────────┬──────────┐     │

│┌─────────│+────┤

││         ││   └────┐

0.3        1.2        2.4        4.8         9.6     1.2/0.075                    NA

UR--------------------------------------│----------│-----------│----------- ----------------------------

┌──────────────────────────────┴──────────┴───────────┘

│

none     V.21       V.22      V.22bis     V.26ter     V.32       V.23            auto1      NA

MT------------------------------------------------------------------------------------------------------

1) for NIRR:6kb/s (not for the SETUP message)

.c.1.6	Bearer Service 61, Alternate Speech/Data

The information element of the "repeat indicator" is set to the value "circular for successive selection (alternate)".

.c.1.6.1	Bearer Service 61,"Speech"

UDI      3.1 kHz      FAX3       speech

ITC-----------------------------------------│-----------

┌────────────────────────────────────────┘

├─Layer/protocol related:──────────────────────────────────────────────────────────────────────────────────┐

│                                                                                                          │

│              NA       I.440       X.28deIN        X.28deUN       X.28nond     X.32        X.21          NAV

│    SAP---------------------------------------------------------------------------------------------------│-

│                                                                                                          │

│                                                                                                          │

│              NA             V.110              X.31Flag               No                                NAV

│    RA----------------------------------------------------------------------------------------------------│-

│                                                                                                          │

│                               A                                S                          NA            NAV

│    SA----------------------------------------------------------------------------------------------------│-

│                                                                                                          │

│                                                                                                          │

│                               NT               bothNT                 bothT                T        NA  NAV

│    CE ---------------------------------------------------------------------------------------------------│-

│                                                                                                          │

│                                                                                                          │

│              NA                                                                                         NAV

│    S-----------------------------------------------------------------------------------------------------│-

│                                                                                                          │

│                                                                                              ┌───────────┘

│                   X.25     ISO6429                      COPnoFlCt                           NAV

│    UIL2P ------------------------------------------------------------------------------------│-------------

│                                                                                              └───────────┐

│                                                                                                          │

│                      NA                             7                            8                      NAV

│    NDB --------------------------------------------------------------------------------------------------│-

│                                                                                                          │

│                                                                                                          │

│                      NA      odd          even          none         0           1                      NAV

│    NPB --------------------------------------------------------------------------------------------------│-

│                                                                                                          │

│                                                                                                          │

│                      NA                             1                            2                      NAV

│    NSB ----------------------------------------------------------------------------------------------------

│

│

│

└─── Radio channel related: ──────────┐

┌─────┴────────────┬──────────────────────┬──────────────────┐

│                  │                      │                  │

HR               dualHR                 dualFR               FR

RCR ------------------------│------------------│----------------------│------------------│---------------

└──────────────────┴──────────────────────┴──────────────────┴─────────────┐

│

8                            16             not used        NA           NAV

IR----------------------------------------------------------------------------------------------------│-

│

0.3        1.2        2.4        4.8         9.6      1.2/0.075                   NA NAV

UR----------------------------------------------------------------------------------------------------│-

│

│

none     V.21      V.22       V.22bis     V.26ter    V.32     V.23            auto1      NA  NAV

MT------------------------------------------------------------------------------------------------------

.c.1.6.2	Bearer Service 61, 3.1 kHz audio ex-PLMN information transfer capability

.c.1.6.2.1	Asynchronous

UDI      3.1 kHz      FAX3       speech

ITC-----------------│------------------------------------

│

┌────────────────┘

│

├─Layer/protocol related:─┐

│                         │

│              NA       I.440        X.28deIN        X.28deUN         X.28nond       X.32          X.21    res

│    SAP------------------│----------------------------------------------------------------------------------

│                         └─────────────────────────────────────────────┐

│                                                                       │

│              NA             V.110              X.31Flag               No

│    RA-----------------------------------------------------------------│------------------------------------

│                               ┌───────────────────────────────────────┘

│                               A                                S                          NA

│    SA-------------------------│----------------------------------------------------------------------------

│                               ├──────────────────┬──────────────────────┬───────────────────┐

│                               │                  │                      │                   │

│                               NT               bothNT                 bothT                 T        NA

│    CE-------------------------│------------------│----------------------│-------------------│--------------

│                               ├──────────────────┴──────────────────────┘                   │

│                               │                                                             │

│              NA               SDU                                                     unstructured

│    S---------------------------│------------------------------------------------------------│--------------

│                                │                                                            │

│                                ├──────────────────────────┬────────────────────────────────┐│

│                                │                        5)│                              4)││

│                   X.25      ISO6429                    COPnoFlCt                           NAV

│    UIL2P ----------------------│--------------------------│---------------------------------│--------------

│                                └────────────────────┬─────┴──────────────────────┬──────────┘

│                                                     │                            │

│                     NA                              7                            8

│    NDB ---------------------------------------------│----------------------------│-------------------------

│                               ┌────────────┬────────┴───┬────────────┬──────────┐│

│                               │            │            │            │          ││

│                     NA       odd          even          0            1          none

│    NPB -----------------------│------------│------------│------------│-----------│-------------------------

│                               └────────────┴────────┬───┴────────────┴───────────┤

│                                                     │                            │

│                     NA                              1                            2

│    NSB ----------------------------------------------------------------------------------------------------

│

│

└─── Radio channel related: ────┬──────────────────┬──────────────────────┬──────────────────┐

│                  │                      │                  │

HR               dualHR                 dualFR               FR

RCR ------------------------│------------------│----------------------│------------------│---------------

│                  └──────────┬───────────┴──────────────────┘

│                             │

│+----------------------------┤

│|               2)           │

8                             16             not_used        NA

IR-------------------------│-----------------------------│----------------------------------------------

┌────────┴─┬──────────┬──────────┬─────│─────────────────┐

│+---------│+---------│+---------│+----└────┐-----------+│

│|         │|         │|         │| 1)      │           |│

0.3        1.2        2.4        4.8         9.6      1.2/0.075 6)            NA

UR----------------│|---------│|---------│|---------│|----------│|----------│|---------------------------

│+---------│+---------│+---------│+----------│+----------│+--------------+

│          │          │          └──────────┐│           │               | 1)

│          │          ├───────────┐         ││           │               |

none     V.21       V.22      V.22bis     V.26ter     V.32       V.23            auto1       NA

MT------------------------------------------------------------------------------------------------------

1) for CE:NT or "both"

2) for CE:T only or CE:NT and NIRR:6kb/s (not for the SETUP message)

3) for MOC only

4) for MTC in the SETUP message or MOC/MTC with "out-band" flow control requested

(not for V.21 and V.23 modem types)

5) for MOC/MTC with no flow control requested

6) MOC only, 75 bit/s in the uplink, 1200 bit/s in the downlink direction

.c.1.6.2.2	Synchronous

UDI      3.1 kHz      FAX3       speech

ITC:----------------│-----------------------------------

│

┌────────────────┘

│

├─Layer/protocol related:─┐

│                         │

│              NA       I.440        X.28deIN        X.28deUN         X.28nond       X.32          X.21    res

│    SAP------------------│----------------------------------------------------------------------------------

│                         └─────────────────────────────────────────────┐

│                                                                       │

│              NA             V.110              X.31Flag               No

│    RA-----------------------------------------------------------------│------------------------------------

│                                                                ┌──────┘

│                               A                                S                          NA

│    SA----------------------------------------------------------│-------------------------------------------

│                                                                └────────────────────────────┐

│                                                                                             │

│                               NT               bothNT                 bothT                 T        NA

│    CE---------------------------------------------------------------------------------------│--------------

│                                                                                             │

│                                                                                             │

│              NA               SDU                                                     unstructured

│    S----------------------------------------------------------------------------------------│--------------

│                                                                                             │

│                                                                                             │

│                                                                                             │

│                   X.25      ISO6429                    COPnoFlCt                            NAV

│    UIL2P -----------------------------------------------------------------------------------│--------------

│                     ┌───────────────────────────────────────────────────────────────────────┘

│                     │

│                     NA                              7                            8

│    NDB -------------│--------------------------------------------------------------------------------------

│                     │

│                     │

│                     NA       odd          even          0            1          none

│    NPB -------------│--------------------------------------------------------------------------------------

│                     │

│                     │

│                     NA                              1                            2

│    NSB ----------------------------------------------------------------------------------------------------

│

│

└─── Radio channel related: ────┬──────────────────┬──────────────────────┬──────────────────┐

│                  │                      │                  │

HR               dualHR                 dualFR               FR

RCR ------------------------│------------------│----------------------│------------------│---------------

│                  └──────────┬───────────┴──────────────────┘

│                             │

│┌────────────────────────────┤

││                            │

8                             16              not_used       NA

IR-------------------------│-----------------------------│----------------------------------------------

└─┬──────────┬──────────┐     │

│          │          │     │

│          │          │     └────┐

0.3        1.2        2.4        4.8         9.6      1.2/0.075                     NA

UR---------------------------│----------│----------│-----------│----------------------------------------

│          │          │           │

│          │          └──────────┐│

│          ├───────────┐         ││

none     V.21       V.22      V.22bis     V.26ter     V.32       V.23            auto1       NA

MT------------------------------------------------------------------------------------------------------

.c.1.7	Bearer Service 81, Speech followed by Data

The information element of the "repeat indicator" is set to the value "sequential for successive selection (followed by)".

.c.1.7.1 Bearer Service 81,"Speech"

Ref. section 1.6.1

.c.1.7.2	Bearer Service 81, 3.1 kHz audio ex-PLMN information transfer capability

.c.1.7.2.1	Asynchronous

Ref. section 1.6.2.1

.c.1.7.2.2	Synchronous

Ref. section 1.6.2.2

.c.1.8	Teleservice 11 ... 12, Speech

Ref. section 1.6.1

.c.1.9	Teleservice 21 ... 23, Short Message

not applicable

.c.1.10	Teleservice 61, Alternate Speech and Facsimile group 3

The information element of the "repeat indicator" is set to the value "circular for successive selection (alternate)".

.c.1.10.1	Teleservice 61, "Speech"

Ref. section 1.6.1.

.c.1.10.2	Teleservice 61, "Facsimile group 3"

UDI      3.1 kHz      FAX3       speech

ITC----------------------------│------------------------

┌───────────────────────────┘

│

├─Layer/protocol related:───┐

│              ┌────────────┘

│              NA       I.440        X.28deIN        X.28deUN         X.28nond       X.32          X.21    res

│    SAP-------│---------------------------------------------------------------------------------------------

│              └────────────────────────────────────────────────────────┐

│              NA             V.110              X.31Flag               No

│    RA-----------------------------------------------------------------│------------------------------------

│                                                                ┌──────┘

│                               A                                S                          NA

│    SA----------------------------------------------------------│-------------------------------------------

│                               ┌──────────────────┬─────────────┴────────┬───────────────────┐

│                               │                  │                      │                   │

│                               NT               bothNT                 bothT                 T        NA

│    CE-------------------------│------------------│----------------------│-------------------│--------------

│                               ├──────────────────┴──────────────────────┘                   │

│                               │                                                             │

│              NA               SDU                                                     unstructured

│    S---------------------------│------------------------------------------------------------│--------------

│                                │                                                            │

│                     ┌──────────┴───────────────────────────────────────────────────────────┐│

│                     │                                           3)                         ││

│                   X.25      ISO6429                      COPnoFlCt                         NAV

│    UIL2P -----------│-----------------------------------------------------------------------│--------------

│                     │┌──────────────────────────────────────────────────────────────────────┘

│                     NA                              7                            8

│    NDB -------------││-------------------------------------------------------------------------------------

│                     ││

│                     NA       odd          even          0            1          none

│    NPB -------------││-------------------------------------------------------------------------------------

│                     ││

│                     NA                              1                            2

│    NSB -----------------------------------------------------------------------------------------------------

│

│

└─── Radio channel related: ─────────────────────────────────────────────────────────────────┐

HR               dualHR                 dualFR               FR

RCR -------------------------------------------------------------------------------------│---------------

┌──────────────────────────────┘

+-----------------------------┤

|                2)           │

8                             16             not_used        NA

IR-------------------------│-----------------------------│----------------------------------------------

└───────────┬──────────┐      │

│+---------│+-----│

│|         │| 1)  └────┐

0.3        1.2        2.4        4.8         9.6      1.2/0.075                    NA

UR--------------------------------------│----------│-----------│----------------------------------------

┌──────────────────────────────┴──────────┴───────────┘

│

none     V.21       V.22      V.22bis     V.26ter     V.32       V.23            auto1      NA

MT------------------------------------------------------------------------------------------------------

1) for CE:NT or "both"

2) for CE:T only

3) for MTC in the SETUP message only

.c.1.11	Teleservice 62, Automatic Facsimile Group 3

Ref. section 1.10, the information element "repeat indicator" is not available/valid.

.c.2	Low Layer/High Layer Compatibility Information Element

.c.2.1	Introduction

.c.2.1.1	General Consideration

The purpose of the Low Layer/High Layer Compatibility Information Element (LLC/HLC-IE) is to provide a means for additional end-to-end compatibility checking by an addressed entity (e.g. a remote user, an interworking unit or a high layer function network node). The LLC/HLC-IE is transferred transparently by the GSM PLMN and an ISDN between the call originating entity (e.g. the calling user) and the addressed entity.

With respect to the individual parameter settings at the MS the following cases may be distinguished (ref. GSM 07.02 and GSM 07.03):

-	Mobile-originated call set up by a MS consisting of a MT with R interface:

The setting results from respective MMI actions and/or MT internal settings.

-	Mobile-originated call set up by a MS consisting of a MT with S interface:

The LLC/HLC-IEs which are contained in the ISDN SETUP message received from the terminal are passed unchanged to the MSC.

-	Mobile-terminated call set up to a MS consisting of a MT with R interface:

The LLC/HLC related part of the compatibility check is carried out according to the knowledge of the MT concerning its implemented functions (i.e. answering the call). The offered field values determine the selection of the terminal function for the intended connection.

-	Mobile-terminated call set up to a MS consisting of a MT with S interface:

The LLC/HLC received from the MSC is passed to the terminal by the MT. The LLC/HLC related part of the compatibility check is up to the terminal connected to the S interface of the MT, as is the selection of the terminal function (i.e. answering the call).

Where applicable, the same settings and rules concerning LLC and/or HLC apply as for ISDN use (ref. ETS 300 102-1 and ETR 018). However, considering that GSM PLMN data transmission is based on CCITT V.110 rate adaptation, the MS shall provide the LLC-IE for mobile-originated calls when using unrestricted digital information transfer capability. This is to assure the conveyance of the e.g. "V.110" indication towards the called entity, as the comparable indication in the ISDN BC-IE may be lost. It shall also be possible to choose whether or not the LLC-IE is provided for the case of an information transfer capability "3.1 kHz audio ex PLMN".

There shall be no contradiction of the information between the BC-IE and LLC-IE at the originating side. However, as some parts of the bearer capability may be modified during the transport of the call, there should be minimum duplication of this information between the BC-IE and the LLC-IE.

If as a result of duplication, a contradiction occurs between the BC-IE and the LLC-IE at the terminating side, the receiving entity shall ignore the conflicting information in the LLC-IE.

.c.2.1.2	Interpretation of the Tables

The individual contents of the LLC/HLC-IE are represented in the following tables. The indication of the applicable service group defines the link between the GSM BC-IE and its associated LLC/HLC-IEs.

If the appropriate message includes multiple BC-IEs and if LLC and/or HLC information is available, multiple LLCs and HLCs shall be included in the message. The LLC/HLC associated with the BC-IE indicating speech shall be marked as "not applicable" (see GSM 04.08).

Legend:	{ xxxx | yyyy }	choice of values

---- 	not relevant for this service (set to appropriate value)

[ zzzz ]	optional

.c.2.2	LLC Bearer Service 21 ... 26

.c.2.2.1	Unrestricted digital information transfer capability

Low layer compatibility information element:

╔═════╤═════════════════════════════════════════╤═══════════════════════════════════════════════════════════╗

║Octet│ Information element field               │ field value                                               ║

╠═════╪═════════════════════════════════════════╪═══════════════════════════════════════════════════════════╣

║  3  │ Coding standard                         │ CCITT                                                     ║

║     │ Information transfer capability         │ unrestricted digital                                      ║

╟─────┼─────────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  4  │ Transfer mode                           │ circuit mode                                              ║

║     │ Information transfer rate               │ 64 kbit/s                                                 ║

╠═════╪═╤═══════════════════════════════════════╪═══════════════════════════════════════════════════════════╣

║  5  │ │ User information layer 1 protocol     │ V.110/X.30                                                ║

╟─────┼─┼───────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  5a │ │ Synchronous / asynchronous            │ asynchronous                                              ║

║     │ │ Negotiation                           │ in-band not possible                                      ║

║     │ │ User rate                             │ { 0.3 | 1.2 | 2.4 | 4.8 | 9.6 | 1.2/0.075 } kbit/s        ║

╟─────┼─┼───────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  5b │ │ Intermediate rate                     │ { 8 | 16 } kbit/s                                         ║

║     │ │ NIC on Tx                             │ -----                                                     ║

║     │ │ NIC on Rx                             │ -----                                                     ║

║     │ │ Flow control on Tx                    │ { not required 1)| required }                             ║

║     │ │ Flow control on Rx                    │ { not accepted 1)| accepted }                             ║

╟─────┼─┼───────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  5c │ │ Number of stop bits                   │ { 1 | 2 } bits                                            ║

║     │ │ Number of data bits                   │ { 7 | 8 } bits                                            ║

║     │ │ Parity                                │ { odd | even | none | forced to 0 | forced to 1 }         ║

╟─────┼─┼───────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  5d │ │ Duplex mode                           │ [ duplex ]                                                ║

║     │ │ Modem type                            │ [ { V.21 | V.22 | V.22bis | V.23 | V.26ter | V.32 } ]     ║

╚═════╧═╧═══════════════════════════════════════╧═══════════════════════════════════════════════════════════╝

1) only these values are applicable to Mobile Originated Calls

.c.2.2.2	3,1 kHz audio ex-PLMN information transfer capability

Low layer compatibility information element:

╔═════╤═════════════════════════════════════════╤═══════════════════════════════════════════════════════════╗

║Octet│ Information element field               │ field value                                               ║

╠═════╪═════════════════════════════════════════╪═══════════════════════════════════════════════════════════╣

║  3  │ Coding standard                         │ CCITT                                                     ║

║     │ Information transfer capability         │ 3.1kHz audio                                              ║

╟─────┼─────────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  4  │ Transfer mode                           │ circuit mode                                              ║

║     │ Information transfer rate               │ 64 kbit/s                                                 ║

╠═════╪═╤═══════════════════════════════════════╪═══════════════════════════════════════════════════════════╣

║  5  │ │ User information layer 1 protocol     │ G.711 A-law                                               ║

╟─────┼─┼───────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  5a │ │ Synchronous / asynchronous            │ (may be set depending on                                  ║

║     │ │ Negotiation                           │  user's requirement)                                      ║

║     │ │ User rate                             │                                                           ║

╟─────┼─┼───────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  5b │ │ Intermediate rate                     │                                                           ║

║     │ │ NIC on Tx                             │ not relevant                                              ║

║     │ │ NIC on Rx                             │ but cannot be omitted                                     ║

║     │ │ Flow control on Tx                    │ in order to have octet 5d                                 ║

║     │ │ Flow control on Rx                    │                                                           ║

╟─────┼─┼───────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  5c │ │ Number of stop bits                   │ (may be set depending on                                  ║

║     │ │ Number of data bits                   │  the user's requirement)                                  ║

║     │ │ Parity                                │                                                           ║

╟─────┼─┼───────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  5d │ │ Duplex mode                           │ [ duplex ]                                                ║

║     │ │ Modem type                            │ [ { V.21 | V.22 | V.22bis | V.23 | V.26ter | V.32 } ]     ║

╚═════╧═════════════════════════════════════════╧═══════════════════════════════════════════════════════════╝

Note:	If octet 5d is not specified, the whole LLC is not required.

.c.2.3	LLC Bearer Service 31 ... 34

.c.2.3.1	Unrestricted digital information transfer capability

Low layer compatibility information element:

╔═════╤═════════════════════════════════════════╤═══════════════════════════════════════════════════════════╗

║Octet│ Information element field               │ field value                                               ║

╠═════╪═════════════════════════════════════════╪═══════════════════════════════════════════════════════════╣

║  3  │ Coding standard                         │ CCITT                                                     ║

║     │ Information transfer capability         │ digital unrestricted                                      ║

╟─────┼─────────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  4  │ Transfer mode                           │ circuit mode                                              ║

║     │ Information transfer rate               │ 64 kbit/s                                                 ║

╠═════╪═╤═══════════════════════════════════════╪═══════════════════════════════════════════════════════════╣

║  5  │ │ User information layer 1 protocol     │ { V.110/X.30 | X.31 flag stuffing }                       ║

╟─────┼─┼───────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  5a │ │ Synchronous / asynchronous            │ synchronous                                               ║

║     │ │ Negotiation                           │ in-band not possible                                      ║

║     │ │ User rate                             │ { 1.2 | 2.4 | 4.8 | 9.6 } kbit/s                          ║

╟─────┼─┼───────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  5b │ │ Intermediate rate                     │ { 8 | 16 } kbit/s                                         ║

║     │ │ NIC on Tx                             │ { not required | required }                               ║

║     │ │ NIC on Rx                             │ { not accepted | accepted }                               ║

║     │ │ Flow control on Tx                    │ -----                                                     ║

║     │ │ Flow control on Rx                    │ -----                                                     ║

╟─────┼─┼───────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  5c │ │ Number of stop bits                   │ not relevant                                              ║

║     │ │ Number of data bits                   │ but cannot be omitted                                     ║

║     │ │ Parity                                │ in order to have octet 5d                                 ║

╟─────┼─┼───────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  5d │ │ Duplex mode                           │ [ duplex ]                                                ║

║     │ │ Modem type                            │ [ { V.21 | V.22bis | V.26ter | V.32 } ]                   ║

╠═════╪═╧═══════════════════════════════════════╪═══════════════════════════════════════════════════════════╣

║  6  │ User information layer 2 protocol       │ [ X.25 ]                                                  ║

╟─────┼─────────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  7  │ User information layer 3 protocol       │ [ X.25 ]                                                  ║

╚═════╧═════════════════════════════════════════╧═══════════════════════════════════════════════════════════╝

Note:	If octet 5d is not specified, octet 5c may be omitted.

.c.2.3.2	3.1kHz audio ex-PLMN information transfer capability

Low layer compatibility information element:

╔═════╤═════════════════════════════════════════╤═══════════════════════════════════════════════════════════╗

║Octet│ Information element field               │ field value                                               ║

╠═════╪═════════════════════════════════════════╪═══════════════════════════════════════════════════════════╣

║  3  │ Coding standard                         │ CCITT                                                     ║

║     │ Information transfer capability         │ 3.1kHz audio                                              ║

╟─────┼─────────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  4  │ Transfer mode                           │ circuit mode                                              ║

║     │ Information transfer rate               │ 64 kbit/s                                                 ║

╠═════╪═╤═══════════════════════════════════════╪═══════════════════════════════════════════════════════════╣

║  5  │ │ User information layer 1 protocol     │ G.711 A-law                                               ║

╟─────┼─┼───────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  5a │ │ Synchronous / asynchronous            │ (may be set depending on                                  ║

║     │ │ Negotiation                           │  the user's requirement)                                  ║

║     │ │ User rate                             │                                                           ║

╟─────┼─┼───────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  5b │ │ Intermediate rate                     │                                                           ║

║     │ │ NIC on Tx                             │ not relevant                                              ║

║     │ │ NIC on Rx                             │ but cannot be omitted                                     ║

║     │ │ Flow control on Tx                    │ in order to have octet 5d                                 ║

║     │ │ Flow control on Rx                    │                                                           ║

╟─────┼─┼───────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  5c │ │ Number of stop bits                   │ (may be set depending on                                  ║

║     │ │ Number of data bits                   │  the user's requirement)                                  ║

║     │ │ Parity                                │                                                           ║

╟─────┼─┼───────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  5d │ │ Duplex mode                           │ [ duplex ]                                                ║

║     │ │ Modem type                            │ [ { V.22 | V.22bis | V.26ter | V.32 } ]                   ║

╠═════╪═╧═══════════════════════════════════════╪═══════════════════════════════════════════════════════════╣

║  6  │ User information layer 2 protocol       │ [  X.25  ]                                                ║

╟─────┼─────────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  7  │ User information layer 3 protocol       │ [  X.25  ]                                                ║

╚═════╧═════════════════════════════════════════╧═══════════════════════════════════════════════════════════╝

Note:	If octet 5d is not specified, octets 5a..5d may be omitted.

.c.2.4	LLC Bearer Services 41 ... 46

May be optionally available with the settings according to 2.2.1

.c.2.5	LLC Bearer Services 51 ... 53

.c.2.5.1	Unrestricted digital information transfer capability

Low layer compatibility information element:

╔═════╤═════════════════════════════════════════╤═══════════════════════════════════════════════════════════╗

║Octet│ Information element field               │ field value                                               ║

╠═════╪═════════════════════════════════════════╪═══════════════════════════════════════════════════════════╣

║  3  │ Coding standard                         │ CCITT                                                     ║

║     │ Information transfer capability         │ unrestricted digital                                      ║

╟─────┼─────────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  4  │ Transfer mode                           │ circuit mode                                              ║

║     │ Information transfer rate               │ 64 kbit/s                                                 ║

╠═════╪═╤═══════════════════════════════════════╪═══════════════════════════════════════════════════════════╣

║  5  │ │ User information layer 1 protocol     │ X.31 flag stuffing                                        ║

╟─────┼─┼───────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  5a │ │ Synchronous / asynchronous            │ synchronous                                               ║

║     │ │ Negotiation                           │ in-band not possible                                      ║

║     │ │ User rate                             │ { 2.4 | 4.8 | 9.6 } kbit/s                                ║

╠═════╪═╧═══════════════════════════════════════╪═══════════════════════════════════════════════════════════╣

║  6  │ User information layer 2 protocol       │ X.25                                                      ║

╟─────┼─────────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  7  │ User information layer 3 protocol       │ X.25                                                      ║

╚═════╧═════════════════════════════════════════╧═══════════════════════════════════════════════════════════╝

.c.2.6	LLC Bearer Service 61

.c.2.6.1	3,1 kHz audio ex-PLMN information transfer capability, Asynchronous

Ref. section 2.2.2

.c.2.6.2	3,1 kHz audio ex-PLMN information transfer capability, Synchronous

Ref. section 2.3.2

.c.2.7	LLC Bearer Service 81

.c.2.7.1	3,1 kHz audio ex-PLMN information transfer capability, Asynchronous

Ref. section 2.2.2

.c.2.7.2	3,1 kHz audio ex-PLMN information transfer capability, Synchronous

Ref. section 2.3.2

.c.2.8	HLC Teleservices 11 ... 12

High layer compatibility information element:

╔═════╤═════════════════════════════════════════╤═══════════════════════════════════════════════════════════╗

║Octet│ Information element field               │ Field value                                               ║

╠═════╪═════════════════════════════════════════╪═══════════════════════════════════════════════════════════╣

║  3  │ Coding standard                         │ CCITT                                                     ║

║     │ Interpretation                          │ first high layer characteristic identification to be      ║

║     │                                         │ used in the call                                          ║

║     │ Presentation method of protocol profile │ high layer protocol profile                               ║

╟─────┼─────────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  4  │ High layer characteristics identific.   │ Telephony                                                 ║

╚═════╧═════════════════════════════════════════╧═══════════════════════════════════════════════════════════╝

.c.2.9	HLC Teleservices 21 ... 23

not applicable

.c.2.10	HLC Teleservice 61

High layer compatibility information element:

╔═════╤═════════════════════════════════════════╤═══════════════════════════════════════════════════════════╗

║Octet│ Information element field               │ Field value                                               ║

╠═════╪═════════════════════════════════════════╪═══════════════════════════════════════════════════════════╣

║  3  │ Coding standard                         │ CCITT                                                     ║

║     │ Interpretation                          │ first high layer characteristic identification to be      ║

║     │                                         │ used in the call                                          ║

║     │ Presentation method of protocol profile │ high layer protocol profile                               ║

╟─────┼─────────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  4  │ High layer characteristics identific.   │ Facsimile G2/G3                                           ║

╚═════╧═════════════════════════════════════════╧═══════════════════════════════════════════════════════════╝

.c.2.11	HLC Teleservice 62

High layer compatibility information element:

╔═════╤═════════════════════════════════════════╤═══════════════════════════════════════════════════════════╗

║Octet│ Information element field               │ Field value                                               ║

╠═════╪═════════════════════════════════════════╪═══════════════════════════════════════════════════════════╣

║  3  │ Coding standard                         │ CCITT                                                     ║

║     │ Interpretation                          │ first high layer characteristic identification to be      ║

║     │                                         │ used in the call                                          ║

║     │ Presentation method of protocol profile │ high layer protocol profile                               ║

╟─────┼─────────────────────────────────────────┼───────────────────────────────────────────────────────────╢

║  4  │ High layer characteristics identific.   │ Facsimile G2/G3                                           ║

╚═════╧═════════════════════════════════════════╧═══════════════════════════════════════════════════════════╝

History

Document history

September 1994	First Edition

March 1995	Unified Approval Procedure	UAP 26:	1995-03-06 to 1995-06-30

July 1995	Second Edition

ISBN 2-7437-0189-7

Dépôt légal : Juillet 1995

NOTE:	- an idle frame sent by the BSS and received by the MS has the same pattern as the synchronisation pattern 1/OFF

- at the moment when the message CONNECT (MOC) or CONNACK (MTC) is received at the MS, it is guarantied that this pattern is received from the MSC/IWF with the exception of a loss of frame synchronisation on the Abis interface

- the handling of frame stealing in case of 2400bit/s full rate data channels is implementation dependent

1) This parameter is correlated with the value of the parameter connection element

2) For non-transparent services this paramter is correlated with the values of the parameters radio channel requirements and negotiation of intermediate rate requested.