II.11	TESTING OF TELESERVICES

II.11.1     CATEGORY 1: SPEECH TELESERVICES - TRANSMISSION REQUIREMENTS

Ref: GSM 03.50, 06.10, (Draft NET 33)

This section provides for the testing of both handset (including hand held portable) and handsfree MSs. An MS may be of either type alone or a combined handset and handsfree MS. The test descriptions for handsfree operation, however, only covers the stability margin.

Note:	The handsfree tests may be enhanced at a later date to include SLR and RLR etc., until now no agreement was found on the exact methods to be used.

In this section all references to CCITT recommendations, unless otherwise stated, are to the Blue Book (1988).

Frequency settings in the following tests are taken from ISO 3, R10 series or R40 series or from Table 2 of REC CCITT P.79.  A departure from the nominal frequencies of + 5% below 240Hz and + 2% at 240Hz and above is accepted. Any submultiple of the sampling frequency of 8kHz shall be avoided. In the case of 4kHz the departure is restricted to -2%.

The measurement accuracy for signal level is + 0.2dB and for sound pressure + 0.6dB.

The tests are performed with "DTX = off".

II.11.1.1	Sending sensitivity/frequency response

Definition

The sending sensitivity frequency response is, as a function of the input test tone frequency, the ratio expressed in dB between the output level, represented by the PCM bit stream at the Digital Audio Interface (DAI) (III. 6.4) and the input sound pressure in the artificial mouth required to obtain this.

Method of measurement

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51.

A pure tone with a sound pressure of -4.7dBPa shall be applied at the mouth reference point (MRP) as described in REC CCITT P.64 using an artificial mouth conforming to REC CCITT P.51.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

Measurements are made at one-twelfth-octave intervals as given by the R40 series of preferred numbers in ISO 3 for frequencies from 100Hz to 4000Hz inclusive.

At each frequency the output level represented by the PCM bit stream at the DAI (pin 23) is measured.

Requirement

The sending sensitivity/frequency response (from MRP to the DAI) shall be within a mask given in the following table. The mask can be drawn with straight lines between the breaking points in the table on a logarithmic (frequency) vs linear (dB sensitivity) scale.

All sensitivity levels are dB on an arbitrary scale.

┌────────────────┬──────────────────┬──────────────────┐

│ Frequency [Hz] │ Upper Limit [dB] │ Lower Limit [dB] │

├────────────────┼──────────────────┼──────────────────┤

│    a =  100    │      -12         │                  │

│    b =  200    │        0         │                  │

│    c =  300    │        0         │     -12          │

│    d = 1000    │        0         │      -6          │

│    e = 2000    │        4         │      -6          │

│    f = 3000    │        4         │      -6          │

│    g = 3400    │        4         │      -9          │

│    h = 4000    │        0         │                  │

└────────────────┴──────────────────┴──────────────────┘

Sending sensitivity/frequency mask

II.11.1.2	Sending Loudness Rating

Definition

The Sending Loudness Rating (SLR) is a means of expressing the sending frequency response based on objective single tone measurements in a way which relates to how a speech signal would be perceived by a listener.

Method of measurement

The sending sensitivity (II.11.1.1) is measured at each of the 14 frequencies listed in Table 2 of REC CCITT P.79, bands 4 to 17.

The sensitivity is expressed in terms of dBV/Pa and the SLR is calculated according to formula 4.19b of REC CCITT P.79, over bands 4 to 17, using the sending weighting factors from Table 2/CCITT P.79, adjusted according to

Table 3/CCITT P.79.

Requirement

The SLR shall be 8 +/- 3dB.

II.11.1.3	Receiving sensitivity/frequency response

Definition

The receiving sensitivity frequency response is, as a function of the input test tone frequency, the ratio expressed in dB between the output sound pressure in the artificial ear and the input level, represented by the PCM bit stream at the Digital Audio Interface (DAI) (III. 6.4), required to obtain this.

Method of measurement

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

The SS sends a PCM bit stream equivalent to a pure tone level of -16dBmO (see NET 33) over the DAI (pin 25) to the MS.

Measurements are made at one-twelfth-octave intervals as given by the R40 series of preferred numbers in ISO 3 for frequencies from 100Hz to 4000Hz inclusive.

At each frequency, the sound pressure in the artificial ear (the ear reference point - ERP) is measured by connecting a suitable measuring set to the artificial ear.

Requirement

The receiving sensitivity/frequency response (from the DAI to the ERP) shall be within the mask given by the following table. The mask can be drawn with straight lines between the breaking points in the following table on a logarithmic (frequency) vs linear (dB sensitivity) scale.

All sensitivity levels are dB on an arbitrary scale.

┌────────────────┬──────────────────┬──────────────────┐

│ Frequency [Hz] │ Upper Limit [dB] │ Lower Limit [dB] │

├────────────────┼──────────────────┼──────────────────┤

│    a =  100    │      -12         │                  │

│    b =  200    │        0         │                  │

│    c =  300    │        2         │      -7          │

│    d =  500    │        *)        │      -5          │

│    e = 1000    │        0         │      -5          │

│    f = 3000    │        2         │      -5          │

│    g = 3400    │        2         │      -10         │

│    h = 4000    │        2         │                  │

└────────────────┴──────────────────┴──────────────────┘

Receiving sensitivity/frequency mask

*)  The limit at intermediate frequencies lies on a straight

line drawn between the given values on a log (frequency)

vs linear (dB) scale.

II.11.1.4	Receiving Loudness Rating

Definition

The Receiving Loudness Rating (RLR) is a means of expressing the receiving frequency response based on objective single tone measurements in a way which relates to how a speech signal would be perceived by a listener.

Method of measurement

The receiving sensitivity (II.11.1.3) is measured at each of the 14 frequencies listed in Table 2 of REC CCITT P.79, bands 4 to 17.

The sensitivity is expressed in terms of dBPa/V and the RLR is calculated according to formula 4.19c of REC CCITT P.79, over bands 4 to 17, using the receiving weighting factors from Table 2/CCITT P.79, adjusted according to Table 3/CCITT P.79. The artificial ear sensitivity must be corrected according to the real ear correction Table 4 of REC CCITT P.79.

Requirement

If no user controlled receive volume control is provided, the RLR shall be 2 +/- 3dB, (later referred as the nominal value).

If a user controlled receive volume control is provided, the RLR shall meet this value for at least one setting of the control.

When the control is set to maximum the RLR shall not be less than (ie louder than) -8dB.

II.11.1.5	Talker sidetone

II.11.1.5.1	Definition

The sidetone loudness ratings are a means of expressing the path loss from the artificial mouth to the artificial ear based on objective single tone measurements in a way that relates to how a speaker will perceive his own voice when speaking (talker sidetone, expressed by the sidetone masking rating - STMR), or how a listener will perceive the background noise picked up by the microphone (listener sidetone rating - LSTR).

Note:	"Listener sidetone" testing is not included in this test specification.

II.11.1.5.2	Method of measurement

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A".

The SS sends a PCM bit stream coded with the value No 1 over the DAI (pin 25) to the MS.

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51.

A pure tone with a sound pressure of -4.7dBPa shall be applied at the mouth reference point as described in REC CCITT P.64 using an artificial mouth conforming to REC CCITT P 51.

For each frequency given in Table 2 of REC CCITT P.79, bands 4 to 17, the sound pressure in the artificial ear is measured.

The sidetone path loss is expressed in dB and the STMR (in dB) is calculated from the formula 8.4 of REC CCITT P.79, using the weighting factors of column (3) in Table 6/CCITT P.79 (unsealed), and values of LE in accordance with Table 4/CCITT P.79.

II.11.1.5.3	Requirement

The STMR shall be  13 + 5dB.

Where a user controlled receive volume control is provided, the STMR shall meet the requirement given above at the setting where the RLR is equal to the nominal value.

Note:	It is recommended that the talker sidetone is independent of the volume control.

II.11.1.6	Acoustic shock

The receiving equipment shall limit the peak sound pressure.

No tone (eg alerting signal, supervisory tone or information tone) howsoever generated shall exceed this limit.

Conformance methods are for further study. Until such methods exist, compliance shall be checked by the suppliers declaration of conformance.

The alert module of a MS may use a transducer which is located in the handset. To protect the user against acoustic shock caused by ringing the following test is provided.

Method of measurement

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51.

By establishing a mobile terminated call the MS is made to activate its alert module.

The sound pressure in the artificial ear is measured.

Requirement

The maximum peak output sound pressure shall not exceed 30dBPa.

II.11.1.7	Telephone acoustic coupling loss (TAL)

II.11.1.7.1	Echo return loss (ERL)

Definition

The echo return loss is the path loss from the digital input of the reference speech encoder of the SS to the digital output of the reference speech decoder of the SS.

Method of measurement

The DAI of the MS is connected to the SS and is set to the operating mode "Normal operation".

A speech call is set up between MS and SS.

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51.

Where a user controlled volume control is provided it shall be set to maximum.

A digital signal generator delivering a signal equivalent to a pure tone level of 0dBm0 (see REC CCITT P.64) is connected to the digital input of the reference speech encoder of the SS.

Measurements are made at one-twelfth-octave intervals as given by the R40 series of preferred numbers in ISO 3 for frequencies from 300Hz to 3400Hz.

At each frequency the level equivalent to the digital signal at the reference speech decoder output of the SS is measured.

The echo return loss is calculated according to REC CCITT G.122.

Note:	The normal operation of the speech codec results in modulation of the amplitude of pure tone signals at a frequency that depends upon the frequency of the tone. This makes the measurement of TAL with pure tones uncertain. Choice of test signal is therefore important but, whilst a speech-like signal may be more suitable, insufficient information is available for a definitive choice.

Requirement

The echo return loss from the digital input to the digital output of the reference speech codec in the SS shall be at least 46 dB.

II.11.1.7.2	Stability margin

Definition

The receive-transmit stability margin is a measure of the gain that would have to be inserted between the go and return paths of the reference speech coder in the SS for oscillation to occur.

Method of measurement

A gain equivalent to the minimum stability margin is inserted in the loop between the go and return paths of the reference speech coder in the SS and any acoustic echo control is enabled.

A test signal according to REC CCITT O.131 is injected into the loop at the digital input of the reference speech codec of the SS and the stability is observed. The test signal shall have a level of -10dBmO and a duration

of 1s.

Where a user controlled volume control is provided it shall be set to maximum.

For handset operation the handset is placed on a hard plane surface with the transducers facing the surface.

For handsfree operation the test setup is shown in REC CCITT P.34

(Fig 3/CCITT P.34), but omitting the test table.

Requirement

The minimum stability margin shall be 6dB and no audible oscillation shall be detected.

II.11.1.8	Distortion

II.11.1.8.1	Sending

Definition

The transmit signal to total distortion ratio is a measure of the linearity of the transmitter equipment (excluding the speech codec).

Method of measurement

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

A sine-wave signal with a frequency in the range 1004Hz to 1025Hz is applied to the MRP. The level of this signal is adjusted until the PCM bitstream at the DAI output (pin 23) corresponds to -10dBmO. The level of the signal at the MRP is then the acoustic reference level (ARL).

The test signal is applied at the following levels: -35, -30, -25, -20,

-15, -10, -5, 0, 5, 10dB relative to the ARL.

The ratio of signal to total distortion power is measured at the DAI with the psophometric noise weighting (see RECs CCITT G.714 and O.132) at each signal level.

The measurement shall be carried out at sound pressures not exceeding +10dBPa.

Requirement

The ratio of signal to total distortion power measured at the DAI with the psophometric noise weighting (see Table 4/CCITT G.223) shall be above the limits given in the following table.

┌───────────────┬────────────┐

│     level     │  Level     │

│  dB relative  │  Ratio     │

│   to ARL      │            │

├───────────────┼────────────┤

│    -35  dB    │  17.5 dB   │

│    -30  dB    │  22.5 dB   │

│    -20  dB    │  30.7 dB   │

│    -10  dB    │  33.3 dB   │

│      0  dB    │  33.7 dB   │

│      7  dB    │  31.7 dB   │

│     10  dB    │  25.5 dB   │

└───────────────┴────────────┘

Limits for the signal to total distortion ratio

(sending) when using the sine wave method.

Limits for intermediate levels are found by drawing

a straight line between breaking points in a linear

(dB signal level) vs linear (dB ratio) scale.

II.11.1.8.2	Receiving

Definition

The receive signal to total distortion ratio is a measure of the linearity in the receive equipment.

Method of measurement

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

The SS sends via the DAI (Pin 25) a PCM bit stream simulating a sine-wave signal corresponding to REC CCITT O.132 at the following levels: -45, -40, -35, -30, -25, -20, -15, -10, -5, 0dBmO.

The ratio of signal to total distortion power is measured with the psophometric noise weighting in the artificial ear (see REC CCITT G.714) at each signal level.

The measurement shall be carried out at sound pressures between -50dBPa and +10dBPa.

Requirement

The ratio of signal to total distortion power measured at the artificial ear with the psophometric noise weighting (see Table 4/CCITT G.223) shall be above the limits given in the following table.

┌───────────────────┬─────────────┐

│       Level       │   Level     │

│  at the digital   │   Ratio     │

│  audio interface  │             │

├───────────────────┼─────────────┤

│     -45  dBm0     │   17.5 dB   │

│     -40  dBm0     │   22.5 dB   │

│     -30  dBm0     │   30.5 dB   │

│     -20  dBm0     │   33.0 dB   │

│     -10  dBm0     │   33.5 dB   │

│      -3  dBm0     │   31.2 dB   │

│       0  dBm0     │   25.5 dB   │

└───────────────────┴─────────────┘

Limits for the signal to total distortion

ratio (receiving) when using the sine wave method.

Limits for intermediate levels are found by drawing

a straight line between breaking points in a linear

(dB signal level) vs linear (dB ratio) scale.

II.11.1.8.3	Sidetone

Definition

The sidetone distortion expresses the linearity of the sidetone path in the handset.

Method of measurement

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A".

The SS sends the PCM bit stream coded with the value No 1 over the DAI

(pin 25) to the MS.

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51.

An instrument capable of measuring the third harmonic distortion of signals with fundamental frequencies in the range 315Hz to 1000Hz is connected to the artificial ear.

A pure-tone  signal of -4.7dBPa is applied at the mouth reference point at frequencies of 315Hz, 500Hz, and 1000Hz. For each frequency the third harmonic distortion is measured in the artificial ear.

Requirement

The third harmonic distortion generated shall not be greater than 10%.

II.11.1.9	Out-of-band signals

II.11.1.9.1	Sending

Definition

The discrimination against out-of-band input signals in the sending direction is a requirement on the in-band image frequencies created by any out-of-band input signals.

Method of measurement

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51.

A pure tone with a sound presure of -4.7 dBPa shall be applied at the mouth reference point as described in REC CCITT P.64 using an artificial mouth conforming to REC CCITT P 51.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

For input signals at frequencies of 4.65, 5, 6, 6.5, 7, and 7.5 kHz, the level represented by the PCM bit stream at the DAI (Pin 23) of any image frequency is measured.

Requirement

The level of any image frequency shall be below a reference obtained at

1 kHz by at least the amount as specified in the following table.

┌─────────────────────┬───────────────┐

│  Applied sine-wave  │    Limit      │

│  frequency          │   (minimum)   │

├─────────────────────┼───────────────┤

│      4.6 kHz        │    30 dB      │

│      8   kHz        │    40 dB      │

└─────────────────────┴───────────────┘

Limits for the image frequency discrimination

The limit at intermediate frequencies lies on a

straight line drawn between the given values on a

log(freqency) vs linear(dB) scale.

II.11.1.9.2	Receiving

Definition

The discrimination against out-of-band signals in the receiving direction is a requirement on the out-of-band signals generated in the artificial ear from in-band input signals.

Method of measurement

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

The SS sends over the DAI (pin 25) a PCM bit stream simulating a sine-wave signal with a level of 0dBmO .

For input signals at the nominal frequencies 500, 1000, 2000, and 3350Hz (bearing in mind the restriction on submultiples of the sampling frequency) the level of any out-of-band signals at frequencies up to 8kHz is measured in the artificial ear.

Requirement

The level of out-of-band signals shall be lower than the in-band acoustic level obtained by a digital signal at 1kHz set at the level specified in the following table.

┌─────────────────┬──────────────────────┐

│  Image Signal   │   Equivalent Input   │

│  frequency      │   Signal Level       │

├─────────────────┼──────────────────────┤

│    4.6 kHz      │     -35 dBmO         │

│    8   KHz      │     -45 dBmO         │

└─────────────────┴──────────────────────┘

Limits for the image frequency discrimination

The limit at intermediate frequencies lies on a

straight line drawn between the given values on

a log(frequency) vs linear(dB) scale.

II.11.1.10	Idle channel noise

II.11.1.10.1	Sending

Definition

The idle channel noise in the sending direction is the equivalent noise level produced at the DAI, when the mouth reference point is in a quiet environment.

Method of measurement

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51 in a quiet environment (ambient noise less than 30dBA).

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

The noise level represented by the PCM bit stream output at the DAI

(pin 23) is measured with psophometric weighting according to REC CCITT G.223, Table 4.

Requirement

The noise produced by the MS in the sending direction  shall not exceed

-64dBmOp.

II.11.1.10.2	Receiving

Definition

The idle channel noise in the receiving direction is the acoustic sound pressure in the artificial ear when the digital input signal at the DAI, is the PCM coded value No 1.

Method of measurement

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

The SS sends a PCM bit stream coded with the value No 1 over the DAI

(Pin 25) to the MS.

The level of the noise is measured in the artificial ear with any volume control set at the position at which the RLR is equal to the nominal value.

Requirement

The measured noise generated by the MS shall not exceed -57dBPa (A).

Where a volume control is provided, the measured noise shall also not exceed -54dBPa (A) at the maximum setting of the volume control.

II.11.2	    CATEGORY 2:  SHORT MESSAGE SERVICE (SMS)

Ref : GSM 03.14, 03.40, 04.11 (point to point)

GSM 03.41, 04.12 (cell broadcast)

II.11.2.1	GENERAL

The purpose of these tests is to verify the MS can handle GSM functions when submitting or receiving short messages between MS and a Short Message Service Centre as described in GSM 03.40.

The procedures are based upon services provided by the Mobility Management (MM) sublayer which is not tested in this case.

The SMS comprises three basic services which shall work in an active MS at any time independent of whether or not there is speech or data call in progress.

II.11.2.2	Short Message Service Point to Point

II.11.2.2.1	SMS Mobile Terminated

Purpose of test :

This test verifies the ability of a Mobile Station to receive and decode the SMS where provided for the Point to Point service.

Method of test :

a)	The Mobile Station shall be in idle mode.

b)	The System Simulator shall send an SM by setting up an SDCCH using the paging request, immediate assignment, authentication and ciphering procedures followed by a SABM frame with SAPI-3. When a UA frame (SAPI-3) is received in response, the System Simulator shall send a CP_DATA message. The information element of the CP_DATA message shall be RP_DATA.

c)	The System Simulator shall wait for the CP_ACK message and then the CP_DATA message containing the RP_ACK.

d)	The System Simulator shall send a CP_ACK to the Mobile Station within TC1M with no further CP-DATA messages and the System Simulator shall initiate channel release.

e)	Steps b), c) and d) shall be repeated but the first CP-DATA message from the Mobile Station shall not be acknowledged. The second CP-DATA message from the Mobile Station shall be acknowledged by a CP-ACK within a time TC1M.

f)	Steps b), c) and d) shall be repeated but the CP-DATA messages from the Mobile Station shall not be acknowledged.

g)	A data or speech call shall be established with the System Simulator and steps b) to f) shall be repeated. In this case the SACCH will be used in the setup procedure.

Requirements :

1)	In the case of step b) and c), the Mobile Station shall respond by sending a UA frame with SAPI-3 and then a CP-ACK message within TC1N.

2)	In the case of step c), the Mobile Station shall send a CP-DATA message containing the RP-ACK to the System Simulator within TR1N.

3)	In the case of step d), the Mobile Station shall return to idle mode. The Mobile Station shall indicate that a short message has arrived and provide a way to display the message.

4)	In the case of step e), the Mobile Station shall repeat once the CP-DATA message containing the RP-ACK within TR1N and then return to idle mode. The MS shall indicate that a short message has arrived and provide a way to display the message.

5)	In the case of step f), the Mobile Station shall repeat, once, the CP-DATA message containing the RP-ACK and then return to idle mode. The MS shall indicate that a short message has arrived and provide a way to display the message.

6)	In the case of step g), the requirements for steps b) to f) shall be satisfied.

Mobile Terminated Test Short Message content :

Information element              Comment                       Value

Message type indication                                        MT

More message to send                                            0

Originating address              an E164 number

Protocol identifier                                             0

Data coding scheme                                              0

Service Centre timestamp         see GSM 03.40

User Data length                                              160

User Data (140 octets)           160 characters (text of message)

II.11.2.2.2	SMS Mobile Originated

Purpose of test :

This test verifies the Mobile Station's ability to correctly send a short message where the SMS is provided for the Point to Point service. It also verifies the Mobile Station's capability to simultaneously receive a network originated SM whilst sending a mobile originated SM.

Method of test :

a)	The System Simulator shall be configured to support SMS and the Mobile Station shall be in idle mode.

b)	The Mobile Station shall be set up to send an SM to the System Simulator and the System Simulator shall respond by allocating an SDCCH, and then perform the normal authentication and ciphering procedures.

c)	The System Simulator shall respond with a UA frame SAPI-3 to the Mobile Station.

d)	The System Simulator shall respond with a CP-ACK message within TC1M followed by a CP-DATA message containing the correct RP-ACK.

e)	The System Simulator shall send a channel release message to the Mobile Station.

f)	Steps b) and c) shall be repeated with the System Simulator configured not to send the CP-ACK.

g)	Steps b) and c) shall be repeated with the System Simulator configured to send CP-ERROR within TC1M containing a "Network Failure" message as the cause.

h)	Steps b) to f) shall be repeated with the Mobile Station in data or speech call established mode.

i)	The System Simulator shall be configured to receive a mobile originated SM. Step b) shall be repeated and, using the end of the CP-DATA message from the Mobile Station as a trigger, the System Simulator shall send SM to the Mobile Station. In this case a new transaction identifier shall be used in the CP messages of SMS Mobile terminated.

j)	The System Simulator shall be configured not to support SMS and the Mobile Station shall set up a SM.

Requirements :

1)	In the case of step b), the Mobile Station shall perform a normal channel request procedure, indicating SMS in the request type. After SDCCH allocation, the Mobile Station shall perform normal authentication and ciphering procedures. Once the ciphering procedure is complete the Mobile Station shall send a SABM frame with SAPI-3.

2)	In the case of step c), the Mobile Station shall send the CP-DATA which shall be displayed as correct by the System Simulator.

3)	In the case of step d), the Mobile Station shall send a CP-ACK message within time TC1N and then shall return to the idle mode.

4)	In the case of step e), the Mobile Station shall respond with

a Disconnect message with SAPI-O.

5)	In the case of step f), the Mobile Station shall send the CP-DATA message and after time TC1M this CP-DATA message shall be repeated once only. The Mobile Station shall then return to idle mode.

6)	In the case of step g), the Mobile Station shall return to idle mode and may indicate the cause or error.

7)	In the case of step h), the requirements for steps a) to f) shall be satisfied. The SABM with SAPI = 3 shall be sent on the SACCH by the MS.

8)	In the case of step i), the Mobile Station shall correctly receive the short message and indicate that a message has arrived and provide a way to display the message. In the mobile originated case the Mobile Station shall send the CP-ACK message with transaction identifier assigned to this transfer. In the mobile terminated case the Mobile Station shall send a CP-ACK message and a CP-DATA message containing the RP-ACK. The transaction identifier shall be the same as the one chosen by the System Simulator for the mobile terminated transfer.

9)	In the case of step j), the Mobile Station shall return to idle mode.

Mobile Originated Test Short Message content :

Information element              Comment                      Value

Validity period                  Hours                         24

Destination address              an E164 number

Protocol identifier              all zeros                      0

Data coding scheme               standard alphabet              0

User data length                                              160

User Data (140 octets)           160 characters

(text of message)

II.11.2.3 	Short Message Service Cell Broadcast

Purpose of test :

This test verifies the ability of a Mobile Station in idle mode to correctly receive and decode the SMS as offered by the SMS Cell Broadcast Point to Multipoint (SMSCB) facility from PLMNs to Mobile Stations.

Method of test :

a)	The Mobile Station shall be in idle mode.

b)	The System Simulator shall provide a BCCH/CCCH to support the MS in idle mode.

c)	A SM shall be sent by the System Simulator on the Cell Broadcast Channel (CBCH) and the contents of the message shall use all the characters of the default 7-bit coded alphabet (see fig. GSM 03.41, Annex 1) filling all the available character positions of the possible 64 pages. The message identifier shall be set to the first reserved code for the GSM system.

d)	 Step c) shall be repeated for all other GSM reserved message identifier codes supported by the MS under test.

Requirements :

1)	In the case of step c), all characters for all pages sent by the SS shall be correctly decoded and fill the correct positions for the pages displayed by the MS.

2)	In the case of step d), the requirements for step c) shall apply for all GSM reserved identifier codes supported by the MS under test.

II.11.3     Category 3:  Message Handling Service

Ref: GSM 03.42

(Reserved.)

II.11.4     Category 4:  Videotex

Ref: GSM 03.43

(Reserved.)

II.11.5     Category 5:  Teletex

Ref: GSM 03.44

(Reserved.)

II.11.6     Category 6:  Facsimile

Ref: GSM 03.45 and 03.46

(Reserved.)