II.16	MOBILE STATION FEATURES

ref: GSM 02.07

Note:	GSM 02.07 shows mandatory and optional Mobile Station features. Mandatory Mobile Station features must be implemented in the MS and their implementation shall be verified in the process of type approval. From GSM 02.07, the following list of mandatory Mobile Station features is derived.

The following Mobile Station features shall be provided in the GSM Mobile Station. Their presence and appropriate functioning shall be verified.

II.16.1	Entry and display of called number

Purpose of test

To verify that the MS correctly accepts the entry and display of the called number.

Initial Condition

The MS must be registered in a cell of the SS.

Procedure

a)	A number (not including "+ function") is entered and then a call is set up.

b)	After the SS has accepted the call the number displayed on the MS and the number received in the SS are compared.

c)	The NPI and TON are examined in the SS.

d)	Steps a) to c) are repeated, but in a), the number entered starts with the "+ function"

Note:	This test may also be performed automatically using the EMMI.

Requirements

1)	In step b), both numbers shall be identical

2)	In step c), the NPI shall be "E164" and the TON shall be "unknown".

3)	In step d), the NPI shall be "E164" and the TON shall be "international".

II.16.2	Indication of call progress signals

ref: GSM 02.07 and GSM 02.40

II.16.2.1	Purpose of the tests

This test will verify the different call progress signals initiated by the

SS emulating the PLMN and the resulting announcements generated by the MS

if this feature is implemented. The feature, and thus the test is mandatory for an MS provided with a human interface.

It is acceptable that the tests be conducted by human comparison of the generated audio signals to a master tape copy. The pass criteria is then that the generated tones are easily recognisable.

II.16.2.2	Ringing tone

II.16.2.2.1	Method of test

a)	According to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to j).

II.16.2.2.2	Requirements

1)	According to section "Structured procedures, Mobile originating call, early assignment, Requirements", paragraphs 1) to 7).

2)	The ringing tone characteristics shall be as follows:

Tone               Frequency  Tolerance       Type

-------------------------------------------------------

Ringing tone         425 Hz     15 Hz       Periodic

Tone on 1 s,

Silence 4 s

II.16.2.3	Busy tone

II.16.2.3.1	Method of test

a)	According to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to h).

b)	The SS then sends message DISCONNECT with cause number 17.

Message  DISCONNECT (GSM 04.08, 9.3.7) to the MS:

Comment                 Value

Protocol Discriminator     CM                        001

Transaction Identifier     MS orig.                 1xxx

Message Type                                   0010 0101

Cause

- Coding standard          GSM                        11

- Location                 User                     0000

- Cause value              #17 "user busy"      001 0001

II.16.2.3.2	Requirements

1)	According to section "Structured procedures, Mobile originating call, early assignment, Requirements", paragraphs 1) to 5).

2)	After the reception of DISCONNECT a busy tone shall be generated. The busy tone characteristics shall be as follows:

Tone            Frequency  Tolerance       Type

---------------------------------------------------------

Busy tone         425 Hz     15 Hz       Periodic

Tone on 500 ms,

Silence 500 ms

II.16.2.4	Congestion tone

II.16.2.4.1	Method of test

a)	According to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to h).

b)	The SS then sends message DISCONNECT with cause number 42.

DISCONNECT message: As in II.16.2.3.1 with cause value #42 "Switching

equipment congestion" (0101010).

II.16.2.4.2	Requirements

1)	According to section "Structured procedures, Mobile originating call, early assignment, Requirements", paragraphs 1) to 5).

2)	After the reception of DISCONNECT a congestion tone shall be generated.

3)	The congestion tone characteristics shall be as follows:

Tone               Frequency  Tolerance       Type

------------------------------------------------------------

Congestion tone      425 Hz     15 Hz       Periodic

Tone on 200 ms,

Silence 200 ms

II.16.2.5	Authentication failure tone

II.16.2.5.1  Method of test

a)	According to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to e).

b)	After reception of message AUTHENTICATION RESPONSE the SS sends message AUTHENTICATION REJECT.

Message  AUTHENTICATION REJECT (GSM 04.08, 9.2.1) to the MS:

Comment                 Value

Protocol Discriminator     MM                       0101

Transaction Identifier     not relevant             1xxx

Message Type                                   0001 0001

II.16.2.5.2	Requirements

1)	According to section "Structured procedures, Mobile originating call, early assignment, Requirements", paragraphs 1) to 4).

2)	After reception of AUTHENTICATION REJECT a tone shall be generated indicating authentication failure.

3)	The authentication failure tone is the error/special information tone with characteristics as follows:

Tone               Frequency  Tolerance       Type

-----------------------------------------------------------

Error/Special        950 Hz     50 Hz       Triple tone

Information tone    1400 Hz     50 Hz       Tones on 330 ms

1800 Hz     50 Hz       Silence 1.0 s

II.16.2.6	Number unobtainable tone

II.16.2.6.1	Method of test

a)	According to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to h).

b)	The SS then sends message DISCONNECT with cause number 1.

DISCONNECT message: As in II.16.2.3.1 with cause value #1 "Unassigned

(unallocated) number" (0000001).

II.16.2.6.2	Requirements

1)	According to section "Structured procedures, Mobile originating call, early assignment, Requirements", paragraphs 1) to 5).

2)	After reception of DISCONNECT a tone shall be generated indicating that the called number is unobtainable.

The number unobtainable tone is the error/special information tone with characteristics as in II.16.2.5.2.

II.16.2.7	Call dropped tone

II.16.2.7.1	Method of test

a)	According to section "Structured procedures, Mobile originating call, early assignment, Method of test", paragraphs a) to l).

However, it shall be indicated in the system information messages that call re-establishment shall not be attempted (RACH control parameters).

b)	When the call has been established the SS stops transmitting on the TCH/SACCH.

II.16.2.7.2	Requirements

1)	According to section "Structured procedures, Mobile originating call, early assignment, Requirements", paragraphs 1) to 8).

2)	After the radio link timeout period has expired a tone shall be generated indicating that the call has been dropped.

The call dropped tone characteristics shall be as follows:

Tone               Frequency  Tolerance       Type

------------------------------------------------------------

Call dropped tone    425 Hz     15 Hz     Tone on 200 ms,

Silence 200 ms

3 bursts of on/off

II.16.3	Network Selection / Indication

II.16.3.1	Definition

Network selection and indication is defined in GSM 02.11.

Purpose of the tests

These tests verify the selection and indication of PLMNs by the MS.

II.16.3.2	Method of Test

Both procedures 1 and 2 are carried out.

PROCEDURE 1:  This procedure applies to the automatic mode for PLMN

selection.

PROCEDURE 2:  This procedure applies to the manual mode for PLMN selection.

PROCEDURE 1

1a)	The MS is set up with a SIM which contains, in the "PLMN selector" data field, a list of 3 PLMNs in the priority order PLMN2 (highest priority), PLMN3, PLMN4 (lowest priority).  PLMN1 is the Home PLMN of the MS as defined in the IMSI.  The "Forbidden PLMN" data field shall contain NULL values.

1b)	The SS transmits 4 BCCH carriers with the following parameters:

PLMN		Level dBµVemf( )

Carrier 1		PLMN1		28

Carrier 2		PLMN2		33

Carrier 3		PLMN3		38

Carrier 4		PLMN4		43

Each carrier has the "IMSI attach" (ATT) flag set in the BCCH data.  (The purpose of this is to force the MS to do Location Updating whenever it is switched on, so that the SS can determine which PLMN has been selected).

The other system information parameters are as in Table II.16.3.

1c)	The MS is switched on in automatic PLMN selection mode.

1d)	The SS checks that the MS sends a "Location updating request" on Carrier 1.

1e)	The SS sends a "Location updating accept" message to the MS on Carrier 1.  After 5s, the MS's "selected PLMN indicator" is checked.

1f)	The SS switches off Carriers 1 and 2.

1g)	The SS checks that the MS sends a "Location updating request" on Carrier 3.

Note:	Steps 1h) to 1j) test that the MS writes the forbidden PLMN to the correct location in the Test-SIM.  The procedure will be invalid if the "Forbidden PLMN" data field in the SIM does not contain null values at the start of the procedure.

1h)	The SS sends a "Location updating reject" message on Carrier 3 with the cause "PLMN not allowed".

1i)	The MS is turned off.  The SS switches on Carrier 4 again with the same parameters as in step 1b) above.  The MS is turned on in automatic PLMN selection mode.

1j)	The SS checks that the MS sends a "Location updating request" on Carrier 4.  (PLMN3 should not be selected because it should have been written to the SIM's "forbidden PLMN" data field in step 1h) above.)

1k)	The SS sends a "Location updating accept" message on Carrier 4.  After 5s, the MS's "selected PLMN indicator" is checked.

1l)	Carriers 1 and 2 are turned on with the same parameters as in step 1b) above.

1m)	The SS checks that the MS does not send a "Location updating request" on either Carrier 1 or 2.

1n)	Carrier 4 is turned off.  The SS then checks that the MS sends a "Location updating request" on Carrier 1.

1o)	The SS sends a "Location updating accept" message on Carrier 1.  After 5s, the MS's "selected PLMN indicator" is checked.

1p)	The MS is turned off and then turned on again in manual PLMN selection mode.

1q)	The SS checks that the MS sends a "Location updating request" on Carrier 1.

1r)	Carrier 1 is turned off.  After 30s, the indication on the MS of available PLMNs is checked.

1s)	PLMN3 is selected manually.

1t)	The SS checks that the MS sends a "Location updating request" on Carrier 3.  (Although PLMN3 is in the "forbidden PLMN" list, it should still be able to be selected when in manual mode).

1u)	The SS sends a "Location updating accept" message on Carrier 3.  After 5s, the MS's "selected PLMN indicator" is checked.

1v)	Carrier 3 is turned off.  The "PLMN search procedure" is initiated manually.  After 30s, the indication on MS of available PLMNs is checked.

1w)	The MS is turned off.  Carrier 2 is turned off and Carriers 3 and 4 are turned on with the parameters as in step 1b) above.  The MS is turned on in automatic PLMN selection mode.

1x)	The SS checks that the MS sends a "Location updating request" on Carrier 3. (PLMN 3 should have been deleted from the SIM's "forbidden PLMN" list in step 1u) above.)

1y)	The SS sends a "Location updating accept" message on Carrier 3.  After 5s, the MS's "selected PLMN indicator" is checked.

1z)	Carrier 3 is turned off. The SS checks that the MS sends a "Location updating request" on Carrier 4.

1aa)  The SS sends a "Location updating reject" message on Carrier 4 with the cause "Location Area not allowed".  The SS checks that the MS camps on Carrier 4 in accordance with GSM 05.08.

Requirements 1

Requirement 1.1) is mandatory for all MSs.  Requirements 1.2) and 1.3) only apply to MSs with a human interface.

1.1)	The MS shall make a response as indicated in steps 1d), 1g), 1j), 1m), 1q), 1t), 1x), 1z) and 1aa), above.  In all cases, the MS shall respond within 30s.

1.2)	The selected PLMN shall be indicated:

End of Step		  1e)    1k)    1o)    1u)    1y)

PLMN indicated:	PLMN1  PLMN4  PLMN1  PLMN3  PLMN3

1.3)	The following PLMNs shall be indicated as available:

Step 1r): PLMN2, PLMN3

Step 1v): PLMN2

PROCEDURE 2

2a)	The MS is set up with a SIM which contains, in the "PLMN selector" data field, a list of 3 PLMNs in the priority order PLMN2 (highest priority), PLMN3, PLMN4 (lowest priority).  PLMN1 is the Home PLMN of the MS as defined in the IMSI.  The "Forbidden PLMN" data field shall contain PLMN3.

2b)	The SS transmits 4 BCCH carriers with the following parameters:

PLMN		Level dBµVemf(  )

Carrier 1		PLMN1		28

Carrier 2		PLMN2		33

Carrier 3		PLMN3		38

Carrier 4		PLMN4		43

Each carrier has the "IMSI attach" (ATT) flag set in the BCCH data.  (The purpose of this is to force the MS to do Location Updating whenever it is switched on, so that the SS can determine which PLMN has been selected.)

The other system information parameters are as in Table II.16.3.

2c)	The MS is turned on in manual PLMN selection mode.

2d)	The SS checks that the MS sends a "Location updating request" on Carrier 1.

2e)	Carrier 1 is turned off.  After 30s, the indication on the MS of available PLMNs is checked.

2f)	PLMN3 is selected manually.

2g)	The SS checks that the MS sends a "Location updating request" on Carrier 3.  (Although PLMN3 is in the "forbidden PLMN" list, it should still be able to be selected when in manual mode).

2h)	The SS sends a "Location updating accept" message on Carrier 3.  After 5s, the MS's "selected PLMN indicator" is checked.

2i)	Carriers 3 and 4 are turned off.  The "PLMN search procedure" is initiated manually.  After 30s, the indication on MS of available PLMNs is checked.

2j)	PLMN2 is selected manually.

2k)	The SS checks that the MS sends a "Location updating request" on Carrier 2.

2l)	The SS sends a "Location updating reject" message on Carrier 2 with the cause "Location Area not allowed".  The SS checks that the MS camps on Carrier 2 in accordance with GSM 05.08.

Requirements 2

Requirement 2.1) is mandatory for all MSs.  Requirements 2.2) and 2.3) only apply to MSs with a human interface.

2.1)	The MS shall make a response as indicated in steps 2d), 2g), 2k) and 2l), as above. In all cases, the MS shall respond within 30s.

2.2)	The selected PLMN shall be indicated:

End of Step		  2h)

PLMN indicated:	PLMN3

2.3)	The following PLMNs shall be indicated as available:

Step 2e): PLMN2, PLMN3, PLMN4

Step 2i): PLMN2

Table  II.16.3 :  Normal System Information Fields

Parameter			    Reference in	Abbre-	Normal Setting

GSM 04.08	viation

________________________________________________________________________

Cell Channel Description	10.5.2.1	 -		Any values

Max retrans				10.5.2.17	 -		1

Tx-integer				10.5.2.17	 -		Any value

CELL_BAR_ACCESS			10.5.2.17	CBA		0 (i.e. no barred)

AC CN					10.5.2.17	AC		All 0

RE					10.5.2.17	RE		0 (i.e. re-estab-

lishment allowed)

BA ARFCN				10.5.2.13	BA		One entry equal

to the ARFCN of

the carrier

NCC					10.5.2.15	NCC		Any value

Cell Identity			10.5.1.1	 -		Any value

MCC, MNC				10.5.1.3	PLMN		Ref. II.16.3.2, 1b)

and  II.16.3.2, 2b)

LAC					10.5.1.3	LAC		1111 (Hex)

ATT, B_AG_BLKS_RES,T3212,					ATT = '1'

CCCH_CONF				10.5.2.8	 -		Other parameters

any values.

BS_PA_MFRMS				10.5.2.8	BPM		5 frames

Cell Options			10.5.2.3	 -		Any values

CELL_RESELECT_HYSTERESIS	10.5.2.4	CRH		10 dB

MS_TXPWR_MAX_CCH			10.5.2.4	MTMC		Maximum RF output

power of MS.

RXLEV_ACCESS_MIN			10.5.2.4	RAM		-95dBm

II.16.4	( RESERVED )

II.16.5	Invalid PIN indicator

Purpose of the test

This test verifies that an entry of an invalid PIN is indicated correctly.

Initial condition

The MS is in idle mode.

Note:	The test SIM which is used to perform this test, should not lock itself after a certain amount of incorrect entries.

Procedure:

a)	The SIM must be inserted.

b)	Several wrong PINs are entered.

Activation may be either manual or via the EMMI.

Requirement:

1)	The MS shall be equiped with a facility to indicate that an invalid PIN has been entered.

2)	For each incorrect PIN, the MS shall indicate that the code has been denied.

3)	When the correct PIN has been entered, the indication shall disappear.

II.16.6	Service indicator

Purpose of the test

This test verifies the service indicator.

Initial condition:

a)	The MS is in idle mode, unregistered.

b)	The SS shall emulate perfect radio conditions so that the MS is able to register and to set up or receive a call.

Procedure:

a)	The MS is brought in an active state by either switching it on or by inserting a SIM.

Requirements

1)	The successfull registration and the good condition shall be indicated by the MS indicator and by the SS.

II.16.7	Subscription identity management

The following test is mandatory.

Procedure

a)	A call is set up.

b)	The SIM is removed.

c)	The SS observes wether the MS performs IMSI detach.

d)	An attempt to establish a MO call is made (not an emergency call).

e)	An attempt to establish a MT call is made.

Requirements

1)	In step c), the MS shall perform an IMSI detach.

2)	In step d) the MS shall not initiate  to set up a new call via the Um interface.

3)	In step e), the MS shall not initiate to set up a new call via the Um interface.

II.16.8	Barring of outgoing calls

Purpose of the test

To verify that, if an MS is equiped with a local facility to bar outgoing calls, then this facility does not limit the possibility to establish an emergency call.

This test does not verify that the local facility to block outgoing calls really functions.

Procedure

a)	The local facility to bar outgoing calls is activated.

b)	Via MMI, the MS is actioned to establish an emergency call.

Requirements

1)	The MS shall establish an emergency call.

II.16.9	Prevention of unauthorized calls

If an MS can only be used by operating a key, or by using a keyword, then this shall not be required to establish an emergency call.

Purpose of the test

To verify that, if an MS is or can be configured/constructed such that it can only be used by operating a key, or by using a password, then this facility does not limit the possibility to establish an emergency call.

This test does not verify that operation of the MS is really limited and enabled only by the key or by the keyword.

Procedure

a)	The local facility to restrict operation such that the MS can only be operated by using a key or a keyword is activated. The most restrictive situation is created.

b)	Via MMI, the MS is actioned to establish an emergency call.

Requirements

1)	The MS shall establish an emergency call.