DRAFT

EUROPEAN	pr	ETS 300 607-1

TELECOMMUNICATION		August 1997

STANDARD		Sixth edition

Source: ETSI SMG		Reference: RE/SMG-07111OPR4-1

ICS:	33.020

Key words:	Digital cellular telecommunications system, Global System for Mobile Communication (GSM)

Digital cellular telecommunications system (Phase 2);

Mobile Station (MS) conformance specification;

Part 1: Conformance specification

(GSM 11.10-1 version 4.19.0)

ETSI

European Telecommunications Standards Institute

ETSI Secretariat

Postal address: F-06921 Sophia Antipolis CEDEX - FRANCE

Office address: 650 Route des Lucioles - Sophia Antipolis - Valbonne - FRANCE

X.400: c=fr, a=atlas, p=etsi, s=secretariat - Internet: secretariat@etsi.fr

Tel.: +33 4 92 94 42 00 - Fax: +33 4 93 65 47 16

Copyright Notification: No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media.

© European Telecommunications Standards Institute 1997. All rights reserved.

Whilst every care has been taken in the preparation and publication of this document, errors in content, typographical or otherwise, may occur. If you have comments concerning its accuracy, please write to "ETSI Editing and Standards Approval Dept." at the address shown on the title page.

Contents

Foreword	29

1	Scope	31

2	Normative references	31

3	Definitions, conventions, and applicability	37

3.1	Mobile station definition and configurations	37

3.2	Applicability	38

3.2.1	Applicability of this specification	38

3.2.2	Applicability of the individual tests	38

3.2.3	Applicability to terminal equipment	58

3.3	Definitions	58

3.4	Conventions for mathematical notations	59

3.4.1	Mathematical signs	59

3.4.2	Powers to the base 10	59

3.5	Conventions on electrical terms	59

3.5.1	Radio Frequency (RF) input signal level	59

3.5.2	Reference sensitivity level	59

3.5.3	Power level of fading signal	60

3.6	Terms on test conditions	60

3.6.1	Radio test conditions	60

4	Test Equipment	60

4.1	Terms used to describe test equipment in this ETS	60

4.2	Functional requirements of test equipment	61

5	Testing methodology in general (layers 1, 2, and 3)	61

5.1	Testing of optional functions and procedures	61

5.2	Test interfaces and facilities	61

5.3	Different protocol layers	62

5.4	Information to be provided by the apparatus supplier	62

5.5	Definitions of transmit and receive times	62

6	Reference test methods.	62

6.1	General	62

6.2	Choice of frequencies in the frequency hopping mode	62

6.3	"Ideal" radio conditions	63

6.4	Standard test signals.	63

6.5	Power (control) levels.	63

7	Implicit testing	63

8	Measurement uncertainty	63

9	Format of tests	64

10	Generic call set up procedures	64

10.1	Generic call set-up procedure for mobile terminating speech calls	64

10.1.1	Initial conditions	64

10.1.2	Definition of system information messages	65

10.1.3	Procedure	67

10.1.4	Specific message contents	68

10.2	Generic call set-up procedure for mobile originating speech calls	69

10.2.1	Initial conditions	69

10.2.2	Definition of system information messages	69

10.2.3	Procedure	70

10.2.4	Specific message contents	70

10.3	Generic call set-up procedure for mobile terminating data calls	72

10.3.1	Initial conditions	72

10.3.2	Definition of system information messages	72

10.3.3	Procedure	73

10.3.4	Specific message contents	73

10.4	Generic call set-up procedure for mobile originating data calls	75

10.4.1	Initial conditions	75

10.4.2	Definition of system information messages	75

10.4.3	Procedure	76

10.4.4	Specific message contents	76

11	General tests	79

11.1	Verification of support and non-support of services (multiple numbering scheme or ISDN)	79

11.1.1	Mobile Terminated (MT) calls	79

11.1.2	Mobile Originated (MO) calls	80

11.2	Verification of support of the single numbering scheme	81

11.3	Verification of non-support of services (Advice of Charge Charging (AoCC))	82

11.4	Verification of non-support of services (call hold)	83

11.5	Verification of non-support of services (multiparty)	84

11.6	Verification of non-support of feature (Fixed Dialling Number (FDN))	85

11.7	IMEI Security	86

11.8	Coding of the Bearer Capability information element	87

11.8.1	Network to MS Direction	88

11.8.1.1	BS 21 to 26 - Asynchronous Service	88

11.8.1.1.1	BS 21	88

11.8.1.1.2	BS 22	92

11.8.1.1.3	BS 24	92

11.8.1.1.4	BS 25	92

11.8.1.1.5	BS 26	92

11.8.1.1.6	BS 23	92

11.8.1.2	BS 31 to 34 - Synchronous Service	93

11.8.1.2.1	BS 32	93

11.8.1.2.2	BS 31	97

11.8.1.2.3	BS 33	98

11.8.1.2.4	BS 34	98

11.8.1.3	BS 61 - Alternate Speech / Data	98

11.8.1.3.1	Speech/Asynchronous Data, Transparent	99

11.8.1.3.2	Speech/Asynchronous Data, Non Transparent	101

11.8.1.3.3	Speech/Synchronous Data	103

11.8.1.4	BS 81 - Speech followed by Data	104

11.8.1.4.1	Speech followed by Asynchronous Data	104

11.8.1.4.2	Speech followed by Synchronous Data	104

11.8.1.5	TS 61 - Alternate Speech / Facsimile group 3	104

11.8.1.5.1	TS 61 - Alternate Speech / Facsimile group 3, Transparent	105

11.8.1.5.2	TS 61 - Alternate Speech / Facsimile group 3, Non-Transparent	106

11.8.1.6	TS 62 - Automatic Facsimile group 3	107

11.8.2	MS to SS direction	107

11.8.2.1	BS 21 to 26 - Asynchronous Service	107

11.8.2.1.1	BS 21	108

11.8.2.1.2	BS 22	112

11.8.2.1.3	BS 24	112

11.8.2.1.4	BS 25	112

11.8.2.1.5	BS 26	112

11.8.2.1.6	BS 23	112

11.8.2.2	BS 31 to 34 - Synchronous Service	113

11.8.2.2.1	BS 32	113

11.8.2.2.2	BS 31	117

11.8.2.2.3	BS 33	118

11.8.2.2.4	BS 34	118

11.8.2.3	BS 41 to 46 - PAD Access Asynchronous	119

11.8.2.3.1	2) BS 41	119

11.8.2.3.2	BS 42	120

11.8.2.3.3	BS 44	121

11.8.2.3.4	BS 45	121

11.8.2.3.5	BS 46	121

11.8.2.3.6	BS 43	121

11.8.2.4	BS 51 to 53 - Packet Service Synchronous	122

11.8.2.4.1	2) BS 51	122

11.8.2.4.2	BS 52	122

11.8.2.4.3	BS 53	122

11.8.2.5	BS 61 - Alternate Speech / Data	123

11.8.2.5.1	Speech/Asynchronous Data, Transparent	123

11.8.2.5.2	Speech/Asynchronous Data, Non Transparent	125

11.8.2.5.3	Speech/Synchronous Data	127

11.8.2.6	BS 81 - Speech followed by Data	128

11.8.2.6.1	Speech followed by Asynchronous Data	128

11.8.2.6.2	Speech followed by Synchronous Data	128

11.8.2.7	TS 61 - Alternate Speech / Facsimile group 3	128

11.8.2.7.1	TS 61 - Alternate Speech / Facsimile group 3, Transparent	129

11.8.2.7.2	TS 61 - Alternate Speech / Facsimile group 3, Non Transparent	130

11.8.2.8	TS 62 - Automatic Facsimile group 3	131

11.8.2.9	TS 11 and TS 12-  Speech	131

11.8.2.9.1	Support of only full/half rate speech version 1	131

11.8.2.9.2	Support of speech full rate version 2 (Enhanced Full Rate)	131

12	Transceiver	133

12.1	Conducted spurious emissions	133

12.1.1	MS allocated a channel	133

12.1.2	MS in idle mode	134

12.2	Radiated spurious emissions	136

12.2.1	MS allocated a channel	136

12.2.2	MS in idle mode	138

13	Transmitter	141

13.1	Frequency error and phase error	141

13.2	Frequency error under multipath and interference conditions	144

13.3	Transmitter output power and burst timing	146

13.4	Output RF spectrum	153

13.5	Intermodulation attenuation	159

14	Receiver	161

14.1	Bad frame indication	166

14.1.1	Bad frame indication - TCH/FS	166

14.1.1.1	Bad frame indication - TCH/FS - Random RF input	166

14.1.1.2	Bad frame indication - TCH/FS - Frequency hopping and downlink DTX	167

14.1.2	Bad frame indication - TCH/HS	169

14.1.2.1	Bad frame indication - TCH/HS - Random RF input	169

14.1.2.2	Bad frame indication - TCH/HS - Frequency hopping and downlink DTX	170

14.1.3	Bad frame indication - TCH/FS - Frequency hopping and downlink DTX - Phase 2 MS in a phase 1 network	171

14.1.4	Bad frame indication - TCH/HS - Frequency hopping and downlink DTX - Phase 2 MS in a phase 1 network	173

14.2	Reference sensitivity	175

14.2.1	Reference sensitivity - TCH/FS	175

14.2.2	Reference sensitivity - TCH/HS (Speech frames)	177

14.2.3	Reference sensitivity - FACCH/F	180

14.2.4	Reference sensitivity - FACCH/H	181

14.2.5	Reference sensitivity - full rate data channels	182

14.2.6	Reference sensitivity - half rate data channels	183

14.3	Usable receiver input level range	184

14.4	Co-channel rejection	185

14.4.1	Co-channel rejection - TCH/FS.	185

14.4.2	Co-channel rejection - TCH/HS	187

14.4.3	Co-channel rejection - TCH/HS (SID frames)	189

14.4.4	Co-channel rejection - FACCH/F	191

14.4.5	Co-channel rejection - FACCH/H	192

14.5	Adjacent channel rejection	193

14.5.1	Adjacent channel rejection - speech channels	193

14.5.2	Adjacent channel rejection - control channels	195

14.6	Intermodulation rejection	197

14.6.1	Intermodulation rejection - speech channels	197

14.6.2	Intermodulation rejection - control channels	199

14.7	Blocking and spurious response	201

14.7.1	Blocking and spurious response - speech channels	201

14.7.2	Blocking and spurious response - control channels	204

14.8	AM suppression	207

14.8.1	AM suppression - speech channels	207

14.8.2	AM suppression - control channels	208

15	Timing advance and absolute delay	211

16	Reception time tracking speed	213

17	Access times during handover	215

17.1	Intra cell channel change	215

17.2	Inter cell handover	217

18	Temporary reception gaps	220

19	Channel release after unrecoverable errors	222

19.1	Channel release after unrecoverable errors -1	222

19.2	Channel release after unrecoverable errors - 2	223

19.3	Channel release after unrecoverable errors - 3	224

20	Cell selection and reselection	226

20.1	Cell selection	227

20.2	Cell selection with varying signal strength values	229

20.3	Basic cell reselection	231

20.4	Cell reselection using TEMPORARY_OFFSET, CELL_RESELECT_OFFSET, POWER_OFFSET and PENALTY_TIME parameters	233

20.5	Cell reselection using parameters transmitted in the System Information type 2bis, type 2ter, type 7 and type 8 messages.	235

20.5.5	Test Requirements	236

20.6	Cell reselection timings	237

20.6.1	Definition and applicability	237

20.6.2	Conformance requirement	237

20.6.3	Test purpose	237

20.6.4	Method of test	237

20.6.4.1	Initial conditions	237

20.6.4.2	Procedure	237

20.6.5	Test requirements	238

20.7	Priority of cells	238

20.7.4.2	Procedure	239

20.7.5	Test requirements	240

20.8	Cell reselection when C1 (serving cell) < 0 for 5 seconds	240

20.8.1	Definition and applicability	240

20.8.2	Conformance requirement	240

20.8.3	Test purpose	240

20.8.4	Method of test	240

20.8.4.1	Initial conditions	240

20.8.4.2	Procedure	240

20.8.5	Test requirements	241

20.9	Running average of the surrounding cell BCCH carrier signal levels	241

20.9.1	Definition and applicability	241

20.9.2	Conformance requirement	241

20.9.3	Test purpose	241

20.9.4	Method of test	241

20.9.4.1	Initial conditions	241

20.9.4.2	Procedure	242

20.9.5	Test requirements	242

20.10	Running average of the serving cell BCCH carrier signal level	242

20.10.1	Definition and applicability	242

20.10.2	Conformance requirement	242

20.10.3	Test purpose	242

20.10.4	Method of test	242

20.10.4.1	Initial conditions	242

20.10.4.2	Procedure	243

20.10.5	Test requirement	243

20.11	Updating the list of six strongest neighbour carriers and decoding the BCCH information of a new carrier on the list	243

20.11.1	Definition and applicability	243

20.11.2	Conformance requirement	243

20.11.3	Test purpose	243

20.11.4	Method of test	244

20.11.4.1	Initial conditions	244

20.11.4.2	Procedure	244

20.11.5	Test requirements	244

20.12	Decoding the BCCH information of the neighbour carriers on the list of six strongest neighbour carriers	244

20.12.1	Definition and applicability	244

20.12.2	Conformance requirement	244

20.12.3	Test purpose	244

20.12.4	Method of test	245

20.12.4.1	Initial conditions	245

20.12.4.2	Procedure	245

20.12.5	Test requirements	245

20.13	Decoding the BSIC of the neighbour carriers on the list of six strongest neighbour carriers	245

20.13.1	Definition and applicability	245

20.13.2	Conformance requirement	245

20.13.3	Test purpose	245

20.13.4	Method of test	246

20.13.4.1	Initial conditions	246

20.13.4.2	Procedure	246

20.13.5	Test requirements	246

20.14	Emergency calls	246

20.14.1	Definition and applicability	246

20.14.2	Conformance requirement	246

20.14.3	Test purpose	247

20.14.4	Method of test	247

20.14.4.1	Initial conditions	247

20.14.4.2	Procedure	247

20.14.5	Test requirements	247

20.15	Cell reselection due to MS rejection "LA not allowed"	248

20.15.1	Definition and applicability	248

20.15.2	Conformance requirement	248

20.15.3	Test purpose	248

20.15.4	Method of test	248

20.15.4.1	Initial conditions	248

20.15.4.2	Procedure	249

20.15.5	Test requirements	249

20.16	Downlink signalling failure	249

20.16.1	Definition and applicability	249

20.16.2	Conformance requirement	249

20.16.3	Test purpose	250

20.17	Cell selection if no suitable cell found in 10 s	251

20.18	Cell reselection due to MS rejection "Roaming not allowed in this LA"	252

20.18.5	Test requirements	253

20.19	Cell selection on release of SDCCH and TCH	253

20.19.1	Definition and applicability	253

20.19.2	Conformance requirement	253

20.19.3	Test purpose	254

20.19.4	Method of test	254

20.19.4.1	Initial conditions	254

20.19.4.2	Test procedure	254

20.19.5	Test requirements	254

20.20	Multiband cell selection and reselection	255

20.20.1	Multiband cell selection and reselection / Cell Selection	255

20.20.2	Multiband cell selection and reselection / Cell reselection	257

21	Received signal measurements	259

21.1	Signal strength	259

21.2	Signal strength selectivity	262

21.3	Signal quality under static conditions	263

21.3.1	Signal quality under static conditions - TCH/FS	263

21.3.2	Signal quality under static conditions - TCH/HS	265

21.3.2.4.2	Procedure	266

21.4	Signal quality under TU50 propagation conditions	267

21.4.4.1	Initial conditions	268

21.4.4.2	Procedure	268

22	Transmit power control timing and confirmation	270

23	Single frequency reference	271

24	Tests of the layer 1 signalling functions	272

25	Tests of the layer 2 signalling functions	273

25.1	Introduction, objective and scope	273

25.1.1	General	273

25.1.2	Test configurations	273

25.1.3	Pre-conditions	273

25.1.4	Layer 2 test frames	274

25.1.5	Establishment of the dedicated physical resource	274

25.1.6	Release of the dedicated physical resource	275

25.2	Test sequences	275

25.2.1	Initialization	276

25.2.1.1	Initialization when contention resolution required	276

25.2.1.1.1	Normal initialization	276

25.2.1.1.2	Initialization failure	277

25.2.1.1.2.1	Loss of UA frame	277

25.2.1.1.2.2	UA frame with different information field	278

25.2.1.1.2.3	Information frame and supervisory frames in response to an SABM frame	279

25.2.1.1.3	Initialization denial	279

25.2.1.1.4	Total initialization failure	280

25.2.1.2	Initialization, contention resolution not required	281

25.2.1.2.1	Normal initialization without contention resolution	281

25.2.1.2.2	Initialization failure	282

25.2.1.2.3	Initialization denial	284

25.2.1.2.4	Total initialization failure	284

25.2.2	Normal information transfer	285

25.2.2.1	Sequence counting and I frame acknowledgements	285

25.2.2.2	Receipt of an I frame in the timer recovery state	288

25.2.2.3	Segmentation and concatenation	290

25.2.3	Normal layer 2 disconnection	293

25.2.4	Test of link failure	293

25.2.4.1	I frame loss (MS to SS)	293

25.2.4.2	RR response frame loss (SS to MS)	294

25.2.4.3	RR response frame loss (MS to SS)	295

25.2.5	Test of frame transmission with incorrect C/R values	296

25.2.5.1	I frame with C bit set to zero	296

25.2.5.2	SABM frame with C bit set to zero	297

25.2.6	Test of errors in the control field	298

25.2.6.1	N(S) sequence error	298

25.2.6.2	N(R) sequence error	300

25.2.6.3	Improper F bit	300

25.2.7	Test on receipt of invalid frames	301

26	Testing of layer 3 functions	306

26.1	Default conditions and structured sequence of tests	306

26.1.1	Default test conditions during layer 3 tests	306

26.1.2	Structured sequence of the tests	307

26.1.3	General rules for message parameters	308

26.1.4	General rules for layer 3 testing	308

26.1.5	Format of layer 3 test descriptions	308

26.2	Initial tests	310

26.2.1	Channel request	310

26.2.1.1	Channel request / initial time	310

26.2.1.2	Channel request / repetition time	311

26.2.1.3	Channel request / random reference	313

26.2.2	IMSI detach and IMSI attach	314

26.2.3	Sequenced MM / CM message transfer	318

26.2.4	Establishment cause	319

26.3	Test of MS functions in idle mode	328

26.3.1	Initial conditions	328

26.3.2	MS indication of available PLMNs	330

26.3.3	MS will send only if BSS is "on air"	331

26.3.4	Manual mode of PLMN selection	331

26.4	Lower layer failures in layer 3 testing	333

26.4.1	Introduction	333

26.4.2	Layer 1 reception failures	333

26.4.3	Data link layer failures	333

26.4.4	Lower layer failures, used for the tests in clause 25	333

26.5	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions	334

26.5.1	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / unknown protocol discriminator	334

26.5.2	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / TI and skip indicator	335

26.5.2.1	TI and skip indicator / RR	335

26.5.2.1.1	TI and skip indicator / RR / Idle Mode	335

26.5.2.1.2	TI and skip indicator / RR / RR-Connection established	336

26.5.2.2	TI and skip indicator / MM	337

26.5.2.3	TI and skip indicator / CC	339

26.5.3	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / undefined or unexpected message type	341

26.5.3.1	Undefined or unexpected message type / undefined message type / CC	341

26.5.3.2	Undefined or unexpected message type / undefined message type / MM	342

26.5.3.3	Undefined or unexpected message type / undefined message type / RR	344

26.5.3.4	Undefined or unexpected message type / unexpected message type / CC	345

26.5.4	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / unforeseen information elements in the non-imperative message part	346

26.5.4.1	Unforeseen information elements in the non-imperative message part / duplicated information elements	346

26.5.5	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / non-semantical mandatory IE errors	348

26.5.5.1	Non-semantical mandatory IE errors / RR	348

26.5.5.1.1	Non-semantical mandatory IE errors / RR / missing mandatory IE error	348

26.5.5.1.1.1	Non-semantical mandatory IE errors / RR / missing mandatory IE error / special case	348

26.5.5.1.1.2	Non-semantical mandatory IE errors / RR / missing mandatory IE error / general case	349

26.5.5.1.2	Non-semantical mandatory IE errors / RR / comprehension required	350

26.5.5.2	Non-semantical mandatory IE errors / MM	351

26.5.5.2.1	Non-semantical mandatory IE errors / MM / syntactically incorrect mandatory IE	351

26.5.5.2.2	Non-semantical mandatory IE errors / MM / syntactically incorrect mandatory IE	352

26.5.5.2.3	Non-semantical mandatory IE errors / MM / comprehension required	354

26.5.5.3	Non-semantical mandatory IE errors / CC	356

26.5.5.3.1	Non-semantical mandatory IE errors / CC / missing mandatory IE	356

26.5.5.3.1.1	Non-semantical mandatory IE errors / CC / missing mandatory IE / disconnect message	356

26.5.5.3.1.2	Non-semantical mandatory IE errors / CC / missing mandatory IE / general case	357

26.5.5.3.2	Non-semantical mandatory IE errors / CC / comprehension required	358

26.5.6	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / unknown IE, comprehension not required	359

26.5.6.1	Unknown information elements in the non-imperative message part / MM	359

26.5.6.1.1	Unknown IE, comprehension not required / MM / IE unknown in the protocol	359

26.5.6.1.2	Unknown IE, comprehension not required / MM / IE unknown in the message	361

26.5.6.2	Unknown information elements in the non-imperative message part / CC	362

26.5.6.2.1	Unknown information elements in the non-imperative message part / CC / Call establishment	362

26.5.6.2.2	Unknown information elements in the non-imperative message part / CC / disconnect	363

26.5.6.2.3	Unknown information elements in the non-imperative message part / CC / release	364

26.5.6.2.4	Unknown information elements in the non-imperative message part / CC / release complete	365

26.5.6.3	Unknown IE in the non-imperative message part, comprehension not required / RR	366

26.5.7	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / spare bits	368

26.5.7.1	Spare bits / RR	368

26.5.7.1.1	Spare bits / RR / paging channel	368

26.5.7.1.2	Spare bits / RR / BCCH	369

26.5.7.1.3	Spare bits / RR / AGCH	371

26.5.7.1.4	Spare bits / RR / Connected Mode	373

26.5.7.2	Spare bits / MM	375

26.5.7.3	Spare bits / CC	376

26.5.8	Default contents of messages	379

26.6	Test of the elementary procedures for radio resource management	382

26.6.1	Immediate assignment	382

26.6.1.1	Immediate assignment / SDCCH or TCH assignment	382

26.6.1.2	Immediate assignment / extended assignment	383

26.6.1.3	Immediate assignment / assignment rejection	385

26.6.1.4	Immediate assignment / ignore assignment	387

26.6.1.5	Immediate assignment after immediate assignment reject	389

26.6.2	Test of paging	390

26.6.2.1	Normal paging	390

26.6.2.1.1	Paging / normal / type 1	390

26.6.2.1.2	Paging / normal / type 2	392

26.6.2.1.3	Paging / normal / type 3	394

26.6.2.2	Paging / extended	396

26.6.2.3	Paging / reorganization	400

26.6.2.3.1	Paging / reorganization / procedure 1	400

26.6.2.3.2	Paging / reorganization / procedure 2	403

26.6.2.4	Paging / same as before	404

26.6.2.5	Paging / multislot CCCH	405

26.6.3	Test of measurement report	406

26.6.3.1	Measurement / no neighbours	407

26.6.3.2	Measurement / all neighbours present	410

26.6.3.3	Measurement / barred cells and non-permitted NCCs	414

26.6.3.4	Measurement / DTX	419

26.6.3.5	Measurement / Frequency Formats	423

26.6.3.6	Measurement / multiband environment	426

26.6.4	Test of the channel assignment procedure	431

26.6.4.1	Dedicated assignment / successful case	431

26.6.4.2	Dedicated assignment / failure	442

26.6.4.2.1	Dedicated assignment / failure / failure during active state	442

26.6.4.2.2	Dedicated assignment / failure / general case	443

26.6.5	Test of handover	445

26.6.5.1	Handover / successful / active call / non-synchronized	446

26.6.5.2	Handover / successful / call under establishment / non-synchronized	461

26.6.5.3	Handover / successful / active call / finely synchronized	479

26.6.5.4	Handover / successful / call under establishment / finely synchronized	484

26.6.5.5	Pre-synchronized handovers	494

26.6.5.5.1	Handover / successful / active call / pre-synchronized / Timing Advance IE not included	494

26.6.5.5.2	Handover / successful / call being established / pre-synchronized / timing advance IE is included / reporting of observed time difference requested.	496

26.6.5.6	Handover / successful / active call / pseudo synchronized	498

26.6.5.7	Handover / successful / active call / non-synchronized / reporting of observed time difference requested.	500

26.6.5.8	Handover / layer 3 failure	501

26.6.5.9	Handover / layer 1 failure	503

26.6.6	Test of frequency redefinition	504

26.6.6.1	Frequency redefinition	504

26.6.7	Test of the channel mode modify procedure	509

26.6.7.1	Test of the channel mode modify procedure / full rate	510

26.6.7.2	Test of the channel mode modify procedure / half rate	512

26.6.8	Test of ciphering mode setting	514

26.6.8.1	Ciphering mode / start ciphering	514

26.6.8.2	Ciphering mode / no ciphering	515

26.6.8.3	Ciphering mode / old cipher key	517

26.6.8.4	Ciphering mode / change of mode, algorithm and key	518

26.6.8.5	Ciphering mode / IMEISV request	526

26.6.9	Test of additional assignment	528

26.6.10	Test of partial release	528

26.6.11	Test of classmark	528

26.6.11.1	Classmark change	528

26.6.11.2	Classmark interrogation	530

26.6.12	Test of channel release	532

26.6.12.1	Channel release / SDCCH	532

26.6.12.2	Channel release / SDCCH - no L2 ACK	533

26.6.12.3	Channel release / TCH-F	535

26.6.12.4	Channel release / TCH-F - no L2 ACK	536

26.6.13	Test of starting time	537

26.6.13.1	Dedicated assignment with starting time / successful case / time not elapsed	539

26.6.13.2	Dedicated assignment with starting time / successful case / time elapsed	540

26.6.13.3	Dedicated assignment with starting time and frequency redefinition / failure case / time not elapsed	542

26.6.13.4	Dedicated assignment with starting time and frequency redefinition / failure case / time elapsed	545

26.6.13.5	Handover with starting time / successful case / time not elapsed	547

26.6.13.6	Handover with starting time / successful case / time elapsed	549

26.6.13.7	Handover with starting time and frequency redefinition / failure case / time not elapsed	551

26.6.13.8	Handover with starting time and frequency redefinition / failure case / time elapsed	554

26.6.13.9	Immediate assignment with starting time / successful case / time not elapsed	556

26.6.13.10	Immediate assignment with starting time / successful case / time elapsed	558

26.6.14	Default contents of GSM 900 layer 3 messages for RR tests	560

26.6.15	Default contents of DCS 1 800 layer 3 messages for RR tests	571

26.7	Elementary procedures of mobility management	583

26.7.1	TMSI reallocation	583

26.7.2	Authentication	586

26.7.2.1	Authentication accepted	586

26.7.2.2	Authentication rejected	587

26.7.3	Identification	590

26.7.3.1	General Identification	590

26.7.3.2	Handling of IMSI shorter than the maximum length	592

26.7.4	Location updating	595

26.7.4.1	Location updating / accepted	595

26.7.4.2	Location updating / rejected	598

26.7.4.2.1	Location updating / rejected / IMSI invalid	598

26.7.4.2.2	Location updating / rejected / PLMN not allowed	601

26.7.4.2.3	Location updating / rejected / location area not allowed	605

26.7.4.2.4	Location updating / rejected / roaming not allowed in this location area	608

26.7.4.3	Location updating / abnormal cases	615

26.7.4.3.1	Location updating / abnormal cases / random access fails	615

26.7.4.3.2	Location updating / abnormal cases / attempt counter less or equal to 4, LAI different	617

26.7.4.3.3	Location updating / abnormal cases / attempt counter equal to 4	623

26.7.4.3.4	Location updating / abnormal cases / attempt counter less or equal to 4, stored LAI equal to broadcast LAI	631

26.7.4.4	Location updating / release / expiry of T3240	639

26.7.4.5	Location updating / periodic	640

26.7.4.5.1	Location updating / periodic spread	640

26.7.4.5.2	Location updating / periodic normal / test 1	642

26.7.4.5.3	Location updating / periodic normal / test 2	644

26.7.4.5.4	Location updating / periodic HPLMN search	647

26.7.4.5.4.1	Location updating / periodic HPLMN search / MS waits time T	647

26.7.4.5.4.2	Location updating / periodic HPLMN search / MS in manual mode	648

26.7.4.5.4.3	Location updating / periodic HPLMN search / MS waits at least two minutes and at most T minutes	649

26.7.4.6	Location updating / interworking of attach and periodic	651

26.7.5	MM connection	653

26.7.5.1	Introduction	653

26.7.5.2	MM connection / establishment with cipher	653

26.7.5.3	MM connection / establishment without cipher	655

26.7.5.4	MM connection / establishment rejected	656

26.7.5.5	MM connection / establishment rejected cause 4	657

26.7.5.6	MM connection / expiry T3230	658

26.7.5.7	MM connection / abortion by the network	659

26.7.5.7.1	MM connection / abortion by the network / cause #6	659

26.7.5.7.2	MM connection / abortion by the network / cause not equal to #6	662

26.7.5.8	MM connection / follow-on request pending	663

26.7.5.8.1	MM connection / follow-on request pending / test 1	663

26.7.5.8.2	MM connection / follow-on request pending / test 2	664

26.7.5.8.3	MM connection / follow-on request pending / test 3	665

26.7.6	Default contents of messages	667

26.8	Tests related to circuit switched call control	671

26.8.1	Circuit switched Call Control (CC) state machine verification	671

26.8.1.1	General on CC state machine verification	671

26.8.1.2	Establishment of an outgoing call	672

26.8.1.2.1	Outgoing call / U0 null state	674

26.8.1.2.1.1	Outgoing call / U0 null state / MM connection requested	674

26.8.1.2.2	Outgoing call / U0.1 MM connection pending	675

26.8.1.2.2.1	Outgoing call / U0.1 MM connection pending / CM service rejected	675

26.8.1.2.2.2	Outgoing call / U0.1 MM connection pending / CM service accepted	676

26.8.1.2.2.3	Outgoing call / U0.1 MM connection pending / lower layer failure	678

26.8.1.2.3	Outgoing call / U1 call initiated	679

26.8.1.2.3.1	Outgoing call / U1 call initiated / receiving CALL PROCEEDING	679

26.8.1.2.3.2	Outgoing call / U1 call initiated / rejecting with RELEASE COMPLETE	680

26.8.1.2.3.3	Outgoing call / U1 call initiated / T303 expiry	682

26.8.1.2.3.4	Outgoing call / U1 call initiated / lower layer failure	683

26.8.1.2.3.5	Outgoing call / U1 call initiated / receiving ALERTING	684

26.8.1.2.3.6	Outgoing call / U1 call initiated / entering state U10	685

26.8.1.2.3.7	Outgoing call / U1 call initiated / unknown message received	686

26.8.1.2.4	Outgoing call / U3 MS originating call proceeding	687

26.8.1.2.4.1	Outgoing call / U3 MS originating call proceeding / ALERTING received	687

26.8.1.2.4.2	Outgoing call / U3 MS originating call proceeding / CONNECT received	688

26.8.1.2.4.3	Outgoing call / U3 MS originating call proceeding / PROGRESS received without in band information	690

26.8.1.2.4.4	Outgoing call / U3 MS originating call proceeding / PROGRESS with in band information	691

26.8.1.2.4.5	Outgoing call / U3 MS originating call proceeding / DISCONNECT with in band tones	693

26.8.1.2.4.6	Outgoing call / U3 MS originating call proceeding / DISCONNECT without in band tones	694

26.8.1.2.4.7	Outgoing call / U3 MS originating call proceeding / RELEASE received	695

26.8.1.2.4.8	Outgoing call / U3 MS originating call proceeding / termination requested by the user	696

26.8.1.2.4.9	Outgoing call / U3 MS originating call proceeding / traffic channel allocation	698

26.8.1.2.4.10	Outgoing call / U3 MS originating call proceeding / timer T310 time-out	699

26.8.1.2.4.11	Outgoing call / U3 MS originating call proceeding / lower layer failure	700

26.8.1.2.4.12	Outgoing call / U3 MS originating call proceeding / unknown message received	701

26.8.1.2.4.13	Outgoing call / U3 MS originating call proceeding / Internal alerting indication	702

26.8.1.2.5	Outgoing call / U4 call delivered	703

26.8.1.2.5.1	Outgoing call / U4 call delivered / CONNECT received	703

26.8.1.2.5.2	Outgoing call / U4 call delivered / termination requested by the user	705

26.8.1.2.5.3	Outgoing call / U4 call delivered / DISCONNECT with in band tones	706

26.8.1.2.5.4	Outgoing call / U4 call delivered / DISCONNECT without in band tones	707

26.8.1.2.5.5	Outgoing call / U4 call delivered / RELEASE received	708

26.8.1.2.5.6	Outgoing call / U4 call delivered / lower layer failure	709

26.8.1.2.5.7	Outgoing call / U4 call delivered / traffic channel allocation	711

26.8.1.2.5.8	Outgoing call / U4 call delivered / unknown message received	712

26.8.1.2.6	U10 call active	713

26.8.1.2.6.1	U10 call active / termination requested by the user	713

26.8.1.2.6.2	U10 call active / RELEASE received	714

26.8.1.2.6.3	U10 call active / DISCONNECT with in band tones	715

26.8.1.2.6.4	U10 call active / DISCONNECT without in band tones	717

26.8.1.2.6.5	U10 call active / RELEASE COMPLETE received	718

26.8.1.2.6.6	U10 call active / SETUP received	719

26.8.1.2.7	U11 disconnect request	721

26.8.1.2.7.1	U11 disconnect request / clear collision	721

26.8.1.2.7.2	U11 disconnect request / RELEASE received	722

26.8.1.2.7.3	U11 disconnect request / timer T305 time-out	723

26.8.1.2.7.4	U11 disconnect request / lower layer failure	725

26.8.1.2.7.5	U11 disconnect request / unknown message received	726

26.8.1.2.8	U12 disconnect indication	727

26.8.1.2.8.1	U12 disconnect indication / call releasing requested by the user	727

26.8.1.2.8.2	U12 disconnect indication / RELEASE received	728

26.8.1.2.8.3	U12 disconnect indication / lower layer failure	729

26.8.1.2.8.4	U12 disconnect indication / unknown message received	731

26.8.1.2.9	Outgoing call / U19 release request	732

26.8.1.2.9.1	Outgoing call / U19 release request / timer T308 time-out	732

26.8.1.2.9.2	Outgoing call / U19 release request / 2nd timer T308 time-out	733

26.8.1.2.9.3	Outgoing call / U19 release request / RELEASE received	735

26.8.1.2.9.4	Outgoing call / U19 release request / RELEASE COMPLETE received	736

26.8.1.2.9.5	Outgoing call / U19 release request / lower layer failure	737

26.8.1.3	Establishment of an incoming call / Initial conditions	738

26.8.1.3.1	Incoming call / U0 null state	741

26.8.1.3.1.1	Incoming call / U0 null state / SETUP received with a non supported bearer capability	741

26.8.1.3.2.	Incoming call / U6 call present	742

26.8.1.3.2.1	Incoming call / U6 call present / automatic call rejection	742

26.8.1.3.3	Incoming call / U9 mobile terminating call confirmed	744

26.8.1.3.3.1	Incoming call / U9 mobile terminating call confirmed / alerting or immediate connecting	744

26.8.1.3.3.2	Incoming call / U9 mobile terminating call confirmed / TCH assignment	745

26.8.1.3.3.3	Incoming call / U9 mobile terminating call confirmed / termination requested by the user	746

26.8.1.3.3.4	Incoming call / U9 mobile terminating call confirmed / DISCONNECT received	747

26.8.1.3.3.5	Incoming call / U9 mobile terminating call confirmed / RELEASE received	749

26.8.1.3.3.6	Incoming call / U9 mobile terminating call confirmed / lower layer failure	750

26.8.1.3.3.7	Incoming call / U9 mobile terminating call confirmed / unknown message received	751

26.8.1.3.4	Incoming call / U7 call received	752

26.8.1.3.4.1	Incoming call / U7 call received / call accepted	752

26.8.1.3.4.2	Incoming call / U7 call received / termination requested by the user	754

26.8.1.3.4.3	Incoming call / U7 call received / DISCONNECT received	755

26.8.1.3.4.4	Incoming call / U7 call received / RELEASE received	756

26.8.1.3.4.5	Incoming call / U7 call received / lower layer failure	757

26.8.1.3.4.6	Incoming call / U7 call received / unknown message received	759

26.8.1.3.4.7	Incoming call / U7 call received / TCH assignment	760

26.8.1.3.4.8	Incoming call / U7 call received / RELEASE COMPLETE received	761

26.8.1.3.5	Incoming call / U8 connect request	762

26.8.1.3.5.1	Incoming call / U8 connect request / CONNECT acknowledged	762

26.8.1.3.5.2	Incoming call / U8 connect request / timer T313 time-out	763

26.8.1.3.5.3	Incoming call / U8 connect request / termination requested by the user	765

26.8.1.3.5.4	Incoming call / U8 connect request / DISCONNECT received with in-band information	766

26.8.1.3.5.5	Incoming call / U8 connect request / DISCONNECT received without in-band information	767

26.8.1.3.5.6	Incoming call / U8 connect request / RELEASE received	768

26.8.1.3.5.7	Incoming call / U8 connect request / lower layer failure	770

26.8.1.3.5.8	Incoming call / U8 connect request / TCH assignment	771

26.8.1.3.5.9	Incoming call / U8 connect request / unknown message received	772

26.8.1.4	In call functions	773

26.8.1.4.1	In-call functions / DTMF information transfer	773

26.8.1.4.1.1	In-call functions / DTMF information transfer / basic procedures	773

26.8.1.4.2	In-call functions / user notification	775

26.8.1.4.2.1	In-call functions / User notification / MS terminated	775

26.8.1.4.3	In-call functions / channel changes	776

26.8.1.4.3.1	In-call functions / channel changes / a successful channel change in active state/ Handover and Assignment Command	776

26.8.1.4.3.2	In-call functions / channel changes / an unsuccessful channel change in active mode/ Handover and Assignment Command	778

26.8.1.4.4	In-call functions / MS terminated in-call modification	780

26.8.1.4.4.1	In-call functions / MS terminated in-call modification / modify when new mode is not supported	780

26.8.1.4.5	In-call functions / MS originated in-call modification	781

26.8.1.4.5.1	In-call functions / MS originated in-call modification / a successful case of modifying	781

26.8.1.4.5.2	In-call functions / MS originated in-call modification / modify rejected	783

26.8.1.4.5.3	In-call functions / MS originated in-call modification / an abnormal case of acceptance	785

26.8.1.4.5.4	In-call functions / MS originated in-call modification / an abnormal case of rejection	786

26.8.1.4.5.5	In-call functions / MS originated in-call modification / time-out of timer T323	787

26.8.1.4.5.6	In-call functions / MS originated in-call modification / a successful channel change in state mobile originating modify	789

26.8.1.4.5.7	In-call functions / MS originated in-call modification / an unsuccessful channel change in state mobile originating modify	791

26.8.1.4.5.8	In-call functions / MS originated in-call modification / unknown message received	793

26.8.1.4.5.9	In-call functions / MS originated in-call modification / a release complete received	794

26.8.1.4.5.10	In-call functions/MS originated in-call modification/contents of some of the messages	796

26.8.2	Call Re-establishment	798

26.8.2.1	Call Re-establishment/call present, re-establishment allowed	798

26.8.2.2	Call Re-establishment/call present, re-establishment not allowed	800

26.8.2.3	Call Re-establishment/call under establishment, transmission stopped	801

26.8.3	User to user signalling	803

26.8.4	Default contents of message	805

26.9	Structured procedures	812

26.9.1	Structured procedures / general	812

26.9.2	Structured procedures / MS originated call / early assignment	812

26.9.3	Structured procedures / MS originated call / late assignment	815

26.9.4	Structured procedures / MS terminated call / early assignment	817

26.9.5	Structured procedures / MS terminated call / late assignment	820

26.9.6	Structured procedures / emergency call	823

26.9.6.1	Structured procedures / emergency call / idle updated	823

26.9.6.1.1	Structured procedures / emergency call / idle updated / preferred channel rate	823

26.9.6.1.2	Structured procedures / emergency call / idle updated, non-preferred channel rate	826

26.9.6.2	Structured procedures / emergency call / idle, no IMSI	826

26.9.6.2.1	Structured procedures / emergency call / idle, no IMSI / accept case	826

26.9.6.2.2	Structured procedures / emergency call / idle, no IMSI / reject case	828

26.9.7	Default contents of messages	830

26.10	E-GSM signalling	836

26.10.1	E-GSM signalling / general considerations	836

26.10.2	E-GSM signalling / RR	836

26.10.2.1	E-GSM signalling / RR / Measurement	836

26.10.2.2	E-GSM signalling / RR / Immediate assignment	841

26.10.2.3	E-GSM signalling / RR / channel assignment procedure	842

26.10.2.4	E-GSM signalling / RR / Handover	845

26.10.2.4.1	E-GSM signalling / RR / Handover / Successful handover	845

26.10.2.4.2	E-GSM signalling / RR / Handover / layer 1 failure	849

26.10.2.5	E-GSM signalling / RR / Frequency Redefinition	850

26.10.3	E-GSM signalling / Structured procedure	852

26.10.3.1	E-GSM signalling / Structured procedure / Mobile originated call	853

26.10.3.2	E-GSM signalling / Structured procedures / emergency call	855

26.10.3.3	Default contents of messages	858

26.10.4	E-GSM signalling / Default message contents	858

26.11	Multiband signalling	863

26.11.1	General considerations	863

26.11.2	Multiband signalling / RR	863

26.11.2.1	Multiband signalling / RR / Immediate assignment procedure	863

26.11.2.2	Multiband signalling / RR / Handover	865

26.11.2.2.1	Multiband signalling / RR / Handover / successful / active call / non-synchronized	865

26.11.2.2.2	Multiband signalling / RR / Handover / layer 1 failure	870

26.11.2.3	Multiband signalling / RR / Measurement reporting	872

26.11.3	Multiband signalling / MM	875

26.11.3.1	Multiband signalling / MM / Location updating	875

26.11.3.1.1	Location updating / accepted	875

26.11.3.1.2	Location updating / periodic	878

26.11.4	Multiband signalling / CC	880

26.11.5	Multiband signalling / Structured procedures	880

26.11.5.1	Multiband signalling / Structured procedures / MS originated call / early assignment	880

26.11.5.2	Structured procedures / MS terminated call / late assignment	884

26.11.6	Multiband signalling / Default messages contents	888

26.12	Enhanced Full Rate signalling	894

26.12.1	EFR signalling/ test of the channel mode modify procedure	894

26.12.2	EFR signalling/ tests of handover	897

26.12.2.1	EFR signalling / Handover /  active call / successful case	898

26.12.2.2	EFR signalling/ Handover / successful / call under establishment / non-synchronized	915

26.12.3	EFR Signalling / Structured procedures / MS originated call / late assignment	930

26.12.4	Structured procedures / MS terminated call / l early assignment	933

26.12.5	Structured procedures / emergency call	936

26.12.6	Default contents of  layer 3 messages for Enhanced Full rate speech tests	939

27	Testing of the SIM/ME interface	943

27.1	MS identification by short IMSI	946

27.1.1	MS identification by short IMSI - Normal case	946

27.1.2	MS identification by short IMSI, Phase 1 DCS SIM	947

27.2	MS identification by short TMSI	948

27.3	MS identification by long TMSI	949

27.4	MS identification by long IMSI, TMSI updating and cipher key sequence number assignment	950

27.5	Forbidden PLMNs, location updating and undefined cipher key	952

27.6	MS updating forbidden PLMNs	955

27.7	MS deleting forbidden PLMNs	957

27.8	MS updating the PLMN selector list	958

27.9	MS recognizing the priority order of the PLMN selector list	959

27.10	MS access control management	961

27.11	Exchange protocol tests	971

27.11.1	Character transmission	971

27.11.1.1	Bit/character duration during the transmission from the ME to the SIM	971

27.11.1.2	Bit/character duration during the transmission from the SIM simulator to the ME	971

27.11.1.3	Inter-character delay	972

27.11.1.4	Error handling during the transmission from the ME to the SIM	973

27.11.1.5	Error handling during transmission from the SIM to the ME	974

27.11.2	Answer to reset (RST)	975

27.11.2.1	Acceptance of SIMs with internal RST	975

27.11.2.2	Acceptance of SIMs with active low RST	975

27.11.2.3	Characters of the answer to reset	976

27.11.2.4	PTS procedure	977

27.11.2.5	Reset repetition	978

27.11.3	Command processing, procedure bytes	979

27.12	Evaluation of directory characteristics	979

27.12.1	Operating speed in authentication procedure	979

27.12.2	Clock stop	980

27.13	Mechanical tests	982

27.13.1	Contact pressure	982

27.13.2	Shape of contacts for IC card SIM card reader	982

27.14	Secret code usage	983

27.14.1	Entry of PIN	983

27.14.2	Change of PIN	984

27.14.3	Disabling the PIN	985

27.14.4	PUK entry	986

27.14.5	Entry of PIN2	987

27.14.6	Change of PIN2	987

27.14.7	PUK2 entry	988

27.15	Abbreviated Dialling Numbers (ADN)	989

27.16	MMI reaction to SIM status encoding	991

27.17	Electrical tests	991

27.17.1	Test of the power transition phases	993

27.17.1.1	Phase preceding ME power on	993

27.17.1.2	Phase during SIM power on	993

27.17.1.3	Phase during ME power off with clock stop forbidden	995

27.17.1.4	Phase during ME power off with clock stop allowed	996

27.17.1.5	SIM Type Recognition and Voltage Switching	998

27.17.1.5.1	Reaction of 3V only MEs on SIM type recognition failure	998

27.17.1.5.2	Reaction of 3V only MEs on type recognition of 5V only SIMs	999

27.17.1.5.3	Reaction of 3V technology MEs on type recognition of 5V only SIMs	1000

27.17.1.5.4	Reaction of 3V technology MEs on type recognition of 3V technology SIMs	1001

27.17.2	Electrical tests on each ME contact	1002

27.17.2.1	Electrical tests on contact C1	1003

27.17.2.1.1	Test 1	1003

27.17.2.1.2	Test 2	1004

27.17.2.2	Electrical tests on contact C2	1007

27.17.2.3	Electrical tests on contact C3	1008

27.17.2.4	[Not used]	1009

27.17.2.5	Electrical tests on contact C7	1009

27.18	Fixed Number Dialling (FND)	1011

27.18.1	ME and SIM with FND activated	1011

27.18.2	ME and SIM with FND deactivated	1012

27.18.3	Enabling, disabling and updating of FND	1013

27.19	Phase identification	1015

27.20	SIM presence detection	1015

27.21	Advice of Charge (AoC)	1016

27.21.1	AoC not supported by SIM	1016

27.21.2	Maximum frequency of ACM updating	1018

27.21.3	Call terminated when ACM greater than ACMmax	1020

27.21.4	Response codes of increase command	1023

28	Test of autocalling restrictions	1026

28.1	General	1026

28.2	Constraining the access to a single number (GSM 02.07 category 3)	1026

28.3	Constraining the access to a single number (GSM 02.07 categories 1 and 2)	1028

28.4	Behaviour of the MS when its list of blacklisted numbers is full	1030

29	Testing of bearer services	1032

29.1	General	1032

29.2	Testing of transparent data services	1032

29.2.1	Verification of synchronization	1032

29.2.2	Filtering of channel control information for transparent BCs	1035

29.2.3	Correct Terminal Compatibility Decision	1037

29.2.3.1	Negotiation of Radio Channel Requirement (RCR)	1037

29.2.3.2	Negotiation of Connection Element (CE)	1037

29.2.3.3	Negotiation of Number of Stop Bits, Number of Data bits, and Parity	1038

29.2.3.4	Negotiation of Modem Type	1039

29.2.3.5	Negotiation of Intermediate Rate	1039

29.2.3.6	Negotiation of User Information Layer 2 Protocol	1040

29.2.3.7	Negotiation between TS 61 and TS 62: Mobile Originated call.	1041

29.2.3.8	Negotiation between TS 61 and TS 62: Mobile Terminated call.	1041

29.2.4	Data Rate Adaptation for Synchronous Transparent Bearer Capabilities	1042

29.2.5	Network Independent Clocking	1043

29.2.6	Asynchronous Transparent Bearer Capabilities	1043

29.2.6.1	Data Rate Adaptation	1043

29.2.6.2	Passage of the Break Signal	1044

29.2.6.3	Overspeed/Underspeed Handling (Local Terminal)	1046

29.2.6.4	Overspeed/Underspeed Handling (Remote Terminal)	1047

29.2.7	Interchange circuit mapping for transparent bearer capabilities	1048

29.3	Testing of non transparent data services (RLP tests)	1049

29.3.1	Initialization	1049

29.3.1.1	Normal initialization done by the MS	1049

29.3.1.2	Initialization failure	1050

29.3.1.2.1	Loss of UA frame	1050

29.3.1.2.2	Total loss of UA frame	1051

29.3.2	Data transfer	1052

29.3.2.1	Default conditions	1052

29.3.2.2	MS sends I+S frames	1053

29.3.2.2.1	N(S) sequence number	1053

29.3.2.2.2	Transmission window	1054

29.3.2.2.3	Busy condition	1056

29.3.2.3	SS sends I+S frames	1058

29.3.2.3.1	N(R) sequence number	1058

29.3.2.3.2	Busy condition	1059

29.3.2.4	SS rejects I+S frames	1060

29.3.2.4.1	REJ frame	1060

29.3.2.4.2.	SREJ frame	1062

29.3.2.4.3	I+S reject frame	1064

29.3.2.5	MS rejects I+S frames	1067

29.3.2.5.1	Rejection with REJ or SREJ supervisory frames	1067

29.3.2.5.2	Retransmission of REJ or SREJ frames	1071

29.3.2.5.3	I+S reject frame	1074

29.3.2.6	Checkpoint recovery	1076

29.3.2.6.1	SS in checkpoint recovery mode	1076

29.3.2.6.2	End of the window	1079

29.3.2.6.3	End of a sequence	1081

29.3.2.6.4	Time-out of one frame	1082

29.3.2.6.5	No response to checkpointing	1083

29.3.2.6.6	Incorrect response to checkpointing	1085

29.3.2.6.7	Total loss of response to checkpointing	1087

29.3.2.6.8	Retransmission of a sequence	1089

29.3.2.6.9	N2 retransmission of a sequence	1092

29.3.3	Negotiation of the RLP parameters	1094

29.3.3.1	Negotiation initiated by the SS	1094

29.3.3.2	Negotiation initiated by the MS	1099

29.3.3.3	Collision of XID frames	1102

29.3.3.4	Loss of XID frames	1106

29.3.3.5	Total loss of XID frames	1107

29.4	Facsimile tests for the transparent network support	1108

29.4.1	General	1108

29.4.2	Mobile originated call	1109

29.4.2.1	Call establishment procedure	1109

29.4.2.1.1	Alternate speech / facsimile	1109

29.4.2.1.2	Automatic facsimile	1111

29.4.2.2	Pre-message procedure	1112

29.4.2.3	Message procedure	1113

29.4.2.4	Post-message procedure	1115

29.4.2.5	Call release procedure	1116

29.4.2.6	CTC processing - 4th PPR for the same block	1117

29.4.2.7	Transition from Facsimile to Speech - Procedure interrupt generated by receiving station	1118

29.4.2.8	Transition from Facsimile to Speech - Procedure interrupt generated by transmitting station	1120

29.4.2.9	Quality check	1121

29.4.3	Mobile terminated call	1122

29.4.3.1	Call Establishment Procedure	1122

29.4.3.1.1	Alternate Speech/Facsimile	1122

29.4.3.1.1.1	DCD Mobile Terminated	1122

29.4.3.1.1.2	DCD mobile originated	1123

29.4.3.1.2	Automatic facsimile	1124

29.4.3.2	Pre-message procedure	1126

29.4.3.3	Message procedure	1127

29.4.3.4	Post-message procedure	1128

29.4.3.5	Call release procedure	1129

29.4.3.6	Speed conversion factor	1130

29.4.3.7	Quality Check	1133

29.4.3	Notes	1133

30	Speech teleservices	1135

30.1	Sending sensitivity/frequency response	1135

30.2	Sending loudness rating	1136

30.3	Receiving sensitivity/frequency response	1137

30.4	Receiving loudness rating	1138

30.5	Side tones	1139

30.5.1	Side Tone Masking Rating (STMR)	1139

30.5.2	Listener Side Tone Rating (LSTR)	1140

30.6	Telephone Acoustic coupling Loss (TAL)	1141

30.6.1	Echo Loss (EL)	1141

30.6.2	Stability margin	1142

30.7	Distortion	1143

30.7.1	Sending	1143

30.7.2	Receiving	1144

30.8	Sidetone distortion	1145

30.9	Out-of-band signals	1146

30.9.1	Sending	1146

30.9.2	Receiving	1147

30.10	Idle channel noise	1148

30.10.1	Sending	1148

30.10.2	Receiving	1148

31	Test of supplementary services	1150

31.1	Number identification supplementary services	1150

31.2	Call offering supplementary services	1150

31.2.1	Call forwarding supplementary services	1150

31.2.1.1	Registration	1150

31.2.1.1.1	Registration accepted	1150

31.2.1.1.2	Registration rejected	1153

31.2.1.2	Erasure by the subscriber	1156

31.2.1.2.1	Erasure accepted	1156

31.2.1.2.2	Erasure rejected	1158

31.2.1.3	Activation	1161

31.2.1.4	Deactivation	1163

31.2.1.5	Invocation	1165

31.2.1.6	Interrogation	1165

31.2.1.6.1	Interrogation accepted	1165

31.2.1.6.2	Interrogation rejected	1167

31.2.1.7	Normal operation	1170

31.2.1.7.1	Served mobile subscriber side	1170

31.2.1.7.1.1	Notification during an incoming call	1170

31.2.1.7.1.2	Notification during an outgoing call	1172

31.2.1.7.2	Forwarded-to mobile subscriber side	1174

31.2.2	Call transfer and mobile access hunting supplementary services	1175

31.3	Call completion supplementary services	1175

31.4	Multi-party supplementary services	1175

31.5	Community of interest supplementary services	1175

31.6	Charging supplementary services	1176

31.6.1	Advice of Charge Charging	1176

31.6.1.1	AOC time related charging / MS originated call	1176

31.6.1.2	AOC time related charging / MS terminated call	1179

31.6.1.3	AOC volume related charging / MS originated call	1182

31.6.1.4	AOC volume related charging / MS terminated call	1182

31.6.1.5	Change in charging information during a call	1182

31.6.1.6	Different formats of charging information	1185

31.6.1.7	AOC on a Call Hold call	1188

31.6.1.8	AOC on a Multi-party call	1191

31.6.2	Charge Storage	1195

31.6.2.1	Removal of SIM during an active call	1195

31.6.2.2	Interruption of power supply during an active call	1197

31.6.2.3	MS going out of coverage during an active AoCC call	1199

31.6.2.4	ACMmax operation / Mobile Originating	1201

31.6.2.5	ACMmax operation / Mobile Terminating	1204

31.6.3	Advice of Charge Information	1207

31.6.4	Default contents of messages	1207

31.7	Additional information transfer supplementary services	1208

31.8	Call restriction supplementary services	1208

31.8.1	Registration of a password	1209

31.8.1.1	Registration accepted	1209

31.8.1.2	Registration rejected	1211

31.8.1.2.1	Rejection after invoke of the RegisterPassword operation	1211

31.8.1.2.2	Rejection after password check with negative result	1213

31.8.1.2.3	Rejection after new password mismatch	1215

31.8.2	Erasure	1218

31.8.3	Activation	1218

31.8.3.1	Activation accepted	1218

31.8.3.2	Activation rejected	1221

31.8.3.2.1	Rejection after invoke of ActivateSS operation	1221

31.8.3.2.2	Rejection after use of password procedure	1223

31.8.4	Deactivation	1225

31.8.4.1	Deactivation accepted	1225

31.8.4.2	Deactivation rejected	1228

31.8.4.2.1	Rejection after invoke of DeactivateSS operation	1228

31.8.4.2.2	Rejection after use of password procedure	1230

31.8.5	Invocation	1232

31.8.6	Interrogation	1232

31.8.6.1	Interrogation accepted	1232

31.8.6.2	Interrogation rejected	1235

31.8.7	Normal operation	1237

31.9	Handling of undefined (future) GSM supplementary services	1238

31.9.1	Mobile station initiated Unstructured supplementary service data operation	1238

31.9.1.1	ProcessUnstructuredSS-request/accepted	1238

31.9.1.2	ProcessUnstructuredSS-request/cross phase compatibility and error handling	1243

31.9.2	Network initiated unstructured supplementary service operations	1248

31.9.2.1	UnstructuredSS-Notify/accepted	1248

31.9.2.3	UnstructuredSS-Request/accepted	1253

31.10	MMI input for USSD	1259

31.11	Specific message contents and ASN.1 codings	1260

32	Testing of speech transcoding functions	1310

32.1	Full Rate Downlink speech transcoding	1310

32.2	Full Rate Downlink receiver DTX functions	1311

32.3	Full Rate Uplink speech transcoding	1314

32.4	Full Rate Uplink transmitter DTX functions	1315

32.5	Full Rate Speech channel transmission delay	1316

32.5.4	Downlink processing delay	1317

32.5.5	Downlink coding delay	1317

32.5.6	Uplink processing delay	1318

32.5.7	Uplink coding delay	1318

32.6	Half Rate Downlink speech transcoding	1319

32.7	Half Rate Downlink receiver DTX functions	1320

32.8	Half Rate Uplink speech transcoding	1321

32.9	Half Rate Uplink transmitter DTX functions	1322

32.10	Half Rate Speech channel transmission delay	1324

32.10.4	Downlink processing delay	1325

32.10.5	Downlink coding delay	1325

32.10.6	Uplink processing delay	1326

32.10.7	Uplink coding delay	1326

32.11	Intra cell channel change from a TCH/HS to a TCH/FS	1327

32.12	Intra cell channel change from a TCH/FS to a TCH/HS	1329

33	Mobile station features	1332

33.1	Entry and display of called number	1332

33.2	Indication of call progress signals	1333

33.2.4	Ringing tone	1333

33.2.5	Busy tone	1333

33.2.6	Congestion tone	1334

33.2.7	Authentication failure tone	1334

33.2.8	Number unobtainable tone	1335

33.2.9	Call dropped tone	1335

33.3	Network selection / indication	1336

33.3.1	Definition and applicability	1336

33.3.2	Conformance requirement	1336

33.3.3	Test purpose	1336

33.3.4	Method of Test	1337

33.3.4.1	Procedure 1	1337

33.3.4.2	Requirements 1	1339

33.3.4.3	Procedure 2	1339

33.3.4.4	Requirements 2	1340

33.4	Invalid and blocked PIN indicators	1340

33.5	Service indicator	1341

33.6	Subscription identity management	1341

33.7	Barring of outgoing calls	1342

33.8	Prevention of unauthorized calls	1343

34	Short message service (SMS)	1344

34.1	General	1344

34.2	Short message service point to point	1344

34.2.1	SMS mobile terminated	1344

34.2.2	SMS mobile originated	1350

34.2.3	Test of memory full condition and memory available notification:	1355

34.2.4	Test of the status report capabilities and of SMS-COMMAND:	1360

34.2.5	Test of message class 0 to 3	1364

34.2.5.1	Short message class 0	1364

34.2.5.2	Test of class 1 short messages	1367

34.2.5.3	Test of class 2 short messages	1369

34.2.5.4	Test of class 3 short messages	1373

34.2.6	Test of short message type 0	1373

34.2.7	Test of the replace mechanism for SM type 1-7	1373

34.2.8	Test of the reply path scheme	1377

34.2.9	Multiple SMS mobile originated	1380

34.2.9.1	MS in idle mode	1380

34.2.9.2	MS in active mode	1384

34.3	Short message service cell broadcast	1387

34.4	 Default message contents:	1389

35	Low battery voltage detection	1392

36	Individual equipment type requirements and interworking - special conformance testing functions	1394

36.1	General	1394

36.2	Activation and deactivation of special test functions in the MS	1394

36.2.1	Internal test loops	1394

36.2.1.1	TCH loop	1395

36.2.1.1.1	TCH loop including signalling of erased frames (A)	1396

36.2.1.1.2	Speech TCH loop without signalling of erased frames (B)	1396

36.2.1.1.3	TCH burst-by-burst loop (C)	1396

36.2.1.1.4	TCH loop including signalling of erased frames and unreliable frames (D)	1397

36.2.1.1.5	TCH loop including signalling of erased SID frames (E)	1398

36.2.1.1.6	TCH loop including signalling of erased valid SID frames (F)	1398

36.2.1.1.7	Deactivating TCH loops	1398

36.2.1.1.8	Additional non-mandatory operating characteristics	1399

36.2.2	Activating and deactivating EMMI	1399

36.2.3	Activating and deactivating DAI tests	1399

36.2.4	Message definitions and contents	1400

36.2.4.1	CLOSE_TCH_LOOP_CMD	1400

36.2.4.2	CLOSE_TCH_LOOP_ACK	1400

36.2.4.3	OPEN_LOOP_CMD	1401

36.2.4.4	Command for the activation of the EMMI, ACT_EMMI_CMD	1401

36.2.4.5	Acknowledge of the activation of the EMMI, ACT_EMMI_ACK	1401

36.2.4.6	Deactivation of the EMMI, DEACT_EMMI	1402

36.2.4.7	Test_Interface	1402

36.2.5	Timer values	1402

36.3	Electrical Man Machine Interface (EMMI)	1402

36.3.1	Use of the EMMI	1402

36.3.2	Formal aspects	1403

36.3.3	Layered structure of the interface	1403

36.3.4	Terminology	1403

36.3.5	Description of the EMMI	1403

36.3.5.1	EMMI, Layer 1	1403

36.3.5.1.1	Mechanical and electrical characteristics	1403

36.3.5.1.2	Transmission and reception characteristics	1404

36.3.5.2	EMMI, layer 2	1404

36.3.5.2.1	General structure	1404

36.3.5.2.2	Control frames	1404

36.3.5.2.3	Frame structure	1404

36.3.5.2.4	Flow of I-frames on layer 2	1405

36.3.5.2.4.1	Transmission of frames	1405

36.3.5.2.4.2	Reception of frames	1405

36.3.5.2.4.3	Use of ACK and NAK on receiving side	1405

36.3.5.2.4.4	Use of XON and XOF	1405

36.3.5.2.4.5	Parameters on layer 2	1406

36.3.5.3	EMMI, layer 3	1406

36.3.5.3.1	Message structure	1406

36.3.5.3.2	Definition of messages	1407

36.4	Digital audio interface	1413

36.4.1	General	1413

36.4.2	Formal aspects	1413

36.4.3	Hardware aspect of the interface	1414

36.4.3.1	Mechanical characteristics of the interface	1414

36.4.3.2	Electrical characteristics of the interface	1414

36.4.3.3	Timing characteristics of the interface	1414

36.4.4	Logical interface	1415

36.4.5	Functionality of the DAI	1415

36.5	SIM/ME test interface	1416

36.5.1	General	1416

36.5.2	Formal aspects	1416

36.5.3	Hardware and logical aspects of the interface	1416

36.5.4	Mechanical characteristics of the interface	1416

Annex 1 (Normative):	 Reference test methods	1418

A1.1 General Conditions (GC)	1418

A1.1.1	Outdoor test site and general arrangements for measurements involving the use of radiated fields (GC4)	1418

A1.1.2	Anechoic shielded chamber (GC5)	1418

A1.1.3	Temporary antenna connector (GC7)	1419

A1.1.4	Temporary antenna connector characteristics	1419

A1.1.5	Calibration of the temporary antenna connector	1420

A1.1.5.1	Antenna radiation pattern	1420

A1.1.5.2	Test range calibration	1422

A1.1.5.3	Temporary antenna connector coupling factor	1422

A1.2 Normal and extreme Test Conditions (TC)	1423

A1.2.1	Power sources and ambient temperatures (TC2)	1423

A1.2.2	Normal test conditions (TC2.1)	1424

A1.2.3	Extreme test conditions (TC2.2)	1424

A1.2.4	Vibration requirements (TC4)	1425

Annex 2:	 Not used	1426

Annex 3:	 Protocol implementation information	1426

A3.1 Protocol Implementation Conformance Statement (PICS)	1426

A3.1.1	LAPDm protocol (GSM 04.05 and 04.06)	1426

A3.1.1.1	Simplified protocol - GSM 04.06 section 6	1426

A3.1.1.2	Management of SAPI = 3 - GSM 04.11 section 2.3	1426

A3.1.2	Mobility management	1426

A3.1.2.1	IMSI detach initiation by the MS - GSM 04.08 section 4.3.4.1	1426

A3.1.2.2	IMSI detach completion by the MS - GSM 04.08 section 4.3.4.3	1426

A3.1.2.3	MM specific procedures - GSM 04.08 section 4.4 and 4.5.1.1	1426

A3.1.2.4	Receiving an MM STATUS message - GSM 04.08 section 4.6	1426

A3.1.3	Call control	1427

A3.1.3.1	Status enquiry procedures - GSM 04.08 section 5.5.3.1	1427

A3.1.3.2	Receiving a STATUS message by a CC entity - GSM 04.08 section 5.5.3.2	1427

A3.1.3.3	Called side compatibility checking - GSM 04.08 annex B.3	1427

A3.1.3.4	Disconnect on incoming call	1427

A3.1.4	Layer 1	1427

A3.1.4.1	Optional storage of BCCH carrier information - GSM 05.08 section 6.3	1427

A3.1.5	Autocalling - (ref.: GSM 02.07, annex 1)	1427

A3.1.6	Transient states	1427

A3.2 Protocol Implementation Extra Information for Testing (PIXIT)	1428

A3.2.0	Introduction	1428

A3.2.1	Basic characteristics	1428

A3.2.1.1	Type of antenna	1428

A3.2.1.2	Power supply	1429

A3.2.1.3	Power class of the MS	1429

A3.2.1.4	Channel modes supported	1429

A3.2.1.5	Teleservices supported	1429

A3.2.1.6	Supplementary services supported	1429

A3.2.1.7	Bearer services supported	1429

A3.2.1.8	SIM removal	1430

A3.2.1.9	Classmark	1430

A3.2.1.10	Type of SIM/ME interface (ref. GSM 11.11 and GSM 11.12)	1430

A3.2.2	Man machine interface	1430

A3.2.2.1	Mobile station features	1430

A3.2.2.2	Short message service	1431

A3.2.2.3	Supplementary services	1431

A3.2.2.3.1	Call forwarding	1431

A3.2.2.3.2	Call restriction	1432

A3.2.2.3.3	Handling of (undefined) GSM supplementary services	1432

A3.2.3	Electrical Man Machine Interface (EMMI)	1432

A3.2.3.1	Methods supported for activation/deactivation of EMMI	1432

A3.2.3.2	Transmission rate supported by the ME on the EMMI	1432

A3.2.3.3	Layer 3 messages supported on the EMMI	1432

A3.2.3.4	Keystroke sequence messages	1432

A3.2.3.5	Internal malfunction detected messages	1432

A3.2.4	Digital Audio Interface (DAI)	1432

A3.2.5	Characteristics related to bearer services or teleservices	1433

A3.2.5.1	Access interface	1433

A3.2.5.2	Configuration of the MT	1433

A3.2.5.3	Capability information	1433

A3.2.5.4	Subaddress or DDI number	1433

A3.2.5.5	User to user signalling	1433

A3.2.5.6	Data call set-up and data call clearing	1433

A3.2.5.7	Characteristics of non-transparent data services	1434

A3.2.5.8	Possible ways of setting-up a call from either an external interface or internally	1434

A3.2.5.9	Application layer causing automatic call termination.	1434

A3.2.5.10	Call re-establishment for MS not supporting speech	1434

A3.2.6	International mobile station equipment identity	1434

A3.2.7	Receiver intermediate frequencies	1434

Annex 4:	 Test SIM Parameters	1435

A4.1 Introduction	1435

A4.1.1	Definitions	1435

A4.1.2	Definition of the test algorithm for authentication	1435

A4.2 Default Parameters for the test SIM	1435

A4.3. Default settings for the Elementary Files (EFs).	1435

A4.3.1	EFICCID (ICC Identification)	1436

A4.3.2	EFLP (Language preference)	1436

A4.3.3	EFIMSI (IMSI)	1436

A4.3.4	EFKc (Ciphering key Kc)	1436

A4.3.5	EFPLMNsel (PLMN selector)	1436

A4.3.6	EFHPLMN (HPLMN search period)	1436

A4.3.7	EFACMmax (ACM maximum value)	1437

A4.3.8	EFSST (SIM service table)	1437

A4.3.9	EFACM (Accumulated call meter)	1437

A4.3.10	EFPUCT (Price per unit and currency table)	1437

A4.3.11	EFCBMI (Cell broadcast Message Identifier Selection)	1437

A4.3.12	EFBCCH (Broadcast control channels)	1438

A4.3.13	EFACC (Access control class)	1438

A4.3.14	EFFPLMN (Forbidden PLMNs)	1438

A4.3.15	EFLOCI (Location information)	1438

A4.3.16	EFAD (Administrative data)	1438

A4.3.17	EFPhase (Phase identification)	1438

A4.3.18	EFADN (Abbreviated dialling numbers)	1439

A4.3.19	EFFDN (Fixed dialling numbers)	1439

A4.3.20	EFSMS (Short messages)	1439

A4.3.21	EFCCP (Capability configuration parameters)	1439

A4.3.22	EFMSISDN (MSISDN)	1439

A4.3.23	EFSMSP (Short message service parameters)	1439

A4.3.24	EFSMSS (SMS status)	1439

A4.3.25	EFEXT1 (Extension 1)	1439

A4.3.26	EFEXT2 (Extension 2)	1439

Annex 5:	Test equipment	1440

A5.1 Introduction	1440

A5.1.1	General	1440

A5.1.2	Test equipment terms	1440

A5.1.3	Confidence level	1440

A5.2 Standard test signals	1440

A5.3 SS functional requirements	1441

A5.3.1	Level setting range	1441

A5.3.2	Level Measurement / operation range	1441

A5.3.3	MS power supply interface	1441

A5.3.4	MS antenna interface	1441

A5.3.4.1	Uplink receiver error	1441

A5.3.4.2	Power versus time measurements	1441

A5.3.4.3	Wideband selective power measurement	1444

A5.3.4.4	Inband selective power measurements	1444

A5.3.4.5	Phase trajectory and frequency error measurements	1444

A5.3.4.6	RF delay measurements relative to nominal times	1445

A5.3.4.7	The wanted signal or traffic channel of serving cell	1445

A5.3.4.8	The first interfering signal or traffic channel of the first adjacent cell	1445

A5.3.4.9	The second interfering signal	1446

A5.3.4.10	BCCH carriers of serving and adjacent cells	1446

A5.3.4.11	The wide frequency range signal	1447

A5.3.4.12	The multipath fading function	1447

A5.3.5	MS audio interface and DAI	1447

A5.3.5.1	General uncertainties	1447

A5.3.5.2	Analogue single test tone	1448

A5.3.5.3	Delay measurement between Um and DAI	1448

A5.4 SIM simulator functional requirements	1448

A5.4.1	General	1448

A5.4.2	Contacts C1, C2, C6, C7	1448

A5.4.2.1	Default measurement / setting uncertainties	1448

A5.4.2.2	Contact C1	1448

A5.4.2.3	Contact C7	1449

A5.4.3	Contact C3	1449

A5.4.4	Definition of timing	1449

History	1450