## 26.12	Enhanced Full Rate signalling

This section only applies to MS supporting enhanced  full rate speech.

As an EFR mobile station necessarily supports  the speech full rate version 1 or both  speech full rate version 1  and speech half rate version 1, conformance requirements of section 26 fully apply to this mobile.

The purpose of this extra section is to test the different procedures which may be impacted when Enhanced full rate speech codec is used.

### 26.12.1	EFR signalling/ test of the channel mode modify procedure

NOTE: 	This test is derived from the tests in sections 26.6.7.1 and 26.6.7.2 respectively entitled “Test of the channel mode modify procedure / full rate” and “Test of the channel mode modify procedure / half rate”.

This test is only applicable to an enhanced full rate speech MS.

26.12.1.1	Conformance requirement

The MS with a TCH/F allocated acknowledges a CHANNEL MODE MODIFY message by sending a CHANNEL MODE MODIFY ACKNOWLEDGE message specifying  the new mode and by swithcing to this mode when this one is set to :

-	speech full rate or half rate version 1

-	speech full rate or half rate version 2

- 	any other mode declared supported by the mobile

If the mobile station does not support the indicated mode, it shall retain the old mode and return the associated channel mode information in the CHANNEL MODE MODIFY ACKNOWLEDGE message.

References

TS GSM  02.06 , subclauses  3.2.3

TS GSM 04.08, subclauses  3.4.6.1.1 and 3.4.6.1.2

26.12.1.2	Test purpose

To verify that the MS  with a TCH/F allocated acknowledges a CHANNEL MODE MODIFY message by sending a CHANNEL MODE MODIFY ACKNOWLEDGE message specifying  the new mode and by swithcing to this mode when this one is set to :

-	speech full rate or half rate version 1

-	speech full rate or half rate version 2

- 	any other mode declared supported by the mobile

To verify that the MS, in an RR connected state, acknowledges a CHANNEL MODE MODIFY message by sending a CHANNEL MODE MODIFY ACKNOWLEDGE message specifying  the old mode when the new  mode is not declared as supported by the mobile

26.12.1.3	Method of test

Initial Conditions

System Simulator:	1 cell, default parameters.

Mobile Station:	The MS is "idle updated", with TMSI allocated.

Related PICS/PIXIT Statements

-	Type of MS (P-GSM900 or EGSM or DCS1800).

-	The MS supports speech full rate version 2

-	Bearer Capabilities supported by the MS

-	Channel modes supported by the MS

*	MS supports speech full rate version 3 (p1 = Y/N)

*	MS supports data 12 Kb/s (p2 = Y/N)

*	MS supports data 6 Kb/s full rate (p3 = Y/N)

*	MS supports data 3,6 Kb/s full rate (p4 = Y/N)

Foreseen Final State of the MS

"Idle, updated", with TMSI allocated.

Test procedure

A Mobile Terminated call is initiated , however following the CHANNEL REQUEST received from the Mobile Station, the SS sends an IMMEDIATE ASSIGNMENT to the MS commanding it to go to a TCH/F. This sets the Channel Mode automatically to "Signalling Only".

The SS then sends a series of CHANNEL MODE MODIFY messages to the MS. Each time it is checked that the MS responds with a CHANNEL MODE MODIFY ACKNOWLEDGE message specifying

-	the channel mode that has been specified in the CHANNEL MODE MODIFY message, if the MS supports that mode (this mode then becomes the "channel mode in use"). If necessary, the MS shall be correctly configured in order to accept this mode .

-	the channel mode that was in use when the CHANNEL MODE MODIFY message has been received, if the MS does not support the channel mode specified in the CHANNEL MODE MODIFY message.

Maximum Duration of Test

50 seconds.

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | SS->MS | PAGING REQUEST TYPE 1 | Sent on correct paging subchannel |
| 2 | MS->SS | CHANNEL REQUEST | Establishment cause indicates "answer to paging" |
| 3 | SS->MS | IMMEDIATE ASSIGNMENT | Assignment to a non hopping TCH/F |
| 4 | SS->MS | CHANNEL MODE MODIFY |  |
| 5 | MS->SS | CHANNEL MODE MODIDY ACKNOWLEDGE |  |
| 6 | SS->MS | CHANNEL MODE MODIFY |  |
| 7 | MS->SS | CHANNEL MODE MODIDY ACKNOWLEDGE |  |
| 8 | SS->MS | CHANNEL MODE MODIFY |  |
| 9 | MS->SS | CHANNEL MODE MODIDY ACKNOWLEDGE |  |
| 10 | SS->MS | CHANNEL MODE MODIFY |  |
| 11 | MS->SS | CHANNEL MODE MODIDY ACKNOWLEDGE |  |
| 12 | SS->MS | CHANNEL MODE MODIFY |  |
| 13 | MS->SS | CHANNEL MODE MODIDY ACKNOWLEDGE |  |
| 14 | SS->MS | CHANNEL MODE MODIFY |  |
| 15 | MS->SS | CHANNEL MODE MODIDY ACKNOWLEDGE |  |
| 16 | SS->MS | CHANNEL MODE MODIFY |  |
| 17 | MS->SS | CHANNEL MODE MODIDY ACKNOWLEDGE |  |
| 18 | SS->MS | CHANNEL MODE MODIFY |  |
| 19 | MS->SS | CHANNEL MODE MODIDY ACKNOWLEDGE |  |
| 20 | SS->MS | CHANNEL MODE MODIFY |  |
| 21 | MS->SS | CHANNEL MODE MODIDY ACKNOWLEDGE |  |
| 22 | SS->MS | CHANNEL MODE MODIFY |  |
| 23 | MS->SS | CHANNEL MODE MODIDY ACKNOWLEDGE |  |
| 24 | SS->MS | CHANNEL RELEASE |  |

Specific Message Contents

CHANNEL MODE MODIFY

| Information Element | value/remark |
| --- | --- |
| Channel description | describes the already assigned dedicated channel |
| Channel mode |  |
| Mode | in step 4: speech full or half rate version 2 in step 6: data 3,6 Kb/s in step 8: speech full or half rate version 2 in step 10: data 6 Kb/s in step 12: speech full or half rate version 2 in step 14: data 12 Kb/s in step 16: speech full or half rate version 2 in step 18: speech full or half rate version 1 in step 20: speech full or half rate version 2 in step 22: speech full or half rate version 3 |

CHANNEL MODE MODIFY ACKNOWLEDGE

| Channel mode |  |
| --- | --- |
| Mode | in steps 5, 9, 13, 17, 21 : speech full rate version 2 in step 7: if p 4= Y: data 3,6 Kb/s if p4 = N: same as in step 5 in step 11 : if p3 = Y: data 6,0 Kb/s full rate if p3 = N: same as in step 9 in step 15: if p2 = Y: data 12 Kb/s full rate if p2 = N: same as in step 13 in step 19: speech full rate version 2 in step 23: if p1 = Y: speech full rate version 3 if p1 = N: same as in step 21 |

### 26.12.2	EFR signalling/ tests of handover

With the Handover procedure, it is possible to completely alter the channels allocated to a MS. This makes it possible in particular to switch a call in progress from one cell to another. The procedure is always initiated by the network and with the MS in a dedicated mode.

Sections 26.12.2.1 - 26.12.2.2 contain generic test procedures to be used for executing successful Handover tests dealing with EFR mode.

section 26.12.2.1 deals with EFR signalling in the Handover/successful/active call/non synchronised case.

Section 26.12.2.2 deals with EFR signalling in the Handover/successful/call under establishment/non synchronised case.

Table 1 contains a summary of the different combinations of parameters which have to be tested, together with a reference to the appropriate generic test procedure. If a test uses a channel rate which the MS under test does not support, the test shall be skipped.

sv1 stands for speech full/half rate version 1.

sv2 stands for speech full/half rate version 2 (enhanced full rate).

Table 1

| From | To | Timing Adv. | Start Time | Syn ? | State of call | Section | Exec Coun-ter |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TCH/F, sv2, no FH | TCH/F, sv2, no FH | 20 | none | no | U10 | 26.12.2.1 | 1 |
| TCH/F, sv2, no FH | TCH/F, sv2, FH | arbitrary | none | no | U10 | 26.12.2.1 | 2 |
| TCH/F, sv2, FH | TCH/F, sv2, no FH | 20 | 1,1s | no | U10 | 26.12.2.1 | 3 |
| TCH/F, sv2, no FH | TCH/F, sv1, no FH | 20 | none | no | U10 | 26.12.2.1 | 4 |
| TCH/F, sv1,no FH | TCH/F, sv2, no FH | arbitrary | none | no | U10 | 26.12.2.1 | 5 |
| TCH/F, sv2, no FH | TCH/F, sv1, FH | arbitrary | none | no | U10 | 26.12.2.1 | 6 |
| TCH/F, sv1, FH | TCH/F, sv2,FH | 20 | 1,1 | no | U10 | 26.12.2.1 | 7 |
| TCH/F, sv2, FH | TCH/F, sv1, FH | arbitrary | none | no | U10 | 26.12.2.1 | 8 |
| TCH/F, sv1, FH | TCH/F, sv2, no FH | arbitrary | none | no | U10 | 26.12.2.1 | 9 |
| TCH/F, sv2, no FH | TCH/H, sv1, FH | arbitrary | none | no | U10 | 26.12.2.1 | 10 |
| TCH/H, sv1, FH | TCH/F, sv2, FH | 20 | 1,1 | no | U10 | 26.12.2.1 | 11 |
| TCH/F, sv2, FH | TCH/H, sv1, FH | arbitrary | none | no | U10 | 26.12.2.1 | 12 |
| TCH/H, sv1, FH | TCH/F, sv2, noFH | 20 | none | no | U10 | 26.12.2.1 | 13 |
| TCH/F, sv2, noFH | TCH/H, sv1, noFH | 20 | none | no | U10 | 26.12.2.1 | 14 |
| TCH/H, sv1, noFH | TCH/F, sv2, noFH | 20 | none | no | U10 | 26.12.2.1 | 15 |
| SDCCH/4, no FH | TCH/F, sv2, FH | 20 | none | no | estab | 26.12.2.2 | 1 |
| SDCCH/8, FH | TCH/F, sv2, no FH | 20 | none | no | estab | 26.12.2.2 | 2 |
| SDCCH/8, no FH | TCH/F, sv2, FH | 20 | none | no | estab | 26.12.2.2 | 3 |
| TCH/F, no FH | TCH/F, sv2, FH | 20 | none | no | estab | 26.12.2.2 | 4 |
| TCH/F, no FH | TCH/F, sv2, no FH | 20 | none | no | estab | 26.12.2.2 | 5 |
| TCH/F, FH | TCH/F, sv2, FH | 20 | 1,1 | no | estab | 26.12.2.2 | 6 |
| TCH/F, FH | TCH/F, sv2, no FH | 20 | none | no | estab | 26.12.2.2 | 7 |
| TCH/H, no FH | TCH/F, sv2, FH | 20 | none | no | estab | 26.12.2.2 | 8 |
| TCH/H, no FH | TCH/F, sv2, no FH | 20 | 1,1 | no | estab | 26.12.2.2 | 9 |
| TCH/H, FH | TCH/F, sv2, FH | 20 | none | no | estab | 26.12.2.2 | 10 |
| TCH/H, FH | TCH/F, sv2, no FH | 20 | none | no | estab | 26.12.2.2 | 11 |

Table 2

|  | TCH/FS | TCH/HS | SDCCH |
| --- | --- | --- | --- |
| n | 10-20 | 5-10 | 2-5 |

n : number of access bursts.

#### 26.12.2.1	EFR signalling / Handover /  active call / successful case

NOTE:	This test is derived from the one defined in section 26.6.5.1 “Handover/successful/active call/non-synchronized”

This test only applies for MS supporting full rate speech version 2 (enhanced full rate speech).

26.12.2.1.1	Conformance requirements

The MS shall correctly apply the handover procedure in the non-synchronized case when :

-	a call is in progress  and,

-	handover is performed from a TCH/F with/without frequency hopping towards a TCH/F with/without frequency hopping  and,

-	the mode of  either the current or the target channel is set to full rate speech version 2 (enhanced full rate speech).

The MS also supporting half rate shall correctly apply the handover procedure in the non-synchronized case when:

-	a call is in progress and,

-	a handover is performed between a TCH/H with/without frequency hopping and  a TCH/F with/without frequency hopping and,

-	the mode of the TCH/F is set to  full rate speech version 2.

References

GSM 04.08 sections 3.4.4 and 9.1.15

GSM 04.13 section 5.2.6.2.

26.12.2.1.2	Test purpose

To test that the MS shall correctly apply the handover procedure in the non-synchronized case when :

-	a call is in progress  and,

-	handover is performed from a TCH/F with/without frequency hopping towards a TCH/F with/without frequency hopping  and,

-	the mode of  either the current or the target channel is set to full rate speech version 2 (enhanced full rate speech).

To test that the MS also supporting half rate shall correctly apply the handover procedure in the non-synchronized case when :

-	a call is in progress and,

-	a handover is performed between a TCH/H with/without frequency hopping and  a TCH/F with/without frequency hopping and,

-	the mode of the TCH/F is set to  full rate speech version 2.

26.12.2.1.3	Method of test

Initial Conditions

System Simulator:

2 cells, A and B with same LAI, default parameters except:

GSM900 :

Cell A has :	BCCH ARFCN = 20

Cell Allocation = (10, 17, 20, 26, 34, 42, 45, 46, 52, 59, 66, 73, 74, 75, 76, 108, 114)

Cell B has :	BCCH ARFCN = 40

Cell Allocation = (14, 18, 22, 24, 30, 31, 38, 40, 60, 66, 73, 74, 75, 76, 108, 114)

The frame numbers of cells A and B shall be different by 100.

The timebase of cells A and B shall be such that the edges of their timeslots are not coincident at the antenna connector.

DCS1800 :

Cell A has :	BCCH ARFCN = 747

Cell Allocation = ( 734, 741, 747, 754, 759, 762, 766, 767, 773, 775, 779, 782, 791, 798, 829, 832, 844 )

Cell B has :	BCCH ARFCN = 764

Cell Allocation = ( 739, 743, 746, 749, 756, 758, 761, 764, 771, 779, 782, 791, 798, 829, 832, 844 )

The Cell Allocation of both Cell A and Cell B shall be coded using range 256 format.

The frame numbers of cells A and B shall be different by 100.

The timebase of cells A and B shall be such that the edges of their timeslots are not coincident at the antenna connector.

Mobile Station:	The MS is in the active state (U10) of a call on cell A.

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/F and/or TCH/H.

Support for state U10 of the Call Control protocol.

Support  for full rate speech version 2 (enhanced full rate speech)

supported radio interface rates : 12kbps, 6kbps, 3,6kbps

Type of Mobile Station ( P-GSM900 or EGSM or DCS1800 ).

Foreseen Final State of the MS

The active state (U10) of a call on cell A.

Test Procedure

This procedure is repeated for execution counter M = 1 to 15.

The MS is in the active state (U10) of a call. The SS sends a HANDOVER COMMAND on the main DCCH. The MS shall (at the time defined by the Starting Time information element, if included in the message) begin to send access bursts on the new DCCH of the target cell. The SS observes the access bursts and after receiving n (n being arbitrarily chosen between values according to table 2 of section 26.12.2) access bursts, the SS sends one PHYSICAL INFORMATION message with a Timing Advance as specified in table 1 of section 26.12.2. The MS shall activate the channel in sending and receiving mode. The MS shall establish a signalling link. The MS shall be ready to transmit a HANDOVER COMPLETE message, before 'x' MS after the end of the PHYSICAL INFORMATION message, but not before a UA frame has been sent by the SS.

The term 'ready to transmit' is defined in TS GSM 04.13. The value of 'x' depends upon the target channel and is specified in the specific message contents section.

Maximum Duration of Test

10 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

This sequence is performed for an execution counter M = 1, 2, 9 for an MS which supports  enhanced full speech codec and only TCH/F .

This sequence is performed for an execution counter M = 1, 2.. 15 for an MS which supports  enhanced full speech codec and TCH/F and H.

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 0 | MS -> SS |  | The MS and SS are in the active state of a call on the channel described below. |
| 1 | SS -> MS | HANDOVER COMMAND | See Specific message contents |
| 2 | MS -> SS | HANDOVER ACCESS | Repeated on every burst of the uplink main DCCH until reception of PHYSICAL INFORMATION. Handover Reference as included in the HANDOVER COMMAND. If the HANDOVER COMMAND includes a starting time IE then the first HANDOVER ACCESS message shall be transmitted in the indicated frame (unless the indicated frame is not used by that channel, in which case the next frame used by that channel shall be used) |
| 3 | SS -> MS | PHYSICAL INFORMATION | Sent after reception of n HANDOVER ACCESS messages. See specific message contents. |
| 4 | MS -> SS | SABM | Sent without information field |
| 5 | SS -> MS | UA |  |
| 6 | MS -> SS | HANDOVER COMPLETE | The message shall be ready to be transmitted before 'x' ms after the completion of step 3. |
| 7 | MS -> SS |  | The MS and SS are in the active state of a call on the TCH described below. The SS checks that the TCH is through connected in the correct mode. |

Specific Message Contents

For M = 1 :

P-GSM900

Step 0 : The MS and SS are using a full rate TCH with speech full rate version 2 and in non-hopping mode on cell A.

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base Station Colour Code | 5 |
| - BCCH Carrier Number | 40 |
| Synchronisation Indication |  |
| - Report Observed Time Difference | Shall not be included. |
| - Synchronisation Indication | 'Non synchronised'. |
| - Normal Cell Indication | Ignore out of range timing advance. |

PHYSICAL INFORMATION

| Information Element | value/remarks |
| --- | --- |
| As default message contents. |  |

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH with speech full rate version 2 and in non hopping mode on cell B.

DCS1800

Step 0 : The MS and SS are using a full rate TCH with speech full rate version 2 and in non-hopping mode on cell A.

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base Station Colour Code | 5 |
| - BCCH Carrier Number | 764 |
| Synchronisation Indication |  |
| - Report Observed Time Difference | Shall not be included. |
| - Synchronisation Indication | 'Non synchronised'. |
| - Normal Cell Indication | Ignore out of range timing advance. |

PHYSICAL INFORMATION

| Information Element | value/remarks |
| --- | --- |
| As default message contents. |  |

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH with speech full rate version 2 and in non hopping mode on cell B.

For M = 2 :

GSM900

Step 0 : The MS and SS are using a full rate TCH  with speech full rate version 2 and in non-hopping mode on cell B.

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base Station Colour Code | 5 |
| - BCCH Carrier Number | 20 |
| Channel Description |  |
| - Channel Type | TCH/F + ACCHs |
| - Timeslot number | Chosen arbitrarily, but not Zero |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | RF hopping channel. |
| - MAIO | Chosen arbitrarily from the set ( 0, 1 to N-1 ), where N is the number of frequencies encoded in the Frequency Channel Sequence IE. |
| - HSN | Chosen arbitrarily from the set ( 1,2,..63 ) |
| Synchronisation Indication IE is not included. |  |
| Channel Mode IE is not included. |  |
| Frequency Channel Sequence after time |  |
| - Frequency Channel Sequence | Allocates the following 12 frequencies ( 10, 17, 20, 26, 59, 66, 73, 74, 75, 76,108, 114 ) |

PHYSICAL INFORMATION

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Timing advance | Arbitrarily selected but different to default value. |

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH with speech full rate version 2 and in hopping mode on cell A.

DCS1800

Step 0 : The MS and SS are using a full rate TCH with speech full rate version 2 and in non-hopping mode on cell B.

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base Station Colour Code | 5 |
| - BCCH Carrier Number | 747 |
| Channel Description |  |
| - Channel Type | TCH/F + ACCHs |
| - Timeslot number | Chosen arbitrarily, but not Zero |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | RF hopping channel. |
| - MAIO | Chosen arbitrarily from the set ( 0, 1 to N-1 ), where N is the number of frequencies encoded in the Frequency Short List IE. |
| - HSN | Chosen arbitrarily from the set ( 1,2,..63 ) |
| Synchronisation Indication IE is not included. |  |
| Channel Mode IE is not included. |  |
| Frequency Short List after time |  |
| - Frequency List | Use Range 256 to encode the following 9 frequencies : ( 747, 775, 779, 782, 791, 798, 829, 832, 844 ) |

PHYSICAL INFORMATION

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Timing advance | Arbitrarily selected but different to default value. |

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH with speech full rate version 2 and in hopping mode on cell A.

For M = 3 :

GSM900

Step 0 : The MS and SS are using a full rate TCH ith speech full rate version 2 and in hopping mode on cell A.

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base Station Colour Code | 5 |
| - BCCH Carrier Number | 40 |
| Synchronisation Indication |  |
| - Report Observed Time Difference | Shall not be included. |
| - Synchronisation Indication | 'Non synchronised'. |
| - Normal Cell Indication | Out of range timing advance shall trigger a handover failure procedure. |
| Starting Time | Indicates the frame number of cell B that will occur approximately 1,1 seconds ( 238 frames have elapsed ) after the HANDOVER COMMAND is sent by cell A. |

PHYSICAL INFORMATION

| Information Element | value/remarks |
| --- | --- |
| As default message contents. |  |

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH  with speech full rate version 2 and in non-hopping mode on cell B.

DCS1800

Step 0 : The MS and SS are using a full rate TCH  with speech full rate version 2 and in hopping mode on cell A.

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base Station Colour Code | 5 |
| - BCCH Carrier Number | 764 |
| Synchronisation Indication |  |
| - Report Observed Time Difference | Shall not be included. |
| - Synchronisation Indication | 'Non synchronised'. |
| - Normal Cell Indication | Out of range timing advance shall trigger a handover failure procedure. |
| Starting Time | Indicates the frame number of cell B that will occur approximately 1,1 seconds ( 238 frames have elapsed ) after the HANDOVER COMMAND is sent by cell A. |

PHYSICAL INFORMATION

| Information Element | value/remarks |
| --- | --- |
| As default message contents. |  |

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH with speech version 2 and in non-hopping mode on cell B.

For M = 4 :

P-GSM900

Step 0 : The MS and SS are using a full rate TCH with  full rate speech version 2 and in non-hopping mode on cell  B.

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base Station Colour Code | 5 |
| - BCCH Carrier Number | 20 |
| Synchronisation Indication |  |
| - Report Observed Time Difference | Shall not be included. |
| - Synchronisation Indication | 'Non synchronised'. |
| - Normal Cell Indication | Ignore out of range timing advance. |
| Mode of the first channel |  |
| - Mode | speech full rate version 1 |

PHYSICAL INFORMATION

| Information Element | value/remarks |
| --- | --- |
| As default message contents. |  |

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH with speech full rate version 1  and in non hopping mode on cell A.

DCS1800

Step 0 : The MS and SS are using a full rate TCH with speech full rate version 2 and in non-hopping mode on cell B.

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base Station Colour Code | 5 |
| - BCCH Carrier Number | 747 |
| Synchronisation Indication |  |
| - Report Observed Time Difference | Shall not be included. |
| - Synchronisation Indication | 'Non synchronised'. |
| - Normal Cell Indication | Ignore out of range timing advance. |
| Mode of the first channel |  |
| - Mode | speech full rate version 1 |

PHYSICAL INFORMATION

| Information Element | value/remarks |
| --- | --- |
| As default message contents. |  |

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH with speech full rate version 1 and in non hopping mode on cell A.

For M = 5 :

GSM900

Step 0 : The MS and SS are using a full rate TCH with full rate speech version 1 and in non-hopping mode on cell  A.

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base Station Colour Code | 5 |
| - BCCH Carrier Number | 40 |
| Synchronisation Indication |  |
| - Report Observed Time Difference | Shall not be included. |
| - Synchronisation Indication | 'Non synchronised'. |
| - Normal Cell Indication | Ignore out of range timing advance. |

PHYSICAL INFORMATION

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : Timing advance | Arbitrarily selected but different to default value. |

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH with speech full rate version 2  and in non hopping mode on cell  B.

DCS1800

Step 0 : The MS and SS are using a full rate TCH with speech full rate version 1 and  in non-hopping mode on cell A.

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base Station Colour Code | 5 |
| - BCCH Carrier Number | 764 |
| Synchronisation Indication |  |
| - Report Observed Time Difference | Shall not be included. |
| - Synchronisation Indication | 'Non synchronised'. |
| - Normal Cell Indication | Ignore out of range timing advance. |

PHYSICAL INFORMATION

| Information Element | value/remarks |
| --- | --- |
| As default message contents. |  |

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH with speech full rate version 2 mode and non hopping mode on cell B.

For M=6 :

GSM900

Step 0 : The MS and SS are using a full rate TCH with speech full rate version 2 and non-hopping mode on cell B.

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base Station Colour Code | 5 |
| - BCCH Carrier Number | 20 |
| Channel Description |  |
| - Channel Type | TCH/F + ACCHs |
| - Timeslot number | Chosen arbitrarily, but not Zero |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | RF hopping channel. |
| - MAIO | Chosen arbitrarily from the set ( 0, 1 to N-1 ), where N is the number of frequencies encoded in the Frequency Channel Sequence IE. |
| - HSN | Chosen arbitrarily from the set ( 1,2,..63 ) |
| Synchronisation Indication IE is not included. |  |
| Mode of the first channel. |  |
| - Mode | speech full rate version 1 |
| Frequency Channel Sequence after time |  |
| - Frequency Channel Sequence | Allocates the following 12 frequencies ( 10, 17, 20, 26, 59, 66, 73, 74, 75, 76,108, 114 ) |

PHYSICAL INFORMATION

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Timing advance | Arbitrarily selected but different to default value. |

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH with speech full rate version 1 and  hopping mode on cell A.

DCS1800

Step 0 : The MS and SS are using a full rate TCH  with full rate speech version 2 and in non-hopping mode on cell B.

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base Station Colour Code | 5 |
| - BCCH Carrier Number | 747 |
| Channel Description |  |
| - Channel Type | TCH/F + ACCHs |
| - Timeslot number | Chosen arbitrarily, but not Zero |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | RF hopping channel. |
| - MAIO | Chosen arbitrarily from the set ( 0, 1 to N-1 ), where N is the number of frequencies encoded in the Frequency Short List IE. |
| - HSN | Chosen arbitrarily from the set ( 1,2,..63 ) |
| Synchronisation Indication IE is not included. |  |
| Channel Mode IE is not included. |  |
| Frequency Short List after time |  |
| - Frequency List | Use Range 256 to encode the following 9 frequencies : ( 747, 775, 779, 782, 791, 798, 829, 832, 844 ) |

PHYSICAL INFORMATION

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Timing advance | Arbitrarily selected but different to default value. |

Step 6 : x = 500

Step 7 : The MS and SS are using a full rate TCH with full rate speech version 1 and in hopping mode on cell A.

For M=7 :

GSM900

Step 0 : The MS and SS are using a full rate TCH with full rate speech version 1 and in hopping mode on cell A.

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base Station Colour Code | 5 |
| - BCCH Carrier Number | 40 |
| Channel Description |  |
| - Channel Type | TCH/F + ACCHs |
| - TDMA offset | Chosen arbitrarily |
| - Timeslot number | Chosen arbitrarily, but not Zero |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | RF hopping channel. |
| - MAIO | Chosen arbitrarily from the set ( 0, 1 to N-1 ), where N is the number of frequencies encoded in the Frequency List IE. |
| - HSN | Chosen arbitrarily from the set |
|  | ( 1,2,..63 ) |
| Frequency List after time |  |
| - Frequency List | use bit map 0 to allocates the following 12 frequencies : ( 14, 18, 22, 24, 60, 66, 73, 74, 75, 76,108, 114 ) |
| Synchronisation Indication |  |
| - Report Observed Time Difference | Shall not be included. |
| - Synchronisation Indication | 'Non synchronised'. |
| - Normal Cell Indication | Ignore out of range timing advance. |
| Starting Time | Indicates the frame number of cell B. that will occur approximately 1,1 seconds ( 238 frames have elapsed ) after the HANDOVER COMMAND is sent by cell A. |

PHYSICAL INFORMATION

| Information Element | value/remarks |
| --- | --- |
| As default message contents. |  |

Step 6 : x = 750

Step 7 : The MS and SS are using a full rate TCH with full rate speech version 2 and in hopping mode on cell B.

DCS1800

Step 0 : The MS and SS are using a full rate TCH with full rate speech version 1 and in hopping mode on cell A.

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base Station Colour Code | 5 |
| - BCCH Carrier Number | 764 |
| Channel Description |  |
| - Channel Type | TCH/F + ACCHs |
| - TDMA offset | Chosen arbitrarily |
| - Timeslot number | Chosen arbitrarily but not Zero |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | RF hopping channel. |
| - MAIO | Chosen arbitrarily from the set ( 0, 1 to N-1 ), where N is the number of frequencies encoded in the Frequency List IE. |
| - HSN | Chosen arbitrarily from the set ( 1,2,..63 ) |
| Frequency List after time |  |
| - Frequency List | Use Range 1024 to allocate the following 12 frequencies : (749, 758, 761, 764, 771, 779, 782, 791, 798, 829, 832, 844 ) |
| Synchronisation Indication |  |
| - Report Observed Time Difference | Shall not be included. |
| - Synchronisation Indication | 'Non synchronised'. |
| - Normal Cell Indication | Ignore out of range timing advance. |
| Starting Time | Indicates the frame number of cell B that will occur approximately 1,1 seconds ( 238 frames have elapsed ) after the HANDOVER COMMAND is sent by cell A. |

PHYSICAL INFORMATION

| Information Element | value/remarks |
| --- | --- |
| As default message contents. |  |

Step 6 : x = 750

Step 7 : The MS and SS are using a full rate TCH with full rate speech version 2 and in hopping mode on cell B.

For M = 8 :

GSM900

Step 0 : The MS and SS are using a full rate TCH with full rate speech version 2 and in hopping mode on cell B.

HANDOVER COMMAND

same as for M=6

PHYSICAL INFORMATION

same as for M=6

Step 6 : x = 750

Step 7 : The MS and SS are using a full rate TCH with speech full rate version 1 and in hopping mode on cell A.

DCS1800

Step 0 : The MS and SS are using a full rate TCH with full rate speech version 2 and in hopping mode on cell B.

HANDOVER COMMAND

same as for M=6

PHYSICAL INFORMATION

same as for M=6

Step 6 : x = 750

Step 7 : The MS and SS are using a full rate TCH with speech full rate version 1 and in hopping mode on cell A.

For M = 9 :

GSM900

Step 0 : The MS and SS are using a full rate TCH with speech full rate version 1 and in hopping mode on cell A.

HANDOVER COMMAND

same as for M=5.

PHYSICAL INFORMATION

same as For M=5.

Step 6 : x = 750

Step 7 : The MS and SS are using a full rate TCH with speech full rate version 2 and in non hopping mode on cell B.

DCS1800

Step 0 : The MS and SS are using a full rate TCH with speech full rate version 1 and in hopping mode on cell A.

HANDOVER COMMAND

same as for M=5.

PHYSICAL INFORMATION

same as for M=5.

Step 6 : x = 750

Step 7 : The MS and SS are using a full rate TCH with speech full rate version 2 and in non hopping mode on cell B.

For M=10 :

GSM900

Step 0 : The MS and SS are using a full rate TCH with full rate speech version 2 and in non hopping mode on cell B.

HANDOVER COMMAND

same as for M=6 except :

Channel Description

- Channel Type		TCH/H + ACCHs

PHYSICAL INFORMATION

same as for M=6

Step 6 : x = 750

Step 7 : The MS and SS are using a half rate TCH with speech full rate version 1 and in hopping mode on cell A.

DCS1800

Step 0 : The MS and SS are using a full rate TCH with full rate speech version 2 and in non hopping mode on cell B.

HANDOVER COMMAND

same as for M=6 except :

Channel Description

- Channel Type		TCH/H + ACCHs

PHYSICAL INFORMATION

same as for M=6

Step 6 : x = 750

Step 7 : The MS and SS are using a half rate TCH with speech full rate version 1 and in hopping mode on cell A.

For M = 11 :

GSM900

Step 0 : The MS and SS are using a half rate TCH with speech full rate version 1 and in hopping mode on cell A.

HANDOVER COMMAND

same as for M=7

PHYSICAL INFORMATION

same as For M=7.

Step 6 : x = 750

Step 7 : The MS and SS are using a full rate TCH with speech full rate version 2 and in hopping mode on cell B.

DCS1800

Step 0 : The MS and SS are using a half rate TCH with speech full rate version 1 and in hopping mode on cell A.

HANDOVER COMMAND

same as for M=7

PHYSICAL INFORMATION

same as for M=7.

Step 6 : x = 750

Step 7 : The MS and SS are using a full rate TCH with speech full rate version 2 and in hopping mode on cell B.

For M = 12 :

GSM900

Step 0 : The MS and SS are using a full rate TCH with full rate speech version 2 and in hopping mode on cell B.

HANDOVER COMMAND

same as for M=6 except :

| Channel Description |  |
| --- | --- |
| - Channel Type | TCH/H + ACCHs |

PHYSICAL INFORMATION

same as for M=6

Step 6 : x = 750

Step 7 : The MS and SS are using a half rate TCH with speech full rate version 1 and in hopping mode on cell A.

DCS1800

Step 0 : The MS and SS are using a full rate TCH with full rate speech version 2 and in hopping mode on cell B.

HANDOVER COMMAND

same as for M=6 except :

| Channel Description |  |
| --- | --- |
| - Channel Type | TCH/H + ACCHs |

PHYSICAL INFORMATION

same as for M=6

Step 6 : x = 750

Step 7 : The MS and SS are using a half rate TCH with speech full rate version 1 and in hopping mode on cell A.

For M = 13 :

GSM900

Step 0 : The MS and SS are using a half rate TCH with speech full rate version 1 and in hopping mode on cell A.

HANDOVER COMMAND

same as for M=1

PHYSICAL INFORMATION

same as For M=1.

Step 6 : x = 750

Step 7 : The MS and SS are using a full rate TCH with speech full rate version 2 and in non hopping mode on cell B.

DCS1800

Step 0 : The MS and SS are using a half rate TCH with speech full rate version 1 and in hopping mode on cell A.

HANDOVER COMMAND

same as for M=1

PHYSICAL INFORMATION

same as for M=1.

Step 6 : x = 750

Step 7 : The MS and SS are using a full rate TCH with speech full rate version 2 and in non hopping mode on cell B.

For M = 14 :

GSM900

Step 0 : The MS and SS are using a full rate TCH with full rate speech version 2 and in non hopping mode on cell B.

HANDOVER COMMAND

same as for M=4 except :

| Channel Description |  |
| --- | --- |
| - Channel Type | TCH/H + ACCHs |

PHYSICAL INFORMATION

same as for M=4

Step 6 : x = 750

Step 7 : The MS and SS are using a half rate TCH with speech full rate version 1 and in non hopping mode on cell A.

DCS1800

Step 0 : The MS and SS are using a full rate TCH with full rate speech version 2 and in non hopping mode on cell B.

HANDOVER COMMAND

same as for M=4 except :

| Channel Description |  |
| --- | --- |
| - Channel Type | TCH/H + ACCHs |

PHYSICAL INFORMATION

same as for M=4

Step 6 : x = 750

Step 7 : The MS and SS are using a half rate TCH with speech full rate version 1 and in non hopping mode on cell A.

For M = 15 :

GSM900

Step 0 : The MS and SS are using a half rate TCH with speech full rate version 1 and in hopping mode on cell A.

HANDOVER COMMAND

same as for M=1

PHYSICAL INFORMATION

same as For M=1.

Step 6 : x = 750

Step 7 : The MS and SS are using a full rate TCH with speech full rate version 2 and in non hopping mode on cell B.

DCS1800

Step 0 : The MS and SS are using a half rate TCH with speech full rate version 1 and in hopping mode on cell A.

HANDOVER COMMAND

same as for M=1

PHYSICAL INFORMATION

same as for M=1.

Step 6 : x = 750

Step 7 : The MS and SS are using a full rate TCH with speech full rate version 2 and in non hopping mode on cell B.

#### 26.12.2.2	EFR signalling/ Handover / successful / call under establishment / non-synchronized

NOTE:	This test is derived from the one described in section 26.6.5.2 and entitled “Handover/successful/ call under establishment/ non-synchronized”.

This test only applies to MS supporting speech full rate version 2 (enhanced full rate) and at least one MO circuit switched basic service.

26.12.2.2.1	Conformance requirements

1.	The MS shall correctly apply the handover procedure from SDCCH/8 with or without frequency hopping to TCH/F  using full rate speech version 2 with or without frequency hopping in the non-synchronized case during call establishment. The mobile shall correctly apply the handover procedures from non frequency hopping SDCCH/4  to TCH/F using full rate speech version 2 with or without frequency hopping.

2.	If during call establishment a Layer 3 MM or CC message just sent by the MS is not Layer 2 acknowledged before the channel change caused by the HANDOVER COMMAND message, the MS shall send the Layer 3 message to the new cell, using the same value in the N(SD) field, after the handover procedure.

References

GSM 04.08, sections 3.1.4.1, 3.4.4 and 9.1.15

GSM 04.13, section 5.2.6.2

26.12.2.2.2	Test purpose

1.	To verify that the MS  correctly applies the handover procedure from SDCCH/8 with or without frequency hopping to TCH/F  using full rate speech version 2 with or without frequency hopping in the non-synchronized case during call establishment. To verify that the mobile correctly applies the handover procedures from non frequency hopping SDCCH/4  to TCH/F using full rate speech version 2 with or without frequency hopping.

2.	To verify that if during call establishment a Layer 3 MM or CC message just sent by the MS is not Layer 2 acknowledged before the channel change caused by the HANDOVER COMMAND message, the MS shall send the Layer 3 message to the new cell, using the same value in the N(SD) field, after the handover procedure.

26.12.2.2.3	Method of test

Initial Conditions

System Simulator:

2 cells A and B with same LAI, default parameters, except:

GSM900 :

Cell A has :	BCCH ARFCN = 20

Cell Allocation = (10, 17, 20, 26, 34, 42, 45, 46, 52, 59, 66, 73, 74, 75, 76, 108, 114)

PLMN colour code, NCC = as defaults.

BS colour code, BCC = as defaults.

Cell B has :	BCCH ARFCN = 40

Cell Allocation = (14, 18, 22, 24, 30, 31, 38, 40, 60, 66, 73, 74, 75, 76, 108, 114)

PLMN colour code, NCC = 3.

BS colour code, BCC = 0.

Both cells send SYSTEM INFORMATION TYPE 1 messages containing the complete Cell Allocation of the cell, using bit map 0 format.

The timebase of Cells A and B shall be such that the edges of their timeslots are not coincident at the antenna connector.

For execution counter M = 1 a combined CCH/SDCCH is used.

For execution counter M = 2 and 3 a non combined SDCCH is used.

DCS1800 :

Cell A has :	BCCH ARFCN = 747

Cell Allocation = (734, 741, 747, 754, 759, 762, 766, 767, 773, 775, 779, 782, 791, 798, 829, 832, 844)

PLMN colour code, NCC = as defaults.

BS colour code, BCC = as defaults.

Cell B has :	BCCH ARFCN = 764

Cell Allocation = ( 739, 743, 746, 749, 756, 758, 761, 764, 771, 779, 782, 791, 798, 829, 832, 844)

PLMN colour code, NCC = 3.

BS colour code, BCC = 0.

Both cells send SYSTEM INFORMATION TYPE 1 messages containing the complete Cell Allocation of the cell, using Range 512 format.

The timebase of Cells A and B shall be such that the edges of their timeslots are not coincident at the antenna connector.

For execution counter M = 1 a combined CCH/SDCCH is used.

For execution counter M = 2 and 3 a non combined SDCCH is used.

Mobile Station:	The MS is in the "idle, updated" state, with a TMSI allocated and camped on cell A.

Related PICS/PIXIT Statements

Supported rate(s) of TCH: TCH/FS and/or TCH/HS.

Support for state U10 of the Call Control protocol.

Supported speech full rate version 2 (enhanced full rate)

Type of MS (P-GSM900 or EGSM or DCS1800).

Foreseen Final State of the MS

"Idle, updated" with TMSI allocated and camped on cell B.

Test Procedure

This procedure is repeated for execution counter M = 1 through 11 ( See Table 1 of section 26.12.2 )

A Mobile Originating Call is initiated on Cell A. After the MS has sent the SETUP message (and before the last L2 frame carrying the SETUP message is acknowledged by the SS) the SS sends a HANDOVER COMMAND message, ordering the MS to switch to cell B. The MS shall then begin to send access bursts on the new DCCH to cell B. The SS observes the access bursts and after receiving n (n being arbitrarily chosen between values according to table 2 of section 26.12.2) access bursts, the SS sends one PHYSICAL INFORMATION message with a Timing Advance as specified in table 1 of section 26.12.2.The MS shall activate the channel in sending and receiving mode. The MS shall establish a signalling link. The MS shall be ready to transmit a HANDOVER COMPLETE message before x ms after the end of the PHYSICAL INFORMATION message, but not before a UA frame has been sent by the SS. The MS shall then again send the SETUP message to the SS, using the same value in the N(SD) field. Finally the SS sends a CHANNEL RELEASE to end the test.

The term 'ready to transmit' is defined in TS GSM 04.13. The value of 'x' depends upon the target channel and is specified in the specific message contents section.

Maximum Duration of Test

5 minutes, including 1 minute for any necessary operator actions.

Expected Sequence

The sequence is performed for execution counter M = 1,through 11.

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | -------- | -------- | A MO call is initiated on cell A. |
| 2 | MS -> SS | CHANNEL REQUEST | Establ. Cause = "Originating call, NECI not set to 1 |
| 3 | SS -> MS | IMMEDIATE ASSIGNMENT | See specific message contents. |
| 4 | MS -> SS | CM SERVICE REQUEST | CM Service Type = Mobile Originating Call Establishment. |
| 5 | SS -> MS | CIPHERING MODE COMMAND |  |
| 6 | MS -> SS | CIPHERING MODE COMPLETE |  |
| 7 | MS -> SS | SETUP | Last L2 frame not acknowledged by the SS. |
| 8 | SS -> MS | HANDOVER COMMAND | See specific message contents. |
| 9 | MS -> SS | HANDOVER ACCESS | Repeated on every burst of the uplink main DCCH until reception of PHYSICAL INFORMATION. Handover Reference as included in the HANDOVER COMMAND |
| 10 | SS -> MS | PHYSICAL INFO | Sent after reception of n HANDOVER ACCESS message. Timing Advance as specified in table 1 of section 26.12.2. |
| 11 | MS -> SS | SABM | Sent without information field |
| 12 | SS -> MS | UA |  |
| 13 | MS -> SS | HANDOVER COMPLETE | This message shall be ready to be transmitted before 'x' ms after the completion of step 10. |
| 14 | MS -> SS | SETUP | Same N(SD) as in step 7. |
| 15 | SS -> MS | CHANNEL RELEASE |  |

Specific Message Contents

M = 1

DCS1800 :

IMMEDIATE ASSIGNMENT

| Information Element | value/remark |
| --- | --- |
| As default message contents. |  |

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 3 |
| - Base Station Colour Code | 0 |
| - BCCH Carrier Number | 764 |
| Channel Description |  |
| - Channel Type | TCH/F + ACCHs |
| - Timeslot number | zero |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | RF hopping channel. |
| - MAIO | Chosen arbitrarily from the set ( 0, 1 to N-1 ), where N is the number of frequencies encoded in the Frequency Short List IE. |
| - HSN | Chosen arbitrarily from the set |
|  | ( 1,2,..63 ) |
| Synchronisation IE is not included. |  |
| Frequency Short List after time |  |
| - Frequency Short List | Use Range 128 to encode the following 11 frequencies : ( 756, 758, 761, 771, 779, 782, 791, 798, 829, 832, 844 ) |

Step 13 : 'x' = 500

For GSM900 :

IMMEDIATE ASSIGNMENT

| Information Element | value/remark |
| --- | --- |
| As default message contents. |  |

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 3 |
| - Base Station Colour Code | 0 |
| - BCCH Carrier Number | 40 |
| Channel Description |  |
| - Channel Type | TCH/F + ACCHs |
| - Timeslot number | Zero |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | RF hopping channel. |
| - MAIO | Chosen arbitrarily from the set ( 0, 1 to N-1 ), where N is the number of frequencies encoded in the Frequency Channel Sequence IE. |
| - HSN | Chosen arbitrarily from the set |
|  | ( 1,2,..63 ) |
| Synchronisation IE is not included. |  |
| Frequency Channel Sequence, after time |  |
| - Frequency Channel Sequence | Allocates the following 15 frequencies : (14, 18, 22, 24, 30, 31, 38, 53, 66, 73, 74, 75, 76, 108, 114 ) |

Step 13 : 'x' = 500

M = 2 :

DCS1800 :

IMMEDIATE ASSIGNMENT

| Information Element | value/remark |
| --- | --- |
| As default message contents except : |  |
| L2 pseudo length | 14 octets ( 11 + contents of the MA ) |
| Channel Description | Channel Description |
| - Channel Type | SDCCH/8 |
| - TDMA offset | As default message contents |
| - Timeslot number | Arbitrary value but not zero. |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | RF hopping channel. |
| - MAIO | Chosen arbitrarily from the set ( 0, 1 to N-1 ), where N is the number of frequencies encoded in the Mobile Allocation. |
| - HSN | Chosen arbitrarily from the set |
|  | ( 1,2,..63 ) |
| Mobile Allocation |  |
| - Length | 3octets. |
| - Contents | Indicates only three frequencies : ( 773, 775, 779 ). |

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 3 |
| - Base Station Colour Code | 0 |
| - BCCH Carrier Number | 764 |
| Channel Description |  |
| - Channel Type | TCH/F + ACCHs |
| - Timeslot number | Zero |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | Single RF Channel |
| - ARFCN | Chosen arbitrarily from the Cell Allocation of Cell B, but not the BCCH carrier of Cell B. |
| Synchronisation Indication IE not included. |  |

Step 13 : 'x' = 500

GSM900 :

IMMEDIATE ASSIGNMENT

| Information Element | value/remark |
| --- | --- |
| As default message contents except : |  |
| L2 pseudo length | 14 octets ( 11 + contents of the MA ) |
| Channel Description |  |
| - Channel Type | SDCCH/8 |
| - TDMA offset | As default message contents |
| - Timeslot number | Arbitrary value, but not zero. |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | RF hopping channel. |
| - MAIO | Chosen arbitrarily from the set ( 0, 1 to N-1 ), where N is the number of frequencies encoded in the Mobile Allocation. |
| - HSN | Chosen arbitrarily from the set ( 1,2,..63 ) |
| Mobile Allocation |  |
| - Length | 3 octets. |
| - Contents | Indicates only three frequencies : ( 73, 74, 75 ). |

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 3 |
| - Base Station Colour Code | 0 |
| - BCCH Carrier Number | 40 |
| Channel Description |  |
| - Channel Type | TCH/F + ACCHs |
| - Timeslot number | Zero |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | Single RF Channel |
| - ARFCN | Chosen arbitrarily from the Cell Allocation of Cell B, but not the BCCH carrier of Cell B. |

Step 13 : 'x' = 500

M = 3 :

DCS1800 :

IMMEDIATE ASSIGNMENT

| Information Element | value/remark |
| --- | --- |
| As default message contents. |  |

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 3 |
| - Base Station Colour Code | 0 |
| - BCCH Carrier Number | 764 |
| Channel Description |  |
| - Channel Type | TCH/F + ACCHs |
| - Timeslot number | Arbitrary value, but not zero. |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | RF hopping channel. |
| - MAIO | Chosen arbitrarily from the set ( 0, 1 to N-1 ), where N is the number of frequencies encoded in the Frequency Short List IE. |
| - HSN | Zero ( this gives cyclic hopping ) |
| Frequency Short List after time |  |
| - Frequency Short List | Use Range 256 to encode the following 3 frequencies : ( 764, 779, 782 ) |
| Synchronisation Indication |  |
| - Report Observed Time Difference | Shall not be included. |
| - Synchronisation Indication | 'Non synchronised'. |
| - Normal Cell Indication | Ignore out of range timing advance. |

Step 13 : 'x' = 500

GSM900 :

IMMEDIATE ASSIGNMENT

| Information Element | value/remark |
| --- | --- |
| As default message contents. |  |

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 3 |
| - Base Station Colour Code | 0 |
| - BCCH Carrier Number | 40 |
| Channel Description |  |
| - Channel Type | TCH/F + ACCHs |
| - Timeslot number | zero |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | RF hopping channel |
| - MAIO | Chosen arbitrarily from the set {0, 1}. |
| - HSN | Chosen arbitrarily from the set {1, 2,..., 63}. |
| Synchronisation Indication IE is not included |  |
| Channel Mode IE | speech full rate |
| Frequency Channel Sequence, after time. |  |
| - Frequency Channel Sequence IE | allocates the following two frequencies {14, 114} |

Step 13 : 'x' = 500

M =4 :

DCS1800 :

IMMEDIATE ASSIGNMENT

| Information Element | value/remark |
| --- | --- |
| As default message contents except : |  |
| L2 pseudo length | 11 |
| Channel Description | Channel Description |
| - Channel Type | TCH/F |
| - TDMA offset | As default message contents |
| - Timeslot number | Arbitrary value but not zero. |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | single RF channel. |
| - ARFCN | 754 |

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base Station Colour Code | 5 |
| - BCCH Carrier Number | 764 |
| Channel Description |  |
| - Channel Type | TCH/F + ACCHs |
| - TDMA offset | Chosen arbitrarily |
| - Timeslot number | Chosen arbitrarily, but not Zero |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | RF hopping channel. |
| - MAIO | Chosen arbitrarily from the set ( 0, 1 to N-1 ), where N is the number of frequencies encoded in the Frequency List IE. |
| - HSN | Chosen arbitrarily from the set |
|  | ( 1,2,..63 ) |
| Frequency List after time |  |
| - Frequency List | use bit map 0 to allocates the following 12 frequencies : (746, 756, 758, 761, 764, 771, 779, 782, 798, 829, 832, 844 ) |

Step 13 : 'x' = 500

GSM900 :

IMMEDIATE ASSIGNMENT

| Information Element | value/remark |
| --- | --- |
| As default message contents except : |  |
| L2 pseudo length | 11 |
| Channel Description | Channel Description |
| - Channel Type | TCH/F |
| - TDMA offset | As default message contents |
| - Timeslot number | Arbitrary value but not zero. |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | single RF channel. |
| - ARFCN | 66 |

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base Station Colour Code | 5 |
| - BCCH Carrier Number | 40 |
| Channel Description |  |
| - Channel Type | TCH/F + ACCHs |
| - TDMA offset | Chosen arbitrarily |
| - Timeslot number | Chosen arbitrarily, but not Zero |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | RF hopping channel. |
| - MAIO | Chosen arbitrarily from the set ( 0, 1 to N-1 ), where N is the number of frequencies encoded in the Frequency List IE. |
| - HSN | Chosen arbitrarily from the set |
|  | ( 1,2,..63 ) |
| Frequency List after time |  |
| - Frequency List | use bit map 0 to allocates the following 12 frequencies : ( 14, 18, 22, 24, 60, 66, 73, 74, 75, 76,108, 114 ) |
| Synchronisation Indication |  |
| - Report Observed Time Difference | Shall not be included. |
| - Synchronisation Indication | 'Non synchronised'. |
| - Normal Cell Indication | Ignore out of range timing advance. |

Step 13 : 'x' = 500 ??

For  M=5

DCS1800 :

IMMEDIATE ASSIGNMENT

| Information Element | value/remark |
| --- | --- |
| As default message contents except : |  |
| L2 pseudo length | 11 |
| Channel Description | Channel Description |
| - Channel Type | TCH/F |
| - TDMA offset | As default message contents |
| - Timeslot number | Arbitrary value but not zero. |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | single RF channel. |
| - ARFCN | 791 |

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base Station Colour Code | 5 |
| - BCCH Carrier Number | 40 |
| Channel Description |  |
| - Channel Type | TCH/F + ACCHs |
| - TDMA offset | Chosen arbitrarily |
| - Timeslot number | Chosen arbitrarily, but not Zero |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | single RF channel. |
| - ARFCN | 749 |

Step 13 : 'x' = 500

GSM900 :

IMMEDIATE ASSIGNMENT

| Information Element | value/remark |
| --- | --- |
| As default message contents except : |  |
| L2 pseudo length | 11 |
| Channel Description | Channel Description |
| - Channel Type | TCH/F |
| - TDMA offset | As default message contents |
| - Timeslot number | Arbitrary value but not zero. |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | single RF channel. |
| - ARFCN | 66 |

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base Station Colour Code | 5 |
| - BCCH Carrier Number | 40 |
| Channel Description |  |
| - Channel Type | TCH/F + ACCHs |
| - TDMA offset | Chosen arbitrarily |
| - Timeslot number | Chosen arbitrarily, but not Zero |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | single RF channel. |
| - ARFCN | 31 |

Step 13 : 'x' = 500

M=6

DCS1800 :

IMMEDIATE ASSIGNMENT

| Information Element | value/remark |
| --- | --- |
| As default message contents except : |  |
| L2 pseudo length | 14 octets ( 11 + contents of the MA ) |
| Channel Description | Channel Description |
| - Channel Type | TCH/F |
| - TDMA offset | As default message contents |
| - Timeslot number | Arbitrary value but not zero. |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | RF hopping channel. |
| - MAIO | Chosen arbitrarily from the set ( 0, 1 to N-1 ), where N is the number of frequencies encoded in the Mobile Allocation. |
| - HSN | Chosen arbitrarily from the set |
|  | ( 1,2,..63 ) |
| Mobile Allocation |  |
| - Length | 3 octets. |
| - Contents | Indicates only five frequencies : ( 734, 741, 762, 791, 832). |

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 3 |
| - Base Station Colour Code | 0 |
| - BCCH Carrier Number | 764 |
| Channel Description |  |
| - Channel Type | TCH/F + ACCHs |
| - TDMA offset | Chosen arbitrarily |
| - Timeslot number | Chosen arbitrarily, but not Zero |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | RF hopping channel. |
| - MAIO | Chosen arbitrarily from the set ( 0, 1 to N-1 ), where N is the number of frequencies encoded in the Frequency List IE. |
| - HSN | Chosen arbitrarily from the set |
|  | ( 1,2,..63 ) |
| Frequency List after time |  |
| - Frequency List | use range 256 to allocates the following 8 frequencies : ( 739, 743, 746, 749, 761, 771, 782, 791) |
| Synchronisation Indication |  |
| - Report Observed Time Difference | Shall not be included. |
| - Synchronisation Indication | 'Non synchronised'. |
| - Normal Cell Indication | Ignore out of range timing advance. |
| Starting Time | Indicates the frame number of cell B. that will occur approximately 1,1 seconds ( 238 frames have elapsed ) after the HANDOVER COMMAND is sent by cell A. |

Step 13 : 'x' = 500

GSM900 :

IMMEDIATE ASSIGNMENT

| Information Element | value/remark |
| --- | --- |
| As default message contents except : |  |
| L2 pseudo length | 14 octets ( 11 + contents of the MA ) |
| Channel Description |  |
| - Channel Type | TCH/F |
| - TDMA offset | As default message contents |
| - Timeslot number | Arbitrary value, but not zero. |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | RF hopping channel. |
| - MAIO | Chosen arbitrarily from the set ( 0, 1 to N-1 ), where N is the number of frequencies encoded in the Mobile Allocation. |
| - HSN | Chosen arbitrarily from the set ( 1,2,..63 ) |
| Mobile Allocation |  |
| - Length | 3 octets. |
| - Contents | Indicates five frequencies : ( 17, 34, 52, 46, 73 ). |

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 3 |
| - Base Station Colour Code | 0 |
| - BCCH Carrier Number | 40 |
| Channel Description |  |
| - Channel Type | TCH/F + ACCHs |
| - Timeslot number | Zero |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | hopping RF channel |
| - MAIO | Chosen arbitrarily from the set ( 0, 1 to N-1 ), where N is the number of frequencies encoded in the Frequency List IE. |
| - HSN | Chosen arbitrarily from the set |
|  | ( 1,2,..63 ) |
| Frequency List after time |  |
| - Frequency List | use bit map 0 to allocates the following 8 frequencies : ( 14, 18, 24, 30, 38, 60, 73, 74) |
| Synchronisation Indication |  |
| - Report Observed Time Difference | Shall not be included. |
| - Synchronisation Indication | 'Non synchronised'. |
| - Normal Cell Indication | Ignore out of range timing advance. |
| Starting Time | Indicates the frame number of cell B. that will occur approximately 1,1 seconds ( 238 frames have elapsed ) after the HANDOVER COMMAND is sent by cell A. |

Step 13 : 'x' = 500

M = 7

DCS1800 :

IMMEDIATE ASSIGNMENT

| Information Element | value/remark |
| --- | --- |
| As default message contents except : |  |
| L2 pseudo length | 14 octets ( 11 + contents of the MA ) |
| Channel Description | Channel Description |
| - Channel Type | TCH/F |
| - TDMA offset | As default message contents |
| - Timeslot number | Arbitrary value but not zero. |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | RF hopping channel. |
| - MAIO | Chosen arbitrarily from the set ( 0, 1 to N-1 ), where N is the number of frequencies encoded in the Mobile Allocation. |
| - HSN | Chosen arbitrarily from the set |
|  | ( 1,2,..63 ) |
| Mobile Allocation |  |
| - Length | 3 octets. |
| - Contents | Indicates only three frequencies : ( 734, 762, 791). |

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 3 |
| - Base Station Colour Code | 0 |
| - BCCH Carrier Number | 764 |
| Channel Description |  |
| - Channel Type | TCH/F + ACCHs |
| - Timeslot number | Zero |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | Single RF Channel |
| - ARFCN | Chosen arbitrarily from the Cell Allocation of Cell B, but not the BCCH carrier of Cell B. |

Step 13 : 'x' = 500

GSM900 :

IMMEDIATE ASSIGNMENT

| Information Element | value/remark |
| --- | --- |
| As default message contents except : |  |
| L2 pseudo length | 14 octets ( 11 + contents of the MA ) |
| Channel Description |  |
| - Channel Type | SDCCH/8 |
| - TDMA offset | As default message contents |
| - Timeslot number | Arbitrary value, but not zero. |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | RF hopping channel. |
| - MAIO | Chosen arbitrarily from the set ( 0, 1 to N-1 ), where N is the number of frequencies encoded in the Mobile Allocation. |
| - HSN | Chosen arbitrarily from the set ( 1,2,..63 ) |
| Mobile Allocation |  |
| - Length | 3 octets. |
| - Contents | Indicates only three frequencies : ( 73, 74, 75 ). |

HANDOVER COMMAND

| Information Element | value/remarks |
| --- | --- |
| As default message contents, except : |  |
| Cell Description |  |
| - Network Colour Code | 3 |
| - Base Station Colour Code | 0 |
| - BCCH Carrier Number | 40 |
| Channel Description |  |
| - Channel Type | TCH/F + ACCHs |
| - Timeslot number | Zero |
| - Training Sequence Code | Chosen arbitrarily |
| - Hopping | Single RF Channel |
| - ARFCN | Chosen arbitrarily from the Cell Allocation of Cell B, but not the BCCH carrier of Cell B. |

Step 13 : 'x' = 500

M = 8

DCS1800 :

IMMEDIATE ASSIGNMENT

same as for M= 4

except :

| Channel Description |  |
| --- | --- |
| - Channel Type | TCH/H + ACCHs |

HANDOVER COMMAND

same as for M= 4

GSM  900 :

IMMEDIATE ASSIGNMENT

same as for M= 4

except :

| Channel Description |  |
| --- | --- |
| - Channel Type | TCH/H + ACCHs |

HANDOVER COMMAND

same as for M= 4

M = 9

DCS1800 :

IMMEDIATE ASSIGNMENT

same as for M= 5

except :

| Channel Description |  |
| --- | --- |
| - Channel Type | TCH/H + ACCHs |

HANDOVER COMMAND

same as for M= 5

except :

| Starting Time | Indicates the frame number of cell B. that will occur approximately 1,1 seconds ( 238 frames have elapsed ) after the HANDOVER COMMAND is sent by cell A. |
| --- | --- |

GSM  900 :

IMMEDIATE ASSIGNMENT

same as for M= 5

except :

| Channel Description |  |
| --- | --- |
| - Channel Type | TCH/H + ACCHs |

HANDOVER COMMAND

same as for M= 5

M = 10

DCS1800 :

IMMEDIATE ASSIGNMENT

same as for M= 6

except :

| Channel Description |  |
| --- | --- |
| - Channel Type | TCH/H + ACCHs |

HANDOVER COMMAND

same as for M= 6

GSM  900 :

IMMEDIATE ASSIGNMENT

same as for M= 6

except :

| Channel Description |  |
| --- | --- |
| - Channel Type | TCH/H + ACCHs |

HANDOVER COMMAND

same as for M= 6

M = 11

DCS1800 :

IMMEDIATE ASSIGNMENT

same as for M= 7

except :

| Channel Description |  |
| --- | --- |
| - Channel Type | TCH/H + ACCHs |

HANDOVER COMMAND

same as for M= 7

GSM  900 :

IMMEDIATE ASSIGNMENT

same as for M= 7

except :

| Channel Description |  |
| --- | --- |
| - Channel Type | TCH/H + ACCHs |

HANDOVER COMMAND

same as for M= 7

### 26.12.3	EFR Signalling / Structured procedures / MS originated call / late assignment

NOTE:	this test is derived from the one defined in section 26.9.3 and entitled “Structured procedures / MS originated call / late assignment”

26.12.3.1	Conformance requirement

1)	The MS shall indicate and include  in the mobile originating SETUP for speech call all the speech versions that it supports.

2)	Upon receipt of the ASSIGNMENT COMMAND message specifying either speech full rate version 1 or  speech full rate version 2 or speech half rate version 1 (for an MS also supporting half rate), the  Mobile Station starts a normal channel assignment procedure. It means that the MS initiates a local end release of link layer connections, disconnects the physical channels, commands the switching to the assigned channels and initiates the establishment of lower layer connections (this includes the activation of the channels, their connection and the establishment of the data links). After the main signalling link is successfully established, the MS returns an ASSIGNMENT COMPLETE message, specifying cause 'normal event', to the  network on the main DCCH.

3,4)	The call control entity of the Mobile Station in the "call initiated" state, in the "mobile originating call proceeding" state or in the "call delivered" state, shall, upon receipt of a CONNECT message:

-	attach the user connection to the radio path;

-	return a CONNECT ACKNOWLEDGE message.

References

| Conformance requirement 1: | TS GSM 04.08 sections 10.5.4.5 and 10.5.4.5.1 and TS GSM 02.06 section 3.2.3 |
| --- | --- |
| Conformance requirement 2: | TS GSM 04.08 section 3.4.3.1, 3.4.3.2 |
| Conformance requirement 3: | TS GSM 04.08 section 5.2.1.6. |

26.12.3.2	Test purpose

1)	To verify that the MS indicates and includes in the mobile originating SETUP for speech call all the speech versions that it supports.

2)	To verify that subsequently after receipt of an IMMEDIATE ASSIGNMENT message allocating an SDCCH, after completion of establishment of the main signalling link, after having sent a CM SERVICE REQUEST message, after having successfully performed authentication and cipher mode setting procedures, after having sent a SETUP message, after having received a CALL PROCEEDING message followed by an ALERTING message and an ASSIGNMENT COMMAND message allocating either speech full rate version 1  TCH or  speech full rate version 2  TCH or speech half rate version 1 TCH (for an MS also supporting half rate), the MS sends an ASSIGNMENT COMPLETE message.

3)	To verify that subsequently, after the suite of actions specified in test purposes 1 and 2, the MS after receiving a CONNECT message returns a CONNECT ACKNOWLEDGE message.

4)	To verify that after the suite of actions specified in test purposes 1 and 2, the MS after receiving a CONNECT message attaches the user connection to the radio path. (This is checked by verifying that there is a point in time after transmission of the first L2 frame containing the (complete) CONNECT message, where the MS is sending appropriate speech or data frames whenever it doesn't have to transmit or acknowledge an I frame on layer 2 of the FACCH.)

26.12.3.3	Method of test

Related PICS/PIXIT statements

-	Enhanced full rate speech MS

- 	Supported speech versions

-	Interface to the human user (p1 = Y/N)

-	Way to display the called number (only applicable if the MS has an interface to the human user)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Supported teleservices

-	Classmark

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS has a MO call in state U10, "active".

Test procedure

The following test is performed for all rates (full rate/half rate) supported by the MS:

The MS is made to initiate a speech call. The call is established with late assignment.

Maximum Duration of Test

3 minutes.

Expected Sequence

This test is repeated for execution counter M=1, 2 for an MS supporting  full rate channels only.

This test is repeated for execution counter M=1, 2, 3 for an MS supporting both half and full rate channels.

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | MS |  | The "called number" is entered |
| 2 | MS |  |  |
| 3 | MS -> SS | CHANNEL REQUEST | Establishment cause is "originating call and the network does not set the NECI bit to 1". |
| 4 | SS -> MS | IMMEDIATE ASSIGNMENT |  |
| 5 | MS -> SS | CM SERVICE REQUEST | Message is contained in SABM. |
| 6 | SS -> MS | AUTHENTICATION REQ |  |
| 7 | MS -> SS | AUTHENTICATION RESP | SRES specifies correct value. |
| 8 | SS -> MS | CIPHER MODE COMMAND | SS starts deciphering after sending the message. |
| 9 | MS -> SS | CIPHER MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 10 | SS |  | SS starts ciphering. |
| 11 | MS -> SS | SETUP | If the mobile only supports full rate speech , it is checked that it indicates support of full rate speech version 1 and version 2. If the mobile supports both rates, it is checked that it indicates full rate speech version 1, half rate speech version 1 and full rate speech version 2. |
| 12 | SS -> MS | CALL PROCEEDING |  |
| 13 | SS -> MS | ALERTING |  |
| 14 | MS |  | Depending on the PICS, an alerting indication is given. |
| 15 | SS -> MS | ASSIGNMENT COMMAND |  |
| 16 | MS -> SS | ASSIGNMENT COMPLETE |  |
| 17 | SS -> MS | CONNECT |  |
| 18 | MS -> SS | CONNECT ACKNOWLEDGE |  |
| 19 | MS |  | The appropriate bearer channel is through connected in both directions. |

Specific Message Contents:

For M = 1 :

Assignment command

see default message contents section 26.12.6.

Setup

Same contents as 26.12.6 but all the speech versions supported by the MS shall be indicated in octet_3a_etc(s).

For M = 2  :

Assignment command

same as for default message contents except :

| Mode of the first channel |  |
| --- | --- |
| - Mode | speech full rate or half rate version 1 |

Setup

Same contents as 26.12.6 but all the speech versions supported by the MS shall be indicated in octet_3a_etc(s).

For M = 3 :

Assignment command

same as for default message contents except :

| Channel Description |  |
| --- | --- |
| - TDMA offset | TCH/H+ACCHs |
| Mode of the first channel |  |
| - Mode | speech full rate or half rate version 1 |

Setup

same contents as 26.12.6 but the supported speech versions and their preferred order indicated in octet_3a_etc(s) shall be as declared by the manufacturer.

### 26.12.4	Structured procedures / MS terminated call / l early assignment

NOTE:	this test is derived from the one described in section  26.9.4 and entitled “Structured procedures / MS terminated call / early assignment”

26.12.4.1	Conformance requirement

1)	In acceptance to a SETUP message indicating speech, the MS shall indicate and include  in the CALL CONFIRMED message all the speech versions that it supports.

2)	Upon receipt of the ASSIGNMENT COMMAND message specifying either speech full rate version 1 or  speech full rate version 2 or speech half rate version 1 (for an MS also supporting half rate), the  Mobile Station  continues  a mobile terminating call establishment with early assignment of traffic channel

a)	by replying to the ASSIGNMENT command with an ASSIGNMENT COMPLETE  message and ,

b) 	by continuing the call establishment by through connecting TCH in both directions  if it supports immediate connect or by sending an ALERTING message otherwise,

3)	for  speech calls, the mobile station shall attach the user connection at latest when sending the connect message, except if there is no compatible radio resource available at this time. In this case the attachment  shall be delayed until such a resource becomes available.

References

| Conformance requirement 1: | TS GSM 04.08 sections 10.5.4.5,and 10.5.4.5.1 |
| --- | --- |
|  | TS GSM 02.06 section 3.2.3 |
| Conformance requirement 2: | TS GSM 04.08 section 3.4.3.1, 3.4.3.2 |
| Conformance requirement 3: | TS GSM 04.08 section 5.2.2.9 |

26.12.4.2	Test purpose

1)	To verify that, in acceptance to a SETUP message indicating speech, the MS indicates and includes in the CALL CONFIRMED message all the speech versions that it supports.

2)	To verify that upon receipt of the ASSIGNMENT COMMAND message specifying either speech full rate version 1 or  speech full rate version 2 or speech half rate version 1 (for an MS also supporting half rate), the  Mobile Station  continues  a mobile terminating call establishment with early assignment of traffic channel

a)	by replying to the ASSIGNMENT command with an ASSIGNMENT COMPLETE  message and ,

b) 	by continuing the call establishment by through connecting TCH in both directions  if it supports immediate connect or by sending an ALERTING message otherwise,

3)	To verify that for  speech calls, the mobile station shall attach the user connection at latest when sending the connect message, except if there is no compatible radio resource available at this time. In this case the attachment  shall be delayed until such a resource becomes available.

26.12.4.3	Method of test

Related PICS/PIXIT statements

-	Enhanced full rate speech MS

- 	Supported speech versions

-	Interface to the human user (p1 = Y/N)

-	Way to indicate alerting (only applicable if the MS supports the feature)

-	Way to make the MS accept an incoming call after alerting (possibly dependent on teleservice and configuration)

-	Supported teleservices

-	Classmark

-	Immediate connect supported (Y/N)

Initial Conditions

System Simulator:	1 cell, default parameters.

Mobile Station:	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

CC state U10-call active.

Test procedure

The following test is performed for all rates (full rate/half rate) supported by the MS:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice.

The MS is paged and the resulting call is established. Having reached the active state, the MS is made to clear the call.

Maximum Duration of Test

3 minutes.

Expected Sequence

This test is repeated for execution counter M=1, 2 for an MS supporting  full rate channels only.

This test is repeated for execution counter M=1, 2, 3 for an MS supporting both half and full rate channels.

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | SS -> MS | PAGING REQUEST TYPE 1 | Sent on the correct paging subchannel |
| 2 | MS -> SS | CHANNEL REQUEST |  |
| 3 | SS -> MS | IMMEDIATE ASSIGNMENT |  |
| 4 | MS -> SS | PAGING RESPONSE | Message is contained in SABM. |
| 5 | SS -> MS | AUTHENTICATION REQUEST |  |
| 6 | MS -> SS | AUTHENTICATION RESPONSE | SRES specifies correct value. |
| 7 | SS -> MS | CIPHERING MODE COMMAND | SS starts deciphering after sending the message. |
| 8 | MS -> SS | CIPHERING MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 9 | SS |  | SS starts ciphering. |
| 10 | SS -> MS | SETUP | Message does not contain the signal IE. |
| 11 | MS -> SS | CALL CONFIRMED | If the mobile only supports full rate speech, it is checked that it indicates support of full rate speech version 1 and version 2. If the mobile supports both rates, it is checked that it indicates full rate speech version 1, half rate speech version 1 and full rate speech version 2. |
|  |  |  | If the MS supports an Immediate connection then branch A applies. If the MS doesn’t support an immediate connection then branch B applies. |
| A12 | MS -> SS | CONNECT | sent on the old channel |
| A13 | SS -> MS | ASSIGNMENT COMMAND |  |
| A14 | MS -> SS | ASSIGNMENT COMPLETE |  |
| B12 | SS -> MS | ASSIGNMENT COMMAND |  |
| B13 | MS -> SS | ASSIGNMENT COMPLETE | Sent on the new channel. |
| B14 | MS -> SS | ALERTING |  |
| B15 | MS |  | An alerting indication as defined in an PICS/PIXIT statement is given by the MS. |
| B16 | MS |  | The MS is made to accept the call in the way described in a PICS/PIXIT statement. |
| B17 | MS -> SS | CONNECT |  |
| 18 | MS |  | the TCH shall be through connected in both directions. in the indicated mode. |
| 19 | SS -> MS | CONNECT ACKNOWLEDGE |  |
| 20 | MS |  | The MS is made to release the call. |
| 21 | MS -> SS | DISCONNECT |  |
| 22 | SS -> MS | RELEASE |  |
| 23 | MS -> SS | RELEASE COMPLETE |  |
| 24 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |

Specific Message Contents:

For M = 1 :

Assignment command

see default message contents section 26.12.6.

call confirmed

Same contents as 26.12.6 but all the speech versions supported by the MS shall be indicated in octet_3a_etc(s).

For M = 2  :

Assignment command

same as for default message contents except :

| Mode of the first channel |  |
| --- | --- |
| - Mode | speech full rate or half rate version 1 |

call confirmed

Same contents as 26.12.6 but all the speech versions supported by the MS shall be indicated in octet_3a_etc(s).

For M = 3 :

Assignment command

same as for default message contents except :

| Channel Description |  |
| --- | --- |
| - TDMA offset | TCH/H+ACCHs |
| Mode of the first channel |  |
| - Mode | speech full rate or half rate version 1 |

CALL CONFIRMED

same contents as 26.12.6 but all the speech versions supported by the MS shall be indicated in octet_3a_etc(s).

### 26.12.5	Structured procedures / emergency call

NOTE :	This test is derived from the ones described in sections  26.9.6.1.1 and 26.9.6.1.2 and respectively entitled “Structured procedures / emergency call / idle updated, preferred channel rate “ and  “Structured procedures / emergency call / idle updated, non-preferred channel rate”.

This test applies to mobiles supporting Enhanced Full Rate speech.

26.12.5.1	Conformance requirement

1)	The MS in the "idle, updated" state, as after a successful location update, after the number 112 has been entered  by user, shall send a CHANNEL REQUEST message with correct establishment cause ("emergency call").

2)	After assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel shall be a CM SERVICE REQUEST message specifying the correct CKSN and TMSI , with CM Service Type "emergency call establishment".

3)	Authentication and cipher mode setting shall be performed successfully.

4)	After cipher mode setting acceptance by the network, the MS shall send an EMERGENCY SETUP message.

5)	The EFR mobile station shall  accept  channel assignment to a TCH  full rate speech version 1 or 2  and if it supports  half rate, in addition  it shall accept  channel assignment to a TCH half rate speech version 1.

6)	After receipt of a CONNECT ACKNOWLEDGE message during correct establishment of the emergency call the TCH shall be through connected in both directions if an appropriate TCH is available.

7)	The call shall be cleared correctly.

Requirement Reference:

| For conformance requirement 1 and 2: | TS GSM 04.08 section 3.3.1, TS GSM 04.08 section 5.2.1, TS GSM 04.08 section 4.5.1.5., TS GSM 02.30 section 4. |
| --- | --- |
| For conformance requirement 3: | TS GSM 04.08, section 3.4.7, TS GSM 04.08 section 4.3.2. |
| For conformance requirement 4: | TS GSM 04.08, section 5.2.1.1. |
| For conformance requirement 5 : | TS GSM 04.08, sections 5.2.1.1 and 3.4.3. |
|  | TS GSM 02.06 , subclause 3.2.3. |
| For conformance requirement 6: | TS GSM 04.08, section 5.2.1.6 |
| For conformance requirement 7: | TS GSM 04.08, section 5.4. |

26.12.5.2	Test purpose

1)	To verify that an MS supporting speech in the MM state "idle, updated", when made to call the number 112, sends a CHANNEL REQUEST message with establishment cause "emergency call".

2)	To verify that after assignment of a dedicated channel the first layer message sent by the MS on the assigned dedicated channel is a CM SERVICE REQUEST message specifying the correct CKSN and TMSI, with CM Service Type "emergency call establishment" .

3)	To verify that authentication and cipher mode setting are performed successfully.

4)	To verify that after cipher mode setting acceptance by the SS, the MS sends an EMERGENCY SETUP message.

5)	To verify that the EFR mobile station shall both accept  channel assignment to a TCH  full rate speech version 1 or 2  and if it supports  half rate, in addition  it shall accept  channel assignment to a TCH half rate speech version 1.

6)	To verify that subsequently the MS has through connected the TCH in both directions.

7)	To verify that the call is cleared correctly.

26.12.5.3	Method of test

Related PICS/PIXIT Statements

- Speech supported (Y/N).

- Supported rate for speech: (F/H, F).

- Speech version supproted :

- Classmark.

- Inclusion of the bearer capability IE in the emergency setup

Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen Final State of the MS

The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Test procedure

The MS is made to initiate an emergency call. The call is established with late assignment. Having reached the active state, the call is cleared by the SS. This procedure is repeated so that the assignment is made with all the channel rates and speech versions supported by the mobile station.

Maximum Duration of Test

3 minutes

Expected Sequence

The expected sequence is executed for M=1 and 2, for a full rate only mobile station which includes the bearer capabilityv IE in the emergency setup message .

The expected sequence is executed for M=1, 2 and 3, for a dual rate  mobile station which includes the bearer capabilityv IE in the emergency setup message.

The expected sequence is executed for M=1, for a mobille which does not include the bearer capability IE in the emergency setup message.

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | MS |  | The "called number" 112 is entered |
| 3 | MS -> SS | CHANNEL REQUEST | Establishment cause is emergency call establishment. |
| 4 | SS -> MS | IMMEDIATE ASSIGNMENT |  |
| 5 | MS -> SS | CM SERVICE REQUEST | Message is contained in SABM. The CM service type IE indicates "emergency call establishment". |
| 6 | SS -> MS | AUTHENTICATION REQUEST |  |
| 7 | MS -> SS | AUTHENTICATION RESPONSE | SRES specifies correct value. |
| 8 | SS -> MS | CIPHERING MODE COMMAND | SS starts deciphering after sending the message. |
| 9 | MS -> SS | CIPHERING MODE COMPLETE | Shall be sent enciphered. All following messages shall be sent enciphered. |
| 10 | SS |  | SS starts ciphering. |
| 11 | MS -> SS | EMERGENCY SETUP | If the bearer capability IE is including, it shall be checked that all the speech versions supported by the MS are present. |
| 12 | SS -> MS | CALL PROCEEDING |  |
| 13 | SS -> MS | ALERTING |  |
| 14 | SS -> MS | ASSIGNMENT COMMAND | See specific message contents. |
| 15 | MS -> SS | ASSIGNMENT COMPLETE |  |
| 16 | SS -> MS | CONNECT |  |
| 17 | MS -> SS | CONNECT ACKNOWLEDGE |  |
| 18 | MS |  | The TCH is through connected in both directions in the correct mode. |
| 19 | SS -> MS | DISCONNECT |  |
| 20 | MS -> SS | RELEASE |  |
| 21 | SS -> MS | RELEASE COMPLETE |  |
| 23 | SS -> MS | CHANNEL RELEASE | The main signalling link is released. |

Specific Message Contents:

For M= 1

ASSIGNMENT COMMAND

same as for default message contents except :

| Mode of the first channel |  |
| --- | --- |
| - Mode | Speech full rate or half rate version 1 |

For M= 2

ASSIGNMENT COMMAND

same as for default message contents

For M= 3

ASSIGNMENT  COMMAND

same as for default message contents except :

| Channel Description |  |
| --- | --- |
| - TDMA offset | TCH/H+ACCHs |
| Mode of the first channel |  |
| - Mode | Speech full rate or half rate version 1 |

### 26.12.6	Default contents of  layer 3 messages for Enhanced Full rate speech tests

same as section 26.6.14 for GSM900 MS and 26.6.15 for DCS1800 MS except for :

Contents of ASSIGNMENT COMMAND message in the GSM900  band:

| Protocol Discriminator | RR Management |
| --- | --- |
| Skip Indicator | 0000 |
| Message Type | 00101110 |
| Channel Description |  |
| - Channel Type and TDMA offset | TCH/F + ACCHs |
| - Timeslot Number | Chosen arbitrarily by the test house |
| - Training Sequence Code | Chosen arbitrarily by the test house |
| - Hopping | Single RF channel |
| - ARFCN | Channel number 30 |
| Power Command |  |
| - Power level | Chosen arbitrarily by the test house |
| Mode of the first channel |  |
| - Mode | speech full rate or half rate version 2 |
| All other information elements | Not present |

Contents of ASSIGNMENT COMMAND message in the DCS1800 band :

| Protocol Discriminator | RR Management |
| --- | --- |
| Skip Indicator | 0000 |
| Message Type | 00101110 |
| Channel Description |  |
| - Channel Type and TDMA offset | TCH/F + ACCHs |
| - Timeslot Number | Chosen arbitrarily by the test house |
| - Training Sequence Code | Chosen arbitrarily by the test house |
| - Hopping | Single RF channel |
| - ARFCN | Channel number 650 |
| Power Command |  |
| - Power level | Chosen arbitrarily by the test house |
| Mode of the first channel |  |
| - Mode | speech full rate or half rate version 2 |
| All other information elements | Not present |

CALL CONFIRMED

| Information element | Value/remark |
| --- | --- |
| Repeat indicator | Omitted |
| Bearer capability 1 | coding as described in section 11.8.2.9.2 |
| Bearer capability 2 | Omitted |
| Cause | Omitted |

Contents of CHANNEL MODE MODIFY message in the GSM900  band :

| Protocol Discriminator | RR Management |
| --- | --- |
| Skip Indicator | 0000 |
| Message Type | 00010000 |
| Channel Description | same as the dedicated channel currently allocated |
| Channel Mode |  |
| - Mode | speech full rate version 2 |

Contents of CHANNEL MODE MODIFY ACKNOWLEDGE message in the GSM900  band :

| Protocol Discriminator | RR Management |
| --- | --- |
| Skip Indicator | 0000 |
| Message Type | 00010111 |
| Channel Description | same as the dedicated channel currently allocated |
| Channel Mode |  |
| - Mode | Speech full rate version 2 |

Contents of CHANNEL MODE MODIFY message in the DCS1800 band :

| Protocol Discriminator | RR Management |
| --- | --- |
| Skip Indicator | 0000 |
| Message Type | 00010000 |
| Channel Description | same as the dedicated channel currently allocated |
| Channel Mode |  |
| - Mode | speech full rate version 2 |

Contents of CHANNEL MODE MODIFY ACKNOWLEDGE message in DCS 1800 band :

| Protocol Discriminator | RR Management |
| --- | --- |
| Skip Indicator | 0000 |
| Message Type | 00010111 |
| Channel Description | same as the dedicated channel currently allocated |
| Channel Mode |  |
| - Mode | Speech full/half rate version 2 |

Contents of HANDOVER COMMAND message in the GSM900 band :

| Protocol Discriminator | RR Management |
| --- | --- |
| Skip Indicator | 0000 |
| Message Type | 00101011 |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base station Colour Code | 5 |
| - BCCH Carrier Number | Set to the BCCH carrier number of cell B. (one of 10, 20, 80, 90, 100, 110 or 120) |
| Channel Description |  |
| - Channel Type and TDMA offset | TCH/F + ACCHs |
| - Timeslot Number | Chosen arbitrarily by the test house |
| - Training Sequence Code | Chosen arbitrarily by the test house |
| - Hopping | Single RF channel |
| - ARFCN | Chosen arbitrarily by the test house from those supported on the target cell |
| Handover Reference |  |
| - Handover Reference Value | Chosen arbitrarily by the test house. |
| Power Command |  |
| - Power level | Chosen arbitrarily by the test house |
| Mode of the first channel |  |
| - Mode | speech full/half rate version 2 |
| All other information elements | Not present |

Contents of HANDOVER COMMAND message  in the DCS 1800  band :

| Protocol Discriminator | RR Management |
| --- | --- |
| Skip Indicator | 0000 |
| Message Type | 00101011 |
| Cell Description |  |
| - Network Colour Code | 1 |
| - Base station Colour Code | 5 |
| - BCCH Carrier Number | Set to the BCCH carrier number of cell B. (one of 520, 590, 600, 700, 780, 810 or 870) |
| Channel Description |  |
| - Channel Type and TDMA offset | TCH/F + ACCHs |
| - Timeslot Number | Chosen arbitrarily by the test house |
| - Training Sequence Code | Chosen arbitrarily by the test house |
| - Hopping | Single RF channel |
| - ARFCN | Chosen arbitrarily by the test house from those supported on the target cell |
| Handover Reference |  |
| - Handover Reference Value | Chosen arbitrarily by the test house. |
| Power Command |  |
| - Power level | Chosen arbitrarily by the test house |
| Mode of the first channel |  |
| - Mode | speech full/half rate version 2 |
| All other information elements | Not present |

Contents of SETUP message; (MS to SS);

| Protocol Discriminator | Call Control |
| --- | --- |
| Transaction Identifier |  |
| TI value | any value from the set {0, ..., 6} |
| TI flag | 0 |
| Message Type | 0X000101 |
| Other information elements | Not checked |
| Protocol Discriminator | Call Control |
| Transaction Identifier | set {0, ..., 6} |
| TI flag | 0 |
| BC repeat indicator | Not present |
| Bearer capability 1 | codind as described in section 11.8.2.9.2 |
| All other information elements | Not present |

Contents of SETUP message; (SS to MS for speech teleservice)

| Protocol Discriminator | Call Control |
| --- | --- |
| Transaction Identifier |  |
| TI value | any value from the set {0, ..., 6} |
| TI flag | 0 |
| Message Type | 0X000101 |
| Other information elements | Not checked |
| Protocol Discriminator | Call Control |
| Transaction Identifier | set {0, ..., 6} |
| TI flag | 0 |
| BC repeat indicator | Not present |
| Bearer capability 1 |  |
| octet 2 |  |
| length | 01 H |
| octet 3 |  |
| extension | 1 |
| radio channel requirement | 01 |
| coding standard | GSM standardized coding |
| transfer mode | circuit mode |
| information transfer capability | speech |
| All other information elements | Not present |