TD <>

TS 100 607-1 V5.4.0 (1998-02)

Technical Specification

Digital cellular telecommunications system (Phase2+);

Mobile Station (MS) conformance specification;

Part 1: Conformance specification

(GSM 11.10-1 version 5.4.0)

Reference

RTS/SMG-071110QR2-1 (5mc92ic3.PDF)

Keywords

Digital cellular telecommunications system, Global System for Mobile Communication (GSM)

ETSI Secretariat

Postal address

F-06921 Sophia Antipolis Cedex - FRANCE

Office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° 7803/88

X.400

c= fr; a=atlas; p=etsi; s=secretariat

Internet

secretariat@etsi.fr

http://www.etsi.fr

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© European Telecommunications Standards Institute 1998.

All rights reserved.

Contents

Foreword	26

1	Scope	27

2	Normative references	27

3	Definitions, conventions, and applicability	34

3.1	Mobile station definition and configurations	34

3.2	Applicability	34

3.2.1	Applicability of this specification	34

3.2.2	Applicability of the individual tests	34

3.2.3	Applicability to terminal equipment	57

3.3	Definitions	57

3.4	Conventions for mathematical notations	58

3.4.1	Mathematical signs	58

3.4.2	Powers to the base 10	58

3.5	Conventions on electrical terms	58

3.5.1	Radio Frequency (RF) input signal level	58

3.5.2	Reference sensitivity level	59

3.5.3	Power level of fading signal	59

3.6	Terms on test conditions	59

3.6.1	Radio test conditions	59

4	Test Equipment	61

4.1	Terms used to describe test equipment in this TS	61

4.2	Functional requirements of test equipment	61

5	Testing methodology in general (layers 1, 2, and 3)	61

5.1	Testing of optional functions and procedures	61

5.2	Test interfaces and facilities	61

5.3	Different protocol layers	62

5.4	Information to be provided by the apparatus supplier	62

5.5	Definitions of transmit and receive times	62

6	Reference test methods	62

6.1	General	62

6.2	Choice of frequencies in the frequency hopping mode	63

6.3	"Ideal" radio conditions	63

6.4	Standard test signals	63

6.5	Power (control) levels	63

7	Implicit testing	63

8	Measurement uncertainty	64

9	Format of tests	64

10	Generic call set up procedures	64

10.1	Generic call set-up procedure for mobile terminating speech calls	64

10.2	Generic call set-up procedure for mobile originating speech calls	69

10.3	Generic call set-up procedure for mobile terminating data calls	72

10.4	Generic call set-up procedure for mobile originating data calls	75

10.5	Generic call set-up procedure for mobile terminating multislot configuration, minimum number of timeslots allocated	78

10.6	Generic call set-up procedure for mobile originating multislot configuration, minimum number of timeslots allocated	81

11	General tests	85

11.1	Verification of support and non-support of services (multiple numbering scheme or ISDN)	85

11.1.1	Mobile Terminated (MT) calls	85

11.1.2	Mobile Originated (MO) calls	86

11.2	Verification of support of the single numbering scheme	87

11.3	Verification of non-support of services (Advice of Charge Charging (AoCC))	88

11.4	Verification of non-support of services (call hold)	89

11.5	Verification of non-support of services (multiparty)	90

11.6	Verification of non-support of feature (Fixed Dialling Number (FDN))	91

11.7	IMEI Security	92

11.8	Coding of the Bearer Capability information element	93

11.8.1	Network to MS Direction	94

11.8.1.1	BS 21 to 26 - Asynchronous Service	94

11.8.1.1.1	BS 21	94

11.8.1.1.2	BS 22	98

11.8.1.1.3	BS 24	98

11.8.1.1.4	BS 25	98

11.8.1.1.5	BS 26	98

11.8.1.1.6	BS 23	98

11.8.1.2	BS 31 to 34 - Synchronous Service	99

11.8.1.2.1	BS 32	99

11.8.1.2.2	BS 31	103

11.8.1.2.3	BS 33	104

11.8.1.2.4	BS 34	104

11.8.1.3	BS 61 - Alternate Speech / Data	104

11.8.1.3.1	Speech/Asynchronous Data, Transparent	105

11.8.1.3.2	Speech/Asynchronous Data, Non Transparent	107

11.8.1.3.3	Speech/Synchronous Data	109

11.8.1.4	BS 81 - Speech followed by Data	110

11.8.1.4.1	Speech followed by Asynchronous Data	110

11.8.1.4.2	Speech followed by Synchronous Data	110

11.8.1.5	TS 61 - Alternate Speech / Facsimile group 3	110

11.8.1.5.1	TS 61 - Alternate Speech / Facsimile group 3, Transparent	111

11.8.1.5.2	TS 61 - Alternate Speech / Facsimile group 3, Non-Transparent	112

11.8.1.6	TS 62 - Automatic Facsimile group 3	113

11.8.2	MS to SS direction	113

11.8.2.1	BS 21 to 26 - Asynchronous Service	113

11.8.2.1.1	BS 21	114

11.8.2.1.2	BS 22	118

11.8.2.1.3	BS 24	118

11.8.2.1.4	BS 25	118

11.8.2.1.5	BS 26	118

11.8.2.1.6	BS 23	118

11.8.2.2	BS 31 to 34 - Synchronous Service	119

11.8.2.2.1	BS 32	119

11.8.2.2.2	BS 31	123

11.8.2.2.3	BS 33	124

11.8.2.2.4	BS 34	124

11.8.2.3	BS 41 to 46 - PAD Access Asynchronous	125

11.8.2.3.1	2) BS 41	125

11.8.2.3.2	BS 42	126

11.8.2.3.3	BS 44	127

11.8.2.3.4	BS 45	127

11.8.2.3.5	BS 46	127

11.8.2.3.6	BS 43	127

11.8.2.4	BS 51 to 53 - Packet Service Synchronous	128

11.8.2.4.1	2) BS 51	128

11.8.2.4.2	BS 52	128

11.8.2.4.3	BS 53	128

11.8.2.5	BS 61 - Alternate Speech / Data	129

11.8.2.5.1	Speech/Asynchronous Data, Transparent	129

11.8.2.5.2	Speech/Asynchronous Data, Non Transparent	131

11.8.2.5.3	Speech/Synchronous Data	133

11.8.2.6	BS 81 - Speech followed by Data	134

11.8.2.6.1	Speech followed by Asynchronous Data	134

11.8.2.6.2	Speech followed by Synchronous Data	134

11.8.2.7	TS 61 - Alternate Speech / Facsimile group 3	134

11.8.2.7.1	TS 61 - Alternate Speech / Facsimile group 3, Transparent	135

11.8.2.7.2	TS 61 - Alternate Speech / Facsimile group 3, Non Transparent	136

11.8.2.8	TS 62 - Automatic Facsimile group 3	137

11.8.2.9	TS 11 and TS 12-  Speech	137

11.8.2.9.1	Support of only full/half rate speech version 1	137

11.8.2.9.2	Support of speech full rate version 2 (Enhanced Full Rate)	137

12	Transceiver	139

12.1	Conducted spurious emissions	139

12.1.1	MS allocated a channel	139

12.1.2	MS in idle mode	140

12.2	Radiated spurious emissions	142

12.2.1	MS allocated a channel	142

12.2.2	MS in idle mode	144

13	Transmitter	147

13.1	Frequency error and phase error	147

13.2	Frequency error under multipath and interference conditions	150

13.3	Transmitter output power and burst timing	152

13.4	Output RF spectrum	159

13.5	Intermodulation attenuation	165

13.6	Frequency error and phase error in HSCSD multislot configurations	166

13.7	Reserved for future use	170

13.8	Output RF spectrum in HSCSD multislot configuration	170

14	Receiver	177

14.1	Bad frame indication	183

14.1.1	Bad frame indication - TCH/FS	183

14.1.1.1	Bad frame indication - TCH/FS - Random RF input	183

14.1.1.2	Bad frame indication - TCH/FS - Frequency hopping and downlink DTX	184

14.1.2	Bad frame indication - TCH/HS	186

14.1.2.1	Bad frame indication - TCH/HS - Random RF input	186

14.1.2.2	Bad frame indication - TCH/HS - Frequency hopping and downlink DTX	187

14.1.3	Bad frame indication - TCH/FS - Frequency hopping and downlink DTX - Phase 2 MS in a phase 1 network	188

14.1.4	Bad frame indication - TCH/HS - Frequency hopping and downlink DTX - Phase 2 MS in a phase 1 network	190

14.2	Reference sensitivity	192

14.2.1	Reference sensitivity - TCH/FS	192

14.2.2	Reference sensitivity - TCH/HS (Speech frames)	194

14.2.3	Reference sensitivity - FACCH/F	197

14.2.4	Reference sensitivity - FACCH/H	198

14.2.5	Reference sensitivity - full rate data channels	199

14.2.6	Reference sensitivity - half rate data channels	200

14.2.7	Reference sensitivity - TCH/EFS	201

14.2.8	Reference sensitivity - full rate data channels in multislot configuration	203

14.3	Usable receiver input level range	204

14.4	Co-channel rejection	206

14.4.1	Co-channel rejection - TCH/FS.	206

14.4.2	Co-channel rejection - TCH/HS	207

14.4.3	Co-channel rejection - TCH/HS (SID frames)	209

14.4.4	Co-channel rejection - FACCH/F	211

14.4.5	Co-channel rejection - FACCH/H	212

14.4.6	Co-channel rejection - TCH/EFS.	213

14.5	Adjacent channel rejection	215

14.5.1	Adjacent channel rejection - speech channels	215

14.5.2	Adjacent channel rejection - control channels	217

14.6	Intermodulation rejection	219

14.6.1	Intermodulation rejection - speech channels	219

14.6.2	Intermodulation rejection - control channels	221

14.7	Blocking and spurious response	222

14.7.1	Blocking and spurious response - speech channels	222

14.7.2	Blocking and spurious response - control channels	225

14.8	AM suppression	229

14.8.1	AM suppression - speech channels	229

14.8.2	AM suppression - control channels	230

15	Timing advance and absolute delay	233

16	Reception time tracking speed	235

17	Access times during handover	237

17.1	Intra cell channel change	237

17.2	Inter cell handover	239

18	Temporary reception gaps	242

19	Channel release after unrecoverable errors	244

19.1	Channel release after unrecoverable errors -1	244

19.2	Channel release after unrecoverable errors - 2	245

19.3	Channel release after unrecoverable errors - 3	246

20	Cell selection and reselection	248

20.1	Cell selection	249

20.2	Cell selection with varying signal strength values	251

20.3	Basic cell reselection	253

20.4	Cell reselection using TEMPORARY_OFFSET, CELL_RESELECT_OFFSET, POWER_OFFSET and PENALTY_TIME parameters	255

20.5	Cell reselection using parameters transmitted in the System Information type 2bis, type 2ter, type 7 and type 8 messages.	257

20.6	Cell reselection timings	259

20.7	Priority of cells	261

20.8	Cell reselection when C1 (serving cell) < 0 for 5 seconds	262

20.9	Running average of the surrounding cell BCCH carrier signal levels	263

20.10	Running average of the serving cell BCCH carrier signal level	264

20.11	Updating the list of six strongest neighbour carriers and decoding the BCCH information of a new carrier on the list	266

20.12	Decoding the BCCH information of the neighbour carriers on the list of six strongest neighbour carriers	267

20.13	Decoding the BSIC of the neighbour carriers on the list of six strongest neighbour carriers	268

20.14	Emergency calls	269

20.15	Cell reselection due to MS rejection "LA not allowed"	270

20.16	Downlink signalling failure	272

20.17	Cell selection if no suitable cell found in 10 s	273

20.18	Cell reselection due to MS rejection "Roaming not allowed in this LA"	274

20.19	Cell selection on release of SDCCH and TCH	275

20.20	Multiband cell selection and reselection	277

20.20.1	Multiband cell selection and reselection / Cell Selection	277

20.20.2	Multiband cell selection and reselection / Cell reselection	279

21	Received signal measurements	281

21.1	Signal strength	281

21.2	Signal strength selectivity	284

21.3	Signal quality under static conditions	285

21.3.1	Signal quality under static conditions - TCH/FS	285

21.3.2	Signal quality under static conditions - TCH/HS	287

21.4	Signal quality under TU50 propagation conditions	289

22	Transmit power control timing and confirmation	292

22.1	Transmit power control timing and confirmation, single slot	292

22.2	Transmit power control timing and confirmation in HSCSD multislot configurations	293

23	Single frequency reference	295

24	Tests of the layer 1 signalling functions	296

25	Tests of the layer 2 signalling functions	297

25.1	Introduction, objective and scope	297

25.1.1	General	297

25.1.2	Test configurations	297

25.1.3	Pre-conditions	297

25.1.4	Layer 2 test frames	298

25.1.5	Establishment of the dedicated physical resource	298

25.1.6	Release of the dedicated physical resource	299

25.2	Test sequences	299

25.2.1	Initialization	300

25.2.1.1	Initialization when contention resolution required	300

25.2.1.1.1	Normal initialization	300

25.2.1.1.2	Initialization failure	301

25.2.1.1.2.1	Loss of UA frame	301

25.2.1.1.2.2	UA frame with different information field	302

25.2.1.1.2.3	Information frame and supervisory frames in response to an SABM frame	303

25.2.1.1.3	Initialization denial	303

25.2.1.1.4	Total initialization failure	304

25.2.1.2	Initialization, contention resolution not required	305

25.2.1.2.1	Normal initialization without contention resolution	305

25.2.1.2.2	Initialization failure	306

25.2.1.2.3	Initialization denial	308

25.2.1.2.4	Total initialization failure	308

25.2.2	Normal information transfer	309

25.2.2.1	Sequence counting and I frame acknowledgements	309

25.2.2.2	Receipt of an I frame in the timer recovery state	312

25.2.2.3	Segmentation and concatenation	314

25.2.3	Normal layer 2 disconnection	317

25.2.4	Test of link failure	317

25.2.4.1	I frame loss (MS to SS)	317

25.2.4.2	RR response frame loss (SS to MS)	318

25.2.4.3	RR response frame loss (MS to SS)	319

25.2.5	Test of frame transmission with incorrect C/R values	320

25.2.5.1	I frame with C bit set to zero	320

25.2.5.2	SABM frame with C bit set to zero	321

25.2.6	Test of errors in the control field	322

25.2.6.1	N(S) sequence error	322

25.2.6.2	N(R) sequence error	324

25.2.6.3	Improper F bit	324

25.2.7	Test on receipt of invalid frames	325

26	Testing of layer 3 functions	330

26.1	Default conditions and structured sequence of tests	330

26.1.1	Default test conditions during layer 3 tests	330

26.1.2	Structured sequence of the tests	331

26.1.3	General rules for message parameters	332

26.1.4	General rules for layer 3 testing	332

26.1.5	Format of layer 3 test descriptions	332

26.2	Initial tests	334

26.2.1	Channel request	334

26.2.1.1	Channel request / initial time	334

26.2.1.2	Channel request / repetition time	335

26.2.1.3	Channel request / random reference	337

26.2.2	IMSI detach and IMSI attach	338

26.2.3	Sequenced MM / CM message transfer	342

26.2.4	Establishment cause	343

26.3	Test of MS functions in idle mode	352

26.3.1	Initial conditions	352

26.3.2	MS indication of available PLMNs	354

26.3.3	MS will send only if BSS is "on air"	355

26.3.4	Manual mode of PLMN selection	355

26.4	Lower layer failures in layer 3 testing	357

26.4.1	Introduction	357

26.4.2	Layer 1 reception failures	357

26.4.3	Data link layer failures	357

26.4.4	Lower layer failures, used for the tests in clause 25	357

26.5	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions	358

26.5.1	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / unknown protocol discriminator	358

26.5.2	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / TI and skip indicator	359

26.5.2.1	TI and skip indicator / RR	359

26.5.2.1.1	TI and skip indicator / RR / Idle Mode	360

26.5.2.1.2	TI and skip indicator / RR / RR-Connection established	361

26.5.2.2	TI and skip indicator / MM	362

26.5.2.3	TI and skip indicator / CC	364

26.5.3	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / undefined or unexpected message type	366

26.5.3.1	Undefined or unexpected message type / undefined message type / CC	366

26.5.3.2	Undefined or unexpected message type / undefined message type / MM	367

26.5.3.3	Undefined or unexpected message type / undefined message type / RR	369

26.5.3.4	Undefined or unexpected message type / unexpected message type / CC	370

26.5.4	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / unforeseen information elements in the non-imperative message part	371

26.5.4.1	Unforeseen information elements in the non-imperative message part / duplicated information elements	371

26.5.5	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / non-semantical mandatory IE errors	373

26.5.5.1	Non-semantical mandatory IE errors / RR	373

26.5.5.1.1	Non-semantical mandatory IE errors / RR / missing mandatory IE error	373

26.5.5.1.1.1	Non-semantical mandatory IE errors / RR / missing mandatory IE error / special case	373

26.5.5.1.1.2	Non-semantical mandatory IE errors / RR / missing mandatory IE error / general case	374

26.5.5.1.2	Non-semantical mandatory IE errors / RR / comprehension required	375

26.5.5.2	Non-semantical mandatory IE errors / MM	376

26.5.5.2.1	Non-semantical mandatory IE errors / MM / syntactically incorrect mandatory IE	376

26.5.5.2.2	Non-semantical mandatory IE errors / MM / syntactically incorrect mandatory IE	377

26.5.5.2.3	Non-semantical mandatory IE errors / MM / comprehension required	379

26.5.5.3	Non-semantical mandatory IE errors / CC	381

26.5.5.3.1	Non-semantical mandatory IE errors / CC / missing mandatory IE	381

26.5.5.3.1.1	Non-semantical mandatory IE errors / CC / missing mandatory IE / disconnect message	381

26.5.5.3.1.2	Non-semantical mandatory IE errors / CC / missing mandatory IE / general case	382

26.5.5.3.2	Non-semantical mandatory IE errors / CC / comprehension required	383

26.5.6	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / unknown IE, comprehension not required	384

26.5.6.1	Unknown information elements in the non-imperative message part / MM	384

26.5.6.1.1	Unknown IE, comprehension not required / MM / IE unknown in the protocol	384

26.5.6.1.2	Unknown IE, comprehension not required / MM / IE unknown in the message	386

26.5.6.2	Unknown information elements in the non-imperative message part / CC	387

26.5.6.2.1	Unknown information elements in the non-imperative message part / CC / Call establishment	387

26.5.6.2.2	Unknown information elements in the non-imperative message part / CC / disconnect	388

26.5.6.2.3	Unknown information elements in the non-imperative message part / CC / release	389

26.5.6.2.4	Unknown information elements in the non-imperative message part / CC / release complete	390

26.5.6.3	Unknown IE in the non-imperative message part, comprehension not required / RR	391

26.5.7	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / spare bits	393

26.5.7.1	Spare bits / RR	393

26.5.7.1.1	Spare bits / RR / paging channel	393

26.5.7.1.2	Spare bits / RR / BCCH	394

26.5.7.1.3	Spare bits / RR / AGCH	396

26.5.7.1.4	Spare bits / RR / Connected Mode	398

26.5.7.2	Spare bits / MM	400

26.5.7.3	Spare bits / CC	401

26.5.8	Default contents of messages	404

26.6	Test of the elementary procedures for radio resource management	407

26.6.1	Immediate assignment	407

26.6.1.1	Immediate assignment / SDCCH or TCH assignment	407

26.6.1.2	Immediate assignment / extended assignment	408

26.6.1.3	Immediate assignment / assignment rejection	410

26.6.1.4	Immediate assignment / ignore assignment	412

26.6.1.5	Immediate assignment after immediate assignment reject	414

26.6.2	Test of paging	415

26.6.2.1	Normal paging	415

26.6.2.1.1	Paging / normal / type 1	415

26.6.2.1.2	Paging / normal / type 2	417

26.6.2.1.3	Paging / normal / type 3	419

26.6.2.2	Paging / extended	421

26.6.2.3	Paging / reorganization	425

26.6.2.3.1	Paging / reorganization / procedure 1	425

26.6.2.3.2	Paging / reorganization / procedure 2	428

26.6.2.4	Paging / same as before	429

26.6.2.5	Paging / multislot CCCH	430

26.6.3	Test of measurement report	431

26.6.3.1	Measurement / no neighbours	432

26.6.3.2	Measurement / all neighbours present	435

26.6.3.3	Measurement / barred cells and non-permitted NCCs	439

26.6.3.4	Measurement / DTX	444

26.6.3.5	Measurement / Frequency Formats	448

26.6.3.6	Measurement / multiband environment	451

26.6.4	Test of the channel assignment procedure	456

26.6.4.1	Dedicated assignment / successful case	456

26.6.4.2	Dedicated assignment / failure	467

26.6.4.2.1	Dedicated assignment / failure / failure during active state	467

26.6.4.2.2	Dedicated assignment / failure / general case	468

26.6.5	Test of handover	470

26.6.5.1	Handover / successful / active call / non-synchronized	471

26.6.5.2	Handover / successful / call under establishment / non-synchronized	486

26.6.5.3	Handover / successful / active call / finely synchronized	504

26.6.5.4	Handover / successful / call under establishment / finely synchronized	509

26.6.5.5	Pre-synchronized handovers	519

26.6.5.5.1	Handover / successful / active call / pre-synchronized / Timing Advance IE not included	519

26.6.5.5.2	Handover / successful / call being established / pre-synchronized / timing advance IE is included / reporting of observed time difference requested.	521

26.6.5.6	Handover / successful / active call / pseudo synchronized	523

26.6.5.7	Handover / successful / active call / non-synchronized / reporting of observed time difference requested.	525

26.6.5.8	Handover / layer 3 failure	526

26.6.5.9	Handover / layer 1 failure	528

26.6.6	Test of frequency redefinition	529

26.6.6.1	Frequency redefinition	529

26.6.7	Test of the channel mode modify procedure	534

26.6.7.1	Test of the channel mode modify procedure / full rate	535

26.6.7.2	Test of the channel mode modify procedure / half rate	537

26.6.8	Test of ciphering mode setting	539

26.6.8.1	Ciphering mode / start ciphering	539

26.6.8.2	Ciphering mode / no ciphering	540

26.6.8.3	Ciphering mode / old cipher key	542

26.6.8.4	Ciphering mode / change of mode, algorithm and key	543

26.6.8.5	Ciphering mode / IMEISV request	551

26.6.9	Test of additional assignment	553

26.6.10	Test of partial release	553

26.6.11	Test of classmark	553

26.6.11.1	Classmark change	553

26.6.11.2	Classmark interrogation	555

26.6.12	Test of channel release	557

26.6.12.1	Channel release / SDCCH	557

26.6.12.2	Channel release / SDCCH - no L2 ACK	558

26.6.12.3	Channel release / TCH-F	560

26.6.12.4	Channel release / TCH-F - no L2 ACK	561

26.6.13	Test of starting time	562

26.6.13.1	Dedicated assignment with starting time / successful case / time not elapsed	564

26.6.13.2	Dedicated assignment with starting time / successful case / time elapsed	565

26.6.13.3	Dedicated assignment with starting time and frequency redefinition / failure case / time not elapsed	567

26.6.13.4	Dedicated assignment with starting time and frequency redefinition / failure case / time elapsed	570

26.6.13.5	Handover with starting time / successful case / time not elapsed	572

26.6.13.6	Handover with starting time / successful case / time elapsed	574

26.6.13.7	Handover with starting time and frequency redefinition / failure case / time not elapsed	576

26.6.13.8	Handover with starting time and frequency redefinition / failure case / time elapsed	579

26.6.13.9	Immediate assignment with starting time / successful case / time not elapsed	581

26.6.13.10	Immediate assignment with starting time / successful case / time elapsed	583

26.6.14	Default contents of GSM 900 layer 3 messages for RR tests	585

26.6.15	Default contents of DCS 1 800 layer 3 messages for RR tests	596

26.7	Elementary procedures of mobility management	608

26.7.1	TMSI reallocation	608

26.7.2	Authentication	611

26.7.2.1	Authentication accepted	611

26.7.2.2	Authentication rejected	612

26.7.3	Identification	615

26.7.3.1	General Identification	615

26.7.3.2	Handling of IMSI shorter than the maximum length	617

26.7.4	Location updating	620

26.7.4.1	Location updating / accepted	620

26.7.4.2	Location updating / rejected	623

26.7.4.2.1	Location updating / rejected / IMSI invalid	623

26.7.4.2.2	Location updating / rejected / PLMN not allowed	626

26.7.4.2.3	Location updating / rejected / location area not allowed	630

26.7.4.2.4	Location updating / rejected / roaming not allowed in this location area	633

26.7.4.3	Location updating / abnormal cases	640

26.7.4.3.1	Location updating / abnormal cases / random access fails	640

26.7.4.3.2	Location updating / abnormal cases / attempt counter less or equal to 4, LAI different	642

26.7.4.3.3	Location updating / abnormal cases / attempt counter equal to 4	648

26.7.4.3.4	Location updating / abnormal cases / attempt counter less or equal to 4, stored LAI equal to broadcast LAI	656

26.7.4.4	Location updating / release / expiry of T3240	664

26.7.4.5	Location updating / periodic	665

26.7.4.5.1	Location updating / periodic spread	665

26.7.4.5.2	Location updating / periodic normal / test 1	667

26.7.4.5.3	Location updating / periodic normal / test 2	669

26.7.4.5.4	Location updating / periodic HPLMN search	672

26.7.4.5.4.1	Location updating / periodic HPLMN search / MS waits time T	672

26.7.4.5.4.2	Location updating / periodic HPLMN search / MS in manual mode	673

26.7.4.5.4.3	Location updating / periodic HPLMN search / MS waits at least two minutes and at most T minutes	674

26.7.4.6	Location updating / interworking of attach and periodic	676

26.7.5	MM connection	678

26.7.5.1	Introduction	678

26.7.5.2	MM connection / establishment with cipher	678

26.7.5.3	MM connection / establishment without cipher	680

26.7.5.4	MM connection / establishment rejected	681

26.7.5.5	MM connection / establishment rejected cause 4	682

26.7.5.6	MM connection / expiry T3230	683

26.7.5.7	MM connection / abortion by the network	684

26.7.5.7.1	MM connection / abortion by the network / cause #6	684

26.7.5.7.2	MM connection / abortion by the network / cause not equal to #6	687

26.7.5.8	MM connection / follow-on request pending	688

26.7.5.8.1	MM connection / follow-on request pending / test 1	688

26.7.5.8.2	MM connection / follow-on request pending / test 2	689

26.7.5.8.3	MM connection / follow-on request pending / test 3	690

26.7.6	Default contents of messages	692

26.8	Tests related to circuit switched call control	696

26.8.1	Circuit switched Call Control (CC) state machine verification	696

26.8.1.1	General on CC state machine verification	696

26.8.1.2	Establishment of an outgoing call	697

26.8.1.2.1	Outgoing call / U0 null state	699

26.8.1.2.1.1	Outgoing call / U0 null state / MM connection requested	699

26.8.1.2.2	Outgoing call / U0.1 MM connection pending	700

26.8.1.2.2.1	Outgoing call / U0.1 MM connection pending / CM service rejected	700

26.8.1.2.2.2	Outgoing call / U0.1 MM connection pending / CM service accepted	701

26.8.1.2.2.3	Outgoing call / U0.1 MM connection pending / lower layer failure	703

26.8.1.2.3	Outgoing call / U1 call initiated	704

26.8.1.2.3.1	Outgoing call / U1 call initiated / receiving CALL PROCEEDING	704

26.8.1.2.3.2	Outgoing call / U1 call initiated / rejecting with RELEASE COMPLETE	705

26.8.1.2.3.3	Outgoing call / U1 call initiated / T303 expiry	707

26.8.1.2.3.4	Outgoing call / U1 call initiated / lower layer failure	708

26.8.1.2.3.5	Outgoing call / U1 call initiated / receiving ALERTING	709

26.8.1.2.3.6	Outgoing call / U1 call initiated / entering state U10	710

26.8.1.2.3.7	Outgoing call / U1 call initiated / unknown message received	711

26.8.1.2.4	Outgoing call / U3 MS originating call proceeding	712

26.8.1.2.4.1	Outgoing call / U3 MS originating call proceeding / ALERTING received	712

26.8.1.2.4.2	Outgoing call / U3 MS originating call proceeding / CONNECT received	713

26.8.1.2.4.3	Outgoing call / U3 MS originating call proceeding / PROGRESS received without in band information	715

26.8.1.2.4.4	Outgoing call / U3 MS originating call proceeding / PROGRESS with in band information	716

26.8.1.2.4.5	Outgoing call / U3 MS originating call proceeding / DISCONNECT with in band tones	718

26.8.1.2.4.6	Outgoing call / U3 MS originating call proceeding / DISCONNECT without in band tones	719

26.8.1.2.4.7	Outgoing call / U3 MS originating call proceeding / RELEASE received	720

26.8.1.2.4.8	Outgoing call / U3 MS originating call proceeding / termination requested by the user	721

26.8.1.2.4.9	Outgoing call / U3 MS originating call proceeding / traffic channel allocation	723

26.8.1.2.4.10	Outgoing call / U3 MS originating call proceeding / timer T310 time-out	724

26.8.1.2.4.11	Outgoing call / U3 MS originating call proceeding / lower layer failure	725

26.8.1.2.4.12	Outgoing call / U3 MS originating call proceeding / unknown message received	726

26.8.1.2.4.13	Outgoing call / U3 MS originating call proceeding / Internal alerting indication	727

26.8.1.2.5	Outgoing call / U4 call delivered	728

26.8.1.2.5.1	Outgoing call / U4 call delivered / CONNECT received	728

26.8.1.2.5.2	Outgoing call / U4 call delivered / termination requested by the user	730

26.8.1.2.5.3	Outgoing call / U4 call delivered / DISCONNECT with in band tones	731

26.8.1.2.5.4	Outgoing call / U4 call delivered / DISCONNECT without in band tones	732

26.8.1.2.5.5	Outgoing call / U4 call delivered / RELEASE received	733

26.8.1.2.5.6	Outgoing call / U4 call delivered / lower layer failure	734

26.8.1.2.5.7	Outgoing call / U4 call delivered / traffic channel allocation	736

26.8.1.2.5.8	Outgoing call / U4 call delivered / unknown message received	737

26.8.1.2.6	U10 call active	738

26.8.1.2.6.1	U10 call active / termination requested by the user	738

26.8.1.2.6.2	U10 call active / RELEASE received	739

26.8.1.2.6.3	U10 call active / DISCONNECT with in band tones	740

26.8.1.2.6.4	U10 call active / DISCONNECT without in band tones	742

26.8.1.2.6.5	U10 call active / RELEASE COMPLETE received	743

26.8.1.2.6.6	U10 call active / SETUP received	744

26.8.1.2.7	U11 disconnect request	746

26.8.1.2.7.1	U11 disconnect request / clear collision	746

26.8.1.2.7.2	U11 disconnect request / RELEASE received	747

26.8.1.2.7.3	U11 disconnect request / timer T305 time-out	748

26.8.1.2.7.4	U11 disconnect request / lower layer failure	750

26.8.1.2.7.5	U11 disconnect request / unknown message received	751

26.8.1.2.8	U12 disconnect indication	752

26.8.1.2.8.1	U12 disconnect indication / call releasing requested by the user	752

26.8.1.2.8.2	U12 disconnect indication / RELEASE received	753

26.8.1.2.8.3	U12 disconnect indication / lower layer failure	754

26.8.1.2.8.4	U12 disconnect indication / unknown message received	756

26.8.1.2.9	Outgoing call / U19 release request	757

26.8.1.2.9.1	Outgoing call / U19 release request / timer T308 time-out	757

26.8.1.2.9.2	Outgoing call / U19 release request / 2nd timer T308 time-out	758

26.8.1.2.9.3	Outgoing call / U19 release request / RELEASE received	760

26.8.1.2.9.4	Outgoing call / U19 release request / RELEASE COMPLETE received	761

26.8.1.2.9.5	Outgoing call / U19 release request / lower layer failure	762

26.8.1.3	Establishment of an incoming call / Initial conditions	763

26.8.1.3.1	Incoming call / U0 null state	766

26.8.1.3.1.1	Incoming call / U0 null state / SETUP received with a non supported bearer capability	766

26.8.1.3.2.	Incoming call / U6 call present	767

26.8.1.3.2.1	Incoming call / U6 call present / automatic call rejection	767

26.8.1.3.3	Incoming call / U9 mobile terminating call confirmed	769

26.8.1.3.3.1	Incoming call / U9 mobile terminating call confirmed / alerting or immediate connecting	769

26.8.1.3.3.2	Incoming call / U9 mobile terminating call confirmed / TCH assignment	770

26.8.1.3.3.3	Incoming call / U9 mobile terminating call confirmed / termination requested by the user	771

26.8.1.3.3.4	Incoming call / U9 mobile terminating call confirmed / DISCONNECT received	772

26.8.1.3.3.5	Incoming call / U9 mobile terminating call confirmed / RELEASE received	774

26.8.1.3.3.6	Incoming call / U9 mobile terminating call confirmed / lower layer failure	775

26.8.1.3.3.7	Incoming call / U9 mobile terminating call confirmed / unknown message received	776

26.8.1.3.4	Incoming call / U7 call received	777

26.8.1.3.4.1	Incoming call / U7 call received / call accepted	777

26.8.1.3.4.2	Incoming call / U7 call received / termination requested by the user	779

26.8.1.3.4.3	Incoming call / U7 call received / DISCONNECT received	780

26.8.1.3.4.4	Incoming call / U7 call received / RELEASE received	781

26.8.1.3.4.5	Incoming call / U7 call received / lower layer failure	782

26.8.1.3.4.6	Incoming call / U7 call received / unknown message received	784

26.8.1.3.4.7	Incoming call / U7 call received / TCH assignment	785

26.8.1.3.4.8	Incoming call / U7 call received / RELEASE COMPLETE received	786

26.8.1.3.5	Incoming call / U8 connect request	787

26.8.1.3.5.1	Incoming call / U8 connect request / CONNECT acknowledged	787

26.8.1.3.5.2	Incoming call / U8 connect request / timer T313 time-out	788

26.8.1.3.5.3	Incoming call / U8 connect request / termination requested by the user	790

26.8.1.3.5.4	Incoming call / U8 connect request / DISCONNECT received with in-band information	791

26.8.1.3.5.5	Incoming call / U8 connect request / DISCONNECT received without in-band information	792

26.8.1.3.5.6	Incoming call / U8 connect request / RELEASE received	793

26.8.1.3.5.7	Incoming call / U8 connect request / lower layer failure	795

26.8.1.3.5.8	Incoming call / U8 connect request / TCH assignment	796

26.8.1.3.5.9	Incoming call / U8 connect request / unknown message received	797

26.8.1.4	In call functions	798

26.8.1.4.1	In-call functions / DTMF information transfer	798

26.8.1.4.1.1	In-call functions / DTMF information transfer / basic procedures	798

26.8.1.4.2	In-call functions / user notification	800

26.8.1.4.2.1	In-call functions / User notification / MS terminated	800

26.8.1.4.3	In-call functions / channel changes	801

26.8.1.4.3.1	In-call functions / channel changes / a successful channel change in active state/ Handover and Assignment Command	801

26.8.1.4.3.2	In-call functions / channel changes / an unsuccessful channel change in active mode/ Handover and Assignment Command	803

26.8.1.4.4	In-call functions / MS terminated in-call modification	805

26.8.1.4.4.1	In-call functions / MS terminated in-call modification / modify when new mode is not supported	805

26.8.1.4.5	In-call functions / MS originated in-call modification	806

26.8.1.4.5.1	In-call functions / MS originated in-call modification / a successful case of modifying	806

26.8.1.4.5.2	In-call functions / MS originated in-call modification / modify rejected	808

26.8.1.4.5.3	In-call functions / MS originated in-call modification / an abnormal case of acceptance	810

26.8.1.4.5.4	In-call functions / MS originated in-call modification / an abnormal case of rejection	811

26.8.1.4.5.5	In-call functions / MS originated in-call modification / time-out of timer T323	812

26.8.1.4.5.6	In-call functions / MS originated in-call modification / a successful channel change in state mobile originating modify	814

26.8.1.4.5.7	In-call functions / MS originated in-call modification / an unsuccessful channel change in state mobile originating modify	816

26.8.1.4.5.8	In-call functions / MS originated in-call modification / unknown message received	818

26.8.1.4.5.9	In-call functions / MS originated in-call modification / a release complete received	819

26.8.1.4.5.10	In-call functions/MS originated in-call modification/contents of some of the messages	821

26.8.2	Call Re-establishment	823

26.8.2.1	Call Re-establishment/call present, re-establishment allowed	823

26.8.2.2	Call Re-establishment/call present, re-establishment not allowed	825

26.8.2.3	Call Re-establishment/call under establishment, transmission stopped	826

26.8.3	User to user signalling	828

26.8.4	Default contents of message	830

26.9	Structured procedures	837

26.9.1	Structured procedures / general	837

26.9.2	Structured procedures / MS originated call / early assignment	837

26.9.3	Structured procedures / MS originated call / late assignment	840

26.9.4	Structured procedures / MS terminated call / early assignment	842

26.9.5	Structured procedures / MS terminated call / late assignment	845

26.9.6	Structured procedures / emergency call	848

26.9.6.1	Structured procedures / emergency call / idle updated	848

26.9.6.1.1	Structured procedures / emergency call / idle updated / preferred channel rate	848

26.9.6.1.2	Structured procedures / emergency call / idle updated, non-preferred channel rate	851

26.9.6.2	Structured procedures / emergency call / idle, no IMSI	851

26.9.6.2.1	Structured procedures / emergency call / idle, no IMSI / accept case	851

26.9.6.2.2	Structured procedures / emergency call / idle, no IMSI / reject case	853

26.9.7	Default contents of messages	855

26.10	E-GSM signalling	861

26.10.1	E-GSM signalling / general considerations	861

26.10.2	E-GSM signalling / RR	861

26.10.2.1	E-GSM signalling / RR / Measurement	861

26.10.2.2	E-GSM signalling / RR / Immediate assignment	866

26.10.2.3	E-GSM signalling / RR / channel assignment procedure	867

26.10.2.4	E-GSM signalling / RR / Handover	870

26.10.2.4.1	E-GSM signalling / RR / Handover / Successful handover	870

26.10.2.4.2	E-GSM signalling / RR / Handover / layer 1 failure	874

26.10.2.5	E-GSM signalling / RR / Frequency Redefinition	875

26.10.3	E-GSM signalling / Structured procedure	877

26.10.3.1	E-GSM signalling / Structured procedure / Mobile originated call	878

26.10.3.2	E-GSM signalling / Structured procedures / emergency call	880

26.10.3.3	Default contents of messages	883

26.10.4	E-GSM signalling / Default message contents	883

26.11	Multiband signalling	888

26.11.1	General considerations	888

26.11.2	Multiband signalling / RR	888

26.11.2.1	Multiband signalling / RR / Immediate assignment procedure	888

26.11.2.2	Multiband signalling / RR / Handover	890

26.11.2.2.1	Multiband signalling / RR / Handover / successful / active call / non-synchronized	890

26.11.2.2.2	Multiband signalling / RR / Handover / layer 1 failure	895

26.11.2.2.3	Multiband signalling / RR / Handover / Multiband BCCH / successful / active call / non synchronized.	897

26.11.2.2.4	Multiband signalling / RR / Handover/ Multiband BCCH / Intracell Handover - Interband Assignment.	900

26.11.2.3	Multiband signalling / RR / Measurement reporting	907

26.11.3	Multiband signalling / MM	910

26.11.3.1	Multiband signalling / MM / Location updating	910

26.11.3.1.1	Location updating / accepted	910

26.11.3.1.2	Location updating / periodic	913

26.11.4	Multiband signalling / CC	916

26.11.5	Multiband signalling / Structured procedures	916

26.11.5.1	Multiband signalling / Structured procedures / MS originated call / early assignment	916

26.11.5.2	Structured procedures / MS terminated call / late assignment	919

26.11.6	Multiband signalling / Default messages contents	923

26.12	Enhanced Full Rate signalling	928

26.12.1	EFR signalling/ test of the channel mode modify procedure	928

26.12.2	EFR signalling/ tests of handover	931

26.12.2.1	EFR signalling / Handover /  active call / successful case	932

26.12.2.2	EFR signalling/ Handover / successful / call under establishment / non-synchronized	949

26.12.3	EFR Signalling / Structured procedures / MS originated call / late assignment	964

26.12.4	Structured procedures / MS terminated call / l early assignment	967

26.12.5	Structured procedures / emergency call	970

26.12.6	Default contents of  layer 3 messages for Enhanced Full rate speech tests	973

26.13	Multislot signalling	977

26.13.1	Multislot signalling / RR	977

26.13.1.1	Multislot signalling / RR / Measurement	977

26.13.1.1.1	Multislot signalling / RR / Measurement / symmetric	977

26.13.1.1.2	Multislot signalling / RR / Measurement / asymmetric	980

26.13.1.2	Multislot signalling / RR / Dedicated assignment	985

26.13.1.2.1	Multislot signalling / RR / Dedicated assignment / successful case	985

26.13.1.2.2	Multislot signalling / RR / Dedicated assignment / failure / general case	994

26.13.1.3	Test of handover	1002

26.13.1.3.1	Multislot signalling / RR / Handover / successful / active call / non-synchronized	1003

26.13.1.3.2	Multislot signalling / RR / Handover / successful / call under establishment / non synchronized / resource upgrading	1006

26.13.1.3.3	Multislot signalling / RR / Handover / successful / active call / finely synchronized / resource downgrading	1011

26.13.1.3.4	Multislot signalling / RR / Handover / successful / call under establishment / finely synchronized / relocation of channels	1014

26.13.1.3.5	Multislot signalling / RR /Handover / successful / call under establishment / pre-synchronized / resource upgrading	1021

26.13.1.4	Multislot signalling / RR / Test of the channel mode modify procedure	1025

26.13.1.5	Multislot signalling / RR / Early classmark sending	1027

26.13.1.6	Default contents of layer 3 messages for RR tests	1029

26.13.1.6.1	Default contents of GSM 900 layer 3 messages for RR tests	1029

26.13.1.6.2	Default contents of DCS 1 800 layer 3 messages for RR tests	1040

26.13.2	Multislot signalling / CC	1051

26.13.2.1	Multislot signalling / CC / In-call functions	1051

26.13.2.1.1	Multislot signalling / CC / In-call functions / User initiated service level upgrade / successful	1051

26.13.2.1.2	Multislot signalling / CC / In-call functions / User initiated service level downgrade / successful	1052

26.13.2.1.3	Multislot signalling / CC / In-call functions / User initiated service level upgrade / Time-out of timer T323	1054

26.13.2.1.4	Multislot signalling / CC / In-call functions / User initiated service level upgrade / modify reject	1056

26.13.2.1.5	Multislot signalling / CC / In call functions / contents of some of the messages	1057

26.13.3	Multislot signalling / Structured procedures	1058

26.13.3.1	Multislot signalling / Structured procedures / MS originated call / early assignment / HSCSD / non-transparent	1058

26.13.3.2	Multislot signalling / Structured procedures / MS originated call / late assignment / HSCSD / non-transparent	1061

26.13.3.3	Multislot signalling / Structured procedures / MS originated call / early assignment / HSCSD / Transparent	1064

26.13.3.4	Multislot signalling / Structured procedures / MS Terminated call / early assignment / HSCSD / non-transparent	1067

26.13.3.5	Multislot signalling / Structured procedures / MS Terminated call / early assignment / HSCSD / Transparent	1072

26.13.3.6	Default test conditions during layer 3 tests	1075

26.13.3.7	Default contents of messages	1077

27	Testing of the SIM/ME interface	1084

27.1	MS identification by short IMSI	1087

27.1.1	MS identification by short IMSI - Normal case	1087

27.1.2	MS identification by short IMSI, Phase 1 DCS SIM	1088

27.2	MS identification by short TMSI	1089

27.3	MS identification by long TMSI	1090

27.4	MS identification by long IMSI, TMSI updating and cipher key sequence number assignment	1091

27.5	Forbidden PLMNs, location updating and undefined cipher key	1093

27.6	MS updating forbidden PLMNs	1096

27.7	MS deleting forbidden PLMNs	1098

27.8	MS updating the PLMN selector list	1099

27.9	MS recognizing the priority order of the PLMN selector list	1100

27.10	MS access control management	1102

27.11	Exchange protocol tests	1112

27.11.1	Character transmission	1112

27.11.1.1	Bit/character duration during the transmission from the ME to the SIM	1112

27.11.1.2	Bit/character duration during the transmission from the SIM simulator to the ME	1112

27.11.1.3	Inter-character delay	1113

27.11.1.4	Error handling during the transmission from the ME to the SIM	1114

27.11.1.5	Error handling during transmission from the SIM to the ME	1115

27.11.2	Answer to reset (RST)	1116

27.11.2.1	Acceptance of SIMs with internal RST	1116

27.11.2.2	Acceptance of SIMs with active low RST	1116

27.11.2.3	Characters of the answer to reset	1117

27.11.2.4	PTS procedure	1118

27.11.2.5	Reset repetition	1119

27.11.2.6	Speed Enhancement	1120

27.11.3	Command processing, procedure bytes	1121

27.12	Evaluation of directory characteristics	1122

27.12.1	Operating speed in authentication procedure	1122

27.12.2	Clock stop	1122

27.13	Mechanical tests	1124

27.13.1	Contact pressure	1124

27.13.2	Shape of contacts for IC card SIM card reader	1124

27.14	Secret code usage	1125

27.14.1	Entry of PIN	1125

27.14.2	Change of PIN	1126

27.14.3	Disabling the PIN	1127

27.14.4	PUK entry	1128

27.14.5	Entry of PIN2	1129

27.14.6	Change of PIN2	1130

27.14.7	PUK2 entry	1131

27.15	Abbreviated Dialling Numbers (ADN)	1132

27.16	MMI reaction to SIM status encoding	1133

27.17	Electrical tests	1134

27.17.1	Test of the power transition phases	1135

27.17.1.1	Phase preceding ME power on	1135

27.17.1.2	Phase during SIM power on	1136

27.17.1.3	Phase during ME power off with clock stop forbidden	1137

27.17.1.4	Phase during ME power off with clock stop allowed	1139

27.17.1.5	SIM Type Recognition and Voltage Switching	1141

27.17.1.5.1	Reaction of 3V only MEs on SIM type recognition failure	1141

27.17.1.5.2	Reaction of 3V only MEs on type recognition of 5V only SIMs	1142

27.17.1.5.3	Reaction of 3V technology MEs on type recognition of 5V only SIMs	1143

27.17.1.5.4	Reaction of 3V technology MEs on type recognition of 3V technology SIMs	1144

27.17.2	Electrical tests on each ME contact	1145

27.17.2.1	Electrical tests on contact C1	1145

27.17.2.1.1	Test 1	1146

27.17.2.1.2	Test 2	1147

27.17.2.2	Electrical tests on contact C2	1149

27.17.2.3	Electrical tests on contact C3	1150

27.17.2.4	[Not used]	1152

27.17.2.5	Electrical tests on contact C7	1152

27.18	Fixed Number Dialling (FND)	1154

27.18.1	ME and SIM with FND activated	1154

27.18.1.1	EFADN invalidated and not readable or updatable	1154

27.18.2	ME and SIM with FND deactivated	1156

27.18.3	Enabling, disabling and updating of FND	1157

27.19	Phase identification	1158

27.20	SIM presence detection	1159

27.21	Advice of Charge (AoC)	1160

27.21.1	AoC not supported by SIM	1160

27.21.2	Maximum frequency of ACM updating	1161

27.21.3	Call terminated when ACM greater than ACMmax	1164

27.21.4	Response codes of increase command	1166

28	Test of autocalling restrictions	1169

28.1	General	1169

28.2	Constraining the access to a single number (GSM 02.07 category 3)	1169

28.3	Constraining the access to a single number (GSM 02.07 categories 1 and 2)	1171

28.4	Behaviour of the MS when its list of blacklisted numbers is full	1173

29	Testing of bearer services	1175

29.1	General	1175

29.2	Testing of transparent data services	1175

29.2.1	Verification of synchronization	1175

29.2.2	Filtering of channel control information for transparent BCs	1178

29.2.3	Correct Terminal Compatibility Decision	1180

29.2.3.1	Negotiation of Radio Channel Requirement (RCR)	1180

29.2.3.2	Negotiation of Connection Element (CE)	1180

29.2.3.3	Negotiation of Number of Stop Bits, Number of Data bits, and Parity	1181

29.2.3.4	Negotiation of Modem Type	1182

29.2.3.5	Negotiation of Intermediate Rate	1182

29.2.3.6	Negotiation of User Information Layer 2 Protocol	1183

29.2.3.7	Negotiation between TS 61 and TS 62: Mobile Originated call.	1184

29.2.3.8	Negotiation between TS 61 and TS 62: Mobile Terminated call.	1184

29.2.4	Data Rate Adaptation for Synchronous Transparent Bearer Capabilities	1185

29.2.5	Network Independent Clocking	1186

29.2.6	Asynchronous Transparent Bearer Capabilities	1186

29.2.6.1	Data Rate Adaptation	1186

29.2.6.2	Passage of the Break Signal	1187

29.2.6.3	Overspeed/Underspeed Handling (Local Terminal)	1189

29.2.6.4	Overspeed/Underspeed Handling (Remote Terminal)	1190

29.2.7	Interchange circuit mapping for transparent bearer capabilities	1191

29.3	Testing of non transparent data services (RLP tests)	1192

29.3.1	Initialization	1192

29.3.1.1	Normal initialization done by the MS	1192

29.3.1.2	Initialization failure	1193

29.3.1.2.1	Loss of UA frame	1193

29.3.1.2.2	Total loss of UA frame	1194

29.3.2	Data transfer	1196

29.3.2.1	Default conditions	1196

29.3.2.2	MS sends I+S frames	1196

29.3.2.2.1	N(S) sequence number	1196

29.3.2.2.2	Transmission window	1197

29.3.2.2.3	Busy condition	1199

29.3.2.3	SS sends I+S frames	1201

29.3.2.3.1	N(R) sequence number	1201

29.3.2.3.2	Busy condition	1202

29.3.2.4	SS rejects I+S frames	1203

29.3.2.4.1	REJ frame	1203

29.3.2.4.2.	SREJ frame	1205

29.3.2.4.3	I+S reject frame	1207

29.3.2.5	MS rejects I+S frames	1210

29.3.2.5.1	Rejection with REJ or SREJ supervisory frames	1210

29.3.2.5.2	Retransmission of REJ or SREJ frames	1215

29.3.2.5.3	I+S reject frame	1217

29.3.2.6	Checkpoint recovery	1219

29.3.2.6.1	SS in checkpoint recovery mode	1219

29.3.2.6.2	End of the window	1222

29.3.2.6.3	End of a sequence	1224

29.3.2.6.4	Time-out of one frame	1226

29.3.2.6.5	No response to checkpointing	1227

29.3.2.6.6	Incorrect response to checkpointing	1229

29.3.2.6.7	Total loss of response to checkpointing	1231

29.3.2.6.8	Retransmission of a sequence	1233

29.3.2.6.9	N2 retransmission of a sequence	1236

29.3.3	Negotiation of the RLP parameters	1238

29.3.3.1	Negotiation initiated by the SS	1238

29.3.3.2	Negotiation initiated by the MS	1243

29.3.3.3	Collision of XID frames	1246

29.3.3.4	Loss of XID frames	1250

29.3.3.5	Total loss of XID frames	1251

29.4	Facsimile tests for the transparent network support	1252

29.4.1	General	1252

29.4.2	Mobile originated call	1253

29.4.2.1	Call establishment procedure	1253

29.4.2.1.1	Alternate speech / facsimile	1253

29.4.2.1.2	Automatic facsimile	1255

29.4.2.2	Pre-message procedure	1256

29.4.2.3	Message procedure	1257

29.4.2.4	Post-message procedure	1259

29.4.2.5	Call release procedure	1260

29.4.2.6	CTC processing - 4th PPR for the same block	1261

29.4.2.7	Transition from Facsimile to Speech - Procedure interrupt generated by receiving station	1262

29.4.2.8	Transition from Facsimile to Speech - Procedure interrupt generated by transmitting station	1264

29.4.2.9	Quality check	1265

29.4.3	Mobile terminated call	1266

29.4.3.1	Call Establishment Procedure	1266

29.4.3.1.1	Alternate Speech/Facsimile	1266

29.4.3.1.1.1	DCD Mobile Terminated	1266

29.4.3.1.1.2	DCD mobile originated	1267

29.4.3.1.2	Automatic facsimile	1268

29.4.3.2	Pre-message procedure	1270

29.4.3.3	Message procedure	1271

29.4.3.4	Post-message procedure	1272

29.4.3.5	Call release procedure	1273

29.4.3.6	Speed conversion factor	1274

29.4.3.7	Quality Check	1277

29.4.4	Notes	1278

30	Speech teleservices	1279

30.1	Sending sensitivity/frequency response	1279

30.2	Sending loudness rating	1280

30.3	Receiving sensitivity/frequency response	1281

30.4	Receiving loudness rating	1282

30.5	Side tones	1283

30.5.1	Side Tone Masking Rating (STMR)	1283

30.5.2	Listener Side Tone Rating (LSTR)	1284

30.6	Telephone Acoustic coupling Loss (TAL)	1285

30.6.1	Echo Loss (EL)	1285

30.6.2	Stability margin	1286

30.7	Distortion	1287

30.7.1	Sending	1287

30.7.2	Receiving	1288

30.8	Sidetone distortion	1289

30.9	Out-of-band signals	1290

30.9.1	Sending	1290

30.9.2	Receiving	1291

30.10	Idle channel noise	1292

30.10.1	Sending	1292

30.10.2	Receiving	1293

30.11	 Ambient Noise Rejection	1294

30.11.1	  Definition and Applicability.	1294

30.11.2	 Conformance Requirement.	1294

30.11.3	  Test Purpose.	1294

30.11.4	 Method of Test.	1294

31	Test of supplementary services	1296

31.1	Number identification supplementary services	1296

31.2	Call offering supplementary services	1296

31.2.1	Call forwarding supplementary services	1296

31.2.1.1	Registration	1296

31.2.1.1.1	Registration accepted	1296

31.2.1.1.2	Registration rejected	1299

31.2.1.2	Erasure by the subscriber	1302

31.2.1.2.1	Erasure accepted	1302

31.2.1.2.2	Erasure rejected	1304

31.2.1.3	Activation	1307

31.2.1.4	Deactivation	1309

31.2.1.5	Invocation	1311

31.2.1.6	Interrogation	1311

31.2.1.6.1	Interrogation accepted	1311

31.2.1.6.2	Interrogation rejected	1313

31.2.1.7	Normal operation	1316

31.2.1.7.1	Served mobile subscriber side	1316

31.2.1.7.1.1	Notification during an incoming call	1316

31.2.1.7.1.2	Notification during an outgoing call	1318

31.2.1.7.2	Forwarded-to mobile subscriber side	1320

31.2.2	Call transfer and mobile access hunting supplementary services	1321

31.3	Call completion supplementary services	1321

31.4	Multi-party supplementary services	1321

31.5	Community of interest supplementary services	1321

31.6	Charging supplementary services	1322

31.6.1	Advice of Charge Charging	1322

31.6.1.1	AOC time related charging / MS originated call	1322

31.6.1.2	AOC time related charging / MS terminated call	1325

31.6.1.3	AOC volume related charging / MS originated call	1328

31.6.1.4	AOC volume related charging / MS terminated call	1328

31.6.1.5	Change in charging information during a call	1328

31.6.1.6	Different formats of charging information	1331

31.6.1.7	AOC on a Call Hold call	1334

31.6.1.8	AOC on a Multi-party call	1337

31.6.2	Charge Storage	1341

31.6.2.1	Removal of SIM during an active call	1341

31.6.2.2	Interruption of power supply during an active call	1344

31.6.2.3	MS going out of coverage during an active AoCC call	1346

31.6.2.4	ACMmax operation / Mobile Originating	1348

31.6.2.5	ACMmax operation / Mobile Terminating	1351

31.6.3	Advice of Charge Information	1354

31.6.4	Default contents of messages	1354

31.7	Additional information transfer supplementary services	1355

31.8	Call restriction supplementary services	1355

31.8.1	Registration of a password	1356

31.8.1.1	Registration accepted	1356

31.8.1.2	Registration rejected	1358

31.8.1.2.1	Rejection after invoke of the RegisterPassword operation	1358

31.8.1.2.2	Rejection after password check with negative result	1360

31.8.1.2.3	Rejection after new password mismatch	1362

31.8.2	Erasure	1365

31.8.3	Activation	1365

31.8.3.1	Activation accepted	1365

31.8.3.2	Activation rejected	1368

31.8.3.2.1	Rejection after invoke of ActivateSS operation	1368

31.8.3.2.2	Rejection after use of password procedure	1370

31.8.4	Deactivation	1372

31.8.4.1	Deactivation accepted	1372

31.8.4.2	Deactivation rejected	1375

31.8.4.2.1	Rejection after invoke of DeactivateSS operation	1375

31.8.4.2.2	Rejection after use of password procedure	1377

31.8.5	Invocation	1379

31.8.6	Interrogation	1379

31.8.6.1	Interrogation accepted	1379

31.8.6.2	Interrogation rejected	1382

31.8.7	Normal operation	1384

31.9	Handling of undefined (future) GSM supplementary services	1385

31.9.1	Mobile station initiated Unstructured supplementary service data operation	1385

31.9.1.1	ProcessUnstructuredSS-request/accepted	1385

31.9.1.2	ProcessUnstructuredSS-request/cross phase compatibility and error handling	1390

31.9.2	Network initiated unstructured supplementary service operations	1395

31.9.2.1	UnstructuredSS-Notify/accepted	1395

31.9.2.3	UnstructuredSS-Request/accepted	1400

31.10	MMI input for USSD	1406

31.11	Specific message contents and ASN.1 codings	1407

32	Testing of speech transcoding functions	1457

32.1	Full Rate Downlink speech transcoding	1457

32.2	Full Rate Downlink receiver DTX functions	1458

32.3	Full Rate Uplink speech transcoding	1461

32.4	Full Rate Uplink transmitter DTX functions	1462

32.5	Full Rate Speech channel transmission delay	1463

32.5.4	Downlink processing delay	1464

32.5.5	Downlink coding delay	1464

32.5.6	Uplink processing delay	1465

32.5.7	Uplink coding delay	1465

32.6	Half Rate Downlink speech transcoding	1466

32.7	Half Rate Downlink receiver DTX functions	1467

32.8	Half Rate Uplink speech transcoding	1468

32.9	Half Rate Uplink transmitter DTX functions	1469

32.10	Half Rate Speech channel transmission delay	1471

32.10.4	Downlink processing delay	1471

32.10.5	Downlink coding delay	1472

32.10.6	Uplink processing delay	1472

32.10.7	Uplink coding delay	1473

32.11	Intra cell channel change from a TCH/HS to a TCH/FS	1474

32.12	Intra cell channel change from a TCH/FS to a TCH/HS	1475

33	Mobile station features	1478

33.1	Entry and display of called number	1478

33.2	Indication of call progress signals	1479

33.2.4	Ringing tone	1479

33.2.5	Busy tone	1479

33.2.6	Congestion tone	1480

33.2.7	Authentication failure tone	1480

33.2.8	Number unobtainable tone	1481

33.2.9	Call dropped tone	1481

33.3	Network selection / indication	1482

33.4	Invalid and blocked PIN indicators	1486

33.5	Service indicator	1487

33.6	Subscription identity management	1487

33.7	Barring of outgoing calls	1488

33.8	Prevention of unauthorized calls	1489

34	Short message service (SMS)	1490

34.1	General	1490

34.2	Short message service point to point	1490

34.2.1	SMS mobile terminated	1490

34.2.2	SMS mobile originated	1496

34.2.3	Test of memory full condition and memory available notification:	1501

34.2.4	Test of the status report capabilities and of SMS-COMMAND:	1506

34.2.5	Test of message class 0 to 3	1510

34.2.5.1	Short message class 0	1510

34.2.5.2	Test of class 1 short messages	1513

34.2.5.3	Test of class 2 short messages	1515

34.2.5.4	Test of class 3 short messages	1519

34.2.6	Test of short message type 0	1519

34.2.7	Test of the replace mechanism for SM type 1-7	1519

34.2.8	Test of the reply path scheme	1523

34.2.9	Multiple SMS mobile originated	1526

34.2.9.1	MS in idle mode	1526

34.2.9.2	MS in active mode	1530

34.3	Short message service cell broadcast	1533

34.4	 Default message contents:	1535

35	Low battery voltage detection	1538

36	Individual equipment type requirements and interworking - special conformance testing functions	1540

36.1	General	1540

36.2	Activation and deactivation of special test functions in the MS	1540

36.2.1	Internal test loops	1540

36.2.1.1	TCH loop	1541

36.2.1.1.1	TCH loop including signalling of erased frames (A)	1542

36.2.1.1.2	Speech TCH loop without signalling of erased frames (B)	1542

36.2.1.1.3	TCH burst-by-burst loop (C)	1542

36.2.1.1.4	TCH loop including signalling of erased frames and unreliable frames (D)	1543

36.2.1.1.5	TCH loop including signalling of erased SID frames (E)	1543

36.2.1.1.6	TCH loop including signalling of erased valid SID frames (F)	1544

36.2.1.1.7	Deactivating TCH loops	1544

36.2.1.1.8	Additional non-mandatory operating characteristics	1544

36.2.2	Activating and deactivating EMMI	1545

36.2.3	Activating and deactivating DAI tests	1545

36.2.4	Message definitions and contents	1545

36.2.4.1	CLOSE_TCH_LOOP_CMD	1546

36.2.4.2	CLOSE_TCH_LOOP_ACK	1546

36.2.4.3	OPEN_LOOP_CMD	1546

36.2.4.4	Command for the activation of the EMMI, ACT_EMMI_CMD	1547

36.2.4.5	Acknowledge of the activation of the EMMI, ACT_EMMI_ACK	1547

36.2.4.6	Deactivation of the EMMI, DEACT_EMMI	1547

36.2.4.7	Test_Interface	1548

36.2.5	Timer values	1548

36.3	Electrical Man Machine Interface (EMMI)	1548

36.3.1	Use of the EMMI	1548

36.3.2	Formal aspects	1548

36.3.3	Layered structure of the interface	1549

36.3.4	Terminology	1549

36.3.5	Description of the EMMI	1549

36.3.5.1	EMMI, Layer 1	1549

36.3.5.1.1	Mechanical and electrical characteristics	1549

36.3.5.1.2	Transmission and reception characteristics	1549

36.3.5.2	EMMI, layer 2	1550

36.3.5.2.1	General structure	1550

36.3.5.2.2	Control frames	1550

36.3.5.2.3	Frame structure	1550

36.3.5.2.4	Flow of I-frames on layer 2	1550

36.3.5.2.4.1	Transmission of frames	1550

36.3.5.2.4.2	Reception of frames	1551

36.3.5.2.4.3	Use of ACK and NAK on receiving side	1551

36.3.5.2.4.4	Use of XON and XOF	1551

36.3.5.2.4.5	Parameters on layer 2	1551

36.3.5.3	EMMI, layer 3	1552

36.3.5.3.1	Message structure	1552

36.3.5.3.2	Definition of messages	1553

36.4	Digital audio interface	1559

36.4.1	General	1559

36.4.2	Formal aspects	1559

36.4.3	Hardware aspect of the interface	1559

36.4.3.1	Mechanical characteristics of the interface	1559

36.4.3.2	Electrical characteristics of the interface	1560

36.4.3.3	Timing characteristics of the interface	1560

36.4.4	Logical interface	1560

36.4.5	Functionality of the DAI	1561

36.5	SIM/ME test interface	1561

36.5.1	General	1561

36.5.2	Formal aspects	1562

36.5.3	Hardware and logical aspects of the interface	1562

36.5.4	Mechanical characteristics of the interface	1562

Annex 1 (Normative):	 Reference test methods	1564

A1.1	General Conditions (GC)	1564

A1.1.1	Outdoor test site and general arrangements for measurements involving the use of radiated fields (GC4)	1564

A1.1.2	Anechoic shielded chamber (GC5)	1564

A1.1.3	Temporary antenna connector (GC7)	1565

A1.1.4	Temporary antenna connector characteristics	1565

A1.1.5	Calibration of the temporary antenna connector	1566

A1.1.5.1	Antenna radiation pattern	1566

A1.1.5.2	Test range calibration	1568

A1.1.5.3	Temporary antenna connector coupling factor	1568

A1.2	Normal and extreme Test Conditions (TC)	1569

A1.2.1	Power sources and ambient temperatures (TC2)	1569

A1.2.2	Normal test conditions (TC2.1)	1570

A1.2.3	Extreme test conditions (TC2.2)	1570

A1.2.4	Vibration requirements (TC4)	1571

Annex 2:  Not used	1572

Annex 3:  Protocol implementation information	1572

A3.1	Protocol Implementation Conformance Statement (PICS)	1572

A3.1.1	LAPDm protocol (GSM 04.05 and 04.06)	1572

A3.1.1.1	Simplified protocol - GSM 04.06 section 6	1572

A3.1.1.2	Management of SAPI = 3 - GSM 04.11 section 2.3	1572

A3.1.2	Mobility management	1572

A3.1.2.1	IMSI detach initiation by the MS - GSM 04.08 section 4.3.4.1	1572

A3.1.2.2	IMSI detach completion by the MS - GSM 04.08 section 4.3.4.3	1572

A3.1.2.3	MM specific procedures - GSM 04.08 section 4.4 and 4.5.1.1	1572

A3.1.2.4	Receiving an MM STATUS message - GSM 04.08 section 4.6	1572

A3.1.3	Call control	1573

A3.1.3.1	Status enquiry procedures - GSM 04.08 section 5.5.3.1	1573

A3.1.3.2	Receiving a STATUS message by a CC entity - GSM 04.08 section 5.5.3.2	1573

A3.1.3.3	Called side compatibility checking - GSM 04.08 annex B.3	1573

A3.1.3.4	Disconnect on incoming call	1573

A3.1.4	Layer 1	1573

A3.1.4.1	Optional storage of BCCH carrier information - GSM 05.08 section 6.3	1573

A3.1.5	Autocalling - (ref.: GSM 02.07, annex 1)	1573

A3.1.6	Transient states	1573

A3.2	Protocol Implementation Extra Information for Testing (PIXIT)	1574

A3.2.0	Introduction	1574

A3.2.1	Basic characteristics	1574

A3.2.1.1	Type of antenna	1574

A3.2.1.2	Power supply	1574

A3.2.1.3	Power class of the MS	1574

A3.2.1.4	Channel modes supported	1575

A3.2.1.5	Teleservices supported	1575

A3.2.1.6	Supplementary services supported	1575

A3.2.1.7	Bearer services supported	1575

A3.2.1.8	SIM removal	1575

A3.2.1.9	Classmark	1575

A3.2.1.10	Type of SIM/ME interface (ref. GSM 11.11 and GSM 11.12)	1576

A3.2.2	Man machine interface	1576

A3.2.2.1	Mobile station features	1576

A3.2.2.2	Short message service	1577

A3.2.2.3	Supplementary services	1577

A3.2.2.3.1	Call forwarding	1577

A3.2.2.3.2	Call restriction	1577

A3.2.2.3.3	Handling of (undefined) GSM supplementary services	1577

A3.2.3	Electrical Man Machine Interface (EMMI)	1578

A3.2.3.1	Methods supported for activation/deactivation of EMMI	1578

A3.2.3.2	Transmission rate supported by the ME on the EMMI	1578

A3.2.3.3	Layer 3 messages supported on the EMMI	1578

A3.2.3.4	Keystroke sequence messages	1578

A3.2.3.5	Internal malfunction detected messages	1578

A3.2.4	Digital Audio Interface (DAI)	1578

A3.2.5	Characteristics related to bearer services or teleservices	1578

A3.2.5.1	Access interface	1578

A3.2.5.2	Configuration of the MT	1579

A3.2.5.3	Capability information	1579

A3.2.5.4	Subaddress or DDI number	1579

A3.2.5.5	User to user signalling	1579

A3.2.5.6	Data call set-up and data call clearing	1579

A3.2.5.7	Characteristics of non-transparent data services	1580

A3.2.5.8	Possible ways of setting-up a call from either an external interface or internally	1580

A3.2.5.9	Application layer causing automatic call termination.	1580

A3.2.5.10	Call re-establishment for MS not supporting speech	1580

A3.2.6	International mobile station equipment identity	1580

A3.2.7	Receiver intermediate frequencies	1580

A3.2.8	Artificial ear	1580

Annex 4:  Test SIM Parameters	1581

A4.1	Introduction	1581

A4.1.1	Definitions	1581

A4.1.2	Definition of the test algorithm for authentication	1581

A4.2	Default Parameters for the test SIM	1581

A4.3.	Default settings for the Elementary Files (EFs).	1581

A4.3.1	EFICCID (ICC Identification)	1582

A4.3.2	EFLP (Language preference)	1582

A4.3.3	EFIMSI (IMSI)	1582

A4.3.4	EFKc (Ciphering key Kc)	1582

A4.3.5	EFPLMNsel (PLMN selector)	1582

A4.3.6	EFHPLMN (HPLMN search period)	1582

A4.3.7	EFACMmax (ACM maximum value)	1583

A4.3.8	EFSST (SIM service table)	1583

A4.3.9	EFACM (Accumulated call meter)	1583

A4.3.10	EFPUCT (Price per unit and currency table)	1583

A4.3.11	EFCBMI (Cell broadcast Message Identifier Selection)	1583

A4.3.12	EFBCCH (Broadcast control channels)	1584

A4.3.13	EFACC (Access control class)	1584

A4.3.14	EFFPLMN (Forbidden PLMNs)	1584

A4.3.15	EFLOCI (Location information)	1584

A4.3.16	EFAD (Administrative data)	1584

A4.3.17	EFPhase (Phase identification)	1584

A4.3.18	EFADN (Abbreviated dialling numbers)	1585

A4.3.19	EFFDN (Fixed dialling numbers)	1585

A4.3.20	EFSMS (Short messages)	1585

A4.3.21	EFCCP (Capability configuration parameters)	1585

A4.3.22	EFMSISDN (MSISDN)	1585

A4.3.23	EFSMSP (Short message service parameters)	1585

A4.3.24	EFSMSS (SMS status)	1585

A4.3.25	EFEXT1 (Extension 1)	1585

A4.3.26	EFEXT2 (Extension 2)	1585

Annex 5: Test equipment	1586

A5.1	Introduction	1586

A5.1.1	General	1586

A5.1.2	Test equipment terms	1586

A5.1.3	Confidence level	1586

A5.2	Standard test signals	1586

A5.3	SS functional requirements	1587

A5.3.1	Level setting range	1587

A5.3.2	Level Measurement / operation range	1587

A5.3.3	MS power supply interface	1587

A5.3.4	MS antenna interface	1587

A5.3.4.1	Uplink receiver error	1587

A5.3.4.2	Power versus time measurements	1587

A5.3.4.3	Wideband selective power measurement	1590

A5.3.4.4	Inband selective power measurements	1590

A5.3.4.5	Phase trajectory and frequency error measurements	1590

A5.3.4.6	RF delay measurements relative to nominal times	1591

A5.3.4.7	The wanted signal or traffic channel of serving cell	1591

A5.3.4.8	The first interfering signal or traffic channel of the first adjacent cell	1591

A5.3.4.9	The second interfering signal	1592

A5.3.4.10	BCCH carriers of serving and adjacent cells	1592

A5.3.4.11	The wide frequency range signal	1593

A5.3.4.12	The multipath fading function	1593

A5.3.5	MS audio interface and DAI	1593

A5.3.5.1	General uncertainties	1593

A5.3.5.2	Analogue single test tone	1594

A5.3.5.3	Delay measurement between Um and DAI	1594

A5.4	SIM simulator functional requirements	1594

A5.4.1	General	1594

A5.4.2	Contacts C1, C2, C6, C7	1594

A5.4.2.1	Default measurement / setting uncertainties	1594

A5.4.2.2	Contact C1	1594

A5.4.2.3	Contact C7	1595

A5.4.3	Contact C3	1595

A5.4.4	Definition of timing	1595

History	1596