Annex B:	Partial IXIT proforma

Notwithstanding the provisions of the copyright clause related to the text of this ETS, ETSI grants that users of this ETS may freely reproduce the PIXIT proforma in this annex so that it can be used for its intended purposes and may further publish the completed PIXIT.

# Introduction

This partial IXIT proforma contained in this ETS, after augmented by the Test Realizer, is proposed to be provided to the client for completion, when the related Abstract Test Suite is to be used against client's Implementation Under Test (IUT).

Text in italics is comments for guidance for the production of a IXIT, and is not to be included in the actual IXIT.

The completed IXIT will normally be used in conjunction with the completed ICS, as it adds precision to the information provided by the ICS.

# B.1	Identification Summary

This table is completed by the test laboratory. The item "Contract References" is optional.

Table 1: Identification Summary

| IXIT Reference Number |  |
| --- | --- |
| Test Laboratory Name |  |
| Date of Issue |  |
| Issued to (name of client) |  |
| Contract References |  |

# B.2	Abstract Test Suite Summary

In the following table the test laboratory provides the version number of the protocol specification and the version number of ATS which are used in the conformance testing.

Table 2: ATS Summary

| Protocol Specification | ETS 300 557 |
| --- | --- |
| Version of Protocol Specification |  |
| TSS & TP Specification | ETS 300 607-1 |
| Version of TSS & TP Specification |  |
| ATS Specification | ETS 300 607-3 |
| Version of ATS Specification |  |
| Abstract Test Method | Remote Test Method |

# B.3	Test Laboratory

## B.3.1	Test Laboratory Identification

The test laboratory provides the following information.

Table 3: Test Laboratory Identification

| Name of Test Laboratory |  |
| --- | --- |
| Postal Address |  |
| Office address |  |
| Telephone Number |  |
| FAX Number |  |

## B.3.2	Accreditation status of the test service

The test laboratory provides the following information.

Table 4: Accreditation status of the test service

| Accreditation status |  |
| --- | --- |
| Accreditation Reference |  |

## B.3.3	Manager of Test Laboratory

The test laboratory provides the information about the manager of test laboratory in the following table.

Table 5: Manager of Test Laboratory

| Name of Manager of Test Laboratory |  |
| --- | --- |
| Telephone Number |  |
| FAX Number |  |
| E-mail Address |  |

## B.3.4	Contact person of Test Laboratory

The test laboratory provides the information about the contact person of test laboratory in the following table.

Table 6: Contact person of Test Laboratory

| Name of Contact of Test Laboratory |  |
| --- | --- |
| Telephone Number |  |
| FAX Number |  |
| E-mail Address |  |

## B.3.5	Means of Testing

In the table below, the test laboratory provides a statement of conformance of the Means Of Testing (MOT) to the reference standardized ATS, and identifies all restrictions for the test execution required by the MOT beyond those stated in the reference standardized ATS.

Table 7: Means of Testing

Means of Testing

## B.3.6	Instructions for Completion

In this table, the test laboratory provides any specific instructions necessary for completion and return of the proforma from the client.

Table 8: Instruction for Completion

Instructions for Completion

# B.4	Client

## B.4.1	Client Identification

The client provides the identification in the following table.

Table 9: Client Identification

| Name of Client |  |
| --- | --- |
| Postal Address |  |
| Office Address |  |
| Telephone Number |  |
| FAX Number |  |

## B.4.2	Client Test Manager

In this table the client provides information about the test manager.

Table 10: Client Test Manager

| Name of Client Test Manager |  |
| --- | --- |
| Telephone Number |  |
| FAX Number |  |
| E-mail Address |  |

## B.4.3	Client Contact person

In this table the client provides information about the test contact person.

Table 11: Client Contact person

| Name of Client contact person |  |
| --- | --- |
| Telephone Number |  |
| FAX Number |  |
| E-mail Address |  |

## B.4.4	Test Facilities Required

In the following table, the client records the particular facilities required for testing, if a range of facilities is provided by the test laboratory.

Table 12: Test Facilities Required

Test Facilities Required

# B.5	System Under Test

## B.5.1	SUT Information

The client provides information about the SUT in the table below.

Table 13: SUT Information

| System Name |  |
| --- | --- |
| System Version |  |
| SCS Reference |  |
| Machine Configuration |  |
| Operating System Identification |  |
| IUT Identification |  |
| ICS Reference for the IUT |  |

## B.5.2	Limitations of the SUT

In the table below, the client provides information explaining if any of the abstract tests cannot be executed.

Table 14: Limitation of the SUT

Limitations of the SUT

## B.5.3	Environmental Conditions

In the table below the client provides information about any tighter environmental conditions for the correct operation of the SUT.

Table 15: Environmental Conditions

Environmental Conditions

# B.6	Ancillary Protocols

This clause is completed by the client in conjunction with the test laboratory.

In the following tables, the client identifies relevant information concerning each ancillary protocol in the SUT other than the IUT itself. One table for one ancillary protocol.

Based on the MOT the test laboratory should create question proforma for each ancillary protocol in the blank space following each table. The information required is dependent on the MOT and the SUT, and covers all the addressing, parameter values, timer values and facilities (relevant to ETSs) as defined by the ICS for the ancillary protocol.

## B.6.1	Ancillary Protocols 1

Table 16: Ancillary Protocol 1

| Protocol Name | ETS 300 |
| --- | --- |
| Version number |  |
| ICS Reference (optional) |  |
| IXIT Reference (optional) |  |
| PCTR Reference (optional) |  |

## B.6.2	Ancillary Protocols 2

Table 17: Ancillary Protocol 2

| Protocol Name | ETS 300 |
| --- | --- |
| Version number |  |
| ICS Reference (optional) |  |
| IXIT Reference (optional) |  |
| PCTR Reference (optional) |  |

# B.7	Protocol Layer Information for L3 of Mobile Station

## B.7.1	Information provided for test purposes by the MS supplier

| Item | Description | Type/Allowed values | Supported Value |
| --- | --- | --- | --- |
| TSPC_24DataF | 2.4 k full rate data mode supported | BOOLEAN |  |
| TSPC_24DataH | 2.4 k half rate data mode supported | BOOLEAN |  |
| TSPC_48DataF | 4.8 k full rate data mode supported | BOOLEAN |  |
| TSPC_48DataH | 4.8 k half rate data mode supported | BOOLEAN |  |
| TSPC_96Data | 9.6 k full rate data mode supported | BOOLEAN |  |
| TSPC_Feat_A51 | ciphering algorithm A5/1 supported | BOOLEAN |  |
| TSPC_Feat_A52 | ciphering algorithm A5/2 supported | BOOLEAN |  |
| TSPC_AddCharSet | A, B, C, D chars supported | BOOLEAN |  |
| TSPC_AddInfo_PseudoSynch | Pseudo synchronised supported | BOOLEAN |  |
| TSPC_AlertInd | alerting indication to the user supported | BOOLEAN |  |
| TSPC_SvcOnTCH | at least one service on traffic channel supported | BOOLEAN |  |
| TSPC_SMS | at least one short message service supported | BOOLEAN |  |
| TSPC_SS | at least one supplementary service supported | BOOLEAN |  |
| TSPC_AutoAutoMode | automatically enter automatic selection of PLMN mode supported | BOOLEAN |  |
| TSPC_BasCharSet | Chars 0-9, *, # supported | BOOLEAN |  |
| TSPC_CalledNumDisp | called number display supported | BOOLEAN |  |
| TSPC_DCS | testing DCS1 800 | BOOLEAN |  |
| TSPC_DetachOnPwrDn | detach on power down supported | BOOLEAN |  |
| TSPC_DetachOnSIMRmv | detach on SIM remove supported | BOOLEAN |  |
| TSPC_DispRcvSMS | display of received SMS supported | BOOLEAN |  |
| TSPC_DualRate | dual rate channel types supported | BOOLEAN |  |
| TSPC_EGSM | both standard and extended GSM band supported | BOOLEAN |  |
| TSPC_EmgOnly | the only circuit switched basic service is emergency call | BOOLEAN |  |
| TSPC_followOnReq | follow-on request procedure supported | BOOLEAN |  |
| TSPC_HalfRateData | half rate data supported | BOOLEAN |  |
| TSPC_HalfRateSpeech | half rate speech mode supported | BOOLEAN |  |
| TSPC_InCallMod | In-Call modification supported | BOOLEAN |  |
| TSPC_NoimmConn | at least one service not support immediate connection | BOOLEAN |  |
| TSPC_Feat_FND | FND feature supported | BOOLEAN |  |
| TSPC_NonCallSS | non call related supplementary service supported | BOOLEAN |  |
| TSPC_MTsvc | at least one MT circuit switched basic service supported | BOOLEAN |  |
| TSPC_MOsvc | at least one MO circuit switched basic service supported | BOOLEAN |  |
| TSPC_PGSM | only standard GSM band supported | BOOLEAN |  |
| TSPC_RefusalCall | refusal of call supported | BOOLEAN |  |
| TSPC_ReplaceSMS | replace SMS supported | BOOLEAN |  |
| TSPC_ReplyProc | (SMS) reply procedures supported | BOOLEAN |  |
| TSPC_RFAmp | RF amplification supported | BOOLEAN |  |
| TSPC_SDCCHOnly | only SDCCH supported | BOOLEAN |  |
| TSPC_Serv_SS_AoCC | AOCC SS supported | BOOLEAN |  |
| TSPC_Serv_SS_BAIC | BAIC SS supported | BOOLEAN |  |
| TSPC_Serv_SS_BI | BI SS supported | BOOLEAN |  |
| TSPC_Serv_SS_BICRoam | BICRoam SS supported | BOOLEAN |  |
| TSPC_Serv_SS_BAOC | BAOC SS supported | BOOLEAN |  |
| TSPC_Serv_SS_BOIC | BOIC SS supported | BOOLEAN |  |
| TSPC_Serv_SS_BOICexHC | BOICExHC SS supported | BOOLEAN |  |
| TSPC_Serv_SS_CFB | CFB supported | BOOLEAN |  |
| TSPC_Serv_SS_CFNRy | CFNry supported | BOOLEAN |  |
| TSPC_Serv_SS_CFNRc | CFNRc supported | BOOLEAN |  |
| TSPC_Serv_SS_CFU | CFU supported | BOOLEAN |  |
| TSPC_Serv_SS_HOLD | Call Hold SS supported | BOOLEAN |  |
| TSPC_Serv_SS_MPTY | multiparty SS supported | BOOLEAN |  |
| TSPC_Serv_SS_unstruct | USSD supported | BOOLEAN |  |
| TSPC_Serv_TS11 | telephony supported | BOOLEAN |  |
| TSPC_Serv_TS12 | emergency call supported | BOOLEAN |  |
| TSPC_Serv_TS21 | SMS MT/PP supported | BOOLEAN |  |
| TSPC_Serv_TS22 | SMS MO/PP supported | BOOLEAN |  |
| TSPC_Serv_TS23 | SMS cell broadcast supported | BOOLEAN |  |
| TSPC_Serv_TS61 | alternate speech and G3 fax (TS61) supported | BOOLEAN |  |
| TSPC_Serv_TS62 | automatic G3 fax (TS62) supported | BOOLEAN |  |
| TSPC_Serv_BS21 | data circuit duplex async 300 bit/s supported | BOOLEAN |  |
| TSPC_Serv_BS22 | data circuit duplex async 1 200 bit/s supported | BOOLEAN |  |
| TSPC_Serv_BS23 | data circuit duplex async 1 200/75 bit/s supported | BOOLEAN |  |
| TSPC_Serv_BS24 | data circuit duplex async 2 400 bit/s supported | BOOLEAN |  |
| TSPC_Serv_BS25 | data circuit duplex async 4 800 bit/s supported | BOOLEAN |  |
| TSPC_Serv_BS26 | data circuit duplex async 9 600 bit/s supported | BOOLEAN |  |
| TSPC_Serv_BS31 | data circuit duplex sync 1 200 bit/s supported | BOOLEAN |  |
| TSPC_Serv_BS32 | data circuit duplex sync 2 400 bit/s supported | BOOLEAN |  |
| TSPC_Serv_BS33 | data circuit duplex sync 4 800 bit/s supported | BOOLEAN |  |
| TSPC_Serv_BS34 | data circuit duplex sync 9 600 bit/s supported | BOOLEAN |  |
| TSPC_Serv_BS41 | PAD access 300 bit/s supported | BOOLEAN |  |
| TSPC_Serv_BS42 | PAD access 1 200 bit/s supported | BOOLEAN |  |
| TSPC_Serv_BS43 | PAD access 1 200/75 bit/s supported | BOOLEAN |  |
| TSPC_Serv_BS44 | PAD access 2 400 bit/s supported | BOOLEAN |  |
| TSPC_Serv_BS45 | PAD access 4 800 bit/s supported | BOOLEAN |  |
| TSPC_Serv_BS46 | PAD access 9 600 bit/s supported | BOOLEAN |  |
| TSPC_Serv_BS51 | packet access 2 400 bit/s supported | BOOLEAN |  |
| TSPC_Serv_BS52 | packet access 4 800 bit/s supported | BOOLEAN |  |
| TSPC_Serv_BS53 | packet access 9 600 bit/s supported | BOOLEAN |  |
| TSPC_Serv_BS61 | alternate speech/data (BS61) supported | BOOLEAN |  |
| TSPC_Serv_BS81 | speech followed data (BS81) supported | BOOLEAN |  |
| TSPC_SMSStatusRepCap | SMS status report capabilities supported | BOOLEAN |  |
| TSPC_StoreRcvSMSME | Storage of received SMS in ME supported | BOOLEAN |  |
| TSPC_StoreRcvSMSSIM | Storage of received SMS in SIM supported | BOOLEAN |  |
| TSPC_SIMRmv | SIM removable without power down supported | BOOLEAN |  |
| TSPC_SwitchOnOff | switch on/off supported | BOOLEAN |  |
| TSPC_TranspDataOnly | only transparent data service supported | BOOLEAN |  |
| TSPX_TE_stopbit | TE conf. number stop bits. default value : '0'B, 1 bit | BITSTRING[1] |  |
| TSPX_TE_databit | TE conf. number data bits. default value : '1'B, 8 bits | BITSTRING[1] |  |
| TSPX_TE_FLCT | TE flow control. default : 0, outband flow control | 0, 1, 2 |  |
| TSPX_TE_parity | TE conf. parity. default : '001'B, no parity | BITSTRING[3] |  |
| TSPX_BS_21_itc1 | itc value supported for BS21. default value : '001'B, UDI | BITSTRING[3] |  |
| TSPX_BS_21_more_itc | more than one itc value supported by BS21. default value : FALSE, only one itc | BOOLEAN |  |
| TSPX_BS_21_T_NT | both T and NT supported by BS21. default value : FALSE, not support both transparent and non transparent modes | BOOLEAN |  |
| TSPX_BS_21_ce | ce value used in MO call and if the answer to TSPX_BS_21_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '00'B, transparent mode | BITSTRING[2] |  |
| TSPX_BS_21_sacp | sacp value used in MO call. default value : '001'B, I440/450 | BITSTRING[3] |  |
| TSPX_BS_21_itc2 | other itc value supported by BS21, it shall be differenet from the previous one if the answer to TSPX_BS_21_more_itc is TRUE. default value :'001'B, UDI | BITSTRING[3] |  |
| TSPX_BS_22_itc1 | itc value supported by BS22. default value : '010'B, 3.1kHz audio, exPLMN | BITSTRING[3] |  |
| TSPX_BS_22_more_itc | more than one itc value supported by BS22. default value : FALSE, only one itc | BOOLEAN |  |
| TSPX_BS_22_T_NT | both T and NT supported by BS22. default value : FALSE, not support both transparent and non transparent modes | BOOLEAN |  |
| TSPX_BS_22_ce | ce value used in MO call and if the answer to TSPX_BS_22_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '01'B, non transparent | BITSTRING[2] |  |
| TSPX_BS_22_sacp | sacp value used in MO call. default value : '001'B, I440/450 | BITSTRING[3] |  |
| TSPX_BS_22_itc2 | other itc value supported by BS22, it shall be differenet from the previous one if the answer to TSPX_BS_22_more_itc is TRUE. default value : '010'B, 3.1kHz audio, exPLMN | BITSTRING[3] |  |
| TSPX_BS_23_itc | itc value supported by BS23. default value : '010'B, 3.1kHz audio, exPLMN | BITSTRING[3] |  |
| TSPX_BS_23_T_NT | both T and NT supported for BS23. default value : FALSE, not support both transparent and non transparent modes | BOOLEAN |  |
| TSPX_BS_23_ce | ce value used in MO call. default value : '00'B, transparent | BITSTRING[2] |  |
| TSPX_BS_23_sacp | sacp value used in MO call. default value : '001'B, I440/450 | BITSTRING[3] |  |
| TSPX_BS_24_itc1 | itc value supported by BS24. default value : '010'B, 3.1kHz audio, exPLMN | BITSTRING[3] |  |
| TSPX_BS_24_more_itc | more than one itc value supported by BS24. default value : FALSE, only one itc | BOOLEAN |  |
| TSPX_BS_24_T_NT | both T and NT supported by BS24. default value : FALSE, not support both transparent and non transparent modes | BOOLEAN |  |
| TSPX_BS_24_ce | ce value used in MO call and if the answer to TSPX_BS_24_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '00'B, transparent | BITSTRING[2] |  |
| TSPX_BS_24_sacp | sacp value used in MO call. default value : '001'B, I440/450 | BITSTRING[3] |  |
| TSPX_BS_24_itc2 | other itc value supported by BS24, it shall be differenet from the previous one if the answer to TSPX_BS_24_more_itc is TRUE. default value : '010'B, 3.1kHz audio, exPLMN | BITSTRING[3] |  |
| TSPX_BS_25_itc1 | itc value supported by BS25. default value : '001'B, UDI | BITSTRING[3] |  |
| TSPX_BS_25_more_itc | more than one itc value supported by BS25. default value : FALSE, only one itc | BOOLEAN |  |
| TSPX_BS_25_T_NT | both T and NT supported by BS25. default value : FALSE, not support both transparent and non transparent modes | BOOLEAN |  |
| TSPX_BS_25_ce | ce value used in MO call and if the answer to TSPX_BS_25_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '00'B, transparent | BITSTRING[2] |  |
| TSPX_BS_25_sacp | sacp value used in MO call. default value : '001'B, I440/450 | BITSTRING[3] |  |
| TSPX_BS_25_itc2 | other itc value supported by BS25, it shall be differenet from the previous one if the answer to TSPX_BS_25_more_itc is TRUE. default value : '001'B, UDI | BITSTRING[3] |  |
| TSPX_BS_26_itc1 | itc value supported by BS26. default value : '010'B, 3.1kHz audio, exPLMN | BITSTRING[3] |  |
| TSPX_BS_26_more_itc | more than one itc value supported by BS26. default value : FALSE, only one itc | BOOLEAN |  |
| TSPX_BS_26_T_NT | both T and NT supported by BS26. default value : FALSE, not support both transparent and non transparent modes | BOOLEAN |  |
| TSPX_BS_26_ce | ce value used in MO call and if the answer to TSPX_BS_26_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '00'B, transparent | BITSTRING[2] |  |
| TSPX_BS_26_sacp | sacp value used in MO call. default value : '001'B, I440/450 | BITSTRING[3] |  |
| TSPX_BS_26_itc2 | other itc value supported by BS26, it shall be differenet from the previous one if the answer to TSPX_BS_26_more_itc is TRUE. default value : '010'B, 3.1kHz audio, exPLMN | BITSTRING[3] |  |
| TSPX_BS_31_more_itc | more than one itc value supported by BS31. default value : FALSE, only one itc | BOOLEAN |  |
| TSPX_BS_31_more_sacp | more than one sacp value supported by BS31. default value : FALSE, only one sacp | BOOLEAN |  |
| TSPX_BS_31_itc1 | itc value supported by BS31. default value : '001'B, UDI | BITSTRING[3] |  |
| TSPX_BS_31_sacp1 | sacp value supported by BS31. default value : '001'B, I440/450 | BITSTRING[3] |  |
| TSPX_BS_31_itc2 | other itc value supported by BS31, it shall be differenet from the previous one if the answer to TSPX_BS_31_more_itc is TRUE. default value : '001'B, UDI | BITSTRING[3] |  |
| TSPX_BS_31_sacp2 | other sacp value supported by BS31, it shall be differenet from the previous one if the answer to TSPX_BS_31_more_sacp is TRUE. default value : '001'B, I440/450 | BITSTRING[3] |  |
| TSPX_BS_32_itc1 | itc value supported by BS32. default value : '010'B, 3.1kHz audio, exPLMN | BITSTRING[3] |  |
| TSPX_BS_32_more_itc | more than one itc value supported by BS32. default value : FALSE, only one itc | BOOLEAN |  |
| TSPX_BS_32_more_sacp | more than one sacp value supported by BS32. default value : FALSE, only one sacp | BOOLEAN |  |
| TSPX_BS_32_sacp1 | sacp value supported by BS32. default value : '110'B, X.32 | BITSTRING[3] |  |
| TSPX_BS_32_X32_T_NT | BS 32 both T and NT supported for X32 3.1kHz. default value : FALSE, not support both transparent and non transparent modes | BOOLEAN |  |
| TSPX_BS_32_X32_ce | if the answer to TSPX_BS_32_X32_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '01'B, non transparent | BITSTRING[2] |  |
| TSPX_BS_32_itc2 | other itc value supported by BS32, it shall be differenet from the previous one if the answer to TSPX_BS_32_more_itc is TRUE. default value : '010'B, 3.1kHz audio, exPLMN | BITSTRING[3] |  |
| TSPX_BS_32_sacp2 | other sacp value supported by BS32, it shall be differenet from the previous one if the answer to TSPX_BS_32_more_sacp is TRUE. default value : '110'B, X.32 | BITSTRING[3] |  |
| TSPX_BS_33_itc1 | itc value supported by BS33. default value : '001'B, UDI | BITSTRING[3] |  |
| TSPX_BS_33_more_itc | more than one itc value supported by BS33. default value : FALSE, only one itc | BOOLEAN |  |
| TSPX_BS_33_more_sacp | more than one sacp value supported by BS33. default value : FALSE, only one sacp | BOOLEAN |  |
| TSPX_BS_33_sacp1 | sacp value supported by BS33. default value : '010'B, X.21 | BITSTRING[3] |  |
| TSPX_BS_33_X32_T_NT | BS 33 both T and NT supported for X32 3.1kHz. default value : FALSE, not support both transparent and non transparent modes | BOOLEAN |  |
| TSPX_BS_33_X32_ce | if the answer to TSPX_BS_33_X32_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '00'B, transparent | BITSTRING[2] |  |
| TSPX_BS_33_itc2 | other itc value supported by BS32, it shall be differenet from the previous one if the answer to TSPX_BS_33_more_itc is TRUE. default value : '001'B, UDI | BITSTRING[3] |  |
| TSPX_BS_33_sacp2 | other sacp value supported by BS33, it shall be differenet from the previous one if the answer to TSPX_BS_33_more_sacp is TRUE. default value : '010'B, X.21 | BITSTRING[3] |  |
| TSPX_BS_34_itc1 | itc value supported by BS34. default value : '010'B, 3.1kHz audio, exPLMN | BITSTRING[3] |  |
| TSPX_BS_34_more_itc | more than one itc value supported by BS34. default value : FALSE, only one itc | BOOLEAN |  |
| TSPX_BS_34_more_sacp | more than one sacp value supported by BS34. default value : FALSE, only one sacp | BOOLEAN |  |
| TSPX_BS_34_sacp1 | sacp value supported by BS34. default value : '001'B, I440/450 | BITSTRING[3] |  |
| TSPX_BS_34_X32_T_NT | BS 34 both T and NT supported for X32 3.1kHz. default value : FALSE, not support both transparent and non transparent modes | BOOLEAN |  |
| TSPX_BS_34_X32_ce | if the answer to TSPX_BS_34_X32_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '00'B, transparent | BITSTRING[2] |  |
| TSPX_BS_34_itc2 | other itc value supported by BS34, it shall be differenet from the previous one if the answer to TSPX_BS_34_more_itc is TRUE. default value : '010'B, 3.1kHz audio, exPLMN | BITSTRING[3] |  |
| TSPX_BS_34_sacp2 | other sacp value supported by BS34, it shall be differenet from the previous one if the answer to TSPX_BS_34_more_sacp is TRUE. default value : '001'B, I440/450 | BITSTRING[3] |  |
| TSPX_BS_41_T_NT | BS41 both T and NT supported . default value : FALSE, not support both | BOOLEAN |  |
| TSPX_BS_41_ce | ce value used in BS41 MO call. default value : '00'B, transparent | BITSTRING[2] |  |
| TSPX_BS_42_T_NT | BS42 both T and NT supported . default value : FALSE, not support both transparent and non transparent modes | BOOLEAN |  |
| TSPX_BS_42_ce | ce value used in BS42 MO call. default value : '010'B, non transparent | BITSTRING[2] |  |
| TSPX_BS_43_T_NT | BS43 both T and NT supported . default value : FALSE, not support both transparent and non transparent modes | BOOLEAN |  |
| TSPX_BS_43_ce | ce value used in BS43 MO call. default value : '00'B, transparent | BITSTRING[2] |  |
| TSPX_BS_44_T_NT | BS44 both T and NT supported . default value : FALSE, not support both transparent and non transparent modes | BOOLEAN |  |
| TSPX_BS_44_ce | ce value used in BS44 MO call. default value : '00'B, transparent | BITSTRING[2] |  |
| TSPX_BS_45_T_NT | BS45 both T and NT supported . default value : FALSE, not support both transparent and non transparent modes | BOOLEAN |  |
| TSPX_BS_45_ce | ce value used in BS45 MO call. default value : '01'B, non transparent | BITSTRING[2] |  |
| TSPX_BS_46_T_NT | BS46 both T and NT supported . default value : FALSE, not support both transparent and non transparent modes | BOOLEAN |  |
| TSPX_BS_46_ce | ce value used in BS46 MO call. default value : '00'B, transparent | BITSTRING[2] |  |
| TSPX_BS_61_S | synchronous data supported by BS 61 . default value : TRUE, synchronous mode | BOOLEAN |  |
| TSPX_BS_61_S_more_ur | more than one user rate for synchronous data service supported by BS61. default value : FALSE, only ine user rate | BOOLEAN |  |
| TSPX_BS_61_S_ur1 | usrer rate value supported for synchronous data service of BS61. default value : '0100'B, 4.8 kbit/s | BITSTRING[4] |  |
| TSPX_BS_61_S_ur2 | another usrer rate value supported for synchronous data service of BS61. it shall be different from the previous one if the answer to TSPX_BS_61_S_more_ur is TRUE. default value : '0100'B, 4.8 kbit/s | BITSTRING[4] |  |
| TSPX_BS_61_A | asynchronous data service supported by BS61 . default value : TRUE, asynchronous | BOOLEAN |  |
| TSPX_BS_61_A_ur1 | usrer rate value supported for asynchronous data service of BS61. default value : '0101'B, 9.6 kbit/s | BITSTRING[4] |  |
| TSPX_BS_61_A_ur1_T_NT | both T and NT supported for the rate TSPX_BS_61_A_ur1 of BS61. default value : FALSE, not support both transparent and non transparent modes | BOOLEAN |  |
| TSPX_BS_61_A_ur1_ce | if the answer to TSPX_BS_61_A_ur1_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '00'B, transparent | BITSTRING[2] |  |
| TSPX_BS_61_A_more_ur | more than one user rate for asynchronous data service supported by BS61. default value : FALSE, only one user rate | BOOLEAN |  |
| TSPX_BS_61_A_ur2 | another usrer rate value supported for asynchronous data service of BS61. it shall be different from the previous one if the answer to TSPX_BS_61_A_more_ur is TRUE. default value : '0101'B, 9.6 kbit/s | BITSTRING[4] |  |
| TSPX_BS_61_A_ur2_T_NT | both T and NT supported for the rate TSPX_BS_61_A_ur2 of BS61. default value : FALSE, not support both transparent and non transparent modes | BOOLEAN |  |
| TSPX_BS_81_S | synchronous data supported by BS 81. default value : TRUE, synchronous mode | BOOLEAN |  |
| TSPX_BS_81_S_more_ur | more than one user rate for synchronous data service supported by BS81. default value : FALSE, only one user rate | BOOLEAN |  |
| TSPX_BS_81_S_ur1 | usrer rate value supported for synchronous data service of BS81. default value : '0101'B, 9.6 kbit/s | BITSTRING[4] |  |
| TSPX_BS_81_S_ur2 | another usrer rate value supported for synchronous data service of BS81. it shall be different from the previous one if the answer to TSPX_BS_81_S_more_ur is TRUE. default value : '0101'B, 9.6 kbit/s | BITSTRING[4] |  |
| TSPX_BS_81_A | asynchronous data service supported by BS81. default value : TRUE, asynchronous mode | BOOLEAN |  |
| TSPX_BS_81_A_ur1 | usrer rate value supported for asynchronous data service of BS81. default value : '0100'B, 4.8 kbit/s | BITSTRING[4] |  |
| TSPX_BS_81_A_ur1_T_NT | both T and NT supported for the rate TSPX_BS_81_A_ur1 of BS81. default value : FALSE, not support both transparent and non transparent modes | BOOLEAN |  |
| TSPX_BS_81_A_ur1_ce | if the answer to TSPX_BS_81_A_ur1_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '01'B, non transparent | BITSTRING[2] |  |
| TSPX_BS_81_A_more_ur | more than one user rate for asynchronous data service supported by BS81. default value : FALSE, only one user rate | BOOLEAN |  |
| TSPX_BS_81_A_ur2 | another usrer rate value supported for asynchronous data service of BS81. it shall be different from the previous one if the answer to TSPX_BS_81_A_more_ur is TRUE. default value : '0100'B, 4.8 kbit/s | BITSTRING[4] |  |
| TSPX_BS_81_A_ur2_T_NT | both T and NT supported for the rate TSPX_BS_81_A_ur2 of BS81. default value : FALSE, not support both transparent and non transparent modes | BOOLEAN |  |
| TSPX_TS_61_T_NT | both T and NT supported for TS61(fax). default value : FALSE, not support both transparent and non transparent modes | BOOLEAN |  |
| TSPX_TS_61_ce | if the answer to TSPX_TS_61_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '01'B, non transparent | BITSTRING[2] |  |
| TSPX_TS_61_ur1 | user rate for TS61(fax). default value : '0101'B, 9.6 kbit/s | BITSTRING[4] |  |
| TSPX_TS_61_more_ur | more than one user rate supported for TS61. default value : FALSE, only one user rate | BOOLEAN |  |
| TSPX_TS_61_ur2 | another user rate for TS61, it shall be different from the previous one if the answer to TSPX_TS_61_more_ur is TRUE. default value : '0101'B, 9.6 kbit/s | BITSTRING[4] |  |
| TSPX_TS_62_T_NT | both T and NT supported for TS62. default value : FALSE, not support both transparent and non transparent modes | BOOLEAN |  |
| TSPX_TS_62_ce | if the answer to TSPX_TS_62_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '00'B, transparent | BITSTRING[2] |  |
| TSPX_TS_62_ur1 | user rate for TS62. default value : '0100'B, 4.8 kbit/s | BITSTRING[4] |  |
| TSPX_TS_62_more_ur | more than one user rate supported by TS62. default value : FALSE, only one user rate | BOOLEAN |  |
| TSPX_TS_62_ur2 | another user rate for TS62, it shall be different from the previous one if the answer to TSPX_TS_62_more_ur is TRUE. default value : '0100'B, 4.8 kbit/s | BITSTRING[4] |  |
| TSPX_DTMF | call control capabilities: value '1'B means MS supports DTMF | BITSTRING[1] |  |
| TSPX_modF | any non signalling full rate channel mode for TC_26_6_4_1 | BITSTRING[8] |  |
| TSPX_modH | any non signalling half channel mode for TC_26_6_4_1 | BITSTRING[8] |  |
| TSPX_anymod | channel mode for TC_62_10_2_4_2. Any supported value except signalling and Full rate speech. possible values are '00000011'B' 12k data, '00001011'B 6k data, '0010011'B 3.6k data | BITSTRING[8] |  |
| TSPX_CKSNA | cipher key sequence number | BITSTRING[3] |  |
| TSPX_CKSNB | cipher key sequence number | BITSTRING[3] |  |
| TSPX_CKSNC | cipher key sequence number | BITSTRING[3] |  |
| TSPX_CKSNDef | default cipher key sequence number | BITSTRING[3] |  |
| TSPX_RfPwrCap | RF power capability | BITSTRING[3] |  |
| TSPX_RevLevel | Revision level of classmark 1 | BITSTRING[2] |  |
| TSPX_CiphAlgA5_1 | A5/1 algorithm: '1'B : available, '0'B: not available | BITSTRING[1] |  |
| TSPX_CiphAlgA5_2 | A5/2 algorithm: '1'B : available, '0'B: not available | BITSTRING[1] |  |
| TSPX_CiphAlgA5_3 | A5/3 algorithm: '1'B : available, '0'B: not available | BITSTRING[1] |  |
| TSPX_CiphAlgA5_4 | A5/4 algorithm: '1'B : available, '0'B: not available | BITSTRING[1] |  |
| TSPX_CiphAlgA5_5 | A5/5 algorithm: '1'B : available, '0'B: not available | BITSTRING[1] |  |
| TSPX_CiphAlgA5_6 | A5/6 algorithm: '1'B : available, '0'B: not available | BITSTRING[1] |  |
| TSPX_CiphAlgA5_7 | A5/7 algorithm: '1'B : available, '0'B: not available | BITSTRING[1] |  |
| TSPX_pSyncCap | pseudo synchronisation capability, '0'B: not present '1'B: present | BITSTRING[1] |  |
| TSPX_SSscrnInd | SS screen indicator | BITSTRING[2] |  |
| TSPX_SMCap | short message capability, '0'B: not present '1'B: present | BITSTRING[1] |  |
| TSPX_frqCap | frequency capability. DCS: reserved, GSM: '0'B not support extension band, '1'B support extension band | BITSTRING[1] |  |
| TSPX_ClsMk3 | classmark 3 indicator, '0'B no addition capability info, '1'B additional capability info in class mark 3 | BITSTRING[1] |  |
| TSPX_RfPwrCapAmp | RF power capability with external RF amplifier | BITSTRING[3] |  |
| TSPX_DTMFInd | DTMF indication to user supported | BOOLEAN |  |
| TSPX_IMEI | IMEI of the MS. | HEXSTRING |  |
| TSPX_IMEISV | IMEISV of the MS. Used in: TC_26_6_8_5, TC_26_7_3_1 | HEXSTRING |  |
| TSPX_TC1M | timer value for GSM timer TC1M (for SMS) | INTEGER |  |
| TSPX_MaxCPDataRetx | max. number of CP data retransmissions for SMS | INTEGER |  |
| TSPX_Telephony_Immconn | Immediate connect for telephony supported ? | BOOLEAN |  |
| TSPX_Uupd | user-user protocol discriminator | BITSTRING[8] |  |
| TSPX_UuInfo | user-user information | OCTETSTRING |  |

## B.7.2	MMI information

How is the Accumulated Call Meter (ACM) on the SIM read ? NOTE: Used in Operation OO_ACMIncCHK and OO_ACMReading.

How is the ACM on the SIM reset to zero, and ACMmax set to 2 units? NOTE: Used in Operation OO_ACMSetting.

How is power amplification added to the mobile and removed? NOTE: Used in Operation OO_AddPwrAmp and OO_RemvPwrAmp.

What alerting indication is given by the mobile? NOTE: Used in Operation OO_AltIndCHK.

How is the called party number displayed on the mobile? NOTE: Used in Operation OO_CalledNumCHK and OO_CalledPtyNumCHK.

How is a call placed on Hold? NOTE: Used in Operation OO_CallHold.

How is the presence of short messages checked in the mobile and how are they displayed? NOTE: Used in Operation OO_CheckAllSMPresentBut4th and OO_CheckMessageDisplayed and OO_DisplaySMAndSendReplySM and OO_ReadSMAndRemove and OO_RecallAndDisplaySM.

How is the presence of Cell Broadcast short messages in the mobile checked? NOTE: Used in Operation OO_CheckCBSMReceived.

How is it checked whether the memory capacity of the SIM has been exceeded? NOTE: Used in Operation OO_CheckMCEFOnSIM and OO_CheckMCEFOnSIMUnset.

How and where is the SIM inserted and removed? NOTE: Used in Operation OO_ConnectSIMSimulator and OO_SIMIns and OO_SIM2Ins and OO_SIM3Ins and OO_SIMRmv.

Does the MS have an 'END' key? If not, what method is used to replace its function? NOTE: Used in Operation OO_DepressEndKey.

How is a number entered in order to make an outgoing call? NOTE: Used in Operation OO_DiallCalledNum.

How is a reply to a received short message sent? NOTE: Used in Operation OO_DisplaySMAndSendReplySM.

How is it checked which DTMF character has been sent? NOTE: Used in Operation OO_DTMFIndCHK.

How is the message store of the MS emptied? NOTE: Used in OO_EmptyMessageStorage.

How is a password on the MS entered? NOTE: Used in Operation OO_EnterPswd.

How is an incoming call answered? NOTE: Used in Operation OO_HookOff.

For each service on the MS which requires the MS to be connected to an interface on other equipment (e.g. a computer), how is that interface set up for the purpose? NOTE: Used in Operation OO_IFsetup.

How is an In-Call modification initiated on the MS? NOTE: Used in Operation OO_InCallMod.

For each mobile originating basic service supported by the MS, how is the MS configured to make an outgoing call on that service? NOTE: Used in Operation OO_InitCall.

How is the MS to configured accept standard MMI sequences for the initiation of supplementary services? NOTE: Used in Operation OO_InitSS

How is the MS checked whether it is in the normal service state (idle, updated). NOTE: Used in Operation OO_InServiceCHK and OO_PressKeyWhenInService.

How is a Multi-Party call initiated on the MS. NOTE: Used in Operation OO_MptyCall

How is the MS configured to store received Class 1 short messages in the mobile equipment (rather than in the SIM)? NOTE: Used in Operation OO_MSSetupStoreClass1SMInMEMemory.

How is the list of available PLMNs checked? NOTE: Used in Operation OO_PLMNsCHK.

How is the PLMN selection mode switched to automatic selection. NOTE: Used in operation OO_PLMNselModeAuto.

How is the PLMN selection mode switched to manual selection. NOTE: Used in operation OO_PLMNselModeMan.

How is the power source for the MS removed and replaced. (Note, this is not the same as switching the Mobile on and off using the on/off switch.) NOTE: Used in operation OO_PowerUp and OO_PowerDown.

How is a short message removed from the message store? NOTE: Used in operation OO_ReadSMAndRemove.

When in manual PLMN selection mode, how is a PLMN selected? NOTE: Used in OperationOO_SelPLMN

How is a short message sent from the MS? NOTE: Used in operation OO_SendMOShortMessage.

How is an SMS COMMAND message sent to delete the last short message sent? NOTE: Used in OO_SendSMSCOMMANDDe.

How is an SMS COMMAND message sent enquiring about the last short message sent? NOTE: Used in OO_SendSMS_COMMANDEnq

How is Call Refusal set on the MS? NOTE: Used in Operation OO_SetRefuseCall.

How is a DTMF digit sent? NOTE: Used in operation OO_ShortKeyDepr.

How is it checked whether a supplementary service result is correct? NOTE: Used in Operation OO_SSresultCHK

How is the MS switched on and off? NOTE: Used in operation OO_SwitchOn and OO_SwitchOff.

How is a call cleared? NOTE: Used in operation OO_TermCall.

## B.7.3	Test house specified parameters

| Item | Description | Type/Allowed values | Value chosen |
| --- | --- | --- | --- |
| TSPX_AltNb | use alternative neighbour cells description. default : FALSE | BOOLEAN |  |
| TSPX_CphAlgA | any supported ciphering algorithm | OCTETSTRING [1] |  |
| TSPX_CphAlgB | ciphering algorithm supported | OCTETSTRING [1] |  |
| TSPX_CphAlgC | ciphering algorithm supported | OCTETSTRING [1] |  |
| TSPX_CphAlgD | ciphering algorithm supported | OCTETSTRING [1] |  |
| TSPX_CphAlgE | ciphering algorithm supported | OCTETSTRING [1] |  |
| TSPX_CphAlgDef | default ciphering algorithm | OCTETSTRING [1] |  |
| TSPX_NoOfHoAccA | number of handover access bursts, value range 10 - 20 | INTEGER |  |
| TSPX_NoOfHoAccB | number of handover access bursts, value range 10 - 20 | INTEGER |  |
| TSPX_NoOfHoAccC | number of handover access bursts, value range 10 - 20 | INTEGER |  |
| TSPX_NoOfHoAccD | number of handover access bursts, value range 5 - 10 | INTEGER |  |
| TSPX_NoOfHoAccE | number of handover access bursts, value range 5 - 10 | INTEGER |  |
| TSPX_NoOfHoAccF | number of handover access bursts, value range 5 - 10 | INTEGER |  |
| TSPX_NoOfHoAccG | number of handover access bursts, value range 2 - 5 | INTEGER |  |
| TSPX_NoOfHoAccH | number of handover access bursts, value range 2 - 5 | INTEGER |  |
| TSPX_NoOfHoAccI | number of handover access bursts, value range 2 - 5 | INTEGER |  |
| TSPX_HoRefA | Hand over reference, GSM 04.08, 10.5.2.15 | BITSTRING [8] |  |
| TSPX_HoRefB | Hand over reference, GSM 04.08, 10.5.2.15 | BITSTRING [8] |  |
| TSPX_HoRefC | Hand over reference, GSM 04.08, 10.5.2.15 | BITSTRING [8] |  |
| TSPX_HoRefD | Hand over reference, GSM 04.08, 10.5.2.15 | BITSTRING [8] |  |
| TSPX_HoRefE | Hand over reference, GSM 04.08, 10.5.2.15 | BITSTRING [8] |  |
| TSPX_HoRefF | Hand over reference, GSM 04.08, 10.5.2.15 | BITSTRING [8] |  |
| TSPX_HoRefG | Hand over reference, GSM 04.08, 10.5.2.15 | BITSTRING [8] |  |
| TSPX_HoRefH | Hand over reference, GSM 04.08, 10.5.2.15 | BITSTRING [8] |  |
| TSPX_HoRefI | Hand over reference, GSM 04.08, 10.5.2.15 | BITSTRING [8] |  |
| TSPX_HoRefJ | Hand over reference, GSM 04.08, 10.5.2.15 | BITSTRING [8] |  |
| TSPX_HSN | Hopping sequence number value range: 0 - 63 | INTEGER |  |
| TSPX_IMSI | IMSI of the MS | HEXSTRING |  |
| TSPX_Ki | default authentication key used in testing | BITSTRING |  |
| TSPX_MAIO | mobile allocation index offset, value range: 0 - 63 | INTEGER |  |
| TSPX_MaxRetrans | Max-Retrans | INTEGER |  |
| TSPX_MSTxpwrMax | maximum output power from MS | 0..31 |  |
| TSPX_PwrlvlA | power level, value between 2 to 15 | INTEGER |  |
| TSPX_PwrlvlB | power level, value between 2 to 15 | INTEGER |  |
| TSPX_PwrlvlC | power level, value between 2 to 15 | INTEGER |  |
| TSPX_PwrlvlD | power level, value between 2 to 15 | INTEGER |  |
| TSPX_RANDA | challenge RAND | BITSTRING [128] |  |
| TSPX_RANDB | challenge RAND | BITSTRING [128] |  |
| TSPX_RANDC | challenge RAND | BITSTRING [128] |  |
| TSPX_RANDDef | default challenge RAND | BITSTRING [128] |  |
| TSPX_SDCCH4SubA | TDMA offset of SDCCH/4 subchannel | BITSTRING[2] |  |
| TSPX_SDCCH4SubB | TDMA offset of SDCCH/4 subchannel | BITSTRING[2] |  |
| TSPX_SDCCH4SubC | TDMA offset of SDCCH/4 subchannel | BITSTRING[2] |  |
| TSPX_SDCCH4SubDef | TDMA offset of default SDCCH/4 subchannel | BITSTRING[2] |  |
| TSPX_SDCCH8SubA | TDMA offset of SDCCH/8 subchannel | BITSTRING[3] |  |
| TSPX_SDCCH8SubB | TDMA offset of SDCCH/8 subchannel | BITSTRING[3] |  |
| TSPX_SDCCH8SubC | TDMA offset of SDCCH/8 subchannel | BITSTRING[3] |  |
| TSPX_SDCCH8SubD | TDMA offset of SDCCH/8 subchannel | BITSTRING[3] |  |
| TSPX_SDCCH8SubE | TDMA offset of SDCCH/8 subchannel | BITSTRING[3] |  |
| TSPX_SDCCH8SubF | TDMA offset of SDCCH/8 subchannel | BITSTRING[3] |  |
| TSPX_SDCCH8SubG | TDMA offset of SDCCH/8 subchannel | BITSTRING[3] |  |
| TSPX_SDCCH8SubDef | TDMA offset of default SDCCH/8 subchannel | BITSTRING[3] |  |
| TSPX_TimadvA | timing advance | INTEGER |  |
| TSPX_TimadvB | timing advance | INTEGER |  |
| TSPX_TimadvC | timing advance | INTEGER |  |
| TSPX_TmSltA | time slot | BITSTRING [3] |  |
| TSPX_TmSltB | time slot | BITSTRING [3] |  |
| TSPX_TmSltC | time slot | BITSTRING [3] |  |
| TSPX_TmSltD | time slot | BITSTRING [3] |  |
| TSPX_TmSltE | time slot | BITSTRING [3] |  |
| TSPX_TmSltF | time slot | BITSTRING [3] |  |
| TSPX_TmSltG | time slot | BITSTRING [3] |  |
| TSPX_TmSltDef | default time slot | BITSTRING [3] |  |
| TSPX_TmSltNotZero | time slot, arbitrarily value, but not zero. | BITSTRING [3] |  |
| TSPX_TmSltNotZero1 | time slot, arbitrarily value, but not zero and not TSPX_TmSltNotZero | BITSTRING [3] |  |
| TSPX_Txint | Tx-Integer | INTEGER |  |
| TSPX_TscA | training sequence code | BITSTRING [3] |  |
| TSPX_TscB | training sequence code | BITSTRING [3] |  |
| TSPX_TscC | training sequence code | BITSTRING [3] |  |
| TSPX_TscD | training sequence code | BITSTRING [3] |  |
| TSPX_TscE | training sequence code | BITSTRING [3] |  |
| TSPX_TscF | training sequence code | BITSTRING [3] |  |
| TSPX_TscG | training sequence code | BITSTRING [3] |  |
| TSPX_TscDef | default TSC | BITSTRING [3] |  |
| TSPX_T3122 | value of timer T3122, HEXSTRING[2] | HEXSTRING [2] |  |
| TSPX_TCHcarrierA_ho | the value can be choosen arbitrarily from cell allocation of cell A (GSM), but not BCCH carrier .possible value is : 10, 17, 26, 34, 42, 45, 46, 52, 59, 66, 73, 74, 75, 76, 108, 114 | INTEGER |  |
| TSPX_TCHcarrierA_hod | the value can be choosen arbitrarily from cell allocation of cell A (DCS), but not BCCH carrier . possible value is : 734,741,754,759,762,766,767,773,775,779,782,791,798,829,832,844 | INTEGER |  |
| TSPX_TCHcarrierB | TCH and SDCCH channel frequency number of cell B (GSM) | INTEGER |  |
| TSPX_TCHcarrierBd | TCH and SDCCH channel frequency number of cell B (DCS) | INTEGER |  |
| TSPX_TCHcarrierB_ho | the value can be choosen arbitrarily from cell allocation of cell B (GSM), but not BCCH carrier . possibles value is : 14,18,22,24,30,31,38,60,66,73,74,75,76,108,114 | INTEGER |  |
| TSPX_TCHcarrierB_hod | not BCCH carrier of cell B. the value can be choosen arbitrarily from cell allocation of cell B (DCS), which is (739,743,746,749,756,758,761,771,779,782,791,798,829,832,844) | INTEGER |  |
| TSPX_TCHcarrierB2_ho | Choosen arbitrarly from cell allocation B for GSM HO cases, but not BCCH carrier! | INTEGER |  |
| TSPX_TCHcarrierB2_hod | Choosen arbitrarly from cell allocation B for DCS HO cases, but not BCCH carrier! | INTEGER |  |
| TSPX_TCHHSubA | TDMA offset of half rate subchannel | BITSTRING[1] |  |
| TSPX_TCHHSubDef | TDMA offset of default half rate subchannel | BITSTRING[1] |  |
| TSPX_TMSI | TMSI of the MS used in test | OCTETSTRING[4] |  |
| TSPX_TMSI1 | another TMSI used in test which shall differ from TSPX_TMSI, TSPX_TMSI + '01'O TSPX_TMSI + '02'O TSPX_TMSI + '03'O | OCTETSTRING[4] |  |
| TSPX_k | timing of cell A before cell B k bit periods for TC_26_6_5_4, TC_26_6_5_3. | INTEGER |  |
| TSPX_y | timing advance for TC_26_6_5_4, TC_26_6_5_3. the values of TSPX_k and TSPX_y shall be 0 < (2*TSPX_k + TSPX_y) MOD 256 < 60. | INTEGER |  |
| TSPX_k1 | timing of cell A before cell B k bit periods for TC_26_6_5_5 | INTEGER |  |
| TSPX_k2 | timing of cell A before cell B k bit periods for TC_26_6_5_6 | INTEGER |  |
| TSPX_y2 | timing advance for TC_26_6_5_6, value range : 11 - 62. | INTEGER |  |
| TSPX_k3 | timing of cell A before cell B k bit periods for TC_26_6_5_7 | INTEGER |  |
| TSPX_y3 | timing advance for TC_26_6_5_7 | INTEGER |  |
| TSPX_nPara | the n'th ChReq for TC_26_6_1_2, shall be choose to [1..8], MAXRETRANS for TC_26_6_1_2 is 7 (See 11.10 for more explaination). | INTEGER |  |
| TSPX_kPara | the k'th ChReq for TC_26_6_1_2 | INTEGER |  |
| TSPX_rPara | the r'th ChReq for TC_26_6_1_2 | INTEGER |  |
| TSPX_i1Para | for TC_26_1_2 | INTEGER |  |
| TSPX_i2Para | for TC_26_1_2 | INTEGER |  |
| TSPX_i3Para | the r'th ChReq for TC_26_6_1_2 | INTEGER |  |
| TSPX_n1Para | the n'th ChReq for TC_26_6_1_3 | INTEGER |  |
| TSPX_i4Para | for TC_26_6_1_3 | INTEGER |  |
| TSPX_xPara | t3122 for TC_26_6_1_3 | INTEGER |  |
| TSPX_AGBLKS1 | BS-AG-BLKS-RES for TC_26_6_2_3_1, TC_26_6_2_1_1 | INTEGER |  |
| TSPX_PAMFRMS1 | BS-PA-MFRMS for TC_26_6_2_3_1 (shall not be set to 9), TC_26_6_2_1_1 | INTEGER |  |
| TSPX_CcchConf1 | paging subchannel for TC_26_6_2_3_1, TC_26_6_2_1_1 | BITSTRING[3] |  |
| TSPX_PgSubch | paging subchannel for TC_26_6_2_3_1 | INTEGER |  |
| TSPX_AGBLKS2 | BS-AG-BLKS-RES for TC_26_6_2_3_2, TC_26_6_2_1_2 | INTEGER |  |
| TSPX_PAMFRMS2 | BS-PA-MFRMS for TC_26_6_2_3_2, TC_26_6_2_1_2 | INTEGER |  |
| TSPX_CcchConf2 | paging subchannel for TC_26_6_2_3_2, TC_26_6_2_1_2 | BITSTRING[3] |  |
| TSPX_AGBLKS3 | BS-AG-BLKS-RES for TC_26_6_2_4, TC_26_6_2_2, TC_26_6_2_1_3 | INTEGER |  |
| TSPX_PAMFRMS3 | BS-PA-MFRMS for TC_26_6_2_4, TC_26_6_2_2, TC_26_6_2_1_3 | INTEGER |  |
| TSPX_CcchConf3 | paging subchannel for TC_26_6_2_4, TC_26_6_2_2, TC_26_6_2_1_3 | BITSTRING[3] |  |
| TSPX_AGBLKS4 | BS-AG-BLKS-RES for TC_26_6_2_5 | INTEGER |  |
| TSPX_PAMFRMS4 | BS-PA-MFRMS for TC_26_6_2_5 | INTEGER |  |
| TSPX_CcchConf4 | paging subchannel for TC_26_6_2_5 (shall be in the set ('010', '100', '110')) | BITSTRING[3] |  |
| TSPX_Chtp1 | channel type for TC_26_6_13_1, any value supported by the MS | BITSTRING[5] |  |
| TSPX_ChMod1 | channel mode for TC_26_6_13_1, any value for the channel type of TSPX_Chtp1 | BITSTRING[8] |  |
| TSPX_Tm1 | a value to calculate the starting time for TC_26_6_13_1, value between 60 -100 | INTEGER |  |
| TSPX_Maio1 | hopping parameter for TC_26_6_13_1, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma1, default : 2 | INTEGER |  |
| TSPX_Ma1 | mobile allocation for TC_26_6_13_1, its value shall indicate number of frequencies between 1 and 17. default : '010101'O | OCTETSTRING[3] |  |
| TSPX_Hsn1 | hopping parameter for TC_26_6_13_1. default: 1 | INTEGER |  |
| TSPX_Maio2 | hopping parameter for TC_26_6_13_1, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma2, default : 8 | INTEGER |  |
| TSPX_Ma2 | mobile allocation for TC_26_6_13_1, its value shall indicate number of frequencies between 1 and 17. default :'01362A5'O | OCTETSTRING[3] |  |
| TSPX_Hsn2 | hopping parameter for TC_26_6_13_1, default : 6 | INTEGER |  |
| TSPX_Maio3 | hopping parameter for TC_26_6_13_1, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma3, default : 14 | INTEGER |  |
| TSPX_Ma3 | mobile allocation for TC_26_6_13_1, its value shall indicate number of frequencies between 1 and 17. default :'01FFFA'O | OCTETSTRING[3] |  |
| TSPX_Hsn3 | hopping parameter for TC_26_6_13_1, default : 0 | INTEGER |  |
| TSPX_Chtp2 | channel type for TC_26_6_13_2, any value supported by the MS | BITSTRING[5] |  |
| TSPX_ChMod2 | channel mode for TC_26_6_13_2 | BITSTRING[8] |  |
| TSPX_Maio4 | hopping parameter for TC_26_6_13_2, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma4, default : 3 | INTEGER |  |
| TSPX_Ma4 | mobile allocation for TC_26_6_13_2, its value shall indicate number of frequencies between 1 and 17. default : '001141'O | OCTETSTRING[3] |  |
| TSPX_Hsn4 | hopping parameter for TC_26_6_13_2, default : 0 | INTEGER |  |
| TSPX_Maio5 | hopping parameter for TC_26_6_13_2, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma5, default : 9 | INTEGER |  |
| TSPX_Ma5 | mobile allocation for TC_26_6_13_2, its value shall indicate number of frequencies between 1 and 17. default : '01ABCB'O | OCTETSTRING[3] |  |
| TSPX_Hsn5 | hopping parameter for TC_26_6_13_2, default : 16 | INTEGER |  |
| TSPX_Chtp3 | channel type for TC_26_6_13_3, any value supported by the MS | BITSTRING[5] |  |
| TSPX_Maio6 | hopping parameter for TC_26_6_13_3, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma6, default : 4 | INTEGER |  |
| TSPX_Ma6 | mobile allocation for TC_26_6_13_3, its value shall indicate number of frequencies between 1 and 17. default :'006248'O | OCTETSTRING[3] |  |
| TSPX_Hsn6 | hopping parameter for TC_26_6_13_3, default : 4 | INTEGER |  |
| TSPX_Maio7 | hopping parameter for TC_26_6_13_3, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma7, default : 10 | INTEGER |  |
| TSPX_Ma7 | mobile allocation for TC_26_6_13_3, its value shall indicate number of frequencies between 2 and 17. default : '009AFB'O | OCTETSTRING[3] |  |
| TSPX_Chtp4 | channel type for TC_26_6_13_3, any value supported by the MS | BITSTRING[5] |  |
| TSPX_Maio8 | hopping parameter for TC_26_6_13_3, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma8, default : 1 | INTEGER |  |
| TSPX_Ma8 | mobile allocation for TC_26_6_13_3, its value shall indicate number of frequencies between 2 and 17. default : '002800'O | OCTETSTRING[3] |  |
| TSPX_Hsn8 | hopping parameter for TC_26_6_13_3, default : 40 | INTEGER |  |
| TSPX_Maio9 | hopping parameter for TC_26_6_13_3, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma9, default : 15 | INTEGER |  |
| TSPX_Ma9 | mobile allocation for TC_26_6_13_3, its value shall indicate number of frequencies between 2 and 17. default : '01FFFB'O | OCTETSTRING[3] |  |
| TSPX_Hsn9 | hopping parameter for TC_26_6_13_3, default : 8 | INTEGER |  |
| TSPX_Chtp5 | channel type for TC_26_6_13_4 | BITSTRING[5] |  |
| TSPX_Maio10 | hopping parameter for TC_26_6_13_4, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma10, default : 5 | INTEGER |  |
| TSPX_Ma10 | mobile allocation for TC_26_6_13_4, its value shall indicate number of frequencies between 1 and 17. default : '01D082'O | OCTETSTRING[3] |  |
| TSPX_Hsn10 | hopping parameter for TC_26_6_13_4, default: 0 | INTEGER |  |
| TSPX_Maio11 | hopping parameter for TC_26_6_13_4, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma11, default : 11 | INTEGER |  |
| TSPX_Ma11 | mobile allocation for TC_26_6_13_4, its value shall indicate number of frequencies between 2 and 17. default : '00CFF3'O | OCTETSTRING[3] |  |
| TSPX_Chtp6 | channel type for TC_26_6_13_4 | BITSTRING[5] |  |
| TSPX_Maio12 | hopping parameter for TC_26_6_13_4, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma12, default : 1 | INTEGER |  |
| TSPX_Ma12 | mobile allocation for TC_26_6_13_4, its value shall indicate number of frequencies between 1 and 17. default : '010100'O | OCTETSTRING[3] |  |
| TSPX_Hsn12 | hopping parameter for TC_26_6_13_4, default : 39 | INTEGER |  |
| TSPX_Maio13 | hopping parameter for TC_26_6_13_4, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma13, default : 6 | INTEGER |  |
| TSPX_Ma13 | mobile allocation for TC_26_6_13_4, its value shall indicate number of frequencies between 1 and 17. default : '00E690'O | OCTETSTRING[3] |  |
| TSPX_Hsn13 | hopping parameter for TC_26_6_13_4, default: 42 | INTEGER |  |
| TSPX_Chtp7 | channel type for TC_26_6_13_5 | BITSTRING[5] |  |
| TSPX_ChMod4 | channel mode for TC_26_6_13_5 | BITSTRING[8] |  |
| TSPX_Tm2 | a value to calculate the starting time for TC_26_6_13_5, value between 60 -100 | INTEGER |  |
| TSPX_Maio14 | hopping parameter for TC_26_6_13_5, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma14, default : 5 | INTEGER |  |
| TSPX_Ma14 | mobile allocation for TC_26_6_13_5, its value shall indicate number of frequencies between 1 and 17. default : '014A28'O | OCTETSTRING[3] |  |
| TSPX_Hsn14 | hopping parameter for TC_26_6_13_5, default : 50 | INTEGER |  |
| TSPX_Maio15 | hopping parameter for TC_26_6_13_5, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma15, default : 12 | INTEGER |  |
| TSPX_Ma15 | mobile allocation for TC_26_6_13_5, its value shall indicate number of frequencies between 1 and 16. default : '00FF79'O | OCTETSTRING[3] |  |
| TSPX_Hsn15 | hopping parameter for TC_26_6_13_5, default : 33 | INTEGER |  |
| TSPX_Maio16 | hopping parameter for TC_26_6_13_5, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma16, default : 2 | INTEGER |  |
| TSPX_Ma16 | mobile allocation for TC_26_6_13_5, its value shall indicate number of frequencies between 1 and 16. default : '001110'O | OCTETSTRING[3] |  |
| TSPX_Hsn16 | hopping parameter for TC_26_6_13_5, default : 21 | INTEGER |  |
| TSPX_Chtp8 | channel type for TC_26_6_13_6 | BITSTRING[5] |  |
| TSPX_ChMod5 | channel mode for TC_26_6_13_6 | BITSTRING[8] |  |
| TSPX_Maio17 | hopping parameter for TC_26_6_13_6, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma17, default : 7 | INTEGER |  |
| TSPX_Ma17 | mobile allocation for TC_26_6_13_6, its value shall indicate number of frequencies between 1 and 17. default : '00E6A1'O | OCTETSTRING[3] |  |
| TSPX_Hsn17 | hopping parameter for TC_26_6_13_6, default : 22 | INTEGER |  |
| TSPX_Maio18 | hopping parameter for TC_26_6_13_6, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma18, default : 13 | INTEGER |  |
| TSPX_Ma18 | mobile allocation for TC_26_6_13_6, its value shall indicate number of frequencies between 1 and 16. default : '00FFF9'O | OCTETSTRING[3] |  |
| TSPX_Hsn18 | hopping parameter for TC_26_6_13_6, default : 9 | INTEGER |  |
| TSPX_Maio19 | hopping parameter for TC_26_6_13_6, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma19, default : 3 | INTEGER |  |
| TSPX_Ma19 | mobile allocation for TC_26_6_13_6, its value shall indicate number of frequencies between 1 and 16. default : '001111'O | OCTETSTRING[3] |  |
| TSPX_Hsn19 | hopping parameter for TC_26_6_13_6, default : 44 | INTEGER |  |
| TSPX_Chtp9 | channel type for TC_26_6_13_7 | BITSTRING[5] |  |
| TSPX_ChMod6 | channel mode for TC_26_6_13_7 | BITSTRING[8] |  |
| TSPX_Maio20 | hopping parameter for TC_26_6_13_7, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma20, default : 8 | INTEGER |  |
| TSPX_Ma20 | mobile allocation for TC_26_6_13_7, its value shall indicate number of frequencies between 1 and 17. default : '016699'O | OCTETSTRING[3] |  |
| TSPX_Hsn20 | hopping parameter for TC_26_6_13_7, default : 30 | INTEGER |  |
| TSPX_Maio21 | hopping parameter for TC_26_6_13_7, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma21, default : 14 | INTEGER |  |
| TSPX_Ma21 | mobile allocation for TC_26_6_13_7, its value shall indicate number of frequencies between 1 and 17. default : '01FEFB'O | OCTETSTRING[3] |  |
| TSPX_Chtp10 | channel type for TC_26_6_13_7 | BITSTRING[5] |  |
| TSPX_Maio22 | hopping parameter for TC_26_6_13_7, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma22, default : 4 | INTEGER |  |
| TSPX_Ma22 | mobile allocation for TC_26_6_13_7, its value shall indicate number of frequencies between 1 and 16. default : '006241'O | OCTETSTRING[3] |  |
| TSPX_Hsn22 | hopping parameter for TC_26_6_13_7, default : 11 | INTEGER |  |
| TSPX_Maio23 | hopping parameter for TC_26_6_13_7, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma23, default : 11 | INTEGER |  |
| TSPX_Ma23 | mobile allocation for TC_26_6_13_7, its value shall indicate number of frequencies between 1 and 16. default : '00FF1B'O | OCTETSTRING[3] |  |
| TSPX_Hsn23 | hopping parameter for TC_26_6_13_7, default : 60 | INTEGER |  |
| TSPX_Chtp11 | channel type for TC_26_6_13_8 | BITSTRING[5] |  |
| TSPX_ChMod7 | channel mode for TC_26_6_13_8 | BITSTRING[8] |  |
| TSPX_Maio24 | hopping parameter for TC_26_6_13_8, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma24, default : 9 | INTEGER |  |
| TSPX_Ma24 | mobile allocation for TC_26_6_13_8, its value shall indicate number of frequencies between 1 and 17. default : '00E6E9'O | OCTETSTRING[3] |  |
| TSPX_Hsn24 | hopping parameter for TC_26_6_13_8, default : 7 | INTEGER |  |
| TSPX_Maio25 | hopping parameter for TC_26_6_13_8, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma25, default : 15 | INTEGER |  |
| TSPX_Ma25 | mobile allocation for TC_26_6_13_8, its value shall indicate number of frequencies between 1 and 17. default : '01FFFB'O | OCTETSTRING[3] |  |
| TSPX_Chtp12 | channel type for TC_26_6_13_8 | BITSTRING[5] |  |
| TSPX_Maio26 | hopping parameter for TC_26_6_13_8, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma26, default : 5 | INTEGER |  |
| TSPX_Ma26 | mobile allocation for TC_26_6_13_8, its value shall indicate number of frequencies between 1 and 16. default : '009168'O | OCTETSTRING[3] |  |
| TSPX_Hsn26 | hopping parameter for TC_26_6_13_8, default : 9 | INTEGER |  |
| TSPX_Maio27 | hopping parameter for TC_26_6_13_8, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma27, default : 1 | INTEGER |  |
| TSPX_Ma27 | mobile allocation for TC_26_6_13_8, its value shall indicate number of frequencies between 1 and 16. default : '004080'O | OCTETSTRING[3] |  |
| TSPX_Hsn27 | hopping parameter for TC_26_6_13_8, default : 38 | INTEGER |  |
| TSPX_Chtp13 | channel type for TC_26_6_13_9 | BITSTRING[5] |  |
| TSPX_Tm3 | channel mode for TC_26_6_13_9, value between 60 -100 | INTEGER |  |
| TSPX_Maio28 | hopping parameter for TC_26_6_13_9, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma28, default : 10 | INTEGER |  |
| TSPX_Ma28 | mobile allocation for TC_26_6_13_9, its value shall indicate number of frequencies between 1 and 17. default : '00EE7A'O | OCTETSTRING[3] |  |
| TSPX_Hsn28 | hopping parameter for TC_26_6_13_9, default : 11 | INTEGER |  |
| TSPX_Maio29 | hopping parameter for TC_26_6_13_9, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma29, default : 6 | INTEGER |  |
| TSPX_Ma29 | mobile allocation for TC_26_6_13_9, its value shall indicate number of frequencies between 1 and 17. default : '00F070'O | OCTETSTRING[3] |  |
| TSPX_Chtp14 | channel type for TC_26_6_13_10 | BITSTRING[5] |  |
| TSPX_Maio30 | hopping parameter for TC_26_6_13_10, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma30, default : 11 | INTEGER |  |
| TSPX_Ma30 | mobile allocation for TC_26_6_13_10, its value shall indicate number of frequencies between 1 and 17. default : '017F69'O | OCTETSTRING[3] |  |
| TSPX_Hsn30 | hopping parameter for TC_26_6_13_10, default : 62 | INTEGER |  |
| TSPX_Maio31 | hopping parameter for TC_26_6_13_10, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma31, default : 1 | INTEGER |  |
| TSPX_Ma31 | mobile allocation for TC_26_6_13_10, its value shall indicate number of frequencies between 1 and 17. default : '000101'O | OCTETSTRING[3] |  |
| TSPX_MOBscSvcA | any supported MO basic service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MOBscSvcB | any supported MO basic service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MOBscSvcC | any supported MO basic service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MOBscSvcD | any supported MO basic service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MOBscSvcE | any supported MO basic service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MOBscSvcF | any supported MO basic service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MOBscSvcG | any supported MO basic service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MOBscSvcH | any supported MO basic service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MOBscSvcI | any supported MO basic service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MOBscSvcJ | any supported MO basic service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MT_DualModSvc | any supported MT dual mode service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MT_NonSptSvc | any not supported MT basic service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MTBscSvcA | any supported MT basic service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MTBscSvcB | any supported MT basic service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MTBscSvcC | any supported MT basic service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MTBscSvcD | any supported MT basic service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MTBscSvcE | any supported MT basic service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MTBscSvcF | any supported MT basic service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MTBscSvcG | any supported MT basic service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MTBscSvcH | any supported MT basic service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MTBscSvcI | any supported MT basic service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MTBscSvcJ | any supported MT basic service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MTNIC_BscSvcA | any supported MT basic service without immediate connection | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MTNIC_BscSvcB | any supported MT basic service without immediate connection | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MTNIC_BscSvcC | any supported MT basic service without immediate connection | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MTNIC_BscSvcD | any supported MT basic service without immediate connection | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MTNIC_BscSvcE | any supported MT basic service without immediate connection | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MTNIC_BscSvcF | any supported MT basic service without immediate connection | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MTNIC_BscSvcG | any supported MT basic service without immediate connection | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MTNIC_BscSvcH | any supported MT basic service without immediate connection | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MTNIC_BscSvcI | any supported MT basic service without immediate connection | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MTNIC_BscSvcJ | any supported MT basic service without immediate connection | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_Telephony_Rate | channel rate for TS11, default value : C_Full | "F", "H" |  |
| TSPX_MOChRateA | channel rate for TSPX_MOBscSvcA, default value : C_Full | "F", "H" |  |
| TSPX_MOChRateB | channel rate for TSPX_MOBscSvcB, default value : C_Full | "F", "H" |  |
| TSPX_MOChRateC | channel rate for TSPX_MOBscSvcC, default value : C_Full | "F", "H" |  |
| TSPX_MOChRateD | channel rate for TSPX_MOBscSvcD, default value : C_Full | "F", "H" |  |
| TSPX_MOChRateE | channel rate for TSPX_MOBscSvcE, default value : C_Full | "F", "H" |  |
| TSPX_MOChRateF | channel rate for TSPX_MOBscSvcF, default value : C_Full | "F", "H" |  |
| TSPX_MOChRateG | channel rate for TSPX_MOBscSvcG, default value : C_Full | "F", "H" |  |
| TSPX_MOChRateH | channel rate for TSPX_MOBscSvcH, default value : C_Full | "F", "H" |  |
| TSPX_MOChRateI | channel rate for TSPX_MOBscSvcI, default value : C_Full | "F", "H" |  |
| TSPX_MOChRateJ | channel rate for TSPX_MOBscSvcJ, default value : C_Full | "F", "H" |  |
| TSPX_MTChRateA | channel rate for TSPX_MTBscSvcA, default value : C_Full | "F", "H" |  |
| TSPX_MTChRateB | channel rate for TSPX_MTBscSvcB, default value : C_Full | "F", "H" |  |
| TSPX_MTChRateC | channel rate for TSPX_MTBscSvcC, default value : C_Full | "F", "H" |  |
| TSPX_MTChRateD | channel rate for TSPX_MTBscSvcD, default value : C_Full | "F", "H" |  |
| TSPX_MTChRateE | channel rate for TSPX_MTBscSvcE, default value : C_Full | "F", "H" |  |
| TSPX_MTChRateF | channel rate for TSPX_MTBscSvcF, default value : C_Full | "F", "H" |  |
| TSPX_MTChRateG | channel rate for TSPX_MTBscSvcG, default value : C_Full | "F", "H" |  |
| TSPX_MTChRateH | channel rate for TSPX_MTBscSvcH, default value : C_Full | "F", "H" |  |
| TSPX_MTChRateI | channel rate for TSPX_MTBscSvcI, default value : C_Full | "F", "H" |  |
| TSPX_MTChRateJ | channel rate for TSPX_MTBscSvcJ, default value : C_Full | "F", "H" |  |
| TSPX_MTNIC_ChRateA | channel rate for TSPX_MTNIC_BscSvcA, default value : C_Full | "F", "H" |  |
| TSPX_MTNIC_ChRateB | channel rate for TSPX_MTNIC_BscSvcB, default value : C_Full | "F", "H" |  |
| TSPX_MTNIC_ChRateC | channel rate for TSPX_MTNIC_BscSvcC, default value : C_Full | "F", "H" |  |
| TSPX_MTNIC_ChRateD | channel rate for TSPX_MTNIC_BscSvcD, default value : C_Full | "F", "H" |  |
| TSPX_MTNIC_ChRateE | channel rate for TSPX_MTNIC_BscSvcE, default value : C_Full | "F", "H" |  |
| TSPX_MTNIC_ChRateF | channel rate for TSPX_MTNIC_BscSvcF, default value : C_Full | "F", "H" |  |
| TSPX_MTNIC_ChRateG | channel rate for TSPX_MTNIC_BscSvcG, default value : C_Full | "F", "H" |  |
| TSPX_MTNIC_ChRateH | channel rate for TSPX_MTNIC_BscSvcH, default value : C_Full | "F", "H" |  |
| TSPX_MTNIC_ChRateI | channel rate for TSPX_MTNIC_BscSvcI, default value : C_Full | "F", "H" |  |
| TSPX_MTNIC_ChRateJ | channel rate for TSPX_MTNIC_BscSvcJ, default value : C_Full | "F", "H" |  |
| TSPX_EmgCallRate | Rate for the basic service supported for MO emergency calls | "F", "H" |  |
| TSPX_MO_NonCallSS | any supported MO non-call releated supplementary service | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MO_BscSvc_SMS | any supported MO SMS calls | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MO_DualModSvc | any supported MO dual mode call | "C_Telephony", "C_EmgCallSRV", "C_AltSpchG3", "C_AutoG3", "C_300cda", "C_1200cda", "C_120075cda", "C_2400cda", "C_4800cda", "C_2400cds", "C_4800cds", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Pkt2400", "C_Pkt4800", "C_Pkt9600", "C_AltSpchData", "C_SpchData" |  |
| TSPX_MO_DualModRate | Rate for the supported MO dual mode call in TSPX_MO_DualModSvc | "F", "H" |  |

Annex C:	PCTR Proforma

Notwithstanding the provisions of the copyright clause related to the text of this ETS, ETSI grants that users of this ETS may freely reproduce the PCTR proforma in this annex so that it can be used for its intended purposes and may further publish the completed PCTR.

PROTOCOL

Conformance Test Report

(PCTR)

Global System for Mobile Communication, GSM,

User-Network Access

Layer 3 Signalling Functions

| Test Candidate |  |
| --- | --- |
| Name : | SUT name |
| Model : | model |
| H/W version : | hw |
| S/W version : | sw |
| Serial No. : | serienr |

| Client |  |
| --- | --- |
| Name : |  |
| Street / No. : |  |
| Postal Code / City: |  |
| Country : |  |

This Test Report shall not be reproduced except in full without the written permission of TEST LAB REFERENCE, and shall not be quoted out of context.

# History

| Document history |  |
| --- | --- |
| December 1995 | Public Enquiry PE 97: 1995-12-04 to 1996-03-29 |
| May 1996 | Vote V 103: 1996-05-20 to 1996-08-23 |
| September 1996 | First Edition |
| December 1995 | Public Enquiry PE 97: 1995-12-04 to 1996-03-29 (Second Edition) |
| May 1996 | Vote V 103: 1996-05-20 to 1996-07-26 (Second Edition) |
| August 1996 | Unified Approval Procedure UAP 52 1996-08-19 to 1996-12-13 (Second Edition) |
| November 1996 | Unified Approval Procedure UAP 58 1996-11-18 to 1997-03-14 (Third Edition) |
| January 1997 | Second Edition |
| April 1997 | One-step Approval Procedure OAP 9731: 1997-04-04 to 1997-08-01 (Fourth Edition) |
| May 1997 | Third Edition |
| August 1997 | One-step Approval Procedure OAP 9748: 1997-08-01 to 1997-11-28 (Fifth Edition) |