DRAFT

EUROPEAN	pr	ETS 300 607-3

TELECOMMUNICATION		August 1997

STANDARD		Fifth Edition

Source: ETSI TC-SMG		Reference: RE/SMG-071110PR2-3

ICS:	33.020

Key words:	Digital cellular telecommunications system, Global System for Mobile communications (GSM)

Digital cellular telecommunications system (Phase 2);

Mobile Station (MS) conformance specification;

Part 3: Layer 3 (L3) Abstract Test Suite (ATS)

(GSM 11.10-3)

ETSI

European Telecommunications Standards Institute

ETSI Secretariat

Postal address: F-06921 Sophia Antipolis CEDEX - FRANCE

Office address: 650 Route des Lucioles - Sophia Antipolis - Valbonne - FRANCE

X.400: c=fr, a=atlas, p=etsi, s=secretariat - Internet: secretariat@etsi.fr

Tel.: +33 4 92 94 42 00 - Fax: +33 4 93 65 47 16

Copyright Notification: No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media.

© European Telecommunications Standards Institute 1997. All rights reserved.

Whilst every care has been taken in the preparation and publication of this document, errors in content, typographical or otherwise, may occur. If you have comments concerning its accuracy, please write to "ETSI Editing and Committee Support Dept." at the address shown on the title page.

Contents

Foreword	7

1	Scope	9

2	Normative references	9

3	Definitions, symbols and abbreviations	12

3.1	Definitions	12

3.2	Abbreviations	12

4	Test Suite Structure (TSS)	13

4.1	Test suite naming convention	13

4.2	Suite Overview	14

4.3	Test groups	14

4.3.1	Valid Behaviour tests (BV)	14

4.3.2	Invalid Behaviour/Inopportune Behaviour tests (BI-BO)	14

4.4	Test Step Structure	15

4.4.1	Preambles	15

4.4.2	Postambles	15

5	Test Purposes (TP)	15

5.1	TP and test case naming convention	15

6	Abstract test method and test configurations	15

6.1	Test system model	15

6.2	Test Method	16

Annex A: Abstract Test Suite	17

Suite overview	18

Suite structure	18

Test case index	19

Test step index	29

Default index	37

Declarations Part	38

Test suite type definitions	38

Simple type definitions	38

Structured Type definitions	40

ASN1 type definitions	62

Test suite operation definitions	87

Test suite parameter declarations	144

Test case selection expression definitions	164

Test suite constant declarations	167

Test case variable declarations	186

PCO declarations	191

Timer declarations	191

ASP type definitions	191

TTCN ASP Type definitions	191

PDU type definitions	218

TTCN PDU Type definitions	218

Constraints Part	244

Test suite type constraint declarations	244

Structured type constraint declarations	244

ASN1 type constraints	497

ASP constraint declarations	511

TTCN ASP constraint declarations	511

PDU constraint declarations	543

TTCN PDU constraint declarations	543

Dynamic Part	652

Test Cases	652

Test Group General	652

Test Group InitialTest	667

Test Group IdleMode	693

Test Group BiBo	696

Test Group RR	734

Test Group MM	892

Test Group CC	955

Test Group StructureProc	1063

Test Group EGSMSignalling	1085

Test Group SS	1107

Test Group SM	1197

Test Step Library	1225

Test Step Group management	1225

Test Step Group ChConfig	1226

Test Step Group Miscellaneous	1251

Test Step Group SysInfo	1290

Test Step Group OperatorOP	1301

Test Step Group Postambles	1302

Test Step Group Preambles	1304

Defaults Library	1397

Annex B: Partial IXIT proforma	1404

Introduction	1404

B.1	Identification Summary	1405

B.2	Abstract Test Suite Summary	1405

B.3	Test Laboratory	1405

B.3.1	Test Laboratory Identification	1405

B.3.2	Accreditation status of the test service	1406

B.3.3	Manager of Test Laboratory	1406

B.3.4	Contact person of Test Laboratory	1406

B.3.5	Means of Testing	1406

B.3.6	Instructions for Completion	1407

B.4	Client	1408

B.4.1	Client Identification	1408

B.4.2	Client Test Manager	1409

B.4.3	Client Contact person	1409

B.4.4	Test Facilities Required	1409

B.5	System Under Test	1410

B.5.1	SUT Information	1410

B.5.2	Limitations of the SUT	1411

B.5.3	Environmental Conditions	1412

B.6	Ancillary Protocols	1413

B.6.1	Ancillary Protocols 1	1414

B.6.2	Ancillary Protocols 2	1414

B.7	Protocol Layer Information for L3 of Mobile Station	1414

B.7.1	Information provided for test purposes by the MS supplier	1414

B.7.2	MMI information	1427

B.7.3	Test house specified parameters	1432

Annex C: PCTR Proforma	1464

History	1465

Blank page