# Suite overview

## Suite structure

| Test Suite Structure |  |
| --- | --- |
| Suite Name: | GSM_L3_MS_v4190 |
| Standards Ref: | ETS 300 557 |
| PICS Ref: |  |
| PIXIT Ref: |  |
| Test Method(s): | Distributed Single Layer Test Method |
| Comments: | Version 4.19.0 |

| Structure and Objectives |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Group Reference |  | Selection Ref | Test Group Objective | Page Nr |
| GSM_L3_MS_v4190/General/ |  | SelExpr_0000 | To verify the supported and non-supported services. | 637 |
| GSM_L3_MS_v4190/InitialTest/ |  | SelExpr_0100 | To verify random access procedure, IMSI attatch and detach procedure, sequenced MM/CC message transfer and establishment causes. | 652 |
| GSM_L3_MS_v4190/IdleMode/ |  | SelExpr_0200 | To verify MS functions in idle mode | 678 |
| GSM_L3_MS_v4190/BiBo/ |  | SelExpr_0300 | To verify the MS handling unknown, unforeseen and erroneous protocol data, and parallel transactions | 681 |
| GSM_L3_MS_v4190/RR/ |  | SelExpr_0400 | To verify the elementary procedures for radio resource management | 719 |
| GSM_L3_MS_v4190/MM/ |  | SelExpr_0500 | To verify the elementary procedures of mobility management. | 877 |
| GSM_L3_MS_v4190/CC/ |  | SelExpr_0600 | To verify the circuit switched call cntrol functions. | 940 |
| GSM_L3_MS_v4190/StructureProc/ |  | SelExpr_0700 | To verify the structured procedures. | 1048 |
| GSM_L3_MS_v4190/EGSMSignalling/ |  | SelExpr_1000 | To verify the different procedures which may be impacted when some channel uses E- GSM frequencies. | 1070 |
| GSM_L3_MS_v4190/SS/ |  | SelExpr_0800 | To verify the functions of supplementary services. | 1093 |
| GSM_L3_MS_v4190/SM/ |  | SelExpr_0900 | To verify the functions of short message service. | 1183 |
| Detailed Comments: |  |  |  |  |

## Test case index

| Test Case Index |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Group Reference | Test Case Id |  | Selection ref | Description | Page Nr |
| GSM_L3_MS_v4190/General/ | TC_11_1_1 |  | SelExpr_0002 | Verification of support and non-support of services (MT). | 637 |
| GSM_L3_MS_v4190/General/ | TC_11_1_2 |  | SelExpr_0007 | Verification of support and non-support of services (MO). | 639 |
| GSM_L3_MS_v4190/General/ | TC_11_2 |  | SelExpr_0002 | Verification of support of the single numbering scheme. | 644 |
| GSM_L3_MS_v4190/General/ | TC_11_3 |  | SelExpr_0003 | Verification of non-support of services. (Advice of Charge Charging, AOCC) | 645 |
| GSM_L3_MS_v4190/General/ | TC_11_4 |  | SelExpr_0004 | Verification of non-support of services. (Call Hold) | 648 |
| GSM_L3_MS_v4190/General/ | TC_11_5 |  | SelExpr_0005 | Verification of non-support of services. (MultiParty) | 649 |
| GSM_L3_MS_v4190/General/ | TC_11_6 |  | SelExpr_0006 | Verification of non-support of feature. (Fixed dialling number) | 651 |
| GSM_L3_MS_v4190/InitialTest/ | TC_26_2_1_1 |  | SelExpr_0101 | Initial Layer 3 tests - Channel request / initial time. 11.10 Ref. ver.4.10; CR | 652 |
| GSM_L3_MS_v4190/InitialTest/ | TC_26_2_1_2 |  | SelExpr_0101 | Initial Layer 3 tests - Channel request / repetition time. 11.10 Ref. ver.4.10; CR: C46 | 654 |
| GSM_L3_MS_v4190/InitialTest/ | TC_26_2_1_3 |  | SelExpr_0101 | Initial Layer 3 tests - Channel request / random reference. 11.10 Ref. ver.4.10; CR | 656 |
| GSM_L3_MS_v4190/InitialTest/ | TC_26_2_2 |  | SelExpr_0101 | IMSI detach and IMSI attach. 11.10 Ref. ver 4.10.0; CR. 0295-6r1. CR.C62r1. | 657 |
| GSM_L3_MS_v4190/InitialTest/ | TC_26_2_3 |  | SelExpr_0101 | Sequenced MM / CM message transfer. | 659 |
| GSM_L3_MS_v4190/InitialTest/ | TC_26_2_4_1 |  | SelExpr_0102 | Establishment Cause /pr1 (TCH) | 661 |
| GSM_L3_MS_v4190/InitialTest/ | TC_26_2_4_2 |  | SelExpr_0103 | Establishment Cause /pr2 ( /H) | 663 |
| GSM_L3_MS_v4190/InitialTest/ | TC_26_2_4_3 |  | SelExpr_0107 | Establishment Cause /pr3 (TCH/FS) | 665 |
| GSM_L3_MS_v4190/InitialTest/ | TC_26_2_4_4 |  | SelExpr_0104 | Establishment Cause /pr4 (data) | 667 |
| GSM_L3_MS_v4190/InitialTest/ | TC_26_2_4_5 |  | SelExpr_0101 | Establishment Cause /pr5 | 669 |
| GSM_L3_MS_v4190/InitialTest/ | TC_26_2_4_6 |  | SelExpr_0101 | Establishment Cause /pr6 | 672 |
| GSM_L3_MS_v4190/InitialTest/ | TC_26_2_4_7 |  | SelExpr_0105 | Establishment Cause /pr7 (non-call-SS) | 675 |
| GSM_L3_MS_v4190/InitialTest/ | TC_26_2_4_8 |  | SelExpr_0106 | Establishment Cause /pr8 (SMS/PP MO) | 677 |
| GSM_L3_MS_v4190/IdleMode/ | TC_26_3_2 |  | SelExpr_0201 | MS indication of available PLMNs | 678 |
| GSM_L3_MS_v4190/IdleMode/ | TC_26_3_3 |  | SelExpr_0201 | MS will send only if BSS is "on air". | 679 |
| GSM_L3_MS_v4190/IdleMode/ | TC_26_3_4 |  | SelExpr_0201 | Manual mode of PLMN selection 11.10 Ref. ver. 4.10.0; CR. 11.10-661. C64 | 680 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_1 |  | SelExpr_0302 | Handling of unknown protocol discriminator | 681 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_2_1_1 |  | SelExpr_0302 | Handling of unknown TI and skip indicator / RR | 682 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_2_1_2 |  | SelExpr_0302 | TI Skip indicator / RR / RR Connection established | 683 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_2_2 |  | SelExpr_0302 | TI and skip indicator / MM | 685 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_2_3 |  | SelExpr_0301 | TI and skip indicator / CC | 687 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_3_1 |  | SelExpr_0301 | Undefined or unexpected Message type / undefined message type / CC | 689 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_3_2 |  | SelExpr_0301 | Undefined or unexpected message type / undefined message type / MM | 690 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_3_3 |  | SelExpr_0302 | Undefined or unexpected message type / undefined message type / RR | 691 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_3_4 |  | SelExpr_0301 | Undefined or unexpected message type / unexpected message type / CC | 692 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_4_1 |  | SelExpr_0302 | Unforeseen info elements in non-imperative message part / duplicated info elements | 693 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_5_1_1_1 |  | SelExpr_0302 | Non-semantical mandatory IE errors / RR / missing mandatory IE error / special case | 694 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_5_1_1_2 |  | SelExpr_0302 | Non-semantical mandatory IE errors / RR / missing mandatory IE error / general case | 695 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_5_1_2 |  | SelExpr_0302 | Non-semantical mandatory ie errors / RR / comprehension required | 696 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_5_2_1 |  | SelExpr_0301 | Non-semantical mandatory IE errors / MM / syntactically incorrect mandatory IE | 697 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_5_2_2 |  | SelExpr_0302 | Non-semantical mandatory IE errors / MM / syntactically incorrect mandatory IE 11.10 Ref. ver. 4.10.0; CR. 11.10-688 | 698 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_5_2_3 |  | SelExpr_0302 | Non-semantical mandatory IE errors / MM / syntactically incorrect mandatory IE | 699 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_5_3_1_1 |  | SelExpr_0301 | Non-semantical mandatory IE errors / CC / missing mandatory IE / disconnect message | 701 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_5_3_1_2 |  | SelExpr_0301 | Non-semantical mandatory IE errors / CC / missing mandatory IE / general case | 702 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_5_3_2 |  | SelExpr_0301 | Non-semantical mandatory IE errors / CC / comprehension required | 703 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_6_1_1 |  | SelExpr_0302 | Unknown IE, comprehension not required / MM / IE unknown in the protocol | 704 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_6_1_2 |  | SelExpr_0302 | Unknown IE, comprehension not required / MM / IE unknown in the message | 706 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_6_2_1 |  | SelExpr_0301 | Unknown info elements in the non-imperative message part / CC / Call establishment | 707 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_6_2_2 |  | SelExpr_0301 | Unknown information elements in the non-imperative message part / CC / disconnect | 708 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_6_2_3 |  | SelExpr_0301 | Unknown information elements in the non-imperative message part / CC / release | 709 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_6_2_4 |  | SelExpr_0301 | Unknown info elements in the non-imperative message part / CC / release complete | 710 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_6_3 |  | SelExpr_0302 | Unknown IE in the non-imperative message part, comprehension not required / RR / unknown in the protocol. | 711 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_7_1_1 |  | SelExpr_0302 | Spare bits / RR / paging channel | 712 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_7_1_2 |  | SelExpr_0302 | Spare bits / RR / BCCH | 713 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_7_1_3 |  | SelExpr_0302 | pare bits / RR / AGCH | 714 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_7_1_4 |  | SelExpr_0302 | Spare bits / RR / connected mode | 715 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_7_2 |  | SelExpr_0302 | Spare bits / MM | 716 |
| GSM_L3_MS_v4190/BiBo/ | TC_26_5_7_3 |  | SelExpr_0301 | Spare bits / CC | 717 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_1_1 |  | SelExpr_0401 | Immediate Assignment / SDCCH or TCH assignment | 719 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_1_2 |  | SelExpr_0401 | Immediate Assignment / extended assignment | 721 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_1_3 |  | SelExpr_0401 | Immediate Assignment / assignment rejection | 724 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_1_4 |  | SelExpr_0401 | Immediate Assignment / ignore assignment | 727 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_1_5 |  | SelExpr_0401 | Immediate Assignment after immediate assignment reject | 729 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_2_1_1 |  | SelExpr_0401 | Paging / normal / type 1 | 730 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_2_1_2 |  | SelExpr_0401 | Paging / normal / type 2 | 732 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_2_1_3 |  | SelExpr_0401 | Paging / normal / type 3 | 734 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_2_2 |  | SelExpr_0401 | Paging / extended | 736 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_2_3_1 |  | SelExpr_0401 | Paging / reorganisation / procedure 1 | 740 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_2_3_2 |  | SelExpr_0401 | Paging / reorganisation / procedure 2 | 742 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_2_4 |  | SelExpr_0401 | Paging / same as before | 743 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_2_5 |  | SelExpr_0401 | Multislot CCCH handling | 744 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_3_1 |  | SelExpr_0402 | Measurement / no neighbour | 746 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_3_2 |  | SelExpr_0402 | Measurement / all neighbours present | 748 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_3_3 |  | SelExpr_0402 | Measurement / barred cells and non-permitted NCCs | 750 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_3_4 |  | SelExpr_0402 | Measurement / DTX | 752 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_3_5 |  | SelExpr_0402 | Measurement / frequency formats | 756 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_4_1 |  | SelExpr_0401 | Dedicated assignment / Successful case | 759 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_4_2_1 |  | SelExpr_0402 | Dedicated assignment / failure / failure during active state | 765 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_4_2_2 |  | SelExpr_0401 | Dedicated assignment / failure / genaral case | 767 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_1 |  | SelExpr_0402 | Handover / successful / active call / non-synchronized | 769 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_2_1 |  | SelExpr_0410 | Retransmit L3-msg during non synchr. HO from SDCCH/4_nonFH to TCH/F_FH. | 774 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_2_2 |  | SelExpr_0408 | Retransmit L3-msg during non synchr. HO from SDCCH/4_nonFH to TCH/H_FH. | 776 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_2_3 |  | SelExpr_0410 | Retransmit L3-msg during non synchr. HO from SDCCH/4_nonFH to SDCCH/8_FH. | 778 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_2_4 |  | SelExpr_0410 | Retransmit L3-msg during non synchr. HO from SDCCH/8_nonFH in cell A to SDCCH/8_FH in cellB. | 780 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_2_5 |  | SelExpr_0408 | Retransmit L3-msg during non synchr. HO from TCH/F_NonFH in cell A to TCH/H_NonFH in cellB. | 782 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_2_6 |  | SelExpr_0408 | Retransmit L3-msg during non synchr. HO from TCH/H_FH in cell A to TCH/F_FH in cellB. | 784 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_2_7 |  | SelExpr_0410 | Retransmit L3-msg during non synchr. HO from TCH/F_FH in cell A to TCH/F_FH in cellB. | 786 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_2_8 |  | SelExpr_0410 | Retransmit L3-msg during non synchr. HO from TCH/SDCCH8_FH in cell A to TCH/F_NonFH in cellB. | 788 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_2_9 |  | SelExpr_0410 | Retransmit L3-msg during non synchr. HO from SDCCH8_NoFH in cell A to TCH/F_FH in cellB. | 790 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_2_10 |  | SelExpr_0408 | Retransmit L3-msg during non synchr. HO from TCH/SDCCH8_NoFH in cell A to TCH/H_FH in cellB. | 792 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_3_1 |  | SelExpr_0402 | synchronised HO from TCH/F_FH in cellA to TCH/F_nonFH in cellB. | 794 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_3_2 |  | SelExpr_0408 | synchronised HO from TCH/H_FH in cellA to TCH/H_nonFH in cellB. | 796 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_4_1 |  | SelExpr_0410 | Handover/ successful/ call under establishment/ finely synchronized (HO: SDCCH8/FH -> SDCCH/8_FH) | 798 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_4_2 |  | SelExpr_0410 | Handover/ successful/ call under establishment/ finely synchronized (HO: SDCCH8/FH -> SDCCH/4_noFH) | 800 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_4_3 |  | SelExpr_0410 | Handover/ successful/ call under establishment/ finely synchronized (HO: TCH/F_NoFH -> TCH/F_FH) | 802 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_4_4 |  | SelExpr_0410 | Handover/ successful/ call under establishment/ finely synchronized (HO: SDCCH8_NoFH -> TCH/F_NoFH) | 804 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_5_1 |  | SelExpr_0402 | Handover / successful / active call / pre-synchronised / Timing Advance IE not included | 806 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_5_2 |  | SelExpr_0410 | Handover / successful / call being estab. / pre-synch. /Tim. Advance IE is included / reporting of obsrvd. time diff requested | 809 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_6 |  | SelExpr_0409 | Handover / successful / active call / pseudo -synchronised | 812 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_7 |  | SelExpr_0402 | Handover / successful / active call / non-synchronised / reporting of obsrvd. Time diff requested | 814 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_8 |  | SelExpr_0402 | Handover / L3-failure 11.10 Ref. ver.4.10.0; CR 11.10-720 | 816 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_5_9 |  | SelExpr_0402 | Handover / L1-failure 11.10 Ref. ver.4.10.0; CR 11.10-720 | 818 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_6_1 |  | SelExpr_0401 | Frequency redefinition | 820 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_7_1 |  | SelExpr_0403 | Channel mode modify / full rate | 824 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_7_2 |  | SelExpr_0404 | Channel mode modify / half rate 11.10 Ref ver. 4.10.0;CR.0295-17r1; 11.10-780 | 827 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_8_1 |  | SelExpr_0405 | Ciphering mode / start ciphering | 830 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_8_2 |  | SelExpr_0402 | Ciphering mode / no ciphering | 832 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_8_3 |  | SelExpr_0405 | Ciphering mode / old cipher key | 833 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_8_4 |  | SelExpr_0401 | Ciphering mode / Change of mode, algorithm and key | 834 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_8_5 |  | SelExpr_0401 | Ciphering mode / IMEI request 11.10 REF. VER.4.10.0; CR. C92 | 841 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_11_1 |  | SelExpr_0406 | Test of Classmark change | 842 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_11_2 |  | SelExpr_0401 | Test of Classmark Interrogation | 844 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_12_1 |  | SelExpr_0401 | Channel release / SDCCH | 845 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_12_2 |  | SelExpr_0401 | Channel release / SDCCH - no L2 ACK | 847 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_12_3 |  | SelExpr_0403 | Channel release / TCH-F | 849 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_12_4 |  | SelExpr_0403 | Channel release / TCH-F - no L2 ACK | 851 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_13_1 |  | SelExpr_0401 | Dedicated assignment with starting time / successful case / time not elapsed | 853 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_13_2 |  | SelExpr_0401 | Dedicated assignment with starting time / successful case / time elapsed | 855 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_13_3 |  | SelExpr_0401 | Dedicated assignment with starting time and frequency redefinition/ failure case / time not elapsed | 857 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_13_4 |  | SelExpr_0401 | Dedicated assignment with starting time and frequency redefinition/ failure case / time elapsed | 859 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_13_5 |  | SelExpr_0401 | Handover with starting time / successful case / time not elapsed | 861 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_13_6 |  | SelExpr_0401 | Handover with starting time / successful case / time elapsed | 864 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_13_7 |  | SelExpr_0401 | Handover with starting time and frequency redefinition / failure case / time not elapsed | 867 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_13_8 |  | SelExpr_0401 | Handover with starting time and frequency redefinition / failure case / time elapsed | 870 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_13_9 |  | SelExpr_0401 | Immediate assignment with starting time / successful case / time not elapsed | 873 |
| GSM_L3_MS_v4190/RR/ | TC_26_6_13_10 |  | SelExpr_0401 | Immediate assignment with starting time / successful case / time elapsed | 875 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_1 |  | SelExpr_0501 | TMSI reallocation. | 877 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_2_1 |  | SelExpr_0501 | Authentication accepted. | 878 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_2_2 |  | SelExpr_0501 | Authentication Reject | 879 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_3_1 |  | SelExpr_0501 | General identification with IMSI and TMSI in non ciphered mode and IMEI in ciphered mode. | 881 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_3_2 |  | SelExpr_0501 | Handling of IMSI shorter than the maximum length. | 882 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_1 |  | SelExpr_0501 | Location Updating Accepted | 885 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_2_1 |  | SelExpr_0501 | Location Update Reject with the cause "IMSI unknown in HLR", "illegal MS" or "Illegal ME". | 887 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_2_2_1 |  | SelExpr_0501 | Location Update Rejected with the cause "PLMN not allowed" - test 1. | 889 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_2_2_2 |  | SelExpr_0501 | Location Update Rejected with the cause "PLM not allowed" - test 2. | 891 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_2_3 |  | SelExpr_0501 | Location Update Rejected with the cause "Location area not allowed". | 893 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_2_4_1 |  | SelExpr_0501 | Location Updating / rejected / roaming / procedure1 | 895 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_2_4_2 |  | SelExpr_0501 | Location Updating / rejected / roaming / procedure2 | 896 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_2_4_3 |  | SelExpr_0501 | Location Updating / rejected / roaming / procedure3 | 898 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_2_4_4 |  | SelExpr_0501 | Location Updating / rejected / roaming / procedure4 | 900 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_2_4_5 |  | SelExpr_0502 | Location Updating / rejected / roaming / procedure5 | 901 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_3_1 |  | SelExpr_0501 | Location Updating / abnormal cases / random access fails | 902 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_3_2 |  | SelExpr_0501 | Location updating / abnormal cases / attempt counter less or equal to 4, LAI different | 904 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_3_3 |  | SelExpr_0501 | Location Updating/Abnormal cases/Attempt Counter Equal to 4 | 908 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_3_4 |  | SelExpr_0501 | Location updating/Abnormal cases/Attempt Counter Equal to 4, stored LAI equal to broadcast LAI | 912 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_4 |  | SelExpr_0502 | To verify that the MS aborts the RR-connection at the expiry of timer T3240. | 916 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_5_1 |  | SelExpr_0501 | Periodic Updating Procedure/ Change of T3212 value. | 917 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_5_2 |  | SelExpr_0501 | Periodic Updating Procedure/ Reset of T3212 after receiving of the first L3 message. | 919 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_5_3 |  | SelExpr_0501 | Periodic Updating Procedure/ Reset of T3212 after generic LUP or IMSI attach procedure | 921 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_5_4_1 |  | SelExpr_0501 | Location updating/ periodic HPLMN search/ MS waits time T. | 922 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_5_4_2 |  | SelExpr_0501 | Location updating/ periodic HPLMN search/ MS in manual mode. | 923 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_5_4_3 |  | SelExpr_0501 | Location updating/ periodic HPLMN search/ MS waits at least two minutes | 924 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_4_6 |  | SelExpr_0501 | Location updating/ interworking of attach and periodic. | 926 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_5_2 |  | SelExpr_0501 | MM connection/ establishment with cipher. | 928 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_5_3 |  | SelExpr_0501 | MM connection/ establishment without cipher. | 930 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_5_4 |  | SelExpr_0501 | MM connection/ establishment rejected. | 931 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_5_5 |  | SelExpr_0501 | MM connection/ establishment rejected cause 4. | 932 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_5_6 |  | SelExpr_0501 | MM connection/ expiry T3230 | 933 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_5_7_1 |  | SelExpr_0501 | MM connection/ abortion by the network/ cause #6 | 934 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_5_7_2 |  | SelExpr_0504 | MM connection/ abortion by the network/ cause not equal #6 | 936 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_5_8_1 |  | SelExpr_0501 | MM connection / follow-on request pending / test1 | 937 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_5_8_2 |  | SelExpr_0501 | MM connection / follow-on request pending / test2 | 938 |
| GSM_L3_MS_v4190/MM/ | TC_26_7_5_8_3 |  | SelExpr_0501 | MM connection / follow-on request pending / test3 | 939 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_1_1 |  | SelExpr_0601 | Outgoing call/ / U10 null state. | 940 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_2_1 |  | SelExpr_0601 | Outgoing call / U0.1 MM connection pending / CM service rejected | 941 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_2_2 |  | SelExpr_0601 | Outgoing call / U0.1 MM connection pending / CM service accepted | 942 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_2_3 |  | SelExpr_0601 | Outgoing call / U0.1 MM connection pending / lower layer failure | 943 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_3_1 |  | SelExpr_0601 | Outgoing call / U1 call initiated / receiving CALL PROCEEDING | 944 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_3_2 |  | SelExpr_0601 | Outgoing call / U1 call initiated / rejecting with RELEASE COMPLETE | 945 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_3_3 |  | SelExpr_0601 | Outgoing call / U1 call initiated / T303 expiry | 946 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_3_4 |  | SelExpr_0601 | Outgoing call / U1 call initiated / lower layer failure | 947 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_3_5 |  | SelExpr_0601 | Outgoing call / U1 call initiated / receiving ALERTING | 948 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_3_6 |  | SelExpr_0601 | Outgoing call / U1 call initiated / entering state U10 | 949 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_3_7 |  | SelExpr_0601 | Outgoing call / U1 call initiated / unknown message received | 950 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_4_1 |  | SelExpr_0601 | Outgoing call / U3 MS originating call proceeding / ALERTING received | 951 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_4_2 |  | SelExpr_0601 | Outgoing call / U3 MS originating call proceeding / CONNECT received | 952 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_4_3 |  | SelExpr_0601 | Outgoing call / U3 MS origintg. call proceeding / PROGRESS received without in band information. | 953 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_4_4 |  | SelExpr_0601 | Outgoing call / U3 MS originating call proceeding / PROGRESS with in band information | 954 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_4_5 |  | SelExpr_0601 | Outgoing call / U3 MS originating call proceeding / DISCONNECT with in band tones | 956 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_4_6 |  | SelExpr_0601 | Outgoing call / U3 MS originating call proceeding / DISCONNECT without in band tones | 957 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_4_7 |  | SelExpr_0601 | Outgoing call / U3 MS originating call proceeding / RELEASE received | 958 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_4_8 |  | SelExpr_0601 | Outgoing call / U3 MS originating call proceeding / termination requested by the user | 959 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_4_9 |  | SelExpr_0601 | Outgoing call / U3 MS originating call proceeding / traffic channel allocation | 960 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_4_10 |  | SelExpr_0601 | Outgoing call / U3 MS originating call proceeding / timer T310 timeout | 961 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_4_11 |  | SelExpr_0601 | Outgoing call / U3 MS originating call proceeding / lower layer failure | 962 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_4_12 |  | SelExpr_0601 | Outgoing call / U3 MS originating call proceeding / unknown message received | 963 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_4_13 |  | SelExpr_0605 | Outgoing call / U3 MS originating call proceeding / Internal alerting indication | 964 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_5_1 |  | SelExpr_0601 | Outgoing call / U4 call delivered / CONNECT received | 965 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_5_2 |  | SelExpr_0601 | Outgoing call / U4 call delivered / termination requested by the user | 966 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_5_3 |  | SelExpr_0601 | Outgoing call / U4 call delivered / DISCONNECT with in band tones | 967 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_5_4 |  | SelExpr_0601 | Outgoing call / U4 call delivered / DISCONNECT without in band tones | 968 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_5_5 |  | SelExpr_0601 | Outgoing call / U4 call delivered / RELEASE received | 969 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_5_6 |  | SelExpr_0601 | Outgoing call / U4 call delivered / lower layer failure | 970 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_5_7 |  | SelExpr_0601 | Outgoing call / U4 call delivered / traffic channel allocation | 971 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_5_8 |  | SelExpr_0601 | Outgoing call / U4 call delivered / unknown message received | 972 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_6_1 |  | SelExpr_0601 | U10 call active / termination requested by the user | 973 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_6_2 |  | SelExpr_0601 | 10 call active / RELEASE received | 974 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_6_3 |  | SelExpr_0601 | U10 call active / DISCONNECT with in band tones | 975 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_6_4 |  | SelExpr_0601 | U10 call active / DISCONNECT without in band tones | 976 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_6_5 |  | SelExpr_0601 | U10 call active / RELEASE COMPLETE received | 977 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_6_6 |  | SelExpr_0601 | U10 call active / SETUP received | 978 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_7_1 |  | SelExpr_0601 | U11 disconnect request / clear collision | 979 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_7_2 |  | SelExpr_0601 | U11 disconnect request / RELEASE received | 980 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_7_3 |  | SelExpr_0601 | U11 disconnect request / timer T305 timeout | 981 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_7_4 |  | SelExpr_0601 | U11 disconnect request / lower layer failure | 982 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_7_5 |  | SelExpr_0601 | U11 disconnect request / unknown message received | 983 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_8_1 |  | SelExpr_0604 | U12 disconnect indication / call releasing requested by the user | 984 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_8_2 |  | SelExpr_0604 | U12 disconnect indication / RELEASE received | 985 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_8_3 |  | SelExpr_0604 | U12 disconnect indication / lower layer failure | 986 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_8_4 |  | SelExpr_0604 | U12 disconnect indication / unknown message received | 987 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_9_1 |  | SelExpr_0601 | Outgoing call / U19 release request / timer T308 timeout | 988 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_9_2 |  | SelExpr_0601 | Outgoing call / U19 release request / 2nd timer T308 timeout | 989 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_9_3 |  | SelExpr_0601 | Outgoing call / U19 release request / RELEASE received | 990 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_9_4 |  | SelExpr_0601 | Outgoing call / U19 release request / RELEASE COMPLETE received | 991 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_2_9_5 |  | SelExpr_0601 | Outgoing call / U19 release request / lower layer failure | 992 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_1_1 |  | SelExpr_0600 | Incoming call / U0 null state / SETUP received with a non supported bearer capability | 993 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_2_1 |  | SelExpr_0603 | Incoming call / U6 call present / automatic call rejection | 994 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_3_1 |  | SelExpr_0606 | Incoming call / U9 mobile terminating call confirmed / alerting or immediate connecting 11.10 Ref. ver. 4.10.0; CR. | 995 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_3_2 |  | SelExpr_0602 | Incoming call / U9 mobile terminating call confirmed / TCH assignment | 996 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_3_3 |  | SelExpr_0602 | Incoming call / U9 mobile terminating call confirmed / termination requested by the user | 997 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_3_4 |  | SelExpr_0602 | Incoming call / U9 mobile terminating call confirmed / DISCONNECT received | 998 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_3_5 |  | SelExpr_0602 | Incoming call / U9 mobile terminating call confirmed / RELEASE received | 999 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_3_6 |  | SelExpr_0602 | Incoming call / U9 mobile terminating call confirmed / lower layer failure | 1000 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_3_7 |  | SelExpr_0602 | Incoming call / U9 mobile terminating call confirmed / unknown message received | 1001 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_4_1 |  | SelExpr_0602 | Incoming call / U7 call received / call accepted | 1002 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_4_2 |  | SelExpr_0602 | Incoming call / U7 call received / termination requested by the user 11.10 Ref. ver. 4.10.0; CR. | 1003 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_4_3 |  | SelExpr_0602 | Incoming call / U7 call received / DISCONNECT received | 1004 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_4_4 |  | SelExpr_0602 | Incoming call / U7 call received / RELEASE received | 1005 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_4_5 |  | SelExpr_0602 | Incoming call / U7 call received / lower layer failure | 1006 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_4_6 |  | SelExpr_0602 | Incoming call / U7 call received / unknown message received | 1007 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_4_7 |  | SelExpr_0602 | Incoming call / U7 call received / TCH assignment | 1008 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_4_8 |  | SelExpr_0602 | Incoming call / U7 call received / RELEASE COMPLETE received | 1009 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_5_1 |  | SelExpr_0602 | Incoming call / U8 connect request / CONNECT acknowledged | 1010 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_5_2 |  | SelExpr_0602 | Incoming call / U8 connect request / timer T313 timeout | 1011 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_5_3 |  | SelExpr_0602 | Incoming call / U8 connect request / termination requested by the user | 1012 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_5_4 |  | SelExpr_0602 | Incoming call / U8 connect request / DISCONNECT received with in-band information | 1013 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_5_5 |  | SelExpr_0602 | Incoming call / U8 connect request / DISCONNECT received without in-band information | 1014 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_5_6 |  | SelExpr_0602 | Incoming call / U8 connect request / RELEASE received | 1015 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_5_7 |  | SelExpr_0602 | Incoming call / U8 connect request / lower layer failure | 1016 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_5_8 |  | SelExpr_0602 | Incoming call / U8 connect request / TCH assignment | 1017 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_3_5_9 |  | SelExpr_0602 | Incoming call / U8 connect request / unknown message received | 1018 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_4_1_1 |  | SelExpr_0604 | In-call functions / DTMF information transfer / basic procedure | 1019 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_4_2_1 |  | SelExpr_0606 | In-call functions / User notification / MS terminated | 1021 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_4_3_1 |  | SelExpr_0606 | In-call functions / Channel changes / A successful channel change in active state/ Handover and assignment command | 1022 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_4_3_2 |  | SelExpr_0606 | In-call functions / Channel changes / An unsuccessful channel change in active mode/ Handover and assignment command | 1025 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_4_4_1 |  | SelExpr_0607 | In-call functions / MS terminated in-call modification / modify when new mode is not supported | 1027 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_4_5_1 |  | SelExpr_0607 | In-call functions / MS originated in-call modification / A successful case of modifying | 1029 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_4_5_2 |  | SelExpr_0607 | In-call functions / MS originated in-call modification / modify rejected | 1030 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_4_5_3 |  | SelExpr_0607 | In-call functions / MS originated in-call modification / an abnormal case of acceptance | 1031 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_4_5_4 |  | SelExpr_0607 | In-call functions / MS originated in-call modification / an abnormal case of rejection | 1032 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_4_5_5 |  | SelExpr_0607 | In-call functions / MS originated in-call modification / timeout of timer T323 | 1033 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_4_5_6 |  | SelExpr_0607 | In-call functions / MS orignated in-call modification / A successful channel change in state mobile originating modify | 1034 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_4_5_7 |  | SelExpr_0607 | In-call functions / MS originated in-call modification / An unsuccessful channel change in state mobile originating modify | 1036 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_4_5_8 |  | SelExpr_0607 | In-call functions / MS originated in-call modification / unknown message received | 1038 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_1_4_5_9 |  | SelExpr_0607 | In-call functions / MS originated in-call modification / a release complete received | 1039 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_2_1 |  | SelExpr_0601 | Call Re-establishment / Call Present, re-establishment allowed | 1040 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_2_2 |  | SelExpr_0601 | Call Re-establishment / Call Present, re-establishment not allowed 11.10 Ref. ver. 4.10.0; CR. | 1043 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_2_3 |  | SelExpr_0601 | Call Re-establishment / Call under establishment, transmission stopped 11.10 Ref. ver. 4.10.0; CR. | 1045 |
| GSM_L3_MS_v4190/CC/ | TC_26_8_3 |  | SelExpr_0606 | User to user signalling | 1046 |
| GSM_L3_MS_v4190/StructureProc/ | TC_26_9_2 |  | SelExpr_0701 | Structured procedures / MS originated call / early assignment | 1048 |
| GSM_L3_MS_v4190/StructureProc/ | TC_26_9_3 |  | SelExpr_0701 | Structured procedures / MS originated call / late assignment | 1051 |
| GSM_L3_MS_v4190/StructureProc/ | TC_26_9_4 |  | SelExpr_0701 | Structured procedures / MS terminated call / early assignment | 1054 |
| GSM_L3_MS_v4190/StructureProc/ | TC_26_9_5 |  | SelExpr_0701 | Structured procedures / MS terminated call / late assignment | 1058 |
| GSM_L3_MS_v4190/StructureProc/ | TC_26_9_6_1_1 |  | SelExpr_0702 | Structured procedures / emergency call / idle updated / preferred channel rate | 1061 |
| GSM_L3_MS_v4190/StructureProc/ | TC_26_9_6_1_2 |  | SelExpr_0703 | Structured procedures / emergency call / idle updated, non-preferred channel rate | 1064 |
| GSM_L3_MS_v4190/StructureProc/ | TC_26_9_6_2_1 |  | SelExpr_0702 | Structured procedures / emergency call / idle, no IMSI / accept case | 1067 |
| GSM_L3_MS_v4190/StructureProc/ | TC_26_9_6_2_2 |  | SelExpr_0702 | Structured procedures / emergency call / idle, no IMSI / reject case | 1069 |
| GSM_L3_MS_v4190/EGSMSignalling/ | TC_26_10_2_1 |  | SelExpr_1001 | E-GSM signalling/ RR / measurement | 1070 |
| GSM_L3_MS_v4190/EGSMSignalling/ | TC_26_10_2_2 |  | SelExpr_1000 | E-GSM signalling/ RR / immediate assignment | 1074 |
| GSM_L3_MS_v4190/EGSMSignalling/ | TC_26_10_2_3 |  | SelExpr_1000 | E-GSM signalling/ RR / channel assignment procedure | 1076 |
| GSM_L3_MS_v4190/EGSMSignalling/ | TC_26_10_2_4_1 |  | SelExpr_1001 | E-GSM signalling/ RR / Handover / successful case | 1080 |
| GSM_L3_MS_v4190/EGSMSignalling/ | TC_26_10_2_4_2 |  | SelExpr_1001 | E-GSM signalling/ RR / Handover / layer 1 failure | 1084 |
| GSM_L3_MS_v4190/EGSMSignalling/ | TC_26_10_2_5 |  | SelExpr_1000 | E-GSM signalling/ RR / frequency redifinition | 1086 |
| GSM_L3_MS_v4190/EGSMSignalling/ | TC_26_10_3_1 |  | SelExpr_1002 | E-GSM signalling/ structured procedure / mobile originating call | 1089 |
| GSM_L3_MS_v4190/EGSMSignalling/ | TC_26_10_3_2 |  | SelExpr_1003 | E-GSM signalling/ structured procedure / emergency call | 1091 |
| GSM_L3_MS_v4190/SS/ | TC_31_2_1_1_1 |  | SelExpr_0809 | Call forwarding supplementary services / Registration - Reg. accepted | 1093 |
| GSM_L3_MS_v4190/SS/ | TC_31_2_1_1_2 |  | SelExpr_0810 | Call forwarding supplementary services / Registration - Reg. rejected | 1095 |
| GSM_L3_MS_v4190/SS/ | TC_31_2_1_2_1 |  | SelExpr_0811 | Call forwarding supplementary services / Erasure by the subscriber - Erasure Accepted | 1098 |
| GSM_L3_MS_v4190/SS/ | TC_31_2_1_2_2 |  | SelExpr_0809 | Call forwarding supplementary services / Erasure by the subscriber - Erasure rejected | 1100 |
| GSM_L3_MS_v4190/SS/ | TC_31_2_1_3 |  | SelExpr_0810 | Call forwarding supplementary services / Activation | 1102 |
| GSM_L3_MS_v4190/SS/ | TC_31_2_1_4 |  | SelExpr_0811 | Call forwarding supplementary services / Deactivation | 1104 |
| GSM_L3_MS_v4190/SS/ | TC_31_2_1_6_1 |  | SelExpr_0811 | Call forwarding supplementary services / Interrogation - Interrogation Accepted | 1106 |
| GSM_L3_MS_v4190/SS/ | TC_31_2_1_6_2 |  | SelExpr_0812 | Call forwarding supplementary services / Interrogation - Interrogation Rejected | 1108 |
| GSM_L3_MS_v4190/SS/ | TC_31_2_1_7_1_1 |  | SelExpr_0813 | Normal operation - Served mobile subscriber side / Notification during an incomming call | 1110 |
| GSM_L3_MS_v4190/SS/ | TC_31_2_1_7_1_2 |  | SelExpr_0810 | Normal operation / served mobile subscriber side / Notification during outgoing call | 1112 |
| GSM_L3_MS_v4190/SS/ | TC_31_2_1_7_2 |  | SelExpr_0810 | Normal operation / Forwarded-to mobile subscriber side | 1114 |
| GSM_L3_MS_v4190/SS/ | TC_31_6_1_1 |  | SelExpr_0802 | AOC time related charging / MS originated call | 1116 |
| GSM_L3_MS_v4190/SS/ | TC_31_6_1_2 |  | SelExpr_0802 | AOC time related charging / MS terminated call | 1118 |
| GSM_L3_MS_v4190/SS/ | TC_31_6_1_5 |  | SelExpr_0802 | Change in charging information during a call | 1120 |
| GSM_L3_MS_v4190/SS/ | TC_31_6_1_6 |  | SelExpr_0802 | Different formats of charging information | 1122 |
| GSM_L3_MS_v4190/SS/ | TC_31_6_1_7 |  | SelExpr_0819 | OC on a Call Hold call | 1124 |
| GSM_L3_MS_v4190/SS/ | TC_31_6_1_8 |  | SelExpr_0820 | AOC on a MultiParty call | 1126 |
| GSM_L3_MS_v4190/SS/ | TC_31_6_2_1 |  | SelExpr_0818 | Charge Storage - Removal of SIM during an active call | 1128 |
| GSM_L3_MS_v4190/SS/ | TC_31_6_2_2 |  | SelExpr_0821 | Charge Storage - Interruption of power supply during an active call | 1130 |
| GSM_L3_MS_v4190/SS/ | TC_31_6_2_3 |  | SelExpr_0802 | Charge Storage - MS going out of coverage during an active AOCC Call | 1131 |
| GSM_L3_MS_v4190/SS/ | TC_31_6_2_4 |  | SelExpr_0802 | Charge Storage - ACMM operation / Mobile Originating | 1132 |
| GSM_L3_MS_v4190/SS/ | TC_31_6_2_5 |  | SelExpr_0802 | Charge Storage - ACMM operation / Mobile Terminating | 1135 |
| GSM_L3_MS_v4190/SS/ | TC_31_8_1_1 |  | SelExpr_0801 | Call restriction supplementary services / Registration of a password/ Registration accepted | 1138 |
| GSM_L3_MS_v4190/SS/ | TC_31_8_1_2_1 |  | SelExpr_0801 | Rejection after invoke of the RegisterPassword operation | 1140 |
| GSM_L3_MS_v4190/SS/ | TC_31_8_1_2_2 |  | SelExpr_0801 | Rejection after password check with negative result | 1142 |
| GSM_L3_MS_v4190/SS/ | TC_31_8_1_2_3 |  | SelExpr_0801 | Rejection after new password mismatch | 1144 |
| GSM_L3_MS_v4190/SS/ | TC_31_8_3_1 |  | SelExpr_0814 | Activation accepted | 1146 |
| GSM_L3_MS_v4190/SS/ | TC_31_8_3_2_1 |  | SelExpr_0803 | Rejection after invoke of ActivateSS operation | 1149 |
| GSM_L3_MS_v4190/SS/ | TC_31_8_3_2_2 |  | SelExpr_0804 | Rejection after use of password procedure | 1151 |
| GSM_L3_MS_v4190/SS/ | TC_31_8_4_1 |  | SelExpr_0801 | Deactivation accepted | 1153 |
| GSM_L3_MS_v4190/SS/ | TC_31_8_4_2_1 |  | SelExpr_0808 | Deactivation rejected / Rejection after invoke of DeactivateSS operation | 1156 |
| GSM_L3_MS_v4190/SS/ | TC_31_8_4_2_2 |  | SelExpr_0805 | Deactivation rejected / Rejection after use of password procedure | 1158 |
| GSM_L3_MS_v4190/SS/ | TC_31_8_6_1 |  | SelExpr_0806 | Interrogation / Interrogation accepted | 1160 |
| GSM_L3_MS_v4190/SS/ | TC_31_8_6_2 |  | SelExpr_0807 | Interrogation / Interrogation rejected | 1163 |
| GSM_L3_MS_v4190/SS/ | TC_31_8_7 |  | SelExpr_0808 | Normal operation | 1165 |
| GSM_L3_MS_v4190/SS/ | TC_31_9_1_1 |  | SelExpr_0816 | mobile stattion initiated / unstructured supplementary service request accepted | 1166 |
| GSM_L3_MS_v4190/SS/ | TC_31_9_1_2 |  | SelExpr_0817 | mobile station initiated / unstructured supplementary service data request rejected | 1170 |
| GSM_L3_MS_v4190/SS/ | TC_31_9_2_1 |  | SelExpr_0817 | network initaited / unstructured supplementary service request accepted | 1175 |
| GSM_L3_MS_v4190/SS/ | TC_31_9_2_2 |  | SelExpr_0817 | network initiated / unstructured supplementary service request rejected | 1177 |
| GSM_L3_MS_v4190/SS/ | TC_31_9_2_3 |  | SelExpr_0817 | network initiated / unstructured supplementary service data notification accepted | 1178 |
| GSM_L3_MS_v4190/SS/ | TC_31_9_2_4 |  | SelExpr_0817 | network initiated / unstructured supplementary service data notification rejected | 1180 |
| GSM_L3_MS_v4190/SS/ | TC_31_10 |  | SelExpr_0815 | MMI input for USSD | 1181 |
| GSM_L3_MS_v4190/SM/ | TC_34_2_1 |  | SelExpr_0901 | Short message service / SMS point to point - SMS mobile terminated | 1183 |
| GSM_L3_MS_v4190/SM/ | TC_34_2_2 |  | SelExpr_0902 | Short message service / SMS point to point - SMS mobile originated | 1187 |
| GSM_L3_MS_v4190/SM/ | TC_34_2_3 |  | SelExpr_0903 | Short message service / SMS point to point - Test of the memory available notification | 1191 |
| GSM_L3_MS_v4190/SM/ | TC_34_2_4 |  | SelExpr_0904 | Short message service / Test of the status report capabilities and of SMS-COMMAND | 1195 |
| GSM_L3_MS_v4190/SM/ | TC_34_2_5_1 |  | SelExpr_0905 | Short message service / Test of message class 0 to 3 - Test of Class 0 Short Messages | 1198 |
| GSM_L3_MS_v4190/SM/ | TC_34_2_5_2 |  | SelExpr_0906 | Short message service / Test of message class 0 to 3 - Test of Class 1 Short Messages | 1200 |
| GSM_L3_MS_v4190/SM/ | TC_34_2_5_3 |  | SelExpr_0907 | Short message service / Test of message class 0 to 3 - Test of Class 2 Short Messages | 1202 |
| GSM_L3_MS_v4190/SM/ | TC_34_2_7 |  | SelExpr_0908 | Short message service / Test of the replace mechanism for SM type 1-7 | 1204 |
| GSM_L3_MS_v4190/SM/ | TC_34_2_8 |  | SelExpr_0909 | Short message service / Test of the reply path scheme | 1206 |
| GSM_L3_MS_v4190/SM/ | TC_34_3 |  | SelExpr_0900 | Short message service cell broadcast | 1209 |
| Detailed Comments: |  |  |  |  |  |

## Test step index

| Test Step Index |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Step Group Reference |  | Test Step Id | Description | Page Nr |
| GSM_L3_MS_v4190/management/ |  | StopAllBCCH | To stop the RF transmission of all BCCH channels in all active cells. | 1211 |
| GSM_L3_MS_v4190/management/ |  | Stopmaindcch | To stop the transmission of main dcch channel. | 1211 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | CCConfigTCH |  | 1212 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | CCConfigTCH_B |  | 1212 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | CCConfigTCH_nociph |  | 1213 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_CBMS_A_1 | To set a physical channel and map FCCH, SCH, BCCH, CCCH, SDCCH4 and CBCH onto the physical channel which represents cell A. | 1213 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_FACCHF_A_1 | To set one physical channel used as TCHF_ACCH's of cell A. | 1214 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_FACCHF_A_2 | To set one physical channel used as TCHF_ACCH's for instance 2 of cell A. | 1214 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_FACCHF_B_1 | To set one physical channel used as TCHF_ACCH's of cell B. | 1214 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_FACCHF_H_1 | To set one physical channel used as TCHF_ACCH's of cell H. | 1214 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_FACCHH_A_1 | To set one physical channel used as TCHH_ACCH's of cell A. | 1215 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_FACCHH_A_2 | To set one physical channel used as TCHH_ACCH's of cell A. | 1215 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_FACCHH_B_1 | To set one physical channel used as TCHH_ACCH's of cell B. | 1215 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_BCCH_CCCH_A_1 | To set a physical channel and map FCCH, SCH, BCCH and CCCH onto the physical channel which represents cell A. | 1216 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_BCCH_CCCH_A_2 | To set a physical channel and map the second BCCH, CCCH onto the physical channel which represents cell A. | 1216 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_BCCH_CCCH_A_3 | To set a physical channel and map the third BCCH, CCCH onto the physical channel which represents cell A. | 1216 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_BCCH_CCCH_A_4 | To set a physical channel and map the fourth BCCH, CCCH onto the physical channel which represents cell A. | 1217 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_BCCH_CCCH_B_1 | To set a physical channel and map FCCH, SCH, BCCH and CCCH onto the physical channel which represents cell B. | 1217 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_BCCH_CCCH_B_ho_1 | To set a physical channel and map FCCH, SCH, BCCH and CCCH onto the physical channel which represents cell B with controlable td and fn. | 1217 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_SDCCH4_A_1 | To set a physical channel and map FCCH, SCH, BCCH, CCCH and SDCCH4 onto the physical channel which represents cell A. | 1218 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_SDCCH4_B_1 | To set a physical channel and map FCCH, SCH, BCCH, CCCH and SDCCH4 onto the physical channel which represents cell B. | 1218 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_SDCCH4_B_ho_1 | To set a physical channel and map FCCH, SCH, BCCH, CCCH and SDCCH4 onto the physical channel which represents cell B with controlable td and fn. | 1219 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_SDCCH4_C_1 | To set a physical channel and map FCCH, SCH, BCCH, CCCH and SDCCH4 onto the physical channel which represents cell C | 1219 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_SDCCH4_D_1 | To set a physical channel and map FCCH, SCH, BCCH, CCCH and SDCCH4 onto the physical channel which represents cell D | 1220 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_SDCCH4_E_1 | To set a physical channel and map FCCH, SCH, BCCH, CCCH and SDCCH4 onto the physical channel which represents cell E | 1220 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_SDCCH4_F_1 | To set a physical channel and map FCCH, SCH, BCCH, CCCH and SDCCH4 onto the physical channel which represents cell F | 1221 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_SDCCH4_G_1 | To set a physical channel and map FCCH, SCH, BCCH, CCCH and SDCCH4 onto the physical channel which represents cell G | 1221 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_SDCCH4_H_1 | To set a physical channel and map FCCH, SCH, BCCH, CCCH and SDCCH4 onto the physical channel which represents cell H | 1222 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_SDCCH8_A_1 | To set one physical channel used as SDCCH8 channel for instance 1 of cell A. | 1222 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_SDCCH8_A_2 | To set one physical channel used as SDCCH8 channel for instance 2 of cell A. | 1223 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | Config_SDCCH8_B_1 | To set one physical channel used as SDCCH8 channel for instance 1 of cell B. | 1223 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | CombinedBCCH_A_CBMS | To set a physical channel and map FCCH, SCH, BCCH, CCCH and SDCCH4 onto the physical channel which represents cell A. | 1224 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | CombinedBCCH_A | To set a physical channel and map FCCH, SCH, BCCH, CCCH and SDCCH4 onto the physical channel which represents cell A. | 1224 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | CombinedBCCH_B | To set a physical channel and map FCCH, SCH, BCCH, CCCH and SDCCH4 onto the physical channel which represents cell B. | 1224 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | CombinedBCCH_B_ho | To set a physical channel and map FCCH, SCH, BCCH, CCCH and SDCCH4 onto the physical channel which represents cell B. | 1225 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | CombinedBCCH_C | To set a physical channel and map FCCH, SCH, BCCH, CCCH and SDCCH4 onto the physical channel which represents cell C. | 1225 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | CombinedBCCH_D | To set a physical channel and map FCCH, SCH, BCCH, CCCH and SDCCH4 onto the physical channel which represents cell D. | 1225 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | CombinedBCCH_E | To set a physical channel and map FCCH, SCH, BCCH, CCCH and SDCCH4 onto the physical channel which represents cell E. | 1226 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | CombinedBCCH_F | To set a physical channel and map FCCH, SCH, BCCH, CCCH and SDCCH4 onto the physical channel which represents cell F. | 1226 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | CombinedBCCH_G | To set a physical channel and map FCCH, SCH, BCCH, CCCH and SDCCH4 onto the physical channel which represents cell G. | 1226 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | CombinedBCCH_G_sp | To set a physical channel and map FCCH, SCH, BCCH, CCCH and SDCCH4 onto the physical channel which represents cell G. | 1227 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | CombinedBCCH_H | To set a physical channel and map FCCH, SCH, BCCH, CCCH and SDCCH4 onto the physical channel which represents cell H. | 1227 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | NonCombinedBCCH_A | To set one physical channel used as FCHH_SCH_BCCH_CCCH for cell A. | 1228 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | NonCombinedBCCH_A_2 | To set a physical channel and map the second BCCH, CCCH onto the physical channel which represents cell A. | 1228 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | NonCombinedBCCH_A_3 | To set a physical channel and map the third BCCH, CCCH onto the physical channel which represents cell A. | 1228 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | NonCombinedBCCH_A_4 | To set a physical channel and map the fourth BCCH, CCCH onto the physical channel which represents cell A. | 1229 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | NonCombinedBCCH_B | To set a physical channel and map FCCH, SCH, BCCH, CCCH onto the physical channel which represents cell B for RR testing. | 1229 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | NonCombinedBCCH_B_ho | To set a physical channel and map FCCH, SCH, BCCH, CCCH onto the physical channel which represents cell B for RR testing. | 1230 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | FullRateCh_A_1 | To set one physical channel used as TCHF_ACCH's for instance 1 of cell A. | 1230 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | FullRateCh_A_1_nociph | To set one physical channel used as TCHF_ACCH's for instance 1 of cell A. | 1231 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | FullRateCh_A_2 | To set one physical channel used as TCHF_ACCH's for instance 2 of cell A for TC_26_6_13_1. | 1231 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | FullRateCh_A_2_nociph | To set one physical channel used as TCHF_ACCH's for instance 2 of cell A for TC_26_6_13_1. | 1231 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | FullRateCh_B_1 | To set one physical channel used as TCHF_ACCH's for instance 1 of cell B. | 1232 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | FullRateCh_B_1_nociph | To set one physical channel used as TCHF_ACCH's for instance 1 of cell B. | 1232 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | HalfRateCh_A_1 | To set one physical channel used as TCHH_ACCH's for instance 1 of cell A. | 1232 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | HalfRateCh_A_1_nociph | To set one physical channel used as TCHH_ACCH's for instance 1 of cell A. | 1233 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | HalfRateCh_A_2 | To set one physical channel used as TCHH_ACCH's for instance 2 of cell A for TC_26_6_13_1. | 1233 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | HalfRateCh_A_2_nociph | To set one physical channel used as TCHH_ACCH's for instance 2 of cell A for TC_26_6_13_1. | 1234 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | HalfRateCh_B_1 | To set one physical channel used as TCHH_ACCH's for instance 1 of cell B | 1234 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | HalfRateCh_B_1_nociph | To set one physical channel used as TCHH_ACCH's for instance 1 of cell B | 1235 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | SDCCH8_A_1 | To set one physical channel used as SDCCH8 channel for instance 1 of cell A. | 1235 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | SDCCH8_A_1_nociph | To set one physical channel used as SDCCH8 channel for instance 1 of cell A. | 1235 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | SDCCH8_A_2_nociph | To set one physical channel used as hopping SDCCH8 channel for instance 2 of cell A, for TC_26_6_13_1. | 1236 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | SDCCH8_B_1 | To set one physical channel used as hopping SDCCH8 channel for instance 1 of cell B . | 1236 |
| GSM_L3_MS_v4190/management/ChConfig/ |  | SDCCH8_B_1_nociph | To set one physical channel used as hopping SDCCH8 channel for instance 1 of cell B For TC_26_6_13_5. | 1237 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | Adjust_gsmanddcs_powerlvl |  | 1237 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | AOC_CHK_FAC | Check the receive of FACILITY within one second at AOCC | 1238 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | AssCh_complete |  | 1238 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | AssCh_failure |  | 1239 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | AssCmdGen |  | 1240 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | AssCmdGen_fh |  | 1241 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | Authentication | To execute the authentication procedure. | 1241 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | CallProcGen | To generate a CALL PROCEEDING message . | 1242 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | CCAssignTCH |  | 1243 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | CCCH_group_Paging_group |  | 1244 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | CCEstablishMO_SDCCH4 |  | 1245 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | CCEstablishMO_TCH |  | 1245 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | CCEstablishMT_SDCCH4 |  | 1245 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | CCEstablishMT_TCH |  | 1246 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | CC_Est_MT_Call |  | 1246 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | CC_EstMsTermCall |  | 1247 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | CCModifyTCH |  | 1247 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | CCPage |  | 1248 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | CCstatuschk_01 | To check whether the MS under test is in the CC state 'st'. | 1248 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | CCstatuschk_02 | To check whether the MS under test is in the CC state 'st'. | 1248 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | CCstatuschk_03 | To check whether the MS under test is in the CC state 'st'. | 1249 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | CCstatuschk_05 | To check whether the MS under test is in the CC state 'st'. | 1249 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | Check_Time | The test step calls FnArith to calculate the MS return time for AOC acknowledge stored in TCV_Time. | 1249 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | CheckTIsInStateU0 |  | 1250 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | ChkMsrmntRpt |  | 1250 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | Ciphering_off | To switch off the ciphering. In presteps the ciphering shall be switched on. | 1250 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | Ciphering_on | To switch on the ciphering. | 1251 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | CMsrvcRq | To request a CM service | 1251 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | DTMFSignalling | To handle DTMF signalling initiated by the MS, which sends 'n' tones to the network. | 1251 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | EstablishFacMO |  | 1252 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | FnArith | To calcurate the difference of two frame numbers 'fn1' and 'fn'. The test step uses TCV_Cnt1, TCV_Cnt2, TCV_K and TCV_Time. | 1252 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | IdentityRequest | To execute the identity procedure depending on the given identity type. Used var's: TCV_ch, TCV_tmsi | 1253 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | ImsiAttach | If possible, SIM_Insert or PowerUp or SwitchOn, and then to execute the IMSI Attach procedure when ATT=1. This test step does not allowed to execute the switch-on even if the MS supports both the SIM insert attach and the soft switch-on attach. | 1254 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | ImsiAttachIni | If possible, SIM_Insert or PowerUp or SwitchOn, and then to execute the IMSI Attach procedure when ATT=1. This test step does not allowed to execute the switch-on even if the MS supports both the SIM insert attach and the soft switch-on attach. | 1255 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | ImsiAttachNoReaction | If possible, SIM_Insert or PowerUp or SwitchOn, and then to execute the IMSI Attach procedure when ATT=1. This test step does not allowed to execute the switch-on even if the MS supports both the SIM insert attach and the soft switch-on attach. | 1256 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | ImsiDetach | If possible, SIM_Remove or PowerDown or SwitchOff, and then to execute the IMSI Detach procedure. This test step does not allowed to execute the switch-off even if the MS supports both the SIM remove detach and the soft switch-off detach. | 1257 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | ImsiDetachNoReaction | If possible, SIM_Remove or PowerDown or SwitchOff, and then wait for no reaction from the MS. | 1258 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | InCallModi1 |  | 1258 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LUP | To execute the Location Update Procedure. The parameter lup_mi is the actual mi of MS before MM_LUP. | 1258 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LUP2 | To execute the Location Update Procedure. The parameter lup_mi is the actual mi of MS before MM_LUP. | 1259 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LUP3 | To execute the normal Location Update Procedure. No check of parameter. | 1259 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LupAndStop | To execute the initialisation of the Location Update Procedure. | 1259 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LupAuthRpt | To initialise Location Update and repeated Authentication procedures till T3210 times out. | 1260 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LUPauth1 | To execute the Location Update Procedure with authentication. Check of the parameter LAI, MSClassMark and Mobile Identity is not required and are not checked. | 1260 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LUPauth2 | To execute the Location Update Procedure with authentication. Checking of the parameter LAI, MSClassMark and Mobile Identity is required. | 1261 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LUP_imsi | To execute the Location Update Procedure only for IMSI. The parameter lup_mi is the actual mi of MS before MM_LUP. It is called by TC_26_7_4_3_1. | 1261 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LupInit | To execute the initialisation of the Location Update Procedure. | 1262 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LupInit2 | To execute the initialisation of theLocation Update Procedure. | 1262 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LupInit3 | To execute the initialisation of the Location Update Procedure. | 1263 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LupInit4 | To execute the initialisation of theLocation Update Procedure. | 1263 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LupInit5 | To execute the initialisation of the Location Update Procedure. | 1264 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LupInit6 | To execute the initialisation of the Location Update Procedure. | 1264 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LUP_tmsirealloc | To execute the Location Update Procedure. The parameter expectedlup_mi is the actual mi of MS before MM_LUP. The expectedlup_mi is not used in this teststep, it is not required to check it. The test step is called in TC_26_7_1. | 1264 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LUPper | To execute the periodic Location Update Procedure. | 1265 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LUPper2 | To execute the periodic Location Update Procedure. | 1265 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LUPperauth | To execute the periodic Location Update Procedure. | 1265 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LUPperrej | To execute the periodic Location Update Procedure. | 1266 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LUPperrej2 | To execute the Location Update Procedure, which shall be rejected. | 1266 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LUPperrej3 | To execute the Location Updating Procedure, which shall be stopped with channel release. | 1266 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LupRej | To execute the Location Update Procedure, which shall be rejected. | 1267 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_LupRej2 | To execute the Location Update Procedure, which shall be rejected. | 1267 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_PwrOrSimOff | If possible, emove SIM card or to switch off the MS or to remove power source. | 1267 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_PwrOrSimOn | If possible to insert the SIM card or to switch on the MS or to restore the power source. | 1267 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_check_ecall1 | To check, if MS execute the emergency call. | 1268 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_check_ecall2 | To check, if MS execute the emergency call with IMSI | 1269 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_no_cmservices | To check, if MS doesn't execute a MO CM connection procedure. | 1269 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | MM_no_paging | To check, if MS doesn't initiate a RR connection. The duration of the checking is set in given par. | 1270 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | NoReaction | To check whether MS doesn't initiate any RR connections. | 1270 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | RR_hocomp1 | To finish the HO-procedure in non-synchronized HO cases. | 1270 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | RR_hocomp3 | To finish the HO-procedure in synchronized HO cases. | 1271 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | RRmtcallprepare | To establish a full rate call with non hopping in cell A for GSM900 and DCS1800. IUT should be in idle updated state. | 1271 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | RcvSetupOrEsetup |  | 1271 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | SendSeqNo_chk | check sending sequence number. | 1272 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | SelectPagingCh |  | 1272 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | Set_CellChDescr | To set the cell channel description | 1273 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | SetupRcvMo2 |  | 1273 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | Switchcell |  | 1273 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | SvcSupportedChk | To check wether the Basic Service svc is supported against PICS answers. | 1274 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | Timadv_Pwrlvl_chk | To check the power level and the timing advance in L1 head. | 1275 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | TmsiReallocation | To execute the TMSI reallocation procedure. Used var's: TCV_ch | 1275 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | Varinit_fix |  | 1275 |
| GSM_L3_MS_v4190/Miscellaneous/ |  | WaitForInService | To wait until the MS enters the Idle and updated state. | 1276 |
| GSM_L3_MS_v4190/Miscellaneous/SysInfo/ |  | ChgLAC_A | To change the LAC of cell A. | 1276 |
| GSM_L3_MS_v4190/Miscellaneous/SysInfo/ |  | ChgLAC_B | To change the LAC of cell B. | 1276 |
| GSM_L3_MS_v4190/Miscellaneous/SysInfo/ |  | ChgLAI_C | To change the LAI of cell C to HPLMN. | 1277 |
| GSM_L3_MS_v4190/Miscellaneous/SysInfo/ |  | SysInfoSending_cbms | To send system information messages for the CBMS tests. | 1278 |
| GSM_L3_MS_v4190/Miscellaneous/SysInfo/ |  | SysInfoSending_fh | To send system information messages for the L3 tests. The following parameters specified by input parameters. | 1279 |
| GSM_L3_MS_v4190/Miscellaneous/SysInfo/ |  | SysInfoSending_nfh | To send system information messages with default parameters defined for L3 tests for which no other special parameters indicated. | 1280 |
| GSM_L3_MS_v4190/Miscellaneous/SysInfo/ |  | SysInfoSending_nfhCb | To send system information messages with default parameters defined for L3 tests except that the cell is barred. | 1282 |
| GSM_L3_MS_v4190/Miscellaneous/SysInfo/ |  | SysInfoSending_e | Sending of Systeminformation for cell A in EGSM test case TC_26_10_2_2. | 1283 |
| GSM_L3_MS_v4190/Miscellaneous/SysInfo/ |  | SysInfo_SacchSending | To send SYSTEM INFORMATION 5 and 6 messages defined by parameters 'sysinfo5_pdu' and 'sysinfo6_pdu' in the parametrised 'ch' channel. | 1284 |
| GSM_L3_MS_v4190/Miscellaneous/SysInfo/ |  | SysInfo_5bisSending | To send SYSTEM INFORMATION 5bis message defined by parameters 'sysinfo5bis_pdu' in the parametrised 'ch' channel. | 1284 |
| GSM_L3_MS_v4190/Miscellaneous/SysInfo/ |  | SetNECI | To set the NECI =1. | 1285 |
| GSM_L3_MS_v4190/Miscellaneous/SysInfo/ |  | SetATT | To set the ATT flag to "MS's in the cell should apply IMSI attach and detach procedure" | 1286 |
| GSM_L3_MS_v4190/OperatorOP/ |  | InitCall | To initiate a call of the basic service 'srv'. | 1287 |
| GSM_L3_MS_v4190/OperatorOP/ |  | InitNonCallSupp | To attempt a non call related supplementary service at the MS under test. | 1287 |
| GSM_L3_MS_v4190/OperatorOP/ |  | AtmpShortMsg | To attempt an MO short message service transaction at the MS under test. | 1287 |
| GSM_L3_MS_v4190/OperatorOP/ |  | CheckUssdStringDisplayed | To check whether the correct USSD String is displayed on the MS | 1287 |
| GSM_L3_MS_v4190/OperatorOP/ |  | PLMNsCHK | To check whether the MS presents a list of available PLMNs. | 1287 |
| GSM_L3_MS_v4190/OperatorOP/ |  | RFtransCHK | To check whether the MS transmits any radio signal. | 1288 |
| GSM_L3_MS_v4190/Postambles/ |  | PostLinkRelEnd | To release the RR connection and bring the MS back to Idle state. This teststep decides the final verdict and therefor it shall be used only at the end of testcases. | 1288 |
| GSM_L3_MS_v4190/Postambles/ |  | PostMainLinkRel | To release the main signalling link 'ch', and bring the MS back to Idle state. | 1288 |
| GSM_L3_MS_v4190/Postambles/ |  | ChanRel | To release the RR connection on the channel TCV_ch and bring the MS back to Idle state. | 1288 |
| GSM_L3_MS_v4190/Postambles/ |  | ChanRel_P | To release the RR connection on the channel TCV_chmaindcch and bring the MS back to Idle state. This teststep decides the verdict (P). | 1289 |
| GSM_L3_MS_v4190/Postambles/ |  | ChanRel_end | To release the RR connection and bring the MS back to Idle state. This teststep decides the final verdict and therefor it shall be used only at the end of testcases. | 1289 |
| GSM_L3_MS_v4190/Postambles/ |  | RmvForbiddenPlmn | To remove a forbidden PLMN (C_PLMN_2) of cell B in the list on SIM. | 1289 |
| GSM_L3_MS_v4190/Preambles/ |  | BasicServiceMO | 1. This test step generates an appropriate MO SETUP message according to the IXIT parameters of Bearer services/Teleservices for an MO call. 2. The detailed algorithms for Bcap derivation for each Bearer service/Teleservice are described in test steps BS2xMO, BS3xMO, BS4xMO, BS5xMO, BS61or81MO, TS61MO and TS62MO. | 1290 |
| GSM_L3_MS_v4190/Preambles/ |  | BasicServiceMOorTelephony | To get a MO SETUP message with right BC IE. | 1292 |
| GSM_L3_MS_v4190/Preambles/ |  | BasicServiceMT | 1. This test step generates an appropriate MT SETUP message according to the IXIT parameters of Bearer services/Teleservices for an MT call. If possible the test step generates a 2nd MT SETUP message for TC_11_1_1. 2. The Bearer Capability in the 2nd SETUP message is chosen in such a way that as many parameters as possible are different from the Bcap in the first MT SETUP message. The detailed algorithms for Bcap derivation for each Bearer service/Teleservice are described in test steps BS2xMT, BS3xMT, BS61or81MT, TS61MT and TS62MT. | 1293 |
| GSM_L3_MS_v4190/Preambles/ |  | BasicServiceMTorTelephony | To get a MT SETUP message with right BC IE. | 1294 |
| GSM_L3_MS_v4190/Preambles/ |  | BS2xMO | MO SETUP message with right Bcap for BS2x service. | 1295 |
| GSM_L3_MS_v4190/Preambles/ |  | BS2xMT | MT SETUP message with right Bcap for BS2x service. | 1298 |
| GSM_L3_MS_v4190/Preambles/ |  | BS3xMO | MO SETUP message with right Bcap for BS3x service. | 1301 |
| GSM_L3_MS_v4190/Preambles/ |  | BS3xMT | MT SETUP message with right Bcap for BS3x service. | 1304 |
| GSM_L3_MS_v4190/Preambles/ |  | BS4xMO | MO SETUP message with right Bcap for BS4x service. | 1309 |
| GSM_L3_MS_v4190/Preambles/ |  | BS5xMO | MO SETUP message with right Bcap for BS5x service. | 1311 |
| GSM_L3_MS_v4190/Preambles/ |  | BS61or81MO | To get a MO SETUP message with right BC IE for BS61 or BS81 service. | 1312 |
| GSM_L3_MS_v4190/Preambles/ |  | BS61or81MT | To get a MT SETUP message with right BC IE for BS61 or BS81 service. | 1316 |
| GSM_L3_MS_v4190/Preambles/ |  | CallCfmGen | generate a CALL COMFIRMED for CALL COMFIRMED message verifying | 1320 |
| GSM_L3_MS_v4190/Preambles/ |  | Chmod | To assign correct values to the test case variables TCV_ChMod and TCV_ChModb. | 1325 |
| GSM_L3_MS_v4190/Preambles/ |  | Est_MO_Call | To establish a MO call in any cell. | 1326 |
| GSM_L3_MS_v4190/Preambles/ |  | Est_MO_Call_init | To initiate a mobile originating call for the supported bearer capability. Only in HO cases. | 1327 |
| GSM_L3_MS_v4190/Preambles/ |  | Est_MT_Call_FH | To establish a call with FH (in CELL A or B). | 1328 |
| GSM_L3_MS_v4190/Preambles/ |  | Est_MT_CallNonFH | To establish a call with non hopping. | 1329 |
| GSM_L3_MS_v4190/Preambles/ |  | IdleUpdated | This test step is called at the preamble of each test case. | 1330 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterIdleState_r02 | To broadcast SYSTEM INFORMATION messages 2, 3, 4, 5 and 6 with some options in cell A, and wait for the SUT (MS) entering the Idle updated state. 2 parameters: max retransmission, and tx integer can be set/assigned and legal combination of CCCH-CONF, BS-AG-BLKS-RES, BS-PA-MFRMS are specified by parameter 'ccd'. | 1338 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU01_21 | To bring the MS into CC state U0.1 by procedure in table 26.8.1.2/1. This is used in CC testing. | 1339 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU1_21 | To bring the MS into CC state U1 by procedure in table 26.8.1.2/1. This is used in CC testing. | 1340 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU1_22 | To bring the MS into CC state U1 by procedure in table 26.8.1.2/2. This is used in CC testing. | 1340 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU1_22Timer | To bring the MS into CC state U1 by procedure in table 26.8.1.2/2. This is used in CC testing. | 1341 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU1_24 | To bring the MS into CC state U1 by procedure in table 26.8.1.2/4. This is used in CC testing. | 1341 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU1 | To establish a mobile originating call and put the MS under test in the CC state U1. | 1342 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU3 | To establish a mobile originating call and put the MS under test in the CC state U3. | 1342 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU3_21 | To bring the MS into CC state U3 by procedure in table 26.8.1.2/1. This is used in CC testing. | 1343 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU3_22 | To bring the MS into CC state U3 by procedure in table 26.8.1.2/2. This is used in CC testing. | 1343 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU3_23 | To bring the MS into CC state U3 by procedure in table 26.8.1.2/3. This is used in CC testing. | 1343 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU3_24 | To bring the MS into CC state U3 by procedure in table 26.8.1.2/4. This is used in CC testing. | 1344 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU4_21 | To bring the MS into CC state U4 by procedure in table 26.8.1.2/1. This is used in CC testing. | 1344 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU4_22 | To bring the MS into CC state U4 by procedure in table 26.8.1.2/2. This is used in CC testing. | 1344 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU4_23 | To bring the MS into CC state U4 by procedure in table 26.8.1.2/3. This is used in CC testing. | 1345 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU4_24 | To bring the MS into CC state U4 by procedure in table 26.8.1.2/4. This is used in CC testing. | 1345 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU6_32 | To bring the MS into CC state U6 by procedure in table 26.8.1.3/2. This is used in CC testing. | 1345 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU7_31 | To bring the MS into CC state U7 by procedure in table 26.8.1.3/1. This is used in CC testing. | 1346 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU7_32 | To bring the MS into CC state U7 by procedure in table 26.8.1.3/2. This is used in CC testing. | 1346 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU7_33 | To bring the MS into CC state U7 by procedure in table 26.8.1.3/3. This is used in CC testing. | 1346 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU8_31 | To bring the MS into CC state U8 by procedure in table 26.8.1.3/1. This is used in CC testing. | 1347 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU8_32 | To bring the MS into CC state U8 by procedure in table 26.8.1.3/2. This is used in CC testing. | 1347 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU8_33 | To bring the MS into CC state U8 by procedure in table 26.8.1.3/3. This is used in CC testing. | 1348 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU8_34_SS | Entering U8, non hopping, no ciphering. | 1348 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU9_31 |  | 1349 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU9_32 | To bring the MS into CC state U9 by procedure in table 26.8.1.3/2. This is used in CC testing. | 1349 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU9_33 | To bring the MS into CC state U9 by procedure in table 26.8.1.3/3. This is used in CC testing. | 1349 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU9_34 | To bring the MS into CC state U9 by procedure in table 26.8.1.3/4. This is used in CC testing. | 1350 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU10 | To establish a mobile terminating call for the supported bearer capability and put the MS under test in the CC state U10. The supported bearer capability is specified in the input partameter setup. With early assigment, SETUP message does not contain SIGNAL information element. | 1351 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU10_late | To establish a mobile terminating call for the supported bearer capability and put the MS under test in the CC state U10. The supported bearer capability is specified in the input partameter setup. Late assigment only. The SETUP message shall contain a SIGNAL information element. | 1352 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU10_r01 | To establish a mobile terminating call for the supported bearer capability and put the MS in the CC state U10 with late assignment. | 1353 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU10_21 | To bring the MS into CC state U10 by procedure in table 26.8.1.2/1. This is used in CC testing. | 1354 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU10_22 | To bring the MS into CC state U10 by procedure in table 26.8.1.2/2. This is used in CC testing. | 1354 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU11_23 | To bring the MS into CC state U11 by procedure in table 26.8.1.2/3. This is used in CC testing. | 1355 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU11_23Timer | To bring the MS into CC state U11 by procedure in table 26.8.1.2/3. This is used in CC testing. | 1355 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU11_24 | To bring the MS into CC state U11 by procedure in table 26.8.1.2/4. This is used in CC testing. | 1356 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU12_21 | To bring the MS into CC state U12 by procedure in table 26.8.1.2/1. This is used in CC testing. | 1356 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU12_22 | To bring the MS into CC state U12 by procedure in table 26.8.1.2/2. This is used in CC testing. | 1356 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU12_23 | To bring the MS into CC state U12 by procedure in table 26.8.1.2/3. This is used in CC testing. | 1357 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU19_21 | To bring the MS into CC state U19 by procedure in table 26.8.1.2/1. This is used in CC testing. | 1357 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU19_24 | To bring the MS into CC state U19 by procedure in table 26.8.1.2/4. This is used in CC testing. | 1358 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEnterCCstateU19_24Timer | To bring the MS into CC state U19 by procedure in table 26.8.1.2/4. This is used in CC testing. | 1358 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEstRRConn | To establish a RR connection on TSPX_SDCCH4SubDef | 1359 |
| GSM_L3_MS_v4190/Preambles/ |  | PreEstRRC_MM | To establish a RR connection for MM testcases. | 1359 |
| GSM_L3_MS_v4190/Preambles/ |  | PreModifySetup | To setup dual mode call and initiate MO incall modification. | 1360 |
| GSM_L3_MS_v4190/Preambles/ |  | SpeechService | To selecte a speech service. | 1360 |
| GSM_L3_MS_v4190/Preambles/ |  | StartCellA | To broadcast SYSTEM INFORMATION messages 2, 3, 4, 5 and 6 with default parameters in cell A, frequency hopping. | 1361 |
| GSM_L3_MS_v4190/Preambles/ |  | StartCellA_1 | To start cell A without hopping. | 1361 |
| GSM_L3_MS_v4190/Preambles/ |  | StartCellB | To broadcast SYSTEM INFORMATION messages 2, 3, 4, 5 and 6 with default parameters in cell B. | 1362 |
| GSM_L3_MS_v4190/Preambles/ |  | StartCellB_1 | To start cell B with default parameters. | 1363 |
| GSM_L3_MS_v4190/Preambles/ |  | StartCellB_egsm | To broadcast SYSTEM INFORMATION messages 2, 3, 4, 5 and 6 with default parameters in cell B. | 1364 |
| GSM_L3_MS_v4190/Preambles/ |  | StartCellB_ho | To broadcast SYSTEM INFORMATION messages 2, 3, 4, 5 and 6 with default parameters in cell B. Cell B has a frame number difference 'fn' and the BCCH of cell B has a delay of 'td' bits to the BCCH of cell A. | 1365 |
| GSM_L3_MS_v4190/Preambles/ |  | StartCellC | To broadcast SYSTEM INFORMATION messages 2, 3, 4, 5 and 6 with default parameters in cell C. | 1366 |
| GSM_L3_MS_v4190/Preambles/ |  | Start_2cells |  | 1366 |
| GSM_L3_MS_v4190/Preambles/ |  | StartMultiCells_01 | To broadcast SYSTEM INFORMATION messages 2, 3, 4, 5 and 6 in multiple cells for idle mode testing. | 1367 |
| GSM_L3_MS_v4190/Preambles/ |  | StartMultiCells_02 | To broadcast SYSTEM INFORMATION messages 1, 2, 3, 4, 5 and 6 in multiple cells for measurement testing. Neighbour cells description for cell S1 is a formal parameter. | 1370 |
| GSM_L3_MS_v4190/Preambles/ |  | StartMultiCells_02e | To broadcast SYSTEM INFORMATION messages 1, 2, 3, 4, 5 and 6 in multiple cells for measurement testing. Neighbour cells description for cell S1 is a formal parameter. | 1372 |
| GSM_L3_MS_v4190/Preambles/ |  | StartMultiCells_03 | To broadcast SYSTEM INFORMATION messages 1, 2, 3, 4, 5 and 6 in multiple cells for measurement testing. Neighbour cells description for cell S1 is a formal parameter. | 1374 |
| GSM_L3_MS_v4190/Preambles/ |  | TS61MO | MO SETUP message with right Bcap for TS61 service. | 1376 |
| GSM_L3_MS_v4190/Preambles/ |  | TS62MO | MO SETUP message with right Bcap for TS62 service. | 1378 |
| GSM_L3_MS_v4190/Preambles/ |  | TS61MT | MT SETUP message with right Bcap for TS61 service. | 1380 |
| GSM_L3_MS_v4190/Preambles/ |  | TS62MT | MT SETUP message with right Bcap for TS62 service. | 1382 |
| Detailed Comments: |  |  |  |  |

## Default index

| Default Index |  |  |  |  |
| --- | --- | --- | --- | --- |
| Default Group Reference |  | Default Id | Description | Page Nr |
| GSM_L3_MS_v4190/ |  | OtherEvents | To match unexpected events and sign final verdict for preambles. | 1384 |
| GSM_L3_MS_v4190/ |  | OtherEventsFail | To match unexpected events and fail the test case. | 1385 |
| GSM_L3_MS_v4190/ |  | OtherEventsFail_01 | To match irrelevant CHANNEL REQUEST msg and return or match other unexpected events and fail the test case. | 1386 |
| GSM_L3_MS_v4190/ |  | OtherEventsFail_02 | To match unexpected events and fail the test case but ignore channel request messages that are sent before the lower tester has sent (and the MS received) the Immediate Assigment message. | 1387 |
| GSM_L3_MS_v4190/ |  | OtherEvents_01 | To match irrelevant messages and return | 1388 |
| GSM_L3_MS_v4190/ |  | OtherEvents_02 | To match unexpected events and sign final verdict for preambles but ignore channel request messages that are sent before the lower tester has sent (and the MS received) the Immediate Assigment message. | 1389 |
| GSM_L3_MS_v4190/ |  | RcvHdOvAcc | To match any HANDOVER ACCESS then return to calling tree. | 1390 |
| Detailed Comments: |  |  |  |  |