# Declarations Part

## Test suite type definitions

### Simple type definitions

| Simple Type Definitions |  |  |  |
| --- | --- | --- | --- |
| Type Name |  | Type Definition | Comments |
| B_1 |  | BITSTRING [1] | Generic type for 1 bit value |
| B_2 |  | BITSTRING [2] | Generic type for 2 bits value |
| B_3 |  | BITSTRING [3] | Generic type for 3 bits value |
| B_4 |  | BITSTRING [4] | Generic type for 4 bits value |
| B_5 |  | BITSTRING [5] | Generic type for 5 bits value |
| B_6 |  | BITSTRING [6] | Generic type for 6 bits value |
| B_8 |  | BITSTRING [8] | Generic type for 8 bits value |
| BCCHFRQ |  | BITSTRING [5] | position of a bcch carrier in the bcch channel list |
| BCC |  | BITSTRING [3] | BS colour code |
| BCDN |  | OCTETSTRING [1..10] | BCD numbers, GSM 04.08, 10.5.4.7, octet 4-13 |
| BSIC |  | BITSTRING [6] | base station identity code |
| CCCH_CON |  | BITSTRING[3] | number of physical channels for ccch GSM 04.08, 10.5.2.11 |
| CCSTATE |  | INTEGER(1,3,4,6,7,8,9,10,11,12,19,26,27) |  |
| CellID |  | IA5String | Cell identifier |
| CHANNEL |  | BITSTRING [2] | needed channel type |
| CHMOD_VAL |  | BITSTRING[8] | value for channel mode GSM 04.08, 10.5.2.6, 10.5.2.7 |
| CH_TDMA |  | BITSTRING[5] | channel type and TDMA offset GSM 04.08, 10.5.2.5 |
| CI |  | OCTETSTRING [2] | cell identity, GSM 04.08, 10.5.1.1 |
| CKSN |  | BITSTRING[3] | ciphering key sequence number(only key sequence) GSM 04.08, 10.5.1.2 |
| CLRSUP |  | BITSTRING('10100001'B) | CLIR suppression GSM 04.08, 10.5.4.11a |
| CLRINV |  | BITSTRING('10100010'B) | CLIR invocation GSM 04.08, 10.5.4.11b |
| CMSVTYPE |  | BITSTRING [4] | CM service type |
| CP_CAU |  | OCTETSTRING [1] | cp cause GSM 04.11, 8.1.4.2 |
| CPHALG |  | BITSTRING[3] | Ciphering algorithm identifier GSM 04.08, 10.5.2.9 |
| CS |  | BITSTRING [2] | coding standard |
| EstMode |  | OCTETSTRING [1] | establish mode |
| EXTB |  | BITSTRING [1] | extension bit |
| FCS |  | HEXSTRING [2] | TP failure cause |
| FLWCNTL |  | INTEGER(0, 1, 2) | type of flow control. 0--outband flow control; 1--inband flow control; 2-- no flow control |
| HORF |  | BITSTRING [8] | handover reference |
| HSN |  | BITSTRING[6] | Hopping sequence number e.g.GSM 04.08 10.5.2.5 |
| IDTYPE |  | B_4 | identity type |
| IEI_4 |  | BITSTRING [4] | information element identifier, type 1 |
| IEI_8 |  | BITSTRING [8] | information element identifier, type 2-4 |
| L2FMTYPE |  | INTEGER(1..3) | L2 frame type |
| LENGTH |  | OCTETSTRING [1] | IE length or L2 pseudo length. The L2 pseudo length is composed of a L2 pseudo length value, which is 6 bits long, and of 2 additional bits. See GSM 04.08 section 10.5.2.19 |
| LOGICCH |  | IA5String |  |
| LOGCH |  | IA5String |  |
| MAC |  | OCTETSTRING[3] | mac, e.g. GSM 04.08 10.5.2.21 |
| MAIO |  | BITSTRING[6] | MAIO,GSM 04.08 10.5.2.5 |
| MAXTXPOW |  | INTEGER(0..31) | Max Tx Power Level GSM 04.08, 10.5.2.4 |
| MR |  | OCTETSTRING [1] | SMS RP or TP message reference |
| MT |  | BITSTRING[8] | message type, structure of it see MT0 |
| MTI |  | BITSTRING [3] | SMS RP message type indicator |
| NCC |  | BITSTRING [3] | PLMN colour code |
| NCCP |  | OCTETSTRING [1] | ncc permitted |
| NTI |  | BITSTRING('10000000'B, '10000001'B, '10000010'B) | notification indicator GSM 04.08, 10.5.4.20 |
| PD |  | B_4 | Protocol discriminator |
| PGG |  | BITSTRING [8] | the paging group of an MS to be paged in binary presentation |
| RATE |  | IA5String("F", "H") |  |
| RAND |  | BITSTRING [128] | random number |
| RCSD |  | BITSTRING [8] | reverse call setup direction GSM 04.08, 10.5.4.22a |
| RelMode |  | OCTETSTRING [1] | release mode |
| REJCAU |  | OCTETSTRING [1] | reject cause GSM 04.08, 10.5.3.6 |
| RPI |  | BITSTRING [8] | repeat indicator |
| RRCAU |  | BITSTRING [8] | RR cause GSM 04.08, 10.5.2.31 |
| RXLEV |  | BITSTRING [6] | received signal strength |
| SAPID |  | OCTETSTRING [1] | sap identifier |
| SENDINGMODE |  | INTEGER(1..4) | the mode of sending two consecutive messages containing paging mode IE |
| SERVICES |  | IA5String("C_Telephony", "C_EmgCall", "C_AltSpchFax", "C_AutoFax", "C_Async300", "C_Async1200", "C_Async120075", "C_Async2400", "C_Async4800", "C_Async9600", "C_Sync1200", "C_Sync2400", "C_Sync4800", "C_Sync9600", "C_PAD300", "C_PAD1200", "C_PAD120075", "C_PAD2400", "C_PAD4800", "C_PAD9600", "C_Packet2400", "C_Packet4800", "C_Packet9600", "C_AltSpchData", "C_SpchData") |  |
| SHOCT |  | B_4 | spare half octet for type 1 information element GSM 04.08, 10.5.1.8 |
| SKI |  | B_4 | Skip indicator |
| SN |  | BITSTRING [3] | all possible slot number, GSM 04.08 10.5.2.5 |
| SPB |  | B_1 | spare bit |
| SP2B |  | B_2 | 2 spare bits |
| SP3B |  | B_3 | 3 spare bits |
| SP5B |  | B_5 | 5 spare bits |
| SP6B |  | B_6 | 6 spare bits |
| SRES |  | OCTETSTRING [4] | authentication response signature |
| TPCD |  | OCTETSTRING [1..157] | TP command data |
| TPSCTS |  | HEXSTRING [14] | TP service centre time stamp |
| TPUD |  | OCTETSTRING [1..140] | TP user data |
| TSC |  | B_3 | Training sequence code,GSM 04.08 10.5.2.5 |
| T1 |  | BITSTRING [11] |  |
| T1_ |  | B_5 | T1' |
| T2 |  | B_5 |  |
| T3 |  | BITSTRING [6] |  |
| T3_ |  | B_3 | T3' |
| TI_V |  | B_3 | TI value |
| TMSI_V |  | OCTETSTRING[4] | TMSI value is octetstring of length 4 |
| TZONES |  | INTEGER(-79..79) | time zones in 15 minute steps |
| WI |  | HEXSTRING [2] | wait indication, unit in second |
| Detailed Comments: |  |  |  |

### Structured Type definitions

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | ACST |  |  |
| Comments: | Auxiliary states (CC information element) GSM 04.08, 10.5.4.4 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| extb |  | EXTB |  |
| sp3b |  | SP3B |  |
| has |  | BITSTRING [2] |  |
| mpas |  | BITSTRING [2] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | BCAP |  |  |
| Comments: | Bearer capability (CC information element) GSM 04.08, 10.5.4.5 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| extb3 |  | EXTB |  |
| rchr |  | BITSTRING [2] |  |
| cs |  | BITSTRING [1] |  |
| tm |  | BITSTRING [1] |  |
| itc |  | BITSTRING [3] |  |
| extb4 |  | EXTB |  |
| spb |  | SPB |  |
| strc |  | BITSTRING [2] |  |
| dplxm |  | BITSTRING [1] |  |
| config |  | BITSTRING [1] |  |
| nirr |  | BITSTRING [1] |  |
| est |  | BITSTRING [1] |  |
| extb5 |  | EXTB |  |
| accid |  | BITSTRING [2] |  |
| ra |  | BITSTRING [2] |  |
| sacp |  | BITSTRING [3] |  |
| extb6 |  | EXTB |  |
| l1id |  | BITSTRING [2] |  |
| uil1 |  | BITSTRING [4] |  |
| sb |  | BITSTRING [1] |  |
| extb6a |  | EXTB |  |
| nsb |  | BITSTRING [1] |  |
| nb |  | BITSTRING [1] |  |
| ndb |  | BITSTRING [1] |  |
| ur |  | BITSTRING [4] |  |
| extb6b |  | EXTB |  |
| ir |  | BITSTRING [2] |  |
| nictx |  | BITSTRING [1] |  |
| nicrx |  | BITSTRING [1] |  |
| pi |  | BITSTRING [3] |  |
| extb6c |  | EXTB |  |
| ce |  | BITSTRING [2] |  |
| modemt |  | BITSTRING [5] |  |
| extb7 |  | EXTB |  |
| l2id |  | BITSTRING [2] |  |
| uil2 |  | BITSTRING [5] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | BLOCKTYPE |  |  |
| Comments: | Block type, GSM 04.12, 3.3.1 |  |  |
| Element Name |  | Type Definition | Comments |
| spare1 |  | SPB |  |
| lpd |  | BITSTRING[2] |  |
| lb |  | BITSTRING[1] |  |
| sequence_number |  | BITSTRING[4] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CAU |  |  |
| Comments: | Cause (CC information element) GSM 04.08, 10.5.4.11 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| extb3 |  | EXTB |  |
| cs |  | CS |  |
| spb |  | SPB |  |
| location |  | BITSTRING [4] |  |
| extb3a |  | EXTB |  |
| rec |  | BITSTRING [7] |  |
| extb4 |  | EXTB |  |
| cau_class |  | BITSTRING [3] |  |
| cau_va |  | BITSTRING [4] |  |
| cau_di |  | OCTETSTRING [1..28] |  |
| Detailed Comments: | &COMMON_U06 |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CCCAP |  |  |
| Comments: | Call Control Capabilities GSM 04.08, 10.5.4.5a |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| spr |  | BITSTRING [7] |  |
| dtmf |  | BITSTRING [1] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CCD |  |  |
| Comments: | Control channel description GSM 04.08, 10.5.2.11 |  |  |
| Element Name |  | Type Definition | Comments |
| spr1 |  | BITSTRING [1] |  |
| att |  | BITSTRING [1] |  |
| babr |  | BITSTRING [3] |  |
| ccch_con |  | BITSTRING [3] |  |
| spr2 |  | BITSTRING [5] |  |
| bpm |  | BITSTRING [3] |  |
| t3212 |  | OCTETSTRING [1] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CCHD |  |  |
| Comments: | Cell channel description GSM 04.08, 10.5.2.1 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| rfl |  | OCTETSTRING [16] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CD |  |  |
| Comments: | Cell description GSM 04.08, 10.5.2.2 |  |  |
| Element Name |  | Type Definition | Comments |
| bcch_arfcn_h |  | BITSTRING [2] |  |
| ncc |  | NCC |  |
| bcc |  | BCC |  |
| bcch_arfcn_l |  | BITSTRING [8] |  |
| Detailed Comments: | The info element is two octets long. |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CDPN |  |  |
| Comments: | Called party BCD number (CC information element) GSM 04.08, 10.5.4.7 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| tonnpi |  | TON_NPI |  |
| digits |  | BCDN |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CDPS |  |  |
| Comments: | Called party subaddress (CC information element) GSM 04.08, 10.5.4.8 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| subad |  | SUBAD |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CE |  |  |
| Comments: | connection element |  |  |
| Element Name |  | Type Definition | Comments |
| TorBothTorBothNT |  | B_2 |  |
| NTorBothTorBothNT |  | B_2 |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CGPN |  |  |
| Comments: | Calling party BCD number (CC information element) GSM 04.08, 10.5.4.9 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| tonnpi |  | TON_NPI |  |
| pisi |  | PI_SI |  |
| digits |  | BCDN |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CGPS |  |  |
| Comments: | Calling party subaddress (CC information element) GSM 04.08, 10.5.4.10 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| subad |  | SUBAD |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CHD |  |  |
| Comments: | Channel description GSM 04.08, 10.5.2.5 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| cht_schn |  | BITSTRING [5] |  |
| tn |  | BITSTRING [3] |  |
| tsc |  | BITSTRING [3] |  |
| hch |  | BITSTRING [1] |  |
| maio |  | BITSTRING [6] |  |
| hsn |  | BITSTRING [6] |  |
| spr |  | BITSTRING [2] |  |
| arfcn |  | BITSTRING [10] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CHMOD |  |  |
| Comments: | Channel mode, channel mode2 GSM 04.08, 10.5.2.6, 10.5.2.7 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| mode |  | B_8 |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CHNEED |  |  |
| Comments: | Channels needed GSM 04.08, 10.5.2.8 |  |  |
| Element Name |  | Type Definition | Comments |
| ch2 |  | CHANNEL |  |
| ch1 |  | CHANNEL |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CNN |  |  |
| Comments: | Connected number (CC information element) GSM 04.08, 10.5.4.13 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| tonnpi |  | TON_NPI |  |
| pisi |  | PI_SI |  |
| digits |  | BCDN |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CNS |  |  |
| Comments: | Connected subaddress (CC information element) GSM 04.08, 10.5.4.14 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| subad |  | SUBAD |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CO |  |  |
| Comments: | Cell options (BCCH / SACCH) GSM 04.08, 10.5.2.3 |  |  |
| Element Name |  | Type Definition | Comments |
| sprb |  | BITSTRING [1] |  |
| pwrc |  | BITSTRING [1] |  |
| dtx |  | BITSTRING [2] |  |
| rlt |  | BITSTRING [4] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | Component_T |  |  |
| Comments: | Component for sending (downlink) GSM 04.80, 3.6 |  |  |
| Element Name |  | Type Definition | Comments |
| comp_part1 |  | OCTETSTRING |  |
| invokeId |  | OCTETSTRING [1] |  |
| comp_part2 |  | OCTETSTRING |  |
| comp_part3 |  | OCTETSTRING |  |
| comp_part4 |  | OCTETSTRING |  |
| comp_part5 |  | OCTETSTRING |  |
| comp_part6 |  | OCTETSTRING |  |
| comp_part7 |  | OCTETSTRING |  |
| comp_part8 |  | OCTETSTRING |  |
| comp_part9 |  | OCTETSTRING |  |
| comp_part10 |  | OCTETSTRING |  |
| comp_part11 |  | OCTETSTRING |  |
| comp_part12 |  | OCTETSTRING |  |
| Detailed Comments: | This component is defined for the use in the downlink (from the test system to the IUT) direction. |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CPDATA |  |  |
| Comments: | CP-User data element GSM 04.11, 8.1.4.1 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| rpack |  | RPACK |  |
| rpdata |  | RPDATA |  |
| rperr |  | RPERR |  |
| rpsmma |  | RPSMMA |  |
| Detailed Comments: | One of the 4 types of rpdu is contained in the CPDATA. |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CPHMS |  |  |
| Comments: | Cipher mode setting GSM 04.08, 10.5.2.9 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_4 |  |
| algid |  | BITSTRING [3] |  |
| sc |  | BITSTRING [1] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CPHKSN |  |  |
| Comments: | Ciphering key sequence number GSM 04.08, 10.5.1.2 |  |  |
| Element Name |  | Type Definition | Comments |
| sprb |  | BITSTRING [1] |  |
| ks |  | BITSTRING [3] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CPH_RES |  |  |
| Comments: | Cipher response GSM 04.08, 10.5.2.10 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_4 |  |
| sprb |  | BITSTRING [3] |  |
| cr |  | BITSTRING [1] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CSP |  |  |
| Comments: | Cell selection parameters GSM 04.08, 10.5.2.4 |  |  |
| Element Name |  | Type Definition | Comments |
| crh |  | BITSTRING [3] |  |
| mtmc |  | BITSTRING [5] |  |
| acs |  | BITSTRING [1] |  |
| neci |  | BITSTRING [1] |  |
| ram |  | BITSTRING [6] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | CST |  |  |
| Comments: | Call status (CC information element) GSM 04.08, 10.5.4.6 |  |  |
| Element Name |  | Type Definition | Comments |
| cs |  | CS |  |
| csv |  | BITSTRING [6] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | FIE |  |  |
| Comments: | Facility information element GSM 04.80, 3.6 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| components_1 |  | Components |  |
| components_t |  | Component_T |  |
| Detailed Comments: | When sending normally only one component is sent, but when receiving any number of components can be received even though normally we are only interested in one component during the testing. |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | FN |  |  |
| Comments: | The time in reference to the frame numbering corresponding to the absolute frame number modulo 42432. |  |  |
| Element Name |  | Type Definition | Comments |
| t1_ |  | T1_ |  |
| t3 |  | T3 |  |
| t2 |  | T2 |  |
| Detailed Comments: | In STRT and RQR |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | FRQCHS |  |  |
| Comments: | Frequency channel sequence GSM 04.08, 10.5.2.12 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| larfcn |  | OCTETSTRING [1] |  |
| incs |  | OCTETSTRING [8] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | FRQL |  |  |
| Comments: | Frequency list, frequency short list GSM 04.08, 10.5.2.5, 10.5.2.6 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| fl |  | OCTETSTRING |  |
| Detailed Comments: | frequency short list has a fixed length of 10 octets and does not contain the length indicator. |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | FRQPARA |  |  |
| Comments: | Parameters for Description of basic physical channel in frequency domain. |  |  |
| Element Name |  | Type Definition | Comments |
| hch |  | BITSTRING[1] |  |
| maio |  | BITSTRING [6] |  |
| hsn |  | BITSTRING [6] |  |
| spr |  | BITSTRING [2] |  |
| arfcn |  | BITSTRING [10] |  |
| maclength |  | LENGTH |  |
| mac_8n |  | BITSTRING [8] |  |
| mac_7n |  | BITSTRING [8] |  |
| mac_6n |  | BITSTRING [8] |  |
| mac_5n |  | BITSTRING [8] |  |
| mac_4n |  | BITSTRING [8] |  |
| mac_3n |  | BITSTRING [8] |  |
| mac_2n |  | BITSTRING [8] |  |
| mac_1n |  | BITSTRING [8] |  |
| flst |  | FRQL |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | HLCMP |  |  |
| Comments: | High layer compatibility (CC information element) GSM 04.08, 10.5.4.16, ITU Q.931 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| extb3 |  | EXTB |  |
| cs |  | BITSTRING [2] |  |
| in |  | BITSTRING [3] |  |
| pmpp |  | BITSTRING [2] |  |
| extb4 |  | EXTB |  |
| hlci |  | BITSTRING [7] |  |
| extb4a |  | EXTB |  |
| ehlci |  | BITSTRING [7] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | IARESTOCT |  |  |
| Comments: |  |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iaroct1 |  | OCTETSTRING[1..11] |  |
| iaroct2 |  | IARESTOCT2 |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | IARESTOCT2 |  |  |
| Comments: |  |  |  |
| Element Name |  | Type Definition | Comments |
| p |  | BITSTRING[2] |  |
| frqparalen |  | BITSTRING[6] |  |
| spbt1 |  | BITSTRING[2] |  |
| maio |  | BITSTRING[6] |  |
| ma |  | OCTETSTRING[0..8] |  |
| spbt2 |  | OCTETSTRING[0..8] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | KPF (keypad facility) |  |  |
| Comments: | keypad facility GSM 04.08 clause 10.5.4.17 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| extb |  | EXTB |  |
| kpf_info |  | IA5String [1] |  |
| Detailed Comments: | Is extb needed? |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | L1HD |  |  |
| Comments: | Layer 1 Header |  |  |
| Element Name |  | Type Definition | Comments |
| spr1 |  | BITSTRING[3] |  |
| mspwrlvl |  | BITSTRING[5] |  |
| spr2 |  | BITSTRING[1] |  |
| ta |  | BITSTRING[7] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | LAI |  |  |
| Comments: | Location area identification GSM 04.08, 10.5.1.3 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| mcc |  | OCTETSTRING [2] |  |
| mnc |  | OCTETSTRING [1] |  |
| lac |  | OCTETSTRING [2] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | LLCMP |  |  |
| Comments: | Low layer compatibility (CC information element) GSM 04.08, 10.5.4.18 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| extb3 |  | EXTB |  |
| cs |  | BITSTRING [2] |  |
| itc |  | BITSTRING [5] |  |
| extb3a |  | EXTB |  |
| negind |  | BITSTRING[1] |  |
| spb3a |  | SP6B |  |
| extb4 |  | EXTB |  |
| tm |  | BITSTRING [2] |  |
| itr |  | BITSTRING [5] |  |
| extb4a |  | EXTB |  |
| strc |  | BITSTRING [3] |  |
| config |  | BITSTRING [2] |  |
| est |  | BITSTRING [2] |  |
| extb4b |  | EXTB |  |
| sym |  | BITSTRING [2] |  |
| itrdo |  | BITSTRING [5] |  |
| extb5 |  | EXTB |  |
| l1id |  | BITSTRING [2] |  |
| uil1 |  | BITSTRING [5] |  |
| extb5a |  | EXTB |  |
| sb |  | BITSTRING [1] |  |
| neg |  | BITSTRING [1] |  |
| ur |  | BITSTRING [5] |  |
| extb5b1 |  | EXTB |  |
| ir |  | BITSTRING [2] |  |
| nictx |  | BITSTRING [1] |  |
| nicrx |  | BITSTRING [1] |  |
| fctx |  | BITSTRING [1] |  |
| fcrx |  | BITSTRING [1] |  |
| spb5b1 |  | SPB |  |
| extb5b2 |  | EXTB |  |
| hdrb |  | BITSTRING[1] |  |
| mfs |  | BITSTRING[1] |  |
| mode |  | BITSTRING[1] |  |
| llineg |  | BITSTRING[1] |  |
| ass |  | BITSTRING[1] |  |
| ibob |  | BITSTRING[1] |  |
| spb5b2 |  | SPB |  |
| extb5c |  | EXTB |  |
| nsb |  | BITSTRING [2] |  |
| ndb |  | BITSTRING [2] |  |
| pi |  | BITSTRING [3] |  |
| extb5d |  | EXTB |  |
| dplxm |  | BITSTRING [1] |  |
| modemt |  | BITSTRING [6] |  |
| extb6 |  | EXTB |  |
| l2id |  | BITSTRING [2] |  |
| uil2 |  | BITSTRING [5] |  |
| extb6a |  | EXTB |  |
| ol2pi |  | BITSTRING[7] |  |
| extb7 |  | EXTB |  |
| l3id |  | BITSTRING [2] |  |
| uil3 |  | BITSTRING [5] |  |
| extb7a |  | EXTB |  |
| ol3pi |  | BITSTRING[7] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | LUT |  |  |
| Comments: | Location updating type GSM 04.08, 10.5.3.5 |  |  |
| Element Name |  | Type Definition | Comments |
| foreq |  | BITSTRING [1] |  |
| sprb |  | BITSTRING [1] |  |
| lut |  | BITSTRING [2] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | MA |  |  |
| Comments: | Mobile allocation GSM 04.08, 10.5.2.21 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| mac_8n |  | BITSTRING [8] |  |
| mac_7n |  | BITSTRING [8] |  |
| mac_6n |  | BITSTRING [8] |  |
| mac_5n |  | BITSTRING [8] |  |
| mac_4n |  | BITSTRING [8] |  |
| mac_3n |  | BITSTRING [8] |  |
| mac_2n |  | BITSTRING [8] |  |
| mac_1n |  | BITSTRING [8] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | MI |  |  |
| Comments: | Mobile identity GSM 04.08, 10.5.1.4 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| idigit_1 |  | BITSTRING [4] |  |
| oei |  | BITSTRING [1] |  |
| toi |  | BITSTRING [3] |  |
| idigits_other |  | OCTETSTRING [0..7] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | MODEMTYPE |  |  |
| Comments: |  |  |  |
| Element Name |  | Type Definition | Comments |
| V22bisOrV26ter |  | B_5 |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | MSCLM1 |  |  |
| Comments: | mobile station classmark 1 GSM 04.08, 10.5.1.5 |  |  |
| Element Name |  | Type Definition | Comments |
| spr1 |  | BITSTRING [1] |  |
| rl |  | BITSTRING [2] |  |
| spr2 |  | BITSTRING [1] |  |
| a5_1 |  | BITSTRING [1] |  |
| rfpc |  | BITSTRING [3] |  |
| Detailed Comments: | 2 octets long |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | MSCLM2 |  |  |
| Comments: | mobile station classmark 2 GSM 04.08, 10.5.1.6 |  |  |
| Element Name |  | Type Definition | Comments |
| iel |  | LENGTH |  |
| spr1 |  | BITSTRING [1] |  |
| rl |  | BITSTRING [2] |  |
| spr2 |  | BITSTRING [1] |  |
| a5_1 |  | BITSTRING [1] |  |
| rfpc |  | BITSTRING [3] |  |
| spr3 |  | BITSTRING [1] |  |
| psc |  | BITSTRING [1] |  |
| sssi |  | BITSTRING [2] |  |
| smc |  | BITSTRING [1] |  |
| spr4 |  | BITSTRING [2] |  |
| fc |  | BITSTRING [1] |  |
| cm3 |  | BITSTRING [1] |  |
| spr5 |  | BITSTRING [5] |  |
| a5_3 |  | BITSTRING [1] |  |
| a5_2 |  | BITSTRING [1] |  |
| Detailed Comments: | 4 octets long |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | MSCLM3 |  |  |
| Comments: | Mobile station classmark 3 GSM 04.08, 10.5.1.7 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| spr1 |  | BITSTRING [4] |  |
| a5_7 |  | BITSTRING [1] |  |
| a5_6 |  | BITSTRING [1] |  |
| a5_5 |  | BITSTRING [1] |  |
| a5_4 |  | BITSTRING [1] |  |
| spr2 |  | OCTETSTRING [11] |  |
| Detailed Comments: | The info element has 14 octets |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | MSRR |  |  |
| Comments: | measurement results GSM 04.08, 10.5.2.20 |  |  |
| Element Name |  | Type Definition | Comments |
| ba_used |  | BITSTRING [1] |  |
| dtx_used |  | BITSTRING [1] |  |
| rxlev_fsc |  | RXLEV |  |
| spr1 |  | BITSTRING [1] |  |
| meas_valid |  | BITSTRING [1] |  |
| rxlev_ssc |  | RXLEV |  |
| spr2 |  | BITSTRING [1] |  |
| rxqual_fsc |  | BITSTRING [3] |  |
| rxqual_ssc |  | BITSTRING [3] |  |
| no_nc |  | BITSTRING [3] |  |
| rxlev_nc1 |  | RXLEV |  |
| bcchfrq_nc1 |  | BCCHFRQ |  |
| bsic_nc1 |  | BSIC |  |
| rxlev_nc2 |  | RXLEV |  |
| bcchfrq_nc2 |  | BCCHFRQ |  |
| bsic_nc2 |  | BSIC |  |
| rxlev_nc3 |  | RXLEV |  |
| bcchfrq_nc3 |  | BCCHFRQ |  |
| bsic_nc3 |  | BSIC |  |
| rxlev_nc4 |  | RXLEV |  |
| bcchfrq_nc4 |  | BCCHFRQ |  |
| bsic_nc4 |  | BSIC |  |
| rxlev_nc5 |  | RXLEV |  |
| bcchfrq_nc5 |  | BCCHFRQ |  |
| bsic_nc5 |  | BSIC |  |
| rxlev_nc6 |  | RXLEV |  |
| bcchfrq_nc6 |  | BCCHFRQ |  |
| bsic_nc6 |  | BSIC |  |
| Detailed Comments: | The info element has a fixed length of 16 octets. |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | MTDIF |  |  |
| Comments: | Mobile time difference GSM 04.08, 10.5.1.21a |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| value |  | OCTETSTRING [3] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | NCD |  |  |
| Comments: | Neighbour cells description GSM 04.08, 10.5.2.22 |  |  |
| Element Name |  | Type Definition | Comments |
| rfl2 |  | BITSTRING [2] |  |
| extind |  | BITSTRING [1] |  |
| baind |  | BITSTRING [1] |  |
| rfl4 |  | BITSTRING [4] |  |
| rfl |  | OCTETSTRING [15] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | NCD2 |  |  |
| Comments: | Neighbour cells description2 GSM 04.08, 10.5.2.22b |  |  |
| Element Name |  | Type Definition | Comments |
| b128 |  | BITSTRING [1] |  |
| mbrpt |  | BITSTRING [2] |  |
| baind |  | BITSTRING [1] |  |
| b122_124 |  | BITSTRING [3] |  |
| rfl1 |  | BITSTRING [1] |  |
| rfl |  | OCTETSTRING [15] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | PCMD |  |  |
| Comments: | Power command and access type GSM 04.08, 10.5.2.28, 10.5.2.28a |  |  |
| Element Name |  | Type Definition | Comments |
| sprb |  | BITSTRING [3] |  |
| pl |  | BITSTRING [5] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | PI |  |  |
| Comments: | Progress indicator GSM 04.08, 10.5.4.21 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| extb3 |  | EXTB |  |
| cs |  | BITSTRING [2] |  |
| spb |  | SPB |  |
| loc |  | BITSTRING [4] |  |
| extb4 |  | EXTB |  |
| prd |  | BITSTRING [7] |  |
| Detailed Comments: | total 4 octets |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | PI_SI |  |  |
| Comments: | Presentation indicator & screening indicator GSM 04.08, 10.5.4.9, 10.5.4.13 |  |  |
| Element Name |  | Type Definition | Comments |
| extb |  | EXTB |  |
| pi |  | BITSTRING [2] |  |
| sp3b |  | SP3B |  |
| si |  | BITSTRING [2] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | PM |  |  |
| Comments: | Page mode GSM 04.08, 10.5.2.26 |  |  |
| Element Name |  | Type Definition | Comments |
| sprb |  | B_2 |  |
| pgm |  | B_2 |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | RACHCP |  |  |
| Comments: | Rach control parameters GSM 04.08, 10.5.2.29 |  |  |
| Element Name |  | Type Definition | Comments |
| maxrtx |  | BITSTRING [2] |  |
| txint |  | BITSTRING [4] |  |
| cba |  | BITSTRING [1] |  |
| re |  | BITSTRING [1] |  |
| acc_2 |  | BITSTRING [5] |  |
| ec |  | BITSTRING [1] |  |
| acc_1 |  | BITSTRING [10] |  |
| Detailed Comments: | The info element has a fixed length of 3 octets. |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | RPACK |  |  |
| Comments: | SMS RP ACKNOWLEDGEMENT ms <-> n GSM 04.11, 7.3.3 |  |  |
| Element Name |  | Type Definition | Comments |
| sprb |  | SP5B |  |
| rpmti |  | MTI |  |
| rpmr |  | MR |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | RPCAU |  |  |
| Comments: | RP user data element GSM 04.11, 8.2.5.4 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| extb2 |  | EXTB |  |
| rpcau_class |  | BITSTRING [3] |  |
| rpcau_va |  | BITSTRING [4] |  |
| rpcau_di |  | OCTETSTRING [1] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | RPDATA |  |  |
| Comments: | SMS RP DATA ms <-> n GSM 04.11, 7.3.1 |  |  |
| Element Name |  | Type Definition | Comments |
| sprb |  | SP5B |  |
| rpmti |  | MTI |  |
| rpmr |  | MR |  |
| rpOaddr |  | CDPN |  |
| rpDaddr |  | CDPN |  |
| rpusrdat |  | RPUSRDAT |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | RPERR |  |  |
| Comments: | SMS RP ERROR ms <-> n GSM 04.11, 7.3.4 |  |  |
| Element Name |  | Type Definition | Comments |
| sprb |  | SP5B |  |
| rpmti |  | MTI |  |
| rpmr |  | MR |  |
| rpcau |  | RPCAU |  |
| rpusrdat |  | RPUSRDAT |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | RPSMMA |  |  |
| Comments: | SMS RP SMMA ms -> n GSM 04.11, 7.3.2 |  |  |
| Element Name |  | Type Definition | Comments |
| sprb |  | SP5B |  |
| rpmti |  | MTI |  |
| rpmr |  | MR |  |
| Detailed Comments: | SM memory available |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | RPUSRDAT |  |  |
| Comments: | RP user data element GSM 04.11, 8.2.5.3, GSM 03.40, 9.2.2 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| tpdeliver |  | SMDLVR |  |
| tpsubmit |  | SMSBMT |  |
| tpstatus_rpt |  | SMST_RPT |  |
| tpcommand |  | SMCMD |  |
| tpdlvr_sbmt_rpt |  | SMDLVR_RPT |  |
| Detailed Comments: | One of the six tpdu is contained in the RPUSRDATA. Since the structures of the messages SMS-Deliver-Report and SMS-Submit-Report are identical, they have been combined to tpdlvr_sbmt_rpt and therefore only five tp message types exist in this type definition. |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | RQR |  |  |
| Comments: | Request reference GSM 04.08, 10.5.2.30 |  |  |
| Element Name |  | Type Definition | Comments |
| ra |  | BITSTRING [8] |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | SIGNAL |  |  |
| Comments: | Signal (CC information element) GSM 04.08, 10.5.4.23 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| sigv |  | BITSTRING [8] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | SERIAL_NUMBER |  |  |
| Comments: | Serial number for SMSCB, GSM 3.41, 9.3.2 |  |  |
| Element Name |  | Type Definition | Comments |
| gs |  | BITSTRING[2] |  |
| message_code |  | BITSTRING[10] |  |
| update_number |  | BITSTRING[4] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | SMCMD |  |  |
| Comments: | SMS COMMAND, ms - > n. GSM 03.40, 9.2.2.4 |  |  |
| Element Name |  | Type Definition | Comments |
| sprb1 |  | SP2B |  |
| srr |  | BITSTRING[1] |  |
| sprb2 |  | SP3B |  |
| mti |  | BITSTRING [2] |  |
| mr |  | MR |  |
| pid |  | TPPID |  |
| ct |  | OCTETSTRING [1] |  |
| mn |  | OCTETSTRING [1] |  |
| da |  | TPA |  |
| cdl |  | LENGTH |  |
| cd |  | TPCD |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | SMDLVR |  |  |
| Comments: | SMS DELIVER, n - > ms. GSM 03.40, 9.2.2.1 |  |  |
| Element Name |  | Type Definition | Comments |
| rp |  | BITSTRING [1] |  |
| udhi |  | BITSTRING [1] |  |
| sri |  | BITSTRING [1] |  |
| sprb2 |  | SP2B |  |
| mms |  | BITSTRING [1] |  |
| mti |  | BITSTRING [2] |  |
| oa |  | TPA |  |
| pid |  | TPPID |  |
| dcs |  | TPDCS |  |
| scts |  | TPSCTS |  |
| udl |  | LENGTH |  |
| ud |  | TPUD |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | SMDLVR_RPT |  |  |
| Comments: | SMS DELIVER or SUBMIT REPORT contained in the RP ERROR PDU, n <-> ms. GSM 03.40, 9.2.2.1a, 9.2.2.2a. |  |  |
| Element Name |  | Type Definition | Comments |
| sprb1 |  | SP6B |  |
| mti |  | BITSTRING [2] |  |
| fcs |  | FCS |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | SMSBMT |  |  |
| Comments: | SMS SUBMIT, ms -> n. GSM 03.40, 9.2.2.2 |  |  |
| Element Name |  | Type Definition | Comments |
| rp |  | BITSTRING [1] |  |
| udhi |  | BITSTRING [1] |  |
| srr |  | BITSTRING [1] |  |
| vpf |  | BITSTRING [2] |  |
| rd |  | BITSTRING [1] |  |
| mti |  | BITSTRING [2] |  |
| mr |  | MR |  |
| da |  | TPA |  |
| pid |  | TPPID |  |
| dcs |  | TPDCS |  |
| vp1 |  | OCTETSTRING [1] |  |
| vp7 |  | TPSCTS |  |
| udl |  | LENGTH |  |
| ud |  | TPUD |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | SMSCBpack |  |  |
| Comments: | SMS cell broadcasting packing data n -> ms. GSM 03.38, 6.1.2.2.1 |  |  |
| Element Name |  | Type Definition | Comments |
| block1 |  | OCTETSTRING [16] |  |
| block2 |  | OCTETSTRING [22] |  |
| block3 |  | OCTETSTRING [22] |  |
| block4 |  | OCTETSTRING [22] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | SMST_RPT |  |  |
| Comments: | SMS STATUS REPORT, n - > ms. GSM 03.40, 9.2.2.3 |  |  |
| Element Name |  | Type Definition | Comments |
| sprb1 |  | SP5B |  |
| mms |  | BITSTRING [1] |  |
| mti |  | BITSTRING [2] |  |
| mr |  | MR |  |
| ra |  | TPA |  |
| scts |  | TPSCTS |  |
| dt |  | TPSCTS |  |
| st |  | TPST |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | SSVI |  |  |
| Comments: | SS version indicator GSM 04.08, 10.5.4.24 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| ssv |  | OCTETSTRING [1] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | STRT |  |  |
| Comments: | Starting time GSM 04.08, 10.5.2.38 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | SUBAD |  |  |
| Comments: | Subaddress GSM 04.08, 10.5.4.8, 10.5.4.10, 10.5.4.14 |  |  |
| Element Name |  | Type Definition | Comments |
| extb |  | EXTB |  |
| tos |  | BITSTRING [3] |  |
| oei |  | BITSTRING [1] |  |
| sp3b |  | SP3B |  |
| si |  | OCTETSTRING [2..23] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | SYNCHI |  |  |
| Comments: | Synchronization indication GSM 04.08, 10.5.2.39 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_4 |  |
| nci |  | B_1 |  |
| rot |  | B_1 |  |
| si |  | B_2 |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | TA |  |  |
| Comments: | Timing advance GSM 04.08, 10.5.2.40 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| sprb |  | BITSTRING [2] |  |
| value |  | BITSTRING [6] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | TDIF |  |  |
| Comments: | Time difference GSM 04.08, 10.5.1.41 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| value |  | OCTETSTRING [1] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | TI |  |  |
| Comments: | Transaction identifier GSM 04.08, 10.1 |  |  |
| Element Name |  | Type Definition | Comments |
| ti_f |  | BITSTRING [1] |  |
| ti_v |  | TI_V |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | TMSI |  |  |
| Comments: | Temporary GSM 04.08, 10.5.2.42 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| tmsi_val |  | OCTETSTRING [4] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | TON_NPI |  |  |
| Comments: | Type of number and numbering plan GSM 04.08, 10.5.4.7, 10.5.4.9, 10.5.4.13, GSM 03.40, 9.1.2.5 |  |  |
| Element Name |  | Type Definition | Comments |
| extb |  | EXTB |  |
| ton |  | BITSTRING [3] |  |
| npi |  | BITSTRING [4] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | TPA |  |  |
| Comments: | TP address GSM 03.40, 9.1.2.5 |  |  |
| Element Name |  | Type Definition | Comments |
| iel |  | LENGTH |  |
| tonnpi |  | TON_NPI |  |
| digits |  | BCDN |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | TPDCS |  |  |
| Comments: | SMS data coding scheme GSM 03.38, 4, 5 |  |  |
| Element Name |  | Type Definition | Comments |
| cg |  | BITSTRING [4] |  |
| code |  | BITSTRING [4] |  |
| Detailed Comments: | Identifying the coding scheme within the TP user data. |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | TPPID |  |  |
| Comments: | TP protocol identifier GSM 03.40, 9.2.3.9 |  |  |
| Element Name |  | Type Definition | Comments |
| type |  | BITSTRING [2] |  |
| value |  | BITSTRING [6] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | TPST |  |  |
| Comments: | TP status GSM 03.40, 9.2.3.15 |  |  |
| Element Name |  | Type Definition | Comments |
| sprb1 |  | SPB |  |
| value |  | BITSTRING [7] |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | UNKWN |  |  |
| Comments: | unknown IE |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| contents |  | OCTETSTRING |  |
| Detailed Comments: |  |  |  |

| Structured Type Definition |  |  |  |
| --- | --- | --- | --- |
| Type Name: | UU |  |  |
| Comments: | User-user GSM 04.08, 10.5.4.25 |  |  |
| Element Name |  | Type Definition | Comments |
| iei |  | IEI_8 |  |
| iel |  | LENGTH |  |
| uupd |  | BITSTRING [8] |  |
| uui |  | OCTETSTRING [1..128] |  |
| Detailed Comments: | In SETUP, ALERTING, CONNECT, DISCONNECT, RELEASE and RELEASE COMPLETE messages the uui length is of 0 - 32 bytes. In USER INFORMATION messages the uui length is of 1 - 128. |  |  |

### ASN1 type definitions

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | Component |
| Comments: | ASN1_Encoding: BER |
| Type Definition |  |
| CHOICE { registerSSComponents RegisterSS_Components, eraseSSComponents EraseSS_Components, activateSSComponents ActivateSS_Components, deactivateSSComponents DeactivateSS_Components, interrogateSSComponents InterrogateSS_Components, notifySSComponents NotifySS_Components, registerPasswordComponents RegisterPassword_Components, getPasswordComponents GetPassword_Components, processUnstructuredSSDataComponents ProcessUnstructuredSSData_Components, forwardCheckSSIndicationComponents ForwardCheckSSIndication_Components, processUnstructuredSSRequestComponents ProcessUnstructuredSSRequest_Components, unstructuredSSRequestComponents UnstructuredSSRequest_Components, unstructuredSSNotifyComponents UnstructuredSSNotify_Components, forwardCUGInfoComponents ForwardCUGInfo_Components, splitMPTYComponents SplitMPTY_Components, retrieveMPTYComponents RetrieveMPTY_Components, holdMPTYComponents HoldMPTY_Components, buildMPTYComponents BuildMPTY_Components, forwardChargeAdviceComponents ForwardChargeAdvice_Components, generalComponents General_Components } |  |
| Detailed Comments: | Plural components as each type represents invoke, return result, return error etc. |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | Components |
| Comments: | GSM 04.80, 3.6 |
| Type Definition |  |
| SET OF Component |  |
| Detailed Comments: | ASN.1 transfer encoding rules: BER is not wholy used for the type Components. The contents of Components, without the octets encoding the tag and the length of SET OF, is carried as the Components value. |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | ActivateSS_Components |
| Comments: | GSM 09.02, 14.5, 14.6.4, 14.6.6, 14.7.4, 14.7.7 |
| Type Definition |  |
| CHOICE { activateSS_InvokeComp [1] IMPLICIT SEQUENCE { invokeID InvokeIDType, localValue INTEGER (12), ss_ForBS SS_ForBS_Code }, activateSS_ReturnResultComp [2] IMPLICIT SEQUENCE { invokeID InvokeIDType, result SEQUENCE { localValue INTEGER (12), ss_Info SS_Info } } , activateSS_ReturnErrorComp CHOICE { errorCodes [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( bearerServiceNotProvisioned ¦ teleserviceNotProvisioned ¦ illegalSS_Operation ¦ dataMissing ¦ unexpectedDataValue ¦ negativePW_Check ¦ numberOfPW_AttemptsViolation) }, callBarredErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( callBarred ), parameter CallBarringCause }, ss_ErrorStatusErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( ss_ErrorStatus ), parameter SS_Status }, ss_SubscriptionViolationeErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues (ss_SubscriptionViolation), parameter SS_SubscriptionOption }, ss_IncompatibilityErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( ss_Incompatibility ), parameter SS_IncompatibilityCause }, systemFailureErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( systemFailure ), parameter NetworkResource} }, activateSS_RejectComp [4] IMPLICIT RejectComponent } |  |
| Detailed Comments: | Timer m (15 - 30 s) |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | BuildMPTY_Components |
| Comments: | GSM 04.80, 4.2 |
| Type Definition |  |
| CHOICE { buildMPTY_InvokeComp [1] IMPLICIT SEQUENCE { invokeID InvokeIDType, localValue INTEGER (124) }, buildMPTY_ReturnResultComp [2] IMPLICIT SEQUENCE { invokeID InvokeIDType, result SEQUENCE { localValue INTEGER (124) } } , buildMPTY_ReturnErrorComp CHOICE { errorCodes [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( illegalSS_Operation ¦ ss_NotAvailable ¦ maxNumberOfMPTY_ParticipantsExceeded ¦ resourcesNotAvailable ) }, ss_ErrorStatusErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( ss_ErrorStatus ), parameter SS_Status }, ss_IncompatibilityErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( ss_Incompatibility ), parameter SS_IncompatibilityCause }, systemFailureErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( systemFailure ), parameter NetworkResource} }, buildMPTY_RejectComp [4] IMPLICIT RejectComponent } |  |
| Detailed Comments: | Timer T_buildMPTY (5 - 30 s) |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | DeactivateSS_Components |
| Comments: | GSM 09.02, 14.5, 14.6.4, 14.6.6, 14.7.4, 14.7.7 |
| Type Definition |  |
| CHOICE { deactivateSS_InvokeComp [1] IMPLICIT SEQUENCE { invokeID InvokeIDType, localValue INTEGER (13), ss_ForBS SS_ForBS_Code }, deactivateSS_ReturnResultComp [2] IMPLICIT SEQUENCE { invokeID InvokeIDType, result SEQUENCE { localValue INTEGER (13), ss_Info SS_Info } } , deactivateSS_ReturnErrorComp CHOICE { errorCodes [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( bearerServiceNotProvisioned ¦ teleserviceNotProvisioned ¦ illegalSS_Operation ¦ dataMissing ¦ unexpectedDataValue ¦ negativePW_Check ¦ numberOfPW_AttemptsViolation) }, callBarredErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( callBarred ), parameter CallBarringCause }, ss_ErrorStatusErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( ss_ErrorStatus ), parameter SS_Status }, ss_SubscriptionViolationeErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues (ss_SubscriptionViolation), parameter SS_SubscriptionOption }, systemFailureErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( systemFailure ), parameter NetworkResource} }, deactivateSS_RejectComp [4] IMPLICIT RejectComponent } |  |
| Detailed Comments: | Timer m |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | EraseSS_Components |
| Comments: | GSM 09.02, 14.5, 14.6.4, 14.6.6, 14.7.4, 14.7.7 |
| Type Definition |  |
| CHOICE { eraseSS_InvokeComp [1] IMPLICIT SEQUENCE { invokeID InvokeIDType, localValue INTEGER (11), ss_ForBS SS_ForBS_Code }, eraseSS_ReturnResultComp [2] IMPLICIT SEQUENCE { invokeID InvokeIDType, result SEQUENCE { localValue INTEGER (11), ss_Info SS_Info } } , eraseSS_ReturnErrorComp CHOICE { errorCodes [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( bearerServiceNotProvisioned ¦ teleserviceNotProvisioned ¦ illegalSS_Operation ¦ dataMissing ¦ unexpectedDataValue ) }, callBarredErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( callBarred ), parameter CallBarringCause }, ss_ErrorStatusErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( ss_ErrorStatus ), parameter SS_Status }, systemFailureErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( systemFailure ), parameter NetworkResource} }, eraseSS_RejectComp [4] IMPLICIT RejectComponent } |  |
| Detailed Comments: | Timer m |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | ForwardChargeAdvice_Components |
| Comments: | GSM 04.80, 4.2 |
| Type Definition |  |
| CHOICE { forwardChargeAdvice_InvokeComp [1] IMPLICIT SEQUENCE { invokeID InvokeIDType, localValue INTEGER (125), forwardChargeAdviceArg SEQUENCE { ss_Code [0] SS_Code, chargingInformation [1] SEQUENCE{ e1 [1] INTEGER (0..max10TimesUnitsPerTime) OPTIONAL, e2 [2] INTEGER (0..max10TimesTimeInterval) OPTIONAL, e3 [3] INTEGER (0..max100TimesScalingFactor) OPTIONAL, e4 [4] INTEGER (0..max10TimesIncrement) OPTIONAL, e5 [5] INTEGER (0..max10TimesIncrementPerDataInterval) OPTIONAL, e6 [6] INTEGER (0..maxNumberOfSegmentsPerDataInterval) OPTIONAL, e7 [7] INTEGER (0..max10TimesInitialTime) OPTIONAL } } }, forwardChargeAdvice_ReturnResultComp [2] IMPLICIT SEQUENCE { invokeID InvokeIDType, result SEQUENCE { localValue INTEGER (125) } } , forwardChargeAdvice_RejectComp [4] IMPLICIT RejectComponent } |  |
| Detailed Comments: | Timer T_aoc = 1 - 40 s |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | ForwardCheckSSIndication_Components |
| Comments: | GSM 09.02, 14.5, 14.6.4, 14.6.6, 14.7.4, 14.7.7 |
| Type Definition |  |
| CHOICE { forwardCheckSSIndication_InvokeComp [1] IMPLICIT SEQUENCE { invokeID InvokeIDType, localValue INTEGER (38) }, forwardCheckSSIndication_RejectComp [4] IMPLICIT RejectComponent } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | ForwardCUGInfo_Components |
| Comments: | GSM 04.80, 4.2 |
| Type Definition |  |
| CHOICE { forwardCUGInfo_InvokeComp [1] IMPLICIT SEQUENCE { invokeID InvokeIDType, localValue INTEGER (120), forwardCUGInfo_Arg SEQUENCE{ cug_Index [0] IMPLICIT CUG_Index OPTIONAL, suppressPrefCUG [1] IMPLICIT NULL OPTIONAL, suppressOA [2] IMPLICIT NULL OPTIONAL } }, forwardCUGInfo_RejectComp [4] IMPLICIT RejectComponent } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | General_Components |
| Comments: | Non specified components must match this type definition. |
| Type Definition |  |
| CHOICE { general_InvokeComp [1] IMPLICIT General_InvokeComponent, general_ReturnResultComp [2] IMPLICIT General_ReturnResultComponent, general_ReturnErrorComp [3] IMPLICIT General_ReturnErrorComponent, general_RejectComp [4] IMPLICIT RejectComponent } -- This is the General InvokeComponent -- General_InvokeComponent ::= SEQUENCE { invokeID InvokeIDType, linked_ID [0] IMPLICIT InvokeIDType OPTIONAL, operation_value Operation, argument ANY OPTIONAL } -- This is the General ReturnResultComponent -- General_ReturnResultComponent ::= SEQUENCE { invokeID InvokeIDType, valueAndResult SEQUENCE { operation_value Operation, result ANY } OPTIONAL } -- This is the General ReturnErrorComponent -- General_ReturnErrorComponent ::= SEQUENCE { invokeID InvokeIDType, error ANY } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | GetPassword_Components |
| Comments: | GSM 09.02, 14.5, 14.6.4, 14.6.6, 14.7.4, 14.7.7 |
| Type Definition |  |
| CHOICE { getPassword_InvokeComp [1] IMPLICIT SEQUENCE { invokeID InvokeIDType, linkedID [0] IMPLICIT InvokeIDType, localValue INTEGER (18), guidanceInfo ENUMERATED { enterPW (0), enterNewPW (1), enterNewPW_Again (2) } }, getPassword_ReturnResultComp [2] IMPLICIT SEQUENCE { invokeID InvokeIDType, result SEQUENCE { localValue INTEGER (18), currentPassword Password } } , getPassword_RejectComp [4] IMPLICIT RejectComponent } |  |
| Detailed Comments: | Timer m |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | HoldMPTY_Components |
| Comments: | GSM 04.80, 4.2 |
| Type Definition |  |
| CHOICE { holdMPTY_InvokeComp [1] IMPLICIT SEQUENCE { invokeID InvokeIDType, localValue INTEGER (123) }, holdMPTY_ReturnResultComp [2] IMPLICIT SEQUENCE { invokeID InvokeIDType, result SEQUENCE { localValue INTEGER (123) } } , holdMPTY_ReturnErrorComp CHOICE { errorCodes [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( illegalSS_Operation ¦ facilityNotSupported ) }, ss_ErrorStatusErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( ss_ErrorStatus ), parameter SS_Status }, ss_IncompatibilityErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( ss_Incompatibility ), parameter SS_IncompatibilityCause }, systemFailureErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( systemFailure ), parameter NetworkResource} }, holdMPTY_RejectComp [4] IMPLICIT RejectComponent } |  |
| Detailed Comments: | Timer T_holdMPTY (5 - 30 s) |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | InterrogateSS_Components |
| Comments: | GSM 09.02, 14.5, 14.6.4, 14.6.6, 14.7.4, 14.7.7 |
| Type Definition |  |
| CHOICE { interrogateSS_InvokeComp [1] IMPLICIT SEQUENCE { invokeID InvokeIDType, localValue INTEGER (14), ss_ForBS SS_ForBS_Code }, interrogateSS_ReturnResultComp [2] IMPLICIT SEQUENCE { invokeID InvokeIDType, result SEQUENCE { localValue INTEGER (14), interrogateSS_Res CHOICE { ss_Status [0] IMPLICIT SS_Status, basicServiceGroupList [2] IMPLICIT BasicServiceGroupList, forwardingFeatureList [3] IMPLICIT ForwardingFeatureList, cli_RestrictionInfo [4] IMPLICIT SEQUENCE { ss_Status SS_Status, cliRestrictionOption CliRestrictionOption OPTIONAL } } } } , interrogateSS_ReturnErrorComp CHOICE { errorCodes [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( bearerServiceNotProvisioned ¦ teleserviceNotProvisioned ¦ illegalSS_Operation ¦ ss_NotAvailable ¦ dataMissing ¦ unexpectedDataValue) }, callBarredErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( callBarred ), parameter CallBarringCause }, systemFailureErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( systemFailure ), parameter NetworkResource} }, interrogateSS_RejectComp [4] IMPLICIT RejectComponent } |  |
| Detailed Comments: | Timer m |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | NotifySS_Components |
| Comments: | GSM 04.80, 4.2 |
| Type Definition |  |
| CHOICE { notifySS_InvokeComp [1] IMPLICIT SEQUENCE { invokeID InvokeIDType, localValue INTEGER (16), notifySS_Arg SEQUENCE{ ss_Code [1] IMPLICIT SS_Code OPTIONAL, ss_Status [4] IMPLICIT SS_Status OPTIONAL, ss_Notification [5] IMPLICIT OCTET STRING (SIZE (1)) OPTIONAL, callIsWaiting_Indicator [14] IMPLICIT NULL OPTIONAL, callOnHold_Indicator [15] IMPLICIT ENUMERATED { callRetrieved (0), callOnHold (1) } OPTIONAL, mpty_Indicator [16] IMPLICIT NULL OPTIONAL, cug_Index [17] IMPLICIT CUG_Index OPTIONAL, clirSuppressionRejected [18] IMPLICIT NULL OPTIONAL } }, notifySS_RejectComp [4] IMPLICIT RejectComponent } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | ProcessUnstructuredSSData_Components |
| Comments: | GSM 09.02, 14.5, 14.6.4, 14.6.6, 14.7.4, 14.7.7 |
| Type Definition |  |
| CHOICE { processUnstructuredSSData_InvokeComp [1] IMPLICIT SEQUENCE { invokeID InvokeIDType, localValue INTEGER (19), ss_UserData SS_UserData }, processUnstructuredSSData_ReturnResultComp [2] IMPLICIT SEQUENCE { invokeID InvokeIDType, result SEQUENCE { localValue INTEGER (19), ss_UserData SS_UserData } } , processUnstructuredSSData_ReturnErrorComp CHOICE { errorCodes [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues (unexpectedDataValue) }, systemFailureErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( systemFailure ), parameter NetworkResource} }, processUnstructuredSSData_RejectComp [4] IMPLICIT RejectComponent } |  |
| Detailed Comments: | Timer m |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | ProcessUnstructuredSSRequest_Components |
| Comments: | GSM 09.02, 14.5, 14.6.4, 14.6.6, 14.7.4, 14.7.7 |
| Type Definition |  |
| CHOICE { processUnstructuredSSRequest_InvokeComp [1] IMPLICIT SEQUENCE { invokeID InvokeIDType, localValue INTEGER (59), ussd_Arg USSD_Arg }, processUnstructuredSSRequest_ReturnResultComp [2] IMPLICIT SEQUENCE { invokeID InvokeIDType, result SEQUENCE { localValue INTEGER (59), ussd_Res USSD_Res} } , processUnstructuredSSRequest_ReturnErrorComp CHOICE { errorCodes [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues (dataMissing ¦ unexpectedDataValue ¦ unknownAlphabet) }, callBarredErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( callBarred ), parameter CallBarringCause }, systemFailureErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( systemFailure ), parameter NetworkResource} }, processUnstructuredSSRequest_RejectComp [4] IMPLICIT RejectComponent } |  |
| Detailed Comments: | Timer m |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | RegisterPassword_Components |
| Comments: | GSM 09.02, 14.5, 14.6.4, 14.6.6, 14.7.4, 14.7.7 |
| Type Definition |  |
| CHOICE { registerPassword_InvokeComp [1] IMPLICIT SEQUENCE { invokeID InvokeIDType, localValue INTEGER (17), ss_Code SS_Code }, registerPassword_ReturnResultComp [2] IMPLICIT SEQUENCE { invokeID InvokeIDType, result SEQUENCE { localValue INTEGER (17), newPassword Password } } , registerPassword_ReturnErrorComp CHOICE { errorCodes [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( dataMissing ¦ unexpectedDataValue ¦ negativePW_Check ¦ numberOfPW_AttemptsViolation) }, callBarredErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( callBarred ), parameter CallBarringCause }, ss_SubscriptionViolationeErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues (ss_SubscriptionViolation), parameter SS_SubscriptionOption }, systemFailureErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( systemFailure ), parameter NetworkResource}, pw_RegistrationFailureErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( pw_RegistrationFailure ), parameter PW_RegistrationFailureCause } }, registerPassword_RejectComp [4] IMPLICIT RejectComponent } |  |
| Detailed Comments: | Timer m |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | RegisterSS_Components |
| Comments: | GSM 09.02, 14.5, 14.6.4, 14.6.6, 14.7.4, 14.7.7 |
| Type Definition |  |
| CHOICE { registerSS_InvokeComp [1] IMPLICIT SEQUENCE { invokeID InvokeIDType, localValue INTEGER (10), registerSS_Arg SEQUENCE{ ss_Code SS_Code, basicService BasicServiceCode OPTIONAL, forwardedToNumber [4] IMPLICIT AddressString OPTIONAL, forwardedToSubaddress [6] IMPLICIT ISDN_SubaddressString OPTIONAL, noReplyConditionTime [5] IMPLICIT NoReplyConditionTime OPTIONAL } }, registerSS_ReturnResultComp [2] IMPLICIT SEQUENCE { invokeID InvokeIDType, result SEQUENCE { localValue INTEGER (10), ss_Info SS_Info } } , registerSS_ReturnErrorComp CHOICE { errorCodes [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( bearerServiceNotProvisioned ¦ teleserviceNotProvisioned ¦ illegalSS_Operation ¦ dataMissing ¦ unexpectedDataValue ) }, callBarredErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( callBarred ), parameter CallBarringCause }, ss_IncompatibilityErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( ss_Incompatibility ), parameter SS_IncompatibilityCause }, systemFailureErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( systemFailure ), parameter NetworkResource} }, registerSS_RejectComp [4] IMPLICIT RejectComponent } |  |
| Detailed Comments: | Timer m |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | RejectComponent |
| Comments: | ITU-T Q.767 |
| Type Definition |  |
| SEQUENCE { invokedID CHOICE { derivable InvokeIDType, notDerivable NULL }, problem CHOICE { generalProblem [0] IMPLICIT GeneralProblem, invokeProblem [1] IMPLICIT InvokeProblem, returnResultProblem [2] IMPLICIT ReturnResultProblem, returnErrorProblem [3] IMPLICIT ReturnErrorProblem } } |  |
| Detailed Comments: | Reject Component is not specific to any particular operation. The invokeID may be used to identify a specific operation. |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | RetrieveMPTY_Components |
| Comments: | GSM 04.80, 4.2 |
| Type Definition |  |
| CHOICE { retrieveMPTY_InvokeComp [1] IMPLICIT SEQUENCE { invokeID InvokeIDType, localValue INTEGER (122) }, retrieveMPTY_ReturnResultComp [2] IMPLICIT SEQUENCE { invokeID InvokeIDType, result SEQUENCE { localValue INTEGER (122) } } , retrieveMPTY_ReturnErrorComp CHOICE { errorCodes [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( illegalSS_Operation ¦ facilityNotSupported ) }, ss_ErrorStatusErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( ss_ErrorStatus ), parameter SS_Status }, ss_IncompatibilityErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( ss_Incompatibility ), parameter SS_IncompatibilityCause }, systemFailureErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( systemFailure ), parameter NetworkResource} }, retrieveMPTY_RejectComp [4] IMPLICIT RejectComponent } |  |
| Detailed Comments: | Timer T_retrieveMPTY (5 - 30 s) |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | SplitMPTY_Components |
| Comments: | GSM 04.80, 4.2 |
| Type Definition |  |
| CHOICE { splitMPTY_InvokeComp [1] IMPLICIT SEQUENCE { invokeID InvokeIDType, localValue INTEGER (121) }, splitMPTY_ReturnResultComp [2] IMPLICIT SEQUENCE { invokeID InvokeIDType, result SEQUENCE { localValue INTEGER (121) } } , splitMPTY_ReturnErrorComp CHOICE { errorCodes [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( illegalSS_Operation ¦ facilityNotSupported ) }, ss_ErrorStatusErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( ss_ErrorStatus ), parameter SS_Status }, ss_IncompatibilityErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( ss_Incompatibility ), parameter SS_IncompatibilityCause }, systemFailureErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( systemFailure ), parameter NetworkResource} }, splitMPTY_RejectComp [4] IMPLICIT RejectComponent } |  |
| Detailed Comments: | Timer T_splitMPTY (5 - 30 s) |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | UnstructuredSSNotify_Components |
| Comments: | GSM 09.02, 14.5, 14.6.4, 14.6.6, 14.7.4, 14.7.7 |
| Type Definition |  |
| CHOICE { unstructuredSSNotify_InvokeComp [1] IMPLICIT SEQUENCE { invokeID InvokeIDType, localValue INTEGER (61), ussd_Arg USSD_Arg }, unstructuredSSNotify_ReturnResultComp [2] IMPLICIT SEQUENCE { invokeID InvokeIDType, result SEQUENCE { localValue INTEGER (61) } } , unstructuredSSNotify_ReturnErrorComp CHOICE { errorCodes [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( illegalSubscriber ¦ illegalEquipment ¦ absentSubscriber ¦ dataMissing ¦ unexpectedDataValue ¦ unknownAlphabet ¦ ussd_Busy) }, systemFailureErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( systemFailure ), parameter NetworkResource} }, unstructuredSSNotify_RejectComp [4] IMPLICIT RejectComponent } |  |
| Detailed Comments: | Timer m |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | UnstructuredSSRequest_Components |
| Comments: | GSM 09.02, 14.5, 14.6.4, 14.6.6, 14.7.4, 14.7.7 |
| Type Definition |  |
| CHOICE { unstructuredSSRequest_InvokeComp [1] IMPLICIT SEQUENCE { invokeID InvokeIDType, localValue INTEGER (60), ussd_Arg USSD_Arg }, unstructuredSSRequest_ReturnResultComp [2] IMPLICIT SEQUENCE { invokeID InvokeIDType, result SEQUENCE { localValue INTEGER (60), ussd_Res USSD_Res } } , unstructuredSSRequest_ReturnErrorComp CHOICE { errorCodes [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( illegalSubscriber ¦ illegalEquipment ¦ absentSubscriber ¦ dataMissing ¦ unexpectedDataValue ¦ unknownAlphabet ¦ ussd_Busy) }, systemFailureErr [3] IMPLICIT SEQUENCE { invokeID InvokeIDType, errorCode ErrorLocalValues ( systemFailure ), parameter NetworkResource} }, unstructuredSSRequest_RejectComp [4] IMPLICIT RejectComponent } |  |
| Detailed Comments: | Timer m |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | AddressString |
| Comments: | GSM 09.02, 14.7.8 |
| Type Definition |  |
| OCTET STRING (SIZE ( 1 .. maxAddressLength )) |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | Asn1Integer |
| Comments: | INTEGER of ASN.1 type |
| Type Definition |  |
| INTEGER |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | BasicServiceCode |
| Comments: | GSM 09.02, 14.7.8 |
| Type Definition |  |
| CHOICE { bearerService [2] IMPLICIT BearerServiceCode, teleservice [3] IMPLICIT TeleserviceCode } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | BasicServiceGroupList |
| Comments: | GSM 09.02, 14.7.8 |
| Type Definition |  |
| SEQUENCE SIZE ( 1 .. maxNumOfBasicServiceGroups ) OF BasicServiceCode |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | BearerServiceCode |
| Comments: | GSM 09.02, 14.7.10 |
| Type Definition |  |
| OCTET STRING (SIZE (1)) |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | CallBarringCause |
| Comments: | GSM 09.02, 14.7.7 |
| Type Definition |  |
| ENUMERATED { barringServiceActive (0), operatorBarring (1) } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | CallBarringFeature |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| SEQUENCE { basicService BasicServiceCode OPTIONAL, ss_Status [4] IMPLICIT SS_Status OPTIONAL } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | CallBarringFeatureList |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| SEQUENCE SIZE (1..maxNumOfBasicServiceGroups) OF CallBarringFeature |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | CallBarringInfo |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| SEQUENCE { ss_Code SS_Code OPTIONAL, callBarringFeatureList CallBarringFeatureList } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | CliRestrictionOption |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| ENUMERATED { permanent (0), temporaryDefaultRestricted (1), temporaryDefaultAllowed (2) } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | CUG_Feature |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| SEQUENCE { basicService BasicServiceCode OPTIONAL, preferentialCUG_Indicator CUG_Index OPTIONAL, interCUG_Restrictions InterCUG_Restrictions } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | CUG_FeatureList |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| SEQUENCE SIZE (1.. maxNumOfBasicServiceGroups ) OF CUG_Feature |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | CUG_Index |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| INTEGER (0..32767) |  |
| Detailed Comments: | The internal structure is defined in ETS 300 138. |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | CUG_Interlock |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| OCTET STRING (SIZE (4)) |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | CUG_Info |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| SEQUENCE { cug_SubscriptionList CUG_SubscriptionList, cug_FeatureList CUG_FeatureList OPTIONAL } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | CUG_Subscription |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| SEQUENCE { cug_Index CUG_Index, cug_Interlock CUG_Interlock, intraCUG_Options IntraCUG_Options, basicServiceGroupList BasicServiceGroupList OPTIONAL } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | CUG_SubscriptionList |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| SEQUENCE SIZE (1.. maxNumOfCUG ) OF CUG_Subscription |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | ErrorLocalValues |
| Comments: | GSM 04.80, 4.5 |
| Type Definition |  |
| INTEGER { unknownSubscriber (1), illegalSubscriber (9), bearerServiceNotProvisioned (10), teleserviceNotProvisioned (11), illegalEquipment (12), callBarred (13), illegalSS_Operation (16), ss_ErrorStatus (17), ss_NotAvailable (18), ss_SubscriptionViolation (19), ss_Incompatibility (20), facilityNotSupported (21), absentSubscriber (27), systemFailure (34), dataMissing (35), unexpectedDataValue (36), pw_RegistrationFailure (37), negativePW_Check (38), numberOfPW_AttemptsViolation (43), unknownAlphabet (71), ussd_Busy (72), maxNumberOfMPTY_ParticipantsExceeded (126), resourcesNotAvailable (127) } |  |
| Detailed Comments: | Elements of INTEGER are global for the ATS. |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | ForwardingFeature |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| SEQUENCE { basicService BasicServiceCode OPTIONAL, ss_Status [4] IMPLICIT SS_Status OPTIONAL, forwardedToNumber [5] IMPLICIT ISDN_AddressString OPTIONAL, forwardedToSubaddress [8] IMPLICIT ISDN_SubaddressString OPTIONAL, forwardingOptions [6] IMPLICIT ForwardingOptions OPTIONAL, noReplyConditionTime [7] IMPLICIT NoReplyConditionTime OPTIONAL } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | ForwardingFeatureList |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| SEQUENCE SIZE (1..maxNumOfBasicServiceGroups) OF ForwardingFeature |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | ForwardingInfo |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| SEQUENCE { ss_Code SS_Code OPTIONAL, forwardingFeatureList ForwardingFeatureList } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | ForwardingOptions |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| OCTET STRING (SIZE (1)) |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | GeneralProblem |
| Comments: | ITU-T Q.767 |
| Type Definition |  |
| TCAP_Problems ( unrecognizedComponent ¦ mistypedComponent ¦ badlyStructuredComponent ) |  |
| Detailed Comments: | Type restricted to these three. |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | InterCUG_Restrictions |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| OCTET STRING (SIZE (1)) |  |
| Detailed Comments: | bits: 876543: 000000 (unused) bits 21: 00 CUG only facilities 01 CUG with outgoing access 10 CUG with incoming access 11 CUG with both outgoing and incoming access |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | IntraCUG_Options |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| ENUMERATED { noCUG_Restrictions (0), cugIC_CallBarred (1), cugOG_CallBarred (2) } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | InvokeIDType |
| Comments: | ITU-T, Q.773 |
| Type Definition |  |
| INTEGER (-128 .. 127) |  |
| Detailed Comments: | Values: Sending Components: If it is an invoke component then use Test Case Variable (with default) to set value. If another invoke component is sent the TCV should be incremented beforehand. If it is a return result, error or reject component in response to a received invoke component then use TCV also, making sure the value is set to the value of the received component beforehand. Receiving Components: If it is an invoke comp then use '?'. If it is a return result, error or reject component in response to a sent invoke component then use TCV value (as used in sent invoke component). |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | InvokeProblem |
| Comments: | ITU-T Q.767 |
| Type Definition |  |
| TCAP_Problems ( duplicateInvokeID ¦ unrecognizedOperation ¦ mistypedArgument ¦ resourceLimitation ¦ initiatingRelease ¦ unrecognizedLinkedID ¦ linkedResponseUnexpected ¦ unexpectedLinkedOperation ) |  |
| Detailed Comments: | Type restricted to these 8. |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | ISDN_AddressString |
| Comments: | GSM 09.02, 14.7.8 |
| Type Definition |  |
| AddressString (SIZE ( 1 .. maxISDN_AddressLength )) |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | ISDN_SubaddressString |
| Comments: | GSM 09.02, 14.7.8 |
| Type Definition |  |
| OCTET STRING (SIZE ( 1 .. maxISDN_SubaddressLength )) |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | NetworkResource |
| Comments: | GSM 09.02, 14.7.8, |
| Type Definition |  |
| ENUMERATED { plmn (0), hlr (1), vlr (2), pvlr (3), controllingMSC (4), vmsc (5), eir (6), rss (7) } |  |
| Detailed Comments: | Elements of INTEGER are global for the ATS. |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | NoReplyConditionTime |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| INTEGER (5 .. 30 ) |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | Operation |
| Comments: | ITU-T Q.767 |
| Type Definition |  |
| CHOICE { localValue INTEGER, globalValue OBJECT IDENTIFIER} |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | OverrideCategory |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| ENUMERATED { overrideEnabled (0), overrideDisabled (1) } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | Password |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| NumericString (FROM ("0"¦"1"¦"2"¦"3"¦"4"¦"5"¦"6"¦"7"¦"8"¦"9")) (SIZE (4)) |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | PW_RegistrationFailureCause |
| Comments: | GSM 09.02, 14.7.7 |
| Type Definition |  |
| ENUMERATED { undetermined (0), invalidFormat (1), newPasswordsMismatch (2) } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | ReturnErrorProblem |
| Comments: | ITU-T Q.767 |
| Type Definition |  |
| TCAP_Problems ( unrecognizedInvokeID ¦ returnErrorUnexpected ¦ unrecognizedError ¦ unexpectedError ¦ mistypedParameter ) |  |
| Detailed Comments: | Type restricted to these 5. |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | ReturnResultProblem |
| Comments: | ITU-T Q.767 |
| Type Definition |  |
| TCAP_Problems ( unrecognizedInvokeID ¦ returnResultUnexpected ¦ mistypedResult ) |  |
| Detailed Comments: | Type restricted to these three. |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | SS_Code |
| Comments: | GSM 09.02, 14.7.5 |
| Type Definition |  |
| OCTET STRING (SIZE (1)) |  |
| Detailed Comments: | group (bits 8765), and specific service (bits 4321) |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | SS_Data |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| SEQUENCE { ss_Code SS_Code OPTIONAL, ss_Status [4] IMPLICIT SS_Status OPTIONAL, ss_SubscriptionOption SS_SubscriptionOption OPTIONAL, basicServiceGroupList BasicServiceGroupList OPTIONAL } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | SS_ForBS_Code |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| SEQUENCE { ss_Code SS_Code, basicService BasicServiceCode OPTIONAL } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | SS_IncompatibilityCause |
| Comments: | GSM 09.02, 14.7.8 |
| Type Definition |  |
| SEQUENCE { ss_Code [1] IMPLICIT SS_Code OPTIONAL, basicService BasicServiceCode OPTIONAL, ss_Status [4] SS_Status OPTIONAL } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | SS_Info |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| CHOICE { forwardingInfo [0] IMPLICIT ForwardingInfo, callBarringInfo [1] IMPLICIT CallBarringInfo, cug_Info [2] IMPLICIT CUG_Info, ss_Data [3] IMPLICIT SS_Data } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | SS_Status |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| OCTET STRING (SIZE (1)) |  |
| Detailed Comments: | bits 8765: 0000 unused, bits 4: SS state information Q bit, bits 3: SS state information P bit, bits 2: SS state information R bit, bits 1: SS state information A bit. |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | SS_SubscriptionOption |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| CHOICE { overrideCategory [1] IMPLICIT OverrideCategory, cliRestrictionOption [2] IMPLICIT CliRestrictionOption } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | SS_UserData |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| IA5String (SIZE (1 .. maxSignalInfoLength)) |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | TCAP_Problems |
| Comments: | ITU-T Q.767 |
| Type Definition |  |
| INTEGER { unrecognizedComponent (0), -- GeneralProblem unrecognizedInvokeID (0), -- ReturnResultProblem, ReturnErrorProblem duplicateInvokeID (0), -- InvokeProblem mistypedComponent (1), -- GeneralProblem returnErrorUnexpected (1), -- ReturnErrorProblem returnResultUnexpected (1), -- ReturnResultProblem unrecognizedOperation (1), -- InvokeProblem badlyStructuredComponent (2), -- GeneralProblem unrecognizedError (2), -- ReturnErrorProblem mistypedArgument (2), -- InvokeProblem, originally called mistypedParameter in TCAP mistypedResult (2), -- ReturnResultProblem resourceLimitation (3), -- InvokeProblem unexpectedError (3), -- ReturnErrorProblem mistypedParameter (4), -- ReturnErrorProblem initiatingRelease (4), -- InvokeProblem unrecognizedLinkedID (5), -- InvokeProblem linkedResponseUnexpected (6), -- InvokeProblem unexpectedLinkedOperation (7) -- InvokeProblem } |  |
| Detailed Comments: | Errors of the same integer value are distinguished by their different parent types (General, Invoke, ReturnResult, ReturnError). |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | TeleserviceCode |
| Comments: | GSM 09.02, 14.7.9 |
| Type Definition |  |
| OCTET STRING (SIZE (1)) |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | USSD_Arg |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| SEQUENCE { ussd_DataCodingScheme USSD_DataCodingScheme, ussd_String USSD_String } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | USSD_Res |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| SEQUENCE { ussd_DataCodingScheme USSD_DataCodingScheme, ussd_String USSD_String } |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | USSD_DataCodingScheme |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| OCTET STRING (SIZE (1)) |  |
| Detailed Comments: |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name: | USSD_String |
| Comments: | GSM 09.02, 14.7.4 |
| Type Definition |  |
| OCTET STRING (SIZE (1..maxUSSD_StringLength)) |  |
| Detailed Comments: | The structure of the contents of the USSD-String is dependent on the USSD-DataCodingScheme as described in TS GSM03.38. |

## Test suite operation definitions

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_Asn1intToOct(n:Asn1Integer; l: INTEGER) |
| Result Type: | OCTETSTRING |
| Comments: |  |
| Description |  |
| OC_Asn1intToOct converts the ASN.1 INTEGER `n` into OCTETSTRING with length = `l`. for example: OC_Asn1intToOct(14,1) = '0E'O; OC_Asn1intToOct(18,1) = '12'O; OC_Asn1intToOct(18,2) = '0012'O; OC_Asn1intToOct(-128,1) = '80'O (MSB (position p out of 1...p) represents -2exp(p-1) ); OC_Asn1intToOct(-32768,2) = '8000'O (MSB (position p out of 1...p) represents -2exp(p-1) ). |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_Bcap(setup:SETUP_MO_PDU; callproc:CALL_PROC_PDU; mem:INTEGER) |
| Result Type: | BCAP |
| Comments: |  |
| Description |  |
| OC_Bcap operation returns a bearer capability IE according the following rule: - for `mem` = 1: - if bearer capability IE were presented in `callproc` the returned BC is the bearer capability 1 of the `callproc`; - otherwise, the returned bearer capability is the bearer capability 1 of the `setup`. - for `mem` = 2: - if bearer capability IE were presented in `callproc` the returned BC is the bearer capability 2 of the `callproc`; - otherwise, the returned bearer capability is the bearer capability 2 of the `setup`. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_BinAdd(bitstr1:BITSTRING; bitstr2:BITSTRING) |
| Result Type: | BITSTRING |
| Comments: |  |
| Description |  |
| OC_BinAdd operation performs binary addition of two input parameters `bitstr1` and `bitstr2`, then returns the result of the addition. These two input parameters shall have the same length, the result of the operation has the same length as the input parameters. for example: OC_BinAdd('01000'B, '00110'B) = '01110'B; OC_BinAdd('01000'B, '00011'B) = '01011'B; OC_BinAdd('00100'B, '00010'B) = '00110'B. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_Bit7(bstring: B_8) |
| Result Type: | BITSTRING |
| Comments: | The input parameter bstring is of type BITSTRING[8]. |
| Description |  |
| OC_Bit7(bstring) returns the value of bit 7 in the `bstring`. for example: OC_Bit7('01010101'B) = '1'B, OC_Bit7('10101010'B) = '0'B |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_BCDtoInt(bcdstring:HEXSTRING; relevant_digits: INTEGER) |
| Result Type: | INTEGER |
| Comments: |  |
| Description |  |
| The operation OC_BCDtoInt converts last 'relevant_digits' of an HEXSTRING containing BCD coded digits to an integer representation of these relevant digits. Example: OC_BCDtoInt('12345'H, 3) := 345 |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_CalledNumCHK(callednum:OCTETSTRING; dialnum:IA5String) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| the operation OC_CalledNumCHK to check whether the called party number `callednum`, which is represented by OCTETSTRING, is the same as the dialed number 'dialnum`, which is represented by IA5String. It returns TRUE if they are the same, otherwise FALSE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_ChkSRES(sres:OCTETSTRING; ki: BITSTRING; rand:BITSTRING) |
| Result Type: | BOOLEAN |
| Comments: | sres is 32 bits value. |
| Description |  |
| OC_ChkSRES checks the input parameter `sres` according to the authentication algorithm defined in the following procedure. It returns TRUE if the `sres` is correct, otherwise it returns FALSE. - firstly the `ki` XOR to the `rand` results in RES1; - then compare the most significant 32 bits of the RES1 with the `sres`; - if they are equal, the `sres` is correct and the operation returns TRUE; - if they are not equal, the `sres` is wrong and the operation returns FALSE. NOTE: this procedure is the test algorithm for authentication defined by GSM. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_CnvtMax(max:INTEGER) |
| Result Type: | B_2 |
| Comments: |  |
| Description |  |
| The allowed integer values for max are 1, 2, 4, 7 (maximum number of retransmissions). The result BITSTRING is 2 bits long. OC_CnvtMax converts the input integer `max` into a BITSTRING according to the following rule: 1. the result is '00'B if the `max` = 1, 2. the result is '01'B if the `max` = 2, 3. the result is '10'B if the `max` = 4, 4. the result is '11'B if the `max` = 7. for example: OC_CnvtMax(1) = '00'B, OC_CvntMax(7) = '11'B. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_CnvtTx(tx:INTEGER) |
| Result Type: | B_4 |
| Comments: | The result BITSTRING is 4 bits long. |
| Description |  |
| OC_CnvtTx converts the input integer `tx` into a BITSTRING according the following rule: 1. the result is '0000'B if the `tx` = 3, 2. the result is '0001'B if the `tx` = 4, 3. the result is '0010'B if the `tx` = 5, 4. the result is '0011'B if the `tx` = 6, 5. the result is '0100'B if the `tx` = 7, 6. the result is '0101'B if the `tx` = 8, 7. the result is '0110'B if the `tx` = 9, 8 . the result is '0111'B if the `tx` = 10, 9. the result is '1000'B if the `tx` = 11, 10. the result is '1001'B if the `tx` = 12, 11. the result is '1010'B if the `tx` = 14, 12. the result is '1011'B if the `tx` = 16, 13. the result is '1100'B if the `tx` = 20, 14. the result is '1101'B if the `tx` = 25, 15. the result is '1110'B if the `tx` = 32, 16. the result is '1111'B if the `tx` = 50. for example: OC_CnvtTx(3) = '0000'B, OC_CvntTx(5) = '0010'B. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_CodeSMSCBMessage(firstoct: INTEGER; lastoct: INTEGER) |
| Result Type: | OCTETSTRING |
| Comments: |  |
| Description |  |
| The operation codes a part of the contents for a cell broadcast short message. The cell broadcast short message, is 82 octets long, i.e. 93 characters, each represented by 7 bits. As many as possible different characters are sent, the characters are those corresponding to the 7-bit representation of the integers 0, 1, 2, ..., 92. The bits are arranged acc. to GSM 03.38, clause 6.1.2.1.1. The result of this operation is the octetstring of the octets 'firstoct' to 'lastoct' (16 octets for the first message block, 22 octets for the 2nd, 3rd and 4th blocks), with the octets of the cell broadcast short message being numbered from 1 to 82. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_CodingOfUssdString(text: IA5String) |
| Result Type: | OCTETSTRING |
| Comments: |  |
| Description |  |
| This operation provides the coding for a USSD String 'text' in the default alpabet, each character represented by 7 bits. The bits are arranged acc. to GSM 03.38, clause 6.1.2.2.1. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_ComputeSMContents(NumberOfCharacters: INTEGER) |
| Result Type: | OCTETSTRING |
| Comments: | max. 160 characters, i.e. 140 octets. |
| Description |  |
| This operation provides a short message's contents with a specified number of characters 'NumberOfCharacters', each represented by 7 bits. As possible different characters are sent, the characters are those corresponding to the 7-bit representation of 0, 1, 2, ... up to ('NumberOfCharacters' - 1). If more than 128 characters are sent, the rest of the characters is the corresponding to 0, 1, ... up to (NumberOfCharacters - 128 - 1), e.g. for 160 characters: 0, 1, ..., 127, 0, 1, ..., 31. The bits are arranged acc. to GSM 03.38, clause 6.1.2.1.1. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_ComputeSMContentsSpecText(NumbOfIA5chara: INTEGER; text: IA5String) |
| Result Type: | OCTETSTRING |
| Comments: | max. 160 characters, i.e. 140 octets. |
| Description |  |
| This operation provides a short message's contents with a specified number of characters 'NumbOfIA5chara', each represented by 7 bits. 'text' is used as contents of the short message. If "text' contains less than 'NumberOfCharacters' characters, 'text' is repeated until the short message reaches the 'NumberOfCharacters' characters long. The bits packing is arranged according. to GSM 03.38, clause 6.1.2.1.1. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_CphKeyGen(ki:BITSTRING; rand:BITSTRING) |
| Result Type: | BITSTRING |
| Comments: | both ki and rand are 128 bits values, the result of the operation is 64 bits value |
| Description |  |
| OC_CphKeyGen generates the ciphering key from the input parameters according to the following procedure: - firstly the `ki` XOR to the `rand` results in RES1; - then discard the most significant 32 bits of the RES; - the next 64 bits of RES1 are the ciphering key, the operation returns this value. - the 32 least singnificant bits are not used. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_FirstDigi(bcddigits:HEXSTRING) |
| Result Type: | B_4 |
| Comments: |  |
| Description |  |
| The input parameter bcddigits shall be a BCD string (subset of HEXSTRING), the resut is a BITSTRING[4] of a binary representation of one BCD digit. The function of the OC_FirstDigi is to return the first (most significant) digit of the input parameter `bcddigits`. for example: OC_FirstDigi('12345') = '0001'B, OC_FirstDigi('012345678') = '0000'B. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_FnInc(fn:FN; inc:INTEGER) |
| Result Type: | FN |
| Comments: |  |
| Description |  |
| The function of OC_FnInc operation to increase frame number with `inc`. The frame number to be incremented is the input parameter `fn` in FN type and the increment `inc` is in INTEGER type, the incremented frame number is returned in FN type. OC_FnInc(fn, inc) FN fn; INTEGER inc; { int fmrin; FN frmout; frmin = 51 * ((fn.t3 - fn.t2) MOD 26) + fn.t3 + 1326 * fn.t1_ ; frmout.t1_ = ((frmin + inc) DIV 1326) MOD 32; frmout.t2 = (frmin + inc) MOD 26; frmout.t3 = (frmin + inc) MOD 51; return (frmout); } |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_GetSCTimeStamp(timezone:TZONES) |
| Result Type: | TPSCTS |
| Comments: | TPSCTS is HEXSTRING[14] |
| Description |  |
| This Operation provides the hexstring containing the service center time stamp (SCTS) according to GSM 03.40, clauses 9.2.2.1 and 9.2.3.11. The TSO reads the current time of the test systems clock and transforms the time in conbination with the input parameter 'timezone' into a service center time stamp. Example: 1996 April 18, 15:32:46, timezone=4 OC_GetSCTimeStamp returns 69408151236440 |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_IncTmsi(tmsi:TMSI_V; inc:OCTETSTRING) |
| Result Type: | TMSI_V |
| Comments: | the `tmsi` is 4 OCTETs long |
| Description |  |
| OC_IncTmsi operation adds the two input parameters and returns the result. An overflow of addition is allowed. For example : OC_IncTmsi('33542140'O + '01'O) = '33542141'O; OC_IncTmsi('21322140'O + '08'O) = '21322148'O. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_InRang(tx: INTEGER; maxret:INTEGER; m: INTEGER) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| The operation returns TRUE if (m DIV (maxret * ((230+maxret -1)/maxret))) is inside the following interval: [0.8 - ((tx+1)/2) DIV tx; 1.2 - ((tx+1)/2) DIV tx] where / is integer division, DIV is float division, m, tx and maxret are input parameters of the operation. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_IntToOct(n:INTEGER; l: INTEGER) |
| Result Type: | OCTETSTRING |
| Comments: |  |
| Description |  |
| OC_IntToOct converts the INTEGER `n` into OCTETSTRING with length = `l`. for example: OC_IntToOct(14,1) = '0E'O; OC_IntToOct(18,1) = '12'O; OC_IntToOct(18,2) = '0012'O. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_LeastBits(bstring:BITSTRING; lg:INTEGER) |
| Result Type: | BITSTRING |
| Comments: |  |
| Description |  |
| OC_LeastBits operation returns the `lg` least significant bits of the original `bstring`. for example: OC_LeastBits('110011000101010'B, 3) = '010'B, OC_LeastBits('110011000101010'B, 6) = '101010'B. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_LengthOf(identity_field :MI) |
| Result Type: | INTEGER |
| Comments: |  |
| Description |  |
| OC_LengthOf operation returns the actual length of the `identity_field`, the unit of length is in OCTET. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_LengthOfBCDN(bcdn :BCDN) |
| Result Type: | INTEGER |
| Comments: |  |
| Description |  |
| OC_LengthOfBCDN operation returns the actual length of an IE of type BCDN 'bcdn', the unit of length is in OCTET. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_LengthOfComp(comp: Component_T) |
| Result Type: | LENGTH |
| Comments: |  |
| Description |  |
| OC_LengthOfComp operation returns the actual length of the Component_T `comp`, the unit of length is in OCTET. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_LengthOfString(strg: IA5String) |
| Result Type: | INTEGER |
| Comments: |  |
| Description |  |
| OC_LengthOfString operation returns the actual length (number of characters) of the string `strg`. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_LookupS(t: INTEGER; combined :BOOLEAN) |
| Result Type: | INTEGER |
| Comments: | the algorithm is derived from Table 3.1/GSM 04.08 for values of parameter S. |
| Description |  |
| This operation returns an INTEGER according the following algorithm: OC_LookupS( t, comb) INTEGER t; BOOLEAN combined; { switch (t) { case 3: case 8: case 14: case 50: if (combined) then return(41) else return(55); break; case 4: case 9: case 16: if (combined) then return(52) else return(76); break; case 5: case 10: case 20: if (combined) then return(58) else return(109); break; case 6: case 11: case 25: if (combined) then return(86) else return(163); break; case 7: case 12: case 32: if (combined) then return(115) else return(217); break; } } |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_MostBits(bstring:BITSTRING; lg :INTEGER) |
| Result Type: | BITSTRING |
| Comments: |  |
| Description |  |
| OC_MostBits operation returns the `lg` most significant bits of the original `bstring`. for example: OC_LeastBits('110011000101010'B, 3) = '110'B, OC_LeastBits('110011000101010'B, 6) = '110011'B. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_MsrReptChk(msrres: MSRR; index:INTEGER) |
| Result Type: | BOOLEAN |
| Comments: | parameter index is used for selecting different match pattern |
| Description |  |
| OC_MsrReptChk operation checks whether the received measurement report contains correct values: 1. when the `index` = 1: if set { { msrres.bcchfrq_nc1, msrres.bsic_nc1}, { msrres.bcchfrq_nc2, msrres.bsic_nc2}, { msrres.bcchfrq_nc3, msrres.bsic_nc3}, { msrres.bcchfrq_nc4, msrres.bsic_nc4}, { msrres.bcchfrq_nc5, msrres.bsic_nc5}, { msrres.bcchfrq_nc6, msrres.bsic_nc6} } equals to set { {'00000'B, '001011'B}, {'01111'B, '001001'B}, {'10101'B, '001011'B}, {'10110'B, '001101'B}, {'11100'B, '001111'B}, {'11111'B, '001001'B} } then the operation returns TRUE otherwise FALSE. 2. when the `index` = 2: if set { { msrres.bcchfrq_nc1, msrres.bsic_nc1}, { msrres.bcchfrq_nc2, msrres.bsic_nc2}, { msrres.bcchfrq_nc3, msrres.bsic_nc3}, { msrres.bcchfrq_nc4, msrres.bsic_nc4}, { msrres.bcchfrq_nc5, msrres.bsic_nc5}, { msrres.bcchfrq_nc6, msrres.bsic_nc6} } equals to set { {'00000'B, '001011'B}, {'00100'B, '001001'B}, {'00110'B, '001011'B}, {'00101'B, '001101'B}, {'00010'B, '001111'B}, {'00111'B, '001001'B} } then the operation returns TRUE otherwise FALSE. 3. when the `index` = 3: if set { { msrres.bcchfrq_nc1, msrres.bsic_nc1}, { msrres.bcchfrq_nc2, msrres.bsic_nc2}, { msrres.bcchfrq_nc3, msrres.bsic_nc3}, { msrres.bcchfrq_nc4, msrres.bsic_nc4}, { msrres.bcchfrq_nc5, msrres.bsic_nc5}, { msrres.bcchfrq_nc6, msrres.bsic_nc6} } equals to set { {'00001'B, '001011'B}, {'00101'B, '001001'B}, {'00111'B, '001011'B}, {'00110'B, '001101'B}, {'00011'B, '001111'B}, {'01000'B, '001001'B} } then the operation returns TRUE otherwise FALSE. 4. when the `index` = 4 : if set { { msrres.bcchfrq_nc1, msrres.bsic_nc1}, { msrres.bcchfrq_nc2, msrres.bsic_nc2}, { msrres.bcchfrq_nc3, msrres.bsic_nc3}, { msrres.bcchfrq_nc4, msrres.bsic_nc4}, } equals to set { {'00000'B, '001011'B}, {'00010'B, '001111'B}, {'00001'B, '001111'B}, {'00011'B, '001001'B} } then the operation returns TRUE otherwise FALSE. 5. when the `index` = 5 : if set { { msrres.bcchfrq_nc1, msrres.bsic_nc1}, { msrres.bcchfrq_nc2, msrres.bsic_nc2}, { msrres.bcchfrq_nc3, msrres.bsic_nc3}, { msrres.bcchfrq_nc4, msrres.bsic_nc4}, } equals to set { {'00000'B, '001011'B}, {'00010'B, '001111'B}, {'00001'B, '001111'B}, {'00100'B, '001001'B} } then the operation returns TRUE otherwise FALSE. 6. when the `index` = 6: if set { { msrres.bcchfrq_nc1, msrres.bsic_nc1}, { msrres.bcchfrq_nc2, msrres.bsic_nc2}, { msrres.bcchfrq_nc3, msrres.bsic_nc3}, { msrres.bcchfrq_nc4, msrres.bsic_nc4}, { msrres.bcchfrq_nc5, msrres.bsic_nc5}, { msrres.bcchfrq_nc6, msrres.bsic_nc6} } equals to set { {'00000'B, '001011'B}, {'00011'B, '001001'B}, {'00100'B, '001011'B}, {'00101'B, '001101'B}, {'00110'B, '001111'B}, {'00111'B, '001001'B} } then the operation returns TRUE otherwise FALSE. 7. when the `index` = 7: if set { { msrres.bcchfrq_nc1, msrres.bsic_nc1}, { msrres.bcchfrq_nc2, msrres.bsic_nc2}, { msrres.bcchfrq_nc3, msrres.bsic_nc3}, { msrres.bcchfrq_nc4, msrres.bsic_nc4}, { msrres.bcchfrq_nc5, msrres.bsic_nc5}, { msrres.bcchfrq_nc6, msrres.bsic_nc6} } equals to set { {'00000'B, '001011'B}, {'00100'B, '001001'B}, {'00110'B, '001011'B}, {'00101'B, '001101'B}, {'00010'B, '001111'B}, {'00111'B, '001001'B} } then the operation returns TRUE otherwise FALSE. 8. when the `index` = 8 : if set { { msrres.bcchfrq_nc1, msrres.bsic_nc1}, { msrres.bcchfrq_nc2, msrres.bsic_nc2} } equals to set { {'00000'B, '001011'B}, {'00010'B, '001101'B} } then the operation returns TRUE otherwise FALSE. 9. when the `index` = 9 : if set { { msrres.bcchfrq_nc1, msrres.bsic_nc1}, { msrres.bcchfrq_nc2, msrres.bsic_nc2} } equals to set { {'00110'B, '001011'B}, {'00111'B, '001101'B} } then the operation returns TRUE otherwise FALSE. 10. when the `index` = 10 : if set { { msrres.bcchfrq_nc1, msrres.bsic_nc1}, { msrres.bcchfrq_nc2, msrres.bsic_nc2} } equals to set { {'00011'B, '001011'B}, {'00101'B, '001101'B} } then the operation returns TRUE otherwise FALSE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_OeBit(bcddigits:HEXSTRING) |
| Result Type: | BITSTRING |
| Comments: | The input parameter `bcddigits` is really BCD string (subset of HEXSTRING), the result is BITSTRING[1]. |
| Description |  |
| The function of the OC_OeBit is as the following: 1. it returns '1'B, if the length of the `bcddigits` is odd, 2. it returns '0'B, if the length of the `bcddigits` is even. for example: OC_OeBit('12583') = '1'B, OC_OeBit('87259957') ='0'B. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_OctToInt(ostr: OCTETSTRING) |
| Result Type: | INTEGER |
| Comments: |  |
| Description |  |
| OC_OctToInt converts the OCTETSTRING `ostr` into INTEGER. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_OctToInvokeIDType(o:OCTETSTRING) |
| Result Type: | InvokeIDType |
| Comments: |  |
| Description |  |
| OC_OctToInvokeIDType converts the OCTETSTRING `o` into InvokeIDType, with the MSB of 'o' representing the negative value -2exp(n-1), for example '80'O->'10000000'B-> -128. The rest of the bits can code positive values up to +2exp(n-2)-1, for example '4F'O->'01111111'B-> +127. for example: OC_OctToInvokeIDType('80'O) = -128; OC_OctToInvokeIDType('81'O) = -128+1 = -127; OC_OctToInvokeIDType('40'O) = +64. OC_OctToInvokeIDType('7F'O) = +127. OC_OctToInvokeIDType('C0'O) = -128+64 = -64. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_OtherDigi(bcddigits:HEXSTRING) |
| Result Type: | OCTETSTRING |
| Comments: | The input parameter` bcddigits` is really BCD string (subset of HEXSTRING), the result is an even BCD digits except that the next to last may either be 'F'H or a BCD digit. |
| Description |  |
| The function of the OC_OtherDigi is as the following: 1. If the number of the `bcddigits` is odd, the operation removes the most significant digit, and then reverses the order of each pair of digits; 2. If the number of the `bcddigits` is even, first the operation suffixes the `bcddigits` with 'F'H, then removes the most significant digit, and then reverses the order of each pair of digits. for example: OC_OtherDigi('12345') = '3254', OC_OtherDigi('12345678') ='325476F8'. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_PosinSet(set: Components; comp: Component) |
| Result Type: | INTEGER |
| Comments: |  |
| Description |  |
| The operation OC_PosinSet returns the position of component `comp` within the SET `set`. for example: if the set = { registerSSComponents, eraseSSComponents, activateSSComponents, deactivateSSComponents } OC_PosinSet(set, registerSSComponents) = 0, OC_PosinSet(set, activateSSComponents) = 2. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_RachSlots(fn1:FN; fn2:FN; cmbn: BOOLEAN; mode:INTEGER) |
| Result Type: | INTEGER |
| Comments: |  |
| Description |  |
| OC_RachSlots calculates the number of RACH slots between frame number `fn1` and `fn2` excluding the slots in `fn1` and `fn2`, then return the result of the calculation. If mode=0, 'fn1' denotes the (first) frame number for sending Paging Request, while 'fn2' denotes the (first) frame number for the received Channel Request. If mode=1 'fn1' and 'fn2' denote the (first) frame number for the received two subsequent Channel Requests. Any other values for mode is not allowed. Since the number of RACH slots depends on the RACH being combined or not with dedicated channels, the parameter 'cmbn' is needed: TRUE --> combined, FALSE--> not combined. It is assumed that the distance between fn1 and fn2 is not more than one group of 42432 TDMA frames (modulo 42432 is used to calculate t1' of fn1 and fn2). This is equivalent to approximately 196 seconds. The 51 TDMA frames in a multiframe are numbered 0 to 50. fn1' := fn1 mod 51 fn2' := fn2 mod 51 CASE 1: cmbn is FALSE ('not combined') When 'not combined', all slots are RACH slots. 1.1 mode=0 (sending paging message at fn1) The paging uses the numbered frames 6 - 9, 12 -19, 22-29, 32-39 and 42-49. 1.1.1 If fn1 is not in the paging TDMA frame mapping, it leads testing to a test system error. IF (fn1' < 6) OR (9 < fn1' < 12) OR (19 < fn1' < 22) OR (29 < fn1' < 32) OR (39 < fn1' < 42) OR (fn1' >49), A test system error! The calling test case shall re-run again. OTHERWISE 1.1.2 The number of RACH slots is equal to the number of frames between fn1 + 3 and fn2. The fn1 is added by 3 because a page message occupies 4 slots. The fn1 indicates the first slot of the paging message. IF (fn1 + 3) < fn2, OC_RachSlots := fn2 - (fn1 + 3) -1 = fn2 - fn1 - 4, 1.1.3 If fn1+3 is equal to or greater than fn2, then this is due to fn2 being in the next group of 42432 frames. In this case 42432 frames have to be added. IF fn2 <= (fn1 + 3), OC_RachSlots := fn2 -fn1 - 3 + 42432 -1 = fn2 -fn1 + 42428 1.2 mode=1 (receiving channel request at fn1) The number of RACH slots is equal to the number of frames between fn1 and fn2 1.2.1 IF fn1 < fn2, OC_RachSlots := fn2 - fn1 - 1, 1.2.2 IF fn2 <= fn1, OC_RachSlots := fn2 -fn1 + 42432 -1 = fn2 -fn1 + 42431 CASE 2: cmbn is TRUE ('combined') When combined only the slots of the numbered frames 4, 5, 14 to 36, 45 and 46 in each multiframe are RACH slots, i.e.total 27 frames per multiframe. 2.1 mode=0 (sending paging message at fn1) The paging uses the numbered frames 6 - 9 and 12 -19. 2.1.1 If fn1 is not in the paging TDMA frame mapping, it leads testing to a test system error. If fn2 is not in the RACH TDMA frame mapping, it leads testing to fail. IF (fn2` < 4) OR (5 < fn2` < 14) OR (36 < fn2` < 45) OR (fn2' > 46), OC_RachSlots := -9999 IF (fn1' < 6) OR (9 < fn1' < 12) OR (fn1' >19), A test system error! The calling test case shall re-run again. OTHERWISE 2.1.2 Calculation of the number 'c' multiframes between fn1 + 3 and fn2. If fn1 + 3 is equal to or greater than fn2, then this is due to fn2 being in the next group of 42432 frames. In this case 42432 frames have to be added. '/' shall be the integer division, i.e. the result is also integer. Fractions are discarded. 2.1.2.1 IF (fn1 + 3) < fn2, c := fn2 / 51 - (fn1 + 3) / 51 2.1.2.2 IF fn2 <= (fn1 + 3), c := fn2 / 51 - (fn1 + 3) / 51 + 42432 / 51 = fn2 / 51 - (fn1 + 3) / 51 + 832 2.1.3 Calculation of the number of frames 'a' to be subtracted according to the position of fn1' within the multiframe IF (5 < fn1` < 9) , a := 2 IF (fn1` =9), a := 3 IF (11<fn1` <20), a := fn1'-8 2.2 mode=1 (receiving channel request at fn1) 2.2.1 If fn1 or fn2 are not in the RACH TDMA frame mapping, it leads testing to fail. IF (fn2` < 4) OR (5 < fn2` < 14) OR (36 < fn2` < 45) OR (fn2' > 46) OR (fn1' < 6) OR (9 < fn1' < 12) OR (fn1' >19), OC_RachSlots := -9999 2.2.2 Calculation of the number 'c' multiframes between fn1 and fn2 2.2.2.1 IF fn1 < fn2, c := fn2 / 51 - fn1 / 51 2.2.2.2 IF fn2 <= fn1 c := fn2 / 51 - fn1 / 51 + 42432 / 51 = fn2 / 51 - fn1/ 51 + 832 2.2.3 Calculation of the number of frames 'a' to be subtracted according to the position of fn1' within the multiframe IF (3 < fn1' < 6:), a:= fn1' - 3 IF (13 < fn1' < 37), a:= fn1' - 11 IF (44 < fn1' < 47), a:= fn1' - 19 2.3 Calculation of the number of slots 'b' to be added according to the position of fn2' within the multiframe IF (3 < fn2' < 6:), b:= fn2' - 4 IF (13 < fn2' < 37), b:= fn2' - 12 IF (44 < fn2' < 47), b:= fn2' - 20 2.4 Calculation of the number of RACH slots. There are 27 RACH slots in each multiframe. OC_RachSlots := 27 * c + b - a |  |
| Detailed Comments: | TC_26_2_1_1 uses mode=0. TC_26_2_1_2 uses mode=1. |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_Random(n1:INTEGER; n2:INTEGER) |
| Result Type: | INTEGER |
| Comments: |  |
| Description |  |
| This operation randomly returns one number from the following candidates: 'n1', 'n1'+1, ..., 'n2' |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_RcsdPresent(msg:MODIFY_PDU) |
| Result Type: | BOOLEAN |
| Comments: | To check if RCSD IE is present or not in Modify PDU |
| Description |  |
| IF RCSD IE is present in the PDU passed on input paremeter return TRUE ELSE return FALSE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_SaveAndProc1(val:INTEGER; mode :INTEGER; cnt:INTEGER; cmbnd:BOOLEAN) |
| Result Type: | BOOLEAN |
| Comments: | This operation is used for storing and analysing the CHANNEL REQUEST message distribution. |
| Description |  |
| The function of this operation is defined as an pseudo C code: #define SAVE 0; #define PROC 1; OC_SaveAndProc(val, mode,cnt) INTEGER val, mode,cnt; BOOLEAN combined; { static INTEGER buf[200]; int i, j, n; if (mode == SAVE) { cnt = cnt mod 200; if (cmbnd == C_NotCombined) AND (val<(151 + 8)) then { buf[cnt] = val; return (TRUE); } if (cmbnd == C_Combined) AND (val<(81 + 8)) then { buf[cnt] = val; return (TRUE); } else return (FALSE); } if (mode == PROC) { for (i=0, i<200, i++) { n=0; for (j=0, j<200, j++) if (buf[j] == buf[i]) n = n+1; if (n > 41) return (FALSE); } return(TRUE); } } |  |
| Detailed Comments: | This test suite operation has two operation modes : - when the mode is SAVE and the val is less than 89 for combined or 159 for non-combined, it saves the value val into internal buffer and returns TRUE, otherwise returns FALSE. 200 values will be stored in the internal buffer when the operation is invoked 200 times. - When the mode is PROC, it analyses the values stored in the internal buffer, if no more than 41 of them are equal the operation returns TRUE, otherwise returns FALSE. ( i.e. for all n, CARD{ k¦f(k) = n } <= 41) |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_SaveAndProc3(val:BITSTRING; mode :INTEGER; cnt:INTEGER) |
| Result Type: | BOOLEAN |
| Comments: | This operation is used for storing and analysing the random reference. |
| Description |  |
| The function of this operation is defined as an pseudo C code: #define SAVE 0; #define PROC 1; OC_SaveAndProc(val, mode,cnt) BITSTRING[8] val; INTEGER mode,cnt; { static BITSTRING[8] buf[7]; int i, j, n=7; if (mode == SAVE) { cnt = cnt mod 7; buf[cnt] = val; return(TRUE); } if (mode == PROC) { for (i=0, i<6, i++) { for (j=i+1, j<7, j++) if (buf[j] == buf[i]) { n = n-1; break; } } if (n>=4) then return(TRUE) else return (FALSE); } } |  |
| Detailed Comments: | The function of this test suite operation is : - it saves the value val into internal buffer when the mode = SAVE. 7 values will be stored in the internal buffer when the operation is invoked 7 times. - it compares the values stored in the internal buffer when the mode = PROC, if 4 or more than 4 of them are different the operation returns TRUE, otherwise returns FALSE. |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_SaveAndRetrv(val:RQR; mode, idx, ex: INTEGER) |
| Result Type: | RQR |
| Comments: | This operation is used for storing and retrieving the random reference. |
| Description |  |
| The function of this test suite operation is (in prose): - it saves the value val into internal buffer buf[idx] when the mode = SAVE and ex = 0. Maximum 9 values can be stored. The idx is ranged between 1-9. - it returns the value stored in the internal buffer buf[idx] when the mode = RETRV and ex = 0. - it returns a value which is different from any value stored in the buf and also different from any values which have been returned by the consecutive invocation of this operation with ex <> 0, if ex <> 0. The idx is ranged between 10-13. The function of this operation is defined as an pseudo C code: #define SAVE 0; #define RETRV 1; OC_SaveAndRetrv(val, mode, idx, ex) RQR val; INTEGER mode, idx, ex; { int i, j; static RQR buf[14]; if ((mode == SAVE) AND (ex == 0)) { buf[idx] = val; buf[10].ra = '00000000'B; buf[10].fn = '00'O; buf[11].ra = '00000000'B; buf[11].fn = '00'O; buf[12].ra = '00000000'B; buf[12].fn = '00'O; buf[13].ra = '00000000'B; buf[13].fn = '00'O; return (buf[10]; } if ((mode == RETRV) AND (ex == 0)) return (buf[idx]); if (ex != 0) { buf[0].ra = buf[1].ra + '00000001'B; buf[0].fn = buf[1].fn; for (j=1, j <13, j++) { for (i=1, i<14, i++) if( buf[0] == buf[i]) { buf[0].ra = buf[i].ra + '00000001'B; break; } if (i == 14) { buf[idx].ra = buf[0].ra: return (buf[0]); } } buf[idx].ra = buf[i].ra+'00000001'B; return (buf[idx]); } } |  |
| Detailed Comments: | 0. The pseudo C code gives one of the possible implementations for the OC description in prose. 1. '00'O should be understood as equivalence of Fn_01, a TTCN structured type constraint. 2. The OC can save maximum 13 RQR values. In SAVE mode, idx is ranged between 1-9. 3. For ex<>0, there are maximum 13 scans. In each scan a new value is assigned to buf[0].ra. If there is a 'non-match' in one of the 13 scans the buf[0] is returned. In case of 'all-match' for the 13 scans, the pair of (buf[i].ra + '00000001' at the last scan, buf[idx].fn is returned (idx is ranged between 10 - 13). |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_StartTime(frmn:FN; t, i:INTEGER) |
| Result Type: | STRT |
| Comments: |  |
| Description |  |
| OC_StartTime operation generates the STARTING TIME IE according to the input parameters. (* frmn is the current frame number, t is the "delay" in applying the new frequencies, i is the contents of the starting time IE. *) OC_StartTime(frmn, t, i) INTEGER t, i; FN frmn; { int tmp; STRT strt; tmp = 51 * ((frmn.t3 - frmn.t2) MOD 26) + frmn.t3 + 1326 * frmn.t1_ ; tmp = (tmp + t) MOD 42432; strt.fn.t1_ = (tmp DIV 1326) MOD 32; strt.fn.t2 = tmp MOD 26; strt.fn.t3 = tmp MOD 51; strt.iei = OMIT; if (i == 1) strt.iei := '01111100'B /* if i=1 the information element identifier shall be included \ |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_SubOctet(src:OCTETSTRING; len :INTEGER) |
| Result Type: | OCTETSTRING |
| Comments: |  |
| Description |  |
| OC_SubOctet(src, len) is the octetstring of length `len` starting from the leftmost position of the source octetstring `src`. For example : OC_SubOctet('123456789ABCDEF'O, 4) = '12345678'O |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_SubchOfFacchh(subch: B_1; cell:CellID; inst:INTEGER) |
| Result Type: | LOGICCH |
| Comments: | subch is of type BITSTRING[1] |
| Description |  |
| OC_SubchOfFacchh operation returns a logic channel identifier for FACCHH subchannel indicated by the input parameters `subch`, `cell` and `inst`, where 'subch' is the subchannel number, `cell` is the cell identifier which the channel belongs to, `inst` is the instance of the channel. for example: OC_SubchOfFacchh('0'B, C_CellA, 1) = C_FACCHH0_A_1; OC_SubchOfFacchh('1'B, C_CellA, 1) = C_FACCHH1_A_1; OC_SubchOfFacchh('0'B, C_CellB, 1) = C_FACCHH0_B_1; OC_SubchOfFacchh('1'B, C_CellB, 1) = C_FACCHH1_B_1. pseudo C code definition for the operation as following : OC_SubchOfFacchh(subch, cell, inst) BITSTRING subch; CellID cell; INTEGER inst; { LOGICCH logch; if (subch=='0'B && cell == C_CellA) switch (inst) { case 1 : logch = C_FACCHH0_A_1; break; case 2 : logch = C_FACCHH0_A_2; break; case 3 : logch = C_FACCHH0_A_3; break; } if (subch=='0'B && cell == C_CellB) switch (inst) { case 1 : logch = C_FACCHH0_B_1; break; case 2 : logch = C_FACCHH0_B_2; break; case 3 : logch = C_FACCHH0_B_3; break; } if (subch=='0'B && cell == C_CellC) switch (inst) { case 1 : logch = C_FACCHH0_C_1; break; case 2 : logch = C_FACCHH0_C_2; break; case 3 : logch = C_FACCHH0_C_3; break; } if (subch=='1'B && cell == C_CellA) switch (inst) { case 1 : logch = C_FACCHH1_A_1; break; case 2 : logch = C_FACCHH1_A_2; break; case 3 : logch = C_FACCHH1_A_3; break; } if (subch=='1'B && cell == C_CellB) switch (inst) { case 1 : logch = C_FACCHH1_B_1; break; case 2 : logch = C_FACCHH1_B_2; break; case 3 : logch = C_FACCHH1_B_3; break; } if (subch=='1'B && cell == C_CellC) switch (inst) { case 1 : logch = C_FACCHH1_C_1; break; case 2 : logch = C_FACCHH1_C_2; break; case 3 : logch = C_FACCHH1_C_3; break; } return(logch); } |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_SubchOfSacchh(subch: B_1; cell:CellID; inst:INTEGER) |
| Result Type: | LOGICCH |
| Comments: | subch is of type BITSTRING[1] |
| Description |  |
| OC_SubchOfSacchh operation returns a logic channel identifier for SACCHH subchannel indicated by the input parameters `subch`, `cell` and `inst`, where 'subch' is the subchannel number, `cell` is the cell identifier which the channel belongs to, `inst` is the instance of the channel. for example: OC_SubchOfSacchh('0'B, C_CellA, 1) = C_SACCHH0_A_1; OC_SubchOfSacchh('1'B, C_CellA, 1) = C_SACCHH1_A_1; OC_SubchOfSacchh('0'B, C_CellB, 1) = C_SACCHH0_B_1; OC_SubchOfSacchh('1'B, C_CellB, 1) = C_SACCHH1_B_1. pseudo C code definition for the operation as following : OC_SubchOfSacchh(subch, cell, inst) BITSTRING subch; CellID cell; INTEGER inst; { LOGICCH logch; if (subch=='0'B && cell == C_CellA) switch (inst) { case 1 : logch = C_SACCHH0_A_1; break; case 2 : logch = C_SACCHH0_A_2; break; case 3 : logch = C_SACCHH0_A_3; break; } if (subch=='0'B && cell == C_CellB) switch (inst) { case 1 : logch = C_SACCHH0_B_1; break; case 2 : logch = C_SACCHH0_B_2; break; case 3 : logch = C_SACCHH0_B_3; break; } if (subch=='0'B && cell == C_CellC) switch (inst) { case 1 : logch = C_SACCHH0_C_1; break; case 2 : logch = C_SACCHH0_C_2; break; case 3 : logch = C_SACCHH0_C_3; break; } if (subch=='1'B && cell == C_CellA) switch (inst) { case 1 : logch = C_SACCHH1_A_1; break; case 2 : logch = C_SACCHH1_A_2; break; case 3 : logch = C_SACCHH1_A_3; break; } if (subch=='1'B && cell == C_CellB) switch (inst) { case 1 : logch = C_SACCHH1_B_1; break; case 2 : logch = C_SACCHH1_B_2; break; case 3 : logch = C_SACCHH1_B_3; break; } if (subch=='1'B && cell == C_CellC) switch (inst) { case 1 : logch = C_SACCHH1_C_1; break; case 2 : logch = C_SACCHH1_C_2; break; case 3 : logch = C_SACCHH1_C_3; break; } return(logch); } |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_SubchOfSacch4(subch: B_2; cell:CellID) |
| Result Type: | LOGICCH |
| Comments: |  |
| Description |  |
| OC_SubchOfSacch4 operation returns a logic channel identifier for SACCHC4 subchannel indicated by the input parameters `subch ` and `cell`, where `subch` is the TDMA offset, `cell` is the cell identifier which the channel belongs to. for example: OC_SubchOfSacch4('00'B, C_CellA) = C_SACCHC40_A; OC_SubchOfSacch4('01'B, C_CellA) = C_SACCHC41_A; OC_SubchOfSacch4('10'B, C_CellA) = C_SACCHC42_A; OC_SubchOfSacch4('11'B, C_CellA) = C_SACCHC43_A; OC_SubchOfSacch4('00'B, C_CellB) = C_SACCHC40_B; OC_SubchOfSacch4('01'B, C_CellB) = C_SACCHC41_B. pseudo C code definition for the operation as following : OC_SubchOfSacch4(subch, cell) BITSTRING subch; CellID cell; { LOGICCH logch; if (cell == C_CellA) switch (subch) { case '00'B : logch = C_SACCHC40_A; break; case '01'B : logch = C_SACCHC41_A; break; case '10'B : logch = C_SACCHC42_A; break; case '11'B : logch = C_SACCHC43_A; break; } if (cell == C_CellB) switch (subch) { case '00'B : logch = C_SACCHC40_B; break; case '01'B : logch = C_SACCHC41_B; break; case '10'B : logch = C_SACCHC42_B; break; case '11'B : logch = C_SACCHC43_B; break; } if (cell == C_CellC) switch (subch) { case '00'B : logch = C_SACCHC40_C; break; case '01'B : logch = C_SACCHC41_C; break; case '10'B : logch = C_SACCHC42_C; break; case '11'B : logch = C_SACCHC43_C; break; } return(logch); } |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_SubchOfSacch8(subch: B_3; cell:CellID; inst:INTEGER) |
| Result Type: | LOGICCH |
| Comments: | offset is of type BITSTRING[3] |
| Description |  |
| OC_SubchOfSacch8 operation returns a logic channel identifier for SACCHC8 subchannel indicated by the input parameters `subch`, `cell` and `inst`, where `subch` is the TDMA offset, `cell` is the cell identifier which the channel belongs to, `inst` is the instance of the channel. for example: OC_SubchOfSacch8('000'B, C_CellA, 1) = C_SACCHC80_A_1; OC_SubchOfSacch8('001'B, C_CellA, 1) = C_SACCHC81_A_1; OC_SubchOfSacch8('010'B, C_CellA, 1) = C_SACCHC82_A_1; OC_SubchOfSacch8('011'B, C_CellA, 1) = C_SACCHC83_A_1; OC_SubchOfSacch8('100'B, C_CellA, 1) = C_SACCHC84_A_1; OC_SubchOfSacch8('101'B, C_CellA, 1) = C_SACCHC85_A_1; OC_SubchOfSacch8('110'B, C_CellA, 1) = C_SACCHC86_A_1; OC_SubchOfSacch8('111'B, C_CellA, 1) = C_SACCHC87_A_1; OC_SubchOfSacch8('000'B, C_CellB, 1) = C_SACCHC80_B_1; OC_SubchOfSacch8('001'B, C_CellB, 1) = C_SACCHC81_B_1; OC_SubchOfSacch8('000'B, C_CellC, 2) = C_SACCHC80_C_2; OC_SubchOfSacch8('001'B, C_CellC, 2) = C_SACCHC81_C_2. pseudo C code definition for the operation as following : OC_SubchOfSacch8(subch, cell, inst) BITSTRING subch; CellID cell; INTEGER inst; { LOGICCH logch; if (inst==1 && cell == C_CellA) switch (subch) { case '000'B : logch = C_SACCHC80_A_1; break; case '001'B : logch = C_SACCHC81_A_1; break; case '010'B : logch = C_SACCHC82_A_1; break; case '011'B : logch = C_SACCHC83_A_1; break; case '100'B : logch = C_SACCHC84_A_1; break; case '101'B : logch = C_SACCHC85_A_1; break; case '110'B : logch = C_SACCHC86_A_1; break; case '111'B : logch = C_SACCHC87_A_1; break; } if (inst==2 && cell == C_CellA) switch (subch) { case '000'B : logch = C_SACCHC80_A_2; break; case '001'B : logch = C_SACCHC81_A_2; break; case '010'B : logch = C_SACCHC82_A_2; break; case '011'B : logch = C_SACCHC83_A_2; break; case '100'B : logch = C_SACCHC84_A_2; break; case '101'B : logch = C_SACCHC85_A_2; break; case '110'B : logch = C_SACCHC86_A_2; break; case '111'B : logch = C_SACCHC87_A_2; break; } if (inst==3 && cell == C_CellA) switch (subch) { case '000'B : logch = C_SACCHC80_A_3; break; case '001'B : logch = C_SACCHC81_A_3; break; case '010'B : logch = C_SACCHC82_A_3; break; case '011'B : logch = C_SACCHC83_A_3; break; case '100'B : logch = C_SACCHC84_A_3; break; case '101'B : logch = C_SACCHC85_A_3; break; case '110'B : logch = C_SACCHC86_A_3; break; case '111'B : logch = C_SACCHC87_A_3; break; } if (inst==1 && cell == C_CellB) switch (subch) { case '000'B : logch = C_SACCHC80_B_1; break; case '001'B : logch = C_SACCHC81_B_1; break; case '010'B : logch = C_SACCHC82_B_1; break; case '011'B : logch = C_SACCHC83_B_1; break; case '100'B : logch = C_SACCHC84_B_1; break; case '101'B : logch = C_SACCHC85_B_1; break; case '110'B : logch = C_SACCHC86_B_1; break; case '111'B : logch = C_SACCHC87_B_1; break; } if (inst==2 && cell == C_CellB) switch (subch) { case '000'B : logch = C_SACCHC80_B_2; break; case '001'B : logch = C_SACCHC81_B_2; break; case '010'B : logch = C_SACCHC82_B_2; break; case '011'B : logch = C_SACCHC83_B_2; break; case '100'B : logch = C_SACCHC84_B_2; break; case '101'B : logch = C_SACCHC85_B_2; break; case '110'B : logch = C_SACCHC86_B_2; break; case '111'B : logch = C_SACCHC87_B_2; break; } if (inst==3 && cell == C_CellB) switch (subch) { case '000'B : logch = C_SACCHC80_B_3; break; case '001'B : logch = C_SACCHC81_B_3; break; case '010'B : logch = C_SACCHC82_B_3; break; case '011'B : logch = C_SACCHC83_B_3; break; case '100'B : logch = C_SACCHC84_B_3; break; case '101'B : logch = C_SACCHC85_B_3; break; case '110'B : logch = C_SACCHC86_B_3; break; case '111'B : logch = C_SACCHC87_B_3; break; } if (inst==1 && cell == C_CellC) switch (subch) { case '000'B : logch = C_SACCHC80_C_1; break; case '001'B : logch = C_SACCHC81_C_1; break; case '010'B : logch = C_SACCHC82_C_1; break; case '011'B : logch = C_SACCHC83_C_1; break; case '100'B : logch = C_SACCHC84_C_1; break; case '101'B : logch = C_SACCHC85_C_1; break; case '110'B : logch = C_SACCHC86_C_1; break; case '111'B : logch = C_SACCHC87_C_1; break; } if (inst==2 && cell == C_CellC) switch (subch) { case '000'B : logch = C_SACCHC80_C_2; break; case '001'B : logch = C_SACCHC81_C_2; break; case '010'B : logch = C_SACCHC82_C_2; break; case '011'B : logch = C_SACCHC83_C_2; break; case '100'B : logch = C_SACCHC84_C_2; break; case '101'B : logch = C_SACCHC85_C_2; break; case '110'B : logch = C_SACCHC86_C_2; break; case '111'B : logch = C_SACCHC87_C_2; break; } if (inst==3 && cell == C_CellC) switch (subch) { case '000'B : logch = C_SACCHC80_C_3; break; case '001'B : logch = C_SACCHC81_C_3; break; case '010'B : logch = C_SACCHC82_C_3; break; case '011'B : logch = C_SACCHC83_C_3; break; case '100'B : logch = C_SACCHC84_C_3; break; case '101'B : logch = C_SACCHC85_C_3; break; case '110'B : logch = C_SACCHC86_C_3; break; case '111'B : logch = C_SACCHC87_C_3; break; } return(logch); } |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_SubchOfSdcch4(subch: B_2; cell:CellID) |
| Result Type: | LOGICCH |
| Comments: |  |
| Description |  |
| OC_SubchOfSdcch4 operation returns a logic channel identifier for SDCCH4 subchannel indicated by the input parameters `subch ` and `cell`, where `subch` is the TDMA offset, `cell` is the cell identifier which the channel belongs to. for example: OC_SubchOfSdcch4('00'B, C_CellA) = C_SDCCH40_A; OC_SubchOfSdcch4('01'B, C_CellA) = C_SDCCH41_A; OC_SubchOfSdcch4('10'B, C_CellA) = C_SDCCH42_A; OC_SubchOfSdcch4('11'B, C_CellA) = C_SDCCH43_A; OC_SubchOfSdcch4('00'B, C_CellB) = C_SDCCH40_B; OC_SubchOfSdcch4('01'B, C_CellB) = C_SDCCH41_B. pseudo C code definition for the operation as following : OC_SubchOfSdcch4(subch, cell) BITSTRING subch; CellID cell; { LOGICCH logch; if (cell == C_CellA) switch (subch) { case '00'B : logch = C_SDCCH40_A; break; case '01'B : logch = C_SDCCH41_A; break; case '10'B : logch = C_SDCCH42_A; break; case '11'B : logch = C_SDCCH43_A; break; } if (cell == C_CellB) switch (subch) { case '00'B : logch = C_SDCCH40_B; break; case '01'B : logch = C_SDCCH41_B; break; case '10'B : logch = C_SDCCH42_B; break; case '11'B : logch = C_SDCCH43_B; break; } if (cell == C_CellC) switch (subch) { case '00'B : logch = C_SDCCH40_C; break; case '01'B : logch = C_SDCCH41_C; break; case '10'B : logch = C_SDCCH42_C; break; case '11'B : logch = C_SDCCH43_C; break; } return(logch); } |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OC_SubchOfSdcch8(subch: B_3; cell:CellID; inst:INTEGER) |
| Result Type: | LOGICCH |
| Comments: | offset is of type BITSTRING[3] |
| Description |  |
| OC_SubchOfSdcch8 operation returns a logic channel identifier for SDCCH8 subchannel indicated by the input parameters `subch`, `cell` and `inst`, where `subch` is the TDMA offset, `cell` is the cell identifier which the channel belongs to, `inst` is the instance of the channel. for example: OC_SubchOfSdcch8('000'B, C_CellA, 1) = C_SDCCH80_A_1; OC_SubchOfSdcch8('001'B, C_CellA, 1) = C_SDCCH81_A_1; OC_SubchOfSdcch8('010'B, C_CellA, 1) = C_SDCCH82_A_1; OC_SubchOfSdcch8('011'B, C_CellA, 1) = C_SDCCH83_A_1; OC_SubchOfSdcch8('100'B, C_CellA, 1) = C_SDCCH84_A_1; OC_SubchOfSdcch8('101'B, C_CellA, 1) = C_SDCCH85_A_1; OC_SubchOfSdcch8('110'B, C_CellA, 1) = C_SDCCH86_A_1; OC_SubchOfSdcch8('111'B, C_CellA, 1) = C_SDCCH87_A_1; OC_SubchOfSdcch8('000'B, C_CellB, 1) = C_SDCCH80_B_1; OC_SubchOfSdcch8('001'B, C_CellB, 1) = C_SDCCH81_B_1; OC_SubchOfSdcch8('000'B, C_CellC, 2) = C_SDCCH80_C_2; OC_SubchOfSdcch8('001'B, C_CellC, 2) = C_SDCCH81_C_2. pseudo C code definition for the operation as following : OC_SubchOfSdcch8(subch, cell, inst) BITSTRING subch; CellID cell; INTEGER inst; { LOGICCH logch; if (inst==1 && cell == C_CellA) switch (subch) { case '000'B : logch = C_SDCCH80_A_1; break; case '001'B : logch = C_SDCCH81_A_1; break; case '010'B : logch = C_SDCCH82_A_1; break; case '011'B : logch = C_SDCCH83_A_1; break; case '100'B : logch = C_SDCCH84_A_1; break; case '101'B : logch = C_SDCCH85_A_1; break; case '110'B : logch = C_SDCCH86_A_1; break; case '111'B : logch = C_SDCCH87_A_1; break; } if (inst==2 && cell == C_CellA) switch (subch) { case '000'B : logch = C_SDCCH80_A_2; break; case '001'B : logch = C_SDCCH81_A_2; break; case '010'B : logch = C_SDCCH82_A_2; break; case '011'B : logch = C_SDCCH83_A_2; break; case '100'B : logch = C_SDCCH84_A_2; break; case '101'B : logch = C_SDCCH85_A_2; break; case '110'B : logch = C_SDCCH86_A_2; break; case '111'B : logch = C_SDCCH87_A_2; break; } if (inst==3 && cell == C_CellA) switch (subch) { case '000'B : logch = C_SDCCH80_A_3; break; case '001'B : logch = C_SDCCH81_A_3; break; case '010'B : logch = C_SDCCH82_A_3; break; case '011'B : logch = C_SDCCH83_A_3; break; case '100'B : logch = C_SDCCH84_A_3; break; case '101'B : logch = C_SDCCH85_A_3; break; case '110'B : logch = C_SDCCH86_A_3; break; case '111'B : logch = C_SDCCH87_A_3; break; } if (inst==1 && cell == C_CellB) switch (subch) { case '000'B : logch = C_SDCCH80_B_1; break; case '001'B : logch = C_SDCCH81_B_1; break; case '010'B : logch = C_SDCCH82_B_1; break; case '011'B : logch = C_SDCCH83_B_1; break; case '100'B : logch = C_SDCCH84_B_1; break; case '101'B : logch = C_SDCCH85_B_1; break; case '110'B : logch = C_SDCCH86_B_1; break; case '111'B : logch = C_SDCCH87_B_1; break; } if (inst==2 && cell == C_CellB) switch (subch) { case '000'B : logch = C_SDCCH80_B_2; break; case '001'B : logch = C_SDCCH81_B_2; break; case '010'B : logch = C_SDCCH82_B_2; break; case '011'B : logch = C_SDCCH83_B_2; break; case '100'B : logch = C_SDCCH84_B_2; break; case '101'B : logch = C_SDCCH85_B_2; break; case '110'B : logch = C_SDCCH86_B_2; break; case '111'B : logch = C_SDCCH87_B_2; break; } if (inst==3 && cell == C_CellB) switch (subch) { case '000'B : logch = C_SDCCH80_B_3; break; case '001'B : logch = C_SDCCH81_B_3; break; case '010'B : logch = C_SDCCH82_B_3; break; case '011'B : logch = C_SDCCH83_B_3; break; case '100'B : logch = C_SDCCH84_B_3; break; case '101'B : logch = C_SDCCH85_B_3; break; case '110'B : logch = C_SDCCH86_B_3; break; case '111'B : logch = C_SDCCH87_B_3; break; } if (inst==1 && cell == C_CellC) switch (subch) { case '000'B : logch = C_SDCCH80_C_1; break; case '001'B : logch = C_SDCCH81_C_1; break; case '010'B : logch = C_SDCCH82_C_1; break; case '011'B : logch = C_SDCCH83_C_1; break; case '100'B : logch = C_SDCCH84_C_1; break; case '101'B : logch = C_SDCCH85_C_1; break; case '110'B : logch = C_SDCCH86_C_1; break; case '111'B : logch = C_SDCCH87_C_1; break; } if (inst==2 && cell == C_CellC) switch (subch) { case '000'B : logch = C_SDCCH80_C_2; break; case '001'B : logch = C_SDCCH81_C_2; break; case '010'B : logch = C_SDCCH82_C_2; break; case '011'B : logch = C_SDCCH83_C_2; break; case '100'B : logch = C_SDCCH84_C_2; break; case '101'B : logch = C_SDCCH85_C_2; break; case '110'B : logch = C_SDCCH86_C_2; break; case '111'B : logch = C_SDCCH87_C_2; break; } if (inst==3 && cell == C_CellC) switch (subch) { case '000'B : logch = C_SDCCH80_C_3; break; case '001'B : logch = C_SDCCH81_C_3; break; case '010'B : logch = C_SDCCH82_C_3; break; case '011'B : logch = C_SDCCH83_C_3; break; case '100'B : logch = C_SDCCH84_C_3; break; case '101'B : logch = C_SDCCH85_C_3; break; case '110'B : logch = C_SDCCH86_C_3; break; case '111'B : logch = C_SDCCH87_C_3; break; } return(logch); } |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_2Msgs(ch:LOGICCH; pgg:PGG; mode:SENDINGMODE) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| OM_2Msgs puts the layer 2 emulator into a special operation mode then returns. In this special mode the layer 2 emulator sends the next two consecutive messages in the following way: - send the first message on the paging subchannel indicated by the `pgg` and 'ch'; - if the `mode` = C_NxtButOne, send the second message in the next but one paging sub block; - if the `mode` = C_FmrAGB, send the second message in a former access grant block; - if the `mode` = C_BfReOcc, send the second message before the MS's original paging subchannel re-occurs but later than the next paging block of that CCCH (paging block not belong to the MS); - if the `mode` = C_NxtButOneNxt, nothing is sent in the next but one paging sub block, then send the second message in the next paging subblock of the MS's paging subchannel. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_Assoc(lgch1:LOGICCH; sublgch1:LOGICCH; sublgch2:LOGICCH; sublgch3:LOGICCH; sublgch4:LOGICCH; sublgch5:LOGICCH; sublgch6:LOGICCH; sublgch7: LOGICCH; sublgch8:LOGICCH; lgch2:LOGICCH; sublgch9:LOGICCH; sublgch10:LOGICCH; sublgch11:LOGICCH; sublgch12:LOGICCH; sublgch13:LOGICCH; sublgch14:LOGICCH; sublgch15: LOGICCH; sublgch16:LOGICCH) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| OM_Assoc operation associates the sub logic channel identifiers `sublgch1` to `sublgch8` and `sublgch9` to `sublgch16` with the generic "parent" channel identifier ` lgch1` and `lgch2` respectively, therefore the subchannel identifiers can refer to the corresponding channels configured by OM_ChConf operation. The `lgch1` can be an identifier for SDCCH4, SDCCH8, FACCHH and the `lgch2` can be SACCHC4, SACCHC8, and SACCHH. "dummy" in the actual parameter list means the corresponding parameter is not used. Naming convention for logical channel constants: C_<channel type>[<rate>][<subchannel>][_<cell>][_<instance>] e.g.: C_FACCH H 0 _C _3 => C_FACCHH0_C_3 channel type: FCCH, SCH, BCCH, CBCH, PCH, RACH, FACCH, SACCH, SDCCH, DCCH rate: F(ull), H(alf), 4 (fourths, C4 for SACCHC4), 8 (eighths, C8 for SACCHC8). subchannel: H: 0, 1 (C)4: 0, ..., 3 (C)8: 0, ..., 7 cell: A, ..., H instance: 1, ..., 3 When several channels of the same type can exist in one cell simultaneously (in different time slots and / or on different frequencies) they are identified by their instance. The parameters of ChConf are used to specify the frequency and slot of this instance of the channel type. Not all possible combinations of this naming syntax have been used. According to the elements that are used / omitted the name can have a special meaning. Examples: Comments: C_FCCH_A FCCH of cell A. C_SCH_A SCH of cell A. C_BCCH_A_1 First instance of a BCCH in cell A. C_CBCH_A CBCH of cell A. C_PCH_A_1 First instance of a PCH in cell A (there is one instance of the PCH per BCCH in the cell). C_AGCH_A_1 First instance of a AGCH in cell A (there is one instance of the AGCH per BCCH in the cell). C_RACH_A_1 First instance of a RACH in cell A. C_FACCHF_A_1 First instance of a full rate FACCH in cell A. C_FACCHH_A_1 First instance of a half rate FACCH in cell A (all subchannels). C_FACCHH0_A_1 Subchannel 0 of the first instance of a half rate FACCH in cell A. C_SACCHF_A_1 First instance of a full rate SACCH in cell A. C_SACCHH_A_1 First instance of a half rate SACCH in cell A (all subchannels). C_SACCHH1_A_1 Subchannel 1 of the first instance of a half rate SACCH in cell A. C_SACCHC4_A SACCH/4 of cell A (all subchannels). C_SACCHC43_A Subchannel 3 of the SACCH/4 of cell A. C_SACCHC8_A_1 First instance of a SACCH/8 in cell A (all subchannels). C_SACCHC87_A_1 Subchannel 7 of the first instance of a SACCH/8 in cell A. C_SDCCH4_A SDCCH/4 of cell A (all subchannels). C_SDCCH42_A Subchannel 2 of the SDCCH/4 of cell A. C_SDCCH87_A_1 First instance of a SDCCH/8 in cell A (all subchannels). C_SDCCH87_A_1 Subchannel 7 of the first instance of a SDCCH/8 in cell A. This list covers all types of combinations used in the TTCN for the time being. When the meaning refers to 'all' subchannels of a channel (eg. C_FACCHH_A_1), it means that all active subchannels of this channel activated previously by OM_ChConf, are associated with a logical channel identifier in the OM_Assoc. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_BmInfo(ch:LOGICCH; mode:B_8) |
| Result Type: | BOOLEAN |
| Comments: | mode is of type BITSTRING[8], imode is a part of the channel mode IE |
| Description |  |
| OM_BmInfo operation checks whether the MS starts transmiting Bm channel information according to the new channel mode `mode'. The operation returns TRUE if the result of the check is correct otherwise the TSO returns FALSE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_ChangeRFOf2Cells( cellid1:CellID; bspwr1:INTEGER; cellid2:CellID; bspwr2:INTEGER) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This TSO is to change the RF level of the cell 'cellid1' to the RF level 'bspwr1' and the RF level of the cell 'cellid2' to the RF level 'bspwr2' [dBuVemf], then the TSO returns TRUE. The major purpose of the TSO is to force the MS to reselect a new cell according to the GSM 05.08, section 6.6.2. The following example shows the use of the TSO. The default cell selection parameters in the ATS are set as: Cell reselect hysteresis: 12db MX TXPWR MX CCH: minimu level (0) ACS: no additional cell selection parameter (0) RXLEV ACCESS MIN: minimum (0). Therefore the path loss criterion parameter C1 equals to the reselection criterion parameter C2. In the whole ATS the C1 and C2 keeps greater than 10 (e.g. TC_26_7_4_2_3) for a cell not suitable. The MS received level average from a not suitable cell keeps also greater than 10 (-100 dBm) corresponding to the reference sensitivity level of DCS 1800 MS. For GSM arfcn = 20, F = 941 MHz , the fields strengths (E) on tester integral antenna: E (dB microV/m) >= (-110 + 10) (dBm) + 20log(941) + 77.2 = 36.7, For DCS arfcn = 590, F = (-110 + 10) (dBm) + 20log(1820.8) + 77.2 = 42.4 (the formula see GSM 05.05, section 5 and GSM 05.08, section 8.1.4). In order to keep C1, C2 greater 10, a reasonable field strength making a cell not suitable is set to C_E_notsuitable = 45 in comparison with the default field strength C_E_default = 63. Considering that the cell recelect hysteresis is set to 12 db, the field strength of a cell being reselected need to be set to greater than 57. In the ATS the default value C_E_default (63) and C_E_notsuitable (45) are used for this TSO to force MS making cell reselection assuming that the two cells have different location area.. If two cells have the same location area (e.g. in TC_26_6_1_3), or although two cells have different LAI, the test cases needs to lower a cell power to a certain low level (e.g. in TC_26_7_4_2_4_1 and TC_26_7_4_2_4_5), C1 needs to be less than zero to force the MS reselect a better cell. For GSM, E <= 26.7, for DCS, E<=32.4. The value C_E_notsuitable_sameLAI = 24 applies to the TSO to force the MS making cell reselection. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_ChConf(bspwr: INTEGER; mspwr:INTEGER; acttype:BITSTRING; chmod:CHMOD; ta:TA; slot:SN; tsc:TSC; rf:FRQPARA; chcmbn:LOGCH; td:INTEGER; fn:INTEGER; babr, cch_con, bpm:B_3; pgfil:PG1_RQ_PDU; dtxu:BITSTRING; dtxd:BITSTRING; cell:CellID; lgch1:LOGICCH; lgch2:LOGICCH; lgch3:LOGICCH; lgch4:LOGICCH; lgch5:LOGICCH; lgch6:LOGICCH; lgch7:LOGICCH; lgch8:LOGICCH; lgch9:LOGICCH) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| The operation sets the configuration of a basic physical channel according to the input parameters and map logic channel combination onto it: - bspwr: base station power level in dBuv; - mspwr: mobile station power level in "power control level"; - acttype: type of activation: - '000'B activation for intra-cell channel change ---- immediate assignment; - '001'B activation for intra-cell channel change ---- assignment; - '010'B activation for inter-cell channel change ---- asynchronous handover; - '011'B activation for inter-cell channel change ---- synchronous handover; - '100'B activation for additional assignment; NOTE: the types defined above are all the same, no real difference - '101'B activation for receiving only; - chmod: channel mode; - ta: timing advance ---- receive timing ta bit periods in advance of the normal receive timing if the channel is a dedicated channel ; - slot: time slot; - tsc: Training sequence code for normal bursts; - rf: ARFCN or hopping parameters; - chcmbn: logic channel combination mapping to the physical channel; - td: timing difference between the cell and conceptual timing reference ---- to make 2 cells with different timebase; - fn: initial frame number offset to the timing base counter ---- to make 2 cells with different frame numbers; - babr number of blocks reserved for access grant - cch-con CCCH/SDCCHs configuration - bpm multiframe period for transmission of PAGING REQUEST - pgfil: paging filling contents; - dtxu: mobile station discontinuous transmission: - '0'B mobile station discontinuous transmission is not applied; - '1'B mobile station discontinuous transmission is applied; - dtxd: base station discontinuous transmission: - '0'B base station discontinuous transmission is not applied; - '1'B base station discontinuous transmission is applied; - cell: cell identifier; - lgch1: logic channel identifier for the channel FCCH; - lgch2: logic channel identifier for the channel SCH; - lgch3: logic channel identifier for the channel BCCH; - lgch4: logic channel identifier for the channel PCH; - lgch5: logic channel identifier for the channel AGCH; - lgch6: logic channel identifier for the channel RACH; - lgch7: logic channel identifier for the channel FACCH or SDCCH; - lgch8: logic channel identifier for the channel SACCH; - lgch9: logic channel identifier for the channel CBCH; For parameters lgch1 to lgch9 "dummy" in the actual parameter list means that the corresponding parameters are not used. In the test cases the logic channel identifiers lgch1 to lgch9 are used to refer the logic channels configured by the operation. There are only generic identifiers for SDCCH4 or SDCCH8 or FACCHH channel and SACCHC4 or SACCHC8 or SACCHH, identifiers for subchannels of SDCCH4 or SDCCH8 or FACCHH and SACCHC4 or SACCHC8 or SACCHH are linked to the generic identifiers by OM_Assoc operation and in turn refer to the sub logic channels configured by the operation. Channel re-use whithiin OM_ChConf: When an OM_ChConf calls is made then the following rules can be applied for deciding whether or not to re-use existing physical channel. Decode the 'chcmbn' (channel combination) parameter to determine the new channel type. If new channel = "C_FCCH_SCH_BCCH_CCCH_SDCCH4_SACCHC4" (combined) or if new channel = "C_CBCH_FCCH_SCH_BCCH_CCCH_SDCCH4_SACCHC4" (combined + SMS) then re-use any existing control channel configuration with same CellID. If new channel = "C_FCCH_SCH_BCCH_CCCH" (non-combined) then use any combined, combined + SMS or non-combined control channel configuration with the same CellID. If new channel = "C_BCCH_CCCH_2", "C_BCCH_CCCH_3" or "C_BCCH_CCCH_4" (ccch-group) then re-use any existing ccch-group control channel configuration with the same CellID and the slot, or re-use any existing combined, combined + SMS with the same CellID. If new channel = "C_TCHF_ACCHF_1", "C_TCHF_ACCHF_2", "C_TCHH_ACCHH_1", "C_TCHH_ACCHH_2" (traffic) or "C_SDCCH8_SACCHC8_1", "C_SDCCH8_SACCHC8_2", "C_SDCCH8_SACCHC8_3" (dedicated) then re-use any existing traffic or dedicated channel with the same channel description. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_ChMdModi(ch:LOGICCH; chmod: CHMOD) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| OM_ChMdModi operation passes channel mode IE for channel `ch` to lower layer emulator and requests the emulator to set the `ch` to the mode `chmod`. After the mode changed, it returns. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_CphMdChg(ch:LOGICCH; cphmod: CPHMS; key:BITSTRING) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| OM_CphMdChg operation passes ciphering parameters for channel `ch` to lower layer and sets the lower layer emulator into a special operation mode in which lower layer starts the 3-step ciphering mode setting sequence when the next L3 message on the channel `ch` arrives. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_CphMd(ch:LOGICCH; cphmod: CPHMS; key:BITSTRING) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| OM_CphMd operation passes ciphering parameters for channel `ch` to lower layer and sets the channel `ch` in the ciphering mode `cphmod`. This operation puts the channel `ch` into specified ciphering mode `cphmod` immediately without the 3-step ciphering mode setting sequence. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_ComingFn(ch:LOGICCH) |
| Result Type: | FN |
| Comments: |  |
| Description |  |
| OM_ComingFn operation returns the frame number (FN modulo 42432) which is about 5 seconds later than current frame number and is able to carry L3 message on the channel `ch`. |  |
| Detailed Comments: | The delay of about 5 seconds ensures that there is still enough time left for higher layer controller to prepare next TTCN send event after the OM_ComingFn returns. the exact delay value is up to the implementor. |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_Deactivate(ch1, ch2:LOGICCH) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| OM_Deactivate operation deactivates the logical channels `ch1` and 'ch2', then returns TRUE. The ch1 can be a DCCH or a TCH, the ch2 is the associated SACCH. Deactivation of an active SDCCH channel can be done by stopping transmission; this means that for the bursts on the BCCH carriers, dummy bursts shall be transmitted, for other bursts nothing shall be transmitted. Alternatively the deactivation of a SDCCH can be done by transmitting bad blocks on the SACCH with the parity bits set incorrectly. This will lead to a radio link failure in the MS. Deactivation of a TCH channel can be done by stopping transmission if it is non-BCCH. Deactivation of a SACCH_T can be done by transmitting bad blocks on the SACCH_T with the parity bits set incorrectly. The wrong channel coding on the SACCH will lead to a radio link failure in the MS after several seconds depending on the system information messages.. |  |
| Detailed Comments: | The TSO is called in TC_26_7_3_2, TC_26_7_4_3_2, TC_26_7_4_3_3, TC_26_7_4_3_4 on a SDCCH/4/SACCH/C4 channel in downlink, TC_26_2_4_1, TC_26_2_4_2, TC_26_8_2_1, TC_26_8_2_2, TC_26_8_2_3 on TCH/SACCH_T channel in both directions. |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_FHCHK(ch:LOGICCH; ca:CCHD; ma:MA; cd:CHD; t :INTEGER; fn:FN) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| After the SS has been configured to start / change frequency hopping at a particular start time, this TSO instructs the SS to check whether the MS correctly uses the hopping frequencies on and after the expected start time according to the frequency parameters. If `t` = 0 or 32024 <= `t` <= 42431 the operation checks that the new frequencies starts "without delay". If 0 < `t` <= 31000 the operation checks that the new frequencies starts "on" the frame number of (X + t) mod 42432, where x = 51 * ((fn.t3 - fn.t2) MOD 26) + fn.t3 + 1326 * fn.t1. If 31000 < `t` < 32024, or `t` < 0 or `t` > 42431 then the test has been configured incorrectly and should be re-run with a permitted value for `t`. This check is performed at the uplink RF burst level by sampling for a period of 3 seconds from the start time and should therefore be called immediately after the SS has been configured to start / change hopping. The operation returns TRUE if the MS behaviour is correct, otherwise it returns FALSE. Note: before this TSO is called, a layer 3 message must be sent to the MS to command the MS to start / change hopping using the same frequency hopping parameters. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_FreeResource(ch :LOGICCH) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| OM_FreeResource operation instructs the lower layer emulator to release the resources which have been allocated to the logical channel "ch". After the resources are released the operation returns TRUE. |  |
| Detailed Comments: | In some test cases (most of them are hanhover test cases) diffirent channels are needed while the test goes on. the purpose of the operation is to reuse lower layer resources (e.g. tranceivers), which have been allocated to the previous channel, for configing another channel required in the next testing step. |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_FreqDef(st:STRT; ma:MA; ch :LOGICCH; chd: CHD; ca:CCHD; frmn: FN) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| OM_FreqDef operation passes the frequency hopping parameters for channel `ch` to lower layer emulator. The emulator starts using these frequency parameters at the frame number indicated by starting time `st`. The parameter 'frmn' indicates a reference frame number at which the first burst of the message, FREQUENCY REDEFINITION, ASSIGNMENT COMMAND or HANDOVER COMMAND, containing starting time is sent in the concerned cell. If 0 <= (st - frmn) mod 42432 <= 31623, the indicated time is the next time when frame number mod 42432 is equal to st. If 32024 <= (st - frmn) mod 42432 <= 42431, the indicated time has already elapsed. After completely sending out the message containing the parameter starting time the tester starts frequency hopping. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_GetHoaccPara(ch:LOGICCH) |
| Result Type: | L1HD |
| Comments: |  |
| Description |  |
| OM_GetHoaccPara instructs the L 2 emulator to get the timing advance and power level of the handover access burst on channel `ch` and return this value in the format of L 1 head. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_GetL1Hd(ch:LOGICCH) |
| Result Type: | L1HD |
| Comments: |  |
| Description |  |
| OM_GetL1Hd instructs the L 2 emulator to read the L 1 header of the next SACCH frame received on channel `ch` and return this value. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_LowerLayerFail(ch:LOGICCH) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation is to force the lower layer failure, then it returns. The lower layer failure can be any one of layer 1 failures or layer 2 failures. - layer 1 failure, see GSM 05.08; - layer 2 failure, see GSM 04.06. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_NoL2Ack(fmtype:L2FMTYPE; i:INTEGER; ch: LOGICCH) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation sets the layer 2 emulator into a special operation mode then returns. In this special mode the layer 2 emulator shall not acknowledge the `i`th occurrence of the L 2 frame which has the type `fmtype` and the more data bit "M" = 0 on channel `ch`. The layer 2 emulator automatically resumes normal operation after this L 2 frame. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_NotAckSetup(ch:LOGICCH) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation puts the layer 2 emulator into a special operation mode then returns. In this special mode the layer 2 emulator will not acknowledge the last L 2 frame which carries the SETUP message, and the layer 2 emulator resumes normal operation after this L 2 frame. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_NoUAforDISC(ch:LOGICCH) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| OM_NoUAforDISC and OM_ResumUAforDISC are a pair of TSOs. OM_NoUAforDISC forces the L2 emulator of the tester to enter a specific mode. OM_ResumUAforDISC resumes the L2 emulator to a normal L2 operational mode. In the specific mode on receiving a DISC on the channel 'ch', the L2 emulator - does not respond with a UA, - remains in the multiple-frame established state, - indicates that the DISC has been received (by means of subsequent DL_RELEASE_INDICATION ASPs). Then the TSO returns TRUE. |  |
| Detailed Comments: | The TSO is called in TC_26_6_12_2 and TC_26_6_12_4. |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_PgFill(cellid:CellID; msg:PG1_RQ_PDU) |
| Result Type: | BOOLEAN |
| Comments: | The result returned by the operation is not used. |
| Description |  |
| OM_PgFill operation sets up the contents of paging filling frame to be sent on all paging subchannels continuously. If the contents of paging filling frame in cell `cellid` have not been set up, the operation sets the contents to `msg`. If paging filling frame in cell `cellid` has been set up, the operation changes the paging filling message to `msg`. After the required action has been taken it returns. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_Reactivate(ch1, ch2:LOGICCH) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| The TSO OM_Reactivate reactivates the logical channels `ch1` and 'ch2' which were previously deactivated by the OM_Deactivate. Then the TSO returns TRUE. The ch1 is an SDCCH, the ch2 is the associated SACCH. Reactivation of deactivated SDCCH/SACCH channels means to restart normal transmissions both on the SDCCH and the associated SACCH as specified in the ETS 300 574 (GSM 05.02). |  |
| Detailed Comments: | The TSO is called in TC_26_7_3_2, TC_26_7_4_3_2, TC_26_7_4_3_3, TC_26_7_4_3_4 on the SDCCH/4 / SACCH/C4 channels. |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_ReturnFn(ch:LOGICCH) |
| Result Type: | FN |
| Comments: |  |
| Description |  |
| OM_ReturnFn operation returns the frame number (FN modulo 42432) on which the last L3 message was sent on logic channel `ch`. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_ResumUAforDISC(ch:LOGICCH) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| OM_NoUAforDISC and OM_ResumUAforDISC are a pair of TSOs. OM_NoUAforDISC forces the L2 emulator to enter a specific mode. OM_ResumUAforDISC resumes the L2 emulator to a normal L2 operational mode. OM_ResumeUAforDISC cancels the effect of OM_NoUAforDISC, returning L2 on the channel 'ch' to normal operation in the multiple-frame established state. Then the TSO returns TRUE. |  |
| Detailed Comments: | The TSO is called in TC_26_6_12_2 and TC_26_6_12_4. |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_SendSMSCBWhilePaging(ch:LOGICCH) |
| Result Type: | BOOLEAN |
| Comments: | The result returned by the operation is not used. |
| Description |  |
| The TSO makes the lower layers send the second SMSCB message at the same time as the MS is paged. This shall be achieved by paging the MS immediately after the first block of the CB message has been sent. The SS shall ensure that the page is transmitted on the radio interface prior to the transmission of the 4th block of the CB message. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_SendNextOn(ch:LOGICCH; fn:FN) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| OM_SendNextOn operation sets the lower layer emulator into a special operation mode then returns. In the special mode the lower layer emulator sends the next L3 message on the indicated frame number `fn` on the 'ch' channel. After the next L3 message is sent, the lower layer resumes normal operation automatically. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_StartMsrReport(ch:LOGICCH) |
| Result Type: | BOOLEAN |
| Comments: | The result returned by the operation is not used. |
| Description |  |
| The operation starts the reporting of received MEASUREMENT REPORT message to layer 3 emulator, then it returns with value TRUE. Measurement Reports shall be disable by default. By default means that the tester filters them out from the TTCN in-buffer. This operation enables the reports entering the TTCN in-buffer. The OM_StopMsrReport operation disables the reports entering the TTCN in-buffer. When starting each test case, the measurement reports are filtered out by default. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_StopAllBCCH(cell1, cell2, cell3, cell4, cell5, cell6, cell7, cell8:CellID) |
| Result Type: | BOOLEAN |
| Comments: | The result returned by the operation is not used. |
| Description |  |
| The TSO stops the RF transmission on all BCCH channels including dummy bursts in the indicated cells, but keeps the uplink CCCH reception active. After all transmission have been stopped the TSO returns TRUE. If a cell in the parameter list is not in use a "dummy" value needs to be assigned to it by the calling test case. The TSO is called in TC_26_3_3. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_StopCell(cell:CellID) |
| Result Type: | BOOLEAN |
| Comments: | The result returned by the operation is not used. |
| Description |  |
| The TSO stops completely all RF transmission of the specific cell `cell`, including the dummy bursts on other slots. After all RF transmission have been stopped the TSO returns TRUE. The cell shall be deleted from the lower layer tester and therefore if the cell 'cell' is required again by the calling test case it needs to be re-configured. The TSO is called in TC_26_3_4, TC_26_6_6_1, TC_26_7_4_2_2_1, TC_26_7_4_2_2_2, TC_26_7_4_6, TC_31_6_2_3. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OM_StopMsrReport(ch:LOGICCH) |
| Result Type: | BOOLEAN |
| Comments: | The result returned by the operation is not used. |
| Description |  |
| The operation stops the reporting of received MEASUREMENT REPORT message to layer 3 emulator, after measurement report stops it returns with value TRUE. Measurement Reports shall be disable by default. By default means that the tester filters them out from the TTCN in-buffer. This operation disables the reports entering the TTCN in-buffer. The OM_StartMsrReport operation enables the reports entering the TTCN in-buffer. When starting each test case, the measurement reports are filtered out by default. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_ACMIncCHK(para:IA5String) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| the OO_ACMIncCHK operation requests (e.g. from the control console of the test system) the test operator to read the value of ACM on SIM and check the increment of the value. Then the operator informs the test system whether the increment is as description `para`, if it is as the description the operation returns TRUE, otherwise returns FALSE. The value of ACM can be read either via MMI or by removing the SIM and using SIM reader. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_ACMReading |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| the OO_ACMReading operation requests (e.g. from the control console of the test system) the test operator to read and note the value of ACM on SIM. After the operator finishes the action, he informs the test system and the operation returns with the value TRUE. The value of ACM can be read either via MMI or by removing the SIM and using SIM reader. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_ACMSetting |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| the OO_ACMSetting operation requests (e.g. from the control console of the test system) the test operator to reset the ACM to zero and set the ACMmax to 2 units. After the operator finishes the action and informs the test system it returns TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_AddPwrAmp |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to add power amplification at the Mobile Station under test. After the operator finishes the action, he informs the test system and the operation returns with the value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_AltIndCHK |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation first requests (e.g. from the control console of the test system) the test operator to check whether the alerting indication is generated. If the operator informs (e.g. from the control console of the test system) the test system that the alerting indication is generated, the operation returns TRUE. If the operator informs the test system that the alerting indication is not generated, the operation returns FALSE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_CalledNumCHK |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation first requests (e.g. from the control console of the test system) the test operator to check whether the called party number is correctly displayed on the mobile station. If the operator informs (e.g. from the control console of the test system) the test system that the display is correct, the operation returns TRUE. If the operator informs the test system that the display is wrong, the operation returns FALSE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_CalledPtyNumCHK(num:CDPN) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation first requests (e.g. from the control console of the test system) the test operator to check whether the called party number `num` is the one that was entered into the MS. If the operator informs (e.g. from the control console of the test system) the test system that the number is correct, the operation returns TRUE. If the operator informs the test system that the number is wrong, the operation returns FALSE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_CheckAllSMPresentBut4th |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to check if the mobile station under test has stored all short messages during the test case but not the 4th one which should have been replaced by the 5th(cf. GSM 11.10, clause 34.2.7.4). The messages are displayed. Then the operation returns. TRUE: All SM present but not the 4th FALSE: 4th SM present or 1st, 2nd, 3rd or 5th missing. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_CheckCBSMReceived(NumbOfIA5chara: INTEGER; msg:SMSCBpack) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to check whether the MS under test has correctly received the cell broadcast short message represented by 'msg' . Then the TSO returns TRUE: if the CBSM has been correctly received, FALSE: if non or an incorrect CBSM has been received. The TSO needs to convert the 'msg' into IA5 string according to the packing character rules specified in the section 6.1.2 of ETS 300 628 (GSM 03.38). The 'NumbOfIA5chara' indicates the number of IA5 characters in the 'msg'. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_CheckMCEFOnSIM |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to check if the Memory Capacity Exceeded Flag has been set on the SIM simulator and to inform the test system of the result of the checking. If the checking succeds the operation returns TRUE, FALSE otherwise. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_CheckMCEFOnSIMUnset |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to check if the Memory Capacity Exceeded Flag has been unset on the SIM simulator and to inform the test system of the result of the checking. If the checking succeds the operation returns TRUE, FALSE otherwise. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_CheckMessageDisplayed(NumbOfIA5chara:INTEGER; msg:OCTETSTRING) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to check if the mobile station under test indicates that an SM has arrived. If the MS provides the functionality to display MT messages, it is checked that the message contents represented by 'msg' is correctly displayed. Then the operation returns TRUE: if the MS indicates SM reception (and displays correct message), FALSE: if the MS does not indicate SM reception (or displays incorrect message). The TSO needs to convert the 'msg' into IA5 string according to the packing character rules specified in the section 6.1.2 of ETS 300 628 (GSM 03.38). The 'NumbOfIA5chara' indicates the number of IA5 characters in the 'msg'. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_CheckUssdStringDisplayed(strg: IA5String) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to check if the mobile station under test displays the Ussd String 'strg'. Then the operation returns. TRUE: MS displays the correct Ussd String. FALSE: MS does not display the correct Ussd String. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_ConnectSIMSimulator |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to connect the SIM simulator to the mobile station under test. For the contents of the SIM simulator the operator refers to she initial conditions of the test case acc. GSM 11.10, clause 34.2.3.3. After the operator finishes the action, he informs the test system and the operation returns with the value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_DepressEndKey |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to depress the 'END' key of the Mobile Stations keypad, then returns. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_DialCalledNum |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to dial the called party number at the Mobile Station under test but not press the "SEND" key (i.e. not initiate the call setup). After the operator finishes the action, he informs the test system and the operation returns with the value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_DisplaySMAndSendReplySM(n, NumbOfIA5chara: INTEGER; msg:OCTETSTRING) |
| Result Type: | OCTETSTRING |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to display the 'n'-th short message received in the arriving order and to check whether the short message contents represented by 'msg' is correctly displayed. Then the TSO asks the operator to send a reply short message from the mobile. After the operator finishes the action, the operation returns with the message contents sent in OCTETSTRING. The TSO needs to convert the 'msg' into IA5 string and the IA5 text entered by the operator into OCTETSTRING according to the packing character rules specified in the section 6.1.2 of ETS 300 628 (GSM 03.38). The 'NumbOfIA5chara' indicates the number of IA5 characters in the 'msg'. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_DTMFIndCHK(character:IA5String) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation first requests (e.g. from the control console of the test system) the test operator to check whether the `character` is indicated by the DTMF indicator of the MS. If the operator informs (e.g. from the control console of the test system) the test system that the MS does not give the indication of `character`, the operation returns FALSE. If the operator informs the test system that the MS gives the indication, the operation returns TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_EmptyMessageStorage |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to empty the message storage of the mobile station under test. After the operator finishes the action, he informs the test system and the operation returns with the value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_EnterPswd(pswd:IA5String) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This TSO displays a message on the control console to inform the operator that he is to enter a given password (the actual password is given by 'pswd') on the mobile. The operation waits for the operator to acknowledge the request, which may be done before or after entering the password into the mobile, and then returns the value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_HookOff |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to answer the mobile terminating call at the Mobile Station under test. This operation waits for the operator to acknowledge the request which may be done before or after the call has been answered and then returns the value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_IFsetup_Telephony |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This TSO requests (e.g. from the control console of the test system) the test operator to setup the external interface, or configuration of the MS in such a way that the MS is able to successfully receive the call for telephony service. If the MS supports both telephony without immediate connection mode and telephony with immediate connection mode, set the MS in telephony without immediate connection mode. After the operator finishes the required action and informs the test system, the TSO returns with value TRUE. This TSO is used in MO call test cases except TC_11_2, where this TSO is call for single-numbering scheme MT testing. |  |
| Detailed Comments: | used for TS11 and TS12 |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_IFsetup_BS2xorBS3x_MO(srv:SERVICES; itc, sacp:B_3; ce:B_2; rate:IA5String) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This TSO requests (e.g. from the control console of the test system) the test operator to setup the external interface, or the MS configuration in such a way that the MS, when trigged, is able to initiate the call for the BS2x or BS3x service `srv` with the specified itc, sacp, ce values. After the operator finishes the required action and informs the test system, the TSO returns with value TRUE. This TSO is used in MO call test cases except TC_11_2, where this TSO is call for single-numbering scheme MT testing. Three negotiable parameters, number of stop bit, number of data bit, parity information, are asked as global IXIT paramters for all TE configurations. If necessary, the test operator refers to the manual provided by manufacturer to configure the MS. The possible values are: 1) srv ----- basic service - C_Async300 Basic Service 21, Asynchronous 300 bps - C_Async1200 Basic Service 22, Asynchronous 1.2 kbps - C_Async120075 Basic Service 23, Asynchronous 1200/75 bps - C_Async2400 Basic Service 24, Asynchronous 2.4 kbps - C_Async4800 Basic Service 25, Asynchronous 4.8 kbps - C_Async9600 Basic Service 26, Asynchronous 9.6 kbps - C_Sync1200 Basic Service 31, Synchronous 1.2 kbps - C_Sync2400 Basic Service 32, Synchronous 2.4 kbps - C_Sync4800 Basic Service 33, Synchronous 4.8 kbps - C_Sync9600 Basic Service 34, Synchronous 9.6 kbps 2) itc ------ information transfer capability - '001'B unrestricted digital information - '010'B 3.1 kHz audio, ex PLMN 3) sacp ---- signalling access protocol - '001'B I.440/450 - '010'B X.21 - '101'B X.28 - non dedicated PAD - '110'B X.32 4) ce ------ connection element - '00'B transparent - '01'B non transparent 5) rate ----- channel rate - C_Full full rate channel - C_Half half rate channel |  |
| Detailed Comments: | Used for BS21, BS22, BS23, BS24, BS25, BS26, BS31, BS32, BS33, BS34 |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_IFsetup_BS4x_MO(srv:SERVICES; ce:B_2; rate:IA5String) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This TSO requests (e.g. from the control console of the test system) the test operator to setup the external interface, orthe MS configuration in such a way that the MS, when trigged, is able to initiate the call for the BS4x service `srv` with specified ur and ce values. After the operator finishes the required action and informs the test system, the TSO returns with value TRUE. Three negotiable parameters, number of stop bit, number of data bit, parity information, are asked as global IXIT parameters for all TE configurations. If necessary, the test operator refers to the manual provided by manufacturer to configure the MS. The possible values are: 1) srv ----- basic service - C_PAD300 Basic Service 41, PAD access 300 bps - C_PAD1200 Basic Service 42, PAD access 1.2 kbps - C_PAD120075 Basic Service 43, PAD access 1200/75 bps - C_PAD2400 Basic Service 44, PAD access 2.4 kbps - C_PAD4800 Basic Service 45, PAD access 4.8 kbps - C_PAD9600 Basic Service 46, PAD access 9.6 kbps 2) ce ------ connection element - '00'B transparent - '01'B non transparent 3) rate ----- channel rate - C_Full full rate channel - C_Half half rate channel |  |
| Detailed Comments: | Used for BS41, BS42, BS43, BS44, BS45, BS46 |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_IFsetup_BS5x_MO(srv:SERVICES; ur:B_4; rate:IA5String) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This TSO requests (e.g. from the control console of the test system) the test operator to setup the external interface, orthe MS configuration in such a way that the MS, when trigged, is able to initiate the call for the BS5x service `srv` with the specifyied ur value. After the operator finishes the required action and informs the test system, the TSO returns with value TRUE. Three negotiable parameters, number of stop bit, number of data bit, parity information, are asked as global IXIT parameters for all TE configurations. If necessary, the test operator refers to the manual provided by manufacturer to configure the MS. The possible values are: 1) srv ----- basic service - C_Packet2400 Basic Service 51, Packet access 2.4 kbps - C_Packet4800 Basic Service 52, Packet access 4.8 kbps - C_Packet9600 Basic Service 53, Packet access 9.6 kbps 2) ur ------ user rate - '0011'B 2400 bit/s - '0100'B 4800 bit/s - '0101'B 9600 bit/s 3) rate ----- channel rate - C_Full full rate channel - C_Half half rate channel |  |
| Detailed Comments: | Used for BS51, BS52, BS53 |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_IFsetup_BS61orBS81_MO(srv:SERVICES; ur:B_4; sa:B_1; ce:B_2; rate:IA5String) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This TSO requests (e.g. from the control console of the test system) the test operator to setup the external interface, or the MS configuration in such a way that the MS, when trigged, is able to initiate the call for BS61 or BS81 with the specified ur, ce and sa values, and the first bearer capability is speech. After the operator finishes the required action and informs the test system, the TSO returns with value TRUE. This TSO is used in MO call test cases except TC_11_2, where this TSO is call for single-numbering scheme MT testing. Three negotiable parameters, number of stop bit, number of data bit, parity information, are asked as global IXIT parameters for all TE configurations. If necessary, the test operator refers to the manual provided by manufacturer to configure the MS. The possible values are: 1) srv ----- basic service - C_AltSpchData Basic Service 61, Alternatie Speech/data - C_SpchData Basic Service 81, Speech followed by data 2) ur ------ user rate - '0001'B 300 bit/s - '0010'B 1200 bit/s - '0111'B 1200/75 bit/s - '0011'B 2400 bit/s - '0100'B 4800 bit/s - '0101'B 9600 bit/s 3) sa ------ synchronous/asynchronous - '0'B synchronous - '1'B asynchronous 4) ce ------ connection element - '00'B transparent - '01'B non transparent 5) rate ----- channel rate - C_Full full rate channel - C_Half half rate channel |  |
| Detailed Comments: | used for BS61 and BS81 |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_IFsetup_TS6x_MO(srv:SERVICES; ur:B_4; ce:B_2; rate:IA5String) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This TSO requests (e.g. from the control console of the test system) the test operator to setup the external interface, or the MS configuration in such a way that the MS, when trigged, is able to initiate the call for TS61 and TS62 with the specified ur and ce values, and the first bearer capability is speech. After the operator finishes the required action and informs the test system, the TSO returns with value TRUE. This TSO is used in MO call test cases except TC_11_2, where this TSO is call for single-numbering scheme MT testing. Three negotiable parameters, number of stop bit, number of data bit, parity information, are asked as global IXIT parameters for all TE configurations. If necessary, the test operator refers to the manual provided by manufacturer to configure the MS. The possible values are: 1) srv ----- basic service - C_AltSpchFax Teleservice 61, Alternatie Speech and Facsimile group 3 - C_AutoFax Teleservice 62, Automatic Facsimile group 3 2) ur ------ user rate - '0011'B 2400 bit/s - '0100'B 4800 bit/s - '0101'B 9600 bit/s 3) ce ------ connection element - '00'B transparent - '01'B non transparent 4) rate ----- channel rate - C_Full full rate channel - C_Half half rate channel |  |
| Detailed Comments: | used for TS61 and TS62 |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_IFsetup_BS2x_MT(srv:SERVICES; itc:B_3) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This TSO requests (e.g. from the control console of the test system) the test operator to setup the external interface, or the MS configuration in such a way that the MS is able to successfully receive the call for the BS2x service `srv` with the specified itc value. After the operator finishes the required action and informs the test system, the TSO returns with value TRUE. Three negotiable parameters, number of stop bit, number of data bit, parity information, are asked as global IXIT paramters for all TE configurations. If necessary, the test operator refers to the manual provided by manufacturer to configure the MS. |  |
| Detailed Comments: | Used for BS21, BS22, BS24, BS25, BS26 |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_IFsetup_BS3x_MT(srv:SERVICES; itc, sacp:B_3) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This TSO requests (e.g. from the control console of the test system) the test operator to setup the external interface, orthe MS configuration in such a way that the MS is able to successfully receive the call for the BS3x service `srv` with specified itc and sacp values. After the operator finishes the required action and informs the test system, the TSO returns with value TRUE. Three negotiable parameters, number of stop bit, number of data bit, parity information, are asked as global IXIT parameters for all TE configurations. If necessary, the test operator refers to the manual provided by manufacturer to configure the MS. |  |
| Detailed Comments: | Used for BS31, BS32, BS33, BS34 |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_IFsetup_BS61orBS81_MT(srv:SERVICES; ur:B_4; sa:B_1) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This TSO requests (e.g. from the control console of the test system) the test operator to setup the external interface, or the MS configuration in such a way that the MS is able to successfully receive the call for BS61 or BS81 with the specified ur and sa values. After the operator finishes the required action and informs the test system, the TSO returns with value TRUE. Three negotiable parameters, number of stop bit, number of data bit, parity information, are asked as global IXIT parameters for all TE configurations. If necessary, the test operator refers to the manual provided by manufacturer to configure the MS. |  |
| Detailed Comments: | used for BS61 and BS81 |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_IFsetup_TS6x_MT(srv:SERVICES; ur:B_4) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This TSO requests (e.g. from the control console of the test system) the test operator to setup the external interface, or the MS configuration in such a way that the MS is able to successfully receive the call for TS61 and TS62 with the specified ur value. After the operator finishes the required action and informs the test system, the TSO returns with value TRUE. Three negotiable parameters, number of stop bit, number of data bit, parity information, are asked as global IXIT parameters for all TE configurations. If necessary, the test operator refers to the manual provided by manufacturer to configure the MS. |  |
| Detailed Comments: | used for TS61 and TS62 |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_InCallModi(srv:SERVICES) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to initiate a in-call modification at the Mobile Station under test, which is triggered by the calling tone identification (CNG) received by the MS, for the basic service 'srv', then returns TRUE. The calling test step can pass the 'srv' value either as TSPX or constant to the TSO. The possible service values include (see also test suite constant declarations): 1) Teleservices - C_Telephony - C_EmgCall - C_AltSpchFax - C_AutoFax 2) Bearer services - C_Async300, C_Async1200, C_Async120075, C_Async2400, C_Async4800, C_Async9600 - C_Sync1200, C_Sync2400, C_Sync4800, C_Sync9600 - C_PAD300, C_PAD1200, C_PAD120075, C_PAD2400, C_PAD4800, C_PAD9600 - C_Packet2400, C_Packet4800, C_Packet9600 - C_AltSpchData - C_SpchData |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_InitCall(srv:SERVICES) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to initiate an MS originating call for the basic service `srv`, then returns TRUE. The calling test step can pass the 'srv' value either as TSPX or constant to the TSO. The necessary MS configuration has been done in the corresponding TSO OO_IFsetup_xx...xx_MO before this TSO is called. The possible service values include (see also test suite constant declarations): 1) Teleservices - C_Telephony ----- TS11 - C_EmgCall ----- TS12 - C_AltSpchFax ----- TS61 - C_AutoFax ----- TS62 2) Bearer services - C_Async300 ----- BS21 - C_Async1200 ----- BS22 - C_Async120075 ----- BS23 - C_Async2400 ----- BS24 - C_Async4800 ----- BS25 - C_Async9600 ----- BS26 - C_Sync1200 ----- BS31 - C_Sync2400 ----- BS32 - C_Sync4800 ----- BS33 - C_Sync9600 ----- BS34 - C_PAD300 ----- BS41 - C_PAD1200 ----- BS42 - C_PAD120075 ----- BS43 - C_PAD2400 ----- BS44 - C_PAD4800 ----- BS45 - C_PAD9600 ----- BS46 - C_Packet2400 ----- BS51 - C_Packet4800 ----- BS52 - C_Packet9600 ----- BS53 - C_AltSpchData ----- BS61 - C_SpchData ----- BS81 |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_InitNonCallSS |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to initiate a non call related supplementary service which is supported by the MS. After the operator finishes the action, he informs the test system and the operation returns with the value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_InitSS(action: IA5String) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to initiate the required supplementary service by the MMI sequence `action`. After the operator finishes the action, he informs the test system and the operation returns with the value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_InServiceCHK |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation first requests (e.g. from the control console of the test system) the test operator to check whether the Mobile Station under test is in normal service state ("Idle, updated")-- listening to CCCH and BCCH and with U1 UPDATED status. If the operator informs (e.g. from the control console of the test system) the test system that the MS does not give any service indication, the operation returns FALSE. If the operator informs the test system that the MS gives service indication, the operation returns TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_Key |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation returns TRUE if a key of the SS' keyboard has been pressed. Otherwise it returns FALSE. After OO_PressKeyWhenInService was called, the TSO is then continuously called until the test operator hits a key or a timer expires. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_MptyCall |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to enter MultiParty MMI command at the Mobile Station under test, then it returns with the value TRUE after the operator finishes the action and informs the test system. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_MSSetupStoreClass1SMInMEMemory |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to setup the MS under test to store class 1 SM in the ME memory (by way of MMI, as described in PICS/PIXIT statement). After the operator finishes the action, he informs the test system and the operation returns with the value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_PLMNsCHK |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation first requests (e.g. from the control console of the test system) the test operator to check whether the Mobile Station presents a list of available PLMNs. The list shall include the MNC and MCC of cells 1 to 7, but not cell 8 (for GSM900) or cells 1 to 6, but not cell 7 (for DCS1800). If the operator informs (e.g. from the control console of the test system) the test system that the MS correctly presents the list, the operation returns TRUE. If the operator informs the test system that the list is incorrect, the operation returns FALSE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_PLMNselModeAuto |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation switches the PLMN selection mode of the MS to automatic selection. After correctly switching of the mode the operation returns TRUE otherwise FALSE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_PLMNselModeMan |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation switches the PLMN selection mode of the MS to manual selection. After correctly switching of the mode the operation returns TRUE otherwise FALSE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_PowerUp |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to power up the Mobile Station under test and to inform after the test system, then it returns with value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_PowerDown |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to power down the Mobile Station under test and to inform after the test system, then it returns with value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_PressKeyWhenInService |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to press any key when the MS shows the service indicator, then returns. The return value is always TRUE. Working with OO_Key together, the TSO displays a prompt to the test operator and then immediately returns. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_RecallAndDisplaySM(NumbOfIA5chara: INTEGER; msg: OCTETSTRING) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to recall and display an SM stored in the ME of the MS under test e.g. by means of the MMI. Then the operator checks whether the short message contents represented by 'msg' is correctly displayed. The TSO returns TRUE: if SM can be recalled and displayed correctly in comparison with the 'msg', FALSE: if SM can not be recalled or displayed, or displayed incorrectly in comparison with the 'msg'. The TSO needs to convert the 'msg' into IA5 string according to the packing character rules specified in the section 6.1.2 of ETS 300 628 (GSM 03.38). The 'NumbOfIA5chara' indicates the number of IA5 characters in the 'msg'. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_RemoveOneSM |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to remove one of the SM from the message store and then to inform the test system. It does not matter which short message has been removed. After that the operation returns with value TRUE. The returned value is not used in the ATS. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_RemvPwrAmp |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to remove the added power amplification at the Mobile Station under test and to inform after the test system, then the operation returns with value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_RFoutputCHK |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation first requests (e.g. from the control console of the test system) the test operator to check whether the Mobile Station produces any RF output. If the operator informs (e.g. from the control console of the test system) the test system that the MS does not produce any RF output, the operation returns FALSE. If the operator informs the test system that the MS produces RF output, the operation returns TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_SelPLMN(par_plmn:OCTETSTRING) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to select the given PLMN in par_plmn manually, and to inform after the test system then the operation returns with value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_SendMOShortMessage |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to send a short message from the mobile station under test. The operation waits for the operator to acknowldge the request which may be done before or after sending the MO Short Message, and then returns the value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_SendSMSCOMMANDDel |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to send a SMS COMMAND message from the mobile station under test containing requiring to delete the previouly submited SM and then to inform the test system. Then, the operation returns with value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_SendSMSCOMMANDEnq |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to send a SMS COMMAND message from the mobile station under test containing an enquiry about the previouly submited SMand then to inform the test system. Then, the operation returns with value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_SetRefuseCall |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation first requests (e.g. from the control console of the test system) the test operator to enable call refusal on the MS. After the operator completes the action and informs (e.g. from the control console of the test system) the test system, the operation returns with value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_ShortKeyDepr(ch:IA5String) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to cause a DTMF tone to be generated, e.g. by short depression of the key `ch` on the Mobile Station under test. The operation waits for the operator to acknowldge the request which may be done before or after generating the DTMF tone, and then returns the value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_SIMIns |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to insert the SIM card into the Mobile Station under test. After the operator completes the action and informs (e.g. from the control console of the test system) the test system, the operation returns with value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_SIM2Ins |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to insert the SIM card 2 into the Mobile Station under test. After the operator completes the action and informs (e.g. from the control console of the test system) the test system, the operation returns with value TRUE. The SIM Card 2 shall contain the following parameter values, which are different from dafault values(SIM Card 1): IMSI= '001011234' HPLMN_search_period=6min. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_SIM3Ins |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to insert the SIM card 3 into the Mobile Station under test and turn the MS power on, after the test operator finishes the action and informs the test system it returns with value TRUE. The SIM Card 3 shall contain fixed dialling number allocated and activated. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_SIMRmv |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to remove the SIM card from the Mobile Station under test. After the operator completes the action and informs (e.g. from the control console of the test system) the test system, the operation returns with value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_SIMSimulAttIndOK |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to check whether the SIM simulator indicates an attempt made by the ME to store the short message in the SIM. The SIM simulator returns the status response "OK" ('90 00"). Then returns. Attempt indicated: TRUE Attempt NOT indicated: FALSE |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_SIMSimulAttIndMemProblem |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to check whether the SIM simulator indicates an attempt made by the ME to store the short message in the SIM. The SIM simulator returns the status response "Memory Problem" ('92 40"). Then returns. Attempt indicated: TRUE Attempt NOT indicated: FALSE |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_SSresultCHK(svc:INTEGER) |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation first requests (e.g. from the control console of the test system) the test operator to check the user indication of the result of the supplementary service ` svc`. If the operator informs (e.g. from the control console of the test system) the test system that the indication is correct, the operation returns TRUE. If the operator informs the test system that the indication is wrong, the operation returns FALSE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_SwitchOn |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to switch on (or if there is no switch then to restore the power to) the Mobile Station under test. After the operator completes the action and informs (e.g. from the control console of the test system) the test system, the operation returns with value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_SwitchOff |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation requests (e.g. from the control console of the test system) the test operator to switch off (or if there is no switch then to remove the power from) the Mobile Station under test. After the operator completes the action and informs (e.g. from the control console of the test system) the test system, the operation returns with value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_TCHThroConnCHK |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation first requests (e.g. from the control console of the test system) the test operator to check whether the TCH is through connected. The SS has to generate a noise pattern so that the operator can check this. If the operator informs (e.g. from the control console of the test system) the test system that the TCH is through connected, the operation returns TRUE. If the operator informs the test system that the TCH is not through connected, the operation returns FALSE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_TermCall |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation first requests (e.g. from the control console of the test system) the test operator to terminate the ongoing call. After the operator completes the action and informs (e.g. from the control console of the test system) the test system, it returns TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_TguardTimeOut |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation informs (e.g. from the control console of the test system) the test operator that the T_guard timer has expired during the test case leading to an inconclusive or fail verdict and returns immediately with value TRUE. |  |
| Detailed Comments: |  |

| Test Suite Operation Definition |  |
| --- | --- |
| Operation Name: | OO_ToneStopCHK |
| Result Type: | BOOLEAN |
| Comments: |  |
| Description |  |
| This operation first requests (e.g. from the control console of the test system) the test operator to check whether the MS stops the tone generation If the operator informs (e.g. from the control console of the test system) the test system that the MS does not stop tone generation, the operation returns FALSE. If the operator informs the test system that the MS stops tone generation, the operation returns TRUE. Definition of "tone generation": -During MO call: MS generates alerting tone/presents inband information sent by the network. -During MT call: MS generates ringing tone. |  |
| Detailed Comments: |  |

## Test suite parameter declarations

| Test Suite Parameter Declarations |  |  |  |
| --- | --- | --- | --- |
| Parameter Name | Type | PICS/PIXIT Ref | Comments |
| TSPC_24DataF | BOOLEAN | PICS Table A.25 Item 12 | 2.4 k full rate data mode supported |
| TSPC_24DataH | BOOLEAN | PICS Table A.25 Item 13 | 2.4 k half rate data mode supported |
| TSPC_48DataF | BOOLEAN | PICS Table A.25 Item 14 | 4.8 k full rate data mode supported |
| TSPC_48DataH | BOOLEAN | PICS Table A.25 Item 15 | 4.8 k half rate data mode supported |
| TSPC_96Data | BOOLEAN | PICS Table A.25 Item 16 | 9.6 k full rate data mode supported |
| TSPC_Feat_A51 | BOOLEAN | PICS Table A.2 Item 17 | ciphering algorithm A5/1 supported |
| TSPC_Feat_A52 | BOOLEAN | PICS Table A.2 Item 18 | ciphering algorithm A5/2 supported |
| TSPC_AddCharSet | BOOLEAN | PICS Table A.25 Item 47 | A, B, C, D chars supported |
| TSPC_AddInfo_PseudoSynch | BOOLEAN | PICS | Pseudo synchronised supported |
| TSPC_AlertInd | BOOLEAN | PICS Table A.25 Item 49 | alerting indication to the user supported |
| TSPC_SvcOnTCH | BOOLEAN | PICS Table A.25 Item 22 | at least one service on traffic channel supported |
| TSPC_SMS | BOOLEAN | PICS Table A.25 Item 31 | at least one short message service supported |
| TSPC_SS | BOOLEAN | PICS Table A.25 Item 29 | at least one supplementary service supported |
| TSPC_AutoAutoMode | BOOLEAN | PICS Table A.25 Item 48 | automatically enter automatic selection of PLMN mode supported |
| TSPC_BasCharSet | BOOLEAN | PICS Table A.25 Item 46 | Chars 0-9, *, # supported |
| TSPC_CalledNumDisp | BOOLEAN | PICS Table A.2 Item 1 | called number display supported |
| TSPC_DCS | BOOLEAN | PICS Table A.1 Item 3 | testing DCS1800 |
| TSPC_DetachOnPwrDn | BOOLEAN | PICS Table A.25 Item 38 | detach on power down supported |
| TSPC_DetachOnSIMRmv | BOOLEAN | PICS Table A.25 Item 39 | detach on SIM remove supported |
| TSPC_DispRcvSMS | BOOLEAN | PICS Table A.25 Item 34 | display of received SMS supported |
| TSPC_DualRate | BOOLEAN | PICS Table A.25 Item 23 | dual rate channel types supported |
| TSPC_EGSM | BOOLEAN | PICS Table A.1 Item 2 | both standard and extended GSM band supported |
| TSPC_EmgOnly | BOOLEAN | PICS Table A.25 Item 27 | the only circuit switched basic service is emergency call |
| TSPC_followOnReq | BOOLEAN | PICS Table A.25 Item 53 | follow-on request procedure supported |
| TSPC_HalfRateData | BOOLEAN | PICS Table A.25 Item 6 | half rate data supported |
| TSPC_HalfRateSpeech | BOOLEAN | PICS Table A.25 Item 3 | half rate speech mode supported |
| TSPC_InCallMod | BOOLEAN | PICS Table A.25 Item 52 | In-Call modification supported |
| TSPC_NoimmConn | BOOLEAN | PICS Table A.25 Item 51 | at least one service not support immediate connection |
| TSPC_Feat_FND | BOOLEAN | PICS Table A.2 Item 21 | FND feature supported |
| TSPC_NonCallSS | BOOLEAN | PICS Table A.25 Item 30 | non call related supplementary service supported |
| TSPC_MTsvc | BOOLEAN | PICS Table A.25 Item 19 | at least one MT circuit switched basic service supported |
| TSPC_MOsvc | BOOLEAN | PICS Table A.25 Item 20 | at least one MO circuit switched basic service supported |
| TSPC_PGSM | BOOLEAN | PICS Table A.1 Item 1 | only standard GSM band supported |
| TSPC_RefusalCall | BOOLEAN | PICS Table A.25 Item 54 | refusal of call supported |
| TSPC_ReplaceSMS | BOOLEAN | PICS Table A.25 Item 33 | replace SMS supported |
| TSPC_ReplyProc | BOOLEAN | PICS Table A.25 Item 32 | (SMS) reply procedures supported |
| TSPC_RFAmp | BOOLEAN | PICS Table A.25 Item 55 | RF amplification supported |
| TSPC_SDCCHOnly | BOOLEAN | PICS Table A.25 Item 21 | only SDCCH supported |
| TSPC_Serv_SS_AoCC | BOOLEAN | PICS Table A.5 Item 14 | AOCC SS supported |
| TSPC_Serv_SS_BAIC | BOOLEAN | PICS Table A.5 Item 18 | BAIC SS supported |
| TSPC_Serv_SS_BI | BOOLEAN | PICS | BI SS supported |
| TSPC_Serv_SS_BICRoam | BOOLEAN | PICS Table A.5 Item 19 | BICRoam SS supported |
| TSPC_Serv_SS_BAOC | BOOLEAN | PICS Table A.5 Item 15 | BAOC SS supported |
| TSPC_Serv_SS_BOIC | BOOLEAN | PICS Table A.5 Item 16 | BOIC SS supported |
| TSPC_Serv_SS_BOICexHC | BOOLEAN | PICS Table A.5 Item 17 | BOICExHC SS supported |
| TSPC_Serv_SS_CFB | BOOLEAN | PICS Table A.5 Item 6 | CFB supported |
| TSPC_Serv_SS_CFNRy | BOOLEAN | PICS Table A.5 Item 7 | CFNry supported |
| TSPC_Serv_SS_CFNRc | BOOLEAN | PICS Table A.5 Item 8 | CFNRc supported |
| TSPC_Serv_SS_CFU | BOOLEAN | PICS Table A.5 Item 5 | CFU supported |
| TSPC_Serv_SS_HOLD | BOOLEAN | PICS Table A.5 Item 10 | Call Hold SS supported |
| TSPC_Serv_SS_MPTY | BOOLEAN | PICS Table A.5 Item 11 | multiparty SS supported |
| TSPC_Serv_SS_unstruct | BOOLEAN | PICS | USSD supported |
| TSPC_Serv_TS11 | BOOLEAN | PICS Table A.3 Item 1 | telephony supported |
| TSPC_Serv_TS12 | BOOLEAN | PICS Table A.3 Item 2 | emergency call supported |
| TSPC_Serv_TS21 | BOOLEAN | PICS Table A.3 Item 3 | SMS MT/PP supported |
| TSPC_Serv_TS22 | BOOLEAN | PICS Table A.3 Item 4 | SMS MO/PP supported |
| TSPC_Serv_TS23 | BOOLEAN | PICS Table A.3 Item 5 | SMS cell broadcast supported |
| TSPC_Serv_TS61 | BOOLEAN | PICS | alternate speech and G3 fax (TS61) supported |
| TSPC_Serv_TS62 | BOOLEAN | PICS | automatic G3 fax (TS62) supported |
| TSPC_Serv_BS21 | BOOLEAN | PICS Table A.4 Item 1 | data circuit duplex async 300 bit/s supported |
| TSPC_Serv_BS22 | BOOLEAN | PICS Table A.4 Item 2 | data circuit duplex async 1200 bit/s supported |
| TSPC_Serv_BS23 | BOOLEAN | PICS Table A.4 Item 3 | data circuit duplex async 1200/75 bit/s supported |
| TSPC_Serv_BS24 | BOOLEAN | PICS Table A.4 Item 4 | data circuit duplex async 2400 bit/s supported |
| TSPC_Serv_BS25 | BOOLEAN | PICS Table A.4 Item 5 | data circuit duplex async 4800 bit/s supported |
| TSPC_Serv_BS26 | BOOLEAN | PICS Table A.4 Item 6 | data circuit duplex async 9600 bit/s supported |
| TSPC_Serv_BS31 | BOOLEAN | PICS Table A.4 Item 7 | data circuit duplex sync 1200 bit/s supported |
| TSPC_Serv_BS32 | BOOLEAN | PICS Table A.4 Item 8 | data circuit duplex sync 2400 bit/s supported |
| TSPC_Serv_BS33 | BOOLEAN | PICS Table A.4 Item 9 | data circuit duplex sync 4800 bit/s supported |
| TSPC_Serv_BS34 | BOOLEAN | PICS Table A.4 Item 10 | data circuit duplex sync 9600 bit/s supported |
| TSPC_Serv_BS41 | BOOLEAN | PICS Table A.4 Item 11 | PAD access 300 bit/s supported |
| TSPC_Serv_BS42 | BOOLEAN | PICS Table A.4 Item 12 | PAD access 1200 bit/s supported |
| TSPC_Serv_BS43 | BOOLEAN | PICS Table A.4 Item 13 | PAD access 1200/75 bit/s supported |
| TSPC_Serv_BS44 | BOOLEAN | PICS Table A.4 Item 14 | PAD access 2400 bit/s supported |
| TSPC_Serv_BS45 | BOOLEAN | PICS Table A.4 Item 15 | PAD access 4800 bit/s supported |
| TSPC_Serv_BS46 | BOOLEAN | PICS Table A.4 Item 16 | PAD access 9600 bit/s supported |
| TSPC_Serv_BS51 | BOOLEAN | PICS Table A.4 Item 17 | packet access 2400 bit/s supported |
| TSPC_Serv_BS52 | BOOLEAN | PICS Table A.4 Item 18 | packet access 4800 bit/s supported |
| TSPC_Serv_BS53 | BOOLEAN | PICS Table A.4 Item 19 | packet access 9600 bit/s supported |
| TSPC_Serv_BS61 | BOOLEAN | PICS | alternate speech/data (BS61) supported |
| TSPC_Serv_BS81 | BOOLEAN | PICS | speech followed data (BS81) supported |
| TSPC_SMSStatusRepCap | BOOLEAN | PICS Table A.25 Item 35 | SMS status report capabilities supported |
| TSPC_StoreRcvSMSME | BOOLEAN | PICS Table A.25 Item 37 | Storage of received SMS in ME supported |
| TSPC_StoreRcvSMSSIM | BOOLEAN | PICS Table A.25 Item 36 | Storage of received SMS in SIM supported |
| TSPC_SIMRmv | BOOLEAN | PICS Table A.25 Item 40 | SIM removable without power down supported |
| TSPC_SwitchOnOff | BOOLEAN | PICS Table A.2 Item 15 | switch on/off supported |
| TSPC_TranspDataOnly | BOOLEAN | PICS Table A.25 Item 9 | only transparent data service supported |
| TSPX_AltNb | BOOLEAN | PIXIT | use alternative neighbour cells description. default : FALSE |
| TSPX_TE_stopbit | B_1 | PIXIT | TE conf. number stop bits. default value : '0'B, 1 bit |
| TSPX_TE_databit | B_1 | PIXIT | TE conf. number data bits. default value : '1'B, 8 bits |
| TSPX_TE_FLCT | FLWCNTL | PIXIT | TE flow control. default : 0, outband flow control |
| TSPX_TE_parity | B_3 | PIXIT | TE conf. parity. default : '001'B, no parity |
| TSPX_BS_21_itc1 | B_3 | PIXIT | itc value supported for BS21. default value : '001'B, UDI |
| TSPX_BS_21_more_itc | BOOLEAN | PIXIT | more than one itc value supported by BS21. default value : FALSE, only one itc |
| TSPX_BS_21_T_NT | BOOLEAN | PIXIT | both T and NT supported by BS21. default value : FALSE, not support both transparent and non transparent modes |
| TSPX_BS_21_ce | B_2 | PIXIT | ce value used in MO call and if the answer to TSPX_BS_21_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '00'B, transparent mode |
| TSPX_BS_21_sacp | B_3 | PIXIT | sacp value used in MO call. default value : '001'B, I440/450 |
| TSPX_BS_21_itc2 | B_3 | PIXIT | other itc value supported by BS21, it shall be differenet from the previous one if the answer to TSPX_BS_21_more_itc is TRUE. default value :'001'B, UDI |
| TSPX_BS_22_itc1 | B_3 | PIXIT | itc value supported by BS22. default value : '010'B, 3.1kHz audio, exPLMN |
| TSPX_BS_22_more_itc | BOOLEAN | PIXIT | more than one itc value supported by BS22. default value : FALSE, only one itc |
| TSPX_BS_22_T_NT | BOOLEAN | PIXIT | both T and NT supported by BS22. default value : FALSE, not support both transparent and non transparent modes |
| TSPX_BS_22_ce | B_2 | PIXIT | ce value used in MO call and if the answer to TSPX_BS_22_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '01'B, non transparent |
| TSPX_BS_22_sacp | B_3 | PIXIT | sacp value used in MO call. default value : '001'B, I440/450 |
| TSPX_BS_22_itc2 | B_3 | PIXIT | other itc value supported by BS22, it shall be differenet from the previous one if the answer to TSPX_BS_22_more_itc is TRUE. default value : '010'B, 3.1kHz audio, exPLMN |
| TSPX_BS_23_itc | B_3 | PIXIT | itc value supported by BS23. default value : '010'B, 3.1kHz audio, exPLMN |
| TSPX_BS_23_T_NT | BOOLEAN | PIXIT | both T and NT supported for BS23. default value : FALSE, not support both transparent and non transparent modes |
| TSPX_BS_23_ce | B_2 | PIXIT | ce value used in MO call. default value : '00'B, transparent |
| TSPX_BS_23_sacp | B_3 | PIXIT | sacp value used in MO call. default value : '001'B, I440/450 |
| TSPX_BS_24_itc1 | B_3 | PIXIT | itc value supported by BS24. default value : '010'B, 3.1kHz audio, exPLMN |
| TSPX_BS_24_more_itc | BOOLEAN | PIXIT | more than one itc value supported by BS24. default value : FALSE, only one itc |
| TSPX_BS_24_T_NT | BOOLEAN | PIXIT | both T and NT supported by BS24. default value : FALSE, not support both transparent and non transparent modes |
| TSPX_BS_24_ce | B_2 | PIXIT | ce value used in MO call and if the answer to TSPX_BS_24_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '00'B, transparent |
| TSPX_BS_24_sacp | B_3 | PIXIT | sacp value used in MO call. default value : '001'B, I440/450 |
| TSPX_BS_24_itc2 | B_3 | PIXIT | other itc value supported by BS24, it shall be differenet from the previous one if the answer to TSPX_BS_24_more_itc is TRUE. default value : '010'B, 3.1kHz audio, exPLMN |
| TSPX_BS_25_itc1 | B_3 | PIXIT | itc value supported by BS25. default value : '001'B, UDI |
| TSPX_BS_25_more_itc | BOOLEAN | PIXIT | more than one itc value supported by BS25. default value : FALSE, only one itc |
| TSPX_BS_25_T_NT | BOOLEAN | PIXIT | both T and NT supported by BS25. default value : FALSE, not support both transparent and non transparent modes |
| TSPX_BS_25_ce | B_2 | PIXIT | ce value used in MO call and if the answer to TSPX_BS_25_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '00'B, transparent |
| TSPX_BS_25_sacp | B_3 | PIXIT | sacp value used in MO call. default value : '001'B, I440/450 |
| TSPX_BS_25_itc2 | B_3 | PIXIT | other itc value supported by BS25, it shall be differenet from the previous one if the answer to TSPX_BS_25_more_itc is TRUE. default value : '001'B, UDI |
| TSPX_BS_26_itc1 | B_3 | PIXIT | itc value supported by BS26. default value : '010'B, 3.1kHz audio, exPLMN |
| TSPX_BS_26_more_itc | BOOLEAN | PIXIT | more than one itc value supported by BS26. default value : FALSE, only one itc |
| TSPX_BS_26_T_NT | BOOLEAN | PIXIT | both T and NT supported by BS26. default value : FALSE, not support both transparent and non transparent modes |
| TSPX_BS_26_ce | B_2 | PIXIT | ce value used in MO call and if the answer to TSPX_BS_26_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '00'B, transparent |
| TSPX_BS_26_sacp | B_3 | PIXIT | sacp value used in MO call. default value : '001'B, I440/450 |
| TSPX_BS_26_itc2 | B_3 | PIXIT | other itc value supported by BS26, it shall be differenet from the previous one if the answer to TSPX_BS_26_more_itc is TRUE. default value : '010'B, 3.1kHz audio, exPLMN |
| TSPX_BS_31_more_itc | BOOLEAN | PIXIT | more than one itc value supported by BS31. default value : FALSE, only one itc |
| TSPX_BS_31_more_sacp | BOOLEAN | PIXIT | more than one sacp value supported by BS31. default value : FALSE, only one sacp |
| TSPX_BS_31_itc1 | B_3 | PIXIT | itc value supported by BS31. default value : '001'B, UDI |
| TSPX_BS_31_sacp1 | B_3 | PIXIT | sacp value supported by BS31. default value : '001'B, I440/450 |
| TSPX_BS_31_itc2 | B_3 | PIXIT | other itc value supported by BS31, it shall be differenet from the previous one if the answer to TSPX_BS_31_more_itc is TRUE. default value : '001'B, UDI |
| TSPX_BS_31_sacp2 | B_3 | PIXIT | other sacp value supported by BS31, it shall be differenet from the previous one if the answer to TSPX_BS_31_more_sacp is TRUE. default value : '001'B, I440/450 |
| TSPX_BS_32_itc1 | B_3 | PIXIT | itc value supported by BS32. default value : '010'B, 3.1kHz audio, exPLMN |
| TSPX_BS_32_more_itc | BOOLEAN | PIXIT | more than one itc value supported by BS32. default value : FALSE, only one itc |
| TSPX_BS_32_more_sacp | BOOLEAN | PIXIT | more than one sacp value supported by BS32. default value : FALSE, only one sacp |
| TSPX_BS_32_sacp1 | B_3 | PIXIT | sacp value supported by BS32. default value : '110'B, X.32 |
| TSPX_BS_32_X32_T_NT | BOOLEAN | PIXIT | BS 32 both T and NT supported for X32 3.1kHz. default value : FALSE, not support both transparent and non transparent modes |
| TSPX_BS_32_X32_ce | B_2 | PIXIT | if the answer to TSPX_BS_32_X32_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '01'B, non transparent |
| TSPX_BS_32_itc2 | B_3 | PIXIT | other itc value supported by BS32, it shall be differenet from the previous one if the answer to TSPX_BS_32_more_itc is TRUE. default value : '010'B, 3.1kHz audio, exPLMN |
| TSPX_BS_32_sacp2 | B_3 | PIXIT | other sacp value supported by BS32, it shall be differenet from the previous one if the answer to TSPX_BS_32_more_sacp is TRUE. default value : '110'B, X.32 |
| TSPX_BS_33_itc1 | B_3 | PIXIT | itc value supported by BS33. default value : '001'B, UDI |
| TSPX_BS_33_more_itc | BOOLEAN | PIXIT | more than one itc value supported by BS33. default value : FALSE, only one itc |
| TSPX_BS_33_more_sacp | BOOLEAN | PIXIT | more than one sacp value supported by BS33. default value : FALSE, only one sacp |
| TSPX_BS_33_sacp1 | B_3 | PIXIT | sacp value supported by BS33. default value : '010'B, X.21 |
| TSPX_BS_33_X32_T_NT | BOOLEAN | PIXIT | BS 33 both T and NT supported for X32 3.1kHz. default value : FALSE, not support both transparent and non transparent modes |
| TSPX_BS_33_X32_ce | B_2 | PIXIT | if the answer to TSPX_BS_33_X32_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '00'B, transparent |
| TSPX_BS_33_itc2 | B_3 | PIXIT | other itc value supported by BS32, it shall be differenet from the previous one if the answer to TSPX_BS_33_more_itc is TRUE. default value : '001'B, UDI |
| TSPX_BS_33_sacp2 | B_3 | PIXIT | other sacp value supported by BS33, it shall be differenet from the previous one if the answer to TSPX_BS_33_more_sacp is TRUE. default value : '010'B, X.21 |
| TSPX_BS_34_itc1 | B_3 | PIXIT | itc value supported by BS34. default value : '010'B, 3.1kHz audio, exPLMN |
| TSPX_BS_34_more_itc | BOOLEAN | PIXIT | more than one itc value supported by BS34. default value : FALSE, only one itc |
| TSPX_BS_34_more_sacp | BOOLEAN | PIXIT | more than one sacp value supported by BS34. default value : FALSE, only one sacp |
| TSPX_BS_34_sacp1 | B_3 | PIXIT | sacp value supported by BS34. default value : '001'B, I440/450 |
| TSPX_BS_34_X32_T_NT | BOOLEAN | PIXIT | BS 34 both T and NT supported for X32 3.1kHz. default value : FALSE, not support both transparent and non transparent modes |
| TSPX_BS_34_X32_ce | B_2 | PIXIT | if the answer to TSPX_BS_34_X32_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '00'B, transparent |
| TSPX_BS_34_itc2 | B_3 | PIXIT | other itc value supported by BS34, it shall be differenet from the previous one if the answer to TSPX_BS_34_more_itc is TRUE. default value : '010'B, 3.1kHz audio, exPLMN |
| TSPX_BS_34_sacp2 | B_3 | PIXIT | other sacp value supported by BS34, it shall be differenet from the previous one if the answer to TSPX_BS_34_more_sacp is TRUE. default value : '001'B, I440/450 |
| TSPX_BS_41_T_NT | BOOLEAN | PIXIT | BS41 both T and NT supported . default value : FALSE, not support both |
| TSPX_BS_41_ce | B_2 | PIXIT | ce value used in BS41 MO call. default value : '00'B, transparent |
| TSPX_BS_42_T_NT | BOOLEAN | PIXIT | BS42 both T and NT supported . default value : FALSE, not support both transparent and non transparent modes |
| TSPX_BS_42_ce | B_2 | PIXIT | ce value used in BS42 MO call. default value : '010'B, non transparent |
| TSPX_BS_43_T_NT | BOOLEAN | PIXIT | BS43 both T and NT supported . default value : FALSE, not support both transparent and non transparent modes |
| TSPX_BS_43_ce | B_2 | PIXIT | ce value used in BS43 MO call. default value : '00'B, transparent |
| TSPX_BS_44_T_NT | BOOLEAN | PIXIT | BS44 both T and NT supported . default value : FALSE, not support both transparent and non transparent modes |
| TSPX_BS_44_ce | B_2 | PIXIT | ce value used in BS44 MO call. default value : '00'B, transparent |
| TSPX_BS_45_T_NT | BOOLEAN | PIXIT | BS45 both T and NT supported . default value : FALSE, not support both transparent and non transparent modes |
| TSPX_BS_45_ce | B_2 | PIXIT | ce value used in BS45 MO call. default value : '01'B, non transparent |
| TSPX_BS_46_T_NT | BOOLEAN | PIXIT | BS46 both T and NT supported . default value : FALSE, not support both transparent and non transparent modes |
| TSPX_BS_46_ce | B_2 | PIXIT | ce value used in BS46 MO call. default value : '00'B, transparent |
| TSPX_BS_61_S | BOOLEAN | PIXIT | synchronous data supported by BS 61 . default value : TRUE, synchronous mode |
| TSPX_BS_61_S_more_ur | BOOLEAN | PIXIT | more than one user rate for synchronous data service supported by BS61. default value : FALSE, only ine user rate |
| TSPX_BS_61_S_ur1 | B_4 | PIXIT | usrer rate value supported for synchronous data service of BS61. default value : '0100'B, 4.8 kbit/s |
| TSPX_BS_61_S_ur2 | B_4 | PIXIT | another usrer rate value supported for synchronous data service of BS61. it shall be different from the previous one if the answer to TSPX_BS_61_S_more_ur is TRUE. default value : '0100'B, 4.8 kbit/s |
| TSPX_BS_61_A | BOOLEAN | PIXIT | asynchronous data service supported by BS61 . default value : TRUE, asynchronous |
| TSPX_BS_61_A_ur1 | B_4 | PIXIT | usrer rate value supported for asynchronous data service of BS61. default value : '0101'B, 9.6 kbit/s |
| TSPX_BS_61_A_ur1_T_NT | BOOLEAN | PIXIT | both T and NT supported for the rate TSPX_BS_61_A_ur1 of BS61. default value : FALSE, not support both transparent and non transparent modes |
| TSPX_BS_61_A_ur1_ce | B_2 | PIXIT | if the answer to TSPX_BS_61_A_ur1_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '00'B, transparent |
| TSPX_BS_61_A_more_ur | BOOLEAN | PIXIT | more than one user rate for asynchronous data service supported by BS61. default value : FALSE, only one user rate |
| TSPX_BS_61_A_ur2 | B_4 | PIXIT | another usrer rate value supported for asynchronous data service of BS61. it shall be different from the previous one if the answer to TSPX_BS_61_A_more_ur is TRUE. default value : '0101'B, 9.6 kbit/s |
| TSPX_BS_61_A_ur2_T_NT | BOOLEAN | PIXIT | both T and NT supported for the rate TSPX_BS_61_A_ur2 of BS61. default value : FALSE, not support both transparent and non transparent modes |
| TSPX_BS_81_S | BOOLEAN | PIXIT | synchronous data supported by BS 81. default value : TRUE, synchronous mode |
| TSPX_BS_81_S_more_ur | BOOLEAN | PIXIT | more than one user rate for synchronous data service supported by BS81. default value : FALSE, only one user rate |
| TSPX_BS_81_S_ur1 | B_4 | PIXIT | usrer rate value supported for synchronous data service of BS81. default value : '0101'B, 9.6 kbit/s |
| TSPX_BS_81_S_ur2 | B_4 | PIXIT | another usrer rate value supported for synchronous data service of BS81. it shall be different from the previous one if the answer to TSPX_BS_81_S_more_ur is TRUE. default value : '0101'B, 9.6 kbit/s |
| TSPX_BS_81_A | BOOLEAN | PIXIT | asynchronous data service supported by BS81. default value : TRUE, asynchronous mode |
| TSPX_BS_81_A_ur1 | B_4 | PIXIT | usrer rate value supported for asynchronous data service of BS81. default value : '0100'B, 4.8 kbit/s |
| TSPX_BS_81_A_ur1_T_NT | BOOLEAN | PIXIT | both T and NT supported for the rate TSPX_BS_81_A_ur1 of BS81. default value : FALSE, not support both transparent and non transparent modes |
| TSPX_BS_81_A_ur1_ce | B_2 | PIXIT | if the answer to TSPX_BS_81_A_ur1_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '01'B, non transparent |
| TSPX_BS_81_A_more_ur | BOOLEAN | PIXIT | more than one user rate for asynchronous data service supported by BS81. default value : FALSE, only one user rate |
| TSPX_BS_81_A_ur2 | B_4 | PIXIT | another usrer rate value supported for asynchronous data service of BS81. it shall be different from the previous one if the answer to TSPX_BS_81_A_more_ur is TRUE. default value : '0100'B, 4.8 kbit/s |
| TSPX_BS_81_A_ur2_T_NT | BOOLEAN | PIXIT | both T and NT supported for the rate TSPX_BS_81_A_ur2 of BS81. default value : FALSE, not support both transparent and non transparent modes |
| TSPX_TS_61_T_NT | BOOLEAN | PIXIT | both T and NT supported for TS61(fax). default value : FALSE, not support both transparent and non transparent modes |
| TSPX_TS_61_ce | B_2 | PIXIT | if the answer to TSPX_TS_61_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '01'B, non transparent |
| TSPX_TS_61_ur1 | B_4 | PIXIT | user rate for TS61(fax). default value : '0101'B, 9.6 kbit/s |
| TSPX_TS_61_more_ur | BOOLEAN | PIXIT | more than one user rate supported for TS61. default value : FALSE, only one user rate |
| TSPX_TS_61_ur2 | B_4 | PIXIT | another user rate for TS61, it shall be different from the previous one if the answer to TSPX_TS_61_more_ur is TRUE. default value : '0101'B, 9.6 kbit/s |
| TSPX_TS_62_T_NT | BOOLEAN | PIXIT | both T and NT supported for TS62. default value : FALSE, not support both transparent and non transparent modes |
| TSPX_TS_62_ce | B_2 | PIXIT | if the answer to TSPX_TS_62_T_NT is FALSE, the manufacturer must precise which mode is supported by the MS. default value : '00'B, transparent |
| TSPX_TS_62_ur1 | B_4 | PIXIT | user rate for TS62. default value : '0100'B, 4.8 kbit/s |
| TSPX_TS_62_more_ur | BOOLEAN | PIXIT | more than one user rate supported by TS62. default value : FALSE, only one user rate |
| TSPX_TS_62_ur2 | B_4 | PIXIT | another user rate for TS62, it shall be different from the previous one if the answer to TSPX_TS_62_more_ur is TRUE. default value : '0100'B, 4.8 kbit/s |
| TSPX_DTMF | B_1 | PIXIT | call control capabilities: value '1'B means MS supports DTMF |
| TSPX_modF | CHMOD_VAL | PIXIT | any non signalling full rate channel mode for TC_26_6_4_1 |
| TSPX_modH | CHMOD_VAL | PIXIT | any non signalling half channel mode for TC_26_6_4_1 |
| TSPX_anymod | CHMOD_VAL | PIXIT | channel mode for TC_62_10_2_4_2. Any supported value except signalling and Full rate speech. possible values are '00000011'B' 12k data, '00001011'B 6k data, '0010011'B 3.6k data |
| TSPX_CKSNA | CKSN | PIXIT | cipher key sequence number |
| TSPX_CKSNB | CKSN | PIXIT | cipher key sequence number |
| TSPX_CKSNC | CKSN | PIXIT | cipher key sequence number |
| TSPX_CKSNDef | CKSN | PIXIT | default cipher key sequence number |
| TSPX_RfPwrCap | B_3 | PIXIT | RF power capability |
| TSPX_RevLevel | B_2 | PIXIT | Revision level of classmark 1 |
| TSPX_CiphAlgA5_1 | B_1 | PIXIT | A5/1 algorithm: '1'B : available, '0'B: not available |
| TSPX_CiphAlgA5_2 | B_1 | PIXIT | A5/2 algorithm: '1'B : available, '0'B: not available |
| TSPX_CiphAlgA5_3 | B_1 | PIXIT | A5/3 algorithm: '1'B : available, '0'B: not available |
| TSPX_CiphAlgA5_4 | B_1 | PIXIT | A5/4 algorithm: '1'B : available, '0'B: not available |
| TSPX_CiphAlgA5_5 | B_1 | PIXIT | A5/5 algorithm: '1'B : available, '0'B: not available |
| TSPX_CiphAlgA5_6 | B_1 | PIXIT | A5/6 algorithm: '1'B : available, '0'B: not available |
| TSPX_CiphAlgA5_7 | B_1 | PIXIT | A5/7 algorithm: '1'B : available, '0'B: not available |
| TSPX_pSyncCap | B_1 | PIXIT | pseudo synchronisation capability, '0'B: not present '1'B: present |
| TSPX_SSscrnInd | B_2 | PIXIT | SS screen indicator |
| TSPX_SMCap | B_1 | PXIIT | short message capability, '0'B: not present '1'B: present |
| TSPX_frqCap | B_1 | PIXIT | frequency capability. DCS: reserved, GSM: '0'B not support extension band, '1'B support extension band |
| TSPX_ClsMk3 | B_1 | PIXIT | classmark 3 indicator, '0'B no addition capability info, '1'B additional capability info in class mark 3 |
| TSPX_RfPwrCapAmp | B_3 | PIXIT | RF power capability with external RF amplifier |
| TSPX_CphAlgA | CPHALG | PIXIT | any supported ciphering algorithm |
| TSPX_CphAlgB | CPHALG | PIXIT | ciphering algorithm supported |
| TSPX_CphAlgC | CPHALG | PIXIT | ciphering algorithm supported |
| TSPX_CphAlgD | CPHALG | PIXIT | ciphering algorithm supported |
| TSPX_CphAlgE | CPHALG | PIXIT | ciphering algorithm supported |
| TSPX_CphAlgDef | CPHALG | PIXIT | default ciphering algorithm |
| TSPX_DTMFInd | BOOLEAN | PIXIT | DTMF indication to user supported |
| TSPX_NoOfHoAccA | INTEGER | PIXIT | number of handover access bursts, value range 10 - 20 |
| TSPX_NoOfHoAccB | INTEGER | PIXIT | number of handover access bursts, value range 10 - 20 |
| TSPX_NoOfHoAccC | INTEGER | PIXIT | number of handover access bursts, value range 10 - 20 |
| TSPX_NoOfHoAccD | INTEGER | PIXIT | number of handover access bursts, value range 5 - 10 |
| TSPX_NoOfHoAccE | INTEGER | PIXIT | number of handover access bursts, value range 5 - 10 |
| TSPX_NoOfHoAccF | INTEGER | PIXIT | number of handover access bursts, value range 5 - 10 |
| TSPX_NoOfHoAccG | INTEGER | PIXIT | number of handover access bursts, value range 2 - 5 |
| TSPX_NoOfHoAccH | INTEGER | PIXIT | number of handover access bursts, value range 2 - 5 |
| TSPX_NoOfHoAccI | INTEGER | PIXIT | number of handover access bursts, value range 2 - 5 |
| TSPX_HoRefA | HORF | PIXIT | Hand over reference, GSM 04.08, 10.5.2.15 |
| TSPX_HoRefB | HORF | PIXIT | Hand over reference, GSM 04.08, 10.5.2.15 |
| TSPX_HoRefC | HORF | PIXIT | Hand over reference, GSM 04.08, 10.5.2.15 |
| TSPX_HoRefD | HORF | PIXIT | Hand over reference, GSM 04.08, 10.5.2.15 |
| TSPX_HoRefE | HORF | PIXIT | Hand over reference, GSM 04.08, 10.5.2.15 |
| TSPX_HoRefF | HORF | PIXIT | Hand over reference, GSM 04.08, 10.5.2.15 |
| TSPX_HoRefG | HORF | PIXIT | Hand over reference, GSM 04.08, 10.5.2.15 |
| TSPX_HoRefH | HORF | PIXIT | Hand over reference, GSM 04.08, 10.5.2.15 |
| TSPX_HoRefI | HORF | PIXIT | Hand over reference, GSM 04.08, 10.5.2.15 |
| TSPX_HoRefJ | HORF | PIXIT | Hand over reference, GSM 04.08, 10.5.2.15 |
| TSPX_HSN | INTEGER | PIXIT | Hopping sequence number value range: 0 - 63 |
| TSPX_IMSI | HEXSTRING | PIXIT | IMSI of the MS |
| TSPX_IMEI | HEXSTRING | PIXIT | IMEI of the MS. |
| TSPX_IMEISV | HEXSTRING | PIXIT | IMEISV of the MS. Used in: TC_26_6_8_5, TC_26_7_3_1 |
| TSPX_Ki | BITSTRING | PIXIT | default authentication key used in testing |
| TSPX_MAIO | INTEGER | PIXIT | mobile allocation index offset, value range: 0 - 63 |
| TSPX_MaxRetrans | INTEGER | PIXIT | Max-Retrans |
| TSPX_MSTxpwrMax | MAXTXPOW | PIXIT | maximum output power from MS |
| TSPX_PwrlvlA | INTEGER | PIXIT | power level, value between 2 to 15 |
| TSPX_PwrlvlB | INTEGER | PIXIT | power level, value between 2 to 15 |
| TSPX_PwrlvlC | INTEGER | PIXIT | power level, value between 2 to 15 |
| TSPX_PwrlvlD | INTEGER | PIXIT | power level, value between 2 to 15 |
| TSPX_RANDA | RAND | PIXIT | challenge RAND |
| TSPX_RANDB | RAND | PIXIT | challenge RAND |
| TSPX_RANDC | RAND | PIXIT | challenge RAND |
| TSPX_RANDDef | RAND | PIXIT | default challenge RAND |
| TSPX_SDCCH4SubA | B_2 | PIXIT | TDMA offset of SDCCH/4 subchannel |
| TSPX_SDCCH4SubB | B_2 | PIXIT | TDMA offset of SDCCH/4 subchannel |
| TSPX_SDCCH4SubC | B_2 | PIXIT | TDMA offset of SDCCH/4 subchannel |
| TSPX_SDCCH4SubDef | B_2 | PIXIT | TDMA offset of default SDCCH/4 subchannel |
| TSPX_SDCCH8SubA | B_3 | PIXIT | TDMA offset of SDCCH/8 subchannel |
| TSPX_SDCCH8SubB | B_3 | PIXIT | TDMA offset of SDCCH/8 subchannel |
| TSPX_SDCCH8SubC | B_3 | PIXIT | TDMA offset of SDCCH/8 subchannel |
| TSPX_SDCCH8SubD | B_3 | PIXIT | TDMA offset of SDCCH/8 subchannel |
| TSPX_SDCCH8SubE | B_3 | PIXIT | TDMA offset of SDCCH/8 subchannel |
| TSPX_SDCCH8SubF | B_3 | PIXIT | TDMA offset of SDCCH/8 subchannel |
| TSPX_SDCCH8SubG | B_3 | PIXIT | TDMA offset of SDCCH/8 subchannel |
| TSPX_SDCCH8SubDef | B_3 | PIXIT | TDMA offset of default SDCCH/8 subchannel |
| TSPX_TC1M | INTEGER | PIXIT | timer value for GSM timer TC1M (for SMS) |
| TSPX_MaxCPDataRetx | INTEGER | PIXIT | max. number of CP data retransmissions for SMS |
| TSPX_Immconn | BOOLEAN | PIXIT | Immediate connect supported |
| TSPX_TimadvA | INTEGER | PIXIT | timing advance |
| TSPX_TimadvB | INTEGER | PIXIT | timing advance |
| TSPX_TimadvC | INTEGER | PIXIT | timing advance |
| TSPX_TmSltA | SN | PIXIT | time slot |
| TSPX_TmSltB | SN | PIXIT | time slot |
| TSPX_TmSltC | SN | PIXIT | time slot |
| TSPX_TmSltD | SN | PIXIT | time slot |
| TSPX_TmSltE | SN | PIXIT | time slot |
| TSPX_TmSltF | SN | PIXIT | time slot |
| TSPX_TmSltG | SN | PIXIT | time slot |
| TSPX_TmSltDef | SN | PIXIT | default time slot |
| TSPX_TmSltNotZero | SN | PIXIT | time slot, arbitrarily value, but not zero. |
| TSPX_TmSltNotZero1 | SN | PIXIT | time slot, arbitrarily value, but not zero and not TSPX_TmSltNotZero |
| TSPX_Txint | INTEGER | PIXIT | Tx-Integer |
| TSPX_TscA | TSC | PIXIT | training sequence code |
| TSPX_TscB | TSC | PIXIT | training sequence code |
| TSPX_TscC | TSC | PIXIT | training sequence code |
| TSPX_TscD | TSC | PIXIT | training sequence code |
| TSPX_TscE | TSC | PIXIT | training sequence code |
| TSPX_TscF | TSC | PIXIT | training sequence code |
| TSPX_TscG | TSC | PIXIT | training sequence code |
| TSPX_TscDef | TSC | PIXIT | default TSC |
| TSPX_T3122 | WI | PIXIT | value of timer T3122, HEXSTRING[2] |
| TSPX_TCHcarrierA_ho | INTEGER | PIXIT | the value can be choosen arbitrarily from cell allocation of cell A (GSM), but not BCCH carrier .possible value is : 10, 17, 26, 34, 42, 45, 46, 52, 59, 66, 73, 74, 75, 76, 108, 114 |
| TSPX_TCHcarrierA_hod | INTEGER | PIXIT | the value can be choosen arbitrarily from cell allocation of cell A (DCS), but not BCCH carrier . possible value is : 734,741,754,759,762,766,767,773,775,779,782,791,798,829,832,844 |
| TSPX_TCHcarrierB | INTEGER | PIXIT | TCH and SDCCH channel frequency number of cell B (GSM) |
| TSPX_TCHcarrierBd | INTEGER | PIXIT | TCH and SDCCH channel frequency number of cell B (DCS) |
| TSPX_TCHcarrierB_ho | INTEGER | PIXIT | the value can be choosen arbitrarily from cell allocation of cell B (GSM), but not BCCH carrier . possibles value is : 14,18,22,24,30,31,38,60,66,73,74,75,76,108,114 |
| TSPX_TCHcarrierB_hod | INTEGER | PIXIT | not BCCH carrier of cell B. the value can be choosen arbitrarily from cell allocation of cell B (DCS), which is (739,743,746,749,756,758,761,771,779,782,791,798,829,832,844) |
| TSPX_TCHcarrierB2_ho | INTEGER | PIXIT | Choosen arbitrarly from cell allocation B for GSM HO cases, but not BCCH carrier! |
| TSPX_TCHcarrierB2_hod | INTEGER | PIXIT | Choosen arbitrarly from cell allocation B for DCS HO cases, but not BCCH carrier! |
| TSPX_TCHHSubA | B_1 | PIXIT | TDMA offset of half rate subchannel |
| TSPX_TCHHSubDef | B_1 | PIXIT | TDMA offset of default half rate subchannel |
| TSPX_TMSI | TMSI_V | PIXIT | TMSI of the MS used in test |
| TSPX_TMSI1 | TMSI_V | PIXIT | another TMSI used in test which shall differ from TSPX_TMSI, TSPX_TMSI + '01'O TSPX_TMSI + '02'O TSPX_TMSI + '03'O |
| TSPX_Uupd | B_8 | PIXIT | user-user protocol discriminator |
| TSPX_UuInfo | OCTETSTRING | PIXIT | user-user information |
| TSPX_k | INTEGER | PIXIT | timing of cell A before cell B k bit periods for TC_26_6_5_1, TC_26_6_5_2_? TC_26_6_5_3_?, TC_26_6_5_4_?. |
| TSPX_y | INTEGER | PIXIT | timing advance for TC_26_6_5_3_?, TC_26_6_5_4_?, TC_26_6_5_5_? . The values of TSPX_k (or TSPX_k1) and TSPX_y: 0 < (2*TSPX_k + TSPX_y) MOD 256 < 60. |
| TSPX_k1 | INTEGER | PIXIT | timing of cell A before cell B k bit periods for TC_26_6_5_5_? |
| TSPX_k2 | INTEGER | PIXIT | timing of cell A before cell B k bit periods for TC_26_6_5_6 |
| TSPX_y2 | INTEGER | PIXIT | timing advance for TC_26_6_5_6, value range : 11 - 62. |
| TSPX_k3 | INTEGER | PIXIT | timing of cell A before cell B k bit periods for TC_26_6_5_7 |
| TSPX_y3 | INTEGER | PIXIT | timing advance for TC_26_6_5_7 |
| TSPX_nPara | INTEGER | PIXIT | the n'th ChReq for TC_26_6_1_2, shall be choose to [1..8], MAXRETRANS for TC_26_6_1_2 is 7 (See 11.10 for more explaination). |
| TSPX_kPara | INTEGER | PIXIT | the k'th ChReq for TC_26_6_1_2 |
| TSPX_rPara | INTEGER | PIXIT | the r'th ChReq for TC_26_6_1_2 |
| TSPX_i1Para | INTEGER | PIXIT | for TC_26_1_2 |
| TSPX_i2Para | INTEGER | PIXIT | for TC_26_1_2 |
| TSPX_i3Para | INTEGER | PIXIT | the r'th ChReq for TC_26_6_1_2 |
| TSPX_n1Para | INTEGER | PIXIT | the n'th ChReq for TC_26_6_1_3 |
| TSPX_i4Para | INTEGER | PIXIT | for TC_26_6_1_3 |
| TSPX_xPara | INTEGER | PIXIT | t3122 for TC_26_6_1_3 |
| TSPX_AGBLKS1 | INTEGER | PIXIT | BS-AG-BLKS-RES for TC_26_6_2_3_1, TC_26_6_2_1_1 |
| TSPX_PAMFRMS1 | INTEGER | PIXIT | BS-PA-MFRMS for TC_26_6_2_3_1 (shall not be set to 9), TC_26_6_2_1_1 |
| TSPX_CcchConf1 | CCCH_CON | PIXIT | paging subchannel for TC_26_6_2_3_1, TC_26_6_2_1_1 |
| TSPX_PgSubch | INTEGER | PIXIT | paging subchannel for TC_26_6_2_3_1 |
| TSPX_AGBLKS2 | INTEGER | PIXIT | BS-AG-BLKS-RES for TC_26_6_2_3_2, TC_26_6_2_1_2 |
| TSPX_PAMFRMS2 | INTEGER | PIXIT | BS-PA-MFRMS for TC_26_6_2_3_2, TC_26_6_2_1_2 |
| TSPX_CcchConf2 | CCCH_CON | PIXIT | paging subchannel for TC_26_6_2_3_2, TC_26_6_2_1_2 |
| TSPX_AGBLKS3 | INTEGER | PIXIT | BS-AG-BLKS-RES for TC_26_6_2_4, TC_26_6_2_2, TC_26_6_2_1_3 |
| TSPX_PAMFRMS3 | INTEGER | PIXIT | BS-PA-MFRMS for TC_26_6_2_4, TC_26_6_2_2, TC_26_6_2_1_3 |
| TSPX_CcchConf3 | CCCH_CON | PIXIT | paging subchannel for TC_26_6_2_4, TC_26_6_2_2, TC_26_6_2_1_3 |
| TSPX_AGBLKS4 | INTEGER | PIXIT | BS-AG-BLKS-RES for TC_26_6_2_5 |
| TSPX_PAMFRMS4 | INTEGER | PIXIT | BS-PA-MFRMS for TC_26_6_2_5 |
| TSPX_CcchConf4 | CCCH_CON | PIXIT | paging subchannel for TC_26_6_2_5 (shall be in the set ('010', '100', '110')) |
| TSPX_Chtp1 | CH_TDMA | PIXIT | channel type for TC_26_6_13_1, any value supported by the MS |
| TSPX_ChMod1 | CHMOD_VAL | PIXIT | channel mode for TC_26_6_13_1, any value for the channel type of TSPX_Chtp1 |
| TSPX_Maio1 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_1, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma1, default : 2 |
| TSPX_Ma1 | MAC | PIXIT | mobile allocation for TC_26_6_13_1, its value shall indicate number of frequencies between 1 and 17. default : '010101'O |
| TSPX_Hsn1 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_1. default: 1 |
| TSPX_Maio2 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_1, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma2, default : 8 |
| TSPX_Ma2 | MAC | PIXIT | mobile allocation for TC_26_6_13_1, its value shall indicate number of frequencies between 1 and 17. default :'01362A5'O |
| TSPX_Hsn2 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_1, default : 6 |
| TSPX_Maio3 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_1, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma3, default : 14 |
| TSPX_Ma3 | MAC | PIXIT | mobile allocation for TC_26_6_13_1, its value shall indicate number of frequencies between 1 and 17. default :'01FFFA'O |
| TSPX_Hsn3 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_1, default : 0 |
| TSPX_Chtp2 | CH_TDMA | PIXIT | channel type for TC_26_6_13_2, any value supported by the MS |
| TSPX_ChMod2 | CHMOD_VAL | PIXIT | channel mode for TC_26_6_13_2 |
| TSPX_Maio4 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_2, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma4, default : 3 |
| TSPX_Ma4 | MAC | PIXIT | mobile allocation for TC_26_6_13_2, its value shall indicate number of frequencies between 1 and 17. default : '001141'O |
| TSPX_Hsn4 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_2, default : 0 |
| TSPX_Maio5 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_2, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma5, default : 9 |
| TSPX_Ma5 | MAC | PIXIT | mobile allocation for TC_26_6_13_2, its value shall indicate number of frequencies between 1 and 17. default : '01ABCB'O |
| TSPX_Hsn5 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_2, default : 16 |
| TSPX_Chtp3 | CH_TDMA | PIXIT | channel type for TC_26_6_13_3, any value supported by the MS |
| TSPX_Maio6 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_3, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma6, default : 4 |
| TSPX_Ma6 | MAC | PIXIT | mobile allocation for TC_26_6_13_3, its value shall indicate number of frequencies between 1 and 17. default :'006248'O |
| TSPX_Hsn6 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_3, default : 4 |
| TSPX_Maio7 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_3, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma7, default : 10 |
| TSPX_Ma7 | MAC | PIXIT | mobile allocation for TC_26_6_13_3, its value shall indicate number of frequencies between 2 and 17. default : '009AFB'O |
| TSPX_Chtp4 | CH_TDMA | PIXIT | channel type for TC_26_6_13_3, any value supported by the MS |
| TSPX_Maio8 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_3, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma8, default : 1 |
| TSPX_Ma8 | MAC | PIXIT | mobile allocation for TC_26_6_13_3, its value shall indicate number of frequencies between 2 and 17. default : '002800'O |
| TSPX_Hsn8 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_3, default : 40 |
| TSPX_Maio9 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_3, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma9, default : 15 |
| TSPX_Ma9 | MAC | PIXIT | mobile allocation for TC_26_6_13_3, its value shall indicate number of frequencies between 2 and 17. default : '01FFFB'O |
| TSPX_Hsn9 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_3, default : 8 |
| TSPX_Chtp5 | CH_TDMA | PIXIT | SDCCH8 subchannel immediately assigned, TC_26_6_13_4 |
| TSPX_Maio10 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_4, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma10, default : 5 |
| TSPX_Ma10 | MAC | PIXIT | mobile allocation for TC_26_6_13_4, its value shall indicate number of frequencies between 1 and 17. default : '01D082'O |
| TSPX_Hsn10 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_4, default: 0 |
| TSPX_Maio11 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_4, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma11, default : 11 |
| TSPX_Ma11 | MAC | PIXIT | mobile allocation for TC_26_6_13_4, its value shall indicate number of frequencies between 2 and 17. default : '00CFF3'O |
| TSPX_Chtp6 | CH_TDMA | PIXIT | channel type of a non- existing channel in Assignment Cmd, TC_26_6_13_4 |
| TSPX_Maio12 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_4, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma12, default : 1 |
| TSPX_Ma12 | MAC | PIXIT | mobile allocation for TC_26_6_13_4, its value shall indicate number of frequencies between 1 and 17. default : '010100'O |
| TSPX_Hsn12 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_4, default : 39 |
| TSPX_Maio13 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_4, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma13, default : 6 |
| TSPX_Ma13 | MAC | PIXIT | mobile allocation for TC_26_6_13_4, its value shall indicate number of frequencies between 1 and 17. default : '00E690'O |
| TSPX_Hsn13 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_4, default: 42 |
| TSPX_Chtp7 | CH_TDMA | PIXIT | channel type for TC_26_6_13_5 |
| TSPX_ChMod4 | CHMOD_VAL | PIXIT | channel mode for TC_26_6_13_5 |
| TSPX_Maio14 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_5, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma14, default : 5 |
| TSPX_Ma14 | MAC | PIXIT | mobile allocation for TC_26_6_13_5, its value shall indicate number of frequencies between 1 and 17. default : '014A28'O |
| TSPX_Hsn14 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_5, default : 50 |
| TSPX_Maio15 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_5, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma15, default : 12 |
| TSPX_Ma15 | MAC | PIXIT | mobile allocation for TC_26_6_13_5, its value shall indicate number of frequencies between 1 and 16. default : '00FF79'O |
| TSPX_Hsn15 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_5, default : 33 |
| TSPX_Maio16 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_5, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma16, default : 2 |
| TSPX_Ma16 | MAC | PIXIT | mobile allocation for TC_26_6_13_5, its value shall indicate number of frequencies between 1 and 16. default : '001110'O |
| TSPX_Hsn16 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_5, default : 21 |
| TSPX_Chtp8 | CH_TDMA | PIXIT | channel type for TC_26_6_13_6 |
| TSPX_ChMod5 | CHMOD_VAL | PIXIT | channel mode for TC_26_6_13_6 |
| TSPX_Maio17 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_6, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma17, default : 7 |
| TSPX_Ma17 | MAC | PIXIT | mobile allocation for TC_26_6_13_6, its value shall indicate number of frequencies between 1 and 17. default : '00E6A1'O |
| TSPX_Hsn17 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_6, default : 22 |
| TSPX_Maio18 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_6, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma18, default : 13 |
| TSPX_Ma18 | MAC | PIXIT | mobile allocation for TC_26_6_13_6, its value shall indicate number of frequencies between 1 and 16. default : '00FFF9'O |
| TSPX_Hsn18 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_6, default : 9 |
| TSPX_Maio19 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_6, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma19, default : 3 |
| TSPX_Ma19 | MAC | PIXIT | mobile allocation for TC_26_6_13_6, its value shall indicate number of frequencies between 1 and 16. default : '001111'O |
| TSPX_Hsn19 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_6, default : 44 |
| TSPX_Chtp9 | CH_TDMA | PIXIT | channel type for TC_26_6_13_7 |
| TSPX_ChMod6 | CHMOD_VAL | PIXIT | channel mode for TC_26_6_13_7 |
| TSPX_Maio20 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_7, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma20, default : 8 |
| TSPX_Ma20 | MAC | PIXIT | mobile allocation for TC_26_6_13_7, its value shall indicate number of frequencies between 1 and 17. default : '016699'O |
| TSPX_Hsn20 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_7, default : 30 |
| TSPX_Maio21 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_7, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma21, default : 14 |
| TSPX_Ma21 | MAC | PIXIT | mobile allocation for TC_26_6_13_7, its value shall indicate number of frequencies between 1 and 17. default : '01FEFB'O |
| TSPX_Chtp10 | CH_TDMA | PIXIT | channel type for TC_26_6_13_7 |
| TSPX_Maio22 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_7, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma22, default : 4 |
| TSPX_Ma22 | MAC | PIXIT | mobile allocation for TC_26_6_13_7, its value shall indicate number of frequencies between 1 and 16. default : '006241'O |
| TSPX_Hsn22 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_7, default : 11 |
| TSPX_Maio23 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_7, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma23, default : 11 |
| TSPX_Ma23 | MAC | PIXIT | mobile allocation for TC_26_6_13_7, its value shall indicate number of frequencies between 1 and 16. default : '00FF1B'O |
| TSPX_Hsn23 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_7, default : 60 |
| TSPX_Chtp11 | CH_TDMA | PIXIT | SDCCH8 subchannel immediately assigned, TC_26_6_13_8 |
| TSPX_ChMod7 | CHMOD_VAL | PIXIT | channel mode for TC_26_6_13_8 |
| TSPX_Maio24 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_8, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma24, default : 9 |
| TSPX_Ma24 | MAC | PIXIT | mobile allocation for TC_26_6_13_8, its value shall indicate number of frequencies between 1 and 17. default : '00E6E9'O |
| TSPX_Hsn24 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_8, default : 7 |
| TSPX_Maio25 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_8, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma25, default : 15 |
| TSPX_Ma25 | MAC | PIXIT | mobile allocation for TC_26_6_13_8, its value shall indicate number of frequencies between 1 and 17. default : '01FFFB'O |
| TSPX_Chtp12 | CH_TDMA | PIXIT | channel type of a non- existing channel in Assignment Cmd, TC_26_6_13_8 |
| TSPX_Maio26 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_8, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma26, default : 5 |
| TSPX_Ma26 | MAC | PIXIT | mobile allocation for TC_26_6_13_8, its value shall indicate number of frequencies between 1 and 16. default : '009168'O |
| TSPX_Hsn26 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_8, default : 9 |
| TSPX_Maio27 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_8, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma27, default : 1 |
| TSPX_Ma27 | MAC | PIXIT | mobile allocation for TC_26_6_13_8, its value shall indicate number of frequencies between 1 and 16. default : '004080'O |
| TSPX_Hsn27 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_8, default : 38 |
| TSPX_Chtp13 | CH_TDMA | PIXIT | channel type for TC_26_6_13_9 |
| TSPX_Tm3 | INTEGER | PIXIT | channel mode for TC_26_6_13_9, value between 60 -100 |
| TSPX_Maio28 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_9, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma28, default : 10 |
| TSPX_Ma28 | MAC | PIXIT | mobile allocation for TC_26_6_13_9, its value shall indicate number of frequencies between 1 and 17. default : '00EE7A'O |
| TSPX_Hsn28 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_9, default : 11 |
| TSPX_Maio29 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_9, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma29, default : 6 |
| TSPX_Ma29 | MAC | PIXIT | mobile allocation for TC_26_6_13_9, its value shall indicate number of frequencies between 1 and 17. default : '00F070'O |
| TSPX_Chtp14 | CH_TDMA | PIXIT | channel type for TC_26_6_13_10 |
| TSPX_Maio30 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_10, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma30, default : 11 |
| TSPX_Ma30 | MAC | PIXIT | mobile allocation for TC_26_6_13_10, its value shall indicate number of frequencies between 1 and 17. default : '017F69'O |
| TSPX_Hsn30 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_10, default : 62 |
| TSPX_Maio31 | INTEGER | PIXIT | hopping parameter for TC_26_6_13_10, its value between 0 and (the number of frequencies) -1, which is defined in TSPX_Ma31, default : 1 |
| TSPX_Ma31 | MAC | PIXIT | mobile allocation for TC_26_6_13_10, its value shall indicate number of frequencies between 1 and 17. default : '000101'O |
| TSPX_MOBscSvcA | SERVICES | PIXIT | any supported MO basic service |
| TSPX_MOBscSvcB | SERVICES | PIXIT | any supported MO basic service |
| TSPX_MOBscSvcC | SERVICES | PIXIT | any supported MO basic service |
| TSPX_MOBscSvcD | SERVICES | PIXIT | any supported MO basic service |
| TSPX_MOBscSvcE | SERVICES | PIXIT | any supported MO basic service |
| TSPX_MOBscSvcF | SERVICES | PIXIT | any supported MO basic service |
| TSPX_MOBscSvcG | SERVICES | PIXIT | any supported MO basic service |
| TSPX_MOBscSvcH | SERVICES | PIXIT | any supported MO basic service |
| TSPX_MOBscSvcI | SERVICES | PIXIT | any supported MO basic service |
| TSPX_MOBscSvcJ | SERVICES | PIXIT | any supported MO basic service |
| TSPX_MT_DualModSvc | SERVICES | PIXIT | any supported MT dual mode service |
| TSPX_MT_NonSptSvc | SERVICES | PIXIT | any not supported MT basic service |
| TSPX_MTBscSvcA | SERVICES | PIXIT | any supported MT basic service |
| TSPX_MTBscSvcB | SERVICES | PIXIT | any supported MT basic service |
| TSPX_MTBscSvcC | SERVICES | PIXIT | any supported MT basic service |
| TSPX_MTBscSvcD | SERVICES | PIXIT | any supported MT basic service |
| TSPX_MTBscSvcE | SERVICES | PIXIT | any supported MT basic service |
| TSPX_MTBscSvcF | SERVICES | PIXIT | any supported MT basic service |
| TSPX_MTBscSvcG | SERVICES | PIXIT | any supported MT basic service |
| TSPX_MTBscSvcH | SERVICES | PIXIT | any supported MT basic service |
| TSPX_MTBscSvcI | SERVICES | PIXIT | any supported MT basic service |
| TSPX_MTBscSvcJ | SERVICES | PIXIT | any supported MT basic service |
| TSPX_MTNIC_BscSvcA | SERVICES | PIXIT | any supported MT basic service without immediate connection |
| TSPX_MTNIC_BscSvcB | SERVICES | PIXIT | any supported MT basic service without immediate connection |
| TSPX_MTNIC_BscSvcC | SERVICES | PIXIT | any supported MT basic service without immediate connection |
| TSPX_MTNIC_BscSvcD | SERVICES | PIXIT | any supported MT basic service without immediate connection |
| TSPX_MTNIC_BscSvcE | SERVICES | PIXIT | any supported MT basic service without immediate connection |
| TSPX_MTNIC_BscSvcF | SERVICES | PIXIT | any supported MT basic service without immediate connection |
| TSPX_MTNIC_BscSvcG | SERVICES | PIXIT | any supported MT basic service without immediate connection |
| TSPX_MTNIC_BscSvcH | SERVICES | PIXIT | any supported MT basic service without immediate connection |
| TSPX_MTNIC_BscSvcI | SERVICES | PIXIT | any supported MT basic service without immediate connection |
| TSPX_MTNIC_BscSvcJ | SERVICES | PIXIT | any supported MT basic service without immediate connection |
| TSPX_Telephony_Rate | RATE | PIXIT | channel rate for TS11, default value : C_Full |
| TSPX_MOChRateA | RATE | PIXIT | channel rate for TSPX_MOBscSvcA, default value : C_Full |
| TSPX_MOChRateB | RATE | PIXIT | channel rate for TSPX_MOBscSvcB, default value : C_Full |
| TSPX_MOChRateC | RATE | PIXIT | channel rate for TSPX_MOBscSvcC, default value : C_Full |
| TSPX_MOChRateD | RATE | PIXIT | channel rate for TSPX_MOBscSvcD, default value : C_Full |
| TSPX_MOChRateE | RATE | PIXIT | channel rate for TSPX_MOBscSvcE, default value : C_Full |
| TSPX_MOChRateF | RATE | PIXIT | channel rate for TSPX_MOBscSvcF, default value : C_Full |
| TSPX_MOChRateG | RATE | PIXIT | channel rate for TSPX_MOBscSvcG, default value : C_Full |
| TSPX_MOChRateH | RATE | PIXIT | channel rate for TSPX_MOBscSvcH, default value : C_Full |
| TSPX_MOChRateI | RATE | PIXIT | channel rate for TSPX_MOBscSvcI, default value : C_Full |
| TSPX_MOChRateJ | RATE | PIXIT | channel rate for TSPX_MOBscSvcJ, default value : C_Full |
| TSPX_MTChRateA | RATE | PIXIT | channel rate for TSPX_MTBscSvcA, default value : C_Full |
| TSPX_MTChRateB | RATE | PIXIT | channel rate for TSPX_MTBscSvcB, default value : C_Full |
| TSPX_MTChRateC | RATE | PIXIT | channel rate for TSPX_MTBscSvcC, default value : C_Full |
| TSPX_MTChRateD | RATE | PIXIT | channel rate for TSPX_MTBscSvcD, default value : C_Full |
| TSPX_MTChRateE | RATE | PIXIT | channel rate for TSPX_MTBscSvcE, default value : C_Full |
| TSPX_MTChRateF | RATE | PIXIT | channel rate for TSPX_MTBscSvcF, default value : C_Full |
| TSPX_MTChRateG | RATE | PIXIT | channel rate for TSPX_MTBscSvcG, default value : C_Full |
| TSPX_MTChRateH | RATE | PIXIT | channel rate for TSPX_MTBscSvcH, default value : C_Full |
| TSPX_MTChRateI | RATE | PIXIT | channel rate for TSPX_MTBscSvcI, default value : C_Full |
| TSPX_MTChRateJ | RATE | PIXIT | channel rate for TSPX_MTBscSvcJ, default value : C_Full |
| TSPX_MTNIC_ChRateA | RATE | PIXIT | channel rate for TSPX_MTNIC_BscSvcA, default value : C_Full |
| TSPX_MTNIC_ChRateB | RATE | PIXIT | channel rate for TSPX_MTNIC_BscSvcB, default value : C_Full |
| TSPX_MTNIC_ChRateC | RATE | PIXIT | channel rate for TSPX_MTNIC_BscSvcC, default value : C_Full |
| TSPX_MTNIC_ChRateD | RATE | PIXIT | channel rate for TSPX_MTNIC_BscSvcD, default value : C_Full |
| TSPX_MTNIC_ChRateE | RATE | PIXIT | channel rate for TSPX_MTNIC_BscSvcE, default value : C_Full |
| TSPX_MTNIC_ChRateF | RATE | PIXIT | channel rate for TSPX_MTNIC_BscSvcF, default value : C_Full |
| TSPX_MTNIC_ChRateG | RATE | PIXIT | channel rate for TSPX_MTNIC_BscSvcG, default value : C_Full |
| TSPX_MTNIC_ChRateH | RATE | PIXIT | channel rate for TSPX_MTNIC_BscSvcH, default value : C_Full |
| TSPX_MTNIC_ChRateI | RATE | PIXIT | channel rate for TSPX_MTNIC_BscSvcI, default value : C_Full |
| TSPX_MTNIC_ChRateJ | RATE | PIXIT | channel rate for TSPX_MTNIC_BscSvcJ, default value : C_Full |
| TSPX_EmgCallRate | RATE | PIXIT | Rate for the basic service supported for MO emergency calls |
| TSPX_MO_NonCallSS | SERVICES | PIXIT | any supported MO non-call releated supplementary service |
| TSPX_MO_BscSvc_SMS | SERVICES | PIXIT | any supported MO SMS calls |
| TSPX_MO_DualModSvc | SERVICES | PIXIT | any supported MO dual mode call |
| TSPX_MO_DualModRate | RATE | PIXIT | Rate for the supported MO dual mode call in TSPX_MO_DualModSvc |
| Detailed Comments: |  |  |  |

## Test case selection expression definitions

| Test Case Selection Expression Definitions |  |  |  |
| --- | --- | --- | --- |
| Expression Name |  | Selection Expression | Comments |
| SelExpr_0000 |  | TRUE | General test group always selected |
| SelExpr_0001 |  | TRUE | always selected |
| SelExpr_0002 |  | TSPC_MTsvc | MT circuit switched basic service supported |
| SelExpr_0003 |  | NOT TSPC_Serv_SS_AoCC | AOCC not supported |
| SelExpr_0004 |  | TSPC_Serv_SS_AoCC AND(NOT TSPC_Serv_SS_HOLD) | AOCC supported but Call Hold not supported |
| SelExpr_0005 |  | TSPC_Serv_SS_AoCC AND TSPC_Serv_SS_HOLD AND(NOT TSPC_Serv_SS_MPTY) | AOCC and Call Hold supported but multiparty not supported |
| SelExpr_0006 |  | NOT TSPC_Feat_FND | FND feature not supported |
| SelExpr_0007 |  | TSPC_MOsvc | MO circuit switched basic service supported |
| SelExpr_0100 |  | TRUE | initial test group always selected |
| SelExpr_0101 |  | TRUE | always selected |
| SelExpr_0102 |  | TSPC_SvcOnTCH | at least one service on traffic channel supported |
| SelExpr_0103 |  | TSPC_DualRate | half rate channel supported |
| SelExpr_0104 |  | TSPC_Serv_BS21 OR TSPC_Serv_BS22 OR TSPC_Serv_BS23 OR TSPC_Serv_BS24 OR TSPC_Serv_BS25 OR TSPC_Serv_BS26 OR TSPC_Serv_BS31 OR TSPC_Serv_BS32 OR TSPC_Serv_BS33 OR TSPC_Serv_BS34 OR TSPC_Serv_BS41 OR TSPC_Serv_BS42 OR TSPC_Serv_BS43 OR TSPC_Serv_BS44 OR TSPC_Serv_BS45 OR TSPC_Serv_BS46 OR TSPC_Serv_BS51 OR TSPC_Serv_BS52 OR TSPC_Serv_BS53 OR TSPC_Serv_BS61 OR TSPC_Serv_BS81 OR TSPC_Serv_TS61 OR TSPC_Serv_TS62 | at least one data service supported |
| SelExpr_0105 |  | TSPC_NonCallSS | non call related supplementary service supported |
| SelExpr_0106 |  | TSPC_Serv_TS22 | MO short message service supported |
| SelExpr_0107 |  | TSPC_Serv_TS11 OR TSPC_Serv_TS12 | speech supported |
| SelExpr_0200 |  | TRUE | idleMode test group always selected |
| SelExpr_0201 |  | TRUE | always selected |
| SelExpr_0300 |  | TRUE | BiBo test group always selected |
| SelExpr_0301 |  | TSPC_Serv_TS11 OR TSPC_Serv_TS12 OR SelExpr_0104 | CC protocol for at least one BC supported |
| SelExpr_0302 |  | TRUE | always selected |
| SelExpr_0400 |  | TRUE | RR test group always selected |
| SelExpr_0401 |  | TRUE | always selected |
| SelExpr_0402 |  | SelExpr_0301 | CC protocol for at least one BC supported |
| SelExpr_0403 |  | SelExpr_0301 | Full rate traffic channel supported |
| SelExpr_0404 |  | TSPC_DualRate | Dual rate traffic channel supported |
| SelExpr_0405 |  | SelExpr_0301 AND(TSPC_Feat_A51 OR TSPC_Feat_A52) | call control protocol and (A5/1 or A5/2) algorithm supported |
| SelExpr_0406 |  | SelExpr_0301 AND TSPC_RFAmp | call control protocol and RF amplification supported |
| SelExpr_0407 |  | SelExpr_0301 | call control protocol and full rate traffic channel supported |
| SelExpr_0408 |  | SelExpr_0301 AND TSPC_DualRate | call control protocol and dual rate traffic channel supported |
| SelExpr_0409 |  | SelExpr_0301 AND TSPC_AddInfo_PseudoSynch | CC protocol for at least one BC supported and Pseudo synchronised supported |
| SelExpr_0410 |  | TSPC_MOsvc | at least one MO circuit switched basic service supported |
| SelExpr_0500 |  | TRUE | MM test group always selected |
| SelExpr_0501 |  | TRUE | always selected |
| SelExpr_0502 |  | TSPC_SIMRmv | SIM removable without power down supported |
| SelExpr_0503 |  | TSPC_Serv_TS11 OR TSPC_Serv_TS12 | speech supported |
| SelExpr_0504 |  | TSPC_NonCallSS | The MS supports a non call related supplementary service operation during an active call |
| SelExpr_0600 |  | TSPC_MOsvc OR TSPC_MTsvc | CC test group |
| SelExpr_0601 |  | TSPC_MOsvc AND(NOT TSPC_EmgOnly) | at least one mobile originating circuit switched basic service supported and not only emergency call supported |
| SelExpr_0602 |  | (TSPC_NoimmConn) AND TSPC_MTsvc | immediate connect not supported and at least one mobile terminated circuit switched basic service supported |
| SelExpr_0603 |  | TSPC_RefusalCall AND TSPC_MTsvc | refusal of call supported and at least one mobile terminated circuit switched basic service supported |
| SelExpr_0604 |  | TSPC_Serv_TS11 OR TSPC_Serv_TS12 OR TSPC_Serv_TS61 OR TSPC_Serv_BS61 OR TSPC_Serv_BS81 | speech supported |
| SelExpr_0605 |  | SelExpr_0604 AND TSPC_AlertInd | speech & alerting supported |
| SelExpr_0606 |  | TSPC_MTsvc | at least one mobile terminated circuit switched basic service supported |
| SelExpr_0607 |  | TSPC_Serv_TS61 OR TSPC_Serv_BS61 OR TSPC_Serv_BS81 | dual mode services supported |
| SelExpr_0700 |  | SelExpr_0301 | StrcutureProc test group |
| SelExpr_0701 |  | TSPC_Serv_TS11 OR TSPC_Serv_TS62 | at least one teleservice (except emergency call and dual service) supported |
| SelExpr_0702 |  | TSPC_Serv_TS12 | emergency call supported |
| SelExpr_0703 |  | TSPC_Serv_TS12 AND TSPC_DualRate | dual rate emergency call supported |
| SelExpr_0800 |  | TSPC_SS | SS test group |
| SelExpr_0801 |  | TSPC_Serv_SS_BOIC OR TSPC_Serv_SS_BAIC OR TSPC_Serv_SS_BOICexHC OR TSPC_Serv_SS_BICRoam OR TSPC_Serv_SS_BAOC | at least one of call restrictions supported |
| SelExpr_0802 |  | TSPC_Serv_SS_AoCC | AOCC supported |
| SelExpr_0803 |  | TSPC_Serv_SS_BOIC | BOIC supported |
| SelExpr_0804 |  | TSPC_Serv_SS_BAIC | BAIC supported |
| SelExpr_0805 |  | TSPC_Serv_SS_BOICexHC | BOICExHC supported |
| SelExpr_0806 |  | TSPC_Serv_SS_BOICexHC OR TSPC_Serv_SS_BAIC | BOICExHC OR BAIC supported |
| SelExpr_0807 |  | TSPC_Serv_SS_BOIC OR TSPC_Serv_SS_BICRoam | BOIC OR BICRoam supported |
| SelExpr_0808 |  | TSPC_Serv_SS_BI | BI supported |
| SelExpr_0809 |  | TSPC_Serv_SS_CFNRy OR TSPC_Serv_SS_CFU | CFNNRy or CFU supported |
| SelExpr_0810 |  | TSPC_Serv_SS_CFB OR TSPC_Serv_SS_CFU OR TSPC_Serv_SS_CFNRc OR TSPC_Serv_SS_CFNRy | CFB or CFU or CFNRy or CFNRc supported |
| SelExpr_0811 |  | TSPC_Serv_SS_CFB OR TSPC_Serv_SS_CFNRc OR TSPC_Serv_SS_CFNRy | CFNRy or CFNRc or CFB supported |
| SelExpr_0812 |  | TSPC_Serv_SS_CFNRc OR TSPC_Serv_SS_CFB | CFNNRc or CFB supported |
| SelExpr_0813 |  | TSPC_Serv_SS_CFB | CFB supported |
| SelExpr_0814 |  | TSPC_Serv_SS_BICRoam OR TSPC_Serv_SS_BAOC | BAOC or BICRoam supported |
| SelExpr_0815 |  | TRUE | always selected |
| SelExpr_0816 |  | TSPC_Serv_SS_unstruct | USSD supported |
| SelExpr_0817 |  | TSPC_Serv_SS_unstruct AND SelExpr_0301 | USSD and CC protocol for at least one BC supported |
| SelExpr_0818 |  | TSPC_Serv_SS_AoCC AND TSPC_SIMRmv | AOCC and SIM removable without power down supported |
| SelExpr_0819 |  | TSPC_Serv_SS_AoCC AND TSPC_Serv_SS_HOLD | AOCC and Call Hold supported |
| SelExpr_0820 |  | TSPC_Serv_SS_AoCC AND TSPC_Serv_SS_MPTY | AOCC and Multi Party service supported |
| SelExpr_0821 |  | TSPC_Serv_SS_AoCC AND TSPC_SwitchOnOff | AOCC and switch on/off supported |
| SelExpr_0900 |  | TSPC_SMS | SMS test group |
| SelExpr_0901 |  | TSPC_Serv_TS21 AND SelExpr_0301 | MT/PP supported and CC protocol for at least one BC supported |
| SelExpr_0902 |  | TSPC_Serv_TS22 AND TSPC_Serv_TS21 AND SelExpr_0301 | MO/PP supported and MT/PP supported and CC protocol for at least one BC supported |
| SelExpr_0903 |  | TSPC_Serv_TS22 AND TSPC_StoreRcvSMSME AND TSPC_StoreRcvSMSSIM | MO/PP supported AND storage of SMS in the ME supported AND storage of SMS in the SIM supported |
| SelExpr_0904 |  | TSPC_Serv_TS22 AND TSPC_Serv_TS21 AND TSPC_SMSStatusRepCap | MO/PP supported AND MT/PP supported AND SMS Status report capabilities supported |
| SelExpr_0905 |  | TSPC_Serv_TS21 AND TSPC_DispRcvSMS | MT/PP supported AND display of received short message supported |
| SelExpr_0906 |  | TSPC_Serv_TS21 AND TSPC_DispRcvSMS AND(TSPC_StoreRcvSMSME OR TSPC_StoreRcvSMSSIM) | MT/PP supported AND display of received short message supported AND (storage of SMS in the ME supported OR storage of SMS in the SIM supported ) |
| SelExpr_0907 |  | TSPC_Serv_TS21 AND TSPC_StoreRcvSMSSIM | MT/PP supported AND display of received short message supported AND storage of SMS in the SIM supported |
| SelExpr_0908 |  | TSPC_Serv_TS21 AND TSPC_ReplaceSMS AND TSPC_DispRcvSMS | MT/PP supported AND "replace short message" and "display of received short message" supported |
| SelExpr_0909 |  | TSPC_Serv_TS22 AND TSPC_Serv_TS21 AND TSPC_ReplyProc AND TSPC_DispRcvSMS | MO/PP supported AND MT/PP supported AND "reply procedure" and "display of received short message" supported |
| SelExpr_1000 |  | TSPC_EGSM | E- band supported |
| SelExpr_1001 |  | SelExpr_0301 | CC protocol for at least one BC supported |
| SelExpr_1002 |  | TSPC_Serv_TS11 OR TSPC_Serv_TS61 OR TSPC_Serv_TS62 | telephony supported or alternate speech/data supported or Teleservice automatic G3 fax supported |
| SelExpr_1003 |  | TSPC_Serv_TS12 | emergency call supported |
| Detailed Comments: |  |  |  |

## Test suite constant declarations

| Test Suite Constant Declarations |  |  |  |
| --- | --- | --- | --- |
| Constant Name | Type | Value | Comments |
| C_arfcnA | INTEGER | 20 | ARFCN 20 |
| C_arfcn_1 | INTEGER | 1 | ARFCN 1 |
| C_arfcn_2 | INTEGER | 2 | ARFCN 2 |
| C_arfcn_1015 | INTEGER | 1015 | ARFCN 1015 |
| C_arfcn_10 | INTEGER | 10 | ARFCN 10 |
| C_arfcn_34 | INTEGER | 34 | ARFCN 34 |
| C_arfcnB | INTEGER | 10 | ARFCN of cell B |
| C_arfcnC | INTEGER | 30 | ARFCN 30 |
| C_arfcnH | INTEGER | 44 | ARFCN 44 |
| C_arfcnAd | INTEGER | 590 | ARFCN 590 |
| C_arfcnAd_1 | INTEGER | 520 | ARFCN 520 |
| C_arfcnAd_2 | INTEGER | 514 | ARFCN 514 |
| C_arfcnAd_3 | INTEGER | 715 | ARFCN 715 |
| C_arfcnAd_4 | INTEGER | 747 | ARFCN 747 |
| C_arfcnAd_5 | INTEGER | 734 | ARFCN 734 |
| C_arfcnAd_6 | INTEGER | 759 | ARFCN 759 |
| C_arfcnBd | INTEGER | 520 | ARFCN 520 |
| C_arfcnCd | INTEGER | 700 | ARFCN 700 |
| C_arfcnHd | INTEGER | 810 | ARFCN 810 |
| C_arfcn_tchA | INTEGER | 30 | ARFCN f30 |
| C_arfcn_tchAd | INTEGER | 650 | ARFCN 650 |
| C_arfcn_tchB | INTEGER | 114 | ARFCN 114 |
| C_arfcn_tchBd | INTEGER | 844 | ARFCN 844 |
| C_arfcnEgsm | INTEGER | 990 | ARFCN 990 |
| C_arfcnEgsm_iacmd | INTEGER | 20 | ARFCN 20 |
| C_arfcnEgsm_asscmd | INTEGER | 20 | ARFCN 20 |
| C_BCCHcarrierB_ho | INTEGER | 40 | BCCH frequency number of cell B for Ho cases (GSM: 40) |
| C_BCCHcarrierB_hod | INTEGER | 764 | BCCH frequency number of cell B for Ho cases (DCS:764) |
| C_Noarfcn | INTEGER | 65534 | No arfcn defined ( hopping channel) |
| C_ATT_0 | INTEGER | 0 | Attach value 0 |
| C_ATT_1 | INTEGER | 1 | Attach value 1 |
| C_NCC | NCC | '001'B | Network color code |
| C_NCC_0 | NCC | '000'B | Network color code value '000'B |
| C_NCC_1 | NCC | '001'B | Network color code value '001'B |
| C_NCC_2 | NCC | '010'B | Network color code value '010'B |
| C_NCC_3 | NCC | '011'B | Network color code value '011'B |
| C_NCC_4 | NCC | '100'B | Network color code value '100'B |
| C_NCC_5 | NCC | '101'B | Network color code value '101'B |
| C_NCC_6 | NCC | '110'B | Network color code value '110'B |
| C_NCC_7 | NCC | '111'B | Network color code value '111'B |
| C_NCCP_2 | NCCP | '02'O | Network color code permitted value '02'O |
| C_NCCP_10 | NCCP | '0A'O | Network color code permitted value '0A'O |
| C_BCC | BCC | '101'B | Base Station color code |
| C_BCC_0 | BCC | '000'B | Base Station color code value 0 |
| C_BCC_1 | BCC | '001'B | Base Station color code value 1 |
| C_BCC_3 | BCC | '011'B | Base Station color code value 3 |
| C_BCC_5 | BCC | '101'B | Base Station color code value 5 |
| C_BCC_7 | BCC | '111'B | Base Station color code value 7 |
| C_cch_1nonComb | B_3 | '000'B | cch_con = 1 basic ch non combined |
| C_cch_1Comb | B_3 | '001'B | cch_con = 1 basic ch combined |
| C_cch_2nonComb | B_3 | '010'B | cch_con = 2 basic chs non combined |
| C_cch_3nonComb | B_3 | '100'B | cch_con = 3 basic chs non combined |
| C_cch_4nonComb | B_3 | '110'B | cch_con = 4 basic chs non combined |
| C_cch_XnonComb | B_3 | '??0'B | cch_con = non combined all number value permitted |
| C_cch_NotAllowed | B_3 | '??1'B | cch_con = value reserved after testing '001'B |
| C_cchd_e_407 | OCTETSTRING | '8DEA0DF4CC6C4AFAFB10000000000000'O | f-list for cellchdescr using of 128 format. The coded set is {980, 981, 982, 983, 990, 991, 992, 993, 994, 1000, 1005, 1010, 1015}. Length = 16 TC_26_10_4, k=2, c=1 TC_26.10.6, c=1 |
| C_cchd_e_408 | OCTETSTRING | '8A0A1CFD3EF4610E2FFFFA0000000000'O | f-list for cellchdescr using of 256 format. The coded set is {20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108, 114,115} Length = 16 TC_26_10_4, k=2, c=2 TC_26.10.6, c=2 |
| C_cchd_e_409 | OCTETSTRING | '89EA037F433C7B042BFEBFEC10000000'O | f-list for cellchdescr using of 512 format. The coded set is {980, 981, 982, 983, 990, 991, 992, 993, 994, 1000, 1005, 1010, 1015}. Length = 16 TC_26_10_4, k=2, c=3 TC: 26.10.6 c=3 |
| C_cchd_e_410 | OCTETSTRING | '841EEA893EF98143B161000000000000'O | f-list for cellchdescr using of 1024 format. Length = 16 |
| C_cchd_e_411 | OCTETSTRING | '8FEA703E084210000000000000000000'O | f-list for cellchdescr using of variable bit format. The coded set is {980, 981, 982, 983, 990, 991, 992, 993, 994, 1000, 1005, 1010, 1015}. Length = 16 TC_26_10_4, k=2, c=5 TC: 26.10.6 c=5 |
| C_cchd_e_412 | OCTETSTRING | '0002080000007F020000008000080000'O | f-list for cellchdescr using of bit map 0 format. The coded set is {20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108, 114,115} Length = 16 TC_26_10_4, k=2, c=6 |
| C_cchd_e_414 | OCTETSTRING | '0000000000000000200002000020000000'O | f-list for cellchdescr using of bit map 0 format. Length = 16 f_list: 30, 50 ,70 TC: 26.10.6 c=6 |
| C_cchd_e_415 | OCTETSTRING | '841EEA893EF981438000000000000000'O | f-list for cellchdescr using range 1024 format. The coded set is {0, 30, 40, 66, 80, 1005, 1010, 1015}. Length = 16 TC_26_10_4, k=2, c=4 TC: 26.10.6 c=4 |
| C_ChMod_s | CHMOD_VAL | '00000000'B | Mode : signalling only. |
| C_ChMod_r | CHMOD_VAL | '00000001'B | Mode : speech full or half rate. |
| C_ChMod_12k | CHMOD_VAL | '00000011'B | Mode : 12k radio rate. |
| C_ChMod_6k | CHMOD_VAL | '00001011'B | Mode : 6k radio rate. |
| C_ChMod_3k | CHMOD_VAL | '00010011'B | Mode : 3.6k radio rate. |
| C_Sap0 | SAPID | '00'O | service access point 0 |
| C_Sap3 | SAPID | '03'O | service access point 3 |
| C_CellA | CellID | "C_CellA" | cell A -- cell 1 |
| C_CellB | CellID | "C_CellB" | cell B -- cell 2 |
| C_CellC | CellID | "C_CellC" | cell C -- cell 3 |
| C_CellD | CellID | "C_CellD" | cell D -- cell 4 |
| C_CellE | CellID | "C_CellE" | cell E -- cell 5 |
| C_CellF | CellID | "C_CellF" | cell F -- cell 6 |
| C_CellG | CellID | "C_CellG" | cell G -- cell 7 |
| C_CellH | CellID | "C_CellH" | cell H -- cell 8 |
| C_ci_cellA | CI | '0001'O | Cell Id for cell A |
| C_ci_cellB | CI | '0002'O | Cell Id for cell B |
| C_ci_cellC | CI | '0003'O | Cell Id for cell C |
| C_ci_cellD | CI | '0004'O | Cell Id for cell D |
| C_ci_cellE | CI | '0005'O | Cell Id for cell E |
| C_ci_cellF | CI | '0006'O | Cell Id for cell F |
| C_ci_cellG | CI | '0007'O | Cell Id for cell G |
| C_ci_cellH | CI | '0008'O | Cell Id for cell H |
| C_cksn_nokey | BITSTRING | '111'B | No key available |
| C_E_default | INTEGER | 63 | Default field strength |
| C_E_suitable | INTEGER | 53 | Field strength of a suitable cell |
| C_E_notsuitable | INTEGER | 45 | Field strength of a not suitable cell for reselection |
| C_E_notsuitable_fSel | INTEGER | 24 | Field strength of a not suitable cell for selection |
| C_flist_e_401 | OCTETSTRING | '8DF68AEC00'O | It includes the list of f's in EGSM test cases. The coded set is {1005, 1010, 1015}. range 128 format Length = 5 TC_26_10_4, k=1, c=1 TC: 26.10.5.1 k=1/2,c=1 |
| C_flist_e_402 | OCTETSTRING | '8A2481FF03F8'O | It includes the list of f's in EGSM test cases. The coded set is {73, 74, 75, 76, 77}, range 256 format. Length = 6 TC_26_10_4, k=1, c=2 TC: 26.10.5.1 k=1/2,c=2 |
| C_flist_e_403 | OCTETSTRING | '89EA00BFC040'O | It includes the list of f's in EGSM test cases. The coded set is {980, 981, 982, 983}, range 512 format. Length = 6 TC_26_10_4, k=1, c=3 TC: 26.10.5.1 k=1/2,c=3 |
| C_flist_e_404 | OCTETSTRING | '801EED02BEC0'O | It includes the list of f's in EGSM test cases. The coded set is {30, 40, 1010, 1015}, range 1024 format. Length = 6 TC_26_10_4, k=1, c=4 TC: 26.10.5.1 k=1/2,c=4 |
| C_flist_e_405 | OCTETSTRING | '8FEC001F000010'O | It includes the list of f's in EGSM test cases. The coded set is {980, 991, 992, 993, 994, 1015}. Length = 7 TC_26_10_4, k=1, c=5 TC: 26.10.5.1 k=1/2,c=5 |
| C_flist_e_406 | OCTETSTRING | '00000000000000020000008000080000'O | It includes the list of f's in EGSM test cases. The coded set is {20, 40, 66}, bitmap 0 format. Length = 16 TC_26_10_4, k=1, c=6 TC: 26.10.5.1 k=2,c=6 |
| C_flist_e_407 | OCTETSTRING | '8DEA0DF4CC6C4AFAFB10000000000000'O | It includes the list of f's in EGSM test cases. The coded set is {980, 981, 982, 983, 990, 991, 992, 993, 994, 1000, 1005, 1010, 1015}, range 128 format. Length = 16 TC_26_10_4, k=2, c=1 TC: 26.10.5.1 k=3,c=1 TC: 26.10.6 c=1 |
| C_flist_e_408 | OCTETSTRING | '8A0A1CFD3EF4610E2FFFFA0000'O | It includes the list of f's in EGSM test cases. The coded set is {20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108, 114, 115}, range 256 format. Length = 13 TC_26_10_4, k=2, c=2 TC: 26.10.5.1 k=3,c=2 TC: 26.10.6 c=2 |
| C_flist_e_409 | OCTETSTRING | '89EA037F433C7B042BFEBFEC10000000'O | It includes the list of f's in EGSM test cases. The coded set is {980, 981, 982, 983, 990, 991, 992, 993, 994, 1000, 1005, 1010, 1015}, range 512 format. Length = 16 TC_26_10_4, k=2, c=3 TC: 26.10.5.2 k=3,c=3 TC: 26.10.6 c=3 |
| C_flist_e_410 | OCTETSTRING | '841EEA893EF98143B161'O | Length = 10 TC: 26.10.5.2 k=3,c=4 |
| C_flist_e_411 | OCTETSTRING | '8FEA703E084210'O | It includes the list of f's in EGSM test cases. The coded set is {980, 981, 982, 983, 990, 991, 992, 993, 994, 1000, 1005, 1010, 1015}, variable bitmap format. Length = 7 TC_26_10_4, k=2, c=5 TC: 26.10.5.1 k=3,c=5 TC: 26.10.6 c=5 |
| C_flist_e_412 | OCTETSTRING | '0002080000007F020000008000080000'O | It includes the list of f's in EGSM test cases. The coded set is {20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108, 114, 115}, bitmap 0 format. Length = 16 TC_26_10_4, k=2, c=6 TC: 26.10.5.1 k=3,c=6 |
| C_flist_e_415 | OCTETSTRING | '841EEA893EF9814380'O | It includes the list of f's in EGSM test cases. The coded set is {0, 30, 40, 66, 80, 1005, 1010, 1015}, range 1024 format. Length = 9 Format: 1024 TC_26_10_4, k=2, c=4 TC: 26.10.6 c=4 |
| C_flist_e_416 | OCTETSTRING | '0002080000007F020000008000080000'O | bit map 0 format. The coded set is {20, 40, 66, 73, 74, 75, 76, 77, 78, 79, 108, 114} Length = 16 |
| C_BABR_0 | B_3 | '000'B | BABR value 0 |
| C_BABR_1 | B_3 | '001'B | BABR value 1 |
| C_BABR_2 | B_3 | '010'B | BABR value 2 |
| C_BPM_0 | B_3 | '000'B | BPM value 0, 2 multiframes |
| C_BPM_2 | B_3 | '010'B | BPM value 2, 4 multiframes |
| C_BPM_3 | B_3 | '011'B | BPM value 3, 5 multiframes |
| C_BPM_7 | B_3 | '111'B | BPM value 7, 9 multiframes |
| C_T3212_0 | OCTETSTRING | '00'O | T3212 infinite |
| C_T3212_1 | OCTETSTRING | '01'O | T3212 6 mn / 1 deci hours |
| C_T3212_2 | OCTETSTRING | '02'O | T3212 12 mn / 2 deci hours |
| C_T3212_5 | OCTETSTRING | '05'O | T3212 30 mn / 5 deci hours |
| C_IMSI | INTEGER | 0 | MI type: IMSI |
| C_TMSI | INTEGER | 1 | MI type: TMSI |
| C_IMEI | INTEGER | 2 | MI type: IMEI |
| C_IMEISV | INTEGER | 3 | MI type: IMEISV |
| C_shortIMSI | HEXSTRING | '001011234'H | A short IMSI |
| C_LAC_1 | OCTETSTRING | '0001'O | lac 1. |
| C_LAC_2 | OCTETSTRING | '0002'O | lac 2. |
| C_LAC_3 | OCTETSTRING | '0003'O | lac 3. |
| C_LAC_4 | OCTETSTRING | '0004'O | lac 4. |
| C_LAC_5 | OCTETSTRING | '0005'O | lac 5. |
| C_LAC_6 | OCTETSTRING | '0006'O | lac 6. |
| C_LAC_7 | OCTETSTRING | '0007'O | lac 7. |
| C_LAC_8 | OCTETSTRING | '0008'O | lac 8. |
| C_LAC_spe | OCTETSTRING | '5344'O | lac '5344'H. |
| C_lacdeleted | OCTETSTRING | 'FFFE'O | lac value deleted. |
| C_MCC_1 | OCTETSTRING | '00F1'O | MCC 1 |
| C_MCC_2 | OCTETSTRING | '00F2'O | MCC 2 |
| C_MCC_3 | OCTETSTRING | '00F3'O | MCC 3 |
| C_MCC_4 | OCTETSTRING | '00F4'O | MCC 4 |
| C_MCC_5 | OCTETSTRING | '00F5'O | MCC 5 |
| C_MCC_6 | OCTETSTRING | '00F6'O | MCC 6 |
| C_MCC_7 | OCTETSTRING | '00F7'O | MCC 7 |
| C_MCC_8 | OCTETSTRING | '00F8'O | MCC 8 |
| C_MCC_9 | OCTETSTRING | '013B'O | MCC 9 |
| C_Max_1 | INTEGER | 1 | Max Retrans value 1 |
| C_Max_2 | INTEGER | 2 | Max Retrans value 2 |
| C_Max_7 | INTEGER | 7 | Max Retrans value 7 |
| C_NECI_0 | INTEGER | 0 | NECI value 0, not supported |
| C_NECI_1 | INTEGER | 1 | NECI value 1, supported |
| C_PLMN_1 | OCTETSTRING | '10'O | PLMN 1 (mnc=01) |
| C_PLMN_2 | OCTETSTRING | '20'O | PLMN 2 (mnc=02) |
| C_PLMN_3 | OCTETSTRING | '30'O | PLMN 3 (mnc=03) |
| C_PLMN_4 | OCTETSTRING | 'F0'O | PLMN 4 |
| C_PLMN_5 | OCTETSTRING | 'F2'O | PLMN 5 |
| C_PLMN_6 | OCTETSTRING | 'F3'O | PLMN 6 |
| C_PLMN_7 | OCTETSTRING | 'F4'O | PLMN 7 |
| C_PLMN_8 | OCTETSTRING | 'F5'O | PLMN 8 |
| C_PLMN_9 | OCTETSTRING | 'F6'O | PLMN 9 |
| C_PLMN_10 | OCTETSTRING | 'F7'O | PLMN 10 |
| C_PLMN_Home | OCTETSTRING | '10'O | PLMN 1 (mnc=01) |
| C_TxInt_5 | INTEGER | 5 | Tx Integer value 5 |
| C_TxInt_7 | INTEGER | 7 | Tx Integer value 7 |
| C_TxInt_10 | INTEGER | 10 | Tx Integer value 10 |
| C_NotCombined | BOOLEAN | FALSE | CCCH not combined with SDCCH |
| C_Combined | BOOLEAN | TRUE | CCCH combined with SDCCH |
| C_FCCH_A | LOGICCH | "C_FCCH_A" | frequency correction channel of cell A(1) |
| C_FCCH_B | LOGICCH | "C_FCCH_B" | frequency correction channel of cell B(2) |
| C_FCCH_C | LOGICCH | "C_FCCH_C" | frequency correction channel of cell C(3) |
| C_FCCH_D | LOGICCH | "C_FCCH_D" | frequency correction channel of cell D(4) |
| C_FCCH_E | LOGICCH | "C_FCCH_E" | frequency correction channel of cell E(5) |
| C_FCCH_F | LOGICCH | "C_FCCH_F" | frequency correction channel of cell F(6) |
| C_FCCH_G | LOGICCH | "C_FCCH_G" | frequency correction channel of cell G(7) |
| C_FCCH_H | LOGICCH | "C_FCCH_H" | frequency correction channel of cell H(8) |
| C_SCH_A | LOGICCH | "C_SCH_A" | sync channel of cell A(1) |
| C_SCH_B | LOGICCH | "C_SCH_B" | sync channel of cell B(2) |
| C_SCH_C | LOGICCH | "C_SCH_C" | sync channel of cell C(3) |
| C_SCH_D | LOGICCH | "C_SCH_D" | sync channel of cell D(4) |
| C_SCH_E | LOGICCH | "C_SCH_E" | sync channel of cell E(5) |
| C_SCH_F | LOGICCH | "C_SCH_F" | sync channel of cell F(6) |
| C_SCH_G | LOGICCH | "C_SCH_G" | sync channel of cell G(7) |
| C_SCH_H | LOGICCH | "C_SCH_H" | sync channel of cell H(8) |
| C_BCCH_A_1 | LOGICCH | "C_BCCH_A_1" | 1st broadcast channel of cell A |
| C_BCCH_A_2 | LOGICCH | "C_BCCH_A_2" | 2nd broadcast channel of cell A |
| C_BCCH_A_3 | LOGICCH | "C_BCCH_A_3" | 3rd broadcast channel of cell A |
| C_BCCH_A_4 | LOGICCH | "C_BCCH_A_4" | 4th broadcast channel of cell A |
| C_BCCH_B_1 | LOGICCH | "C_BCCH_B_1" | 1st broadcast channel of cell B |
| C_BCCH_C_1 | LOGICCH | "C_BCCH_C_1" | 1st broadcast channel of cell C |
| C_BCCH_D_1 | LOGICCH | "C_BCCH_D_1" | 1st broadcast channel of cell D |
| C_BCCH_E_1 | LOGICCH | "C_BCCH_E_1" | 1st broadcast channel of cell E |
| C_BCCH_F_1 | LOGICCH | "C_BCCH_F_1" | 1st broadcast channel of cell F |
| C_BCCH_G_1 | LOGICCH | "C_BCCH_G_1" | 1st broadcast channel of cell G |
| C_BCCH_H_1 | LOGICCH | "C_BCCH_H_1" | 1st broadcast channel of cell H |
| C_CBCH_A | LOGICCH | "C_CBCH_A" | Cell broadcast channel in cell A |
| C_AGCH_A_1 | LOGICCH | "C_AGCH_A_1" | 1st down link CCCH (AGCH) of cell A |
| C_AGCH_A_2 | LOGICCH | "C_AGCH_A_2" | 2nd down link CCCH (AGCH) of cell A |
| C_AGCH_A_3 | LOGICCH | "C_AGCH_A_3" | 3rd down link CCCH (AGCH) of cell A |
| C_AGCH_A_4 | LOGICCH | "C_AGCH_A_4" | 4th down link CCCH (AGCH) of cell A |
| C_AGCH_B_1 | LOGICCH | "C_AGCH_B_1" | 1st down link CCCH (AGCH) of cell B |
| C_AGCH_B_2 | LOGICCH | "C_AGCH_B_2" | 2nd down link CCCH (AGCH) of cell B |
| C_AGCH_B_3 | LOGICCH | "C_AGCH_B_3" | 3rd down link CCCH (AGCH) of cell B |
| C_AGCH_B_4 | LOGICCH | "C_AGCH_B_4" | 4th down link CCCH (AGCH) of cell B |
| C_AGCH_C_1 | LOGICCH | "C_AGCH_C_1" | 1st down link CCCH (AGCH) of cell C |
| C_AGCH_C_2 | LOGICCH | "C_AGCH_C_2" | 2nd down link CCCH (AGCH) of cell C |
| C_AGCH_C_3 | LOGICCH | "C_AGCH_C_3" | 3rd down link CCCH (AGCH) of cell C |
| C_AGCH_C_4 | LOGICCH | "C_AGCH_C_4" | 4th down link CCCH (AGCH) of cell C |
| C_AGCH_D_1 | LOGICCH | "C_AGCH_D_1" | 1st down link CCCH (AGCH) of cell D |
| C_AGCH_E_1 | LOGICCH | "C_AGCH_E_1" | 1st down link CCCH (AGCH) of cell E |
| C_AGCH_F_1 | LOGICCH | "C_AGCH_F_1" | 1st down link CCCH (AGCH) of cell F |
| C_AGCH_G_1 | LOGICCH | "C_AGCH_G_1" | 1st down link CCCH (AGCH) of cell G |
| C_AGCH_H_1 | LOGICCH | "C_AGCH_H_1" | 1st down link CCCH (AGCH) of cell H |
| C_PCH_A_1 | LOGICCH | "C_PCH_A_1" | 1st down link CCCH (PCH) of cell A |
| C_PCH_A_2 | LOGICCH | "C_PCH_A_2" | 2nd down link CCCH (PCH) of cell A |
| C_PCH_A_3 | LOGICCH | "C_PCH_A_3" | 3rd down link CCCH (PCH) of cell A |
| C_PCH_A_4 | LOGICCH | "C_PCH_A_4" | 4th down link CCCH (PCH) of cell A |
| C_PCH_B_1 | LOGICCH | "C_PCH_B_1" | 1st down link CCCH (PCH) of cell B |
| C_PCH_B_2 | LOGICCH | "C_PCH_B_2" | 2nd down link CCCH (PCH) of cell B |
| C_PCH_B_3 | LOGICCH | "C_PCH_B_3" | 3rd down link CCCH (PCH) of cell B |
| C_PCH_B_4 | LOGICCH | "C_PCH_B_4" | 4th down link CCCH (PCH ) of cell B |
| C_PCH_C_1 | LOGICCH | "C_PCH_C_1" | 1st down link CCCH (PCH) of cell C |
| C_PCH_C_2 | LOGICCH | "C_PCH_C_2" | 2nd down link CCCH (PCH) of cell C |
| C_PCH_C_3 | LOGICCH | "C_PCH_C_3" | 3rd down link CCCH (PCH) of cell C |
| C_PCH_C_4 | LOGICCH | "C_PCH_C_4" | 4th down link CCCH (PCH) of cell C |
| C_PCH_D_1 | LOGICCH | "C_PCH_D_1" | 1st down link CCCH (PCH and AGCH) of cell D |
| C_PCH_E_1 | LOGICCH | "C_PCH_E_1" | 1st down link CCCH (PCH) of cell E |
| C_PCH_F_1 | LOGICCH | "C_PCH_F_1" | 1st down link CCCH (PCH) of cell F |
| C_PCH_G_1 | LOGICCH | "C_PCH_G_1" | 1st down link CCCH (PCH) of cell G |
| C_PCH_H_1 | LOGICCH | "C_PCH_H_1" | 1st down link CCCH (PCH) of cell H |
| C_RACH_A_1 | LOGICCH | "C_RACH_A_1" | 1st uplink CCCH (RACH) of cell A |
| C_RACH_A_2 | LOGICCH | "C_RACH_A_2" | 2nd uplink CCCH (RACH) of cell A |
| C_RACH_A_3 | LOGICCH | "C_RACH_A_3" | 3rd uplink CCCH (RACH) of cell A |
| C_RACH_A_4 | LOGICCH | "C_RACH_A_4" | 4th uplink CCCH (RACH) of cell A |
| C_RACH_B_1 | LOGICCH | "C_RACH_B_1" | 1st uplink CCCH (RACH) of cell B |
| C_RACH_C_1 | LOGICCH | "C_RACH_C_1" | 1st uplink CCCH (RACH) of cell C |
| C_RACH_D_1 | LOGICCH | "C_RACH_D_1" | 1st uplink CCCH (RACH) of cell D |
| C_RACH_E_1 | LOGICCH | "C_RACH_E_1" | 1st uplink CCCH (RACH) of cell E |
| C_RACH_F_1 | LOGICCH | "C_RACH_F_1" | 1st uplink CCCH (RACH) of cell F |
| C_RACH_G_1 | LOGICCH | "C_RACH_G_1" | 1st uplink CCCH (RACH) of cell G |
| C_RACH_H_1 | LOGICCH | "C_RACH_H_1" | 1st uplink CCCH (RACH) of cell H |
| C_FACCHF_A_1 | LOGICCH | "C_FACCHF_A_1" | FACCH associated with 1st TCH/F of cell A |
| C_FACCHF_A_2 | LOGICCH | "C_FACCHF_A_2" | FACCH associated with 2nd TCH/F of cell A |
| C_FACCHF_A_3 | LOGICCH | "C_FACCHF_A_3" | FACCH associated with 3rd TCH/F of cell A |
| C_FACCHF_B_1 | LOGICCH | "C_FACCHF_B_1" | FACCH associated with 1st TCH/F of cell B |
| C_FACCHF_B_2 | LOGICCH | "C_FACCHF_B_2" | FACCH associated with 2nd TCH/F of cell B |
| C_FACCHF_B_3 | LOGICCH | "C_FACCHF_B_3" | FACCH associated with 3rd TCH/F of cell B |
| C_FACCHF_C_1 | LOGICCH | "C_FACCHF_C_1" | FACCH associated with 1sd TCH/F of cell C |
| C_FACCHF_C_2 | LOGICCH | "C_FACCHF_C_2" | FACCH associated with 2nd TCH/F of cell C |
| C_FACCHF_C_3 | LOGICCH | "C_FACCHF_C_3" | FACCH associated with 3rd TCH/F of cell C |
| C_FACCHF_H_1 | LOGICCH | "C_FACCHF_H_1" | FACCH associated with 1st TCH/F of cell H |
| C_FACCHH_A_1 | LOGICCH | "C_FACCHH_A_1" | FACCH associated with 1st TCH/H of cell A |
| C_FACCHH_A_2 | LOGICCH | "C_FACCHH_A_2" | FACCH associated with 2nd TCH/H of cell A |
| C_FACCHH_A_3 | LOGICCH | "C_FACCHH_A_3" | FACCH associated with 3rd TCH/H of cell A |
| C_FACCHH_B_1 | LOGICCH | "C_FACCHH_B_1" | FACCH associated with 1st TCH/H of cell B |
| C_FACCHH_B_2 | LOGICCH | "C_FACCHH_B_2" | FACCH associated with 2nd TCH/H of cell B |
| C_FACCHH_B_3 | LOGICCH | "C_FACCHH_B_3" | FACCH associated with 3rd TCH/H of cell B |
| C_FACCHH_C_1 | LOGICCH | "C_FACCHH_C_1" | FACCH associated with 1st TCH/H of cell C |
| C_FACCHH_C_2 | LOGICCH | "C_FACCHH_C_2" | FACCH associated with 2nd TCH/H of cell C |
| C_FACCHH_C_3 | LOGICCH | "C_FACCHH_C_3" | FACCH associated with 3rd TCH/H of cell C |
| C_FACCHH0_A_1 | LOGICCH | "C_FACCHH0_A_1" | FACCH associated with 1st TCH/H0 of cell A |
| C_FACCHH1_A_1 | LOGICCH | "C_FACCHH1_A_1" | FACCH associated with 1st TCH/H1 of cell A |
| C_FACCHH0_A_2 | LOGICCH | "C_FACCHH0_A_2" | FACCH associated with 2nd TCH/H0 of cell A |
| C_FACCHH1_A_2 | LOGICCH | "C_FACCHH1_A_2" | FACCH associated with 2nd TCH/H1 of cell A |
| C_FACCHH0_A_3 | LOGICCH | "C_FACCHH0_A_3" | FACCH associated with 3rd TCH/H0 of cell A |
| C_FACCHH1_A_3 | LOGICCH | "C_FACCHH1_A_3" | FACCH associated with 3rd TCH/H1 of cell A |
| C_FACCHH0_B_1 | LOGICCH | "C_FACCHH0_B_1" | FACCH associated with 1st TCH/H0 of cell B |
| C_FACCHH1_B_1 | LOGICCH | "C_FACCHH1_B_1" | FACCH associated with 1st TCH/H1 of cell B |
| C_FACCHH0_B_2 | LOGICCH | "C_FACCHH0_B_2" | FACCH associated with 2nd TCH/H0 of cell B |
| C_FACCHH1_B_2 | LOGICCH | "C_FACCHH1_B_2" | FACCH associated with 2nd TCH/H1 of cell B |
| C_FACCHH0_B_3 | LOGICCH | "C_FACCHH0_B_3" | FACCH associated with 3rd TCH/H0 of cell B |
| C_FACCHH1_B_3 | LOGICCH | "C_FACCHH1_B_3" | FACCH associated with 3rd TCH/H1 of cell B |
| C_FACCHH0_C_1 | LOGICCH | "C_FACCHH0_C_1" | FACCH associated with 1st TCH/H0 of cell C |
| C_FACCHH1_C_1 | LOGICCH | "C_FACCHH1_C_1" | FACCH associated with 1st TCH/H1 of cell C |
| C_FACCHH0_C_2 | LOGICCH | "C_FACCHH0_C_2" | FACCH associated with 2nd TCH/H0 of cell C |
| C_FACCHH1_C_2 | LOGICCH | "C_FACCHH1_C_2" | FACCH associated with 2nd TCH/H1 of cell C |
| C_FACCHH0_C_3 | LOGICCH | "C_FACCHH0_C_3" | FACCH associated with 3rd TCH/H0 of cell C |
| C_FACCHH1_C_3 | LOGICCH | "C_FACCHH1_C_3" | FACCH associated with 3rd TCH/H1 of cell C |
| C_SACCH_A | LOGICCH | "C_SACCH_A" | all SACCHs of cell A |
| C_SACCH_B | LOGICCH | "C_SACCH_B" | all SACCHs of cell B |
| C_SACCH_C | LOGICCH | "C_SACCH_C" | all SACCHs of cell C |
| C_SACCH_D | LOGICCH | "C_SACCH_D" | all SACCHs of cell D |
| C_SACCH_E | LOGICCH | "C_SACCH_E" | all SACCHs of cell E |
| C_SACCH_F | LOGICCH | "C_SACCH_F" | all SACCHs of cell F |
| C_SACCH_G | LOGICCH | "C_SACCH_G" | all SACCHs of cell G |
| C_SACCH_H | LOGICCH | "C_SACCH_H" | all SACCHs of cell H |
| C_SACCHF_A_1 | LOGICCH | "C_SACCHF_A_1" | SACCH associated with 1st TCH/F of cell A |
| C_SACCHF_A_2 | LOGICCH | "C_SACCHF_A_2" | SACCH associated with 2nd TCH/F of cell A |
| C_SACCHF_B_1 | LOGICCH | "C_SACCHF_B_1" | SACCH associated with 1st TCH/F of cell B |
| C_SACCHF_B_2 | LOGICCH | "C_SACCHF_B_2" | SACCH associated with 2nd TCH/F of cell B |
| C_SACCHF_H_1 | LOGICCH | "C_SACCHF_H_1" | SACCH associated with 1st TCH/F of cell H |
| C_SACCHH_A_1 | LOGICCH | "C_SACCHH_A_1" | SACCH associated with 1st TCH/H of cell A |
| C_SACCHH_A_2 | LOGICCH | "C_SACCHH_A_2" | SACCH associated with 2nd TCH/H of cell A |
| C_SACCHH_A_3 | LOGICCH | "C_SACCHH_A_3" | SACCH associated with 3rd TCH/H of cell A |
| C_SACCHH_B_1 | LOGICCH | "C_SACCHH_B_1" | SACCH associated with 1st TCH/H of cell B |
| C_SACCHH_B_2 | LOGICCH | "C_SACCHH_B_2" | SACCH associated with 2nd TCH/H of cell B |
| C_SACCHH_B_3 | LOGICCH | "C_SACCHH_B_3" | SACCH associated with 3rd TCH/H of cell B |
| C_SACCHH_C_1 | LOGICCH | "C_SACCHH_C_1" | SACCH associated with 1st TCH/H of cell C |
| C_SACCHH_C_2 | LOGICCH | "C_SACCHH_C_2" | SACCH associated with 2nd TCH/H of cell C |
| C_SACCHH_C_3 | LOGICCH | "C_SACCHH_C_3" | SACCH associated with 3rd TCH/H of cell C |
| C_SACCHH0_A_1 | LOGICCH | "C_SACCHH0_A_1" | SACCH associated with 1st TCH/H_0 of cell A |
| C_SACCHH1_A_1 | LOGICCH | "C_SACCHH1_A_1" | SACCH associated with 1st TCH/H_1 of cell A |
| C_SACCHH0_A_2 | LOGICCH | "C_SACCHH0_A_2" | SACCH associated with 2nd TCH/H_0 of cell A |
| C_SACCHH1_A_2 | LOGICCH | "C_SACCHH1_A_2" | SACCH associated with 2nd TCH/H_1 of cell A |
| C_SACCHH0_A_3 | LOGICCH | "C_SACCHH0_A_3" | SACCH associated with 3rd TCH/H_0 of cell A |
| C_SACCHH1_A_3 | LOGICCH | "C_SACCHH1_A_3" | SACCH associated with 3rd TCH/H_1 of cell A |
| C_SACCHH0_B_1 | LOGICCH | "C_SACCHH0_B_1" | SACCH associated with 1st TCH/H_0 of cell B |
| C_SACCHH1_B_1 | LOGICCH | "C_SACCHH1_B_1" | SACCH associated with 1st TCH/H_1 of cell B |
| C_SACCHH0_B_2 | LOGICCH | "C_SACCHH0_B_2" | SACCH associated with 2nd TCH/H_0 of cell B |
| C_SACCHH1_B_2 | LOGICCH | "C_SACCHH1_B_2" | SACCH associated with 2nd TCH/H_1 of cell B |
| C_SACCHH0_B_3 | LOGICCH | "C_SACCHH0_B_3" | SACCH associated with 3rd TCH/H_0 of cell B |
| C_SACCHH1_B_3 | LOGICCH | "C_SACCHH1_B_3" | SACCH associated with 3rd TCH/H_1 of cell B |
| C_SACCHH0_C_1 | LOGICCH | "C_SACCHH0_C_1" | SACCH associated with 1st TCH/H_0 of cell C |
| C_SACCHH1_C_1 | LOGICCH | "C_SACCHH1_C_1" | SACCH associated with 1st TCH/H_1 of cell C |
| C_SACCHH0_C_2 | LOGICCH | "C_SACCHH0_C_2" | SACCH associated with 2nd TCH/H_0 of cell C |
| C_SACCHH1_C_2 | LOGICCH | "C_SACCHH1_C_2" | SACCH associated with 2nd TCH/H_1 of cell C |
| C_SACCHH0_C_3 | LOGICCH | "C_SACCHH0_C_3" | SACCH associated with 3rd TCH/H_0 of cell C |
| C_SACCHH1_C_3 | LOGICCH | "C_SACCHH1_C_3" | SACCH associated with 3rd TCH/H_1 of cell C |
| C_SACCHC4_A | LOGICCH | "C_SACCHC4_A" | SACCHC4 channel of cell A |
| C_SACCHC4_B | LOGICCH | "C_SACCHC4_B" | SACCHC4 channel of cell B |
| C_SACCHC4_C | LOGICCH | "C_SACCHC4_C" | SACCHC4 channel of cell C |
| C_SACCHC4_D | LOGICCH | "C_SACCHC4_D" | SACCHC4 channel of cell D |
| C_SACCHC4_E | LOGICCH | "C_SACCHC4_E" | SACCHC4 channel of cell E |
| C_SACCHC4_F | LOGICCH | "C_SACCHC4_F" | SACCHC4 channel of cell F |
| C_SACCHC4_G | LOGICCH | "C_SACCHC4_G" | SACCHC4 channel of cell G |
| C_SACCHC4_H | LOGICCH | "C_SACCHC4_H" | SACCHC4 channel of cell H |
| C_SACCHC40_A | LOGICCH | "C_SACCHC40_A" | SACCH/C4(0) associated with SDCCH/4(0) of cell A |
| C_SACCHC41_A | LOGICCH | "C_SACCHC41_A" | SACCH/C4(1) associated with SDCCH/4(1) of cell A |
| C_SACCHC42_A | LOGICCH | "C_SACCHC42_A" | SACCH/C4(2) associated with SDCCH/4(2) of cell A |
| C_SACCHC43_A | LOGICCH | "C_SACCCH43_A" | SACCH/C4(3) associated with SDCCH/4(3) of cell A |
| C_SACCHC40_B | LOGICCH | "C_SACCHC40_B" | SACCH/C4(0) associated with SDCCH/4(0) of cell B |
| C_SACCHC41_B | LOGICCH | "C_SACCHC41_B" | SACCH/C4(1) associated with SDCCH/4(1) of cell B |
| C_SACCHC42_B | LOGICCH | "C_SACCHC42_B" | SACCH/C4(2) associated with SDCCH/4(2) of cell B |
| C_SACCHC43_B | LOGICCH | "C_SACCHC43_B" | SACCH/C4(3) associated with SDCCH/4(3) of cell B |
| C_SACCHC40_C | LOGICCH | "C_SACCHC40_C" | SACCH/C4(0) associated with SDCCH/4(0) of cell C |
| C_SACCHC41_C | LOGICCH | "C_SACCHC41_C" | SACCH/C4(1) associated with SDCCH/4(1) of cell C |
| C_SACCHC42_C | LOGICCH | "C_SACCHC42_C" | SACCH/C4(2) associated with SDCCH/4(2) of cell C |
| C_SACCHC43_C | LOGICCH | "C_SACCHC43_C" | SACCH/C4(3) associated with SDCCH/4(3) of cell C |
| C_SACCHC40_D | LOGICCH | "C_SACCHC40_D" | SACCH/C4(0) associated with SDCCH/4(0) of cell D |
| C_SACCHC41_D | LOGICCH | "C_SACCHC41_D" | SACCH/C4(1) associated with SDCCH/4(1) of cell D |
| C_SACCHC42_D | LOGICCH | "C_SACCHC42_D" | SACCH/C4(2) associated with SDCCH/4(2) of cell D |
| C_SACCHC43_D | LOGICCH | "C_SACCCH43_D" | SACCH/C4(3) associated with SDCCH/4(3) of cell D |
| C_SACCHC40_E | LOGICCH | "C_SACCHC40_E" | SACCH/C4(0) associated with SDCCH/4(0) of cell E |
| C_SACCHC41_E | LOGICCH | "C_SACCHC41_E" | SACCH/C4(1) associated with SDCCH/4(1) of cell E |
| C_SACCHC42_E | LOGICCH | "C_SACCHC42_E" | SACCH/C4(2) associated with SDCCH/4(2) of cell E |
| C_SACCHC43_E | LOGICCH | "C_SACCHC43_E" | SACCH/C4(3) associated with SDCCH/4(3) of cell E |
| C_SACCHC40_F | LOGICCH | "C_SACCHC40_F" | SACCH/C4(0) associated with SDCCH/4(0) of cell F |
| C_SACCHC41_F | LOGICCH | "C_SACCHC41_F" | SACCH/C4(1) associated with SDCCH/4(1) of cell F |
| C_SACCHC42_F | LOGICCH | "C_SACCHC42_F" | SACCH/C4(2) associated with SDCCH/4(2) of cell F |
| C_SACCHC43_F | LOGICCH | "C_SACCHC43_F" | SACCH/C4(3) associated with SDCCH/4(3) of cell F |
| C_SACCHC40_G | LOGICCH | "C_SACCHC40_G" | SACCH/C4(0) associated with SDCCH/4(0) of cell G |
| C_SACCHC41_G | LOGICCH | "C_SACCHC41_G" | SACCH/C4(1) associated with SDCCH/4(1) of cell G |
| C_SACCHC42_G | LOGICCH | "C_SACCHC42_G" | SACCH/C4(2) associated with SDCCH/4(2) of cell G |
| C_SACCHC43_G | LOGICCH | "C_SACCHC43_G" | SACCH/C4(3) associated with SDCCH/4(3) of cell G |
| C_SACCHC40_H | LOGICCH | "C_SACCHC40_C" | SACCH/C4(0) associated with SDCCH/4(0) of cell H |
| C_SACCHC41_H | LOGICCH | "C_SACCHC41_C" | SACCH/C4(1) associated with SDCCH/4(1) of cell H |
| C_SACCHC42_H | LOGICCH | "C_SACCHC42_C" | SACCH/C4(2) associated with SDCCH/4(2) of cell H |
| C_SACCHC43_H | LOGICCH | "C_SACCHC43_C" | SACCH/C4(3) associated with SDCCH/4(3) of cell H |
| C_SACCHC8_A_1 | LOGICCH | "C_SACCHC8_A_1" | 1st SACCH/8 channel of cell A |
| C_SACCHC8_A_2 | LOGICCH | "C_SACCHC8_A_2" | 2nd SACCH/8 channel of cell A |
| C_SACCHC8_A_3 | LOGICCH | "C_SACCHC8_A_3" | 3rd SACCH/8 channel of cell A |
| C_SACCHC8_B_1 | LOGICCH | "C_SACCHC8_B_1" | 1st SACCH/8 channel of cell B |
| C_SACCHC8_B_2 | LOGICCH | "C_SACCHC8_B_2" | 2nd SACCH/8 channel of cell B |
| C_SACCHC8_B_3 | LOGICCH | "C_SACCHC8_B_3" | 3rd SACCH/8 channel of cell B |
| C_SACCHC8_C_1 | LOGICCH | "C_SACCHC8_C_1" | 1st SACCH/8 channel of cell C |
| C_SACCHC8_C_2 | LOGICCH | "C_SACCHC8_C_2" | 2nd SACCH/8 channel of cell C |
| C_SACCHC8_C_3 | LOGICCH | "C_SACCHC8_C_3" | 3rd SACCH/8 channel of cell C |
| C_SACCHC80_A_1 | LOGICCH | "C_SACCHC80_A_1" | SACCH/C8(0) associated with 1st SDCCH/8(0) of cell A |
| C_SACCHC81_A_1 | LOGICCH | "C_SACCHC81_A_1" | SACCH/C8(1) associated with 1st SDCCH/8(1) of cell A |
| C_SACCHC82_A_1 | LOGICCH | "C_SACCHC82_A_1" | SACCH/C8(2) associated with 1st SDCCH/8(2) of cell A |
| C_SACCHC83_A_1 | LOGICCH | "C_SACCHC83_A_1" | SACCH/C8(3) associated with 1st SDCCH/8(3) of cell A |
| C_SACCHC84_A_1 | LOGICCH | "C_SACCHC84_A_1" | SACCH/C8(4) associated with 1st SDCCH/8(4) of cell A |
| C_SACCHC85_A_1 | LOGICCH | "C_SACCHC85_A_1" | SACCH/C8(5) associated with 1st SDCCH/8(5) of cell A |
| C_SACCHC86_A_1 | LOGICCH | "C_SACCHC86_A_1" | SACCH/C8(6) associated with 1st SDCCH/8(6) of cell A |
| C_SACCHC87_A_1 | LOGICCH | "C_SACCHC87_A_1" | SACCH/C8(7) associated with 1st SDCCH/8(7) of cell A |
| C_SACCHC80_A_2 | LOGICCH | "C_SACCHC80_A_2" | SACCH/C8(0) associated with 2nd SDCCH/8(0) of cell A |
| C_SACCHC81_A_2 | LOGICCH | "C_SACCHC81_A_2" | SACCH/C8(1) associated with 2nd SDCCH/8(1) of cell A |
| C_SACCHC82_A_2 | LOGICCH | "C_SACCHC82_A_2" | SACCH/C8(2) associated with 2nd SDCCH/8(2) of cell A |
| C_SACCHC83_A_2 | LOGICCH | "C_SACCHC83_A_2" | SACCH/C8(3) associated with 2nd SDCCH/8(3) of cell A |
| C_SACCHC84_A_2 | LOGICCH | "C_SACCHC84_A_2" | SACCH/C8(4) associated with 2nd SDCCH/8(4) of cell A |
| C_SACCHC85_A_2 | LOGICCH | "C_SACCHC85_A_2" | SACCH/C8(5) associated with 2nd SDCCH/8(5) of cell A |
| C_SACCHC86_A_2 | LOGICCH | "C_SACCHC86_A_2" | SACCH/C8(6) associated with 2nd SDCCH/8(6) of cell A |
| C_SACCHC87_A_2 | LOGICCH | "C_SACCHC87_A_2" | SACCH/C8(7) associated with 2nd SDCCH/8(7) of cell A |
| C_SACCHC80_A_3 | LOGICCH | "C_SACCHC80_A_3" | SACCH/C8(0) associated with 3rd SDCCH/8(0) of cell A |
| C_SACCHC81_A_3 | LOGICCH | "C_SACCHC81_A_3" | SACCH/C8(1) associated with 3rd SDCCH/8(1) of cell A |
| C_SACCHC82_A_3 | LOGICCH | "C_SACCHC82_A_3" | SACCH/C8(2) associated with 3rd SDCCH/8(2) of cell A |
| C_SACCHC83_A_3 | LOGICCH | "C_SACCHC83_A_3" | SACCH/C8(3) associated with 3rd SDCCH/8(3) of cell A |
| C_SACCHC84_A_3 | LOGICCH | "C_SACCHC84_A_3" | SACCH/C8(4) associated with 3rd SDCCH/8(4) of cell A |
| C_SACCHC85_A_3 | LOGICCH | "C_SACCHC85_A_3" | SACCH/C8(5) associated with 3rd SDCCH/8(5) of cell A |
| C_SACCHC86_A_3 | LOGICCH | "C_SACCHC86_A_3" | SACCH/C8(6) associated with 3rd SDCCH/8(6) of cell A |
| C_SACCHC87_A_3 | LOGICCH | "C_SACCHC87_A_3" | SACCH/C8(7) associated with 3rd SDCCH/8(7) of cell A |
| C_SACCHC80_B_1 | LOGICCH | "C_SACCHC80_B_1" | SACCH/C8(0) associated with 1st SDCCH/8(0) of cell B |
| C_SACCHC81_B_1 | LOGICCH | "C_SACCHC81_B_1" | SACCH/C8(1) associated with 1st SDCCH/8(1) of cell B |
| C_SACCHC82_B_1 | LOGICCH | "C_SACCHC82_B_1" | SACCH/C8(2) associated with 1st SDCCH/8(2) of cell B |
| C_SACCHC83_B_1 | LOGICCH | "C_SACCHC83_B_1" | SACCH/C8(3) associated with 1st SDCCH/8(3) of cell B |
| C_SACCHC84_B_1 | LOGICCH | "C_SACCHC84_B_1" | SACCH/C8(4) associated with 1st SDCCH/8(4) of cell B |
| C_SACCHC85_B_1 | LOGICCH | "C_SACCHC85_B_1" | SACCH/C8(5) associated with 1st SDCCH/8(5) of cell B |
| C_SACCHC86_B_1 | LOGICCH | "C_SACCHC86_B_1" | SACCH/C8(6) associated with 1st SDCCH/8(6) of cell B |
| C_SACCHC87_B_1 | LOGICCH | "C_SACCHC87_B_1" | SACCH/C8(7) associated with 1st SDCCH/8(7) of cell B |
| C_SACCHC80_B_2 | LOGICCH | "C_SACCHC80_B_2" | SACCH/C8(0) associated with 2nd SDCCH/8(0) of cell B |
| C_SACCHC81_B_2 | LOGICCH | "C_SACCHC81_B_2" | SACCH/C8(1) associated with 2nd SDCCH/8(1) of cell B |
| C_SACCHC82_B_2 | LOGICCH | "C_SACCHC82_B_2" | SACCH/C8(2) associated with 2nd SDCCH/8(2) of cell B |
| C_SACCHC83_B_2 | LOGICCH | "C_SACCHC83_B_2" | SACCH/C8(3) associated with 2nd SDCCH/8(3) of cell B |
| C_SACCHC84_B_2 | LOGICCH | "C_SACCHC84_B_2" | SACCH/C8(4) associated with 2nd SDCCH/8(4) of cell B |
| C_SACCHC85_B_2 | LOGICCH | "C_SACCHC85_B_2" | SACCH/C8(5) associated with 2nd SDCCH/8(5) of cell B |
| C_SACCHC86_B_2 | LOGICCH | "C_SACCHC86_B_2" | SACCH/C8(6) associated with 2nd SDCCH/8(6) of cell B |
| C_SACCHC87_B_2 | LOGICCH | "C_SACCHC87_B_2" | SACCH/C8(7) associated with 2nd SDCCH/8(7) of cell B |
| C_SACCHC80_B_3 | LOGICCH | "C_SACCHC80_B_3" | SACCH/C8(0) associated with 3rd SDCCH/8(0) of cell B |
| C_SACCHC81_B_3 | LOGICCH | "C_SACCHC81_B_3" | SACCH/C8(1) associated with 3rd SDCCH/8(1) of cell B |
| C_SACCHC82_B_3 | LOGICCH | "C_SACCHC82_B_3" | SACCH/C8(2) associated with 3rd SDCCH/8(2) of cell B |
| C_SACCHC83_B_3 | LOGICCH | "C_SACCHC83_B_3" | SACCH/C8(3) associated with 3rd SDCCH/8(3) of cell B |
| C_SACCHC84_B_3 | LOGICCH | "C_SACCHC84_B_3" | SACCH/C8(4) associated with 3rd SDCCH/8(4) of cell B |
| C_SACCHC85_B_3 | LOGICCH | "C_SACCHC85_B_3" | SACCH/C8(5) associated with 3rd SDCCH/8(5) of cell B |
| C_SACCHC86_B_3 | LOGICCH | "C_SACCHC86_B_3" | SACCH/C8(6) associated with 3rd SDCCH/8(6) of cell B |
| C_SACCHC87_B_3 | LOGICCH | "C_SACCHC87_B_3" | SACCH/C8(7) associated with 3rd SDCCH/8(7) of cell B |
| C_SACCHC80_C_1 | LOGICCH | "C_SACCHC80_C_1" | SACCH/C8(0) associated with 1st SDCCH/8(0) of cell C |
| C_SACCHC81_C_1 | LOGICCH | "C_SACCHC81_C_1" | SACCH/C8(1) associated with 1st SDCCH/8(1) of cell C |
| C_SACCHC82_C_1 | LOGICCH | "C_SACCHC82_C_1" | SACCH/C8(2) associated with 1st SDCCH/8(2) of cell C |
| C_SACCHC83_C_1 | LOGICCH | "C_SACCHC83_C_1" | SACCH/C8(3) associated with 1st SDCCH/8(3) of cell C |
| C_SACCHC84_C_1 | LOGICCH | "C_SACCHC84_C_1" | SACCH/C8(4) associated with 1st SDCCH/8(4) of cell C |
| C_SACCHC85_C_1 | LOGICCH | "C_SACCHC85_C_1" | SACCH/C8(5) associated with 1st SDCCH/8(5) of cell C |
| C_SACCHC86_C_1 | LOGICCH | "C_SACCHC86_C_1" | SACCH/C8(6) associated with 1st SDCCH/8(6) of cell C |
| C_SACCHC87_C_1 | LOGICCH | "C_SACCHC87_C_1" | SACCH/C8(7) associated with 1st SDCCH/8(7) of cell C |
| C_SACCHC80_C_2 | LOGICCH | "C_SACCHC80_C_2" | SACCH/C8(0) associated with 2nd SDCCH/8(0) of cell C |
| C_SACCHC81_C_2 | LOGICCH | "C_SACCHC81_C_2" | SACCH/C8(1) associated with 2nd SDCCH/8(1) of cell C |
| C_SACCHC82_C_2 | LOGICCH | "C_SACCHC82_C_2" | SACCH/C8(2) associated with 2nd SDCCH/8(2) of cell C |
| C_SACCHC83_C_2 | LOGICCH | "C_SACCHC83_C_2" | SACCH/C8(3) associated with 2nd SDCCH/8(3) of cell C |
| C_SACCHC84_C_2 | LOGICCH | "C_SACCHC84_C_2" | SACCH/C8(4) associated with 2nd SDCCH/8(4) of cell C |
| C_SACCHC85_C_2 | LOGICCH | "C_SACCHC85_C_2" | SACCH/C8(5) associated with 2nd SDCCH/8(5) of cell C |
| C_SACCHC86_C_2 | LOGICCH | "C_SACCHC86_C_2" | SACCH/C8(6) associated with 2nd SDCCH/8(6) of cell C |
| C_SACCHC87_C_2 | LOGICCH | "C_SACCHC87_C_2" | SACCH/C8(7) associated with 2nd SDCCH/8(7) of cell C |
| C_SACCHC80_C_3 | LOGICCH | "C_SACCHC80_C_3" | SACCH/C8(0) associated with 3rd SDCCH/8(0) of cell C |
| C_SACCHC81_C_3 | LOGICCH | "C_SACCHC81_C_3" | SACCH/C8(1) associated with 3rd SDCCH/8(1) of cell C |
| C_SACCHC82_C_3 | LOGICCH | "C_SACCHC82_C_3" | SACCH/C8(2) associated with 3rd SDCCH/8(2) of cell C |
| C_SACCHC83_C_3 | LOGICCH | "C_SACCHC83_C_3" | SACCH/C8(3) associated with 3rd SDCCH/8(3) of cell C |
| C_SACCHC84_C_3 | LOGICCH | "C_SACCHC84_C_3" | SACCH/C8(4) associated with 3rd SDCCH/8(4) of cell C |
| C_SACCHC85_C_3 | LOGICCH | "C_SACCHC85_C_3" | SACCH/C8(5) associated with 3rd SDCCH/8(5) of cell C |
| C_SACCHC86_C_3 | LOGICCH | "C_SACCHC86_C_3" | SACCH/C8(6) associated with 3rd SDCCH/8(6) of cell C |
| C_SACCHC87_C_3 | LOGICCH | "C_SACCHC87_C_3" | SACCH/C8(7) associated with 3rd SDCCH/8(7) of cell C |
| C_SDCCH4_A | LOGICCH | "C_SDCCH4_A" | SDCCH/4 channel of cell A |
| C_SDCCH4_B | LOGICCH | "C_SDCCH4_B" | SDCCH/4 channel of cell B |
| C_SDCCH4_C | LOGICCH | "C_SDCCH4_C" | SDCCH/4 channel of cell C |
| C_SDCCH4_D | LOGICCH | "C_SDCCH4_D" | SDCCH/4 channel of cell D |
| C_SDCCH4_E | LOGICCH | "C_SDCCH4_E" | SDCCH/4 channel of cell E |
| C_SDCCH4_F | LOGICCH | "C_SDCCH4_F" | SDCCH/4 channel of cell F |
| C_SDCCH4_G | LOGICCH | "C_SDCCH4_G" | SDCCH/4 channel of cell G |
| C_SDCCH4_H | LOGICCH | "C_SDCCH4_H" | SDCCH/4 channel of cell H |
| C_SDCCH40_A | LOGICCH | "C_SDCCH40_A" | SDCCH/4(0) of cell A |
| C_SDCCH41_A | LOGICCH | "C_SDCCH41_A" | SDCCH/4(1) of cell A |
| C_SDCCH42_A | LOGICCH | "C_SDCCH42_A" | SDCCH/4(2) of cell A |
| C_SDCCH43_A | LOGICCH | "C_SDCCH43_A" | SDCCH/4(3) of cell A |
| C_SDCCH40_B | LOGICCH | "C_SDCCH40_B" | SDCCH/4(0) of cell B |
| C_SDCCH41_B | LOGICCH | "C_SDCCH41_B" | SDCCH/4(1) of cell B |
| C_SDCCH42_B | LOGICCH | "C_SDCCH42_B" | SDCCH/4(2) of cell B |
| C_SDCCH43_B | LOGICCH | "C_SDCCH43_B" | SDCCH/4(3) of cell B |
| C_SDCCH40_C | LOGICCH | "C_SDCCH40_C" | SDCCH/4(0) of cell C |
| C_SDCCH41_C | LOGICCH | "C_SDCCH41_C" | SDCCH/4(1) of cell C |
| C_SDCCH42_C | LOGICCH | "C_SDCCH42_C" | SDCCH/4(2) of cell C |
| C_SDCCH43_C | LOGICCH | "C_SDCCH43_C" | SDCCH/4(3) of cell C |
| C_SDCCH40_D | LOGICCH | "C_SDCCH40_D" | SDCCH/4(0) of cell D |
| C_SDCCH41_D | LOGICCH | "C_SDCCH41_D" | SDCCH/4(1) of cell D |
| C_SDCCH42_D | LOGICCH | "C_SDCCH42_D" | SDCCH/4(2) of cell D |
| C_SDCCH43_D | LOGICCH | "C_SDCCH43_D" | SDCCH/4(3) of cell D |
| C_SDCCH40_E | LOGICCH | "C_SDCCH40_E" | SDCCH/4(0) of cell E |
| C_SDCCH41_E | LOGICCH | "C_SDCCH41_E" | SDCCH/4(1) of cell E |
| C_SDCCH42_E | LOGICCH | "C_SDCCH42_E" | SDCCH/4(2) of cell E |
| C_SDCCH43_E | LOGICCH | "C_SDCCH43_E" | SDCCH/4(3) of cell E |
| C_SDCCH40_F | LOGICCH | "C_SDCCH40_F" | SDCCH/4(0) of cell F |
| C_SDCCH41_F | LOGICCH | "C_SDCCH41_F" | SDCCH/4(1) of cell F |
| C_SDCCH42_F | LOGICCH | "C_SDCCH42_F" | SDCCH/4(2) of cell F |
| C_SDCCH43_F | LOGICCH | "C_SDCCH43_F" | SDCCH/4(3) of cell F |
| C_SDCCH40_G | LOGICCH | "C_SDCCH40_G" | SDCCH/4(0) of cell G |
| C_SDCCH41_G | LOGICCH | "C_SDCCH41_G" | SDCCH/4(1) of cell G |
| C_SDCCH42_G | LOGICCH | "C_SDCCH42_G" | SDCCH/4(2) of cell G |
| C_SDCCH43_G | LOGICCH | "C_SDCCH43_G" | SDCCH/4(3) of cell G |
| C_SDCCH40_H | LOGICCH | "C_SDCCH40_H" | SDCCH/4(0) of cell H |
| C_SDCCH41_H | LOGICCH | "C_SDCCH41_H" | SDCCH/4(1) of cell H |
| C_SDCCH42_H | LOGICCH | "C_SDCCH42_H" | SDCCH/4(2) of cell H |
| C_SDCCH43_H | LOGICCH | "C_SDCCH43_H" | SDCCH/4(3) of cell H |
| C_SDCCH8_A_1 | LOGICCH | "C_SDCCH8_A_1" | 1st SDCCH/8 channel of cell A |
| C_SDCCH8_A_2 | LOGICCH | "C_SDCCH8_A_2" | 2nd SDCCH/8 channel of cell A |
| C_SDCCH8_A_3 | LOGICCH | "C_SDCCH8_A_3" | 3rd SDCCH/8 channel of cell A |
| C_SDCCH8_B_1 | LOGICCH | "C_SDCCH8_B_1" | 1st SDCCH/8 channel of cell B |
| C_SDCCH8_B_2 | LOGICCH | "C_SDCCH8_B_2" | 2nd SDCCH/8 channel of cell B |
| C_SDCCH80_A_1 | LOGICCH | "C_SDCCH80_A_1" | 1st SDCCH/8(0) of cell A |
| C_SDCCH81_A_1 | LOGICCH | "C_SDCCH81_A_1" | 1st SDCCH/8(1) of cell A |
| C_SDCCH82_A_1 | LOGICCH | "C_SDCCH82_A_1" | 1st SDCCH/8(2) of cell A |
| C_SDCCH83_A_1 | LOGICCH | "C_SDCCH83_A_1" | 1st SDCCH/8(3) of cell A |
| C_SDCCH84_A_1 | LOGICCH | "C_SDCCH84_A_1" | 1st SDCCH/8(4) of cell A |
| C_SDCCH85_A_1 | LOGICCH | "C_SDCCH85_A_1" | 1st SDCCH/8(5) of cell A |
| C_SDCCH86_A_1 | LOGICCH | "C_SDCCH86_A_1" | 1st SDCCH/8(6) of cell A |
| C_SDCCH87_A_1 | LOGICCH | "C_SDCCH87_A_1" | 1st SDCCH/8(7) of cell A |
| C_SDCCH80_A_2 | LOGICCH | "C_SDCCH80_A_2" | 2nd SDCCH/8(0) of cell A |
| C_SDCCH81_A_2 | LOGICCH | "C_SDCCH81_A_2" | 2nd SDCCH/8(1) of cell A |
| C_SDCCH82_A_2 | LOGICCH | "C_SDCCH82_A_2" | 2nd SDCCH/8(2) of cell A |
| C_SDCCH83_A_2 | LOGICCH | "C_SDCCH83_A_2" | 2nd SDCCH/8(3) of cell A |
| C_SDCCH84_A_2 | LOGICCH | "C_SDCCH84_A_2" | 2nd SDCCH/8(4) of cell A |
| C_SDCCH85_A_2 | LOGICCH | "C_SDCCH85_A_2" | 2nd SDCCH/8(5) of cell A |
| C_SDCCH86_A_2 | LOGICCH | "C_SDCCH86_A_2" | 2nd SDCCH/8(6) of cell A |
| C_SDCCH87_A_2 | LOGICCH | "C_SDCCH87_A_2" | 2nd SDCCH/8(7) of cell A |
| C_SDCCH80_A_3 | LOGICCH | "C_SDCCH80_A_3" | 3rd SDCCH/8(0) of cell A |
| C_SDCCH81_A_3 | LOGICCH | "C_SDCCH81_A_3" | 3rd SDCCH/8(1) of cell A |
| C_SDCCH82_A_3 | LOGICCH | "C_SDCCH82_A_3" | 3rd SDCCH/8(2) of cell A |
| C_SDCCH83_A_3 | LOGICCH | "C_SDCCH83_A_3" | 3rd SDCCH/8(3) of cell A |
| C_SDCCH84_A_3 | LOGICCH | "C_SDCCH84_A_3" | 3rd SDCCH/8(4) of cell A |
| C_SDCCH85_A_3 | LOGICCH | "C_SDCCH85_A_3" | 3rd SDCCH/8(5) of cell A |
| C_SDCCH86_A_3 | LOGICCH | "C_SDCCH86_A_3" | 3rd SDCCH/8(6) of cell A |
| C_SDCCH87_A_3 | LOGICCH | "C_SDCCH87_A_3" | 3rd SDCCH/8(7) of cell A |
| C_SDCCH80_B_1 | LOGICCH | "C_SDCCH80_B_1" | 1st SDCCH/8(0) of cell B |
| C_SDCCH81_B_1 | LOGICCH | "C_SDCCH81_B_1" | 1st SDCCH/8(1) of cell B |
| C_SDCCH82_B_1 | LOGICCH | "C_SDCCH82_B_1" | 1st SDCCH/8(2) of cell B |
| C_SDCCH83_B_1 | LOGICCH | "C_SDCCH83_B_1" | 1st SDCCH/8(3) of cell B |
| C_SDCCH84_B_1 | LOGICCH | "C_SDCCH84_B_1" | 1st SDCCH/8(4) of cell B |
| C_SDCCH85_B_1 | LOGICCH | "C_SDCCH85_B_1" | 1st SDCCH/8(5) of cell B |
| C_SDCCH86_B_1 | LOGICCH | "C_SDCCH86_B_1" | 1st SDCCH/8(6) of cell B |
| C_SDCCH87_B_1 | LOGICCH | "C_SDCCH87_B_1" | 1st SDCCH/8(7) of cell B |
| C_SDCCH80_B_2 | LOGICCH | "C_SDCCH80_B_2" | 2nd SDCCH/8(0) of cell B |
| C_SDCCH81_B_2 | LOGICCH | "C_SDCCH81_B_2" | 2nd SDCCH/8(1) of cell B |
| C_SDCCH82_B_2 | LOGICCH | "C_SDCCH82_B_2" | 2nd SDCCH/8(2) of cell B |
| C_SDCCH83_B_2 | LOGICCH | "C_SDCCH83_B_2" | 2nd SDCCH/8(3) of cell B |
| C_SDCCH84_B_2 | LOGICCH | "C_SDCCH84_B_2" | 2nd SDCCH/8(4) of cell B |
| C_SDCCH85_B_2 | LOGICCH | "C_SDCCH85_B_2" | 2nd SDCCH/8(5) of cell B |
| C_SDCCH86_B_2 | LOGICCH | "C_SDCCH86_B_2" | 2nd SDCCH/8(6) of cell B |
| C_SDCCH87_B_2 | LOGICCH | "C_SDCCH87_B_2" | 2nd SDCCH/8(7) of cell B |
| C_SDCCH80_B_3 | LOGICCH | "C_SDCCH80_B_3" | 3rd SDCCH/8(0) of cell B |
| C_SDCCH81_B_3 | LOGICCH | "C_SDCCH81_B_3" | 3rd SDCCH/8(1) of cell B |
| C_SDCCH82_B_3 | LOGICCH | "C_SDCCH82_B_3" | 3rd SDCCH/8(2) of cell B |
| C_SDCCH83_B_3 | LOGICCH | "C_SDCCH83_B_3" | 3rd SDCCH/8(3) of cell B |
| C_SDCCH84_B_3 | LOGICCH | "C_SDCCH84_B_3" | 3rd SDCCH/8(4) of cell B |
| C_SDCCH85_B_3 | LOGICCH | "C_SDCCH85_B_3" | 3rd SDCCH/8(5) of cell B |
| C_SDCCH86_B_3 | LOGICCH | "C_SDCCH86_B_3" | 3rd SDCCH/8(6) of cell B |
| C_SDCCH87_B_3 | LOGICCH | "C_SDCCH87_B_3" | 3rd SDCCH/8(7) of cell B |
| C_SDCCH80_C_1 | LOGICCH | "C_SDCCH80_C_1" | 1st SDCCH/8(0) of cell C |
| C_SDCCH81_C_1 | LOGICCH | "C_SDCCH81_C_1" | 1st SDCCH/8(1) of cell C |
| C_SDCCH82_C_1 | LOGICCH | "C_SDCCH82_C_1" | 1st SDCCH/8(2) of cell C |
| C_SDCCH83_C_1 | LOGICCH | "C_SDCCH83_C_1" | 1st SDCCH/8(3) of cell C |
| C_SDCCH84_C_1 | LOGICCH | "C_SDCCH84_C_1" | 1st SDCCH/8(4) of cell C |
| C_SDCCH85_C_1 | LOGICCH | "C_SDCCH85_C_1" | 1st SDCCH/8(5) of cell C |
| C_SDCCH86_C_1 | LOGICCH | "C_SDCCH86_C_1" | 1st SDCCH/8(6) of cell C |
| C_SDCCH87_C_1 | LOGICCH | "C_SDCCH87_C_1" | 1st SDCCH/8(7) of cell C |
| C_SDCCH80_C_2 | LOGICCH | "C_SDCCH80_C_2" | 2nd SDCCH/8(0) of cell C |
| C_SDCCH81_C_2 | LOGICCH | "C_SDCCH81_C_2" | 2nd SDCCH/8(1) of cell C |
| C_SDCCH82_C_2 | LOGICCH | "C_SDCCH82_C_2" | 2nd SDCCH/8(2) of cell C |
| C_SDCCH83_C_2 | LOGICCH | "C_SDCCH83_C_2" | 2nd SDCCH/8(3) of cell C |
| C_SDCCH84_C_2 | LOGICCH | "C_SDCCH84_C_2" | 2nd SDCCH/8(4) of cell C |
| C_SDCCH85_C_2 | LOGICCH | "C_SDCCH85_C_2" | 2nd SDCCH/8(5) of cell C |
| C_SDCCH86_C_2 | LOGICCH | "C_SDCCH86_C_2" | 2nd SDCCH/8(6) of cell C |
| C_SDCCH87_C_2 | LOGICCH | "C_SDCCH87_C_2" | 2nd SDCCH/8(7) of cell C |
| C_SDCCH80_C_3 | LOGICCH | "C_SDCCH80_C_3" | 3rd SDCCH/8(0) of cell C |
| C_SDCCH81_C_3 | LOGICCH | "C_SDCCH81_C_3" | 3rd SDCCH/8(1) of cell C |
| C_SDCCH82_C_3 | LOGICCH | "C_SDCCH82_C_3" | 3rd SDCCH/8(2) of cell C |
| C_SDCCH83_C_3 | LOGICCH | "C_SDCCH83_C_3" | 3rd SDCCH/8(3) of cell C |
| C_SDCCH84_C_3 | LOGICCH | "C_SDCCH84_C_3" | 3rd SDCCH/8(4) of cell C |
| C_SDCCH85_C_3 | LOGICCH | "C_SDCCH85_C_3" | 3rd SDCCH/8(5) of cell C |
| C_SDCCH86_C_3 | LOGICCH | "C_SDCCH86_C_3" | 3rd SDCCH/8(6) of cell C |
| C_SDCCH87_C_3 | LOGICCH | "C_SDCCH87_C_3" | 3rd SDCCH/8(7) of cell C |
| C_CHSDCCH8_FH | OCTETSTRING | '00'O | ch.type SDCCH8 with FH |
| C_CHSDCCH8_NonFH | OCTETSTRING | '01'O | ch.type SDCCH8 No FH |
| C_CHSDCCH4_NonFH | OCTETSTRING | '03'O | ch.type SDCCH4 NoFH |
| C_CHTCHF_FH | OCTETSTRING | '04'O | ch.type TCHF FH |
| C_CHTCHF_NonFH | OCTETSTRING | '05'O | ch.type TCHF NonFH |
| C_CHTCHH_FH | OCTETSTRING | '06'O | ch.type TCHH FH |
| C_TCHF_ACCHF_1 | LOGCH | "C_TCHF_ACCHF_1" | first TCH/F + ACCHs type channel |
| C_TCHF_ACCHF_2 | LOGCH | "C_TCHF_ACCHF_2" | second TCH/F + ACCHs type channel |
| C_TCHH_ACCHH_1 | LOGCH | "C_TCHH_ACCHH_1" | first TCH/H + ACCHs type channel |
| C_TCHH_ACCHH_2 | LOGCH | "C_TCHH_ACCHH_2" | second TCH/H + ACCHs type channel |
| C_FCCH_SCH_BCCH_CCCH | LOGCH | "C_FCCH_SCH_BCCH_CCCH" | FCCH_SCH_BCCH_CCCH type channel |
| C_BCCH_CCCH_2 | LOGCH | "C_BCCH_CCCH_2" | second BCCH_CCCH type channel |
| C_BCCH_CCCH_3 | LOGCH | "C_BCCH_CCCH_3" | third BCCH_CCCH type channel |
| C_BCCH_CCCH_4 | LOGCH | "C_BCCH_CCCH_4" | fourth BCCH_CCCH type channel |
| C_FCCH_SCH_BCCH_CCCH_SDCCH4_SACCHC4 | LOGCH | "C_FCCH_SCH_BCCH_CCCH_SDCCH4_SACCHC4" | combined CCCH type channel |
| C_CBCH_FCCH_SCH_BCCH_CCCH_SDCCH4_SACCHC4 | LOGCH | "C_CBCH_FCCH_SCH_BCCH_CCCH_SDCCH4_SACCHC4" | combined CCCH type channel for cell broadcast SM |
| C_SDCCH8_SACCHC8_1 | LOGCH | "C_SDCCH8_SACCHC8_1" | first SDCCH/8 type channel |
| C_SDCCH8_SACCHC8_2 | LOGCH | "C_SDCCH8_SACCHC8_2" | second SDCCH/8 type channel |
| C_SDCCH8_SACCHC8_3 | LOGCH | "C_SDCCH8_SACCHC8_3" | 3rd SDCCH/8 type channel |
| C_S0 | SN | '000'B | time slot 0 |
| C_S2 | SN | '010'B | time slot 2 |
| C_S3 | SN | '011'B | time slot 3 |
| C_S4 | SN | '100'B | time slot 4 |
| C_S6 | SN | '110'B | time slot 6 |
| C_SAVE | INTEGER | 0 | the OC_SaveAndProc saves input values |
| C_PROC | INTEGER | 1 | the OC_SvaAndProc processes saved values |
| C_RETRV | INTEGER | 1 |  |
| C_U1 | CCSTATE | 1 | CC state U1 |
| C_U3 | CCSTATE | 3 | CC state U3 |
| C_U4 | CCSTATE | 4 | CC state U4 |
| C_U6 | CCSTATE | 6 | CC state U6 |
| C_U7 | CCSTATE | 7 | CC state U7 |
| C_U8 | CCSTATE | 8 | CC state U8 |
| C_U9 | CCSTATE | 9 | CC state U9 |
| C_U10 | CCSTATE | 10 | CC state U10 |
| C_U11 | CCSTATE | 11 | CC state U11 |
| C_U12 | CCSTATE | 12 | CC state U12 |
| C_U19 | CCSTATE | 19 | CC state U19 |
| C_U26 | CCSTATE | 26 | CC state U26 |
| C_NxtButOne | SENDINGMODE | 1 | send the second message on the next but one paging subblock |
| C_FmrAGB | SENDINGMODE | 2 | send the second message on the former access grant block |
| C_BfReOcc | SENDINGMODE | 3 | send the second message before the MS's original paging subchannel re-occurs, but later than the next paging block of that CCCH |
| C_NxtButOneNxt | SENDINGMODE | 4 | nothing is sent in the next but one paging sub block, then send the second message in the next paging subblock of the MS's paging subchannel |
| C_CMServiceTypeE | CMSVTYPE | '0010'B | CM Service Type for emergency call. |
| C_Telephony | IA5String | "C_Telephony" | telephony service (TS11) |
| C_EmgCall | IA5String | "C_EmgCall" | emergency call service (TS12) |
| C_AltSpchFax | IA5String | "C_AltSpchFax" | alternate speech and G3 fax service (TS61) |
| C_AutoFax | IA5String | "C_AutoFax" | automatic G3 fax service (TS62) |
| C_Async300 | IA5String | "C_Async300" | data circuit duplex async. 300 bit/s service (BS21) |
| C_Async1200 | IA5String | "C_Async1200" | data circuit duplex async. 1200 bit/s service (BS22) |
| C_Async120075 | IA5String | "C_Async120075" | data circuit duplex async. 1200/75 bit/s service (BS23) |
| C_Async2400 | IA5String | "C_Async2400" | data circuit duplex async. 2400 bit/s service (BS24) |
| C_Async4800 | IA5String | "C_Async4800" | data circuit duplex async. 4800 bit/s service (BS25) |
| C_Async9600 | IA5String | "C_Async9600" | data circuit duplex async 9600 bit/s service (BS26) |
| C_Sync1200 | IA5String | "C_Sync1200" | data circuit duplex sync. 1200 bit/s service (BS31) |
| C_Sync2400 | IA5String | "C_Sync2400" | data circuit duplex sync. 2400 bit/s service (BS32) |
| C_Sync4800 | IA5String | "C_Sync4800" | data circuit duplex sync. 4800 bit/s service (BS33) |
| C_Sync9600 | IA5String | "C_Sync9600" | data circuit duplex sync 9600 bit/s service (BS34) |
| C_PAD300 | IA5String | "C_PAD300" | PAD access 300 bit/s service (BS41) |
| C_PAD1200 | IA5String | "C_PAD1200" | PAD access 1200 bit/s service (BS42) |
| C_PAD120075 | IA5String | "C_PAD120075" | PAD access 1200/75 bit/s service (BS43) |
| C_PAD2400 | IA5String | "C_PAD2400" | PAD access 2400 bit/s service (BS44) |
| C_PAD4800 | IA5String | "C_PAD4800" | PAD access 4800 bit/s service (BS45) |
| C_PAD9600 | IA5String | "C_PAD9600" | PAD access 9600 bit/s service (BS46) |
| C_Packet2400 | IA5String | "C_Packet2400" | packt access 2400 bit/s service (BS51) |
| C_Packet4800 | IA5String | "C_Packet4800" | packt access 4800 bit/s service (BS52) |
| C_Packet9600 | IA5String | "C_Packet9600" | packt access 9600 bit/s service (BS53) |
| C_AltSpchData | IA5String | "C_AltSpchData" | alternate speech/data service (BS61) |
| C_SpchData | IA5String | "C_SpchData" | speech followed data service (BS81) |
| C_I | INTEGER | 1 | I command of L 2 |
| C_one | INTEGER | 1 | one frequency, no hopping |
| C_Immass | BITSTRING | '000'B | activation for immediate assignment |
| C_Ass | BITSTRING | '001'B | activation for assignment |
| C_Asynho | BITSTRING | '010'B | activation for non synchronous handover |
| C_Synho | BITSTRING | '011'B | activation for synchronous handover |
| C_Rcv | BITSTRING | '101'B | activation for receiving only |
| C_LocEndRel | RelMode | '01'O | local end release |
| C_MaxPwrLvlD | INTEGER | 15 | Max power level for MS DCS |
| C_MaxPwrLvlG | INTEGER | 19 | Max power level for MS GSM |
| C_rc_conditIEerror | REJCAU | '64'O | reject cause: Conditional IE error |
| C_rc_congestion | REJCAU | '16'O | reject cause: congestion |
| C_rc_illegal_ms | REJCAU | '03'O | reject cause: illegal MS |
| C_rc_illegal_me | REJCAU | '06'O | reject cause: illegal ME |
| C_rc_imsiunknownhlr | REJCAU | '02'O | reject cause: IMSI unknown in HLR |
| C_rc_imsiunknownvlr | REJCAU | '04'O | reject cause: IMSI unknown in VLR |
| C_rc_invalidmaninfo | REJCAU | '60'O | reject cause: invalid mandatory information |
| C_rc_LAnotallowed | REJCAU | '0C'O | reject cause: LocationArea is not allowed |
| C_rc_networkfailure | REJCAU | '11'O | reject cause: network failure |
| C_rc_notidentified | REJCAU | '26'O | reject cause: can not be identified |
| C_rc_plmn_not | REJCAU | '0B'O | reject cause: PLMN not allowed |
| C_rc_protocolerror | REJCAU | '6F'O | reject cause: Protocol error unspecified |
| C_rc_reqservoptnotsub | REJCAU | '21'O | reject cause: requested service option not subscribed |
| C_rc_roamingnot | REJCAU | '0D'O | reject cause: Roaming not allowed |
| C_Restablishment | B_1 | '0'B | Restablishment allowed |
| C_noRestablishment | B_1 | '1'B | No restablishment allowed |
| C_SlotNU | SN | '111'B | Slot Number not used |
| C_CellReselectHys12 | INTEGER | 6 | Cell Reselection Hysteresis value 12db |
| C_CellReselectHys4 | INTEGER | 2 | Cell Reselection Hysteresis value 4db |
| C_RI_alternate | B_8 | '11010001'B | Bcap repeat indicator for alternate (circular selection) |
| C_RI_follow | B_8 | '11010011'B | Bcap repeat indicator for successive (sequential selection) |
| C_SIMIn | BOOLEAN | FALSE | SIM is always inserted in a test case |
| C_SIMneedRmv | BOOLEAN | TRUE | Removal of SIM is needed in a test case (SIM is not always inserted.) |
| C_Yes | BOOLEAN | TRUE |  |
| C_No | BOOLEAN | FALSE |  |
| C_Start_Imsi | INTEGER | 1 | Test case body starts without TMSI |
| C_Start_Tmsi | INTEGER | 2 | Test case body starts with Tmsi_01 |
| C_Start_TmsiOff | INTEGER | 5 | Test case body starts with Tmsi_01. MS has been deactivated. |
| C_Start_Rej | INTEGER | 3 | Test case body starts with LAI deleted. MS has been deactivated. |
| C_Start_PLMNnot | INTEGER | 4 | Test case body starts with PLMN not allowed |
| C_Test_fh | INTEGER | 1 | Test case with possibly frequency hopping |
| C_Test_nfh | INTEGER | 2 | Test case with no frequency hopping |
| C_Test_egsm | INTEGER | 3 | Test case for egsm |
| C_Test_cbms | INTEGER | 4 | Test case for CBMS |
| C_normal_updating | B_2 | '00'B | normal location updating |
| C_periodic_updating | B_2 | '01'B | periodic location updating |
| C_normalOrperiodic | B_2 | '0?'B | periodic or normal location updating |
| C_imsi_attach | B_2 | '10'B | location updating lMSI attach |
| C_norm_period_attach | B_2 | '??'B | imsi attach or periodic location updating or normal location updating |
| C_normal_paging | B_2 | '00'B | normal page mode |
| C_extended_paging | B_2 | '01'B | extended paging |
| C_reorg_paging | B_2 | '10'B | paging reorganisation |
| C_sab_paging | B_2 | '11'B | same as before paging |
| C_non_synchronised | B_2 | '00'B | non-synchronised |
| C_synchronised | B_2 | '01'B | synchronised |
| C_pre_synchronised | B_2 | '10'B | pre-synchronised |
| C_pseudo_synchronised | B_2 | '11'B | pseudo-synchronised |
| C_not_report_otd | B_1 | '0'B | not report observed time difference |
| C_report_otd | B_1 | '1'B | report observed time difference |
| C_Tzone0 | TZONES | 0 | Time zone 0 (used in TC_34_2_1, TC_34_2_5_3) |
| C_Tzone1 | TZONES | 4 | Time zone 4 (used in TC_34_2_2, TC_34_2_7) |
| C_Tzone2 | TZONES | 10 | Time zone 10 (used in TC_34_2_3) |
| C_Tzone3 | TZONES | 15 | Time zone 15 (used in TC_34_2_4) |
| C_Tzone4 | TZONES | 25 | Time zone 25 (used in TC_34_2_5_1) |
| C_Tzone5 | TZONES | 35 | Time zone 35 (used in TC_34_2_5_2) |
| C_Tzone6 | TZONES | 45 | Time zone 45 (used in TC_34_2_8) |
| C_T_AssCmd | INTEGER | 900 | 600ms + 300ms(tolerance) |
| C_T_Wait1stChReq | INTEGER | 35000 | max time for full BCCH data decoding |
| C_T_mrsrp | INTEGER | 1500 | 3 times time btw 2 msr rpt. |
| C_T_T3124 | INTEGER | 675 | T3124(675ms) |
| C_T_T3211plus | INTEGER | 20000 | 20s |
| C_T_T3212 | INTEGER | 360000 | T3212(6min) |
| C_T_T3212min | INTEGER | 345000 | T3212(6min) - 15sec |
| C_T_T3212dif | INTEGER | 15000 | Difference between T3212 and T3212min |
| C_T_T3210 | INTEGER | 20000 | timer T3210 in ms |
| C_T_T3211min | INTEGER | 13500 | 90% T3211 in ms |
| C_T_T3230min | INTEGER | 13500 | 90% T3230 in ms |
| C_T_T3240min | INTEGER | 9000 | 90% T3240 in ms |
| C_T_T3240tol | INTEGER | 2000 | 20% T3240 in ms |
| C_T_Wait | INTEGER | 5000 | 5s. T3126max |
| C_T_Wait_FAC | INTEGER | 1500 | 1.5s Facility response time (1s) + tolerance (0.5s) |
| C_release_time | INTEGER | 11 | release timer value |
| C_RegCFNRy | INTEGER | 1 | registration of call forwarding service for CFNRy (speech) |
| C_RegCFU | INTEGER | 2 | registration of call forwarding service for CFU (all facsimile) |
| C_RegCF | INTEGER | 3 | registration of call forwarding service for CF (all facsimile) |
| C_ErsCFC | INTEGER | 4 | erasure of call forwarding service for CFC (all facsimile) |
| C_ErsCFNRc | INTEGER | 5 | erasure of call forwarding service for CFNRc (all basic services) |
| C_ErsCFNRy | INTEGER | 6 | erasure of call forwarding service for CFNRy (all facsimile) |
| C_ActCF | INTEGER | 7 | activation of call forwarding service for CF (all synchronous services) |
| C_ActCFU | INTEGER | 8 | activation of call forwarding service for CFU (all basic services) |
| C_DeactCFC | INTEGER | 9 | deactivation of call forwarding service for CFC (speech) |
| C_DeactCFNRc | INTEGER | 10 | deactivation of call forwarding service for CFNRc (all facsimile) |
| C_InterrogCFC | INTEGER | 11 | interrogation of call forwarding service for CFC (Speech) |
| C_InterrogCFB | INTEGER | 12 | interrogation of call forwarding service for CFB (all basic services) |
| C_NotifyCFB | INTEGER | 13 | notification of call forwarding service for CFB (incoming call is forwarded) |
| C_NotifyCFU | INTEGER | 14 | notification of call forwarding service for CFU (provisioned, registered, active) |
| C_NotifyCFC | INTEGER | 15 | notification of call forwarding service for CFC (provisioned, registered, active) |
| C_NotifyCFNRc | INTEGER | 16 | notification of call forwarding service for CFNRc (Ms not reachable) |
| C_RegPswd | INTEGER | 17 | Registration of password for all call barring services |
| C_ActBOAC | INTEGER | 18 | Activation of BOAC |
| C_ActBICRoam | INTEGER | 19 | Activation of BICRoam |
| C_ActBOIC | INTEGER | 20 | Activation of BOIC |
| C_ActBAIC | INTEGER | 21 | Activation of BAIC |
| C_DeactBO | INTEGER | 22 | Deactivation of BO |
| C_DeactBI | INTEGER | 23 | Deactivation of BI |
| C_DeactBOICExHC | INTEGER | 24 | Deactivation of BOICExHC |
| C_NotifyBI | INTEGER | 25 | Notify of BI |
| C_InterrogBOIC | INTEGER | 26 | Interrogation of BOIC |
| C_InterrogBOICExHC | INTEGER | 27 | Interrogation of BOICExHC |
| C_Full | IA5String | "F" | full rate channel |
| C_Half | IA5String | "H" | half rate channel |
| C_nirr_dontcare | B_1 | '?'B | NIRR no meaning or requested |
| C_Rchr_dontcare | B_2 | '??'B | radio channel requirement not checked |
| C_Rchr_Full | B_2 | '01'B | radio channel requirement : full rate |
| C_Rchr_DualForDualH | B_2 | '1?'B | radio channel requirement : dual rate support/half rate preferred or dual rate support/full rate preferred |
| C_StartingTimeHO | INTEGER | 238 | Starting time 238 (1.1s), in TC_26_6_5_1, TC_26_6_5_4_2 |
| C_StrT_1000 | INTEGER | 1000 | Starting time 1000, in TC_26_6_13_1 |
| C_Norm | OCTETSTRING | '00'O | Establish mode: normal |
| C_Speech | B_3 | '000'B | Info. Transfer Cap. speech |
| C_3100Hz | B_3 | '010'B | Info. Transfer Cap. 3.1 kHz |
| C_UDI | B_3 | '001'B | Info. Transfer Cap. UDI |
| C_FAX3 | B_3 | '011'B | Info. Transfer Cap. FAX3 |
| C_SDUintegrity | B_2 | '00'B | Structure SDU integrity |
| C_Unstructured | B_2 | '11'B | Structure Unstructured |
| C_struc_dontchk | B_2 | '??'B | Structure not checked |
| C_nirr_nomeaning | B_1 | '0'B | NIRR no meaning |
| C_no_rate_adaption | B_2 | '00'B | Rate adaption no |
| C_rate_adaption_V110 | B_2 | '01'B | Rate adaption V110 |
| C_rate_adaption_X31 | B_2 | '10'B | Rate adaption X31 flag |
| C_sacp_NA | B_3 | '001'B | Signaling access NA (sec.11.8.1.5.1) |
| C_I440_450 | B_3 | '001'B | Signaling access I440/450 |
| C_X21 | B_3 | '010'B | Signaling access X21 |
| C_X28_unui | B_3 | '100'B | Signaling access X28 U NUI |
| C_X28_nond | B_3 | '101'B | Signaling access X28 non dedicated |
| C_X32 | B_3 | '110'B | Signaling access X32 |
| C_Synchronous | B_1 | '0'B | Synchronous |
| C_Asynchronous | B_1 | '1'B | Asynchronous |
| C_stopbit_NA | B_1 | '0'B | Number of Stop bits NA |
| C_stopbit_dontcare | B_1 | '?'B | Number of Stop bits not care |
| C_databit_NA | B_1 | '1'B | Number of Data bits not care |
| C_databit_dontcare | B_1 | '?'B | Number of Data bits NA |
| C_300bs | B_4 | '0001'B | User rate 300 bs |
| C_1200bs | B_4 | '0010'B | User rate 1200 bs |
| C_120075bs | B_4 | '0111'B | User rate 1200/75 bs |
| C_2400bs | B_4 | '0011'B | User rate 2400 bs |
| C_4800bs | B_4 | '0100'B | User rate 4800 bs |
| C_9600bs | B_4 | '0101'B | User rate 9600 bs |
| C_parity_NA | B_3 | '011'B | Parity NA |
| C_parity_dontcare | B_3 | '???'B | Parity not care |
| C_ir_8kbs | B_2 | '10'B | Intermediate rate 8 Kbs |
| C_ir_16kbs | B_2 | '11'B | Intermediate rate 16 Kbs |
| C_ir_8or16kbs | B_2 | '1?'B | Intermediate rate 8 or 16 Kbs |
| C_transparent | B_2 | '00'B | Connection element T |
| C_notransparent | B_2 | '01'B | Connection element NT |
| C_BothT | B_2 | '10'B | Connection element both T |
| C_BothNT | B_2 | '11'B | Connection element both NT |
| C_modemt_none | B_5 | '00000'B | no modem |
| C_modemt_V21 | B_5 | '00001'B | modem V21 |
| C_modemt_V22 | B_5 | '00010'B | modem V22 |
| C_modemt_V22bis | B_5 | '00011'B | modem V22bis |
| C_modemt_V23 | B_5 | '00100'B | modem V23 |
| C_modemt_V32 | B_5 | '00110'B | modem V32 |
| C_ISO6429 | B_5 | '01000'B | Inband flow control |
| C_COPnoFLCT | B_5 | '01100'B | No flow control |
| C_X25_flct | B_5 | '00110'B | X25 flow control |
| C_Outband | FLWCNTL | 0 | outband flow control |
| C_Inband | FLWCNTL | 1 | outband flow control |
| C_Nocontrol | FLWCNTL | 2 | outband flow control |
| C_uil2p_dontcare | B_5 | '?????'B | not checked |
| C_HSN_0 | INTEGER | 0 | cyclic hopping |
| maxAddressLength | Asn1Integer | 20 |  |
| maxISDN_AddressLength | Asn1Integer | 9 |  |
| maxISDN_SubaddressLength | Asn1Integer | 21 |  |
| maxNumOfBasicServiceGroups | Asn1Integer | 13 |  |
| maxNumOfCUG | Asn1Integer | 10 |  |
| maxNumberOfSegmentsPerDataInterval | Asn1Integer | 8191 |  |
| maxSignalInfoLength | Asn1Integer | 200 |  |
| maxUSSD_StringLength | Asn1Integer | 160 |  |
| max10TimesIncrement | Asn1Integer | 8191 |  |
| max10TimesIncrementPerDataInterval | Asn1Integer | 8191 |  |
| max10TimesInitialTime | Asn1Integer | 8191 |  |
| max100TimesScalingFactor | Asn1Integer | 8191 |  |
| max10TimesTimeInterval | Asn1Integer | 8191 |  |
| max10TimesUnitsPerTime | Asn1Integer | 8191 |  |
| Detailed Comments: |  |  |  |

## Test case variable declarations

| Test Case Variable Declarations |  |  |  |
| --- | --- | --- | --- |
| Variable Name | Type | Value | Comments |
| TCV_CC | BOOLEAN | TSPC_Serv_TS11 OR TSPC_Serv_TS12 OR TSPC_Serv_TS61 OR TSPC_Serv_TS62 OR TSPC_Serv_BS21 OR TSPC_Serv_BS22 OR TSPC_Serv_BS23 OR TSPC_Serv_BS24 OR TSPC_Serv_BS25 OR TSPC_Serv_BS26 OR TSPC_Serv_BS31 OR TSPC_Serv_BS32 OR TSPC_Serv_BS33 OR TSPC_Serv_BS34 OR TSPC_Serv_BS41 OR TSPC_Serv_BS42 OR TSPC_Serv_BS43 OR TSPC_Serv_BS44 OR TSPC_Serv_BS45 OR TSPC_Serv_BS46 OR TSPC_Serv_BS51 OR TSPC_Serv_BS52 OR TSPC_Serv_BS53 OR TSPC_Serv_BS61 OR TSPC_Serv_BS81 |  |
| TCV_AssCmd | ASS_CMD_PDU |  | to hold ASSIGNMENT COMMAND PDU |
| TCV_Bcap1 | BCAP |  | to hold bearer capability |
| TCV_BcapMT1 | BCAP |  | to hold BC for MT call |
| TCV_BcapMO1 | BCAP |  | to hold BC for MO call |
| TCV_Bcap2 | BCAP |  | to hold bearer capability |
| TCV_BcapMT2 | BCAP |  | to hold BC for MT call |
| TCV_bs_ag_blks_res | INTEGER |  |  |
| TCV_bs_cc_chans | INTEGER |  |  |
| TCV_bs_pa_mfrms | INTEGER |  |  |
| TCV_CalledNum | CDPN |  | to hold called party number |
| TCV_CallProc | CALL_PROC_PDU |  | to hold CALL PROCEEDING PDU |
| TCV_CallCfm | CALL_CO_PDU |  | to hold CALL COMFIRM PDU |
| TCV_Cau | CAU |  | to hold cause |
| TCV_Cau0 | CAU |  | to hold cause |
| TCV_CAgsm | CCHD |  | To compute Cell Channel Descriptor for gsm |
| TCV_CAdcs | CCHD |  | To compute Cell Channel Descriptor for dcs |
| TCV_CB1 | OCTETSTRING |  | store SMSCB block data |
| TCV_CB2 | OCTETSTRING |  | store SMSCB block data |
| TCV_CB3 | OCTETSTRING |  | store SMSCB block data |
| TCV_CB4 | OCTETSTRING |  | store SMSCB block data |
| TCV_CE | CE |  |  |
| TCV_Ccchg | INTEGER |  | CCCH_GROUP |
| TCV_Ccd0A | CCD |  | a copy of control channel description in use for cell A |
| TCV_Ccd0B | CCD |  | a copy of control channel description in use for cell B |
| TCV_Ccd0C | CCD |  | a copy of control channel description in use for cell C |
| TCV_Ccd0D | CCD |  | a copy of control channel description in use for cell D |
| TCV_Ccd0E | CCD |  | a copy of control channel description in use for cell E |
| TCV_Ccd0F | CCD |  | a copy of control channel description in use for cell F |
| TCV_Ccd0G | CCD |  | a copy of control channel description in use for cell G |
| TCV_Ccd0H | CCD |  | a copy of control channel description in use for cell H |
| TCV_cchdescr | OCTETSTRING |  | Frequency list of Cell Channel Descr, used in EGSM |
| TCV_CCst | CST |  | to hold call status |
| TCV_ch | LOGICCH | "dummy" | to hold logic channel |
| TCV_ch1 | LOGICCH | "dummy" | to hold logic channel |
| TCV_sacch | LOGICCH | "dummy" | to hold logic channel |
| TCV_sacch_B | LOGICCH | "dummy" | to hold logic channel |
| TCV_sacch_C | LOGICCH | "dummy" | to hold logic channel |
| TCV_sacch_D | LOGICCH | "dummy" | to hold logic channel |
| TCV_sacch_E | LOGICCH | "dummy" | to hold logic channel |
| TCV_sacch_F | LOGICCH | "dummy" | to hold logic channel |
| TCV_sacch_G | LOGICCH | "dummy" | to hold logic channel |
| TCV_sacch_H | LOGICCH | "dummy" | to hold logic channel |
| TCV_sacch8 | LOGICCH | "dummy" | to hold logic channel |
| TCV_sacchTch | LOGICCH | "dummy" | to hold logic channel |
| TCV_sacchTch1 | LOGICCH | "dummy" | to hold logic channel |
| TCV_sacchTch_H | LOGICCH | "dummy" | to hold logic channel |
| TCV_chTch | LOGICCH | "dummy" | to hold logic channel |
| TCV_chTch1 | LOGICCH | "dummy" | to hold logic channel |
| TCV_chSms | LOGICCH | "dummy" | to hold logic channel |
| TCV_agch | LOGICCH | "dummy" | downlink access grant channel |
| TCV_WorkingCh | LOGICCH | "dummy" | to hold logic channel |
| TCV_cellid | CellID |  | to hold the cell identifier |
| TCV_Char | IA5String |  | to hold the IA5 char. corresponding to a DTMF tone |
| TCV_chd1 | CHD |  | to hold channel description IE |
| TCV_cchd1 | CCHD |  | to hold cell channel description IE |
| TCV_ChMod | CHMOD |  | to hold channel mode IE |
| TCV_ChModb | CHMOD |  | to hold channel mode IE for second mode in dual mode services |
| TCV_Tchtype | CH_TDMA | '00001'B | Traffic channel type and TDMA offset in channel description IE |
| TCV_chtype | CH_TDMA |  | to channel type and TDMA offset in channel description IE |
| TCV_cksn | BITSTRING |  | to hold the CKSN. |
| TCV_Cnt | INTEGER |  | general loop counter |
| TCV_Cnt1 | INTEGER |  | general loop counter |
| TCV_Cnt2 | INTEGER |  | general loop counter |
| TCV_Cntstart | BOOLEAN | FALSE | general loop counter start decision |
| TCV_Cntref | INTEGER |  | Reference for general loop counter |
| TCV_CphAlg | BITSTRING |  | ciphering algorithm |
| TCV_CphKey | BITSTRING |  | ciphering key |
| TCV_CphMd | CPHMS |  | ciphering mode setting |
| TCV_CPDataRetx | INTEGER |  | Number of CP Data retransmissions for SMS |
| TCV_ChRate | IA5String | C_Full | to hold channel rate |
| TCV_ChRate1 | IA5String | C_Full | to hold channel rate |
| TCV_counter_c | INTEGER |  | loop counter c |
| TCV_counter_k | INTEGER |  | loop counter k |
| TCV_Fk | INTEGER |  | to hold an interval between events |
| TCV_flist | OCTETSTRING |  | Frequency List, used in EGSM for IE "frequency list" or "frequency short list" |
| TCV_flistl | LENGTH |  | Length of Frequency List, used in EGSM for IE "frequency list" or "frequency short list" |
| TCV_Fn | FN |  | to hold the frame number (T1' , T2, T3) |
| TCV_Fn1 | FN |  | to hold the frame number (T1' , T2, T3) |
| TCV_FnAss | FN |  | to hold the frame number in AssCmd statement |
| TCV_FollowingOctets | OCTETSTRING |  | Contains the octets following the invoke id |
| TCV_Horf | HORF |  | Variable for HO reference in HO-Messages of HO cases. |
| TCV_HoCMD | HO_CMD_PDU |  | to hold handover PDU |
| TCV_Hrf | HORF |  | to hold handover reference |
| TCV_iel | LENGTH |  | Length of IE |
| TCV_kcnt | INTEGER |  | loop counter |
| TCV_K | INTEGER |  |  |
| TCV_L1Head0 | L1HD |  | to hold L 1 header |
| TCV_L1Head | L1HD |  | to hold L 1 header |
| TCV_lac | OCTETSTRING |  | to hold the lac. 2 octets |
| TCV_n | INTEGER |  |  |
| TCV_Neci | BITSTRING |  |  |
| TCV_M | INTEGER |  | counter |
| TCV_maio | MAIO |  | to hold MAIO |
| TCV_mae1 | BITSTRING |  | Mobile Allocation in EGSM. It includes the 1.octet of MA |
| TCV_mae2 | BITSTRING |  | Mobile Allocation in EGSM. It includes the 2.octet of MA |
| TCV_Max | BITSTRING |  | to hold Max-retrans bitstring |
| TCV_MemCapExcd | BOOLEAN |  | RP error Memory Capacity Exceeded was sent |
| TCV_MsrRes | MSRR |  | to hold measurement results |
| TCV_Mt | BITSTRING |  | to hold message type |
| TCV_Mt1 | BITSTRING |  | to hold message type |
| TCV_Modify | MODIFY_PDU |  | To hold Modify message |
| TCV_Null | BOOLEAN |  | to collect useless result from some test suite operations |
| TCV_PgCh | LOGICCH | "dummy" | logical channel for paging |
| TCV_Pgg | PGG |  | paging group |
| TCV_PreviousOctets | OCTETSTRING |  | Contains the octets previous to the invoke id |
| TCV_ProtErrorUnspec | BOOLEAN |  | RP error Protocol Error Unspecified was sent |
| TCV_Pwrlvl | BITSTRING |  | to hold the power level |
| TCV_Pwrlvl_ho | BITSTRING |  | power level for HO_CMD, used in HO cases. |
| TCV_Rchr | B_2 |  | to hold Radio channel requirement |
| TCV_Res | BOOLEAN |  | to hold the result of a test suite operation |
| TCV_Rpmr | MR |  | RP message reference for SMS |
| TCV_Rqr | RQR |  | to hold the request reference |
| TCV_Rqr9 | RQR |  | to hold the request reference differing from TCV_Rqr |
| TCV_Rqr10 | RQR |  | to hold the request reference differing from TCV_Rqr, TCV_Rqr9 |
| TCV_Rqr11 | RQR |  | to hold the request reference differing from TCV_Rqr, TCV_Rqr9, TCV_Rqr10 |
| TCV_Rr | BITSTRING |  | to hold the random request reference |
| TCV_Rr1 | BITSTRING |  | to hold the random reference |
| TCV_Service | SERVICES |  | to hold basicservice selected |
| TCV_Service1 | SERVICES |  | to hold basicservice selected |
| TCV_Setup_mo | SETUP_MO_PDU |  | to hold the SETUP PDU (MO) |
| TCV_Setup_mo1 | SETUP_MO_PDU |  | to hold the SETUP PDU (MO) |
| TCV_Setup_mt | SETUP_MT_PDU |  | to hold the SETUP PDU (MT) |
| TCV_Setup_mt1 | SETUP_MT_PDU |  | to hold the 2nd SETUP PDU (MT) |
| TCV_2ndtest | BOOLEAN |  | 2nd test required in TC_11_1_1 |
| TCV_Esetup | ESETUP_PDU |  | to hold the emergency SETUP PDU |
| TCV_Esetup1 | ESETUP_PDU |  | to hold the emergency SETUP PDU |
| TCV_Ecall | BOOLEAN | FALSE | it is set to TRUE, if the service under test is emergency call |
| TCV_slot | SN |  | To hold the default slot used during the entire TC. |
| TCV_slt2 | SN |  | To hold a second working slot number. |
| TCV_Sres | OCTETSTRING |  | to hold the SRES returned |
| TCV_sysinf5 | SYSINFO5_PDU |  | to hold the SysInfo5 PDU |
| TCV_sysinfo5 | SYSINFO5_PDU |  | to hold the SysInfo5 PDU for cell A |
| TCV_sysinfo5bis | SYSINFO5bis_PDU |  | to hold the SysInfo5bis PDU for cell A |
| TCV_sysinfo5_B | SYSINFO5_PDU |  | to hold the SysInfo5 PDU for cell B |
| TCV_sysinfo5_C | SYSINFO5_PDU |  | to hold the SysInfo5 PDU for cell c |
| TCV_sysinfo5_D | SYSINFO5_PDU |  | to hold the SysInfo5 PDU for cell D |
| TCV_sysinfo5_E | SYSINFO5_PDU |  | to hold the SysInfo5 PDU for cell E |
| TCV_sysinfo5_F | SYSINFO5_PDU |  | to hold the SysInfo5 PDU for cell F |
| TCV_sysinfo5_G | SYSINFO5_PDU |  | to hold the SysInfo5 PDU for cell G |
| TCV_sysinfo5_H | SYSINFO5_PDU |  | to hold the SysInfo5 PDU for cell H |
| TCV_sysinfo5bis_A | SYSINFO5bis_PDU |  | to hold the SysInfo5bis PDU for cell A(TC_26_10_2_4_1) |
| TCV_sysinfo5bis_B | SYSINFO5bis_PDU |  | to hold the SysInfo5bis PDU for cell B |
| TCV_sysinf6 | SYSINFO6_PDU |  | to hold the SysInfo6 PDU |
| TCV_sysinfo6 | SYSINFO6_PDU |  | to hold the SysInfo6 PDU for cell A |
| TCV_sysinfo6_B | SYSINFO6_PDU |  | to hold the SysInfo6 PDU for cell B |
| TCV_sysinfo6_C | SYSINFO6_PDU |  | to hold the SysInfo6 PDU for cell C |
| TCV_sysinfo6_D | SYSINFO6_PDU |  | to hold the SysInfo6 PDU for cell D |
| TCV_sysinfo6_E | SYSINFO6_PDU |  | to hold the SysInfo6 PDU for cell E |
| TCV_sysinfo6_F | SYSINFO6_PDU |  | to hold the SysInfo6 PDU for cell F |
| TCV_sysinfo6_G | SYSINFO6_PDU |  | to hold the SysInfo6 PDU for cell G |
| TCV_sysinfo6_H | SYSINFO6_PDU |  | to hold the SysInfo6 PDU for cell H |
| TCV_ti_v | TI_V |  | to hold the transaction id. value |
| TCV_ti_v_2 | TI_V |  | to hold the transaction id. value for another transaction |
| TCV_T | INTEGER |  | to hold Tx-integer chosen randomly |
| TCV_T_TwiceTC1M | INTEGER | TSPX_TC1M * 2000 | Twice TC1M in ms |
| TCV_chdescr_arfcn | INTEGER |  | ARFCN carrier of actual cell |
| TCV_tch_arfcn | INTEGER |  | ARFCN carrier for traffic channel |
| TCV_asscmd_ts | BITSTRING |  | Time slot for channel description in assignment cmd. |
| TCV_ts | BITSTRING |  | Time slot for channel description in handover cmd. |
| TCV_ia_ts | BITSTRING |  | Time slot for channel description in immediate assignment |
| TCV_tsc | TSC |  | To hold the default training sequence used during the entire TC. |
| TCV_Td | OCTETSTRING |  | to hold timing difference |
| TCV_Time | INTEGER |  | to hold the measured value of T310 or T305 or T308 |
| TCV_TI | TI |  | to hold the transaction ID for MO calls in sending evets |
| TCV_TI0 | TI |  | to hold the transaction ID for MO calls in receiving evets |
| TCV_TI1 | TI |  | to hold the transaction ID |
| TCV_TI2 | TI |  | to hold the transaction ID |
| TCV_TI3 | TI |  | to hold the transaction ID |
| TCV_tmp | INTEGER |  | temporary integer variable |
| TCV_Tx | BITSTRING |  | to hold Tx bitstring |
| TCV_S | INTEGER |  | to hold the S parameter |
| TCV_Upd | BOOLEAN |  | to hold the information whether the MS is updated or not,. |
| TCV_UssdString | IA5String |  | String for USS Data |
| TCV_UssdString1 | IA5String |  | String for USS Data |
| TCV_UssdString2 | IA5String |  | String for USS Data |
| TCV_InvkId | OCTETSTRING |  | to hold SS transaction ID |
| TCV_InvkId0 | OCTETSTRING |  | to hold SS transaction ID |
| TCV_InvkId1 | OCTETSTRING |  | to hold SS transaction ID |
| TCV_Comp | Components |  | to hold SS Components |
| TCV_Strt | STRT |  | to hold starting time |
| TCV_CBch | LOGICCH | "dummy" | Cell Broadcast channel |
| TCV_Tpmr | MR |  | TP message reference |
| TCV_TPOA1 | BCDN |  | TP originating address digits |
| TCV_TPOA2 | BCDN |  | TP originating address digits |
| TCV_TPDA | BCDN |  | TP destination address digits |
| TCV_RPOA_MT | BCDN |  | RP originating address digits for MT short messages |
| TCV_RPOA_MO | BCDN |  | RP originating address digits for MO short messages |
| TCV_RPOA1 | BCDN |  | RP originating address digits for MT short messages |
| TCV_RPOA2 | BCDN |  | RP originating address digits for MT short messages |
| TCV_RPDA_MT | BCDN |  | RP destination address digits for MT short messages |
| TCV_RPDA_MO | BCDN |  | RP destination address digits for MO short messages |
| TCV_SMTypeM | INTEGER |  | Replace short message type |
| TCV_SMTypeN | INTEGER |  | Replace short message type |
| TCV_SMcntns | OCTETSTRING |  | short message contents |
| TCV_SMSCBpack | SMSCBpack |  | CB message contents |
| TCV_freq | FRQPARA |  | to hold Freq type constraints |
| TCV_srv | SERVICES |  | to hold MT service |
| TCV_ur | B_4 |  | to hold user rate |
| TCV_itc | B_3 |  | to hold information transfer capability |
| TCV_sacp | B_3 |  | to hold service access protocol |
| TCV_ce | B_2 |  | to hold connection element |
| TCV_ce1 | B_2 |  | to hold connection element |
| TCV_ce3 | B_2 |  | to hold connection element |
| TCV_sa | B_1 |  | to hold synchronous/asynchronous |
| TCV_ir | B_2 |  | to hold intermediate rate |
| TCV_uil1p | B_2 |  | to hold user information layer 1 protocol |
| TCV_modemt | B_5 |  | to hold modem type |
| TCV_MODEM | MODEMTYPE |  |  |
| TCV_ur2 | B_4 |  | to hold user rate for second configuration |
| TCV_itc2 | B_3 |  | to hold information transfer capability for second configuration |
| TCV_sacp2 | B_3 |  | to hold service access protocol for second configuration |
| TCV_strc | B_2 |  | to hold structure |
| TCV_sa2 | B_1 |  | to hold synchronous/asynchronous for second configuration |
| TCV_supported | BOOLEAN | FALSE | used for indicating a selected service is declared supported or not |
| TCV_SpecialCase | BOOLEAN | FALSE | used for indicating whether a test case tests non-supported service |
| Detailed Comments: |  |  |  |

## PCO declarations

| PCO Declarations |  |  |  |
| --- | --- | --- | --- |
| PCO Name | Type | Role | Comments |
| L | SAP0_3 | LT |  |
| Detailed Comments: | 1. The PCO consists of multiple SAPs: the SAP0 and SAP3. 2. The lower tester (LT) is the user of the data link layer service. 3. The SAP0 at the lower tester controlling and observing the exchange of CC, MM, RR and SS PDUs (messages) on the DCCH, SACCH and/or RACH, BCCH, CCCH channels. 4. The SAP3 at the lower tester controlling and observing the exchange of SMS PDUs (messages) on the DCCH or SACCH channels. |  |  |

## Timer declarations

| Timer Declarations |  |  |  |
| --- | --- | --- | --- |
| Timer Name | Duration | Unit | Comments |
| T_dly |  | ms | general purpose delay timer |
| T_dly1 |  | ms | general purpose delay timer |
| T_dly2 |  | ms | general purpose delay timer |
| T_dlyAss | 5000 | ms | AssCmd timer, 1) |
| T_guard | 300 | s | guard timer |
| T_release | C_release_time | s | release timer |
| Detailed Comments: | 1) In order to give to the lower tester enough time to dequeue and to send out the call SETUP and the other call establishment related messages in the sending buffer before starting sending of ASSINGMENT COMMAND message, the default value 5s is used in the test step AssCh_complete. In the test step AssCh_failure the timer value is set to 4s. For the TC_26_6_4_1, 600ms is used. |  |  |

## ASP type definitions

### TTCN ASP Type definitions

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_EstRq (DL_ESTABLISH_REQUEST) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the establishment of multiple frame operation (L3 -> L2). The normal establishment procedure is initiated. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| establish_mode |  | EstMode |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_EstIn (DL_ESTABLISH_INDICATION) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the establishment of multiple frame operation (L2 -> L3). The normal establishment procedure has been initiated. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| establish_mode |  | EstMode |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_EstInCmsRq (DL_ESTABLISH_INDICATION_CM_SERVICE_REQUEST) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the establishment of multiple frame operation due to the receipt of the MM CM SERVICE REQUEST message (L2 -> L3). The contention resolution establishment procedure has been initiated. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| establish_mode |  | EstMode |  |
| msg |  | CMS_RQ_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_EstInLupRq (DL_ESTABLISH_INDICATION_LOCATION_UPDATING_REQUEST) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the establishment of multiple frame operation due to the receipt of the MM LOCATION UPDATING REQUEST message (L2 -> L3). The contention resolution establishment procedure has been initiated. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| establish_mode |  | EstMode |  |
| msg |  | LUP_RQ_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_EstInImsidIn (DL_ESTABLISH_INDICATION_IMSI_DETACH_INDICATION) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the establishment of multiple frame operation due to the receipt of the MM IMSI DETACH INDICATION message (L2 -> L3). The contention resolution establishment procedure has been initiated. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| establish_mode |  | EstMode |  |
| msg |  | IMSID_IN_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_EstInPgRes (DL_ESTABLISH_INDICATION_PAGING_RESPONSE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the establishment of multiple frame operation due to the receipt of the RR PAGING RESPONSE message (L2 -> L3). The contention resolution establishment procedure has been initiated. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| establish_mode |  | EstMode |  |
| msg |  | PG_RES_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_EstInCmreRq (DL_ESTABLISH_INDICATION_CM_REESTABLISHMENT_REQUEST) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the establishment of multiple frame operation due to the receipt of the MM CM REESTABLISHMENT REQUEST message (L2 -> L3). The contention resolution establishment procedure has been initiated. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| establish_mode |  | EstMode |  |
| msg |  | CMRE_RQ_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_EstCo (DL_ESTABLISH_CONFIRM) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used by the L2 to inform the L3 about the establishment of multiple frame link (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| establish_mode |  | EstMode |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_RelIn (DL_RELEASE_INDICATION) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the termination of an established multiple frame operation or to report an unsuccessful establishment attempt (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| release_mode |  | RelMode |  |
| outstanding_indicator |  | BOOLEAN |  |
| fn |  | FN |  |
| Detailed Comments: | The outstanding indicator indicates whether or not there are outstanding acknowledgements or unsolved DL_DATA_REQUEST primitives. |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_RacInChRq (DL_RANDOM_ACCESS_INDICATION_CHANNEL_REQUEST) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the arrival of an RR CHANNEL REQUEST message (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| fn |  | FN |  |
| msg |  | CH_RQ_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_RacInHoacc (DL_RADOM_ACCESS_INDICATION_HANDOVER_ACCESS) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the arrival of an RR HANDOVER ACCESS message (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | HOACC_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatRqImmass (DL_UNIT_DATA_REQUEST_IMMEDIATE_ASSIGNMENT) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR IMMEDIATE ASSIGNMENT message using unacknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | IMMASS_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatRqImmass_sp (DL_UNIT_DATA_REQUEST_IMMEDIATE_ASSIGNMENT) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR IMMEDIATE ASSIGNMENT message using unacknowledged operation (L3 -> L2) on the MS paging channel. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| pgg |  | PGG |  |
| msg |  | IMMASS_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatRqImmassx (DL_UNIT_DATA_REQUEST_IMMEDIATE_ASSIGNMENT_EXTENDED) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR IMMEDIATE ASSIGNMENT EXTENDED message using unacknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | IMMASSX_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatRqImmassx_sp (DL_UNIT_DATA_REQUEST_IMMEDIATE_ASSIGNMENT_EXTENDED) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR IMMEDIATE ASSIGNMENT EXTENDED message using unacknowledged operation (L3 -> L2) on the MS paging channel. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| pgg |  | PGG |  |
| msg |  | IMMASSX_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatRqImmassRej (DL_UNIT_DATA_REQUEST_IMMEDIATE_ASSIGNMENT_REJECT) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR IMMEDIATE ASSIGNMENT REJECT message using unacknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | IMMASS_REJ_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatRqImmassRej_sp (DL_UNIT_DATA_REQUEST_IMMEDIATE_ASSIGNMENT_REJECT) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR IMMEDIATE ASSIGNMENT REJECT message using unacknowledged operation (L3 -> L2) on the MS paging channel. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| pgg |  | PGG |  |
| msg |  | IMMASS_REJ_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatInMsrRpt (DL_DATA_INDICATION_MEASUREMENT_REPORT) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the RR MEASUREMENT REPORT message using unacknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | MSR_RPT_PDU |  |
| fn |  | FN |  |
| Detailed Comments: | The ASPs are continuously received during the testing. |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatRqSMSCBData (DL_UNIT_DATA_REQUEST_SMSCB_DATA) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the SMSCB data using unacknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | SMSCB_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatRqPg1Rq (DL_UNIT_DATA_REQUEST_PAGING_REQUEST_TYPE1) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR PAGING REQUEST TYPE 1 message using unacknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| pgg |  | PGG |  |
| msg |  | PG1_RQ_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatRqPg2Rq (DL_UNIT_DATA_REQUEST_PAGING_REQUEST_TYPE2) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR PAGING REQUEST TYPE 2 message using unacknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| pgg |  | PGG |  |
| msg |  | PG2_RQ_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatRqPg3Rq (DL_UNIT_DATA_REQUEST_PAGING_REQUEST_TYPE3) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR PAGING REQUEST TYPE 3 message using unacknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| pgg |  | PGG |  |
| msg |  | PG3_RQ_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatRqSchinfo (DL_UNIT_DATA_REQUEST_SYNCHRONIZATION_CHANNEL_INFORMATION) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR SYNCHRONIZATION CHANNEL INFORMATION message using unacknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | SCHINFO_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatRqSysinfo1 (DL_UNIT_DATA_REQUEST_SYSTEM_INFORMATION_TYPE1) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR SYSTEM INFORMATION TYPE 1 message using unacknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | SYSINFO1_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatRqSysinfo1_nh (DL_UNIT_DATA_REQUEST_SYSTEM_INFORMATION_TYPE1) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR SYSTEM INFORMATION TYPE 3 message in case of non hopping |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | SYSINFO3_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatRqSysinfo2 (DL_UNIT_DATA_REQUEST_SYSTEM_INFORMATION_TYPE2) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR SYSTEM INFORMATION TYPE 2 message using unacknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | SYSINFO2_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatRqSysinfo2bis (DL_UNIT_DATA_REQUEST_SYSTEM_INFORMATION_TYPE2bis) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR SYSTEM INFORMATION TYPE 2bis message using unacknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | SYSINFO2bis_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatRqSysinfo3 (DL_UNIT_DATA_REQUEST_SYSTEM_INFORMATION_TYPE3) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR SYSTEM INFORMATION TYPE 3 message using unacknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | SYSINFO3_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatRqSysinfo4 (DL_UNIT_DATA_REQUEST_SYSTEM_INFORMATION_TYPE4) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR SYSTEM INFORMATION TYPE 4 message using unacknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | SYSINFO4_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatRqSysinfo5 (DL_UNIT_DATA_REQUEST_SYSTEM_INFORMATION_TYPE5) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR SYSTEM INFORMATION TYPE 5 message using unacknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | SYSINFO5_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatRqSysinfo5bis (DL_UNIT_DATA_REQUEST_SYSTEM_INFORMATION_TYPE5bis) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR SYSTEM INFORMATION TYPE 5bis message using unacknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | SYSINFO5bis_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatRqSysinfo6 (DL_UNIT_DATA_REQUEST_SYSTEM_INFORMATION_TYPE6) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR SYSTEM INFORMATION TYPE 6 message using unacknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | SYSINFO6_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_UdatInClmChn (DL_UdatINDICATION_CLASSMARK_CHANGE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the RR CLASSMARK CHANGE message using unacknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CLM_CHN_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqAssCmd (DL_DATA_REQUEST_ASSIGNMENT_COMMAND) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR ASSIGNMENT COMMAND message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | ASS_CMD_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInAssCom (DL_DATA_INDICATION_ASSIGNMENT_COMPLETE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the RR ASSIGNMENT COMPLETE message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | ASS_COM_PDU |  |
| fn |  | FN |  |
| Detailed Comments: | The ASP is a result of the ASP DL_RESUME_REQUEST of the SUT sending the ASSIGNMENT COMPLETE message. |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInAssfl (DL_DATA_INDICATION_ASSIGNMENT_FAILURE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the RR ASSIGNMENT FAILURE message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | ASSFL_PDU |  |
| fn |  | FN |  |
| Detailed Comments: | The ASP is a result of the ASP DL_RECONNECT_REQUEST of the SUT sending the ASSIGNMEN FAILURE message. |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInCmsRq (DL_DATA_INDICATION_CM_SERVICE_REQUEST) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the MM CM SERVICE REQUEST message (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CMS_RQ_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqChmmo (DL_DATA_REQUEST_CHANNEL_MODE_MODIFY) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR CHANNEL MODE MODIFY message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CHMMO_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInChmmoAck (DL_DATA_INDICATION_CHANNEL_MODE_MODIFY_ACKNOWLEDGE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the RR CHANNEL MODE MODIFY ACKNOWLEDGE message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CHMMO_ACK_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqChRel (DL_DATA_REQUEST_CHANNEL_RELEASE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR CHANNEL_RELEASE message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CH_REL_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqChRelErr (DL_DATA_REQUEST_CHANNEL_RELEASE_ERROR) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR CHANNEL_RELEASE message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CH_REL_PDU_ERR |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqCphmCmd (DL_DATA_REQUEST_CIPHERING_MODE_COMMAND) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR CIPHERING_MODE_COMMAND message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CPHM_CMD_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqCphmCmdErr (DL_DATA_REQUEST_CIPHERING_MODE_COMMAND_ERROR) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the invalid RR CIPHERING_MODE_COMMAND message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CPHM_CMD_PDU_ERR |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInCphmCom (DL_DATA_INDICATION_CIPHERING_MODE_COMPLETE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the RR CIPHERING MODE COMPLETE message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CPHM_COM_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInClmChn (DL_DAT_INDICATION_CLASSMARK_CHANGE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the RR CLASSMARK CHANGE message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CLM_CHN_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqClmEnq (DL_DATA_REQUEST_CLASSMARK_ENQUIRY) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR CLASSMARK ENQUIRY message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CLM_ENQ_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqFrqre (DL_DATA_REQUEST_FREQUENCY_REDEFINITION) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR FREQUENCY REDEFINITION message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | FRQRE_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqHoCmd (DL_DATA_REQUEST_HANDOVER_COMMAND) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR HANDOVER COMMAND message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | HO_CMD_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInHoCom (DL_DATA_INDICATION_HANDOVER_COMPLETE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the RR HANDOVER COMPLETE message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | HO_COM_PDU |  |
| fn |  | FN |  |
| Detailed Comments: | The ASP is a result of the ASP DL_RESUME_REQUEST of the SUT sending the HANDOVER COMPLETE message. |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInHofl (DL_DATA_INDICATION_HANDOVER_FAILURE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the RR HANDOVER FAILURE message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | HOFL_PDU |  |
| fn |  | FN |  |
| Detailed Comments: | The ASP is a result of the ASP DL_RECONNECT_REQUEST of the SUT sending the HANDOVER FAILURE message. |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqPhyinfo (DL_DATA_REQUEST_PHYSICAL_INFORMATION) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the RR PHYSICAL INFORMATION message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | PHYINFO_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInRrst (DL_DATA_INDICATION_RR_STATUS) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the RR STATUS message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | RRST_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqAuthRej (DL_DATA_REQUEST_AUTHENTICATION_REJECT) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the MM AUTHENTICATION REJECT message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | AUTH_REJ_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqAuthRq (DL_DATA_REQUEST_AUTHENTICATION_REQUEST) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the MM AUTHENTICATION REQUEST message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | AUTH_RQ_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInAuthRes (DL_DATA_INDICATION_AUTHENTICATION_RESPONSE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the MM AUTHENTICATION RESPONSE message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | AUTH_RES_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqCmsAcp (DL_DATA_REQUEST_CM_SERVICE_ACCEPT) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the MM CM SERVICE ACCEPT message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CMS_ACP_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqCmsRej (DL_DATA_REQUEST_CM_SERVICE_REJECT) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the MM CM SERVICE REJECT message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CMS_REJ_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqAbrt (DL_DATA_REQUEST_ABORT) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the MM ABORT message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | ABRT_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqIdRq (DL_DATA_REQUEST_IDENTITY_REQUEST) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the MM IDENTITY REQUEST message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | ID_RQ_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInIdRes (DL_DATA_INDICATION_IDENTIFY_RESPONSE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the MM IDENTITY RESPONSE message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | ID_RES_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqLupAcp (DL_DATA_REQUEST_LOCATION_UPDATING_ACCEPT) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the MM LOCATION UPDATING ACCEPT message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | LUP_ACP_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqLupAcpErr (DL_DATA_REQUEST_LOCATION_UPDATING_ACCEPT_ERR) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the MM LOCATION UPDATING ACCEPT message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | LUP_ACP_PDU_ERR |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqLupRej (DL_DATA_REQUEST_LOCATION_UPDATING_REJECT) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the MM LOCATION UPDATING REJECT message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | LUP_REJ_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInMmst (DL_DATA_INDICATION_MM_STATUS) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the MM STATUS message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | MMST_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqTmsireCmd (DL_DATA_REQUEST_TMSI_REALLOCATION_COMMAND) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the MM TMSI REALLOCATION COMMAND message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | TMSIRE_CMD_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInTmsireCom (DL_DATA_INDICATION_TMSI_REALLOCATION_COMPLETE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the MM TMSI REALLOCATION COMPLETE message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | TMSIRE_COM_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqAlert (DL_DATA_REQUEST_ALERTING) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC ALERTING message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | ALERT_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInAlert (DL_DATA_INDICATION_ALERTING) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the CC ALERTING message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | ALERT_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInCallCo (DL_DATA_INDICATION_CALL_CONFIRMED) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the CC CALL CONFIRMED message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CALL_CO_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqCallProc (DL_DATA_REQUEST_CALL_PROCEEDING) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC CALL PROCEEDING message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CALL_PROC_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqConn (DL_DATA_REQUEST_CONNECT) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC CONNECT message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CONN_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqConnErr (DL_DATA_REQUEST_CONNECT_ERR) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC CONNECT message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CONN_PDU_ERR |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInConn (DL_DATA_INDICATION_CONNECT) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the CC CONNECT message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CONN_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqConnAck (DL_DATA_REQUEST_CONNECT_ACKNOWLEDGE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC CONNECT ACKNOWLEDGE message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CONN_ACK_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInConnAck (DL_DATA_INDICATION_CONNECT_ACKNOWLEDGE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the reception of the CC CONNECT ACKNOWLEDGE message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CONN_ACK_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqDisc (DL_DATA_REQUEST_DISCONNECT) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC DISCONNECT message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | DISC_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqDiscErr (DL_DATA_REQUEST_DISCONNECT_ERR) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC DISCONNECT message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | DISC_PDU_ERR |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInDisc (DL_DATA_INDICATION_DISCONNECT) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the CC DISCONNECT message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | DISC_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInESetup (DL_DATA_INDICATION_EMERGENCY_SETUP) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the CC EMERGENCY_SETUP message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | ESETUP_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqFac (DL_DATA_REQUEST_FACILITY) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC FACILITY message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | FAC_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInFac (DL_DATA_INDICATION_FACILITY) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the CC FACILITY message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | FAC_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInHold (DL_DATA_INDICATION_HOLD) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the CC HOLD message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | HOLD_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqHoldAck (DL_DATA_REQUEST_HOLD_ACKNOWLEDGE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC HOLD ACKNOWLEDGE message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | HOLD_ACK_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqModify (DL_DATA_REQUEST_MODIFY) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC MODIFY message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | MODIFY_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInModify (DL_DATA_INDICATION_MODIFY) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the CC MODIFY message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | MODIFY_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqModifyCom (DL_DATA_REQUEST_MODIFY_COMPLETE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC MODIFY COMPLETE message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | MODIFY_COM_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqModifyRej (DL_DATA_REQUEST_MODIFY_REJECT) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC MODIFY REJECT message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | MODIFY_REJ_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInModifyRej (DL_DATA_INDICATION_MODIFY_REJECT) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the CC MODIFY REJECT message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | MODIFY_REJ_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqNotify (DL_DATA_REQUEST_NOTIFY) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC NOTIFY message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | NOTIFY_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqProg (DL_DATA_REQUEST_PROGRESS) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC PROGRESS message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | PROG_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqRegister (DL_DATA_REQUEST_REGISTER) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the SS REGISTER message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | REGISTER_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInRegister (DL_DATA_INDICATION_REGISTER) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to receive the transmission of the SS REGISTER message using acknowledged operation. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | REGISTER_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqRel (DL_DATA_REQUEST_RELEASE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC RELEASE message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | REL_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInRel (DL_DATA_INDICATION_RELEASE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the CC RELEASE message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | REL_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqRelCmp (DL_DATA_REQUEST_RELEASE_COMPLETE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC RELEASE COMPLETE message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | REL_COM_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInRelCmp (DL_DATA_INDICATION_RELEASE_COMPLETE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the CC RELEASE COMPLETE message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | REL_COM_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqSetup (DL_DATA_REQUEST_SETUP) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC SETUP message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | SETUP_MT_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInSetup (DL_DATA_INDICATION_SETUP) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the CC SETUP message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | SETUP_MO_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInStartDtmf (DL_DATA_INDICATION_START_DTMF) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the CC START_DTMF message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | START_DTMF_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqStartDtmfAck (DL_DATA_REQUEST_START_DTMF_ACKNOWLEDGE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC START DTMF ACKNOWLEDGE message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | START_DTMF_ACK_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqStartDtmfRej (DL_DATA_REQUEST_START_DTMF_REJCT) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC START DTMF REJCT message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | START_DTMF_REJ_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInStopDtmf (DL_DATA_INDICATION_STOP_DTMF) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the CC STOP_DTMF message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | STOP_DTMF_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqStopDtmfAck (DL_DATA_REQUEST_STOP_DTMF_ACKNOWLEDGE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC STOP DTMF ACKNOWLEDGE message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | STOP_DTMF_ACK_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqCcst (DL_DATA_REQUEST_CC_STATUS) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC STATUS message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CCST_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInCcst (DL_DATA_INDICATION_CC_STATUS) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the CC STATUS message using acknowledged operation (L2 -> L3). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CCST_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqCcstEnq (DL_DATA_REQUEST_CC_STATUS_ENQ) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CC STATUS_ENQ message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CCST_ENQ_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqUndefCC (DL_DATA_REQUEST) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to transmit any message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CONN_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqUndefMM (DL_DATA_REQUEST) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to transmit any message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | ID_RES_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqUndefRR (DL_DATA_REQUEST) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to transmit any message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | PART_REL_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqUnknown (DL_DATA_REQUEST) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to transmit any message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CCST_ENQ_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInCpData (DL_DATA_INDICATION_CP_DATA) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the SMS CP DATA message using acknowledged operation (L2 -> L3) for MT. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CP_DATA_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqCpData (DL_DATA_REQUEST_CP_DATA) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the SMS CP DATA message using acknowledged operation (L3 -> L2) for MO. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CP_DATA_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatInCpDataAck (DL_DATA_INDICATION_CPDATA_ACKNOWLEDGE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to indicate the receipt of the SMS CP DATA message using acknowledged operation (L2 -> L3) . |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CPDATA_ACK_PDU |  |
| fn |  | FN |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqCpDataAck (DL_DATA_REQUEST_CPDATA_ACKNOWLEDGE) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the CP DATA ACKNOWLEDGE message using acknowledged operation (L3 -> L2). |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CPDATA_ACK_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | DL_DatRqCpError (DL_DATA_REQUEST_CP_ERROR) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the transmission of the SMS CP ERROR message using acknowledged operation (L3 -> L2) for MO. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| msg |  | CPERR_PDU |  |
| Detailed Comments: |  |  |  |

| ASP Type Definition |  |  |  |
| --- | --- | --- | --- |
| ASP Name: | MDL_RelRq (MDL_RELEASE_REQUEST) |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | The ASP is used to request the local end termiination of a previous established acknowledged mode operation (L3 -> L2). The local end termination procedure is initiated. |  |  |
| Parameter Name |  | Parameter Type | Comments |
| sapi |  | SAPID |  |
| logic_ch |  | LOGICCH |  |
| release_mode |  | RelMode |  |
| Detailed Comments: |  |  |  |

## PDU type definitions

### TTCN PDU Type definitions

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | ASS_CMD_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR ASSSIGNMENT COMMAND n -> ms GSM 04.08, 9.1.2 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| ch1d_at |  | CHD |  |
| pcmd |  | PCMD |  |
| frql_at |  | FRQL |  |
| cchd |  | CCHD |  |
| ch1mod |  | CHMOD |  |
| ch2d_at |  | CHD |  |
| ch2mod |  | CHMOD |  |
| ma_at |  | MA |  |
| strt |  | STRT |  |
| frql_bt |  | FRQL |  |
| ch1d_bt |  | CHD |  |
| ch2d_bt |  | CHD |  |
| frqchs_bt |  | FRQCHS |  |
| ma_bt |  | MA |  |
| cphms |  | CPHMS |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | ASS_COM_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR ASSSIGNMENT COMPLETE ms -> n GSM 04.08, 9.1.3 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| rrcau |  | RRCAU |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | ASSFL_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR ASSSIGNMENT FAILURE ms -> n GSM 04.08, 9.1.4 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| rrcau |  | RRCAU |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CHMMO_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR CHANNEL MODE MODIFY n -> ms GSM 04.08, 9.1.5 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| chd |  | CHD |  |
| chmod |  | CHMOD |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CHMMO_ACK_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR CHANNEL MODE MODIFY ACKNOWLEDGE ms -> n GSM 04.08, 9.1.6 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| chd |  | CHD |  |
| chmod |  | CHMOD |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CH_REL_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR CHANNEL RELEASE n -> ms GSM 04.08, 9.1.7 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| rrcau |  | RRCAU |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CH_REL_PDU_ERR |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR CHANNEL RELEASE containing additional unknown IE n -> ms GSM 04.08, 9.1.7 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| rrcau |  | RRCAU |  |
| add |  | OCTETSTRING |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CH_RQ_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR CHANNEL REQUEST ms -> n GSM 04.08, 9.1.8 |  |  |
| Field Name |  | Field Type | Comments |
| ecau_rrf |  | BITSTRING [8] |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CPHM_CMD_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR CIPHERING MODE COMMAND n -> ms GSM 04.08, 9.1.9 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| cph_res |  | CPH_RES |  |
| cphms |  | CPHMS |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CPHM_CMD_PDU_ERR |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR CIPHERING MODE COMMAND with additional unknown IE GSM 04.08, 9.1.9 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| cph_res |  | CPH_RES |  |
| cphms |  | CPHMS |  |
| add |  | OCTETSTRING |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CPHM_COM_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR CIPHERING MODE COMPLETE ms -> n GSM 04.08, 9.1.10 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| mei |  | MI |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CLM_CHN_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR CLASSMARK CHANGE ms -> n GSM 04.08, 9.1.11 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| msclm |  | MSCLM2 |  |
| msclm_adi |  | MSCLM3 |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CLM_ENQ_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR CLASSMARK ENQUIRY n -> ms GSM 04.08, 9.1.12 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | FRQRE_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR FREQUENCY REDEFINITIONY n -> ms GSM 04.08, 9.1.13 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| chd |  | CHD |  |
| ma |  | MA |  |
| strt |  | STRT |  |
| cchd |  | CCHD |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | HOACC_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR HANDOVER ACCESS ms -> n GSM 04.08, 9.1.14 |  |  |
| Field Name |  | Field Type | Comments |
| horf |  | HORF |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | HO_CMD_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR HANDOVER COMMAND n -> ms GSM 04.08, 9.1.15 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| cd |  | CD |  |
| ch1d_at |  | CHD |  |
| horf |  | HORF |  |
| pcmd |  | PCMD |  |
| synchi |  | SYNCHI |  |
| frqsl_at |  | FRQL |  |
| frql_at |  | FRQL |  |
| cchd |  | CCHD |  |
| ch1mod |  | CHMOD |  |
| ch2d_at |  | CHD |  |
| ch2mod |  | CHMOD |  |
| frqchs_at |  | FRQCHS |  |
| ma_at |  | MA |  |
| strt |  | STRT |  |
| rtdif |  | TDIF |  |
| ta |  | TA |  |
| frqsl_bt |  | FRQL |  |
| frql_bt |  | FRQL |  |
| ch1d_bt |  | CHD |  |
| ch2d_bt |  | CHD |  |
| frqchs_bt |  | FRQCHS |  |
| ma_bt |  | MA |  |
| cphms |  | CPHMS |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | HO_COM_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR HANDOVER COMPLETE ms -> n GSM 04.08, 9.1.16 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| rrcau |  | RRCAU |  |
| motdif |  | MTDIF |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | HOFL_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR HANDOVER FAILURE ms -> n GSM 04.08, 9.1.17 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| rrcau |  | RRCAU |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | IMMASS_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR IMMEDIATE ASSIGNMENT n -> ms GSM 04.08, 9.1.18 |  |  |
| Field Name |  | Field Type | Comments |
| l2_pl |  | LENGTH |  |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| shoct |  | SHOCT |  |
| pm |  | PM |  |
| chd |  | CHD |  |
| rqr |  | RQR |  |
| ta |  | TA |  |
| ma |  | MA |  |
| strt |  | STRT |  |
| iaroct |  | IARESTOCT |  |
| Detailed Comments: | The message has a fixed length of 23 octets. |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | IMMASSX_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR IMMEDIATE ASSIGNMENT n -> ms GSM 04.08, 9.1.19 |  |  |
| Field Name |  | Field Type | Comments |
| l2_pl |  | LENGTH |  |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| shoct |  | SHOCT |  |
| pm |  | PM |  |
| chd1 |  | CHD |  |
| rqr1 |  | RQR |  |
| ta1 |  | TA |  |
| chd2 |  | CHD |  |
| rqr2 |  | RQR |  |
| ta2 |  | TA |  |
| ma |  | MA |  |
| strt |  | STRT |  |
| iaxroct |  | OCTETSTRING [1..4] |  |
| Detailed Comments: | The message has a fixed length of 23 octets. |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | IMMASS_REJ_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR IMMEDIATE REJECT n -> ms GSM 04.08, 9.1.20 |  |  |
| Field Name |  | Field Type | Comments |
| l2_pl |  | LENGTH |  |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| shoct |  | SHOCT |  |
| pm |  | PM |  |
| rqr1 |  | RQR |  |
| wi1 |  | WI |  |
| rqr2 |  | RQR |  |
| wi2 |  | WI |  |
| rqr3 |  | RQR |  |
| wi3 |  | WI |  |
| rqr4 |  | RQR |  |
| wi4 |  | WI |  |
| iarroct |  | OCTETSTRING [3] |  |
| Detailed Comments: | The message has a fixed length of 23 octets. |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | MSR_RPT_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR MEASUREMENT REPORT ms -> n GSM 04.08, 9.1.21 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| msrr |  | MSRR |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | PG1_RQ_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR PAGING REQUEST_TYPE1 n -> ms GSM 04.08, 9.1.22 |  |  |
| Field Name |  | Field Type | Comments |
| l2_pl |  | LENGTH |  |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| chn_m1_2 |  | CHNEED |  |
| pm |  | PM |  |
| mi1 |  | MI |  |
| mi2 |  | MI |  |
| p1roct |  | OCTETSTRING [0..17] |  |
| Detailed Comments: | The message has a fixed length of 23 octets. |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | PG2_RQ_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR PAGING REQUEST TYPE2 n -> ms GSM 04.08, 9.1.23 |  |  |
| Field Name |  | Field Type | Comments |
| l2_pl |  | LENGTH |  |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| chn_m1_2 |  | CHNEED |  |
| pm |  | PM |  |
| mi1 |  | TMSI |  |
| mi2 |  | TMSI |  |
| mi3 |  | MI |  |
| p2roct |  | OCTETSTRING [1..11] |  |
| Detailed Comments: | The message has a fixed length of 23 octets. |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | PG3_RQ_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR PAGING REQUEST TYPE3 n -> ms GSM 04.08, 9.1.24 |  |  |
| Field Name |  | Field Type | Comments |
| l2_pl |  | LENGTH |  |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| chn_m1_2 |  | CHNEED |  |
| pm |  | PM |  |
| mi1 |  | TMSI |  |
| mi2 |  | TMSI |  |
| mi3 |  | TMSI |  |
| mi4 |  | TMSI |  |
| p3roct |  | OCTETSTRING [3] |  |
| Detailed Comments: | The message has an L2 pseudo length of 19 octets and a total length of 23 octets. |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | PG_RES_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR PAGING RESPONSE ms -> n GSM 04.08, 9.1.25 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| shoct |  | SHOCT |  |
| cphksn |  | CPHKSN |  |
| msclm |  | MSCLM2 |  |
| mi |  | MI |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | PART_REL_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR PARTIAL RELEASE n -> ms GSM 04.08, 9.1.26 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| chd |  | MAC |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | PHYINFO_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR PHYSICAL INFORMATION n -> ms GSM 04.08 clause 9.1.28 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| ta |  | TA |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | RRST_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR STATUS n <-> ms GSM 04.08, 9.1.29 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| rrcau |  | RRCAU |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | SCHINFO_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR SYNCHRONIZATION CHANNEL INFORMATION n -> ms GSM 04.08, 9.1.30 |  |  |
| Field Name |  | Field Type | Comments |
| ncc |  | NCC |  |
| bcc |  | BCC |  |
| t1 |  | T1 |  |
| t2 |  | T2 |  |
| t3_ |  | T3_ |  |
| Detailed Comments: | SCHINFO_PDU has a total lengh of 25 bits (GSM 04.04). |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | SYSINFO1_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR SYSTEM INFORMATION TYPE1 n -> ms GSM 04.08, 9.1.31 |  |  |
| Field Name |  | Field Type | Comments |
| l2_pl |  | LENGTH |  |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| cchd |  | CCHD |  |
| rachcp |  | RACHCP |  |
| si1roct |  | OCTETSTRING [1] |  |
| Detailed Comments: | The message has an L2 pseudo length of 21 octets and a total length of 23 octets. |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | SYSINFO2_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR SYSTEM INFORMATION TYPE2 n -> ms GSM 04.08, 9.1.32 |  |  |
| Field Name |  | Field Type | Comments |
| l2_pl |  | LENGTH |  |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| bcchfl |  | NCD |  |
| nccp |  | NCCP |  |
| rachcp |  | RACHCP |  |
| Detailed Comments: | The message has an L2 pseudo length of 22 octets and a total length of 23 octets. |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | SYSINFO2bis_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR SYSTEM INFORMATION TYPE2bis or 2ter n -> ms GSM 04.08, 9.1.33, 9.1.33a |  |  |
| Field Name |  | Field Type | Comments |
| l2_pl |  | LENGTH |  |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| xbcchfl |  | NCD |  |
| rachcp |  | RACHCP |  |
| si2bisroct |  | OCTETSTRING [1..4] |  |
| Detailed Comments: | This PDU type is used both for SYSINFO2bis, as well as for 2ter. The 2bis message has an L2 pseudo length of 21 octets and a total length of 23 octets. The 2ter message has an L2 pseudo length of 18 octets and a total length of 23 octets. |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | SYSINFO3_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR SYSTEM INFORMATION TYPE3 n -> ms GSM 04.08, 9.1.34 |  |  |
| Field Name |  | Field Type | Comments |
| l2_pl |  | LENGTH |  |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| ci |  | CI |  |
| lai |  | LAI |  |
| ccd |  | CCD |  |
| co |  | CO |  |
| csp |  | CSP |  |
| rachcp |  | RACHCP |  |
| si3roct |  | OCTETSTRING [4] |  |
| Detailed Comments: | The message has an L2 pseudo length of 18 octets and a total length of 23 octets. |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | SYSINFO4_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR SYSTEM INFORMATION TYPE4 n -> ms GSM 04.08, 9.1.35 |  |  |
| Field Name |  | Field Type | Comments |
| l2_pl |  | LENGTH |  |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| lai |  | LAI |  |
| csp |  | CSP |  |
| rachcp |  | RACHCP |  |
| cbchd |  | CHD |  |
| cbchma |  | MA |  |
| si4roct |  | OCTETSTRING [1..10] |  |
| Detailed Comments: | The message has a total length of 23 octets. |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | SYSINFO5_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR SYSTEM INFORMATION TYPE5 n -> ms GSM 04.08, 9.1.36 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| bcchfl |  | NCD |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | SYSINFO5bis_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR SYSTEM INFORMATION TYPE5bis n -> ms GSM 04.08, 9.1.38 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| xbcchfl |  | NCD |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | SYSINFO6_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | RR SYSTEM INFORMATION TYPE6 n -> ms GSM 04.08, 9.1.38 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| rrpd |  | PD |  |
| mt |  | MT |  |
| ci |  | CI |  |
| lai |  | LAI |  |
| co |  | CO |  |
| nccp |  | NCCP |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | ABRT_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | MM ABORT n -> ms GSM 04.08, 9.2.8 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| mmpd |  | PD |  |
| mt |  | MT |  |
| rejcau |  | REJCAU |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | AUTH_REJ_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | MM AUTHENTICATION REJECT n -> ms GSM 04.08, 9.2.1 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| mmpd |  | PD |  |
| mt |  | MT |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | AUTH_RQ_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | MM AUTHENTICATION REQUEST n -> ms GSM 04.08, 9.2.2 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| mmpd |  | PD |  |
| mt |  | MT |  |
| shoct |  | SHOCT |  |
| cphksn |  | CPHKSN |  |
| rand |  | RAND |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | AUTH_RES_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | MM AUTHENTICATION RESPONSE ms -> n GSM 04.08, 9.2.3 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| mmpd |  | PD |  |
| mt |  | MT |  |
| sres |  | SRES |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CMRE_RQ_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | MM CM REESTABLISHMENT REQUEST ms -> n GSM 04.08, 9.2.4 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| mmpd |  | PD |  |
| mt |  | MT |  |
| shoct |  | SHOCT |  |
| cphksn |  | CPHKSN |  |
| msclm |  | MSCLM2 |  |
| mi |  | MI |  |
| lai |  | LAI |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CMS_ACP_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | MM CM SERVICE ACCEPT n -> ms GSM 04.08, 9.2.5 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| mmpd |  | PD |  |
| mt |  | MT |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CMS_REJ_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | MM CM SERVICE REJECT n -> ms GSM 04.08, 9.2.6 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| mmpd |  | PD |  |
| mt |  | MT |  |
| mmcau |  | REJCAU |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CMS_RQ_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | MM CM SERVICE REQUEST ms -> n GSM 04.08, 9.2.9 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| mmpd |  | PD |  |
| mt |  | MT |  |
| cphksn |  | CPHKSN |  |
| svtype |  | CMSVTYPE |  |
| msclm |  | MSCLM2 |  |
| mi |  | MI |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | ID_RQ_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | MM IDENTITY REQUEST n -> ms GSM 04.08, 9.2.10 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| mmpd |  | PD |  |
| mt |  | MT |  |
| shoct |  | SHOCT |  |
| idtype |  | IDTYPE |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | ID_RES_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | MM IDENTITY RESPONSE ms -> n GSM 04.08, 9.2.11 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| mmpd |  | PD |  |
| mt |  | MT |  |
| mi |  | MI |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | IMSID_IN_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | MM IMSI DETACH INDICATION ms -> n GSM 04.08, 9.2.12 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| mmpd |  | PD |  |
| mt |  | MT |  |
| msclm |  | MSCLM1 |  |
| mi |  | MI |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | LUP_ACP_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | MM LOCATION UPDATING ACCEPT n -> ms GSM 04.08, 9.2.13 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| mmpd |  | PD |  |
| mt |  | MT |  |
| lai |  | LAI |  |
| mi |  | MI |  |
| fop |  | IEI_8 |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | LUP_ACP_PDU_ERR |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | MM LOCATION UPDATING ACCEPT n -> ms GSM 04.08, 9.2.13 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| mmpd |  | PD |  |
| mt |  | MT |  |
| lai |  | LAI |  |
| mi |  | MI |  |
| dupmi |  | MI |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | LUP_REJ_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | MM LOCATION UPDATING REJECT n -> ms GSM 04.08, 9.2.14 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| mmpd |  | PD |  |
| mt |  | MT |  |
| rejcau |  | REJCAU |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | LUP_RQ_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | MM LOCATION UPDATING REQUEST ms -> n GSM 04.08, 9.2.15 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| mmpd |  | PD |  |
| mt |  | MT |  |
| cphksn |  | CPHKSN |  |
| lutype |  | LUT |  |
| lai |  | LAI |  |
| msclm |  | MSCLM1 |  |
| mi |  | MI |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | MMST_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | MM STATUS n <-> ms GSM 04.08, 9.2.16 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| mmpd |  | PD |  |
| mt |  | MT |  |
| rejcau |  | REJCAU |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | TMSIRE_CMD_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | MM TMSI REALLOCATION COMMAND n -> ms GSM 04.08, 9.2.17 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| mmpd |  | PD |  |
| mt |  | MT |  |
| lai |  | LAI |  |
| mi |  | MI |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | TMSIRE_COM_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | MM TMSI REALLOCATION COMPLETE ms -> n GSM 04.08, 9.2.18 |  |  |
| Field Name |  | Field Type | Comments |
| ski |  | SKI |  |
| mmpd |  | PD |  |
| mt |  | MT |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | ALERT_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC ALERTING ms <-> n GSM 04.08, 9.3.1 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| fie |  | FIE |  |
| pi |  | PI |  |
| uu |  | UU |  |
| ssvi |  | SSVI |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CALL_CO_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC CALL CONFIRMED ms -> n GSM 04.08, 9.3.2 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| bcri |  | RPI |  |
| bcap1 |  | BCAP |  |
| bcap2 |  | BCAP |  |
| cau |  | CAU |  |
| cccap |  | CCCAP |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CALL_PROC_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC CALL PROCEEDING n -> ms GSM 04.08, 9.3.3 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| bcri |  | RPI |  |
| bcap1 |  | BCAP |  |
| bcap2 |  | BCAP |  |
| fie |  | FIE |  |
| pi |  | PI |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CONN_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC CONNECT n <-> ms GSM 04.08, 9.3.5 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| fie |  | FIE |  |
| pi |  | PI |  |
| cnn |  | CNN |  |
| cns |  | CNS |  |
| uu |  | UU |  |
| ssvi |  | SSVI |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CONN_PDU_ERR |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC CONNECT n <-> ms Used as invalid message |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| unknown |  | UNKWN |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CONN_ACK_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC CONNECT ACKNOWLEDGE ms <-> n GSM 04.08, 9.3.6 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | DISC_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC DISCONNECT ms <-> n (both directions) GSM 04.08, 9.3.7 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| cau |  | CAU |  |
| fie |  | FIE |  |
| pi |  | PI |  |
| uu |  | UU |  |
| ssvi |  | SSVI |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | DISC_PDU_ERR |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: |  |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| cau |  | CAU |  |
| unknown |  | UNKWN |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | ESETUP_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC EMERGENCY SETUP ms -> n GSM 04.08, 9.3.8 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| bcap |  | BCAP |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | FAC_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC FACILITY n <-> ms GSM 04.08, 9.3.9 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| cc_sspd |  | PD |  |
| mt |  | MT |  |
| fie |  | FIE |  |
| ssvi |  | SSVI |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | HOLD_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC HOLD ms -> n GSM 04.08, 9.3.10 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | HOLD_ACK_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC HOLD ACKNOWLEDGE n -> ms GSM 04.08, 9.3.11 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | MODIFY_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC MODIFY ms <-> n GSM 04.08, 9.3.13 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| bcap |  | BCAP |  |
| llcmp |  | LLCMP |  |
| hlcmp |  | HLCMP |  |
| rcsd |  | RCSD |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | MODIFY_COM_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC MODIFY COMPLETE ms <-> n GSM 04.08, 9.3.14 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| bcap |  | BCAP |  |
| llcmp |  | LLCMP |  |
| hlcmp |  | HLCMP |  |
| rcsd |  | RCSD |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | MODIFY_REJ_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC MODIFY REJECT ms <-> n GSM 04.08, 9.3.15 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| bcap |  | BCAP |  |
| cau |  | CAU |  |
| llcmp |  | LLCMP |  |
| hlcmp |  | HLCMP |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | NOTIFY_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC NOTIFY ms <-> n GSM 04.08, 9.3.16 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| nti |  | NTI |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | REGISTER_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | SS REGISTER ms <-> n GSM 04.80, 2.4 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| sspd |  | PD |  |
| mt |  | MT |  |
| fie |  | FIE |  |
| ssvi |  | SSVI |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | PROG_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC PROGRESS n -> ms GSM 04.08, 9.3.17 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| pi |  | PI |  |
| uu |  | UU |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | REL_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC RELEASE n <-> ms GSM 04.08, 9.3.18 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| cau |  | CAU |  |
| cau2 |  | CAU |  |
| fie |  | FIE |  |
| uu |  | UU |  |
| ssvi |  | SSVI |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | REL_COM_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC or SS RELEASE COMPLETE n <-> ms GSM 04.08, 9.3.19; GSM 04.80, 2.5 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| cc_sspd |  | PD |  |
| mt |  | MT |  |
| cau |  | CAU |  |
| fie |  | FIE |  |
| uu |  | UU |  |
| ssvi |  | SSVI |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | SETUP_MO_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC SETUP n <- ms GSM 04.08, 9.3.23.2 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| bcri |  | RPI |  |
| bcap1 |  | BCAP |  |
| bcap2 |  | BCAP |  |
| fie |  | FIE |  |
| cgps |  | CGPS |  |
| cdpn |  | CDPN |  |
| cdps |  | CDPS |  |
| llcri |  | RPI |  |
| llcmp1 |  | LLCMP |  |
| llcmp2 |  | LLCMP |  |
| hlcri |  | RPI |  |
| hlcmp1 |  | HLCMP |  |
| hlcmp2 |  | HLCMP |  |
| uu |  | UU |  |
| ssvi |  | SSVI |  |
| clirsup |  | CLRSUP |  |
| clirinv |  | CLRINV |  |
| cccap |  | CCCAP |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | SETUP_MT_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC SETUP n -> ms GSM 04.08, 9.3.23.1 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| bcri |  | RPI |  |
| bcap1 |  | BCAP |  |
| bcap2 |  | BCAP |  |
| fie |  | FIE |  |
| pi |  | PI |  |
| sig |  | SIGNAL |  |
| cgpn |  | CGPN |  |
| cgps |  | CGPS |  |
| cdpn |  | CDPN |  |
| cdps |  | CDPS |  |
| llcri |  | RPI |  |
| llcmp1 |  | LLCMP |  |
| llcmp2 |  | LLCMP |  |
| hlcri |  | RPI |  |
| hlcmp1 |  | HLCMP |  |
| hlcmp2 |  | HLCMP |  |
| uu |  | UU |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | SMSCB_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | SMSCB message GSM 03.41, 9.3 |  |  |
| Field Name |  | Field Type | Comments |
| blocktype |  | BLOCKTYPE |  |
| serial_number |  | SERIAL_NUMBER |  |
| message_id |  | OCTETSTRING[2] |  |
| dcs |  | TPDCS |  |
| page_param |  | BITSTRING[8] |  |
| message_contents |  | OCTETSTRING |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | START_DTMF_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC START DTMF ms -> n GSM 04.08, 9.3.24 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| kpf |  | KPF |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | START_DTMF_ACK_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC START DTMF ACKNOWLEDGE n -> ms GSM 04.08, 9.3.25 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| kpf |  | KPF |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | START_DTMF_REJ_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC START DTMF REJECT n -> ms GSM 04.08, 9.3.26 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| cau |  | CAU |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CCST_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC STATUS n <-> ms GSM 04.08, 9.3.27 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| cau |  | CAU |  |
| cst |  | CST |  |
| acst |  | ACST |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CCST_ENQ_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC STATUS ENQUIRY ms <-> n GSM 04.08, 9.3.28 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | STOP_DTMF_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC STOP DTMF ms -> n GSM 04.08, 9.3.29 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | STOP_DTMF_ACK_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CC STOP DTMF acknowledge n -> ms GSM 04.08, 9.3.30 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| ccpd |  | PD |  |
| mt |  | MT |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CP_DATA_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | SMS CP DATA ms <-> n GSM 04.11, 7.2.1 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| smspd |  | PD |  |
| mt |  | MT |  |
| CPdata |  | CPDATA |  |
| Detailed Comments: | CPDATA contains RPDU - RP_ACK or RP_ERROR, either returning to the MS in case of MO, or reporting the outcome of a MT messaging attempt in case of MT . |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CPDATA_ACK_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CP DATA ACKNOWLEDGE ms <-> n GSM 04.11, 7.2.2 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| smspd |  | PD |  |
| mt |  | MT |  |
| Detailed Comments: |  |  |  |

| PDU Type Definition |  |  |  |
| --- | --- | --- | --- |
| PDU Name: | CPERR_PDU |  |  |
| PCO Type: | SAP0_3 |  |  |
| Comments: | CP_ERROR n <-> ms GSM 04.11, 7.2.3 |  |  |
| Field Name |  | Field Type | Comments |
| ti |  | TI |  |
| smspd |  | PD |  |
| mt |  | MT |  |
| cp_cause |  | CP_CAU |  |
| Detailed Comments: | CP_UDAT contains RPDU, RP_ACK or RP_ERROR, either returning to the MS in case of MO, or reporting the outcome of a MT messaging attempt in case of MT . |  |  |