# Foreword

This draft European Telecommunication Standard (ETS) has been produced by the Special Mobile Group (SMG) Technical Committee of the European Telecommunications Standards Institute (ETSI) and is now submitted for the Voting phase of the ETSI standards approval procedure.

This ETS describes the technical characteristics and methods of test for Mobile Stations (MSs), operating in the 900 MHz and 1 800 MHz frequency band (GSM 900 and DCS 1 800) within the digital cellular telecommunications system.

This ETS corresponds to GSM technical specification GSM 11.10-3 version 4.15.0.

This part of the ETS (Part 3), contains Tree and Tabular Combined Notation (TTCN) for Layer 3 conformity specifications for which mobile stations, within the digital cellular telecommunications system (Phase 2), are tested for compliance.

The machine processable ATS

The electronic forms of the machine processable files (TTCN MP format) corresponding to the ATS for Layer 3 are contained in the self-extracting archive file oev06073.exe (Layer 3) on the diskette included as a part of this ETS (Part 3).

ETS 300 607 consists of three parts, which have the following ETS numbers and titles:

ETS 300 607-1	Digital cellular telecommunications system (Phase 2);

Mobile Station (MS) conformance specification;

Part 1: Conformance specification

Reference: GSM 11.10-1.

ETS 300 607-2	Digital cellular telecommunications system (Phase 2);

Mobile Station (MS) conformance specification;

Part 2: Protocol Implementation Conformance Statement (PICS)

proforma specification

Reference: GSM 11.10-2.

ETS 300 607-3	Digital cellular telecommunications system (Phase 1);

Mobile Station (MS) conformance specification;

Part 3: Layer 3 (L3) Abstract Test Suite (ATS)

Reference: GSM 11.10-3.

Reference is made within this draft ETS to GSM-TSs (note).

NOTE:	TC-SMG has produced documents which give the technical specifications for the implementation of the European digital cellular telecommunications system. Historically, these documents have been identified as GSM Technical Specifications (GSM-TSs). These TSs may have subsequently become I-ETSs (Phase 1), or ETSs (Phase 2), whilst others may become ETSI Technical Reports (ETRs). GSM-TSs are, for editorial reasons, still referred to in current GSM ETSs.

| Proposed transposition dates |  |
| --- | --- |
| Date of latest announcement of this ETS (doa): | 3 months after ETSI publication |
| Date of latest publication of new National Standard or endorsement of this ETS (dop/e): | 6 months after doa |
| Date of withdrawal of any conflicting National Standard (dow): | 6 months after doa |

Blank page

# 1	Scope

This European Telecommunications Standard specifies the Abstract Test Suite (ATS) and partial PIXIT proforma for the Network Layer (Layer 3) at the mobile radio interface of the GSM or DCS (Phase 2) mobile stations (MS) conforming to the ETSs for the Radio Resource management, the Mobility Management, the circuit-switched Call Control, the Supplementary Services and Short Message Services for the digital cellular telecommunications systems (Phase 2).

The ISO standards for the methodology of conformance testing are used as the basis for the test specifications.

# 2	Normative references

This ETS incorporates by dated and undated reference, provisions from other publications. These normative references are cited at the appropriate places in the text and the publications are listed hereafter. For dated references, subsequent amendments to or revisions of any of these publications apply to this ETS only when incorporated in it by amendment or revision. For undated references, the latest edition of the publication referred to applies.

[1]	ISO/IEC 9646-1: “Information Technology-OSI- Conformance Testing Methodology and Framework, Part 1: General Concepts”.

[2]	ISO/IEC 9646-2: “Information Technology-OSI- Conformance Testing Methodology and Framework, Part 2: Abstract Test Suite Specification”.

[3]	ISO/IEC 9646-3: “Information Technology-OSI- Conformance Testing Methodology and Framework, Part 3: The Tree and Tabular Combined Notation”.

[4]	ISO/IEC 9646-5: “Information Technology-OSI- Conformance Testing Methodology and Framework, Part 5: Requirements on test laboratories and clients for the conformance assessment process”.

[5]	ISO/IEC 8824: “Information Technology-OSI- Specification of Abstract Syntax Notation One (ASN.1) “.

[6]	ISO/IEC 8825: “Information Technology-OSI- Specification of Basic Encoding Rules for Abstract Syntax Notation One (ASN.1)“.

[7]	ITU-T Recommendation Q.773: “Specifications of Signalling System No.7; Transaction capabilities formats and encoding”.

[8]	ETS 300 287: “Integrated Services Digital Network (ISDN); CCITT Signalling System No.7, Transaction Capabilities Application Part (TCAP) version 2”.

[9]	ETS 300 406 (January 1995): “Methods for Testing and Specification (MTS); Protocol and profile conformance testing specifications: Standardization methodology”.

[10]	ETS 300 501 (GSM 02.02): “Digital cellular telecommunication system (Phase 2); Bearer Services (BS) supported by a GSM Public Land Mobile Network (PLMN)”.

[11]	ETS 300 502 (GSM 02.03): “Digital cellular telecommunication system (Phase 2); Teleservices supported by a GSM Public Land Mobile Network (PLMN)”.

[12]	ETS 300 504 (GSM 02.06): “Digital cellular telecommunication system (Phase 2); Types of Mobile Stations (MS)”.

[13]	ETS 300 505 (GSM 02.07): “Digital cellular telecommunication system (Phase 2); Mobile Station (MS) features”.

[14]	ETS 300 511 (GSM 02.30): “Digital cellular telecommunication system (Phase 2); Man-Machine Interface (MMI) of the Mobile Station (MS)”.

[15]	ETS 300 523 (GSM 03.03): “Digital cellular telecommunication system (Phase 2); Numbering, addressing and identification”.

[16]	ETS 300 527 (GSM 03.09): “Digital cellular telecommunication system (Phase 2); Handover procedures”.

[17]	ETS 300 528 (GSM 03.10): “Digital cellular telecommunication system (Phase 2); GSM Public Land Mobile Network (PLMN) connection types”.

[18]	ETS 300 529 (GSM 03.11): “Digital cellular telecommunication system (Phase 2); Technical realization of supplementary services”.

[19]	ETS 300 530 (GSM 03.12): “Digital cellular telecommunication system (Phase 2); Location registration procedures”.

[20]	ETS 300 532 (GSM 03.14): “Digital cellular telecommunication system (Phase 2); Support of Dual Tone Multi-Frequency signalling (DTMF) via the GSM system”.

[21]	ETS 300 535 (GSM 03.22): “Digital cellular telecommunication system (Phase 2); Functions related to Mobile Station (MS) in idle mode”.

[22]	ETS 300 536 (GSM 03.40): “Digital cellular telecommunication system (Phase 2); Technical realization of the Short Message Service (SMS) Point to Point (PP)”.

[23]	ETS 300 537 (GSM 03.41): “Digital cellular telecommunication system (Phase 2); Technical realization of Short Message Service Cell Broadcast (SMSCB)”.

[24]	ETS 300 550 (GSM 04.01): “Digital cellular telecommunication system (Phase 2); Mobile Station - Base Station System (MS - BSS) interface  General aspects and principles”.

[25]	ETS 300 552 (GSM 04.03): “Digital cellular telecommunication system (Phase 2); Mobile Station - Base Station System (MS - BSS) interface  Channel structures and access capabilities”.

[26]	ETS 300 553 (GSM 04.04): “Digital cellular telecommunication system (Phase 2); layer 1  General requirements”.

[27]	ETS 300 554 (GSM 04.05): “Digital cellular telecommunication system (Phase 2); Data Link (DL) layer  General aspects”.

[28]	ETS 300 555 (GSM 04.06): “Digital cellular telecommunication system (Phase 2); Mobile Station - Base Station System (MS - BSS) interface  Data Link (DL) layer specification”.

[29]	ETS 300 556 (GSM 04.07): “Digital cellular telecommunication system (Phase 2); Mobile radio interface signalling layer 3  General aspects”.

[30]	ETS 300 557 (GSM 04.08): “Digital cellular telecommunication system (Phase 2); Mobile radio interface layer 3 specification”.

[31]	ETS 300 558 (GSM 04.10): “Digital cellular telecommunication system (Phase 2); Mobile radio interface layer 3  Supplementary services specification  General aspects”.

[32]	ETS 300 559 (GSM 04.11): “Digital cellular telecommunication system (Phase 2); Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface”.

[33]	ETS 300 560 (GSM 04.12): “Digital cellular telecommunication system (Phase 2); Short Message Service Cell Broadcast (SMSCB) support on the mobile radio interface”.

[34]	ETS 300 564 (GSM 04.80): “Digital cellular telecommunication system (Phase 2); Mobile radio interface layer 3 supplementary services specification  Formats and coding”.

[35]	ETS 300 565 (GSM 04.81): “Digital cellular telecommunication system (Phase 2); Line identification supplementary services - Stage 3”.

[36]	ETS 300 566 (GSM 04.82): “Digital cellular telecommunication system (Phase 2); Call Forwarding (CF) supplementary services - Stage 3”.

[37]	ETS 300 567 (GSM 04.83): “Digital cellular telecommunication system (Phase 2); Call Waiting (CW) and Call Hold (HOLD) supplementary services - Stage 3”.

[38]	ETS 300 568 (GSM 04.84): “Digital cellular telecommunication system (Phase 2); MultiParty (MPTY) supplementary services - Stage 3”.

[39]	ETS 300 569 (GSM 04.85): “Digital cellular telecommunication system (Phase 2); Closed User Group (CUG) supplementary services - Stage 3”.

[40]	ETS 300 570 (GSM 04.86): “Digital cellular telecommunication system (Phase 2); Advice of Charge (AoC) supplementary services - Stage 3”.

[41]	ETS 300 571 (GSM 04.88): “Digital cellular telecommunication system (Phase 2); Call Barring (CB) supplementary services - Stage 3”.

[42]	ETS 300 572 (GSM 04.90): “Digital cellular telecommunication system (Phase 2); Unstructured supplementary services operation - Stage 3”.

[43]	ETS 300 573 (GSM 05.01): “Digital cellular telecommunication system (Phase 2); Physical layer on the radio path  General description”.

[44]	ETS 300 574 (GSM 05.02): “Digital cellular telecommunication system (Phase 2); Multiplexing and multiple access on the radio path”.

[45]	ETS 300 575 (GSM 05.03): “Digital cellular telecommunication system (Phase 2); Channel coding”.

[46]	ETS 300 576 (GSM 05.04): “Digital cellular telecommunication system (Phase 2); Modulation”.

[47]	ETS 300 577 (GSM 05.05): “Digital cellular telecommunication system (Phase 2); Radio transmission and reception”.

[48]	ETS 300 578 (GSM 05.08): “Digital cellular telecommunication system (Phase 2); Radio subsystem link control”.

[49]	ETS 300 579 (GSM 05.10): “Digital cellular telecommunication system (Phase 2); Radio subsystem synchronisation”.

[50]	ETS 300 582 (GSM 07.01): “Digital cellular telecommunication system (Phase 2); General on Terminal Adaption Functions (TAF) for Mobile Stations (MS)”.

[51]	ETS 300 590 (GSM 08.08): “Digital cellular telecommunication system (Phase 2); Mobile Switching Centre - Base Station System (MSC - BSS) interface Layer 3 specification”.

[52]	ETS 300 596 (GSM 08.58): “Digital cellular telecommunication system (Phase 2); Base Station Controller - Base Transceiver Station (BSC - BTS) interface Layer 3 specification”.

[53]	ETS 300 599 (GSM 09.02): “Digital cellular telecommunication system (Phase 2); Mobile Application Part (MAP) specification”.

[54]	ETS 300 607-1 (GSM 11.10-1): “Digital cellular telecommunication system (Phase 2); Mobile Station (MS) conformity specification”.

[55]	ETS 300 607-2 (GSM 11.10-2): “Digital cellular telecommunication system (Phase 2); Mobile Station (MS)  Conformance specification, Protocol Implementation Conformance Statement (PICS) Proforma”.

# 3	Definitions, symbols and abbreviations (Third numbered Clause)

## 3.1	Definitions

For the purpose of this ETS the following definitions apply:

Abstract Test Suite (ATS): 					refer to ISO/IEC 9646-1 [1].

Implementation Under Test (IUT):				refer to ISO/IEC 9646-1 [1].

partial Protocol Implementation eXtra Information for Testing (PIXIT):  refer to ISO/IEC 9646-1 [1].

Point of Controls and Observations (PCO): 		refer to ISO/IEC 9646-1 [1].

Protocol Implementation Conformance Statement (PICS): 	refer to ISO/IEC 9646-1 [1].

System Under Test (SUT):					refer to ISO/IEC 9646-1 [1].

## 3.2	Abbreviations

For the purpose of this ETS the following abbreviations apply:

ATS	Abstract Test Suite

BI	Invalid Behaviour tests

BO	Inopportune Behaviour tests

BV	Valid Behaviour tests

CA	CApability tests

EDP-N	Event Detection Point - Notification

EDP-R	Event Detection Point - Request

ETS	European Telecommunication Standard

FE	Functional Entity

FSM	Finite State Machine

IUT	Implementation Under Test

PDU	Protocol Data Unit

PICS	Protocol Implementation Conformance Statements

PIXIT	Protocol Implementation eXtra Information for Testing

SUT	System Under Test

TP	Test Purpose

TSS	Test Suite Structure

Further abbreviations used within GSM related ETS may be found in ETR 100.

Abbreviations for test case group names may be found in figure 1 and 2.

# 4	Test Suite Structure (TSS)

## 4.1	Test suite naming convention

The test group identifier for each group and subgroup is built according to the scheme in figure 1.

Identifier:	L3<c><s><g0><g1><g2><nn>

<c> = category:	BIT	BIT, Basic Interconnection tests	(not used)
CA	CA, Capability tests	(not used)
BV	BV, Valid Behaviour tests
BI-BO	BI and BO, Invalid and Inopportune Behaviour tests

<g0> = group:	IN	Initial tests
ID	Idle mode tests
RR	Radio Resource management
MM	Mobility Management
CC	Call Control
SP	Structured Procedures
SS	Supplementary  services
LLF	Lower Layer Failures
UUE	Unknown, Unforeseen or erroneous Elements

<g1> = group:	CR	Channel Request
AT	IMSI detach and IMSI attach
SMT	Sequenced MM / CC message transfer 
EC	Establishment Cause
IA	Immediate assignment 
PG	Test of paging 
MR	Test of measurement report 
ASS	Test of the channel 
HO	Test of handover 
FR	Test of frequency redefinition 
CMM	Test of the channel mode modify procedure 
CY	Test of ciphering mode setting 
CM	Test of classmark 
CHR	Test of channel release 
START	Test of starting time 
IDAU	Identification and authentication 
LU	Location updating 
CON	MM connection 
CCSMO	State Machine Verification, Mobile Originating Call
CCSMT	State Machine Verification, Mobile Terminating Call 
SMICF	State machine verification, In Call Functions 
CRE	Call Re-establishment 
UUS	User to user signalling 
UPD	Unknown protocol discriminator 
TIS	TI and skip indicator 
UMT	Undefined or unexpected message type 
UIE	Unforeseen information elements in the non-imperative 
NMIE	Non-semantical mandatory IE errors 
CNR	Unknown IE, comprehension not required 
SB	Spare bits

<g2> = group:	AC	Handover / successful / active call 
CUE	Handover / successful / call under establishment 
FSY	Handover / successful / active call / finely synchronized
PRS	Pre-synchronized handovers	 
PRF	Protocol failures	 
ACC	Location updating / accepted 
REJ	Location updating / rejected 
ABN	Location updating / abnormal cases 
REL	Location updating / release 
PER	Location updating / periodic 
HPER	Location updating / periodic HPLMN search 
IWAT	Location updating / interworking of attach and periodic
EST	MM connection / establishment 
EXP	MM connection / expiry 
NWAB	MM connection / abortion by the network 
FRQP	MM connection / follow-on request pending 
U0	Mobile Originating Call U0 State 
U0.1	Mobile Originating Call U0.1 MM Connection pending 
U1	Mobile Originating Call U1 State 
U3	Mobile Originating Call U3 State 
U4	Mobile Originating Call U4 State 
U10	Mobile Originating Call U10 State 
U11	Mobile Originating Call U11 State 
U12	Mobile Originating Call U12 State 
U19	Mobile Originating Call U19 State 
U0	Mobile Terminating Call U0 State 
U6	Mobile Terminating Call U6 State 
U9	Mobile Terminating Call U9 State 
U7	Mobile Terminating Call U7 State 
U8	Mobile Terminating Call U8 State 
DTMF	State machine verification, In Call Functions / transfer 
CHC	State machine verification, In Call Functions / 
TICM	State machine verification, In Call Functions / in-
OICM	State machine verification, In Call Functions / in-

Figure 1: Test group identifier naming convention scheme.

## 4.2	Suite Overview

Figure 2 shows the structure of the test suites for L3.

| L3 ATS |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BV |  |  |  |  |  |  |  | BI/O |  |  |
| IN | ID | RR | MM | CC | SP | SS | SMS |  | LLF | UUE |

Figure 2: Test suite structure of the L3 tests

## 4.3	Test groups

### 4.3.1	Valid Behaviour tests (BV)

Predefined state transitions are considered as valid. The test purposes in the valid behaviour test subgroup cover the verification of the normal and exceptional procedures of the various Finite State Macnies (FSMs), i.e. a valid behaviour test is a test where the message sequence and the message contents are considered as valid.

### 4.3.2	Invalid Behaviour/Inopportune Behaviour tests (BI-BO)

This test sub group verifies that the Implementation Under Test (IUT) is able to react properly having received an invalid Protocol Data Unit (PDU) or in the case an inopportune protocol event occurs. An invalid PDU is defined as a syntactically incorrect message. An inopportune event is syntactically correct but occurs when it is not expected, e.g. a correctly coded operation is received in a wrong state (the IUT may respond Error UnexpectedComponentSequence).

## 4.4	Test Step Structure

### 4.4.1	Preambles

The preamble is defined for each test purpose.

### 4.4.2	Postambles

After each test case the IUT shall be brought to the state as defined in the postamble for each test purpose.

# 5	Test Purposes (TP)

For each conformance requirement a Test Purpose (TP) is defined. The test purposes are specified in the ATS Dynamic part (annex A).

## 5.1	TP and test case naming convention

In order clearly to map the conformance requirements specified in the prETS 300 607-1 and TTCN test cases in the ATS, the subclause (section) numbers in the prETS 300 607-1 are used as test case names.

The identifier of each TP is identical to the name of the implemented TTCN test case.

“Test Purpose Identifier” = “Test Case Name” = TPI = TC_NN_A_B_C_D_E, where NN, A, B, C, D and E are digits used in the corresponding subclause numbers of prETS 300 607-1. For example, the test case name TC_26_5_6_2 is the TTCN specification corresponding to the conformance requirements in the subclause (section) 26.5.6.1.2 in the prETS 300 607-1. In case where the subclause has been implemented in more than one test case sub numbering has been introduced. For example, the section 26.7.4.2.4 of prETS 300 607-1, Location updating/rejected/national roaming testing is split into 5 test cases. The corresponding TCs are identified as TC_26_7_4_2_4_1, TC_26_7_4_2_4_2, TC_26_7_4_2_4_3, TC_26_7_4_2_4_4 and TC_26_7_4_2_4_5, respectively.

# 6	Abstract test method and test configurations

The distributed test method applies to the L3 MS testing. The test method uses a lower tester and a man-machine interface as an upper tester at the SUT.

## 6.1	Test system model

The model of the L3 test system is based on the original protocol architecture at the air interface. The test system consists conceptually of a lower tester LT, the L3 test programme (executable test suite), a L2 radio link emulator, a management functional unit, the L1 service provider and a TRx set (see fig. 1).

Figure 3: Test system and distributed test method

The LT provides the test environment and for test execution and the means of control and observation at the L3 lower service boundary within the test system.

The L3 TTCN test specification uses the three LT interfaces to communicate with the MS,  the system under test, and with the other parts of the test system:

-	Interface to the L2 emulator via the PCO,

-	Interface to the management functional unit via TTCN test suite operations,

-	Interface to the MS man-machine interface (MMI) via a test operator.

## 6.2	Test Method

The PCO in the LT is defined as L2 SAP (SAP 0 + 3). The PCO has two FIFO queues (data buffers) to store all sending and receiving test events. The L2 primitives in the ATS which constitute mainly the interface to the L2 emulator are specified via the L2 primitives. In order to simulate multicell testing as required in some test cases, the defined primitives are able to address individual cells of the test system and the logic channels of each cell for the L3 message exchanges. The L2 emulator together with the underlying L1 and the TRx set support all message exchanges via correct radio links.

The management function unit has three management functions:

•	L2 and L1 management,

•	Channel management,

•	TRx management.

The interface to the management function unit is presented in the ATS via a set of test suite operations. The major functions of the test suite management operations are:

•	To load configuration parameters necessary for the test system,

•	To control and get the necessary values of radio resources/ channels for tests.

The SUT (MS) has a more or less standardised MMI, such as keys, digital display, tones, etc. The ATS uses such kind of functions to provoke some procedures or to observe simple results at the SUT side. A human operator is needed during the test. The test system shall have an interface to the human operator to enable the test coordination.

Annex A:	Abstract Test Suite