II.5.3.1	Default conditions and structured sequence of tests

II.5.3.1.1	Default test conditions during L3-tests

During tests in section II.5.3 the following default test conditions shall apply if not otherwise stated within the test description. In the table below, decimal values are normally used. Sometimes a hexadecimal value, indicated with a 'H', or a binary value, indicated with a 'B' is given.

General signalling conditions for all carriers

Cyphering                               	yes

General RF-conditions for all carriers

Frequency hopping mode                  	Non-hopping

Propagation profile                     	Static

Downlink Input Level                    	63 dBmicroVolt emf

Uplink output power                     	MinimumMaximum according to MS power class

Serving cell, BCCH/CCCH carrier

Channel                                 	ARFCN 20 600

Alternative channels                    	ARFCN 40 or 60 700 or 800

Serving cell, Traffic channel, SDCCH

Channel                                 	ARFCN 30 650

Alternative channels                    	ARFCN 50 or 70  750 or 850

Power Control Indicator (PWRC)          	0

Neighbouring cells BCCH/CCCH carriers

Channels (ARFCN):                       	10,80,90,100,110,120 520,600,700,780,810,870

Alternative channels:                   	15,85,95,105,115,122 530,610,710,790,820,880

Input level                             	53 dBmicroVolt emf

Network dependent parameters

Cell identity                           	0001H

Mobile country code, MCC                	001  (decimal)

Mobile network code, MNC                	01   (decimal)

Location area code, LAC                 	0001H

BCCH allocation number, BA-NO           	00B   band number 0

BCCH allocation sequence number(BA-IND) 	0

Cell allocation number, CA-NO           	00B   band number 0

PLMN colour code, NCC                   	1

BS colour code, BCC                     	5

SMS Cell Broadcast                      	not active

DTX                                     	MS must not use

IMSI Attach-detach allowed              	0 MS shall not apply

CCCH-CONF                               	set to 1 basic physical channel used for

CCCH,combined with SDCCHS, encoded as 001B

BS-AG-BLKS-RES                          	0 blocks reserved for access grant

BS-PA-MFRMS                             	5 paging subgroups, encoded as 011B

CELL-BAR-ACCESS                         	0 the cell is not barred

Call-reestablishment allowed (RE)       	1

Emergency Call allowed (EC)             	0

Access Control Class (AC) (0..9,11..15) 	0

Network dependent timers

Radio Link Timeout                      	8

T3212 Periodic updating in decihours    	Infinite

Access control parameters

Max retrans  (1, 2, 4 or 7)             	1

Tx-integer, number of slots             	5

CELL-RESELECT-HYSTERESIS                	12 dB

MS-TXPWR-MAX-CCH                        	minimum level

RXLEV-ACCESS-MIN                       	minimum

These informations are provided by system information 1, 2, 3 and 4 messages.

The system information elements which are broadcasted on the SACCH during the dedicated mode should be consistent with those sent on the BCCH when the MS was in idle mode prior to the channel request.

II.5.3.1.2	Structured sequence of the tests

The tests shall be performed in the order as indicated in the following table.

The validity of the tests depends upon the results of the tests performed before.

Channel request (basic test)                       		RR   	II.5.3.2.1

Immediate assignment                               	RR   	II.5.3.6.1

IMSI attach/detach (basic)                         		RR   	II.5.3.2.2

Paging                                             		RR   	II.5.3.6.2

Test of the mobile station functions in idle mode  	RR   	II.5.3.3

Frequency redefinition                             		RR   	II.5.3.6.6

Measurement report (incl. system info not idle)    	RR   	II.5.3.6.3

Authentication                                     		MM   	II.5.3.7.2

Cipher mode setting                                		RR   	II.5.3.6.8

Identification                                     		MM   	II.5.3.7.3

Sequenced MM/CC message transfer                   	..   	II.5.3.2.3

Channel release                                    		RR   	II.5.3.6.12

Location updating                                  		MM   	II.5.3.7.4

TMSI reallocation                                  		MM   	II.5.3.7.1

Classmark change                                   		RR   	II.5.3.6.11

Call controll (verification on CC state diagram)   	CC   	II.5.3.8.1

Call rearrangement                                	 	CC   	II.5.3.8.* missing

DTMF information transfer                          	CC   	II.5.3.8.4

Handover                                           		RR   	II.5.3.6.5

Additional assignment                              		RR   	II.5.3.6.9

Partial release                                    		RR   	II.5.3.6.10

Re-establishment                                   		CC   	II.5.3.8.3

Dedicated channel assignment (during calls)        	RR   	II.5.3.6.4

Emergency call establishment                       	CC   	II.5.3.8.2

Transmission mode change                           	RR   	II.5.3.6.7

Mobility management connection establishment       	MM   	II.5.3.7.5

Test of Layer 3 error handling                     	     	II.5.3.5

User to user signalling                            		CC   	II.5.3.8.5

Testing of structured procedures                   	     	II.5.3.9

II.5.3.1.3	General Rules for message parameters

The following rules concerning message parameters apply to section II.5.3:

1)	Those values of parameters which are a consequence of the context of a test and which are not specific to that test need not be defined.

2)	If the value of a parameter of an uplink message (MS to Network) is specifiedin a test, the implicit meaning is that it has to be checked; if the value is not specified, it is not to be checked unless stated otherwise.

3)	An optional field or optional Information Element of a downlink message (Network to MS), the presence of which is not a consequence of a test description, shall be absent in that test.

4)	If an optional field or Information Element is not indicated for the uplink (MS to Network) - unless specified otherwise -, it may be included or not.

5)	The Protocol Discriminator, Transaction Identifier and Message Type of all uplink messages have to be checked.

II.5.3.1.4	General Rules for L3 Testing

Unless otherwise specified, before the SS pages the MS, the MS must be given the necessary time to camp on the cell (see section II.6.1.6.1)