II.2 	The transceiver

Ref. GSM 05.05-DCS

General

This section addresses those aspects of a transceiver which are broader than only the transmitter or the receiver. It may be noted that frequency hopping and encryption are not tested explicitly. As many of the measurements on the transceiver are carried out whilst frequency hopping and encryption are active it is expected that these aspects will be tested implicitly to a sufficient degree.

However, the ability to switch to frequency hopping or non-hopping, and the ability to change the encryption key mode setting are specifically verified in other chapters.

A special Test-SIM (section III.1) is required throughout this test procedure, and all test methods assume the functions provided by that SIM are available.

II.2.1	Radio frequency aspects

II.2.1.1	Frequency bands				ref: 05.05-DCS clause 2

A GSMDCS 1800 mobile station (M.S.) shall be able to transmit in the frequency band 890 - 9151710-1785 MHz and receive in the frequency band 935 - 9601805-1880 MHz.

II.2.1.2	RF Channels and channel numbering	ref: 05.05-DCS cl.2

The channel spacing shall be 200 kHz.

A GSMDCS 1800 MS shall be equipped to transmit on each of the RF channels within the transmit band.  The nominal centre frequency for each RF channel is given by the following expression:

Fc = 8901710.2 + n x 0.2 *(n-512) MHz, where n ranges from 1512 to 124885.

The value n is called the ABSOLUTE RADIO FREQUENCY CHANNEL NUMBER (ARFCN).

A GSMDCS 1800 MS shall be equipped to receive on each of the RF channels within the receive band.  The nominal centre frequency for each RF channel is given by the following expression:

Fc = 9351805.2 + n x 0.2 * (n-512) MHz. where n ranges from 1512 to 124885.

II.2.1.3	Frequency hopping

Frequency hopping is described in GSM 05.02, section 6.2.

Frequency hopping is an optional feature for the network. For the MS however, this function is mandatory.

The parameters to be used for frequency hopping tests throughout this document are described in Annex 1, Part General Conditions, GC, unless otherwise indicated in the individual tests.

II.2.2	Spurious emissions

II.2.2.1	Definition

Spurious emissions are emissions at frequencies other than those of the carrier and sidebands associated with normal modulation.  The level of these spurious emissions shall be measured as:

(a)Their power level in a specified load.

(b)Their effective radiated power when radiated by the cabinet and structure of the mobile station, including all interconnecting cables.

Note: (b) is also known as "cabinet radiation".

For Mobile Stations having an antenna connector both (a) and (b) shall be measured.

For Mobile Stations with an integral antenna and no means of connecting an external antenna only (b) shall be measured.

For Mobile Stations with an integral antenna and means for connecting an external antenna both (a) and (b) shall be measured.

II.2.2.2	Method of Measurement (a)

II.2.2.2.1	Mobile Allocated Channel

Spurious emissions shall be measured as the power level of any discrete signal, greater than 6dB below the appropriate limit value, delivered into a

50 ohms load. The transceiver is connected to the System Simulator (SS) and the absolute level of any such emission at the connector of the transceiver is measured.

a)The SS shall originate a call to the MS on a TCH/FS in the range ARFCN 60 to 65 690 to 710.

b)The SS shall command the mobile to loop back its channel decoder output to channel encoder input.

c)The SS shall command the MS to operate in encrypted mode, and shall generate Standard Test Signal C1.

d)The MS shall be set to operate at its maximum output power, with DTX off.

e)Measurements are made in the frequency range 100 kHz to 12.75 GHz.

The measurement bandwidth based on a 5 pole synchronously tuned filter shall be according to Table II.2.1. The power indication shall be the peak power detected by the measuring system.

f)The test shall be conducted under normal test conditions and under extreme test conditions (Annex 1, TC). For the extreme test conditions, the following combinations shall be applied:

Temp   	Hi  Hi  Lo  Lo

Volt   	Hi  Lo  Hi  Lo

Table II.2.1

Frequency	Frequency Offset	Filter Bandwidth	Approx Video

Range			Bandwidth

100 kHz

to 50 MHz	-	10 kHz	30 kHz

50 MHz

to 500 MHz	-	100 kHz	300 kHz

500 MHz	>= 2 MHz	30 kHz	100 kHz

to 12.75 GHz	>= 5 MHz	100 kHz	300 kHz

Excl. 890 MHz	>= 10MHz	300 kHz	1 MHz

to 915 MHz	>= 20 MHz	1 MHz	3 MHz

and 935 MHz	> 30 MHz	3 MHz	10 MHz

to 960 MHz

(offset from edge of 		=> 3 MHz

MS TX Band)

and 1710 MHz

to 1785 MHz

and 1805 MHZ

to 1880 MHz

890 1710 MHz	1.8 MHz to 6.0 MHz	30 kHz	100 kHz

to 915 1785 MHz	> 6.0 MHz	100 kHz	300 kHz

(offset from carrier)

Note:	a) Due to practical implementation of a SS, the video bandwidth is restricted to a maximum of 3 MHz.

b) The filter bandwidth for frequencies below 500 MHz is reduced from 3 MHz due to increasing internal noise of available test equipment.

c) The frequency ranges 935 MHz to 960 MHz and 1805 MHz to 1880 MHz are excluded as these ranges are tested in section II.3.4.

II.2.2.2.2	MS in Idle Mode

Spurious emissions shall be measured as the power level of any discrete signal, greater than 6dB below the appropriate limit value.

Precondition:	The BCCH message content from the serving cell shall ensure that the MS DRX function is not activated.

a)The MS is connected to the SS.

b)The MS shall be in idle mode, already camped on the serving cell.

c)The absolute level of all spurious emissions at the antenna connector of the MS shall be measured over the frequency range  100 kHz to 12.75 GHz.

The measurement bandwidth based on a 5 pole synchronously tuned filter shall be according to Table II.2.2. The power indication shall be the peak power detected by the measuring system.

TABLE II.2.2

Frequency range	Filter Bandwidth	Video Bandwidth

100 kHz to 50 MHz	10 kHz	30 kHz

50 MHz to 12.75 GHz	100 kHz	300 kHz

d)The test shall be conducted under normal test conditions and under extreme test conditions (Annex 1, TC). For the extreme test conditions, the following combinations shall be applied:

Temp   	Hi  Hi  Lo  Lo

Volt   	Hi  Lo  Hi  Lo

II.2.2.3	Method of Measurement (b)

On an outdoor test site, fulfilling the requirements of GC4 of Annex 1 or in an anechoic shielded chamber (GC5 of Annex 1), the sample shall be placed at the specified height on the support.

Note:	The test method described has been written for measurement in an anechoic shielded chamber. If an outdoor test site is used then additional precautions are necessary to ensure correct measurement. These measures are familiar to test houses which perform spurious emissions tests and are:

a) Raise/lower the test antenna through the specified   height range during both the emission detection and   substitution parts of the test.

b) Perform a qualitative pre-search in a shielded   environment for test sites where the ambient RF   environment can prevent the detection of spurious   emissions which exceed the limit.

c) Detect emissions at a more sensitive threshold to   that specified in II.2.2.3.1 e) to allow for   destructive interference due to ground plane   reflections at the test antenna search height.

II.2.2.3.1	MS Allocated a Channel

a)The SS shall originate a call to the MS on a TCH/FS in the range ARFCN 60 to 65690 to 710.

b)The SS shall command the MS  to loop back its channel decoder output to its channel encoder input.

c)The SS shall command the MS to operate in encrypted mode and shall generate Standard Test Signal C1.

d)The transmitter is set to its maximum power and DTX off.

e)Initially the test antenna is closely coupled to the MS and any spurious emission radiated by the MS shall be detected by the test antenna and receiver in the range 30MHz to 4GHz.

Note:	This is a qualitative step to identify the frequency and presence of spurious emissions which are to be measured in subsequent steps.

f)The test antenna separation is set to the appropriate measurement distance and at each frequency at which an emission has been detected, the MS shall be rotated to obtain maximum response and the effective radiated power of the emission determined by a substitution measurement. In case of an anechoic shielded chamber pre-calibration may be used instead of a substitution measurement.

g)The measurements shall be repeated with the test antenna in the orthogonal polarisation plane.

h)The measurement bandwidth, based on a 5 pole synchronously tuned filter, shall be according to Table II.2.3.

i)The power indication shall be the peak power detected by the measuring system.

j)The test shall be conducted under normal test conditions and under extreme voltage test conditions (see Annex 1, TC).

TABLE II.2.3

Frequency	Frequency Offset	Filter Bandwidth	Approx Video

Range			Bandwidth

30 MHz

to 50 MHz	-	10 kHz	30 kHz

50 MHz

to 500 MHz	-	 100 kHz 	300 kHz

500 MHz

to 14 GHz	> 2 MHz-	 30 kHz 	100 kHz

Excl. 890 MHz 	>= 5 MHz	100 kHz	300 kHz

to 915 MHz 	>= 10 MHz	300 kHz	1 MHz

>= 20 MHz	1 MHz	3 MHz

1710 MHz	>= 30 MHz	3 MHz	10 MHz

to 1785 MHz	offset from edge 		=> 3 MHz

of band

890 MHz	1.8 MHz to 6.0 MHz 	30 kHz 	100 kHz

to 915 MHz

> 6.0 MHz	100 kHz 	300 kHz

1710 MHz	(offset from carrier)

to 1785 MHz

Note:	a) Due to practical implementation of a SS, the video bandwidth is restricted to a maximum of 3 MHz.

b) The filter bandwidth for frequencies below 500 MHz is reduced from 3 MHz due to increasing internal noise of available test equipment.

II.2.2.3.2	MS in Idle Mode

Precondition:  	The MS shall be in idle mode, already camped on the serving  cell. The BCCH message content from the serving cell shall ensure that the DRX function of the MS is not active.

a)Initially the test antenna is closely coupled to the MS and any spurious emission radiated by the MS shall be detected by the test antenna and receiver in the range 30MHz to 4GHz.

Note: 	This is a qualitative step to identify the frequency and presence of spurious emissions which are to be measured in subsequent steps.

b)The test antenna separation is set to the appropriate measurement distance and at each frequency at which a spurious emission has been detected the MS shall be rotated to obtain a maximum response. The effective radiated power of the emission shall be determined by a substitution measurement. In case of an anechoic shielded chamber pre-calibration may be used instead of a substitution measurement.

The measurement bandwidth based on a 5 pole synchronously tuned filter shall be according to Table II.2.4. The power indication shall be the peak power detected by the measuring system.

TABLE II.2.4

Frequency range	Filter Bandwidth	Video Bandwidth

30 MHz to 50 MHz	10 kHz	30 kHz

50 MHz to 4 GHz 	100 kHz 	300 kHz

c)The measurements shall be repeated with the test antenna in the orthogonal polarisation plane.

d)The power indication shall be the peak power detected by the measuring system.

e)The test shall be conducted under normal test conditions and under extreme voltage test conditions (see Annex 1, TC).

II.2.2.4	Requirements

The power of any spurious emission shall not exceed the values given below:

100 kHz to 1 GHz	1 GHz to 12.75 GHz

Tx operating	-36 dBm 	-30 dBm

(0.25 microwatt)(1 microwatt)

IDLE-mode	57 dBm 	47 dBm

(2 nanowatt)(20 nanowatt)

100 kHz 	1 GHz to 1710 MHz

to 1000 MHz	1785 to 1805 MHz	1710 to 1785 MHz 	1805 to 1880 MHz

1880 MHz to 12.75 MHz

Allocated	- 36 dBm	- 30 dBm	- 36 dBm-	- 30 dBm

Channel

Idle

Mode	- 57 dBm	- 47 dBm	- 57 dBm	- 57 dBm

Note 	For the filter bandwidths quoted in the test method some difficulties may be experienced with noise floor above required measurement limit.  This will depend on the gain of the test antenna and adjustment of the measuring system bandwidth is permissible when carrying out method (b).  Alternatively, for test frequencies above 900 MHz, the test antenna separation from the MS may be  reduced to 1 metre.