II.14	Testing of support for non-transparEnt services

Ref: GSM 04.22 / 07.01 / 07.02 / 07.03 / 09.07

II.14.1	General

The MS is connected to the System Simulator (SS) at the Um interface and to the LTE over the appropriate R or S interface. Tests will be performed under ideal radio conditions.

GSM 11.40 defines the configuration for tests of user data services. User information is transferred over the radio interface between two Layer 2 Relay (L2R) entities in fixed length Protocol Data Units. As these are directly related to the fixed length information fields of the frames transmitted by the Radio Link Protocol (RLP), the tests will be done by analyzing the functionality of the RLP. In particular for testing of RLP the SS shall be able to analyse the RLP frames and all references to RLP messages shall be observed at this point in the SS. There is no requirement for test access to the RLP entity in the MS.

Before testing the L2R and RLP functions the terminal compatibility checking shall be verified.

States referred to in the following tests are those given in the annex of GSM 04.22.

Tests wich validate error free transmission of user data by the RLP require that the MS or the SS shall transmit a continuous known user data pattern. This data is assembled by the RLP into frames, each of which can contain a maximum of 24 characters each contained in 8 bits.

The test configuration depends on whether the MS supports asynchronous data services or synchronous services (or both).

Test configuration for testing the support of asynchronous non-transparent data services:

The MT is connected to an equipment being capable of sending test data which should be the following:

continuously repeated IA5 character string

ABCDEFGHIJKLMNOPQRSTUVWXYZ<123456

carriage return

All the possible parity conditions according to GSM 07.02 Table 4 shall be dealt with correctly, i.e. the respective parity bit offered at the MT interface shall be passed transparently to the SS, even if the parity condition currently set at the MT is violated at the

MT interface.

Note:	This string comprising 33 characters ensures that I frames within a RLP window each contain different information, and can therefore be identified for correct sequencing.

Test configuration for testing the support of synchronous non-transparent data services:

The MT is connected to an equipment simulating LAP B i.e. performing link set-up, link reset, link disconnection and information transfer in both directions.

During the data transfer phase the LAP B equipment has to send test data which should be a continuously repeated HDLC frame.

II.14.2	Scope of the tests

The following tests verify :

i)	the establishment and clearing or the RLP entity and

ii)	error free transmission of user data managed by the ARQ scheme,

iii)	correct synchronization.

In the following procedures the following convention is used:

Message ------>  =  Message from ME to SS

Message <------  =  Message from SS to ME.

II.14.3	Test sequences

Test 1	To verify normal call set-up and establishment of RLP at the request of the MS and that correct synchronization occurs.

Method of Test

Procedure	a)	Set up a call from the ME using the R interface, S interface or MMI.

Requirement	1)	In the case of a V24 'R' interface CT106, CT107 and CT109 shall be 'off'.

Procedure	b)	Following CONN ACK (part of GSM 04.08 procedure), observe SABM ----->.

c)	Within T1 send UA <-----.

Requirement	2)	In the case of a V24 'R' interface CT107 shall be "on".

There shall be no further SABM ---->.

Procedure 	d)	Send DISC <-----.

Requirement	3)	The MS shall send : UA ------>. (This confirms that the MS had entered State 4 (ABM and Connection Established).

Procedure	e)	Terminate call.

Test 2	To verify MS operation if the SS does not respond to MS request for establishment or RLP.

Method of Test

Procedure	a)	Set up a call from the ME using the R interface,

S interface or MMI.

Requirement	1)	Following CONN ACK (part of GSM 04.08 procedure), the MS shall send SABM ---->.

Procedure	b)	The SS shall not respond.

Requirement	2)	At the expiry of T1, the MS shall send SABM ----->.

This shall occur a further 4 times.

3)	Following transmission of the 6th SABM, unacknowledged by the SS, and subsequent expiry of T1, the MS shall automatically terminate the call, using GSM 04.08 procedures.

Test 3	Clash of SABM

Two tests are required to allow for different network implementations.

Test A assumes the network does not respond to SABM ----> with UA <----

if SABM <---- is sent.

Test B assumes the network responds to SABM ----> with UA <---- in spite of having sent SABM <----.

Method of Test (A)

Procedure	a)	Set up a call from the ME using the R interface,

S interface or MMI.

Requirement	1)	Following CONN ACK (part of GSM 04.08 procedure), the MS shall send SABM ---->.

Procedure	b)	send SABM <----.

Requirement	2)	Within T1, the MS shall send UA ---->.

Procedure	c)	Send DISC <----.

Requirement	3)	The MS shall send UA ------>. This confirms that the MS had entered State 4 (ABM and Connection Established).

Procedure	d)	Terminate call.

Method of Test (B)

Procedure	a)	Set up a call from the ME using the R interface,

S interface or MMI.

Requirement	1)	Following CONN ACK (part of GSM 04.08 procedure), observe SABM ---->.

Procedure	b)	Send in consecutive frames SABM <-- followed by

UA <----.

Requirement	2)	Within T1 from SABM ----, the MS shall send

UA ---->.

Procedure	c)	Send DISC <----.

Requirement	3)	Observe UA ---->. This confirms that the MS had entered State 4 (ABM and Connection Established).

Test 4	To verify the MS response to SABM from SS. This test follows requirement 3 of Test 3(B).

Method of Test

Procedure	d)	Send SABM <---- (C/R = 0, P/F = 0).

Requirement	4)	There shall be no response from the MS.

Procedure	e)	Send SABM <---- (C/R = 0, P/F = 1).

Requirement	5)	There shall be no response from the MS.

Procedure	f)	Send SABM <---- (C/R = 1, P/F = 0).

Requirement	6)	There shall be no response from the MS.

Procedure	g)	Send SABM <---- (C/R = 1, PF = 1).

Requirement	7)	The MS shall send UA ---->.

Procedure	h)	send DISC <----.

Requirement	8)	Observe UA ---->. This confirms that the MS had entered State 4 (ABM and Connection Established).

Procedure	i)	Terminate call.

Test 5	To verify MS action on receiving a command to abort establishment of RLP.

Method of Test

Procedure	a)	Set up a call from the ME using the R interface,

S interface or MMI.

Requirement	1)	Following CONN ACK (part of GSM 04.08 procedure), the MS shall send SABM.

Procedure	b)	Send UA <----, followed by DISC <----.

Requirement	2)	The MS shall send UA ---->. This confirms that the MS is now in State 1 (ADM and Attached).

Procedure	c)	Send in consecutive frames, SABM <----

followed by DISC <----.

Requirement	3)	Observe UA ---->.

Procedure	d)	Send DISC <----.

Requirement	4)	The MS shall send DM ---->. This confirms that the MS has remained in State 1 (ADM and Attached).

Procedure	e)	Terminate call.

Test 6	To confirm that the MS remains in the Idle State if the Network is unable to accept a request for establishment of RLP.

Method of Test

Procedure	a)	Set up a call from the ME using the R interface,

S interface or MMI.

Requirement	1)	Following CONN ACK (part of GSM 04.08 procedure), the MS shall send SABM ---->.

Procedure	b)	Within T1 send DM <----.

c)	Send DISC <----.

Requirement	2)	The MS shall send DM ---->. This confirms that the MS has remained in State 1 (ADM and Attached).

Test 7	To confirm correct exchange of XID messages. This test follows Test 6.

Method of Test

Procedure	d)	Send XID <---- using non default parameter values for the window sizes.

Requirement	3	The MS shall send XID ----> with the parameter values of step f) or modified in accordance with GSM 04.22 Table 1.

Procedure	e)	Send <---- SABM.

Requirement	4)	Observe UA ---->.

Procedure	f)	Terminate call.

Test 8	To confirm correct transmission of user data in each direction.

(It is assumed in the description which follows that the window is set

to 31.)

Method of Test

Procedure	a)	Set up a call from the ME using the R interface,

S interface or MMI.

Requirement	1)	Following CONN ACK (part of GSM 04.08 procedure), the MS shall send SABM ---->.

Procedure	b)	Within T1 send UA <----. This exchange of SABM and UA ensures that the sequence of information frames to follow commences at I(0).

c)	Send continuous User Data ----> as defined in Initial Conditions above, starting with ABC ...

(There shall be no change of control signals, thereby ensuring that there is a maximum of 24 user data characters in each I Frame).

Requirement	2)	The MS shall send I(0) ---->, send RR(1) <----.

The MS shall send I(1) ---->, send RR(2) <----.

The MS shall send I(2) ---->, send RR(3) <----.

through to

"   I(31) ---->    "    RR(0) <----

"   I(0)  ---->    "    RR(1) ----

etc.

The user data in these frames shall be :

I(0) :	ABCDEFGHIJKLMNOPQRSTUVWX

I(1) :	YZ123456ABCDEFGHIJKLMNO

I(2) :	PQRSTUVWXYZ123456ABCDEF

etc.

Procedure	d)	Simultaneously with steps a) to c) and

Requirement 2) above, send

I(10) <----, User Data : ABCDEFGHIJKLMNOPQRSTUVWX

I(1)  <----, User Data : YZ 123456ABCDEFGHIJKLMNO

I(2)  <----, User Data : PQRSTUVWXYZ-123456ABCDEF

etc.

The value of the SA and SB bits in the status octet shall be set to '1'.

These information  frames may be coincident with the RR <--- frames of of requirement 2) above; in which case a combined I + S frame shall be sent by the SS.

Requirement	3)	The MS shall send:

RR(1) ---->

RR(2) ---->

RR(3) ---->

etc.

These supervisory frames may be coincident with the I(X) ---->frames of step e) above; in which case a combined I + S frame shall be received by the SS.

The User Data <---- shall be the continuous sequence defined in Initial Conditions above.

Requirement	4)	In the case of a V24 'R' interface CT106 and CT109 shall be 'off'.

Procedure	e)	Set the SA and SB bits of the status octet of the

I frames <--- to 'O'.

Requirement	5)	In the case of a V24 'R' interface CT106 and CT109 shall be set 'on'.

Test 9	To confirm that application of Flow Control at the 'R' interface causes correct setting of status octet bits, and vice-versa.

This test follows on from Test 8 above.

Method of Test

Procedure	f)	Apply Flow Control on the 'R' interface.

Requirement	6)	The MS shall send the initial status octet in the next I Frame --> and the X bit shall be set.

Procedure	g)	Remove Flow Control on the 'R' interface.

Requirement	7)	The MS shall send the initial status octet in the next I Frame ----> and the X bit shall no longer be set.

Procedure	h)	Set X bit of the status octet byte to '1' in the I Frames <----.

Requirement	8)	The MS shall apply flow control on the 'R' interface.

Procedure	i)	Set the X bit of the status octet byte to 'O' in the I Frames <---.

Requirement	9)	The MS shall remove flow control on the 'R' interface.

Test 10	To confirm correct application of RNR under receive buffer full conditions.

This test follows on from Test 9 above.

Method of Test

Procedure	j)	Set local flow control from terminal to mobile over the R interface, the initial status octet in the next I frame ----> should have the X bit set. The system simulator should ignore this flow control setting and continue to send I frames <---- containing user data.

Requirement	10)	When the receive buffer in the mobile becomes full, the MS shall send RNR ---->.

Procedure	k)	The system simulator should then stop sending user data and the local flow control in the mobile should be removed.

Requirement	11)	The user data held in the mobile's receive buffer shall be passed to the 'R' interface without error.

12)	The MS shall send RR ---->.

Test 11	To confirm correct application of Flow Control under transmit buffer half full conditions.

This test follows on from Test 10 above.

Method of Test

Procedure	l)	Send RNR <---- continuously.

Requirement	13)	Observe that Flow Control operates on the 'R'interface (this occurs when the MS transmit buffer becomes half full).

Procedure	m)	Resume sending RR <----.

Requirement	14)	As the buffer empties, observe that flow control on the 'R' interface is removed.

Test 12	To confim that the MS ceases transmission of I frames when the window size for outstanding frames is reached; sends RR until T1 expires, and then sends RR with p=1.

To confirm that transmission of I frames continues after the MS receives RR with F=1 and N(R)=n. If n is the number of the next frame to be transmitted by the MS all frames have been correctly received by the SS and transmission from the MS continues with the next frame. Otherwise frames from I(n) shall be repeated. All values of n are checked.

The tests which follow will require the use of flow control across the user interface.

Method of Test

Procedure	a)	Set up a call from the ME using the R interface,

S interface or MMI.

Requirement	1)	Following CONN ACK (part of GSM 04.08 procedure), observe SABM ---->.

Procedure	b)	Within T1 send UA <----.

c)	Send continuous User Data ----> as defined in Initial Conditions above, starting with ABC... .

Requirement	2)	The MS shall send:

I(n) ---->

I(n+1) ---->

I(n+2) ---->

....

I(n+30) ---->         // The window is now closed.

RR(P=0) ---->         // An RR with P=0 is repeated

//  until T1 expires.

....

RR(P=0) ---->

RR(P=1) ---->         // T1 has expired.

Procedure	d)	The SS selects y from {0,1,...,31} and sends RR(F=1, N(R)=n+31-y)

Requirement	3)	The MS shall send:

I(n+31-y) ----> .

Procedure	e)	The SS shall answer by sending:

RR(n+31-y+1) <----

Requirement	4.1)	The MS shall continue with:

I(n+31-y+1) ----> .

4.2)	In case of repetitions (if y>0), the content (user data) of the repeated frame must be identical to the earlier sent user data!

Procedure	f)	The SS shall select the new n to be n+31-y+1.

Procedure	g)	Repeat step c) to f) and choose different values for y in step d), until all values of {0,1,...,31} have been used.

Requirement	5)	Requirements 3), 4.1) and 4.2) applie for every case in step g).

Test 13	To confirm that correct action is taken by the MS in response to SABM whilst in Data Transfer State, REJect and Selective REJect.

Method of Test

Procedure	a)	Set up a call from the ME using the R interface,

S interface or MMI.

Requirement	1)	Following CONN ACK (part of GSM 04.08 procedure), the MS shall send SABM ---->.

Procedure	b)	Within T1 send UA <----. This exchange of SABM and UA ensures that the sequence of information frames to follow commences at I(0).

c)	Send continuous User Data  ----> as defined in Initial Conditions above, starting with ABC... (There shall be no change of control signals, thereby ensuring that there is a maximum of

24 users data characters in each I Frame).

Requirement	2)	The MS shall send:

I(0) ----> ,   send RR(1) <----,

I(1) ----> ,    "   RR(2) <----,

I(2) ----> ,    "   RR(3) <----,

I(3) ----> ,   send  SABM <----

I(4) ----> ,        }

I(5) ----> ,        } send nothing

I(6) ----> ,        }

I(7) ----> ,        }

I(8) ----> ,        }

I(9) ----> ,        }

Note:	At this point in the procedure frames I(10), I(11) ... I(2) ----> may be received.

Requirement	3)	The MS shall send:

UA. ---->

I(0) ----> ,  send RR(1) <----,

I(1) ----> ,  send RR(2) <----,

etc.

Note:	I(0) now carries other user data compared to the user data in the earlier sent I(0)!

Procedure	d)	Allow the sequence to continue.

Note:	At this point in the procedure, frames I(10), I(11) ... I(2)

may be received.

Observe I(0) ---->, send RR(1) <----,

Observe I(1) ---->, send RR(2) <----,

Observe I(2) ---->, send RR(3) <----,

Observe I(3) ---->,

Observe I(4) ---->,

Observe I(5) ---->,

Observe I(6) ---->,

Observe I(7) ---->,

Observe I(8) ---->,

Observe I(9) ---->, send REJ(3) <----

Requirement	4)	The MS shall send:

I(3) ---->,  send RR(4)

I(4) ---->,  send RR(5)

etc.

4.1)	The user data of these frames shall be identical with that previously sent for the same frame sequence number in step g).

Procedure	e)	Allow the sequence to continue.

.                    .

.                    .

Observe I(0) ---->, send RR(1) <----,

Observe I(1) ---->, send RR(2) <----,

Observe I(2) ---->, send RR(3) <----,

Observe I(3) ---->, send RR(3) <----,

Observe I(4) ---->, send SREJ(3) <----

Requirement	5.1)	The sequential transmission of I frames shall continue with I(3) inserted into the sequence before the sequence reaches I(2).

I(5) ---->,  send RR(3)   <----,

.              .

.              .

I(n)    ---->,  send RR(3)   <----,

I(3)    ---->,  send RR(n+1) <----,

I(n+1)  ---->,  send RR(n+2)<----,

.              .

.              .

.              .

I(2)    ---->,      .

5.2)	The user data in the selective requested and repeated I(3) must be identical to the earlier sent user data in I(3).