II.15 	Tests for mobile stations equipped with a connector, to connect data terminal equipment

Ref: GSM 04.02, 07.01

Note:	The transmission of user data shall be verified in the case where the MS is equipped with a data connector. This could be, for example, a V.24 type of interface or an I.420 (S-bus) connection. In that case the mobile station shall perform a number of functions which shall make it possible to connect data terminal equipment. If the MS is equipped with an I.420 interface, it is possible to connect a standard ISDN terminal adapter (TA) equipment to allow the connection of other types of terminals (V or X series interfaces). As the bit rate on the radio channel is lower than the standard ISDN bit rate, the data terminals or data terminal adapters need to support the standard ISDN rate adaptation for use on the MS's I.420 interface.

II.15.1	Mobile station equipped with a connector accommodating an S-interface (MT1)

Ref: GSM 07.01, 07.02, 07.03, 07.04

(Reserved.)

II.15.2 	Mobile station equipped with a connector accommodating a V-interface (MT2)

Ref: GSM 07.01, 07.02, 07.03

General:

The tests in this section shall only be applied to those Mobile Stations which do have a connector, accommodating a V-interface.

II.15.2.1 	V.25 bis

V.25bis is defined in Fascicle VIII.1 of the CCITT recommendations.  The complete title is "Automatic Calling and/or Answering Equipment on the General Switched Telephone Network (GSTN) Using the 100-series Interchange Circuits".

It defines procedures between data terminal equipment (DTE) and data circuit terminating equipment (DCE) related to mainly automatic call setup and release. An MS using V.25bis will, together with the Interworking Function IWF within the network, act as a DCE towards any DTE capable of handling V.25bis. The following restrictions for the testing apply:

-	Only MS part of the DCE functions is tested.

-	Only the "108/2" mode of operation of the MS is tested meaning MS supporting V.24 signal 108/2 but not 108/1 (in V.25bis referred to as "addressed call and/or answered authorized by DTE mode").

-	Only minimum set of call setup commands and indications as specified by GSM 07.02 is tested.

-	Electrical characteristics according to V.28.

-	Mechanical characteristics up to the manufacturer, but mechanical adapter shall be supplied, if necessary, to the testhouse. The SS/DTE will use ISO 2110 (25 pin DSUB).

┌──────────────┐          ┌──────────────┐          ┌──────────────┐

│      2       │          │      1       │          │      8       │

│  DTE ready   ├─────────>│ DTE not ready├─────────>│ Incoming Call│

│   ON  OFF    │<─────────┤   OFF OFF    │<─────────┤   OFF OFF    │

│   OFF OFF    │          │   OFF OFF    │          │   OFF ON     │

└────┬─────────┘          └──────────────┘          └─┬────────────┘

A    │                                                │

│    │                                                │

│    V                                                V

├──────────────┐          ┌──────────────┐          ┌──────────────┐

│      3       │          │     10       │          │      9       │

│DTE/DCE dialog├─────────>│IC accepting  │<─────────┤IC recognized │

│   ON  OFF    │ <────────┤   ON  OFF    │          │   ON  OFF    │

│   ON  OFF    │          │   ON  ON     │     ┌────┤   OFF ON     │

└─────────┬────┘          └─────────┬────┘     │    └──────────────┘

A    │                    A    │          │

│    │                    │    │          │

│    V                    │    │          │

┌────┴─────────┐               │    │          │    ┌──────────────┐

│      4       │               │    │          │    │      11      │

│Call establish┼───────────────┘    └───────── │ ──>│ IC Accepted  │

│   ON  OFF    │                               │    │   ON  OFF    │

│   ON  OFF    │                               │    │   OFF ON     │

└──────┬───────┘                               │    └──────┬───────┘

│                                       │           │

│                                       │           │

V                                       │           V

┌──────────────┐          ┌──────────────┐     │    ┌──────────────┐

│      5       │          │      6       │     │    │     13       │

│Ans tone detec├─────────>│Call connect  │     └───>│ Line Seized  │

│   ON  OFF    │     ┌────┤   ON  OFF    │<─────────┤   ON  OFF    │

│   OFF OFF    │     │    │   ON  ON     │          │   OFF OFF    │

└──────────────┘     │    └──────────────┘          └──────────────┘

│

│

V

┌──────────────┐    ┌───────────┐   ┌────────────┐   ┌───┐

┌───┐     │     12       │    │     X     │   │     7      ├──>│ 8 │

│ 2 │<────┤Data transfer │    │  Any state├──>│Clear by DTE│   └───┘

└───┘     │   ON  ON     │    │   ON  X   │   │   OFF X    │   ┌───┐

│   X   OFF    │    │   X   X   │   │   X   X    ├──>│ 1 │

└──────────────┘    └───────────┘   └────────────┘   └───┘

The figure is a primitive reproduction of           ┌──────────────┐

Figure 5 in REC CCITT V.25bis. Transition           │  State no    │

from state 12 to 2 is not specified                 │     Name     │

(or tested). Numbers and names according            │ 108/2  107   │

to the figure on the right.                         │  106   125   │

└──────────────┘

Figure 1 :   V.25bis, 108/2-mode, state diagram

II.15.2.2 	Test configuration

During the test, the MS is connected on the R-interface to the Local Terminal Emulator (LTE), and on the Um-interface to the Remote Terminal Emulator (RTE).

R-inter-              Um-inter-

face                   face

──────────────┐      :                      :      ┌─────────────

SS │      :      ┌────────┐      :      │ SS

Local ├──────:──────┤   MS   ├──────:──────┤ Remote

Terminal │      :      └────────┘      :      │ Terminal

Emulator │      :                      :      │ Emulator

──────────────┘      :                      :      └─────────────

DTE- :  DCE-                :

side :  side                :

Figure 2 :  Test Configuration

Requirements on the Um-interface includes Layer 3 messages and parameters. Requirements on the R-interface includes signals (value ON or OFF) and messages (transmitted as characters over the data lines).

On the R-interface, only the following signals are connected and used during the test:

No.     Circuit Name                     Direction

102     Common Return                    DTE <-> DCE

103     Transmitted Data                     -->

104     Received Data                        <--

105     Request To Send                      -->

106     Ready for Sending                    <--

107     Data Set Ready                       <--

108/2   Data Terminal Ready                  -->

109     Data Ch. Rec. Line Sign. Detect.     <--

125     Calling indicator                    <--

II.15.2.3	Test procedures

Test procedures are numbered and have to be performed sequentially.

II.15.2.3.1	Outgoing call, normal outcome

The test simulates a situation where the terminal equipment initiates a call and proceeds until a data transfer occurs.

V.25"        LTE        R-inter-   MS       Um-inter-       SS/RTE

state       (DTE)       face      (DCE)      face

:                    :

1          108/2=OFF  <--+-->  106=107=109    :

1                     <--+-->     =OFF        :

1                        :                    :

2          108/2=ON    --+-->                 :

3                     <--+--   106=ON         :

3                        :                    :

4            ----------!CRN!------->          :

4                        :                    :

4                        :          -------!SETUP!--------->

4                        :                    :

4                        :         <-------!CONN!----------

4                        :                    :

5                     <--+--   106=OFF        :

5                        :                    :

5                        :        I----(synchronization)---I

5                        :                    :

5                     <--+-----106=109=mapped-+------------

5                        :                    :

6                     <--+--   107=ON         :

6                        :                    :

12            I-----------+--(data transfer)---+------------I

Figure 3 :  Mobile originated call, normal outcome

P1	Procedure: 	The MS is listening to the BCCH under default radio conditions. (Editorial: This could be specified in a separate section as done for L3-tests in II.5.3.1.) The MS is connected to the SS via Um and R-interfaces. LTE 108/2=OFF.

Requirement: The MS shall signal 106=107=109=OFF.

P2 	Procedure: LTE signals Data Terminal Ready (108/2=ON). R-interface moved from state 1 to 2.

Requirement: The MS shall signal "Ready for sending" (106=ON) moving the interface from state 2 to 3.

P3 	Procedure: The LTE sends a Call Request With Number Provided to the MS, moving from state 3 to 4. The request shall contain the B-party number used by the SS during the tests.

Requirement: The MS shall send a SETUP message with correct B-party number and service request on the Um-interface.

P4 	Procedure: The call is established.

Requirement: The MS shall reset Ready For Sending (106=OFF) moving the interface from state 4 to 5.

P5 	Procedure: Synchronization is established.

Requirement: The MS shall set Data Set Ready (107=ON) and shall map signal 106 and 109 moving the interface from state 5 to 6.

P6 	Procedure: Data transfer between RTE and LTE in both directions.

Requirement: Correct mapping of data. Checked by SS by comparing received and transmitted data in both RTE and LTE.

Message:      SETUP (GSM 04.08, 9.3.16)

Information element        Comment                             Value

Protocol discriminator     CC                                    001

Transaction identifier     Originating side, value 0            0000

Message type               Setup (note seq variable        0x00 0101

Bearer capabilities        (GSM 04.08, 10.5.4.4)

- IEI                      Bearer capability               0000 0100

- Length of contents       fill in  value                  zzzz zzzz

- Radio ch req             full rate channel                      01

- Coding standard          gsm                                     0

- Transfer mode            Circuit mode                            0

- Info transf capability   Digital information                   100

- Structure                Unstructured                           11

- Dupl mode                Full duplex                             1

- Configuration            Point to Point                          0

- Establishment            Demand                                  0

- Rate adaption            V.100/X.30 rate adaption               01

- Signalling access prot   Not present indicates V.25bis!         -

- User info L1 proto       Default                              0000

- Synch/Asynch             Asynch                                  0

- Negotiation              Inband negotiation not possible         0

- Number of data bits      8 bits or bitoriented                   1

- User rate                Fill in                              zzzz

- Intermediate rate        Not used                               00

- NW indepentent clk tx    Fill in                                 z

- NW indepentent clk rx    Fill in                                 z

- Parity info              Fill in                               zzz

- Number of stop bits      Fill in                                 z

- Connection element       Fill in                                 z

- Modem type               Fill in, V.21, V.22 ...            z zzzz

Mobile identity            (GSM 04.08, 10.5.1.4) (optional!)

- IEI                      Mobile Identity                 0000 0111

- Length                   Enter correct value             zzzz zzzz

- Odd/even indic           indicate even when using TMSI           0

- Type of identity         TMSI                                  100

- Identity digit one       Fill in                              zzzz

..       ..    ..        ..   ..                                ..

- Identity digit p         Last digit                           zzzz

Called party BCD number    (GSM 04.08, 10.5.4.6)

- IEI                      Mobile Identity                 0101 1110

- Length                   Enter correct value             zzzz zzzz

- Type of number           Enter correct type (nat/int..)        zzz

- Numbering plan           ISDN/telephony                       0001

- Number digit one         Fill in                              zzzz

..       ..    ..        ..   ..                                ..

- Number digit p           Last digit                           zzzz

Figure 4 :  Layer 3 SETUP Message used on Um-interface

in Test II.15.2.3.1

II.15.2.3.2 	Call clearing by DTE

Note:	Since call clearing by DTE is not clearly specified in the GSM Standard, in this test procedure MMI intervention has to be used preliminarily for clearing the call.

V.25"        LTE        R-inter-   MS       Um-inter-       SS/RTE

state       (DTE)       face      (DCE)      face

:                    :

12            I-----------+--(data transfer)---+------------I

12                        :                    :

12          108/2=ON   <--+-->   107=ON        :

12                        :                    :

12                        :   (call cleared    :

12                        :    through MMI)    :

12                        :                    :

12                        :          -------!DISC!---------->

12                        :                    :

1                     <--+--  106=107=109     :

1                     <--+--     =125=OFF     :

1                        :                    :

Figure 5 :  Data Connection in Progress, Cleared by DTE

MMI intervention

P7 	Procedure: During data transfer, the call is cleared by MMI intervention.

Requirement: The MS shall make call release and reset all relevant indications (106, 107, 109, 125 all OFF). This moves the interface into state 1.

Message:      DISCONNECT (GSM 04.08, 9.3.7)

Information element        Comment                         Value (1)

Protocol discriminator     CC                                    001

Transaction identifier     Originating side, value 0            0000

Message type               Disc  (note seq variable)       0x10 0101

Cause                      (GSM 04.08, 10.5.4.8)

- IEI                                                      0000 1000

- Length                   Fill in                         zzzz zzzz

- Coding standard          GSM                                    11

- Location                 User                                 0000

- Recommendation           Not present indicates GSM 04.08!        -

- Cause value              No user responding               001 0101

II.15.2.3.3 	Outgoing Call, Radio Resource Failure

The test simulates a situation where the MS does not have any connection on the Um interface.

V.25"        LTE        R-inter-   MS       Um-inter-       SS/RTE

state       (DTE)       face      (DCE)      face

:                    :

1          108/2=OFF  <--+-->  106=107=109    :

1                     <--+-->     =OFF        :

1                        :                    :

2          108/2=ON    --+-->                 :

3                     <--+--   106=ON         :

3                        :                    :

4            ----------!CRN!------->          :

4                        :           (no BCCH on Um-

4                        :              interface)

4           <----------!CFI!-------           :

4                        :                    :

1          108/2=OFF   --+-->                 :

1                        :                    :

Figure 6 :  Outgoing call, RR Failure

P8 	Procedure: The MS is listening to the BCCH under default radio conditions. The MS is connected to the SS via Um and R-interfaces. LTE 108/2=OFF.

Requirement: The MS shall signal 106=107=109=OFF.

P9 	Procedure: LTE signals Data Terminal Ready (108/2=ON). R-interface moved from state 1 to 2.

Requirement: The MS shall signal "Ready for sending" (106=ON) moving the interface from state 2 to 3.

P10 	Procedure: The LTE sends a Call Request With Number Provided (CRN) to the MS, moving from state 3 to 4.  The request shall contain the

B-party number used by the SS during the tests.

Requirement: The MS shall send a "Call Failure Indication" (CFI). The parameter value should be one of the following: "Engaged Tone" (ET), "Answer Tone not Detected" (NT), "Forbidden Call (FC)".

P11 	Procedure: The LTE resets 108/2.

Requirement: The MS shall signal 106=107=109=OFF.

II.15.2.3.4	Outgoing Call, Network Reject

The test simulates a situation where the network rejects the out-going call because the B-party is busy.

V.25"        LTE        R-inter-   MS       Um-inter-       SS/RTE

state       (DTE)       face      (DCE)      face

:                    :

1          108/2=OFF  <--+-->  106=107=109    :

1                     <--+-->     =OFF        :

1                        :                    :

2          108/2=ON    --+-->                 :

3                     <--+--   106=ON         :

3                        :                    :

4            ----------!CRN!------->          :

4                        :                    :

4                        :          -------!SETUP!--------->

4                        :                    :

4                        :         <---!RELEASE COM!-------

4                        :                    :

4           <----------!CFI!-------           :

4                        :                    :

7          108/2=OFF   --+-->                 :

1                     <--+--   106=OFF        :

1                        :                    :

Figure 7 :  Outgoing call, Network Reject

P12 	Procedure: LTE signals Data Terminal Ready (108/2=ON). R-interface moved from state 1 to 2.

Requirement: The MS shall signal "Ready for sending" (106=ON) moving the interface from state 2 to 3.

P13 	Procedure: The LTE sends a Call Request With Number Provided (CRN) to the MS, moving from state 3 to 4.  The request  shall  contain  the B-party number used by the SS during the tests.

Requirement: The MS shall send a "Call Failure Indication" (CFI). The parameter value should be "Engaged Tone" (ET) or "Answer Tone not Detected" (NT).

P14 	Procedure: The LTE resets 108/2.

Requirement: The MS shall signal 106=107=109=OFF.

II.15.2.3.5 	Incoming call, normal data transfer

The test simulates an incoming call to the DTE.

V.25"        LTE        R-inter-   MS       Um-inter-       SS/RTE

state       (DTE)       face      (DCE)      face

:                    :

1          108/2=OFF  <--+-->  106=107=109    :

1                     <--+-->     =OFF        :

1                        :         <-------!SETUP!---------

1                        :                    :

8                     <--+--   125=ON         :

9          108/2=ON    --+-->                 :

10                     <--+--   106=ON         :

10                        :                    :

10           <----------!IC!--------           :

10         (waiting)      :                    :

10                        :                    :

11                     <--+--   106=OFF        :

13                     <--+--   125=OFF        :

13                        :          ----!CALL CONFIRM!----->

13                        :                    :

13                        :        I----(synchronization)---I

13                     <--+-----106=109=mapped-+------------

6                     <--+--   107=ON         :

12            I-----------+--(data transfer)---+------------I

12                        :                    :

Figure 8 :  Incoming call, normal outcome

P15 	Procedure: The SS set up a data call.

Requirement: The MS shall react on the incoming call. The MS signals incoming call (125=ON). The interface is moved into state 8.

P16 	Procedure: The LTE signals Data Terminal Ready (108/2=ON) moving the interface into state 9.

Requirement: The MS sets signal "Ready for Sending"(106=ON) and indicates "Incoming Call" (IC).

P17 	Procedure: LTE waits for 10 s.

Requirement: After 5 to 10 s, the MS shall reset "Ready for sending" (106=OFF) and shall reset "Calling Indicator" (125=OFF). The MS shall confirm the incoming call on the Um-interface.

P18 	Procedure: Synchronization is established.

Requirement: The MS shall map signal 106 and 109 from the RTE to the DTE. The MS shall set signal "Data Set Ready" (107=ON).

P19 	Procedure: The call is cleared by the LTE by resetting "Data Terminal Ready" (108/2).

Requirement: The MS shall signal 106=107=109=OFF.

Message:     CALL CONFIRMED (GSM 04.08, 9.3.2)

Information element        Comment                         Value (1)

Protocol discriminator     CC                                    001

Transaction identifier     Not orig side, value 0               0000

Message type               Call conf(note seq variable)    0x00 1000

II.15.2.3.6	Incoming call, call rejected by DTE

The test simulates an incoming call to the DTE. The call is rejected

by the LTE.

V.25"        LTE        R-inter-   MS       Um-inter-       SS/RTE

state       (DTE)       face      (DCE)      face

:                    :

1          108/2=OFF  <--+-->  106=107=109    :

1                     <--+-->     =OFF        :

1                        :                    :

1                        :         <-------!SETUP!---------

1                        :                    :

8                     <--+--   125=ON         :

9          108/2=ON    --+-->                 :

10                     <--+--   106=ON         :

10                        :                    :

10           <----------!IC!--------           :

10            ----------!DIC!------->          :

10                        :                    :

7          108/2=OFF   --+-->                 :

7                        :                    :

1                     <--+--   106=OFF        :

1                     <--+--   125=OFF        :

1                        :                    :

1                        :          ----!RELEASE!---------->

Figure 9 :  Incoming call, call rejected by DTE

P20 	Procedure: The SS sets up a data call.

Requirement: The MS shall react on the incoming call. The MS signals incoming call (125=ON). The interface is moved into state 8.

P21 	Procedure: The LTE signals Data Terminal Ready (108/2=ON) moving the interface into state 9.

Requirement: The MS set signal "Ready for Sending"(106=ON) and indicates "Incoming Call" (IC).

P22 	Procedure: Within 5 s the LTE answers with "Disregard Incoming Call" (DIC). LTE resets signal "Data Terminal Ready" (108/2=OFF).

Requirement: The MS shall reset "Calling Indicator" (125=OFF). The MS shall reset "Ready for sending" (106=OFF). The MS shall reject the incoming call on the Um-interface.

Message:     RELEASE (GSM 04.08, 9.3.14)

Information element        Comment                         Value (1)

Protocol discriminator     CC                                    001

Transaction identifier     Not orig side, value 0               1000

Message type               Release (note seq variable)     0x10 0101

Cause                      (GSM 04.08, 10.5.4.8)

- IEI                                                      0000 1000

- Length                   Fill in                         zzzz zzzz

- Coding standard          GSM                                    11

- Location                 User                                 0000

- Recommendation           Not present indicates GSM 04.08!        -

- Cause value              No user responding               001 0101

II.15.2.3.7 	Errors in parity or format

The test simulates a situation where the terminal equipment initiates a call, but there is errors in the request from the DTE.

V.25"        LTE        R-inter-   MS       Um-inter-       SS/RTE

state       (DTE)       face      (DCE)      face

:                    :

1          108/2=OFF  <--+-->  106=107=109    :

1                     <--+-->     =OFF        :

1                        :                    :

2          108/2=ON    --+-->                 :

3                     <--+--   106=ON         :

3                        :                    :

4            ---!CRN(parity error)->          :

4                        :                    :

4                        :    (no action!)    :

4                        :                    :

4            ---!CRN(format error)->          :

4                        :                    :

4            <-------!INV!---------           :

4                        :                    :

Figure 10 :  Mobile originated call, errors in format

P23 	Procedure: LTE signals Data Terminal Ready (108/2=ON). R-interface moved from state 1 to 2.

Requirement: The MS shall signal "Ready for sending" (106=ON) moving the interface from state 2 to 3.

P24 	Procedure: The LTE sends a Call Request With Number Provided (CRN) to the MS, moving from state 3 to 4. The request shall contain the

B-party number used by the SS during the tests. In digit number three, a parity error is introduced by reversing the parity bit.

Requirement: During a time period of 3 seconds, the MS shall not send Channel Request on the Um-interface or make any change in state of control lines or exchange any indication on the R-interface.

P25 	Procedure: The LTE sends a Call Request With Number Provided (CRN) to the MS, moving from state 3 to 4. The request shall contain the

B-party number used by the SS during the tests. In the CRN command, the third IA5-letter is changed from the letter N to the letter P with correct parity.

Requirement: During a time period of 3 seconds, the MS shall not send Channel Request on the Um-interface or make any change in state of control lines on the R-interface. The MS shall however send the indication "Invalid" on the R-interface.

II.15.3 	Mobile station equipped with a connector accommodating an X-interface (MT2)

Ref: GSM 07.01, 07.02, 07.03

The tests in this section shall only be applied to those Mobile Stations which do have a connector, accommodating an X-interface.

-	MTs accommodating an X.21 interface shall be tested along section II.15.3.1.

-	MTs accommodating an X.21bis interface shall be tested along section II.15.3.2.

II.15.3.1 	X.21

CCITT Recommendation X.21 defines an interface to be used between Data Terminal Equipment and Data Circuit-terminating Equipment for synchronous operation on Public Data Networks.

GSM 07.02 defines Terminal Adaptation Functions integrated in the Mobile Station which enables the attachment of synchronous terminals.

Together with the Interworking Function within the PLMN, a Mobile Station supporting X.21 Terminal Adaptation Functions, could act as a normal Data Circuit-termination to a Data Terminal Equipment. Mobile Stations supporting this feature is tested with a limited set of specific test procedures. The test procedures verifies that the MS is capable of handling incoming and outgoing calls under normal conditions. Some error handling is also verified.

II.15.3.1.1 	Introduction

X.21 is a high-speed balanced interface between DCE and DTE. The interface is defined in Fascicle VIII.2 of CCITT Recommendations. The full title is "Interface Between Data Terminal equipment (DTE) and Data Circuit-Terminating equipment (DTE) for Synchronous Operation on Public Data Networks.

II.15.3.1.2 	Restrictions on the testing

Only a very limited set of tests is performed. It simulates situations which verifies functions that are regarded to be essential to the operation of X.21 in the PLMN. This includes:

-   Monitoring the radio path while the interface is not used.

-   Normal outcome of incoming and outgoing calls.

-   Loss of radio path during call in progress.

II.15.3.1.3 	Test Conditions

II.15.3.1.3.1 	Mechanical, Electrical and Radio Characteristics

Electrical characteristics shall comply with X.27 with or without cable termination in the load, or with X.26. The SS uses electrical characteristics according to X.27, without cable termination in the load. Mechanical characteristics and contact number assignments according to ISO 4903 (15-pole DTE/DCE interface connector). The test of electrical and/or mechanical characteristics is specified elsewhere.

On the Um interface, default radio conditions as specified in section "Default test conditions during L3-test" is used by the SS.

II.15.3.1.3.2 	Interchange circuits

Interchange circuits G, Ga, T, R C, I, and S are connected during test. Interchange circuits B and X are not connected during test.

┌───────┬───────────────────────────┬───────┬───────┬───────┬───────┐

│Inter- │ Interchange           Used│Data   │Ctrl   │Timing │Other  │

│change │ circuit name         │    │DTE│DCE│DTE│DCE│DTE│DCE│DTE│DCE│

│circuit│                      │    │LTE│MS │LTE│MS │LTE│MS │LTE│MS │

├───────┼──────────────────────┼────┼───┼───┼───┼───┼───┼───┼───┼───┤

│   G   │ Common return        │ X  │   │   │   │   │   │   │ X │ X │

│   Ga  │ TE2 common return    │ X  │   │   │ X │   │   │   │   │   │

│   T   │ Transmit             │ X  │ X │   │   │   │   │   │   │   │

│   R   │ Receive              │ X  │   │ X │   │   │   │   │   │   │

│   C   │ Control              │ X  │   │   │ X │   │   │   │   │   │

│   I   │ Indication           │ X  │   │   │   │ X │   │   │   │   │

│   S   │ Signal element tim.  │ X  │   │   │   │   │   │ X │   │   │

│   B   │ Byte timing          │    │   │   │   │   │   │ X │   │   │

│   X   │ DTE signal elem. tim.│    │   │   │   │   │ X │   │   │   │

└───────┴──────────────────────┴────┴───┴───┴───┴───┴───┴───┴───┴───┘

Figure 11 :  Interchange Circuits

II.15.3.1.3.3 	Test configuration

During the test, the MS is connected on the R-interface to the Local Terminal Emulator (LTE), and on the Um-interface to the Remote Terminal Emulator (RTE).

Requirements on the Um-interface includes Layer 3 messages and parameters. Requirements on the R-interface includes signals (value ON or OFF) and messages (transmitted as characters over the data lines).

R-inter-              Um-inter-

face                   face

──────────────┐      :                      :      ┌───────────────

SS │      :      ┌────────┐      :      │ SS

Local ├──────:──────┤   MS   ├──────:──────┤ Remote

Terminal │      :      └────────┘      :      │ Terminal

Emulator │      :                      :      │ Emulator

──────────────┘      :                      :      └───────────────

DTE- :  DCE-                :

side :  side                :

Figure 12 :  Test Configuration

II.15.3.1.4 	Test procedures

Test procedures are numbered and have to be performed sequentially. A general signalling figure is provided to each test.

┌────┬───────────────┬─────────────────────────────────────────────┬

│X.21│ Inter. Circ.  │             R                Um             │

│stat│ T : C : R : I │SS/LTE action│ MS action      │SS/RTE action │

│ (1)│      (2)      │   (4)       │     (5)        │    (6)       │

├────┼───┬───┬───┬───┼─────────────┼────────────────┼──────────────┤

│ X  │ D │OFF│ + │ X │Set T and C  │T=1, ready      │Default cond. │

│ X  │IA5│OFF│SYN│ON │wait         │                │              │

│    │(3)│(3)│(3)│(3)│             │                │              │

└────┴───┴───┴───┴───┴─────────────┴────────────────┴──────────────┘

Figure 13 :  Definitions in signalling tables

(1)	"X.21 stat." identifies the X.21 Interface Signalling state by the number used in the CCITT specification.

(2)	"Inter. Circ." indicates the status of four of the Interchange Circuits: T=Transmit, C=Control, R=Receive, I=Indication.

(3)	The status is indicated by the following abbreviations:

D		= data signals,

0		= steady binary 0,

1		= steady binary 1,

01		= Alternate binary 0 and binary 1,

X		= any value or  value not checked,

OFF	= continuous OFF (binary 1),

ON		= continuous ON (binary 0),

IA5	= Characters from International Alphabet No 5

(REC CCITT V.3 and X.4),

+		= IA5 character 2/11,

BEL	= IA5 character 0/7,

SYN	= IA5 character 1/6.

(4)	"SS/LTE action" indicates the actions taken by the Local Terminal Emulator in the System Simulator, which during the tests acts like a Data Terminal Equipment towards the MS.

(5)	"MS action" indicates the required actions made by the MS during the tests. The MS shall act as a Data Circuit-terminating equipment towards the LTE during the tests.

(6)	"SS/RTE action" indicates the actions taken on the Um interface by the Remote Terminal Emulator in the System Simulator.

II.15.3.1.4.1 	Initialisation

The test verifies that the MS is able to indicate to the DTE the conditions on the network side (radio path). The MS is connected to the LTE and RTE. The interface is moved into "Ready" state.

┌────┬───────────────┬─────────────────────────────────────────────┐

│X.21│ Inter. Circ.  │             R                Um             │

│stat│ T │ C │ R │ I │SS/LTE action│ MS action      │SS/RTE action │

├────┼───┼───┼───┼───┼─────────────┼────────────────┼──────────────┤

│ X  │ X │ X │ X │ X │  Connect    │  Connect       │  Connect     │

│ X  │ 1 │OFF│ X │ X │ Set T and C │                │ Default cond │

│ X  │ 1 │OFF│ X │ X │  wait 30 s  │  Set R and I   │              │

│ 1  │ 1 │OFF│ 1 │OFF│             │                │              │

└────┴───┴───┴───┴───┴─────────────┴────────────────┴──────────────┘

Figure 14 :  Initialisation

P1 	Procedure:

The MS is connected to the SS on the R- and Um-interfaces under normal conditions according to section "Test conditions" as described before. The MS is switched on, and by using appropriate actions on MMI or EMMI, the MS is put into the same mode a normal user would put it into after connecting a terminal equipment.

Requirement:

30 s after the completion of the procedure. The status of the indication lines shall be C=OFF, T=1.

P2 	Procedure: (reserved)

P3 	Procedure: (reserved)

P4 	Procedure: (reserved)

P5 	Procedure: (reserved)

II.15.3.1.4.2 	Outgoing call, normal outcome, DTE release

The test verifies that the MS is able to handle a DTE originated call request ended with a DTE release. The interface is in state "Ready". The LTE requests a call and transfers the called address. The MS sets up a call to that address on the Um interface. A data session is made. The test ends with the DTE making a release.

┌────┬───────────────┬─────────────────────────────────────────────┐

│X.21│ Inter. Circ.  │             R                Um             │

│stat│ T │ C │ R │ I │SS/LTE action│ MS action      │SS/RTE action │

├────┼───┼───┼───┼───┼─────────────┼────────────────┼──────────────┤

│  1 │ 1 │OFF│ 1 │OFF│             │                │              │

│  2 │ 0 │ON │ 1 │OFF│     T=0     │                │              │

│  3 │ 0 │ON │ + │OFF│             │      R=+       │              │

│  4 │IA5│ON │ + │OFF│     -----!select!---->       │              │

│  5 │ 1 │ON │ + │OFF│     T=1     │                │              │

│  6A│ 1 │ON │SYN│OFF│             │      R=SYN     │              │

│  6A│ 1 │ON │SYN│OFF│             │   ----------!SETUP!----->     │

│  7 │ 1 │ON │IA5│OFF│    <--!progress!-----        │              │

│ 11 │ 1 │ON │ 1 │OFF│             │      R=1       │              │

│ 11 │ 1 │ON │ 1 │OFF│             │   <---------!CONN!-------     │

│ 11 │ 1 │ON │ 1 │OFF│             │   --------!CONN ACK!---->     │

│ 11 │ 1 │ON │ 1 │OFF│             │      <--(synchronization)-->  │

│ 12 │ 1 │ON │ 1 │ON │             │      I=ON      │              │

│ 13 │ D │ON │ D │ON │    <------(data)--->   <---(data)----->     │

│ 16 │ 0 │OFF│ X │ X │ T=0, C=OFF  │                │              │

│ 16 │ 0 │OFF│ X │ X │             │           --!RELEASE!--->     │

│ 17 │ 0 │OFF│ 0 │OFF│             │  R=0, I=OFF    │              │

│ 21 │ 0 │OFF│ 1 │OFF│             │     R=1        │              │

│  1 │ 1 │OFF│ 1 │OFF│T=1          │                │              │

└────┴───┴───┴───┴───┴─────────────┴────────────────┴──────────────┘

Figure 15 :  Outgoing call, normal outcome, DTE release

P6 	Procedure:

The LTE indicates a request for a call by signalling T=0, C=ON, both changes made at the same instant.

Requirement:

The MS shall start transmitting continuously character "+" preceeded by two or more "SYN" on the R circuit with I = OFF.

P7 	Procedure:

The LTE sends a "selection signal sequence" on the T circuit, with C=ON. The sequence contains the called address. The sequence is preceeded by two "SYN". There is no "SYN" between individual selection characters. After selection, the LTE will signal "DTE waiting" with T=1.

Requirement:

The MS shall signal "SYN" on R circuit with i=OFF, indicating DCE waiting. The MS shall also initiate a call setup on the Um interface. After sending SETUP, and received acknowledge, the MS shall indicate connection in progress with R=1. It shall also accept synchronize on the Um-interface. After synchronization, it signals I=ON.

P8 	Procedure:

Data transfer.

Requirement:

Correct mapping.

P9 	Procedure:

The LTE ends the session with T=0, C=OFF.

Requirement:

The MS releases the call. The MS shall confirm the clearing with R=0, I=OFF and then indicate DCE Ready with R=1.

P10 	Procedure:

The LTE indicates Ready with T=1.

Requirement:

No requirement.

The SETUP message sent by the MS during the test is verified by the SS. The content sometimes depends on the environment in which the interface is used, and some of the values (i. e. data rate, connection element) cannot be defined here although they are defined before the test is executed.

Note that information elements may be optional.

Message:      SETUP (GSM 04.08, 9.3.16)

Information element        Comment                             Value

Protocol discriminator     CC                                    001

Transaction identifier     Originating side, value 0            0000

Message type               Setup (note seq variable)       0x00 0101

Bearer capabilities        (GSM 04.08, 10.5.4.4)

- IEI                      Bearer capability               0000 0100

- Length of contents       fill in  value                  zzzz zzzz

- Radio ch req             fill in value

- Coding standard          gsm                                     0

- Transfer mode            Circuit mode                            0

- Info transf capability   Digital information                   100

- Structure                Unstructured                           11

- Dupl mode                Full duplex                             1

- Configuration            Point to Point                          0

- Establishment            Demand                                  0

- Rate adaption            V.100/X.30 rate adaption               01

- Signalling access prot   X.21                                  010

- User info L1 proto       Default                              0000

- Synch/Asynch             Synchronous                             0

- Number of  stop bits     1 stop bit or synchronous               0

- Negotiation              Inband negotiation not possible         0

- Number of data bits      8 bits or bitoriented                   1

- User rate                Fill in                              zzzz

- Intermediate rate        Fill in                                zz

- NW independent clk tx    Fill in                                 z

- NW independent clk rx    Fill in                                 z

- Parity info              None                                  011

- Connection element       Fill in                                 z

- Modem type               Fill in V.21, V.22  ...            z zzzz

Mobile identity            (GSM 04.08, 10.5.1.4) (optional)

- IEI                      Mobile Identity                 0000 0111

- Length                   Enter correct value             zzzz zzzz

- Odd/even indic           indicate even when using TMSI           0

- Type of identity         TMSI                                  100

- Identity digit one       Fill in                              zzzz

..       ..    ..        ..   ..                                ..

- Identity digit p         Last digit                           zzzz

Called party BCD number    (GSM 04.08, 10.5.4.6)

- IEI                      Called party BCD number         0101 1110

- Length                   Enter correct value             zzzz zzzz

- Type of number           Enter correct type (nat/int..)        zzz

- Numbering plan           ISDN/telephony                       0001

- Number digit one         Fill in                              zzzz

..       ..    ..        ..   ..                                ..

- Number digit p           Last digit                           zzzz

Figure 16 :  Layer 3 SETUP Message used in Test II.15.3.1.4.2

II.15.3.1.4.3 	Incoming call, normal outcome, remote release

The test verifies that the MS is able to handle an incoming call. The interface is in "Ready" state. The RTE initiates a call, and the MS is paged on the Um interface. The MS responds and a data transfer sessions is made. The RTE initiates a remote release.

┌────┬───────────────┬─────────────────────────────────────────────┐

│X.21│ Inter. Circ.  │             R                Um             │

│stat│ T │ C │ R │ I │SS/LTE action│ MS action      │SS/RTE action │

├────┼───┼───┼───┼───┼─────────────┼────────────────┴──────────────┤

│  1 │ 1 │OFF│ 1 │OFF│             │         <---!SETUP!----       │

│  8 │ 1 │OFF│BEL│OFF│             │    R=BEL       │              │

│  9 │ 1 │ON │BEL│OFF│C=ON         │                │              │

│  6B│ 1 │ON │SYN│OFF│             │    R=SYN       │              │

│  6B│ 1 │ON │SYN│OFF│             │         ----!CONN!----->      │

│  6B│ 1 │ON │SYN│OFF│             │         <-!CONN ACK!----      │

│  6B│ 1 │ON │SYN│OFF│             │         --!CALL-CONF!-->      │

│  6B│ 1 │ON │SYN│OFF│             │         <-(synchronization)-> │

│ 12 │ 1 │ON │ 1 │ON │             │ R=1, I=ON      │              │

│ 13 │ D │ON │ D │ON │    <------(data)--->   <---(data)----->     │

│ 13 │ D │ON │ D │ON │             │          <--!DISCONN!---      │

│ 19 │ X │ X │ 0 │OFF│             │  R=0, I=OFF    │              │

│ 20 │ 0 │OFF│ 0 │OFF│  T=0, C=OFF │                │              │

│ 21 │ 0 │OFF│ 1 │OFF│             │      R=1       │              │

│  1 │ 1 │OFF│ 1 │OFF│     T=1     │                │              │

└────┴───┴───┴───┴───┴─────────────┴────────────────┴──────────────┘

Figure 17 :  Incoming call, remote release

P11 	Procedure: The interface is in Ready state. The MS is paged and

a SETUP message is transferred to the MS.

Requirement: The MS shall access the radio interface and receive the SETUP message. After reception of the SETUP message, the MS shall indicate Incoming Call (R=BEL).

P12 	Procedure: The LTE indicates Call Accepted (C=ON).

Requirement: The MS shall indicated DCE waiting (R=SYN). On the Um-interface, Call Confirm shall be sent. Synchronization shall take place. After synchronization, the MS shall indicate Ready for Data (I=ON, R=1).

P13 	Procedure:  Data session.

Requirement: Correct mapping.

P14 	Procedure: DISCONN is sent from the RTE.

Requirement: The MS shall indicate DCE Clear Indication (R=0,I=OFF).

P15 	Procedure: The LTE indicates DTE Clear Confirmation (T=0,C=OFF).

Requirement: The MS shall indicate DCE Ready (R=1).

P16 	Procedure: The LTE indicates Ready with T=1.

Requirement: No requirement.

II.15.3.2	CCITT X.21bis compatible MT interface

II.15.3.2.1	Scope

REC CCITT X.21bis describes a high speed, synchronous interface between a DTE and DCE. The requirement for a pseudo-X.21 interface mapped onto the physical circuits of a CCITT V.24 interface was first proposed to support the large investment made, by both end users and network operators, into V.24 style equipment. The X.21bis interface, therefore, allows conventional V.24 equipment to be used in place of true X.21 equipment.

The protocol used in X.21bis is essentially the same as that defined for X.21 except that the V.24 interchange circuits are used instead of the circuits specified by REC CCITT X.21.

II.15.3.2.2	Test Conditions

II.15.3.2.2.1	Mechanical, Electrical and Radio Characteristics

The mechanical characteristics comply wiyh ISO 2110. The electrical characteristics shall comply with V.28 (see section II.15.6.2: Test of CCITT V.28 interface).

II.15.3.2.2.2	Interchange Circuits

The following interchange circuits for the 'R' interface shall be operative during the tests:-

CT102	Reference and protective ground.

CT103	Equivalent to the X.21 T circuit (Data transmitted to the DCE (MT)).

CT104	Equivalent to the X.21 R circuit (Data received from the DCE (MT)).

CT105	Equivalent to the X.21 C circuit, ie.

ON	DTE is in an operable state (Ready).

OFF	DTE is inoperable (Controlled Not Ready).

CT106	ON	DCE (MS) is operable (Ready).

OFF	DCE (MS) is inoperable (Not Ready).

CT107	ON	Ready for Data.

OFF	DCE (MS) Clear Indication or Confirmation.

CT108/1	ON	Call Request by DTE or Call Accepted by DTE.

OFF	DTE Clear Request or DTE Clear Confirmation.

CT108/2	ON	Call Accepted by DTE.

OFF	DTE Clear Request or DTE Clear Confirmation.

CT109	Equivalent to the X.21 I circuit, ie.

ON	DCE (MT) operable.

OFF	DCE (MT) faulty.

CT114	Clock supplied from the DCE (MT) to control the DTE transmitter.

CT115	Clock supplied from the DCE (MT) to control the DTE receiver.

CT125	ON	Incoming Call from DCE (MT).

OFF	Default (no Incoming Call pending).

II.15.3.2.2.3	Test Configuration

For the purposes of the following tests the MT shall be  connected on the 'R' interface to the Local Terminal Emulator (LTE), and on the 'Um' interface to the Remote Terminal Emulator (RTE).

II.15.3.2.3	Test Procedures

The test procedures are to be sequentially performed in strict numerical order. Each test procedure is accompanied by a signalling diagram defining the stimuli and expected responses for the MT under test.

II.15.3.2.3.1	Mobile Originated Call - Auto Calling

The test verifies that the MS is able to handle a DTE originated call request ended with a DTE release. The interface is in state "Ready". The LTE requests a call and transfers the called address. The MS sets up a call to that address on the Um interface. A data session is made. The test ends with the DTE making a release. The signalling diagram below also shows the corresponding X.21 state as the session proceeds.

╔════Ð═══════════════Ð═══════════════════════════════════════╗

║X.21│    Circuit    │            R            Um            ║

║stat│ T   C   R   I │    LTE     │     MT     │    RTE      ║

╠════Ï═══Ð═══Ð═══Ð═══Ï════════════Ï════════════Ï═════════════╣

║  1 │ 1 │OFF│ 1 │OFF│            │            │             ║

║  2 │ 0 │ON │ 1 │OFF│    T=0     │            │             ║

║  3 │ 0 │ON │ + │OFF│            │    R=+     │             ║

║  4 │IA5│ON │ + │OFF│   ------!select!-->     │             ║

║  5 │ 1 │ON │ + │OFF│    T=1     │            │             ║

║  6A│ 1 │ON │SYN│OFF│            │    R=SYN   │             ║

║  6A│ 1 │ON │SYN│OFF│            │ --------!SETUP!----->    ║

║  7 │ 1 │ON │IA5│OFF│<-!call progress!- <-!call proceeding!-║

║    │   │   │   │   │     message             causes        ║

║  6A│ 1 │ON │SYN│OFF│                  ──┐                  ║

║  6A│ 1 │ON │SYN│OFF│                    │ shall be tested  ║

║ 10A│ 1 │ON │IA5│OFF│ <--DCE provided--  │ if implemented,  ║

║    │   │   │   │   │    information     │ otherwise skip   ║

║  6A│ 1 │ON │SYN│OFF│                  ──┘                  ║

║ 11 │ 1 │ON │ 1 │OFF│            │    R=1     │             ║

║ 11 │ 1 │ON │ 1 │OFF│            │ <-------!CONN!-------    ║

║ 11 │ 1 │ON │ 1 │OFF│            │ <-(synchronization)->    ║

║ 12 │ 1 │ON │ 1 │ON │            │    I=ON    │             ║

║ 13 │ D │ON │ D │ON │   <----(data)-->  <---(data)----->    ║

║ 16 │ 0 │OFF│ X │ X │ T=0, C=OFF │            │             ║

║ 16 │ 0 │OFF│ X │ X │            │       --!RELEASE!--->    ║

║ 17 │ 0 │OFF│ 0 │OFF│            │ R=0, I=OFF │             ║

║ 21 │ 0 │OFF│ 1 │OFF│            │    R=1     │             ║

║  1 │ 1 │OFF│ 1 │OFF│    T=1     │            │             ║

╚════¤═══¤═══¤═══¤═══¤════════════¤════════════¤═════════════╝

P1	Procedure: The LTE indicates a request for a call by signalling T=0, C=ON, both changes made at the same instant.

Requirement: The MS shall start transmitting continuously character "+" preceeded by two or more "SYN" on the R circuit with I = OFF.

P2	Procedure: The LTE sends a "selection signal sequence" on the T circuit, with C=ON. The sequence contains the called address.

The sequence is preceeded by two or more "SYN". The period, if any, between individual selection characters shall be filled by characters "SYN" with C=ON.

Requirement: The MS shall signal "SYN" on R circuit with I=OFF, indicating DCE waiting. The MS shall also initiate a call setup on the Um interface.

-	After sending SETUP, the MT shall provide the LTE with the Call progress message(s) which have been supplied by L3 messages (CALL PROCEEDING,...). The DCE may provide further information, some of those may cause the MT to request the related supplementary services, see REC CCITT X.21 Table F1 and GSM 04.08.

-	After synchronisation, on the Um interface, the MS shall signal I=ON.

P3	Procedure: Data transfer.

Requirement: Correct mapping.

P4	Procedure: The LTE ends the session with T=0, C=OFF.

Requirements:

1)	The MS releases the call.

2)	The MS shall confirm the clearing with R=0, I=OFF and then indicate DCE Ready with R=1.

P5 	Procedure: The LTE indicates Ready with T=1.

Requirement: No requirement.

Note: See REC CCITT X.21, Figure B1 for the timer values.

II.15.3.2.3.2	Mobile Terminated Call - Auto Answer

The test verifies that the MS is able to handle an incoming call. The interface is in "Ready" state. The RTE initiates a call, and the MS is paged on the Um interface. The MS responds and a data transfer sessions is made. The RTE initiates a remote release. The signalling diagram below also shows the corresponding X.21 state as the session proceeds.

╔════Ð═══════════════Ð═══════════════════════════════════════╗

║X.21│    Circuit    │            R            Um            ║

║stat│ T   C   R   I │    LTE     │     MS     │    RTE      ║

╠════Ï═══Ð═══Ð═══Ð═══Ï════════════Ï════════════Ï═════════════╣

║  1 │ 1 │OFF│ 1 │OFF│            │    <---!SETUP!---------- ║

║  8 │ 1 │OFF│BEL│OFF│            │    R=BEL   │             ║

║    │   │   │   │   │    <--CT125 ON---       │             ║

║  9 │ 1 │ON │BEL│OFF│    C=ON    │            │             ║

║  6B│ 1 │ON │BEL│OFF│            │            │             ║

║ 10B│ 1 │ON │SYN│OFF│            │  R=SYN ──┐               ║

║ 10B│ 1 │ON │SYN│OFF│            │          │shall be tested║

║ 10B│ 1 │ON │IA5│OFF│   <--DCE provided--   │if implemented,║

║    │   │   │   │   │       information   ──┘otherwise skip ║

║  6B│ 1 │ON │SYN│OFF│            │    ---- -!CONN!--------> ║

║  6B│ 1 │ON │SYN│OFF│            │    <-(synchronization)-> ║

║ 11 │ 1 │ON │ 1 │ON │            │     R=1    │             ║

║ 12 │ 1 │ON │ 1 │ON │            │     I=ON   │             ║

║ 13 │ D │ON │ D │ON │    <---(data)--->  <--(data)--------> ║

║ 13 │ D │ON │ D │ON │            │    <----!RELEASE!------- ║

║ 19 │ X │ X │ 0 │OFF│            │ R=0, I=OFF │             ║

║ 20 │ 0 │OFF│ 0 │OFF│ T=0, C=OFF │            │             ║

║ 21 │ 0 │OFF│ 1 │OFF│            │    R=1     │             ║

║  1 │ 1 │OFF│ 1 │OFF│    T=1     │            │             ║

╚════¤═══¤═══¤═══¤═══¤════════════¤════════════¤═════════════╝

P1	Procedure: The interface is in Ready state. The MS is paged and a SETUP message is transferred to the MS.

Requirement: The MS shall access the radio interface and receive the SETUP message. After reception of the SETUP message, the MS shall indicate Incoming Call by sending (R=SYN,SYN,BEL,..) and setting the CT125 (RI) circuit to the LTE.

P2	Procedure: The LTE indicates Call Accepted (C=ON).

Requirements:

1)	The MS shall indicated DCE waiting (R=SYN).

Note: The DCE may provide further information, some of those may

cause the MT to request the related supplementary services.

2)	On the Um-interface, Call Confirm shall be sent. Synchronization shall take place.

3)	After synchronization, the MS shall indicate Ready for Data (I=ON, R=1).

P3	Procedure: Data session.

Requirement: Correct mapping.

P4	Procedure: RELEASE is sent from the RTE.

Requirement: The MT shall indicate DCE Clear Indication (R=0,I=OFF).

P5	Procedure: The LTE indicates DTE Clear Confirmation. (T=0,C=OFF).

Requirement: The MT shall indicate DCE Ready (R=1).

P6	Procedure: The LTE indicates Ready with T=1.

Requirement: No requirement.

II.15.4 	Two wire analogue interface

Note:	This is not a recognized configuration. An MT, providing such a user (data) interface is not accepted for conformance testing on the basis of these test specifications.

Ref: GSM 02.03, 03.45

II.15.5 	Non-standard Connector

The MT, which uses a proprietary interface to a TE, can only be accepted for conformance testing including that TE. MT+TE will be tested together as an MT0, and the TE with which the MT has been tested is documented very precisely in the test report.

If the user (data) interface differs only mechanically from a public interface, then it can be tested for conformance, provided that the manufacturer provides a mechanical adapter.

II.15.6	Test of R-type interfaces

II.15.6.1	Introduction

The tests in the following sections are on a MT which is equipped with a DCE connector with electrical characteristics stated to be according to CCITT Recommendations V.28, V.10/X.26 and V.11/X.27.

II.15.6.2	TEST OF CCITT V.28 INTERFACE

Initial Conditions :

i)	The MT shall be connected to the System Simulator at the Um interface and to the LTE over the R-Interface.

ii)	Tests will be performed under ideal radio conditions.

iii)	During the measurements, the mobile termination must be powered-on.

II.15.6.2.1	Tests on the receiving lines

The equivalent test circuit is shown in Figure 21 where

i)	EL, RL and CL are the open circuit load voltage (bias), d.c. resistance and capacitance respectively, pretended by the MT at the interchange point ;

ii)	EM is the applied test voltage from the SS LTE controlled by switch SM ;

iii)	V1 is the voltage at the interchange point with respect to signal ground ; and

iv)	IL is the current flowing in the interchange circuit.

The following interchange circuits must be tested :

CT103   -  Transmitted data

CT105   -  Request to send

CT108/2 -  Data terminal ready.

II.15.6.2.1.1	Open-circuit load voltage

Method of Test

a)	Switch SM is set 'open' and the voltage V1 is measured.

Requirement:

1)	│V1│ < 2 volts.

II.15.6.2.1.2	Load d.c. resistance

Method of Test

a)	Switch SM is set 'closed' and EM set to the values given in the following table.

Requirement

1)	The measured value of IL shall be between the corresponding maximum and minimum values.

┌────────────────────────────┬───────────────────────────────┐

│    Test conditions         │         IL (mA)               │

├────────────────────────────┼───────────────────────────────┤

│    EM = + 15 V             │    + 13/7   < IL   < + 17/3   │

│    EM = +  3 V             │    +  1/7   < IL   < +  5/3   │

│    EM = -  3 V             │    -  5/3   < IL   < -  1/7   │

│    EM = - 15 V             │    - 17/3   < IL   < - 13/7   │

└────────────────────────────┴───────────────────────────────┘

EM + - EL max

IL min, max = ───────────────

RL max, min

II.15.6.2.1.3	Load reactance

Method of Test :

a)	The System Simulator shall measure the effective shunt capacitance CL across the interface.

Requirement :

1)	The value shall not exceed 2500 pF ; There shall be no inductive component.

II.15.6.2.1.4	Significant input levels

II.15.6.2.1.4.1	Control Interchange Circuits

The test will be performed with interchanges circuits CT105, then CT108.

Method of Test :

a)	A call shall be established to the SS.

b)	Switch SM is set 'closed' and EM set to the value as given in the following table.

c)	The receiver V.110 frames shall be analysed by the SS.

d)	The test is repeated for different test input conditions:

┌──────────────────────┬────────────────────────┐

│ Test input condition │     Receiver output    │

│                      │     binary condition   │

├──────────────────────┼────────────────────────┤

│    EM = + 15 V       │       0 or ON          │

│    EM = +  3 V       │       0 or ON          │

│                      │                        │

│    EM = -  3 V       │       1 or OFF         │

│    EM = - 15 V       │       1 or OFF         │

└──────────────────────┴────────────────────────┘

Requirement

1)	The bits of the V.110 frame corresponding to the interchange circuit under test shall have the binary value given in the table.

Note:	Generally CT108 = OFF is used to abort or clear the call (V.25bis). In that case V.110 frame will not necessarily be sent by the MT, but the call will be cleared. This call clearing has to be verified. See also the note in section II.15.6.2.2.1.1.

II.15.6.2.1.5	Transmitted Data (CT103)

Method of Test :

a)	A call shall be established to the SS, using a full rate TCH and the highest data rate supported by the MT.

b)	Pseudo random data is presented at the R-interface. The value of EM for this data shall be +/- 3 volts.

c)	The received V.110 frames shall be analysed by the SS.

Requirement :

1)	The bits of the V.110 frame shall correspond to the pseudo random data.

2)	The test is repeated with a value of +/- 15 volts for EM.

II.15.6.2.2	Test on the transmitting lines

The equivalent test circuit is shown in Figure 22, where

i)	E0, R0 and C0 are the open circuit generator voltage, resistance and capacitance presented by the MT at the interchange point ;

ii)	EL, RL and CL are the open circuit load voltage (bias), resistance and capacitance of the load presented by the LTE at the interchange point;

iii)	V1 is the voltage at the interchange point with respect to signal ground; and

iv)	IL is the current flowing in the interchange circuit.

The following interchange circuits are tested :

CT104  -  Received data

CT106  -  Ready for sending

CT107  -  Data set ready

CT109  -  Data channel received line signal detector

CT114  -  Transmitter signal element timing

CT115  -  Receiver signal element timing

CT125  -  Calling indicator.

The interchange circuits CT114 and CT115 are tested only in case of synchronous data services using X.21bis.

II.15.6.2.2.1	Open-circuit generator voltage, short-circuit current and generator resistance

II.15.6.2.2.1.1	Circuits 104, 106, 107, 109 and 125

Method of Test :

a)	A call shall be established to the SS.

b)	V.110 frames shall be generated by the SS and sent over the Um interface with the appropriate bits for each interchange circuit set to 'O' or ON.

c)	The test is repeated with appropriate bits set in the V.110 frames for each interchange circuit set to '1' or OFF, and the value of EL in step d) set to + 15 volts.

Note:	Some states may be interpreted as fault conditions by the MT, ie CT107 = OFF.  See REC CCITT V.110 section 3.6 and

REC CCITT V.28 section 7.

Requirement :

1)	With no load applied by the LTE (SL open), │V1│ shall not exceed

25 volts.

2)	With SL closed, EL set to -15 volts and RL set zero, │IL│ shall not exceed 0.5 Amps.

3)	With SL closed, EL set to zero and RL set to 3000 ohms, │V1│ shall not be less than 5 volts.

4)	With SL closed, EL set to zero and RL set to 7000 ohms, │V1│ shall not be greater than 15 volts.

II.15.6.2.2.1.2	Circuits 114 and 115

This test is only necessary in case of synchonous data services using

X.21bis.

Method of test :

a)	See steps a) and b) of II.15.6.2.2.1.1.

Requirement :

1)	The MT shall always when powered on and configured to support

X.21bis, provide a clock on circuits CT114 and CT115, with a rate according to the data rate of the attached DTE.

2)	With no load applied by the LTE (SL open), │V1│ shall not exceed

25 volts.

3)	With SL closed, EL set to -15 volts and RL set zero, │IL│ shall not exceed 0.5 Amps.

4)	With SL closed, EL set to +15 volts and RL set zero, │IL│ shall not exceed 0.5 Amps.

5)	With SL closed, EL set to zero and RL set to 3000 ohms, │V1│ shall not be less than 5 volts.

6)	With SL closed, EL set to zero and RL set to 7000 ohms, │V1│ shall not be greater than 15 volts.

II.15.6.2.2.2	Generator resistance - power-off condition

Method of test :

a)	The MT shall be powered-off.

Requirement :

1)	With SL closed, EL set to +2 volts and RL set zero, │IL│ shall not exceed 6.67 mA.

2)	The test is repeated with EL set to -2 volts.

II.15.6.2.2.3	Dynamic characteristics

II.15.6.2.2.3.1	Performance in Transition Region and Rise/Fall Time

Definitions :

1)	The region between +3 volts and -3 volts is defined as the

transition region.

2)	The rise/fall time (ttrf) is the time required to pass through the transition region from one binary state to the other.

Method of Test :

a)	A call shall be established to the SS.

b)	V.110 frames shall be generated by the SS and sent over the Um interface with the appropriate bits for each interchange circuit toggling between 'O' and '1'.

Requirement :

1)	With RL set to 7000 ohms and CL set to 2500 pF all interchange signals shall proceed from one signal state through the transition region to the opposite signal state without re-entering this region until the next significant change of signal condition. There shall be no reversal of the direction of voltage change while the signal is in the transition region.

2)	For Control Interchange circuits, (CT106, CT107, CT109 and CT125),

200 ns < ttrf < 1 ms.

3)	For Timing and Data Interchange circuits, (CT104, CT114 and CT115), ttrf shall comply with the following table.

┌───────────┬──────────┬────────────┐

│ Data rate │ TRF min  │   TRF max  │

├───────────┼──────────┼────────────┤

│   75 Baud │  200 ns  │ 400.000 µs │

│  300 Baud │  200 ns  │ 100.000 µs │

│ 1200 Baud │  200 ns  │  25.000 µs │

│ 2400 Baud │  200 ns  │  12.500 µs │

│ 4800 Baud │  200 ns  │   6.250 µs │

│ 9600 Baud │  200 ns  │   3.125 µs │

└───────────┴──────────┴────────────┘