II.5.3.6	The elementary procedures for radio resource management

II.5.3.6.1	Immediate assignment

II.5.3.6.1.1	Introduction

The immediate assignment is used by the network to allocate Radio Resource to a MS requesting service. This is necessary to establish a dedicated control channel for the MS and network to communicate the detail of the MS service requested. The immediate assignment is described in section 3.3.1 of GSM 04.08.

II.5.3.6.1.2	Purpose of the tests

The purpose of these tests is to verify that the MS can :

-	correctly set up the dedicated control channel (SDCCH or TCH/FACCH),

-	correctly identify its own assignment in case of an extended assignment,

-	accept an assignment rejection,

-	ignore an assignment for another MS while waiting for an assignment of its own,

-	correctly react in the case of many random accesses, it must react only to Immediate Assignment messages including one of its three last request references and ignore the previous ones.

At the end of each test, the SS releases the dedicated resource when it has been successfully established.

II.5.3.6.1.3	SDCCH Assignment

II.5.3.6.1.3.1	Initial conditions

CCCH_CONF is set to non combined case.

The MS has successfully performed a location update.

II.5.3.6.1.3.2	Procedure

a)	The SS sends a Paging Request with correct TMSI.

b)	The SS draws a random value between 0 and 7, coded as TTT.

c)	The SS responds to the MS Channel Request with an immediate assignment message allocating a SDCCH.

Message:     IMMEDIATE ASSIGNMENT

Information Element	Comment	Value

Protocol Discriminator	RR man. message		0110

Transaction Identifier	not relevant	0000

Message Type	immediate assignment	0011	1111

Page Mode	normal paging	00

Channel Description	SDCCH /8	01TT	Txxx

Request Reference

Timing Advance	30 bit periods	011110

Mobile Allocation	empty

Starting Time	optional field

II.5.3.6.1.3.3	Requirement

1)	The MS shall respond to the Paging Request message by sending a Channel Request message with establishment cause set to "Answer to Paging".

2)	The MS goes to the correct SDCCH indicated in the Immediate Assignment message and sends a Paging Response message.

Message:     PAGING RESPONSE

Information Element	Comment	Value

Protocol Discriminator	RR		0110

Transaction Identifier	not relevant	0000

Message Type	paging response	0010	0111

Ciph. key sequence number	not relevant	000

Mobile Station Classmark 2	as specified by the manufacturer

Mobile Identity	TMSI

II.5.3.6.1.4	TCH Assignment

Max_retrans is set to 1 (coding 00).

CCCH_CONF is set to non combined.

II.5.3.6.1.4.1	Procedure

a)	The SS sends a Paging Request with the correct TMSI.

b)	The SS responds to the MS Channel Request with an Immediate Assignment message allocating a TCH.

Message:     IMMEDIATE ASSIGNMENT

Information Element	Comment	Value

Protocol Discriminator	RR man. message		0110

Transaction Identifier	not relevant	0000

Message Type	immediate assignment	0011	1111

Page Mode	normal paging	00

Channel Description	Bm+ACCHs	0000	1xxx

Request Reference

Timing Advance	30 bit periods	011110

Mobile Allocation	empty

Starting Time	optional field

II.5.3.6.1.4.2	Requirements

1)	The MS shall respond to the Paging Request message by sending a Channel Request message with establishment cause set to "Answer to Paging".

2)	The MS shall go to the correct TCH indicated in the Immediate Assignment message.

3)	The MS shall send a Paging Response message on the FACCH.

Message:     PAGING RESPONSE

Information Element	Comment	Value

Protocol Discriminator	RR		0110

Transaction Identifier	not relevant	0000

Message Type	paging response	0010	0111

Ciph. key sequence number	not relevant	000

Mobile Station Classmark 2	as specified by the manufacturer

Mobile Identity	TMSI

II.5.3.6.1.5	Extended Assignment

Initial conditions

CCCH-CONF is set to combined case.

The maximum number of retransmissions Max-retrans shall be set to 7.

II.5.3.6.1.5.1	Procedure 1

a)	The SS sends a Paging Request with correct TMSI.

b)	The SS draws a random integer value n between 1 and 8, and another value between 0 and 3, coded as TT.

c)	Immediately after having received n Channel Requests, the SS responds to the MS with an Immediate Assignment Extended message allocating an SDCCH.

Message:     IMMEDIATE ASSIGNMENT EXTENDED

information Element	Comment	Value

Protocol Discriminator	RR man.	0110

Transaction Identifier	not relevant	0000

Message Type	immediate assignment extended	0011	1001

Page Mode	normal paging	00

Channel Description 1	} SDCCH/4	001T	Txxx

Request Reference 1	} for MS under test

Timing Advance 1	} see the following note

Channel Description 2	}

Request Reference 2	} arbitrary (not MS under test)

Timing Advance 2	}

Mobile Allocation

Starting Time.

Note:	The request reference is the one which pertains to the i-th Channel Request sent by the MS, where i is an integer between max(1,n-2) and n, its value being randomly drawn by the SS.

II.5.3.6.1.5.2	Requirements 1

1)	The MS shall respond to the Paging Request message by sending a Channel Request message with establishment cause set to "Answer to Paging".

2)	The MS goes to the correct SDCCH indicated in the Immediate Assignment message and sends a Paging Response message.

Message:     PAGING RESPONSE

Information Element	Comment	Value

Protocol Discriminator	RR		0110

Transaction Identifier	not relevant	0000

Message Type	paging response	0010	0111

Ciph. key sequence number	not relevant	000

Mobile Station Classmark 2	as specified by the manufacturer

Mobile Identity	TMSI

II.5.3.6.1.5.3	Procedure 2

a)	The SS sends a Paging Request with the correct TMSI.

b)	The SS draws a random integer value n between 4 and 8, and another value between 0 and 3, coded as TT.

c)	Immediately after having received n Channel Requests, the SS responds to the MS with an Immediate Assignment Extended message allocating an SDCCH.

Note:	The SS will allow 30 seconds between this test and the following test, for the MS to complete the channel request procedure and to perform cell reselection.

Message:     IMMEDIATE ASSIGNMENT EXTENDED

information Element	Comment	Value

Protocol Discriminator	RR man.	0110

Transaction Identifier	not relevant	0000

Message Type	immediate assignment extended	0011	1001

Page Mode	normal paging	00

Channel Description 1	} SDCCH/4	001T	Txxx

Request Reference 1	} for-MS under test

Timing Advance 1	} see the following note

Channel Description 2	}

Request Reference 2	} arbitrary (not MS under test)

Timing Advance 2	}

Mobile Allocation

Starting Time.

Note:	The request reference is the one which pertains to the i-th Channel Request sent by the MS, where i is an integer between 1 and n-3, its value being randomly drawn by the SS.

II.5.3.6.1.5.4	Requirements 2

1)	The MS shall respond to the Paging Request message by sending a Channel Request message with establishment cause set to "Answer to Paging".

2)	The MS shall ignore the immediate assignment.

II.5.3.6.1.5.5	Procedure 3

a)	The SS draws:

a random integer value n between 4 and 8

a random integer value S between 0 and 7, and

a random integer value l between n-2 and n.

b)	The SS sends a PAGING REQUEST message addressing the MS with the correct TMSI.

c)	Immediately after having received n CHANNEL REQUEST messages, the SS responds to the MS with an IMMEDIATE ASSIGNMENT EXTENDED message allocating in the "Channel description 2" Information Element the SDCCH number S of an SDCCH(8).

The "Request reference 2" Information Element refers to the l-th Channel Request sent by the MS.

II.5.3.6.1.5.6	Requirements 3

1)	After step b) the MS shall respond to the PAGING REQUEST message by sending between n and (Max retrans) + 1 CHANNEL REQUEST messages with establishment cause set to "Answer to Paging)

2)	After step c) the MS shall go to the correct SDCCH indicated in the IMMEDIATE ASSIGNMENT EXTENDED message and send a PAGING RESPONSE message.

II.5.3.6.1.6	Assignment Rejection

Initial condition

Max-retrans is set to 7.

II.5.3.6.1.6.1 	Procedure 1

a)	The SS sends a Paging Request with correct TMSI.

b)	x is randomly drawn between 1 and 255 s by the SS. n is an integer randomly drawn by the SS between 1 and 8.

c)	Immediately after having received n channel requests, the SS responds to the MS with an Immediate Assignment Reject message and continues to send Paging messages for more than x seconds.

Message:     IMMEDIATE ASSIGNMENT REJECT

Information Element	Comment	Value

Protocol Discriminator	Radio Resource management		0110

Transaction Identifier	Not relevant	000

Message Type	immediate assignment reject	0011	1010

Page Mode	Normal paging	00

Request Reference

Wait Indication	not the MS under test

Request Reference	MS

Wait Indication	x seconds (maximum=255)

Request Reference

Wait Indication	not the MS under test

Request Reference

Wait Indication	not the MS under test

Note:	The request reference is the one which pertains to the i-th Channel Request sent by the MS, where i is an integer between max(1,n-2) and n, its value being randomly drawn by the SS.

II.5.3.6.1.6.2	Requirements 1

1)	The MS shall respond to the Paging Request message by sending a Channel Request message with establishment cause set to "Answer to Paging".

2)	After the reception of IMMEDIATE ASSIGNMENT REJECT, the MS shall not transmit for x seconds, and then answer to the paging request.

II.5.3.6.1.6.3	Procedure 2

a)	The SS responds to the MS Channel Request with an Immediate Assignment Reject message.

b)	The MS is made to perform a cell reselection.

Message:     IMMEDIATE ASSIGNMENT REJECT

Information Element	Comment	Value

Protocol Discriminator	Radio Resource management		0110

Transaction Identifier	Not relevant	000

Message Type	immediate assignment reject	0011	1010

Page Mode	Normal paging	00

Request Reference

Wait Indication	not the MS under test

Request Reference

Wait Indication	not the MS under test

Request Reference	MS

Wait Indication	255 seconds

Request Reference

c)	The SS pages the MS in the new cell.

II.5.3.6.1.6.4	Requirement 2

1)	The MS shall respond to the Paging Request message by sending a Channel Request message with establishment cause set to "Answer to Paging".

2)	The MS shall not transmit until the cell reselection, and after that it shall answer to the paging.

II.5.3.6.1.7	Ignore Assignment for another MS

II.5.3.6.1.7.1	Procedure

a)	The SS sends a Paging Request with the correct TMSI.

b)	The SS responds to the MS Channel Request with an Immediate Assignment message allocating a SDCCH, but to a different Request Reference:

b.1)	wrong request reference = actual frame number +2 and correct random access info,

b.2)	wrong request reference = actual frame number and wrong random access info.

Message:     IMMEDIATE ASSIGNMENT

Information Element	Comment	Value

Protocol Discriminator	RR man. message		0110

Transaction Identifier	not relevant	0000

Message Type	immediate assignment	0011	1111

Page Mode	normal paging	00

Channel Description	SDCCH/8	01TT	Txxx

Request Reference	Wrong request reference

Timing Advance	30 bit periods	011110

Mobile Allocation	empty

Starting Time	optional field

II.5.3.6.1.7.2	Requirement

1)	The MS shall respond to the Paging Request message by sending a Channel Request message with establishment cause set to "Answer to Paging".

2)	The MS shall continue sending its Channel Requests and not go to the assigned SDCCH.

II.5.3.6.2	Test of paging

The paging procedures are specified in sections 3.3.2 of GSM 04.08 and in section 6.5 of GSM 05.02.

Note:	Mobile Identity information fields, not specified in the method of test, shall either be non-existent or not address the mobile under test.

II.5.3.6.2.1 	Normal paging

II.5.3.6.2.1.1  Purpose

To test that the MS is able to determine its CCCH group and paging group correctly and that the MS will respond to a PAGING REQUEST message (page mode set to "normal paging").

II.5.3.6.2.1.2  Initial conditions

1)	The SS simulates a BSS. The SYSTEM INFORMATION messages on the BCCH are indicating a cell with: Max-retrans is set to 2 (encoded as 01). A legal combination of parameters (CCCH-CONF,BS-AG-BLKS-RES, BS-PA-MFRMS) has to be chosen at the beginning of each following test case.

2)	The MS is in idle updated state. It has been switched on 90 seconds ago. The SS knows the MS's IMSI and the MS has been assigned a TMSI.

Note:	The 90 seconds are needed by the MS, so that it can acquire the synchronisation of the six strongest BCCHs of its list and also read the SYSTEM INFO of these BCCHs (required in GSM 05.08, section 6.6.1 "Monitoring of Received level and BCCH data"). After this initialisation has been performed there remains a probability that a correct MS will be rejected because of non reception of a paging request message. This probability will be less than 0.04 %.

II.5.3.6.2.1.3	Procedure 1: Paging request type 1

a)	The SS sends a PAGING REQUEST TYPE 1 message on the paging subchannel which corresponds to the MS's IMSI. The page mode is set to "normal paging". The test is carried out for each of the following 6 cases, as there is 6 times a return from step d) to step a).

Message:     PAGING REQUEST TYPE 1

Information Element	Comment	Value

Protocol Discriminator	RR management		0110

Transaction Identifier	not relevant	0000

Message Type	Paging Request type 1	0010 0001

Page mode	Normal paging	0000

Mobile identity	Mandatory

Mobile identity	Optional

Normal cases

a.1)	The MS is addressed with its IMSI which is conveyed in the first Mobile Identity information field. The optional Mobile Identity information element is not present.

a.2)	The MS is addressed with its TMSI which is conveyed in the first Mobile Identity information field. The optional Mobile Identity information element specifies an IMSI different from that of the MS's.

a.3)	The first Mobile Identity information element specifies a TMSI different from that of the MS. The optional Mobile Identity information element addresses the MS by its IMSI.

a.4)	The first Mobile Identity information element specifies a TMSI which is different from that of the MS. The optional Mobile Identity information element contains the correct TMSI for the MS.

a.5)	The MS is addressed in the first Mobile Identity field with its IMEI. The optional Mobile Identity information element is not included in the Paging Request message.

a.6)	The MS is addressed in the first Mobile Identity field with its TMSI, but the type of identity in this field is set to "No Identity". The optional Mobile Identity information element is not included in the Paging Request message.

b)	The SS provides a positive response to the 2nd channel request, i.e. the SS responds with an immediate assignment.

Message:     IMMEDIATE ASSIGNMENT

Information Element	Comment	Value

Protocol Discriminator	RR management		0110

Transaction Identifier	not relevant	0000

Message Type	Immediate Assignment	0011	1111

Page mode	normal paging		  00

Channel Description	SDCCH/8	01TT	Txxx

TTT is value randomly chosen between 0 and 7

Request Reference

Timing Advance	30 bit periods	  01	1110

Mobile Allocation	empty

Starting Time	optional field

c)	The SS receives a Paging Response message in each of the test cases a.1) to a.4).

d)	For test cases a.1) to a.4) the SS sends a Channel Release message. For test cases a.5) and a.6) the SS waits 1 second to confirm that the MS does not respond to the paging.

Message:     CHANNEL RELEASE

Information Element	Comment	Value

Protocol Discriminator	RR management		0110

Transaction Identifier	not relevant	0000

Message Type	Channel Release	0000	1101

RR cause	Normal release	0000	0000

II.5.3.6.2.1.4	Requirements 1

1)	For cases a.1) to a.4) the MS shall respond with CHANNEL REQUEST (ESTABL. CAUSE = Answer to paging) until the SS answers. The number of CHANNEL REQUEST messages is limited by the parameter Max-retrans.

For case a.5) and a.6) the MS shall ignore the message.

2)	In each of the test cases a.1) to a.4), the MS shall send a PAGING RESPONSE message on the channel assigned by the SS.

Message:     PAGING RESPONSE

Information Element	Comment	Value

Protocol Discriminator	RR management		0110

Transaction Identifier	not relevant	0000

Message Type	Paging Response	0010 0111

Ciphering key seq. number	not relevant	000

Mobile Station classmark 2	according to manufacturer specification

Mobile Identity	TMSI

II.5.3.6.2.1.5	Procedure 2: PAGING REQUEST TYPE 2

a)	The SS sends a PAGING REQUEST TYPE 2 message on the paging subchannel which corresponds to the MS's IMSI. The page mode is set to "normal paging". The test is carried out for each of the following 6 cases, as there is 6 times a return from step d) to step a).

Message:     PAGING REQUEST TYPE 2

Information Element	Comment	Value

Protocol Discriminator	RR management		0110

Transaction Identifier	not relevant	0000

Message Type	Paging Request type 2	0010 0010

Page mode	normal paging	0000

TMSI	Mandatory

TMSI	Mandatory

Mobile Identity	Optional

Normal cases

a.1)	The MS is addressed in the first TMSI field.

a.2)	The MS is addressed in the second TMSI field.

a.3)	The MS is addressed in the Mobile Identity information field with its TMSI.

a.4)	The MS is addressed in the Mobile Identity information field with its IMSI.

a.5)	The MS is addressed in the optional Mobile Identity field with its IMEI.

a.6)	The MS is addressed in the optional Mobile Identity field with its TMSI, but the type of identity in this field is set to "No Identity".

b)	The SS provides a positive response to the 2nd channel request, i.e. the SS responds with an immediate assignment.

Message:     IMMEDIATE ASSIGNMENT

Information Element	Comment	Value

Protocol Discriminator	RR management		0110

Transaction Identifier	not relevant	0000

Message Type	Immediate Assignment	0011	1111

Page mode	normal paging		  00

Channel Description	SDCCH/8	01TT	Txxx

TTT is value randomly chosen between 0 and 7

Request Reference

Timing Advance	30 bit periods	  01	1110

Mobile Allocation	empty

Starting Time	optional field

c)	The SS receives a Paging Response message in each of the test cases a.1) to a.4).

d)	For test cases a.1) to a.4) the SS sends a Channel Release message. For test cases a.5) and a.6) the SS waits 1 second to confirm that the MS does not respond to the paging.

Message:     CHANNEL RELEASE

Information Element	Comment	Value

Protocol Discriminator	RR management		0110

Transaction Identifier	not relevant	0000

Message Type	Channel Release	0000	1101

RR cause	Normal release	0000	0000

II.5.3.6.2.1.6	Requirements 2

1)	For cases a.1) to a.4) the MS shall respond with CHANNEL REQUEST (ESTABL. CAUSE = Answer to paging) until the SS answers. The number of CHANNEL REQUEST messages is limited by the parameter Max-retrans.

For case a.5) and a.6) the MS shall ignore the message.

2)	In each of the test cases a.1) to a.4), the MS shall send a PAGING RESPONSE message on the channel assigned by the SS.

Message:     PAGING RESPONSE

Information Element	Comment	Value

Protocol Discriminator	RR management		0110

Transaction Identifier	not relevant	0000

Message Type	Paging Response	0010	0111

Ciphering key seq. number	not relevant	000

Mobile Station classmark 2	according to manufacturer specification

Mobile Identity	TMSI

II.5.3.6.2.1.7	Procedure 3: PAGING REQUEST TYPE 3

a)	The SS sends a PAGING REQUEST TYPE 3 message on the paging subchannel which corresponds to the MS's IMSI. The page mode is set to "normal paging". The test is carried out for each of the following 4 cases, as there is 4 times a return from step d) to step a).

Message:     PAGING REQUEST TYPE 3

Information Element	Comment	Value

Protocol Discriminator	RR management		0110

Transaction Identifier	not relevant	0000

Message Type	Paging Request type 3	0010	0100

Page mode	normal paging	0000

TMSI	Mandatory

TMSI	Mandatory

TMSI	Mandatory

TMSI	Mandatory

Normal cases

a.1)	The MS is addressed in the first TMSI field.

a.2)	The MS is addressed in the second TMSI field.

a.3)	The MS is addressed in the third TMSI field.

a.4)	The MS is addressed in the fourth TMSI field.

b)	The SS provides a positive response to the 2nd channel request, i.e. the SS responds with an immediate assignment.

Message:     IMMEDIATE ASSIGNMENT

Information Element	Comment	Value

Protocol Discriminator	RR management		0110

Transaction Identifier	not relevant	0000

Message Type	Immediate Assignment	0011	1111

Page mode	Normal paging		  00

Channel Description	SDCCH/8	01TT	Txxx

TTT is value randomly chosen between 0 and 7

Request Reference

Timing Advance	30 bit periods	  01	1110

Mobile Allocation	empty

Starting Time	optional field

c)	The SS receives a Paging Response message in each of the test cases a.1) to a.4).

d)	For test cases a.1) to a.4) the SS sends a Channel Release message.

Message:     CHANNEL RELEASE

Information Element	Comment	Value

Protocol Discriminator	RR management		0110

Transaction Identifier	not relevant	0000

Message Type	Channel Release	0000	1101

RR cause	Normal release	0000	0000

II.5.3.6.2.1.8	Requirements 3

1)	For cases a.1) to a.4) the MS shall respond with CHANNEL REQUEST (ESTABL. CAUSE = Answer to paging) until the SS answers. The number of CHANNEL REQUEST messages is limited by the parameter Max-retrans.

2)	In each of the test cases a.1) to a.4), the MS shall send a PAGING RESPONSE message on the channel assigned by the SS.

Message:     PAGING RESPONSE

Information Element	Comment	Value

Protocol Discriminator	RR management		0110

Transaction Identifier	not relevant	0000

Message Type	Paging Response	0010 0111

Ciphering key seq. number	not relevant	000

Mobile Station classmark 2	according to manufacturer specification

Mobile Identity	TMSI

II.5.3.6.2.1.9	Procedure 4

The MS is made to go in "idle roaming not allowed" state (the MS is made to perform a location updating procedure to which the SS answers by a location updating reject with cause #11), that is no more TMSI available.

All those tests from II.5.3.6.2.1.3 to II.5.3.6.2.1.8 which address the MS by its TMSI are repeated in this condition.

II.5.3.6.2.1.10	Response Requirement 4

The MS shall ignore the paging requests.

II.5.3.6.2.2	Extended paging

Purpose:

To test that the MS is operating in the extended page mode when this is ordered by the BSS as specified in GSM 04.08 sect. 3.3.2.1 (and answers paging messages in the next but one paging sub block).

Procedure 1

a)	The SS simulates a BSS which is configured as in II.5.3.6.2.1.

b)	The MS is in idle updated state. The MS's IMSI is known to the SS and the MS has been assigned a TMSI.

c)	The SS sends a PAGING REQUEST TYPE 1 message not addressing the MS under test but on the paging subchannel which corresponds to the MS's identity. The page mode is set to "extended paging".

Message:     PAGING REQUEST TYPE 1

Information Elements	Comments	Value

Protocol Discriminator	RR management	0110

Transaction Identifier		0000

Message Type	Paging request type 1	0010	0001

Page Mode	Extended Paging	0000	0001

Mobile Identity	NOT the identity of MS under test

-type of identity	IMSI	001

Note:	Extended paging can be chained only once.

d)	In the next but one paging subblock on the same CCCH the SS sends a PAGING REQUEST TYPE 1 message specifying paging reorganisation and addressing the MS by TMSI.

Message:     PAGING REQUEST TYPE 1

Information Elements	Comments	Value

Protocol Discriminator	RR management	0110

Transaction Identifier		0000

Message Type	Paging request type 1	0010	0001

Page Mode	Paging reorganisation	0000	0010

Mobile Identity	the identity of MS under test

-type of identity	TMSI	100

e)	The SS shall respond to the second CHANNEL REQUEST by sending a IMMEDIATE ASSIGNMENT REJECT message (to avoid a cell reselection).

Message:     IMMEDIATE ASSIGNMENT REJECT

Information Elements	Comments	Value

Protocol Discriminator	RR		0110

Transaction Identifier		0000

Message Type	Immediate Assignment Rej	0011	1010

Page Mode	normal	0000	0000

Request Reference	the 2nd channel request

Wait indication	0 second	0000	0000

other fields not referring to the MS under test.

Requirements 1

1)	In case of step c) the MS shall not respond.

2)	In case of step d) the MS shall send at least two CHANNEL REQUEST messages on its RACH.

Procedure 2

a)	The SS simulates a BSS which is configured as in II.5.3.6.2.1.

b)	The MS is in idle updated state. The MS's IMSI is known to the SS and the MS has been assigned a TMSI.

c)	The SS sends an IMMEDIATE ASSIGNMENT on the paging subchannel which corresponds to the MS's identity. The page mode is set to "extended paging".

Message:     IMMEDIATE ASSIGNMENT

Information Elements	Comments	Value

Protocol Discriminator	RR management	0110

Transaction Identifier		0000

Message Type	immediate assignment	0011	1111

Page Mode	Extended Paging	0000	0001

Channel description	not relevant

request reference	not relevant

timing advance	not relevant

mobile allocation	not relevant

starting time	not relevant

Note: Extended paging can be chained only once.

d)	In the next but one paging subblock on the same CCCH the SS sends a PAGING REQUEST TYPE 1 message specifying paging reorganisation and addressing the MS by TMSI.

Message:     PAGING REQUEST TYPE 1

Information Elements	Comments	Value

Protocol Discriminator	RR management	0110

Transaction Identifier		0000

Message Type	Paging request type 1	0010	0001

Page Mode	Paging reorganisation	0000	0010

Mobile Identity	the identity of MS under test

-type of identity	TMSI	100

e)	The SS shall respond to the second CHANNEL REQUEST by sending a IMMEDIATE ASSIGNMENT REJECT message (to avoid a cell reselection).

Message:     IMMEDIATE ASSIGNMENT REJECT

Information Elements	Comments	Value

Protocol Discriminator	RR		0110

Transaction Identifier		0000

Message Type	Immediate Assignment Rej	0011	1010

Page Mode	normal	0000	0000

Request Reference	the 2nd channel request

Wait indication	0 second	0000	0000

other fields not referring to the MS under test.

Requirements 2

1)	In case of step c) the MS shall not respond.

2)	In case of step d) the MS shall send at least two CHANNEL REQUEST messages on its RACH.

II.5.3.6.2.3	Paging reorganisation

Purpose:

To test that the MS is operating in the paging reorganisation page mode as specified in GSM 04.08 sect. 3.3.2.1 (and answers to paging messages sent on any block of the full downlink CCCH during the reorganisation and receives all BCCH messages during the reorganisation).

Procedure 1

a)	The SS simulates a BSS which is configured as in II.5.3.6.2.1.

b)	The MS is in idle-updated state. The MS has been assigned a TMSI, its IMSI is known to the SS.

c)	The SS sends a IMMEDIATE ASSIGNMENT EXTENDED message containing paging reorganisation page mode.

Message:     IMMEDIATE ASSIGNMENT EXTENDED

Information Elements	Comments	Value

Protocol Discriminator	RR management	0110

Transaction Identifier		0000

Message Type	IMM ASS EXT	0010	1001

Page Mode	paging reorganisation	0000	0010

request reference	Do NOT address the MS under test

wait indication

d)	Before the MS's original paging sub-channel re-occurs, the SS pages it on its old CCCH by TMSI in some paging block which is not belonging to the MS's paging sub-channel. A PAGING REQUEST TYPE 2 message is used, the page mode set to "normal paging".

Message:     PAGING REQUEST TYPE 2

Information Elements	Comments	Value

Protocol Discriminator	RR management	0110

Transaction Identifier		0000

Message Type	Paging request type 2	0010	0010

Page Mode	Normal Paging	0000	0000

TMSI	TMSI of MS under test

TMSI	NOT TMSI of MS under test

The SS shall respond to the second CHANNEL REQUEST by sending a IMMEDIATE ASSIGNMENT REJECT message (to avoid a cell reselection).

Message:     IMMEDIATE ASSIGNMENT REJECT

Information Elements	Comments	Value

Protocol Discriminator	RR		0110

Transaction Identifier		0000

Message Type	Immediate Assignment Rej	0011	1010

Page Mode	normal	0000	0000

Request Reference	the 2nd channel request

Wait indication	0 second	0000	0000

other fields not referring to the MS under test.

e)	The SS changes the following BCCH parameters as indicated:

CCCH-CONF is set to "1 basic physical channel used for CCCH, combined with SDCCH" (encoded as 001).

BS-AG-BLKS-RES is set to 2 blocks reserved for access grant (coded as 2).

f)	On the MS's old paging subchannel a PAGING REQUEST message of arbitrary type is transmitted, not addressing the MS, with the page mode set to "paging re-organisation".

Message:     PAGING REQUEST TYPE 1

Information Elements	Comments	Value

Protocol Discriminator	RR management	0110

Transaction Identifier		0000

Message Type	Paging request type 1	0010	0001

Page Mode	Paging Reorganisation	0000	0010

Mobile Identity	Do NOT address the MS under test

-type of identity	IMSI	001

g)	The SS waits for the duration of five 51 TDMA frame multiframes (four to allow the MS to read all four SYSTEM INFO messages on the BCCH, one to calculate the new paging group). Then the MS is paged on its new paging subchannel.

Message:     PAGING REQUEST TYPE 1

Protocol Discriminator	RR management		0110

Transaction Identifier		0000

Message Type	Paging request type 1	10010	0001

Page Mode	Normal Paging	0000	0000

Mobile Identity	Identity of MS under test

-type of identity	TMSI	100

h)	The SS shall respond to the second CHANNEL REQUEST by sending a IMMEDIATE ASSIGNMENT REJECT message (to avoid a cell reselection).

Message:     IMMEDIATE ASSIGNMENT REJECT

Information Elements	Comments	Value

Protocol Discriminator	RR		0110

Transaction Identifier		0000

Message Type	Immediate Assignment Rej	0011	1010

Page Mode	normal	0000	0000

Request Reference	the 2nd channel request

Wait indication	0 second	0000	0000

other fields not referring to the MS under test.

Requirements 1

1)	In case of step d) the MS shall transmit at least two CHANNEL REQUEST messages on its RACH.

2)	In case of step g) the MS shall transmit at least two CHANNEL REQUEST messages on its new RACH.

Procedure 2

Procedure 1 is repeated, but in step e) "CCCH-CONF" is set to "2 basic physical channels used for CCCH, not combined with SDCCHs" (encoded as 010).

Requirements 2

Same requirements as "Requirements 1".

Procedure 3

a)	The SS simulates a BSS which is configured as in II.5.3.6.2.1.

b)	The MS is in idle-updated state. The MS has been assigned a TMSI, its IMSI is known to the SS.

c)	BCCH info is set such that Max-retrans = 1 (coded as 00).

d)	The SS sends an IMMEDIATE ASSIGNMENT EXTENDED message as in Procedure 1 c).

e)	The MS is paged immediately in a former AG block, as in Procedure

1 d). The SS shall respond to the second CHANNEL REQUEST by sending a IMMEDIATE ASSIGNMENT REJECT message (to avoid a cell reselection).

Requirements 3

1)	In step e), the MS shall produce at least two channel requests.

II.5.3.6.2.4 	No change of page mode

Purpose:

To test that the MS remembers the page mode from the previous paging request message.

Method of test

a)	The SS simulates a BSS which is configured as in II.5.3.6.2.1.

b)	The MS is in the idle updated state. The MS has been assigned a TMSI and its IMSI is known to the SS.

c)	The SS sends an IMMEDIATE ASSIGNMENT REJECT message on the MS's paging channel. The page mode is set to "extended paging".

Message:     IMMEDIATE ASSIGNMENT REJECT

Information Elements	Comments	Value

Protocol Discriminator	RR management	0110

Transaction Identifier		0000

Message Type	Imm Ass Rej	0011	1010

Page Mode	Extended Paging	0000	0010

Request reference	NOT for the MS under test

Wait indication

Comment GSM3: do it again for "immediate assignment extended"

d)	When the MS's specific paging multiframe and paging subblock reoccurs for the second time, PAGING REQUEST TYPE 3 is sent. Page mode set to "no change".

Message:     PAGING REQUEST TYPE 3

Information Elements	Comments	Value

Protocol Discriminator	RR management	0110

Transaction Identifier		0000

Message Type	Paging request type 3	0010	0100

Page Mode	no change	0000	0011

TMSI	Not TMSI of a MS under test

TMSI	Not TMSI of a MS under test

TMSI	Not TMSI of a MS under test

TMSI	Not TMSI of a MS under test

e)	In the next but one paging subblock on the same CCCH the SS sends a PAGING REQUEST TYPE 1 message specifying paging reorganisation and addressing the MS by TMSI.

Message:     PAGING REQUEST TYPE 1

Information Elements	Comments	Value

Protocol Discriminator	RR management	0110

Transaction Identifier		0000

Message Type	Paging request type 1	0010	0001

Page Mode	Paging reorganisation	0000	0010

Mobile Identity	the identity of MS under test

-type of identity	TMSI	100

f)	The SS shall respond to the second CHANNEL REQUEST by sending a IMMEDIATE ASSIGNMENT REJECT message (to avoid a cell reselection).

Requirements

1)	In case of steps c) and d) the MS shall not respond.

2)	In case of step e) the MS shall send at least two CHANNEL REQUEST messages on its RACH.

II.5.3.6.3	Measurement Report

The Measurement Report Procedure is described in section 3.4.1.2 of GSM 04.08.

II.5.3.6.3.1	Introduction and purpose of test

When having a RR-connection, the MS sends measurement reports. The report contains reception characteristics from serving and neighbouring cells.

Note:	The capability of calculating RxLev and RxQual is tested in section II.6.1.2. Here, only the signalling aspect is verified. The interval between two successive Layer 2 frames containing measurements reports shall not exceed one Layer 2 frame.

The MS shall be able to indicate:

-	if no neighbouring cells are identified,

-	the six best BCCH's out of a list taken from System Information,

-	only permitted BCCH's out of a list.

II.5.3.6.3.2	Method of Measurement

a)	The SS uses one serving cell and seven additional transmitters in stable conditions on different frequencies and levels.

b)	On the serving cell the MS is having a call in progress.

c)	First the SS sends an empty BCCH allocation in System Information Type 5.

d)	Then the SS sends a non-empty BCCH allocation in System Information Type 5 and changes the BA ARFCN list and colourcodes to verify the MS.

e)	The MS is forced to make a handover in a new cell when DTX shall be used and PWRC indicates that the MS has not to measure the BCCH carrier.

II.5.3.6.3.3	Procedures

Procedure 1	No Neighbouring Cells

Transmitter			Level		NCC	BS Colour Code	ARFCN

Serving 1	S1		-60		1	 3			  002

Neighbour	N1		-85		1	 5			  008

"    "	N2		-80		1	 7			  014

"    "	N3		-75		1	 1			  020

"    "	N4		-55		1	 3			  026

"    "	N5		-50		1	 5			  032

"    "	N6		-45		1	 7			  038

"    "	N7		-40		1	 1			  044

Message:     SYSTEM INFORMATION TYPE 5 (GSM 04.08, 9.1.33)

Information Element	Comment 	Value

Protocol Discriminator	RR management

Transaction Identifier	not relevant

Message Type	sys info 5

Neighbour cell descr		-

- BA-NO	Band number 0

- BA ARFCN (124..001)	no BCCH allocation	all 0

Message:     SYSTEM INFORMATION TYPE 6 (GSM 04.08, 9.1.34)

Information Element	Comment	Value

Protocol Discriminator	RR management

Transaction Identifier	not relevant

Message Type

Cell Identity	default

LAI	default

Cell Options

- PWRC	PWRC is not set

- DTX	shall not be used

- RADIO-LINK-TIMEOUT	default

PLMN permitted	only NCC 1 permitted

Requirements 1

1)	The MS shall continuously send measurement reports on every SACCH blocks and the measurement valid indication shall be set to valid (0) within the second block at the latest.

Message:     MEASUREMENT REPORT (GSM 04.08, 9.1.20)

Information Element	Comment	Value

Protocol Discriminator	RR management

Transaction Identifier	not relevant

Message type	measurement report

Measurement results		-

- BA-USED	"Same value as BA-IND

sent on SACCH"

- DTX USED	Dtx not used

- RXLEV-FULL-SERVING-CELL	Actual value not checked

- RXLEV-SUB-SERVING-CELL

- MEAS-VALID	as specified in the text

- RXQUAL-FULL-SERVING-CELL	Actual value not checked

- RXQUAL-SUB-SERVING-CELL 	Actual value not checked

- NO-NCELL-M 	"No neighbour cell

measurement result"

or

"Neighbour cell information

not available for serving cell"

- RXLEV-NCELL     1	no result                  all bits set to 0

- BCCH-FREQ-NCELL 1	no result

- BSIC--NCELL     1    	no result

.     .         .	.   .

.     .         .	.   .

- RXLEV-NCELL     6	no result

- BCCH-FREQ-NCELL 6	no result

- BSIC--NCELL     6	no result

Procedure 2	All neighbours present

Transmitter			Level		NCC	BS Colour Code	ARFCN

Serving 1	S1		-60		1	 3			002

Neighbour	N1		-85		1	 5			008

"    "	N2		-80		1	 7			014

"    "	N3		-75		1	 1			020

"    "	N4		-55		1	 3			026

"    "	N5		-50		1	 5			032

"    "	N6		-45		1	 7			038

"    "	N7		-40		1	 1			044

Message:     SYSTEM INFORMATION TYPE 5 (GSM 04.08, 9.1.33)

Information Element	Comment	Value

Protocol Discriminator	RR management		0110

Transaction Identifier	not relevant	0000

Message Type	sys info 5	0001	1101

Neighbour cell descr		-

- BA-NO	gsm band 0	00

- BA-IND		0

- BA ARFCN       	(2,8,14,20,26,32,38,44)=open

-       	(all others)=closed

Message:     SYSTEM INFORMATION TYPE 6 (GSM 04.08, 9.1.34)

Information Element	Comment	Value

Protocol Discriminator	RR management		0110

Transaction Identifier	not relevant	0000

Message Type		0001	1110

Cell Identity	default

LAI	default

Cell Options

- PWRC	PWRC is not set

- DTX	shall not be used	10

- RADIO-LINK-TIMEOUT	default

PLMN permitted	only NCC 1 permitted	0000	0010

Requirements 2

1)	The MS shall continuously send measurement reports on every SACCH blocks and the measurement valid indication shall be set to valid (0) within the second block at the latest.

Message:     MEASUREMENT REPORT (GSM 04.08, 9.1.20)

Information Element	Comment	Value

Protocol Discriminator	RR management		0110

Transaction Identifier	not relevant	0000

Message Type	measurement report	0001	0101

Measurement results		-

- DTX USED	Dtx not used	0

- BA-USED

- MEAS-VALID	as specified in the text

- RXLEV-FULL-SERVING-CELL	Actual value not checked	  xx 	xxxx

- RXLEV-SUB-SERVING-CELL	Actual value not checked	  xx 	xxxx

- RXQUAL-FULL-SERVING-CELL	Actual value not checked	     	 xxx

- RXQUAL-SUB-SERVING CELL	Actual value not checked	     	 xxx

- NO-NCELL-N 	number of neighbours=six 	     	 110

- RXLEV-NCELL     1	only order checked	  xx 	xxxx

- BCCH-FREQ-NCELL 1	the index foi	   x 	xxxx

- BSIC--NCELL     1    	as in table	  xx 	xxxx

.     .         .	.   . 	  .	  .

.     .         .	.   . 	  .	  .

- RXLEV-NCELL     6	only order checked	  xx 	xxxx

- BCCH-FREQ-NCELL 6	as in table	   x 	xxxx

- BSIC--NCELL     6    	as in table	  xx 	xxxx

Note:	The actual values in the MEASUREMENT REPORT are not checked, but the order of the cells, when put in the order of increasing RXLEV should be N3, S1, N4, N5, N6, N7.

Procedure 3:	Combination of barred cells and unpermitted PLMNs

Transmitter				Level	NCC	BS Colour Code		ARFCN

Serving 1	S1			-60	 1	 3				002

Neighbour	N1  (no info)	-85	 1	 5				008

"    "	N2			-80	 1	 7				014

"    "	N3			-75	 2	 1				020

"    "	N4  (no info)	-55	 3	 3				026

"    "	N5  (no info)	-50	 4	 5				032

"    "	N6			-45	 1	 7				038

"    "	N7			-40	 1	 1				044

Message:     SYSTEM INFORMATION TYPE 5 (GSM 04.08, 9.1.33)

Information Element	Comment 	Value

Protocol Discriminator	RR		0110

Transaction Identifier	not relevant	0000

Message Type	sys info 5	0001	1101

Neighbour cell descr		-

- BA-NO	gsm band 0	00

- BA ARFCN	(2,14,20,38,44)=open	1

-       	(all others)=closed	0

Message:     SYSTEM INFORMATION TYPE 6 (GSM 04.08, 9.1.34)

Information Element	Comment	Value

Protocol Discriminator	RR management		0110

Transaction Identifier	not relevant	0000

Message Type		0001	1110

Cell Identity	default

LAI	default

Cell Options

- PWRC	PWRC is not set

- DTX	shall not be used	10

- RADIO-LINK-TIMEOUT	default

PLMN permitted	only NCC 1 permitted	0000	0010

Requirements 3

1)	The MS shall continuously send measurement reports on every SACCH blocks and the measurement valid indication shall be set to valid (0) within the second block at the latest.

Message:     MEASUREMENT REPORT (GSM 04.08, 9.1.20)

Information Element	Comment 	Value

Protocol Discriminator	RR management		0110

Transaction Identifier	not relevant	0000

Message Type	measurement report	0001	0101

Measurement results		-

- BA-USED

- MEAS-VALID	as specified in the text

- DTX USED	Dtx not used	0

- RXLEV-FULL-SERVING-CELL	Actual value not checked	  xx 	xxxx

- RXLEV-SUB-SERVING-CELL	Actual value not checked	  xx 	xxxx

- RXQUAL-FULL-SERVING-CELL	Actual value not checked	     	 xxx

- RXQUAL-SUB-SERVING-CELL 	Actual value not checked	     	 xxx

- NO-NCELL-N 	number of neighbours=four	       100

- RXLEV-NCELL     1	only order checked	  xx 	xxxx

- BCCH-FREQ-NCELL 1	the index in neighbour

- BSIC--NCELL     1    	cell description 	xxxx

.     .         .	.   . 	  .	  .

.     .         .	.   . 	  .	  .

- RXLEV-NCELL     4	only order checked	  xx 	xxxx

- BCCH-FREQ-NCELL 4	as in table

- BSIC--NCELL     4	as in table	  xx 	xxxx

Note:	The actual values in the MEASUREMENT REPORT are not checked, but the order of the cells, when put in the order of increasing RXLEV should be N2, S1, N6, N7.

Procedure 4:	Use of DTX in the Measurement Report

Transmitter			Level		NCC	BS Colour Code	ARFCN

Serving 1	S1		-60		1	 3			002

Neighbour	N1		-85		1	 5			008

"    "	N2		-80		1	 7			014

"    "	N3		-75		1	 1			020

"    "	N4		-55		1	 3			026

"    "	N5		-50		1	 5			032

"    "	N6		-45		1	 7			038

"    "	N7		-40		1	 1			044

The MS is forced to make a handover in a new cell, where the following SYSTEM INFO TYPE 5 and 6 are broadcasted.

Message:     SYSTEM INFORMATION TYPE 5 (GSM 04.08, 9.1.33)

Information Element	Comment 	Value

Protocol Discriminator	RR		0110

Transaction Identifier	not relevant	0000

Message Type	sys info 5	0001	1101

Neighbour cell descr		-

- BA-NO	gsm band 0	00

- BA ARFCN	(2,8,14,20,26,32,38,44)=open

-       	(all others)=closed

Message:     SYSTEM INFORMATION TYPE 6 (GSM 04.08, 9.1.34)

Information Element	Comment	Value

Protocol Discriminator	RR management		0110

Transaction Identifier	not relevant	0000

Message Type		0001	1110

Cell Identity	default

LAI	default

Cell Options

- PWRC	PWRC is set	1

- DTX	shall be used	01

- RADIO-LINK-TIMEOUT	default

PLMN permitted	only NCC 1 permitted	0000	0010

Requirements 4

1)	After the sending of the HANDOVER COMPLETE message, the MS shall continuously send measurement reports on every SACCH blocks, the measurement valid indication shall be set to valid (0) within the second block at the latest and DTX-USED shall be set to "DTX used".

Message:     MEASUREMENT REPORT (GSM 04.08, 9.1.20)

Information Element	Comment 	Value

Protocol Discriminator	RR management		0110

Transaction Identifier	not relevant	0000

Message Type	measurement report	0001	0101

Measurement results		-

- BA-USED

- MEAS-VALID	as specified in the text

- DTX USED	Dtx used	1

- RXLEV-FULL-SERVING-CELL	Actual value not checked	  xx 	xxxx

- RXLEV-SUB-SERVING-CELL	Actual value not checked	  xx 	xxxx

- RXQUAL-FULL-SERVING-CELL	Actual value not checked	     	 xxx

- RXQUAL-SUB-SERVING-CELL 	Actual value not checked	     	 xxx

- NO-NCELL-M 	number of neighbours=six	     	 110

- RXLEV-NCELL     1	only order checked	  xx 	xxxx

- BCCH-FREQ-NCELL 1	the index in neighbour

- BSIC--NCELL     1    	cell description 	xxxx

.     .         .	.   . 	  .	  .

.     .         .	.   . 	  .	  .

- RXLEV-NCELL     6	only order checked	  xx 	xxxx

- BCCH-FREQ-NCELL 6	as in table

- BSIC--NCELL     6    	as in table	  xx 	xxxx

Note:	The actual values in the MEASUREMENT REPORT are not checked, but the order of the cells, when put in the order of increasing RXLEV should be N3, S1, N4, N5, N6, N7.

II.5.3.6.4	Test of Dedicated Channel Assignment

II.5.3.6.4.1	Introduction

An intracell change of channel can be requested by upper layers for changing the channel type, or decided by the RR-sublayer, e.g. for an internal handover. This change is performed through the dedicated channel assignment.

The purpose of the dedicated channel assignment procedure is to completely modify the physical channel configuration of the MS while staying in the same cell.

This is described in section 3.4.3 of GSM 04.08.

II.5.3.6.4.2	Purpose of the test Assignment completion

The purpose of this test is to verify that the MS can seize the channel defined in the command. Just a subset of all possible channels transitions will be tested:

SDCCH  -> TCH/FS

TCH/FS -> TCH/HS

TCH/HS -> TCH/FS

TCH/FS -> TCH/FS,	In this particular test we check that the MS does not increment V(SD) when repeating a message, after completion.

TCH/FS -> TCH/FS,	In this particular test we check that the MS takes into account the value specified in the field "starting time".

Note: TCH/HS is tested only if supported (i.e. step b) might be skipped).

II.5.3.6.4.2.1	Procedure

a)	The SS pages the MS. On response of the MS a SDCCH is allocated. Then the SS sends an ASSIGNMENT COMMAND allocating a traffic channel TCH/FS with frequency hopping used.

b)	After reception of the ASSIGNMENT COMPLETE message, the SS sends a ASSIGNMENT COMMAND message allocating an traffic channel TCH/HS with frequency hopping not used.

c)	Same as b) but the new channel is an TCH/FS with frequency hopping not used.

d)	The SS sends an AUTHENTICATION REQUEST message to the MS.

The SS does not Layer 2 acknowledge the AUTHENTICATION RESPONSE message sent by the MS and stores this message.

e)	The SS sends an ASSIGNMENT COMMAND message allocating a TCH/FS with frequency hopping used, just after having received the AUTHENTICATION RESPONSE.

f)	The SS sends an ASSIGNMENT COMMAND message allocating a traffic channel with frequency hopping not used. This command includes the optional field "starting time" filled with current frame number +100.

Note:	At each step, the channel allocation is changed by increasing

by one the time slot number modulo 8.

Message:     ASSIGNMENT COMMAND

Information Element	Comment	Value

Protocol Discriminator	RR	0110

Transaction Identifier	not relevant	0000

Message Type	ass command	0010	1110

Channel Description	as specified in the text

Power Command	randomly drawn between 0 and 31 (Note 1)

Starting time	Optional field used when specified in the

text

Mobile allocation	Use or not of frequency hopping

Note 1:	The power command is at each step randomly drawn between 0 and 31 and with a different value.

II.5.3.6.4.2.2	Requirements

1)	In step a), the MS shall switch to the assigned channel, establish the link with the power level specified in the command message and send an ASSIGNEMENT COMPLETE message.

2)	In step b), same as 1).

3)	In step c), same as 1).

4)	In step d), the MS shall send an AUTHENTICATION RESPONSE message.

5)	In step e), same as 1).

Then the MS shall send an AUTHENTICATION RESPONSE message with V(SD) at the same value as in the first AUTHENTICATION RESPONSE sent (the SS shall compare the two messages).

6)	In step f) same as 1) but this shall be done after the time specified in the field "Starting Time".

Message:     ASSIGNMENT COMPLETE

II.5.3.6.4.3	Assignment Failure

The purpose of the test is to verify that when not succeeding to seizing the new channel, the MS reactivates the old one with the previous power used on this channel.

II.5.3.6.4.3.1	Procedure

a)	A Mobile Terminated call is initiated. After establishment of the call, an ASSIGNMENT COMMAND message is sent allocating a new TCH/FS.

b)	The SS does not set up the new channel.

Message:     ASSIGNMENT COMMAND

Information Element	Comment	Value

Protocol Discriminator 	RR	0110

Transaction Identifier 	not relevant	0000

Message Type 		0010	1110

Channel Description	as specified in the text

Power Command	Randomly drawn between 0 and 31 and different

from the previous one.

II.5.3.6.4.3.2	Requirement

1)	The MS shall try to activate the new channel. The MS shall reactivate the old one with the previous power used on this channel, reconnect the TCH and trigger the establishment of the main signalling link.

The MS shall then send an ASSIGNMENT FAILURE message.

Message:     ASSIGNMENT FAILURE

II.5.3.6.5	Test of Handover

The Handover procedure is specified in section 3.4.4 of GSM 04.08.

II.5.3.6.5.1	Introduction

With the handover procedure, it is possible to completely alter the channels allocated to a MS. This makes possible in particular to switch a call in progress from one cell to another. The procedure is always initiated by the network, with the MS in a dedicated mode.

II.5.3.6.5.2	Purpose of the Test

The purpose of the test is to verify that the MS during handover

-	disconnects and connects channels,

-	releases and establishes data links,

-	is capable of handling failures and abnormal cases related to the handover procedure.

II.5.3.6.5.3	Method of Measurement

The SS simulates two cells, A and B, where A is the old cell and B is the target for the handover. All test procedures start with the MS being allocated a dedicated resource on cell A. Cell A and B are running under ideal radio conditions. Appropriate information on both cell A and B is given to the MS on the BCCHs of both cells according to the default conditions for the serving cell (A) and the neighbouring cell (B). Depending on the procedure frequency hopping is in use or not. Each time frequency hopping has to be used the SS has to draw randomly the value of N (number of frequency) between 1 and 64, HSN (hopping sequence number), MAIO (mobile allocation offset) and include it in the relevant messages (IMMEDIATE ASSIGNMENT, ASSIGNMENT COMMAND, HANDOVER COMMAND).

The power used by the MS on the channel belonging to the old cell is set to the value encoded by 3.

In the first two procedures, normal handover is made and the next three procedures test handover during call establishment. These five procedures are first tested with non synchronized cells, and then with finely synchronized cells. All tests are repeated with starting time specified in HANDOVER COMMAND message.

The two last procedures test the function of timer T3124, first on a Layer 3 failure, then on a Layer 1 failure. Layer 3 failure is introduced by the SS by not sending PHYSICAL INFORMATION on cell B whereas Layer 1 failure is introduced by the SS by not sending anything at all, with the exception of BCCH, on cell B.

II.5.3.6.5.4	Procedures

II.5.3.6.5.4.1	Procedure 1: Handover during call in progress, TCH/FS, Non Synchronized, without frequency hopping

II.5.3.6.5.4.1.1	Purpose of the test

To verify that when the MS is made to make a handover to a non synchronized cell, it continuously sends access bursts on the main DCCH until it receives a PHYSICAL INFORMATION message from the SS.

To verify that it then activates the channels.

II.5.3.6.5.4.1.2	Test

a)	The SS sends a HANDOVER COMMAND message on the main DCCH on cell A.

b)	The SS observes the access bursts which are now continuously being sent by the MS on its main DCCH of cell B. After receiving n (randomly drawn between values according to table below) access bursts, the SS sends PHYSICAL INFORMATION with a Timing Advance in the range 0..63 (=k) bit periods of 48/13 µs each.

In the first test k=20 will apply.

Target cell	TCH/FS	TCH/HS	SDCCH

channel type

n	10-20	5-10	2-5

Message:     HANDOVER COMMAND

Information Element	Comment	Value

Protocol Discriminator	RR management	0110

Transaction identifier	not relevant	0000

Message type	handover command	0010	1011

Cell description	default

Channel description	Bm + ACCHs, TN = 0	0000	1000

TSC, H=0, FB-NO	xxx0	0000

ARFCN	0xxx	xxxx

Handover reference	100 dec 	0110	0100

Power command	5 dec	0000	0101

Synchronization indic	non synchronized cells	1101	0000

Message:     PHYSICAL INFO

Information Element	Comment	Value

Protocol Discriminator	RR management	0110

Transaction identifier	not relevant	0000

Message type	physical info	0010	1101

Timing advance	k*48/13 µs	0001	0100

c)	The test is repeated for a Timing Advance (k) of 50 bit periods (encoded as 0011 0010).

II.5.3.6.5.4.1.3	Requirements

1)	The MS shall send continuously access bursts at the power level specified in HANDOVER COMMAND including the correct handover reference on the main DCCH of the target cell until the reception of the PHYSICAL INFORMATION.

2)	The MS shall activate the channel in sending and receiving mode, and connect the channel in the range of (0..3) s after the transmission time of the PHYSICAL INFORMATION.

3)	The MS shall use the correct timing advance, as specified by the PHYSICAL INFORMATION sent by the SS.

4)	The MS shall establish a signalling link.

5)	The  MS shall send HANDOVER COMPLETE  on the target cell so that the SS receives the message within T3103 seconds after sending the HANDOVER COMMAND.

Message:     HANDOVER COMPLETE

6)	HANDOVER COMPLETE shall not be received before UA frame has been sent by the SS.

II.5.3.6.5.4.2	Procedure 2: Handover during call in progress, TCH/HS without FH to TCH/HS with FH, non synchronized

This test shall be skipped if the MS does not support TCH/HS.

II.5.3.6.5.4.2.1	Purpose

To verify that when the MS is made to make a handover to a non synchronized cell, it continuously sends access bursts on the main DCCH until it receives a PHYSICAL INFORMATION message from the SS.

To verify that it then activates the channels.

II.5.3.6.5.4.2.2	Test

As in procedure 1 except that call establishment shall be performed on TCH/HS and that channel description in HANDOVER COMMAND message shall indicate TCH/HS.

Message:     HANDOVER COMMAND

Information Element	Comment	Value

Protocol Discriminator	RR management		0110

Transaction identifier	not relevant	0000

Message type	handover command	0010	1011

Cell description	default

Channel description	Lm + ACCHs, TN = 5	0001	1101

TSC, H=1, MAIO (high part)	xxx1	xxxx

MAIO (low part), HSN	xxxx	xxxx

Handover reference	255	1111	1111

Power command	6	0000	0110

Frequency Channel seq.	IEI	0110	1001

as specified in GSM 04.08

II.5.3.6.5.4.2.3	Requirements

1)	The MS shall send continuously access bursts at the power level specified in HANDOVER COMMAND including the correct handover reference on the main DCCH of the target cell until the reception of the PHYSICAL INFORMATION.

2)	The MS shall activate the channel in sending and receiving mode, and connect the channel in the range of (0..3) s after the transmission time of the PHYSICAL INFORMATION.

3)	The MS shall use the correct Timing Advance, as specified by the PHYSICAL INFORMATION sent by the SS.

4)	The MS shall establish a signalling link.

5)	The  MS shall send HANDOVER COMPLETE  on the target cell so that the SS receives the message within T3103 seconds after sending the HANDOVER COMMAND.

Message:     HANDOVER COMPLETE

6)	HANDOVER COMPLETE shall not be received before UA frame has been sent by the SS.

II.5.3.6.5.4.3	Procedure 3: Handover during Call establishment, SDCCH without FH to SDCCH with FH, Non synchronized

II.5.3.6.5.4.3.1	Purpose of the test

When the SS sends HANDOVER COMMAND, it may occur that the MS had just sent a (MM or CC) Layer 3 message which has not yet been acknowledged. The purpose of this test is to check that the MS will send again this message after the HANDOVER COMPLETE (using the same value in the N(SD) field).

II.5.3.6.5.4.3.2	Test:

a)	The Call establishment shall be performed on a SDCCH/8.

The test of Procedure 1 will be used, except that the HANDOVER COMMAND message will be sent at a very well selected time. For example, the HANDOVER COMMAND will be sent in a mobile originated call establishment procedure.

The channel description information will be set to SDCCH/8 with FH.

b)	The SS will store the SETUP message and send a HANDOVER COMMAND message before having acknowledged the last Layer 2 frame of the SETUP message.

II.5.3.6.5.4.3.3	Requirements

1)	The MS shall send continuously access bursts at the power level specified in HANDOVER COMMAND including the correct handover reference on the main DCCH of the target cell until the reception of the PHYSICAL INFORMATION.

2)	The MS shall activate the channel in sending and receiving mode, and connect the channel in the range of (0..3) s after the transmission time of the PHYSICAL INFORMATION.

3)	The MS shall use the correct Timing Advance, as specified by the PHYSICAL INFORMATION sent by the SS.

4)	The MS shall establish a signalling link.

5)	The  MS shall send HANDOVER COMPLETE  on the target cell so that the SS receives the message within T3103 seconds after sending the HANDOVER COMMAND.

Message:     HANDOVER COMPLETE

6)	HANDOVER COMPLETE shall not be received before UA frame has been sent by the SS.

7)	The MS shall send again the SETUP message to the SS using the same value in the N(SD) field, just after the HANDOVER COMPLETE message has been sent.

II.5.3.6.5.4.4	Procedure 4: Handover during call establishment, TCH/FS to TCH/FS all with FH, Non synchronized

Hopping sequence sets have to be drawn for each cell.

II.5.3.6.5.4.4.1	Purpose

When the SS sends HANDOVER COMMAND, it may occur that the MS had just sent a (MM or CC) Layer 3 message which has not yet been acknowledged. The purpose of this test is to check that the MS will send again this message after the HANDOVER COMPLETE (using the same value in the N(SD) field).

II.5.3.6.5.4.4.2	Test

As in procedure 3 except that a TCH/FS shall have been assigned for call establishment with the channel mode set to "signalling only" and that the channel description information element in HANDOVER COMMAND message shall indicate TCH/FS.

a)	The Call establishment shall be performed for a TCH/FS, with the channel mode set to "signalling only".

The test of Procedure 1 will be used, except that the HANDOVER COMMAND message will be sent at a very well selected time. For example, the HANDOVER COMMAND will be sent in a mobile originating call establishment procedure.

The channel description information element in the HANDOVER COMMAND message shall indicate TCH/FS.

b)	The SS will store the SETUP message and send a HANDOVER COMMAND message before having acknowledged the last Layer 2 frame of the SETUP message.

II.5.3.6.5.4.4.3	Requirements

1)	The MS shall send continuously access bursts at the power level specified in HANDOVER COMMAND including the correct handover reference on the main DCCH of the target cell until the reception of the PHYSICAL INFORMATION.

2)	The MS shall activate the channel in sending and receiving mode, and connect the channel in the range of (0..3) s after the transmission time of the PHYSICAL INFORMATION.

3)	The MS shall use the correct Timing Advance, as specified by the PHYSICAL INFORMATION sent by the SS.

4)	The MS shall establish a signalling link.

5)	The  MS shall send HANDOVER COMPLETE  on the target cell so that the SS receives the message within T3103 seconds after sending the HANDOVER COMMAND.

Message:     HANDOVER COMPLETE

6)	HANDOVER COMPLETE shall not be received before UA frame has been sent by the SS.

7)	The MS shall send again the SETUP message to the SS using the same value in the N(SD) field, just after the HANDOVER COMPLETE message has been sent.

II.5.3.6.5.4.5	Procedure 5: Handover during call establishment, SDCCH with FH to TCH/FS without FH, Non synchronized

II.5.3.6.5.4.5.1	Purpose

When the SS sends HANDOVER COMMAND, it may occur that the MS had just sent a (MM or CC) Layer 3 message which has not yet been acknowledged. The purpose of this test is to check that the MS will send again this message after the HANDOVER COMPLETE (using the same value in the N(SD) field).

II.5.3.6.5.4.5.2	Test

a)	The Call establishment shall be performed on a SDCCH/8.

The test of Procedure 1 will be used, except that the HANDOVER COMMAND message will be sent at a very well selected time. For example, the HANDOVER COMMAND will be sent in a mobile originating call establishment procedure.

The channel description information in the HANDOVER COMMAND message shall indicate TCH/FS, and the channel mode Information Element shall be present and set to "signalling only".

b)	The SS will store the SETUP message and send a HANDOVER COMMAND message before having acknowledged the last Layer 2 frame of the SETUP message.

II.5.3.6.5.4.5.3	Requirements

1)	The MS shall send continuously access bursts at the power level specified in HANDOVER COMMAND including the correct handover reference on the main DCCH of the target cell until the reception of the PHYSICAL INFORMATION.

2)	The MS shall activate the channel in sending and receiving mode, and connect the channel in the range of (0..3) s after the transmission time of the PHYSICAL INFORMATION.

3)	The MS shall use the correct timing advance, as specified by the PHYSICAL INFORMATION sent by the SS.

4)	The MS shall establish a signalling link.

5)	The  MS shall send HANDOVER COMPLETE  on the target cell so that the SS receives the message within T3103 seconds after sending the HANDOVER COMMAND.

Message:     HANDOVER COMPLETE

6)	HANDOVER COMPLETE shall not be received before UA frame has been sent by the SS.

7)	The MS shall send again the SETUP message to the SS using the same value in the N(SD) field, just after the HANDOVER COMPLETE message has been sent.

II.5.3.6.5.4.6	Procedure 6: Handover during call in progress, TCH/FS with FH to TCH/FS without FH, Synchronized

II.5.3.6.5.4.6.1	Purpose of the test

To verify that when the MS is made to make a handover to a synchronized cell, it sends a specific number of access burst on the main DCCH, and then activates the channels.

II.5.3.6.5.4.6.2	Test

a)	The SS sends a HANDOVER COMMAND on the main DCCH on cell A, indicating a handover to a synchronized cell (B).

b)	The SS then waits for reception of a HANDOVER COMPLETE message.

Message:     HANDOVER COMMAND

Information Element	Comment	Value

Protocol Discriminator	RR management	0110

Transaction identifier	not relevant	0000

Message type	handover command	0010	1011

Cell description	default

Channel description	Bm + ACCHs, TN = 4	0000	1100

TSC, H=0, FB-NO	xxx0	0000

ARFCN	0xxx	xxxx

Handover reference	64	0100	0000

Power command	7 dec	0000	0111

Synchronization indic	synchronized cells	1101	0001

II.5.3.6.5.4.6.3	Requirements

1)	In four successive slots on the main DCCH, the MS shall send access bursts at the power level specified in HANDOVER COMMAND with the correct handover reference.

2)	The MS shall activate the channel in sending and receiving mode, and connect the channel in the range of (0..3) seconds after the transmission of the HANDOVER COMMAND on the SS-side.

3)	The MS shall establish a link.

4)	The MS shall send HANDOVER COMPLETE on the target cell so that the SS receives the message within T3103 seconds after sending the HANDOVER COMMAND message, using the correct Timing Advance defined according to the previous case in cell A and the value of k (see note).

Note:	The two cells A and B are synchronised, but the SS will simulate the case where distance (A,MS) is unequal to distance (B,MS). For example, the BCCH(A) will be sent a [k] bits period before BCCH(B), Timing Advance in cell B = 2k + Timing Advance in cell A > 0.

Message:     HANDOVER COMPLETE

II.5.3.6.5.4.7	Procedure 7: Handover during call in progress, TCH/HS with FH to TCH/HS without FH, Synchronized

This test shall be skipped if the MS does not support TCH/HS.

II.5.3.6.5.4.7.1	Purpose of the test

To verify that when the MS is made to make a handover to a synchronized cell, it sends a specific number of access burst on the main DCCH, and then activates the channels.

II.5.3.6.5.4.7.2	Test

As in procedure 6 except that the channel description information element shall indicate TCH/HS.

II.5.3.6.5.4.7.3	Requirements

1)	In four successive slots on the main DCCH, the MS shall send access bursts at the power level specified in HANDOVER COMMAND with the correct handover reference.

2)	The MS shall activate the channel in sending and receiving mode, and connect the channel in the range of (0..3) seconds after the transmission of the HANDOVER COMMAND on the SS-side.

3)	The MS shall establish a link.

4)	The MS shall send HANDOVER COMPLETE on the target cell so that the SS receives the message within T3103 seconds after sending the HANDOVER COMMAND message, using the correct Timing Advance defined according to the previous case in cell A and the value of k (see note).

Note:	The two cells A and B are synchronised, but the SS will simulate the case where distance (A,MS) is unequal to distance (B,MS). For example, the BCCH(A) will be sent a [k] bits period before BCCH(B), Timing Advance in cell B = 2k + Timing Advance in cell A > 0.

Message:     HANDOVER COMPLETE

II.5.3.6.5.4.8	Procedure 8: Handover during call establishment, SDCCH to SDCCH all with FH, Synchronized

Hopping sequence sets have to be drawn for each cell.

II.5.3.6.5.4.8.1	Purpose

When the SS sends HANDOVER COMMAND, it may occur that the MS had just sent a (MM or CC) Layer 3 message which has not yet been acknowledged. The purpose of this test is to check that the MS will send again this message after the HANDOVER COMPLETE (using the same value in the N(SD) field).

To verify that when the MS is made to make a handover to a synchronized cell, it sends a specific number of access burst on the main DCCH, and then activates the channels.

II.5.3.6.5.4.8.2	Test

a)	The Call establishment shall be performed on a SDCCH/8.

The test of Procedure 6 will be used, except that the HANDOVER COMMAND message will be sent at a very well selected time. For example, the HANDOVER COMMAND will be sent in a mobile originating call establishment procedure.

The channel description information will be set to SDCCH/8 with FH.

b)	The SS will store the SETUP message and send a HANDOVER COMMAND message before having acknowledged the last Layer 2 frame of the SETUP message.

II.5.3.6.5.4.8.3	Requirements

1)	In four successive slots on the main DCCH, the MS shall send access bursts at the power level specified in HANDOVER COMMAND with the correct handover reference.

2)	The MS shall activate the channel in sending and receiving mode, and connect the channel in the range of (0..3) seconds after the transmission of the HANDOVER COMMAND on the SS-side.

3)	The MS shall establish a link.

4)	The MS shall send HANDOVER COMPLETE on the target cell so that the SS receives the message within T3103 seconds after sending the HANDOVER COMMAND message, using the correct Timing Advance defined according to the previous case in cell A and the value of k (see note).

Note:	The two cells A and B are synchronised, but the SS will simulate the case where distance (A,MS) is unequal to distance (B,MS). For example, the BCCH(A) will be sent a [k] bits period before BCCH(B), Timing Advance in cell B = 2k + Timing Advance in cell A > 0.

Message:     HANDOVER COMPLETE

5)	The MS shall send again the SETUP message to the SS using the same value in the N(SD) field, just after the HANDOVER COMPLETE message has been sent.

II.5.3.6.5.4.9	Procedure 9: Handover during call establishment, TCH/FS without FH to TCH/FS with FH, Synchronized

II.5.3.6.5.4.9.1	Purpose

When the SS sends HANDOVER COMMAND, it may occur that the MS had just sent a (MM or CC) Layer 3 message which has not yet been acknowledged. The purpose of this test is to check that the MS will send again this message after the HANDOVER COMPLETE (using the same value in the N(SD) field).

To verify that when the MS is made to make a handover to a synchronized cell, it sends a specific number of access burst on the main DCCH, and then activates the channels.

II.5.3.6.5.4.9.2	Test

a)	The Call establishment shall be performed on a TCH/FS.

The test of Procedure 6 will be used, except that the HANDOVER COMMAND message will be sent at a very well selected time. For example, the HANDOVER COMMAND will be sent in a mobile originated call establishment procedure.

The channel description information element in the HANDOVER COMMAND message will be set to TCH/FS with FH, and the channel mode Information Element shall be present and set to "signalling only".

b)	The SS will store the SETUP message and send a HANDOVER COMMAND message before having acknowledged the last Layer 2 frame of the SETUP message.

II.5.3.6.5.4.9.3	Requirements

1)	In four successive slots on the main DCCH, the MS shall send access bursts at the power level specified in HANDOVER COMMAND with the correct handover reference.

2)	The MS shall activate the channel in sending and receiving mode, and connect the channel in the range of (0..3) seconds after the transmission of the HANDOVER COMMAND on the SS-side.

3)	The MS shall establish a link.

4)	The MS shall send HANDOVER COMPLETE on the target cell so that the SS receives the message within T3103 seconds after sending the HANDOVER COMMAND message, using the correct Timing Advance defined according to the previous case in cell A and the value of k (see note).

Note:	The two cells A and B are synchronised, but the SS will simulate the case where distance (A,MS) is unequal to distance (B,MS). For example, the BCCH(A) will be sent a [k] bits period before BCCH(B), Timing Advance in cell B = 2k + Timing Advance in cell A > 0.

Message:     HANDOVER COMPLETE

5)	The MS shall send again the SETUP message to the SS using the same value in the N(SD) field, just after the HANDOVER COMPLETE message has been sent.

II.5.3.6.5.4.10	Procedure 10: Handover during call establishment, SDCCH to TCH/FS all without FH, Synchronized

II.5.3.6.5.4.10.1	Purpose

When the SS sends HANDOVER COMMAND, it may occur that the MS had just sent a (MM or CC) Layer 3 message which has not yet been acknowledged. The purpose of this test is to check that the MS will send again this message after the HANDOVER COMPLETE (using the same value in the N(SD) field).

To verify that when the MS is made to make a handover to a synchronized cell, it sends a specific number of access burst on the main DCCH, and then activates the channels.

II.5.3.6.5.4.10.2	Test

a)	The Call establishment shall be performed on a SDCCH/8.

The test of Procedure 6 will be used, except that the HANDOVER COMMAND message will be sent at a very well selected time. For example, the HANDOVER COMMAND will be sent in a mobile originating call establishment procedure.

The channel description information element in the HANDOVER COMMAND message shall indicate TCH/FS, and the channel mode Information Element shall be present and set to "signalling only".

b)	The SS will store the SETUP message and send a HANDOVER COMMAND message before having acknowledged the last Layer 2 frame of the SETUP message.

II.5.3.6.5.4.10.3	Requirements

1)	In four successive slots on the main DCCH, the MS shall send access bursts at the power level specified in HANDOVER COMMAND with the correct handover reference.

2)	The MS shall activate the channel in sending and receiving mode, and connect the channel in the range of (0..3) seconds after the transmission of the HANDOVER COMMAND on the SS-side.

3)	The MS shall establish a link.

4)	The MS shall send HANDOVER COMPLETE on the target cell so that the SS receives the message within T3103 seconds after sending the HANDOVER COMMAND message, using the correct Timing Advance defined according to the previous case in cell A and the value of k (see note).

Note:	The two cells A and B are synchronised, but the SS will simulate the case where distance (A,MS) is unequal to distance (B,MS). For example, the BCCH(A) will be sent a [k] bits period before BCCH(B), Timing Advance in cell B = 2k + Timing Advance in cell A > 0.

Message:     HANDOVER COMPLETE

5)	The MS shall send again the SETUP message to the SS using the same value in the N(SD) field, just after the HANDOVER COMPLETE message has been sent.

II.5.3.6.5.4.11	Procedure 11: Handover with time specified

II.5.3.6.5.4.11.1	Purpose

To verify that the MS can perform any type of handover, according to procedures 1 to 10 taking into account the optional field "starting time".

II.5.3.6.5.4.11.2	Test

All tests according to procedures 1 to 10  are repeated with the following difference: the HANDOVER COMMAND message contains a starting time information element set to a value corresponding to 1 second + 0.1 second after the point in time when the HANDOVER COMMAND message was sent.

II.5.3.6.5.4.11.3	Requirement

As in Procedures 1 to 10, except that the MS shall start transmitting access bursts at the ordered point in time.

II.5.3.6.5.4.12	Procedure 12: Handover Failure, Layer 3 Failure on the Target Cell, TCH/FS with FH to TCH/FS without FH

II.5.3.6.5.4.12.1	Purpose of the test

To verify the function of timer T3124 and the contents in message HANDOVER FAILURE.

Note:	A time out on timer T3124 is produced in the following way: the PHYSICAL INFORMATION does not arrive from the target cell.

II.5.3.6.5.4.12.2	Test

a)	After sending the HANDOVER COMMAND on the cell A, cell B is not transmitting PHYSICAL INFORMATION as expected by the MS.

Message:     HANDOVER COMMAND

Information Element	Comment	Value

Protocol Discriminator	RR management	0110

Transaction identifier	not relevant	0000

Message type	handover command	0010	1011

Cell description	default

Channel description	Bm + ACCHs, TN = 7	0000	1111

TSC, H=0, FB-NO	xxx0	0000

ARFCN	0xxx	xxxx

Handover reference	100 dec	0110	0100

Power command	5 dec	0000	0101

Synchronization indic	non synchronized cells	1101	0000

II.5.3.6.5.4.12.3	Requirements

1)	The MS shall establish a link

2)	The MS shall send HANDOVER FAILURE on cell A at the previously assigned power level. The transmission time shall be in the range of (0..3) s after transmission time of HANDOVER COMMAND on SS-side.

Message:     HANDOVER FAILURE

3)	The MS shall use the power level used before the HANDOVER COMMAND.

II.5.3.6.5.4.13	Procedure 13: Handover Failure, Layer 1 Failure on the Target Cell, TCH/FS without FH to TCH/FS with FH

II.5.3.6.5.4.13.1	Purpose of the test

To verify the function of timer T3124 and the contents of HANDOVER FAILURE.

Note:	A time out on timer T3124 in this test indicates that the Layer 1 signalling on cell B is not operating.

II.5.3.6.5.4.13.2	Test

After sending the HANDOVER COMMAND on cell A, cell B is not transmitting at all (except for normal BCCH signalling). The MS will have a time out on timer T3124. The MS will take no further action at all on cell B. It activates channels on cell A and sends HANDOVER FAILURE  on cell A.

II.5.3.6.5.4.13.3	Requirements

1)	The MS shall send HANDOVER FAILURE on cell A at the previously assigned power level. The transmission time shall be in the range of (0..3) seconds after the transmission time of HANDOVER COMMAND on SS-side.

Message:     HANDOVER FAILURE

II.5.3.6.6	Frequency redefinition

The procedure is specified in GSM 04.08 section 3.4.5.

II.5.3.6.6.1	Purpose of the test

To verify that the MS, after receiving a frequency redefinition message, starts using the new frequencies and hopping sequence at the time indicated in the message.

II.5.3.6.6.2	Setup of the test parameters

A random value ca(1) in the range 80,...,124 is drawn.

A random subset CA(1) of the set {1,...,124} containing ca(1) elements is drawn.

An element B of the set CA(1) is randomly drawn.

Let T(1) = 40, T(2) = 42000.

A random value T(3) in the range 41,...,41999 is drawn.

II.5.3.6.6.3	Initial conditions

The SS simulates a BSS with exactly one cell. The cell uses the frequency hopping mode. CCCH-CONF indicates 1 basic physical channel, not combined with SDCCHs. The cell allocation of the cell corresponds to the set CA(1) (see Note 2 of II.5.3.6.6.6).

II.5.3.6.6.4	Procedure

a)	The procedure described in b) to e) is performed for each combination of a value T(k) (k = 1,2,3) on the one side, and one of the channel types

-	TCH/FS, TCH/HS, or SDCCH, if the MS is able to manage TCH/HS;

-	TCH/FS, or SDCCH, if the MS is not able to manage TCH/HS.

(This results in 9 or 6 test cases.)

b)	Random values ca(2) in the range 20,...,79, and ca(3) in the range 4,...,19 are randomly drawn. Subsets CA(i) of {1,...,124} with ca(i) elements and containing B are randomly drawn (i = 2,3).

For j = 1,2,3, values ma(j) in the range j,...,min(64, ca(j)-1) and values maio(j) in the range 0,...,ma(j)-1 are randomly drawn.

Subsets MA(j) of CA(j) not containing B and having ma(j) elements are randomly drawn.

c)	As described in II.5.3.6.1, the MS is paged, and it is allocated a dedicated channel of the appropriate type by an IMMEDIATE ASSIGNMENT message.

Parameters of this message:

Starting time: not present.

Mobile allocation: corresponds to the set MA(1)

(cf. Note 2 of II.5.3.6.6.6).

Channel description:

Channel type as specific for the test case with

randomly generated subchannel if appropriate.

RF hopping channel indicated (encoded as H = 1).

HSN indicates hopping sequence number 0 (encoded as 000000).

MAIO indicates MAIO(1).

Other values as in II.5.3.6.1.3.2.

d)	The SS sends on the main DCCH a FREQUENCY REDEFINITION message to the MS.

Parameters of this message:

Cell channel description: corresponds to the set CA(2)

(see note 2 of II.5.3.6.6.6).

Mobile allocation: corresponds to the set MA(2)

(see note 2 of II.5.3.6.6.6).

Starting time: corresponds to the frame number modulo 42432 which is T(k) frames ahead of the transmission time of the last burst containing a part of the message

(see note 3 of II.5.3.6.6.6).

MAIO indicates MAIO(2).

All other parameters are as in the IMMEDIATE ASSIGNMENT of c).

e)	The SS sends on the main DCCH a FREQUENCY REDEFINITION message to the MS.

Parameters of this message:

Cell channel description: corresponds to the set CA(3)

(see note 2 of II.5.3.6.6.6).

Mobile allocation: corresponds to the set MA(3)

(see note 2 of II.5.3.6.6.6).

Starting time: corresponds to the frame number modulo 42432 which is T(k) frames ahead of the transmission time of the last burst containing a part of the message

(see note 3 of II.5.3.6.6.6).

MAIO indicates MAIO(3).

II.5.3.6.6.5	Requirements

1)	In step d) and e), the MS must correctly modify the frequencies/hopping sequences it uses at the exact indicated time slot.

II.5.3.6.6.6	Notes

1)	The verification is performed on the RF burst level, the MS transmits the Standard Test Signal C1 as defined in GSM 11.40, and for the TCH case, the SS checks the received pattern with the expected pattern. For the SDCCH test the MS transmits fill frames, and the SS checks for each burst wether a burst is transmitted at the right frequency.

2)	A subset of {1,...,124} corresponds to the cell allocation (or mobile allocation) of those RF channels whose ARFCN belongs to the subset. The cell allocation (mobile allocation) is encoded as described in section 10.5.2.1 of GSM 04.08.

3)	A frame number FN modulo 42432 is encoded as described in section 10.5.2.20 of GSM 04.08.

II.5.3.6.7	Transmission mode change

Axerud

The Transmission Mode Change procedure is described in section 3.4.6 of GSM 04.08.

II.5.3.6.7.1	Purpose of test

The purpose of the test is to verify that the MS changes mode of operation, and acknowledges this to the NW.

Note:	The Transmission Mode Change Procedure is used to change from different modes of coding (e.g. from 9.6/F to 4.8/F).

II.5.3.6.7.2	Method of measurement

a)	The test starts with the MS having a speech call in progress on a TCH/F channel.

b)	The SS sends CHANNEL MODE MODIFY.

Note:	The MS is supposed to respond with CHANNEL MODE MODIFY ACKNOWLEDGE. Only parameter valued within the messages are verified. The coding is not tested here.

II.5.3.6.7.3	Procedure

a)	The MS is put into a speech call in progress on a TCH/F-channel.

b)	The MS sends CHANNEL MODE MODIFY message.

Message:     CHANNEL MODE MODIFY (GSM 04.08, 9.1.5)

Information Element	Comment	Value

Protocol Discriminator	RR management		0110

Transaction Identifier	not relevant	0000

Message Type	Ch mode modif	0001	0000

Channel description		-

- Ch type and offset	Bm + ACCH's no offset	   0	0001

- TN	Timeslot number	(current)

- Hopping channel	Non-hopping	0

- Channel selector	used ARFCN	(current)

- CA-NO	Cell allocation number, band 0	00

- HSN	Non-hopping mode	  00	0000

Channel mode		-

- Mode (select one)	1 Signalling	0000	0000

- "         "	2 speech full rate	0000	0001

- "         "	3 speech half rate	0000	0101

- "         "	4 data 9.6	0000	0011

- "         "	5      4.8/F	0000	1011

- "         "	6      4.8/H         	0000	0000

- "         "	7      2.4/F	0001	0011

- "         "	8      2.4/H	0001	0111

b.1)	b) with Channel mode set to 4.

b.2)	b) with Channel mode set to 5.

b.3)	b) with Channel mode set to 7.

c)	This is then performed with a half rate channel and

c.1)	Channel mode is set to 6.

c.2)	Channel mode is set to 8.

II.5.3.6.7.4	Requirements

1)	For every change of mode that the MS is capable to handle, it sends CHANNEL MODE MODIFY ACKNOWLEDGE with corresponding value.

Message:     CHANNEL MODE MODIFY ACKNOWLEDGE (GSM 04.08, 9.1.6)

Information Element	Comment	Value

Protocol Discriminator	RR management		 011

Transaction Identifier	RR	0000

Message Type	Ch mode modif ack 	0001	0001

Channel description		-

- IEI	Ch descr	0110	0100

- Ch type and offset	Bm + ACCH's no offset	   0	0001

- TN	Timeslot number	(current)

- Hopping channel	Non-hopping	0

- Channel selector	used ARFCN	(current)

- CA-NO	Cell allocation number, band 0	00

- HSN	Non-hopping mode	  00	0000

Channel mode		-

- IEI 	Ch mode	0110	0110

- Mode (select one)	1 Signalling	0000	0000

- "            "	2 speech full rate	0000	0001

- "            "	3 speech half rate	0000	0101

- "            "	4 data 9.6	0000	0011

- "            "	5      4.8/F	0000	1011

- "            "	6      4.8/H	0000	0000

- "            "	7      2.4/F	0001	0011

- "            "	8      2.4/H	0001	0111

II.5.3.6.8	Ciphering mode setting

McCabe/Prohaska

The procedure of Ciphering Mode Setting is specified in section 3.4.7 of GSM 04.08.

II.5.3.6.8.1	Purpose of the test

To verify that the MS responds correctly to the CIPHERING MODE COMMAND message and adopts the mode as indicated in the cipher mode setting.

II.5.3.6.8.2	Initial conditions

a)	The SS shall simulate a BSS with CCCH and BCCH.

b)	The MS shall be placed in the MM-state "idle,updated" (ref. location updating accepted II.5.3.7.4.1).

II.5.3.6.8.3	Method of the test

II.5.3.6.8.3.1	Procedure 1

a)	The MS shall be made to originate a call (sequence described in GSM 04.08 fig 7.8a) and shall successfully authenticate to the SS.

b)	On receipt of the successful AUTHENTICATION RESPONSE message from the MS, the SS shall send the CIPHERING MODE COMMMAND with cipher mode setting = "start ciphering" and start deciphering.

Message:     CIPHERING MODE COMMAND (GSM 04.08, 9.1.9)

Information Element	Comment	Value

Protocol Discriminator	RR management	0110

Transaction identifier	not relevant	0000

Message type		0011	0101

Cipher mode setting	"start ciphering"	0001

c)	The SS sends a message AUTHENTICATION REQUEST.

II.5.3.6.8.3.2	Procedure 2

a)	With initial conditions as specified, repeat Procedure 1, points a) and b) with cipher mode setting = "no ciphering".

Same message, but with cipher mode setting set to no ciphering, encoded as 0000.

II.5.3.6.8.3.3	Procedure 3

a)	With initial conditions as specified, with a cipher key previously stored, and authenticated, the MS shall be made to originate a call.

b)	Upon receipt of the CM SERVICE REQUEST message from the MS, the SS shall send the CIPHERING MODE COMMAND with cipher mode setting = "start ciphering".

II.5.3.6.8.3.4	Procedure 4

a)	With initial conditions as specified, repeat Procedure 1, points a) and b).

b)	The SS ignores messages coming from the MS.

II.5.3.6.8.4	Requirements

The following requirements apply to the corresponding Procedures in the previous section II.5.3.6.8.3.

II.5.3.6.8.4.1	Requirements 1

1)	The MS shall respond with the CIPHERING MODE COMPLETE message in ciphered mode using the cipher key as determined during authentication and continue to establish the call with a SETUP message.

Message:     CIPHERING MODE COMPLETE (GSM 04.08, 9.1.10)

Information Element	Comment	Value

Protocol Discriminator	RR management	0110

Transaction Identifier		0000

Message type		0011	0010

2)	After reception of AUTHENTICATION REQUEST, the MS shall respond with AUTHENTICATION RESPONSE, it shall store the new ciphering key, and continue to use the old one.

II.5.3.6.8.4.2	Requirements 2

1)	The MS shall respond to the CIPHERING MODE COMMAND with a CIPHERING MODE COMPLETE message in non-ciphered mode and continue call establishment.

II.5.3.6.8.4.3	Requirements 3

1)	The MS shall respond with the CIPHERING MODE COMPLETE message in ciphered mode using the old cipher key.

2)	The MS shall continue to establish the call with a SETUP message.

II.5.3.6.8.4.4	Requirements 4

1)	The MS shall respond with the CIPHER MODE COMPLETE message in ciphered mode.

2)	The MS shall repeat this message after T200 expiry, N200 times. Then the MS shall release the RR connection.

II.5.3.6.9	TEST OF Additional Assignment

Honkasalo/Freeman

II.5.3.6.9.1	Introduction

The Additional Assignment procedure is used to change an MS channel configuration from Lm + ACCH to Lm + Lm + ACCH. It is therefore only relevant to those mobiles which perform such operation. The procedure is described in section 3.4.8 of GSM 04.08.

The configuration is not yet applicable to the GSM Standard. A test is not specified.

II.5.3.6.9.2	Purpose of the test

The purpose of this test is to verify that (if equipped) the MS can correctly set up an Lm + Lm + ACCH channel.

II.5.3.6.9.3	Procedure

a)	The MS is put into a Lm + ACCH call as described in test [t.b.d]. [EDITORIAL: CORRECT REFERENCE ?]

b)	The SS sends an ADDITIONAL ASSIGNMENT message to the MS on the FACCH.

Message:     ADDITIONAL ASSIGNMENT

Information Element	Comment	Value

Protocol Discriminator	RR management

Transaction Identifier	RR

Message Type	additional assignment

Channel Description

II.5.3.6.9.4	Response Requirement

The MS sets up the Lm + Lm + ACCH channel described in the ADDITIONAL ASSIGNMENT message and sends an ASSIGNMENT COMPLETE message to the SS on the first FACCH.

Message:     ASSIGNMENT COMPLETE

Information Element	Comment	Value

Protocol Discriminator	RR management

Transaction Identifier 	RR

Message Type 	assignment complete

Cause	normal release

II.5.3.6.10	TEST OF Partial release

The elementary procedure of Partial Release is specified in GSM 04.08 section 3.4.9.

Introduction:

As this procedure applies to the transition of the configuration

Lm + Lm + ACCHs to Lm + ACCHs, it is not carried out on a "simple MS".

The configuration is not yet applicable to the GSM Standard. A test is not specified.

For the distinction of a "simple MS", refer to section [t.b.p.]. [EDITORIAL: REFERENCE ?]

Purpose of the test:

To verify that an MS will correctly deactivate a part of the dedicated channel in use, under various conditions, concerning:

-	channel in use (hopping/non hopping),

-	part to be deactivated,

-	abnormal case.

Method of test:

a)	The procedure of section II.5.3.6.9 (additional assignment) is carried out.

Note 1:	That procedure includes call establishment by referring to section II.5.3.5.1.2.[EDITORIAL: Correct Reference ?]

Note 2:	The MS is now in a state where it operates in the  configuration  Lm + Lm + ACCH.

b)	The SS provides user data on both Lm channels.

c)	The MS is made to transmit user data to the SS on both the return channels.

d)	The SS sends a PARTIAL RELEASE message to the MS on the main DCCH, indicating the release of one of the two Lm channels. However, the SS continues to send on both Lm channels. The PARTIAL RELEASE message shall have the following content:

Message:     PARTIAL RELEASE

Information Element	Comment	Value

Protocol Discriminator	RR management	....	011.

Transaction Identifier	RR	0000	....

Message Type	partial release	0000	1010

Channel Type	0001A  A = subchannel

Timeslot number	real value	xxx

Hop indicator	H = 0 : fix. freq.	H

H = 1 : hopping

Channel selector	ARFCN or MAIO	....	....

Cell allocation		00

Hopping sequence no.		..xx	xxxx

Note 1:	The GSM network shall change it's configuration immediately after transmitting the PARTIAL RELEASE message.

Note 2:	The contents of the PARTIAL RELEASE message is different, depending upon:

- hopping or non-hopping channel,

- way to express the channel selector (binary AFRCN or MAIO).

Note 3:	On reception of PARTIAL RELEASE, the MS should reinitialize the connections on the data link of the connections which it maintains.

e)	The SS verifies reception of the PARTIAL RELEASE COMPLETE message.

f)	The SS verifies the remaining configuration. If a call reestablishment procedure is initiated by the MS then this shall be recorded.

g)	The SS verifies re-initialisation of the L2 links.

h)	Establishing the same original channel configuration, steps a) up to including f) are repeated for release of the other Lm channel.

i)	Steps a) up to including g) are repeated for a hopping channel.

j)	Steps a) up to including h) are repeated for the alternative way of expressing the channel selector in the PARTIAL RELEASE message in step d).

k)	Steps a) up to including i) are repeated under conditions where lower layer failures are introduced. (See II.5.3.4 for information on lower layer failures.)

Requirements:

1)	The MS shall respond with a PARTIAL RELEASE COMPLETE message within [timer T3107/ timer T3109/ reception gap timer] after sending of PARTIAL RELEASE by the SS. [EDITORIAL: HOW TO DROP THE SQUARE BRACKETS?]

2)	After transmission of PARTIAL RELEASE COMPLETE, the MS shall have released the physical channels that are no longer required.

3)	The MS shall not have initiated a call re-establishment before step j).

4)	After step k) the MS shall initiate a call re-establishment.

5)	The MS shall have re-initialized the Layer 2 links, needed after step d).

II.5.3.6.11	Classmark Change

The Classmark Change procedure is specified in section 3.4.10 and Annex A1 of GSM 04.08.

Purpose of the test

The test shall verify that any change in the classmark of the MS shall be communicated on a DCCH to the network. The procedure described shall apply only to an MS which supports this feature.

Initial conditions

1)	The SS shall simulate a BSS with CCCH and BCCH. using the default values as specified in II.5.3.1.

2)	The MS shall be placed in the MM-state "idle,updated" (ref. location updating accepted II.5.3.7.4.1).

3)	The MS shall have no additional power amplification applied.

Procedure 1	Mobile originating

a)	With the MS in the "idle,updated" state, the RF power capability shall be changed by the addition of power amplification.

b)	The MS shall be made to originate a call.

Requirements 1

1)	The MS shall include the new RF power capability in the CLASSMARK2 information element of the CM SERV REQ message (GSM 04.08 section 9.2.7).

Procedure 2

a)	In the course of the call, the RF power capability of the MS shall be changed by removal of the additional power amplification.

Requirements 2

1)	The MS shall send a CLASSMARK CHANGE message to the SS with the new CLASSMARK2 information element.

Procedure 3

a)	In the course of the call, the RF power capability of the MS shall be changed by addition of power amplification.

Requirements 3

1)	The MS shall send a CLASSMARK CHANGE message to the SS with the new CLASSMARK2 information element.

Procedure 4	Mobile terminating

a)	The MS shall be placed in the state "idle,updated" and the additional power amplification removed.

b)	The SS shall page the MS.

Requirements 4

1)	The MS shall include the new RF power capability in the CLASSMARK2 information element of the PAGING RESPONSE message (GSM 04.08 section 9.1.24).

Message:     Classmark Change  (GSM 04.08, 9.1.11)

Information Element	Comment	Value

Protocol Discriminator	RR management		0110

Transaction identifier	not relevant		 000

Message type	Classmark Change	0001	0110

MS Classmark2	length 	0000	0100

Rev/A5/RF Power	xxx0	00??

1000	x000

x - indicates that the value is of no significance to the test

? - indicates ME dependent value.

II.5.3.6.12	Channel release

De Jesus Mendes/Rzeznik/McCabe

The procedure of Channel Release is specified in section 3.5.1 of GSM 04.08.

II.5.3.6.12.1	Purpose of the test

The purpose of the test is to verify that the MS correctly frees its dedicated channels after having received a CHANNEL RELEASE message.

II.5.3.6.12.2	Method of test

a)	The SS shall simulate a BSS with BCCH/CCCH.

b)	The MS is paged and allocated a dedicated channel and the Layer 2 signalling link is established.

c)	The SS sends a Channel Release message.

Note:	Timer T3110 is currently not defined, however, a value of 2s is

used in the test. This is the time taken to transmit N200+1 DISC

frames. (see GSM 04.06)

Procedure 1

a)	The dedicated channel assigned is a SDCCH, the SS sends Channel Release with a valid RR cause value field.

Procedure 2

a)	This procedure is the same as Procedure 1 except that the SS does not acknowledge any Layer 2 disconnection initiated by the MS.

Procedure 3

a)	The dedicated channel assigned is a TCH/FS, the sends a CHANNEL RELEASE message with a valid RR cause field value.

Procedure 4

a)	The dedicated channel assigned is a TCH/FS, the SS sends a CHANNEL RELEASE message with cause 'abnormal release unspecified' (#1).

b)	The SS does not acknowledge any Layer 2 disconnection initiated by the MS.

II.5.3.6.12.3	Requirements

The requirements below apply to the corresponding procedures in section II.5.3.6.12.2.

Requirements 1

1)	The MS shall initiate a Layer 2 disconnection process on the main signalling link.

2)	After the acknowledgement of the Layer 2 disconnection by the SS, the MS shall not produce any further RF-transmission.

3)	The MS shall return to the idle state. It is verified through the paging procedure at which the MS should respond.

Requirements 2

1)	The MS sends at least two L2 disconnect frames.

2)	After having received the first L2 disconnect frame, the SS waits  2 s and verifies that there is no more transmissions from the MS.

3)	The MS shall return to the idle state. It is verified through the paging procedure at which the MS should respond.

Requirements 3

1)	The MS shall initiate a Layer 2 disconnection process on the main signalling link.

2)	After the acknowledgement of the Layer 2 disconnection by the SS, the MS shall not produce any further RF-transmission.

3)	The MS shall return to the idle state. It is verified through the paging procedure at which the MS should respond.

Requirements 4

1)	The MS sends at least two L2 disconnect frames.

2)	After having received the first L2 disconnect frame, the SS waits  2 s and verifies that there is no more transmissions from the MS.

3)	The MS shall return to the idle state. It is verified through the paging procedure at which the MS should respond.

Message:     CHANNEL RELEASE (GSM 04.08, 9.1.7)

Information Element	Comment	Value

Protocol Discriminator	RR	0110

Transaction Identifier	not relevant	0000

Message Type		0000 1101

RR cause	normal release	0000 0000