II.8	Testing of the ME/SIM (Subscriber Identification Module) interface

The SIM is specified in GSM 11.11.

Testing of the SIM itself is not part of this specification.

General

The following tests concentrate upon procedures which have an impact upon the performance of the ME on the network; i.e. following the usual criteria that type approval protects the network and other users, but does not validate the performance of the equipment under test.

The following sequence of tests for the SIM/ME interface confirms:

a)	the correct interpretation of data read by the ME from the SIM;

b)	the correct writing of data by the ME to the SIM;

c)	the initiation of appropriate procedures by the ME involving transfer of information between ME and SIM.

The tests must be considered as a continuous sequence requiring that information written to or deleted from the SIM or ME memories is carried forward into the succeeding tests.

Note:	A SIM Simulator will be required as part of the SS.

A SIM Programmer/Reader is required as part of the SS.

The tests of High Level Procedures (Test Sequences 1 to 8) have been designed using the substitution method of testing. It is probable that instead of a number of SIMs programmed to values defined in Tables II.8-1 and II.8-2 the SIM Simulator will be used. However, as these tests require use also of the Um interface, there is concern that rf may disturb the SIM Simulator. For the present, therefore, the tests are still described for the substitution method. If the SIM Simulator is found reliable for all tests then the requirement for a Programmer/Reader as part of the SS can be dropped.

Test Sequences 1 to 8

To perform these test sequences SIMs will be required programmed with specific data field values as defined in Tables II.8-1 and II.8-2 following.

These tables also give the expected values of the data fields at the completion of each test sequence.

(For a full explanation of each data field see GSM 11.11.)

TABLE II.8-1: SIM DATA FIELD VALUES for the SIM/ME Interface Test Sequences

At start of test sequ.  │ 1 │ 2 │ 3 │ 4 │   │ 5 │   │ 6 │   │   8    │ 13 │

At end of test sequence │ 1 │ 2 │ 3 │   │ 4 │   │ 5 │   │ 6 │   8    │ 13 │

════════════════════════Ï═══Ï═══Ï═══Ï═══Ï═══Ï═══Ï═══Ï═══Ï═══Ï════════Ï════Á

Data Field     \  Value

\

_________________\_____________________________________________________

│   │   │   │   │   │   │   │   │   │        │    │

IMSI                    │ A │ A │ A │ B │ B │ B │ B │ A │ A │ see    │  x │

│   │   │   │   │   │   │   │   │   │ test 8 │    │

────────────────────────┼───┼───┼───┼───┼───┼───┼───┼───┼───┼────────┼────┤

LOCATION INFORMATION    │   │   │   │   │   │   │   │   │   │        │    │

(LAI & TMSI)            │ A │ B │ C │ A │ D │ F │ E │ B │ B │  B     │  x │

────────────────────────┼───┼───┼───┼───┼───┼───┼───┼───┼───┼────────┼────┤

Kc and Cipher Key       │ A │ A │ A │ A │ B │ B │ B │ A │ A │  A     │  x │

Sequence No. (n)        │   │   │   │   │   │   │   │   │   │        │    │

────────────────────────┼───┼───┼───┼───┼───┼───┼───┼───┼───┼────────┼────┤

ACCESS CONTROL          │ A │ A │ A │ A │ A │ A │ A │ A │ A │ see    │  x │

│   │   │   │   │   │   │   │   │   │ test 8 │    │

────────────────────────┼───┼───┼───┼───┼───┼───┼───┼───┼───┼────────┼────┤

FORBIDDEN PLMNs         │ A │ A │ A │ A │ A │ A │ B │ A │ A │   A    │  x │

────────────────────────┼───┼───┼───┼───┼───┼───┼───┼───┼───┼────────┼────┤

SIM SERVICE TABLE       │ A │ A │ A │ A │ A │ A │ A │ A │ A │   A    │  A │

────────────────────────┼───┼───┼───┼───┼───┼───┼───┼───┼───┼────────┼────┤

PIN                     │ x │ x │ x │ x │ x │ x │ x │ x │ x │   x    │  A │

────────────────────────┼───┼───┼───┼───┼───┼───┼───┼───┼───┼────────┼────┤

UNBLOCKING KEY          │ x │ x │ x │ x │ x │ x │ x │ A │ A │   x    │  A │

────────────────────────┼───┼───┼───┼───┼───┼───┼───┼───┼───┼────────┼────┤

PLMN SELECTOR           │ A │ A │ A │ A │ A │ A │ A │ A │ B │   A    │  x │

────────────────────────┴───┴───┴───┴───┴───┴───┴───┴───┴───┴────────┴────┘

Note:	'x' indicates that the value is of no significance to the test.

TABLE II.8-2 :  DEFINITION OF VALUES OF TABLE II.8-1

Data-Field 6F-07  :  IMSI

Value A        Value B

246813579      246811111111111

byte 1          05 H           08 H      length of IMSI

byte 2          29 H           29 H      first digit, parity coding

byte 3          64 H           64 H      digits

byte 4          18 H           18 H

byte 5          53 H           11 H

byte 6          97 H            "

byte 7          FF H            "

byte 8          FF H            "

byte 9          FF H            "

TABLE II.8-2 cont.

Data-Field 6F-7E  :  Location Information

Value A   Value B   Value C   Value D    Value E    Value F

LAI - MCC  234      234      234        234        234        234

MNC  01       01       01         01         06         01

TMSI       Null     1234     12345678   23456789  34567890  23456789

TMSI Time- Null     Null     Null       Null       Null       Null

byte 1            FF H     00 H     21 H       32 H       43 H       23 H

byte 2            FF H     00 H     43 H       54 H       65 H       45 H

byte 3            FF H     21 H     65 H       76 H       87 H       67 H

byte 4            FF H     43 H     87 H       98 H       09 H       89 H

byte 5            32 H     32 H     32 H       32 H       32 H       32 H

byte 6            F4 H     F4 H     F4 H       F4 H       F4 H       F4 H

byte 7            10 H     10 H     10 H       10 H       60 H       10 H

byte 8            00 H     00 H     00 H       xx H       xx H       00 H

byte 9            00 H     00 H     00 H       xx H       xx H       00 H

byte 10           FF H     FF H     FF H       xx H       xx H       FF H

byte 11           07 H     07 H     07 H       xx H       xx H       07 H

xx = Don't care. Depends on MS and SS test implementations.

Data-Field 6F-20  : Cipher Key (Kc) and Cipher Key Sequence No. (n)

Value A            Value B

byte 1          xx   }            bytes 1-8 as

byte 2          xx   }            determined by Authentication

byte 3          xx   }            Algorithm

byte 4          xx   }  Kc

byte 5          xx   }

byte 6          xx   }

byte 7          xx   }

byte 8          xx   }

byte 9          01 H  n=001       02 H n=010

Data-Field 6F-78  : Access Control

One Access Class from 0-9 to be allocated compatible with those allowed by the SS.

TABLE II.8-2 cont.

Data-Field 6F-7B  : Forbidden PLMNs

Value A            Value B

byte 1    32 H  MCC = 234    32 H  MCC = 234   }

byte 2    F4 H               F4 H              }  First forbidden

byte 3    20 H  MNC = 02     30 H  MNC = 03    }  PLMN (oldest)

byte 4    32 H  MCC = 234    32 H  MCC = 234   }

byte 5    F4 H               F4 H              }  2nd forbidden

byte 6    30 H  MNC = 03     40 H  MNC = 04    }  PLMN

byte 7    32 H  MCC = 234    32 H  MCC = 234   }

byte 8    F4 H               F4 H              }  3rd forbidden

byte 9    40 H  MNC = 04     50 H  MNC = 05    }  PLMN

byte 10   32 H  MCC = 234    32 H  MCC = 234   }

byte 11   F4 H               F4 H              }  4th forbidden

byte 12   50 H  MNC = 05     10 H  MNC = 01    }  PLMN (newest)

Data-Field 6F-38  :  SIM Service Table

Value A :

byte 1   xxxxxx11 - PIN Disable function allocated and activated

byte 2   xxxxxx11 - Charging Counter allocated and activated

Value B :

byte 1   xxxxxx01 - PIN Disable function allocated but

not activated

byte 2   xxxxxx11 - Charging Counter allocated and activated

Data-Field 6F-30 : PLMN Selector

Value A             Value B

byte 1           32H  MCC = 234      32H  MCC = 234

byte 2           F4H                 F4H

byte 3           10H  MNC = 01       10H  MNC = 01

byte 4           32H  MCC = 234      65H  MCC = 567

byte 5           F4H                 F7H

byte 6           20H   MNC = 02      10H  MNC = 01

byte 7           32H   MCC = 234     32H  MCC = 234

byte 8           F4H                 F4H

byte 9           30H   MNC = 03      30H  MNC = 03

byte 10          32H   MCC = 234     32H  MCC = 234

byte 11          F4H                 F4H

byte 12          40H   MNC = 04      40H  MNC = 04

byte 13          32H   MCC = 234     32H  MCC = 234

byte 14          F4H                 F4H

byte 15          50H   MNC = 05      50H  MNC = 05

byte 16          32H   MCC = 234     32H  MCC = 234

byte 17          F4H                 F4H

byte 18          60H   MNC = 06      60H  MNC = 06

byte 19          42H   MCC = 246     42H  MCC = 246

byte 20          F6H                 F6H

byte 21          18H   MNC = 81      18H  MNC = 81

byte 22          42H   MCC = 246     42H  MCC = 246

byte 23          F6H                 F6H

byte 24          28H   MNC = 82      28H  MNC = 82

TABLE II.8-2 cont.

PIN

Value A

2468

UNBLOCKING KEY

Value : 13243546

Data-Field Values of other data fields are of no significance for the tests which follow.

II.8.1	Test Sequence 1

Purpose:

1)	To confirm that the ME reads IMSI from SIM as part of the SIM/ME initialisation procedure.

2)	To confirm that the ME can handle IMSIs of less than the maximum allowed value.

Procedure:

a)	The SS is set up to transmit on the BCCH with the following network parameters:

Attached/detach disabled

LAI : MCC = 246

MNC = 81

Access Control: Unrestricted.

b)	The SIM defined in Tables II.8-1 and II.8-2 above is installed into the ME.

c)	The MS is powered-on.

d)	The SS sends PAGe REQuest to the MS using IMSI (value A).

Requirements:

1)	The MS shall send CHANnel REQuest to the SS.

Procedure:

e)	The SS shall send IMM ASSIGN to the MS.

Requirements:

2)	The MS shall send PAG RES to the SS containing IMSI (value A).

Procedure:

f)	The SS shall send RELEASE to the MS.

II.8.2	Test Sequence 2

Purpose:

1)	To confirm that ME reads TMSI from SIM as part of SIM/ME initialisation procedure.

2)	To confirm that the can handle TMSIs of less than maximum length.

Procedure:

a)	The SS sends PAGe REQuest to the MS using TMSI (value B).

Requirements:

b)	The MS shall send CHANnel REQuest to the SS.

Procedure:

c)	The SS shall send IMM ASSIGN to the MS.

Requirements:

d)	The MS shall send PAG RES to the SS containing TMSI (value B).

Procedure:

e)	The SS shall send RELEASE to the MS.

II.8.3	Test Sequence 3

Purpose:

1)	To confirm that when a SIM with a different TMSI value is introduced into the ME, causing a new SIM/ME initialisation procedure to take place, that the values previously used by the ME have been deleted.

2)	To confirm that the ME can handle a TMSI values of maximum length.

Procedure:

a)	The SIM defined in Tables II.8-1 and II.8-2 above is installed into the ME, and the MS powered-on.

b)	The SS sends PAGe REQuest to the MS using TMSI (value B).

Requirements:

c)	The MS shall not respond to the PAGe REQuest.

Procedure:

d)	The SS sends PAGe REQuest to the MS using TMSI (value C).

Requirements:

e)	The MS shall send CHANnel REQuest to the SS.

Procedure:

f)	The SS shall send IMM ASSIGN to the MS.

Requirements:

g)	The MS shall send PAG RES to the SS containing TMSI (value C).

h)	The SS shall send RELEASE to the MS.

II.8.4	Test Sequence 4

Purpose:

1)	To confirm that when a SIM with a different IMSI value is introduced into the ME, causing a new SIM/ME initialisation procedure to take place, that the value previously used by the ME has been deleted.

2)	To confirm that the ME can handle a IMSI value of maximum length.

3)	To confirm that after Ciphering Mode Setting the ME updates the SIM with the new value of Cipher Key Sequence Number (n), at call termination.

4)	To confirm that the TMSI data field in the SIM is correctly updated by the ME at call termination.

Procedure:

a)	The SIM defined in Tables II.8-1 and II.8-2 above is installed into the ME, and the MS powered-on.

b)	The SS sends PAGe REQuest to the MS using IMSI (value A).

Requirements:

1)	The MS shall not respond to the PAGe REQuest.

Procedure:

c)	The SS sends PAGe REQuest to the MS using IMSI (value B).

Requirements:

2)	The MS shall send CHANnel REQuest to the SS.

Procedure:

d)	The SS shall send IMMI ASSIGN to the MS.

Requirements:

3)	The MS shall send PAG RES to the SS containing IMSI (value B).

4)	The SS shall send AUThentication REQuest to the MS containing Cipher Key Sequence Number set to binary 010.

5)	The MS shall send AUThentication RESponse to the SS.

6)	The Cipher Key Sequence Number in the SIM shall be the value '010' when read in Requirement 9) below.

Procedure:

e)	The SIM shall send TMSI REALlocation to the MS containing the TMSI (value D).

Requirement:

7)	The MS shall send TMSI REALlocation Complete to the SS.

8)	The TMSI data field in the SIM shall contain the value D when read in Requirement 9) below.

Procedure:

f)	Within 5 seconds the SS shall send RELEASE to the MS.

g)	The MS is hard powered down, e.g battery removed. (This is to confirm that the call termination procedure causes SIM updating and not soft power-down).

Requirements:

9)	The SIM shall be removed from the MS, and installed in a SIM reader. It shall contain the values listed in Tables II.8-1 and II.8-2 above.

II.8.5	Test Sequence 5

Purpose:

1)	To confirm that the ME reads the value of the Forbidden PLMN field as part of the SIM/ME Initialisation procedure and does not attempt to access these PLMNs in automatic PLMN selection mode.

2)	To confirm that the Forbidden PLMN data field in the SIM is correctly updated by the ME at soft power-down.

3)	To confirm that the Location Information data field in the SIM is correctly updated by the ME at soft power-down.

Procedure:

a)	The SIM defined in Tables II.8-1 and II.8-2 above is installed into the ME, and the MS powered-on. The MS is made to operate in automatic PLMN selection mode.

b)	The SS provides a BCCH with default values and:

LAI : MCC = 234

MNC = 02.

Requirement:

1)	The MS shall not attempt a LOCation UPDate.

Procedure:

c)	The SS stops all RF output on the BCCH for a long enough period of time to cause a reselection in the MS. The BCCH is changed to contain:

LAI : MCC = 234

MNC = 03.

The SS then resumes RF output on the BCCH.

Requirement:

2)	The MS shall not attempt a LOCation UPDate.

Procedure:

d)	The SS stops all RF output on the BCCH for a long enough period of time to cause a reselection in the MS. The BCCH is changed to contain:

LAI : MCC = 234

MNC = 04.

The SS then resumes RF output on the BCCH.

Requirement:

3)	The MS shall not attempt a LOCation UPDate.

Procedure:

e)	The SS stops all RF output on the BCCH for a long enough period of time to cause a reselection in the MS. The BCCH is changed to contain:

LAI : MCC = 234

MNC = 05.

The SS then resumes RF output on the BCCH.

Requirement:

4)	The MS shall not attempt a LOCation UPDate.

Procedure:

f)	The SS stops all RF output on the BCCH for a long enough period of time to cause a reselection in the MS. The BCCH is changed to contain:

LAI : MCC = 234

MNC = 01.

The SS then resumes RF output on the BCCH.

Requirement:

5)	The MS shall send CHANnel REQuest to the SS.

Procedure:

g)	The SS shall send IMM ASSIGN to the MS.

Requirement:

6)	The MS shall send Location UPDate Request to the SS.

Procedure:

h)	The SS shall send LOC UPD REJ to the MS with the reason "PLMN Not Allowed", followed by CHANnel RELease.

Requirement:

7)	The Forbidden PLMN data field in the SIM shall contain the Value B when read in Requirement 12) below.

Procedure:

i)	The SS stops all RF output on the BCCH for a long enough period of time to cause a reselection in the MS. The BCCH is changed to contain:

LAI : MCC = 234

MNC = 06.

Requirement:

8)	The MS shall send CHANnel REQuest to the SS.

Procedure:

j)	The SS shall send IMM ASSIGN to the MS.

Requirement:

9)	The MS shall send LOC UPD REQ to the SS.

Procedure:

k)	The SS shall send LOC UPD ACC with

LAI : MCC = 234

MNC = 06

and TMSI: 34567890

to the MS.

Requirements:

10)	The MS shall respond with TMSI REAL COM.

11)	The Location Area Identification data field in the SIM shall contain the value E when read in Requirement 12 below.

Procedure:

l)	The SS shall send RELEASE to the MS.

m)	The MS is soft powered-down.

Requirements:

12)	The SIM shall be removed from the MS, and installed in a SIM reader.

It shall contain the values listed in Tables II.8-1 and II.8-2 above.

II.8.6	Test Sequence 6

Purpose:

To check PLMN Selector Updating Procedure.

Procedure:

a)	The SIM defined in Tables II.8-1 and II.8-2 above is installed into the ME, and the MS powered-on.

b)	The MS is powered-on.

c)	The second PLMN in the PLMN selector list shall be entered as

MCC = 567  MNC = 01. (MMI dependent)

d)	The SIM shall be removed from the MS, and installed in a SIM reader.

Requirement:

The SIM shall contain the PLMN MCC = 567 MNC = 01 in the second position of the PLMN selector list.

II.8.7	Test Sequence 7 - ELECTRICAL TESTS

Purpose of the test

Testing of Electrical characteristics of the SIM/ME interface.

Whilst non-conformance in this area would be unlikely to cause difficulties to other users or the network (type approval criteria) significant deviations from the specifications (GSM 11.11 and ISO 7816) may damage the SIM. If an attempt is then made to use the SIM in a different ME, then its failure may reflect badly on both that ME and the network.

This sequence lists the electrical tests to be run and when.

The first part describes the tests to be run during each phase of a transaction with the card.

The second part deals with tests run on each contact, and with static and dynamic tests:

1 -	All Power on/off phases;

2 -	All voltages, currents and signals characteristics on

each contacts.

However, due to the likely difficulty of accessing the terminals of the SIM/ME interface for the purposes of measurements, the ME manufacturer shall provide a test interface in accordance with section III.1.5 for the purpose of conformance testing.

These tests are mandatory for all SIM or Plug-in SIM/ME interfaces.

General Measurement conventions

The measurement conventions are specified in ISO 7816-3 section 4.2.1.

II.8.7.1	Electrical test phases

Electrical voltage, given in table here after, must be adhered to under normal and extreme test conditions. This applies in particular to temperature conditions and terminal (or coupler) power supply voltages, which are both described in standard ISO 7816/3 section 4.2.1 and in the individual ME specification.

Normal conditions

These conditions correspond to the current and voltage which must be applied during a functional assembly test for example.

┌───────────┬───────────────┬───────────────────┬─────────────────────┐

│ Contacts  │   Low level   │    High level     │ Max capacitive load │

│           │               │                   │ from test equipment │

├───────────┼───────────────┼───────────────────┼─────────────────────┤

│ C1        │               │  V = 5V  +/-10%   │                     │

│ (Vcc)     │               │  I = 10 mA        │                     │

├───────────┼───────────────┼───────────────────┼─────────────────────┤

│ C2        │  0V<=V<=0.6V  │  4V<=V<=Vcc       │      30 pF          │

│ (RST)     │  I= +200µA    │  I= +20µA         │                     │

├───────────┼───────────────┼───────────────────┼─────────────────────┤

│ C3        │  0V<=V<=0.6V  │  4V<=V<=Vcc       │      30 pF          │

│ (CLK)     │  I= +200µA    │  I= +20µA         │                     │

├───────────┼───────────────┼───────────────────┼─────────────────────┤

│ C5        │  0V           │  0V               │                     │

│ (GND)     │               │                   │                     │

├───────────┼───────────────┼───────────────────┼─────────────────────┤

│ C6        │               │  5V  +/-10%       │                     │

│ (Vpp)     │               │                   │                     │

├───────────┼───────────────┼───────────────────┼─────────────────────┤

│         I │  0V<=V<=0.8V  │  0.7*Vcc<=V<=Vcc  │                     │

│           │  I= 1mA       │  I= +/-20µA       │                     │

│ C7        │               │                   │      30 pF          │

│ (I/O)   O │  0V<=V<=0.4V  │  3.8V<=V<=Vcc     │                     │

│           │  I= 1mA       │  I= -20µA         │                     │

└───────────┴───────────────┴───────────────────┴─────────────────────┘

II.8.7.1.1	PHASE PRECEDING ME POWER ON

Purpose :		Measurement of residual voltages.

Procedure :	The power of the ME is off.

Requirement :	The voltages across the contacts (C1,C2,C3,C6,C7) shall correspond to inactive levels, i.e. between

0 and 0.4V.

II.8.7.1.2	PHASE DURING ME POWER ON

Purpose :		Measurement of interference voltages.

Procedure :	The power of the ME is switched on. To observe the interface ME/SIM during the action.

Requirement :	The voltages across the contacts shall correspond to inactive levels, i.e. between 0 and 0.4V.

II.8.7.1.3	PHASE DURING SIM POWER ON

Purpose :		Verification of contact activation sequence.

Procedure :	To observe the interface till the SIM is fully activated (See ISO 7816-3 section 5.1)

Requirements :	Contacts shall activated in the following order:

RST in State L,

-	VCC powered,

-	I/O in reception mode,

-	Vpp powered (see GSM 11.11 section 6.1.2.)

-	Clock signal provided with a suitable and stable clock.

II.8.7.1.4	SIM POWER OFF PHASE

Purpose :		Verification of contacts deactivation sequence.

Procedure :	To observe the interface till the SIM is fully deactivated (See ISO 7816-3 section 5.1)

Requirements :	Contacts are deactivated in the following order:

-	RST at low status,

-	Clock stopped at low status,

-	I/O at status A,

-	VPP cut off (if VPP was provided, See GSM 11.11 section 6.1.2.),

-	VCC cut off.

II.8.7.1.5	ME POWER OFF PHASE

Purpose :		Measurement of interference voltages.

Procedure :	The power of the ME is switched off. To observe the interface ME/SIM during the action.

Requirements :	The voltages on the contacts shall correspond to inactive levels, i.e. between 0 and 0.4V.

II.8.7.1.6	PHASE AFTER ME power off

Purpose :		Measurement of residual voltages.

Procedure :	The power of the ME is off; To observe the the SIM/ME interface.

Requirements :	The voltages on the contacts shall correspond to inactive levels, i.e. between 0 and 0.4V.

II.8.7.2	ELECTRICAL TESTS ON EACH ME CONTACT

NOTE:	Measurements of contacts voltage levels can be done at any time since the beginning of activation of the SIM and the end of deactivation of the SIM (ISO 7816-3 section 5.1).

The reference point of all measurements is the contact C5 (Ground).

II.8.7.2.1	Electrical tests on contact C1

C1 = Card power supply (Vcc)

II.8.7.2.1.1

Purpose :		To verify the nominal voltage.

Procedure :	Vcc is measured with the remaining contacts in nominal test conditions.

Requirement :	The voltage shall be 5V +/-10% for Icc up to 10 mA.

II.8.7.2.1.2

Purpose :		To verify that the voltage does not collapse when subjected to current spikes.

Procedure :	Vcc is monitored and the following current spikes are applied:

1) a single spike:

Max current 200 mA

Duration (including rise and fall times as specified in 11.40) 200 ns

2) a single spike:

Max current 100 mA

Duration (including rise and fall times as defined in 11.40) 400 ns.

3) continuous spikes:

Max current 20 mA

Min current  0 mA

Duration 100 ns

Pause 100 ns

4) continuous spikes:

Max current 20 mA

Min current  0 mA

Duration 400 ns

Pause 400 ns

5) continuous spikes:

Max current 20 mA

Min current  5 mA

Duration 150 ns

Pause 300 ns

6) Pseudorandom spikes:

Max current 20 mA

Min current  5 mA

Duration 200 ns

200 spikes/second

7) Pseudorandom spikes:

Max current 20 mA

Min current  5 mA

Duration 200 ns

1000 spikes/second

Requirement:	The voltage shall not fall below 4.5V.

II.8.7.2.2	Electrical tests on contact C2

C2 = Reset (RST)

Purpose :		To verify the voltage on contact C2

Procedure :	The voltage on C2 is measured with the remaining contacts in nominal test conditions.

II.8.7.2.2.1	Tests on contact C2 = low level RST

Requirement :	The voltage shall be between -0.3V and +0.6V for a current of 200 µA.

II.8.7.2.2.2	Tests on contact C2 = high level RST

Requirement :	Make sure that the voltage is between 4V and Vcc +0.3V for a current of 20 µA.

II.8.7.2.3	Electrical tests on contact C3

C3 = Clock (CLK)

Purpose :		To verify the voltage, the rise/fall time of the signal and the clock cycle ratio on contact C3

Procedure :	The voltage on C3 is measured with the remaining contacts in nominal test conditions.

II.8.7.2.3.1	Tests on contact C3 = CLK at low level

Requirement :	The voltage shall be between -0.3V and 0.6V for a current of 200 µA.

II.8.7.2.3.2	Tests on contact C3 = CLK for switch from low level to high level. Verification of rise time.

Requirement :	The rise time shall not exceed 9% of the total time period and shall not exceed 0.5 us.

II.8.7.2.3.3	Tests on contact C3 = CLK at high level

Requirement :	The voltage shall be between 2.4 V and Vcc +0.3V for a current of 200 µA.

II.8.7.2.3.4	Tests on contact C3 = CLK for switch from high level to low level. Verification of fall time.

Requirement :	The fall time shall not exceed 9% of the period and it shall not exceed 0.5 µs.

II.8.7.2.3.5	Clock cycle ratio test

Requirement :	The cycle ratio of the clock signal shall be between 40% and 60% of the period, in steady state.

II.8.7.2.4	Electrical tests on contact C6

Purpose :		To verify that the contact is in accordance with GSM 11.11.

Procedure :	The voltage on C6 is measured with the remaining contacts in nominal test conditions.

Requirement :

-	In the case of IC card SIM, the ME shall provide contact C6 and an idle state equal to the condition for Vcc. Contact C6 may be connected to Vcc in the ME.

-	In the case of the Plug-in SIM, contact C6 need not to be provided by the ME. If it is present in the ME, the programming voltage may not be provided. C6 shall not be connected to ground in the ME.

II.8.7.2.5	Electrical tests on contact C7

C7 = Input - output (I/O)

Purpose :		To verify the voltage levels on contact C7

Procedure :	The voltage on C7 is measured with the remaining contacts in nominal test conditions.

Requirements :

-	On the ME input:

The voltage shall be between 0 and 0.4V (low level) for I = 1 mA and between 3.8V and Vcc+0.3V  (high level) for I = -20 µA.

-	On the ME output:

The voltage shall be between 0 and 0.8V (low level) for I = 1 mA and between 0.7*Vcc and Vcc+0.3V (high level) for I = +/-20 µA.

II.8.8	Test Sequence 8:  ACCESS CONTROL

Purpose:

1)	To confirm that the ME reads the Access Control value as part of the SIM/ME initialisation procedure -

2)	To verify that the MS does not attempt a network access if its Access Control Class is invalid -

- in the following cases:

(a)	No SIM in ME - Emergency Calls not allowed by network.

(b)	No SIM in ME - Emergency Calls allowed by network.

(c)	MS with Access Class 0 to 9 - No Calls allowed by network.

(d)	MS with Access Class 0 to 9 - Emergency Calls only allowed

by network.

(e)	MS with Access Class 0 to 9 - All Calls allowed by network.

(f)	MS with Access Class 11 and 15 not in HPLMN,

SIM with Access Class 12, 13 and 14 not in HPLMN country.

(g)	MS with Access Class 11 and 15      )

in HPLMN,            )   - Emergency Calls only

MS with Access Class 12, 13 and 14  )     allowed by the network.

in HPLMN country.    )

(h)	MS with Access Class 11 and 15      )

in HPLMN,            )   - All calls allowed

MS with Access Class 12, 13 and 14  )     by the network.

in HPLMN country.    )

Method of Test:

a)	A SIM is installed in the ME containing IMSI and Access Control Class values as given in the following Table II.8-3.

b)	The SS is set up to transmit on the BCCH with a LAI and RACH control parameters as given in the following Table II.8-3.

c)	The MS is powered-on.

d)	Using the MMI or EMMI a normal call set-up is attempted.

e)	Using the MMI or EMMI an emergency call set-up is attempted.

f)	The test is repeated for each set of values in Table II.8-3.

Requirement:

The MS will access the network, or make no access attempt, to set-up the normal call and the emergency call as stated in the following Table II.8-3.  For the tests 8c, 8d and 8e it is only necessary that one of the access classes is tested.

TABLE  II.8-3

SIM        │     NETWORK          │ Test Result

│                      │

IMSI    Access  │ RACH      BCCH/LAI   │Normal  Emergency

Control │ octet 3     MCC      │Calls   Calls

Class   │ octet 4     MNC      │

│                      │

See end of table for coding details of      │

these parameters.                           │

│                      │

────────────────────────┼──────────────────────┼─────────────────────

TEST 8a  No SIM in ME   │   00000100   234    │ No     No

│   00000000    01    │

│                      │

────────────────────────┼──────────────────────┼─────────────────────

TEST 8b  No SIM in ME   │   00000000   234     │ No     Yes

│   00000000    01     │

│                      │

────────────────────────┼──────────────────────┼─────────────────────

TEST 8c                 │                      │

246813579   0     │   00000100    246   │ No     No

│   00000001    81     │

│                      │

"       1     │   00000100    "      │ No     No

│   00000010           │

│                      │

"       2     │   00000100    "      │ No     No

│   00000100           │

│                      │

"       3     │   00000100    "      │ No     No

│   00001000           │

│                      │

"       4     │   00000100    "      │ No     No

│   00010000           │

│                      │

"       5     │   00000100    "      │ No     No

│   00100000           │

│                      │

"       6     │   00000100    "      │ No     No

│   01000000           │

│                      │

"       7     │   00000100    "      │ No     No

│   10000000           │

│                      │

"       8     │   00000101    "      │ No     No

│   00000000           │

│                      │

"       9     │   00000110    "      │ No     No

│   00000000           │

────────────────────────┴──────────────────────┴─────────────────────

TABLE II.8-3 cont.

SIM        │     NETWORK          │ Test Result

│                      │

IMSI    Access  │ RACH      BCCH/LAI   │Normal  Emergency

Control │ octet 3     MCC      │Calls   Calls

Class   │ octet 4     MNC      │

────────────────────────┼──────────────────────┼─────────────────────

TEST 8d                 │                      │

246813579   0     │   00000000   246    │ No     Yes

│   00000001    81    │

│                      │

"       1     │   00000000    "     │ No     Yes

│   00000010           │

│                      │

"       2     │   00000000    "     │ No     Yes

│   00000100           │

│                      │

"       3     │   00000000    "     │ No     Yes

│   00001000           │

│                      │

"       4     │   00000000    "     │ No     Yes

│   00010000           │

│                      │

"       5     │   00000000    "      │ No     Yes

│   00100000           │

│                      │

"       6     │   00000000    "      │ No     Yes

│   01000000           │

│                      │

"       7     │   00000000    "     │ No     Yes

│   10000000           │

│                      │

"       8     │   00000001    "      │ No     Yes

│   00000000           │

│                      │

"       9     │   00000010    "      │ No     Yes

│   00000000           │

│                      │

────────────────────────┴──────────────────────┴─────────────────────

Note:	See end of table for coding details of the parameters in the table.

TABLE II.8-3 cont.

SIM        │     NETWORK          │ Test Result

│                      │

IMSI    Access  │ RACH      BCCH/LAI   │Normal  Emergency

Control │ octet 3     MCC      │Calls   Calls

Class   │ octet 4     MNC      │

────────────────────────┼──────────────────────┼─────────────────────

TEST 8e                 │                      │

246813579   0     │   11111011   246    │ Yes    Yes

│   11111110    81     │

│                      │

"       1     │   11111011    "     │ Yes    Yes

│   11111101           │

│                      │

"       2     │   11111011    "     │ Yes    Yes

│   11111011           │

│                      │

"       3     │   11111011    "      │ Yes    Yes

│   11110111           │

│                      │

"       4     │   11111011    "     │ Yes    Yes

│   11101111           │

│                      │

"       5     │   11111011    "      │ Yes    Yes

│   11011111           │

│                      │

"       6     │   11111011    "      │ Yes    Yes

│   10111111           │

│                      │

"       7     │   11111011    "     │ Yes    Yes

│   01111111           │

│                      │

"       8     │   11111010    "     │ Yes    Yes

│   11111111           │

│                      │

"       9     │   11111001    "     │ Yes    Yes

│   11111111           │

────────────────────────┴──────────────────────┴─────────────────────

Note:	See end of table for coding details of the parameters in the table.

TABLE II.8-3 cont.

SIM        │     NETWORK          │ Test Result

│                      │

IMSI    Access  │ RACH      BCCH/LAI   │Normal  Emergency

Control │ octet 3     MCC      │Calls   Calls

Class   │ octet 4     MNC      │

────────────────────────┼──────────────────────┼─────────────────────

TEST 8f                 │                      │

2468135x9   11 and x │   00000111   246    │ No     No

│   11111111    82     │

│                      │

"        "     │   00000011    "      │ No     Yes

│   11111111           │

│                      │

"        "     │   00000000           │ Yes    Yes

│   00000000           │

Set 'x' to a random │                      │

value between 0 & 9 │                      │

────────────────────────┼──────────────────────┼─────────────────────

│                      │

2468135x9   12 and x │   00000111   234    │ No     No

│   11111111    01    │

│                      │

"        "     │   00000011    "     │ No     Yes

│   11111111           │

│                      │

"        "     │   00000000           │ Yes    Yes

│   00000000           │

Set 'x' to a random │                      │

value between 0 & 9 │                      │

────────────────────────┼──────────────────────┼─────────────────────

│                      │

2468135x9   13 and x │   00000111   234     │ No     No

│   11111111    01    │

│                      │

"        "     │   00000011    "      │ No     Yes

│   11111111           │

│                      │

"        "     │   00000000           │ Yes    Yes

│   00000000           │

Set 'x' to a random │                      │

value between 0 & 9 │                      │

────────────────────────┼──────────────────────┼─────────────────────

│                      │

2468135x9   14 and x │   00000111   234    │ No     No

│   11111111    01    │

│                      │

"        "     │   00000011    "     │ No     Yes

│   11111111           │

│                      │

"        "     │   00000000           │ Yes    Yes

│   00000000           │

Set 'x' to a random │                      │

value between 0 & 9 │                      │

────────────────────────┼──────────────────────┼─────────────────────

│                      │

2468135x9   15 and x │   00000111   246    │ No     No

│   11111111    82     │

│                      │

"        "     │   00000011    "     │ No     Yes

│   11111111           │

│                      │

"        "     │   00000000           │ Yes    Yes

│   00000000           │

Set 'x' to a random │                      │

value between 0 & 9 │                      │

────────────────────────┴──────────────────────┴─────────────────────

TABLE II.8-3 cont.

SIM        │     NETWORK          │ Test Result

│                      │

IMSI    Access  │ RACH      BCCH/LAI   │Normal  Emergency

Control │ octet 3     MCC      │Calls   Calls

Class   │ octet 4     MNC      │

────────────────────────┼──────────────────────┼─────────────────────

TEST 8g                 │                      │

246813579   11 and x │   00001111   246    │ No     Yes

│   11111111    81     │

│                      │

"       "     │   00001011    "      │ No     Yes

│   11111111           │

│                      │

246813579   12 and x │   00010111   246    │ No     Yes

│   11111111    82    │

│                      │

"       "     │   00010011    "      │ No     Yes

│   11111111           │

│                      │

246813579   13 and x │   00100111   246    │ No     Yes

│   11111111    82    │

│                      │

"       "     │   00100011    "      │ No     Yes

│   11111111           │

│                      │

246813579   14 and x │   01000111   246     │ No     Yes

│   11111111    82    │

│                      │

"       "     │   01000011    "      │ No     Yes

│   11111111           │

│                      │

246813579   15 and x │   10000111   246    │ No     Yes

│   11111111    81    │

│                      │

"       "     │   10000011    "     │ No     Yes

│   11111111           │

Set 'x' to a random │                      │

value between 0 & 9 │                      │

────────────────────────┼──────────────────────┼─────────────────────

TEST 8h                 │                      │

246813579   11 and x │   11110011   246    │ Yes    Yes

│   11111111    81    │

│                      │

246813579   12 and x │   11101011   246    │ Yes    Yes

│   11111111    82    │

│                      │

246813579   13 and x │   11011011   246    │ Yes    Yes

│   11111111    82    │

│                      │

246813579   14 and x │   10111011   246    │ Yes    Yes

│   11111111    82    │

│                      │

246813579   15 and x │   01111011   246     │ Yes    Yes

│   11111111    81    │

Set 'x' to a random │                      │

value between 0 & 9 │                      │

────────────────────────┴──────────────────────┴─────────────────────

Note:	See following page for coding details of the parameters in the table.

CODING DETAILS

SIM   IMSI: Data Field 6F 07

Value 246813579        Value 2468135x9

byte 1      05H                    05H

byte 2      29H                    29H

byte 3      64H                    64H

byte 4      18H                    18H

byte 5      53H                    53H

byte 6      97H                    9xH

byte 7      FFH                    FFH

byte 8      FFH                    FFH

byte 9      FFH                    FFH

Access Class: Data Field 6F 78

See GSM 11.11.

NETWORK (SS)

RACH: As defined in GSM 04.08 section 10.5.2.17.

octet 1   01111000

octet 2   00001000

octet 3   }

octet 4   } as above

II.8.9	Test Sequence 9:  EXCHANGE PROTOCOL TESTS

Purpose of the tests:

Testing of exchange protocol of the SIM/ME interface.

This sequence lists protocol tests used.

The list of SIM exchange protocol tests is divided into three subsections:

The first subsection deals with character transmission, which constitutes the basis for ME-SIM exchanges.

The following two subsections correspond to the two main parts of a transaction with a microprocessor card: the reset and the card answer then command processing. Both subsections describe the protocol tests to be run during each corresponding phase of the transaction.

The characteristics to be tested are :

II.8.9.1	CHARACTER TRANSMISSION

II.8.9.1.1	ETU variation

II.8.9.1.2	I/O line at zero before character transmission

II.8.9.1.3	Inter-character delay

II.8.9.1.4	Error handling

II.8.9.2	RESET AND ANSWER to reset

II.8.9.2.1	Sorts of RST

II.8.9.2.2	Characters of the Answer to Reset

II.8.9.2.3	PTS procedure

II.8.9.2.4	Reset repetition

II.8.9.3	COMMAND PROCESSING

II.8.9.3.1	Procedure bytes ACK

II.8.9.1	CHARACTER TRANSMISSION

Purpose:	To verify the character frame during answer the transmission

(See ISO 7816-3 section 6.1.2). For this test, use a SIM

Simulator.

II.8.9.1.1	Bit/Character duration during the transmission from the ME to the SIM

Procedure :

A character transmitted from the transmitter to the receiver comprises ten elements plus the guard time.

The ten elements comprise:

-	the start bit,

-	eight data bits,

-	the parity bit.

The SIM Simulator shall measure the bit/character duration of the request sent from the ME.

Requirement :

The bit/character duration of the request sent from the ME shall be in the range specified in GSM 11.11, section 5.4.

II.8.9.1.2	Bit/Character duration during the transmission from the SIM Simulator to the ME

Procedure :

The SIM Simulator shall send responses with the maximum and minimum bit/character durations specified in GSM 11.11 section 5.4.

Requirement :

The ME shall evaluate the response.

II.8.9.1.3	Inter-character delay

Procedure :

a)	The requested inter-character delays from the ME to the SIM (or the SIM Simulator) are modified by changing the value of N in TC1 :

a.1)	N = 0;

a.2)	N = 255;

a.3)	N = Other values.

b)	Verification is performed in the following two ways:

-	 observation using the oscilloscope or logical analyzer,

-	 non-acceptance by the SIM (or the SIM Simulator) of the byte

transmitted too soon.

c)	The inter-character delay from the SIM (or the SIM Simulator) to the ME is altered until a value is obtained at which the ME considers the SIM (or the SIM Simulator) to be mute.

Requirement :

In case a.1) the ME shall work with the SIM.

In case a.2) the ME shall work with the SIM.

In case a.3) the ME shall reject the SIM.

II.8.9.1.4	Error Handling

II.8.9.1.4.1	Error Handling during the transmission from the ME to the SIM Simulator

Procedure:

The SIM Simulator shall transmit an error signal in response to a received character in accordance with ISO 7816-3, section 6.1.3.

Requirement:

The ME shall repeat the character in accordance with ISO 7816-3, section 6.1.3.

II.8.9.1.4.2	Error Handling during transmission from the SIM Simulator to the ME

Procedure:

The SIM Simulator shall send a response with a parity error and check that the ME performs error handling in accordance with ISO 7816-3, section 6.1.3.

Requirement:

The ME shall send an error signal in accordance with ISO 7816-3, section 6.1.3, and expect a repetition of the character.

II.8.9.2	ANSWER TO RESET (RST)

Purpose

To verify if the ME accepts the internal and the active low RESET and detect a mute SIM (See ISO 7816-3 section 5.2).

II.8.9.2.1	Sorts of RST

Three sorts of RST are to be simulated:

-	internal RST,

-	active low RST,

-	mute card.

The SIM (or the SIM Simulator) must be synchronized with the successive detection of Vcc and the clock signal in order to begin the reset routine.

II.8.9.2.1.1	Internal RST

Procedure :

The SIM (or the SIM Simulator) verifies that RST is at low status and starts its answer between (400/fi) s and (40000/fi) s after the clock signal has been detected fi is the initial frequency supplied by the ME.

Requirement :

The ME accepts the SIM with internal reset.

II.8.9.2.1.2	Active low RST

Procedure :

The SIM Simulator sends no internal reset and checks that the RST contact is put to state H after a minimum of (40,000/fi)s. The SIM Simulator verifies that the RST contact stays at this level for at least a further (40000/fi) s.

Requirement :

The ME shall accept the SIM with internal reset.

II.8.9.2.1.3	Mute card

Procedure :

a)	There is no answer before (80000/fi) s.

The SIM (or the SIM Simulator) is considered mute and the RST signal returns to low level.

b)	Time measurements are made on the RST signal, using an oscilloscope or a logical analyzer.

Requirement :

The ME shall recognize that the SIM is mute.

II.8.9.2.2	Characters of the Answer to Reset

Purpose :

This paragraph includes tests run on the TS, T0 and all the other bytes in accordance with standard 7816-3. The answer to reset consists of at most 33 characters. The ME shall be able to receive interface characters for other transmission protocols than T=0, historical characters and a check byte, even if only T=0 is used by the ME.

Procedure and Requirements :

The SIM (or the SIM Simulator) sends an Answer to reset with a set of following Characters :

┌────────────────┬───────────┬──────────────────┬─────────────────────────┐

│ Character      │ Character │ sent by SIM      │ Requirement             │

│ Type           │           │                  │                         │

├────────────────┼───────────┼──────────────────┼─────────────────────────┤

│ 1. Initial     │ TS        │ always           │ using appropriate       │

│    character   │           │                  │ convention              │

├────────────────┼───────────┼──────────────────┼─────────────────────────┤

│ 2. Format      │ T0        │ always           │ identifying the         │

│    character   │           │                  │ subsequent character    │

│                │           │                  │ accordingly             │

├────────────────┼───────────┼──────────────────┼─────────────────────────┤

│ 3. Interface   │ TA1       │ optional         │ if TA1 is not '00010001'│

│    character   │           │                  │ use PTS procedure       │

├────────────────┼───────────┼──────────────────┼─────────────────────────┤

│ 4. Interface   │ TB1       │ optional         │ if PI1 is not 0 or not  │

│    character   │           │                  │ 5 then reject SIM       │

├────────────────┼───────────┼──────────────────┼─────────────────────────┤

│ 5. Interface   │ TC1       │ optional         │ if TC1 is not 0 or not  │

│    character   │           │                  │ 255 then reject the SIM │

├────────────────┼───────────┼──────────────────┼─────────────────────────┤

│ 6. Interface   │ TD1       │ optional         │ identifying the         │

│    character   │           │                  │ subsequent characters   │

│                │           │                  │ accordingly             │

├────────────────┼───────────┼──────────────────┼─────────────────────────┤

│ 7. Interface   │ TA2       │ optional         │ no reaction necessary   │

│    character   │           │                  │                         │

├────────────────┼───────────┼──────────────────┼─────────────────────────┤

│ 8. Interface   │ TB2       │ optional         │ if PI2 is not 50        │

│    character   │           │                  │ then reject the SIM     │

├────────────────┼───────────┼──────────────────┼─────────────────────────┤

│ 9. Interface   │ TC2       │ optional         │ using the work waiting  │

│    character   │           │                  │ time accordingly        │

├────────────────┼───────────┼──────────────────┼─────────────────────────┤

│ 10. Interface  │ TDi (i>1) │ optional         │ identifying the         │

│     character  │           │                  │ subsequent characters    │

│                │           │                  │ accordingly             │

├────────────────┼───────────┼──────────────────┼─────────────────────────┤

│ 11. Interface  │ TAi, TBi  │ optional         │ no reaction necessary   │

│     character  │ TCi (i>2) │                  │                         │

├────────────────┼───────────┼──────────────────┼─────────────────────────┤

│ 12. Historical │ T1 ... TK │ optional         │ no reaction             │

│     character  │           │                  │ necessary               │

├────────────────┼───────────┼──────────────────┼─────────────────────────┤

│ 13. Check      │ TCK       │ not sent if only │ no reaction necessary   │

│     character  │           │ T=0 is indicated │                         │

│                │           │ in the ATR;  in  │                         │

│                │           │ other cases TCK  │                         │

│                │           │ shall be sent    │                         │

└────────────────┴───────────┴──────────────────┴─────────────────────────┘

II.8.9.2.3	PTS Procedure

Purpose :		To verify that ME uses PTS procedure as specified in GSM 11.11 section 5.2.

Procedure :	The SIM (or the SIM Simulator) gives an answer to reset with TA1 different of '11H'.

Requirement :	Te ME shall send to the SIM (or the SIM Simulator) 'FF00FF'.

II.8.9.2.4	Reset repetition

Purpose :		To verify that the ME repeats the Reset between

2 and 5 times after receiving a wrong answer to reset.

Procedure :	The SIM Simulator sends a non understandable answer to reset to the ME. (e.g. a wrong TS byte).

Requirement :	The ME shall repeat the Reset between 2 and 5 times after receiving a wrong Answer to reset.

Note: Between 3 up to 6 reset are done before

rejecting the SIM.

II.8.9.3	COMMAND PROCESSING

II.8.9.3.1	Procedure bytes ACK :

Purpose :		To verify that the ME uses correctly the different modes of data transmission defined in ISO 7816-3 section 8.2.2.1.

Procedure :	Store data in the SIM (or the Sim simulator). The SIM (or the SIM Simulator) first answers ACK=INS or (INS+1) complemented and then changes ACK to INS or INS+1 during the transmission.

Requirement :	The command shall be executed correctly.

II.8.10	Test Sequence 10: EVALUATION OF DIRECTORY CHARACTERISTICS

II.8.10.1	Test Sequence 10.1: OPERATING SPEED IN AUTHENTICATION Procedure

Purpose :		To verify that the authentication procedure is done with a frequency of at least 13/4 MHz if the bit b2 of the directory characteristics is set to 1 (see GSM 11.11 section 6.2.1.).

Procedure :	An authentication is made with a SIM (or a SIM Simulator) with bit b2 of the directory characteristics set to 1.

Requirement :	The frequency of the clock shall be at least

13/4 MHz during the authentication procedure.

II.8.10.2:	Test Sequence 10.2: Clock stop

Purpose :		To verify that the clock is only switched off if bit b1 of the directory characteristics is set

to 1.

Procedure :	A SIM (or a SIM Simulator) with b1 set to 0 is used.

Requirement :	The ME shall not switched off the clock.

II.8.11	Test Sequence 11: MECHANICAL TESTS

II.8.11.1	Test Sequence 11.1 : Contact pressure

General		The ME manufacturers shall provide a separate card reader (Mechanical components) to make the measurement possible.

Purpose :		To verify that the contact pressure of each contacting element is not greater than 0.5 N (see GSM 11.11 section 6.1.2.2.) when each of the following types of card is used:

i)   Unembossed

ii)  Embossed on the contact side

iii) Embossed on the opposite side to the contacts.

NOTE:	 Only type i) applies to the plug-in SIM.

Procedure :	The pressure of each contacting element is measured.

Requirement :	The contact pressure of each contacting element shall be not greater than 0.5 N.

II.8.11.2	Test Sequence 11.2: Shape of contacts for IC card SIM card reader

General		The ME manufacturers shall provide a separate card reader (Mechanical components) to make the measurement possible.

Purpose :		To verify that the radius of curvature of the contacting elements is greater than or equal to

0.8 mm in the contact area on both axes (see GSM 11.11 section 6.1.2.3.).

Procedure :	The radius of curvature of the contacting elements is measured on both axes.

Requirement :	The radius of curvature of the contacting elements shall be greater than or equal to 0.8 mm in the contact area on both axes.

II.8.12	Test Sequence 12: MMI reaction to SIM status encoding

Purpose of the test

To verify the MMI reaction on the MS when error codes occur from the SIM on the SIM/ME interface.

Procedure

a)	The SIM simulator is used to send the following error codes as reaction on an instruction from the ME:

-  9240  update impossible

-  9804  Access security policy not fulfilled or secret code rejected

-  9840  Secret code locked

-  6FXX  Technical problem with no diagnostic given as reaction on an

instruction from the ME.

Requirement

The ME shall give an MMI indication.

II.8.13	Test Sequence 13: Secret code usage

Purpose of the test

To verify the usage of the secret code on the MMI, as specified in GSM 02.30.

1)	To confirm that it is possible to change the PIN.

2)	To confirm that a PIN with 4 and 8 digits can be handled.

3)	To check whether it is possible to unblock the SIM.

4)	To check whether it is possible to store and recall an abbreviated number.

Procedure:

a)	The SIM defined in Tables II.8-1 and II.8-2 above is installed into the ME, and the MS powered-on.

b)	When the MS is in Mode "PIN check" enter "2468#".

Requirements:

1)	The MS shall give an indication "OK".

Procedure:

c)	Enter "**04*2468*01234567*01234567#".

Requirements:

2)	The MS shall give an indication that the new PIN is accepted.

Procedure:

d)	The MS is switched off and on.

e)	When the MS is in mode "PIN-check", the sequence "01234567#" is entered.

Requirement

3)	The MS shall give an indication "OK".

Procedure

f)	Switch the MS off and on.

g)	When the MS is in Mode "PIN check" enter "2468#".

Requirements:

4)	The MS shall give an indication that the entered PIN is wrong.

Procedure:

h)	Repeat d) to e) two times with wrong PIN numbers.

Requirements:

5)	In step f), requirement 3 shall be met each time.

6)	At the 2nd repetition the MS shall show that the SIM is blocked.

Procedure:

i)	Enter "**05*13243546*2468*2468#".

Requirements:

7)	The MS shall indicate that the unblocking was successful.

Procedure:

j)	Switch the MS off and on.

k)	When the MS is in Mode "PIN check" enter "2468#".

Requirements:

8)	The MS shall give an indication that the entered PIN is ok.

Procedure:

l)	The code "+123456789012345" is stored (entered) in the MS as abbreviated dialling number 7.

m)	The code "00112233" is stored (entered) in the MS as abbreviated dialling number 6.

n)	Enter "7#".

Requirements:

9)	The number "+123456789012345" shall be displayed.

Procedure:

o)	Enter "6#".

Requirements:

10)	The number "00112233" shall be displayed.