II.5.3.5	TEST OF L3 ERROR HANDLING

ref: GSM 04.08

II.5.3.5.1	ERRORS IN L3 MESSAGES

II.5.3.5.1.1	Wrong protocol discriminator

Purpose

To verify that the MS ignores messages containing protocol discrimination errors.

Test method

a)	The SS simulates a base station with BCCH/CCCH.

b)	The MS is paged and allocated a TCH.

c)	With the MS having a call in progress, the SS sends a Status Enquiry message with an error in the protocol discriminator.

Message:     STATUS ENQUIRY

Information Element	Comment	Value

Protocol discriminator	The value is reserved		0000

Transaction identifier	Selected by SS		0XXX

Message Type		0011	0100

Note:	If a MM message is sent with an error giving it a CC protocol discriminator, this could be treated by the MS as being a CC message with errors in the Message Type information Element. To make the test unambiguous, the test uses a protocol discriminator that is reserved.

Requirements:

1)	The MS shall ignore the message.

II.5.3.5.2	Test of MS reaction to errors in Transaction Identifier

ref: GSM 04.08 section 8.4

II.5.3.5.2.1	Test of TI error in RR management message

Purpose of the test

To verify that the MS ignores RR messages with incorrect TI.

Initial conditions

The SS simulates a base station with BCCH/CCCH.  The MS is in the MM-state 'idle,updated' and shall have successfully passed the test of section II.5.3.6.2.1 ('normal paging') thus ensuring that errors are the result of erroneous TI.

Procedure

Comment GSM3: should be tested with RR message, using a TI that is used for CC for the active call

a)	The MS is consecutively paged with PAGING REQUEST TYPE 1, using TI values in the range 0001 - 0110 (binary) and also with TI set to 1000 i.e. non-zero value.

ref: GSM 04.08 section 10.3 Transaction Identifier

Message:     Paging Request Type 1

Information element	comment	value

Protocol Discriminator	RR		0110

transaction Identifier	invalid	0001

-	0110

and also		1000

message type	type 1	0010	0001

page mode	normal	0000	0000

mobile identity	TMSI

Requirement

1)	The MS shall not respond to any paging request with TI set to non-zero (coding includes TI flag).

II.5.3.5.2.2	Test of TI error in MM message

Purpose of the test

To verify that the MS ignores MM messages with incorrect TI.

Initial conditions

The MS is in the MM-active state of a mobile terminating call (TI assigned by the SS).

Procedure

a)	The SS sends the IDENTITY REQUEST message using TI values 0001 - 0110 inclusive and also with TI set to 1000. A period of 1 second (nominal) shall separate the transmission of messages in order to allow time for a response from the MS.

Requirement

1)	The MS shall ignore each IDENTITY REQUEST message

2)	No response shall be returned to the SS up to and including the duration of T3270 after the last IDENTITY REQUEST is transmitted.

Message:     IDENTITY REQUEST  (GSM 04.08, 9.2.8)

Information element	comment	value

Protocol Discriminator	MM		0101

Transaction Identifier	invalid 	0001 - 0110

and also		1000

message type		0001	1000

identity type	IMSI	0000	0001

II.5.3.5.2.3	Test of TI error in CC message

Purpose of the test

To verify that the MS responds correctly to messages which include Transaction Identifier which is not recognised as relating to active call or call in progress.

Initial conditions

The MS is in the active state of a Mobile-Terminating call (SS has allocated the TI).

Procedure (1)

a)	The SS sends a DISCONNECT message to the MS with a TI value which has not been allocated.

b)	The SS sends a STATUS ENQUIRY message to the MS with correct TI.

Requirement (1)

1)	The MS shall respond to the DISCONNECT message with a RELEASE COMPLETE message including cause #81 and the received value of TI.

2)	The MS shall respond to the STATUS ENQUIRY message by sending a STATUS message with CC-state U10 (active) i.e. no state change.

Procedure (2)

a)	With the MS in the active state (U10),the SS sends a RELEASE COMPLETE message to the MS with a TI value which has not been allocated (this shall be the first call clearing message ).

b)	The SS sends a STATUS ENQUIRY message to the MS with correct TI.

Requirement (2)

1)	The MS shall not respond to the RELEASE COMPLETE message.

2)	The MS shall send a STATUS message with CC-state U10 (active) and with TI relating to the active call.

Procedure (3)

a)	The SS sends a SETUP message to the MS with a TI value with coding related to a mobile-originated call i.e. TI-flag indicates wrong end of TI origin.

b)	The SS waits [5 seconds] and then sends a STATUS ENQUIRY message to the MS with correct TI.

Note:	The value of 5 seconds is set to be sure that the MS would have answered if the SETUP were taken into account.

Requirement (3)

1)	The MS shall ignore the SETUP message.

2)	The MS respond to the STATUS ENQUIRY by sending a STATUS message with CC-state U10 (active) and with TI relating to the initial call.

Procedure (4)

a)	The SS sends a SETUP message to the MS with a TI value identical to that of the active call (as allocated by SS).

b)	The SS sends a STATUS ENQUIRY message to the MS with correct TI.

Requirement (4)

1)	The MS shall ignore the SETUP message .

2)	The MS respond to the STATUS ENQUIRY by sending a STATUS message with CC-state U10 (active) and with TI relating to the active call.

Message:     DISCONNECT from SS    (GSM 04.08, 9.3.7)

Information element	comment	value

Protocol Discriminator	CC		0011

Transaction Identifier	correct TI flag	0xxx

(SS originates the TI)	unallocated TI value

message type	disconnect	0010	0101

cause(mandatory)	length	0000	0010

coding/loc.	1110	0000

(#16)normal	1001	0000

Message:     RELEASE COMPLETE from MS (GSM 04.08, 9.3.15)

(used as first call clearing message)

Information element	comment	value

Protocol Discriminator	CC		0011

Transaction Identifier	the same as sent	1xxx

message type 	rel com	0x10 	1010

cause(optional)	IEI	0000	1000

length	0000	0010

coding/loc.	1110	0000

#81(invalid TI)	1101	0001

Message:     RELEASE COMPLETE from SS (GSM 04.08, 9.3.15)

Information element	comment	value

Protocol Discriminator	CC		0011

Transaction Identifier	correct TI flag	0xxx

(SS originates the TI)	unallocated TI value

Message Type	Release complete	0010	1010

Cause (optional)	IEI	0000	1000

length	0000	0010

coding/loc	1110	0000

#16 normal	1001	0000

Message:      SETUP from SS (GSM 04.08 9.3.16)

Information element	Comment	Value

Protocol Discriminator	CC		0011

Transaction Identifier	3)wrong TI flag	1xxx

correct TI

4)correct TI flag and TI	0xxx

Message Type	SETUP	0000	0101

Message:     STATUS ENQUIRY from SS (GSM 04.08, 9.3.21)

Information element	comment	value

Protocol Discriminator	CC		0011

Transaction Identifier	SS allocated		0xxx

message type	status enq	0011 	0100

Message:     STATUS from MS (GSM 04.08, 9.3.20)

Information element	comment	value

Protocol Discriminator	CC		0011

Transaction Identifier	as allocated		1xxx

message type	status 	0x11 	1101

cause	length is varying

coding/loc.	1110	0000

(#30)response to status 	1001	1110

enquiry

call state	coding(U10)	1100	1010

II.5.3.5.3	Test of message type errors

ref: GSM 04.08 section 8.5

II.5.3.5.3.1	Test of MS reaction to non-existent message type

Purpose of the test

To verify that the MS reacts correctly to a message which is non-existent in the GSM library.

Initial conditions

The MS is in the MM state 'idle,updated' listening to the BCCH/CCCH.

Procedure

a)	The SS pages the MS and a normal call is established (CC state = U10).

b)	The SS sends an ERRONEOUS message to the MS, with TI applying to the active call and PD referring to Call Control.

c)	The SS sends the STATUS ENQUIRY message (correctly coded) to the MS.

Requirement

1)	In response to the ERRONEOUS message, the MS shall return the STATUS message to the SS with cause #97 'message type non-existent or not implemented' or #98.

2)	In response to the STATUS ENQUIRY, the MS shall return the STATUS message with cause #30 'response to status enquiry' and call state U10.

II.5.3.5.3.2	Test of MS reaction to message inconsistent with PD

Purpose of the test

To verify that the MS reacts correctly to a message which is inconsistent with the Protocol Discriminator.

Initial conditions

The MS is in the active (CC state = U10) state of a MT call with the CC TI having the value 0..

Procedure

a)	The SS sends the DISCONNECT message to the MS , correctly coded and using the active call TI but with PD indicating Mobility Management (MM).

b)	The SS sends the STATUS ENQUIRY message (correctly coded) to the MS.

Requirements

1)	In response to the DISCONNECT message, the MS shall return the MM -STATUS message to the SS with cause #97 or #98.

2)	In response to the STATUS ENQUIRY, the MS shall return the STATUS message with cause #30 'response to status enquiry' and call state U10.

II.5.3.5.3.3	Test of MS reaction to message inconsistent with call state

Purpose of the test

To verify that the MS reacts correctly to a message which is inconsistent with the current state.For the purposes of this test, the Call Control entity is chosen since specific states are defined by GSM.

Initial conditions

The MS is in the active state (U10) of a call.

Procedure

a)	The SS sends the CALL PROCEEDING message to the MS , correctly coded and referring to the active transaction.

b)	The SS sends the STATUS ENQUIRY message to the MS.

Requirements

1)	The MS shall respond to the CALL PROCEEDING message with a STATUS message including the call control state U10 and with cause value #98 'message incompatible with call state' or #97.

2)	In response to the STATUS ENQUIRY message, the MS shall return a STATUS message to the SS with cause #30 'response to status enquiry' and with call state U10.

Message:     DISCONNECT with invalid PD (GSM 04.08, 9.3.7)

Information element	comment	  value

Protocol Discriminator	MM		0101

Transaction Identifier	SS allocated	0xxx

message type	disconnect	0010 	0101

cause	length	0000	0010

coding/loc.	1110	0000

#16(normal)	1001	0000

Message:     Call Proceeding (GSM 04.08, 9.3.3)

Information element	comment	value

Protocol Discriminator	CC		0011

Transaction Identifier	SS allocated	0xxx

message type	call proc	0000 	0010

Message:      ERRONEOUS

Information element	comment	  value

Protocol Discriminator	CC		0011

Transaction Identifier	SS allocated	0xxx

message type	non existent	1111	1111

Note:	In messages from the network, bit 7 is always coded 0 in the message type: refer GSM 04.08 Table 10.5.

Message:     STATUS ENQUIRY	from SS (GSM 04.08, 9.3.21)

Information element	comment	value

Protocol Discriminator	CC		0011

Transaction Identifier	SS allocated	0xxx

message type	status enq	0x11 	0100

Message:STATUS from MS	(GSM 04.08, 9.3.20)

Information element	comment	value

Protocol Discriminator	CC		0011

Transaction Identifier	as allocated	1xxx

message type	status 	0x11 	1101

cause	length is varying

coding/loc.	1110	0000

1) #97	1110	0001

#98	1110	0010

2) #30	1001	1110

call state	coding(U10)	1100	1010

Message:     MM-STATUS from MS (GSM 04.08, 9.2.13)

Information element	Comment	Value

Protocol Discriminator	MM		0101

Transaction Identifier		0000

Message Type		0x11	0001

Reject Cause	#98	0110	0010

#97	0110	0001

II.5.3.5.4	Test of general information element errors

ref: GSM 04.08 section 8.6

It is considered that message out-of-sequence errors may occur only as the result of a faulty network and therefore no tests are specified in this section.

Note:	In addition, there are limitations as to what could be tested and these are detailed below.

II.5.3.5.4.1	Test of MS response to information elements out of sequence

The prescribed sequence for information elements is defined in GSM 04.08 sections 9 and 10.5.

a)	Mandatory information elements

For the case of Mandatory Information elements, the information element identifier is not included in the message and therefore no sequence information is available. No test is specified in this section.

b)	Optional information elements

It is not possible for an MS to differentiate between an optional information element occurring early and one occurring late. The MS may therefore report the incorrect information element in a STATUS message to the network. The prescribed STATUS response of section 8.8 does not make provision for sequence errors. Clarification is required since the MS software implementation is being forced by this section i.e. an MS may detect an out-of-sequence information element but still fully understand it.

In conclusion, as it is not possible to design a comprehensive test for out of sequence information elements, no test is specified in this section.

II.5.3.5.4.2	Test of MS response to duplicated information elements

Purpose of the test

To verify that the MS ignores duplicated optional information elements which are not permitted.

Initial conditions

a)	The SS simulates two cells, A and B , belonging to different location areas (default values as specified in II.5.3.1).

b)	The MS is in the MM-state 'idle,updated' listening to the BCCH/CCCH of cell A (TMSI assigned).

c)	The test of 'normal location updating' (section II.5.3.6.5.1) shall have been previously passed thus ensuring that TMSI is deleted if location update accept contains IMSI.

Procedure

a)	The Rf level of cell A is lowered until the MS selects cell B (according to the cell-selection procedures of GSM 05.08).

The MS shall establish an RR connection and perform the 'normal' location updating procedure (using TMSI).

b)	The SS responds to the location update request with the LOCATION UPDATE ACCEPT message containing IMSI and also TMSI (i.e. duplication of information element).

c)	The SS shall page the MS using the PAGING REQUEST TYPE 1 message including the TMSI which was previously used in the LOCATION UPDATE ACCEPT message.

d)	The SS shall wait T3113 (see GSM 04.08 sect. 11.1.2) seconds for the response from the MS.

e)	The SS shall then page the MS using IMSI.

Requirement

1)	The MS shall not respond to the paging request using TMSI but shall respond on cell B to the paging request using IMSI by initiating the immediate assignment procedure using the CHANNEL REQUEST message.

Message:     Location update accept (GSM 04.08, 9.2.11)

Information element	comment	  value

Protocol Discriminator	MM		0101

Transaction Identifier	not relevant	0000

message type	loc upd acc.	0000	0010

location area id	cell B(5 bytes)

mobile identity 	IEI	0000	0111

length	0000	xxxx

ID type	xxxx	x001

IMSI

mobile identity(duplication)	IEI	0000	0111

length	0000	xxxx

ID type	xxxx	x100

TMSI(as allocated)

(where length is dependent on the choice of IMSI/TMSI).

Message:     Paging request type 1 (GSM 04.08, 9.1.21)

Information element	comment	  value

Protocol Discriminator	RR		0110

Transaction Identifier	not relevant	0000

message type	type 1	0010 	0001

page mode	normal	1011 	0000

mobile identity      	(i) TMSI		xxxx

(ii) IMSI

Message:     channel Request (GSM 04.08, 9.1.8)

establishment cause	answer to paging	100. 	....

random reference	(bits 5-1)	...x	xxxx

II.5.3.5.5	Test of MS reaction to Mandatory Information Element errors

II.5.3.5.5.1	Test of MS reaction to Mandatory Information Element errors in RR message

ref: GSM 04.08 section 8.7.1

Purpose of the test

To verify that the MS reacts correctly to RR messages containing error in mandatory information elements.

Initial conditions

The MS shall be in the MM state 'idle,updated' and listening to the BCCH/CCCH.

Procedure (i)

a)	The SS shall page the MS and establish an RR connection with the IMMEDIATE ASSIGNMENT message.

b)	It shall wait for the PAGING RESPONSE from the MS and then send the CHANNEL RELEASE message to the MS with missing mandatory information element RR-CAUSE .

Requirement (i)

1)	The MS shall respond with the PAGING RESPONSE message following the IMMEDIATE ASSIGN message.

2)	The MS shall respond to the CHANNEL RELEASE message by releasing the RR connection and returning to the MM state 'idle,updated'.

Message	CHANNEL RELEASE (GSM 04.08, 9.1.7)

Information element	Comment	Value

Protocol discriminator	RR		0110

Transaction Identifier	not relevant	0000

Message Type		0000	1101

RR cause	missing

Procedure (ii)

a)	Same as in Procedure (i)

b)	The SS shall wait for the PAGING RESPONSE from the MS and then send a CIPHERING MODE COMMAND message without information field "Cipher Mode Setting".

Requirement (ii)

1)	The MS shall respond with the PAGING RESPONSE message following the IMMEDIATE ASSIGNMENT message.

2)	The MS shall send an RR-STATUS message with cause value 'Invalid information element contents'.

Message	CIPHERING MODE COMMAND (GSM 04.08, 9.1.9)

Information element	Comment	Value

Protocol discriminator	RR management		0110

Transaction Identifier	not relevant	0000

Message Type		0011	0101

Cipher Mode Setting	missing

II.5.3.5.5.2	Test of MS reaction to Mandatory Information Elements errors in MM message

ref: GSM 04.08 section 8.7.2

Purpose of the test

To verify that the MS takes appropriate action upon receipt of an MM message containing error in a mandatory information element.

Initial conditions

a)	The SS simulates a BS with BCCH/CCCH.

b)	The MS shall be brought in the active state of a mobile-terminating call ( Call state = U10), as specified in the test of call control state.

Procedure

a)	The SS sends an IDENTITY REQUEST message to the MS with incorrectly coded information element (mandatory).

b)	The SS shall send a STATUS ENQUIRY message to the MS.

Requirement

1)	The MS shall respond to the IDENTITY REQUEST with a MM-STATUS message including cause value #100 ('invalid information element contents').

2)	The MS shall respond to the STATUS ENQUIRY message with a STATUS message including CC state = U10 (active) and cause value #30 ('response to status enquiry').

Message:	IDENTITY REQUEST (GSM 04.08, 9.2.8)

information element	comment	  value

Protocol Discriminator	MM		0101

Transaction Identifier	not relevant	0000

message type	identity req.	0001	1000

erroneous message content	invalid	0000	1111

Message	MM-STATUS (GSM 04.08 9.2.15a)

Information Element	Comment	Value

Protocol discriminator	MM		0101

Transaction identifier		0000

Message type	MM-STATUS	0x11	0001

Reject cause	"invalid i.e. content"	0110	0100

Message	STATUS ENQUIRY (GSM 04.08 9.3.21)

Information element	Comment	Value

Protocol Discriminator	CC		0011

Transaction Identifier	SS allocated

Message Type		0011	0100

Message	STATUS (GSM 04.08 9.3.20)

Information element	Comment	Value

Protocol discriminator	CC		0011

Transaction Identifier

Message Type		0011	1101

Cause

30

Call State	U10	1100	1010

II.5.3.5.5.3	Test of MS reaction to missing Cause IE in the first call clearing message and to Mandatory Information Element errors in CC message

ref: GSM 04.08 section 8.7.3

Purpose of the test

To verify that the MS responds correctly to CC messages which include mandatory information element errors.

Initial conditions

The MS is in the MM-state 'idle,updated' and listening to the BCCH/CCCH in all cases.

Procedure (i)

a)	The MS is paged and brought to the CC-active state (U10) of a call.

b)	The SS sends the RELEASE message to the MS (as the first call-clearing message) with no 'cause' information element.

Requirement (i)

1)	The MS shall respond to the erroneous RELEASE message by returning a RELEASE COMPLETE message.

Message:     RELEASE from SS (GSM 04.08, 9.3.14)

information element	comment	  value

Protocol Discriminator	CC		0011

Transaction Identifier	SS allocated	0xxx

message type	release	0010 	1101

cause	missing	----	----

Procedure (ii)

a)	The MS is paged and brought to the active state of a call (CC state = U10). The SS sends a DISCONNECT message to the MS with a missing mandatory 'cause' information element.

Requirement (ii)

1)	The MS shall respond to the erroneous DISCONNECT message by clearing the call with a RELEASE message including cause #100.

Message:     DISCONNECT from SS (GSM 04.08, 9.3.7)

information element	comment	  value

Protocol Discriminator	CC		0011

Transaction Identifier	SS allocated	0xxx

message type	disconnect	0010 	0101

cause	missing	----	----

Message:     RELEASE from MS (GSM 04.08, 9.3.14)

information element	comment	  value

Protocol Discriminator	CC		0011

Transaction Identifier	SS allocated	1xxx

message type	release	0x10 	1101

cause	IEI	0000	1000

length	0000	0010

coding/loc.	0110	0000

(#100)	1110	0100

Procedure (iii)

(Reserved)

Requirement (iii)

(Reserved)

Procedure (iv)

a)	The MS is paged and brought to the active state of a call (CC state = U10).

b)	The SS sends a STATUS message to the MS with a missing mandatory 'cause' information element.

c)	The SS sends a correctly coded STATUS ENQUIRY message to the MS .

Requirement (iv)

1)	The MS shall respond to the erroneous STATUS message by sending a STATUS message with cause #100.

2)	The MS shall respond to the STATUS ENQUIRY with a STATUS message including  the cause 'response to status enquiry' and CC state = U10 (active).

Message:     STATUS from SS (GSM 04.08, 9.3.20)

information element	comment	  value

Protocol Discriminator	CC		0011

Transaction Identifier	SS allocated	0xxx

message type	status	0011 	1101

Message:     STATUS from MS (GSM 04.08, 9.3.20)

information element	comment	  value

Protocol Discriminator	CC		0011

Transaction Identifier	SS allocated	1xxx

message type	status	0x11 	1101

cause	length	0000	0010

coding/loc.	0110	0000

1)(#100)	1110	0100

2)(#30)	1001	1110

call state	coding(U10)	1100	1010

II.5.3.5.6	Test of MS reaction to non-mandatory information element errors

For the selection of coding of unrecognized optional information elements the following shall apply:

1)	Unrecognized information elements shall be:

a)	coded as "unrecognized in the sublayer indicated by the protocol discriminator of the message";

b)	of type 4, and the length shall be greater than zero.

2)	Unrecognized information elements coded to indicate "Comprehension required" shall have a length greater than zero.

II.5.3.5.6.1	Test of MS reaction to Non-mandatory information element errors in RR messages

ref: GSM 04.08 section 8.8.1.1

II.5.3.5.6.1.1	Reserved

II.5.3.5.6.1.2	Reserved

II.5.3.5.6.1.3	Test of MS reaction to unrecognised information element in RR message on a dedicated control channel

Purpose of the test

To verify that the MS responds correctly to unrecognised information elements included in RR messages on the DCCH.

Initial conditions

The SS simulates two cells A and B. The MS is in the MM state  'idle,updated' in cell A listening to the BCCH/CCCH. The MS is paged and brought to the active state (U10) of a call in cell A.

Procedure 1

The SS sends a HANDOVER COMMAND message on the DCCH (specifying cell B) with an optional information element which is not recognisable as applicable to the RR message but is coded 'comprehension required'.

Requirements 1

1)	The MS shall not initiate the handover procedure

2)	The MS shall return an RR-STATUS message with cause value 'Invalid information element contents' to the SS.

3)	The MS shall maintain the link on the previous channel.

Message:     HANDOVER COMMAND (GSM 04.08, 9.1.14)

information element	comment	value

Protocol Discriminator	not relevant

Transaction Identifier	RR

message type	handover

cell description	as required

channel description	"

handover reference	"

power command	"

comprehension required	IEI	0000	0000

length

0000	0001

unrecognised IE contents	xxxx	xxxx

Procedure 2

The SS sends a HANDOVER COMMAND message on the DCCH with an optional information element which is not recognisable as applicable to the RR message but is NOT coded 'comprehension required'.

Requirements 2

The MS shall initiate the handover procedure and then return an RR-STATUS message with cause #100 to the SS.

Message:     HANDOVER COMMAND (GSM 04.08, 9.1.14)

Information element	comment	value

Protocol Discriminator	RR

Transaction Identifier	not relevant

message type	handover

cell description	as required

channel description	"

handover reference	"

power command	"

unrecognised IEI	(see note)	0110	1111

length	0000	0001

unrecognised IE contents	xxxx	xxxx

Note:	The coding chosen is not recognisable as a valid information element identifier for this message.

II.5.3.5.6.2	Test of MS reaction to Non-mandatory information element errors in MM messages

ref: GSM 04.08 section 8.8.1.2

Purpose of the test

To verify that the MS takes appropriate action upon receipt of an MM message containing error in an optional information element.

Initial conditions

a)	The SS simulates two cells, A and B , belonging to different location areas (default values as specified in II.5.3.1).

b)	The MS is in the MM-state 'idle,updated' listening to the BCCH/CCCH of cell A.

Note:	The test of 'normal location updating' (section II.5.3.7.4) shall have been previously passed.

Procedure 1

a)	The Rf level of cell A is lowered until the MS selects cell B (according to the cell-selection procedures of GSM 05.08).

b)	The SS responds to the location updating request with the LOCATION UPDATING ACCEPT message containing an optional information element coded 'comprehension required'.

c)	On receipt of the new LOCATION UPDATING REQUEST, the SS sends a LOCATION UPDATING ACCEPT correctly coded with TMSI 2.

Requirements 1

1)	The MS shall establish an RR connection and perform the 'normal' location-updating procedure (using TMSI 1).

2)	The MS shall return the MM-STATUS message with cause #100 in response to the LOCATION UPDATING ACCEPT.

3)	The MS shall abort the RR-connection ("local release") after a period of T3210 seconds from transmission of the initial LOCATION UPDATING REQUEST. After a period of T3211 seconds a new location updating procedure shall be started.

4)	Following receipt of the LOCATION UPDATING ACCEPT with TMSI, the MS shall send a TMSI REALLOCATION COMPLETE and then return to the MM-state 'idle,updated' in cell B (this is verified in Procedure 2).

Message:     Location Updating Accept (GSM 04.08, 9.2.11)

information element	comment	  value

Protocol Discriminator	MM

Transaction Identifier	not relevant

message type	loc upd acc.

location area id	cell B (5 bytes)

b) comprehension required	IEI	0000 	0000

length

0000	0001

unrecognised IE contents	xxxx	xxxx

c) Mobile Identity	TMSI 2

Procedure 2

a)	With the MS now in the 'idle,updated' state in cell B, (default  values as specified in II.5.3.1) the RF level of cell B is lowered until the MS selects cell A (according to the cell-selection procedures of GSM 05.08).

b)	The SS responds to the location updating request with the LOCATION UPDATING ACCEPT message containing an optional information element which is incorrect for the message and NOT coded 'comprehension required'.

c)	The SS waits at least a period of T3210.

d)	The SS shall page the MS on cell A with the PAGING REQUEST TYPE 1 message using the TMSI originally supplied by the MS in the LOCATION UPDATING REQUEST.

Requirements 2

1)	The MS shall establish an RR connection and perform the 'normal' location-updating procedure (using TMSI).

2)	The MS shall respond to the LOCATION UPDATING ACCEPT message  with an MM-STATUS message including cause value #99 ('information element non-existent or not implemented').

3)	The MS shall abort the RR connection ("local release")

4)	The MS shall respond to the PAGING REQUEST TYPE 1 message by initiating the immediate assignment procedure i.e. sends a CHANNEL REQUEST message with establishment cause set to "Answer to paging".

Message:     Location Updating Accept (GSM 04.08, 9.2.11)

information element	comment	  value

Protocol Discriminator	MM

Transaction Identifier	not relevant

message type	loc upd acc.

location area id	cell A(5 bytes)

unrecognised IEI	(see note)	0110	1111

length	0000	0001

unrecognised IE contents	xxxx	xxxx

Note:	The coding chosen is not recognisable as a valid information element identifier.

II.5.3.5.6.3	Test of MS reaction to Non-mandatory information element errors in CC messages

ref: GSM 04.08 section 8.8.1.3

Note:	The provision of 'diagnostic' information (as described by GSM 04.08) is optional and therefore not included in the tests described here.

Purpose of the test

To verify that the MS reacts correctly to errors in non-mandatory information elements included in CC messages. To verify that if present the diagnostic field element shall take the value described.

Initial condition for procedure 1

a)	The SS simulates a BS with BCCH/CCCH.

b)	The MS is made to originate a call and is brought to the  CC state U1 (call- initiated).

Procedure 1

a)	With the MS in the U1 state, the SS sends the CALL PROCEEDING message to the MS with an erroneous optional information element.

b)	After response from the MS,the SS sends the STATUS ENQUIRY message to the MS.

Requirements 1

1)	The MS shall respond to the CALL PROCEEDING message by returning a STATUS message (1) with cause #99 and CC-state = U1.

2)	The MS shall respond to the STATUS ENQUIRY message by returning a STATUS message (2) with cause #30 'response to status enquiry' and with CC-state = U3.

Procedure 2

a)	The SS sends the CONNECT message to the MS with the coding 'comprehension required' in an optional indicator information element.

b)	The SS sends a STATUS ENQUIRY message to the MS.

Requirements 2

1)	The MS shall respond to the CC message indicating 'comprehension required' by returning a STATUS message (3) with cause #100 and CC-state U3 (call proceeding).

2)	The MS shall respond to the STATUS ENQUIRY message by returning a STATUS message (2) with cause #30 'response to status enquiry' and with CC-state U3.

Procedure 3

a)	With the MS in the state U10 (active) of the mobile terminating call, the SS sends the DISCONNECT message to the MS with an error in an optional information element (causing it to be unrecognised).

b)	The SS sends a STATUS ENQUIRY message to the MS.

Requirements 3

1)	The MS shall respond to the DISCONNECT message with a RELEASE message including cause #99 'information element non-existent or not implemented'.

2)	The MS shall respond to the STATUS ENQUIRY message by returning a STATUS message (4) with cause #30 and CC-state = U19 (release request).

Procedure 4

a)	The MS is again brought to the active state (U10) of a mobile terminating call.

b)	The MS is made to end the call and consequently sends a DISCONNECT message to the SS.

c)	The SS sends the RELEASE message to the MS with an error in the optional 'cause' information element.

Requirements 4

1)	The MS shall respond to the RELEASE message with a RELEASE COMPLETE message including cause #99 'information element non-existent or not implemented'.

2)	The MS shall release the link.

Procedure 5

a)	The MS is again brought to the  active state (U10)  of a mobile terminating call.

b)	The SS ends the call by sending a DISCONNECT message to the MS.

c)	Following the RELEASE message from the MS, the SS sends a RELEASE COMPLETE message to the MS with an error in the optional 'cause' information element.

Requirements 5

1)	The MS shall respond to the RELEASE COMPLETE message by releasing the link.

Message:     CALL PROCEEDING (GSM 04.08, 9.3.3)

information element	comment	value

Protocol Discriminator	CC

Transaction Identifier	MS allocated

message type	call proc

unrecognised IEI	(see note)	0111	1111

length

0000	0001

unrecognised IE contents	xxxx	xxxx

Note:	The coding chosen is not recognisable as a valid information element identifier for this message.

Message:     CONNECT from SS (including IEI 'comprehension required'

ref: GSM 04.08, Table 10.45)

Note:	This is intended for extension of codesets: no specific message is defined presently by GSM.

information element	comment	value

Protocol Discriminator	CC

Transaction Identifier	MS allocated

message type	connect

comprehension required	IEI	0000 	0000

length

0000	0001

unrecognised IE contents	xxxx	xxxx

Message:     DISCONNECT from SS (GSM 04.08, 9.3.7)

information element	comment	  value

Protocol Discriminator	CC

Transaction Identifier	SS allocated

message type	disconnect

Cause

- length

- coding standard	standard defined for the GSM PLMN

- cause value	16 (normal clearing)

unrecognised IEI	(see note)	0111	1111

length

0000	0001

unrecognised IE contents	xxxx	xxxx

Note:	The coding chosen is not recognisable as a valid information element identifier for this message.

Message:     RELEASE from SS (GSM 04.08, 9.3.14)

information element	comment	  value

Protocol Discriminator	CC

Transaction Identifier	SS allocated

message type	release

unrecognised IEI	(see note)	0111	1111

length

0000	0001

unrecognised IE contents	xxxx	xxxx

Note:	The coding chosen is not recognisable as a valid information element identifier for this message.

Note:	Length may vary by optional diagnostic elements.

Message:     RELEASE COMPLETE from SS (GSM 04.08, 9.3.15)

information element	comment	value

Protocol Discriminator	CC

Transaction Identifier	SS allocated

message type	rel com

unrecognised IEI	(see note)	0111	1111

length

0000	0001

unrecognised IE contents	xxxx	xxxx

Note:	The coding chosen is not recognisable as a valid information element identifier for this message.

II.5.3.5.7	Test of MS reaction to content errors in Non-mandatory information elements

ref: GSM 04.08 section 8.8.2

In this stage, no tests are included to verify the behaviour of the MS under such conditions.

Not for phase 1

II.5.3.5.7.1.1	Test of MS reaction to content error in optional information elements included in RR messages on PCH or BCCH.

purpose of the test

To verify that the MS ignores all non-mandatory information elements with erroneous content which are included in RR messages on the Paging channel or the broadcast control channel.

initial conditions

procedure

requirement

II.5.3.5.7.1.2	Test of MS reaction to content error in optional information elements included in RR messages on DCCH.

purpose of the test

initial conditions

procedure

requirement

II.5.3.5.7.2	Test of MS reaction to content error in optional information elements included in MM messages .

purpose of the test

initial conditions

procedure

requirement

II.5.3.5.7.2	Test of MS reaction to content error in optional information elements included in CC messages .

purpose of the test

initial conditions

procedure

requirement