II.5.3.4	Lower layer failures in Layer 3 testing

II.5.3.4.1	Introduction

The text in this section is intended to develop a standardized way of creating lower layer failures whilst testing the performance of Layer 3 signalling.

References: GSM 04.08, 04.06, 05.08

There are two groups of lower layer failures:

1)  Detected by analysis of reception at Layer 1 (GSM 05.08, 04.08),

2)  Data link layer failures.

II.5.3.4.2	Layer 1 reception failures

-	The absence of reception of correct frames on the SACCH until the

S counter reaches value 0 will be interpreted as a Layer 1 failure.

II.5.3.4.3	Data Link layer failures

-	Many kinds of error cases can be caused in Layer 2. For example to many "T200 - timeout/retrying" - pairs.

Notes:

-	All types of Data Link failures are indicated similarly to the

RR - layer (Release Indication).

-	All types of L1 - failures are indicated similarly to each layer (Abort Indication, Error Indication).

II.5.3.4.4	Lower layer failures, used for the tests in section II.5.3

For L3 - testing different lower layer failures are performed:

1)	T100 - timeout in Layer 1.

2)	To many T200 - timeouts consecutively in Layer 2.