II.5.3.9	Testing of structured procedures

Ref: GSM 04.08 sections 7.3.2 through 7.3.4

II.5.3.9.1	General

The purpose of these tests is to complement the testing of elementary procedures to verify that the MS functions also in the "field" and not only on the "test bench".

The elementary procedures are combined into sequences, "structured procedures" as defined in GSM 04.08 section 7, representing true sequences of events for normal MS operation.

Only the main structured procedures are deemed necessary to test.

Mobile originating and terminating calls are tested in cases of both early and late assignment of traffic channel.

In two of the cases the call establishment test is followed by testing of call release initiated by network and mobile respectively.

The tests are performed only for a successful outcome of each elementary procedure.

Time requirements are indicated typically "(T3240-MM-MS)", showing timer identity, sublayer and (MS/NW) side where the timer is used (NW = Network).

The timer values are defined in GSM 04.08, section 11.

For an MS supporting speech the test procedures in II.5.3.9.2, II.5.3.9.3, II.5.3.9.4 and II.5.3.9.5 are performed for speech (teleservice 11, telephony)

For an MS not supporting speech, for each of the test procedures in II.5.3.9.2, II.5.3.9.3, II.5.3.9.4, and II.5.3.9.5 a teleservice supported by the MS (see PICS/PIXIT statement) is chosen, and the test is performed corresponding to that teleservice.

In cases where the tested teleservice is supported via an R or S interface and initiation of a Mobile Originated Call (MOC) can be performed via the MMI, or through the appropriate interface, procedure II.5.3.9.2 shall be performed when initiating the MOC from the MMI and procedure II.5.3.9.3 shall be performed when initiating the MOC through the appropriate interface.

II.5.3.9.2	MS originating call establishment, early assignment, release init. by network

II.5.3.9.2.1	Purpose of the test

a)	To test the MS ability to originate a call following the sequence of elementary signalling procedures given in GSM 04.08 section 7.3.2.1a.

b)	To test the MS ability to release a call, when the release is initiated by the network, following the sequence of elementary signalling procedures given in GSM 04.08 section 7.3.4a.

c)	To verify the MS display of the called number (cf. PICS/PIXIT statement).

II.5.3.9.2.2	Method of test

a)	The SS simulates one cell using the default parameter values for "cell 1" in section II.5.3.3 "Test of MS functions in Idle Mode".

b)	The MS is brought into the MM state "idle, updated", with a TMSI assigned, listening to the BCCH/CCCH of the active cell.

c)	The MS is made to initiate a call for the teleservice selected for the test and the SS accepts the CHANNEL REQUEST message and returns the IMMEDIATE ASSIGNMENT message before the timer (T3120-RR-MS) has expired, assuming that the randomly drawn value of T3120 is the lowest possible.

d)	The SS accepts the service request from the MS via the data link establishment procedure.

e)	The SS sends message AUTHENTICATION REQUEST.

f)	After reception of AUTHENTICATION RESPONSE the SS sends message CIPHERING MODE COMMAND within (T3230-MM-MS) seconds after the first access burst, and starts deciphering.

g)	After reception of CIPHER MODE COMPLETE the SS starts enciphering.

h)	The SS sends the message CALL PROCEEDING within (T303-CC-MS) seconds after the SS has received the message SETUP.

i)	The SS then sends message ASSIGNMENT COMMAND.

j)	Having received message ASSIGNMENT COMPLETE, the SS sends the message ALERTING within (T310-CC-MS) seconds after the sending of CALL PROCEEDING.

k)	Within (T301-CC-MS) seconds after ALERTING the SS sends the CONNECT message.

l)	The SS initiates clearing of the call by sending the message DISCONNECT.

m)	Within (T308-CC-MS) seconds after reception of the message RELEASE the SS sends RELEASE COMPLETE followed, within a further

(T3240-MM-MS) seconds, by CHANNEL RELEASE.

Message   IMMEDIATE ASSIGNMENT (GSM 04.08, 9.1.17) to the MS:

Comment                   Value

Protocol Discriminator     RR                           011

Transaction Identifier     Not used                    0000

Message Type                                      0011 1111

Page mode                  Normal                        00

Channel description

- Channel type ...         SDCCH/SACCH 1(4)          0 0100

- Time slot number         Arbitrary                    xxx

- Training seq.code                                 (default)

- Hopping                  No                             0

- FB no.                   Band no 0                    000

- ARFCN                                             (default)

Random reference

- Random access info       As in CHAN REQ         111x xxxx

- N51, N32, N26            As applicable               ....

Timing advance             Arbitrary                xx xxxx

Mobile allocation          Length=0 (due to hopping)      0

Message   AUTHENTICATION REQUEST (GSM 04.08, 9.2.2) to the MS:

Comment                   Value

Protocol Discriminator     MM                          0101

Transaction Identifier     Not used                    0000

Message Type                                      0001 0010

Ciphering key seq.number   As applic.from loc.upd       xxx

Authent.parameter RAND     As applicable          (128 bits)

Message   CIPHERING MODE COMMAND (GSM 04.08, 9.1.9) to the MS:

Comment                 Value

Protocol Discriminator     RR                       0110

Transaction Identifier     Not used                 0000

Message Type                                   0011 0101

Cipher mode setting        Start ciphering             1

Message   CALL PROCEEDING (GSM 04.08, 9.3.3) to the MS:

Comment                 Value

Protocol Discriminator     CC                       0011

Transaction Identifier     MS orig.                 1xxx

Message Type                                   0000 0010

Message   ASSIGNMENT COMMAND (GSM 04.08, 9.1.2) to the MS:

Comment                 Value

Protocol Discriminator     RR                       0110

Transaction Identifier     Not used                 0000

Message Type                                   0010 1110

Channel description

- Channel type ...         Bm+ACCHs               0 0001

- Time slot number         Arbitrary                 xxx

- Training seq.code                             (default)

- Hopping                  No                          0

- FB no                    Band no 0                 000

- ARFCN                                         (default)

Power level                As applicable          x xxxx

Channel mode               Appropriate for teleservice

selected for the test

Message   ALERTING (GSM 04.08, 9.3.1) to the MS:

Comment                 Value

Protocol Discriminator     CC                       0011

Transaction Identifier     MS orig.                 1xxx

Message Type                                   0000 0001

Message   CONNECT (GSM 04.08, 9.3.5) to the MS:

Comment                 Value

Protocol Discriminator     CC                       0011

Transaction Identifier     MS orig.                 1xxx

Message Type                                   0000 0111

Message   DISCONNECT (GSM 04.08, 9.3.7) to the MS:

Comment                 Value

Protocol Discriminator     CC                       0011

Transaction Identifier     SS orig.                 0xxx

Message Type                                   0010 0101

Cause

- Coding standard          GSM                        11

- Location                 User                     0000

- Cause value              Normal clearing      001 0000

Message   RELEASE COMPLETE (GSM 04.08, 9.3.15) to the MS:

Comment                 Value

Protocol Discriminator     CC                       0011

Transaction Identifier     SS orig.                 0xxx

Message Type                                   0010 1010

Drafting note: Is the "Cause" information element mandatory here ?

Message   CHANNEL RELEASE (GSM 04.08, 9.1.7) to the MS:

Comment                 Value

Protocol Discriminator     RR                       0110

Transaction Identifier     Not used                 0000

Message Type                                   0000 1101

RR cause                   Normal release      0000 0000

II.5.3.9.2.3	Requirements

1)	In step c) the MS shall display the dialled number in the way described in a PICS/PIXIT statement.

2)	In step c) the MS shall initiate immediate assignment by sending the CHANNEL REQUEST message.

3)	In step d) the MS shall send a layer 2 SABM frame containing the

CM SERVICE REQUEST message to the SS. (The same message is returned in a layer 2 UA frame from the SS). The SABM frame shall be sent after reception of the IMMEDIATE ASSIGNMENT message.

4)	In step e) the MS shall send the message AUTHENTICATION RESPONSE with the correct signed response. (Time requirements should be tested in the elementary procedure)

5)	In step f), within 1 second from CIPHERING MODE COMMAND, the MS shall activate ciphering and send the message CIPHERING MODE COMPLETE followed by the SETUP message

Note:	In requirement 5), the time of 1 second is not intended to be a performance requirement, it is introduced only because it is necessary to indicate how long the SS will wait until it decides that there is a failure.

6)	In step i) the MS shall send the message ASSIGNMENT COMPLETE on the correct channel after reception of the ASSIGNMENT COMMAND.

7)	Reserved.

8)	In step k) the MS shall send the message CONNECT ACKNOWLEDGE after reception of CONNECT, and through-connect the traffic channel in both directions.

9)	In step l) the MS shall send the message RELEASE after it has received the message DISCONNECT from the SS, initiating call release.

10)	After step m) the MS shall enter the idle, updated state.

Message   CHANNEL REQUEST (GSM 04.08, 9.1.8) to the SS:

Comment                 Value

Random reference                                  x xxxx

Cause                      Orig. call                111

Message   CM SERVICE REQUEST (GSM 04.08, 9.2.7) to the SS:

Comment                 Value

Protocol discriminator     MM                       0101

Transaction identifier     Not used                 0000

Message type                                   0x10 0100

CM service type            Mob.orig.call est.       0001

Ciphering key seq.number   As applic.from loc.upd    xxx

MS classmark 2

- Revision level           As applicable             xxx

- Encryption algorithm     A5                         00

- RF power capability      As applicable             xxx

- Short message capability As applicable               x

- Frequency band           Band no 0                 000

Mobile identity

- Odd/even no of digits    As applicable               x

- Type of identity         TMSI                      100

- Identity digits          As applicable            ....

Message   AUTHENTICATION RESPONSE (GSM 04.08, 9.2.3) to the SS:

Comment                 Value

Protocol discriminator     MM                       0101

Transaction identifier     Not used                 0000

Message type                                   0x01 0100

Authentication parameter   SRES, as applicable (32 bits)

Message   CIPHERING MODE COMPLETE (GSM 04.08, 9.1.10) to the SS:

Comment                 Value

Protocol discriminator     RR                       0110

Transaction identifier     Not used                 0000

Message type                                   0011 0010

Message   SETUP (GSM 04.08, 9.3.16) to the SS:

Comment                 Value

Protocol discriminator     CC                       0011

Transaction identifier     MS orig.                 0xxx

Message type                                   0x00 0101

Bearer capability          appropriate to the teleservice

selected for the test

Mobile identity            Info element may be present

- Odd/even no of digits    As applicable               x

- Type of identity         TMSI                      100

- Identity digits          As applicable            ....

Called party BCD number

- Type of number           As applicable             xxx

- Numbering plan id.       As applicable            xxxx

- Digits                   As applicable            ....

Message   ASSIGNMENT COMPLETE (GSM 04.08, 9.1.3) to the SS:

Message   CONNECT ACKNOWLEDGE (GSM 04.08, 9.3.6) to the SS:

Comment                 Value

Protocol discriminator     CC                       0011

Transaction identifier     MS orig.                 0xxx

Message type                                   0x00 1111

Message   RELEASE (GSM 04.08, 9.3.14) to the SS:

Comment                 Value

Protocol Discriminator     CC                       0011

Transaction Identifier     SS orig.                 1xxx

Message Type                                   0x10 1101

II.5.3.9.3	MS originating call establishment, late assignment

II.5.3.9.3.1	Purpose of the test

To test the MS ability to originate a call following the sequence of

elementary signalling procedures given in GSM 04.08 section 7.3.2.1b.

II.5.3.9.3.2	Method of test

a)	The first part of the method of test is identical to the case of early assignment of traffic channel (see previous section) up to and inclusive of the SS sending the CALL PROCEEDING message. Thereafter, the following applies:

b)	The SS sends the message ALERTING within (T310-CC-MS) seconds after the message CALL PROCEEDING was sent. The ALERTING message contents shall be the same as in section II.5.3.9.2.2.

c)	The SS then sends message ASSIGNMENT COMMAND. The message contents shall be the same as in section II.5.3.9.2.2.

d)	Having received message ASSIGNMENT COMPLETE, the SS sends message CONNECT within (T301-CC-MS) seconds after message ALERTING was sent. The CONNECT message contents shall be the same as in section II.5.3.9.2.2.

Note:	Clearing of the call is not part of the test procedure.

II.5.3.9.3.3	Requirements

1)	The first part of the requirements is identical to the case of early assignment of traffic channel (see previous section) up to and inclusive of the MS sending the SETUP message. Thereafter, the following applies:

2)	Reserved.

3)	In step c) the MS shall send the message ASSIGNMENT COMPLETE on the correct channel after reception of ASSIGNMENT COMMAND. The ASSIGNMENT COMPLETE message contents shall be the same as in section II.5.3.9.2.3.

4)	In step d) the MS shall send the message CONNECT ACKNOWLEDGE after reception of CONNECT, and through-connect the traffic channel in both directions.

II.5.3.9.4	MS terminating call establishment, early assignment, release init. by MS

II.5.3.9.4.1	Purpose of the test

a)	To test the MS ability to respond correctly to a mobile terminating call following the sequence of elementary signalling procedures given in GSM 04.08 section 7.3.2.1b.

b)	To test the MS ability to initiate and fulfil the release of a call, following the sequence of elementary signalling procedures given in GSM 04.08 section 7.3.4b.

II.5.3.9.4.2	Method of test

a)	The SS simulates one cell using the default parameter values for "cell 1" in section II.5.3.3 "Test of MS functions in Idle Mode".

b)	The MS is brought into the MM state "idle, updated", with a TMSI assigned, listening to the BCCH/CCCH of the active cell.

c)	The MS is paged by means of a PAGING REQUEST TYPE 1 message on the correct paging subchannel.

d)	After reception of the CHANNEL REQUEST the SS returns the IMMEDIATE ASSIGNMENT message before the timer (T3120-RR-MS) has expired, assuming that the randomly drawn value of T3120 is the lowest possible.

e)	The SS accepts the paging response from the MS via the data link establishment procedure.

f)	Immediately after the data link establishment the SS sends AUTHENTICATION REQUEST.

g)	Following the correct AUTHENTICATION RESPONSE from the MS the SS sends CIPHERING MODE COMMAND and then starts ciphering.

Drafting note: Timer values for authentication/ciphering are not specified.

h)	The SS sends the message SETUP immediately after reception of CIPHERING MODE COMPLETE. The SETUP message shall not include the 'signal' information element.

i)	When the SS has received a CALL CONFIRMED message, it sends message ASSIGNMENT COMMAND.

j)	If and when, after step i) the SS has received the message ASSIGNMENT COMPLETE and then the message ALERT, the MS is made to accept the call (e.g by performing off-hook on the MS, or by an MS internal action, see PICS/PIXIT statement).

Otherwise, if and when the SS has received at least one CONNECT message after step h), and received an ASSIGNMENT COMPLETE message after step i), it shall proceed with step k).

k)	The SS sends message CONNECT ACKNOWLEDGE within (T313-C-MS) seconds after reception of message CONNECT.

l)	The MS is initiated to clear the call.

m)	Within (T305-CC-MS) seconds after reception of the message DISCONNECT the SS sends the RELEASE message.

n)	Within (T3240-MM-MS) seconds after reception of the message RELEASE COMPLETE the SS releases the RR connection by sending the message CHANNEL RELEASE.

Message   PAGING REQUEST TYPE 1 (GSM 04.08, 9.1.21) to the MS:

Comment                 Value

Protocol Discriminator     RR                       0110

Transaction Identifier     Not used                 0000

Message Type                                   0010 0001

Page mode                  Normal                     00

Mobile identity

- Odd/even no of digits    As applicable               x

- Type of identity         TMSI                      100

- Identity digits          As applicable            ....

Message   IMMEDIATE ASSIGNMENT (GSM 04.08, 9.1.17) to the MS:

Comment                 Value

Protocol Discriminator       RR                     0110

Transaction Identifier       Not used               0000

Message Type                                   0011 1111

Page mode                    Normal                   00

Channel description

- Channel type ...           SDCCH/SACCH 1(4)     0 0100

- Time slot number           Arbitrary               xxx

- Training seq.code                             (default)

- Hopping                    No                        0

- FB no                      Band no 0               000

- ARFCN                                         (default)

Random reference

- Random access info         As in CHAN REQ    100x xxxx

- N51, N32, N26              As applicable

Timing advance               Arbitrary           xx xxxx

Mobile allocation            Length=0 (due to hopping) 0

Message   AUTHENTICATION REQUEST (GSM 04.08, 9.2.2) to the MS:

Comment                 Value

Protocol Discriminator     MM                       0101

Transaction Identifier     Not used                 0000

Message Type                                   0001 0010

Ciphering key seq.number   As applic.from loc.upd    xxx

Authent.parameter RAND     As applicable       (128 bits)

Message   CIPHERING MODE COMMAND (GSM 04.08, 9.1.9) to the MS:

Comment                 Value

Protocol Discriminator     RR                       0110

Transaction Identifier     Not used                 0000

Message Type                                   0011 0101

Cipher mode setting        Start ciphering             1

Message   SETUP (GSM 04.08, 9.3.16) to the MS:

Comment                 Value

Protocol discriminator     CC                       0011

Transaction identifier     SS orig.                 0xxx

Message type                                   0000 0101

Bearer Capability          appropriate to teleservice used in test

Message   ASSIGNMENT COMMAND (GSM 04.08, 9.1.2) to the MS:

Comment                 Value

Protocol Discriminator     RR                       0110

Transaction Identifier     Not used                 0000

Message Type                                   0010 1110

Channel description

- Channel type ...         Bm+ACCHs               0 0001

- Time slot number         Arbitrary                 xxx

- Training seq.code                             (default)

- Hopping                  No                          0

- FB no                    Band no 0                 000

- ARFCN                                         (default)

Power level                As applicable          x xxxx

Channel mode               appropriate to teleservice used in test

Message   CONNECT ACKNOWLEDGE (GSM 04.08, 9.3.6) to the MS:

Comment                 Value

Protocol discriminator     CC                       0011

Transaction identifier     SS orig.                 0xxx

Message type                                   0000 1111

Message   RELEASE (GSM 04.08, 9.3.14) to the MS:

Comment                 Value

Protocol Discriminator     CC                       0011

Transaction Identifier     MS orig.                 1xxx

Message Type                                   0010 1101

Message   CHANNEL RELEASE (GSM 04.08, 9.1.7) to the MS:

Comment                 Value

Protocol Discriminator     RR                       0110

Transaction Identifier     Not used                 0000

Message Type                                   0000 1101

RR cause                   Normal release      0000 0000

II.5.3.9.4.3	Requirements

1)	In step c) the MS shall initiate immediate assignment by sending the CHANNEL REQUEST message.

2)	In step d) the MS shall send a layer 2 SABM frame containing the PAGING RESPONSE message to the SS. (The same message is returned in a Layer 2 UA frame from the SS). The SABM frame shall be sent after reception of the IMMEDIATE ASSIGNMENT message.

3)	In step f) the MS shall send the message AUTHENTICATION RESPONSE with the correct signed response. (Time requirements should be tested in the elementary procedure.)

4)	In step g) the MS shall activate ciphering and send the message CIPHERING MODE COMPLETE.

5)	In step h) the MS shall send the message CALL CONFIRMED after reception of SETUP. It may then send a CONNECT message.

6)	After receiving the ASSIGNMENT COMMAND message in step i) the MS shall send the message ASSIGNMENT COMPLETE on the correct channel .

The MS may then send a CONNECT message if one of the following conditions holds:

- the MS did not send a CONNECT message in 5).

- the MS has sent a CONNECT message during 5). In this case the N(SD) of both CONNECT messages must be equal.

If the MS has sent at least one CONNECT message in 5) or 6), the following requirements 7) and 9) do not apply.

7)	After step i) the MS shall, in any order,

- generate an alerting indication (e.g by an appropriate tone),

- send the message ALERTING..

This requirement applies unless the MS has sent at least one CONNECT message during 5) or 6).

8)	Reserved.

9)	In step j), the MS shall send the message CONNECT. This requirement applies unless the MS has sent at least one CONNECT message during 5) or 6).

10)	In step j) the MS shall through-connect the traffic channel in both directions.

11)	In step l) the MS shall send the message DISCONNECT at the initiation of call release.

12)	In step m) the MS shall send the message RELEASE COMPLETE after reception of the message RELEASE.

13)	After step n) the MS shall enter the idle, updated state.

Message   CHANNEL REQUEST (GSM 04.08, 9.1.8) to the SS:

Comment                 Value

Random reference                                  x xxxx

Cause                      Answer to paging          100

Message   PAGING RESPONSE (GSM 04.08, 9.1.24) to the SS:

Comment                 Value

Protocol discriminator     RR                       0110

Transaction identifier     Not used                 0000

Message type                                   0010 0111

Ciphering key seq.number   As applic.from loc.upd    xxx

MS classmark 2

- Revision level           As applicable             xxx

- Encryption algorithm     A5                         00

- RF power capability      As applicable             xxx

- Short message capability As applicable               x

- Frequency band           Band no 0                 000

Mobile identity

- Odd/even no of digits    As applicable               x

- Type of identity         TMSI                      100

- Identity digits          As applicable            ....

Message   AUTHENTICATION RESPONSE (GSM 04.08, 9.2.3) to the SS:

Comment                 Value

Protocol discriminator     MM                       0101

Transaction identifier     Not used                 0000

Message type                                   0x01 0100

Authentication parameter   SRES, as applicable (32 bits)

Message   CIPHERING MODE COMPLETE (GSM 04.08, 9.1.10) to the SS:

Comment                 Value

Protocol discriminator     RR                       0110

Transaction identifier     Not used                 0000

Message type                                   0011 0010

Message   CALL CONFIRMED (GSM 04.08, 9.3.2) to the SS:

Comment                 Value

Protocol discriminator     CC                       0011

Transaction identifier     SS orig.                 1xxx

Message type                                   0x00 1000

Message   ASSIGNMENT COMPLETE (GSM 04.08, 9.1.3) to the SS:

Message   ALERTING (GSM 04.08, 9.3.1) to the SS:

Comment                 Value

Protocol Discriminator     CC                       0011

Transaction Identifier     SS orig.                 1xxx

Message Type                                   0x00 0001

Message   CONNECT (GSM 04.08, 9.3.5) to the SS:

Comment                 Value

Protocol Discriminator     CC                       0011

Transaction Identifier     SS orig.                 1xxx

Message Type                                   0x00 0111

Message   DISCONNECT (GSM 04.08, 9.3.7) to the SS:

Comment                 Value

Protocol Discriminator     CC                       0011

Transaction Identifier     MS orig.                 0xxx

Message Type                                   0x10 0101

Cause

- Coding standard          GSM                        11

- Cause value              normal clearing      001 0000

Message   RELEASE COMPLETE (GSM 04.08, 9.3.15) to the SS:

Comment                 Value

Protocol Discriminator     CC                       0011

Transaction Identifier     MS orig.                 0xxx

Message Type                                   0x10 1010

II.5.3.9.5	MS terminating call establishment, late assignment

II.5.3.9.5.1	Purpose of the test

To test the MS ability to respond correctly to a mobile terminated call following the sequence of elementary signalling procedures given in

GSM 04.08 section 7.3.3.1b.

II.5.3.9.5.2	Method of test

a)	The first part of the method of test is identical to the case of early assignment of traffic channel (see previous section) up to and inclusive of the SS sending the SETUP message, except that the SETUP message shall include the 'signal' information element. Thereafter, the following applies:

b)	If and when the SS has received the message ALERTING, the MS is made to accept the call, see PICS/PIXIT statement.

c)	After the SS has received the message CONNECT the SS sends message ASSIGNMENT COMMAND.

d)	Having received message ASSIGNMENT COMPLETE, the SS sends message CONNECT ACKNOWLEDGE within (T313-CC-MS) seconds after reception of message CONNECT.

Note: Clearing of the call is not part of the test procedure.

II.5.3.9.5.3	Requirements

1)	The first part of the requirements is identical to the case of early assignment of traffic channel (see previous section) up to and inclusive of the MS sending the CIPHER MODE COMPLETE message. Thereafter, the following applies:

2)	After receiving the SETUP message in step a), the MS shall proceed in one of the following ways:

a)	The MS sends a CALL CONFIRMED message followed by an ALERTING message. It shall generate an alerting indication (e.g by an appropriate tone).

b)	The MS sends a CALL CONFIRMED message followed by a CONNECT message.

3)	Reserved.

4)	In step b) the MS shall send the message CONNECT within 0.5 seconds from the acceptance of the call (off hook) at the MS.

This requirement applies only if the MS di not send a CONNECT message in 2).

Note:	In requirement 4), the time of 0.5 second is not intended to be a performance requirement, it is introduced only because it is necessary to indicate how long the SS will wait until it decides that there is a failure.

5)	In step c) the MS shall send the message ASSIGNMENT COMPLETE on the correct channel after reception of ASSIGNMENT COMMAND.

6)	In step d) the MS shall through-connect the traffic channel in both directions.

[within (...) milliseconds after it has received the message CONNECT ACKNOWLEDGE].