II.2 	THE TRANSCEIVER

Ref. GSM 05.05

General

This section addresses those aspects of a transceiver which are broader than only the transmitter or the receiver. It may be noted that frequency hopping and encryption are not tested explicitly. As many of the measurements on the transceiver are carried out whilst frequency hopping and encryption are active it is expected that these aspects will be tested implicitly to a sufficient degree.

However, the ability to switch to frequency hopping or non-hopping, and the ability to change the encryption key mode setting are specifically verified in other chapters.

II.2.1	Radio frequency aspects

II.2.1.1	Frequency bands

ref: GSM 05.05 section 2

A Mobile Station shall be able to operate in the following frequency bands

GSM900	DCS1800

transmit	890 to 915 MHz	1710 to 1785 MHz

receive	935 to 960 MHz	1805 to 1880 MHz

II.2.1.2	RF Channels and channel numbering

ref: GSM 05.05 section 2

The channel spacing shall be 200 kHz.

A Mobile Station shall be equipped to operate on each of the RF channels within its band. The nominal centre frequency Fc for each RF channel is given by the following expression

GSM900 (MHz)	DCS1800 (MHz)

transmit Fc	890 + n * 0.2 	1710.2 + 0.2 * (n - 512)

receive Fc	935 + n * 0.2 	1805.2 + 0.2 * (n - 512)

range for n	1 to 124	512 to 885

The value n is called the ABSOLUTE RADIO FREQUENCY CHANNEL NUMBER (ARFCN).

II.2.1.3	Frequency hopping

Ref: GSM 05.02, section 6.2.

Frequency hopping is an optional feature for the network. For the MS however, this function is mandatory.

The parameters to be used for frequency hopping tests throughout this document are described in Annex 1, Part General Conditions, GC1, unless otherwise indicated in the individual tests.

II.2.2	Spurious emissions

II.2.2.1	Definition

Spurious emissions are emissions at frequencies other than those of the carrier and sidebands associated with normal modulation. The level of these spurious emissions shall be measured as:

(a)	Their power level in a specified load.

(b)	Their effective radiated power when radiated by the cabinet and structure of the mobile station, including all interconnecting cables.

Note: (b) is also known as "cabinet radiation".

For Mobile Stations having an antenna connector both (a) and (b) shall be measured.

For Mobile Stations with an integral antenna and no means of connecting an external antenna only (b) shall be measured.

For Mobile Stations with an integral antenna and means for connecting an external antenna both (a) and (b) shall be measured.

II.2.2.2	Method of Measurement (a)

II.2.2.2.1	Mobile Allocated Channel

a)	A call is set up by the SS according to the generic call set up procedure. The channel shall be in the Mid ARFCN range (ref. Table 1.4).

b)	The SS shall command the mobile to loop back its channel decoder output to channel encoder input.

c)	The SS shall generate Standard Test Signal C1.

d)	The MS shall be set to operate at its maximum output power.

e)	Measurements are made in the frequency range 100 kHz to 12.75 GHz. Spurious emissions shall be measured at the connector of the tranceiver, as the power level of any discrete signal, higher than the requirement in section II.2.2.4 minus 6 dB, delivered into a 50 Ohm load.

The measurement bandwidth based on a 5 pole synchronously tuned filter shall be according to Table II.2.1. The power indication shall be the peak power detected by the measuring system.

f)	The test shall be repeated under extreme test conditions (Annex 1, TC2.2 and TC3).

GSM900 only - begin

TABLE II.2.1

Frequency	Frequency Offset	Filter Bandwidth	Approx Video

Range			Bandwidth

100 kHz

to 50 MHz	-	10 kHz	30 kHz

50 MHz

to 500 MHz	-	100 kHz	300 kHz

(See Note)

0 MHz to 10 MHz	100 kHz	300 kHz

500 MHz	10MHz to 20 MHz	300 kHz	1 MHz

to 12.75 GHz

Excl. 890 MHz	20 MHz to 30 MHz	1 MHz	3 MHz

to 915 MHz

and 935 MHz	> 30 MHz	3 MHz	3 MHz

to 960 MHz

(offset from edge of

MS TX Band)

890 MHz	1.8 MHz to 6.0 MHz	30 kHz	100 kHz

to 915 MHz

> 6.0 MHz	100 kHz	300 kHz

(offset from carrier)

Note:	The Filter, Video bandwidths and frequency offsets are only correct for measurements on a MS transmitting on an ARFCN in the range 60 to 65.

GSM900 only - end

DCS1800 only - begin

TABLE II.2.1

Frequency	Frequency Offset	Filter Bandwidth	Approx Video

Range			Bandwidth

100 kHz

to 50 MHz	-	10 kHz	30 kHz

50 MHz

to 500 MHz	-	100 kHz	300 kHz

500 MHz	>= 2 MHz	30 kHz	100 kHz

to 12.75 GHz	>= 5 MHz	100 kHz	300 kHz

Excl. 	>= 10MHz	300 kHz	1 MHz

935 MHz	>= 20MHz	1 MHz	3 MHz

to 960 MHz	> 30 MHz	3 MHz	=> 3MHz

(offset from edge of

MS TX Band)

and 1710 MHz

to 1785 MHz

and 1805 MHZ

to 1880 MHz

1710 MHz	1.8 MHz to 6.0 MHz	30 kHz	100 kHz

to 1785 MHz	> 6.0 MHz	100 kHz	300 kHz

(offset from carrier)

Note:	1) Due to practical implementation of a SS, the video bandwidth is restricted to a maximum of 3 MHz.

2) The filter bandwidth for frequenceies below 500 MHz is reduced from 3 MHz due to increasing internal noisse of available test equipment.

3) The frequency ranges 935 MHz to 960 MHz and 1805 MHz to 1880 MHz are excluded as these ranges are tested in section II.3.4.

DCS only - end

II.2.2.2.2	MS in Idle Mode

a)	The MS is connected to the SS under normal test conditions. The BCCH message contentfrom the serving cell shall ensure that the MS DRX function is not activated.

b)	The MS shall be brought into idle mode, already camped on the serving cell.

c)	Measurements are made in the frequency range 100 kHz to 12.75 GHz. Spurious emissions shall be measured as the power level of any discrete signal, higher than the requirement in section II.2.2.4 minus 6 dB, delivered into a 50 Ohm load.

The measurement bandwidth based on a 5 pole synchronously tuned filter shall be according to Table II.2.2. The power indication shall be the peak power detected by the measuring system.

TABLE II.2.2

Frequency range	Filter Bandwidth	Video Bandwidth

100 kHz to 50 MHz	10 kHz	30 kHz

50 MHz to 12.75 GHz	100 kHz	300 kHz

d)	The test shall berepeated under extreme test conditions (Annex 1, TC2.2 and TC3).

II.2.2.3	Method of Measurement (b)

On an outdoor test site, fulfilling the requirements of GC4 of Annex 1 or in an anechoic shielded chamber (GC5 of Annex 1), the sample shall be placed at the specified height on the support.

Note:	The test method described has been written for measurement in an anechoic shielded chamber. If an outdoor test site is used then additional precautions are necessary to ensure correct measurement. These measures are familiar to test houses which perform spurious emissions tests and are:

a) Raise/lower the test antenna through the specified height range during both the emission detection and substitution parts of the test.

b) Perform a qualitative pre-search in a shielded environment for test sites where the ambient RF environment can prevent the detection of spurious emissions which exceed the limit.

c) Detect emissions at a more sensitive threshold to that specified in II.2.2.3.1 e) to allow for destructive interference due to ground plane reflections at the test antenna search height.

II.2.2.3.1	MS Allocated a Channel

a)	A call shall be set up by the SS according to the generic call set up procedure on a channel in the Mid ARFCN range (ref. Table 1.4) .

b)	The SS shall command the MS to loop back its channel decoder output to its channel encoder input.

c)	The SS shall generate Standard Test Signal C1.

d)	The transmitter is set to its maximum power.

e)	Initially the test antenna is closely coupled to the MS and any spurious emission radiated by the MS shall be detected by the test antenna and receiver in the range 30 MHz to 4 GHz.

Note:	This is a qualitative step to identify the frequency and presence of spurious emissions which are to be measured in subsequent steps.

f)	The test antenna separation is set to the appropriate measurement distance and at each frequency at which an emission has been detected, the MS shall be rotated to obtain maximum response and the effective radiated power of the emission determined by a substitution measurement. In case of an anechoic shielded chamber pre-calibration may be used instead of a substitution measurement.

The measurement bandwidth, based on a 5 pole synchronously tuned filter, shall be according to Table II.2.3. The power indication shall be the peak power detected by the measuring system.

g)	The measurements shall be repeated with the test antenna in the orthogonal polarisation plane.

h)	The test shall be repeated under extreme voltage test conditions (see Annex 1, TC2.2).

GSM900 only - begin

TABLE II.2.3

Frequency	Frequency Offset	Filter Bandwidth	Approx Video

Range			Bandwidth

30 MHz

to 50 MHz	-	10 kHz	30 kHz

50 MHz

to 500 MHz	-	 100 kHz 	300 kHz

(See Note)

0 MHz to 10 MHz	100 kHz	300 kHz

500 MHz

to 1 GHz	10 MHz to 20 MHz	300 kHz	1 MHz

Excl. 890 MHz

to 915 MHz	20 MHz to 30 MHz	1 MHz	3 MHz

>= 30 MHz	3 MHz	3 MHz

offset from edge

of TX band

890 MHz	1.8 MHz to 6.0 MHz	30 kHz 	100 kHz

to 915 MHz	> 6.0 MHz	100 kHz 	300 kHz

offset from carrier

1 GHz to 4 GHz	-	1 MHz	3 MHz

Note:	The Filter, Video bandwidths and frequency offsets are only correct for measurements on an MS trancmitting on an ARFCN in the range 60 to 65.

GSM900 only - end

DCS1800 only - begin

TABLE II.2.3

Frequency	Frequency Offset	Filter Bandwidth	Approx Video

Range			Bandwidth

30 MHz

to 50 MHz	-	10 kHz	30 kHz

50 MHz

to 500 MHz	-	 100 kHz 	300 kHz

500 MHz

to 4 GHz	> 2 MHz-	 30 kHz 	100 kHz

Excl.  	>= 5 MHz	100 kHz	300 kHz

>= 10 MHz	300 kHz	1 MHz

>= 20 MHz	1 MHz	3 MHz

1710 MHz	>= 30 MHz	3 MHz	=> 3 MHz

to 1785 MHz	offset from edge

of band

1.8 MHz to 6.0 MHz 	30 kHz 	100 kHz

> 6.0 MHz	100 kHz 	300 kHz

1710 MHz	(offset from carrier)

to 1785 MHz

Note:	a) Due to practical implementation of a SS, the video bandwidth is restricted to a maximum of 3 MHz.

b) The filter badwidth for frequencies below 500 MHz is reduced from 3 MHz due to increasing internal noise of available test equipment.

DCS1800 only - end

II.2.2.3.2	MS in Idle Mode

a)	The MS is connected to the SS under normal test conditions. The BCCH message content from the serving cell shall ensure that the MS DRX function is not activated.

The MS shall be brought into idle mode, already camped on the serving cell.

b)	Initially the test antenna is closely coupled to the MS and any spurious emission radiated by the MS shall be detected by the test antenna and receiver in the range 30MHz to 4GHz.

Note: 	This is a qualitative step to identify the frequency and presence of spurious emissions which are to be measured in subsequent steps.

c)	The test antenna separation is set to the appropriate measurement distance and at each frequency at which a spurious emission has been detected the MS shall be rotated to obtain a maximum response. The effective radiated power of the emission shall be determined by a substitution measurement. In case of an anechoic shielded chamber pre-calibration may be used instead of a substitution measurement.

The measurement bandwidth based on a 5 pole synchronously tuned filter shall be according to Table II.2.4. The power indication shall be the peak power detected by the measuring system.

TABLE II.2.4

Frequency range	Filter Bandwidth	Video Bandwidth

30 MHz to 50 MHz	10 kHz	30 kHz

50 MHz to  4 GHz	100 kHz	300 kHz

d)	The measurements shall be repeated with the test antenna in the orthogonal polarisation plane.

e)	The test shall be repeated under extreme voltage test conditions (see Annex 1, TC2.2).

II.2.2.4	Requirements

The power of any spurious emission shall not exceed the values given below:

GSM900	DCS1800

Frequency range	Power level in dBm

Allocated	Idle	Allocated	Idle

channel	mode	channel	mode

100 kHz to 1 GHz	-36	-57	-36	-57

1 to 12.75 GHz	-30	-47	-	-

1 GHz to 1710 MHz	-	-	-30	-47

1710 to 1785 MHz	-	-	-36	-57

1785 to 1805 MHz	-	-	-30	-47

1805 to 1880 MHz	-	-	-30	-57

1880 MHz to 12.75 GHz	-	-	-30	-47

Note:	For the filter bandwidths quoted in the test method some difficulties may be experienced with noise floor above required measurement limit. This will depend on the gain of the test antenna and adjustment of the measuring system  bandwidth is permissible when carrying out method (b). Alternatively, for test frequencies above 900 MHz, the test antenna separation from the MS may be  reduced to 1 metre.