II.17	Selftesting of the MS

Ref: GSM 12.10

Purpose of the test:

Verify that the MS carries out selftesting.

Method of test

a)	The MS is connected to the SS. In the case of a MS with integrated antenna it is coupled via an antenna coupling device.

The SS transmits a BCCH with a location updating time set to 0.1 hours.

b)	The MS is operated under normal test conditions.

c)	The SS sends a paging request message to the MS.

d)	The MS responds with a channel request message.

e)	The SS sends an immediate assignment message establishing an SDCCH.

f)	The power supply voltage is gradually reduced to the value declared in the PIXIT statement to be the shutdown voltage of the MS.

g)	The SS observes whether the MS ceases the production of RF output.

h)	After (7) minutes, the SS sends a paging message to the MS.

i)	The SS observes whether the MS produces any RF output.

j)	The location area of the BCCH is modified.

k)	For (7) minutes, the SS observes whether the MS produces any RF output.

Note:	It is anticipated that the MS might attempt Location Updating.

l)	The MS is switched off and on.

m)	The MS is paged by the SS.

n)	The SS observes whether the MS produces any RF output.

Requirements:

1)	In step g), the MS shall cease the production of RF output.

2)	In steps i), k) and n), the MS shall not produce any RF output.