ANNEX  3

Annex 3 :	Protocol  Implementation  Information

GENERAL

The list of PICS and PIXIT gives all the information needed to perform the tests described in GSM 11.10.

1	Protocol  Implementation  Conformance  Statement  (PICS)

For the points listed the manufacturer has the choice between different solutions in implementation. The manufacturer has to describe his choice if there is any consequence for the tests.

1.1	LAPDm  PROTOCOL (GSM 04.05 and 04.06)

1.1.1	Simplified protocol - GSM 04.06 section 6

Statement about the choice made by the manufacturer.

1.1.2	Management of SAPI = 3 - GSM 04.11 section 2.3

Statement about the handling of SAPI = 3 on the Data Link Layer chosen by the manufacturer.

1.2	MOBILITY  MANAGEMENT

1.2.1	IMSI detach initiation by the MS - GSM 04.08 section 4.3.4.1

During a location updating, if an IMSI detach has to be performed (SIM or power off), the IMSI detach can be delayed until the location updating is performed, or can be deleted.

1.2.2	IMSI detach completion by the MS - GSM 04.08 section 4.3.4.3

The MS should delay the local release of the channel to allow a normal release from the network after a detach by power off command, if possible.

If not possible the RR sublayer on the MS side should be aborted without waiting for something from the network.

1.2.3	MM specific procedures - GSM 04.08 section 4.4

During the lifetime of an MM specific procedure, if a MM connection establishment is required by a CM-entity, this request will either be rejected or delayed until the running MM specific procedure is terminated.

1.2.4	MM connection establishment initiated by the MS - GSM 04.08 section 4.5.1.1

If an MM specific procedure running at the time the request from the CM layer is received, the request will either be rejected, or delayed until the MM specific procedure is finished and the RR connection is released.

1.2.5	Receiving an MM STATUS message - GSM 04.08 section 4.6

Local actions to be taken on receiving an MM-STATUS message from the network are an implementation dependent option.

1.2.6	Call re-establishment - GSM 04.08 section 4.5.1.6

MM indicates a low layer failure to all the CM entities with an MM connection. The re-establishment procedure is done if one of the CM entities asks for this re-establishment, without waiting for the answers of the other CM entities, or when all the answers from the CM entities have been received.

1.2.7	Message type error - GSM 04.08 section 8.5

The MS shall return a status message with either of the causes

#98	"message not compatible with call state or non-existent or not implemented",

#97	"message type non-existent or not implemented".

1.3       CALL  CONTROL

1.3.1	Clearing when tones/announcements provided - GSM 04.08 section 5.4.4.1

If the traffic channel is connected, on receipt of the DISCONNECT message with progress indicator # 8, the MS shall connect to the in-band tone announcement and enter disconnect indication state.

Or continue clearing without connecting the in-band tone announcement.

1.3.2	Status enquiry procedures - GSM 04.08 section 5.5.3.1

The MS does not send STATUS ENQUIRY to the network. It analyses the STATUS ENQUIRY received and answers.

Or, the MS can send STATUS ENQUIRY  and it takes the appropriate actions based on the answer (STATUS) of the network.

1.3.3	Receiving a STATUS message by a CC entity - GSM 04.08 section 5.5.3.2

The determination of which CC states are incompatible between the MS and the network is left as an implementation decision except in some particular cases.

1.3.4	Unrecognised CC optional information element - GSM 04.08 section 8.8.1.3

When a CC message is received from the network with an unrecognized information element, the MS sends a CC-STATUS message with cause # 99 "information element non existent or not implemented", and with an optional diagnostic field giving the unrecognized information element identifier.

1.3.5	CC optional information element content error - GSM 04.08 section 8.8.2.3

When a CC message is received from the network which has one or more non-mandatory information element with invalid content, the MS sends a CC-STATUS message with cause # 100 "invalid information element contents" and an optional diagnostic field giving the invalid information element identifier.

1.3.6	Called side compatibility checking - GSM 04.08 annex B.3

Compatibility checking can be performed in various ways from the viewpoint of execution order and information to be checked, e.g. first DDI number, sub-address and then compatibility or vice versa.

1.4	LAYER 1

1.4.1	Optional storage of BCCH carrier information - GSM 05.08 section 6.3

The MS may include optional storage of BCCH carrier information. For instance, the MS store the BCCH carriers in use by the PLMN accessed when it was last active in the GSM network, or it may store BCCH carriers for more than one PLMN.

1.4.2	Call re-establishment option - GSM 05.08 section 5.2

This procedure allows the MS to resume a connection in progress after a lower layer failure, possibly in a new cell.

1.5	Autocalling - (ref: GSM 02.07, Annex 1)

Cause number 27 implemented in:

-	category 2 (preferred);

-	category 3.

1.6	TRANSIENT STATES

The following states may be transient:

State U6

State U6 may be transient if the implementation allows it.

State U7

If the implementation allows for automatic connect after an implementation specific time T.

State U9

State U9 is not transient if:

- the implementation does not support immediate connect

- an appropriate TCH is not yet assigned

- the signalling element has not been present in the SETUP

State U12

U12 is stable if the implementation supports it and the progress indicator is present in DISCONNECT and indicates the availability of in-band announcements.

2	Protocol Implementation Extra Information for Testing (PIXIT)

2.0	INTRODUCTION

Some of the features listed below are mandatory, others are not ; but in any case for each feature implemented the manufacturer must provide information to enable regulatory testing to be conducted.

2.1	BASIC  CHARACTERISTICS

2.1.1	Type of antenna

-	Integrated without a connector.

-	Position for normal use (if integrated without a connector).

-	With a connector allowing the connection of an external antenna.

If with a connector, declare in band impedance.

2.1.2	Power supply

-	Type of battery (if any).

-	Type of power supply.

-	Nominal voltage(s).

-	End-point voltage(s) of battery(ies) (if any).

-	Details of MS shutdown voltage.

2.1.3	Power class of the MS

-	Different class declared.

-	Class mark change : description of the means to change the RF power capabilities.

2.1.4	Channel modes supported

Speech full rate

Speech half rate

Data 9.6 full rate

Data 4.8 full rate

Data 4.8 half rate

Data 2.4 full rate

Data 2.4 half rate

2.1.5	Teleservices supported

11)	Telephony

12)	Emergency calls

21)	Short message MT/PP

22)	Short message MO/PP

23)	Short message transmission cell broadcast

31)	Advanced MHS access

41)	Videotex access profile 1

42)	Videotex access profile 2

43)	Videotex access profile 3

51)	Teletex

61a)	Alternate speech and facsimile group 3 T

61b)	Alternate speech and facsimile group 3 NT

62a)	Automatic facsimile group 3 T

62b)	Automatic facsimile group 3 NT

2.1.6	Supplementary services supported

Call forwarding.

Call restriction.

Handling of undefined GSM Supplementary Services.

2.1.7	Bearer services supported

21a )	Data circuit Duplex asynchronous	300 bit/s		T

21b )	Data circuit Duplex asynchronous	300 bit/s		NT

22a )	Data circuit Duplex asynchronous	1200 bit/s	T

22b )	Data circuit Duplex asynchronous	1200 bit/s	NT

23a )	Data circuit Duplex asynchronous	1200/75 bit/s	T

23b )	Data circuit Duplex asynchronous	1200/75 bit/s	NT

24a )	Data circuit Duplex asynchronous	2400 bit/s	T

24b )	Data circuit Duplex asynchronous	2400 bit/s	NT

25a )	Data circuit Duplex asynchronous	4800 bit/s	T

25b )	Data circuit Duplex asynchronous	4800 bit/s	NT

26a )	Data circuit Duplex asynchronous	9600 bit/s	T

26b )	Data circuit Duplex asynchronous	9600 bit/s	NT

31  )	Data circuit Duplex synchronous 	1200 bit/s	T

32  )	Data circuit Duplex synchronous 	2400 bit/s	T

33  )	Data circuit Duplex synchronous 	4800 bit/s	T

34  )	Data circuit Duplex synchronous 	9600 bit/s	T

41a )	PAD Access circuit asynchronous	300 bit/s		T

41b )	PAD Access circuit asynchronous	300 bit/s		NT

42a )	PAD Access circuit asynchronous	1200 bit/s	T

42b )	PAD Access circuit asynchronous	1200 bit/s	NT

43a )	PAD Access circuit asynchronous	1200/75 bit/s	T

43b )	PAD Access circuit asynchronous	1200/75 bit/s	NT

44a )	PAD Access circuit asynchronous	2400 bit/s	T

44b )	PAD Access circuit asynchronous	2400 bit/s	NT

45a )	PAD Access circuit asynchronous	4800 bit/s	T

45b )	PAD Access circuit asynchronous	4800 bit/s	NT

46a )	PAD Access circuit asynchronous	9600 bit/s	T

46b )	PAD Access circuit asynchronous	9600 bit/s	NT

51a )	Data Packet Duplex synchronous 	2400 bit/s	T

51b )	Data Packet Duplex synchronous 	2400 bit/s	NT

52a )	Data Packet Duplex synchronous 	4800 bit/s	T

52b )	Data Packet Duplex synchronous 	4800 bit/s	NT

53a )	Data Packet Duplex synchronous 	9600 bit/s	T

53b )	Data Packet Duplex synchronous 	9600 bit/s	NT

61a )	Alternate Speech/Unrestricted Digital			T   for unres dig

(here unres dig offers the same service

as bearer services 21-34)

61b )	Alternate Speech/Unrestricted Digital			NT  for unres dig

(here unres dig offers the same service

as bearer services 21-26)

71  )	12 kbit/s Unrestricted Digital

81a )	Speech followed by data				T   for unres dig

(here unres dig offers the same service

as bearer services 21-34)

81b )	Speech followed by data				NT  for unres dig

(here unres dig offers the same service

as bearer services 21-26)

2.2	MAN  MACHINE  INTERFACE

2.2.1	Mobile Station features

-	Description of manual entry and display of a called number.

-	Description of the basic way to send a call manually.

-	Description of the basic way to take a call manually.

-	Description of the basic way to end a call manually.

-	Description of the basic way to send an emergency call manually.

-	Description of the basic way to send DTMF manually.

-	Description of the manual PLMN selector.

-	Description of the automatic PLMN selector.

-	Description of the indication of the country.

-	Description of the indication of the available PLMN.

-	Description of the indication of the automatic registration to a PLMN.

-	Description of the service indicator.

-	Description of the management of the SIM by the user:

.	keying PIN and changing PIN,

.	indication of acceptance or rejection of keyed PIN,

.	indication of blocked SIM,

.	indication of successful unblocking of the SIM,

.	storing an abbreviated number,

.	displaying an abbreviated number.

-	Description of the selection of the hand free.

-	Description of the volume control.

-	Description of local barring of outgoing calls.

-	Description of prevention of unauthorized calls.

-	Description of the auto calling management:

.	selection of the auto calling,

.	indication that the call failed and a retry is attempted,

.	indication that the call finally failed.

Note:	All the above description could be extracted from the user's manual.

2.2.2	Short Message Service

-	Description of the basic procedures to send a mobile originated Short Message.

-	Description of the basic procedures to display a mobile terminated Short Message.

-	Description of the basic procedures to display a cell broadcasted Short Message.

-	The value of the timer TC1M.

-	Whether SMS messages are stored in the SIM and/or the ME.

-	Maximum length (characters) of a mobile originated Short Message.

2.2.3	Supplementary services

2.2.3.1)	Call forwarding

-	Description of the user's commands and of the display of the answers from the network for:

registration,

erasure,

interrogation,

specific data request.

-	Description of the display of:

.	notification of an incoming call to the "served" mobile or the "forwarded to" mobile,

.	notification during out-going call,

.	information to the calling mobile.

2.2.3.2)	Call restriction

-	Description of the user's commands and the display of the answers from the network for:

registration,

change of the password,

activation,

deactivation,

interrogation.

-	Description of the display of the indication of call barring.

2.2.3.3)	Handling of (undefined) GSM supplementary services

-	Description of the user's commands and the display of the answer from the network.

2.3	EMMI - ELECTRICAL  MAN  MACHINE  INTERFACE

2.3.1	Method(s) supported for activation/deactivation of EMMI

-	all possibilities specified in GSM 11.10, III.1.2.2;

-	all possibilities specified in GSM 11.10, III.1.2.2, except activation by inserting a Test-SIM (when the ME is already switched on);

-	activation/deactivation only via Layer 3 messages on the radio interface according to GSM 11.10, III.1.2.2.

2.3.2	Transmission rate supported by the ME on the EMMI

2.3.3	Layer 3 messages supported on the EMMI

-	Layer 3 messages as specified in GSM 11.10, III.1.3.5.3.2, except: (followed by the list of messages not supported);

-	others than defined in GSM 11.10 Part III Table 9.

2.3.4	Keystroke sequence messages

Non-standardised Keystroke sequences as to be used on the EMMI (in line with GSM 11.10, III.1.3.5.3.2):

-	related to tests of the Mobile Station features (GSM 11.10, II.16)

-	related to testing of the ME/SIM interface (GSM 11.10, II.8)

-	related to tests of Autocalling Restrictions (GSM 11.10, II.9)

-	related to tests of Supplementary Services (GSM 11.10, II.12)

-	related to tests of Data Services (GSM 11.10, II.10, II.14, II.15)

-	related to tests of Short Message Service (GSM 11.10, II.11.2.2.2)

-	related to other tests.

2.3.5	Internal malfunction detected messages

List of the error indicators provided.

2.4	DAI,  DIGITAL  AUDIO  INTERFACE

Description of the speech data routing

-	via the control lines

-	or via the test interface message.

2.5	CHARACTERISTICS  RELATED  TO  BEARER  SERVICES  OR  TELESERVICES

2.5.1	Access interface

Description of the access interface to connect the DTE.

The MT may have various user interfaces, such as:

-	V.21 DTE/DCE interface,

-	V.22 DTE/DCE interface,

-	V.22 bis DTE/DCE interface,

-	V.23 DTE/DCE interface,

-	V.26 ter DTE/DCE interface,

-	V.32 DTE/DCE interface,

-	X.21,

-	X.25,

-	S interface.

In case of a proprietary interface to a DTE (non standard), description of this interface (hardware and software).

In case of a non-standard connector provide a mechanical adapter.

2.5.2	Configuration of the MT

Description of the configuration information to be selected in the MT to connect a Terminal Equipment to the Mobile Termination.

Description of the (different) configuration(s) of the MT for each bearer service and each teleservice supported, with the range or value for the parameters and the configuration procedure.

2.5.3	Bearer capabilities

Description of the bearer capabilities, related to supported bearer services

-	higher layer capabilities,

-	lower layer capabilities,

-	user to user capabilities,

description with all the values for the parameters,

supported by the MT,

-	configuration of the MT.

The manufacturer shall describe precisely how it is possible to put the MT in the different configurations to test the bearer capabilities, the lower and higher layer capabilities, the user to user capabilities, described as supported by the MT.

The manufacturer shall describe for every capability the associated terminal functions and their characteristics. The manufacturer shall describe how to verify the correct selection by the MT of the appropriate function, especially using the messages at the Um interface if there is no R or S interface available (case MTO). The description shall be made for every combination of the parameter value valid for the MT.

2.5.4	Subaddress or DDI number

Subaddress or a DDI number of the MT.

Procedure to allocate or change DDI number or subaddress, if possible.

2.5.5	User to user signalling

Description of the function and the user's access to it.

2.5.6	Data call set-up and data call clearing

For each implemented transparent and non-transparent data service:

-	Description of the Data Call Establishment Mechanism

-	Terminal initiated (CT108) (if possible),

-	MT (MMI/EMMI) initiated.

-	Description of DCE provided information (MT to TE), if any.

-	Declaration of optimal function and procedure, services supported by the MT.

-	Description of the Data Call Clearing Mechanism

-	Terminal initiated (CT109) (if possible),

-	MT (MMI/EMMI) initiated.

-	Description of DCE provided information (MT to TE) related to a mobile or network initiated call clearing, if any.

2.5.7	Characteristics of non-transparent data services

Description of Radio Link Protocol (RLP) features supported

-	SREJ command

-	Piggybacking of I and S frames

Description of Flow Control Mechanism

-	INBAND (XON/XOFF)

-	OUTBAND (CT105 and CT106)

2.5.8	V.25 bis procedures for the V.-series R-interface

All procedures must be described to such a level of detail that the test house can program a terminal connected to the R-interface so that the tests in section II.10.3  can be performed.  The procedures must conform to V.25 bis.  It is recommended that the expected behaviour of the LTE is described.

-	If the procedure of section II.15.2.2.2 is not used for call set-up through the R-interface then it is to be described here.

-	If other procedures than that of section II.15.2.2.3 are used for call clearing through the R-interface then they are to be described here.

-	Describe in detail one or more of the possible procedures to handle an incoming call through the R-interface.

2.5.9	Possible ways of setting-up a call from either an external interface or internally

Describe in detail all possible ways a call can be initiated from the MS or a connected terminal.

2.6	International Mobile Station Equipment Identity

IMEI of the MS.

2.7	Receiver Intermediate Frequencies

Flo - Local Oscillator frequency applied to first receiver mixer.

IF1 ... IFn - intermediate frequencies.