II.4	RECEIVER

Ref: GSM 05.05

In this section on receiver measurements, the procedures to test equipment which is fitted with an antenna connector, and the procedures to test equipment which can only be used with an integral antenna, are in general combined into one single test description. The sections which contain separate descriptions for testing these two kinds of equipment are blocking and spurious response (section II.4.7).

Tests on Mobile Stations fitted with an integral antenna and having no means of connecting an external antenna are specified in terms of received field strength. In order to test such Mobile Stations without the need for separated access to a calibrated test site an antenna coupling device is defined in General Conditions, GC6, of Annex 1 of part GC. The detailed description of its calibration is contained within section II.4.2.2. In practice this antenna coupling device may be used for transmitter measurements described in section II.3, but the calibration factors determined in II.4.2.2 will not be directly usable. The detailed calibration, when needed, for transmission tests are described in the relevant sections of II.3.

Testing philosophy

Certain assumptions concerning the functional mechanisms of GSM receivers have been made in order to define tests that will verify the receiver performance without excessive redundancy and excessive test times.

The receiver functions can be divided into:

-	Analogue RF and IF stages that are affected by input levels, temperature and power supply levels.

-	Demodulator that is affected by input levels and interfering signals.

-	Decoders that are affected by the different logical channels and input levels.

The tests are designed to stress each of these blocks with a minimum of redundancy.

Statistical testing of receiver BER/FER performance

Error Definition

1)	Frame Erasure Ratio (FER)

A frame is defined as erased if the error detection functions in the receiver, operating in accordance with GSM 05.03, indicate an error. For full rate speech this is the result of the 3 bit cyclic redundancy check (CRC) as well as other processing functions that cause a Bad Frame Indication (BFI). For signalling channels it is the result of the FIRE code or any other block code used. For data traffic FER is not defined.

2)	Residual Bit Error Ratio (RBER)

The Residual Bit Error Ratio is defined as the Bit Error Ratio (BER) in frames which have not been declared as erased.

3)	Bit Error Ratio (BER)

The Bit Error Ratio is defined as the ratio of the bits wrongly received to all data bits sent.

Test method

Each test is performed in the following manner:

a)	Set up the required test conditions. Set the parameters Max-samples (maximum number of samples) and Max-events (maximum number of error events) to specific values for each test.

b)	Perform the test and record the number of offered samples (bits or frames sent) and the number of occurred events (bit or frame errors).

c)	Terminate the test and determine the test result ("pass" or "fail") when either of the following conditions become true:

-	Number of offered samples	>=	Max-samples	--->	"PASS"

-	Number of (error) events	>	Max-events	--->	"FAIL"

In practice it may be sufficient to generate "Max-samples" and later look at Number of error events observed.

Test criteria

The limits on number of samples and events shall be defined in order to comply with different requirements :

1)	to keep reasonably low the risk of passing a bad unit through the individual tests;

2)	to have high probability of passing a good unit through the individual tests;

3)	to perform a measurement with a significant statistics;

4)	to keep the test time as low as possible.

The risk of passing a bad unit (point 1) should be kept lower than 0.2%. A unit is generally considered "bad" if its BER (or FER) performance is 1.5 times worse than that specified in AWGN (Additive White Gaussian Noise) and 1.26 times worse than that specified in multipath environment. These values have been adopted (taking into account the expected shapes of the BER performance) in order not to pass a unit with performance worse than the specifications by more than 1 dB.

The probability of passing a good unit (point 2) should be at least 99.7%.

If the error events can be assumed to be random independent variables, outputs of stationary random processes with identical Gaussian distributions, the previous figures lead to consider a number of events (point 3) not lower than 200 in AWGN channel and not lower than 600 in multipath environment, and to test a BER (or FER) performance 1.22 times worse than that specified in AWGN and 1.12 times worse than that specified in multipath environment (this corresponds to test a performance at the most 0.5 dB worse than that specified).

For multipath propagation conditions the hypothesis of stationary random processes does not generally hold. In case of non frequency hopping operation mode, the radio channel may be assumed to change 10 times per wavelength of travelled distance and to be short term stationary in between. So, in this case, the required observation time for having good statistical properties should not be lower (with some rounding) than that reported in the following table.

GSM900	DCS1800

PROPAGATION	TU3	TU50	HT100	RA250	TU1.5	TU50	HT100	RA130

CONDITIONS

MIN. TEST	500	30	15	6	500	15	7.5	6

TIME (s)

The table below details, for the different test conditions, the number of events and samples required in order to meet points 1) to 3): the corresponding test time (point 4) can be consequently computed.

As it can be seen in the table, in the cases in which both FER and RBER have to be tested on the same channel the same time as the FER measurement has been adopted. This is longer than that required for the RBER only according to the discussed criteria, but allows the relevant accuracy to be improved without increasing the total test time.

GSM900 only - begin

TABLE OF TEST CONDITIONS

PROPAGATION/	SPECIFIED	TESTED	Max No	Max No	Prob that	BAD UNIT	Risk that

Type of	Type of		Frequency	FER/BER	FER/BER	of	of	GOOD UNIT	BER/FER	BAD UNIT

test	channel		Conditions	%	%	EVENTS	SAMPLES	will pass	%	will pass

BFI	TCH/FS		STATIC		0.200	0.244	200	82000	99.719	0.300	0.139

Sensitivity	TCH/FS		STATIC/FH	0.100*α	0.122*α	200*α	164000	99.717	0.150*α	0.140

,,	TCH/FS Class Ib	STATIC/FH	0.400/α	0.410/α	82000/α	20000000	100.000	0.600/α	<0.001

,,	TCH/FS Class II	STATIC/FH	2.000	2.439	200	8200	99.714	3.000	0.001

,,	TCH/FS		TU50/No FH	6.000*α	6.742*α	600*α	8900	99.825	7.560*α	0.162

,,	TCH/FS Class Ib	TU50/No FH	0.400/α	0.420/α	4200/α	1000000	99.919	0.504/α	<0.001

,,	TCH/FS Class II	TU50/No FH	8.000	8.333	10000	120000	99.999	10.080	<0.001

,,	TCH/FS Class II	HT100/No FH	9.000	9.333	5600	60000	99.779	11.340	<0.001

,,	TCH/FS Class II	RA250/No FH	7.000	7.500	1800	24000	99.873	8.694	<0.001

,,	FACCH		TU50/No FH	13.000	14.634	600	4100	99.899	16.380	0.119

,,	TCH/F9.6&H4.8	HT100/No FH	0.700	0.778	1400	180000	99.995	0.882	<0.001

,,	TCH/F4.8		HT100/No FH	0.010	0.011	600	5350000	99.732	0.013	0.197

,,	TCH/F2.4		HT100/No FH	0.001	0.001	150	11900000	99.734	0.002	0.139

,,	TCH/H2.4		HT100/No FH	0.010	0.011	600	5350000	99.732	0.013	0.197

INPUT LEVEL	TCH/FS Class II	STATIC		0.010	0.012	200	1640000	99.716	0.015	0.141

RANGE	TCH/FS Class II	EQ50		3.000	3.250	3900	120000	100.000	3.780	<0.001

CO-CHANNEL	TCH/FS		TU3/No FH	21.000*α	24.000*α	6000*α	25000	100.000	27.720*α	<0.001

REJECTION	TCH/FS Class Ib	TU3/No FH	2.000/α	2.091/α	69000/α	3300000	100.000	2.520/α	<0.001

,,	TCH/FS Class II	TU3/NoFH	4.000	4.300	86000	2000000	100.000	5.040	<0.001

,,	TCH/FS		TU3/FH		3.000*α	3.371*α	600*α	17800	99.797	3.780*α	0.194

,,	TCH/FS Class Ib	TU3/FH		0.200/α	0.215/α	4300/α	2000000	100.000	0.252/α	<0.001

,,	TCH/FS Class II	TU3/FH		8.000	8.333	100000	1200000	100.000	10.080	<0.001

,,	FACCH		TU3/No FH	22.000	24.000	6000	25000	100.000	27.720	<0.001

ADJACENT	TCH/FS		TU50/No FH	6.000*α	6.742*α	600*α	8900	99.825	7.560*α	0.162

CHANNEL	TCH/FS Class Ib	TU50/No FH	0.400/α	0.420/α	4200/α	1000000	99.919	0.504/α	<0.001

,,	TCH/FS Class II	TU50/No FH	8.000	8.333	50000	600000	100.000	10.080	<0.001

,,	FACCH		TU3/No FH	22.000	24.000	6000	25000	100.000	27.720	<0.001

INTERMOD.	TCH/FS Class II	STATIC		2.000	2.439	200	8200	99.741	3.000	0.122

FACCH		STATIC		0.100	0.122	200	164000	99.717	0.150	0.140

BLOCKING &	TCH/FS Class II	STATIC		2.000	2.439	200	8200	99.741	4.000	<0.001

SPURIOUS	FACCH		STATIC		0.100	0.122	200	164000	99.717	0.150	0.140

RESP.

GSM900 only - end

DCS1800 only - begin

TABLE OF TEST CONDITIONS

PROPAGATION/	SPECIFIED	TESTED	Max No	Max No	Prob that	BAD UNIT	Risk that

Type of		Type of		Frequency	FER/BER	FER/BER	of	of	GOOD UNIT	BER/FER	BAD UNIT

test		channel		Conditions	%	%	EVENTS	SAMPLES	will pass	%	will pass

BFI		TCH/FS		STATIC		0.200	0.244	200	82000	99.719	0.300	0.139

Sensitivity	TCH/FS		STATIC/FH	0.100*α	0.122*α	200*α	164000	99.717	0.150*α	0.140

,,		TCH/FS Class Ib	STATIC/FH	0.400/α	0.410/α	82000/α	20000000	100.000	0.600/α	<0.001

,,		TCH/FS Class II	STATIC/FH	2.000	2.439	200	8200	99.714	3.000	0.001

,,		TCH/FS		TU50/No FH	4.000*α	4.478*α	600*α	13400	99.743	5.040*α	0.133

,,		TCH/FS Class Ib	TU50/No FH	0.300/α	0.320/α	4800/α	1500000	100.000	0.378/α	<0.001

,,		TCH/FS Class II	TU50/No FH	8.000	8.333	5000	60000	99.865	10.080	<0.001

,,		TCH/FS Class II	HT100/No FH	9.000	9.333	2800	30000	97.826	11.340	<0.001

,,		TCH/FS Class II	RA130/No FH	7.000	7.500	1800	24000	99.873	8.820	<0.001

,,		FACCH		TU50/No FH	9.000	10.084	600	5950	99.813	11.340	0.103

,,		TCH/F9.6		HT100/No FH	0.700	0.784	600	76500	99.721	0.882	0.176

,,		TCH/F4.8		HT100/No FH	0.010	0.011	600	5350000	99.732	0.013	0.197

,,		TCH/F2.4		HT100/No FH	0.001	0.001	150	11900000	99.734	0.002	0.139

INPUT LEVEL	TCH/FS Class II	STATIC-23dBm	0.500	0.610	200	32800	99.769	0.750	0.135

RANGE		TCH/FS Class II	STATIC		0.010	0.012	200	1640000	99.716	0.015	0.141

TCH/FS Class II	EQ50		3.000	3.250	1950	60000	99.981	3.780	<0.001

CO-CHANNEL	TCH/FS		TU1.5/No FH	21.00*α	24.00*α	6000*α	25000	100.000	26.460*α	<0.001

REJECTION	TCH/FS Class Ib	TU1.5/No FH	2.000/α	2.091/α	69000/α	3300000	100.000	2.520/α	<0.01

,,		TCH/FS Class II	TU1.5/No FH	4.000	4.300	86000	2000000	100.000	5.040	<0.001

,,		TCH/FS		TU1.5/FH		3.000*α	3.371*α	600*α	17800	99.797	3.780*α	0.194

,,		TCH/FS Class Ib	TU1.5/FH		0.200/α	0.215/α	4300/α	2000000	100.000	0.252/α	<0.001

,,		TCH/FS Class II	TU1.5/FH		8.000	8.333	100000	1200000	100.000	10.080	<0.001

,,		FACCH		TU1.5/No FH	22.000	24.000	6000	25000	100.000	27.720	<0.001

ADJACENT	TCH/FS		TU50/No FH	3.000*α	3.371*α	600*α	17800	99.797	3.780*α	0.194

CHANNEL	TCH/FS Class Ib	TU50/No FH	0.250/α	0.270/α	5400/α	2000000	100.000	0.315/α	<0.001

,,		TCH/FS Class II	TU50/No FH	8.100	8.333	100000	1200000	100.000	10.206	<0.001

,,		FACCH		TU1.5/No FH	22.000	24.000	6000	25000	100.000	27.720	<0.001

INTERMOD.	TCH/FS Class II	STATIC		2.000	2.439	200	8200	99.741	3.000	0.122

FACCH		STATIC		0.100	0.122	200	164000	99.717	0.150	0.140

BLOCKING &	TCH/FS Class II	STATIC		2.000	2.439	200	8200	99.741	4.000	<0.001

SPURIOUS	FACCH		STATIC		0.100	0.122	200	164000	99.717	0.150	0.140

RESP./

DCS1800 only - end

Note 1	α is a parameter which ranges from 1 to 1.6. The value of α for a RBER test on TCH/FS class Ib bits under particular measurement conditions shall be the same as that determined in the FER test on TCH/FS under the same conditions. For example, the value of α may be different for a TU50 sensitivity test and an RA250 sensitivity test. The value of α is determined by dividing the measured number of events for the FER test by the value of the maximum number of events listed in the table corresponding to α=1; if the result of the division is lower than 1, a value of α=1 shall be used. The probabilities that a good unit will pass and the risks that a bad unit will pass, listed in the table are valid for α=1, and would be slightly different for other values of α.

Note 2:	In order to save time the sensitivity test for the TCH/F2.4 channel does not comply with the above said constraints.

In fact, a bad unit which performs 2 times (instead of 1.26) worse than that specified is accounted for, so reducing the required number of events to 150, instead of 600. On the other hand, the specified RBER is in this case 10E-5 and, on the basis of simulations and hardware validation results, doubling this RBER results in a drop in performance of less than 1 dB.

II.4.1	Bad frame indication performance

ref: GSM 05.05 section 6.4

II.4.1.1	Definition

The performance of the bad frame indication (BFI) is a measure of the effectiveness of the MS under DTX conditions. It includes the effect of the 3 bit Cyclic Redundancy Check (CRC) and all other processing associated with the DTX function. The BFI is measured on a full rate speech TCH (TCH/FS) by counting the number of undetected bad frames whilst the input signal is a randomly modulated carrier described in g) below.

Note:	The DTX is used to prevent that the MS would drop the call.

II.4.1.2	Method of measurement

a)	A call shall be set up according to the generic call set up procedure on a channel with an ARFCN in the Mid ARFCN range (ref Table 1.4)Throughout the test the BCCH shall be transmitted by the SS at  28 dBµVemf(  ).

b)	The SS commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder input and at the same time to signal the bad frame indication to the SS.

c)	The SS shall simulate a BSS in DTX mode. During the period when no transmission would occur the SS shall transmit a GSM carrier modulated with random data at a level of 19 dBµVemf(  ). SACCH shall be transmitted normally at a level of 28 dBµVemf(  ). The SID frame shall be transmitted in its correct time interval with valid information at a level of 28 dBµVemf(  ). During transmission of SACCH or SID the random data shall be discontinued.

d)	The SS will proceed to transmit Max-samples of frames of TCH/FS information and will check the bad frame indication (BFI) of the looped back signal from the MS. The SS shall record the number of frames where the bad frame indication is not set. During transmission by the SS of SID the SS shall check that the BFI is not set.

NOTE:	Further explanations on the mechanism of signalling the BFI to the SS will be found in sections III.1.1 and III.1.3.

II.4.1.3	Limits

The BFI performance is accepted if the number of undetected bad frames does not exceed the Max-events:

Max-events 		-	200

Max-samples	-	82000 (excluding SID frames).

During loop back of SID frames no BFI shall be set.

II.4.2	Sensitivity

Ref: GSM 05.05 section 6.2

The reference sensitivity is the signal level at the MS receiver input at which a certain BER or FER must be achieved.

II.4.2.1	Reference Sensitivity for TCH/FS - compliance

II.4.2.1.1	Definition

Compliance with the requirements for reference sensitivity level is tested by setting the receiver at an input signal at the specification limit for reference sensitivity level and at the nominal frequency with standard test modulation, and checking that, after demodulation and decoding, a data signal with a BER or a FER less than the specified values is produced.

II.4.2.1.2	Method of measurement

a)	Set up a call according to the generic call set up procedure on a channel in the Mid ARFCN range (ref Table 1.4).

The SS shall also set up six adjacent cell BCCH's having signal strengths in the range 15 dBµVemf(  ) to 35 dBµVemf(  ). The choice of ARFCN and amplitude for these adjacent cell BCCH's shall follow the guideline given in II.3.2.1 a).

b)	The MS shall be commanded to the maximum power level.

c)	The SS traffic channel shall be the Standard Test Signal C1

GSM900:	(see GSM 11.40)

DCS1800:	(see GSM 11.40-DCS).

at a MS level of 28 dBµVemf(  ).

d)	The SS commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder's input and at the same time to signal the frame erasure event to the SS. The fading function of the SS is set to TU50.

e)	The amplitude of the wanted signal shall be set to

GSM900:	11 dBµVemf(  ) for hand portable MSs and to 9 dBµVemf(  ) for other MSs.

DCS1800:	13 dBµVemf(  )

f)	The SS compares the data of the signal that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication. Also throughout the test, the SS monitors the status message from the MS to confirm correct reporting of the adjacent cell BCCH's.

g)	The SS determines the number of residual bit error events for the bits of class II, by examining sequences of Max-samples of consecutive bits of class II, where bits are taken only from those frames for which no bad frame indication was given.

h)	The SS determines the number of residual bit error events for the bits of the class Ib, by examining sequences of Max-samples of consecutive bits of class Ib, where bits are taken from those frames for which no bad frame indication was given.

i)	The SS also determines the frame erasure events by examining sequences of Max-samples of consecutive frames and assuming a frame is received successfully when there is no bad frame indication concerning it.

j)	Steps a) up to including g) are repeated under extreme test conditions (Annex 1, TC2.2 and TC3)

k)	Steps a) to j) shall be repeated for  channels with ARFCN in the Low ARFCN range and the High ARFCN range (ref Table 1.4).

l)	Steps a) to g) shall be repeated, except under step d) the SS fading function shall be set in turn to

GSM900:	RA250 and HT100

DCS1800:	RA130 and HT100.

m)	Steps a) to j) shall be repeated, except at step d) the SS fading function shall be set to static and the MS shall be commanded by the SS into hopping mode using the hopping sequence of GC1 of Annex 1, full band.

The amplitude of the wanted signal shall be set as in step e). All the other time slots, except the active ones, shall be set to 28 dBµVemf(  ). This will implicitly test adjacent time slot rejection.

II.4.2.1.3	Limits

The Max-events measured for different channels and under the different propagation conditions, under any combination of normal and extreme test voltages and ambient temperatures, shall not exceed the values given in the table below:

GSM900 only - begin

Propagation		Propagation		Propagation		Static

Conditions		Conditions		Conditions		Conditions

Channels		TU50			RA250			HT100

TCH/FS	Max-		Max-	Max-		Max-	Max-		Max-	Max-		Max-

Events		samples	events		samples	events		samples	events		samples

FER		600*α		8900							200*α		164000

class Ib(RBER)	4200/α		1000000							82000/α		20000000

class II(RBER)	10000		120000	1800		24000	5600		60000	200		8200

where α is a parameter which can range from 1 to 1.6. The value of α for a RBER test on TCH/FS class Ib bits under particular measurement conditions shall be the same as that determined in the FER test on TCH/FS under the same conditions.

GSM900 only - end

DCS1800 only  -begin

Propagation		Propagation		Propagation		Static

Conditions		Conditions		Conditions		Conditions

Channels		TU50			RA250			HT100

TCH/FS	Max-		Max-	Max-		Max-	Max-		Max-	Max-		Max-

Events		samples	events		samples	events		samples	events		samples

FER		600*α		13400							200*α		164000

class Ib(RBER)	4800/α		1500000							82000/α		20000000

class II(RBER)	5000		60000	2800		30000	5200		60000	200		8200

where α is a parameter which can range from 1 to 1.6. The value of α for a RBER test on TCH/FS class Ib bits under particular measurement conditions shall be the same as that determined in the FER test on TCH/FS under the same conditions.

DCS1800 - end

II.4.2.2	Sensitivity Measurements for Equipment without an antenna connector

For equipments fitted with an integral antenna and not provided with means for connection to an external antenna an initial procedure is required before sensitivity measurements can be performed. This procedure consists of calibrating an antenna coupling device. This antenna coupling device is a test fixture, which, when mated with the MS forms a single unit enabling the MS to be tested as if it were a MS with an antenna connector. Once calibrated this antenna coupling device enables all receiver test procedures, except blocking to be identical for equipments with an integral antenna and for equipments with an antenna connector.

The calibration procedure shall be carried out at three frequencies, namely an ARFCN out of each of the possible ranges, Low, Mid and High (ref Table 1.4)

The procedure consists of three distinct stages as follows:

1)	Establish the MS antenna radiation pattern for the three selected frequencies.

2)	Calibrate the test range (or anechoic shielded chamber) for the conditions needed in 1).

3)	Determine the antenna coupling device coupling factor.

II.4.2.2.1	Antenna Radiation Pattern

a)	The MS shall be in the anechoic shielded chamber, or on an outdoor test site, on an isolated support in a vertical position at an orientation specified by the manufacturer. This position is the 0 degree position.

A test antenna, connected to the SS shall be in the anechoic shielded chamber, or on the outdoor test site, at a distance of at least 3 metres from the MS.

b)	A call shall be set up according to the generic call set up procedure with ARFCN in the Low ARFCN range.

The SS shall command the MS to maximum transmit power.

c)	The SS shall, using estimated parameters for the outdoor test site or anechoic shielded chamber, set its output level 'E' (see Fig. II.4-1) to give an MS receiver input level of approximately 32 dBµVemf(  ). This corresponds to a field strength at the MS position of

GSM900:	55.5 dBµV/m

DCS1800:	61.3 dBµV/m.

The signal shall be the Standard Test Signal C1.

Note:	The absolute value of the received signal level is not critical. The value suggested however will ensure that the MS receiver is operating essentially error free, yet is low enough to avoid any non linear effects in the receiver.

d)	The SS shall use the RXLEV message from the MS to determine a measure of the received field strength. The procedure detailed in the flow chart of figure II.4-1 shall now be followed. The signal level from the SS that just results in the transition

GSM900:	from RXLEVa to RXLEVb shall be recorded as Ei

DCS1800: between two values of the parameter RXLEV, defined as RXLEVa and RXLEVb shall be recorded as Ei.

Note:	The actual values of RXLEVa and RXLEVb will need to be recorded, because this transition will be used as the reference point for all further stages of the calibration procedure.

e)	Step d) shall be repeated after the MS has been rotated by n * 45 degrees in the horizontal plane

GSM900:	left intentionally blank

DCS1800:	for all values of n in the range 1-7.

Ensuring that the same RXLEV transition is used, the signal levels from the SS shall be recorded as Ein.

f)	Calculate the effective mean signal level from the RMS value of the eight signal levels obtained in d) and e) above by using the following formula:

┌      8      ┐1/2

│ __________  │

E1  =  │  n=7   1    │

│   Σ  ────   │

└  n=0  Ein²  ┘

g)	Repeat steps b) to f), except in step b) use an ARFCN in the Mid ARFCN range (ref Table 1.4), to obtain a mean signal level E2. Ensure the same RXLEV transition is used.

h)	Repeat steps b) to f), except in step b) use an ARFCN in the High ARFCN range (ref Table 1.4), to obtain a mean signal level E3.

Ensure the same RXLEV transition is used.

FIG II.4-1

II.4.2.2.2	Test Range Calibration

The objective of this step is to determine the actual field strength at the MS corresponding to the three signal levels E1, E2 and E3 established in II.4.2.2.1. The following procedure shall be used:

a)	Replace the MS by a calibrated reception antenna connected to a measuring receiver.

b)	For each frequency used in II.4.2.2.1 measure the field strength Efr corresponding to the respective signal levels Er determined in steps f), g) and h) of II.4.2.2.1 record these values as Ef1, Ef2 and Ef3.

II.4.2.2.3	Antenna coupling Device Coupling Factor

The coupling factor of the antenna coupling device is the relationship expressed in dB, between the output signal of the SS and the effective receiver input signal for the MS. The antenna gain is assumed to be 0 dBi. The procedure shall be as follows:

a)	A call shall be set up according to the generic call set up procedure on a channel with ARFCN in the Low ARFCN range (ref Table 1.4).

The SS shall command the MS to maximum transmit power.

b)	The SS shall, using the procedures of II.4.2.2.1, adjust its output signal level to determine the RXLEVa to RXLEVb transition. This signal level shall be recorded as Ec1.

c)	Repeat steps b) and c) for frequencies in the Mid and High ARFCN ranges (ref Table 1.4).

Record the RXLEV transitions as Ec2 and Ec3 respectively.

d)	The antenna coupling device coupling factor F is then calculated from:

┌   Ecn    ┐

Fn = 20 log10 │  ─────── │

└  Efn*Kn  ┘

where Kn =	conversion factor of an isotropic antenna

expressed as  (µVemf) / (µV/m)  at the frequency

corresponding to the ARFCN used.

e)	The mean antenna coupling factor Fm to be used for measurements requiring hopping shall be calculated from the RMS value of all parameters in e) as follows :

┌             3           ┐1/2

Ecm  =  │  ────────────────────   │

└ 1/Ec1² + 1/Ec2² + 1/Ec3²┘

┌             3           ┐1/2

Efm  =  │  ────────────────────   │

└ 1/Ef1² + 1/Ef2² + 1/Ef3²┘

┌  k1² + k2² + k3²  ┐1/2

km   =  │  ─────────────    │

└         3         ┘

┌   Ecm    ┐

Fm   =  20 log10 │  ─────── │

└  Efm*km  ┘

f)	In all tests in which a handheld MS with integral antenna is the unit under test, the input signal level into the coupling device is determined from :

Ein 	  =	Ereq + F

where:	Ein =	input signal level to coupling device (dBµVemf)

Ereq =	signal level required by the test (dBµVemf)

F =	coupling factor at the respective ARFCN (dB)

This is indicated in the test procedures as Ereq, dBµVemf(  ), where the empty parenthesis is to be read as Ein.

II.4.2.3	Sensitivity for data channels and control channels

II.4.2.3.1	Definition

Compliance with the requirements for reference sensitivity on control channels is tested by setting the receiver at an input signal at the specification limit for reference sensitivity and at the nominal frequency with standard test modulation, and checking that, after demodulation and decoding, a FER less than the specified value be produced. For data channels, reference sensitivity is defined in terms of BER.

II.4.2.3.2	Method of measurement

a)	A call is set up according to the generic call set up procedure on a channel having an ARFCN in the Mid ARFCN range (ref Table 1.4).

b)	The MS is commanded to its maximum power level.

c)	The SS output on the traffic channel is set to 28 dBµVemf(  ), producing Standard Test Signal C1.

d)	The fading function of the SS is set to TU50.

e)	The wanted signal is set to a level of

GSM900:	11 dBµVemf(  ) for hand held MS or 9 dBµVemf(  ) for any other MS

DCS1800:	13 dBµVemf(  ).

f)	The SS sends the STATUS message. Due to the low signal level the MS may not be able to acknowledge the Layer 2 frame with an RR frame and the SS will repeat the Layer 2 frame. Each repeated L2 frame will be counted and will indicate a frame erasure event.

g)	The SS determines the frame erasure events during Max-samples of FACCH frames.

h)	The wanted signal is set to

GSM900:	28 dBµVemf(  )

DCS1800:	30 dBµVemf(  ).

and the SS commands the MS to a TCH/F9.6 channel. It also commands the MS to close its TCH loop, specifying that erased frames are to be signalled by the MS (see section III.1.2.1.1.1) in order to loop back received data from the channel decoder via the channel encoder to the uplink TCH.

i)	Set the SS fading function to HT100 and the wanted signal to

GSM900:	11 dBµVemf(  ) for a hand portable MS or 9dBµVemf(  ) for any other MS

DCS1800:	13 dBµVemf(  ).

j)	The SS compares transmitted data with received data for Max-samples consecutive bits and records every error bit as an error event.

k)	Steps h) to j) are repeated for data channels of

GSM900:	TCH/H4.8, TCH/F4.8, TCH/F2.4 and TCH/H2.4

DCS1800:	TCH/F4.8 and TCH/F2.4.

Note 1:	Not all Mobile Stations will be equipped with all data channels. The test procedure will need to be adapted to cater for limited subsets for Mobile Stations offering a restricted data capability.

Note 2:	The SACCH, SDCCH, AGCH and PCH channels are not tested because the coding is identical to the FACCH. These channels are tested implicitly in layer 3.

II.4.2.3.3	Limits

DCS1800 only - begin

The Max-events measured for different channels and under the different propagation conditions shall not exceed the values given in the table below:

DCS1800 only - end

GSM900	DCS1800

Channels	Type of	Propag-	Max-	Max-	Max-	Max-

measure-	ation	events	samples	events	samples

ments

FACCH	FER	TU50	600	4100	600	6700

TCH/F9.6	BER	HT100	1400	180000	600	76500

TCH/H4.8	BER	HT100	1400	180000	-	-

TCH/F4.8	BER	HT100	600	5350000	600	5350000

TCH/F2.4	BER	HT100	150	11900000	150	11900000

TCH/H2.4	BER	HT100	600	5350000	-	-

II.4.3	Usable receiver input level range

ref: GSM 05.05 section 6.1

II.4.3.1	Definition

The usable receiver input level range is the range of the radio frequency input level of a specified modulated signal over which bit error ratio or frame erasure ratios stay between specified limits.

II.4.3.2	Method of measurement

a)	A call shall be set up according to the generic call set up procedure on a channel having an ARFCN in the Mid ARFCN range (ref Table 1.4).

b)	The SS shall be set on the TCH/FS to produce Standard Test Signal C1 at a level of 28 dBµVemf(  ).

c)	The SS commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder's input without signalling the frame erasure event (III.1.2.1.1.2).

d)	The SS compares the data that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding.

The SS tests the bit error ratio for the non-protected bits of TCH/FS class II, by examining sequences of Max-samples consecutive bits of class II. The number of error events is recorded.

e)	Step d) shall be repeated with the amplitude of the wanted signal increased to input levels at the receiver input of respectively

GSM900:	63 dBµVemf(  ) and 103 dBµVemf(  )

DCS1800:	73 dBµVemf(  ).

DCS1800 only - begin

e1)	Step d) shall be repeated with the amplitude of the wanted signal increased to an input level at the receiver input of 90 dBµVemf(  ).

DCS1800 only - end

f)	The SS fading function is set to EQ50.

g)	Step d) shall be repeated with the amplitude of the wanted signal set to respectively 28 dBµVemf(  ) and 73dBµVemf(  ) at the receiver input.

h)	Steps a) to g) are repeated under extreme test conditions (Annex 1, TC2.2 and TC3).

II.4.3.3	Limits

The number of error events recorded in this test shall not exceed the Max-events values given in the table below when Number of samples = Max-samples. This shall apply for any combination of normal and extreme test voltages and ambient temperature, for the different propagation conditions and for any level of input signal to the receiver.

GSM900	DCS1800

Propagation	Max-	Max-	Max-	Max-

conditions	events	samples	events	samples

Static step e)	200	1640000	200	1640000

Static step e1)	-	-	200	32800

EQ50	3900	120000	1950	60000

II.4.4	Co-channel rejection

II.4.4.1	Definition

The co-channel rejection is a measure of the capability of the receiver to receive a wanted modulated signal without exceeding a given degradation due to the presence of an unwanted modulated signal, both signals being at the nominal frequency of the receiver.

II.4.4.2	Method of Measurement

a)	A call shall be set up according to the generic call set up procedure on a channel with ARFCN in the Mid ARFCN range (ref Table 1.4)

b)	The wanted signal shall be the Standard Test Signal C1. It shall be at the nominal frequency of the receiver and at a level of 28 dBµVemf(  ).

The unwanted signal shall have no fixed relationship with the bit transitions of the wanted signal, and it shall be modulated with random data. Its amplitude shall be 9 dB below that of the wanted signal. The unwanted signal shall be continuous.

c)	The SS commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder's input and at the same time to signal the frame erasure event to the SS. (ref III.1.2.1.1.1)

d)	The SS is set to produce a wanted signal and an independent, uncorrelated interfering (unwanted) signal at the same time. The fading characteristic of the wanted and the interfering signal shall be

GSM900:	TU3, on a channel frequency in the range 60 to 65 (ARFCN)

DCS1800:	TU1.5, on a channel frequency in the range 690 to 710 (ARFCN).

e)	The SS compares the modulation of the signal that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication.

The SS tests the frame erasure ratio compliance for the TCH/FS, by examining Max-samples consecutive frames of TCH/FS information where a frame is assumed to be received successfully if there is no bad frame indication. The number of frame erasure events is recorded. During the Max-samples of the frame erasure measurement, the RBER of the class II and class Ib bits are also determined.

f)	The SS commands the MS to open the TCH loop (ref III.1.2.1.1.1)

g)	The SS sends the STATUS message. Due to the co-channel interference, the MS may not be able to acknowledge the Layer 2 frame. Each repeated L2 frame indicates a frame erasure event. The SS determines the number of frame erasure events during Max-samples of consecutive FACCH frames.

h)	Steps c) to e) shall be repeated except that in step d), both the wanted and interfering signal shall be hopping and the SS shall command the MS into hopping mode with a hop pattern covering at least

GSM900:	10 frequencies in a range not exceeding 5 MHz

DCS1800:	[10] frequencies in a range not exceeding [5] MHz.

The hopping band shall be centred around an ARFCN in the Mid ARFCN range (ref Table 1.4)according to the narrow band hop of GC1 of Annex 1.

DCS1800 only - begin

Note: The frequency range of this test is dependent on the fading simulator.

DCS1800 only - end

II.4.4.3	Limits

The number of events recorded in this test shall not exceed the Max-events values given in the table below when the Number of samples = Max-samples.

GSM900	DCS1800

Channel	Propagation	Propagation	Type of	Max-	Max-

Conditions	Conditions	measurement	events	samples

FACCH	TU3/No FH	TU1.5/No FH	FER	6000	25000

TCH/FS	TU3/No FH	TU1.5/No FH	FER	6000*α	25000

TCH/FS	TU3/No FH	TU1.5/No FH	RBER	69000/α	3300000

Class Ib

TCH/FS	TU3/No FH	TU1.5/No FH	RBER	86000	2000000

Class II

TCH/FS	TU3/FH	TU1.5/FH	FER	600*α	17800

TCH/FS	TU3/FH	TU1.5/FH	RBER	4300/α	2000000

class Ib

TCH/FS	TU3/FH	TU1.5/FH	RBER	100000	1200000

class II

The parameter α can range from 1 to 1.6. The value of α for the RBER test on TCH/FS class Ib bits under particular measurement conditions shall be the same as that determined in the FER test on TCH/FS under the same conditions.

II.4.5	Adjacent channel rejection

ref: GSM 05.05 section 6.3.2

II.4.5.1	Definition

The adjacent channel selectivity is a measure of the capability of the receiver to receive a wanted modulated signal without exceeding a given degradation due to the presence of an unwanted signal in the adjacent channel.

The adjacent channel can be the adjacent in the RF spectrum or in time. There are therefore two types of adjacent channel selectivity:

1)	Adjacent RF channel selectivity which is specifically tested in this section.

2)	Adjacent Time Slot selectivity, which is implicitly tested in test II.4.2.1.

II.4.5.2	Method of Measurement

For a MS supporting speech step e) is not required.

For a data only MS, steps b) and d) are not required.

a)	A call shall be set up according to the generic call set up procedure. For MS's supporting speech a TCH/FS call shall be used, otherwise a data bearer declared in section 2.1.7 of the PIXIT shall be used.

b)	For MS's supporting speech the System Simulator commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder's input and at the same time to signal the frame erasure event to the SS.

c)	The SS is set to produce a wanted Standard Test Signal C1 and an independent, uncorrelated interfering (unwanted) Standard Test Signal I1 at the same time. For MS's supporting speech the fading characteristic of the wanted and the interfering signal is TU50, for all other MS's the fading characteristic of the wanted and the interfering signal is

GSM900:	TU3

DSC1800:	TU1.5.

The channel frequency shall be in the Mid ARFCN range (ref Table 1.4).

The wanted signal shall be the Standard Test Signal C1. It shall be at the nominal frequency of the receiver and at a level of 28 dBµVemf(  ).

The unwanted signal shall have no fixed relationship with the bit transition of the wanted signal and it shall be modulated with random data. Its amplitude shall be 9 dB above that of the wanted signal. The unwanted signal shall be continuous, with a nominal frequency 200 kHz above the nominal frequency of the wanted signal.

d)	For MS's supproting speech the SS compares the data of the signal that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication.

The SS tests the frame erasure compliance for the TCH/FS by examining Max-samples of consecutive frames. The number of frame erasure events is recorded. During Max-samples of the frame erasure measurement, the RBER of the class II and class Ib bits are also determined for speech bearers.

e)	The SS sends the STATUS message. Due to the adjacent channel interference, the MS may not be able to acknowledge the Layer 2 frame. Each repeated L2 frame indicates a frame erasure event. The SS determines the number of frame erasure events during Max-samples of consecutive FACCH frames.

f)	The measurement of steps c) and d) or e), whichever is applicable, shall be repeated with the unwanted signal on a frequency at the same displacement from, but below, the frequency of the wanted signal.

g)	The measurement of steps c) to f) shall be repeated for a displacement of the unwanted signal of 400 kHz, and with the amplitude of the unwanted signal 41 dB above the level of the wanted input signal.

h)	Steps c) to g) are repeated under extreme test conditions (Annex 1, TC2.2 and TC3).

II.4.5.3	Limits

GSM900	DCS1800

Channel	Type of 	Max-	Max-	Max-	Max-

measurement	events	samples	events	samples

TCH/FS	FER	600*α	8900	600	17800

class Ib	RBER	4200/α	1000000	5400	2000000

class II	RBER	50000	600000	100000	1200000

FACCH	FER	6000	25000	6000	25000

The number of events recorded in this test shall not exceed the Max-events given in the table when the Number of samples = Max-samples. This shall apply for any combination of normal and extreme test voltages and ambient temperature, and with the interfering signals at either side of the wanted frequency.

The parameter α can range from 1 to 1.6. The value of α for the RBER test on TCH/FS class Ib bits under particular measurement conditions shall be the same as that determined in the FER test on TCH/FS under the same conditions.

II.4.6	Intermodulation rejection

ref: GSM 05.05 section 5.2

II.4.6.1	Definition

The intermodulation rejection is a measure of the capability of the receiver to receive a wanted modulated signal without exceeding a given degradation due to the presence of two or more unwanted signals with a specific frequency relationship to the wanted signal frequency.

II.4.6.2	Method of measurement

NOTE:	The measurements address the third order intermodulation, which represents the most serious case.

The compliance with the requirements for the bit error ratio of TCH/FS class II bits and FER of FACCH is checked in order to verify the receiver's intermodulation rejection under normal and extreme test conditions.

For a MS supporting speech, step e2) is not required (FACCH not tested)

For a data only MS, steps c) and e1) are not required.

a)	A call shall be set up according to the generic call set up procedure on a TCH with ARFCN in the Mid ARFCN range (ref Table 1.4).

b)	The wanted signal shall be the Standard Test Signal C1. The wanted signal shall be at the nominal frequency of the receiver. The level of the wanted signal shall correspond to 4 dB above the reference sensitivity level (see Table II.4.6-1).

c)	The SS commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder input and at the same time to signal the frame erasure event to the SS.

d)	The SS is set to produce a static wanted signal, and two static interfering (unwanted) signals at the same time. There shall be no correlation in the modulation between the signals.

The first interfering signal shall be on a frequency equal to the centre frequency of an ARFCN four above that of the receiver. This signal shall be static and shall be unmodulated.

The second interfering signal shall be on an ARFCN eight above that of the receiver. This signal shall be static, continuous and shall be modulated by random data.

The amplitude of both the interfering signals shall be set according to Table II.4.6-1.

e1)	The SS compares the data of the signal that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication.

The SS tests the RBER compliance of class II bits by examining Max-samples of consecutive bits, with bits only taken from those frames which do not signal frame erasure. The number of error events are recorded.

e2)	The SS sends the STATUS message. The SS determines the number of frame erasure events during Max-samples of consecutive FACCH frames.

f)	The measurement of step e1) or e2, whichever is applicable, shall be repeated with the two unwanted signals having frequencies corresponding to ARFCN four and eight below the ARFCN of the

GSM900:	unwanted signal

DCS1800:	wanted signal.

g)	Steps a) to f), as applicable, shall be repeated but with the receiver operating on an ARFCN in the Low ARFCN range (ref Table 1.4).

h)	Steps a) to f), as applicable, shall be repeated but with the receiver operating on an ARFCN in the High ARFCN range (ref Table 1.4).

i)	Steps a) to h) are repeated under extreme test conditions (Annex 1, TC2.2 and TC3).

GSM900	DCS1800

MS CLASS	1 and 2	3, 4 and 5	1 and 2

MS HANDHELD	NO	YES	YES

WANTED SIGNAL	13	15	17

dBµVemf(  )

FIRST INTERFERER	74	64	64

dBµVemf(  )

SECOND INTERFERER	63	63	64

dBµVemf(  )

Table II.4.6-1: Intermodulation test signal levels

GSM900 only - begin

Note:	The levels in Table II.4.6-1 are different to those specified in GSM 05.05 due to the consideration of the effect of modulation sideband noise from the second interferer.

GSM900 only - end

II.4.6.3	Limits

The Number of error events recorded in this test shall not exceed the Max-events values given below when the Number of samples = Max-samples.

This shall apply under normal condition and under any combination of normal and extreme test voltages and ambient temperature, and with the two interfering signals at either side of the wanted frequency.

Channel	Propagation	Type of	Max-events	Max-samples

conditions	measurement

TCH/FS	Static	RBER	200	8200

Class II

FACCH	Static	FER	200	164000

II.4.7	Blocking and Spurious Response

ref: GSM 05.05 section 5.1

II.4.7.1	Definition

Blocking is a measure of the ability of the receiver to receive a modulated wanted input signal in the presence of an unwanted input signal, on frequencies other than those of the spurious responses or the adjacent channels, without exceeding a given degradation.

The method of test aligns with ETS 300 086.

For MS not supporting speech, step c1) and h1) are not required. For all other MS, steps c2) and h2) are not required.

II.4.7.2	Method of measurement - equipment with antenna connector

a)	A call shall be set up according to the generic call set up procedure.

b)	The wanted signal shall be the Standard Test Signal C1. The wanted signal shall be at the nominal frequency of the receiver.

c1)	The SS commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder's input and at the same time to signal the frame erasure event to the SS.

c2)	The SS sends the STATUS message. Due to interfering signals, the MS may not be able to acknowledge the Layer 2 frame. Frame erasures are indicated by repeated L2 frames.

d)	The SS is set to produce a static wanted signal and a static interfering signal at the same time. The level of the wanted signal shall be

GSM900:	15 dBµVemf(  ) for hand portable MSs

or 13 dBµVemf(  )for all other MSs

DCS1800:	17 dBµVemf(  ).

This corresponds to 4 dB above the reference sensitivity level.

e)	The unwanted signal shall be a C.W. signal (Standard test signal IO) of frequency FB. It shall in turn be applied on the subset of frequencies calculated at step f) in the overall range 100 kHz to 12.75 GHz, where FB is an integer multiple of 200 kHz.

Frequencies in the range FR +/- 600 kHz shall, however, be excluded.

Note:	Allowance must be made for possible spurious signals arising from the SS. These are particularly likely at subharmonic frequencies nFB where n = 2, 3, 4, 5, etc.

f)	The frequencies at which the test shall be performed (adjusted to an integer multiple of 200 kHz channels most closely approximating the absolute frequency of the calculated blocking signal frequency) are the combined frequencies from i), ii) and iii) below :-

i)	The total frequency range formed by :-

the frequencies between Flo + (IF1 + IF2 + ... +IFn +12.5 MHz)

and Flo - (IF1 + IF2 + ... + IFn + 12.5 MHz).

and

the frequencies + 100 MHz and - 100 MHz from the edge of the relevant receive band.

Measurement shall be made at 200 kHz intervals.

ii)	The three frequencies IF1, IF1 + 200 kHz, IF1 - 200 kHz.

iii)	The frequencies mFlo + IF1, mFlo - IF1, where m is all positive integers greater than or equal to 2 such that either sum lies in the range 100 kHz to 12.75 GHz.

The frequencies in step ii) and iii) lying in the range of frequencies defined by step i) above need not be repeated.

iv)	Outside the range defined in i) above, spot frequencies shall be measured at 10 MHz from the range edge and repeated at 10 MHz intervals from 100 kHz to 12.75 GHz.

Where :-

Flo -		local oscillator applied to first receiver mixer

IF1 ... IFn -	are the n intermediate frequencies

Flo, IF1, IF2 ... IFn shall be declared by the manufacturer in the PIXIT statement GSM 11.10 Annex 3.

g)	The level of the unwanted signal shall be set according to the following table.

GSM900	DCS1800

MS POWER CLASS:	3, 4 and 5	1 and 2	1 and 2

FREQUENCY	LEVEL IN dBµVemf(  )

FR +/- 600 kHz to FR +/- 800 kHz	70	75	70

FR +/- 800 kHz to FR +/- 1.6 MHz	70	80	70

FR +/- 1.6 MHz to FR +/- 3 MHz	80	90	80

915 MHz to FR - 3 MHz	90	90	-

FR + 3 MHz to 980 MHz	90	90	-

1785 MHz to FR - 3 MHz	-	-	87

FR + 3 MHz to 1920 MHz	-	-	87

835 MHz to 915 MHz	113	113

980 MHz to 1000 MHz	113	113

100 kHz to 835 MHz	90	90

1000 MHz to 12.75 GHz	90	90

100 kHz to 1705 MHz	-	-	113

1705 MHz to 1785 MHz	-	-	101

1920 MHz to 1980 MHz	-	-	101

1980 MHz to 12.75 GHz	-	-	113

Note:	These values differ from GSM 05.05 because of practical generator limits in the SS.

h1)	The SS compares the data of the signal that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication.

The SS tests the RBER compliance for the bits of class II, by examining sequences of Max-samples consecutive bits of class II, where bits are taken only from those frames for which no bad frame indication was given.  The number of error events is recorded.

If a failure is indicated it shall be noted and counted towards the allowed exemption totals.

In the case of failures discovered at the predicted frequencies at steps f ii), iii) or iv) the test shall be repeated on the adjacent channels +/- 200 kHz away. If either of these two frequencies fail then the next channel 200 kHz beyond shall also be tested. This process shall be repeated until all channels constituting the group of failures is known.

h2)	The SS determines the number of frame erasure events during Max-samples. If a failure is indicated, it should be noted and counted towards the allowed exemption total.

In the case of failures discovered at the predicted frequencies at steps f ii), iii) or iv) the test shall be repeated on the adjacent channels +/- 200 kHz away. If either of these two frequencies fail then the next channel 200 kHz beyond shall also be tested. This process shall be repeated until all channels constituting the group of failures is known.

i)	Steps e) to h1) or h2), whichever is applicable, shall be repeated, for the MS operating on an ARFCN in the Low ARFCN range.

j)	Steps e) to h1) or h2), whichever is appliacbel, shall be repeated, for the MS operating on an ARFCN in the High ARFCN range.

II.4.7.3	Method of measurement for equipment with integral antenna

General

If the test below is carried out on an outdoor test site, the necessary precautions should be taken to prevent the signals in the frequency range 30 MHz to 4 GHz from causing harmful interference to services that might be in operation in the immediate vicinity.

For MS not supporting speech, steps g1) and l1) are not required. For all other MS, steps g2) and l2) are not required.

Method of measurement

a)	In the anechoic shielded chamber or on the test site a calibrated measuring antenna (vertically polarized) shall be placed, connected to a measuring receiver.

b)	A test antenna, vertically polarized, connected to the System Simulator shall be in the anechoic shielded chamber or on the test site at a distance of at least 3 metres from the MS.

c)	At the position of the measuring antenna, a wanted signal is established with a field strength of

GSM900:	38.5 dBµV/m

DCS1800:	46.3 dBµV/m.

Note:	This field strength is 4dB above reference sensitivity.

d)	The measuring antenna is replaced by the MS, placed on a non conductive support, in the position for normal use.

Note:	The MS is at this point in the procedure exposed to a wanted signal, at a field strength 4 dB in excess of the the reference sensitivity value.

e)	A call shall be set up according to the generic call set up procedure on a channel with an ARFCN in the Mid ARFCN range.

f)	The wanted signal shall be the Standard Test Signal C1. The wanted signal FR all be static and at the nominal frequency of the receiver.

g1)	The SS commands the MS to create the loop back facility from the receiver's speech channel decoder output to the transmitter's encoder input and at the same time to signal the frame erasure event to the SS.

g2)	The SS sends the STATUS message. Due to interfering signals, the MS may not be able to acknowledge the Layer 2 frame. Frame erasure events are indicated by repeated L2 frames.

h)	The SS is set to produce a static wanted signal and a static interfering signal at the same time. The interfering signal shall be available from a separate output port on the SS. The interfering signal shall be fed to an antenna, selected in accordance with the frequency of the interfering signal. This antenna, radiating the interfering signal shall be in the anechoic shielded chamber or on the test site at a distance of at least 3 metres from the MS, except at frequencies above 900 MHz when a distance of 1 metre is acceptable.

i)	The unwanted signal shall be a C.W. signal (Standard test signal I0) of frequency FB. It shall in turn be applied on the subset of frequencies calculated at step j) in the overall range 80 kHz to 4 GHz, where FB is an integer multiple of 200 kHz.

Frequencies in the range FR +/- 600 kHz shall, however, be excluded.

Note:	Allowance must be made for possible spurious signals arising from the SS. These are particularly likely at subharmonic frequencies nFB where n = 2, 3, 4, 5 etc.

j)	The frequencies at which the test shall be performed (adjusted to an integer multiple of 200 kHz channels most closely approximating the absolute frequency of the calculated blocking signal frequency) are the combined frequencies from i), ii) and iii) below :-

i)	The total frequency range formed by :-

the frequencies between Flo + (IF1 + IF2 + ... +IFn +12.5 MHz)

and Flo - (IF1 + IF2 + ... + IFn + 12.5 MHz).

and

the frequencies + 100 MHz and - 100 MHz from the edge of the relevant receive band.

Measurement shall be made at 200 kHz intervals.

ii)	The three frequencies IF1, IF1 + 200 kHz, IF1 - 200 kHz.

iii)	The frequencies mFlo + IF1, mFlo - IF1, where m is all positive integers greater than or equal to 2 such that either sum lies in the range 80 kHz to 4 GHz.

The frequencies in step ii) and iii) lying in the range of frequencies defined by step i) above need not be repeated.

iv)	Outside the range defined in i) above, spot frequencies shall be measured at 10 MHz from the range edge and repeated at 10 MHz intervals from 80 kHz to 4 GHz.

Where :-

Flo - local oscillator applied to first receiver mixer

IF1 ... IFn - are the n intermediate frequencies

Flo, IF1, IF2 ... IFn shall be declared by the manufacturer in the PIXIT statement GSM 11.10 Annex 3.

k)	The level of the unwanted signal shall be set according to the table below.

The required field strength of the unwanted signal will be set to the values given in the table below.

GSM900	DCS1800

Frequency range	Absolute Field Strength in dBµV/m

80 MHz to  200 MHz	98.5	98.5

200 MHz to  500 MHz	101.5	101.5

500 MHz to  835 MHz	113.5	113.5

835 MHz to  915 MHz	136.5	-

835 MHz to 1705 MHz	-	142.3

1705 MHz to 1785 MHz	-	130.3

915 MHz to FR - 3 MHz	113.5	-

FR +/- 3 MHz to FR +/- 1.6 MHz	103.5	-

FR +/- 1.6 MHz to FR +/- 800 kHz	93.5	-

FR +/- 800 kHz to FR +/- 600 kHz	93.5	-

FR + 3 MHz to 980 MHz	113.5	-

1785 MHz to FR - 3 MHz	-	116.3

FR +/- 3 MHz to FR +/- 1.6 MHz	-	109.3

FR +/- 1.6 MHz to FR +/- 800 kHz	-	99.3

FR +/- 800 kHz to FR +/- 600 kHz	-	99.3

FR + 3 MHz to 1920 MHz	-	116.3

980 MHz to 1000 MHz	136.5	-

1000 MHz to 4000 MHz	104.5	-

1920 MHz to 1980 MHz	-	130.3

1980 MHz to 2400 MHz	-	142.3

2400 MHz to 4000 MHz	-	110.3

l1)	The SS compares the data of the signal that it sends to the MS with the data which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication. The SS tests the RBER compliance for the bits of class II, by examining sequences of Max-samples consecutive bits of class II, where bits are taken only from those frames for which no bad frame indication was given. The number of error events shall be recorded.

If a failure is indicated it shall be noted and counted towards the allowed exemption totals.

In the case of failures discovered at the predicted frequencies at steps j ii), iii) or iv) the test shall be repeated on the adjacent channels +/- 200 kHz away. If either of these two frequencies fail then the next channel 200 kHz beyond shall also be tested. This process shall be repeated until all channels constituting the group of failures is known.

l2)	The SS determines the number of frame erasure events during Max-samples. If a failure is indicated, it should be noted and counted towards the allowed exemption total.

In the case of failures discovered at the predicted frequencies at steps j ii), iii) or iv) the test shall be repeated on the adjacent channels +/- 200 kHz away. If either of these two frequencies fail then the next channel 200 kHz beyond shall also be tested. This process shall be repeated until all channels constituting the group of failures is known.

m)	Steps i) to l1) or l2), whichever is applicable, shall be repeated with the MS operating on an ARFCN in the Low ARFCN range.

n)	Steps i) to l1) or l2), whichever is applicable, shall be repeated with the MS operating on an ARFCN in the High ARFCN range.

II.4.7.4	Limits

The number of error events recorded in this test shall not exceed the Max-events values given below when the Number of Samples = Max-samples.

This shall apply under normal test voltage and ambient temperature, and with the interfering signal at any frequency in the range specified.

Channel			Type of		Max-events	Max-samples

measurement

non speech MS

FACCH			FER		200		164000

other MS

TCH/FS

Class II			RBER		200		8200

A maximum of six failures are allowed in the range FR +/- 45 MHz, with the exclusion of FR +/- 800 kHz (which, if grouped, shall not exceed three 200 kHz channels per group).

A maximum of 24 failures are allowed in the range 100 kHz to 12.75 GHz for the equipment with antenna connector (or 80 MHz to 4 GHz for the equipment with integral antenna) outside FR +/- 45 MHz (which, if below FR and grouped, shall not exceed three 200 kHz channels per group).

If the number of failures do not exceed the maximum allowed figures stated before, at those frequencies at which the failures occur the test stated in II.4.7.2 (or II.4.7.3) shall be repeated, but with a level of the unwanted signal set at 70 dBµVemf(  )

GSM900:	or 93.5 dBµV/m for the equipment with integral antenna

DCS1800:	or 99.3 dBµV/m for the equipment with integral antenna.

The performance requirement is once again that stated above, that is:

(non speech MS)

Max-events - 200			Max-samples - 164000

(other MS)

Max-events - 200			Max-samples - 8200

The number of Error Events recorded in this test shall not exceed the Max-events values given above, when the number of samples = Max-samples.

No failures are allowed at this lower unwanted signal level.