II.5.3.9	Testing of structured procedures

Ref: GSM 04.08 sections 7.3.2 through 7.3.4

II.5.3.9.1	General

The purpose of these tests is to complement the testing of elementary procedures to verify that the MS functions also in the "field" and not only on the "test bench".

The elementary procedures are combined into sequences, "structured procedures" as defined in GSM 04.08 section 7, representing true sequences of events for normal MS operation.

Only the main structured procedures are deemed necessary to test.

Mobile originating and terminating calls are tested in cases of both early and late assignment of traffic channel.

In two of the cases the call establishment test is followed by testing of call release initiated by network and mobile respectively.

The tests are performed only for a successful outcome of each elementary procedure.

Time requirements are indicated typically "(T3240-MM-MS)", showing timer identity, sublayer and (MS/NW) side where the timer is used (NW = Network).

The timer values are defined in GSM 04.08, section 11.

II.5.3.9.2	MS originating call establishment, early assignment, release init. by network

II.5.3.9.2.1	Purpose of the test

a)	To test the MS ability to originate a call following the sequence of elementary signalling procedures given in GSM 04.08 section 7.3.2.1a.

b)	To test the MS ability to release a call, when the release is initiated by the network, following the sequence of elementary signalling procedures given in GSM 04.08 section 7.3.4a.

c)	To verify the MS display of the called number.

II.5.3.9.2.2	Method of test

a)	The SS simulates one cell using the default parameter values for "cell 1" in section II.5.3.3 "Test of MS functions in Idle Mode".

b)	The MS is brought into the MM state "idle, updated", with a TMSI assigned, listening to the BCCH/CCCH of the active cell.

c)	The MS is made to initiate a call and the SS accepts the CHANNEL REQUEST message and returns the IMMEDIATE ASSIGNMENT message before the timer (T3120-RR-MS) has expired, assuming that the randomly drawn value of T3120 is the lowest possible.

d)	The SS accepts the service request from the MS via the data link establishment procedure.

e)	The SS sends message AUTHENTICATION REQUEST.

f)	After reception of AUTHENTICATION RESPONSE the SS sends message CIPHERING MODE COMMAND within (T3230-MM-MS) s after the first access burst, and starts deciphering.

g)	After reception of CIPHER MODE COMPLETE the SS starts enciphering.

h)	The SS sends the message CALL PROCEEDING within (T303-CC-MS) s after the SS has received the message SETUP.

i)	The SS then sends message ASSIGNMENT COMMAND.

j)	Having received message ASSIGNMENT COMPLETE, the SS sends the message ALERTING within (T310-CC-MS) s after the sending of CALL PROCEEDING.

k)	Within (T301-CC-MS) s after ALERTING the SS sends the CONNECT message.

l)	The SS initiates clearing of the call by sending the message DISCONNECT.

m)	Within (T308-CC-MS) s after reception of the message RELEASE the SS sends RELEASE COMPLETE followed, within a further (T3240-MM-MS) s, by CHANNEL RELEASE.

GSM900 only - begin

Message:	IMMEDIATE ASSIGNMENT	(GSM 04.08, 9.1.17) to the MS:

Informatin Element		Comment			Value

Protocol Discriminator	RR				011

Transaction Identifier	Not used				0000

Message Type						0011	1111

Page mode			Normal				00

Channel description

- Channel type ...		SDCCH/SACCH 1(4)		0	0100

- Time slot number		Arbitrary				xxx

- Training seq.code		(default)

- Hopping			No				0

- FB no.			Band no 0			000

- ARFCN			(default)

Random reference

- Random access info	As in CHAN REQ			111x	xxxx

- N51, N32, N26		As applicable			....

Timing advance		Arbitrary				xx	xxxx

Mobile allocation		Length=0 (due to hopping)		0

GSM900 only - end

DCS1800 only - begin

Message:	IMMEDIATE ASSIGNMENT	(GSM 04.08, 9.1.17) to the MS:

Information Element		Comment			Value

Protocol Discriminator	RR				011

Transaction Identifier	Not used				0000

Message Type						0011	1111

Page mode			Normal				00

Channel description

- Channel type		SDCCH/SACCH 1(4)		0	0100

- Time slot number		Arbitrary				xxx

- Training seq. code		(default)

- Hopping			No				0

- ARFCN			(default)

Random reference

- Random access info	As in CHAN REQ			111x	xxxx

- N51, N32, N26		As applicable			....

Timing advance		Arbitrary				xx	xxxx

Mobile allocation		Length=0 (due to hopping)		0

DCS1800 only - end

Message:	AUTHENTICATION REQUEST	(GSM 04.08, 9.2.2) to the MS:

Information Element		Comment			Value

Protocol Discriminator	MM				0101

Transaction Identifier	Not used				0000

Message Type						0001	0010

Ciphering key seq.number	As applic.from loc.upd		xxx

Authent.parameter RAND	As applicable			(128 bits)

Message:	CIPHERING MODE COMMAND	(GSM 04.08, 9.1.9) to the MS:

Information Element		Comment			Value

Protocol Discriminator	RR				0110

Transaction Identifier	Not used				0000

Message Type						0011	0101

Cipher mode setting		Start ciphering			1

Message:	CALL PROCEEDING	(GSM 04.08, 9.3.3) to the MS:

Information Element		Comment			Value

Protocol Discriminator	CC				0011

Transaction Identifier	MS orig.				1xxx

Message Type						0000	0010

GSM900 only - begin

Message:	ASSIGNMENT COMMAND	(GSM 04.08, 9.1.2) to the MS:

Information Element		Comment			Value

Protocol Discriminator	RR				0110

Transaction Identifier	Not used				0000

Message Type						0010	1110

Channel description

- Channel type ...		Bm+ACCHs			0	0001

- Time slot number		Arbitrary				xxx

- Training seq.code		(default)

- Hopping			No				0

- FB no			Band no 0			000

- ARFCN			(default)

Power level			As applicable			x	xxxx

Channel mode						0110	0011

- speech full rate						0000	0001

GSM900 only - end

DCS1800 only - begin

Message:	ASSIGNMENT COMMAND	(GSM 04.08, 9.1.2) to the MS:

Information Element		Comment			Value

Protocol Discriminator	RR				0110

Transaction Identifier	Not used				0000

Message Type						0010	1110

Channel description

- Channel type		Bm + ACCHs			0	0001

- Time slot number		Arbitrary				xxx

- Training seq. code		(default)

- Hopping			No				0

- ARFCN			(default)

Power level			As applicable			x	xxxx

Channel mode						0110	0011

- speech full rate						0000	0001

DCS1800 only - end

Message:	ALERTING	(GSM 04.08, 9.3.1) to the MS:

Information Element		Comment			Value

Protocol Discriminator	CC				0011

Transaction Identifier	MS orig.				1xxx

Message Type						0000	0001

Message:	CONNECT	(GSM 04.08, 9.3.5) to the MS:

Information Element		Comment			Value

Protocol Discriminator	CC				0011

Transaction Identifier	MS orig.				1xxx

Message Type						0000	0111

Message:	DISCONNECT	(GSM 04.08, 9.3.7) to the MS:

Information Element		Comment			Value

Protocol Discriminator	CC				0011

Transaction Identifier	SS orig.				0xxx

Message Type						0010	0101

Cause

- Coding standard		GSM				11

- Location			User				0000

- Cause value		Normal clearing			001	0000

Message:	RELEASE COMPLETE	(GSM 04.08, 9.3.15) to the MS:

Information Element		Comment			Value

Protocol Discriminator	CC				0011

Transaction Identifier	SS orig.				0xxx

Message Type						0010	1010

Drafting note: Is the "Cause" information element mandatory here ?

Message:	CHANNEL RELEASE	(GSM 04.08, 9.1.7) to the MS:

Information Element		Comment			Value

Protocol Discriminator	RR				0110

Transaction Identifier	Not used				0000

Message Type						0000	1101

RR cause			Normal release			0000	0000

II.5.3.9.2.3	Requirements

1)	In step c) the MS shall display the dialled number.

2)	In step c) the MS shall initiate immediate assignment by sending the CHANNEL REQUEST message.

3)	In step d) the MS shall send a layer 2 SABM frame containing the CM SERVICE REQUEST message to the SS. (The same message is returned in a layer 2 UA frame from the SS). The SABM frame shall be sent after reception of the IMMEDIATE ASSIGNMENT message.

4)	In step e) the MS shall send the message AUTHENTICATION RESPONSE with the correct signed response. (Time requirements should be tested in the elementary procedure)

5)	In step f), within 1 s from CIPHERING MODE COMMAND, the MS shall activate ciphering and send the message CIPHERING MODE COMPLETE followed by the SETUP message

Note:	In requirement 5), the time of 1 s is not intended to be a performance requirement, it is introduced only because it is necessary to indicate how long the SS will wait until it decides that there is a failure.

6)	In step i) the MS shall send the message ASSIGNMENT COMPLETE on the correct channel after reception of the ASSIGNMENT COMMAND.

7)	In step j) the ringing indication at the called party shall be recognizable in the MS.

8)	In step k) the MS shall send the message CONNECT ACKNOWLEDGE after recption of CONNECT, and through-connect the speech path in both directions.

9)	In step l) the MS shall send the message RELEASE after it has received the message DISCONNECT from the SS, initiating call release.

10)	After step m) the MS shall enter the idle, updated state.

Message:	CHANNEL REQUEST	(GSM 04.08, 9.1.8) to the SS:

Information Element		Comment			Value

Random reference						x	xxxx

Cause			Orig. call			111

Message:	 CM SERVICE REQUEST	(GSM 04.08, 9.2.7) to the SS:

Information Element		Comment			Value

Protocol discriminator	MM				0101

Transaction identifier	Not used				0000

Message type						0x10	0100

CM service type		Mob.orig.call est.			0001

Ciphering key seq.number	As applic.from loc.upd		xxx

MS classmark 2

- Revision level		As applicable			xxx

- Encryption algorithm	A5				00

- RF power capability	As applicable			xxx

- Short message capability	As applicable			x

- Frequency band		Band no 0			000

Mobile identity

- Odd/even no of digits	As applicable			x

- Type of identity		TMSI				100

- Identity digits		As applicable			....

Message:	AUTHENTICATION RESPONSE	(GSM 04.08, 9.2.3) to the SS:

Information Element		Comment			Value

Protocol discriminator	MM				0101

Transaction identifier	Not used				0000

Message type						0x01	0100

Authentication parameter	SRES, as applicable (32 bits)

Message:	CIPHERING MODE COMPLETE	(GSM 04.08, 9.1.10) to the SS:

Information Element		Comment			Value

Protocol discriminator	RR				0110

Transaction identifier	Not used				0000

Message type						0011	0010

Message:	SETUP	(GSM 04.08, 9.3.16) to the SS:

Information Element		Comment			Value

Protocol discriminator	CC				0011

Transaction identifier	MS orig.				0xxx

Message type						0x00	0101

Bearer capability

- Radio channel req.		Full rate				01

- Coding standard		GSM				0

- Transfer mode		Circuit mode			0

- Info transfer capab.	Speech				000

Mobile identity		Info element may be present

- Odd/even no of digits	As applicable			x

- Type of identity		TMSI				100

- Identity digits		As applicable			....

Called party BCD number

- Type of number		As applicable			xxx

- Numbering plan id.		As applicable			xxxx

- Digits			As applicable			....

Message:	ASSIGNMENT COMPLETE	(GSM 04.08, 9.1.3) to the SS:

Message:	CONNECT ACKNOWLEDGE	(GSM 04.08, 9.3.6) to the SS:

Information Element		Comment			Value

Protocol discriminator	CC				0011

Transaction identifier	MS orig.				0xxx

Message type						0x00	1111

Message	 RELEASE (GSM 04.08, 9.3.14) to the SS:

Information Element		Comment			Value

Protocol Discriminator	CC				0011

Transaction Identifier	SS orig.				1xxx

Message Type						0x10	1101

II.5.3.9.3	MS originating call establishment, late assignment

II.5.3.9.3.1	Purpose of the test

To test the MS ability to originate a call following the sequence of

elementary signalling procedures given in GSM 04.08 section 7.3.2.1b.

II.5.3.9.3.2	Method of test

a)	The first part of the method of test is identical to the case of early assignment of traffic channel (see previous section) up to and inclusive of the SS sending the CALL PROCEEDING message. Thereafter, the following applies:

b)	The SS sends the message ALERTING within (T310-CC-MS) s after the message CALL PROCEEDING was sent. The ALERTING message contents shall be the same as in section II.5.3.9.2.2.

c)	The SS then sends message ASSIGNMENT COMMAND. The message contents shall be the same as in section II.5.3.9.2.2.

d)	Having received message ASSIGNMENT COMPLETE, the SS sends message CONNECT within (T301-CC-MS) s after message ALERTING was sent. The CONNECT message contents shall be the same as in section II.5.3.9.2.2.

Note:	Clearing of the call is not part of the test procedure.

II.5.3.9.3.3	Requirements

1)	The first part of the requirements is identical to the case of early assignment of traffic channel (see previous section) up to and inclusive of the MS sending the SETUP message. Thereafter, the following applies:

2)	In step b) the ringing indication at the called party shall be recognizable in the MS as in section II.16.2.2.

3)	In step c) the MS shall send the message ASSIGNMENT COMPLETE on the correct channel after reception of ASSIGNMENT COMMAND. The ASSIGNMENT COMPLETE message contents shall be the same as in section II.5.3.9.2.3.

4)	In step d) the MS shall send the message CONNECT ACKNOWLEDGE after reception of CONNECT, and through-connect the speech path in both directions. The CONNECT ACKNOWLEDGE message contents shall be the same as in section II.5.3.9.2.3.

II.5.3.9.4	MS terminating call establishment, early assignment, release init. by MS

II.5.3.9.4.1	Purpose of the test

a)	To test the MS ability to respond correctly to a mobile terminating call following the sequence of elementary signalling procedures given in GSM 04.08 section 7.3.2.1b.

b)	To test the MS ability to initiate and fulfil the release of a call, following the sequence of elementary signalling procedures given in GSM 04.08 section 7.3.4b.

II.5.3.9.4.2	Method of test

a)	The SS simulates one cell using the default parameter values for "cell 1" in section II.5.3.3 "Test of MS functions in Idle Mode".

b)	The MS is brought into the MM state "idle, updated", with a TMSI assigned, listening to the BCCH/CCCH of the active cell.

c)	The MS is paged by means of a PAGING REQUEST TYPE 1 message on the correct paging subchannel.

d)	After reception of the CHANNEL REQUEST the SS returns the IMMEDIATE ASSIGNMENT message before the timer (T3120-RR-MS) has expired, assuming that the randomly drawn value of T3120 is the lowest possible.

e)	The SS accepts the paging response from the MS via the data link establishment procedure.

f)	Immediately after the data link establishment the SS sends AUTHENTICATION REQUEST.

g)	Following the correct AUTHENTICATION RESPONSE from the MS the SS sends CIPHERING MODE COMMAND and then starts ciphering.

#hcDrafting note: Timer values for authentication/ciphering are not specified.

h)	The SS sends the message SETUP immediately after reception of CIPHERING MODE COMPLETE. The SETUP message shall not include the 'signal' element.

i)	The SS then sends message ASSIGNMENT COMMAND.

j)	When the SS has received messages ASSIGNMENT COMPLETE and ALERT off-hook is performed on the MS.

k)	The SS sends message CONNECT ACKNOWLEDGE within (T313-C-MS) s after reception of message CONNECT and then enters conversation state.

l)	The MS is initiated to clear the call.

m)	Within (T305-CC-MS) s after reception of the message DISCONNECT the SS sends the RELEASE message.

n)	Within (T3240-MM-MS) s after reception of the message RELEASE COMPLETE the SS releases the RR connection by sending the message CHANNEL RELEASE.

Message:	PAGING REQUEST TYPE 1	(GSM 04.08, 9.1.21) to the MS:

Information Element		Comment			Value

Protocol Discriminator	RR				0110

Transaction Identifier	Not used				0000

Message Type						0010	0001

Page mode			Normal				00

Mobile identity

- Odd/even no of digits	As applicable			x

- Type of identity		TMSI				100

- Identity digits		As applicable			....

GSM900 only - begin

Message:	IMMEDIATE ASSIGNMENT	(GSM 04.08, 9.1.17) to the MS:

Information Element		Comment			Value

Protocol Discriminator	RR				0110

Transaction Identifier	Not used				0000

Message Type						0011	1111

Page mode			Normal				00

Channel description

- Channel type ...		SDCCH/SACCH 1(4)		0	0100

- Time slot number		Arbitrary				xxx

- Training seq.code		(default)

- Hopping			No				0

- FB no			Band no 0			000

- ARFCN			(default)

Random reference

- Random access info	As in CHAN REQ			100x	xxxx

- N51, N32, N26		As applicable

Timing advance		Arbitrary				xx	xxxx

Mobile allocation		Length=0 (due to hopping) 0

GSM900 only - end

DCS1800 only - begin

Message:	IMMEDIATE ASSIGNMENT	(GSM 04.08, 9.1.17) to the MS:

Information Element		Comment			Value

Protocol Discriminator	RR				0110

Transaction Identifier	Not used				0000

Message Type						0011	1111

Page mode			Normal				00

Channel description

- Channel type		SDCCH/SACCH 1(4)		0	0100

- Time slot number		Arbitrary				xxx

- Training seq. code		(default)

- Hopping			No				0

- ARFCN			(default)

Random reference

- Random access info	As in CHAN REQ			100x	xxxx

- N51, N32, N26		As applicable

Timing advance		Arbitrary				xx	xxxx

Mobile allocation		Length=0 (due to hopping)		0

DCS1800 only - end

Message:	AUTHENTICATION REQUEST	(GSM 04.08, 9.2.2) to the MS:

Information Element		Comment			Value

Protocol Discriminator	MM				0101

Transaction Identifier	Not used				0000

Message Type						0001	0010

Ciphering key seq.number	As applic.from loc.upd		xxx

Authent.parameter RAND	As applicable			(128 bits)

Message:	CIPHERING MODE COMMAND	(GSM 04.08, 9.1.9) to the MS:

Information Element		Comment			Value

Protocol Discriminator	RR				0110

Transaction Identifier	Not used				0000

Message Type						0011	0101

Cipher mode setting		Start ciphering			1

Message:	SETUP	(GSM 04.08, 9.3.16) to the MS:

Information Element		Comment			Value

Protocol discriminator	CC				0011

Transaction identifier	SS orig.				0xxx

Message type						0000	0101

GSM900 only - begin

Message:	ASSIGNMENT COMMAND	(GSM 04.08, 9.1.2) to the MS:

Information Element		Comment			Value

Protocol Discriminator	RR				0110

Transaction Identifier	Not used				0000

Message Type						0010	1110

Channel description

- Channel type ...		Bm+ACCHs			0	0001

- Time slot number		Arbitrary				xxx

- Training seq.code		(default)

- Hopping			No				0

- FB no			Band no 0			000

- ARFCN			(default)

Power level			As applicable			x	xxxx

Channel mode						0110	0011

- speech full rate						0000	0001

GSM900 only - end

DCS1800 only - begin

Message:	ASSIGNMENT COMMAND	(GSM 04.08, 9.1.2) to the MS:

Information Element		Comment			Value

Protocol Discriminator	RR				0110

Transaction Identifier	Not used				0000

Message Type						0010	1110

Channel description

- Channel type		Bm + ACCHs			0	0001

- Time slot number		Arbitrary				xxx

- Training seq. code		(default)

- Hopping			No				0

- ARFCN			(default)

Power level			As applicable			x	xxxx

Channel mode						0110	0011

- speech full rate						0000	0001

DCS1800 only - end

Message:	CONNECT ACKNOWLEDGE	(GSM 04.08, 9.3.6) to the MS:

Information Element		Comment			Value

Protocol discriminator	CC				0011

Transaction identifier	SS orig.				0xxx

Message type						0000	1111

Message:	RELEASE	(GSM 04.08, 9.3.14) to the MS:

Information Element		Comment			Value

Protocol Discriminator	CC				0011

Transaction Identifier	MS orig.				1xxx

Message Type						0010	1101

Message:	CHANNEL RELEASE	(GSM 04.08, 9.1.7) to the MS:

Information Element		Comment			Value

Protocol Discriminator	RR				0110

Transaction Identifier	Not used				0000

Message Type						0000	1101

RR cause			Normal release			0000	0000

II.5.3.9.4.3	Requirements

1)	In step c) the MS shall initiate immediate assignment by sending the CHANNEL REQUEST message.

2)	In step d) the MS shall send a layer 2 SABM frame containing the PAGING RESPONSE message to the SS. (The same message is returned in a Layer 2 UA frame from the SS). The SABM frame shall be sent after reception of the IMMEDIATE ASSIGNMENT message.

3)	In step f) the MS shall send the message AUTHENTICATION RESPONSE with the correct signed response. (Time requirements should be tested in the elementary procedure.)

4)	In step g) the MS shall activate ciphering and send the message CIPHERING MODE COMPLETE.

5)	In step h) the MS shall send the message CALL CONFIRMED after reception of SETUP.

6)	In step i) the MS shall send the message ASSIGNMENT COMPLETE on the correct channel after reception of ASSIGNMENT COMMAND.

7)	The MS shall 'ring'.

8)	The MS shall send the message ALERTING after sending the message CALL CONFIRMED.

9)	When off-hook has been performed the MS shall send the message CONNECT after sending the message ALERTING.

10)	In step j) the MS shall through-connect the speech path in both directions.

11)	In step l) the MS shall send the message DISCONNECT at the initiation of call release.

12)	In step m) the MS shall send the message RELEASE COMPLETE after reception of the message RELEASE.

13)	After step n) the MS shall enter the idle, updated state.

Message:	CHANNEL REQUEST	(GSM 04.08, 9.1.8) to the SS:

Information Element		Comment			Value

Random reference						x	xxxx

Cause			Answer to paging			100

Message:	PAGING RESPONSE	(GSM 04.08, 9.1.24) to the SS:

Information Element		Comment			Value

Protocol discriminator	RR				0110

Transaction identifier	Not used				0000

Message type						0010	0111

Ciphering key seq.number	As applic.from loc.upd		xxx

MS classmark 2

- Revision level		As applicable			xxx

- Encryption algorithm	A5				00

- RF power capability	As applicable			xxx

- Short message capability	As applicable			x

- Frequency band		Band no 0			000

Mobile identity

- Odd/even no of digits	As applicable			x

- Type of identity		TMSI				100

- Identity digits		As applicable			....

Message:	AUTHENTICATION RESPONSE	(GSM 04.08, 9.2.3) to the SS:

Information Element		Comment			Value

Protocol discriminator	MM				0101

Transaction identifier	Not used				0000

Message type						0x01	0100

Authentication parameter	SRES, as applicable (32 bits)

Message:	CIPHERING MODE COMPLETE	(GSM 04.08, 9.1.10) to the SS:

Information Element		Comment			Value

Protocol discriminator	RR				0110

Transaction identifier	Not used				0000

Message type						0011	0010

Message:	CALL CONFIRMED	(GSM 04.08, 9.3.2) to the SS:

Information Element		Comment			Value

Protocol discriminator	CC				0011

Transaction identifier	SS orig.				1xxx

Message type						0x00	1000

Message:	ASSIGNMENT COMPLETE	(GSM 04.08, 9.1.3) to the SS:

Message:	ALERTING	(GSM 04.08, 9.3.1) to the SS:

Information Element		Comment			Value

Protocol Discriminator	CC				0011

Transaction Identifier	SS orig.				1xxx

Message Type						0x00	0001

Message:	CONNECT	(GSM 04.08, 9.3.5) to the SS:

Information Element		Comment			Value

Protocol Discriminator	CC				0011

Transaction Identifier	SS orig.				1xxx

Message Type						0x00	0111

Message:	DISCONNECT	(GSM 04.08, 9.3.7) to the SS:

Information Element		Comment			Value

Protocol Discriminator	CC				0011

Transaction Identifier	MS orig.				0xxx

Message Type						0x10	0101

Cause

- Coding standard		GSM				11

- Location			user				0000

- Cause value		normal clearing			001	0000

Message:	RELEASE COMPLETE	(GSM 04.08, 9.3.15) to the SS:

Information Element		Comment			Value

Protocol Discriminator	CC				0011

Transaction Identifier	MS orig.				0xxx

Message Type						0x10	1010

II.5.3.9.5	MS terminating call establishment, late assignment

II.5.3.9.5.1	Purpose of the test

To test the MS ability to respond correctly to a mobile terminated call following the sequence of elementary signalling procedures given in GSM 04.08 section 7.3.3.1b.

II.5.3.9.5.2	Method of test

a)	The first part of the method of test is identical to the case of early assignment of traffic channel (see previous section) up to and inclusive of the SS receiving the CALL CONFIRMED message, except that the SETUP message shall include the 'signal' element. Thereafter, the following applies:

b)	After the SS has received the messages CALL CONFIRMED and ALERTING, off-hook is performed after the reception of ALERTING, on the MS.

c)	After the SS has received the message CONNECT the SS sends message ASSIGNMENT COMMAND. The message contents shall be the same as in section II.5.3.9.4.2.

d)	Having received message ASSIGNMENT COMPLETE, the SS sends message CONNECT ACKNOWLEDGE within (T313-CC-MS) s after reception of message CONNECT and then enters conversation state. The CONNECT ACKNOWLEDGE message contents shall be the same as in section II.5.3.9.4.2.

Note:	Clearing of the call is not part of the test procedure.

II.5.3.9.5.3	Requirements

1)	The first part of the requirements is identical to the case of early assignment of traffic channel (see previous section) up to and inclusive of the MS sending the CALL CONFIRM message. Thereafter, the following applies:

2)	In step b) the MS shall 'ring'.

3)	In step b) the MS shall send the message ALERTING after sending the message CALL CONFIRMED. The ALERTING message contents shall be the same as in section II.5.3.9.4.3.

4)	In step b) the MS shall send the message CONNECT within 0.5 s from the acceptance of the call (off hook) at the MS. The message contents shall be the same as in section II.5.3.9.4.3.

Note:	In requirement 4), the time of 0.5 s is not intended to be a performance requirement, it is introduced only because it is necessary to indicate how long the SS will wait until it decides that there is a failure.

5)	In step c) the MS shall send the message ASSIGNMENT COMPLETE on the correct channel after reception of ASSIGNMENT COMMAND. The ASSIGNMENT COMPLETE message contents shall be the same as in section II.5.3.9.4.3.

6)	In step d) the MS shall through-connect the speech path in both directions.

Note:	[within (...) ms after it has received the message CONNECT ACKNOWLEDGE].