II.10	TESTING OF BEARER SERVICES

This section is reserved.

Ref: GSM 02.02, 04.21, 07.01, 07.02, 07.03

II.10.1	Scope of testing of Bearer Services

II.10.1.1	General

The logical function blocks and Testing Access Points of the reference test configuration, which is used in describing the tests on data services are explained in section II.10.2, .

If not stated otherwise, the description of the tests refer to the generic call set-up procedures for data calls, which are defined in section II.1.4, including the State Diagram Model for GSM data services testing.

In the case an MS is designed with the functionality of more than one type of mobile terminal (e.g. MT0 + MT2), tests of all MT functionality shall be made as appropriate to all supported terminal types.

II.10.1.2	Tests related to data call establishment and release

The tests for data call establishment and release are given in section II.10.3. This section will verify which data services have been implemented in the MS and which have not. The tests will be performed to a level of detail to cover all compatibility parameters possible in a call SETUP or CALL CONFIRMED/CALL PROCEEDING message.

II.10.1.3	Tests of user data transmission functions

Tests of data transmission functions have been determined to be of lower priority and are thus not tested in GSM Phase 1.

II.10.1.4	Tests for the non-transparent case

Tests of data transmission functions have been determined to be of lower priority and are thus not tested in GSM Phase 1.

II.10.2	Reference configuration for testing of Data and Telematic Services

II.10.2.1	General reference configuration

The general reference configuration used (either explicitly or implicitly) to specify all the test cases for Data and Telematic services is shown in Figure II.10.2-1.

Four functional blocks are assumed as logically essential within the conformance test equipment (System Simulator):

-	The Radio Subsystem Channel Coder RSCC, directly connected to the Um interface, including the overall task of encoding / decoding as well as multiplexing / demultiplexing information related to the GSM signalling channel and Data traffic channel;

-	The Signalling Channel Termination SCT, including the overall task of transmitting / receiving signalling messages to / from the radio subsystem channel coder;

-	The Data Channel Termination DCT, including the overall task of transmitting /receiving information over the data traffic channel to / from the radio subsystem channel coder;

-	The Local Terminal Emulator LTE, emulating a Data Terminal Equipment (DTE) and including the overall task of transmitting / receiving data (synchronous or asynchronous, formatted or unformatted) to / from the MS data interface.

In the above reference configuration four Testing Access Points TAP 1 to TAP 4 are identified to cover all possible test cases:

-	TAP 1:

The MMI (optionally EMMI) of the MS under test;

-	TAP 2:

The R or S interface connecting the MS under test to the SS (only in case of MT2 or MT1 configuration); the following CCITT standards apply according to the MT configuration and the service being tested:

* Applicable V-series DTE/DCE interfaces.

* V.25 bis

* X.21

* X.21 bis

* X.25 (LAPB) layer 2 specification)

* I.430

-	TAP 3:

A testing point internal to the SS, by means of which the traffic channel information related to the Um interface may be sent / received, formatted as 36 or 60 bit V.110' frames (as per GSM 04.21).  No standard physical interface is specified.

-	TAP 4:

A testing point internal to the SS, by means of which, independently of the physical / logical channel where the transport occurs (as per GSM 04.03, 04.04), Layer 3 signalling messages related to the Um interface may be sent / received (according to GSM 04.05 to 04.12 and 04.80 to 04.88).  No standard physical interface is specified.

Figure II.10.2-1: General reference configuration for testing of Data and Telematic services

Figure II.10.2-2: Protocol model (R interface) for the transparent and non-transparent cases

II.10.2.2	Reference configuration for testing of MSs of the type MT0

In the case of MT0, the MS includes the Data Terminal function which is otherwise emulated by the test equipment.  This function within the MS is controlled via the (E)MMI which is also the way that user data is transmitted and received.

The reference configuration in Figure II.10.2-2 for testing of data services with MSs of the type MT0, therefore, identifies only the three Testing Access Points TAP 1, TAP 3 and TAP 4.  In this case, TAP 1 is not only used for manual intervention but also for control and verification of the data services under test.

Figure II.10.2-3: Reference configuration for testing of data services with MSs of the type MT0

II.10.2.3	Semi-graphical representation of the test suites

In the description of the tests a semi-graphical representation is used referring to the above reference model.  In the following an example of the format is shown, when all Testing Access Points play an active role.  The use of the different indications in the scheme is explained below.

TAP 1	TAP 2	TAP 3	TAP 4

(E)MMI	MS	SS	MS	MS	DCT	MS	SCT

Step			(LTE)		(RSCC)		(RSCC)

────	─────┼─────	─────┼─────	────┼────	─────┼─────

1	I1>			<S1			M1>

2								<M2

3

.					F1>

.						<F2

.				<S2

.			S3>

Note 1)

I1,I2,..:	text (when necessary) to explain the specific interaction / configuration

S1,S2,..:	text (when necessary) to explain the specific signal

F1,F2,..:	text (when necessary) to explain the specific V.110' frame

M1,M2,..:	text (when necessary) to explain the specific signalling message

notes   :	text.

II.10.3	Testing of call set-up and release for data services

II.10.3.1	General

The tests described covers the MS configurations MT0 and MT2.

The suite of tests covers compatibility tests and tests of signalling which might affect the network.  It does not cover, in Phase 1, service related tests.

The measurements are performed at the Testing Access Point TAP 4 (signalling).  See section II.10.2 for the reference configuration.

It is assumed that the tests of section II.5.3 have been performed in all cases applicable to the MS (Layer 3 tests).

For those tests in which a SETUP message for the multiple numbering scheme (one or two bearer capabilities) is sent to the MS simulating an incoming call, the setup message shall contain those information elements required for compatibility checking (see GSM 07.01 Annex II as regards the BC-IE, LLC and HLC, and GSM 04.08 section 9.3.16) coded in accordance with the suppliers PICS statements so that the incoming call will be compatible with the MS characteristics.  See  Annex 3.  The various information elements provided shall be mutually compatible, ie Bearer Capability, Low Layer Compatibility, High Layer Compatibility, Called Party Sub-address information elements must be consistent with the characteristics of the MS under test.

For those tests in which a SETUP message for the single numbering scheme (no Bearer Capability Information Element) is sent to the MS simulating an incoming call, there shall be no Low Layer Compatibility or High Layer Compatibility information element present.

These tests for incoming call shall be repeated as necessary so that all the parameters stated by the supplier are covered.  The MS shall be considered to have passed the test if it accepts the call according to GSM 04.08 section 5.2.2.2 and 04.08 Annex B.

In addition, these tests shall be repeated several times with the information elements involved in compatibility checking (ie  Bearer Capability, Low Layer Compatibility, High Layer Compatibility etc) encoded so that the incoming call is incompatible with the characteristics of the terminal under test as specified in the PICS statements of the supplier.  The MS shall be considered to have passed if it rejects or ignores the incoming call according to GSM 04.08 section 5.2.2.2 and 04.08 Annex B, respectively.

Some tests may require actions at the man-machine interface of the MS, such as initiation of an outgoing call.  In the tests defined below, these actions have been described in terms of the Layer 3 messages, which should be generated by such action: eg invoke SETUP.  For MSs where the generation of such messages is not by action at the man-machine interface (eg MT2s), the equivalent operation can be performed at the appropriate Testing Access Point in order to generate these messages.  This Testing Access Point would be for example, the interface at the R-reference point in the case of an MT2.  The supplier must state the operational details in the PIXIT statements, see Annex 3.

The supplier must state in the PIXIT statements (Annex 3)  all possible ways of initiating a call.  The tests shall cover all these possible ways.

For the R-interface only the V.25 bis autocalling procedure is defined.  The supplier must state whether the MS adheres to the default V.25 bis procedures as stated in section II.15 or there are other ways, which then must be specified in an similar manner in a PIXIT statement in Annex 3.

For all tests in this section the SS shall check that the coding of the information elements contained in all the messages received from the MS under test are in accordance with GSM 07.01 Annex II, and 04.08 sections 9.3 and 9.4.  An MS shall be considered to have failed the tests if an information element is received which is either incorrectly structured or has a code value not covered by GSM 07.01 Annex II, and 04.08 sections 9.3 and 9.4.

In particular when performing the tests of the outgoing call to check the MSs satisfaction of the Layer 3 procedures, it shall be tested that the codings of the Bearer capability (if used) and (if used) other optional information elements contained in the SETUP message or CALL CONFIRMED/CALL PROCEEDING message are in accordance with the capabilities of the MS under test as declared by the supplier in the PICS statements.  This test should be repeated to cover all the capabilities (including parameters) specified by the supplier for the MS under test.  The different implemented options and their parameters need to be tested only once.

[In the case where the SS is required to monitor for no response from the MS under test, the monitoring time shall be for 15 s after the sending of the last stimulus from the SS.]

When the SS is expecting a response from the MS, it shall assume that the MS has made no response if no message has been received from the MS within 15 s after the sending of a stimulus from the SS.

In tests where there is an error situation, a RELEASE COMPLETE message is expected from the MS under test.  The MS under test shall be considered to have passed those tests if it sends a RELEASE COMPLETE message within 15 s.

In general the messages sent towards the MS under test shall not transfer information which may cause higher layers to release the call (except for those tests concentrating on releasing procedures.).  The supplier shall declare in PIXIT statements what the implementation specific conditions are (Annex 3).

II.10.3.2	Um interface tests

II.10.3.2.1	Definitions

In the tests below, a "valid" SETUP message from the MS is a SETUP message containing information elements which the supplier in the PICS statements has declared that the MS is capable of transmitting in accordance with the rules of GSM 07.01 annex II.  All other SETUP messages from the MS are considered "invalid".

In the tests below, a "valid" SETUP message from the SS is a SETUP message containing information elements which the supplier in the PICS statements has declared that the MS is capable of being compatible with and which conform to the rules of GSM 07.01 annex II.  All other SETUP messages from the SS are considered "invalid".

II.10.3.2.2	MS originating call, receipt of a CONNECT message

Purpose of the test

Ensure on receipt of a CONNECT message the MS enters the ACTIVE state U10.

Initial condition:

The generic call set-up  procedure in section II.1.4 should be followed up to and including the CIHPERING MODE COMPLETE message of Requirement 5) in II.5.3.9.2.3.

Procedure

MS:	transmit SETUP

SS:	transmit CALL PROCEEDING

SS:	transmit CONNECT

MS:	transmit CONNECT ACKNOWLEDGE

SS:	transmit STATUS ENQUIRY

MS:	transmit STATUS

Result checking:

Analyse STATUS message.

Requirement:

The STATUS message must contain: call state U10.

Postamble:

Use the generic call set-up / release from method of test l) of II.5.3.9.2.2

For MSs with an external R interface the MS must adhere to the default V.25 bis procedure in section II.15 or provide another way which must be specified in a similar manner in the PIXIT statement (Annex 3).  For an MS with an MMI interface and/or an R interface at least one test for one of the possible sets of parameters must be performed for the MMI case and another for the R-interface case, if provided.

Semi-graphical Representation

TAP 1	TAP 2	TAP 3	TAP 4

(E)MMI	MS	SS	MS	MS	DCT	MS	SCT

Step			(LTE)		(RSCC)		(RSCC)

────	─────┼─────	─────┼─────	────┼────	─────┼─────

1							SETUP >

2								< CALL PROCEEDING

3								<CONNECT

4							CONNECT>

ACKNOWLDGE

5								< STATUS ENQ

6							STATUS>

II.10.3.2.3	MS originating call,receipt of a RELEASE COMPLETE message

Purpose of the test.

Ensure on receipt of a RELEASE COMPLETE message the MS enters the NULL state U0.

Initial condition:

The generic call setup procedure in section II.1.4 should be followed up to and including the CIHPERING MODE COMPLETE message of Requirement 5) in section II.5.3.9.2.3.

Procedure

MS:	transmit SETUP

SS: 1)	transmit RELEASE COMPLETE

SS: 2)	transmit STATUS ENQUIRY

MS: 3)	transmit RELEASE COMPLETE

Note 1)	The RELEASE COMPLETE message should contain a cause value of 17 ('User Busy')

Note 2)	Transmit 7 STATUS ENQUIRY messages with different TI values.

Note 3)	The RELEASE COMPLETE message should contain a cause value of 88

Result checking:

Analyse RELEASE COMPLETE messages.

Requirement:

The MS shall answer with a RELEASE COMPLETE, for every related STATUS ENQUIRY message, with a cause value of 88 'incompatible destination'.

Postamble:

Use the generic call set-up / release from method of test l) of II.5.3.9.2.2

For MSs with an external R interface the MS must adhere to the default V.25 bis procedure in section II.15 or provide another way which must be specified in a similar manner in the PIXIT statement. For an MS with an MMI interface and/or an R interface at least one test for one of the possible sets of parameters must be performed for the MMI case and another for the R-interface case, if provided.

This test shall be performed with one parameter set which is compatible and a set which is incompatible.

Semi-graphical Representation

TAP 1	TAP 2	TAP 3	TAP 4

(E)MMI	MS	SS	MS	MS	DCT	MS	SCT

Step			(LTE)		(RSCC)		(RSCC)

────	─────┼─────	─────┼─────	────┼────	─────┼─────

1							SETUP >

2								< CALL PROCEEDING

3								<RELEASE

COMPLETE

4								< STATUS ENQ

5							RELEASE>

COMPLETE

6								< STATUS ENQ

7							RELEASE>

COMPLETE

8								< STATUS ENQ

9							RELEASE>

COMPLETE

10								< STATUS ENQ

11							RELEASE>

COMPLETE

12								< STATUS ENQ

13							RELEASE>

COMPLETE

14								< STATUS ENQ

15							RELEASE>

COMPLETE

16								< STATUS ENQ

17							RELEASE>

COMPLETE

II.10.3.2.4	MS terminating call, acceptable to the MS

Purpose of the test

Ensures that on a valid incoming call the MS will enter the active state.

Initial condition

The generic call setup procedure in II.1.4 should be followed up to and including step g) of II.5.3.9.4.2.

Procedure

SS:		transmit a valid SETUP message.

MS:		transmit CALL CONFIRMED

MS: 1)	transmit ALERT

MS:		transmit CONNECT

SS:		transmit CONNECT ACKNOWLEDGE

SS:		transmit STATUS ENQUIRY

MS:		transmit STATUS

Note 1)	This is an optional message which is subject to the implementation of the mobile as declared in the PIXIT statement (Annex 3).

Result checking:

Analyse STATUS message.

Requirement:

The STATUS message must contain: call state U10 (active).

Postamble:

Use the generic call set-up / release from method of test l) of II.5.3.9.2.2

Note:	It is deliberate that neither MMI nor the R-interface is mentioned here.  However, the supplier must - if necessary - indicate in a PIXIT statement (Annex 3) how a possible R-interface should be set up to make sure that the MS can go to the active state.  This also applies to MMI.

Semi-graphical Representation

TAP 1	TAP 2	TAP 3	TAP 4

(E)MMI	MS	SS	MS	MS	DCT	MS	SCT

Step			(LTE)		(RSCC)		(RSCC)

────	─────┼─────	─────┼─────	────┼────	─────┼─────

1								<-SETUP

2							CALL->

CONFIRMED

3							ALERT->

4							CONNECT>

5								<CONNECT

ACKNOWLEDGE

6								< STATUS ENQ

7							STATUS->

II.10.3.2.5	MS terminating call, not acceptable to the MS

Purpose of the test

Ensures that on an invalid incoming call the MS will reject the call.

Initial condition

The generic call setup procedure in II.1.4 should be followed up to and including step g) of II.5.3.9.4.2.

Procedure

SS: 1)	transmit a SETUP message.

MS:		transmit RELEASE COMPLETE.

Note 1)	transmit a SETUP message with an incompatible BCIE as declared by the manufacturer.

Result checking:

Requirement:

Expect a RELEASE COMPLETE message.

Note:	It is deliberate that neither MMI nor the R-interface is mentioned here.  However, the supplier must - if necessary - indicate in a PIXIT statement (Annex 3) how a possible R-interface should be set up to make sure that the MS can go to the active state (U10) on receipt of a compatible BC-IE.  This is necessary to eliminate any other reason for failing the call.  This also applies to MMI.

Semi-graphical Representation

TAP 1	TAP 2	TAP 3	TAP 4

(E)MMI	MS	SS	MS	MS	DCT	MS	SCT

Step			(LTE)		(RSCC)		(RSCC)

────	─────┼─────	─────┼─────	────┼────	─────┼─────

1								<-SETUP

2							RELEASE>

COMPLETE

II.10.4	Verification of synchronisation of user data on the traffic channel

Ref: GSM 07.01, 04.21

(Reserved.)

II.10.5	Filtering of the channel control information

Ref: GSM 07.01, 04.21

(Reserved.)

II.10.6	Testing of data rate adaptation functions

(Transparent data services only) ref: GSM 04.21

(Reserved.)