II.12	Test of supplementary services

The general aspects of the specification of supplementary services at the Layer 3 radio interface are given in GSM 04.10.

The formats and coding are given in GSM 04.80. If the value of a parameter of an uplink message (MS to network) is specified in a test, the implicit meaning is that it has to be checked; if the value is not specified, it is not to be checked unless otherwise stated.

Unless otherwise stated, the MS shall be in the idle updated state at teh beginning of each test (including in case of repetition of a test).

In each test, before the MS sends the first REGISTER message, a MM connection shall be established.

GSM 04.81 to 04.88 give the procedures used at the radio interface for normal operation, registration, erasure, activation, deactivation, invocation and interrogation of supplementary services.

The supplementary services are described in GSM 02.04 and 02.81 to 02.88.

Whenever activation via the standard MMI is mentioned, if the MS doesn't support it but supports a different procedure, this procedure is used. In the SS tests only the applicable (see GSM 02.8x series) MMI Service Code groups (GSM 02.30 Annex 4), which are supported by the MAP of the Basic Service Code in GSM 09.02, shall be used.

In all the following test cases, the MS will not be failed if it doesn't send a FACILITY information element inthe initial REGISTER message, but in a subsequent FACILITY message (although only the first case is the normal behaviour).

II.12.1	Number identification supplementary services

(Reserved).

II.12.2	Call offering supplementary services

II.12.2.1	Call forwarding supplementary services

Unless stated otherwise, the test procedures described in the following apply to all kinds of call forwarding supplementary services.

The following abbreviations are used:

CFU :		Call Forwarding Unconditional

CFB :		Call Forwarding on mobile subscriber Busy

CFNRy :	Call Forwarding on No Reply

CFNRc :	Call Forwarding on mobile subscriber Not Reachable

CFC :		Call Forwarding Conditional (common name for CFB, CFNRy and CFNRc)

CF :		Call Forwarding (common name for CFU, CFB, CFNRy and CFNRc).

These abbreviations are also used to represent the corresponding SS-Code; e.g. CFC is the SS-Code for all conditional forwarding services.

II.12.2.1.1	Registration

A)	Registration accepted

Method of test:

By means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30 the MS is forced to send a REGISTER message containing the following parameters (see GSM 04.82 section 1.2):

-	for "call forwarding unconditional",

"call forwarding on mobile subscriber busy" and

"call forwarding on mobile subscriber non reachable" :

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

invoke =	Registration

Supplementary Service Code = CFU (or CFB, or CFNRc)

Forwarded To Number

Basic Service Code.

-	for "call forwarding on no reply":

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

invoke =	Registration

Supplementary Service code = CFNRy

Forwarded To Number

No Reply Condition Time

Basic Service Code.

Note:	If Basic Service Code is not included the Forwarded To Number applies to all Basic Services.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE or FACILITY message with the Facility information element containing a return result of the RegisterSS operation with the following parameters with the same values as above:

-	Forwarded To Number

-	No Reply Condition Time	(CFNRy only)

-	Basic Service Code	(Included if included in the invoke).

The invoke ID must be the same as in the invoke of the Register SS operation.

The test is repeated using different values for the SS-Code, Forwarded To Number, No Reply Condition Time and Basic Service Code and also with the Basic Service Code not included. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

For each case tested, the MS has to transmit successively a CHANNEL REQUEST with establishment cause set to "other services requested by the mobile user": (coded as 111), a CM SERVICE REQUEST with CM service type indicating "supplementary service activation" (coded as 1000) and then the REGISTER message containing the invoke of the RegisterSS operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS has to provide the appropriate user indication (which is to be described by the manufacturer).

B)	Registration rejected

Method of test:

The MS is forced to send the REGISTER message in the same way as described for case A).

Upon receipt of the REGISTER message the system simulator answers with a RELEASE COMPLETE containing all parameter values found in REGISTER and additionally the cause for rejection.

The cause could be a return error (for instance SS not available) or a reject (for instance mistyped parameter).

RELEASE COMPLETE

Protocol discriminator.

Transaction identifier

Message type

Facility :	return error code

Or facility :	reject code.

For the return error and the reject the invoke ID must be the same as in the invoke of the Register SS operation.

The possible errors are:

-	IllegalSS-Operation

-	SS-ErrorStatus	with parameter SS-Status indicating:

-	Not Provisioned, Not Registered and Not Active or

-	Provisioned, Registered and Active

-	SS-NotAvailable

-	SS-SpecificError

-	BearerServiceNotProvisioned

-	TeleserviceNotProvisioned

-	SS-SubscriptionViolation

-	SS-Incompatibility

-	UnexpectedDataValue

-	DataMissing

-	SystemFailure.

The possible problems are:

General problems:

-	Unrecognized Component

-	Mistyped Component

-	Badly Structured Component.

Invoke problems:

-	Unrecognized Operation

-	Mistyped Parameter

-	Resource Limitation

-	Initiating Release.

The test is repeated with different errors and problems and when error is SS-ErrorStatus with different SS-Status indications.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS has to provide the appropriate user indication (which is to be described by the Manufacturer).

II.12.2.1.2	Erasure

II.12.2.1.2.1	Erasure by the subscriber

A)	Erasure accepted

Method of test:

By means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30 the MS is forced to send a REGISTER message with the Facility information element containing an invoke of the EraseSS operation with the following parameters:

Facility Invoke = Erasure

-	SS-Code		(CFU, CFB, CFNRy or CFNRc)

-	BasicServiceCode	(Optional).

If Basic Service Code is not included Erasure applies to all Basic Services.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE or FACILITY message with the Facility information element containing a return result of the EraseSS operation with the following parameter with the same value as above:

Facility Return Result = Erasure

-	BasicServiceCode (Included if included in the invoke).

The invoke ID must be the same as in the invoke of the EraseSS operation.

The test is repeated using different values for the SS-Code and BasicServiceCode and also with the BasicServiceCode not included. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

For each case tested, the MS has to transmit the REGISTER message containing the invoke of the EraseSS operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS has to provide the appropriate user indication (which is to be described by the manufacturer).

B)	Erasure rejected

Method of test:

The MS is forced to send the REGISTER message in the same way as described for case A).

Upon receipt of the REGISTER message the system simulator answers with a RELEASE COMPLETE message with the Facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject, the invoke ID must be the same as in the invoke of the EraseSS operation.

If Facility = return error

The possible errors are:

-	IllegalSS-Operation

-	SS-ErrorStatus	with parameter SS-Status indicating:

-	Not Provisioned, Not Registered and Not Active or

-	Provisioned, Not Registered and Not Active

-	SS-SpecificError

-	SS-SubscriptionViolation

-	UnexpectedDataValue

-	DataMissing

-	SystemFailure.

If Facility = Reject

The possible problems are:

General problems:

-	Unrecognized Component

-	Mistyped Component

-	Badly Structured Component.

Invoke problems:

-	Unrecognized Operation

-	Mistyped Parameter

-	Resource Limitation

-	Initiating Release.

The test is repeated with different errors and problems and when error is SS-ErrorStatus with different SS-Status indications.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS has to provide the appropriate user indication (which is to be described by the Manufacturer).

II.12.2.1.3	Activation

Activation is not applicable to call forwarding services.

II.12.2.1.4	Deactivation

Causes no signalling on the radio path.

II.12.2.1.5	Invocation

Not applicable to the MS.

Causes no signalling on the radio path.

II.12.2.1.6	Interrogation

II.12.2.1.6.1	General data request

A)	Interrogation accepted

Method of test:

By means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30 the MS is forced to send a REGISTER message containing the SS code for the specific call forwarding service to be tested.

REGISTER MESSAGE

-	Protocol discriminator

-	Transaction identifier

-	Message type

-	Facility

invoke =	Interrogation

Supplementary Service Code = CFU or CFB, CFNRy, CFNRc.

As the Basic Service Code is not included, interrogation applies to all Basic Services.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE or FACILITY message with the Facility information element containing a return result of the InterrogateSS operation with zero, one or more than one triplet(s)/pair(s) of the following parameters:

-	in case of Supplementary Service Code = CFNRy :

FACILITY :

Return Result = Interrogation

ForwardedToNumber			│

NoReplyConditionTime (CFNRy only)	│ 1st Basic Service

BasicServiceCode			│

Forwarded to Number			│

No Reply Condition Time		│ 2nd Basic Service

Basic Service Code			│

etc ...

-	in case of Supplementary Service Code = CFU or CFB or CFNRc :

FACILITY :

Return Result = Interrogation

Forwarded to Number			│ 1st Basic Service

Basic Service Code			│

Forwarded to Number			│

Basic Service Code			│ 2nd Basic Service

Forwarded to Number			│

Basic Service Code			│ 3rd Basic Service

etc ...

The invoke ID must be the same as in the invoke of the InterrogateSS operation.

The test is repeated using different SS-Codes and different numbers of parameter triplets/pairs with different values. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

For each case tested, the MS has to transmit the REGISTER message containing the invoke of the InterrogateSS operation with the expected parameter value.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS has to provide the appropriate user indication (which is to be described by the manufacturer).

B)	Interrogation rejected

Method of test:

The MS is forced to send the REGISTER message in the same way as described for case A).

Upon receipt of the REGISTER message the system simulator answers with a RELEASE COMPLETE message with the Facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject, the invoke ID must be the same as in the invoke of the InterrogateSS operation.

If Facility = Return error

the possible errors are:

-	IllegalSS-Operation

-	SS-NotAvailable

-	SS-SpecificError

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure.

If Facility = reject

the possible problems are:

General problems:

-	Unrecognized Component

-	Mistyped Component

-	Badly Structured Component.

Invoke problems:

-	Unrecognized Operation

-	Mistyped Parameter

-	Resource Limitation

-	Initiating Release.

The test is repeated with different errors and problems.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS has to provide the appropriate user indication (which is to be described by the Manufacturer).

II.12.2.1.6.2	Specific data request

A)	Interrogation accepted

Method of test:

By means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30 the MS is forced to send a specific data request REGISTER containing the SS code for the specific call forwarding service and the basic service about which information is interrogated.

REGISTER message

-	Protocol discriminator

-	transaction identifier

-	message type

-	facility

Invoke =	Interrogation

Supplementary Service Code = CFU (or CFB or CFNRy, or CFNRc)

Basic Service Code.

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE or FACILITY message with the Facility information element containing a return result of the InterrogateSS operation with the following parameters:

-	in case of Supplementary Service Code = CFNRy :

FACILITY

Return Result =	Interrogation

Forwarded to Number

No Reply Condition Time

Basic Service Code.

-	in case of Supplementary Service Code = CFU or CFB or CFNRc :

FACILITY

Return Result =	Interrogation

Forwarded to Number

Basic Service Code.

The invoke ID must be the same as in the invoke of the InterrogateSS operation.

The test is repeated using different SS-Codes and BasicServiceCodes. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

1)	For each case tested, the MS has to transmit the REGISTER message containing the invoke of the InterrogateSS operation with the expected parameter values.

2)	Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS has to provide the appropriate user indication (which is to be described by the manufacturer).

B)	Interrogation rejected

Method of test:

The MS is forced to send the REGISTER message in the same way as described for case A).

Upon receipt of the REGISTER message the system simulator answers with a RELEASE COMPLETE message with the Facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject, the invoke ID must be the same as in the invoke of the InterrorgateSS operation.

If Facility = Return error

the possible errors are:

-	IllegalSS-Operation

-	SS-NotAvailable

-	SS-SpecificError

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure

If Facility = Reject

the possible problems are:

General problems:

-	Unrecognized Component

-	Mistyped Component

-	Badly Structured Component.

Invoke problems:

-	Unrecognized Operation

-	Mistyped Parameter

-	Resource Limitation

-	Initiating Release.

The test is repeated with different errors and problems.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS has to provide the appropriate user indication (which is to be described by the Manufacturer).

II.12.2.1.7	Normal operation

II.12.2.1.7.1	Served mobile subscriber side

II.12.2.1.7.1.1	Notification of incoming call

This subscription option is only applicable to CFB and CFNRy.

Method of test:

a)	The MS is brought to the call state U7 of an incoming call.

b)	The System Simulator transmits a FACILITY message with the Facility information element containing an invoke of the NotifySS operation with the following parameters:

FACILITY

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

invoke =	Notification

Supplementary Service Code = CFNRy or CFB

SS notification = incoming call is forwarded

(call is forwarded indication to B suscriber).

c)	The MS is brought into the call state U10 of an incoming call.

d)	The SS transmits a FACILITY message with the facility information element containing an invoke of the NotifySS operation with the following parameters:

FACILITY

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

invoke =	Notification

Supplementary Service code = CFB

SS notification = incoming call is forwarded

(call is forwarded indication to B subscriber).

e)	A STATUS ENQUIRY message is sent to the MS.

Requirements:

After steps b) and d), upon receipt of the FACILITY message, the MS has to provide the appropriate user indication (which is to be described by the manufacturer).

After step e), teh MS shall send a STATUS message with CC-state U10.

II.12.2.1.7.1.2	Notification during outgoing call

a)	Method of test:

The MS is forced to place an outgoing call.

After having received a SETUP message and sent a CALL PROCEEDING message and after a TCH has been allocated the system simulator transmits an ALERTING message with the Facility information element described below and then a STATUS ENQUIRY message

Facility Invoke = Notification

-	SS-Code (CFU or CFC)

-	SS-Status (indicating:

Provisioned, Registered and Active).

The test is repeated using different SS-Codes.

Requirements:

Upon receipt of the ALERTING message with the facility information element, the MS has to provide the appropriate user indication (which is to be described by the manufacturer). On receipt of the STATUS ENQUIRY message the MS shall send a STATUS message with CC-state U4.

b)	Method of test

The MS is forced to place an outgoing call.

After having received a SETUP message and sent a CALL PROCEEDING message and after a TCH has been allocated the system simulator transmits an ALERTING message and then a CONNECT with the facility information element described below. After reception of a CONNECT ACKNOWLEDGE message the SS sends a STATUS ENQUIRY message.

Facility Invoke = Notification

-	SS-Code (CFU or CFC)

-	SS-Status (indicating:

Provisioned, Registered and Active).

The test is repeated using different SS-Codes.

Requirements:

Upon receipt of the CONNECT message with the facility information element, the MS has to provide the appropriate user indication (which is to be described by the manufacturer). On receipt of the CONNECT message and then on receipt of the STATUS ENQUIRY message the MS shall send a STATUS message with CC-state U10.

c)	Method of test

The MS is forced to place an outgoing call.

After having received a SETUP message and sent a CALL PROCEEDING message and after a TCH has been allocated the system simulator transmits a FACILITY message with the facility information element described below. The SS then allocates a TCH and after the TCH has been allocated the SS sends a STATUS ENQUIRY message.

Facility Invoke = Notification

-	SS-Code (CFU or CFC)

-	SS-Status (indicating:

Provisioned, Registered and Active).

The test is repeated using different SS-Codes.

Requirements:

Upon receipt of the FACILITY message with the facility information element, the MS has to provide the appropriate user indication (which is to be described by the manufacturer). After a TCH has been allocated the MS shall, on receipt of the STATUS ENQUIRY message send a STATUS message with CC-state U3.

II.12.2.1.7.2	Forwarded-to mobile subscriber side

Method of test:

An incoming call is given to the MS with the SETUP message with the Facility information element containing an invoke of the NotifySS operation with the following parameters:

Facility Invoke = Notification

-	SS-Code (CFU, CFB, CFNRy, CFNRc or CF)

-	SS-Notification (indicating: Call is forwarded i.e.

Call is forwarded indication to C-subscriber).

The test is repeated using different SS-Codes.

Requirements:

Upon receipt of the SETUP message, the MS has to provide the appropriate user indication (which is to be described by the manufacturer).

II.12.2.1.7.3	Calling mobile subscriber side

Method of test:

A call is initiated by the MS, Upon receipt of the SETUP message the System Simulator transmits a FACILITY message with the Facility information element containing an invoke of the NotifySS operation with the following parameters:

Facility Invoke = Notification

-	SS-Code (CFU, CFB, CFNRy, CFNRc or CF)

-	SS-Notification (Outgoing call has been forwarded i.e.

Call is forwarded indication to A-subscriber).

The test is repeated using different SS-Codes.

Requirements:

Upon receipt of the FACILITY message, the MS has to provide the appropriate user indication (which is to be described by the manufacturer).

II.12.2.2	Call transfer and Mobile access hunting supplementary services

(Reserved).

II.12.3	Call completion supplementary services

(Reserved).

II.12.4	Multy-party supplementary services

(Reserved).

II.12.5	Community of interest supplementary services

(Reserved).

II.12.6	Charging supplementary services

(Reserved).

II.12.7	Additional information transfer supplementary services

(Reserved).

II.12.8	Call restriction supplementary services

Unless stated otherwise, the test procedures in the following apply to all kinds of call restriction supplementary services.

The following abbreviations are used

BO		:	Barring of Outgoing calls

BAOC	:	Barring of All Outgoing calls

BOIC		:	Barring of Outgoing International calls

BOIC-exHC	:	Barring of Outgoing international call Except those directed

to the Home PLMN country

BI		:	Barring of Incoming calls

BAIC		:	Barring of All Incoming calls

BAIC-Roam	:	Barring of incoming when Roaming Outside the home

PLMN country

B		:	Barring (common name for BAOC, BOIC, BOIC-exHC, BAIC

and BAIC-Roam).

These abbreviations are also used to represent the corresponding SS-Code; e.g. B is the SS-Code for all call restriction (barring) services.

Note:	The password(s) to be used during tests of this section II.12.8 may be randomly chosen - unless otherwise stated - in accordance with GSM 02.04 section 5.2.

II.12.8.1	Registration

This section allows the registration protocol for password registration to be tested. Both successful and unsuccessful outcomes are tested.

A)	Registration accepted

General

The following steps shall be executed sequentially.

In the case of brand specific MMI, actions might have to be performed at the MS during the test (eg. entering new password).

Method of test, Step 1:

By means of either the standardized MMI functions as described in GSM 02.30 or the brand specific MMI, the MS is forced to send a REGISTER message with the Facility information element containing an invoke of the RegisterPassword operation with the following parameter:

Facility Invoke = Register Password

-	SS-Code (B).

Upon receipt of the REGISTER message, the System Simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility Invoke = Get Password

-	GuidanceInfo	(indicating: Enter Password).

The linked ID must be the same as the invoke ID in the invoke of the RegisterPassword operation.

Requirement:

The MS has to transmit the REGISTER message with the expected parameter value.

Note:	No indication (on GuidanceInfo) is given to the user in case the standardized MMI functions of GSM 02.30 have been used, and the test continues to the next step without interruption.

Method of test, Step 2:

Upon receipt of the FACILITY message, the MS has to transmit a FACILITY message with the Facility information element containing a return result of the GetPassword operation with the following parameter:

Facility Return Result = Get Password

-	Password	(password).

The invoke ID must be the same as in the invoke of the GetPassword operation.

Upon receipt of the FACILITY message, the System Simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility Invoke = Get Password

GuidanceInfo	(indicating: Enter New Password).

The linked ID must be the same as the invoke ID in the invoke of the RegisterPassword operation.

Requirements:

The MS has to transmit the FACILITY message with the expected parameter value.

Upon receipt of the FACILITY message the MS has to provide the appropriate user indication (which is to be described by the manufacturer).

Note:	No indication (on GuidanceInfo) is given to the user in case the standardized MMI functions of GSM 02.30 have been used, and the test continues to the next step without interruption.

Method of test, Step 3:

Unless the standardized MMI functions of GSM 02.30 have been used, the new password is entered in the MS by means of procedures which are to be described by the manufacturer. The MS has to transmit a FACILITY message with the Facility information element containing a return result of the GetPassword operation with the following parameter:

Facility Return Result

-	Password		(the new password).

The invoke ID must be the same as in the invoke of the GetPassword operation.

Upon receipt of the FACILITY message, the System Simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility Invoke = Get Password

-	GuidanceInfo	(indicating: Enter New Password Again).

The linked ID must be the same as the invoke ID in the invoke of the RegisterPassword operation.

Requirements:

The MS has to transmit the FACILITY message with the expected parameter value.

Upon receipt of the FACILITY message the MS has to provide the appropriate user indication (which is to be described by the manufacturer).

Note:	No indication (on GuidanceInfo) is given to the user in case the standardized MMI functions of GSM 02.30 have been used, and the test continues to the next step without interruption.

Method of test, Step 4:

Unless the standardized MMI functions of GSM 02.30 have been used, the new password is entered again in the MS by means of procedures which are to be described by the manufacturer. The MS has to transmit a FACILITY message with the Facility information element containing a return result of the GetPassword operation with the following parameter:

Facility Return Result = Get Password

-	Password	(the new password).

The invoke ID must be the same as in the invoke of the GetPassword operation.

Upon receipt of the FACILITY message, the System Simulator answers with the RELEASE COMPLETE message with the Facility information element containing a return result of the RegisterPassword operation with the following parameter:

-	Password	(the new password).

The invoke ID must be the same as in the invoke of the RegisterPassword operation.

Requirements:

The MS has to transmit the FACILITY message with the expected parameter value.

Upon receipt of the RELEASE COMPLETE message the MS has to provide the appropriate user indication (which is to be described by the manufacturer).

B)	Registration rejected

B1)	Rejection after invoke of the RegisterPassword operation

Method of test:

The MS is forced to send a REGISTER message in the same way as described for case A), Step 1.

Upon receipt of the REGISTER message the system simulator answers with a RELEASE COMPLETE message with the Facility information element containing a return error with an error or a reject with a problem. The test is repeated with all different errors and problems described below.

For the return error or the reject the invoke ID must be the same as in the invoke of the RegisterPassword operation.

If Facility = Return error

the possible errors are:

-	SS-SubscriptionViolation

-	UnexpectedDataValue

-	SystemFailure.

If Facility = Reject

the possible problems are:

General problems:

-	Unrecognized Component

-	Mistyped Component

-	Badly Structured Component.

Invoke problems:

-	Unrecognized Operation

-	Mistyped Parameter

-	Resource Limitation

-	Initiating Release.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS has to provide the appropriate user indication (which is to be described by the Manufacturer).

B2)	Rejection after password check with negative result

Method of test:

The MS is forced to send a REGISTER message in the same way as described for case A), Step 1, by using the standardized MMI as described in GSM 02.30.

Upon receipt of the REGISTER message, the System Simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility Invoke = Get Password

-	GuidanceInfo	(indicating: Enter Password).

The linked ID must be the same as the invoke ID in the invoke of the RegisterPassword operation.

Upon receipt of the FACILITY message from the MS with the facility information element containing a return result of the GetPassword operation, the system simulator answers with the RELEASE COMPLETE message with the Facility information element containing a return error with the following error:

Facility Return error

-	NegativePasswordCheck.

The invoke ID must be the same as in the invoke of the RegisterPassword operation.

Requirements:

Upon receipt of the RELEASE COMPLETE message the MS has to provide the appropriate user indication (which is to be described by the manufacturer).

B3)	Rejection after use of two different new passwords

Method of test:

The MS is forced to send a REGISTER message in the same way as described for case A), Step 1. However, the new password and its repetition shall be different.

Upon receipt of the REGISTER message, the System Simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility Invoke = Get Password

-	GuidanceInfo	(indicating: Enter Password).

The linked ID must be the same as the invoke ID in the invoke of the RegisterPassword operation.

Note:	No indication (on GuidanceInfo) is given to the user in case the standardized MMI functions of GSM 02.30 have been used, and the test continues without interruption.

Upon receipt of the FACILITY message the MS has to transmit a FACILITY message with the Facility information element containing a return result of the GetPassword operation with the following parameter:

Facility Return Result = Get Password

-	Password	(the current password).

The invoke ID must be the same as in the invoke of the GetPassword operation.

Upon receipt of the FACILITY message, the System Simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility Invoke = Get Password

-	GuidanceInfo	(indicating: Enter New Password).

The linked ID must be the same as the invoke ID in the invoke of the RegisterPassword operation.

Note:	No indication (on GuidanceInfo) is given to the user in case the standardized MMI functions of GSM 02.30 have been used, and the test continues without interruption.

Upon receipt of the FACILITY message the MS has to transmit a FACILITY message with the Facility information element containing a return result of the GetPassword operation with the following parameter:

Facility Return Result = Get Password

-	Password	(the new password).

The invoke ID must be the same as in the invoke of the GetPassword operation.

Upon receipt of the FACILITY message, the System Simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility Invoke = Get Password

-	GuidanceInfo	(indicating: Enter New Password Again).

The linked ID must be the same as the invoke ID in the invoke of the RegisterPassword operation.

Note:	No indication (on GuidanceInfo) is given to the user in case the standardized MMI functions of GSM 02.30 have been used, and the test continues without interruption.

Upon the receipt of the FACILITY message the MS has to transmit a FACILITY message with the Facility information element containing a return result of the GetPassword operation with the following parameter:

Facility Return Result = Get Password

-	Password	(a different new password).

The invoke ID must be the same as in the invoke of the GetPassword operation.

Upon receipt of the FACILITY message, the System Simulator answers with the RELEASE COMPLETE message with the Facility information element containing a return error with the following error:

Facility Return Error

-	PasswordRegistrationFailure.

This error contains the following parameter:

-	Diagnostic	(indicating: NewPasswordMismatch).

The invoke ID must be the same as in the invoke of the RegisterPassword operation.

Requirements:

Upon receipt of the RELEASE COMPLETE message the MS has to provide the appropriate user indication (which is to be described by the manufacturer).

II.12.8.2	Erasure

Causes no signalling on the radio path.

II.12.8.3	Activation

A) 	Activation accepted

General

The following steps shall be executed sequentially.

Brand specific MMI functions which are described by the manufacturer and applicable to Test Step 1 (in addition to the standardized MMI functions as defined in GSM 02.30), shall be tested by repeating Test Step 1 while using these MMI functions.

This whole test case A) shall be repeated using different values for the SS-Code and BasicServiceCode and also with the BasicServiceCode not included. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY. Further repetitions of Test Step 1 for brand specific MMI functions are not required.

Method of test, Step 1:

By means of euther the standardized MMI functions as described in GSM 02.30, or the brand specific MMI the MS is forced to send a REGISTER message with the Facility information element containing an invoke of the ActivateSS operation with the following parameters:

REGISTER

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

Invoke =	Activate SS

Supplementary Service Code (eg BAOC, BOIC, BOIC-exHC, BAIC,

BAIC-Roam)

Basic Service Code.

Note:	If Basic Service Code is not included the SS applies to all Basic Services.

Upon receipt of the REGISTER message, the System Simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility Invoke = Get Password

-	GuidanceInfo	(indicating: Enter Password).

Note:	No indication (on GuidanceInfo) is given to the user in case the standardized MMI functions of GSM 02.30 have been used, and the test continues to the next step without interruption.

Method of test, Step 2:

Unless the standardized MMI functions of GSM 02.30 have been used, the current password is entered in the MS by means of appropriate MMI functions (which are to be described by the manufacturer). The MS has to transmit a FACILITY message with the Facility information element containing a return result of the GetPassword operation with the following parameter:

Facility Return Result = Get Password

-	Password	(the current password).

The invoke ID must be the same as in the invoke of the GetPassword operation.

Upon receipt of the FACILITY message, the System Simulator answers with the RELEASE COMPLETE or FACILITY message with the Facility information element containing a return result of the ActivateSS operation with the following parameter with the same value as above:

RELEASE COMPLETE

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

- Return Result:	Activate SS

Basic Service Code

(Included if included in the invoke).

The invoke ID must be the same as in the invoke of the ActivateSS operation.

Requirements:

For each case tested, the MS has to transmit the REGISTER message containing the invoke of the ActivateSS operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS has to provide the appropriate user indication (which is to be described by the manufacturer).

B)	Activation rejected

B1)	Rejection after invoke of ActivateSS operation

General

Brand specific MMI functions which are described by the manufacturer and applicable to the procedure under test (in addition to the standardized MMI functions as defined in GSM 02.30), shall be tested by repeating the test while using these MMI functions.

This test case B1) shall be repeated with different error and problem indications of the RELEASE COMPLETE message. Further repetitions of the test for brand specific MMI functions are not required.

Method of test:

By means of either the standardized MMI functions as described in GSM 02.30, or the brand specific MMI the MS is forced to send a REGISTER message with the Facility information element containing an invoke of the ActivateSS operation with the following parameters:

Facility Invoke = Activate SS

-	SS-Code	(BAOC, BOIC, BOIC-exHC, BAIC or BAIC-Roam)

-	BasicServiceCode	(Optional).

Upon receipt of the REGISTER message, the System Simulator answers with the RELEASE COMPLETE message with the Facility information element containing a return error with an error or a reject with a problem.

For the return error or the reject the invoke ID must be the same as in the invoke of the ActivateSS operation.

If Facility = Return error

the possible errors are:

-	IllegalSS-Operation

-	SS-ErrorStatus	with parameter SS-Status indicating:

-	Not Provisioned, Not Registered and Not Active or

-	Provisioned, Not Registered and Not Active or

-	Provisioned, Registered and Not Active)

-	SS-NotAvailable

-	SS-SpecificError

-	BearerServiceNotProvisioned

-	TeleserviceNotProvisioned

-	SS-SubscriptionViolation

-	SS-Incompatibility

-	UnexpectedDataValue

-	DataMissing

-	SystemFailure.

If Facility = Reject

the possible problems are:

General problems:

-	Unrecognized Component

-	Mistyped Component

-	Badly Structured Component.

Invoke problems:

-	Unrecognized Operation

-	Mistyped Parameter

-	Resource Limitation

-	Initiating Release.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS has to provide the appropriate user indication (which is to be described by the Manufacturer).

B2)	Rejection after use of password procedure

General

The following steps shall be executed sequentially.

Brand specific MMI functions which are described by the manufacturer and applicable to Test Step 1 (in addition to the standardized MMI functions as defined in GSM 02.30), shall be tested by repeating Test Step 1 while using these MMI functions.

This test case B2) shall be repeated with different error and problem indications of the RELEASE COMPLETE message. Further repetitions of Test Step 1 for brand specific MMI functions are not required.

Method of test, Step 1:

By means of either the standardized MMI functions as described in GSM 02.30, or the brand specific MMI the MS is forced to send a REGISTER message with the Facility information element containing an invoke of the ActivateSS operation with the following parameters:

Facility Invoke = Activate SS

-	SS-Code	(BAOC, BOIC, BOIC-exHC, BAIC or BAIC-Roam)

-	BasicServiceCode	(Optional).

Upon receipt of the REGISTER message, the System Simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility Invoke = Get Password

-	GuidanceInfo	(indicating: Enter Password).

Note:	No indication (on GuidanceInfo) is given to the user in case the standardized MMI functions of GSM 02.30 have been used, and the test continues to the next step without interruption.

Method of test, Step 2:

Unless the standardized MMI functions of GSM 02.30 have been used, the current password is entered in the MS by means of appropriate MMI functions (which are to be described by the manufacturer). The MS has to transmit a FACILITY message with the Facility information element containing a return result of the GetPassword operation with the following parameter:

Facility Return Result = Get Password

-	Password	(the current password).

The invoke ID must be the same as in the invoke of the GetPassword operation.

Upon receipt of the FACILITY message, the System Simulator answers with the RELEASE COMPLETE message with the Facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject, the invoke ID must be the same as in the invoke of the ActivateSS operation.

If Facility = Return Error

the possible errors are:

-	NegativePasswordCheck

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure.

If Facility = Reject

the possible problems are:

General problems:

-	Unrecognized Component

-	Mistyped Component

-	Badly Structured Component.

Return result problems:

-	Unrecognized Invoke ID

-	Mistyped Parameter.

Requirements:

For each case tested, the MS has to transmit the REGISTER message containing the invoke of the ActivateSS operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE message, the MS has to provide the appropriate user indication (which is to be described by the manufacturer).

II.12.8.4	Deactivation

A)	Deactivation accepted

General

The following steps shall be executed sequentially.

Brand specific MMI functions which are described by the manufacturer and applicable to Test Step 1 (in addition to the standardized MMI functions as defined in GSM 02.30), shall be tested by repeating Test Step 1 while using these MMI functions.

This test case A) shall be repeated using the 2 different values for the SS-Code and BasicServiceCode and also with the BasicServiceCode not included. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY. Further repetitions of Test Step 1 for brand specific MMI functions are not required.

Method of test, Step 1:

By means of either the standardized MMI functions as described in GSM 02.30, or the brand specific MMI, the MS is forced to send a REGISTER message with the Facility information element containing an invoke of the DeactivateSS operation with the following parameters:

REGISTER

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

Invoke =	Deactivate SS

Supplementary Service Code (BO or BI)

Basic service code.

Note:	If Basic Service Code is not included the SS applies to all Basic Services.

Upon receipt of the REGISTER message, the System Simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility Invoke = Get Password

-	GuidanceInfo	(indicating: Enter Password).

Note:	No indication (on GuidanceInfo) is given to the user in case the standardized MMI functions of GSM 02.30 have been used, and the test continues to the next step without interruption.

Method of test, Step 2:

Unless the standardized MMI functions of GSM 02.30 have been used, the current password is entered in the MS by means of appropriate MMI functions (which are to be described by the manufacturer). The MS has to transmit a FACILITY message with the Facility information element containing a return result of the GetPassword operation with the following parameter:

Facility Return Result = Get Password

-	Password	(the current password).

The invoke ID must be the same as in the invoke of the GetPassword operation.

Upon receipt of the FACILITY message, the System Simulator answers with the RELEASE COMPLETE or FACILITY message with the Facility information element containing a return result of the DeactivateSS operation with the following parameter with the same value as above:

RELEASE COMPLETE

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

Return Result = Deactivation

Basic Service Code (Included if included in the invoke).

The invoke ID must be the same as in the invoke of the DeactivateSS operation.

Requirements:

For each case tested, the MS has to transmit the REGISTER message containing the invoke of the DeactivateSS operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS has to provide the appropriate user indication (which is to be described by the manufacturer).

B)	Deactivation rejected

B1)	Rejection after invoke of DeactivateSS operation

General

Brand specific MMI functions which are described by the manufacturer and applicable to the procedure under test (in addition to the standardized MMI functions as defined in GSM 02.30), shall be tested by repeating the test while using these MMI functions.

This test case B1) shall be repeated with different error and problem indications of the RELEASE COMPLETE message. Further repetitions of the test for brand specific MMI functions are not required.

Method of test:

By means of either the standardized MMI functions as described in GSM 02.30, or the brand specific MMI, the MS is forced to send a REGISTER message with the Facility information element containing an invoke of the DeactivateSS operation with the following parameters:

Facility Invoke = Deactivate SS

-	SS-Code (BO or BI)

-	BasicServiceCode (Optional).

Upon receipt of the FACILITY message, the System Simulator answers with the RELEASE COMPLETE message with the Facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject the invoke ID must be the same as in the invoke of the DeactivateSS operation.

RELEASE COMPLETE

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

Return error	= error code

Or Reject		= problem code.

The possible errors are:

-	IllegalSS-Operation

-	SS-ErrorStatus	with parameter SS-Status indicating:

-	Not Provisioned, Not Registered and Not Active or

-	Provisioned, Not Registered and Not Active or

-	Provisioned, Registered and Active

-	SS-NotAvailable

-	SS-SpecificError

-	BearerServiceNotProvisioned

-	TeleserviceNotProvisioned

-	SS-SubscriptionViolation

-	SS-Incompatibility

-	UnexpectedDataValue

-	DataMissing

-	SystemFailure.

The possible problems are:

General problems:

-	Unrecognized Component

-	Mistyped Component

-	Badly Structured Component.

Invoke problems:

-	Unrecognized Operation

-	Mistyped Parameter

-	Resource Limitation

-	Initiating Release.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS has to provide the appropriate user indication (which is to be described by the Manufacturer).

B2)	Rejection after use of password procedure

General

The following steps shall be executed sequentially.

However, if the MS returns to the start of this test case B2) by sending the REGISTER message containing:

Facility Invoke = Deactivate SS

then the System Simulator shall repeat this test case B2).

Brand specific MMI functions which are described by the manufacturer and applicable to Test Step 1 (in addition to the standardized MMI functions as defined in GSM 02.30), shall be tested by repeating Test Step 1 while using these MMI functions.

This test case B2) shall be repeated with different error and problem indications of the RELEASE COMPLETE message. Further repetitions of the test for brand specific MMI functions are not required.

Method of test, Step 1:

By means of standardized MMI functions as described in GSM 02.30, including any password here, (and repeated for additional procedures which are to be described by the manufacturer) the MS is forced to send a REGISTER message with the Facility information element containing an invoke of the DeactivateSS operation with the following parameters:

Facility Invoke = Deactivate SS

-	SS-Code (BO or BI)

-	BasicServiceCode (Optional).

Upon receipt of the REGISTER message, the System Simulator answers with the FACILITY message with the Facility information element containing an invoke of the GetPassword operation with the following parameter:

Facility Invoke = Get Password

-	GuidanceInfo	(indicating: Enter Password).

Note:	No indication (on GuidanceInfo) is given to the user in case the standardized MMI functions of GSM 02.30 have been used, and the test continues to the next step without interruption.

Method of test, Step 2:

Unless the standardized MMI functions of GSM 02.30 have been used, the current password is entered in the MS by means of appropriate MMI functions (which are to be described by the manufacturer). The MS has to transmit a FACILITY message with the Facility information element containing a return result of the GetPassword operation with the following parameter:

Facility Return Result = Get Password

-	Password	(the current password).

The invoke ID must be the same as in the invoke of the GetPassword operation.

Upon receipt of the FACILITY message, the System Simulator answers with the RELEASE COMPLETE message with the Facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject,the invoke ID must be the same as in the invoke of the DeactivateSS operation.

If Facility = Return Error

the possible errors are:

-	NegativePasswordCheck

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure.

If Facility = Reject

the possible problems are:

General problems:

-	Unrecognized Component

-	Mistyped Component

-	Badly Structured Component.

Return result problems:

-	Unrecognized Invoke ID

-	Mistyped Parameter.

Requirements:

For each case tested, the MS has to transmit the REGISTER message containing the invoke of the DeactivateSS operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE message, the MS has to provide the appropriate user indication (which is to be described by the manufacturer).

II.12.8.5	Invocation

Causes no signalling on the radio path.

II.12.8.6	Interrogation

A)	Interrogation accepted

Method of test:

By means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30 the MS is forced to send a REGISTER message with the Facility information element containing an invoke of the InterrogateSS operation with the following parameter:

REGISTER

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

invoke :	interrogation

SS code (BAOC, BOIC, BOIC-exHC, BAIC or BAIC-Roam).

Upon receipt of the REGISTER message, the System Simulator answers with the RELEASE COMPLETE or FACILITY message with the Facility information element containing a return result of the InterrogateSS operation with zero, one or more than one number of the following parameter:

RELEASE COMPLETE

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

Return Result =	Interrogation

Basic Service Code n°1

Basic Service Code n°2

etc.

The invoke ID must be the same as in the invoke of the InterrogateSS operation.

The test is repeated using different values of the SS-Code and different numbers and values of the BasicServiceCode. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

For each case tested, the MS has to transmit the REGISTER message containing the invoke of the InterrogateSS operation with the expected parameter value.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS has to provide the appropriate user indication (which is to be described by the manufacturer).

B)	Interrogation rejected

Method of test:

The MS is forced to send the REGISTER message in the same way as described for case A).

Upon receipt of the REGISTER message the system simulator answers with a RELEASE COMPLETE message with the Facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject the invoke ID must be the same as in the invoke of the InterrogateSS operation. For the reject the invoke ID may be the same as in the invoke of the InterrogateSS operation but the invoke ID tag can also be universal null together with the invoke ID length = 0.

RELEASE COMPLETE

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

Return Error =	Error Code

Or Reject = problem code.

The possible errors are:

-	IllegalSS-Operation

-	SS-NotAvailable

-	SS-SpecificError

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure.

The possible problems are:

General problems:

-	Unrecognized Component

-	Mistyped Component

-	Badly Structured Component.

Invoke problems:

-	Unrecognized Operation

-	Mistyped Parameter

-	Resource Limitation

-	Initiating Release.

The test is repeated with different errors and problems.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS has to provide the appropriate user indication (which is to be described by the Manufacturer).

II.12.8.7	Normal operation

In case of barring of outgoing call the calling mobile receives information about the activation of SS subscribed.

In case of barring of incoming call the calling mobile receives information about the activation of SS subscribed by the other party (the mobile called).

Method of test:

The MS is forced to place an outgoing call.

Upon receipt of the SET UP message, the system simulator answers either with the negative acknowledgment RELEASE COMPLETE (to simulate a case where call barring is activated).

RELEASE COMPLETE

-	protocol discriminator

-	transaction identifier

-	message type

-	facility

invoke =	notification

SS code = BO or BI

SS status = activation indicator (indicating :

Provisioned, Registered and Active)

The test is repeated with BO then BI.

Requirements:

For each case tested, the MS has to provide the appropriate user indication (which is to be described by the manufacturer).

II.12.9	Handling of undefined (future) GSM supplementary services

The MMI procedures for handling of GSM supplementary services not yet defined are described in GSM 02.30.

The ProcessUnstructuredSsData operation to be used on the radio path for such services is defined in GSM 09.02 and 04.80.

When the MS detects a MMI procedure for an SS where the coding of MMI and information elements is not yet specifically defined in GSM, it generates a REGISTER message with the Facility information element containing an Invoke of the Process Unstructured SS Data operation.

II.12.9.1	Test sequences

In sections II.12.9.2 to II.12.9.7 the test procedures for registration, erasure, activation, deactivation, interrogation and operations not yet defined in GSM 02.30 are described.

For registration, erasure, activation, deactivation and interrogation, the test sequence below is executed for a set of service codes NN(N) which have not yet been allocated for GSM supplementary services (see GSM 02.30 for service codes already specified).

This set of service codes is: 00, 000, 999 and two service codes randomly chosen in the range 01 to 99 and 001 to 998 (one service code in each range). In that random selection, service codes for which explicit tests are carried out in earlier tests of section II.12 should not be selected.

Registration rejected

Registration accepted

Erasure rejected

Activation rejected

Activation accepted

Interrogation accepted

Interrogation rejected

Deactivation rejected

Deactivation accepted

Erasure accepted.

For operations not yet defined by GSM 02.30 the following test sequence is executed:

-	Operation accepted (see section II.12.9.7)

-	Operation rejected (see section II.12.9.7).

For each test in a test sequence the MS is forced to send a REGISTER message with the Facility information element containing an invoke of the ProcessUnstructuredSsData operation with the following parameter:

-	SS-UserData.

The SS-UserData parameter will in an IA5String contain the keystrokes defined in the test case.

For Registration, Erasure, Activation, Interrogation and Deactivation, each test in a test sequence characterized by the service code NN(N) shall be executed three times:

-	with no supplementary information

-	with one supplementary information field Si containing a value chosen at random

-	with two supplementary information fields Sia and Sib both containing values chosen at random.

Besides this, each test in a test sequence shall be repeated a number of times as described in sections II.12.9.2 to II.12.9.6.

II.12.9.2	Registration

A)	Registration accepted

Method of test:

By means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30 the MS is forced to send a REGISTER message with the Facility information element containing an invoke of the ProcessUnstructuredSsData operation with the following parameter:

Facility Invoke = Process Unstructured SS Data

-	SS-UserData.

The SS-UserData parameter shall in an IA5String contain the following digits and symbols:

**NN(N)#		(No supplementary information included)

or	**NN(N)*Si#	(One field of supplementary information included)

or	**NN(N)*Sia*Sib#	(Two fields of supplementary information included).

Upon receipt of the REGISTER message, the System Simulator answers with the RELEASE COMPLETE or FACILITY message with the Facility information element containing a return result of the ProcessUnstructuredSsData operation with the following parameter:

Facility Return Result = Process Unstructured SS data

-	SS-UserData	(Optional).

The SS-UserData parameter may contain information in an IA5String.

The invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

The test is repeated with and without the SS-UserData in the return result. When SS-UserData is included in the return result the content of the IA5String is chosen at random. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

For each case tested, the MS has to transmit the REGISTER message containing the invoke of the ProcessUnstructuredSsData operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS has to provide the appropriate user indication (which is to be described by the manufacturer). If IA5String information is included this shall be given to the user (in a way described by the manufacturer).

B)	Registration rejected

Method of test:

The MS is forced to send the REGISTER message in the same way as described for case A).

Upon receipt of the REGISTER message the system simulator answers with a RELEASE COMPLETE message with the Facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject the invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

If Facility = Return Error

The possible errors are:

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure.

If Facility = Reject

The possible problems are:

General problems:

-	Unrecognized Component

-	Mistyped Component

-	Badly Structured Component.

Invoke problems:

-	Unrecognized Operation

-	Mistyped Parameter

-	Resource Limitation

-	Initiating Release.

The test is repeated with different errors and problems.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS has to provide the appropriate user indication (which is to be described by the Manufacturer).

II.12.9.3	Erasure

A)	Erasure accepted

Method of test:

By means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30 the MS is forced to send a REGISTER message with the Facility information element containing an invoke of the ProcessUnstructuredSsData operation with the following parameter:

Facility Invoke = Process Unstructured SS Data

-	SS-UserData.

The SS-UserData parameter shall in an IA5String contain the following digits and symbols:

##NN(N)#	(No supplementary information included)

or	##NN(N)*Si#	(One field of supplementary information included)

or	##NN(N)*Sia*Sib#	(Two fields of supplementary information included).

Upon receipt of the REGISTER message, the System Simulator answers with the RELEASE COMPLETE or FACILITY message with the Facility information element containing a return result of the ProcessUnstructuredSsData operation with the following parameter:

Facility Return Result = Process Unstructured SS Data

-	SS-UserData	(Optional).

The SS-UserData parameter may contain information in an IA5String.

The invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

The test is repeated with and without the SS-UserData in the return result. When SS-UserData is included in the return result the content of the IA5String is chosen at random. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

For each case tested, the MS has to transmit the REGISTER message containing the invoke of the ProcessUnstructuredSsData operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS has to provide the appropriate user indication (which is to be described by the manufacturer). If IA5String information is included this shall be given to the user (in a way described by the manufacturer).

B)	Erasure rejected

Method of test:

The MS is forced to send the REGISTER message in the same way as described for case A).

Upon receipt of the REGISTER message the System Simulator answers with a RELEASE COMPLETE message with the Facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject the invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

If Facility = Return error

The possible errors are:

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure.

If Facility = Reject

The possible problems are:

General problems:

-	Unrecognized Component

-	Mistyped Component

-	Badly Structured Component.

Invoke problems:

-	Unrecognized Operation

-	Mistyped Parameter

-	Resource Limitation

-	Initiating Release.

The test is repeated with different errors and problems.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS has to provide the appropriate user indication (which is to be described by the Manufacturer).

II.12.9.4	Activation

A)	Activation accepted

Method of test:

By means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30 the MS is forced to send a REGISTER message with the Facility information element containing an invoke of the ProcessUnstructuredSsData operation with the following parameter:

Facility Invoke = Process Unstructured SS Data

-	SS-UserData.

The SS-UserData parameter shall in an IA5String contain the following digits and symbols:

*NN(N)#		(No supplementary information included)

or	*NN(N)*Si#		(One field of supplementary information included)

or	*NN(N)*Sia*Sib#	(Two fields of supplementary information included).

Upon receipt of the REGISTER message, the system simulator answers with the RELEASE COMPLETE or FACILITY message with the Facility information element containing a return result of the ProcessUnstructuredSsData operation with the following parameter:

Facility Result = Process Unstructured SS data

-	SS-UserData	(Optional).

The SS-UserData parameter may contain information in an IA5String.

The invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

The test is repeated with and without the SS-UserData in the return result. When SS-UserData is included in the return result the content of the IA5String is chosen at random. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

For each case tested, the MS has to transmit the REGISTER message containing the invoke of the ProcessUnstructuredSsData operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS has to provide the appropriate user indication (which is to be described by the manufacturer). If IA5String information is included this shall be given to the user (in a way described by the manufacturer).

B)	Activation rejected

Method of test:

The MS is forced to send the REGISTER message in the same way as described for case A).

Upon receipt of the REGISTER message the System Simulator answers with a RELEASE COMPLETE message with the Facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject the invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

If Facility = Return Error

The possible errors are:

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure.

If Facility = Reject

The possible problems are:

General problems:

-	Unrecognized Component

-	Mistyped Component

-	Badly Structured Component.

Invoke problems:

-	Unrecognized Operation

-	Mistyped Parameter

-	Resource Limitation

-	Initiating Release.

The test is repeated with different errors and problems.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS has to provide the appropriate user indication (which is to be described by the Manufacturer).

II.12.9.5	Deactivation

A)	Deactivation accepted

Method of test:

By means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30 the MS is forced to send a REGISTER message with the Facility information element containing an invoke of the ProcessUnstructuredSsData operation with the following parameter:

Facility Invoke = Process Unstructured SS Data

-	SS-UserData.

The SS-UserData parameter shall in an IA5String contain the following digits and symbols:

#NN(N)#		(No supplementary information included)

or	#NN(N)*Si#	(One field of supplementary information included)

or	#NN(N)*Sia*Sib#	(Two fields of supplementary information included).

Upon receipt of the REGISTER message, the System Simulator answers with the RELEASE COMPLETE or FACILITY message with the Facility information element containing a return result of the ProcessUnstructuredSsData operation with the following parameter:

Facility Return Result = Process Unstructured SS Data

-	SS-UserData	(Optional).

The SS-UserData parameter may contain information in an IA5String.

The invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

The test is repeated with and without the SS-UserData in the return result. When SS-UserData is included in the return result the content of the IA5String is chosen at random. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

For each case tested, the MS has to transmit the REGISTER message containing the invoke of the ProcessUnstructuredSsData operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS has to provide the appropriate user indication (which is to be described by the manufacturer). If IA5String information is included this shall be given to the user (in a way described by the manufacturer).

B)	Deactivation rejected

Method of test:

The MS is forced to send the REGISTER message in the same way as described for case A).

Upon receipt of the REGISTER message the System Simulator answers with a RELEASE COMPLETE message with the Facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject the invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

If Facility = return error

The possible errors are:

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure.

If Facility = reject

The possible problems are:

General problems:

-	Unrecognized Component

-	Mistyped Component

-	Badly Structured Component.

Invoke problems:

-	Unrecognized Operation

-	Mistyped Parameter

-	Resource Limitation

-	Initiating Release.

The test is repeated with different errors and problems.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS has to provide the appropriate user indication (which is to be described by the Manufacturer).

II.12.9.6	Interrogation

A)	Interrogation accepted

Method of test:

By means of appropriate MMI functions as defined by the basic public MMI described in GSM 02.30 the MS is forced to send a REGISTER message with the Facility information element containing an invoke of the ProcessUnstructuredSsData operation with the following parameter:

Facility Invoke = Process Unstructured SS Data

-	SS-UserData.

The SS-UserData parameter shall in an IA5String contain the following digits and symbols:

*#NN(N)#		(No supplementary information included)

or	*#NN(N)*Si#	(One field of supplementary information included)

or	*#NN(N)*Sia*Sib#	(Two fields of supplementary information included).

Upon receipt of the REGISTER message, the System Simulator answers with the RELEASE COMPLETE or FACILITY message with the Facility information element containing a return result of the ProcessUnstructuredSsData operation with the following parameter:

Facility Return Result = Process Unstructured SS Data

-	SS-UserData	(Optional).

The SS-UserData parameter may contain information in an IA5String.

The invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

The test is repeated with and without the SS-UserData in the return result.

When SS-UserData is included in the return result the content of the IA5 string is chosen at random.

The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

For each case tested, the MS has to transmit the REGISTER message containing the invoke of the ProcessUnstructuredSsData operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS has to provide the appropriate user indication (which is to be described by the manufacturer). If IA5String information is included this shall be given to the user (in a way described by the manufacturer).

B)	Interrogation rejected

Method of test:

The MS is forced to send the REGISTER message in the same way as described for case A).

Upon receipt of the REGISTER message the System Simulator answers with a RELEASE COMPLETE message with the Facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject the invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

If Facility = Return Error

The possible errors are:

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure.

If Facility = Reject

The possible problems are:

General problems:

-	Unrecognized Component

-	Mistyped Component

-	Badly Structured Component.

Invoke problems:

-	Unrecognized Operation

-	Mistyped Parameter

-	Resource Limitation

-	Initiating Release.

The test is repeated with different errors and problems.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS has to provide the appropriate user indication (which is to be described by the Manufacturer).

II.12.9.7	Operations not yet specified in GSM 02.30

General test of transparency of MMI inputs

A)	Operation accepted

Method of test:

The following tests shall be performed within a call.

By means of a specified MMI keypad operation (defined below) the MS is forced to send a REGISTER or FACILTY message with the Facility information element containing an invoke of the ProcessUnstructuredSsData operation with the following parameter:

Facility Invoke = ProcessUnstructuredSsData

-	SS-UserData.

The SS-UserData parameter shall in an IA5 string contain exactly the digits and symbols expressed on the mobile equipment keypad. One test for each of the below expressed keypad procedures shall be performed.

MMI key depressions		IA5 string contents

0 SEND (note 1)			0

1 SEND 			1

2 SEND			2

:				:

:				:

9 SEND			9

Note 1:	The keyword "SEND" refers to the SEND function as defined in GSM 02.30.

Upon receipt of the REGISTER or FACILITY message, the system simulator answers with the RELEASE COMPLETE or FACILITY message with the Facility information element containing a return result of the ProcessUnstructuredSsData operation with the following parameter:

Facility Return Result = ProcessUnstructuredSsData

-	SS-UserData (optional).

The SS-UserData parameter may contain information in an IA5 string.

The Invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

The test is repeated with and without the SS-UserData in the return result. When SS-UserData is included in the return result the content of the IA5 string is chosen at random. The test is also repeated using different answer messages RELEASE COMPLETE or FACILITY.

Requirements:

For each case tested, the MS has to transmit the REGISTER or FACILITY message containing the invoke of the ProcessUnstructuredSsData operation with the expected parameter values.

Upon receipt of the RELEASE COMPLETE or FACILITY message, the MS has to provide the appropriate user indication (which is to be described by the manufacturer). If IA5 information is included this shall be given to the user (in a way described by the manufacturer).

B)	Operation rejected

Method of test:

The MS is forced to send the REGISTER or FACILITY message in the same way as described for case A).

Upon receipt of the REGISTER or FACILITY message, the system simulator answers with the RELEASE COMPLETE message with the Facility information element containing a return error with an error or a reject with a problem.

For the return error and the reject the invoke ID must be the same as in the invoke of the ProcessUnstructuredSsData operation.

If Facility = Return Error

The possible errors are:

-	DataMissing

-	UnexpectedDataValue

-	SystemFailure.

If facility = reject

The possible problems are:

General problems:

-	Unrecognised Component

-	Mistyped Component

-	Badly Structured Component.

Invoke Problems:

-	Unrecognised Operation

-	Mistyped Parameter

-	Resource Limitation

-	Initiating Release.

The test is repeated with different errors and problems.

Requirements:

Upon receipt of the RELEASE COMPLETE message, the MS has to provide the appropriate user indication (which is to be described by the manufacturer).