II.3	Transmitter

Ref GSM 05.05

II.3.1	Phase error and frequency error

The GSM radio subsystem uses a GMSK modulation system with an equivalent pre modulation Gaussian filter having a bandwidth defined by B * T = 0.3, where B is the filter bandwidth and T is the modulation symbol time.

In order to measure the accuracy of the modulation process a sampled measurement of the transmitted phase trajectory is obtained. This is compared with the theoretically expected phase trajectory. The regression line of the difference between the expected trajectory and the measured trajectory is an indication of the frequency error, whilst the departure of the phase differences from this trajectory is a measure of the phase error.

II.3.1.1	Method of measurement

a)	The MS is connected to the SS. This connection will be to the permanent antenna connector for a MS which is equipped with one or shall use a modified sample fitted with a temporary antenna connector as defined in Annex GC7 for a MS with an integral antenna and not normally having means of connecting an external antenna.

b)	The SS shall originate a call to the MS and the MS shall be made to answer the call. The SS shall command the MS to hopping mode. The hopping bandwidth shall be according to GC1 of Annex 1. The SS shall activate ciphering mode.

Note:	Ciphering mode is active during that test to give a pseudo-random bit stream to the modulator.

c)	The SS shall command the MS to complete the traffic channel loop back (channel decoder output to channel encoder input). The SS shall generate Standard Test Signal C1.

d)	For each transmitted burst, the receiving section of the SS shall capture the signal as a series of phase samples over the period of the burst. These samples shall be evenly distributed over the duration of the burst with a minimum sampling rate of 2/T, where T is the modulation symbol period. The received phase trajectory is then represented by this array of at least 294 samples.

e)	The SS must then calculate, from the known bit pattern and the formal definition of the modulator contained in GSM 05.04, the expected phase trajectory.

f)	From d) and e) the phase trajectory error is calculated, and a linear regression line plotted through this phase trajectory error. The slope of this regression line is the frequency error of the mobile transmitter relative to the simulator reference. The difference between the regression line and the individual sample points is the phase error of that point.

f.1)	The sampled array of at least 294 phase measurements is represented by the vector

∞m = ∞m(0)...∞m(n)

where the number of samples in the array n+1 >= 294.

f.2)	The calculated array, at the corresponding sampling instants, is represented by the vector

∞c = ∞c(0)...∞c(n).

f.3)	The error array is represented by the vector:

∞e = {∞m(0) - ∞c(0)}.........{∞m(n) - ∞c(n)} = ∞e(0)...∞e(n).

f.4)	The corresponding sample numbers form a vector t = t(0)...t(n).

[f.5)	Plot a scattergram with the abscissa represented by the linear values of the vector t and the ordinate by the corresponding value of ∞e.]

[f.6)	This scattergram can now be considered as a graph of the equation ∞e = k * t, and] by regression theory:

j=n

S t(j) * ∞e(j)

j=0

k = 	────────────────

j=n

S t(j)2

j=0

f.7)	The frequency error is given by k/(360 * g), where g is the sampling interval in s and all phase samples are measured in degrees.

f.8)	The individual phase errors from the regression line are given by

∞e(j) - k*t(j).

f.9)	The RMS value ∞E of the phase errors is given by:

┌			┐1/2

|	j=n		|

|	S {∞e(j) - k*t(j)}2	|

∞E(RMS) =	|	j=0		|

|	───────────	|

|		n+1	|

└			┘

g)	Steps d) to f) shall be repeated for 20 bursts, not necessarily contiguous.

h)	The SS shall instruct the MS to its maximum power level, all other conditions remaining constant. Steps d) to g) shall be repeated.

i)	The SS shall instruct the MS to power level

GSM900:	15

DCS1800:	10 for class 1 MS or 13 for class 2 MS,

all other conditions remaining constant. Steps d) to g) shall be repeated.

j)	The MS shall be hard mounted on a vibration table and vibrated at the frequency/amplitudes specified in Annex 1, TC4.

During the vibration steps a) to i) shall be repeated.

Note:	If the MS does not have an antenna connector, then the MS and coupling device will need to be mounted together onto the vibration table and the test performed under vibration.

k)	The MS shall be repositioned on the vibration table in the two orthogonal planes to the plane used in step j). For each of the orthogonal planes step j) shall be repeated.

l)	Steps d) to i) are repeated under extreme test conditions (see Annex 1, TC2.2 and TC3)

Note:	The series of samples taken to determine the phase trajectory could also be used, with different post-processing, to determine the transmitter burst characteristics of II.3.3. Although described independently, it is at the discretion of the test house whether to combine the tests of II.3.1 and II.3.3, giving both answers from single sets of captured data.

II.3.1.2	Limits

II.3.1.2.1	Frequency error

The frequency error, derived in step f.7), for all measured bursts shall be less than 10E-7.

II.3.1.2.2	Phase error

For every burst, the RMS phase error from step f.9) shall be less than 5 degrees.

An individual phase error from step f.8) shall not exceed 20 degrees.

II.3.2	Frequency error under multipath and interference conditions

Ref. GSM 05.10 section 6

The Mobile Station is required to maintain frequency synchronization with the received signal under conditions of Doppler shift, multipath reception and interference.

By using the techniques of section II.3.1 this section checks the correct functioning of this feature under various conditions.

Note:	GSM 05.10 requires that frequency synchronization should be maintained for input signals 3 dB below reference sensitivity. Due to the Radio Link Failure counter this test condition can not be established. Hence all tests in this section are conducted at reference sensitivity level.

II.3.2.1	Method of measurement

a)	A call shall be set up by the SS according to the generic call set up procedure on a channel:

GSM900	DCS1800

ARFCN range	60 to 65	690 to 710

SS fading function	RA250	RA130

input level dBmVemf(  )	20	22

on serving cell BCCH

The SS shall capture the first transmission burst from the MS during call establishment.

The SS shall also set up six adjacent cell BCCHs having signal levels in the range

GSM900:	14 to 60 dBmVemf(  )

DCS1800:	19 to 60 dBmVemf(  ).

The exact level and ARFCN of each signal is not critical, but some simple rules need to be observed:

-	No adjacent cell BCCH shall be set on ARFCN close to the serving cell BCCH or TCH (e.g. five channels separated) if it is set to maximum level.

-	Of the six adjacent cell BCCHs,

GSM900:	two shall be located near the band edges.

DCS1800:	one should be located near each band edge.

The amplitude and ARFCN of all six could with benefit be varied continuously throughout the test, but observing the simple rules given here.

b)	The SS shall set the serving cell BCCH and TCH to

GSM900:	11 dBmVemf(  ) for handheld MSs and 9 dBmVemf(  ) for all other MSs

DCS1800:	13 dBmVemf(  ).

Wait 30 s for the MS to stabilize to these conditions.

c)	The SS shall capture subsequent bursts from the traffic channel in the manner described in II.3.1.1. Throughout the test the SS shall monitor the status message from the MS to verify that the adjacent cell BCCHs are being correctly measured by the MS.

d)	Due to the very low signal level at the MS receiver input the MS receiver is liable to error.

The "looped back" bits are therefore also liable to error, and hence the SS does not know the expected bit sequence.

From the received burst from the MS transmitter the SS must demodulate the signal to derive (error free) the transmitter burst bit pattern. Using this bit pattern the SS can calculate the expected phase trajectory according to the definition within GSM 05.04.

e)	From c) and d) the phase error trajectory is determined and the regression line calculated. This regression line is used to determine the frequency error for the single burst.

f)	Steps c) to e) are repeated for 5 traffic channel bursts spaced over a period of not less than 20 s.

g)	The SS shall release the call.

The SS increases the level of the serving cell BCCH to

GSM900:	20 dBmVemf(  )

DCS1800:	22 dBmVemf(  ).

and sets up a call on the traffic channel with an MS input level of

GSM900:	20 dBmVemf(  )

DCS1800:	22 dBmVemf(  ).

with the fading function set to HT100.

The SS shall capture the first transmitted burst from the MS during call establishment.

h)	Repeat steps b) to f).

i)	The SS shall release the call.

The SS increases the level of the serving cell BCCH to

GSM900:	20 dBmVemf(  )

DCS1800:	22 dBmVemf(  ).

and sets up a call on the traffic channel with an MS input level of

GSM900:	20 dBmVemf(  )

DCS1800:	22 dBmVemf(  ).

with the fading function set to TU50.

The SS shall capture the first transmitted burst from the MS during call establishment.

j)	Repeat steps b) to f).

k)	The SS shall release the call.

The SS increases the level of the serving cell BCCH to

GSM900:	28 dBmVemf(  )

DCS1800:	30 dBmVemf(  ).

and sets up a call on the traffic channel with an MS input level of

GSM900:	28 dBmVemf(  ), with each channel applied to the MS

via the SS fading function set at TU3.

DCS1800:	30 dBmVemf(  ), with each channel applied to the MS

via the SS fading function set at TU1.5.

The SS shall capture the first transmitted burst from the MS during call establishment.

l)	The SS shall now apply to the MS two independent interfering signals on the same nominal carrier frequencies as the active channels. These interfering signals shall be at a level of

GSM900:	18 dBmVemf(  ) each via a fading function set to TU3

DCS1800:	21 dBmVemf(  ), each via a fading function set to TU1.5.

and shall be modulated with random data, including the midamble period.

m)	Wait 100 s for the MS to stabilize to these conditions.

n)	Repeat steps c) to f), except that at step f) the measurement period must be extended to 200 s and the number of measurements increased to 20.

o)	Steps a) to n) are repeated for ARFCN in the Low ARFCN range (ref. Table 1.4) and High ARFCN range.

p)	Repeat steps i) and j) under extreme test conditions (see Annex 1, TC2.2 and TC3).

II.3.2.2	Limits

For all test conditions specified in II.3.2.1 the frequency error, with reference to the SS carrier frequency, for every burst shall be less than the values shown in the table below.

GSM900	DCS1800

Propagation	Permitted	Propagation	Permitted

condition	frequency error	condition	frequency error

RA250	+/- 300 Hz	RA130	+/- 400 Hz

HT100	+/- 175 Hz	HT100	+/- 350 Hz

TU50	+/- 135 Hz	TU50	+/- 260 Hz

TU3	+/- 95 Hz	TU1.5	+/- 180 Hz

II.3.3	Peak transmitter carrier power and burst timing

ref: GSM 05.05 section 4.1

II.3.3.1	Definition

The transmitter carrier power is the power delivered to an artificial antenna or radiated by the MS and its integral antenna.

The peak transmitter carrier power is the average value of the transmitter carrier power over the time that the useful information bits of one burst are transmitted.

The peak received transmitter carrier power is the peak transmitter carrier power, attenuated by propagation and as received by the SS during measurements on equipment with an integral antenna.

The peak transmitter carrier power is defined for each power control level at which a MS may operate.

The power class (the class of maximum peak transmitter carrier power) shall be stated by the manufacturer.

The power control level is a parameter which indicates a specific value for the peak transmitter carrier power.

The transmit burst under reference conditions should be timed to occur 3 burst periods (Timing Advance = 0) after the corresponding received burst. The timings are referenced to the transition from bit 13 to bit 14 of the Training Sequence ("midamble") before differential decoding.

[Note:	Within the current standard, also an ME implementation with a deviation of the burst by 1/2 bit prior to the defined timing reference is acceptable. In this case the manufacturer has to notify the test house. For type testing to the current standard, this is taken into account by considering the uncertainty margins within the SS as if extended in front of the raising and the falling edge each by 1/2 bit (1.84 ms) as shown in Figure App.3-1 of GSM 11.40, Appendix 3. In case of such an ME failing the test, the test house will have to revise the test result of the SS to assess the appropriate verdict to be given in the Test Report.]

II.3.3.2	Methods of measurement

Two methods of measurement are described, separately for:

1)	equipment fitted with a permanent antenna connector and for

2)	equipment fitted with an integral antenna, and which cannot be connected to an external antenna except by the fitting of a temporary test connector as a test fixture.

II.3.3.2.1	Method of measurement for equipment with a permanent antenna connector

a)	A call shall be set up by the SS according to the generic call set up procedure on a channel with ARFCN in the Mid ARFCN range (ref Table 1.4), power control level set to Max power.

+4 dB	|        ┌───┐

+1.0 dB	|        |   └─────────────/ /──────────────────┐

0 dB	|        |                                      |

-1.0 dB	|        |   ┌─────────────/ /──────────────┐   |

|        |   |                              |   |

-6 dB	|     ┌──┘   |                              |   └──┐

|     |      |                              |      |

-30 dB	| ┌───┘      |                              |      └───┐

| |          |                              |          |

| |          |                              |          |

-70 dB	|─┘          |                              |          └

|            |<-------- (147 bits) -------->|

└──────────────────────────/ /──────────────────────────

10  8  10        7056/13=542.8 ms        10  8  10

Figure II.3.3 : Power / time template

GSM900 only - begin

For a transmitter carrier power lower than 34 dBm, where the level of -70 dB is lower than - 36 dBm, the -70 dB shall be replaced by a value which is equivalent to - 36 dBm.

GSM900 only - end

DCS1800 only - begin

For a transmitter carrier power lower than 23 dBm, where the level of -70 dB is lower than - 47 dBm, the -70 dB shall be replaced by a value which is equivalent to - 47 dBm.

DCS1800 only - end

b)	Measure Peak Transmitter Carrier Power

Using a sampling power measurement method with a sampling rate of at least 2/T, where T is bit duration, capture a representation of the MS transmit burst's amplitude and timing. From the array of samples the SS identifies the centre of the useful 147 transmitted bits, i.e. the transition from bit 13 to bit 14 of the midamble, as the timing reference. This enables the peak transmitted carrier power over the 147 useful bits to be calculated and used as the 0 dB reference.

c)	Measure Burst Timing

The transition point identified in b) above is referred to the corresponding transition in the MS received burst.

d)	Match Power/Time Template

The array of power samples shall be tested for a fit within the template of Fig. II.3.3. The centre of the template must be centred on the measured transition from bit 13 to bit 14 of the midamble of the measured burst.

e)	The SS shall then command the MS to each of the

GSM900:	16 power control levels (see Table II.3.3)

DCS1800:	14 power control levels (see Table II.3.3)

steps b) to d) shall be repeated.

f)	The SS shall command the MS to the maximum power control level and steps b) to d) shall be repeated for ARFCN in the Low, Mid and High ranges (ref Table 1.4).

|

+4 dB	|        ┌───┐

+1.0 dB	|        |   └─────────/ /──────────────┐

0 dB	|        |                              |

-1.0 dB	|        |   ┌─────────/ /──────────┐   |

|        |   |                      |   |

-6 dB	|     ┌──┘   |                      |   └──┐

|     |      |                      |      |

-30 dB	| ┌───┘      |                      |      └───┐

| |          |                      |          |

| |          |                      |          |

-70 dB	|─┘          |                      |          └

|            |<---- (87 bits) ----->|

└──────────────────────/ /─────────────────────-

10  8  10    4176/13=321.2 ms    10  8  10

Figure II.3.4 : Power / time template for access burst

GSM900 only - begin

For a transmitter carrier power lower than 34 dBm, where the level of -70 dB is lower than - 36 dBm, the -70 dB shall be replaced by a value which is equivalent to -36 dBm.

GSM900 only - end

DCS1800 only - begin

For a transmitter carrier power lower than 23 dBm, where the level of -70 dB is lower than - 47 dBm, the -70 dB shall be replaced by a value which is equivalent to -47 dBm.

DCS1800 only - end

g)	The SS shall cause the MS to generate an Access Burst on an ARFCN in the Mid ARFCN range (ref Table 1.4).

The SS shall capture this burst as described in b) above. However, in this case the SS needs to locate the centre of the useful bits of the burst by identifying the transition from the last bit of the synch sequence. The centre of the burst, as the timing reference, is then five data bits prior to this point.

h)	Determine the peak transmitter carrier power of the useful 87 bits of the burst, the 87 bits being symmetrically disposed about the centre point derived in g). This gives the 0 dB reference.

i)	Determine the time of the centre point derived in g) with respect to the MS received data on the common control channel.

j)	The captured burst shall fit the template of Fig. II.3.4 with the centre of the template horizontal axis located on the centre located in g) above.

k)	The SS shall modify the control data on the serving cell BCCH in order to limit the MS transmit power on the Access Burst to

GSM900:	power level 10 (+23 dBm)

DCS1800:	power level 10 (+10 dBm).

Steps g) to j) shall be repeated.

l)	Steps a) to k) shall be repeated under extreme test conditions (see Annex 1, TC2.2 and TC3) except that for step e)

GSM900:	only power control levels 10 and 15

DCS1800:	only the minimum power control level and the level 10 dB above that

shall be tested.

II.3.3.2.2	Method of measurement for equipment with an integral antenna

Note:	If the MS is equipped with a permanent connector, such that the antenna can be disconnected and the SS be connected directly, then the method of section II.3.3.2.1 shall be applied.

The tests in this section shall be performed on an unmodified test sample.

The MS shall be in the anechoic shielded chamber (Annex 1, GC5) or on the outdoor test site, on an isolated support, in the position for normal use.

A test antenna, connected to the SS, shall be in the anechoic shielded chamber or on the test site, at a distance of at least 3 metres from the MS.

Note:	The test method described has been written for measurement in an anechoic shielded chamber. If an outdoor test site is used, then it is additionally necessary to raise/lower the test antenna through the specified height range to maximize the received power levels from both the test sample and the substitution antenna.

a)	A call shall be set up by the SS according to the generic call set up procedure on an ARFCN, in the Mid ARFCN range (ref Table 1.4).

b)	Measure Peak Received Transmitter Carrier Power

Using a sampling power measurement method with a sampling rate of at least 2/T, where T is bit duration, capture a representation of the MS transmit burst's amplitude and timing. From the array of samples the SS identifies the centre of the useful 147 transmitted bits, i.e. the transition from bit 13 to bit 14 of the midamble, as the timing reference. This enables the peak received transmitter carrier power over the 147 useful bits to be calculated and used as the 0 dB reference.

c)	Match Power/Time Template

The array of power samples shall be tested for a fit within the template of Fig. II.3.3. The centre of the template must be centred on the measured transition from bit 13 to bit 14 of the midamble of the measured burst.

d)	Measure Burst Timing

The transition point identified in b) above is referred to the corresponding transition in the MS received burst.

e)	The SS shall then command the MS to each of the 16 power control levels (see Table II.3.3) and steps b) to d) shall be repeated.

f)	The SS shall command the MS to the maximum power control level and steps b) to d) shall be repeated for ARFCN in the ranges 1 to 5, 30 to 35, 90 to 95 and 120 to 124.

g)	The SS shall cause the MS to generate an access burst on an ARFCN in the range 60 to 65. The SS shall capture this burst as described in b) above. However, in this case the SS needs to locate the centre of the useful bits of the burst by identifying the transition from the last bit of the synch sequence. The centre of the burst, as the timing reference, is then five data bits prior to this point.

h)	Determine the peak received transmitter carrier power of the useful 87 bits of the burst, the 87 bits being symmetrically disposed about the centre point derived in g). This gives the 0 dB reference.

i)	The captured burst shall fit the template of Fig. II.3.4 with the centre of the template horizontal axis located on the centre located in g) above.

j)	Determine the time of the centre point derived in g) with respect to the MS received data on the common control channel.

k)	The SS shall modify the control data on the serving cell BCCH in order to limit the transmit power on the Access Burst to power control level 10 (+23 dBm). Steps g) to j) shall be repeated.

l)	The MS shall be rotated by n*45 degrees for all values of n in the range 1 to 7. At each rotation step b) shall be repeated for all channels of steps f) and a) at the maximum power control level.

m)	The MS shall be replaced by a half-wave dipole, resonating at the centre frequency of the transmit band (902.5 MHz), connected to an RF generator.

n)	The output power of the RF generator shall be adjusted to reproduce the peak received transmitter power recorded in steps e) (maximum power control level), f) and l). For each indication the power, delivered by the generator (in Watts) to the half-wave dipole, shall be recorded. These values shall be ordered in a matrix where the columns of the matrix represent the eight orientations of the MS and the rows of the matrix represent the 5 channels specified in steps a) and f). Record values in the form Pnc, where n = MS rotation and c = channel number.

For each channel number used compute:

1		n=7

Pac (Watts into dipole) =	───	*	S Pnc

8		n=0

from which: Pac (Tx dBm) = 10log10(Pac) + 30 + 2.15

o)	The difference for each of the five channels between the actual peak transmitter carrier power level averaged over the 8 measurement orientations determined in step n), and the relative peak transmitter carrier power level for the maximum power control level derived in step e) and f), is used to relate the relative measurement results of steps e), f, g) and k) to actual peak transmitter carrier powers for all measured power control levels, which can be checked against the requirements of Table II.3.3.

Note:	The next steps are to determine the peak transmitter carrier power under extreme test conditions. Basically the procedure is such that:

-	the power/time template is tested in the "normal" way,

-	the radiated power is measured by measuring the difference with respect to the radiation under normal test conditions.

p)	A separate test sample equipped with a temporary antenna connector is placed in a climatic test chamber and is linked to the SS by means of the temporary antenna connector.

q)	Under normal test conditions steps a) to k) shall be repeated. The peak received power level shall be noted again for all power control levels and the access burst on every frequency used except that for step e) only power control levels 10 and 15 shall be tested.

Note:	The values noted here are related to the peak transmitter carrier power levels under normal test conditions, which are known after step o). Therefore frequency dependent calibration factors that account for the effects of the temporary antenna connector can be determined.

r)	Steps a) to k) shall be repeated under extreme test conditions (see Annex 1, TC2.2 and TC3) except that for step e) only power control levels 10 and 15 shall be tested.

s)	The peak transmitter carrier power under extreme test conditions is calculated for each power control level and for every frequency used by adding the calibration factor i.e. the difference, obtained in the readings of step o) with respect to the reading obtained in step q) to the values obtained in step r).

II.3.3.3	Requirements

a)	The peak transmitter carrier power, under normal and under extreme test conditions, at each frequency and for each power class shall be within the tolerances as shown in Table II.3.3.

GSM900 only - begin

Power	Power	Peak transmitted	Tolerances

class	control	carrier power

level

1	2	3	4	5		dBm	N.T.C.	E.T.C.

*					0	43	+/-2.0dB	+/-2.5dB

*					1	41	+/-3dB	+/-4dB

*	*				2	39	+/-3dB*)	+/-4dB*)

*	*	*			3	37	+/-3dB*)	+/-4dB*)

*	*	*			4	35	+/-3dB	+/-4dB

*	*	*	*		5	33	+/-3dB*)	+/-4dB*)

*	*	*	*		6	31	+/-3dB	+/-4dB

*	*	*	*	*	7	29	+/-3dB*)	+/-4dB*)

*	*	*	*	*	8	27	+/-3dB	+/-4dB

*	*	*	*	*	9	25	+/-3dB	+/-4dB

*	*	*	*	*	10	23	+/-3dB	+/-4dB

*	*	*	*	*	11	21	+/-3dB	+/-4dB

*	*	*	*	*	12	19	+/-3dB	+/-4dB

*	*	*	*	*	13	17	+/-3dB	+/-4dB

*	*	*	*	*	14	15	+/-3dB	+/-4dB

*	*	*	*	*	15	13	+/-3dB	+/-4dB

Table II.3.3 : Peak transmitter carrier power for different power classes

*)	When the power control level corresponds to the power class of the Mobile Station, then the maximum tolerances shall be 2.0 dB under normal test conditions and 2.5 dB under extreme test conditions.

GSM900 only - end

DCS1800 only - begin

Power	Power	Peak transmitted	Tolerances

class	control	carrier power

level

1	2		dBm	N.T.C.	E.T.C.

*		0	30	+/-2.0dB	+/-2.5dB

*		1	28	+/-3dB	+/-4dB

*		2	26	+/-3dB*)	+/-4dB*)

*	*	3	24	+/-3dB*)	+/-4dB*)

*	*	4	22	+/-3dB	+/-4dB

*	*	5	20	+/-3dB	+/-4dB

*	*	6	18	+/-3dB	+/-4dB

*	*	7	16	+/-3dB	+/-4dB

*	*	8	14	+/-3dB	+/-4dB

*	*	9	12	+/-4dB	+/-5dB

*	*	10	10	+/-4dB	+/-5dB

*	11	8	+/-4dB	+/-5dB

*	12	6	+/-4dB	+/-5dB

*	13	4	+/-4dB	+/-5dB

Table II.3.3 : Peak transmitter carrier power for different power classes

*)	When the power control level corresponds to the power class of the Mobile Station, then the maximum tolerances shall be 2.0 dB under normal test conditions and 2.5 dB under extreme test conditions.

DCS1800 only - end

b)	The difference of the peak transmitter carrier power for two adjacent power control levels (measured at the same frequency, under the same combination of test conditions), shall not be less than 0.5 dB and not be more than 3.5 dB, and the power control steps must form a monotonic sequence.

c)	The power/time curve shall be within the limits of the power time template of Figure II.3.3 at each frequency, at each power control level and under every combination of normal and extreme test conditions.

d)	All the power control levels, for the power class of the MS as stated by the manufacturer, shall be implemented in the MS

GSM900:	from the maximum power control level down to (including) power control level 15

DCS1800:	from level 10 (10 dBm) for class 1 or 13 (4 dBm) for class 2 up to the maximum peak transmitter power corresponding to the class of the particular mobile station.

e)	When the transmitter is commanded to a power control level outside of the power control level capability corresponding to the power class of the MS as stated by the manufacturer, then the transmitter carrier power shall be within the tolerances for the closest power control level corresponding to the power class as stated by the manufacturer.

f)	The centre of the transmitted burst as defined by the transition of bits 13/14 of the midamble shall be 3 timeslot periods (1731 ms) +/- 1 bit period (+/- 3.69 ms) after the corresponding received burst.

g)	The access burst shall fit the power time template of Fig. II.3.4 for all conditions of test.

h)	The centre of the transmitted access burst as defined by the bit transition described in II.3.3.1 shall be an integer number of timeslot periods less 30 bit periods relative to any CCCH midamble centre. The permitted tolerance is +/- 1 bit period (+/- 3.69 ms).

II.3.4	Output RF spectrum

Ref: GSM 05.05 section 4.2

II.3.4.1	Definition

The output RF power spectrum is the radio frequency spectrum, produced by the MS due to the effect of modulation and power ramping, on frequency bands adjacent to the nominal carrier frequency of the MS.

II.3.4.2	Modulation and switching transients

II.3.4.2.1	Method of measurement

Note:	If the Mobile Station is equipped with a permanent antenna connector, then the MS is connected to the SS and is operated under normal test conditions.

If the MS is normally equipped with an integral antenna, and it cannot be operated via a permanent antenna connector, then a second test sample is used fitted with a temporary antenna connector. The SS is connected to the temporary antenna connector and is operated under normal test conditions.

a)	A call shall be set up according to the generic call set up procedure.

The MS shall be commanded into the hopping mode. The hopping pattern shall include only three channels, namely one with an ARFCN in the Low ARFCN range, a second one with an ARFCN in the Mid ARFCN range and the third one with an ARFCN in the High ARFCN range.

Note:	Although the measurement is made whilst the MS is in hopping mode, each measurement is on one single channel.

DCS1800 only - begin

Range 512 format of frequency list (GSM 04.08 section 10.5.2.9a.4) shall be used to address the mobile allocation used for frequency hopping.

When averaging is in use during frequency hopping mode, the averaging shall only include bursts transmitted when the hopping carrier corresponds to the nominal carrier of the measurement.

DCS1800 only - end

b)	The SS commands the MS to create the loop back facility from the receiver decoder output to the transmitter encoder input.

c)	The SS shall send Standard Test Signal C1 to the MS at a level of 23 dBmVemf(  ). The transmitter signal received from the MS shall be applied to a spectrum analysis function within the SS. The centre frequency of the spectrum analysis function shall be tuned to the hop pattern channel with an ARFCN in the Mid ARFCN range (ref Table 1.4).

d)	The MS shall be commanded to the MS maximum power level. The settings of the spectrum analyser shall be adjusted as follows:

Zero frequency scan

Resolution bandwidth:	30 kHz

Video bandwidth:	30 kHz

Video averaging:	may be used, depending on the implementation of the test.

The video signal of the spectrum analyser is "gated" such that the spectrum generated by at least 40 of the bits 87 to 132 of the burst is the only spectrum measured. This gating may be analogue or numerical, dependent upon the design of the spectrum analyser.

Only measurements during transmitted bursts on the nominal carrier of the measurement shall be included. The spectrum analyser shall average over the gated period and over 200 such bursts, using numerical and/or video averaging.

e)	The power level shall be measured at the following frequencies:

FT

FT + 100 kHz	FT - 100 kHz

FT + 200 kHz	FT - 200 kHz

FT + 250 kHz	FT - 250 kHz

FT + 200 kHz * N	FT - 200 kHz * N

where N = 2, 3, 4, 5, 6, 7, 8 and 9

and FT = RF channel nominal centre frequency.

DCS1800 only - begin

e1)	The analyser settings shall be adjusted to

Zero frequency scan

Resolution bandwidth	100 kHz

Video bandwidth	100 kHz

Video averaging

The power level measurement shall be continued beyond the range of step e) measurement and extending over the whole MS Tx range.

1710.2 + 0.2 * (n - 512) MHz	(512 <= n <= 885)

For each frequency the averaging shall be done for 3 separate groups of bursts, each group corresponding to when the transmitter is on one of the 3 hop frequencies. The averaging shall be over 50 transmitted bursts per group.

DCS1800 only - end

f)	The measurement at frequencies up to FT +/- 1800 kHz shall be repeated for

GSM900:	power control level 15

DCS1800:	the minimum MS power control level.

g)	The analyser settings are adjusted to:

Zero frequency scan

Resolution bandwidth:	 30 kHz

Video bandwidth :	100 kHz

Peak hold

h)	The MS shall be commanded to

GSM900:	power level 0

DCS1800:	the maximum power control level.

The spectrum analyser power levels shall be measured at the following frequencies:

FT + 400 kHz	FT - 400 kHz

FT + 600 kHz	FT - 600 kHz

FT + 1,2 MHz	FT - 1,2 MHz

FT + 1,8 MHz	FT - 1,8 MHz

where FT = RF channel nominal centre frequency.

The duration of each measurement (at each frequency) shall be such as to cover at least 10 burst transmissions.

i)	Step h) shall be repeated for

GSM900:	power control levels 7 and 11

DCS1800:	the minimum power control level.

j)	The spectrum analysis function shall be tuned to the hop pattern channel with an ARFCN in the Low ARFCN range (ref Table 1.4) .

Steps d) to e) and g) to h) shall be repeated, except that at step h) the MS shall be commanded to

GSM900:	power control level 11

DCS1800:	the minimum power control level.

k)	The spectrum analysis function shall be tuned to the hop pattern channel with an ARFCN in the High ARFCN range (ref Table 1.4) .

Steps d) to e) and g) to h) shall be repeated, except that at step h) the MS shall be commanded to

GSM900:	power control level 11

DCS1800:	the minimum power control level.

GSM900 only - begin

l)	Step d) shall be repeated, except that the resolution bandwidth shall be set to 100 kHz. The power level shall be measured over the frequency range 935 to 960 MHz in 200 kHz steps, and at each step, the averaging shall be done in 3 separate groups of bursts, each group corresponding to when the transmitter is on one of the 3 hop frequencies. The averaging shall be over at least 50 transmitted bursts per group.

GSM900 only - end

DCS1800 only - begin

l)	The MS shall be commanded to full power and the System Simulator set to the conditions of step c) and e1). The power level shall be measured at the following frequencies

1805.2 + 0.2 * (n - 512)	(512 <= n <= 885)

935.2 + 0.2 * (n - 1)	(1 <= n <= 124)

and with the spectrum analyser set to a resolution bandwidth of 100 kHz and a video bandwidth of 100 kHz. For each frequency the averaging shall be done for 3 separate groups of bursts, each group corresponding to when the transmitter is on one of the 3 hop frequencies. The averaging shall be over at least 50 transmitted bursts per group.

DCS1800 only - end

m)	Steps c) to e) and g) to h) shall be repeated - except that at step h) the MS shall be commanded to

GSM900:	power control level 11

DCS1800:	the minimum power control level.

-	under extreme test conditions (Annex 1, TC2.2 and TC3).

II.3.4.2.2	Limits

GSM900 only - begin

For the modulation sidebands of steps e) and f), the power level shall not exceed the values shown in Table II.3.4.2.

Power	Power	Maximum level, relative to measurement on the

control	level	carrier frequency (dB) at the listed frequency

level	(dBm)	offsets from the nominal carrier frequency (kHz)

0	100	200	250	400	>=600

ant.	int.	ant.	int.

con.	ant.	con.	ant.

0	43	0	+0.5	-30	-33	-60		-70

2	39	0	+0.5	-30	-33	-60		-66

3	37	0	+0.5	-30	-33	-60	-58	-64	-58

>= 5	<= 33	0	+0.5	-30	-33	-60	-54	-60	-54

ant. con. = for MS with antenna connector

int. ant. = for MS with integral antenna

Table II.3.4.2

For all power levels tested the maximum power measured at step l) in the band 935 MHz to 960 MHz shall be:

-71 dBm for class 1 MSs, and

-79 dBm for class 2, 3, 4 or 5 MSs.

For measurements performed on a temporary antenna connector it is not possible to accurately calibrate the connector coupling loss outside the MS transmit band. Absolute levels less than the -71 dBm or -79 dBm (as appropriate) figure require no further investigation.

For recorded levels above the appropriate limit further investigation shall be performed on an unmodified test sample using the integral antenna.

Using the spectrum analyser settings of step l), measurements shall be performed in a shielded anechoic chamber or on the test site. An 8 position average emission level shall be calculated using the substitution method of II.3.3.2.2.

Note 1:	For each value of FT, 1 measurement in the combined range

FT + 400kHz to FT + 1800 kHz, and FT - 400 kHz to FT - 1800 kHz

may be at -36 dBm (  ).

In the range 935 to 960 MHz, within each group of bursts, measurements within 5 GSM RF channels may be at -36 dBm (  ).

Note 2:	MSs of power class 1 will test power control levels 0 and 15,

class 2 will test power control levels 2 and 15,

class 3 will test power control levels 3 and 15,

class 4 will test power control levels 5 and 15 and

class 5 will test power control levels 7 and 15.

For the power ramp sidebands of steps h) and i) the power levels must not exceed:

Power	Maximum level for various offsets

level	from carrier frequency

400 kHz	600 kHz	1200 kHz	1800 kHz

43 dBm	-9 dBm	-21 dBm	-21 dBm	-24 dBm

41 dBm	-11 dBm	-21 dBm	-21 dBm	-24 dBm

39 dBm	-13 dBm	-21 dBm	-21 dBm	-24 dBm

37 dBm	-15 dBm	-21 dBm	-21 dBm	-24 dBm

35 dBm	-17 dBm	-21 dBm	-21 dBm	-24 dBm

33 dBm	-19 dBm	-21 dBm	-21 dBm	-24 dBm

31 dBm	-21 dBm	-23 dBm	-23 dBm	-26 dBm

29 dBm	-23 dBm	-25 dBm	-25 dBm	-28 dBm

27 dBm	-23 dBm	-26 dBm	-27 dBm	-30 dBm

25 dBm	-23 dBm	-26 dBm	-29 dBm	-32 dBm

23 dBm	-23 dBm	-26 dBm	-31 dBm	-34 dBm

<= +21 dBm	-23 dBm	-26 dBm	-32 dBm	-36 dBm

└	┘

allowance	8 dB	6 dB	3 dB

Note:	These figures are different from the requirements in GSM 05.05 because at higher power levels it is the modulation spectrum which is being measured using a peak hold measurement. This allowance is given in the table.

The results of the measurements on MSs with an antenna connector shall be in dBm.

GSM900 only - end

DCS1800 only - begin

For the modulation sidebands of steps e) and f), the power level relative to a measurement in 30 kHz on the carrier frequency shall not exceed the values shown in Table II.3.4.2. Offsets from the carrier frequency are shown in kHz.

Power	Measurement bandwidth

level

(dBm)	30 kHz	100 kHz

100	200	250	400	600 to	>1800 to	>6000

1800	6000

30	+0.5	-30	-33	-60	-60	-65	-75

28	+0.5	-30	-33	-60	-60	-63	-73

26	+0.5	-30	-33	-60	-60	-61	-71

<= 24	+0.5	-30	-33	-60	-60	-59	-69

Table II.3.4.2

The maximum power measured at step l) in the band 935 to 960 MHz shall be no more than -77 dBm and in the band 1805 to 1880 MHz shall be no more than -71 dBm except in up to 5 measurements where a level of up to -36 dBm is permitted in each set of measurements grouped by the hopping frequencies.

For measurements using frequency offset greater than 6 MHz with no frequency hopping the results only refer to one frequency group and must be interpreted accordingly.

For measurements performed on a temporary antenna connector it is not possible to accurately calibrate the connector coupling loss outside the MS transmit band. Absolute levels less than the -77 dBm or -36 dBm (as appropriate) figure require no further investigation.

For recorded levels above the appropriate limit further investigation shall be performed on an unmodified test sample using the internal antenna.

Using the spectrum analyser settings of step l), measurements shall be performed in a shielded anechoic chamber (clause A1.1.2.5) or on the test site. An 8 position average emission level shall be calculated using the substitution method of II.3.3.2.2.

Note 1:	For each value of FT in the combined frequency range formed both by:

1) Step e) measurements in the range FT-1800 kHz to FT-600 kHz and FT+600 kHz to FT+1800kHz,

and

2) Step e1) measurements in the range FT-6000 kHz to FT-1800 kHz and FT+1800 kHz to FT+6000 kHz.

Up to a total of 3 measurements in the combined set of measurements grouped by the hopping frequencies can be up to -36 dBm.

Note 2 :	Up to 12 measurements (in each set of measurements grouped by the hopping frequencies) above 6 MHz offset from the nominal carrier frequency to the edge of the transmit band in step e1) may be at a level up to -36 dBm.

Note 3 :	As a result of GSM 05.05-DCS section 4.2.1 note iii, relaxations of the measurements in steps e and f are possible where :

1) For offsets between 100 kHz and 600 kHz if the table indicates that measurements performed in a 30kHz bandwidth relative to a measurement in 30 kHz on the carrier would result in an absolute level measured in 30 kHz of less than -36 dBm then a value of -36 dBm shall apply for offsets of up to (and including) 600 kHz from the carrier.

2) For offsets greater than 600 kHz up to (and including) 1800 kHz. if the table indicates that measurements performed in a 30 kHz bandwidth relative to a measurement in 30 kHz on the carrier would result in an absolute level measured in 30 kHz of less than -56 dBm then a value of -56 dBm shall apply.

3) For offsets greater than 1800 kHz if the table indicates that measurements performed in a 100 kHz bandwidth relative to a measurement in 30 kHz on the carrier would result in an absolute level measured in 100 kHz of less than -51 dBm then a value of -51 dBm shall apply.

For the power ramp sidebands of steps h) and i) the power levels must not exceed:

Power	Maximum level for various offsets

level	from carrier frequency:

400 kHz	600 kHz	1200 kHz	1800 kHz

30 dBm	 -22 dBm 	-24 dBm	 -24 dBm	 -27 dBm

24 dBm	 -23 dBm 	-26 dBm	 -30 dBm	 -33 dBm

<= +20 dBm	 -23 dBm 	-26 dBm	 -32 dBm	 -36 dBm

Note:	These figures are different from the requirements in GSM 05.05-DCS because of alignment with GSM 05.05 where it was found that at higher power levels it is the modulation spectrum which is being measured using a peak hold measurement. The GSM 11.10 allowance has an impact down to the lower power levels of GSM 05.05-DCS resulting in the given table. The results of the measurements on MSs with an antenna connector shall be in dBm. The results of the measurements on equipment with no possibility to use an antenna connector shall be in dBs relative to the average transmitter carrier power (see section II.3.3) and then converted into dBm by subtracting this value from the average transmitter carrier power, taking the actual power level and filter bandwidths into account.

DCS1800 only - end

DCS1800 only - begin

II.3.5	Intermodulation attenuation

II.3.5.1	Definition

The intermodulation attenuation is a measure of the capability of an RF transmit equipment to inhibit the generation of signals in its non-linear elements caused by the presence of the carrier and an interfering signal reaching the equipment via its antenna.

The intermodulation attenuation is expressed as the ratio in dB of the output power level of the transmitter under test to the power level of the highest intermodulation component.

For further information see recommendation GSM 05.05-DCS section 4.7.3.

II.3.5.2	Methods of measurement

Two methods of measurement are described separately for:

1)	equipment fitted with an antenna connector and for

2)	equipment fitted with an integral antenna, and which cannot be connected to an external antenna.

SFH shall be disabled during this measurement.

II.3.5.2.1	Method of measurement for equipment with an antenna connector

a)	The MS shall be connected to the SS. A call shall be originated by the SS to the MS and answered by the MS. The call shall be on a radio frequency in the Mid ARFCN range, power control level set to maximum power.

b)	The antenna output of the MS shall be connected to the SS via a coupling device, presenting to the MS a load with an impedance of 50 ohms. The coupling device may consist of a circulator one port of which is to be connected by a coaxial cable to the output terminal of the MS, the second port is to be correctly terminated with 50 ohms into a selective measuring device (eg a spectrum analyser, peak hold) with a measurement bandwidth of 300 kHz. The third port of the circulator is to be connected to the unwanted test signal source by means of an isolator.

c)	The unwanted test signal shall be unmodulated and the frequency shall be 800 kHz above the frequency of the MS under test. The unwanted test signal power level shall be adjusted to be 40 dB below the power level of the MS.

d)	The power level of the test signal shall be measured at the antenna output end of the coaxial cable, when disconnected from the RF transmit equipment and then correctly matched into 50 ohm. The antenna output power of the MS shall be measured directly at the antenna output terminal connected to an artificial antenna.

e)	Any intermodulation components shall then be measured in the band indicated in table xx (of section II.3.5.2.3) by means of the selective measuring device.

f)	This measurement shall be repeated with the test signal at a frequency 800 kHz below the transmitted frequency.

NOTE:	When the above measurements are performed precautions must be taken, so that non-linearities in the selective measuring device do not influence the results appreciably. Furthermore it should be ensured that intermodulation components which may be generated by non-linear elements in the test equipment (eg signal generator, circulators, selective measuring device) are sufficiently reduced. The RF transmit equipment under test and the test signal source shall be physically separated in such a way that the measurement is not influenced by direct radiation.

II.3.5.2.2	Method of measurement for equipment with an integral antenna

a)	The MS shall be placed in the anechoic chamber as in section II.3.3.2.2 and the procedure of part a) of that section shall be repeated. The coupling factor F between the mobile and the test antenna shall be calculated as the difference in dB between Pac as defined in section II.3.3.2.2 and the power measured.

b)	The test antenna shall be connected to the SS via a coupling device, presenting to the antenna a load with an impedance of 50 ohm. The coupling device may consist of a circulator one port of which is to be connected by a coaxial cable to the antenna terminal, the second port is to be correctly terminated with 50 ohm into a selective measuring device (eg a spectrum analyser, peak hold) with a measurement bandwidth of 300 kHz. The third port of the circulator is to be connected to the unwanted test signal source by means of an isolator.

c)	The unwanted test signal shall be unmodulated and the frequency shall be 800 kHz above the frequency of the MS under test. The standard test signal Io, power level shall be adjusted to be 40 dB below the power level of the MS at the MS antenna. This is given by:

Pac(Tx dBm) - 40 + F dBm

d)	The power level of the test signal shall be measured at the antenna output end of the coaxial cable, when disconnected from the RF transmit equipment and then correctly matched into 50 ohms.

e)	Any intermodulation components shall then be measured in the band indicated in table xx (of section II.3.5.2.3) by means of the selective measuring device.

f)	This measurement shall be repeated with the test signal at a frequency 800 kHz below the transmitted frequency.

Note: 	When the above measurements are performed precautions must be taken, so that non-linearities in the selective measuring device do not influence the results appreciably. Furthermore it should be ensured that intermodulation components which may be generated by non-linear elements in the test equipment (eg signal generator, circulators, selective measuring device) are sufficiently reduced. The RF transmit equipment under test and the test signal source shall be physically separated in such a way that the measurement is not influenced by direct radiation.

II.3.5.2.3	Requirements

The intermodulation limit is as indicated below under normal test conditions. The limits express the minimum intermodulation attenuation in dB.

Test Signal		Limit	Frequency Band

displacement (x)

0.8 MHz		50dB	1710-1785 MHz

DCS1800 only - end