II.5.3.1	Default conditions and structured sequence of tests

II.5.3.1.1	Default test conditions during L3-tests

During tests in section II.5.3 the following default test conditions shall apply if not otherwise stated within the test description. In the table below, decimal values are normally used. Sometimes a hexadecimal value, indicated with a 'H', or a binary value, indicated with a 'B' is given.

GSM900	DCS1800

General signalling conditions for all carriers

Cyphering	yes	yes

General RF-conditions for all carriers

Frequency hopping mode	Non-hopping	Non-hopping

Propagation profile	Static	Static

Downlink Input Level	63 dBmVemf(  )	63 dBmVemf(  )

Uplink output power	Minimum according	Minimum according

to MS power class	to MS power class

Serving cell, BCCH/CCCH carrier

Channel ARFCN	20	590

Alternative channels	40 or 60	690 or 830

Serving cell, Traffic channel, SDCCH

Channel ARFCN	30	650

Alternative channels	50 or 70	750 or 850

Power Control Indicator	0	0

Neighbouring cells BCCH/CCCH carriers

Channel ARFCN	10, 80, 90, 100, 110, 120	520, 600, 700, 780, 810, 870

Alternative channels	15, 85, 95, 105, 115, 122	530, 610, 710, 790, 820, 880

Input level	53 dBmVemf(  )	53 dBmVemf(  )

Network dependent parameters

Cell identity	0001H	0001H

Mobile country code, MCC	001 (decimal)	001 (decimal)

Mobile network code, MNC	01 (decimal)	01 (decimal)

Location area code, LAC	0001H	0001H

BCCH allocation number	00B band number 0	00B band number 0

BCCH allocation sequence	0	0

number(BA-IND)

Cell allocation number	00B band number 0	00B band number 0

PLMN colour code, NCC	1	1

BS colour code, BCC	5	5

SMS Cell Broadcast	not active	not active

DTX	MS must not use	MS must not use

IMSI Attach-detach	0 MS shall not apply	0 MS shall not apply

CCCH_CONF	1 basic physical channel for	1 basic physical channel for

CCCH combined with SDCCH	CCCH combined with SDCCH

BS_AG_BLKS_RES	0 blocks reserved	0 blocks reserved

BS_PA_MFRMS	5 paging subgroups	5 paging subgroups

CELL_BAR_ACCESS	0 (not barred)	0 (not barred)

Call-reestablishment (RE)	1 (allowed)	1 (allowed)

Emergency Call allowed	0	0

Access Control Class	0	0

(AC) (0..9, 11..15)

Network dependent timers

Radio_Link_Timeout	8	8

T3212 Periodic	Infinite	Infinite

updating in decihours

GSM900	DCS1800

Access control parameters

Max retrans	1	1

Tx-integer, nr. of slots	5	5

CELL_RESELECT_HYSTERESIS	12 dB	12 dB

MS_TXPWR_MAX_CCH	minimum level	minimum level

RXLEV_ACCESS_MIN	minimum	minimum

These informations are provided by system information 1, 2, 3 and 4 messages.

The system information elements which are broadcast on the SACCH during the dedicated mode should be consistent with those sent on the BCCH when the MS was in idle mode prior to the channel request.

II.5.3.1.2	Structured sequence of the tests

The tests shall be performed in the order as indicated in the following table.

The validity of the tests depends upon the results of the tests performed before.

Channel request (basic test)	RR	II.5.3.2.1

Immediate assignment	RR	II.5.3.6.1

IMSI attach/detach (basic)	RR	II.5.3.2.2

Paging	RR	II.5.3.6.2

Test of the mobile station functions in idle mode	RR	II.5.3.3

Frequency redefinition	RR	II.5.3.6.6

Measurement report (incl. system info not idle)	RR	II.5.3.6.3

Authentication	MM	II.5.3.7.2

Cipher mode setting	RR	II.5.3.6.8

Identification	MM	II.5.3.7.3

Sequenced MM/CC message transfer	..	II.5.3.2.3

Channel release	RR	II.5.3.6.12

Location updating	MM	II.5.3.7.4

TMSI reallocation	MM	II.5.3.7.1

Classmark change	RR	II.5.3.6.11

Call controll (verification on CC state diagram)	CC	II.5.3.8.1

Call rearrangement	CC	II.5.3.8.* missing

DTMF information transfer	CC	II.5.3.8.4

Handover	RR	II.5.3.6.5

Additional assignment	RR	II.5.3.6.9

Partial release	RR	II.5.3.6.10

Re-establishment	CC	II.5.3.8.3

Dedicated channel assignment (during calls)	RR	II.5.3.6.4

Emergency call establishment	CC	II.5.3.8.2

Transmission mode change	RR	II.5.3.6.7

Mobility management connection establishment	MM	II.5.3.7.5

Test of Layer 3 error handling		II.5.3.5

User to user signalling	CC	II.5.3.8.5

Testing of structured procedures		II.5.3.9

II.5.3.1.3	General rules for message parameters

The following rules concerning message parameters apply to section II.5.3:

1)	Those values of parameters which are a consequence of the context of a test and which are not specific to that test need not be defined.

2)	If the value of a parameter of an uplink message (MS to Network) is specified in a test, the implicit meaning is that it has to be checked; if the value is not specified, it is not to be checked unless stated otherwise.

3)	An optional field or optional Information Element of a downlink message (Network to MS), the presence of which is not a consequence of a test description, shall be absent in that test.

4)	If an optional field or Information Element is not indicated for the uplink (MS to Network) - unless specified otherwise -, it may be included or not.

5)	The Protocol Discriminator, Transaction Identifier and Message Type of all uplink messages have to be checked.

II.5.3.1.4	General rules for layer 3 testing

Unless otherwise specified, before the SS pages the MS, the MS must be given the necessary time to be able to receive paging (see section II.6.1.6.1).