II.5.3.7	Elementary procedures of mobility management

The tests are based on GSM 04.08, GSM 03.03 and GSM 03.20.

II.5.3.7.1	Test of TMSI reallocation procedure

The TMSI reallocation procedure is defined in section 4.3.1 of GSM 04.08.

II.5.3.7.1.1	Purpose of test

The test verifies that the MS is able to receive and acknowledge a new TMSI.

II.5.3.7.1.2	Method of measurement

a)	The SS establishes a data link with the MS and activates ciphered mode.

b)	The SS sends a TMSI reallocation command with a TMSI different to the old one and starts timer T3250.

Note:	The MS is supposed to store the received TMSI and LAI in non-volatile memory.

c)	The SS observes the transmission of the TMSI REALLOCATION COMPLETE message by the MS, and verifies its contents and timing.

d)	The network releases the channel.

e)	The MS is switched off, its power supply is then interrupted for 1 minute. Then the power supply is resumed and the MS is switched on again.

f)	The MS is paged by the SS, using the newly distributed TMSI.

g)	The SS then releases the main signalling link by sending a CHANNEL RELEASE message.

h)	The MS is made to perform a location updating as described in the method of test of II.5.3.7.4.

i)	The SS sends a TMSI REALLOCATION COMMAND message with a new TMSI.

j)	The SS sends the LOCATION UPDATING ACCEPT message containing neither IMSI nor TMSI.

k)	The SS then releases the main signalling link by sending a CHANNEL RELEASE message.

l)	The SS sends a PAGING REQUEST TYPE 1 message containing the TMSI allocated with the TMSI REALLOCATION COMMAND message.

II.5.3.7.1.3	Requirements

1)	In step c), the MS shall send a TMSI REALLOCATION COMPLETE message within T3250 (5 s in GSM 04.08) after the SS has sent the TMSI REALLOCATION command.

Message:	TMSI REALLOCATION COMPLETE	(GSM 04.08, 9.2.15)

Information element	Comment	Value

protocol discriminator	MM	0101

transaction identifier	not relevant	0000

message type	TMSI realloc com (note seq var)	0x01 1011

2)	In step f), the MS shall respond to the TMSI, which was allocated to it in step b).

3)	In step h), the MS sends a normal LOCATION UPDATING REQUEST message.

4)	In step i), the MS sends a TMSI REALLOCATION COMPLETE message.

5)	In step j), the MS shall wait for the SS to release the link.

6)	In step l), the MS shall respond to paging by sending the PAGING RESPONSE message containing the new TMSI.

II.5.3.7.2	Test of authentication elementary procedure

The authentication procedure is described in section 4.3.2 of GSM 04.08.

II.5.3.7.2.1	Purpose of test

To verify that the MS

-	correctly handles the authentication elementary procedure,

-	correctly manages the new ciphering key which it should establish in this procedure,

-	correctly manages the ciphering key sequence number, which was distributed to it in this procedure,

-	acts correctly upon failure of the procedure.

This test is not intended to do any test of correct handling of authentication algorithm by the SIM.

II.5.3.7.2.2	Method of measurement

Initial conditions

1)	The MS is provided with a SIM, of which the relation between the incoming RAND and the resulting SRES is known. For the purpose of this test, the algorithm may even be a look-up table, or very simple standard manipulation of the incoming RAND.

2)	The MS is brought in the MM-state "idle updated" and receives the BCCH/CCCH of the serving cell. A dedicated channel has been assigned with an IMMEDIATE ASSIGNMENT message.

Procedure

a)	The SS sends AUTHENTICATION REQUEST message and starts timer T3260

Message:	AUTHENTICATION REQUEST	(GSM 04.08, 9.2.2)

Information element	Comment	Value

protocol discriminator	MM message	0101

transaction identifier	not relevant	0000

message type	auth req	0001 0010

ciphering key seq no	key number 0	1111 0000

auth parameter RAND	MF, 16 octets	-

b)	The SS observes the transmission of the AUTHENTICATION RESPONSE message by the MS.

c)	The MS is brought back into idle mode by releasing the channel.

d)	The MS is paged by the SS.

e)	The SS observes the transmission of the PAGING RESPONSE message by the MS and compares the transmitted cipher key sequence number with the value that was sent to the MS in step a).

f)	The SS releases the RR connection.

g)	Steps a) and b) are repeated after the assignment of a dedicated channel.

h)	The SS sends an AUTHENTICATION REJECT message

Message:	AUTHENTICATION REJECT	(GSM 04.08, 9.2.1)

Information element	Comment	Value

protocol discriminator	MM	0101

transaction identifier	not relevant	0000

message type	auth reject	0001 0001

i)	The SS releases the RR connection.

j)	The SS sends a PAGING REQUEST message to the MS, and the SS observes any RF transmission that the MS might produce (channel request).

k)	After 30 s, the MS is made to make a normal call, and the SS observes any RF transmission that the MS might produce (channel request).

l)	After 30 s, via (E)MMI, the MS is made to make an emergency call, and the SS observes any RF transmission that the MS might produce (channel request).

II.5.3.7.2.3	Requirement

1)	Within timer T3260 (5 s in GSM 04.08) the MS shall respond with an AUTHENTICATION RESPONSE message, as detailed below.

Message:	AUTHENTICATION RESPONSE	(GSM 04.08, 9.2.3)

Information element	Comment	Value

protocol discriminator	MM	0101

transaction identifier	not relevant	0000

message type	auth res (note seq variable)	0x01 0100

auth parameter SRES	32 bits	-

2)	The value SRES should be bit exact with the value as produced by the authentication algorithm.

3)	In step e), the cipher key sequence number shall be the same as the value that was sent in step a).

4)	In step j), the MS shall not produce any RF transmission.

5)	In step k), the MS shall not produce any RF transmission.

6)	In step l), the MS shall produce a CHANNEL REQUEST message.

Step l) of the procedure and requirement 6) apply only if the MS supports speech (see PICS/PIXIT statement).

II.5.3.7.3	Test of identification elementary procedure

The identification procedure is described in section 4.3.3 of GSM 04.08.

II.5.3.7.3.1	Purpose of test

To verify that a MS sends identity information as requested by the system.

II.5.3.7.3.2	Method of test

a)	The SS sends a PAGING REQUEST message to the MS.

b)	The MS sends a CHANNEL REQUEST message.

c)	The SS sends an IMMEDIATE ASSIGN message, establishing an SDCCH.

Note 1:	A TMSI has been allocated before in the structured sequence of tests.

Note 2:	Cyphering mode is not set at this stage.

d)	The SS sends an IDENTITY REQUEST message.

Message:	IDENTITY REQUEST	(GSM 04.08, 9.2.8)

Information element	Comment	Value

protocol discriminator	MM	0101

transaction identifier	not relevant	0000

message type	identity request	0001 1000

identity type		1100 0

- id type	IMSI	.001

e)	The SS observes the message from the MS.

Note:	An IDENTITY RESPONSE message is anticipated.

f)	Step d is repeated, but the IDENTITY REQUEST message now contains id. type 100 (TMSI).

g)	The SS observes the message from the MS.

h)	The SS activates the cyphering mode.

i)	Step d is repeated, the IDENTITY REQUEST message contains id. type 010 (IMEI).

j)	The SS observes the message from the MS.

Requirements

1)	In step e), the MS shall transmit an IDENTITY RESPONSE message

Message:	IDENTITY RESPONSE	(GSM 04.08, 9.2.9)

Information element	Comment	Value

protocol discriminator	MM	0101

transaction identifier	not relevant	0000

message type	identity response	0x01 1001

Mob. identity	length

type of identity	IMSI	.001

identity	as many nibbles as necessary	xxxx ....

2)	In step g), the MS shall transmit an IDENTITY RESPONSE message, with:

Type of identity	TMSI	....	.100

and the TMSI shall correspond to the TMSI allocated to the MS by the SS. (a TMSI is allocated during the test of IMSI attach in the structured sequence of tests).

3)	In step j), the MS shall transmit an IDENTITY RESPONSE message, with:

Type of identity	IMEI	....	.010

and the IMEI shall correspond to the IMEI which is stored in the mobile equipment.

II.5.3.7.4	Test of location updating procedures

Purpose

To verify the normal location updating, the periodic updating and the IMSI attach procedures as specified in GSM 04.08, 4.4.

This test description is invoked frequently by the tests, further on in this section of GSM 11.10.

Initial Conditions

The SS simulates two cells, A and B, belonging to different location areas a and b. The MS shall be placed in the MM-state "idle, updated" listening to the BCCH/CCCH of cell A (i.e. updated in cell A).

The ATT parameter is set to 1 in the SYSTEM INFORMATION TYPE 3 message (control channel description), ie IMSI attach/detach is allowed.

The T3212 timeout value in the SYSTEM INFORMATION TYPE 3 message is equal to 1: corresponding to 1/10 hour, ie 6 minutes between periodic updatings.

Method of test

The RF level of cell A is lowered until the MS selects cell B (according to the cell selection procedures as described in GSM 05.08).

The MS should then establish an RR connection and initialise a normal location updating.

The MS behaviour is tested with different network reactions to the location updating request:

-	Location updating accepted by the network;

-	Location updating rejected by the network, subdivided in:

1)	"IMSI unknown in the HLR" or "illegal MS",

2)	"roaming not allowed",

3)	"location Area not allowed"

and

DCS1800:	4) "national roaming not allowed in this location area".

Subsequent location updatings are triggered by decreasing the RF level of the cell the MS is currently listening to and increasing the RF level of the other cell.

Message descriptions:

Message:	LOCATION UPDATE REQUEST

Information element	Comment	Value

protocol discriminator	MM	0101

transaction identifier	not relevant	0000

message type		0x00 1000

location updating type	00:normal, 10:IMSI attach or 01:periodic

ciphering key sequence number:	00 to 110

or 111

location area identification	(as required by text)

mobile station classmark 1	as specified by the manufacturer.

mobile identity

Message:	LOCATION UPDATE ACCEPT	(GSM 04.08, 9.2.11)

Information element	Comment	Value

protocol discriminator	MM	0101

transaction identifier	not relevant	0000

message type		0000 0010

location area identification	(as required by text)

mobile identity

Message:	LOCATION UPDATE REJECT	(GSM 04.08, 9.2.12)

Information element	Comment	Value

protocol discriminator	MM	0101

transaction identifier	not relevant	0000

message type		0000 0100

reject cause	(as required by text)

Message:	TMSI REALLOCATION COMPLETE	(GSM 04.08, 9.2.15)

Information element	Comment	Value

protocol discriminator	MM	0101

transaction identifier	not relevant	0000

message type		0x01 1011

Procedures:

II.5.3.7.4.1	Location updating accepted

ref: GSM 04.08, section 4.4.4.5

For this network response, three different cases are identified:

1)	TMSI is allocated,

2)	Location updating accept contains neither TMSI nor IMSI,

3)	Location updating accept contains IMSI.

Procedure 1	TMSI is allocated; the MS shall accept the new TMSI.

a)	The MS is made to perform a location updating as described in the method of test of II.5.3.7.4.

b)	The SS sends a LOCATION UPDATING ACCEPT message containing a new TMSI.

c)	The SS pages the MS using the PAGING REQUEST TYPE 1 message with the new TMSI.

Requirements 1

1)	In response to the LOCATION UPDATE ACCEPT message, the MS shall return a TMSI REALLOCATION COMPLETE message and wait for the SS to release the link.

2)	In response to the paging request using the new TMSI, the MS shall send the CHANNEL REQUEST message on the random access channel, it shall establish the main signalling link and respond with a PAGING RESPONSE message containing the correct TMSI.

Procedure 2	Neither TMSI nor IMSI is sent by the network

a)	The MS is made to perform a location updating as described in the method of test of II.5.3.7.4. The SS sends the LOCATION UPDATING ACCEPT message containing neither IMSI nor TMSI.

b)	The SS sends the PAGING REQUEST TYPE 1 message containing the old TMSI.

Requirements 2

1)	The MS shall wait for the SS to release the link.

2)	The MS shall respond to paging by sending the PAGING RESPONSE message containing this TMSI.

Procedure 3:	The location updating accept message contains an IMSI

a)	The MS is made to perform a normal location updating as described in the method of test of II.5.3.7.4, using IMSI.

b)	Following completion of the procedure, the SS pages the MS using:-

b.1)	a PAGING REQUEST TYPE 1 message with the old TMSI.

b.2)	a PAGING REQUEST TYPE 1 message with the IMSI.

Requirements 3

1)	The MS shall wait for the SS to release the link.

2)

2.1)	The MS shall ignore the PAGING REQUEST TYPE 1 message containing TMSI.

2.2)	The MS shall respond to the PAGING REQUEST TYPE 1 message containing its IMSI.

II.5.3.7.4.2	Location update rejected

ref: GSM 04.08, sections 4.4.4.6 and 4.4.4.8

II.5.3.7.4.2.1	Location updating rejected, "IMSI unknown in HLR" or "illegal MS"

Procedure

a)	The MS is made to perform a normal location updating as described in the method of test of II.5.3.7.4.

The SS sends LOCATION UPDATING REJECT message with cause value #2, 'IMSI unknown in HLR' and then releases the link.

b)	The RF levels are then changed again to make the MS reselect cell.

c)	The SS waits at least 7 minutes for a possible periodic updating.

d)	The MS is paged first with IMSI and then with TMSI.

e)	A mobile originating normal call establishment is attempted.

f)	An MS initiated emergency call is established.

g)	The MS is powered down.

Requirements

1)	The MS shall perform cell reselection and initiate a first location updating attempt (and enter the state 'idle, no imsi' when the link is released).

2)	After the second change of RF levels, the MS must perform cell reselection as specified in GSM 05.08 but shall not perform normal location updating.

3)	The MS shall not perform periodic updating. This is checked for a period of 7 minutes after release of the link by the SS following the location updating reject.

4)	The MS shall not respond to paging with IMSI or TMSI.

5)	The MS shall not initiate the immediate assignment procedure to establish a link in response to the normal call establishment request.

6)	The MS shall establish the emergency call correctly as described in test II.5.3.8.2.

7)	When powered down, the MS must not perform an IMSI detach.

Step f) of the procedure and requirement 6) apply only if the MS supports speech (see PICS/PIXIT statement).

II.5.3.7.4.2.2	Location updating rejected, "PLMN not allowed"

Initial conditions

1)	The MS shall be in the 'idle,updated' state (TMSI assigned) listening to BCCH/CCCH of the relevant cell. The MS is in manual mode for PLMN selection.

2)	The ATT flag value is set to 1 that is IMSI attach and detach allowed.

3)	The timer T3212 (periodic updating) is set to 6 minutes encoded as 1.

4)	The MS is switched off.

5)	The PLMN number is changed in the SS.

6)	The MS is switched on.

Procedure

a)	When the MS offers the new PLMN as available, the PLMN is manually selected. The SS responds to a LOCATION UPDATING message with the LOCATION UPDATING REJECT message containing cause value #11: "PLMN not allowed" and then releases the link.

b)	The SS waits for a possible periodic updating for 7 minutes.

c)	The MS is switched off and then switched on again (same location area and same PLMN).

d)	An MS initiated emergency call is established.

e)	The MS is switched off. The RF levels are changed to make the MS select a different cell when switched on. The MS is switched on.

f)	The LAI is changed.

g)	The MS is switched off. The PLMN is changed (back to the default values). The MS is switched on.

Requirements

1)	The MS shall offer the new PLMN as available to the user. After it has been selected, the MS shall initiate the normal location updating procedure (the MS shall store the LAI received on the BCCH in the new location area and delete the TMSI).

2)	The MS shall not perform periodic updating within 7 minutes after release of the link by the SS following the LOCATION UPDATING REJECT message.

3)	The MS shall not perform IMSI detach when powered down nor IMSI attach when powered up.

4)	The MS shall establish the emergency call correctly as described in test II.5.3.8.2.

5)	No access to the network shall be registered by the SS within one minute.

6)	No access to the network shall be registered by the SS within one minute.

7)	The MS shall initiate a normal location updating.

Step d) of the procedure and requirement 4) apply only if the MS supports speech (see PICS/PIXIT statement).

II.5.3.7.4.2.3	Location updating rejected "location area not allowed"

Initial conditions

The MS shall be in the 'idle,updated' state (TMSI assigned) listening to BCCH/CCCH of the relevant cell.

Procedure

a)	The MS is made to perform a normal location updating as described under method of test of II.5.3.7.4. The SS responds with the LOCATION UPDATING REJECT message containing cause value #12: "location area not allowed" and then releases the link.

b)	The SS waits for a possible periodic updating for 7 minutes.

c)	The MS is paged first with TMSI then with IMSI.

d)	An MS originated normal call is attempted.

e)	An MS initiated emergency call is established.

f)	The RF levels are changed to make the MS select a different cell (new cell has different LAI).

The SS continues to respond to a location updating attempt from the MS with a LOCATION UPDATING REJECT message with cause #12.

g)	The MS is switched off.

Requirements

1)	In step a), the MS shall perform the normal location updating procedure and return to the idle state upon link release by the SS.

2)	In step b), the MS shall not perform periodic updating within 7 minutes after release of the link by the SS following the LOCATION UPDATING REJECT message.

3)	In step c), the MS shall not respond to TMSI paging but shall respond to IMSI paging. The CKSN in the PAGING RESPONSE message shall indicate "no key available" as described in GSM 04.08; Table 10.8.

4)	In step d), the MS shall not initiate the immediate assignment procedure for call establishment.

5)	In step e), the MS shall establish the emergency call correctly as described in test II.5.3.8.2.

6)	In step f), the MS shall initiate a normal location updating after the change of RF levels.

7)	In step g), the MS shall not perform IMSI detach when powered down.

Message:	PAGING RESPONSE	(GSM 04.08, 9.1.24)

Information element	Comment	Value

protocol discriminator	RR management	0110

transaction Identifier	not relevant	0000

message Type	paging response	0010 0111

CKSN	no key	0111

MS classmark 2		as specified by the manufacturer

mobile identity	IMSI

Step e) of the procedure and requirement 5) apply only if the MS supports speech (see PICS/PIXIT statement).

DCS1800 only - begin

II.5.3.7.4.2.4	Location updating rejected "National Roaming not allowed in this location area"

Initial conditions

1)	The MS shall be in the 'idle,updated' state (TMSI assigned) listening to the BCCH/CCCH of the relevant cell. The list of "location areas not allowed for national roaming" shall be empty

Note:	this may be achieved by either removing the SIM or switching the MS OFF then ON.

2)	Neither location area identification a) of cell A or Location area identification b) of cell B shall have the MNC as the home PLMN, however both a) and b) shall have the same MNC. The MCC shall be the same as that of the home PLMN

These initial conditions apply to all the following procedures except where different conditions are explicitly stated within a procedure.

Procedure 1

Purpose

To test that on receipt of a rejection using the national roaming cause code, the MS ceases trying to update on that cell, that this situation continues for at least one periodic location interval period, and that the rejected area list is reset by powering down the MS. (The requirement given in GSM 04.08-DCS is that the list shall be retained for at least 12 hours. This aspect is not formally tested.)

a)	The MS is made to perform a normal location updating on cell A as described under method of test II.5.3.7.4. The SS responds with the LOCATION UPDATING REJECT message containing cause value #13: "national roaming not allowed in this location area" and then releases the link.

b)	The SS waits for a possible periodic updating on cell A for 10 minutes

c)	The MS is turned off

d)	The MS is turned on

e)	The SS waits for a location update on cell A for 2 minutes

Requirements 1

1)	In step a, the MS shall perform the normal location updating procedure and return to the idle state upon link release by the SS.

2)	In step b, the MS shall not perform periodic updating within 10 minutes after release of the link by the MS following the LOCATION UPDATING REJECT message.

3)	In step e) the MS shall initiate a periodic location update within 2 minutes.

Procedure 2

Purpose

To test that if no cell is available for national roaming, emergency calls can still be made.

a)	The MS is made to perform a normal location updating request on cell A as described under method of test II.5.3.7.4. The SS responds with the LOCATION UPDATING REJECT message containing cause value #13: "national roaming not allowed in this location area" and then releases the link.

b)	The SS waits for a possible periodic location updating on both cells A and B for 2 minutes

c)	A location updating request on cell B causes the SS to respond with the LOCATION UPDATING REJECT message containing cause value #13: "national roaming not allowed in this location area" and then releases the link.

d)	The SS waits for a possible periodic location updating on both cells A and B for 2 minutes

e)	An emergency call is originated at the MS.

Requirements 2

1)	In step a, the MS shall perform the normal location updating procedure and return to the idle state upon link release by the SS.

2)	In step b, the MS shall not perform periodic updating on cell A within 2 minutes after release of the link by the MS following the LOCATION UPDATING REJECT message. However, the MS shall perform a location updating request on cell B within 2 minutes of the release of the link.

3)	In step d, the MS shall not perform a location updating procedure on either cell A or cell B within 2 minutes after the release of the link by the SS.

4)	In step e, the MS shall correctly originate the emergency call as described in test II.5.3.8.2.

Procedure 3

Purpose

To test that at least 6 entries can be held in the list. (The requirement of the core recommendation is to store at least 10 entries. This is not fully tested by this procedure.)

a)	The MS is made to perform a normal location updating request on cell A as described under method of test II.5.3.7.4. The SS responds with the LOCATION UPDATING REJECT message containing cause value #13: "national roaming not allowed in this location area" and then releases the link.

b)	The SS waits for a possible location update request on both cells A and B for 2 minutes.

c)	The SS responds to a location updating request on cell B with the LOCATION UPDATING REJECT message containing cause value #13: "national roaming not allowed in this location area" and then releases the link.

The location area identifier of cell A shall be changed within 5 seconds of the transmission of the reject message. The code shall be chosen randomly but shall be different from any previously used in this procedure. The code shall have the same MCC as the home PLMN and shall not have the same MNC as the home PLMN.

d)	The SS waits for a possible location update request on both cells A and B for 2 minutes.

e)	The SS responds to a location updating request on cell A with the LOCATION UPDATING REJECT message containing cause value #13: "national roaming not allowed in this location area" and then releases the link.

The location area identifier of cell B shall be changed within 5 seconds of the transmission of the reject message. The code shall be chosen randomly but shall be different from any previously used in this procedure. The code shall have the same MCC as the home PLMN and shall not have the same MNC as the home PLMN.

f)	Procedures b and c are repeated once.

g)	Procedures d and e are repeated once.

h)	The SS waits for a possible location updating request on both cells A and B for 2 minutes.

i)	The SS responds to a location updating request on cell B with the LOCATION UPDATING REJECT message containing cause value #13: "national roaming not allowed in this location area" and then releases the link.

The location area identifier of cell A shall be changed within 5 seconds of the transmisison of the reject message to that of the first location area code used in this procedure.

j)	The SS waits for a possible location updating request on both cells A and B for 10 minutes.

Requirements 3

1)	In step a, the MS shall request a location update and return to the idle state upon link release by the SS.

2)	In step b, the MS shall request a location update on cell B within 2 minutes and shall not request a location update on cell A within 2 minutes.

3)	In step d, the MS shall request a location update on cell A within 2 minutes and shall not request a location update on cell B within 2 minutes.

4)	In step f, the MS shall request a location update on cell B within 2 minutes and shall not request a location update on cell A within 2 minutes.

5)	In step g, the MS shall request a location update on cell A within 2 minutes and shall not request a location update on cell B within 2 minutes.

6)	In step h, the MS shall request a location update on cell B within 2 minutes and shall not request a location update on cell A within 2 minutes.

7)	In step j, the MS shall not request a location update on cells A or B within 10 minutes.

Procedure 4

Purpose

The procedure tests that if a cell of the home PLMN is available then the MS returns to it in preference to any other available cell.

a)	The SS is made to simulate 3 cells A, B, C with 3 different location area codes. None of the cells should be part of the home PLMN.

Cells should differ in signal strength by 10 dB with cell A being the strongest and cell C the weakest. There should be a 20 dB range between A and C. A should be set to a level of -40 dBm.

The SS shall be set so that it accepts any location update requests on cells A and C.

(Note	The uplink frequency of cell B may be monitored by using the SS wideband receiver.)

b)	The SS shall wait for a periodic location updating on cell A for 7 minutes.

c)	The location area identity of cell C shall be changed to that of a location area in the home PLMN.

d)	At the next periodic location update on cell A the SS responds to the location updating request for cell A with the LOCATION UPDATING REJECT message containing cause value #13: "national roaming not allowed in this location area" and then releases the link.

e)	The SS shall wait for a location update on cell C for 2 minutes.

Requirements 4

1)	In step b) the MS should perform a normal location updating procedure on cell A.

2)	In step e) the MS shall not transmit on the uplink frequency of cell B and shall perform a normal location updating procedure on cell C within 2 minutes.

Procedure 5

Purpose

The procedure tests that if the SIM is removed the national roaming forbidden list is cleared.

a)	The MS is made to perform a normal location updating on cell A as described under method of test II.5.3.7.4. The SS responds with the LOCATION UPDATING REJECT message containing cause value #13: "national roaming not allowed in this location area" and then releases the link.

b)	The SS waits for a possible periodic updating on cell A for 7 minutes

c)	The SIM is removed

d)	The SIM is re-inserted

e)	The SS waits for a location update on cell A for 2 minutes

Requirements 5

1)	In step a, the MS will perform the normal location updating procedure and return to the idle state upon link release by the SS.

2) In step b, the MS shall not perform periodic updating within 7 minutes after release of the link by the MS following the LOCATION UPDATING REJECT message.

3) In step e) the MS shall initiate a periodic location update within 2 minutes.

DCS1800 only - end

II.5.3.7.4.3	Abnormal cases

II.5.3.7.4.3.1	The random access fails

Purpose of the test

To verify that when during the RR connection establishment phase of a location updating procedure, channel requests are not answered by the network, after expiry of T3213 the complete procedure is repeated if still necessary.

Initial conditions

For both procedure 1 and procedure 2 the initial conditions of II.5.3.7.4 shall apply.

Procedure 1

a)	The MS is made to perform a normal location updating as described in the method of the test of II.5.3.7.4.

b)	Cell B as simulated by the SS does not answer to (max-retrans + 1) CHANNEL REQUEST messages.

During this step b), the RF level of cell A as simulated by the SS is set sufficiently low to ensure that cell A is not suitable as defined in GSM 05.08 section 6.6.2.

c)	After reception of the first CHANNEL REQUEST message, cell B as simulated by the SS no longer refuses the establishment of the RR connection.

Requirements 1

1)	In step b), the MS must send (max-retrans + 1) CHANNEL REQUEST messages to cell B as simulated by the SS.

2)	Within a period t seconds, where T3213 < t < 15, after having sent the last CHANNEL REQUEST message the MS must restart establishing an RR connection with cell B as simulated by the SS.

3)	The RR connection having been established, the MS must initialize a normal location updating to cell B as simulated by the SS.

Note:	The upper limit of 15 s, for t, is to allow time for the MS to perform cell reselection and because it is unclear at which point the timer T3213 is started.

Procedure 2

a)	The MS is made to perform a normal location updating as described in the method of the test of II.5.3.7.4.

b)	Cell B as simulated by the SS does not answer to (max-retrans + 1) CHANNEL REQUEST messages.

After reception of the (max-retrans + 1)-th CHANNEL REQUEST message, after 2 s the RF level of cell A as simulated by the SS is set sufficiently high to ensure that cell A is suitable as defined in 6.6.2 of GSM 05.08 (taking into account the cell reselection hysteresis).

Requirements 2

1)	In step b), the MS shall send (max-retrans + 1) CHANNEL REQUEST messages to cell B as simulated by the SS.

2)	After step b) the MS shall not initiate a normal location updating procedure. (This is checked during 6 s.)

II.5.3.7.4.3.2	Attempt counter smaller than 4, stored LAI different from broadcast LAI

Initial conditions

The MS shall be in the "idle updated" state (TMSI, LAI, Kc and Kc sequence number are in the MS) listening to BCCH/CCCH of the relevant cell. These initial conditions shall be met before each of the following procedures 1 to 6.

Procedure 1:	To verify that the MS performs normal location updating procedures when its attempt counter is smaller than 4.

a)	The MS is brought to perform a location updating procedure as described in the method of test of II.5.3.7.4

b)	The SS sends a LOCATION UPDATING REJECT message with cause

GSM900:	#i. (This cause value is randomly chosen in table 10.44 of GSM 04.08, causes #2, #3, #11 and #12 being excluded.)

DCS1800:	#17 (network failure) or #22 (congestion).

c)	During the location updating procedure, the SS stops any RF transmission on the dedicated channel.

d)	During the location updating procedure the SS sends a CHANNEL RELEASE message before the normal end of the procedure (i.e. before sending the LOCATION UPDATING ACCEPT message).

Requirements 1

1)	After each of steps b), c) and d) the MS shall:

1.1)	wait T3211 s (the SS checks that there is no procedure attempted by the MS during T3211 after the channel release occuring during the location updating failure),

1.2)	immediately after that send a CHANNEL REQUEST message in order to perform a location updating procedure. The MS shall transfer on the main DCCH a LOCATION UPDATING REQUEST message. The location updating type is put to "normal location updating" encoded as 0 0, the key sequence in the ciphering key sequence number is encoded as 1 1 1 indicating "no key is available", the identity field contains the IMSI of the MS.

Procedure 2:	To check that the MS does not perform the IMSI detach procedure when in "idle not updated" state.

a)	The MS is brought to perform a location updating procedure as described in the method of test of II.5.3.7.4.

b)	The SS chooses randomly one among the three steps b), c), d) of procedure 1 of section II.5.3.7.4.3.2, let (i) be this choice.

The SS performs step (i) of procedure 1 of section II.5.3.7.4.3.2.

c)	The MS is paged continuouosly during 8 seconds with the TMSI it used before the location updating failure.

d)	The MS is switched off.

Requirements 2

1)	After step c) of procedure 2, the MS shall ignore the paging request. The SS checks that there is no answer to the paging from the MS during 12s after the sending of the first paging request.

Note:	If the MS tries a location updating procedure, the SS shall react as in step b) and does not restart the 8s or 12s time.

2)	After step d) during at least 30 s, the MS shall not trigger any procedure.

Procedure 3	To verify that the MS can perform an emergency call.

a)	The MS is brought to perform a location updating procedure as described in the method of test of II.5.3.7.4.

b)	The SS triggers n authentication procedures.

Note:	'n' shall be chosen in such a way that T3210 expires.

c)	The MS is made to perform an emergency call.

d)	The SS allocates the MS a DCCH through the immediate assignment procedure.

Requirements 3

1)	After step b) the MS shall abort the RR connection.

2)	After step c) the MS applies for a dedicated radio resource.

3)	After step d) the MS sends a CM SERVICE REQUEST message with CM service type put to "emergency call establishment" encoded as 0010.

3.1)	The key sequence in the ciphering key sequence number shall be encoded as 1 1 1 indicating "no key is available".

3.2)	The identity field shall contain the IMSI of the MS.

4)	The MS sends an EMERGENCY SETUP message on the main DCCH.

Procedure 3 and requirements 3 only apply to MS supporting speech (see PICS/PIXIT statement).

Procedure 4	To verify that in "idle not updated" state the MS uses requests from CM layer other than emergency call as triggering of a normal location updating procedure.

a)	The MS is brought to perform a location updating procedure as described in the method of test of II.5.3.7.4.

b)	The SS chooses randomly one among the three steps b), c), d) of procedure 1 of section II.5.3.7.4.3.2, let (i) be this choice.

The SS performs step (i) of procedure 1 of section II.5.3.7.4.3.2.

c)	The MS is made to perform a mobile originating call set-up.

d)	The SS allocates the MS a DCCH through the immediate assignment procedure.

Requirements 4

1)	After step c) the MS sends a CHANNEL REQUEST message with establishment cause "all other cases" encoded as 0 0 0 .

2)	After step d) the MS sends a LOCATION UPDATING REQUEST message. This message is encoded as indicated in requirement 1.

Procedure 5	To check that in the "idle not updated" state the MS answers to paging with IMSI.

a)	The MS is brought to perform a location updating procedure as described in the method of test of II.5.3.7.4.

b)	The SS chooses randomly one among the three steps b), c), d) of procedure 1 of section II.5.3.7.4.3.2, let (i) be this choice.

The SS performs step (i) of procedure 1 of section II.5.3.7.4.3.2.

c)	The MS is paged with its IMSI.

d)	The SS allocated a DCCH to the MS.

Requirements 5

1)	After step c) the MS sends a CHANNEL REQUEST message, the establishment cause being "answer to paging" encoded as 100.

2)	After step d) the MS sends a PAGING RESPONSE message, the ciphering key sequence number information element indicates that no key is available (i.e. bits 1, 2 and 3 are encoded as 1 1 1 .) and the mobile identity information element includes the IMSI of the MS.

Procedure 6	To verify that the MS performs a normal location updating procedure if it enters a new cell while being in the "idle not updated" state.

a)	The MS is brought to perform a location updating procedure as described in the method of test of II.5.3.7.4.

b)	The SS chooses randomly one among the three steps b), c), d) of procedure 1 of section II.5.3.7.4.3.2, let (i) be this choice.

The SS performs step (i) of procedure 1 of section II.5.3.7.4.3.2.

c)	The RF power level of cell B is reduced until the MS selects cell A.

d)	The SS measures the time interval between the channel release (this is the one due to the location updating failure occurring during step b)) and the sending of the CHANNEL REQUEST message (this is the one used by the MS because of the normal location updating procedure triggered by step c)).

Requirements 6

1)	The MS shall perform a normal location updating procedure.

2)	The time interval between the channel release and the sending of the CHANNEL REQUEST message measured in step d), shall be less than 0.8*T3211.

II.5.3.7.4.3.3	Attempt counter greater or equal to 4, Stored LAI different from broadcast LAI

Initial conditions

The initial conditions of II.5.3.7.4.3.2 shall be met before each of the following procedures 1 to 5.

Purpose of test procedure 1

To verify that the MS performs only periodic location updating procedures, that it does not perform normal location updating procedures when its attempt counter has reached value 4 and that the MS resets its attempt counter after a timer T3212 expiry.

Procedure 1

a)	The MS is brought to perform a location updating procedure as described in the method of test of II.5.3.7.4.

b)	The SS chooses randomly one among the three steps b), c), d) of procedure 1 of section II.5.3.7.4.3.2, let (i) be this choice.

The SS performs step (i) of procedure 1 of section II.5.3.7.4.3.2.

c)	The SS chooses randomly one among the three steps b), c), d) of procedure 1 of section II.5.3.7.4.3.2, let (i) be this choice.

The SS performs step (i) of procedure 1 of section II.5.3.7.4.3.2.

d)	The SS triggers n authentication procedures.

Note:	'n' shall be chosen in such a way that T3210 expires.

e)	The SS chooses randomly one among the three steps b), c), d) of procedure 1 of section II.5.3.7.4.3.2, let (i) be this choice.

The SS performs step (i) of procedure 1 of section II.5.3.7.4.3.2.

f)	During the periodic location updating procedure caused by step e) the SS sends a LOCATION UPDATING REJECT message with cause #17 "network failure".

Requirements 1

1)	After the channel release procedure occuring during step e) the MS shall not trigger any procedure within a time defined as T3212 minus 15s.

2)	After this delay the MS shall perform a periodic location updating procedure. The MS shall transfer on the main DCCH a LOCATION UPDATING REQUEST message. The location updating type is put to "periodic updating" encoded as 0 1, the key sequence in the ciphering key sequence number is encoded ad 1 1 1 indicating "no key is available", the identity field contains the IMSI of the MS.

3)	After step f) the MS shall:

3.1)	wait T3211 s (the SS checks that there is no procedure attempted by the MS during T3211 after the channel release occuring during the location updating failure),

3.2)	immediately after that send a CHANNEL REQUEST message in order to perform a location updating procedure. The MS shall transfer on the main DCCH a LOCATION UPDATING REQUEST message. The location updating type is put to "normal location updating" encoded as 0 0, the key sequence in the ciphering key sequence number is encoded as 1 1 1 indicating "no key is available", the identity field contains the IMSI of the MS.

Purpose of the procedures 2 to 5

To verify that the MS still follows the "idle not updated" state requirements after its attempt counter has reached value 4. (In the procedures of II.5.3.7.4.3.2 the attempt counter has reached value 1.).

To verify that the attempt counter is propoerly reset when required.

Procedure 2	To verify that the MS can perform an emergency call

a)	The MS is brought to perform a location updating procedure as described in the method of test of II.5.3.7.4.

b)	The SS sends a LOCATION UPDATING REJECT message with cause

GSM900:	#i. (This cause value is randomly chosen in table 10.44 of GSM 04.08, causes #2, #3, #11 and #12 being excluded.)

DCS1800:	#17 (network failure) or #22 (congestion).

c)	During the location updating procedure, the SS stops any RF transmission on the dedicated channel.

d)	During the location updating procedure the SS sends a CHANNEL RELEASE message before the normal end of the procedure (i.e. before sending the LOCATION UPDATING ACCEPT message).

e)	The SS chooses randomly one among the three steps b), c), d) of procedure 1 of section II.5.3.7.4.3.2, let (i) be this choice.

The SS performs step (i) of procedure 1 of section II.5.3.7.4.3.2.

f)	The MS is made to perform an emergency call.

g)	The SS allocates the MS a DCCH through the immediate assignment procedure.

h)	The MS is switched off.

Requirements 2

1)	After step f) the MS applies for a dedicated resource.

2)	After step g), the MS sends a CM SERVICE REQUEST message with CM service type put to "emergency call establishment" encoded as 0010.

2.1)	The key sequence in the ciphering key sequence number shall be encoded as 1 1 1 indicating "no key is available".

2.2)	The identity field shall contain the IMSI of the MS.

3)	The MS sends an EMERGENCY SETUP message on the main DCCH.

Procedure 2 and requirements 2 only apply to MS supports speech (see PICS/PIXIT statement).

Procedure 3	To verify that in "idle not updated" state the MS uses requests from CM layer other than emergency call as triggering of a normal location updating procedure.

To verify that the attempt counter is reset after a location updating triggered by request from CM layer.

a)	The MS is brought to perform a location updating procedure as described in the method of test of II.5.3.7.4.

b)	The SS sends a LOCATION UPDATING REJECT message with cause

GSM900:	#i. (This cause value is randomly chosen in table 10.44 of GSM 04.08, causes #2, #3, #11 and #12 being excluded.)

DCS1800:	#17 (network failure) or #22 (congestion).

c)	During the location updating procedure, the SS stops any RF transmission on the dedicated channel.

d)	During the location updating procedure the SS sends a CHANNEL RELEASE message before the normal end of the procedure (i.e. before sending the LOCATION UPDATING ACCEPT message).

e)	The SS chooses randomly one among the three steps b), c), d) of procedure 1 of section II.5.3.7.4.3.2, let (i) be this choice.

The SS performs step (i) of procedure 1 of section II.5.3.7.4.3.2.

f)	The MS is made to perform a mobile originating call setup.

g)	The SS allocates the MS a DCCH through the immediate assignment procedure.

h)	The SS chooses randomly one among the three steps b), c), d) of procedure 1 of section II.5.3.7.4.3.2, let (i) be this choice.

The SS performs step (i) of procedure 1 of section II.5.3.7.4.3.2.

Requirements 3

1)	After step f) the MS sends a CHANNEL REQUEST message with establishment cause "all other cases" encoded as 0 0 0 .

2)	After step g) the MS sends a LOCATION UPDATING REQUEST message. This message is encoded as indicated in requirement 1.

3)	After step h), the MS shall:

3.1)	wait T3211 s (the SS checks that there is no procedure attempted by the MS during T3211 after the channel release occuring during the location updating failure),

3.2)	immediately after that, send a CHANNEL REQUEST message in order to perform a location updating procedure. The MS shall transfer on the main DCCH a LOCATION UPDATING REQUEST message. The location updating type is put to "normal location updating" encoded as 0 0, the key sequence in the ciphering key sequence number is encoded as 1 1 1 indicating "no key is available", the identity field contains the IMSI of the MS.

Procedure 4	To check that in the "idle not updated" state the MS answers to paging with IMSI.

a)	The MS is brought to perform a location updating procedure as described in the method of test of II.5.3.7.4.

b)	The SS sends a LOCATION UPDATING REJECT message with cause

GSM900:	#i. (This cause value is randomly chosen in table 10.44 of GSM 04.08, causes #2, #3, #11 and #12 being excluded.)

DCS1800:	#17 (network failure) or #22 (congestion).

c)	During the location updating procedure, the SS stops any RF transmission on the dedicated channel.

d)	During the location updating procedure the SS sends a CHANNEL RELEASE message before the normal end of the procedure (i.e. before sending the LOCATION UPDATING ACCEPT message).

e)	The SS chooses randomly one among the three steps b), c), d) of procedure 1 of section II.5.3.7.4.3.2, let (i) be this choice.

The SS performs step (i) of procedure 1 of section II.5.3.7.4.3.2.

f)	The MS is paged with its IMSI.

g)	The SS allocates the MS a DCCH through the immediate assignment procedure.

Requirements 4

1)	After step f) the MS sends a CHANNEL REQUEST message, the establishment cause being "answer to paging" encoded as 100.

2)	After step g) the MS sends a PAGING RESPONSE message, the ciphering key sequence number information element indicates that no key is available (i.e. bits 1, 2 and 3 are encoded as 1 1 1 .) and the mobile identity information element includes the IMSI of the MS.

Procedure 5	To verify that the MS performs a normal location updating procedure if it enters a new cell while being in the "idle not updated" state.

To verify that the attempt counter is reset after a new location area is entered.

a)	The MS is brought to perform a location updating procedure as described in the method of test of II.5.3.7.4.

b)	The SS sends a LOCATION UPDATING REJECT message with cause

GSM900:	#i. (This cause value is randomly chosen in table 10.44 of GSM 04.08, causes #2, #3, #11 and #12 being excluded.)

DCS1800:	#17 (network failure) or #22 (congestion).

c)	During the location updating procedure, the SS stops any RF transmission on the dedicated channel.

d)	During the location updating procedure the SS sends a CHANNEL RELEASE message before the normal end of the procedure (i.e. before sending the LOCATION UPDATING ACCEPT message).

e)	The SS chooses randomly one among the three steps b), c), d) of procedure 1 of section II.5.3.7.4.3.2, let (i) be this choice.

The SS performs step (i) of procedure 1 of section II.5.3.7.4.3.2.

f)	The RF power level of cell B is reduced until the MS selects cell A.

g)	The SS chooses randomly one among the three steps b), c), d) of procedure 1 of section II.5.3.7.4.3.2, let (i) be this choice.

The SS performs step (i) of procedure 1 of section II.5.3.7.4.3.2.

Requirement 5

1)	After step f), the MS shall perform a normal location updating procedure.

2)	After step g) the MS shall:

2.1)	wait T3211 s (the SS checks that there is no procedure attempted by the MS during T3211 after the channel release occuring during the location updating failure),

2.2)	immediately after that send a CHANNEL REQUEST message in order to perform a location updating procedure. The MS shall transfer on the main DCCH a LOCATION UPDATING REQUEST message. The location updating type is put to "normal location updating" encoded as 0 0, the key sequence in the ciphering key sequence number is encoded as 1 1 1 indicating "no key is available", the identity field contains the IMSI of the MS.

II.5.3.7.4.3.4	Attempt counter smaller than 4, Stored LAI equal to broadcast LAI

Initial conditions

The initial conditions described at the beginning of II.5.3.7.4.3.2 shall be met before each of the following procedures of this section.

Purpose of the test procedures 1 and 2

To verify that the MS is the "idle updated" state. To verify that timer T3211 is reset after a MM connection establishment.

Procedure 1

a)	The MS is made to perform a periodic updating procedure.

b)	The SS chooses randomly one among the three steps b), c), d) of procedure 1 of section II.5.3.7.4.3.2, let (i) be this choice.

The SS performs step (i) of procedure 1 of section II.5.3.7.4.3.2.

c)	Immediately after that the MS is made to perform a mobile originated call.

d)	The SS allocates the MS a DCCH through the immediate assignment procedure.

e)	After the SETUP message is received (Note: this transmission shows that the MM connection is succesfully established.) from the MS, the SS sends to the MS a CHANNEL RELEASE message.

f)	The SS observes whether a location updating procedure is attempted by the MS during 0.5*T3212 after the reception of the SETUP message.

Requirements 1

1)	After step c) the MS shall send a CHANNEL REQUEST message with the establishment cause "other services required by the mobile user" encoded as 1 1 1.

2)	After step d) the MS shall transmit on the DCCH a CM SERVICE REQUEST message, the CM service type indicates "mobile originating call establishment" encoded as 0 0 0 1, the ciphering key sequence number and the LAI are those which have been allocated to the MS, the mobile identity field contains the TMSI which has been allocated to the MS.

3)	The MS shall not attempt a location updating procedure during at least 0.5*T3212 after the reception of the SETUP message.

Procedure 2

a)	The MS is made to perform an IMSI attach procedure.

b)	The SS chooses randomly one among the three steps b), c), d) of procedure 1 of section II.5.3.7.4.3.2, let (i) be this choice.

The SS performs step (i) of procedure 1 of section II.5.3.7.4.3.2.

c)	Immediately after that the MS is made to perform a mobile originated call.

d)	The SS allocates the MS a DCCH through the immediate assignment procedure.

e)	The SS observes whether a location updating procedure is attempted by the MS during 0.5*T3212 after the reception of the SETUP message.

Requirements 2

1)	After step c) the MS shall send a CHANNEL REQUEST message with the establishment cause "other services required by the mobile user" encoded as 1 1 1.

2)	After step d) the MS shall transmit on the DCCH a CM SERVICE REQUEST message, the CM service type indicates "mobile originating call establishment" encoded as 0 0 0 1, the ciphering key sequence number and the LAI are those which have been allocated to the MS, the mobile identity field contains the TMSI which has been allocated to the MS.

3)	The MS shall not attempt a location updating procedure during at least 0.5*T3212 after the reception of the SETUP message.

Purpose of the test procedures 3, 4 and 5

To verify that the MS uses the T3211 timer and that it enters the "idle not updated" state when its attempt counter reaches value 4 even in the case where the stored LAI is equal to the broadcast LAI.

Procedure 3

a)	The MS is made to perform a periodic location updating procedure.

b)	The SS chooses randomly one among the three steps b), c), d) of procedure 1 of section II.5.3.7.4.3.2, let (i) be this choice.

The SS performs step (i) of procedure 1 of section II.5.3.7.4.3.2.

c)	The SS chooses randomly one among the three steps b), c), d) of procedure 1 of section II.5.3.7.4.3.2, let (i) be this choice.

The SS performs step (i) of procedure 1 of section II.5.3.7.4.3.2.

d)	The SS chooses randomly one among the three steps b), c), d) of procedure 1 of section II.5.3.7.4.3.2, let (i) be this choice.

The SS performs step (i) of procedure 1 of section II.5.3.7.4.3.2.

e)	The SS chooses randomly one among the three steps b), c), d) of procedure 1 of section II.5.3.7.4.3.2, let (i) be this choice.

The SS performs step (i) of procedure 1 of section II.5.3.7.4.3.2.

Requirements 3

1)	After each of steps a), b), c), d) the MS shall transmit on the DCCH a LOCATION UPDATING REQUEST message. The location updating type is put to "periodic updating" encoded as 0 1, the ciphering key sequence number and the LAI are those which have been allocated to the MS; the mobile identity field contains the TMSI which has been allocated to the MS.

2)	After each of steps b), c) and d) the MS shall

2.1)	wait T3211 s (the SS checks that there is no procedure attempted by the MS during T3211 after the channel release occuring during the location updating failure),

2.2)	immediately after that, send a CHANNEL REQUEST message in order to perform a location updating procedure.

3)	After the channel release procedure occuring during step e) the MS shall not trigger any procedure within a time defined as T3212 minus 15s.

4)	After this delay the MS shall perform a periodic location updating procedure. The MS shall transfer on the main DCCH a LOCATION UPDATING REQUEST message. The location updating type is put to "periodic updating" encoded as 0 1, the key sequence in the ciphering key sequence number is encoded as 1 1 1 indicating "no key is available", the identity field contains the IMSI of the MS.

Procedure 4

a)	Steps a) to e) of procedure 3 are performed.

b)	The MS is made to perform a mobile originated call setup.

c)	The SS allocates the MS a DCCH through the immediate assignment procedure.

Requirements 4

1)	After step b) the MS sends a CHANNEL REQUEST message with establishment cause "all other cases" encoded as 0 0 0 .

2)	After step c) the MS sends a LOCATION UPDATING REQUEST message.

Procedure 5

Perform again procedures 3 and 4 after having replaced each step a) by:

a)	The MS is made to perform an IMSI attach procedure.

Requirements 5

"Requirements 3" and "requirements 4" respectively apply.

II.5.3.7.4.4	Test of RR connection release after location updating

Procedure

a)	The MS is made to perform a successful location updating as described in the method of measurement of II.5.3.7.4.

b)	The SS does not release the RR connection within timer value T3240.

Requirement

1)	The MS shall abort the RR connection ("local release") after T3240 expiry and go to the "idle" state (with appropriate updated status).

II.5.3.7.4.5	Periodic updating

II.5.3.7.4.5.1	Reduction of location updating timer

No unique test could be defined for the following conformance requirement.

When the location updating timer value is reduced MSs, of which the last location updating has taken place longer ago than the new timer value indicates, shall spread their reaction time before performing a location updating, to prevent a collission of many location updatings from all those MSs.

There is no exact requirement on how this spread should be achieved, and various options are open to the manufacturer.

II.5.3.7.4.5.2	Test of periodic updating

Initial Conditions

1)	The T3212 timeout value in the SYSTEM INFORMATION TYPE 3 message is set to 1; corresponding to 1/10 hour, i.e. 6 minutes between periodic updatings.

2)	The MS is brought into the state "idle, updated".

3)	Different cases of resetting timer T3212 are tested. The ATT flag is set to 0 to forbid IMSI attach and detach procedures.

Procedure 1

a)	The MS is made to perform a normal successful location updating as described in II.5.3.7.4.

b)	An MS originated call is established 1 minute after the location updating.

c)	The time when the call is cleared is noted.

d)	The SS waits until the periodic location updating.

e)	The MS is paged with IMSI 1 minute after the periodic updating.

f)	The SS sends an IMMEDIATE ASSIGNMENT message and then releases the link.

g)	The SS waits for periodic updating.

h)	After this periodic updating the MS is switched off for 1 minute (including the time needed for the MS switch off and switch on tasks) and then on again.

i)	The SS waits for a periodic updating.

Requirements 1

1)	The MS shall perform periodic updating:

1.1)	In step d) in a time interval defined as 6 minutes - 15 seconds to 6 minutes + 45 seconds after the release of the RR connection by the SS.

1.2)	In step g) in a time interval defined as 6 minutes - 15 seconds to 6 minutes + 45 seconds after the release of the RR connection by the SS.

1.3)	In step i) in a time interval defined as 7 minutes - 15 seconds to 7 minutes + 45 seconds after periodic updating (this value includes the power off time).

Procedure 2

a)	The ATT parameter in the SYSTEM INFORMATION TYPE 3 message shall be set to 1 (attach/detach allowed).

b)	The MS is brought into the state "idle, updated".

c)	If the MS offers a switch off facility, the MS is switched off. Otherwise, if removal of the SIM without disconnection of the power supply is possible (see PIXIT statement), the SIM is removed; else, the power supply is disconnected.

d)	The SS waits 20s. During that time, the MS may initiate an IMSI detach procedure.

e)	If in step c) the MS had been switched off, the MS is switched on. Otherwise if in step c) the SIM was removed the SIM is re-inserted; else, the power supply is connected to the MS and, if necessary, the MS is activated.

f)	The SS shall answer to the location updating request (with type IMSI attach).

g)	The SS waits for the periodic location updating.

Requirement 2

1)	If in step c) the MS had been switched off or the SIM had been removed, the MS must initiate an IMSI detach procedure after step c).

2)	The MS shall send a LOCATION UPDATING REQUEST with type IMSI attach after step e).

3)	The MS shall perform a periodic location updating in a time interval defined as 6 minutes - 15 seconds to 6 minutes + 45 seconds after 2).

Procedure 3

This procedure is the same as procedure 1 except that the broadcast value of the T3212 is set to 12 minutes, encoded as 2.

Requirements 3

1)	The MS shall perform periodic updating:

1.1)	In step d) in a time interval defined as 12 minutes - 15 seconds to 12 minutes + 45 seconds after the release of the RR connection by the SS.

1.2)	In step g) in a time interval defined as 12 minutes - 15 seconds to 12 minutes + 45 seconds after the release of the RR connection by the SS.

1.3)	In step i) in a time interval defined as 13 minutes - 15 seconds to 13 minutes + 45 seconds after periodic updating (this value includes the power off time).

II.5.3.7.5	MM Connection Establishment

II.5.3.7.5.1	Introduction

MM connection establishment is only of particular relevance in the case of MS origination, this is because MM connection establishment is a CM service provision and for the network originated case that is inherent in the paging elementary procedure.

The call re-establishment aspects of CM service provision are dealt with in the next test.

The emergency call establishment aspects of CM service provision are dealt with in test II.5.3.8.2.

II.5.3.7.5.2	Successful MM connection establishment, with cipher mode setting

II.5.3.7.5.2.1	Purpose of the test

The purpose of this test is to verify that the MS can correctly set up an MM connection in an origination and interpret cipher mode setting as acceptance of its CM service request.

II.5.3.7.5.2.2	Initial conditions

1)	The SS simulates a cell with BCCH, CCCH, SDCCH and TCH.

2)	The MS shall be in the "idle updated" state as after a successful location update as in test II.5.3.7.4.

II.5.3.7.5.2.3	Procedure 1

a)	After the initial conditions have been set up, the MS is made to initiate a call.

b)	The SS responds to the MS channel request with an IMMEDIATE ASSIGNMENT message as described in II.5.3.6.1.3.

II.5.3.7.5.2.4	Requirements 1

1)	The MS goes to the correct SDCCH and sends a CM SERVICE REQUEST message.

Message:	CM SERVICE REQUEST

Information element	Comment	Value

protocol discriminator	MM message	0101

transaction identifier	not relevant	000

message type		0x10 0100

CM service type	MO call establishment	0001

ciphering key sequence number as stored in the MS

mobile station classmark 2	as specified by the manufacturer

mobile identity	TMSI

II.5.3.7.5.2.5	Procedure 2

a)	The SS performs a successful authentication with the MS as tested in II.5.3.7.2.

b)	The SS performs a successful cipher mode setting with the MS as tested in II.5.3.6.8.

c)	The SS ceases all transmissions and allows the MS to timeout.

II.5.3.7.5.2.6	Requirements 2

1)	The MS shall consider the cipher mode setting as acceptance of its CM service request and so send its SETUP message.

II.5.3.7.5.3	Successful MM connection establishment, without cipher mode setting

II.5.3.7.5.3.1	Purpose of the test

The purpose of this test is to verify that the MS can correctly set up an MM connection in an origination when cipher mode setting is not required.

II.5.3.7.5.3.2	Initial conditions

1)	The SS simulates a cell with BCCH, CCCH, SDCCH and TCH.

2)	The MS shall be in the "idle updated" state as after a successful location update in test II.5.3.7.4.

II.5.3.7.5.3.3	Procedure 1

a)	After the initial conditions have been set up, the MS is made to initiate a call.

b)	The SS responds to the MS channel request with an IMMEDIATE ASSIGNMENT message as described in II.5.3.6.1.3.

II.5.3.7.5.3.4	Requirements 1

1)	The MS goes to the correct SDCCH and sends a CM service request message.

Message:	CM SERVICE REQUEST

Information element	Comment	Value

protocol discriminator	MM message	0101

transaction identifier	not relevant	000

message type		0x10 0100

CM service type	MO call establishment	0001

ciphering key sequence number as stored in the MS

mobile station classmark 2	as specified by the manufacturer

mobile identity	TMSI

II.5.3.7.5.3.5	Procedure 2

a)	The SS sends a CM SERVICE ACCEPT message.

Message:	CM SERVICE ACCEPT

Information element	Comment	Value

protocol discriminator	MM message	0101

transaction identifier	not relevant

message type	CM service accept	0010 0001

b)	The SS ceases all transmissions and allows the MS to timeout.

II.5.3.7.5.3.6	Requirements 2

1)	The MS shall commence sending its SETUP message.

II.5.3.7.5.4	MM Connection establishment rejected

II.5.3.7.5.4.1	Purpose of the test

The purpose of this test is to verify that the MS can correctly accept a CM service reject in an origination.

II.5.3.7.5.4.2	Initial conditions

1)	The SS simulates a cell with BCCH, CCCH, SDCCH and TCH.

2)	The MS shall be in the "idle updated" state as after a successful location update in test II.5.3.7.4.

II.5.3.7.5.4.3	Procedure 1

a)	After the initial conditions have been set up, the MS is made to initiate a call.

b)	The SS responds to the MS channel request with an IMMEDIATE ASSIGNMENT message as described in II.5.3.6.1.3.

II.5.3.7.5.4.4	Requirements 1

1)	The MS goes to the correct SDCCH and sends a CM SERVICE REQUEST message.

Message:	CM SERVICE REQUEST

Information element	Comment	Value

protocol discriminator	MM message	0101

transaction identifier	not relevant	000

message type		0x10 0100

CM service type	MO call establishment	0001

ciphering key sequence number	as stored in the MS

mobile station classmark 2	as specified by the manufacturer

mobile identity	TMSI

II.5.3.7.5.4.5	Procedure 2

a)	The SS sends a CM SERVICE REJECT message.

Message:	CM SERVICE REJECT

Information element	Comment	Value

protocol discriminator	MM message	0101

transaction identifier	not relevant

message type		0010 0010

reject cause	service option 33

not subscribed

II.5.3.7.5.4.6	Requirements 2

1)	The MS shall not send its SETUP message.

II.5.3.7.5.5	MM Connection Establishment rejected (cause 4)

II.5.3.7.5.5.1	Purpose of the test

The purpose of this test is to verify that the MS can correctly accept a CM SERVICE REJECT message with cause #4.

II.5.3.7.5.5.2	Initial conditions

1)	The SS simulates a cell with BCCH, CCCH, SDCCH and TCH.

2)	The MS shall be in the "idle updated" state as after a successful location update in test II.5.3.7.4.

II.5.3.7.5.5.3	Procedure 1

a)	After the initial conditions have been set up, the MS is made to initiate a call.

b)	The SS responds to the MS channel request with an IMMEDIATE ASSIGNMENT message as described in II.5.3.6.1.3.

II.5.3.7.5.5.4	Requirement 1

1)	The MS goes to the correct SDCCH and sends a CM SERVICE REQUEST message.

Message:	CM SERVICE REQUEST

Information element	Comment	Value

protocol discriminator	MM message	0101

transaction identifier	not relevant	000

message type		0x10 0100

CM service type	MO call establishment	0001

ciphering key sequence number	as stored in the MS

mobile station classmark 2	as specified by the manufacturer

mobile identity	TMSI

II.5.3.7.5.5.5	Procedure 2

a)	The SS sends a CM SERVICE REJECT message.

Message:	CM SERVICE REJECT

Information element	Comment	Value

protocol discriminator	MM message	0101

transaction identifier	not relevant	000

message type		0010 0010

reject cause	IMSI unknown 4

in VLR

II.5.3.7.5.5.6	Requirements 2

1)	The MS shall wait for the network to release the RR connection and change its status to "not updated".

2)	The MS shall carry out a location updating.

II.5.3.7.5.6	Expiry of timer T 3230

Note:	This test is not applicable.

II.5.3.7.6	MM connection release

Note:	This comes basically down to testing the functioning of timer T3240. Testing of that timer is sufficiently covered by the tests on location updating. No separate test on MM connection release is foreseen.