II.5.3.9	Testing of structured procedures

Ref: GSM 04.08 sections 7.3.2 through 7.3.4

II.5.3.9.1	General

The purpose of these tests is to complement the testing of elementary procedures to verify that the MS functions also in the "field" and not only on the "test bench".

The elementary procedures are combined into sequences, "structured procedures" as defined in GSM 04.08 section 7, representing true sequences of events for normal MS operation.

Only the main structured procedures are deemed necessary to test.

Mobile originating and terminating calls are tested in cases of both early and late assignment of traffic channel.

In two of the cases the call establishment test is followed by testing of call release initiated by network and mobile respectively.

The tests are performed only for a successful outcome of each elementary procedure.

Time requirements are indicated typically "(T3240-MM-MS)", showing timer identity, sublayer and (MS/NW) side where the timer is used (NW = Network).

The timer values are defined in GSM 04.08, section 11.

For an MS supporting speech the test procedures in II.5.3.9.2, II.5.3.9.3, II.5.3.9.4 and II.5.3.9.5 are performed for speech (teleservice 11, telephony).

For an MS not supporting speech, for each of the test procedures in II.5.3.9.2, II.5.3.9.3, II.5.3.9.4 and II.5.3.9.5 a teleservice supported by the MS (see PICS / PIXIT) statement) is chosen, and the test is performed corresponding to that teleservice.

In cases where the tested teleservice is supported via an R or S interface and initiation of a Mobile Originated Call (MOC) can be performed via the MMI or through the appropriate interface, procedure II.5.3.9.2 shall be performed initiating the MOC from the MMI and procedure II.5.3.9.3 initiating the MOC through the appropriate interface.

II.5.3.9.2	MS originating call establishment, early assignment, release init. by network

II.5.3.9.2.1	Purpose of the test

a)	To test the MS ability to originate a call following the sequence of elementary signalling procedures given in GSM 04.08 section 7.3.2.1a.

b)	To test the MS ability to release a call, when the release is initiated by the network, following the sequence of elementary signalling procedures given in GSM 04.08 section 7.3.4a.

c)	To verify the MS display of the called number (see PICS / PIXIT statement).

II.5.3.9.2.2	Method of test

a)	The SS simulates one cell using the default parameter values for "cell 1" in section II.5.3.3 "test of MS functions in idle mode".

b)	The MS is brought into the MM state "idle, updated", with a TMSI assigned, listening to the BCCH/CCCH of the active cell.

c)	The MS is made to initiate a call for the teleservice selected for the test and the SS accepts the CHANNEL REQUEST message and returns the IMMEDIATE ASSIGNMENT message before the timer (T3120-RR-MS) has expired, assuming that the randomly drawn value of T3120 is the lowest possible.

d)	The SS accepts the service request from the MS via the data link establishment procedure.

e)	The SS sends message AUTHENTICATION REQUEST.

f)	After reception of the AUTHENTICATION RESPONSE message the SS sends message CIPHERING MODE COMMAND within (T3230-MM-MS) s after the first access burst, and starts deciphering.

g)	After reception of the CIPHER MODE COMPLETE message the SS starts enciphering.

h)	The SS sends the message CALL PROCEEDING within (T303-CC-MS) s after the SS has received the message SETUP.

i)	The SS then sends message ASSIGNMENT COMMAND.

j)	Having received message ASSIGNMENT COMPLETE, the SS sends the message ALERTING within (T310-CC-MS) s after the sending of CALL PROCEEDING.

k)	Within (T301-CC-MS) s after receiving the message ALERTING the SS sends the CONNECT message.

l)	The SS initiates clearing of the call by sending the message DISCONNECT.

m)	Within (T308-CC-MS) s after reception of the message RELEASE the SS sends RELEASE COMPLETE followed, within a further (T3240-MM-MS) s, by CHANNEL RELEASE.

GSM900 only - begin

Message:	IMMEDIATE ASSIGNMENT	(GSM 04.08, 9.1.17) to the MS:

Informatin element	Comment	Value

protocol discriminator	RR	011

transaction identifier	not used	0000

message type		0011 1111

page mode	normal	00

channel description

- channel type ...	SDCCH/SACCH 1(4)	0 0100

- time slot number	arbitrary	xxx

- training seq.code	(default)

- hopping	no	0

- FB no.	band no 0	000

- ARFCN	(default)

random reference

- random access info	as in CHAN REQ	111x xxxx

- N51, N32, N26	as applicable	....

timing advance	arbitrary	xx xxxx

mobile allocation	length=0 (due to hopping)	0

GSM900 only - end

DCS1800 only - begin

Message:	IMMEDIATE ASSIGNMENT	(GSM 04.08, 9.1.17) to the MS:

Information element	Comment	Value

protocol discriminator	RR	011

transaction identifier	not used	0000

message type		0011 1111

page mode	normal	00

channel description

- channel type	SDCCH/SACCH 1(4)	0 0100

- time slot number	arbitrary	xxx

- training seq. code	(default)

- hopping	no	0

- ARFCN	(default)

random reference

- random access info	as in CHAN REQ	111x xxxx

- N51, N32, N26	as applicable	....

timing advance	arbitrary	xx xxxx

mobile allocation	length=0 (due to hopping)	0

DCS1800 only - end

Message:	AUTHENTICATION REQUEST	(GSM 04.08, 9.2.2) to the MS:

Information element	Comment	Value

protocol discriminator	MM	0101

transaction identifier	not used	0000

message type		0001 0010

ciphering key seq.number	as applic.from loc.upd	xxx

authent.parameter RAND	as applicable	(128 bits)

Message:	CIPHERING MODE COMMAND	(GSM 04.08, 9.1.9) to the MS:

Information element	Comment	Value

protocol discriminator	RR	0110

transaction identifier	not used	0000

message type		0011 0101

cipher mode setting	start ciphering	1

Message:	CALL PROCEEDING	(GSM 04.08, 9.3.3) to the MS:

Information element	Comment	Value

protocol discriminator	CC	0011

transaction identifier	MS orig.	1xxx

message type		0000 0010

GSM900 only - begin

Message:	ASSIGNMENT COMMAND	(GSM 04.08, 9.1.2) to the MS:

Information element	Comment	Value

protocol discriminator	RR	0110

transaction identifier	not used	0000

message type		0010 1110

channel description

- channel type ...	Bm+ACCHs	0 0001

- time slot number	arbitrary	xxx

- training seq.code	(default)

- hopping	no	0

- FB no	band no 0	000

- ARFCN	(default)

power level	as applicable	x xxxx

channel mode	appropriate for teleservice

selected for the test

GSM900 only - end

DCS1800 only - begin

Message:	ASSIGNMENT COMMAND	(GSM 04.08, 9.1.2) to the MS:

Information element	Comment	Value

protocol discriminator	RR	0110

transaction identifier	not used	0000

message type		0010 1110

channel description

- channel type	Bm + ACCHs	0 0001

- time slot number	arbitrary	xxx

- training seq. code	(default)

- hopping	no	0

- ARFCN	(default)

power level	as applicable	x xxxx

channel mode	appropriate for teleservice

selected for the test

DCS1800 only - end

Message:	ALERTING	(GSM 04.08, 9.3.1) to the MS:

Information element	Comment	Value

protocol discriminator	CC	0011

transaction identifier	MS orig.	1xxx

message type		0000 0001

Message:	CONNECT	(GSM 04.08, 9.3.5) to the MS:

Information element	Comment	Value

protocol discriminator	CC	0011

transaction identifier	MS orig.	1xxx

message type		0000 0111

Message:	DISCONNECT	(GSM 04.08, 9.3.7) to the MS:

Information element	Comment	Value

protocol discriminator	CC	0011

transaction identifier	SS orig.	0xxx

message type		0010 0101

cause

- coding standard	GSM	11

- location	user	0000

- cause value	normal clearing	001 0000

Message:	RELEASE COMPLETE	(GSM 04.08, 9.3.15) to the MS:

Information element	Comment	Value

protocol discriminator	CC	0011

transaction identifier	SS orig.	0xxx

message type		0010 1010

Message:	CHANNEL RELEASE	(GSM 04.08, 9.1.7) to the MS:

Information element	Comment	Value

protocol discriminator	RR	0110

transaction identifier	not used	0000

message type		0000 1101

RR cause	normal release	0000 0000

II.5.3.9.2.3	Requirements

1)	In step c) the MS shall display the dialled number in the way described in a PICS / PIXIT statement.

2)	In step c) the MS shall initiate immediate assignment by sending the CHANNEL REQUEST message.

3)	In step d) the MS shall send a layer 2 SABM frame containing the CM SERVICE REQUEST message to the SS. (The same message is returned in a layer 2 UA frame from the SS). The SABM frame shall be sent after reception of the IMMEDIATE ASSIGNMENT message.

4)	In step e) the MS shall send the message AUTHENTICATION RESPONSE with the correct signed response. Time requirements should be tested in the elementary procedure.

5)	In step f), within 1 s from the CIPHERING MODE COMMAND message, the MS shall activate ciphering and send the message CIPHERING MODE COMPLETE followed by the SETUP message

Note:	In requirement 5), the time of 1 s is not intended to be a performance requirement, it is introduced only because it is necessary to indicate how long the SS will wait until it decides that there is a failure.

6)	In step i) the MS shall send the message ASSIGNMENT COMPLETE on the correct channel after reception of the ASSIGNMENT COMMAND message.

7)	Reserved.

8)	In step k) the MS shall send the message CONNECT ACKNOWLEDGE after recption of CONNECT, and through connect the traffic channel in both directions.

9)	In step l) the MS shall send the message RELEASE after it has received the message DISCONNECT from the SS, initiating call release.

10)	After step m) the MS shall enter the idle, updated state.

Message:	CHANNEL REQUEST	(GSM 04.08, 9.1.8) to the SS:

Information element	Comment	Value

random reference		x xxxx

cause	orig. call	111

Message:	CM SERVICE REQUEST	(GSM 04.08, 9.2.7) to the SS:

Information element	Comment	Value

protocol discriminator	MM	0101

transaction identifier	not used	0000

message type		0x10 0100

CM service type	mob.orig.call est.	0001

ciphering key seq.numberas applic.from loc.upd	xxx

MS classmark 2

- revision level	as applicable	xxx

- encryption algorithm	A5	00

- RF power capability	as applicable	xxx

- short message capability	as applicable	x

- frequency band	band no 0	000

mobile identity

- odd/even no of digits	as applicable	x

- type of identity	TMSI	100

- identity digits	as applicable	....

Message:	AUTHENTICATION RESPONSE	(GSM 04.08, 9.2.3) to the SS:

Information element	Comment	Value

protocol discriminator	MM	0101

transaction identifier	not used	0000

message type		0x01 0100

authentication parameter	SRES, as applicable (32 bits)

Message:	CIPHERING MODE COMPLETE	(GSM 04.08, 9.1.10) to the SS:

Information element	Comment	Value

protocol discriminator	RR	0110

transaction identifier	not used	0000

message type		0011 0010

Message:	SETUP	(GSM 04.08, 9.3.16) to the SS:

Information element	Comment	Value

protocol discriminator	CC	0011

transaction identifier	MS orig.	0xxx

message type		0x00 0101

bearer capability	appropriate to the teleservice

selected for the test

mobile identity	info element may be present

- odd/even no of digits	as applicable	x

- type of identity	TMSI	100

- identity digits	as applicable	....

called party BCD number

- type of number	as applicable	xxx

- numbering plan id.	as applicable	xxxx

- digits	as applicable	....

Message:	ASSIGNMENT COMPLETE	(GSM 04.08, 9.1.3) to the SS:

Message:	CONNECT ACKNOWLEDGE	(GSM 04.08, 9.3.6) to the SS:

Information element	Comment	Value

protocol discriminator	CC	0011

Transaction identifier	MS orig.	0xxx

Message type		0x00 1111

Message	 RELEASE (GSM 04.08, 9.3.14) to the SS:

Information element	Comment	Value

protocol discriminator	CC	0011

transaction identifier	SS orig.	1xxx

message type		0x10 1101

II.5.3.9.3	MS originating call establishment, late assignment

II.5.3.9.3.1	Purpose of the test

To test the MS ability to originate a call following the sequence of elementary signalling procedures given in GSM 04.08 section 7.3.2.1b.

II.5.3.9.3.2	Method of test

a)	The first part of the method of test is identical to the case of early assignment of traffic channel (see previous section) up to and inclusive of the SS sending the CALL PROCEEDING message. Thereafter, the following applies:

b)	The SS sends the message ALERTING within (T310-CC-MS) s after the message CALL PROCEEDING was sent. The ALERTING message contents shall be the same as in section II.5.3.9.2.2.

c)	The SS then sends message ASSIGNMENT COMMAND. The message contents shall be the same as in section II.5.3.9.2.2.

d)	Having received message ASSIGNMENT COMPLETE, the SS sends message CONNECT within (T301-CC-MS) s after message ALERTING was sent. The CONNECT message contents shall be the same as in section II.5.3.9.2.2.

Note:	Clearing of the call is not part of the test procedure.

II.5.3.9.3.3	Requirements

1)	The first part of the requirements is identical to the case of early assignment of traffic channel (see previous section) up to and inclusive of the MS sending the SETUP message. Thereafter, the following applies:

2)	Reserved.

3)	In step c) the MS shall send the message ASSIGNMENT COMPLETE on the correct channel after reception of ASSIGNMENT COMMAND. The ASSIGNMENT COMPLETE message contents shall be the same as in section II.5.3.9.2.3.

4)	In step d) the MS shall send the message CONNECT ACKNOWLEDGE after reception of CONNECT, and through connect the traffic channel in both directions.

II.5.3.9.4	MS terminating call establishment, early assignment, release init. by MS

II.5.3.9.4.1	Purpose of the test

a)	To test the MS ability to respond correctly to a mobile terminating call following the sequence of elementary signalling procedures given in GSM 04.08 section 7.3.2.1b.

b)	To test the MS ability to initiate and fulfil the release of a call, following the sequence of elementary signalling procedures given in GSM 04.08 section 7.3.4b.

II.5.3.9.4.2	Method of test

a)	The SS simulates one cell using the default parameter values for "cell 1" in section II.5.3.3 "test of MS functions in idle mode".

b)	The MS is brought into the MM state "idle, updated", with a TMSI assigned, listening to the BCCH/CCCH of the active cell.

c)	The MS is paged by means of a PAGING REQUEST TYPE 1 message on the correct paging subchannel.

d)	After reception of the CHANNEL REQUEST message the SS returns the IMMEDIATE ASSIGNMENT message before the timer (T3120-RR-MS) has expired, assuming that the randomly drawn value of T3120 is the lowest possible.

e)	The SS accepts the paging response from the MS via the data link establishment procedure.

f)	Immediately after the data link establishment the SS sends an AUTHENTICATION REQUEST message.

g)	Following the correct AUTHENTICATION RESPONSE message from the MS the SS sends a CIPHERING MODE COMMAND message and then starts ciphering.

h)	The SS sends the message SETUP immediately after reception of CIPHERING MODE COMPLETE. The SETUP message shall not include the 'signal' information element.

i)	When the SS has received a CALL CONFIRMED message, it sends message ASSIGNMENT COMMAND.

j)	If and when, after step i) the SS has received the message ASSIGNMENT COMPLETE and then the message ALERT, the MS is made to accept the call (eg by performing off-hook on the MS or by an MS internal action, see PICS / PIXIT statement).

Otherwise, if and when the SS has received at least one CONNECT message after step h) and received an ASSIGNMENT COMPLETE message after step i), it shall proceed with step k).

k)	The SS sends message CONNECT ACKNOWLEDGE within (T313-C-MS) s after reception of message CONNECT.

l)	The MS is initiated to clear the call.

m)	Within (T305-CC-MS) s after reception of the message DISCONNECT the SS sends the RELEASE message.

n)	Within (T3240-MM-MS) s after reception of the message RELEASE COMPLETE the SS releases the RR connection by sending the message CHANNEL RELEASE.

Message:	PAGING REQUEST TYPE 1	(GSM 04.08, 9.1.21) to the MS:

Information element	Comment	Value

protocol discriminator	RR	0110

transaction identifier	not used	0000

message type		0010 0001

page mode	normal	00

mobile identity

- odd/even no of digits	as applicable	x

- type of identity	TMSI	100

- identity digits	as applicable	....

GSM900 only - begin

Message:	IMMEDIATE ASSIGNMENT	(GSM 04.08, 9.1.17) to the MS:

Information element	Comment	Value

protocol discriminator	RR	0110

transaction identifier	not used	0000

message type		0011 1111

page mode	normal	00

channel description

- channel type ...	SDCCH/SACCH 1(4)	0 0100

- time slot number	arbitrary	xxx

- training seq.code	(default)

- hopping	no	0

- FB no	band no 0	000

- ARFCN	(default)

random reference

- random access info	as in CHAN REQ	100x xxxx

- N51, N32, N26	as applicable

timing advance	arbitrary	xx xxxx

mobile allocation	length=0 (due to hopping) 0

GSM900 only - end

DCS1800 only - begin

Message:	IMMEDIATE ASSIGNMENT	(GSM 04.08, 9.1.17) to the MS:

Information element	Comment	Value

protocol discriminator	RR	0110

transaction identifier	not used	0000

message type		0011 1111

page mode	normal	00

channel description

- channel type	SDCCH/SACCH 1(4)	0 0100

- time slot number	arbitrary	xxx

- training seq. code	(default)

- hopping	no	0

- ARFCN	(default)

random reference

- random access info	as in CHAN REQ	100x xxxx

- N51, N32, N26	as applicable

timing advance	arbitrary	xx xxxx

mobile allocation	length=0 (due to hopping)	0

DCS1800 only - end

Message:	AUTHENTICATION REQUEST	(GSM 04.08, 9.2.2) to the MS:

Information element	Comment	Value

protocol discriminator	MM	0101

transaction identifier	not used	0000

message type		0001 0010

ciphering key seq.numberas applic.from loc.upd	xxx

authent.parameter RAND	as applicable	(128 bits)

Message:	CIPHERING MODE COMMAND	(GSM 04.08, 9.1.9) to the MS:

Information element	Comment	Value

protocol discriminator	RR	0110

transaction identifier	not used	0000

message type		0011 0101

cipher mode setting	start ciphering	1

Message:	SETUP	(GSM 04.08, 9.3.16) to the MS:

Information element	Comment	Value

protocol discriminator	CC	0011

transaction identifier	SS orig.	0xxx

message type		0000 0101

bearer capability	appropriate to the teleservice

selected for the test

GSM900 only - begin

Message:	ASSIGNMENT COMMAND	(GSM 04.08, 9.1.2) to the MS:

Information element	Comment	Value

protocol discriminator	RR	0110

transaction identifier	not used	0000

message type		0010 1110

channel description

- channel type ...	Bm+ACCHs	0 0001

- time slot number	arbitrary	xxx

- training seq.code	(default)

- hopping	no	0

- FB no	band no 0	000

- ARFCN	(default)

power level	as applicable	x xxxx

channel mode	appropriate to the teleservice

selected for the test

GSM900 only - end

DCS1800 only - begin

Message:	ASSIGNMENT COMMAND	(GSM 04.08, 9.1.2) to the MS:

Information element	Comment	Value

protocol discriminator	RR	0110

transaction identifier	not used	0000

message type		0010 1110

channel description

- channel type	Bm + ACCHs	0 0001

- time slot number	arbitrary	xxx

- training seq. code	(default)

- hopping	no	0

- ARFCN	(default)

power level	as applicable	x xxxx

channel mode	appropriate to the teleservice

selected for the test

DCS1800 only - end

Message:	CONNECT ACKNOWLEDGE	(GSM 04.08, 9.3.6) to the MS:

Information element	Comment	Value

protocol discriminator	CC	0011

transaction identifier	SS orig.	0xxx

message type		0000 1111

Message:	RELEASE	(GSM 04.08, 9.3.14) to the MS:

Information element	Comment	Value

protocol discriminator	CC	0011

transaction identifier	MS orig.	1xxx

message type		0010 1101

Message:	CHANNEL RELEASE	(GSM 04.08, 9.1.7) to the MS:

Information element	Comment	Value

protocol discriminator	RR	0110

transaction identifier	not used	0000

message type		0000 1101

RR cause	normal release	0000 0000

II.5.3.9.4.3	Requirements

1)	In step c) the MS shall initiate immediate assignment by sending the CHANNEL REQUEST message.

2)	In step d) the MS shall send a layer 2 SABM frame containing the PAGING RESPONSE message to the SS. (The same message is returned in a layer 2 UA frame from the SS). The SABM frame shall be sent after reception of the IMMEDIATE ASSIGNMENT message.

3)	In step f) the MS shall send the message AUTHENTICATION RESPONSE with the correct signed response. Time requirements should be tested in the elementary procedure.

4)	In step g) the MS shall activate ciphering and send the message CIPHERING MODE COMPLETE.

5)	In step h) the MS shall send the message CALL CONFIRMED after reception of SETUP. It may then send a CONNECT message.

6)	After receiving the ASSIGNMENT COMMAND message in step i) the MS shall send the message ASSIGNMENT COMPLETE on the correct channel.

The MS may then send a CONNECT message if one of the following conditions holds:

-	the MS did not send a CONNECT message in 5).

-	the MS has sent a CONNECT message during 5). In this case the N(SD) of both CONNECT messages must be equal.

If the MS has sent at least one CONNECT message in 5) or 6), the following requirements 7) an 9) do not apply.

7)	After step i) the MS shall, in any order,

-	generate an alerting indication (eg by an appropriate tone),

-	send the message ALERTING.

This requirement applies unless the MS has sent at least one CONNECT message during 5) or 6).

8)	Reserved.

9)	In step j), the MS shall send the message CONNECT. This requirement applies unless the MS has sent at least one CONNECT message during 5) or 6).

10)	In step j) the MS shall through connect the traffic channel in both directions.

11)	In step l) the MS shall send the message DISCONNECT at the initiation of call release.

12)	In step m) the MS shall send the message RELEASE COMPLETE after reception of the message RELEASE.

13)	After step n) the MS shall enter the idle, updated state.

Message:	CHANNEL REQUEST	(GSM 04.08, 9.1.8) to the SS:

Information element	Comment	Value

random reference		x xxxx

cause	answer to paging	100

Message:	PAGING RESPONSE	(GSM 04.08, 9.1.24) to the SS:

Information element	Comment	Value

protocol discriminator	RR	0110

transaction identifier	Not used	0000

message type		0010 0111

ciphering key seq.numberas applic.from loc.upd	xxx

MS classmark 2

- revision level	as applicable	xxx

- encryption algorithm	A5	00

- RF power capability	as applicable	xxx

- short message capability	as applicable	x

- frequency band	band no 0	000

mobile identity

- odd/even no of digits	as applicable	x

- type of identity	TMSI	100

- identity digits	as applicable	....

Message:	AUTHENTICATION RESPONSE	(GSM 04.08, 9.2.3) to the SS:

Information element	Comment	Value

protocol discriminator	MM	0101

transaction identifier	not used	0000

message type		0x01 0100

authentication parameter	SRES, as applicable (32 bits)

Message:	CIPHERING MODE COMPLETE	(GSM 04.08, 9.1.10) to the SS:

Information element	Comment	Value

protocol discriminator	RR	0110

transaction identifier	not used	0000

message type		0011 0010

Message:	CALL CONFIRMED	(GSM 04.08, 9.3.2) to the SS:

Information element	Comment	Value

protocol discriminator	CC	0011

transaction identifier	SS orig.	1xxx

message type		0x00 1000

Message:	ASSIGNMENT COMPLETE	(GSM 04.08, 9.1.3) to the SS:

Message:	ALERTING	(GSM 04.08, 9.3.1) to the SS:

Information element	Comment	Value

protocol discriminator	CC	0011

transaction identifier	SS orig.	1xxx

message type		0x00 0001

Message:	CONNECT	(GSM 04.08, 9.3.5) to the SS:

Information element	Comment	Value

protocol discriminator	CC	0011

transaction identifier	SS orig.	1xxx

message type		0x00 0111

Message:	DISCONNECT	(GSM 04.08, 9.3.7) to the SS:

Information element	Comment	Value

protocol discriminator	CC	0011

transaction identifier	MS orig.	0xxx

message type		0x10 0101

cause

- coding standard	GSM	11

- cause value	normal clearing	001 0000

Message:	RELEASE COMPLETE	(GSM 04.08, 9.3.15) to the SS:

Information element	Comment	Value

protocol discriminator	CC	0011

transaction identifier	MS orig.	0xxx

message type		0x10 1010

II.5.3.9.5	MS terminating call establishment, late assignment

II.5.3.9.5.1	Purpose of the test

To test the MS ability to respond correctly to a mobile terminated call following the sequence of elementary signalling procedures given in GSM 04.08 section 7.3.3.1b.

II.5.3.9.5.2	Method of test

a)	The first part of the method of test is identical to the case of early assignment of traffic channel (see previous section) up to and inclusive of the SS sending the SETUP message, except that the SETUP message shall include the 'signal' information element. Thereafter, the following applies:

b)	If and when the SS has received the message ALERTING, the MS is made to accept the call, see PICS / PIXIT statement.

c)	After the SS has received the message CONNECT the SS sends message ASSIGNMENT COMMAND.

d)	Having received message ASSIGNMENT COMPLETE, the SS sends message CONNECT ACKNOWLEDGE within (T313-CC-MS) s after reception of message CONNECT.

Note:	Clearing of the call is not part of the test procedure.

II.5.3.9.5.3	Requirements

1)	The first part of the requirements is identical to the case of early assignment of traffic channel (see previous section) up to and inclusive of the MS sending the CIPHER MODE COMPLETE message. Thereafter, the following applies:

2)	After receiving the SETUP message in step a), the MS shall proceed in one of the following ways:

a)	The MS sends a CALL CONFIRMED message followed by an ALERTING message. It shall generate an alerting indication (eg. by an appropriate tone).

b)	The MS sends a CALL CONFIRMED message followed by a CONNECT message.

3)	Reserved.

4)	In step b) the MS shall send the message CONNECT within 0.5 s from the acceptance of the call (off hook) at the MS.

This requirement applies only if the MS did not send a CONNECT message in 2).

Note:	In requirement 4), the time of 0.5 s is not intended to be a performance requirement, it is introduced only because it is necessary to indicate how long the SS will wait until it decides that there is a failure.

5)	In step c) the MS shall send the message ASSIGNMENT COMPLETE on the correct channel after reception of the ASSIGNMENT COMMAND message.

6)	In step d) the MS shall through connect the traffic channel in both directions.