II.8	Testing of the ME/SIM (Subscriber Identification Module) interface

The SIM-ME interface is specified in GSM 11.11.

The DCS 1800 system shall use SIMs which support a DCS 1800 application with an associated DCS 1800 directory.

Testing of the SIM itself is not part of this specification.

General

The following tests concentrate upon procedures which have an impact upon the performance of the ME on the network; i.e. following the usual criteria that type approval protects the network and other users, but does not validate the performance of the equipment under test.

The following sequence of tests for the SIM/ME interface confirms

GSM900:

a)	the correct interpretation of data read by the ME from the SIM;

b)	the correct writing of data by the ME to the SIM;

c)	the initiation of appropriate procedures by the ME involving transfer of information between ME and SIM.

DCS1800:

a)	the access to the correct application directory and data

b)	the correct interpretation of data read by the ME from the SIM;

c)	the correct writing of data by the ME to the SIM;

d)	the initiation of appropriate procedures by the ME involving transfer of information between ME and SIM.

The tests must be considered as a continuous sequence requiring that information written to or deleted from the SIM or ME memories is carried forward into the succeeding tests.

Note:	A SIM simulator will be required as part of the SS.

A SIM programmer/reader is required as part of the SS.

The tests of high level procedures (test sequences 1 to 8) have been designed using the substitution method of testing. It is probable that instead of a number of SIMs programmed to values defined in tables II.8-1 and II.8-2 the SIM simulator will be used. However, as these tests require use also of the Um interface, there is concern that RF may disturb the SIM simulator. For the present, therefore, the tests are still described for the substitution method. If the SIM simulator is found reliable for all tests then the requirement for a programmer/reader as part of the SS can be dropped.

Test sequences 1 to 8

To perform these test sequences SIMs will be required programmed with specific data field values

GSM900:	intentionally left blank

DCS1800:	in the DCS1800 directory

as defined in tables II.8-1 and II.8-2 following.

GSM900:	intentionally left blank

DCS1800:	In some tests the SIM will be programmed with both a GSM 900 and a DCS 1800 in the DCS1800 directory. In that case the GSM 900 directory will be preprogrammed as in test sequence 1.

These tables also give the expected values of the data fields at the completion of each test sequence.

(For a full explanation of each data field see GSM 11.11.)

Table II.8-1: SIM data field values for the SIM/ME interface test sequences

GSM900 only - begin

At start of test sequence	1	2	3	4		5		6

At end of test sequence	1	2	3		4		5		6

Data field	Value

IMSI	A	A	A	B	B	B	B	A	A

Location information

(LAI & TMSI)	A	B	C	A	D	F	E	B	B

Kc and cipher key	A	A	A	A	B	B	B	A	A

sequence No. (n)

Access control	A	A	A	A	A	A	A	A	A

Forbidden PLMNs	A	A	A	A	A	A	B	A	A

SIM service table	A	A	A	A	A	A	A	A	A

PIN	x	x	x	x	x	x	x	x	x

Unblocking key	x	x	x	x	x	x	x	A	A

PLMN selector	A	A	A	A	A	A	A	A	B

8	13

8	13

see	x

test 8

x	x

A	x

see	x

test 8

A	x

A	A

x	A

x	A

A	x

Note:	'x' indicates that the value is of no significance to the test.

GSM900 only - end

DCS1800 only - begin

At start of test sequence	1	2	3	4	5		6		8

At end of test sequence	1	2	3		4	5		6	8

Data field	Value

IMSI	A	A	B	B	B	B	A	A	See

test 8

Location information

(LAI & TMSI)	A	B	C	A	D	E	B	B	B

Kc and cipher key	A	A	A	A	B	B	A	A	A

sequence No. (n)

Access control	A	A	A	A	A	A	A	A	see

test 8

Forbidden PLMNs	A	A	A	A	A	B	A	A	A

SIM service table	A	A	A	A	A	A	A	A	A

PIN	x	x	x	x	x	x	x	x	x

Unblocking key	x	x	x	x	x	x	A	A	x

PLMN selector	A	A	A	A	A	A	A	B	A

Directory	G/D	D	D	D	D	D	D	D	D

13	14	15

13	14	15

x	A	A

A	B	A

x	A	A

x	A	A

x	A	A

A	A	A

A	x	x

A	x	x

x	A	A

D	D	D

Note:	'x' indicates that the value is of no significance to the test.

G indicates GSM directory values

D indicates DCS directory values

DCS1800 only - end

Table II.8-2 :  Definition of values of table II.8-1

Data-Field 6F-07  :  IMSI

Value A		Value B

246813579		246811111111111

byte 1	05 H		08 H		length of IMSI

byte 2	29 H		29 H		first digit, parity coding

byte 3	64 H		64 H		digits

byte 4	18 H		18 H

byte 5	53 H		11 H

byte 6	97 H		"

byte 7	FF H		"

byte 8	FF H		"

byte 9	FF H		"

Table II.8-2 cont.

Data-Field 6F-7E  :  Location information

Value A	Value B	Value C	Value D	Value E	Value F

LAI	- MCC	246	246	246	246	234	234

MNC	81	81	81	81	06	01

TMSI		Null	1234	12345678	23456789	34567890	23456789

TMSI Time-		Null	Null	Null	Null	Null	Null

byte 1	FF H	00 H	21 H	32 H	43 H	32H

byte 2	FF H	00 H	43 H	54 H	65 H	54H

byte 3	FF H	21 H	65 H	76 H	87 H	76H

byte 4	FF H	43 H	87 H	98 H	09 H	98H

byte 5	42 H	42 H	42 H	42 H	32 H	32H

byte 6	F6 H	F6 H	F6 H	F6 H	F4 H	F4H

byte 7	18 H	18 H	18 H	18 H	60 H	10H

byte 8	xx H	xx H	xx H	xx H	xx H	00H

byte 9	xx H	xx H	xx H	xx H	xx H	00H

byte 10	FF H	FF H	FF H	xx H	xx H	FFH

byte 11	00 H	00 H	00 H	xx H	xx H	00H

Data-Field 6F-20  : Cipher key (Kc) and cipher key sequence no. (n)

Value A			Value B

byte 1	xx }			bytes 1-8 as

byte 2	xx }			determined by

byte 3	xx }			authentication

byte 4	xx }			algorithm

byte 5	xx }	Kc

byte 6	xx }

byte 7	xx }

byte 8	xx }

byte 9	01 H n=001			02 H n=010

Data-Field 6F-78  : Access control

One access class from 0-9 to be allocated compatible with those allowed by the SS.

Table II.8-2 cont.

Data-Field 6F-7B  : Forbidden PLMNs

Value A		Value B

byte 1	32 H	MCC = 234	32 H	MCC = 234	}

byte 2	F4 H		F4 H		}	First forbidden

byte 3	20 H	MNC = 02	30 H	MNC = 03	}	PLMN (oldest)

byte 4	32 H	MCC = 234	32 H	MCC = 234	}

byte 5	F4 H		F4 H		}	2nd forbidden

byte 6	30 H	MNC = 03	40 H	MNC = 04	}	PLMN

byte 7	32 H	MCC = 234	32 H	MCC = 234	}

byte 8	F4 H		F4 H		}	3rd forbidden

byte 9	40 H	MNC = 04	50 H	MNC = 05	}	PLMN

byte 10	32 H	MCC = 234	32 H	MCC = 234	}

byte 11	F4 H		F4 H		}	4th forbidden

byte 12	50 H	MNC = 05	60 H	MNC = 01	}	PLMN (newest)

Data-Field 6F-38  :  SIM service table

Value A :

byte 1	xxxxxx11 - PIN disable function allocated and activated

byte 2	xxxxxx11 - Charging counter allocated and activated

Value B :

byte 1	xxxxxx01 - PIN disable function allocated but not activated

byte 2	xxxxxx11 - Charging counter allocated and activated

GSM900 only - begin

Data-Field 6F-30 : PLMN selector

Value A		Value B

byte 1	32H	MCC = 234	32H	MCC = 234

byte 2	F4H		F4H

byte 3	10H	MNC = 01	10H	MNC = 01

byte 4	32H	MCC = 234	65H	MCC = 567

byte 5	F4H		F7H

byte 6	20H	MNC = 02	10H	MNC = 01

byte 7	32H	MCC = 234	32H	MCC = 234

byte 8	F4H		F4H

byte 9	30H	MNC = 03	30H	MNC = 03

byte 10	32H	MCC = 234	32H	MCC = 234

byte 11	F4H		F4H

byte 12	40H	MNC = 04	40H	MNC = 04

byte 13	32H	MCC = 234	32H	MCC = 234

byte 14	F4H		F4H

byte 15	50H	MNC = 05	50H	MNC = 05

byte 16	32H	MCC = 234	32H	MCC = 234

byte 17	F4H		F4H

byte 18	60H	MNC = 06	60H	MNC = 06

byte 19	42H	MCC = 246	42H	MCC = 246

byte 20	F6H		F6H

byte 21	18H	MNC = 81	18H	MNC = 81

byte 22	42H	MCC = 246	42H	MCC = 246

byte 23	F6H		F6H

byte 24	28H	MNC = 82	28H	MNC = 82

Table II.8-2 cont.

PIN

Value A

2468

GSM900 only - end

DCS1800 only - begin

Data-Field 6F-30 : PLMN selector

Value A		Value B

byte 1	32H	MCC = 234	32H	MCC = 234

byte 2	F4H		F4H

byte 3	10H	MNC = 01	10H	MNC = 01

byte 4	32H	MCC = 234	32H	MCC = 234

byte 5	F4H	"	F4H

byte 6	60H	"MNC = 06	60H	MNC = 06

byte 7	FFH	"Null	65H	MCC = 567

byte 8	FFH	"	F7H

byte 9	FFH	"	10H	MNC = 01

byte 10	32H	MCC = 234	32H	MCC = 234

byte 11	F4H		F4H

byte 12	40H	MNC = 04	40H	MNC = 04

byte 13	32H	MCC = 234	32H	MCC = 234

byte 14	F4H		F4H

byte 15	50H	MNC = 05	50H	MNC = 05

byte 16	32H	MCC = 234	32H	MCC = 234

byte 17	F4H		F4H

byte 18	60H	MNC = 06	60H	MNC = 06

byte 19	42H	MCC = 246	42H	MCC = 246

byte 20	F6H		F6H

byte 21	18H	MNC = 81	18H	MNC = 81

byte 22	42H	MCC = 246	42H	MCC = 246

byte 23	F6H		F6H

byte 24	28H	MNC = 82	28H	MNC = 82

PIN

Value A

2468

DCS1800 only - end

Unblocking key value : 13243546

Data-Field values of other data fields are of no significance for the tests which follow.

II.8.1	Test sequence 1

Purpose:

1)	To confirm that the ME reads IMSI from SIM as part of the SIM/ME initialization procedure.

2)	To confirm that the ME can handle IMSIs of less than the maximum allowed value.

Procedure:

a)	The SS is set up to transmit on the BCCH with the following network parameters:

Attach/detach :	disabled

LAI :	MCC = 246

MNC = 81

Access control:	unrestricted.

b)	The SIM defined in tables II.8-1 and II.8-2 above is installed into the ME.

c)	The MS is powered-on.

d)	The SS sends page request to the MS using IMSI (value A).

Requirements:

1)	The MS shall send channel request to the SS.

Procedure:

e)	The SS shall send imm assign to the MS.

Requirements:

2)	The MS shall send pag res to the SS containing IMSI (value A).

Procedure:

f)	The SS shall send release to the MS.

II.8.2	Test sequence 2

Purpose:

1)	To confirm that ME reads TMSI from SIM as part of SIM/ME initialization procedure.

2)	To confirm that the ME can handle TMSIs of less than maximum length.

Procedure:

a)	The SS sends page request to the MS using TMSI (value B).

Requirements:

b)	The MS shall send channel request to the SS.

Procedure:

c)	The SS shall send imm assign to the MS.

Requirements:

d)	The MS shall send pag res to the SS containing TMSI (value B).

Procedure:

e)	The SS shall send release to the MS.

II.8.3	Test sequence 3

Purpose:

1)	To confirm that when a SIM with a different TMSI value is introduced into the ME, causing a new SIM/ME initialization procedure to take place, that the values previously used by the ME have been deleted.

2)	To confirm that the ME can handle TMSI values of maximum length.

Procedure:

a)	The SIM defined in tables II.8-1 and II.8-2 above is installed into the ME, and the MS powered-on.

b)	The SS sends page request to the MS using TMSI (value B).

Requirements:

c)	The MS shall not respond to the page request.

Procedure:

d)	The SS sends page request to the MS using TMSI (value C).

Requirements:

e)	The MS shall send channel request to the SS.

Procedure:

f)	The SS shall send imm assign to the MS.

Requirements:

g)	The MS shall send pag res to the SS containing TMSI (value C).

h)	The SS shall send release to the MS.

II.8.4	Test sequence 4

Purpose:

1)	To confirm that when a SIM with a different IMSI value is introduced into the ME, causing a new SIM/ME initialization procedure to take place, that the value previously used by the ME has been deleted.

2)	To confirm that the ME can handle a IMSI value of maximum length.

3)	To confirm that after ciphering mode setting the ME updates the SIM with the new value of cipher key sequence number (n), at call termination.

4)	To confirm that the TMSI data field in the SIM is correctly updated by the ME at call termination.

Procedure:

a)	The SIM defined in tables II.8-1 and II.8-2 above is installed into the ME, and the MS powered-on.

b)	The SS sends page request to the MS using IMSI (value A).

Requirements:

1)	The MS shall not respond to the page request.

Procedure:

c)	The SS sends page request to the MS using IMSI (value B).

Requirements:

2)	The MS shall send channel request to the SS.

Procedure:

d)	The SS shall send immi assign to the MS.

Requirements:

3)	The MS shall send pag res to the SS containing IMSI (value B).

4)	The SS shall send authentication request to the MS containing cipher key sequence number set to binary 010.

5)	The MS shall send authentication response to the SS.

6)	The cipher key sequence number in the SIM shall be the value '010' when read in requirement 9) below.

Procedure:

e)	The SIM shall send TMSI reallocation to the MS containing the TMSI (value D).

Requirement:

7)	The MS shall send TMSI reallocation complete to the SS.

8)	The TMSI data field in the SIM shall contain the value D when read in requirement 9) below.

Procedure:

f)	Within 5 seconds the SS shall send release to the MS.

g)	The values in the SIM are examined after call termination. This can be done by examining the data in the SIM Simulator or by removing the SIM in the following way:

i)	If the SIm can simply be removed it is removed without powering doen the MS.

ii)	If i) cannot be performed the power supply is removed from the MS and the SIM then removed.

Requirements:

9)	The SIM shall contain the values listed in tables II.8-1 and II.8-2 above.

II.8.5	Test sequence 5

Purpose:

1)	To confirm that the ME reads the value of the forbidden PLMN field as part of the SIM/ME initialization procedure and does not attempt to access these PLMNs in automatic PLMN selection mode.

2)	To confirm that the forbidden PLMN data field in the SIM is correctly updated by the ME at soft power-down.

3)	To confirm that the location information data field in the SIM is correctly updated by the ME at soft power-down.

Procedure:

a)	The SIM defined in tables II.8-1 and II.8-2 above is installed into the ME, and the MS powered-on. The MS is made to operate in automatic PLMN selection mode.

b)	The SS provides a BCCH with default values and:

LAI :	MCC = 234

MNC = 02.

Requirement:

1)	The MS shall not attempt a location update.

Procedure:

c)	The SS stops all RF output on the BCCH for a long enough period of time to cause a reselection in the MS. The BCCH is changed to contain:

LAI :	MCC = 234

MNC = 03.

The SS then resumes RF output on the BCCH.

Requirement:

2)	The MS shall not attempt a location update.

Procedure:

d)	The SS stops all RF output on the BCCH for a long enough period of time to cause a reselection in the MS. The BCCH is changed to contain:

LAI :	MCC = 234

MNC = 04.

The SS then resumes RF output on the BCCH.

Requirement:

3)	The MS shall not attempt a location update.

Procedure:

e)	The SS stops all RF output on the BCCH for a long enough period of time to cause a reselection in the MS. The BCCH is changed to contain:

LAI :	MCC = 234

MNC = 05.

The SS then resumes RF output on the BCCH.

Requirement:

4)	The MS shall not attempt a location update.

Procedure:

f)	The SS stops all RF output on the BCCH for a long enough period of time to cause a reselection in the MS. The BCCH is changed to contain:

LAI :	MCC = 234

MNC = 01.

The SS then resumes RF output on the BCCH.

Requirement:

5)	The MS shall send channel request to the SS.

Procedure:

g)	The SS shall send imm assign to the MS.

Requirement:

6)	The MS shall send location update request to the SS.

Procedure:

h)	The SS shall send loc upd rej to the MS with the reason "PLMN not allowed", followed by channel release.

Requirement:

7)	The forbidden PLMN data field in the SIM shall contain the value B when read in requirement 12) below.

Procedure:

i)	The SS stops all RF output on the BCCH for a long enough period of time to cause a reselection in the MS. The BCCH is changed to contain:

LAI :	MCC = 234

MNC = 06.

Requirement:

8)	The MS shall send channel request to the SS.

Procedure:

j)	The SS shall send imm assign to the MS.

Requirement:

9)	The MS shall send loc upd req to the SS.

Procedure:

k)	The SS shall send loc upd acc with

LAI :	MCC = 234

MNC = 06

TMSI:	34567890

to the MS.

Requirements:

10)	The MS shall respond with TMSI real com.

11)	The location area identification data field in the SIM shall contain the value E when read in requirement 12) below.

Procedure:

l)	The SS shall send release to the MS.

m)	The MS is soft powered-down.

Requirements:

12)	The SIM shall be removed from the MS, and installed in a SIM reader. It shall contain the values listed in tables II.8-1 and II.8-2 above.

II.8.6	Test sequence 6

Purpose:

To check PLMN selector updating procedure.

Procedure:

a)	The SIM defined in tables II.8-1 and II.8-2 above is installed into the ME.

b)	The MS is powered-on.

c)	The

GSM900:	second

DCS1800:	third

PLMN in the PLMN selector list shall be entered as

MCC = 567

MNC = 01. (MMI dependent)

d)	The SIM shall be removed from the MS, and installed in a SIM reader.

Requirement:

The SIM shall contain the PLMN MCC = 567, MNC = 01 in the second position of the PLMN selector list.

II.8.7	Test sequence 7 - Electrical tests

Purpose of the test

Testing of electrical characteristics of the SIM/ME interface.

Whilst non-conformance in this area would be unlikely to cause difficulties to other users or the network (type approval criteria) significant deviations from the specifications (GSM 11.11 and ISO 7816) may damage the SIM. If an attempt is then made to use the SIM in a different ME, then its failure may reflect badly on both that ME and the network.

This sequence lists the electrical tests to be run and when.

The first part describes the tests to be run during each phase of a transaction with the card.

The second part deals with tests run on each contact, and with static and dynamic tests:

1 -	All power on/off phases;

2 -	All voltages, currents and signals characteristics on each contact.

However, due to the likely difficulty of accessing the terminals of the SIM/ME interface for the purposes of measurements, the ME manufacturer shall provide a test interface in accordance with section III.1.5 for the purpose of conformance testing.

These tests are mandatory for all SIM or plug-in SIM/ME interfaces.

General measurement conventions

The measurement conventions are specified in ISO 7816-3 section 4.2.1.

II.8.7.1	Test of Power Transition Phases

II.8.7.1.1	Phase preceding ME power on

Purpose

Measurement of residual voltages.

Procedure

The power of the ME is off.

Requirement

The voltages across the contacts (C1, C2, C3, C6, C7) shall correspond to inactive levels, i.e. between 0 and 0.4V.

II.8.7.1.2	Phase during SIM power on

Purpose

Verification of contact activation sequence.

Procedure

The ME is powered on.

The SIM/ME interface is monitored until it is fully activated (see ISO 7816-3 section 5.1).

Requirements

Contacts shall be activated in the following order:

1 -	RST in state L,

2 -	Vcc powered,

3 -	I/O in (ME) reception mode,

4 -	Vpp powered (see GSM 11.11 section 6.1.2.)

5 -	Clock signal provided with a suitable and stable clock.

When Vpp is connected to Vcc, as allowed by GSM 11.11 (clause 6.1.2), then Vpp is activated together with Vcc, at the time of Vcc (step 2 in the sequence above).

II.8.7.1.3	Phase during power off

Purpose

Verification of contacts deactivation sequence.

Procedure

The ME is powered off.

The SIM/ME interface is monitored until it is fully deactivated (See ISO 7816-3 section 5.1)

Requirements

Contacts shall be deactivated in the following order:

1 -	RST at low status,

2 -	Clock stopped at low status,

3 -	Vpp cut off (if Vpp was provided, See GSM 11.11 section 6.1.2.),

4 -	I/O at status A,

5 -	Vcc cut off.

When Vpp is connected to Vcc, as allowed by GSM 11.11 (clause 6.1.2), then Vpp is deactivated together with Vcc, at the time of Vcc (step 5 in the sequence above).

II.8.7.2	Electrical tests on each ME contact

The electrical values, as given in the following table, must apply to all contacts, other than that under test, under both normal and extreme test conditions as defined in Annex 1, TC2.1 and TC2.2.

Contacts	Low level	High level	Max capacitive load

from test equipment

C1		V = 5V +/-10%

(Vcc)		I = 10 mA

C2	0V <= V <= 0.6V	4V <= V <= Vcc	30 pF

(RST)	I = +200mA	I = +20mA

C3	0V <= V <= 0.6V	4V <= V <= Vcc	30 pF

(CLK)	I = +200mA	I = +20mA

C5	0V	0V

(GND)

C6		5V +/-10%

(Vpp)

I	0V <= V <= 0.8V	0.7*Vcc <= V <= Vcc

I = -1mA	I = +/-20mA

C7			30 pF

(I/O) O	0V <= V <= 0.4V	3.8V <= V <= Vcc

I = 1mA	I = -20mA

Note 1:	Measurements of contacts voltage levels can be done at any time since the beginning of activation of the SIM and the end of deactivation of the SIM (ISO 7816-3 section 5.1).

Note 2:	The reference point of all measurements is the contact C5 (Ground).

Note 3:	Currents flowing into the SIM are considered positive.

II.8.7.2.1	Electrical tests on contact C1

C1 = Card power supply (Vcc)

II.8.7.2.1.1

Purpose

To verify the nominal voltage

Procedure

Vcc is measured with the remaining contacts in nominal test conditions.

Requirement

The voltage shall be 5V +/-10% for Icc up to 10 mA.

II.8.7.2.1.2

Purpose

To verify that the voltage does not collapse when subjected to current spikes.

Procedure

Vcc is monitored and the following current spikes are applied:

1)	a single spike:

Max current 200 mA

Duration (including rise and fall times as specified in 11.40) 200 ns

2)	a single spike:

Max current 100 mA

Duration (including rise and fall times as defined in 11.40) 400 ns.

3)	continuous spikes:

Max current 20 mA

Min current  0 mA

Duration 100 ns

Pause 100 ns

4)	continuous spikes:

Max current 20 mA

Min current  0 mA

Duration 400 ns

Pause 400 ns

5)	continuous spikes:

Max current 20 mA

Min current  5 mA

Duration 150 ns

Pause 300 ns

6)	Pseudo random spikes:

Max current 20 mA

Min current  5 mA

Duration 200 ns

200 spikes/second

7)	Pseudo random spikes:

Max current 20 mA

Min current  5 mA

Duration 200 ns

1000 spikes/second

Requirement

The voltage shall not fall below 4.5V.

II.8.7.2.2	Electrical tests on contact C2

C2 = Reset (RST)

Purpose

To verify the voltage on contact C2

Procedure

The voltage on C2 is measured with the remaining contacts in nominal test conditions.

II.8.7.2.2.1	Tests on contact C2 = low level RST

Requirement

The voltage shall be between -0.3V and +0.6V for a current of 200 mA.

II.8.7.2.2.2	Tests on contact C2 = high level RST

Requirement

Make sure that the voltage is between 4V and Vcc +0.3V for a current of 20 mA.

II.8.7.2.3	Electrical tests on contact C3

C3 = Clock (CLK)

Purpose

To verify the voltage, the rise/fall time of the signal and the clock cycle ratio on contact C3

Procedure

The voltage on C3 is measured with the remaining contacts in nominal test conditions.

II.8.7.2.3.1	Tests on contact C3 = CLK at low level

Requirement

The voltage shall be between -0.3V and 0.6V for a current of 200 mA.

II.8.7.2.3.2	Tests on contact C3 = CLK for switch from low level to high level. Verification of rise time.

Requirement

The rise time shall not exceed 9% of the total time period and shall not exceed 0.5 ms.

II.8.7.2.3.3	Tests on contact C3 = CLK at high level

Requirement

The voltage shall be between 2.4 V and Vcc +0.3V for a current of 200 mA.

II.8.7.2.3.4	Tests on contact C3 = CLK for switch from high level to low level. Verification of fall time.

Requirement

The fall time shall not exceed 9% of the period and it shall not exceed 0.5 ms.

II.8.7.2.3.5	Clock cycle ratio test

Requirement

The cycle ratio of the clock signal shall be between 40% and 60% of the period, in steady state.

II.8.7.2.4	Electrical tests on contact C6

Purpose

To verify that the contact is in accordance with GSM 11.11.

Procedure

The voltage on C6 is measured with the remaining contacts in nominal test conditions.

Requirement

-	In the case of IC card SIM, the ME shall provide contact C6 and an idle state equal to the condition for Vcc. Contact C6 may be connected to Vcc in the ME.

-	In the case of the plug-in SIM, contact C6 need not be provided by the ME. If it is present in the ME, the programming voltage may not be provided. C6 shall not be connected to ground in the ME.

II.8.7.2.5	Electrical tests on contact C7

C7 = Input - output (I/O)

Purpose

To verify the voltage levels on contact C7

Procedure

The voltage on C7 is measured with the remaining contacts in nominal test conditions.

Requirements

-	On the ME input:

The voltage shall be between 0 and 0.4V (low level) for I = 1 mA and between 3.8V and Vcc+0.3V  (high level) for I = -20 mA.

-	On the ME output:

The voltage shall be between 0 and 0.8V (low level) for I = 1 mA and between 0.7*Vcc and Vcc+0.3V (high level) for I = +/-20 mA.

II.8.8	Test sequence 8:  Access control

Note:	The figure at the end of test sequence 8 shows the various scenarii for access control, as defined in GSM 04.08 and 02.11.

Purpose

1)	To confirm that the ME reads the access control value as part of the SIM/ME initialization procedure -

2)	To verify that the MS does not attempt a network access if its access control class is invalid in the following cases:

(a)	No SIM in ME - Emergency calls not allowed by network.

(b)	No SIM in ME - Emergency calls allowed by network.

(c)	MS with access class 0 to 9 - No calls allowed by network.

(d)	MS with access class 0 to 9 - Emergency calls only allowed by network.

(e)	MS with access class 0 to 9 - All calls allowed by network.

(f)	MS with access class 11 and 15 not in HPLMN,

MS with access class 12,13 and 14 not in HPLMN country.

(g)	MS with access class 11 and 15	)

in HPLMN,	) - Emergency calls only

MS with access class 12,13 and 14	)     allowed by the network.

in HPLMN country.	)

(h)	MS with Access Class 11 and 15	)

in HPLMN,	) - All calls allowed

MS with Access Class 12,13 and 14	)     by the network.

in HPLMN country.	)

Method of Test:

a)	A SIM is installed in the ME containing IMSI and access control values as given in the following table II.8-3.

b)	The SS is set up to transmit on the BCCH with a LAI and RACH control parameters as given in the following table II.8-3.

c)	The MS is powered-on.

Note:	Depending on the initial value of the data field 6F-7E, the MS may perform a location update. This should be accepted by the SS.

d)	Using the MMI or EMMI a normal call set-up is attempted.

e)	Using the MMI or EMMI an emergency call set-up is attempted.

f)	The test is repeated for each set of values in table II.8-3.

Requirement:

The MS will access the network, or make no access attempt, to set-up the normal call and the emergency call as stated in the following table II.8-3. For the tests 8c, 8d and 8e it is only necessary that one of the access classes is tested.

Table  II.8-3

SIM	Network	Test result

IMSI	Access	RACH	BCCH/LAI	Normal	Emergency

class	octet 3	MCC	calls	calls

octet 4	MNC

See end of table for coding details of these parameters.

TEST 8a

No SIM in ME		00000100	234	No	No

00000000	01

TEST 8b

No SIM in ME		00000000	234	No	Yes

00000000	01

TEST 8c

246813579	0	00000100	246	No	No

00000001	81

"	1	00000100	"	No	No

00000010

"	2	00000100	"	No	No

00000100

"	3	00000100	"	No	No

00001000

"	4	00000100	"	No	No

00010000

"	5	00000100	"	No	No

00100000

"	6	00000100	"	No	No

01000000

"	7	00000100	"	No	No

10000000

"	8	00000101	"	No	No

00000000

"	9	00000110	"	No	No

00000000

Table II.8-3 cont.

SIM	Network	Test result

IMSI	Access	RACH	BCCH/LAI	Normal	Emergency

class	octet 3	MNC	calls	calls

octet 4	MNC

TEST 8d

246813579	0	00000000	246	No	Yes

00000001	81

"	1	00000000	"	No	Yes

00000010

"	2	00000000	"	No	Yes

00000100

"	3	00000000	"	No	Yes

00001000

"	4	00000000	"	No	Yes

00010000

"	5	00000000	"	No	Yes

00100000

"	6	00000000	"	No	Yes

01000000

"	7	00000000	"	No	Yes

10000000

"	8	00000001	"	No	Yes

00000000

"	9	00000010	"	No	Yes

00000000

TEST 8e

246813579	0	11111011	246	Yes	Yes

11111110	81

"	1	11111011	"	Yes	Yes

11111101

"	2	11111011	"	Yes	Yes

11111011

"	3	11111011	"	Yes	Yes

11110111

"	4	11111011	"	Yes	Yes

11101111

"	5	11111011	"	Yes	Yes

11011111

"	6	11111011	"	Yes	Yes

10111111

"	7	11111011	"	Yes	Yes

01111111

"	8	11111010	"	Yes	Yes

11111111

"	9	11111001	"	Yes	Yes

11111111

Note:	See end of table for coding details of the parameters in the table.

Table II.8-3 cont.

SIM	Network	Test result

IMSI	Access	RACH	BCCH/LAI	Normal	Emergency

class	octet 3	MNC	calls	calls

octet 4	MNC

TEST 8f

2468135x9	11 and x	00000111	246	No	No

11111111	82

"	"	00000011	"	No	Yes

11111111

"	"	00000000		Yes	Yes

00000000

Set 'x' to a random

value between 0 & 9

2468135x9	12 and x	00000111	234	No	No

11111111	01

"	"	00000011	"	No	Yes

11111111

"	"	00000000		Yes	Yes

00000000

Set 'x' to a random

value between 0 & 9

2468135x9	13 and x	00000111	234	No	No

11111111	01

"	"	00000011	"	No	Yes

11111111

"	"	00000000		Yes	Yes

00000000

Set 'x' to a random

value between 0 & 9

2468135x9	14 and x	00000111	234	No	No

11111111	01

"	"	00000011	"	No	Yes

11111111

"	"	00000000		Yes	Yes

00000000

Set 'x' to a random

value between 0 & 9

2468135x9	15 and x	00000111	246	No	No

11111111	82

"	"	00000011	"	No	Yes

11111111

"	"	00000000		Yes	Yes

00000000

Set 'x' to a random

value between 0 & 9

TABLE II.8-3 cont.

SIM	Network	Test Result

IMSI	Access	RACH	BCCH/LAI	Normal	Emergency

Class	octet 3	MNC	Calls	Calls

octet 4	MNC

TEST 8g

246813579	11 and x	00001100	246	No	Yes

11111111	81

"	"	00001000	"	No	Yes

11111111

246813579	12 and x	00010100	246	No	Yes

11111111	82

"	"	00010000	"	No	Yes

11111111

246813579	13 and x	00100100	246	No	Yes

11111111	82

"	"	00100000	"	No	Yes

11111111

246813579	14 and x	01000100	246	No	Yes

11111111	82

"	"	01000000	"	No	Yes

11111111

246813579	15 and x	10000100	246	No	Yes

11111111	81

"	"	10000000	"	No	Yes

11111111

TEST 8h

246813579	11 and x	11110011	246	Yes	Yes

11111111	81

246813579	12 and x	11101011	246	Yes	Yes

11111111	82

246813579	13 and x	11011011	246	Yes	Yes

11111111	82

246813579	14 and x	10111011	246	Yes	Yes

11111111	82

246813579	15 and x	01111011	246	Yes	Yes

11111111	81

Note:	See following page for coding details of the parameters in the table.

Coding details

SIM   IMSI: Data Field 6F 07

Value 246813579		Value 2468135x9

byte 1	05H		05H

byte 2	29H		29H

byte 3	64H		64H

byte 4	18H		18H

byte 5	53H		53H

byte 6	97H		9xH

byte 7	FFH		FFH

byte 8	FFH		FFH

byte 9	FFH		FFH

Access class: Data field 6F 78

See GSM 11.11.

NETWORK (SS)

RACH: As defined in GSM 04.08 section 10.5.2.17.

octet 1	01111000

octet 2	00001000

octet 3	}

octet 4	} as above

Figure on access control

II.8.9	Test Sequence 9:  Exchange protocol tests

Purpose of the tests:

Testing of exchange protocol of the SIM/ME interface.

This sequence lists protocol tests used.

The list of SIM exchange protocol tests is divided into three subsections:

The first subsection deals with character transmission, which constitutes the basis for ME-SIM exchanges.

The following two subsections correspond to the two main parts of a transaction with a microprocessor card: the reset and the card answer then command processing. Both subsections describe the protocol tests to be run during each corresponding phase of the transaction.

The characteristics to be tested are :

II.8.9.1	Character transmission

II.8.9.1.1	ETU variation

II.8.9.1.2	I/O line at zero before character transmission

II.8.9.1.3	Inter-character delay

II.8.9.1.4	Error handling

II.8.9.2	Reset and answer to reset

II.8.9.2.1	Sorts of RST

II.8.9.2.2	Characters of the answer to reset

II.8.9.2.3	PTS procedure

II.8.9.2.4	Reset repetition

II.8.9.3	command processing

II.8.9.3.1	Procedure bytes ACK

II.8.9.1	Character transmission

Purpose

To verify the character frame during answer the transmission (See ISO 7816-3 section 6.1.2). For this test, use a SIM simulator.

II.8.9.1.1	Bit/character duration during the transmission from the ME to the SIM

Procedure

A character transmitted from the transmitter to the receiver comprises ten elements plus the guard time.

The ten elements comprise:

-	the start bit,

-	eight data bits,

-	the parity bit.

The SIM simulator shall measure the bit/character duration of the request sent from the ME.

Requirement

The bit/character duration of the request sent from the ME shall be in the range specified in GSM 11.11, section 5.4.

II.8.9.1.2	Bit/character duration during the transmission from the SIM simulator to the ME

Procedure

The SIM simulator shall send responses with the maximum and minimum bit/character durations specified in GSM 11.11 section 5.4.

Requirement

The ME shall evaluate the response.

II.8.9.1.3	Inter-character delay

Procedure

a)	The requested inter-character delays from the ME to the SIM (or the SIM Simulator) are modified by changing the value of N in TC1 :

a.1)	N = 0;

a.2)	N = Values other than 0 and 255.

b)	Verification is performed in the following two ways:

-	observation using the oscilloscope or logical analyzer,

-	non-acceptance by the SIM (or the SIM Simulator) of the byte transmitted too soon.

c)	The inter-character delay from the SIM (or the SIM simulator) to the ME is altered until a value is obtained at which the ME considers the SIM (or the SIM simulator) to be mute.

Requirement

In case a.1) the ME shall work with the SIM.

In case a.2) the ME shall reject the SIM.

II.8.9.1.4	Error handling

II.8.9.1.4.1	Error handling during the transmission from the ME to the SIM simulator

Procedure

The SIM simulator shall transmit an error signal in response to a received character in accordance with ISO 7816-3, section 6.1.3.

Requirement

The ME shall repeat the character in accordance with ISO 7816-3, section 6.1.3.

II.8.9.1.4.2	Error handling during transmission from the SIM simulator to the ME

Procedure

The SIM simulator shall send a response with a parity error and check that the ME performs error handling in accordance with ISO 7816-3, section 6.1.3.

Requirement

The ME shall send an error signal in accordance with ISO 7816-3, section 6.1.3, and expect a repetition of the character.

II.8.9.2	Answer to reset (RST)

Purpose

To verify if the ME accepts the internal and the active low RESET and detect a mute SIM (See ISO 7816-3 section 5.2).

II.8.9.2.1	Sorts of RST

Three sorts of RST are to be simulated:

-	internal RST,

-	active low RST,

-	mute card.

The SIM (or the SIM simulator) must be synchronized with the successive detection of Vcc and the clock signal in order to begin the reset routine.

II.8.9.2.1.1	Internal RST

Procedure

The SIM (or the SIM simulator) verifies that RST is at low status and starts its answer between (400/fi) s and (40000/fi) s after the clock signal has been detected fi is the initial frequency supplied by the ME.

Requirement

The ME accepts the SIM with internal reset.

II.8.9.2.1.2	Active low RST

Procedure

The SIM simulator sends no internal reset and checks that the RST contact is put to state H after a minimum of (40,000/fi)s. The SIM simulator verifies that the RST contact stays at this level for at least a further (40000/fi) s.

Requirement

The ME shall accept the SIM with internal reset.

II.8.9.2.1.3	Mute card

Procedure

a)	There is no answer before (80000/fi) s.

The SIM (or the SIM simulator) is considered mute and the RST signal returns to low level.

b)	Time measurements are made on the RST signal, using an oscilloscope or a logical analyzer.

Requirement :

The ME shall recognize that the SIM is mute.

II.8.9.2.2	Characters of the answer to reset

Purpose

This paragraph includes tests run on the TS, T0 and all the other bytes in accordance with standard 7816-3. The answer to reset consists of at most 33 characters. The ME shall be able to receive interface characters for other transmission protocols than T = 0, historical characters and a check byte, even if only T = 0 is used by the ME.

Procedure and requirements

The SIM (or the SIM simulator) sends an answer to reset with a set of characters, according to GSM 11.11, clause 5.2.

II.8.9.2.3	PTS procedure

Purpose

To verify that ME uses PTS procedure as specified in GSM 11.11 section 5.2.

Procedure

The SIM (or the SIM simulator) gives an answer to reset with TA1 different of '11H'.

Requirement

The ME shall send to the SIM (or the SIM simulator) 'FF00FF'.

II.8.9.2.4	Reset repetition

Purpose

To verify that the ME repeats the reset between 2 and 5 times after receiving a wrong answer to reset.

Procedure

The SIM simulator sends a non understandable answer to reset to the ME. (e.g. a wrong TS byte).

Requirement

The ME shall repeat the reset between 2 and 5 times after receiving a wrong answer to reset.

Note:	Between 3 up to 6 reset are done before rejecting the SIM.

II.8.9.3	Command processing

II.8.9.3.1	Procedure bytes ACK :

Purpose

To verify that the ME uses correctly the different modes of data transmission defined in ISO 7816-3 section 8.2.2.1.

Procedure

Store data in the SIM (or the SIM simulator). The SIM (or the SIM simulator) first answers ACK=INS or (INS+1) complemented and then changes ACK to INS or INS+1 during the transmission.

Requirement

The command shall be executed correctly.

II.8.10	Test sequence 10: Evaluation of directory characteristics

II.8.10.1	Test sequence 10.1: Operating speed in authentication procedure

Purpose

To verify that the authentication procedure is done with a frequency of at least 13/4 MHz if the bit b2 of the directory characteristics is set to 1 (see GSM 11.11 section 6.2.1.).

Procedure

An authentication is made with a SIM (or a SIM simulator) with bit b2 of the directory characteristics set to 1.

Requirement

The frequency of the clock shall be at least 13/4 MHz during the authentication procedure.

II.8.10.2:	Test sequence 10.2: Clock stop

Purpose

To verify that the clock is only switched off if bit b1 of the directory characteristics is set to 1.

Procedure

A SIM (or a SIM simulator) with b1 set to 0 is used.

Requirement

The ME shall not switch off the clock.

II.8.11	Test sequence 11: Mechanical tests

II.8.11.1	Test sequence 11.1 : Contact pressure

General

The ME manufacturers shall provide a separate card reader (mechanical components) to make the measurement possible.

Purpose

To verify that the contact pressure of each contacting element is not greater than 0.5 N (see GSM 11.11 section 6.1.2.2.) when each of the following types of card is used:

i)	Unembossed

ii)	Embossed on the contact side

iii)	Embossed on the opposite side to the contacts.

Note:	Only type i) applies to the plug-in SIM.

Procedure

The pressure of each contacting element is measured.

Requirement

The contact pressure of each contacting element shall be not greater than 0.5 N.

II.8.11.2	Test sequence 11.2: Shape of contacts for IC card SIM card reader

General

The ME manufacturers shall provide a separate card reader (mechanical components) to make the measurement possible.

Purpose

To verify that the radius of curvature of the contacting elements is greater than or equal to 0.8 mm in the contact area on both axes (see GSM 11.11 section 6.1.2.3.).

Procedure

The radius of curvature of the contacting elements is measured on both axes.

Requirement

The radius of curvature of the contacting elements shall be greater than or equal to 0.8 mm in the contact area on both axes.

II.8.12	Test sequence 12: MMI reaction to SIM status encoding

Purpose of the test

To verify the MMI reaction on the MS when error codes occur from the SIM on the SIM/ME interface.

Procedure

a)	The SIM simulator is used to send the following error codes as reaction on an instruction from the ME:

-	9240	Memory Problem

-	9804	Access security policy not fulfilled or secret code rejected

-	9840	Secret code locked

-	9850	Increment cannot be performed

-	6FXX	Technical problem with no diagnostic given as reaction on an instruction from the ME.

Requirement

The ME shall give an MMI indication.

II.8.13	Test sequence 13: Secret code usage

Purpose of the test

To verify the usage of the secret code on the MMI, as specified in GSM 02.30.

1)	To confirm that it is possible to change the PIN.

2)	To confirm that a PIN with 4 and 8 digits can be handled.

3)	To check whether it is possible to unblock the SIM.

4)	To check whether it is possible to store and recall an abbreviated number.

Procedure:

a)	The SIM defined in tables II.8-1 and II.8-2 above is installed into the ME, and the MS powered-on.

b)	When the MS is in mode "PIN check" enter "2468#".

Requirements:

1)	The MS shall give an indication "OK".

Procedure:

c)	Enter "**04*2468*01234567*01234567#".

Requirements:

2)	The MS shall give an indication that the new PIN is accepted.

Procedure:

d)	The MS is switched off and on.

e)	When the MS is in mode "PIN-check", the sequence "01234567#" is entered.

Requirement

3)	The MS shall give an indication "OK".

Procedure

f)	Switch the MS off and on.

g)	When the MS is in mode "PIN check" enter "2468#".

Requirements:

4)	The MS shall give an indication that the entered PIN is wrong.

Procedure:

h)	Repeat f) to g) two times with wrong PIN numbers.

Requirements:

5)	In step g), requirement 4 shall be met each time.

Procedure:

i)	Enter "**05*13243546*2468*2468#".

Requirements:

6)	The MS shall indicate that the unblocking was successful.

Procedure:

j)	Switch the MS off and on.

k)	When the MS is in mode "PIN check" enter "2468#".

Requirements:

7)	The MS shall give an indication that the entered PIN is ok.

Procedure:

l)	The code "+123456789012345" is stored (entered) in the MS as abbreviated dialling number 7.

m)	The code "00112233" is stored (entered) in the MS as abbreviated dialling number 6.

n)	Enter "7#".

Requirements:

8)	The number "+123456789012345" shall be displayed.

Procedure:

o)	Enter "6#".

Requirements:

9)	The number "00112233" shall be displayed.

DCS1800 only - begin

II.8.14 Test sequence 14:DCS and GSM directories access

Purpose

To verify that a DCS 1800 ME reads the DCS directory of a SIM which has a GSM and DCS directory. The GSM directory is preprogrammed as at the end of test 1 (TMSI value A).

The TMSI value read must be the DCS 1800 one (value B).

Procedure

The procedure is the same as in test sequence 2 with value B for TMSI read from SIM as part of the initialization procedure.

Requirements

The requirements are the same as in test sequence 2, the MS must return the value B for the TMSI.

II.8.15 Test sequence 15: GSM SIM in DCS 1800 ME

Purpose

To verify that a DCS 1800 ME rejects a SIM  with a GSM directory only.

Procedure

a) A SIM with a GSM directory is installed into the ME.

b) The MS is powered on.

Requirement:

The MS shall send a status information indicating' current directory inconsistent'.

DCS1800 only - end