II.11	Testing of teleservices

II.11.1	Category 1: Speech teleservices - transmission requirements

Ref: GSM 03.50, 06.10, (Draft NET 33)

This section provides for the testing of both handset (including hand held portable) and handsfree MS. An MS may be of either type alone or a combined handset and handsfree MS. The test descriptions for handsfree operation, however, only covers the stability margin.

Note 1:	The handsfree tests may be enhanced at a later date to include SLR and RLR etc., until now no agreement was found on the exact methods to be used.

Note 2:	For tests in this section a dedicated channel may need to be assigned in cases where the manufacturer has declared that the DAI is activated by means of a layer 3 message on the air interface.

In this section all references to CCITT recommendations, unless otherwise stated, are to the Blue Book (1988).

Frequency settings in the following tests are taken from ISO 3, R10 series or R40 series or from table 2 of REC CCITT P.79. A departure from the nominal frequencies of + 5% below 240 Hz and + 2% at 240 Hz and above is accepted. Any sub-multiple of the sampling frequency of 8 kHz shall be avoided. In the case of 4 kHz the departure is restricted to -2%.

The measurement accuracy for signal level is +/- 0.2 dB and for sound pressure +/- 0.6 dB.

The tests are performed with "DTX = off".

II.11.1.1	Sending sensitivity/frequency response

Definition

The sending sensitivity frequency response is, as a function of the input test tone frequency, the ratio expressed in dB between the output level, represented by the PCM bit stream at the Digital Audio Interface (DAI) (III.1.4) and the input sound pressure in the artificial mouth required to obtain this.

Method of measurement

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51.

A pure tone with a sound pressure of -4.7 dBPa shall be applied at the mouth reference point (MRP) as described in REC CCITT P.64 using an artificial mouth conforming to REC CCITT P.51.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

Measurements are made at one-twelfth-octave intervals as given by the R40 series of preferred numbers in ISO 3 for frequencies from 100 Hz to 4000 Hz inclusive.

At each frequency the output level represented by the PCM bit stream at the DAI (pin 23) is measured.

Requirement

The sending sensitivity/frequency response (from MRP to the DAI) shall be within a mask given in the following table. The mask can be drawn with straight lines between the breaking points in the table on a logarithmic (frequency) vs linear (dB sensitivity) scale.

All sensitivity levels are dB on an arbitrary scale.

Frequency [Hz]	Upper Limit [dB]	Lower Limit [dB]

a =	100	-12

b =	200	0

c =	300	0	-12

d =	1000	0	-6

e =	2000	4	-6

f =	3000	4	-6

g =	3400	4	-9

h =	4000	0

Sending sensitivity/frequency mask

II.11.1.2	Sending loudness rating

Definition

The Sending Loudness Rating (SLR) is a means of expressing the sending frequency response based on objective single tone measurements in a way which relates to how a speech signal would be perceived by a listener.

Method of measurement

The sending sensitivity (II.11.1.1) is measured at each of the 14 frequencies listed in table 2 of REC CCITT P.79, bands 4 to 17.

The sensitivity is expressed in terms of dBV/Pa and the SLR is calculated according to formula 4.19 b of REC CCITT P.79, over bands 4 to 17, using the sending weighting factors from table 2/CCITT P.79, adjusted according to table 3/CCITT P.79.

Requirement

The SLR shall be 8 +/- 3 dB.

II.11.1.3	Receiving sensitivity/frequency response

Definition

The receiving sensitivity frequency response is, as a function of the input test tone frequency, the ratio expressed in dB between the output sound pressure in the artificial ear and the input level, represented by the PCM bit stream at the Digital Audio Interface (DAI) (III.1.4), required to obtain this.

Method of measurement

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

The SS sends a PCM bit stream equivalent to a pure tone level of -16 dBmO (see NET 33) over the DAI (pin 25) to the MS.

Measurements are made at one-twelfth-octave intervals as given by the R40 series of preferred numbers in ISO 3 for frequencies from 100 Hz to 4000 Hz inclusive.

At each frequency, the sound pressure in the artificial ear (the ear reference point - ERP) is measured by connecting a suitable measuring set to the artificial ear.

Requirement

The receiving sensitivity/frequency response (from the DAI to the ERP) shall be within the mask given by the following table. The mask can be drawn with straight lines between the breaking points in the following table on a logarithmic (frequency) vs linear (dB sensitivity) scale.

All sensitivity levels are dB on an arbitrary scale.

Frequency [Hz]	Upper Limit [dB]	Lower Limit [dB]

a =	100	-12

b =	200	0

c =	300	2	-7

d =	1000	*)	-5

e =	2000	0	-5

f =	3000	2	-5

g =	3400	2	-10

h =	4000	2

Receiving sensitivity/frequency mask

*)	The limit at intermediate frequencies lies on a straight line drawn between the given values on a log (frequency) vs linear (dB) scale.

II.11.1.4	Receiving loudness rating

Definition

The Receiving Loudness Rating (RLR) is a means of expressing the receiving frequency response based on objective single tone measurements in a way which relates to how a speech signal would be perceived by a listener.

Method of measurement

The receiving sensitivity (II.11.1.3) is measured at each of the 14 frequencies listed in table 2 of REC CCITT P.79, bands 4 to 17.

The sensitivity is expressed in terms of dBPa/V and the RLR is calculated according to formula 4.19 c of REC CCITT P.79, over bands 4 to 17, using the receiving weighting factors from table 2/CCITT P.79, adjusted according to table 3/CCITT P.79. The artificial ear sensitivity must be corrected according to the real ear correction table 4 of REC CCITT P.79.

Requirement

If no user controlled receive volume control is provided, the RLR shall be 2 +/- 3 dB, (later referred as the nominal value).

If a user controlled receive volume control is provided, the RLR shall meet this value for at least one setting of the control.

When the control is set to maximum the RLR shall not be less than (ie louder than) -8 dB.

II.11.1.5	Talker sidetone

II.11.1.5.1	Definition

The sidetone loudness ratings are a means of expressing the path loss from the artificial mouth to the artificial ear based on objective single tone measurements in a way that relates to how a speaker will perceive his own voice when speaking (talker sidetone, expressed by the sidetone masking rating - STMR), or how a listener will perceive the background noise picked up by the microphone (listener sidetone rating - LSTR).

Note:	"Listener sidetone" testing is not included in this test specification.

II.11.1.5.2	Method of measurement

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A".

The SS sends a PCM bit stream coded with the value No 1 over the DAI (pin 25) to the MS.

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51.

A pure tone with a sound pressure of -4.7 dBPa shall be applied at the mouth reference point as described in REC CCITT P.64 using an artificial mouth conforming to REC CCITT P 51.

For each frequency given in table 2 of REC CCITT P.79, bands 4 to 17, the sound pressure in the artificial ear is measured.

The sidetone path loss is expressed in dB and the STMR (in dB) is calculated from the formula 8.4 of REC CCITT P.79, using the weighting factors of column (3) in table 6/CCITT P.79 (unsealed), and values of LE in accordance with table 4/CCITT P.79.

II.11.1.5.3	Requirement

The STMR shall be 13 +/- 5 dB.

Where a user controlled receive volume control is provided, the STMR shall meet the requirement given above at the setting where the RLR is equal to the nominal value.

Note:	It is recommended that the talker sidetone is independent of the volume control.

II.11.1.6	Reserved

II.11.1.7	Telephone acoustic coupling loss (TAL)

II.11.1.7.1	Echo return loss (ERL)

Definition

The echo return loss is the path loss from the input of the reference speech encoder of the SS to the output of the reference speech decoder of the SS.

Method of measurement

The DAI of the MS is connected to the SS and is set to the operating mode "Normal operation".

A speech call is set up between MS and SS.

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51.

Where a user controlled volume control is provided it shall be set to maximum.

An analogue or digital signal generator delivering a signal equivalent to a pure tone level of 0 dBmO (see REC CCITT P.64 or 66 respectively) is connected to the corresponding analogue or digital input of the reference speech encoder of the SS.

Measurements are made at one-twelfth-octave intervals as given by the R40 series of preferred numbers in ISO 3 for frequencies from 300 Hz to 3400 Hz.

At each frequency the level or, for a digital signal, equivalent level at the reference speech decoder output of the SS is measured.

The echo return loss is calculated according to REC CCITT G.122.

Note:	The normal operation of the speech codec results in modulation of the amplitude of pure tone signals at a frequency that depends upon the frequency of the tone. This makes the measurement of TAL with pure tones uncertain. Choice of test signal is therefore important but, whilst a speech-like signal may be more suitable, insufficient information is available for a definitive choice.

Requirement

The echo return loss from the input to the output of the reference speech codec in the SS shall be at least 46 dB.

II.11.1.7.2	Stability margin

Definition

The receive-transmit stability margin is a measure of the gain that would have to be inserted between the go and return paths of the reference speech coder in the SS for oscillation to occur.

Method of measurement

A gain equivalent to the minimum stability margin is inserted in the loop between the go and return paths of the reference speech coder in the SS and any acoustic echo control is enabled.

A test signal according to REC CCITT O.131 is injected into the loop at the analogue or digital input of the reference speech codec of the SS and the stability is observed. The test signal shall have a level of -10 dBmO and a duration of 1 s.

Where a user controlled volume control is provided it shall be set to maximum.

For handset operation the handset is placed on a hard plane surface with the transducers facing the surface.

For handsfree operation the test setup is shown in REC CCITT P.34 (Fig 3/CCITT P.34), but omitting the test table.

Requirement

The minimum stability margin shall be 6 dB and no audible oscillation shall be detected.

II.11.1.8	Distortion

II.11.1.8.1	Sending

Definition

The transmit signal to total distortion ratio is a measure of the linearity of the transmitter equipment (excluding the speech codec).

Method of measurement

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

A sine-wave signal with a frequency in the range 1004 Hz to 1025 Hz is applied to the MRP. The level of this signal is adjusted until the PCM bitstream at the DAI output (pin 23) corresponds to -10 dBmO. The level of the signal at the MRP is then the acoustic reference level (ARL).

The test signal is applied at the following levels: -35, -30, -25, -20, -15, -10, -5, 0, 5, 10 dB relative to the ARL.

The ratio of signal to total distortion power is measured at the DAI with the psophometric noise weighting (see REC CCITT G.714 and O.132) at each signal level.

The measurement shall be carried out at sound pressures not exceeding +10 dBPa.

Requirement

The ratio of signal to total distortion power measured at the DAI with the psophometric noise weighting (see table 4/CCITT G.223) shall be above the limits given in the following table.

dB relative to ARL	Level ratio

-35 dB	17.5 dB

-30 dB	22.5 dB

-20 dB	30.7 dB

-10 dB	33.3 dB

0 dB	33.7 dB

7 dB	31.7 dB

10 dB	25.5 dB

Limits for the signal to total distortion ratio (sending) when using the sine wave method.

Limits for intermediate levels are found by drawing a straight line between breaking points in a linear (dB signal level) vs linear (dB ratio) scale.

II.11.1.8.2	Receiving

Definition

The receive signal to total distortion ratio is a measure of the linearity in the receive equipment.

Method of measurement

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

The SS sends, via the DAI (Pin 25), a PCM bit stream simulating a sine-wave signal corresponding to REC CCITT O.132 at the following levels: -45, -40, -35, -30, -25, -20, -15, -10, -5, 0 dBmO.

The ratio of signal to total distortion power is measured with the psophometric noise weighting in the artificial ear (see REC CCITT G.714) at each signal level.

The measurement shall be carried out at sound pressures between -50 dBPa and +10 dBPa.

Requirement

The ratio of signal to total distortion power measured at the artificial ear with the psophometric noise weighting (see table 4/CCITT G.223) shall be above the limits given in the following table.

Level at the digital audio interface	Level ratio

-45 dBmO	17.5 dB

-40 dBmO	22.5 dB

-30 dBmO	30.5 dB

-20 dBmO	33.0 dB

-10 dBmO	33.5 dB

-3 dBmO	31.2 dB

0 dBmO	25.5 dB

Limits for the signal to total distortion ratio (receiving) when using the sine wave method.

Limits for intermediate levels are found by drawing a straight line between breaking points in a linear (dB signal level) vs linear (dB ratio) scale.

II.11.1.8.3	Sidetone

Definition

The sidetone distortion expresses the linearity of the sidetone path in the handset.

Method of measurement

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A".

The SS sends the PCM bit stream coded with the value No 1 over the DAI (pin 25) to the MS.

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51.

An instrument capable of measuring the third harmonic distortion of signals with fundamental frequencies in the range 315 Hz to 1000 Hz is connected to the artificial ear.

A pure-tone signal of -4.7 dBPa is applied at the mouth reference point at frequencies of 315 Hz, 500 Hz, and 1000 Hz. For each frequency the third harmonic distortion is measured in the artificial ear.

Requirement

The third harmonic distortion generated shall not be greater than 10%.

II.11.1.9	Out-of-band signals

II.11.1.9.1	Sending

Definition

The discrimination against out-of-band input signals in the sending direction is a requirement on the in-band image frequencies created by any out-of-band input signals.

Method of measurement

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51.

A pure tone with a sound pressure of -4.7 dBPa shall be applied at the mouth reference point as described in REC CCITT P.64 using an artificial mouth conforming to REC CCITT P 51.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

For input signals at frequencies of 4.65, 5, 6, 6.5, 7, and 7.5 kHz, the level represented by the PCM bit stream at the DAI (Pin 23) of any image frequency is measured.

Requirement

The level of any image frequency shall be below a reference obtained at 1 kHz by at least the amount as specified in the following table.

Applied sine-wave frequency	Limit (minimum)

4.6 kHz	30 dB

8 kHz	40 dB

Limits for the image frequency discrimination

The limit at intermediate frequencies lies on a straight line drawn between the given values on a log(frequency) vs linear(dB) scale.

II.11.1.9.2	Receiving

Definition

The discrimination against out-of-band signals in the receiving direction is a requirement on the out-of-band signals generated in the artificial ear from in-band input signals.

Method of measurement

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

The SS sends over the DAI (pin 25) a PCM bit stream simulating a sine-wave signal with a level of 0 dBmO .

For input signals at the nominal frequencies 500, 1000, 2000, and 3350 Hz (bearing in mind the restriction on sub-multiples of the sampling frequency) the level of any out-of-band signals at frequencies up to 8 kHz is measured in the artificial ear.

Requirement

The level of out-of-band signals shall be lower than the in-band acoustic level obtained by a digital signal at 1 kHz set at the level specified in the following table.

Image signal frequency	Equivalent input signal level

4.6 kHz	-35 dBmO

8 kHz	-45 dBmO

Limits for the image frequency discrimination

The limit at intermediate frequencies lies on a straight line drawn between the given values on a log(frequency) vs linear(dB) scale.

II.11.1.10	Idle channel noise

II.11.1.10.1	Sending

Definition

The idle channel noise in the sending direction is the equivalent noise level produced at the DAI, when the mouth reference point is in a quiet environment.

Method of measurement

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51 in a quiet environment (ambient noise less than 30 dBA).

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

The noise level represented by the PCM bit stream output at the DAI (pin 23) is measured with psophometric weighting according to REC CCITT G.223, table 4.

Requirement

The noise produced by the MS in the sending direction shall not exceed -64 dBmOp.

II.11.1.10.2	Receiving

Definition

The idle channel noise in the receiving direction is the acoustic sound pressure in the artificial ear when the digital input signal at the DAI, is the PCM coded value No 1.

Method of measurement

The handset shall be mounted in the LRGP (see annex 1 of REC CCITT P.76) and the earpiece shall be sealed to the knife-edge of the artificial ear conforming to REC CCITT P.51.

The DAI of the MS is connected to the SS and is set to the operating mode "Test of acoustic devices and A/D & D/A ".

The SS sends a PCM bit stream coded with the value No 1 over the DAI (Pin 25) to the MS.

The level of the noise is measured in the artificial ear with any volume control set at the position at which the RLR is equal to the nominal value.

Requirement

The measured noise generated by the MS shall not exceed -57 dBPa (A).

Where a volume control is provided, the measured noise shall also not exceed -54 dBPa (A) at the maximum setting of the volume control.

II.11.2	Category 2: Short message service (SMS)

Ref : GSM 03.40, 04.11 (point to point)

GSM 03.41, 04.12 (cell broadcast)

II.11.2.1	General

The purpose of these tests is to verify that the MS can handle GSM functions when submitting or receiving Short Messages (SM) between MS and a short message service centre as described in GSM 03.40.

The procedures are based upon services provided by the Mobility Management (MM) sublayer which is not tested in this case.

The SMS comprises three basic services. The SMS point to point services shall work in an active MS at any time independent of whether or not there is a speech or data call in progress. The SMS cell broadcast service only works when the MS is in idle mode.

Since the timer TC1M currently is not standardised, the value of TC1M shall be declared by the manufacturer (to be used in II.11.2.2.1 steps d), e), g2), g3), h), i) and requirements 4), 5) and 6), II.11.2.2.2, steps d), f), g), h2), h3) and requirements 5) and 7)).

The manufacturer shall declare whether SMS messages are stored in the SIM and/or the ME. This shall be referred to as the SMS message store in the following tests.

Related PIXIT statements: according to GSM 11.10, Annex 3, sections 2.1.5, 2.2.2 and 2.3.4.

II.11.2.2	Short message service point to point

II.11.2.2.1	SMS mobile terminated

Purpose of test :

This test verifies the ability of a MS to receive and decode the SMS where provided for the point to point service.

Method of test :

a)	The MS shall be in idle, updated state. The SMS message store shall contain no messages.

b)

-	The SS initiates the transmission of a short message using a paging request. Upon response of the MS to the paging the SS assigns an SDCCH, authenticates the MS and activates ciphering. Then the SS establishes SAPI 3 by sending a SABM frame with SAPI - 3 on the SDCCH.

-	When a UA frame (SAPI-3) is received in response, the SS sends a CP_DATA message. The information element of the CP_DATA message will be RP_DATA.

c)	The SS waits a maximum of 25 seconds for the CP_ACK message and then a maximum of 60 seconds for the CP_DATA message containing the RP_ACK.

d)	The SS sends a CP_ACK to the MS within TC1M with no further CP-DATA messages and the SS initiates channel release.

e)	Steps b), c) and d) are repeated but the first CP-DATA message from the MS is not acknowledged. The second CP-DATA message from the MS is acknowledged by a CP-ACK within a time TC1M.

f1)	Steps b) and c) are repeated but the CP-DATA messages from the MS is not acknowledged. The SS then initiates the channel release.

f2)	The SMS message store shall be cleared.

g1)

-	A data or speech call is established on a TCH with the SS and the state U10 of call control is entered. The SS sends a SABM frame with SAPI-3 on the SACCH associated to the TCH.

-	When a UA frame (SAPI-3) is received in response, the SS sends a CP_DATA message. The information element of the CP_DATA message will be RP_DATA. The SS waits a maximum of 25 seconds for the CP_ACK message and then a maximum of 60 seconds for the CP_DATA message containing the RP_ACK.

g2)	The SS sends a CP_ACK to the MS within TC1M with no further CP_DATA messages and the SS initiates channel release. The SMS message store shall be cleared.

g3)	Steps g1) and g2) are repeated but the first CP_DATA message from the MS is not acknowledged. The second CP_DATA message from the MS is acknowledged by a CP_ACK within a time TC1M.

g4)	Step g1) is repeated but the CP_DATA messages from the MS are not acknowledged. The SS then initiates the channel release.

h)	A data or speech call is established on a TCH with the SS and the state U10 of call control shall be entered. The SS sends a SABM frame with SAPI-3 on the SACCH associated to the TCH. After the UA response on SAPI-3, the speech call is cleared by the SS with a disconnect message. (The call clearing is continued on the FACCH in parallel to the following exchange of messages related to SMS).

The SS sends a CP_DATA message. The information element of the CP_DATA message is RP_DATA.

The SS waits a maximum of 25 seconds for the CP_ACK message and then a maximum of 60 seconds for the CP_DATA message containing the RP_ACK.

The SS sends a CP_ACK to the MS within TC1M with no further CP_DATA messages and the SS initiates channel release.

The SMS message store shall be cleared.

i)	A data or speech call is established on a TCH with the SS and the state U10 of call control is entered. The SS sends a SABM frame with SAPI-3 on the SACCH associated to the TCH. After the UA response on SAPI-3, the SS waits 15 seconds. During this time the speech call shall be cleared from the MS. (The call clearing is continued on the FACCH in parallel to the following exchange of messages related to SMS).

The SS sends a CP_DATA message. The information element of the CP_DATA message is RP_DATA.

The SS waits a maximum of 25 seconds for the CP_ACK message and then a maximum of 60 seconds for the CP_DATA message containing the RP_ACK.

The SS sends a CP_ACK to the MS within TC1M with no further CP_DATA messages and the SS initiates channel release.

The SMS message store shall be cleared.

Requirements :

1)	In the case of step b), after the paging response, authentication and ciphering, the MS shall respond to the SABM frame on SAPI-3 by sending a UA frame with SAPI-3.

2)	In the case of step c), the MS shall send a CP_ACK message within 25 seconds from the reception of the CP_DATA message followed by a CP-DATA message containing the RP-ACK to the SS within 85 seconds.

3)	In the case of step d), the MS shall return to idle mode (this is checked implicitly by paging the MS in step e)). The MS shall indicate that a short message has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed (see PICS/PIXIT).

4)	In the case of step e), the MS shall repeat once the CP-DATA message containing the RP-ACK within twice TC1M. After receipt of the channel release message the MS shall return to idle mode (this is checked implicitly by paging the MS in step f1)). The MS shall indicate that a short message has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed (see PICS/PIXIT).

5)	In the case of step f1), the MS shall repeat, once and only once, the CP-DATA message containing the RP-ACK within twice TC1M. After receipt of the channel release message the MS shall return to idle mode (this is checked implicitly by paging the MS in step g)). The MS shall indicate that a short message has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed (see PICS/PIXIT).

6)	In the case of step g1), the MS shall respond to the SABM frame on SAPI-3 by sending a UA frame with SAPI-3. Then the MS shall send a CP_ACK message within 25 seconds from the reception of the CP_DATA message followed by a CP_DATA message containing the RP_ACK to the SS within 85 seconds.

In the case of step g2), the MS shall return to idle mode (this is checked implicitly by paging the MS in step g3)). The MS shall indicate that a short message has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed (see PICS/PIXIT).

In the case of step g3), the MS shall respond to the SABM frame on SAPI-3 by sending a UA frame with SAPI-3. Then the MS shall send a CP_ACK message within 25 seconds from the reception of the CP_DATA message followed by a CP_DATA message containing the RP_ACK to the SS within 85 seconds.

The MS shall repeat once the CP_DATA message containing the RP_ACK within twice TC1M. After receipt of the channel release message the MS shall return to idle mode (this is checked implicitly by paging the MS in step g4)). The MS shall indicate that a short message has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed (see PICS/PIXIT).

In the case of step g4), the MS shall respond to the SABM frame on SAPI-3 by sending a UA frame with SAPI-3. Then the MS shall send a CP_ACK message within 25 seconds from the reception of the CP_DATA message followed by a CP_DATA message containing the RP_ACK to the SS within 85 seconds.

The MS shall repeat once, and only once, the CP_DATA message containing the RP_ACK within twice TC1M. After receipt of the channel release message the MS shall return to idle mode (this is checked implicitly by paging the MS in step h)). The MS shall indicate that a short message has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed (see PICS/PIXIT).

7)	In the case of step h), the requirements for steps g1) and g2) shall be satisfied (call clearing shall be performed in parallel on the FACCH).

8)	In the case of step i), the requirements for steps g1) and g2) shall be satisfied (call clearing shall be performed in parallel on the FACCH).

Note:	Time values in steps c), g1) and h) are chosen sufficiently high to be sure that the MS has enough time to respond to the different messages.

Mobile terminated test short message content : SMS DELIVER TPDU included in the RP_DATA message, which is included in the CP_DATA message.

Information element	Comment	Value

Message type indication	MT

More message to send	"more message waiting"

Originating address	an E164 number

Protocol identifier		0

Data coding scheme		0

Service centre timestamp	any legal value

User data length		160

User data (140 octets)	160 characters (text of message)

Note:	The 160 characters shall include at least one occurrence of each character in the default alphabet (see GSM 03.40 annex 2).

II.11.2.2.2	SMS mobile originated

Purpose of test :

This test verifies the MS ability to correctly send a short message where the SMS is provided for the point to point service. It also verifies the MS capability to simultaneously receive a network originated SM whilst sending a mobile originated SM.

Method of test :

a)	The SS is configured to support SMS and the MS shall be in idle, updated state. The SMS store shall contain no messages.

b)	The MS shall be set up to send a SM to the SS. The SS responds to the channel request message by allocating an SDCCH. The SS answers correctly to the SABM on SAPI 0 and then performs the authentication and ciphering procedures.

c)	The SS responds with a UA frame SAPI-3 to the MS.

d)	The SS responds to the CP_DATA from the MS with a CP-ACK message within TC1M followed by a CP-DATA message containing the correct RP-ACK. The SS waits a maximum of 25 seconds for the CP_ACK message.

e)	The SS sends a channel release message to the MS.

f)	Steps b) and c) are repeated with the SS configured not to send the CP_ACK. After a duration of 3*TC1M the SS initiates channel release.

g)	Steps b) and c) are repeated. On receipt of the CP_DATA from the MS the SS sends a CP_ERROR message within TC1M containing a "Network Failure" cause. Then the SS initiates channel release.

h1)	A data or speech call is established on a TCH with the SS and the state U10 of call control is entered. The MS is setup to send an SM to the SS. After the reception of the CM SERVICE REQUEST, the SS sends a CM SERVICE ACCEPT message. The SS responds with a UA frame SAPI-3 to the SABM with SAPI-3 received from the MS.

h2)	The SS responds to the CP_DATA from the MS with a CP-ACK message within TC1M followed by a CP-DATA message containing the correct RP-ACK. The SS waits a maximum of 25 seconds for the CP_ACK message. Then the SS sends a channel release message to the MS.

h3)	Step h1) is repeated. The SS is configured not to send the CP_ACK message. After a duration of 3*TC1M the SS initiates channel release.

i)	The SS is configured to receive a mobile originated SM. Step b) is repeated and, using the end of the CP-DATA message from the MS as a trigger, the SS sends a SM to the MS. In this case a new transaction identifier shall be used in the CP messages of SMS mobile terminated.

j)	The MS is set up to send an SM to the SS. On receipt of the CM SERVICE REQUEST the SS sends a CM SERVICE REJECT message with the reject cause set to "Service Option not supported" or "Service Option temporarily out of order". After 5 seconds the SS initiates channel release.

Requirements :

1)	In the case of step b), the MS shall send channel request with the establishment cause set to "Other services requested by the mobile user". After SDCCH allocation, the MS shall establish the layer 2 on SAPI 0 by sending an SABM frame piggy backing a CM SERVICE REQUEST message with the CM service type set to "short message transfer". After the UA response the MS shall correctly answer to the authentication procedure and activate the ciphering when requested by the SS. Once the ciphering procedure is complete the MS shall send a SABM frame with SAPI-3.

2)	In the case of step c), the MS shall send the CP-DATA which shall be displayed as correct by the SS.

3)	In the case of step d), the MS shall send a CP-ACK message within 25 seconds.

4)	In the case of step e), the MS shall respond with a layer 2 DISC frame with SAPI-0.

5)	In the case of step f), the MS shall send the CP-DATA message and after time TC1M this CP-DATA message shall be repeated once only.

6)	In the case of step g), after establishment of SAPI-3, the MS shall send the CP_DATA message which shall be displayed as correct by the SS.

7)	In the case of step h1), the MS shall send a CM SERVICE REQUEST message with the CM service type set to "short message transfer" sent in a layer 2 frame on the FACCH. After reception of the CM SERVICE ACCEPT from the SS, the MS shall send a SABM frame with SAPI-3 on the SACCH.

In the case of step h2), the MS shall send the CP-DATA which shall be displayed as correct by the SS. The MS shall send a CP-ACK message within 25 seconds in response to the CP_DATA from the SS. After receipt of the channel release message, the MS shall respond with a layer 2 DISC frame with SAPI-0.

In the case of step h3) the requirements for step h1) shall be satisfied. The MS shall send the CP_DATA message and after time TC1M this CP_DATA message shall be repeated once only. After receipt of the channel release message, the MS shall respond with a layer 2 DISC frame with SAPI-0.

8)	In the case of step i), the MS shall correctly receive the short message and indicate that a message has arrived. If the MS provides the functionality to display MT messages, it is checked that the correct message is displayed (see PICS/PIXIT). In the mobile originated case the MS shall send the CP-ACK message with transaction identifier assigned to this transfer. In the mobile terminated case the MS shall send a CP-ACK message and a CP-DATA message containing the RP-ACK. The transaction identifier shall be the same as the one chosen by the SS for the mobile terminated transfer.

9)	In the case of step j), the MS shall initiate establishment of MM connection. After receipt of the CM SERVICE REJECT message the MS shall not establish SAPI-3.

Note:	Time values in steps d) and h2) are chosen sufficiently high to be sure that the MS has enough time to respond to the different messages.

Mobile originated test short message content : SMS SUBMIT TPDU

Information element	Comment	Value

Message type indication	MO

Validity period format	"TP-VP field present and integer

represented (relative)"

Destination address	an E164 number

Protocol identifier	all zeros	0

Data coding scheme	standard alphabet	0

Validity period	24 hours

User data length	as applicable

User data (140 octets max)	maximum number of characters

(text of message) as defined

by the manufacturer (see PICS/PIXIT)

II.11.2.3	Short message service cell broadcast

Purpose of test :

This test verifies that a MS is able to respond to a paging requested during the transmission of a cell broadcast short message.

Method of test :

a)	The MS is in idle mode. Periodic location updating is disabled.

b)	The SS provides a BCCH/CCCH to support the MS in idle mode.

c)	Three Cell Broadcast (CB) messages are sent by the SS on the CBCH with sequence numbers 0,1,1.

d)	Step c) is repeated, but the SS pages the MS during the transmission of the second CB message.

Cell broadcast test message content

Information element	Comment	Value

Sequence no.	See above

Message identifier		0

Alpha/language identifier	0

Page parameter		0001 0001

Characters of message	93 user characters

using all characters

of default 7 bit

coded alphabet

Requirements :

1)	In the case of step d), the MS shall respond to the page.

This requirement applies to all MS types whether or not they support SMS cell broadcast.

II.11.3	Category 3: Message handling service

Ref: GSM 03.42

(Reserved.)

II.11.4	Category 4: Videotex

Ref: GSM 03.43

(Reserved.)

II.11.5	Category 5: Teletex

Ref: GSM 03.44

(Reserved.)

II.11.6	Category 6: Facsimile

Ref: GSM 03.45 and 03.46

(Reserved.)