II.13 	Testing of speech transcoding functions

Ref: GSM 06.10, 06.11, 06,12, 06.31 and 06.32

The speech transcoding in the GSM mobile station transforms the 13 kbit/s net bit stream, which is transmitted over the radio path via the functions described in the GSM 05 series of recommendations, to 104 kbit/s 13 bit linear PCM, or vice versa.

Following the 13 bit linear PCM there are audio parts like D/A conversion, filtering, microphones and loud speakers. These are verified by the transmission requirements given in section II.11.1.

For further information see GSM 06.01 and GSM 06.10.

For all tests in this chapter II.13, the layer 1 functions of the MS must already be verified and the tests must be performed under ideal radio conditions.

The test sequence SEQ02.COD should not be used for the downlink transcoding tests in II.13.1 and II.13.5.1.1.

Note:	For a definition of the term "traffic frame" used in this chapter, refer to GSM 06.31. For a description of their allocation within the TDMA structure on the radio interface, see GSM 05.02. For a description of the method to achieve test data synchronism over the radio interface, see III.1.4.

II.13.1	Downlink speech transcoding

Definition

This transcoding transforms the 13 kbit/s net radio path bit stream to 13 bit linear PCM.

For further information see GSM 06.10 and refer to the speech decoder parts.

Method of test

a)	For this test DTX is off.

b)	The SS will send a test sequence at 13 kbit/s before channel coding to the MS.

c)	The test sequences for speech transcoding are defined in GSM 06.10 section 5 and are available on floppy disks on IBM/AT MS/DOS format from ETSI secretariate.

d)	The sequences to be input in this test are contained in the files SEQxx.COD. The files contain 16 bit words for all speech encoded parameters and are justified as described in GSM 06.10 table 5.1. 76 words are input in a period of 20 ms and the speech decoder is reset before the start of the test (see section III.1.4).

e)	The 104 kbit/s output bit stream from the MS is recorded on the digital audio interface described in section III.1.4.

Requirements

1.a)	The bit stream output shall be continuous and bit by bit exactly the same as the sequence given in the files SEQxx.OUT on the floppy disks. These files contain 16 bit words of 13 bit linear PCM left justified. See also GSM 06.10 table 5.1.

1.b)	This requirement shall be applied to all files of the type SEQxx.COD.

II.13.2	Downlink receiver DTX functions

Definition

The DTX/VAD receiver functions consist of a SID frame detector, comfort noise generator functions and speech extrapolation and muting functions.

Details about the overall DTX operation for full rate speech traffic channels are given in GSM 06.31, the speech extrapolation and muting functions are described in GSM 06.11 and the comfort noise aspects in GSM 06.12.

Method of test

a)	At 13 kbits/s level in the SS (before channel encoding) coded "speech" traffic frames containing a special test signal defined below are input and transmitted over the radio path. All traffic frames are identical with the exception of some frames which are SID frames as defined in GSM 06.31.

b)	At the digital audio interface in the MS at 104 kbit/s level (13 bit/8 kHz linear PCM), the energy of the PCM signal is evaluated (as a mean square average) and recorded for each block of 20 ms synchronized to the 20 ms speech frame structure.

c)	The transmission of the TDMA frames of the TCH/FS on the radio path is ramped "on" or "off" on a traffic frame by traffic frame basis, taking into account the block diagonal interleaving scheme defined in GSM 05.03. The first traffic frame in step 1 occurs one frame after the window of the SACCH multiframe (TDMA frame 60 modulo 104), allocated for the SID frame (see GSM 05.02 and 05.08). The SACCH will also be transmitted.

Note:	8 timeslots in 8 consecutive TCH/FS TDMA frames is seen as one traffic frame and the next traffic frame starts in the middle of the previous one (i.e. after 4 TDMA frames of the previous one) due to the block diagonal interleaving scheme defined in GSM 05.03.

d)	The special test frame is an encoded "speech" traffic frame of 260 bits obtained from white Gaussian noise band limited to 300 - 3400 Hz. When repeated, the special test frame results in a humming sound with a fairly constant level when decoded, and is defined in table .

Encoded parameter:	Value:

LARc(1)	38

LARc(2)	42

LARc(3)	24

LARc(4)	20

LARc(5)	10

LARc(6)	9

LARc(7)	5

LARc(8)	3

Sub-block n0:

0	1	2	3

Grid position (Mc)	1	3	2	0

Block amplitude (xmaxc)	40	40	40	40

LTP gain (Bc)	0	0	0	0

LTP lag (Nc)	40	120	40	120

RPE pulses (xmc)

- pulse no 1	4	6	6	6

- pulse no 2	4	5	4	3

- pulse no 3	2	1	3	4

- pulse no 4	6	2	1	3

- pulse no 5	3	6	4	1

- pulse no 6	5	1	6	3

- pulse no 7	5	2	5	5

- pulse no 8	5	6	2	1

- pulse no 9	1	3	4	4

- pulse no 10	3	2	4	3

- pulse no 11	5	5	4	5

- pulse no 12	6	1	2	2

- pulse no 13	1	3	4	3

Table : Table of special test traffic frame for receiver DTX tests

e)	The level of the special test frame is controlled with the block amplitude parameter (xmaxc). Reducing xmaxc from 40 to 32 reduces the decoded level with 3 dB, and reducing xmaxc from 40 to 24 reduces the decoded level with 6 dB.

f)	The sequence of traffic frames on the air interface is as follows:

f.1)	23 test frames "on".

f.2)	20 frames "off".

f.3)	20 test frames "on".

f.4)	1 SID frame followed by 6 frames "off", another identical SID frame and 23 frames "off". Except for the SID codeword, the SID frames are identical to the test frame.

f.5)	1 different SID frame, however with 2 to 15 errors inserted in the SID codeword, followed by 23 frames "off".

f.6)	20 test frames "on", but with the level parameter xmaxc = 24.

f.7)	1 SID frame followed by 50 frames "off". Except for the SID codeword, the SID frames are identical to the test frame.

f.8)	The whole test is repeated, but the frames "off" are replaced by frames "on" with the FACCH flag set.

Requirements

1)	In the case of step f.1), the signal energy shall be fairly constant within +/- 3 dB.

2)	In the case of step f.2), the signal energy shall decrease to less than -60 dBm within 17 frames.

3)	In the case of step f.4), comfort noise shall be generated. The same requirements as in step 1) apply.

4)	In the case of step f.5), the same requirements as in step 4) apply.

5)	In the case of step f.6), the same requirements as in step 1) apply. However, the signal energy shall be 12 dB lower.

6)	In the case of step f.7), the signal energy shall be fairly constant within +/- 3 dB for 28 frames. Then the signal energy shall decrease to less than -60 dBm within 16 frames.

7)	In the case of step f.8), the same requirements as in all previous steps apply.

II.13.3	Uplink speech transcoding

Definition

This transcoding transforms 13 bit linear PCM to the 13 kbit/s net radio path bit stream.

For further information see GSM 06.10 and refer to the speech encoder parts.

Method of test

a)	For this test DTX shall be off.

b)	At 104 kbit/s level, at the digital audio interface in the MS, a test sequence is input.

c)	The test sequences for speech transcoding are defined in GSM 06.10 section 5 and are available on floppy disks on IBM/AT MS/DOS format from ETSI secretariate.

d)	The sequences to be input in this test are contained in the files SEQxx.INP. The files contain 16 bit words for 13 bit linear PCM left justified. See also GSM 06.10 table 5.1. The speech decoder is reset before the start of the test (see section III.1.4).

e)	At 13 kbit/s level in the SS over the radio path the output bit stream is recorded.

Requirements

1.a)	The bit stream output shall be bit by bit exactly the same as the sequence given in the files SEQxx.COD on the floppy disks. These files contain 16 bit words of all the 76 parameters in a speech frame justified as in GSM 06.10 table 5.1. 76 codewords shall occur in a frame of 20 ms.

1.b)	This requirement shall be applied to all files of the type SEQxx.INP.

II.13.4	Uplink transmitter DTX functions

Definition

The DTX/VAD transmitter functions consist of a Voice Activity Detector (VAD) and a surrounding Discontinuous Transmission (DTX) system introducing additional "speech" traffic frames on the air compared to those the VAD itself would classify as speech frames containing real speech. The additional traffic frames on the air are introduced due to:

1)	A "hangover" period at the end of speech bursts in order to be certain that the traffic frames contain only noise and to evaluate the background acoustic noise characteristics when no real speech is present.

2)	Special traffic frames (SID frames) added on the air at regular intervals containing only the evaluated background acoustic noise characteristics. These frames are used for generation of comfort noise in speaker silence periods on the receiving side.

Details about the overall DTX operation for full rate speech traffic channels are given in GSM 06.31, the VAD is described in GSM 06.32 and the comfort noise aspects in GSM 06.12.

Method of test

a)	A call is set up on a full rate speech TCH.

b)	A sequence of PCM samples, which are grouped into frames of 20 ms synchronized to the TDMA and traffic frame structure on the radio path, are input at the digital audio test port in the MS at 104 Kbits/s (13 bit/8 kHz linear PCM).

The start of the test sequences is synchronized with the radio transmission on the air interface so that the first traffic frame on the air occurs just after the traffic frame allocated for the SID frame (TDMA frame 56 modulo 104, see GSM 05.02 and GSM 05.08).

Note:	8 timeslots in 8 consecutive TCH/FS TDMA frames is seen as one traffic frame and the next traffic frame starts in the middle of the previous one (i.e. after 4 TDMA frames of the previous one) due to the block diagonal interleaving scheme defined in GSM 05.03.

c)	For this test the input frames are taken from the file SEQ0x.INP defined in GSM 06.32.

d)	The SS detects whether or not there is any power transmitted over the radio path on a timeslot basis excluding SACCH frames. The speech frame by speech frame on/off transmission (on = 1) is recorded.

Requirements

1)	In step d), the traffic frame on/off sequence recorded shall be bit exact like the sequence of SP flags stored as bit 15 of LAR(2) on the reference files *COD defined in GSM 06.32, with the following exceptions:

1.1)	The occurrence of a SID frame in its allowed window within the SACCH multiframe as defined in GSM 05.08.

1.2)	The occurrence of a SID frame after 1 or more real speech frames consecutively transmitted on the air.

II.13.5	Speech channel transmission delay

The total transmission delay within the various elements of a GSM system are specified as round trip delays. For the MS this would be equivalent to applying an RF equivalent of a speech signal to the MS receiver, closing an acoustic path from the ERP to the MRP, detecting the corresponding RF signal at the MS transmitter output and measuring the time interval between the signal originally fed to the MS receiver and that transmitted by the MS transmitter.

This simple approach cannot be demonstrated to be accurate due to the inherent non linear characteristic of the speech transcoder. The overall delay therefore is split into four identifiable and measurable delays. The delays are respectively:

-	the downlink delay from RF input to DAI output,

-	DAI output to ERP,

-	MRP to DAI output, and

-	DAI to uplink RF output.

Each delay is defined and its method of measurement described in the following sections.

II.13.5.1	Downlink processing delay

The downlink processing delay is the delay from the first bit of a speech block transmitted from the RF output of the SS up to the last bit of the corresponding speech block received at the DAI on the output of the speech transcoder.

II.13.5.1.1	Method of test

a)	The test set up is that described in II.13.1 for downlink speech transcoding.

b)	One of the test patterns described in II.13.1 is transmitted by the SS to the MS.

c)	For each speech block transmitted by the SS the SS notes the time of the first bit at the air interface and the time of the last bit of that speech block on the DAI.

d)	As the processing time could be a variable, step c) is repeated 20 times and the maximum delay noted is recorded as TDP.

II.13.5.2	Downlink coding delay

II.13.5.2.1	Definition

The downlink coding delay is defined as the delay between the digital representation of an acoustic signal on the DAI and the corresponding acoustic signal at the ERP.

II.13.5.2.2	Method of measurement

a)	The handset is mounted in the LRGP (see annex 1 of CCITT recommendation P.76) and the earpiece is sealed to the knife edge of the artificial ear conforming to CCITT recommendation P.51.

b)	The SS generates on the DAI a digital representation of a sine wave with a frequency of 1000 Hz.

c)	The SS measures the "phase shift" ∞1, in the range of 0 to 360 degrees, between the equivalent sine wave generated at the DAI and the sine wave at the input to the artificial ear.

d)	The frequency is increased to 1100 Hz and the resulting phase shift ∞2 noted.

e)	The downlink coding delay TDC is calculated from either:

TDC = (∞2 - ∞1)/36 ms	for ∞2 > ∞1

or

TDC = (∞2 + 360 - ∞1)/36 ms	for ∞2 < ∞1

II.13.5.3	Uplink processing delay

II.13.5.3.1	Definition

The uplink processing delay is the delay from the first bit of a speech block on the DAI to the last bit of that speech block being transmitted on the air interface of the MS.

II.13.5.3.2	Method of measurement

a)	The test set up is that described in II.13.3 for uplink speech transcoding.

b)	One of the test patterns described in II.13.3 is input to the DAI of the MS by the SS.

c)	For each speech block output by the SS on the DAI the SS notes the time of the first bit on the DAI and at the air interface of the MS the time of the last transmitted bit of the block.

d)	As the processing time could be a variable, step c) is repeated 20 times. The maximum delay noted is then recorded as TUP.

II.13.5.4	Uplink coding delay

II.13.5.4.1	Definition

The uplink coding delay is defined as the delay between an acoustic signal at the MRP and the digital representation of that signal on the DAI.

II.13.5.4.2	Method of measurement

a)	The handset is mounted in the LRGP (see annex 1 of CCITT recommendation P.76) and the earpiece is sealed to the knife edge of the artificial ear conforming to CCITT recommendation P.51.

b)	The SS generates an acoustic signal at the artificial mouth of the LRGP, being a pure sine wave with a frequency of 1000 Hz.

c)	The SS measures the "phase shift" ∞1, in the range of 0 to 360 degrees, between the signal at the MRP and its digital representation on the DAI.

d)	The frequency of the SS is increased to 1100 Hz and the resulting phase shift ∞2 shall be noted.

e)	The uplink coding delay TUC is calculated from either:

TDC = (∞2 - ∞1)/36 ms	for ∞2 > ∞1

or

TDC = (∞2 + 360 - ∞1)/36 ms	for ∞2 < ∞1

II.13.5.5	Requirement

1)	The sum {TDP + TDC + TUP + TUC} < 144.9 milliseconds.

Note 1:	This limit includes an allowance of 4*0.25 ms delay from the DAI to the MS transmission path.

Note 2:	No allowances have been made for any delays within the measurement system. These must either be calibrated out or subtracted from the individual delays before performing the sum above.