Appendix C : Test report

C.1	General

This appendix specifies the minimum contents of the test report that is to be issued by the accredited GSM test house as part of the certificate of conformance. It is separated in three main sections : Global, Test case and Summary.

All information required for the test report shall be storable/printable by the system simulator (SS). Some information though, has to be entered by the SS operator, either on a test case basis or globally.

C.2	Global section

Each test report shall contain the following information. Only information valid for all test cases shall be included in this section.

a)	Title and Date of issue.

Name of the report e.g. "GSM test report ... 03-10-90"

b)	Name, Address and Location of the laboratory.

c)	Unique identification of the report (such as serial number), each page and a summary of the total number of pages.

d)	Name and address of client.

e)	Description and identification of test item, including photographs.

f)	Test item reception date.

g)	Test specification identification e.g. "GSM 11.10" without version number.

h)	Test equipment identification.

i)	Audio test date reference.

j)	A statement limiting the validity of the test report to the tested item only.

k)	A statement that partial reproduction of the report is not permitted without a written approval from the issuing laboratory.

l)	A copy (or reference) to PICS and PIXIT (see GSM 11.10 Annex 3).

m)	Nature of conformance testing.

n)	A signature and title or an equivalent marking of person(s) accepting technical responsibility for the test report.

C.3	Test case section

The following information shall be provided for each test case. The information is not regarded as valid for any other test case unless otherwise stated.

C.3.1	RF and audio test cases

a)	A unique reference to GSM 11.10 including version number.

b)	A unique Mobile Equipment (ME) reference. This shall define any optional or test specific equipment connected to the "basis" ME during test, e.g. Booster, HF equipment, EMMI etc.

c)	Date and time.

d)	Operator ID.

e)	System simulator software version.

f)	Selected value of variable undefined in GSM 11.10. This may be globally stated for a number of test cases.

g)	Temperature and supply voltage (only included for tests that are, or can be, tested under extreme conditions).

h)	Absolute result, e.g. indicated value.

i)	Limits (without compensation for measurement uncertainty), i.e. design limits.

j)	Measurement uncertainty.

k)	Verdict (result inside or outside the limits mentioned in 3.1 i)).

C.3.2	Protocol test cases

a)	A unique reference to GSM 11.10 including version number.

b)	A unique ME reference. This shall define any optional or test specific equipment connected to the "basic" ME during test, e.g. Booster, HF equipment, EMMI etc.

c)	Date and time.

d)	Operator ID.

e)	System simulator software version.

f)	Selected value of variables undefined in GSM 11.10. This may be globally stated for a number of test cases.

g)	Actual result (display of resulting or message(s) content).

h)	Verdict.

C.4	Summary section

This shall be valid for all test cases covered by the global section.

a)	Schedule of tests (listing of all tests with marking for any omitted tests).

b)	Listing of test case where :

-	test results are outside the design limits,

-	test results are outside the design limits, but within the uncertainty.

c)	Observations such as borderline results, hazards and other issues outside testing but worthy mentioning.