11.	General tests

11.1	Verification of support and non-support of services.

(Multiple numbering scheme or ISDN)

11.1.1	Definition and applicability

11.1.2	Conformance Requirement

1.	The MS shall check the Information Elements for Bearer Capability and if any, the Information Elements for Lower and Higher Layer Compatibility in a received SETUP message, and if it agrees to the proposed set, it shall respond with a CALL CONFIRMED message.

GSM 04.08, 5.2.2.2; GSM 04.08, 5.2.2.3

2.	The MS in the "Null" state, U0, ready to receive a SETUP shall reject a SETUP with Information Elements for Bearer Capability or Lower or Higher Layer Compatibility, which are incompatible with the Bearer Services / Teleservices supported by the MS shall send a RELEASE COMPLETE message or ignore the SETUP message.

The preferred behaviour is the sending by the MS of a RELEASE COMPLETE message.

GSM 04.08, 5.2.2.2; GSM 04.08, 5.2.2.3.1; GSM 07.01, 6.3.1; GSM 04.08, B.3.2

11.1.3	Test Purpose

1.	To verify that the MS, for the case of the Multinumbering scheme/ISDN, accepts a SETUP message, where the Information Elements for Bearer Capability and Lower and Higher Layer Compatibility are compatible with the Bearer Services / Teleservices declared as supported by the MS, by sending a CALL CONFIRMED message.

This is verified for all Bearer Services/Teleservices described in GSM 07.01 and declared as supported by the MS. Information for Lower and Higher Layer Compatability are present if applicable to the services supported by the MS. For negotiable parameters only one value has to be used and that value must be supported by the MS.

2.	To verify that the MS in the "Null" state, U0, when receiving a SETUP messages containing incompatible Information Elements for Bearer Capability or Lower or Higher Layer Compatibility, will either ignore the message, or respond with a RELEASE COMPLETE message.

This is verified for two among the set of Bearer Capabilities and Lower or Higher Layer Compatability Information described in Annex II of GSM 07.01 and declared as non supported by the MS. This choice of two Bearer Capabilities together with Lower or Higher Layer Compatibility information is randomly performed by the test house.

11.1.4	Method of test

11.1.4.1	Initial conditions

For an MS with an external interface the interface shall be setup in such a way that the MS is able to successfully receive the call for the service in question. The manufacturer shall state how this is done in a PIXIT statement. The same applies to features which must be activated by MMI before an incoming call can be accepted.

The generic call set-up procedure shall be followed up to and including the reception of the CIPHERING MODE COMPLETE message from the MS.

11.1.4.2	Procedure

a)	The SS transmits SETUP.

Note:	The SETUP shall contain a single BC-IE with either speech or one of the possibilities defined in 07.01 Annex I. If applicable to the tested bearer service / teleservice the SETUP contains LLC-IE and HLC-IE.

b)	The test must be performed with a speech BC and with all the possible bearer service / teleservice described in 07.01 Annex II. For negotiable parameters only one value has to be used, and that value must be supported by the MS. Negotiation is not checked here.

11.1.5	Test Requirement

It is permitted for the MS to respond in one of three ways:

-	The MS shall send a RELEASE COMPLETE message with cause value 88 - incompatible destination. The MS is deemed not to support the service.

-	The MS shall send a CALL CONFIRMED message. The MS is deemed to support the service.

11.2	Verification of support of the single numbering scheme

11.2.1	Definition and applicability

11.2.2	Conformance Requirement

1.	The MS shall respond with a CALL CONFIRMED message including the single or multiple Bearer Capability, according to the actual configuration of the MS. Two bearer capabilities can be present only in the cases described in GSM 07.01.

GSM 04.08, 5.2.2.2; GSM 04.08, 5.2.2.3.1; GSM 04.08, 9.3.2; GSM 07.01, 6.3.3

2.	The Repeat Indicator Information Element shall be included in the CALL CONFIRMED message, when the in-call modification procedure is used, and no Bearer Capability Information Element is included in the received SETUP message.

GSM 04.08, 9.3.2

11.2.3	Test Purpose

1.	To verify that the MS, for the case of the Single Numbering Scheme, accepts a SETUP message, where the Information Elements for Bearer Capability and Lower and Higher Layer Compatibility are missing by sending a CALL CONFIRMED message, which includes the single or multiple Bearer Capabilities, according to the actual configuration on the MS.

This is verified for all Bearer Services / Teleservices described in GSM 07.01 and declared as supported by the MS.

2.	To verify that the MS includes a correctly encoded Repeat Indicator if it includes multiple Bearer Capabilities in the CALL CONFIRMED message.

11.2.4	Method of test

11.2.4.1	Initial conditions

If possible, the MS shall be configured to respond with a specified preferred BC, in the CALL CONFIRMED message, in reply to a SETUP message with no BC element. The manufacturer must state how this is done in a PIXIT statement. The same applies to features which must be activated by MMI before an incoming call can be accepted. If the MS can not be configured the test is performed with the MS as supplied.

The generic call set-up procedure shall be followed up to and including the reception of the CIPHERING MODE COMPLETE message from the MS.

11.2.4.2	Procedure

a)	The SS transmits SETUP.

Note:	The SETUP does not contain a BC element.

b)	The test is repeated with the MS configured for all possible preferred Bearer Capabilities, as declared in the PIXIT statement.

Note:	Negotiation of parameters is not checked in this test.

11.2.5	Test Requirement

The MS shall send a CALL CONFIRMED message, which shall contain the BC for which the MS is configured, if applicable. Otherwise the BC shall be among those supported by the MS and as declared by the manufacturer.

Where two BCs are contained in the CALL CONFIRMED message, it shall be checked that the combination is allowed, according to GSM 07.01 and that a REPEAT INDICATOR is also included.