14.	Receiver

In this section on receiver measurements, the procedures to test equipment which is fitted with a permanent antenna connector, and the procedures to test equipment which is designed to only be used with an integral antenna, are in general combined into one single test description.

Tests on Mobile Stations fitted with an integral antenna and having no means of connecting an external antenna are specified in terms of received field strength. In order to perform tests on such Mobile Stations without the need for separated access to a calibrated test site a temporary antenna connector is used as defined in General Conditions, GC7, of Annex 1 of part GC.

In practice the temporary antenna connector may be used for transmitter measurements described in section 3, but the calibration factors determined in [TBD] will not be directly usable. The detailed calibration, when needed, for transmission tests are described in the relevant sections of 3.

Testing philosophy

Certain assumptions concerning the functional mechanisms of GSM receivers have been made in order to define tests that will verify the receiver performance without excessive redundancy and excessive test times.

The receiver functions can be divided into:

-	Analogue RF and IF stages that are affected by input levels, temperature and power supply levels.

-	Demodulator that is affected by input levels and interfering signals.

-	Decoders that are affected by the different logical channels and input levels.

The tests are designed to stress each of these blocks with a minimum of redundancy.

Statistical testing of receiver BER/FER performance

Error Definition

1)	Frame Erasure Ratio (FER)

A frame is defined as erased if the error detection functions in the receiver, operating in accordance with GSM 05.03, indicate an error. For full rate speech this is the result of the 3 bit cyclic redundancy check (CRC) as well as other processing functions that cause a Bad Frame Indication (BFI). For signalling channels it is the result of the FIRE code or any other block code used. For data traffic FER is not defined.

2)	Residual Bit Error Ratio (RBER)

The Residual Bit Error Ratio is defined as the Bit Error Ratio (BER) in frames which have not been declared as erased.

3)	Bit Error Ratio (BER)

The Bit Error Ratio is defined as the ratio of the bits wrongly received to all data bits sent.

Test method

Each test is performed in the following manner:

a)	Set up the required test conditions.

b)	Perform the test for at least the minimum number of samples (frames, bits or bits from non erased frames) and record the number of offered samples and the number of occurred events (frame, bit or residual bit errors).

c)	Terminate the test and determine the test result ("pass" or "fail") by comparing the measured error rate against the test limit error rate.

It is permitted to run the test over more samples than the value stated for minimum number of samples. The effect of increasing the number of samples is always to give a higher probability that a good unit will pass and a lower risk that a bad unit will pass, according to the definitions of good and bad unit in this section.

Test criteria

The limits on number of samples and test limit error rate shall be defined in order to comply with different requirements :

1)	to keep reasonably low the risk of passing a bad unit for each individual test;

2)	to have high probability of passing a good unit for each individual test;

3)	to perform measurements with a high degree of statistical significance;

4)	to keep the test time as low as possible.

The risk of passing a bad unit (point 1) should be kept lower than 0.2%. A unit is generally considered "bad" if its BER (or FER) performance is 1.5 times worse than that specified in AWGN (Additive White Gaussian Noise) and 1.26 times worse than that specified in multipath environment. These values have been adopted (taking into account the expected shapes of the BER performance) in order not to pass a unit with performance worse than the specifications by more than 1 dB.

The probability of passing a good unit operating on the specification limit of performance (point 2) should be at least 99.7%.

If the error events can be assumed to be random independent variables, outputs of stationary random processes with identical Gaussian distributions, the previous figures suggest a number of events (point 3) not lower than 200 in AWGN channel and not lower than 600 in a multipath environment, and to test a BER (or FER) performance 1.22 times worse than that specified in AWGN and 1.12 times worse than that specified in a multipath environment (this corresponds to testing a performance, at the most, 0.5 dB worse than that specified).

For multipath propagation conditions the hypothesis of stationary random processes does not generally hold. In case of non frequency hopping operation mode, the radio channel may be assumed to change 10 times per wavelength of travelled distance and to be short term stationary in between. So, in this case, the required observation time for having good statistical properties should not be lower (with some rounding) than that reported in [table 14-1].

Table 14-1: Minimum test time according to propogation profile

GSM900	DCS1800

Propagation	TUlow	TUhigh	HT	RA	TUlow	TUhigh	HT	RA

Conditions

Min. test	500	30	15	6	500	15	7.5	6

time (s)

[Tables 14-2 and 14-3] detail, for the different test conditions, the minimum number of samples required in order to meet points 1) to 3): the corresponding test time (point 4) can be consequently computed.

As can be seen in the tables, in some of the cases in which both FER and RBER have to be tested on the same channel, the length of time for the FER measurement has been adopted for the RBER measurement. This is longer than that required for the RBER only according to the discussed criteria, but allows the use of a test limit error rate closer to the specified error rate while maintaining the same statistical significance. When, as is normal, it is desired to perform the FER and RBER tests, the closer test limit error rate for the RBER measurement can be achieved without increasing the total test time. It is always possible to extend the length of any test and further improve the statistical significance of that test.

Table 14-2: Test conditions for GSM 900

Type of	Type of	Propagation/	Specified	Test limit	Minimum No	Prob that	Bad unit	Risk that

test	channel	frequency	FER/BER	FER/BER	of	good unit	BER/FER	bad unit

conditions	%	%	samples	will pass	%	will pass

BFI	TCH/FS	Static	0.200	0.244	82000	99.719	0.300	0.139

Sensitivity	TCH/FS	Static/FH	0.100*a	0.122*a	164000	99.717	0.150*a	0.140

,,	TCH/FS Class Ib	Static/FH	0.400/a	0.410/a	20000000	100.000	0.600/a	<0.001

,,	TCH/FS Class II	Static/FH	2.000	2.439	8200	99.714	3.000	0.001

,,	TCH/FS	TUhigh/No FH	6.000*a	6.742*a	8900	99.825	7.560*a	0.162

,,	TCH/FS Class Ib	TUhigh/No FH	0.400/a	0.420/a	1000000	99.919	0.504/a	<0.001

,,	TCH/FS Class II	TUhigh/No FH	8.000	8.333	120000	99.999	10.080	<0.001

,,	TCH/FS Class II	HT/No FH	9.000	9.333	60000	99.779	11.340	<0.001

,,	TCH/FS Class II	RA/No FH	7.000	7.500	24000	99.873	8.694	<0.001

,,	FACCH	TUhigh/No FH	8.000	8.961	6696	99.798	10.080	0.108

,,	TCH/F9.6&H4.8	HT/No FH	0.700	0.778	180000	99.995	0.882	<0.001

,,	TCH/F4.8	HT/No FH	0.010	0.011	5350000	99.732	0.013	0.197

,,	TCH/F2.4	HT/No FH	0.001	0.001	11900000	99.734	0.002	<0.001

,,	TCH/H2.4	HT/No FH	0.010	0.011	5350000	99.732	0.013	0.197

Input level	TCH/FS Class II	Static<-40dBm	0.010	0.012	1640000	99.716	0.015	0.141

Input level	TCH/FS Class II	Static<-15dBm	0.100	0.122	164000	99.717	0.150	0.140

range	TCH/FS Class II	EQ	3.000	3.250	120000	100.000	3.780	<0.001

Co-channel	TCH/FS	TUlow/No FH	21.000*a	24.000*a	25000	100.000	27.720*a	<0.001

rejection	TCH/FS Class Ib	TUlow/No FH	2.000/a	2.091/a	3300000	100.000	2.520/a	<0.001

,,	TCH/FS Class II	TUlow/NoFH	4.000	4.300	2000000	100.000	5.040	<0.001

,,	TCH/FS	TUlow/FH	3.000*a	3.371*a	17800	99.797	3.780*a	0.194

,,	TCH/FS Class Ib	TUlow/FH	0.200/a	0.215/a	2000000	100.000	0.252/a	<0.001

,,	TCH/FS Class II	TUlow/FH	8.000	8.333	1200000	100.000	10.080	<0.001

,,	FACCH	TUlow/No FH	22.000	24.000	25000	100.000	27.720	<0.001

,,	TCH/F9.6 or H4.8	TUlow/FH	0.300	0.336	178500	99.716	0.378	0.180

,,	TCH/F4.8	TUlow/FH	0.010	0.011	5350000	99.732	0.013	0.197

,,	TCH/F2.4	TUlow/FH	0.001	0.001	11900000	99.734	0.002	<0.001

,,	TCH/H2.4	TUlow/FH	0.010	0.011	5350000	99.732	0.013	0.197

Adjacent	TCH/FS	TUhigh/No FH	6.000*a	6.742*a	8900	99.825	7.560*a	0.162

channel	TCH/FS Class Ib	TUhigh/No FH	0.400/a	0.420/a	1000000	99.919	0.504/a	<0.001

200 kHz	TCH/FS Class II	TUhigh/No FH	8.000	8.333	600000	100.000	10.080	<0.001

,,	FACCH	TUhigh/No FH	9.500	10.640	5639	99.812	11.970	0.096

Adjacent	TCH/FS	TUhigh/No FH	10.200*a	11.461*a	8900	99.995	12.852*a	0.004

channel	TCH/FS Class Ib	TUhigh/No FH	0.720/a	0.756/a	1000000	99.999	0.9077/a	<0.001

400 kHz	TCH/FS Class II	TUhigh/No FH	8.800	9.167	600000	100.000	11.088	<0.001

,,	FACCH	TUhigh/No FH	17.100	19.152	3133	99.878	21.546	<0.052

Intermod.	TCH/FS Class II	Static	2.000	2.439	8200	99.741	3.000	0.122

FACCH	TUhigh/No FH	8.000	8.961	6696	99.798	10.080	0.108

Blocking &	TCH/FS Class II	Static	2.000	2.439	8200	99.741	4.000	<0.001

spurious	FACCH	TUhigh/No FH	8.000	8.961	6696	99..798	10.080	0.108

resp.

Table 14-3: Test conditions for DCS1800

Type of	Type of	Propagation/	Specified	Test limit	Minimum No	Prob that	Bad unit	Risk that

test	channel	Frequency	FER/BER	FER/BER	of	good unit	BER/FER	bad unit

conditions	%	%	samples	will pass	%	will pass

BFI	TCH/FS	Static	0.200	0.244	82000	99.719	0.300	0.139

Sensitivity	TCH/FS	Static/FH	0.100*a	0.122*a	164000	99.717	0.150*a	0.140

,,	TCH/FS Class Ib	Static/FH	0.400/a	0.410/a	20000000	100.000	0.600/a	<0.001

,,	TCH/FS Class II	Static/FH	2.000	2.439	8200	99.714	3.000	0.001

,,	TCH/FS	TUhigh/No FH	4.000*a	4.478*a	13400	99.743	5.040*a	0.133

,,	TCH/FS Class Ib	TUhigh/No FH	0.300/a	0.320/a	1500000	100.000	0.378/a	<0.001

,,	TCH/FS Class II	TUhigh/No FH	8.000	8.333	60000	99.865	10.080	<0.001

,,	TCH/FS Class II	HT/No FH	9.000	9.333	30000	97.826	11.340	<0.001

,,	TCH/FS Class II	RA/No FH	7.000	7.500	24000	99.873	8.820	<0.001

,,	FACCH	TUhigh/No FH	3.900	4.368	13736	99.752	4.914	0.140

,,	TCH/F9.6	HT/No FH	0.700	0.784	76500	99.721	0.882	0.176

,,	TCH/F4.8	HT/No FH	0.010	0.011	5350000	99.732	0.013	0.197

,,	TCH/F2.4	HT/No FH	0.001	0.001	11900000	99.734	0.002	<0.001

Input level	TCH/FS Class II	Static-23dBm	0.100	0.122	164000	99.717	0.150	0.140

range	TCH/FS Class II	Static<-40dBm	0.010	0.012	1640000	99.716	0.015	0.141

TCH/FS Class II	EQ	3.000	3.250	60000	99.981	3.780	<0.001

Co-channel	TCH/FS	TUlow/No FH	21.00*a	24.00*a	25000	100.000	26.460*a	<0.001

rejection	TCH/FS Class Ib	TUlow/No FH	2.000/a	2.091/a	3300000	100.000	2.520/a	<0.001

,,	TCH/FS Class II	TUlow/No FH	4.000	4.300	2000000	100.000	5.040	<0.001

,,	TCH/FS	TUlow/FH	3.000*a	3.371*a	17800	99.797	3.780*a	0.194

,,	TCH/FS Class Ib	TUlow/FH	0.200/a	0.215/a	2000000	100.000	0.252/a	<0.001

,,	TCH/FS Class II	TUlow/FH	8.000	8.333	1200000	100.000	10.080	<0.001

,,	FACCH	TUlow/No FH	22.000	24.000	25000	100.000	27.720	<0.001

,,	TCH/F9.6 or H4.8	TUlow/FH	0.300	0.336	178500	99.716	0.378	0.180

,,	TCH/F4.8	TUlow/FH	0.010	0.011	5350000	99.732	0.013	0.197

,,	TCH/F2.4	TUlow/FH	0.001	0.001	11900000	99.734	0.002	<0.001

,,	TCH.H2.4	TUlow/FH	0.010	0.011	5350000	99.732	0.013	0.197

Adjacent	TCH/FS	TUhigh/No FH	3.000*a	3.371*a	17800	99.797	3.780*a	0.194

channel	TCH/FS Class Ib	TUhigh/No FH	0.250/a	0.270/a	2000000	100.000	0.315/a	<0.001

200 kHz	TCH/FS Class II	TUhigh/No FH	8.100	8.333	1200000	100.000	10.206	<0.001

,,	FACCH	TUhigh/No FH	3.400	3.808	15756	99.746	4.284	0.145

Adjacent	TCH/FS	TUhigh/No FH	5.100*a	5.714*a	10500	99.773	6.426*a	0.134

channel	TCH/FS Class Ib	TUhigh/No FH	0.450/a	0.483/a	1200000	100.000	0.567/a	<0.001

400 kHz	TCH/FS Class II	TUhigh/No FH	8.900	9.167	720000	100.000	11.214	<0.001

,,	FACCH	TUhigh/No FH	6.100	6.832	8782	99.777	7.686	0.122

Intermod.	TCH/FS Class II	Static	2.000	2.439	8200	99.741	3.000	0.122

FACCH	TUhigh/No FH	3.900	4.368	13736	99.752	4.914	0.140

Blocking &	TCH/FS Class II	Static	2.000	2.439	8200	99.741	4.000	<0.001

spurious	FACCH	TUhigh/No FH	3.900	4.368	13736	99.752	4.914	0.140

resp.

Note 1	a is a parameter which ranges from 1 to 1.6. The value of a for a RBER test on TCH/FS class Ib bits under particular measurement conditions shall be the same as that determined in the FER test on TCH/FS under the same conditions. For example, the value of a may be different for a TUhigh sensitivity test and an RA sensitivity test. The value of a is determined by dividing the measured error rate for the FER test by the value of the test limit error rate listed in the limits section of the test corresponding to a=1; if the result of the division is lower than 1, a value of a=1 shall be used, if the value of a > 1.6 the FER test has failed (the normal treatment of stimulus uncertainties applies). The probabilities that a good unit will pass and the risks that a bad unit will pass, listed in the table are valid for a=1, and would be slightly different for other values of a.

Note 2:	In order to save time the sensitivity and co-channel rejection tests for the TCH/F2.4 channel does not comply with the above said constraints.

In fact, a bad unit which performs 2 times (instead of 1.26) worse than that specified is accounted for, so reducing the required number of events to 150, instead of 600. On the other hand, the specified RBER is in this case 10E-5 and, on the basis of simulations and hardware validation results, doubling this RBER results in a drop in performance of less than 1 dB.

14.1	Bad frame indication

14.1.1	Definition and applicability

The performance of the bad frame indication (BFI) is a measure of the effectiveness of the MS under DTX conditions. It includes the effect of the 3 bit Cyclic Redundancy Check (CRC) and all other processing associated with the DTX function. The BFI is measured on a full rate speech TCH (TCH/FS) by counting the number of undetected bad frames whilst the input signal is a randomly modulated carrier.

The requirements and this test  only apply to MS supporting speech.

14.1.2	Conformance requirement

On a full rate speech TCH (TCH/FS) with a random RF input, the overall reception performance shall be such that, on average, less than one undetected bad speech frame (false bad frame indication) in 10 seconds will be measured; GSM 05.05 6.4 b

14.1.3	Test purpose

To verify that the Bad Frame Indication performance does not exceed the conformance requirement with an allowance for the statistical significance of the test.

14.1.4	Method of test

14.1.4.1	Initial conditions

A call is set up according to the generic call set up procedure on a TCH/FS with an ARFCN in the Mid ARFCN range, power control level set to maximum power.

The SS commands the MS to complete the traffic channel loop back and signal the bad frame indication.

Note:	DTX is used during the test to prevent the MS dropping the call.

14.1.4.2	Procedure

a)	The SS simulates a BSS in DTX mode. During the period when no transmission would occur the SS transmits a GSM carrier modulated with random data at a level 11 dB above reference sensitivity level(  ). The SACCH is transmitted normally at a level 20 dB above reference sensitivity(  ). The SID frame is transmitted in its correct time interval with valid information at a level 20 dB above reference sensitivity level(  ). During transmission of SACCH or SID frames the random data is discontinued.

b)	The SS transmits at least the minimum number of samples of frames of TCH/FS information and checks the bad frame indication (BFI) of the looped back signal from the MS. The SS records the number of frames where the bad frame indication is not set. During transmission by the SS of SID frames the SS checks that the BFI is not set.

Note:	Further explanations on the mechanism of signalling the BFI to the SS will be found in sections III.1.1 and III.1.3.

14.1.5	Test requirements

The BFI performance is accepted if the measured rate of undetected bad frames does not exceed the test limit error rate:

Test limit error rate -		0.244%

Minimum number of samples -	82000	(excluding SID frames).

During loop back of SID frames no BFI shall be set.

14.2	Reference sensitivity

14.2.1	Reference sensitivity - speech channels

14.2.1.1	Definition and applicability

The reference sensitivity is the signal level at the MS receiver input at which a certain BER and FER must be achieved.

The requirements and this test apply to MS supporting speech.

14.2.1.2	Conformance requirement

1.	At reference sensitivity level, the TCH/FS FER shall meet the reference sensitivity performance of table 1 in GSM 05.05; GSM 05.05 6.2

2	At reference sensitivity level, the TCH/FS class I RBER shall meet the reference sensitivity performance of table 1 in GSM 05.05; GSM 05.05 6.2

3	At reference sensitivity level, the TCH/FS class II RBER shall meet the reference sensitivity, performance of table 1 in GSM 05.05: GSM 05.05, 6.2

4.	At reference sensitivity level, the TCH/FS class II RBER shall meet the reference sensitivity, performance of table 1 in GSM under extreme conditions; GSM 05.05, 6.2, GSM 05.05, Annex D D.2.1, D.2.2.

14.2.1.3	Test purpose

1.	To verify that the MS does not exceed conformance requirement 1 under STATIC and TUhigh propogation conditions with an allowance for the statistical significance of the test

2.	To verify that the MS does not exceed conformance requirement 2 under STATIC and TUhigh propagation conditions shall meetwith an allowance for the statistical significance of the test

3.	To verify that the MS does not exceed conformance requirement  3 under RA and HT propogation conditions with an allowance for the statistical significance of the test:

4.	To verify that the MS does not exceed conformance requirement  4 under STATIC and TUhigh propogation conditions with an allowance for the statistical significance of the test:

14.2.1.4	Method of test

Note 1:	The BA list sent on the BCCH and SACCH will indicate at least six surrounding cells with at least one near to each band edge. It is not necessary to generate any of these BCCHs but, if provided the signal strengths of BCCHs shall be in the range 15 dBmVemf(  ) to 35 dBmVemf(  ).

Note 2:	The ARFCN of any BCCH shall not be co-channel or on adjacent channels to the wanted traffic channel.

Note 3:	When frequency hopping is used, the traffic channel may fall on any of the ARFCNs defined in Annex 1.

14.2.1.4.1	Initial conditions

A call is set up according to the generic call set up procedure on a TCH/FS with an ARFCN in the Mid ARFCN range, power control level set to maximum power.

The SS transmits Standard Test Signal C1 on the traffic channel.

The SS commands the MS to create  traffic channel loop back signalling erased frames.

14.2.1.4.2	Procedure

a)	The fading function is set to TUhigh.

b)	the SS sets amplitude of the wanted signal to reference sensitivity level ( ).

c)	The SS compares the data of the signal that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication.

d)	The SS determines the number of residual bit error events for the bits of class II, by examining sequences of at least the minimum number of samples of consecutive bits of class II. Bits are taken only from those frames not signalled as erased.

e)	The SS determines the number of residual bit error events for the bits of the class Ib, by examining sequences of at least the minimum number of samples of consecutive bits of class Ib. Bits are only taken from those frames not signalled as erased

f)	The SS also determines the frame erasure events by examining sequences of at least the minimum number of samples of consecutive frames and assuming a frame is received successfully it is not signalled as erased.

g)	Steps a) to d) are repeated under extreme test conditions.

h)	Steps a) to g) are repeated for TCH/FS with ARFCNs in the Low ARFCN range and the High ARFCN range .

i)	Steps b) to d) are repeated with the SS fading function  set in turn to RA and HT.

j)	Steps b) to g) are repeated,  with the SS fading function  set to static and the MS is commanded by the SS into hopping mode using the hopping sequence of GC1 of Annex 1, full band.

The amplitude of the wanted signal is set according to step b). All the other time slots, except the active ones, are set to 20 dB above reference sensitivity level(  ). This implicitly tests adjacent time slot rejection.

14.2.1.5	Test requirements

The error rates measured for different channels and under the different propagation conditions, under any combination of normal and extreme test voltages and ambient temperatures, shall not exceed the test limit error rate values given in table 14-4 or 14-5:

Table 14-4: Limits for GSM 900 sensitivity

Channels	Propagation	Propagation	Propagation	Static

conditions	conditions	conditions	conditions

TUhigh	RA	HT

Test limit	Minimum	Test limit	Minimum	Test limit	Minimum	Test limit	Minimum

error	No. of	error	No. of	error	No. of	error	No. of

rate %	samples	rate %	samples	rate %	samples	rate %	samples

TCH/FS

FER	6.742*a	8900					0.122*a	164000

class Ib(RBER)	0.42/a	1000000					0.41/a	20000000

class II(RBER)	8.33	120000	7.5	24000	9.333	60000	2.439	8200

where a is a parameter which can range from 1 to 1.6. The value of a for a RBER test on TCH/FS class Ib bits under particular measurement conditions shall be the same as that determined in the FER test on TCH/FS under the same conditions.

Table 14-5: Limits for DCS 1800 sensitivity

Channels	Propagation	Propagation	Propagation	Static

conditions	conditions	conditions	conditions

TUhigh	RA	HT

Test limit	Minimum	Test limit	Minimum	Test limit	Minimum	Test limit	Minimum

error	No. of	error	No. of	error	No. of	error	No. of

rate %	samples	rate %	samples	rate %	samples	rate %	samples

TCH/FS

FER	4.478*a	13400					0.122*a	164000

class Ib(RBER)	0.32/a	1500000					0.41/a	20000000

class II(RBER)	8.333	60000	7.5	24000	9.333	30000	2.439	8200

where a is a parameter which can range from 1 to 1.6. The value of a for a RBER test on TCH/FS class Ib bits under particular measurement conditions shall be the same as that determined in the FER test on TCH/FS under the same conditions.

14.2.2.	Reference sensitivity  - control channels

14.2.2.1	Definition and applicability

The reference sensitivity for control channels is the signal level at the MS receiver input at which a certain FER must be achieved.

The requirements and this test apply to all types of MS.

14.2.2.2	Conformance requirement.

At reference sensitivity level, the FACCH FER shall meet the reference sensitivity performance of table 1 in GSM 05.05; (GSM 05.05 6.2)

14.2.2.3	Test purpose.

To verify that the MS does not exceed the conformance requirement  under TUhigh propogation condition with an allowance for the statistical significance of the test

14.2.2.4	Method of test

14.2.2.4.1	Initial conditions

A call is set up according to the generic call set up procedure on a TCH with an ARFCN in the Mid ARFCN range, power control level set to maximum power.

The SS transmits Standard Test Signal C1 on the traffic channel.

14.2.2.4.2	Procedure

a)	The fading function is set to TUhigh.

b)	The SS sets the amplitude of the wanted signal to reference sensitivity level ( ).

c)	The SS sends the STATUS message. Due to the low signal level the MS may not be able to acknowledge the Layer 2 frame with an RR frame and the SS will repeat the Layer 2 frame. Each repeated L2 frame will be counted and will indicate a frame erasure event.

d)	The SS determines the frame erasure events during at least the minimum number of samples of FACCH frames.

Note:	These frames will not be consecutive but it is expected that the statistical significance of the tests will not be unduly degraded.

14.2.2.5	Test Requirements

The error rates measured shall not exceed the test limit error rate values given in table 14-6.

Table 14-6: Limits for FACCH sensitivity

GSM900	DCS1800

Channels	Type of	Propag-	Test limit	Minimum No of	Test limit	Minimum No of

measure-	ation	error rate %	samples	error rate %	samples

ments

FACCH	FER	TUhigh	8.961	6696	4.368	13736

14.2.3	Reference sensitivity - data channels

14.2.3.1	Definition and applicability

The reference sensitivity for data channels is the signal level at the MS receiver input at which a certain BER must be achieved.

The requirements and this test apply to MS supporting data.

14.2.3.2	Conformance Requirement.

1.	At reference sensitivity level, the TCH/F9.6, TCH/F4.8 and TCH/F2.4 BER shall meet the reference sensitivity performance of table 1 in GSM 05.05; (GSM 05.05, 6.2)

2.	At reference sensitivity level, the TCH/H4.8 and TCH/H2.4 BER  shall meet the reference sensitivity performance of table 1 in GSM 05.05; (GSM 05.05, 6.2)

14.2.3.3	Test purpose

1.	To verify that the MS does not exceed conformance requirement 1 under HT propogation condition with an allowance for the statistical significance of the test

2.	To verify that the MS does not exceed conformance requirement 2 under HT propogation conditionwith an allowance for the statistical significance of the test

14.2.3.4	Method of Test

14.2.3.4.1	Initial conditions

A call is  set up according to the generic call set up procedure on a TCH with an ARFCN in the Mid ARFCN range. One of the supported TCH/ (F9.6, H4.8, F4.8, F2.4 of H2.4) shall be used. The power control level is set to maximum power.

The SS transmits Standard Test Signal C1 on the traffic channel.

The SS commands the MS to create the traffic channel loop back signalling erased frames. (III.1.2.1.1.1)

14.2.3.4.2	Procedure

a)	The fading function is set to HT.

b)	The SS sets the amplitude of the wanted signal level to reference sensitivity level ( )

c)	The SS compares transmitted data with received data for at least the minimum number of samples of consecutive bits and records every error bit as an error event.

d)	The SS sets the wanted signal level to 28 dBmVemf.

e)	The SS commands the MS to open the TCH loop.

f)	The SS commands the MS to another of the supported data channels.

g)	Steps b) to f) are repeated for all supported data channels.

14.2.3.5	Test requirements

The Max-events measured for different channels shall not exceed the values given in the table 14-7.

Table 14-7: Limits for data channel sensitivity

GSM900	DCS1800

Channels	Type of	Propag-	Test limit	Minimum No of	Test limit	Minimum No of

measure-	ation	error rate %	samples	error rate %	samples

ments

TCH/F9.6	BER	HT	0.778	180000	0.784	76500

TCH/H4.8	BER	HT	0.778	180000	-	-

TCH/F4.8	BER	HT	0.011	5350000	0.011	5350000

TCH/F2.4	BER	HT	0.001	11900000	0.001	11900000

TCH/H2.4	BER	HT	0.011	5350000	-	-

14.3	Usable receiver input level range

14.3.1	Definition and applicability

The usable receiver input level range is the range of the radio frequency input level of a specified modulated signal over which bit error ratio or frame erasure ratios stay between specified limits.

The requirements and this test apply to MS supporting speech.

14.3.2	Conformance requirement

1.	The receiver input level range requirements of GSM 05.05 section 6.1 for TCH/FS class II RBER under static and EQ propagation conditions shall be met:

1.1	Under normal conditions.

1.2	Under extreme conditions.

14.3.3	Test purpose

1.	To verify that the MS does not exceed the conformance requirement with an allowance for the statistical significance of the test.

1.1	Under normal conditions

1.2	Under extreme conditions

14.3.4	Method of test

14.3.4.1	Initial conditions

A call is set up according to the generic call set up procedure on a TCH/FS with an ARFCN in the Mid ARFCN range, power control level set to maximum.

The SS transmits Standard Test Signal C1 on the TCH/FS.

The SS commands the MS to create traffic channel loop back signalling erased frames. (III.1.2.1.1.2).

14.3.4.2	Procedure

a)	The SS compares the data that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding.

The SS tests the bit error ratio for the non-protected bits of TCH/FS class II, by examining sequences of at least the minimum number of samples of consecutive bits of class II. The number of error events is recorded.

b)	Step a) is repeated with the amplitude of the wanted signal increased to an input level at the receiver input of 73 dBmVemf(  )

c)	Step a) is repeated with the amplitude of the wanted signal increased to an input level at the receiver input of

GSM900:	98 dBmVemf(  )

DCS1800:	90 dBmVemf(  ).

d)	The SS fading function is set to EQ.

e)	Step a) is repeated with the amplitude of the wanted signal set to respectively 20 dB above reference sensitivity level(  ) and 73dBmVemf(  ) at the receiver input.

f)	The test is repeated under extreme test conditions.

14.3.5	Test Requirements

The error rate measured in this test shall not exceed the test limit error rate values given in table 14-8. This shall apply for any combination of normal and extreme test voltages and ambient temperature, for the different propagation conditions and for any level of input signal to the receiver.

Table 14-8: Limits for input level range

GSM900	DCS1800

Test		Test

limit		limit

Propagation	error	Minimum No.	error	Minimum No

conditions	rate %	of samples	rate %	of samples

Static <= 73 dBmVemf( )	0.012	1640000	0.012	1640000

Static 98 dBmVemf( )	0.122	164000

Static 90 dBmVemf( )			0.122	164000

EQ	3.25	120000	3.25	60000

14.4	Co-channel rejection

14.4.1	Co-channel rejection - speech channels.

14.4.1.1	Definition and applicability

The co-channel rejection is a measure of the capability of the receiver to receive a wanted modulated signal without exceeding a given degradation due to the presence of an unwanted modulated signal, both signals being at the nominal frequency of the receiver.

The requirements and this test apply to MS supporting speech.

14.4.1.2	Conformance requirement

1.	At reference cochannel interference the TCH/FS FER shall meet the reference interference performance of table 2 in GSM 05.05; GSM 05.05, 6.3

2.	At reference cochannel interference the TCH/FS class Ib BER shall meet the reference interference performance of table 2 in GSM 05.05; GSM 05.05, 6.3

3.	At reference cochannel interference the TCH/FS class II BER shall meet the reference interference performance of table 2 in GSM 05.05; GSM 05.05, 6.3

14.4.1.3	Test purpose

1.	To verify that the MS does not exceed conformance requirement 1 under propagation condition TUlow with frequency hopping and no frequency hopping with an allowance for the statistical significance of the test.

2.	To verify that the MS does not exceed conformance requirement 2 under propagation condition TUlow with frequency hopping and no frequency hopping, with an allowance for the statistical significance of the test.

3.	To verify that the MS does not exceed conformance requirement  3 under propagation condition TUlow with frequency hopping and no frequency hopping with an allowance for the statistical significance of the test.

14.4.1.4	Method of test

14.4.1.2.1	Initial conditions

A call is set up according to the generic call set up procedure on a TCH/FS with an ARFCN in the mid ARFCN range, power control level set to maximum power.

The SS transmits Standard Test Signal C1 on the traffic channel (wanted signal).

The SS commands the MS to create the traffic channel loop back, signalling erased frames. (III.1.2.1.1.1)

14.4.1.2.2	Procedure

a)	In addition to the wanted signal, the SS  produces  an independent, uncorrelated interfering  signal, Standard Test Signal I1 (unwanted signal).

The unwanted signal is continuous and has no fixed relationship with the bit transitions of the wanted signal. Its amplitude is 9 dB below that of the wanted signal.

The fading characteristic of the wanted and the interfering signal is TUlow.

b)	The SS compares the modulation of the signal that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication.

c)	The SS determines the number of residual bit error events for the bits of class II, by examining at least the minimum number of samples of consecutive bits of class II. Bits are taken only from those frames not signalled as erased.

d)	The SS determines the number of residual bit error events for the bits of the class Ib, by examining at least the minimum number of samples of consecutive bits of class Ib. Bits are only taken from those frames not signalled as erased.

e)	The SS also determines the frame erasure events by examining at least the minimum number of samples of consecutive frames and assuming a frame is received successfully it is not signalled as erased.

f)	Steps a) to e) are repeated except that in step a) both the wanted and interfering signal are hopping and the SS commands the MS into hopping mode. A hop pattern covering at least 10 frequencies in a range not exceeding 5 MHz is used

The hopping band is centred around an ARFCN in the Mid ARFCN range (ref Table 1.4) according to the narrow band hop of GC1 of Annex 1.

14.4.1.5	Test requirements

The error rate measured in this test shall not exceed the test limit error rate values given in table 14-9.

Table 14-9: Limits for co-channel rejection

Channel	Type of	Propagation	Test	Minimum No.

measurement	condition	limit	of samples

error rate %

TCH/FS	FER	TUlow/No FH		24*a	25000

TCH/FS	RBER	TUlow/No FH		2.091/a	3300000

Class Ib

TCH/FS	RBER	TUlow/No FH		4.3	2000000

Class II

TCH/FS	FER	TUlow/FH		3.371*a	17800

TCH/FS	RBER	TUlow/FH		0.215/a	2000000

class Ib

TCH/FS	RBER	TUlow/FH		8.333	1200000

class II

The parameter a can range from 1 to 1.6. The value of a for the RBER test on TCH/FS class Ib bits under particular measurement conditions shall be the same as that determined in the FER test on TCH/FS under the same conditions.

14.4.2	Co-channel rejection - control channels

14.4.2.1	Definition and applicability

The co-channel rejection is a measure of the capability of the receiver to receive a wanted modulated signal without exceeding a given degradation due to the presence of an unwanted modulated signal, both signals being at the nominal frequency of the receiver.

The requirements and this test apply to all types of MS

14.4.2.2	Conformance requirement

At reference cochannel interference the FACCH FER shall meet the reference interference performance of table 2 in GSM 05.05; (GSM 05.05, 6.3)

14.4.2.3	Test purpose

To verify that the MS does not exceed the conformance requirement under propagation condition TUlow with an allowance for the statistical significance of the test.

14.4.2.4	Method of test

14.4.2.4.1	Initial conditions

A call is set up according to the generic call set up procedure on a TCH with an ARFCN in the mid ARFCN range. For MS supporting speech this shall be a TCH/FS. For MS not supporting speech one of the supported TCH/ (F9.6, F4.8, or F2.4) shall be used. The power control level is set to maximum power.

The SS transmits Standard Test Signal C1 on the traffic channel (wanted signal).

14.4.2.4.2	Procedure

a)	In addition to the wanted signal, the SS produces an independent, uncorrelated interfering signal, Standard Test Signal I1 (unwanted signal).

The unwanted signal is continuous and has no fixed relationship with the bit transitions of the wanted signal. Its amplitude is 9 dB below that of the wanted signal.

The fading characteristic of the wanted and the interfering signal is TUlow.

d)	The SS sends the STATUS message. Due to the co-channel interference, the MS may not be able to acknowledge the Layer 2 frame. Each repeated L2 frame indicates a frame erasure event. The SS determines the number of frame erasure events during at least the minimum number of samples of FACCH frames.

Note:	These frames will not be consecutive but it is expected that the statistical significance of the tests will not be unduly degraded.

14.4.2.5	Test requirements

The error rate measuured in this test shall not exceed the test limit error rate values given in the table 14-10.

Table 14-10: Limits for co-channel rejection.

Channel	Type of	Propagation	Test	Minimum No.

measurement	condition	limit	of samples

error rate %

FACCH	FER	TUlow/No FH	24	25000

14.5	Adjacent channel rejection

14.5.1	Adjacent channel rejection - speech channels

14.5.1.1	Definition and applicability

The adjacent channel selectivity is a measure of the capability of the receiver to receive a wanted modulated signal without exceeding a given degradation due to the presence of an unwanted signal in the adjacent channel.

The adjacent channel can be adjacent in the RF spectrum or in time. There are therefore two types of adjacent channel selectivity:

1)	Adjacent RF channel selectivity which is specifically tested in this section.

2)	Adjacent Time Slot selectivity, which is implicitly tested in test .2.1.

The requirements and this test apply to MS supporting speech.

14.5.1.2	Conformance requirement

1.	With adjacent channel interference at 200 kHz above and below the wanted signal and signal level 9 dB above the wanted signal level:

1.1	For a TUhigh faded wanted signal and a TUhigh adjacent channel interferer, the FER for TCH/FS shall be within the requirements of table 2 in GSM 05.05; GSM 05.05, 6.3

1.2	For a TUhigh faded wanted signal and a TUhigh adjacent channel interferer,the Class Ib RBER shall be within the requirements of table 2 in GSM 05.05; GSM 05.05, 6.3

1.3	For a TUhigh faded wanted signal and a TUhigh adjacent channel interferer, the Class II RBER shall be within the requirements of table 2 in GSM 05.05; GSM 05.05, 6.3

1.4	For a TUhigh faded wanted signal and a TUhigh adjacent channel interferer, the Class II RBER shall be within the requirements of table 2 in GSM 05.05  under extreme test conditions; GSM 05.05, 6.3, GSM 05.05, Annex D D.2.1, D.2.2

2.	For adjacent channel interference at 400 kHz above and below the wanted signal frequency and signal level 41 dB above the wanted signal level:

2.1	For a TUhigh faded wanted signal and a static adjacent channel interferer, the FER for TCH/FS shall be better than:

GSM 900:	10.2*a %; GSM 05.05, 6.3

DCS1800:	 5.1*a %; GSM 05.05, 6.3

2.2	For a TUhigh faded wanted signal and a static adjacent channel interferer, the Class Ib RBER shall be better than:

GSM 900:	0.72/a %; GSM 05.05, 6.3

DCS1800:	0.45/a %; GSM 05.05, 6.3

2.3	For a TUhigh faded wanted signal and a static adjacent channel interferer, the Class II RBER shall be better than:

GSM 900:	8.8%; GSM 05.05, 6.3

DCS1800:	8.9%; GSM 05.05, 6.3

2.4	For a TUhigh faded wanted signal and a static adjacent channel interferer, the Class II RBER shall be better than:

GSM 900:	8.8%;

DCS1800:	8.9%;

under extreme test conditions; GSM 05.05, 6.3, Annex D D.2.1, D.2.2

14.5.1.3	Test purpose

1	To verify that with a TUhigh adjacent channel interferer at 200 kHz above and below the wanted TUhigh signal frequency and signal level 9 dB above the wanted signal level:

1.1	Conformance requirement 1.1 is met with an allowance for the statistical significance of the test.

1.2	Conformance requirement 1.2 is met with an allowance for the statistical significance of the test.

1.3	Conformance requirement 1.3 is met with an allowance for the statistical significance of the test.

1.4	Conformance requirement 1.4 is met with an allowance for the statistical significance of the test.

2.	To verify that with a static adjacent channel interferer at 400 kHz above and below a TUhigh wanted signal frequency and signal level 41 dB above the wanted signal level:

2.1	Conformance requirement 2.1 is met with an allowance for the statistical significance of the test.

2.2	Conformance requirement 2.2 is met with an allowance for the statistical significance of the test.

2.3	Conformance requirement 2.3 is met with an allowance for the statistical significance of the test.

2.4	Conformance requirement 2.4 is met with an allowance for the statistical significance of the test.

14.5.1.4	Method of test

14.5.1.4.1	Initial conditions

A call is set up according to the generic call set up procedure on a TCH/FS with an ARFCN in the mid ARFCN range, power control level set to maximum power.

The SS commands the MS to create the traffic channel loop back signalling erased frames.

The SS transmits Standard Test Signal C1 on the TCH (wanted signal).

14.5.1.4.2	Procedure

a)	In addition to the wanted signal, the SS transmits an independent, uncorrelated interfering signal, Standard Test Signal I1 (unwanted signal).

The unwanted signal is continuous and has no fixed relationship with the bit transitions of the wanted signal.

The fading characteristic of the wanted and the unwanted signal is set to TUhigh.

The unwanted signal is transmitted at a nominal frequency 200 kHz above the nominal frequency of the wanted signal. Its amplitude is set to 9dB above that of the wanted signal.

b)	The SS compares the data of the signal that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication.

c)	The SS tests the frame erasure compliance for the TCH/FS by examining at least the minimum number of samples of consecutive frames. The number of frame erasure events is recorded.

d)	The SS determines the the number of residual bit error events for the bits of the class Ib and class II, by examining sequences of at least the minimum number of samples of consecutive bits of class Ib and class II, Bits are only taken from those frames for which no bad frame indication was given.

e)	The measurement of steps c) and d) is repeated with the unwanted signal on a frequency at the same displacement from, but below, the frequency of the wanted signal.

f)	The measurement of steps c) to e) shall be repeated for a displacement of the unwanted signal of 400 kHz, and with the amplitude of the unwanted signal 41 dB above the level of the wanted input signal, and the unwanted signal static.

g)	Steps c) to f) are repeated under extreme test conditions.

14.5.1.5	Test requirements

Table 14-11: Limits for adjacent channel selectivity

GSM900	DCS1800

Interference	Channel	Type of 	Test limit	Minimum No.	Test limit	Minimum No

at		measurement	error rate %	of samples	error rate	of samples

200 kHz	TCH/FS	FER	6.742*a	8900	3.371*a	17800

class Ib	RBER	0.420/a	1000000	0.270/a	2000000

class II	RBER	8.333	600000	8.333	1200000

400 kHz	TCH/FS	FER	11.461*a	8900	5.714*a	10500

class Ib	RBER	0.756/a	1000000	0.483/a	1200000

class II	RBER	9.167	600000	9.167	720000

The error rates measured in this test shall not exceed the test limit error rate given in table 14-11. This shall apply for any combination of normal and extreme test voltages and ambient temperature, and with the interfering signals at either side of the wanted frequency.

The parameter a can range from 1 to 1.6. The value of a for the RBER test on TCH/FS class Ib bits under particular measurement conditions shall be the same as that determined in the FER test on TCH/FS under the same conditions.

Note:	A static unwanted signal is used to avoid a potential problem with the implementation of the fading simulator.

14.5.2	Adjacent channel rejection - control channels

14.5.2.1	Definition and applicability

The adjacent channel selectivity is a measure of the capability of the receiver to receive a wanted modulated signal without exceeding a given degradation due to the presence of an unwanted signal in the adjacent channel.

The adjacent channel can be the adjacent in the RF spectrum or in time. There are therefore two types of adjacent channel selectivity:

1)	Adjacent RF channel selectivity which is specifically tested in this section.

2)	Adjacent Time Slot selectivity, which is implicitly tested in test 14.2.1.

The requirements and this test apply to MS that do not support speech.

14.5.2.2	Conformance requirement

1.	For adjacent channel interference at 200 kHz above and below the wanted signal frequency and signal level 9 dB above the wanted signal level:

1.1	For a TUhigh faded wanted signal and a TUhigh adjacent channel interferer, the FER for the FACCH does not exceed the requirements of table 2 in GSM 05.05; GSM 05.05, 6.3

1.2	For a TUhigh faded wanted signal and a TUhigh adjacent channel interferer, the FER for the FACCH does not exceed the requirements of table 2 in GSM 05.05 under extreme test conditions; GSM 05.05, 6.3, Annex D D.2.1, D.2.2

2	With adjacent channel interference at 400 kHz above and below the wanted signal frequency and signal level 41 dB above the wanted signal level:

2.1	For a TUhigh faded wanted signal and a static adjacent channel interferer, the FER for the FACCH shall be better than:

GSM 900:	17.1%; GSM 05.05, 6.3

DCS1800:	 6.1%; GSM 05.05, 6.3

2.2	For a TUhigh faded wanted signal and a static adjacent channel interferer, the FACCH shall be better than:

GSM 900:	17.1%

DCS1800:	 6.1%

under extreme test conditions; GSM 05.05, 6.3, Annex D D.2.1, D.2.2

14.5.2.3	Test purpose

1	To verify that with TUhigh adjacent channel interference at 200 kHz above and below a TUhigh wanted signal frequency and signal level 9 dB above the wanted signal level:

1.1	Conformance requirement 1.1 is met with an allowance for the statistical significance of the test.

1.2	Conformance requirement 1.2 is met with an allowance for the statistical significance of the test.

2.	To verify that with static adjacent channel interference at 400 kHz above and below a TUhigh wanted signal frequency and signal level 41 dB above the wanted signal level:

2.1	Conformance requirement 2.1 is met with an allowance for the statistical significance of the test.

2.2	Conformance requirement 2.2 is met with an allowance for the statistical significance of the test.

14.5.2.4	Method of test

14.5.2.4.1	Initial conditions

A call is set up according to the generic call set up procedure on a TCH in the mid ARFCN range. Any one of the supported TCH/(F9.6, F4.8, or F2.4) shall be used.

The SS transmits the  Standard Test Signal C1 on the TCH (wanted signal).

14.5.2.4.2	Procedure

a)	In addition to the wanted signal, the SS transmits an independent, uncorrelated interfering signal, Standard Test Signal I1 (unwanted signal).

The unwanted signal is continuous and has no fixed relationship with the bit transitions of the wanted signal. The fading characteristic of the wanted and the unwanted signal is TUhigh

The unwanted signal is transmitted at a nominal frequency 200 kHz above the nominal frequency of the wanted signal. Its amplitude is set to 9dB above that of the wanted signal.

b)	The SS sends the STATUS message. Due to the adjacent channel interference, the MS may not be able to acknowledge the Layer 2 frame. Each repeated L2 frame indicates a frame erasure event. The SS determines the number of frame erasure events during at least the minimum number of samples of FACCH frames.

Note:	These frames will not be consecutive but it is expected that the statistical significance of the tests will not be unduly degraded.

c)	The measurement of step b) is repeated with the unwanted signal on a frequency at the same displacement from, but below, the frequency of the wanted signal.

d)	The measurement of steps a) to c) is repeated for a displacement of the unwanted signal of 400 kHz, and with the amplitude of the unwanted signal 41 dB above the level of the wanted input signal, and the unwanted signal static.

e)	Steps a) to d) are repeated under extreme test conditions.

14.5.2.5	Test requirements

Table 14-12: Limits for adjacent channel selectivity

GSM900	DCS1800

Interference	Channel	Type of 	Test limit	Minimum No.	Test limit	Minimum No.

at		measurement	error rate %	of samples	error rate %	of samples

200 kHz	FACCH	FER	10.640	5639	3.808	15756

400 kHz	FACCH	FER	19.152	3133	6.832	8782

The error rates measured in this test shall not exceed the test limit error rates given in table 14-12. This shall apply for any combination of normal and extreme test voltages and ambient temperature, and with the interfering signals at either side of the wanted frequency.

Note:	A static unwanted signal is used to avoid a potential problem with the noise floor of the fading simulator.

14.6	Intermodulation rejection

14.6.1	Intermodulation rejection - speech channels

14.6.1.1	Definition and applicability

The intermodulation rejection is a measure of the capability of the receiver to receive a wanted modulated signal without exceeding a given degradation due to the presence of two or more unwanted signals with a specific frequency relationship to the wanted signal frequency.

The requirements and this test only apply to MS supporting speech.

14.6.1.2	Conformance requirement

In the presence of two unwanted signals with a specific frequency relationship to the  wanted signal frequency the Class II RBER for TCH/FS shall meet the reference sensitivity performance of table 1 in GSM 05.05; GSM 05.05, 5.2

14.6.1.3	Test purpose

To verify that the MS does not exceed the the conformance requirement with an allowance for the statistical significance of the test.

14.6.1.4	Method of test

Note:	The measurements address the third order intermodulation, which represents the most serious case.

14.6.1.4.1	Initial conditions

A call is set up according to the generic call set up procedure on a TCH with an ARFCN in the Mid ARFCN range, power control level set to maximum power.

The SS transmits Standard Test Signal C1 on the traffic channel.

14.6.1.4.2	Procedure

a)	The amplitude of the wanted signal is set to 4 dB above the reference sensitivity level (see Table 14-12).

b)	The SS commands the MS to create the loop back facility signalling erased frames.

c)	The SS produces a static wanted signal, and two static interfering (unwanted) signals at the same time. There is no correlation in the modulation between the signals.

The first interfering signal is on a frequency equal to the centre frequency of an ARFCN four above that of the receiver. This signal is static and unmodulated.

The second interfering signal is on an ARFCN eight above that of the receiver. This signal is static, continuous and modulated by random data.

The amplitude of both the interfering signals is set according to Table 14-13.

d)	The SS compares the data of the signal that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication.

The SS tests the RBER compliance of class II bits by examining at least the minimum number of samples of consecutive bits. Bits only taken from those frames which do not signal frame erasure. The number of error events is recorded.

e)	The measurement of step d) is repeated with the two unwanted signals having frequencies corresponding to ARFCN four and eight below the ARFCN of the wanted signal

f)	Steps b) to e), are repeated but with the receiver operating on an ARFCN in the Low ARFCN range.

g)	Steps b) to e), are repeated but with the receiver operating on an ARFCN in the High ARFCN range.

h)	Steps a) to g) are repeated under extreme test conditions .

Table 14-13: Intermodulation test signal levels

GSM900	DCS1800

MS HANDHELD	NO	YES	YES

WANTED SIGNAL	13	15	17

dBmVemf(  )

FIRST INTERFERER	74	64	64

dBmVemf(  )

SECOND INTERFERER	63	63	64

dBmVemf(  )

Note:	Some of the levels in Table 14-12 are different to those specified in GSM 05.05 due to the consideration of the effect of modulation sideband noise from the second interferer.

14.6.1.5	Test requirements

The error rate measured in this test shall not exceed the test limit error rate values given in table 14-14.

This shall apply under normal condition and under any combination of normal and extreme test voltages and ambient temperature, and with the two interfering signals at either side of the wanted frequency.

Table 14-14: Limits for intermodulation rejection

Channel	Propagation	Type of	Test limit	Minimum No. of

conditions	measurement	error rate %	samples

TCH/FS	Static	RBER	2.439	8200

Class II

14.6.2	Intermodulation rejection - control channels

14.6.2.1	Definition and applicability

The intermodulation rejection is a measure of the capability of the receiver to receive a wanted modulated signal without exceeding a given degradation due to the presence of two or more unwanted signals with a specific frequency relationship to the wanted signal frequency.

The requirements and this test apply to MS not supporting speech.

14.6.2.2	Conformance requirement

In the presence of two unwanted signals with a specific frequency relationship to the  wanted signal frequency the FER for FACCH shall meet the reference sensitivity performance of table 1 in GSM 05.05; GSM 05.05, 5.2

14.6.2.3	Test purpose

To verify that the MS does not exceed the conformance requirement with an allowance for the statistical significance of the test.

14.6.2.4	Method of test

Note:	The measurements address the third order intermodulation, which represents the most serious case.

14.6.2.4.1	Initial conditions

A call is set up according to the generic call set up procedure on a TCH with an ARFCN in the Mid ARFCN range, power control level set to maximum.

The SS transmits Standard Test Signal C1 on the traffic channel. The amplitude of the wanted signal is set to 4 dB above the reference sensitivity level (see Table 14.15).

The SS commands the MS to create traffic channel loop back signalling erased frames.

14.6.2.4.2	Procedure

a)	The SS produces a TUhigh wanted signal, and two static interfering (unwanted) signals at the same time. There is no correlation in the modulation between the signals.

The first interfering signal is on a frequency equal to the centre frequency of an ARFCN four above that of the receiver. This signal is static and unmodulated.

The second interfering signal is on an ARFCN eight above that of the receiver. This signal is static, continuous and modulated by random data.

The amplitude of both the interfering signals is set according to Table 14-15.

b)	The SS sends the STATUS message. The SS determines the number of frame erasure events during at least the minimum number of samples of FACCH frames.

Note:	These frames will not be consecutive but it is expected that the statistical significance of the tests will not be unduly degraded.

c)	The measurement of step b) is repeated with the two unwanted signals having frequencies corresponding to ARFCN four and eight below the ARFCN of the wanted signal

d)	Steps a) to c), are repeated but with the receiver operating on an ARFCN in the Low ARFCN range.

e)	Steps a) to c), are repeated but with the receiver operating on an ARFCN in the High ARFCN range.

f)	Steps a) to e) are repeated under extreme test conditions.

Table 14-15: Intermodulation test signal levels

GSM900	DCS1800

MS HANDHELD	NO	YES	YES

WANTED SIGNAL	13	15	17

dBmVemf(  )

FIRST INTERFERER	74	64	64

dBmVemf(  )

SECOND INTERFERER	63	63	64

dBmVemf(  )

Note:	Some of the levels in Table 14-15 are different to those specified in GSM 05.05 due to the consideration of the effect of modulation sideband noise from the second interferer.

14.6.2.3	Test requirements

The error rate measured in this test shall not exceed the test limit error rate values given in table 14-16.

This shall apply under normal condition and under any combination of normal and extreme test voltages and ambient temperature, and with the two interfering signals at either side of the wanted frequency.

Table 14-16: Limits for intermodulation rejection

GSM900			DCS1800

Channel	Propagation	Type of	Test limit		Minimum No. of	Test limit	Min No. of

conditions	measurement	error rate %		max-samples	error rate %	max-samples

FACCH	TUhigh/No FH	FER	14.634		6696	10.084	13736

14.7	Blocking and spurious response

14.7.1	Blocking and spurious response - speech channels

14.7.1.1	Definition and applicability

Blocking is a measure of the ability of the receiver to receive a modulated wanted input signal in the presence of an unwanted input signal, on frequencies other than those of the spurious responses or the adjacent channels, without exceeding a given degradation.

The requirements and this test only apply to MS supporting speech.

14.7.1.2	Conformance requirement

In the presence of an unwanted signal on frequencies outside the current and the adjacent channels in the frequency range 100 kHz to 12.75 GHz, the RBER for Class II bits for TCH/FS shall meet the reference sensitivity performance of table 1 in GSM 05.05; GSM 05.05, 5.1

14.7.1.3	Test purpose

To verify that the MS does not exceed the conformance requirement with an allowance for the statistical significance of the test.

14.7.1.4	Method of test

14.7.1.4.1	Initial conditions

A call is set up according to the generic call set up procedure. on a TCH with an ARFCN in the mid ARFCN range, power control level set to maximium power.

The SS transmits Standard Test Signal C1 on the traffic channel. (TCH frequency FR)

The SS commands the MS to create traffic channel loop back signalling erased frames.

14.7.1.4.2	Procedure

a)	The SS produces a static wanted signal and a static interfering signal at the same time. The amplitude of the wanted signal is set to 4 dB above the reference sensitivity level.

b)	The unwanted signal is a C.W. signal (Standard test signal IO) of frequency FB. It is applied in turn on the subset of frequencies calculated in step c) in the overall range 100 kHz to 12.75 GHz, where FB is an integer multiple of 200 kHz.

However, frequencies in the range FR +/- 600 kHz are excluded.

Note:	Allowance must be made for possible spurious signals arising from the SS. These are particularly likely at sub harmonic frequencies nFB where n = 2, 3, 4, 5, etc.

c)	The frequencies at which the test is performed (adjusted to an integer multiple of 200 kHz channels most closely approximating the absolute frequency of the calculated blocking signal frequency) are the combined frequencies from i), ii) and iii) below :-

i)	The total frequency range formed by :-

GSM900	the frequencies between Flo + (IF1 + IF2 + ... +IFn +12.5 MHz)

and Flo - (IF1 + IF2 + ... + IFn + 12.5 MHz).

DCS1800	the frequencies between Flo + (IF1 + IF2 + ... +IFn +37.5 MHz)

and Flo - (IF1 + IF2 + ... + IFn + 37.5 MHz).

and

the frequencies + 100 MHz and - 100 MHz from the edge of the relevant receive band.

Measurement are made at 200 kHz intervals.

ii)	The three frequencies IF1, IF1 + 200 kHz, IF1 - 200 kHz.

iii)	The frequencies mFlo + IF1, mFlo - IF1, where m is all positive integers greater than or equal to 2 such that either sum lies in the range 100 kHz to 12.75 GHz.

The frequencies in step ii) and iii) lying in the range of frequencies defined by step i) above need not be repeated.

iv)	Outside the range defined in i) above, spot frequencies are measured at 10 MHz from the range edge and repeated at 10 MHz intervals from 100 kHz to 12.75 GHz.

Where :-

Flo -	local oscillator applied to first receiver mixer

IF1 ... IFn -	are the n intermediate frequencies

Flo, IF1, IF2 ... IFn	shall be declared by the manufacturer in the PIXIT statement GSM 11.10 Annex 3.

d)	The level of the unwanted signal is set according to table 14-17.

Table 14-17: Level of unwanted signals

GSM900	DCS1800

MS HANDHELD	YES	NO	YES

FREQUENCY	LEVEL IN dBmVemf(  )

FR +/- 600 kHz to FR +/- 800 kHz	70	75	70

FR +/- 800 kHz to FR +/- 1.6 MHz	70	80	70

FR +/- 1.6 MHz to FR +/- 3 MHz	80	90	80

915 MHz to FR - 3 MHz	90	90	-

FR + 3 MHz to 980 MHz	90	90	-

1785 MHz to FR - 3 MHz	-	-	87

FR + 3 MHz to 1920 MHz	-	-	87

835 MHz to <915 MHz	113	113

>980 MHz to 1000 MHz	113	113

100 kHz to <835 MHz	90	90

>1000 MHz to 12.75 GHz	90	90

100 kHz to 1705 MHz	-	-	113

>1705 MHz to <1785 MHz	-	-	101

>1920 MHz to 1980 MHz	-	-	101

>1980 MHz to 12.75 GHz	-	-	90

Note:	These values differ from GSM 05.05 because of practical generator limits in the SS.

e)	The SS compares the data of the signal that it sends to the MS with the signal which is looped back from the receiver after demodulation and decoding, and checks the frame erasure indication.

The SS tests the RBER compliance for the bits of class II, by examining sequences of at least the minimum number of samples of consecutive bits of class II, where bits are taken only from those frames for which no bad frame indication was given. The number of error events is recorded.

If a failure is indicated it is noted and counted towards the allowed exemption totals.

In the case of failures discovered at the predicted frequencies at steps f ii), iii) or iv) the test is repeated on the adjacent channels +/- 200 kHz away. If either of these two frequencies fail then the next channel 200 kHz beyond is also tested. This process is repeated until all channels constituting the group of failures is known.

f)	Steps b) to e) are repeated, for the MS operating on an ARFCN in the Low ARFCN range.

g)	Steps b) to e) are repeated, for the MS operating on an ARFCN in the High ARFCN range.

14.7.1.5	Test requirements

The error rate measured in this test shall not exceed the test limit error rate values given in table 14-18.

This shall apply under normal test voltage and ambient temperature, and with the interfering signal at any frequency in the range specified.

Table 14-18: Limits for blocking

Channel	Type of	Test limit 	Minimum number of

measurement	error rate %	samples

TCH/FS	RBER	2.439	8200

Class II

The following exceptions are allowed:

GSM 900:	A maximum of six failures  in the frequency band 915 MHz to 980 MHz (which, if grouped, shall not exceed three 200 kHz channels per group).

A maximum of 24 failures  in the combined bands 100 kHz to 915 MHz and 980 MHz to 12.75 GHz (which, if below FR and grouped, shall not exceed three 200 kHz channels per group).

DCS 1800:	A maximum of twelve failures in the band 1785 MHz to 1920 MHz (which, if grouped, shall not exceed three 200 kHz channels per group)

A maximum of 24 in the combined bands 100 kHz to 1785 MHz and 1920 MHz to 12.75 GHz (which, if below FR and grouped, shall not exceed three 200 kHz channels per group).

If the number of failures do not exceed the maximum allowed figures stated above, the test of 14.7.1.4 is repeated at the frequencies at which the failures occurred.  The level of the unwanted signal is set to 70 dBmVemf(  ) and the performance requirement is once again that that stated in the table above.

The number Error rate measured in this test shall not exceed the test limit error rate values given in table 14-18.

No failures are allowed at this lower unwanted signal level.

14.7.2.	Blocking and spurious response - control channels

14.7.2.1	Definition and applicability

Blocking is a measure of the ability of the receiver to receive a modulated wanted input signal in the presence of an unwanted input signal, on frequencies other than those of the spurious responses or the adjacent channels, without exceeding a given degradation.

The requirements and this test apply to MS not supporting speech.

14.7.2.2	Conformance requirement

In the presence of an unwanted signal on frequencies outside the current and the adjacent channels in frequency range 100 kHz to 12.75 GHz, the FER for FACCH shall meet the reference sensitivity requirements of table 1 in GSM 05.05; GSM 05.05, 5.1

14.7.2.3	Test purpose

To verify that the MS does not exceed the conformance requirement with an allowance for the statistical significance of the test.

14.7.2.4	Method of test

14.7.2.4.1	Initial conditions

A call is set up according to the generic call set up procedure on a TCH with an ARFCN in the mid ARFCN range, power control level set to maximum power.

The SS transmits Standard Test Signal C1 on the traffic channel. (TCH frequency FR)

14.7.2.4.2	Procedure

a)	The SS sends the STATUS message. Due to interfering signals, the MS may not be able to acknowledge the Layer 2 frame. Frame erasures are indicated by repeated L2 frames.

b)	The SS is set to produce a TUhigh wanted signal and a static interfering signal at the same time. The SS sets the amplitude of the wanted signal to 4 dB above the reference sensitivity level.

c)	The unwanted signal is a C.W. signal (Standard test signal IO) of frequency FB. It is applied in turn on the subset of frequencies calculated at step f) in the overall range 100 kHz to 12.75 GHz, where FB is an integer multiple of 200 kHz.

However, frequencies in the range FR +/- 600 kHz are excluded.

Note:	Allowance must be made for possible spurious signals arising from the SS. These are particularly likely at sub harmonic frequencies nFB where n = 2, 3, 4, 5, etc.

d)	The frequencies at which the test is performed (adjusted to an integer multiple of 200 kHz channels most closely approximating the absolute frequency of the calculated blocking signal frequency) are the combined frequencies from i), ii) and iii) below :-

i)	The total frequency range formed by :-

GSM900	the frequencies between Flo + (IF1 + IF2 + ... +IFn +12.5 MHz)

and Flo - (IF1 + IF2 + ... + IFn + 12.5 MHz).

DCS1800	the frequencies between Flo + (IF1 + IF2 + ... +IFn +37.5 MHz)

and Flo - (IF1 + IF2 + ... + IFn + 37.5 MHz).

and

the frequencies + 100 MHz and - 100 MHz from the edge of the relevant receive band.

Measurement are made at 200 kHz intervals.

ii)	The three frequencies IF1, IF1 + 200 kHz, IF1 - 200 kHz.

iii)	The frequencies mFlo + IF1, mFlo - IF1, where m is all positive integers greater than or equal to 2 such that either sum lies in the range 100 kHz to 12.75 GHz.

The frequencies in step ii) and iii) lying in the range of frequencies defined by step i) above need not be repeated.

iv)	Outside the range defined in i) above, spot frequencies are measured at 10 MHz from the range edge and repeated at 10 MHz intervals from 100 kHz to 12.75 GHz.

Where :-

Flo -	local oscillator applied to first receiver mixer

IF1 ... IFn -	are the n intermediate frequencies

Flo, IF1, IF2 ... IFn	shall be declared by the manufacturer in the PIXIT statement GSM 11.10 Annex 3.

e)	The level of the unwanted signal is set according to table 14-19

Table 14-19: Level of unwanted signals

GSM900	DCS1800

MS HANDHELD	YES	NO	YES

FREQUENCY	LEVEL IN dBmVemf(  )

FR +/- 600 kHz to FR +/- 800 kHz	70	75	70

FR +/- 800 kHz to FR +/- 1.6 MHz	70	80	70

FR +/- 1.6 MHz to FR +/- 3 MHz	80	90	80

915 MHz to FR - 3 MHz	90	90	-

FR + 3 MHz to 980 MHz	90	90	-

1785 MHz to FR - 3 MHz	-	-	87

FR + 3 MHz to 1920 MHz	-	-	87

835 MHz to <915 MHz	113	113

>980 MHz to 1000 MHz	113	113

100 kHz to <835 MHz	90	90

>1000 MHz to 12.75 GHz	90	90

100 kHz to 1705 MHz	-	-	113

>1705 MHz to <1785 MHz	-	-	101

>1920 MHz to 1980 MHz	-	-	101

>1980 MHz to 12.75 GHz	-	-	90

Note:	These values differ from GSM 05.05 because of practical generator limits in the SS.

f)	The SS determines the number of frame erasure events during at least the minimum number of samples. If a failure is indicated, it is noted and counted towards the allowed exemption total.

In the case of failures discovered at the predicted frequencies at steps f ii), iii) or iv) the test is repeated on the adjacent channels +/- 200 kHz away. If either of these two frequencies fail then the next channel 200 kHz beyond is also be tested. This process is repeated until all channels constituting the group of failures is known.

g)	Steps c) to f) are repeated, for the MS operating on an ARFCN in the Low ARFCN range.

h)	Steps c) to f) are repeated, for the MS operating on an ARFCN in the High ARFCN range.

14.7.2.5	Test requirements

The error rate measured in this test shall not exceed the test limit error rate given in table 14-20.

This shall apply under normal test voltage and ambient temperature, and with the interfering signal at any frequency in the range specified.

Table 14-20: Limits for blocking

GSM 900			DCS 1800

Channel	Type of	Test limit 	Minimum No. of	Test limit	Minimum No. of

measurement	error rate	samples	error rate	samples

FACCH	FER	14.634	6696	10.084	13736

The following exceptions are allowed:

GSM 900:	A maximum of six failures in the band 915 MHz to 980 MHz (which, if grouped, shall not exceed three 200 kHz channels per group).

A maximum of 24 in the combined bands 100 kHz to 915 MHz and 980 MHz to 12.75 GHz (which, if below FR and grouped, shall not exceed three 200 kHz channels per group).

DCS 1800:	A maximum of twelve failures in the band 1785 MHz to 1920 MHz (which, if grouped, shall not exceed three 200 kHz channels per group)

A maximum of 24 in the combined bands 100 kHz to 1785 MHz and 1920 MHz to 12.75 GHz (which, if below FR and grouped, shall not exceed three 200 kHz channels per group).

If the number of failures do not exceed the maximum allowed figures stated above, the test of 14.7.2.4 is repeated at the frequencies at which the failures occured. The level of the unwanted signal is set t0 70 dBmVemf(  ) and the performance requirement is once again that stated above.

The number of Error Events recorded in this test shall not exceed the Max-events values given above, when the number of samples = Max-samples.

No failures are allowed at this lower unwanted signal level.