22.	Transmit power control timing and confirmation

22.1	Definition and applicability

The RF power level to be employed by the MS is indicated by means of the 5 bit TXPWR field sent in the layer 1 header of each downlink SACCH message block and may be sent in a dedicated signalling block.

When a power change is signalled the MS must change its power control level to the new level at a certain rate of change.

The MS shall confirm the power level that it is currently employing by setting the MS_TXPWR_CONF field in the uplink SACCH L1 header.

The requirements and this test apply to all types of GSM 900 and DCS 1800 MS.

22.2	Conformance requirement

1.	The RF power control level to be employed by the MS is indicated by means of the power control information sent in the layer 1 header of each downlink SACCH message block and may be sent in a dedicated signalling block; GSM 05.08 4.2.

2.	The MS shall confirm the power level that it is currently employing in the uplink SACCH L1 header. The indicated value shall be the power control level actually used by the MS for the last burst of the previous SACCH period; GSM 05.08 4.2.

3.	Upon receipt of a command on the SACCH to change its RF power level, the MS shall change to the new level at a rate of one nominal 2 dB power control step every 60 ms; GSM 05.08 4.7.

4.	The change (in conformance requirement 4) shall commence at the first TDMA frame belonging to the next reporting period; GSM 05.08 4.7.

5.	In case of channel change the commanded power level shall be applied on the new channel immediately; GSM 05.08 4.7.

22.3	Test purpose

1.	To verify that the MS will set its transmitter output power in accordance with conformance requirement 1.

2.	To verify that the MS will confirm the power level it is currently employing according to conformance requirement 2.

3.	To verify that the MS, upon receipt of a command from the SACCH to change its RF power level, will change according to conformance requirement 3.

4.	To verify that the MS will commence the change of power level at least by the sixth TDMA frame belonging to the next reporting period.

5.	To verify that in case of new channel assignment the commanded power level is applied on the new channel according to conformance requirement 5.

22.4	Method of test

Note:	The method of measuring the MS transmitter output power is given in section II.3.3.

22.4.1	Initial conditions

A call is set up by the SS according to the generic call set up procedure on a channel with ARFCN in the Mid ARFCN range (ref Table 1.4), power control level set to maximum power.

22.4.2	Procedure

a)	The SS signals minimum power control level to the MS in the SACCH.

b)	The SS measures the MS transmitter output power on TDMA frames 6, 19, 32 and every subsequent 13th TDMA frame to TDMA frame 214. The SS also monitors the MS_TXPWR_CONF field in the uplink SACCH L1 header for the four SACCH multiframes after the SS signals the power change.

c)	The SS now sets TXPWR in the SACCH to the maximum peak power appropriate to the class of the MS.

d)	The SS measures the MS transmitter output power on TDMA frames 6, 19, 32 and every subsequent 13th TDMA frame to TDMA frame 214. The SS also monitors the MS_TXPWR_CONF field in the uplink SACCH L1 header for the four SACCH multiframes after the SS signals the power change.

e)	The SS now sets the SACCH TXPWR to 8.

f)	After 3 s the SS sets the SACCH TXPWR to 9.

g)	The SS measures the MS transmitter output power on TDMA frame 6.

h)	The SS sets the SACCH TXPWR to 8.

i)	The SS measures the MS transmitter output power on TDMA frame 6.

j)	The channel assignment is changed and the demanded power within the channel assignment is set to the minimum power control level of the MS.

k)	When the MS has changed channel its output power is measured on the first burst on the new channel.

22.5	Test requirements

Note:	Refer to tables II.3-3 and II.3-4 for relationship between the power class, power control level, transmitter output power and the relevant tolerances.

a)	In steps b) and d), the transmitter output power shall change by one power step towards the new level signalled for each measured burst until the MS is operating at the closest supported power control level and from then on, all transmissions shall be at that level.

b)	In steps b) and d), the value of the MS_TXPWR_CONF field in the uplink SACCH L1 header shall correspond to the actual power control level used for the last transmitted burst of the previous SACCH multiframe. The first one shall indicate the initial transmitted power control level, the subsequent ones shall change by 8 each time until the final power control level has been reached in which case that value shall be indicated.

c)	In steps g) and i) the transmitter output power of TDMA frame 6 shall correspond to the new commanded power control level.

d)	In step k) the MS output power, measured on the new channel shall correspond to the power control level in the channel assignment.