26.2	Initial tests

26.2.1	Channel request

The random access procedure is used by the MS to ask for resources to the network. If it is not performed correctly, the MS could prevent other MSs to obtain resources, or the network could be overloaded if the MS does not respect the duration between 2 CHANNEL REQUEST messages.

26.2.1.1	Channel request / test 1

[Editorial note : sections 26.2.1.1.1 and 26.2.1.1.2 are intentionnally missing

- the test was modified from the phase 1 because the immediate assignment procedure has been modified in 04.08.

- the initial time to respond to paging needs still some study. 04.08 specifies in phase 2 that the MS shall take no more than 0.5 s betwenn the paging and the beginning of the imm ass procedure. The note in the purpose may need modification.]

Conformance Requirement:

1)	The MS shall spread an initial CHANNEL REQUEST message, with equal probability on approximately the correct number of time slots, after the sending of a PAGING REQUEST message.

2)	The MS shall spread retransmissions of a CHANNEL REQUEST message, with equal probability, on Tx-Integer timeslots.

3)	All CHANNEL REQUEST messsages sent shall contain Establishment Cause "Answer to Paging".

Purpose:

To verify that the MS spreads an initial CHANNEL REQUEST message with equal probability on approximately the correct number of time slots when the following conditions apply :

-	the CCCH is combined or not combined with SDCCHs

-	the maximum number of retransmissions is equal to 1

-	Tx-Integer is put to 5.

To verify that the MS spreads retransmission of a CHANNEL REQUEST message with equal probability on Tx-Integer time slots and correctly applies the fixed delay when the following conditions apply:

-	the CCCH is combined or not combined with SDCCHs

-	the maximum number of retransmissions is equal to 1

-	Tx-Integer is put to 5.

To verify that the MS retransmits exactly Max_Retrans times a CHANNEL REQUEST message if the network never responds to the CHANNEL REQUEST message.

Note	It is not possible to verify that a MS spreads an initial CHANNEL REQUEST message with equal probabilty on exactly the correct number of time slots, because the exact moment is not known when the MS initially initiates the immediate assignment procedure. It is assumed that for an MS that is paged, the time to initiate the immediate assignment procedure will not vary from more than 5ms between different occurrences of the random access procedure.

Reference(s):

GSM 04.08 section 3.3.1.2

Related PICS/PIXIT Statement(s):

None.

26.2.1.1.1	Channel request / test 1 / test cases 1 & 2

Initial Conditions:

System Simulator :

1 cell, Tx-Integer = 5, Max_Retrans = 1.

Test case 1 : CCCH not combined with SDCCH.

Test case 2 : CCCH combined with SDCCH.

Mobile Station :

The MS has a valid TMSI. It is in the MM-state "idle, updated" and in the RR idle-mode.

Specific test parameters :

N0 = max (8, Tx-Integer).

K = 200.

T = Tx-Integer.

Test case 1 : S = 109.

Test case 2 : S = 58.

Foreseen Final State of the MS:

The MS has a valid TMSI. It is in the MM-state "idle, updated" and in the RR idle-mode.

Method of Test:

The MS is paged. The SS measures and stores the number of CCCH RACH slots between the sending of the PAGING REQUEST message and the reception of the CHANNEL REQUEST from the MS. The MS retransmits CHANNEL REQUEST messages Max_Retrans times. The SS measures and stores the number of CCCH RACH slots between the moment where a CHANNEL REQUEST message has been received, and the reception of the following CHANNEL REQUEST message, excluding the slots containing the messages themselves. The SS does not answer to the CHANNEL REQUEST messages Max_Retrans times. After the last CHANNEL REQUEST message the SS sends an IMMEDIATE ASSIGNMENT REJECT. The sequence is performed K times for each test case.

Maximum Duration of Test:

The execution of one sequence (for one value k) : 5s.

Between two consecutive executions (for k and k+1), the SS must wait for an amount of time which is enough to guarantee that the MS is in service (listening to its paging subchannel).

Expected Sequence:

The sequence is executed for execution counter k = 1, ... , K, for each of the 2 test cases.

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	"Mobile Identity" IE contains

correct TMSI.

2	MS -> SS	CHANNEL REQUEST	"Establishment Cause" = Answer to

paging.

The SS checks that a CHANNEL

REQUEST message is sent by the MS

within 1 second.

3	SS		The SS measures the number f(0,k)

of CCCH RACH slots between the

sending of PAGING REQUEST message

and the reception of a CHANNEL

REQUEST message from the MS.

4	SS		The SS stores f(0,k).

Steps 5,6 and 7 are executed for

execution counter i = 1,...,

Max_Retrans

5	MS -> SS	CHANNEL REQUEST	"Establishment Cause" = Answer to

paging.

6	SS		The SS measures the number f(i,k)

of CCCH RACH slots between :

- the moment where the last CHANNEL

REQUEST message has been received

and

- the reception of the new CHANNEL

REQUEST message from the MS,

excluding the slots containing

the messages themselves.

7	SS		The SS stores f(i,k).

8	SS -> MS	IMMEDIATE ASSIGN REJ	The first (test case 1) or the

second (test case 2) "Request

Reference" IE corresponds to the

last CHANNEL REQUEST message

received.

The first (test case 1) or the

second (test case 2) "Wait

Indication" IE specifies 0 second.

Other fields do not concern the MS

under test.

Note:	The test limit has been computed to give a confidence of [99.74 %] that a unit which follows the requirements will pass. The number of samples (200) has been chosen to get a good compromise between the test time and the risk of passing a bad unit.

Analysis of Test Results:

For each of the 2 test cases S(i,n) is computed:

For i = 0,...,Max_Retrans and n=0,...,Max f(i,k)

let S(i,n) = CARD {k | f(i,k) = n}

and the following requirements must be checked:

a)	S(0,n) <= 41 for all n.

There is an integer n0 such that S(0,n) = 0 for all n<n0 and all n > n0 + N0 +1.

b)	S(1,n) <= 60 for all n

for k = 1,...,K S <= f(1,k) <= S+T-1

Note:	CARD {k | f(i,k) = n} is mathematical notation for the number of times that f(i,k) equals n.

26.2.1.1.2	Channel request / test 1 / test cases 3 & 4

Conformance Requirement:

1)	The MS shall spread retransmissions of a CHANNEL REQUEST message, with equal probability on Tx-Integer timeslots, after the sending of the PAGING REQUEST message when the following conditions apply:

-	the CCCH is combined or not combined with SDCCHs;

-	the maximum number of retransmissions is equal to one of the following values: 1, 2, 4, 7;

-	Tx-Integer is put to any of the allowed values among those which are greater or equal to 6.

2)	The MS shall not retransmit another CHANNEL REQUEST message when Max-retrans is reached.

3)	All CHANNEL REQUEST messages sent shall contain Establishment Cause "Answer to Paging".

Initial Conditions:

System Simulator :

1 cell.

Tx-Integer is randomly chosen in the set {6, 7, 8, 9, 10, 11, 12, 14, 16, 20, 25, 32, 50}.

Max_Retrans is randomly chosen in the set {1,2,4,7}.

Test case 3 : CCCH not combined with SDCCH.

Test case 4 : CCCH combined with SDCCH.

Mobile Station :

The MS has a valid TMSI. It is in the MM-state "idle, updated" and in the RR idle-mode.

Specific test parameters

K equals the upper rounded value of 230/Max_Retrans.

m equals the upper rounded value of 0.5*Tx-Integer.

Counter M = 0.

Parameter S : according to table 3.1/GSM 04.08 (this parameter depend on the value chosen for Tx-Integer).

N0 = max (8, Tx-Integer)

Foreseen Final State of the MS:

The MS has a valid TMSI. It is in the MM-state "idle, updated" and in the RR idle-mode.

Method of Test:

The MS is paged. The SS measures the number of CCCH RACH slots between the sending of the PAGING REQUEST message and the reception of the CHANNEL REQUEST from the MS. The MS retransmits CHANNEL REQUEST messages Max_Retrans times. The SS measures the number of CCCH RACH slots f(i,k) between the moment where a CHANNEL REQUEST message has been received, and the reception of the following CHANNEL REQUEST message, excluding the slots containing the messages themselves. The SS updates the counter M. The SS does not answer to the CHANNEL REQUEST messages Max_Retrans times. After the last CHANNEL REQUEST message in every sequence where k is lower than K, the SS sends an IMMEDIATE ASSIGNMENT REJECT . In the last sequence (k = K), the SS does not respond to the MS. The MS shall perform cell reselection.

Maximum Duration of Test:

The execution of one sequence (for one value k) : 5s.

Between two consecutive executions (for k and k+1), the SS must wait for an amount of time which is enough to garantee that the MS is in service (listening to its paging subchannel).

Expected Sequence:

The sequence is executed for execution counter k = 1, ... , K for each of the 2 test cases.

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	"Mobile Identity"= TMSI of the MS.

2	MS -> SS	CHANNEL REQUEST	"Establishment Cause" = Answer to

paging.

The SS checks that this CHANNEL

REQUEST message was sent from the

MS within 1 second.

Step 4, 5, 6 ,7 are executed for

execution counter

i = 1,...,Max_Retrans.

4	MS -> SS	CHANNEL REQUEST	"Establishment Cause" = Answer to

paging.

5	SS		The SS measures the number f(i,k)

of CCCH RACH slots between :

- the moment where the last CHANNEL

REQUEST message has been received

and

- the reception of the new CHANNEL

REQUEST message from the MS,

excluding the slots containing

the messages themselves.

f(i,k) must be in the

set {S,S+1,...,S+T-1}

6	SS		If f(i,k)-S >= m, M = M+1

Depending on the value of k, step

A7 or B7 is performed :

A7	SS -> MS	IMMEDIATE ASSIGN REJ	k < K

The third "Request Reference" IE

corresponds to the last CHANNEL

REQUEST message received.

The third "Wait Indication" IE

specifies 0 second.

Other fields do not adress the MS

under test.

B7	SS		k = K

The SS checks that the MS sends no

more CHANNEL REQUEST messages. This

is verified during 1 second.

8	SS		M / (K * Max_Retrans) must be

inside the following interval :

[0.8 - m/Tx-Integer ;

1.2 - m/Tx-Integer]

Note:	The confidence interval in step 8, and the number of samples are chosen in such a way that the possibility of non accepting a correct MS is less than [0.26%].

26.2.1.2	Channel request / test 2

[Editorial Note :

- This test is performed only once with non combined CCCH/SDCCH.

- No applicable conformance requirement in TBR 5.

- the test is not modified against phase 1 description.]

Purpose:

To verify that an MS produces different random references for a CHANNEL REQUEST after having been switched off and on. There makes probable that different MSs of one product series do not produce the same sequences of random values for the scheduling the sending on the RACH or for the "Random Reference" used in CHANNEL REQUEST messages.

Reference(s):

GSM 04.08 section 3.3.1.2

Related PICS/PIXIT Statement(s):

Switch on button Yes/No.

Initial Conditions:

System Simulator :

1 cell, CCCH not combined with SDCCH.

Mobile Station :

The MS has a valid TMSI. It is in the MM-state "idle, updated" and in the RR idle-mode.

Specific test parameters:

K = 7.

D = 4.

Foreseen Final State of the MS:

The MS has a valid TMSI. It is in the MM-state "idle, updated" and in the RR idle-mode.

Method of Test:

If possible, the MS is switched off, then switched on, otherwise the power supply is removed, then inserted (see PICS). After enough time in order to allow the MS to be pageable, the SS sends a PAGING REQUEST message. The SS stores the "Random Reference" r(k) contained as a parameter in the CHANNEL REQUEST message sent by the MS. This sequence is performed K times, and it is verified that the MS produces different values r(k) after having been switched off and on or power down and on (see PICS).

Maximum Duration of Test:

6 mn

Expected Sequence:

The sequence is executed for execution counter k = 1, ..., K.

Step	Direction	Message	Comments

1	MS		If possible (see PICS) the MS is

switched off. Otherwise the power

supply is removed.

2	MS		Depending on what has been

performed in step 1 the MS is

brought back to operation.

3	SS -> MS	PAGING REQUEST TYPE 1	The SS waits enough time in order

to allow the MS to be pageable

before sending this message, to

allow the MS to camp on the cell.

"Mobile Identity" IE contains

correct TMSI.

4	MS -> SS	CHANNEL REQUEST	"Establishment Cause" = Answer to

paging.

The SS checks that a CHANNEL

REQUEST message is sent from the MS

within 1 second.

5	SS		The SS stores the "Random

Reference" contained in the CHANNEL

REQUEST message.

Analysis of Test Results:

At least D values of r(1),...,r(k) must be different.

Note:	D has been computed such that the probability of refusing a correct MS is less than [0.027%].

26.2.2	IMSI detach and IMSI attach

The IMSI detach/attach procedures are used to indicate to the network that the MS is deactivated/activated. Theses procedures are allowed or not by the network (ATT flag set to "MSs in the cell should apply IMSI attach and detach" or "MSs in the cell are not allowed to apply IMSI attach and detach").

If the IMSI attach procedure does not work correctly then the network would in certain situations not try to establish Mobile Terminating call even if the MS is "idle, updated".

If an MS performs unwanted IMSI detach procedure network resources are wasted.

[Editorial Note: - Procedures 3 and 4 are chained

- Procedures 5 and 6 are chained

]

Conformance Requirement:

1)	When the Attach-detach flag in the Control Channel Description of the System Information Type 3 indicates "MSs in the cell are not allowed to apply IMSI attach and detach", the MS shall not perform the IMSI detach procedure upon deactivation.

2)	When the Attach-detach flag in the Control Channel Description of the System Information Type 3 indicates "MSs in the cell are not allowed to apply IMSI attach and detach", the MS shall not perform the IMSI attach procedure upon activation.

3)	The MS shall not perform the IMSI detach procedure if the Subscriber Identity Module is removed when the Attach-detach flag in the Control Channel Description of the System Information Type 3 indicates "MSs in the cell are not allowed to apply IMSI attach and detach".

4)	The MS shall not perform the IMSI attach procedure if the Subscriber Identity Module is inserted, when the Attach-detach flag in the Control Channel Description of the System Information Type 3 indicates "MSs in the cell are not allowed to apply IMSI attach and detach".

5)	The MS shall correctly perform the IMSI detach procedure, upon switch off, when it is required by the network to do so.

6)	The MS shall correctly perform the IMSI attach procedure upon switch on when the IMSI attach procedure is required by the network. The MS shall correctly acknowlege the implicit TMSI reallocation procedure, which is part of this IMSI attach procedure, this means that the MS shall send a TMSI REALLOCATION COMPLETE message.

7)	The MS shall correctly perform the IMSI detach procedure upon SIM removal when it is required by the network to do so.

8)	The MS shall correctly perform the IMSI attach procedure, following SIM insertion and switch on when the IMSI attach procedure is required by the network. The MS shall correctly acknowlege the implicit TMSI reallocation procedure which is part of this IMSI attach procedure. This means that the MS shall send a TMSI REALLOCATION COMPLETE message.

Purpose:

To verify that the MS correctly performs IMSI detach/attach procedures when it is required by the network and upon deactivation/activation or SIM removal/insertion and does not perform these procedures when not required.

Reference(s):

GSM 04.08 section 4.4.3 and 4.4.4

Related PICS/PIXIT Statement(s):

a:	SIM removal possible without powering down   Yes/ No.

b:	Switch off on button   Yes/No.

c:	IMSI detach after switch off   Yes/ No.

d:	IMSI detach after powering down   Yes/ No.

Initial Conditions:

System Simulator :

1 cell,default parameters.

For procedures 1 and 2 ATT flag is set to "MSs in the cell are not allowed to apply IMSI attach and detach".

For procedures 3 and 4, 5 and 6 ATT flag is set to "MSs in the cell should apply IMSI attach and detach".

Mobile Station :

The MS has a valid TMSI. It is in the MM-state "idle, updated" and in the RR idle-mode.

Foreseen Final State of the MS:

The MS has a valid TMSI which may be different from the initial one. It is in the MM-state "idle, updated" and in the RR idle-mode.

Method of Test:

The SS indicates that IMSI detach/attach is not allowed. If possible the MS is switched off, then switched on, otherwise it is powered down then powered on (see b in PICS). The SS checks that the MS does not perform IMSI detach/attach procedures. If possible (if a = Yes, see PICS), the SIM is removed, then the SIM is inserted. The SS checks that the MS does not perform IMSI detach/attach procedures.The SS indicates now, that IMSI detach/attach is allowed. If possible (if b = Yes and c = Yes, see PICS) the MS is switched off, otherwise it is powered down (if d = Yes, see PICS). The MS initiates an IMSI detach procedure. Then depending on what has been performed before, the MS is switched on or powered on. It initiates an IMSI attach procedure. The location updating procedure contains an implicit TMSI reallocation. The SIM is removed if a = Yes. The MS initiates an IMSI detach procedure. Then the SIM is inserted, it initiates an IMSI attach procedure, the location updating procedure contains an implicit TMSI reallocation.

Maximum Duration of Test:

4 mn

Expected Sequence:

Procedure 1

Step	Direction	Message	Comments

1	MS		If possible the MS is switched off

(see b in PICS), otherwise the MS

is powered down.

2				The MS shall not initiate the IMSI

detach procedure. This is checked

by the SS during 5 seconds.

3	MS		Depending on what has been

performed in step 1, the MS is

brought back to operation.

4				The MS shall not initiate an IMSI

attach procedure. This is checked

by the SS during 30 seconds.

Procedure 2

1	MS		If possible (a = Yes, see PICS),

the SIM is removed from the MS.

2				The MS shall not initiate the IMSI

detach procedure. This is checked

by the SS during 5 seconds.

3	MS		The SIM is inserted in the MS.

4				The MS shall not initiate an IMSI

attach procedure. This is checked

by the SS during 30 seconds.

Procedures 3 and 4

1	MS		The MS is switched off, or powered

down, depending on value b in the

PICS file.

2	MS -> SS	CHANNEL REQUEST	If possible  (b and c = Yes, or d =

Yes) the MS initiates an IMSI

detach procedure.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	IMSI DETACH INDICATION

5	SS -> MS	CHANNEL RELEASE	After the sending of this message,

the SS waits the disconnection of

the main signalling link.

6	MS		Depending on what has been

performed in step 1, the MS is

brought back to operation.

7	MS -> SS	CHANNEL REQUEST	The MS initiates an IMSI attach

procedure.

8	SS -> MS	IMMEDIATE ASSIGNMENT

9	MS -> SS	LOCATION UPDATING REQ	"Location Updating Type" = IMSI

attach.

10	SS -> MS	LOCATION UPDATING ACC	The SS allocates a new TMSI

11	MS -> SS	TMSI REALLOCATION COMP

12	SS -> MS	CHANNEL RELEASE	After the sending of this message,

the SS waits the disconnection of

the main signalling link.

Procedures 5 and 6

1	MS		The SIM is removed from the MS .

2	MS -> SS	CHANNEL REQUEST	If possible (a = Yes in PICS), the

MS initiates an IMSI detach

procedure.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	IMSI DETACH INDICATION

5	SS -> MS	CHANNEL RELEASE	After the sending of this message,

the SS waits the disconnection of

the main signalling link.

6	MS		The SIM is inserted in the MS.

7	MS -> SS	CHANNEL REQUEST	The MS initiates a IMSI attach

procedure.

8	SS -> MS	IMMEDIATE ASSIGNMENT

9	MS -> SS	LOCATION UPDATING REQ	"Location Updating Type" = IMSI

attach.

10	SS -> MS	LOCATION UPDATING ACC	The SS allocates a new TMSI

11	MS -> SS	TMSI REALLOCATION COMP

12	SS -> MS	CHANNEL RELEASE	After the sending of this message,

the SS waits the disconnection of

the main signalling link.

26.2.3	Sequenced MM / CC message transfer

The RR sublayer of the MS shall have an associated send state variable V(SD) for sending MM and CM messages. This send state variable variable has been introduced to avoid the duplication of MM and CM messages. It is useful for the network after a handover or a change of channel to identify duplicated messages.

If the MS started V(SD) with 1 instead of 0 the network would incorrectly diagnose loss of message.

If the MS later on does not handle correctly incrementation of V(SD) the network would not be able to continue the dialogue.

Conformance Requirement:

The MS shall implement correctly the "send state variable V(SD)" ("Send duplicated"), included in transmitted MM and CM messages.

Purpose:

To verify that V(SD) is correctly set to 0 at the beginning of the establishment of the first RR connection and to verify that the MS handles correctly this variable in the special case of IDENTITY REQUEST messages, which are MM messages.

Reference(s):

GSM 04.08 section 2.4.1

Related PICS/PIXIT Statement(s):

None.

Initial Conditions:

System Simulator :

1 cell,default parameters.

Mobile Station :

The MS has a valid TMSI. It is in the MM-state "idle, updated" and in the RR idle-mode.

Foreseen Final State of the MS:

The MS has a valid TMSI. It is in the MM-state "idle, updated" and in the RR idle-mode.

Method of Test:

The MS is paged. After reception of the PAGING RESPONSE message from the MS, the SS sends an IDENTITY REQUEST message. The MS sends an IDENTITY RESPONSE message where N(SD) = 0. The SS repeats its IDENTITY REQUEST message 10 times. The MS transmits IDENTITY RESPONSE message with the value 1 and 0 in the N(SD) field alternately.

Maximum Duration of Test:

1 mn

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	IDENTITY REQUEST

6	MS -> SS	IDENTITY RESPONSE	N(SD) = 0

7	SS -> MS	IDENTITY REQUEST

Steps 7, 8, 9 and 10 are repeated 5

times.

8	MS -> SS	IDENTITY RESPONSE	N(SD) = 1.

9	SS -> MS	IDENTITY REQUEST

10	MS -> SS	IDENTITY RESPONSE	N(SD) = 0.

11	SS -> MS	CHANNEL RELEASE	After the sending of this message,

the SS waits the disconnection of

the main signalling link.