26.5	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions

Abbreviations for message names used in this section:

CIPH MOD CMD	=	CIPERING MODE COMMAND

IMM ASSIGNMENT	=	IMMEDIATE ASSIGNMENT

IMM ASS REJ	=	IMMEDIATE ASSIGNMENT REJECT

LOC UPD REQ	=	LOCATION UPDATING REQUEST

LOC UPD ACC	=	LOCATION UPDATING ACCEPT

PAG REQ TYPE 1	=	PAGING REQUEST TYPE 1

[Editorial note: Introductions to sections containing justification are missing.

]

26.5.1	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / unknown protocol discriminator

[Dissimilarities to 11.10 phase 1

-	editorial notes :

-	The test in this section was in section 26.5.1.1 in phase 1.

-	It is now specified that the MS is paged with TMSI in a PAGING REQUEST TYPE 1 message.

-	It is now specified that the message with unknown PD is sent in state U10 of a MT call.

-	The test is at the moment only applicable to MS supporting TCH and call control protocol. For other MSes a similar test should be added.

]

An MS ignores messages with unknown protocol discriminator. This allows for the introduction of new messages which will be ignored by MS of earlier phases.

Purpose:

To verify that a MS supporting TCH and the call control protocol ignores a message containing an undefined protocol discriminator in the special case of a message coded otherwise like a CC STATUS ENQUIRY message received by the MS having a mobile terminating call in CC-state U10, "active".

Conformance Requirement(s):

If the mobile station receives a standard L3 message with a protocol discriminator different from those specified in Table 9.2/GSM 04.07, the mobile station shall ignore the message.

Reference(s):

TS GSM 04.07, section 9.2.1.

Related PICS/PIXIT Statement(s):

-	Call control protocol supported for at least one BC (Y/N).

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has been paged and an RR connection has been established.

If the MS supports the call control protocol, the test may alternatively be performed with the MS having a mobile terminating call in the CC-state U10, "active".

Foreseen Final State of the MS:

Same as in the initial conditions.

Method of Test:

The SS sends a message to the MS which is coded like a CC STATUS ENQUIRY message relating to the active call except for the fact that the protocol discriminator of the message is undefined.

Maximum Duration of Test:

11 seconds.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	UNKNOWN MESSAGE

2	SS		The SS waits between 5 and 10 seconds

verifying during this

period that the MS does not

send a L3 message on the main

signalling link.

Specific Message Contents:

UNKNOWN MESSAGE

Information element			Value/remark

Protocol discriminator			0000

TI flag					transaction originated by SS

TI value				TI value of the active call if the test is performed in state

U10 otherwise the value is arbitary.

Message Type			H'34

26.5.2	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / TI and skip indicator

[Dissimilarities to 11.10 phase 1 - editorial notes :

]

26.5.2.1	TI and skip indicator / RR

[Dissimilarities to 11.10 phase 1 - editorial notes :

- It is now specified how long and on which channel the SS verifies "no answer".

]

The MS ignores RR messages with skip indicator different to 0. This allows for the introduction of new RR messages which will be ignored by MS of earlier phases, especially on the downlink CCCH and BCCH.

Purpose:

To verify that the MS ignores an RR message with skip indicator different from H'0 in the special case of a PAGING REQUEST TYPE 1 message received in the MM-state "idle, updated" and in RR-idle mode.

[

Comment GSM3: should also be tested with RR message during an active call, using the TI that is used for CC for the active call

]

Conformance Requirement(s):

A radio resource message received with skip indicator different from 0000 shall be ignored.

Reference(s):

TS GSM 04.08, section 10.3.1.

Related PICS/PIXIT Statement(s):

None.

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle, updated" and in RR-idle mode. It has a valid TMSI.

Foreseen Final State of the MS:

The MS is in the MM-state "idle, updated" and in RR-idle mode. It has a valid TMSI.

Method of Test:

For every binary value x in the range 0001 - 0110 (binary) and for binary value x = 1000, the following procedure is performed: The SS sends a PAGING REQUEST TYPE 1 message to the MS with skip indicator set to x. It is verified that the MS doesn't answer to the paging request message.

Maximum Duration of Test:

5 seconds for each execution.

Expected Sequence:

The sequence is executed for execution counter k = 1,2,3,4,5,6,8.

[Why not for others <> 0?]

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	The value of the

skip indicator IE is the binary

encoding of k.

2	SS 		During 3 seconds the

SS verifies that the MS does not

send any message on the RACH.

Specific Message Contents:

None.

26.5.2.2	TI and skip indicator / MM

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The MM-active state is clarified.

- The test is at the moment only applicable to MSes supporting call control.

]

The MS ignores MM messages with skip indicator different to 0. This allows for the introduction of new MM messages which will be ignored by MS of earlier phases.

Purpose:

To verify that the MS ignores an MM message with skip indicator different from H'0 in the special case of an MS supporting the call control protocol and an IDENTITY REQUEST message received in the active state of a mobile terminating call.

Conformance Requirement(s):

A mobility management message received with skip indicator different from 0000 shall be ignored.

Reference(s):

TS GSM 04.08, section 10.3.1.

Related PICS/PIXIT Statement(s):

Call control protocol supported (Y/N).

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a mobile terminating call in CC-state U10, "active", or alternatively, the MS has been paged and an RR connection has been established.

Foreseen Final State of the MS:

Same as in the initial conditions.

Method of Test:

For every binary value x in the range 0001 - 0110 and for the binary value x = 1000, the following procedure is performed: The SS sends a IDENTITY REQUEST message to the MS with skip indicator set to x. It is verified during 5 seconds that the MS doesn't answer to the IDENTITY REQUEST message.

[Why not for other TIs <> 0? Especially the value of the active CC transaction is interesting; actually this value is only checked if the SS assigned 000.]

Maximum Duration of test:

15 seconds.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	IDENTITY REQUEST	Skip indicator IE has value H'1.

2	SS 		The SS starts verifying that the

MS does not send any L3 message

on the main signalling link.

This verification continues

until step 16 of this test

sequence.

3	SS		The SS waits 1 second.

4	SS -> MS	IDENTITY REQUEST	Skip indicator IE has value H'2.

5	SS		The SS waits 1 second.

6	SS -> MS	IDENTITY REQUEST	Skip indicator IE has value H'3.

7	SS		The SS waits 1 second.

8	SS -> MS	IDENTITY REQUEST	Skip indicator IE has value H'4.

9	SS		The SS waits 1 second.

10	SS -> MS	IDENTITY REQUEST	Skip indicator IE has value H'5.

11	SS		The SS waits 1 second.

12	SS -> MS	IDENTITY REQUEST	Skip indicator IE has value H'6.

13	SS		The SS waits 1 second.

14	SS -> MS	IDENTITY REQUEST	Skip indicator IE has value H'8.

15	SS		The SS waits 5 seconds.

16	SS		The SS stops verifying that the

MS does not send any L3 message

on the main signalling link.

Specific Message Contents:

None.

[In phase 1  it was required that the identity type IE of all IDENTITY REQUEST messages of this test sequence indicates IMSI; this is however not relevant for the test!].

26.5.2.3	TI and skip indicator / CC

[Dissimilarities to 11.10 phase 1 - editorial notes :

- Test at the moment only applicable to MS supporting call control.

Action points:

Define in 04.08 when to use which location in cause IE, and that in specified cases the coding standard of GSM must be used; and that the MS shall not include octet 3a if coding standard is GSM (whereas for the receiver other rules apply).

Correct 8.3 of 04.08:

c) When a SETUP message is received specifying a transaction identifier which is not recognized as relating to an active call or to a call in progress, and with a transaction identifier flag incorrectly set to "1", this message shall be ignored.

Define in 11.10 that for indicated IEs the complete coding must be correct. Esp. IEI and length.

]

Purpose

a)	To verify that the MS having a mobile terminating call in CC-state U10, "active", on receipt of a DISCONNECT message which includes a transaction identifier which is not recognized as relating to an active call or a call in progress, sends a RELEASE COMPLETE message with cause value #81 and refering to the latter TI without changing the state of the active call (this is verified by use of the status enquiry procedure).

b)	To verify that the MS having a mobile terminating call in CC-state U10, "active", on receipt of a

b1)	RELEASE COMPLETE message which includes a transaction identifier which is not recognized as relating to an active call or a call in progress, or a

b2)	SETUP message with TI flag refering to a transaction originated by the MS (in the special case where the TI value is equal to the TI value relating to the active call), or a

b3)	SETUP message with TI refering to the active call,

ignores that message without changing the state of the active call (this is verified by use of the status enquiry procedure).

The test is only applicable to an MS supporting the call control protocol for at least one BC.

Conformance Requirement(s):

a)	Whenever any call control message except SETUP, RELEASE COMPLETE or STATUS is received specifying a transaction identifier which is not recognized as relating to an active call or to a call in progress, the receiving entity shall send a RELEASE COMPLETE message with cause value #81 "invalid transaction identifier value" using the received transaction identifier value and remain in the Null state.

b1)	When a RELEASE COMPLETE message is received specifying a transaction identifier which is not recognized as relating to an active call or to a call in progress, the MM-connection associated with that transaction identifier shall be released.

b2)	When a SETUP message is received with a transaction identifier flag set to "1", this message shall be ignored.

b3)	When a SETUP message is received specifying a transaction identifier which is recognized as relating to an active call or to a call in progress, this SETUP message shall be ignored.

Reference(s):

TS GSM 04.08, section 8.3.

Related PICS/PIXIT Statement(s):

Call control protocol supported for at least one BC (Y/N).

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a mobile terminating call in CC-state U10, "active". No other call is active or in progress.

Foreseen Final State of the MS:

The MS has a mobile terminating call in CC-state U10, "active". No other call is active or in progress.

Method of Test:

The SS sends a DISCONNECT message to the MS with a TI not relating to the active call. The MS shall respond with a RELEASE COMPLETE message including cause value #81 and specifying the same transaction. By means of the status enquiry procedure the SS chaecks that the CC-state of the active call did not change.

Then the SS sends the following call control messages to the MS;

-	a RELEASE COMPLETE message, where the TI does not refer to the active call,

-	a SETUP message with TI flag set to 1,

-	a SETUP message with TI refering to the active call.

Each time the SS verifies that the MS does not respond to the message and each time the SS verifies by means of the status enquiry procedure that the CC-state of the active call has not been changed.

Maximum Duration of Test:

40 seconds.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	TI flag = 0; TI does not refer

to the active call.

2	MS -> SS	RELEASE COMPLETE	TI flag = 1; TI value is equal

to TI value received in step 1;

Cause IE indicates cause value

#81.

3	SS -> MS	STATUS ENQUIRY	TI refers to the active call.

4	MS -> SS	STATUS	TI refers to the active call;

Cause IE indicates cause value

#30.

Call state IE indicates state

U10

5	SS -> MS	RELEASE COMPLETE	TI flag = 0; TI does not refer

to the active call.

SS		The SS verifies during 5 seconds

that the MS does not send any L3

message on the main signalling

link.

SS -> MS	STATUS ENQUIRY	TI refers to the active call.

MS -> SS	STATUS	TI refers to the active call;

Cause IE indicates cause value

#30.

Call state IE indicates state

U10

SS -> MS	SETUP	TI flag = 1; TI value is equal

to TI value of the active call.

SS		The SS verifies during 5 seconds

that the MS does not send any L3

message on the main signalling

link.

SS -> MS	STATUS ENQUIRY	TI refers to the active call.

MS -> SS	STATUS	TI refers to the active call;

Cause IE indicates cause value

#30.

Call state IE indicates state

U10

SS -> MS	SETUP	TI flag = 0; TI refers

to the active call.

SS		The SS verifies during 5 seconds

that the MS does not send any L3

message on the main signalling

link.

SS -> MS	STATUS ENQUIRY	TI refers to the active call.

MS -> SS	STATUS	TI refers to the active call;

Cause IE indicates cause value

#30.

Call state IE indicates state

U10

Specific Message Contents:

None.

26.5.3	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / undefined or unexpected message type

26.5.3.1	Undefined or unexpected Message type / undefined message type / CC

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The MS is no more allowed to use cause value #98 in the first STATUS message.

- It is clarified how the undefined message type may look like.

- The test is only applicable to MSes supporting the CC protocol.

]

Purpose

To verify that a MS supporting the call control protocol for at least one BC, having a mobile terminating call in CC-state U10, "active", on receipt of a message with CC protocol discriminator and message type "1111 1111", returns a STATUS message with cause value #97 to the peer CC entity without changing the state of the active call (this is verified by use of the status enquiry procedure.)

Conformance Requirement(s):

If the Mobile Station receives a message with message type not defined for the PD, it shall ignore the message except for the fact that, if an RR-connection exists, it returns a status message (STATUS, RR-STATUS or MM-STATUS depending on the protocol discriminator) with cause value #97 "message type non-existent or not implemented".

Reference(s):

TS GSM 04.08, section 8.4

Related PICS/PIXIT Statement(s):

Call control protocol supported for at least one BC (Y/N).

Initial Conditions:

System Simulator:

cell, default parameters.

Mobile Station:

he MS has a mobile terminating call in CC-state U10, "active".

Foreseen Final State of the MS:

The MS has a mobile terminating call in CC-state U10, "active".

Method of Test:

The SS sends a message to the MS the PD of which refers to call control, the TI of which refers to the active call, and the message type of which is undefined in the call control protocol (however bit 7 and 8 of the message type are "0"). The SS then checks that the MS responds with a STATUS message specifying cause value #97. THe SS then sends a STATUS ENQUIRY message to the MS and verifies that the MS responds with a STATUS message specifying cause value #30 and call state U10, "active".

Maximum Duration of Test:

10 s.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	see comments	PD = "call control; call

related SS messages"

TI is that of the active call

Message type is undefined for

call control, bit 7 and 8 of the

message type are "0"

2	MS -> SS	STATUS	Cause IE indicates cause value

#97.

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	TI refers to the active call;

Cause IE indicates cause value

#30.

Call state IE indicates state

U10

Specific Message Contents:

None.

26.5.3.2	Undefined or unexpected message type / undefined message type / MM

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The MS is no more allowed to use cause value #98 in the MM-STATUS message.

- The test is only applicable to MSes supporting the CC protocol. For other MSes, a test has to be added.

]

Purpose

To verify that a MS supporting the call control protocol for at least one BC, having a mobile terminating call in CC-state U10, "active", on receipt of a message with MM protocol discriminator and message type undefined for the mobility management protocol, returns an MM-STATUS message with reject cause value #97 without changing the state of the active call (this is verified by use of the status enquiry procedure.) This is tested in the special case where the CC TI has value 0 (so that it has the same encoding as the skip indicator when sent from the SS) and where the message type has the same encoding as DISCONNECT in CC.

Conformance Requirement(s):

If the Mobile Station receives a message with message type not defined for the PD, it shall ignore the message except for the fact that, if an RR-connection exists, it returns a status message (STATUS, RR-STATUS or MM-STATUS depending on the protocol discriminator) with cause value #97 "message type non-existent or not implemented".

Reference(s):

TS GSM 04.08, section 8.4

Related PICS/PIXIT Statement(s):

Call control protocol supported for at least one BC (Y/N).

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a mobile terminating call in CC-state U10, "active". The TI of that mobile terminating call has value 0.

Foreseen Final State of the MS:

The MS has a mobile terminating call in CC-state U10, "active".

Method of Test:

The SS sends a message to the MS the PD of which refers to mobility management, the skip indicator of which is "0000", and the message type of which is "0010 0101". The SS then checks that the MS responds with an MM-STATUS message specifying reject cause value #97. The SS then sends a STATUS ENQUIRY message to the MS and verifies that the MS responds with a STATUS message specifying cause #97 and call state U10, "active".

Maximum Duration of Test:

10 s.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	see comments	PD = "mobility management

messages"

Skip indicator = "0000"

Message type = "0010 0101"

rest of the message is

H'02

H'E0

H'90

2	MS -> SS	MM-STATUS	Reject cause IE indicates

reject cause value #97.

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	TI refers to the active call;

Cause IE indicates cause value

#30.

Call state IE indicates state

U10

Specific Message Contents:

None.

26.5.3.3	Undefined or unexpected message type / undefined message type / RR

[Dissimilarities to 11.10 phase 1 - editorial notes :

- New test.

]

Purpose

To verify that an MS in RR connected mode on receipt of a message with RR protocol discriminator and message type undefined for the RR protocol, returns an RR-STATUS message with reject cause value #97 without changing its state (this is checked by observing that the MS does not send L3 messages.)

Conformance Requirement(s)

If the Mobile Station receives a message with message type not defined for the PD, it shall ignore the message except for the fact that, if an RR-connection exists, it returns a status message (STATUS, RR-STATUS or MM-STATUS depending on the protocol discriminator) with cause value #97 "message type non-existent or not implemented".

Reference(s)

TS GSM 04.08, section 8.4

Related PICS/PIXIT statement(s)

- The MS supports at least one bearer capability (p = Y/N)

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has been paged and an RR connection has been established.

Forseen final state of the MS

The MS is in MM-idle updated state.

Method of test

The SS sends a message to the MS the PD of which refers to radio resources management, the skip indicator of which is "0000", and the message type of which is "0010 1010". The SS then checks that the MS responds with an RR-STATUS message specifying reject cause value #97. The SS then verifies during 5 seconds that the MS does not send a L3 message on the main signalling link but continues sending L2 fill frames on the main signalling link. Then the SS sends a SETUP message to the MS. This message specifies a BC that is supported by the MS, if there exists any; if the MS does not support any BC, the SETUP message specifies an arbitrary BC. The SS then verifies that the MS responds with a CALL CONFIRMED message if the SETUP had specified a BC supported by the MS, and that the MMS responds with a RELEASE COMPLETE message otherwise. Then the SS sends a CHANNEL RELEASE to the MS and waits for the disconnection of the main signalling link.

Maximum duration of test

15 s.

Expected sequence

Step	Direction	Message	Comments

1	SS->MS	see comments	PD = "radio resources

management messages"

Skip indicator = "0000"

Message type = "0010 0101"

rest of the message is

H'02

H'E0

H'90

2	MS->SS	RR-STATUS	RR cause IE indicates

RR cause value #97.

3	SS		During 5 seconds the SS

verifies that the MS does not

send a L3 message on the main

signalling link but still

continues to send L2 fill

frames on the main signalling

link.

4	SS->MS	SETUP  	If the MS supports at least

one BC (p = Y), the SETUP

specifies a bearer capability

supported by the MS.

Otherwise (p = N) the SETUP

message specifies any bearer

capability.

A5	MS->SS	CALL CONFIRMED	This message shall be sent by

the MS if p = Y.

B5	MS->SS	RELEASE COMPLETE	This message shall be sent by

the MS if p = N.

6	SS->MS	CHANNEL RELEASE	The SS waits for disconnection

of the main signalling link.

Specific message contents

None.

26.5.3.4	Undefined or unexpected message type / unexpected message type / CC

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The MS is no more allowed to use cause value #97 in the first STATUS message.

- The test is only applicable to MSes supporting the CC protocol.

- Tests for inopportune messages in other protocols are missing.

]

Purpose

To verify that a MS supporting the call control protocol for at least one BC, having a call in CC-state U10, "active", on receipt of an inopportune CC message, returns a STATUS message with reject cause value #98 without changing the state of the active call (this is verified by use of the status enquiry procedure.) This is tested in the special case where the inopportune CC message is a CALL PROCEEDING message relating to the active call.

Conformance Requirement(s):

If the Mobile Station receives a message not consistent with the protocol state, the Mobile Station shall ignore the message except for the fact that, if an RR-connection exists, it returns a status message (STATUS, RR-STATUS or MM-STATUS depending on the protocol discriminator) with cause value #98 "Message type not compatible with protocol state".

Reference(s):

TS GSM 04.08, section 8.4

Related PICS/PIXIT Statement(s):

Call control protocol supported for at least one BC (Y/N).

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a call in CC-state U10, "active".

Foreseen Final State of the MS:

The MS has a call in CC-state U10, "active".

Method of Test:

The SS sends a CALL PROCEEDING message to the MS. The SS then checks that the MS responds with a STATUS message specifying reject cause value #98. The SS then sends a STATUS ENQUIRY message to the MS and verifies that the MS responds with a STATUS message specifying cause #30 and call state U10, "active".

Maximum Duration of Test:

10 s.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	CALL PROCEEDING

2	MS -> SS	STATUS	Cause IE indicates cause

value #98.

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	TI refers to the active call;

Cause IE indicates cause value

#30.

Call state IE indicates state

U10

Specific Message Contents:

None.

26.5.4	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / unforeseen information elements in the non-imperative message part

[Dissimilarities to 11.10 phase 1 - editorial notes :

- Strange section 26.5.4.1 b) should be amended or deleted in phase 1!.

- Tests are missing that when repetition of information elements is specified, only the contents of specified repeated information elements shall be handled, and if the limit on repetition of information elements is exceeded, the contents of information elements appearing first up to the limit of repetitions shall be handled and all subsequent repetitions of the information element shall be ignored.

]

26.5.4.1	Unforeseen information elements in the non-imperative message part / duplicated information elements

[Dissimilarities to 11.10 phase 1 - editorial notes :

- Tests are missing for the case of a repeated IE with format T or TV. This is important because the decoding mechanism is unclear.

- The verification of no response to the first paging is now specified in seconds rather than in terms of a time our value.

- After return to idle mode the time guaranteeing the MS becoming pageable is given explicitely.

]

Purpose

To verify that the MS ignores an unforeseen second occurrence of an information element with format T, TV, or TLV in the special case of the mobile identity IE which has format TLV in the LOCATION UPDATE ACCEPT message.

Conformance Requirement(s):

If an information element with format T, TV, or TLV is repeated in a message in which repetition of the information element is not specified, only the contents of the information element appearing first shall be handled and all subsequent repetitions of the information element shall be ignored.

Reference(s):

TS GSM 04.08, section 8.6.3.

Related PICS/PIXIT Statement(s):

-

Initial Conditions:

System Simulator:

2 cells A and B belonging to different location areas, default parameters.

Mobile Station:

The MS is in the MM-state "idle, updated" and in RR-idle mode, listening to the BCCH/CCCH of cell A. It has a valid TMSI.

Foreseen Final State of the MS:

The MS is in the MM-state "idle, updated" and in RR-idle mode, listening to the BCCH/CCCH of cell B. It does not have a valid TMSI.

Method of Test:

The RF level of cell A is lowered until the MS selects cell B (according to the cell-selection procedures of TS GSM 05.08). The MS shall establish an RR connection and initiate the normal location updating procedure (using TMSI). The SS responds to the location update request with the LOCATION UPDATE ACCEPT message containing the mobile identity IE specifying the IMSI of the MS followed by an additional mobile identity IE specifying the TMSI that was assigned to the MS in the initial conditions (i.e. duplication of information element).

The SS then pages the MS using the PAGING REQUEST TYPE 1 message including the TMSI which was previously used in the LOCATION UPDATE ACCEPT message. The SS then verifies during 5 seconds that the MS doesn't answer to paging. The SS then pages the MS with its IMSI. The SS verifies that the MS responds on cell B by initiating the immediate assignment procedure using the CHANNEL REQUEST message.

Maximum Duration of Test:

20 s.

Expected Sequence:

During 3 seconds the SS verifies that the MS does not send any message on the RACH.

Step	Direction	Message	Comments

1	SS 		The Rf level of cell A is

lowered until the MS selects

cell B.

2	MS -> SS	LOC UPD REQ	Mobile identity 1 IE specifies

the TMSI of the MS.

3	SS -> MS	LOC UPD ACC	(see below)

4	SS		The SS waits at least 5 s to

give the MS time to become

pageable

4	SS -> MS	PAG REQ TYPE 1	Mobile identity 1 IE specifies

the TMSI of the MS.

Mobile identity 2 is missing.

5	SS		The SS waits at least 5 seconds

During that period the

SS verifies that the MS does not

send any message on the RACH.

6	SS -> MS	PAG REQ TYPE 1	Mobile identity 1 IE specifies

the IMSI of the MS.

Mobile identity 2 is missing.

7	MS -> SS	CHANNEL REQUEST	Establishment cause = answer

to paging.

Specific Message Contents:

LOCATION UPDATE ACCEPT

Information element			value/remark

location area identification		LAI of cell B

Mobile identity 			coded TLV, specifies the IMSI of the MS

Type of identity			IMSI

Odd/even indication		corresponding to IMSI

Identity digit 1 etc.		corresponding to IMSI

Mobile identity (duplication)		coded TLV

Type of identity			TMSI of the MS

Odd/even indication		corresponding to TMSI

Identity digit 1 etc.		corresponding to TMSI

26.5.5.	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / non-semantical mandatory ie errors

[Dissimilarities to 11.10 phase 1 - editorial notes :

]

26.5.5.1.	Non-semantical mandatory IE errors / RR

26.5.5.1.1	Non-semantical mandatory IE errors / RR / missing mandatory IE error

26.5.5.1.1.1	Non-semantical mandatory IE errors / RR / missing mandatory IE error / special case

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The verification method of the MS releasing the RR connection is clarified: it consists in observing a L2 DISC frame.

- The test was numbered 26.5.5.1 procedure/requirement (i) inphase 1.

- Paging is now specified to use PAGING REQUEST TYPE 1 (hough this has no impact on the test).

]

The MS shall accept a CHANNEL RELEASE message whether it contains an RR cause or not. This allows for the shortening of the message in the future.

Purpose

To verify that the MS in RR connected mode releases the connection upon receipt of a CHANNEL RELEASE message with missing RR cause (which is "mandatory" in that message).

Conformance Requirement(s0:

When on receipt of a message a "missing mandatory IE" error is diagnosed the MS shall proceed as follows: If the message is a CHANNEL RELEASE message, the actions taken shall be the same as specified for a normal RR-connection release.

Reference(s):

TS GSM 04.08, section 8.5.

Related PICS/PIXIT Statement(s):

-

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle, updated" and in RR-idle mode. It has a valid TMSI.

Foreseen Final State of the MS:

The MS is in the MM-state "idle, updated" and in RR-idle mode. It has a valid TMSI.

Method of Test:

A mobile terminating RR connection is established. Then the SS sends a CHANNEL RELEASE message in which the RR cause IE is missing. It is verified that the MS releases the main signalling link by sending a L2 DISC frame. The main signalling link release is then completed.

Maximum Duration of Test:

10 s.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMM ASSIGNMENT

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	CHANNEL RELEASE	The mandatory RR cause IE is

missing (the message consists

only of protocol discriminator,

skip indicator, and

message type).

6	MS -> SS		The main signalling link is

released (this is observed by a

L2 DISC frame sent from the MS

to the SS).

Specific Message Contents:

None.

26.5.5.1.1.2	Non-semantical mandatory IE errors / RR / missing mandatory IE error / general case

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was numbered 26.5.5.1 procedure/requirement (ii) in phase 1.

- The RR cause value reported by the MS is 96 instead of 100 in phase 2.

]

In the general case, the MS has to report an RR message with missing mandatory IE bu the use of an RR-STATUS message, but otherwise to ignore it. This is  a recovery mechanism for unforeseen states.

Purpose

To verify that the MS in RR connected mode ignores a ciphering mode command message in which the ciphering mode setting IE and cipher response IE are messing except for the fact that it returns a RR-STATUS message.

Conformance Requirement(s):

When on receipt of a message a "missing mandatory IE" error is diagnosed the MS shall proceed as follows: If the message is not one of the messages listed in sections 8.5.1, 8.5.2, and 8.5.3 of TS GSM 04.08, the Mobile Station shall ignore the message except for the fact that, if an RR-connection exists, it returns a status message (STATUS, RR-STATUS or MM-STATUS depending on the protocol discriminator) with cause value #96 "invalid mandatory information".

Reference(s):

TS GSM 04.08, section 8.5.

Related PICS/PIXIT Statement(s):

-

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle, updated" and in RR-idle mode. It has a valid TMSI.

Foreseen Final State of the MS:

The MS is in RR-connected mode.

Method of Test:

A mobile terminating RR connection is established. Then the SS sends a ciphering mode command message in which the ciphering mode setting IE and cipher response IE are messing. The SS verifies that the MS does not start ciphering and returns a RR-STATUS message.

Maximum Duration of Test:

10 s.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMM ASSIGNMENT

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	CIPH MOD CMD	The mandatory

ciphering mode setting IE and

cipher response IEs are missing.

6	MS -> SS	RR-STATUS	RR cause IE specifies

RR cause value #96.

Specific Message Contents:

None.

26.5.5.1.2	Non-semantical mandatory ie errors / RR / comprehension required

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was numbered 26.5.6.1.3 procedure/requirement 1 in phase 1.

- Test is at the moment only applicable to MSes supporting the CC protocol. For other MSes a test should be added.

- The MS must report cause value #96 in ph. 2.

]

Purpose

To verify that the MS having an RR-connection established ignores a HANDOVER COMMAND message containing in the non-imperative part an IE encoded as comprehension required except for the fact that it returns a RR-STATUS message with cause # 96 "invalid mandatory information".

Conformance Requirement(s):

When an RR message containing an IE unknown in the message, but encoded as "comprehension required" (see Table 10.13/GSM O4.08, Table 10.13/GSM 04.08, and Table 10.45/GSM 04.08) is received, the MS shall proceed as follows: When the message is not one of the messages listed in TS GSM 04.08 sections 8.5.1, 8.5.2, and 8.5.3, the Mobile Station shall ignore the message except for the fact that, if an RR-connection exists, it returns a RR-STATUS message with cause value #96 "invalid mandatory information".

Reference(s):

TS GSM 04.08, section 8.5.

Related PICS/PIXIT Statement(s):

MS supports call control protocol for at least one BC (Y/N).

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has an MT call in state U10, "active"; or alternatively, the MS has been paged and an RR-connection has been established.

Foreseen Final State of the MS:

As in the initial conditions.

Method of Test:

The SS sends a HANDOVER command message containing in the non-imperative part an IE encoded as comprehension required. The SS verifies that the MS returns a RR-STATUS message with cause value #96 without changing the dedicated channel.

Maximum Duration of Test:

10 s.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	See below.

2	MS -> SS	RR-STATUS	Sent on the old channel.

RR cause IE specifies

RR cause value #96.

Specific Message Contents:

HANDOVER COMMAND

Information element			value/remark

cell description			as required

channel description			as required

handover reference			as required

power command			as required

comprehension required IEI		0000	0000

length				0000	0001

unrecognised IE contents		xxxx	xxxx

26.5.5.2	Non-semantical mandatory IE errors / MM

26.5.5.2.1	Non-semantical mandatory IE errors / MM / syntactically incorrect mandatory IE

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was II.3.5.5.2 in ph.1.

- The test is at the moment only applicable to an MS supporting the CC protocol. A corresponding test for other MSes is to be written.

- The reject cause to be reported by the MS is 96 in ph. 2, not 100.

]

The MS shall ignore MM messages with syntactically incorrect mandatory IE. This allows to use reserved values in later phases.

Purpose

To verify that the MS having an RR connection established, ignores an MM message containing a syntactically incorrect mandatory IE except for the fact that it returns an MM-STATUS message with reject cause #98. This is tested in the special case of an IDENTITY REQUEST message in which the (mandatory) identity type IE specifies a reserved value for the type of identity. For this test purpose, one of the test cases 26.5.5.2.1a or 26.5.5.2.1b has to be performed altenatively.

26.5.5.2.2	Non-semantical mandatory IE errors / MM / syntactically incorrect mandatory IE

For an MS, one of the test cases 26.5.5.2.1a or 26.5.5.2.1b is performed alternatively. Test 26.5.5.2.1a is only applicable for an MS supporting at least one BC, whereas test 26.5.5.2.1b is applicable to all types of MSes.

Test purpose:

To verify that an MS supporting at least one BC, having a CC entity in state U10, "active", ignores an MM message with syntactically incorrect IE except for the fact that it sends an MM-STATUS message with reject cause #96. This is tested in the special case of an IDENTITY REQUEST message in which the (mandatory) identity type IE specifies a reserved value for the type of identity; that the MS otherwise ignores the message is checked by means of the status enquiry procedure.

Conformance Requirement(s):

When an MM message containing a syntactically incorrect mandatory IE is received, the Mobile Station shall ignore the message except for the fact that, if an RR-connection exists, it returns a MM-STATUS message with cause value #96 "invalid mandatory information".

Reference(s):

TS GSM 04.08, section 8.5.

Related PICS/PIXIT Statement(s):

MS supports call control protocol for at least one BC (Y/N)

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a mobile terminating call in the CC-state U10, "active".

Foreseen Final State of the MS:

The MS has a mobile terminating call in the CC-state U10, "active".

Method of Test:

The SS sends an identity request message in which the (mandatory) identity type IE specifies a reserved value for the type of identity. The SS verifies that the MS returns an MM-STATUS message specifying cause value #96 but does not change its state (this is verified by use of the status enquiry procedure).

Maximum Duration of Test:

10 s.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	IDENTITY REQUEST	The identity type IE is encoded

as "1111" (so that the type of

identity contains the reserved

value "111").

2	MS -> SS	MM STATUS	Reject cause IE indicates

reject cause value #96.

3	SS -> MS	STATUS ENQUIRY	TI refers to the active call.

4	MS -> SS	STATUS	TI refers to the active call;

Cause IE indicates cause value

#30.

Call state IE indicates state

U10.

Specific Message Contents:

None.

26.5.5.2.3	Non-semantical mandatory IE errors / MM / syntactically incorrect mandatory IE

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was is new in phase 2.

- For an MS supporting at least one BC, test 26.5.5.2.1 is used instead of this test.

]

For an MS, one of the test cases 26.5.5.2.1a or 26.5.5.2.1b is performed alternatively. Test 26.5.5.2.1a is only applicable for an MS supporting at least one BC, whereas test 26.5.5.2.1b is applicable to all types of MSes.

Test purpose

To verify that an MS having been paged and having an RR connection established ignores an MM message with syntactically incorrect IE except for the fact that it sends an MM-STATUS message with reject cause #96. This is tested in the special case of an IDENTITY REQUEST message in which the (mandatory) identity type IE specifies a reserved value for the type of identity; the fact that the MS otherwise ignores the message is checked by testing that it answers as usual to an incoming SETUP message.

Conformance requirement(s)

When an MM message containing a syntactically incorrect mandatory IE is received, the Mobile Station shall ignore the message except for the fact that, if an RR-connection exists, it returns an MM-STATUS message with cause value #96 "invalid mandatory information".

Reference(s)

TS GSM 04.08, section 8.5.

Related PICS/PIXIT statement(s)

MS supports call control protocol for at least one BC (Y/N)

Initial conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has been paged; an RR connection has been established.

Foreseen final state of the MS

The MS has been paged; an RR connection has been established.

Method of test

The SS sends an identity request message in which the (mandatory) identity type IE specifies a reserved value for the type of identity. The SS verifies that the MS returns an MM-STATUS message specifying cause value #96 but does not change its state; this is verified as follows:

The SS sends a SETUP message to the MS. This message specifies a BC that is supported by the MS, if there exists any; if the MS does not support any BC, the SETUP message specifies an arbitrary BC. The SS then verifies that the MS responds with a CALL CONFIRMED message if the SETUP had specified a BC supported by the MS, and that the MMS responds with a RELEASE COMPLETE message otherwise.

Then the SS sends a CHANNEL RELEASE to the MS and waits for the disconnection of the main signalling link.

Maximum duration of test

10 s.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	IDENTITY REQUEST	The identity type IE is encoded

as "1111" (so that the type of

identity contains the reserved

value "111").

2	MS -> SS	MM STATUS	Reject cause IE indicates

reject cause value #96.

3	SS->MS	SETUP  	If the MS supports at least

one BC (p = Y), the SETUP

specifies a bearer capability

supported by the MS.

Otherwise (p = N) the SETUP

message specifies any bearer

capability.

A4	MS->SS	CALL CONFIRMED	This message shall be sent by

the MS if p = Y.

B4	MS->SS	RELEASE COMPLETE	This message shall be sent by

the MS if p = N.

5	SS->MS	CHANNEL RELEASE	The SS waits for disconnection

of the main signalling link.

Specific message contents

None.

26.5.5.2.4	Non-semantical mandatory IE errors / MM / comprehension required

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was II.3.5.6.2, proc/req 1 in ph.1.

- Reject cause to be reported is 96 in ph 2.

]

The "comprehension required" mechanism allows for the introduction of essential new information elements into messages, such that a message is ignored and a report is sent if the new information element is not understood.

Purpose

To verify that the MS on receipt of an MM message containing an IE unknown in the message, but encoded as "comprehension required" ignores the message except for the fact that it returns an MM-STATUS message with cause value #96 "invalid mandatory information"; this in the special case of the MM message being a LOCATION UPDATING ACCEPT responding to a LOCATION UPDATING REQUEST from the MS.

Conformance Requirement(s):

When an MM message containing an IE unknown in the message, but encoded as "comprehension required" (see Table 10.13/GSM 04.08) is received, the MS shall ignore the message except for the fact that, if an RR-connection exists, it returns an MM-STATUS message with cause value #96 "invalid mandatory information".

Reference(s):

TS GSM 04.08, section 8.5.

Related PICS/PIXIT Statement(s):

-

Initial Conditions:

System Simulator:

The SS simulates two cells, A and B, belonging to different location areas, default parameters.

Mobile Station:

The MS is in the MM-state 'idle,updated' listening to the BCCH/CCCH of cell A. It has a valid TMSI.

Foreseen Final State of the MS:

The MS is in the MM-state 'idle,updated' listening to the BCCH/CCCH of cell B. It has a valid TMSI.

Method of Test:

The Rf level of cell A is lowered until the MS selects cell B. The SS verifies that the MS establishes an RR connection and performs the normal location updating procedure using its TMSI. The SS responds to the location updating request with the LOCATION UPDATING ACCEPT message containing an optional information element coded as "comprehension required". The SS verifies that the MS returns the MM-STATUS message with cause #96 in response to the LOCATION UPDATING ACCEPT. The SS then waits ofr the MS to abort the RR-connection. The SS verifies that the MS establishes a new RR connection and starts a new location updating procedure.

On receipt of the new LOCATION UPDATING REQUEST, the SS sends a correctly coded LOCATION UPDATING ACCEPT allocating a new TMSI.

The SS verifies that the MS sends a TMSI REALLOCATION COMPLETE message. The SS then initiates the RR connection release.

Maximum Duration of Test:

30 s.

Expected Sequence:

Step	Direction	Message	Comments

1	SS		The Rf level of cell A is

lowered until the MS selects

cell B.

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMM ASSIGNMENT

4	MS -> SS	LOC UPD REQ	The mobile identity IE specifies

the TMSI of the MS.

5	SS -> MS	LOC UPD ACC	See below.

6	MS -> SS	MM-STATUS	Reject cause IE specifies reject

cause value #96.

7	MS		The MS aborts the RR connection

(it initiates release of L2 on

SAPI 0.

8	MS -> SS	CHANNEL REQUEST

9	SS -> MS	IMM ASSIGNMENT

10	MS -> SS	LOC UPD REQ	The mobile identity IE specifies

the TMSI of the MS.

11	SS -> MS	LOC UPD ACC	see below

12	MS -> SS	TMSI REALLOCATION COMPLETE

13	SS -> MS	CHANNEL RELEASE	The RR connection is released.

Specific Message Contents:

LOCATION UPDATIG ACCEPT - first occurrence

Information element			value/remark

Location area identification		LAI of cell B

Comprehension required IEI		0000 0000

length				1

unrecognised IE contents		xxxx xxxx (arbitrary octet)

LOCATION UPDATIG ACCEPT - second occurrence

Information element			value/remark

Location area identification		specifies LAI of cell B

Mobile Identity			specifies a TMSI

26.5.5.3	Non-semantical mandatory IE errors / CC

26.5.5.3.1	Non-semantical mandatory IE errors / CC / missing mandatory IE

[Dissimilarities to 11.10 phase 1 - editorial notes :

Test in proc/req (i) does no more make sense in phase 2.

- A possible new test would be a comprehension reqired IE.

]

26.5.5.3.1.1	Non-semantical mandatory IE errors / CC / missing mandatory IE / disconnect message

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was 26.5.5.3 proc/req (ii) in ph. 1.

- The test is only applicable to an MS supporting the CC protocol.

- The reject cause to be reported by the MS is #96 in ph. 2, not #100.

- At the end of the test the MS is now brought to a stable state.

]

Purpose

To verify that the MS having an MT call in state U10, "active", on receipt of a DISCONNECT message in which the mandatory cause IE is missing shall return a RELEASE message with cause value #96 "invalid mandatory information".

Conformance Requirement(s):

When on receipt of a message a "missing mandatory IE" error is diagnosed, the MS shall proceed as follows: If the message is a DISCONNECT message, a RELEASE message shall be returned with cause value # 96 "invalid mandatory information" and normal call clearing applies.

Reference(s):

TS GSM 04.08, section 8.5.

Related PICS/PIXIT Statement(s):

MS supports call control protocol for at least one BC (Y/N).

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has an MT call in the CC-state U10, "active".

Foreseen Final State of the MS:

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Method of Test:

The SS sends a DISCONNECT message in which the (mandatory) cause IE is missing. The SS verifies that the MS returns a RELEASE message specifying cause value #96. The SS then sends a RELEASE COMPLETE message and performs the RR connection release.

Maximum Duration of Test:

15 s.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	The mandatory cause IE

is missing.

2	MS -> SS	RELEASE	The cause IE indicates

cause value #96

3	SS -> MS	RELEASE COMPLETE

4	SS -> MS	CHANNEL RELEASE	The RR connection is released.

Specific Message Contents:

None.

26.5.5.3.1.2	Non-semantical mandatory IE errors / CC / missing mandatory IE / general case

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was 26.5.5.3 proc/req (iv) in ph. 1.

- The test is only applicable to an MS supporting the CC protocol.

- The reject cause to be reported by the MS is #96 in ph. 2, not #100.

]

Purpose

To verify that the MS having an MT call in state U10, "active", on receipt of a STATUS message in which the mandatory cause IE and call state IE are missing shall ignore the message except for the fact that it return a STATUS message with cause value #96 "invalid mandatory information" (that the MS doesn't change state is checked by use of the status enquiry procedure).

Conformance Requirement(s):

When on receipt of a message a "missing mandatory IE" error is diagnosed, the MS shall proceed as follows: If the message is not a SETUP, RELEASE, DISCONNECT, RELEASE COMPLETE, HOLD REJECT or HOLS REJECT message, it shall ignore the message except for the fact that it returns a STATUS message specifying cause value #96.

Reference(s):

TS GSM 04.08, section 8.5.

Related PICS/PIXIT Statement(s):

MS supports call control protocol for at least one BC (Y/N).

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has an MT call in the CC-state U10, "active".

Foreseen Final State of the MS:

The MS has an MT call in the CC-state U10, "active".

method of test:

The SS sends a a STATUS message in which the mandatory cause IE and call state IE are missing. The SS verifies that the MS returns a STATUS message with cause value #96 "invalid mandatory information". Then the SS sends a STATUS ENQUIRY message and checksthat the MS returns a STATUS message indicating cause value#30 and call state U10, "active".

Maximum Duration of Test:

15 s.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	STATUS	The mandatory cause IE and

call state IE are missing.

2	MS -> SS	STATUS	The cause IE indicates

cause value #96

3	SS -> MS	STATUS ENQUIRY	TI refers to the active call.

4	MS -> SS	STATUS	TI refers to the active call;

Cause IE indicates cause value

#30.

Call state IE indicates state

U10

Specific Message Contents:

None.

26.5.5.3.2	Non-semantical mandatory IE errors / CC / comprehension required

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was II.3.5.6.3, proc/req 2 in ph.1.

- The behaviour of the MS has been changed: it must now report cause value #96.

]

Purpose

To verify that an MS supporting the call control protocol for at least one BC having a call control entity in state U3 ignores a CONNECT message containing in the non-imperative part an IE encoded as comprehension required except for the fact that it returns a STATUS message with cause value #96 "invalid mandatory information".

Conformance Requirement(s):

When a CC message containing an IE unknown in the message, but encoded as "comprehension required" (see TS GSM 04.08, Table 10.45/GSM 04.08) is received, the MS shall proceed as follows: When the message is not one of the messages listed in TS GSM 04.08 sections 8.5.1, 8.5.2, and 8.5.3, the Mobile Station shall ignore the message except for the fact that, if an RR-connection exists, it returns a STATUS message with cause value #96 "invalid mandatory information".

Reference(s):

TS GSM 04.08, section 8.5.

Related PICS/PIXIT Statement(s):

MS supports call control protocol for at least one BC (Y/N).

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a call contol entity in CC state U3.

Foreseen Final State of the MS:

The MS has a call contol entity in CC state U3.

Method of Test:

The SS sends a CONNECT message containing an optional information element coded as 'comprehension required'. The SS verifies that the MS returns a STATUS message specifying cause value #96 "invalid mandatory information". The SS checks by use of the status enquiry procedure that the MS did not change the state.

Maximum Duration of Test:

5 s.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	CONNECT	See below.

2	MS -> SS	STATUS	TI refers to the call in

progress; cause IE indicates

cause value #96.

3	SS -> MS	STATUS ENQUIRY	TI refers to the call in

progress.

4	MS -> SS	STATUS	TI refers to the call in

progress;

Cause IE indicates cause value

#30.

Call state IE indicates state

U3.

Specific Message Contents:

CONNECT

Information element			value/remark

Unknown IEI			0000 0000

length			1

unknown IE contents		xxxx xxxx (arbitrary octet)

26.5.6	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / unknown IE, comprehension not required

[Dissimilarities to 11.10 phase 1 - editorial notes :

]

26.5.6.1	Unknown information elements in the non-imperative message part / MM

26.5.6.1.1	Unknown IE, comprehension not required / MM / IE unknown in the protocol

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was II.3.5.6.2, proc/req 2 in ph.1. It was intended to be run after proc/req 1 of the same section.

- The behaviour of the MS has been changed: no more MM-STATUS message.

- It is not nice that the SS causes the MS abort the RR connection.

- The MS is brought to the idle state at the end.

- The ph. 1 test must be corrected so that the unknown IE has bit 8 = 0 and is undefined in 04.08. For phase 2 it is sufficient that the IE has bit 8 = 0 and is undefined in the message.

]

Purpose

To verify that the MS on receipt of an MM message containing an IE unknown in the message and unknown in the MM protocol which is not  encoded as "comprehension required" ignores that IE; this in the special case of the MM message being a LOCATION UPDATING ACCEPT responding to a LOCATION UPDATING REQUEST from the MS.

Conformance Requirement(s):

The MS shall ignore all IEs unknown in a message which are not encoded as "comprehension required".

Reference(s):

TS GSM 04.08, section 8.6.1.

Related PICS/PIXIT Statement(s):

-

Initial Conditions:

System Simulator:

The SS simulates two cells, A and B, belonging to different location areas, default parameters.

Mobile Station:

The MS is in the MM-state 'idle,updated' listening to the BCCH/CCCH of cell B. It has a valid TMSI.

Foreseen Final State of the MS:

The MS is in the MM-state 'idle, updated' listening to the BCCH/CCCH of cell A. It has a valid TMSI.

Method of Test:

The RF level of cell B is lowered until the MS selects cell A. The SS verifies that the MS establishes an RR connection and performs the normal location updating procedure using its TMSI. The SS responds to the location updating request with the LOCATION UPDATING ACCEPT message containing an optional information element not coded as 'comprehension required' the IE of which is unknown in the MM protocol. (The LOCATION UPDATING ACCEPT message does not contain a mobile identity IE.) Then the SS initiates the channel release procedure. After at least 20 seconds the SS pages the MS with the TMSI that the MS used in the LOCATION UPDATING REQUEST message. The SS verifies that the MS sends a CHANNEL REQUEST message on the RACH of cell A and responds with an IMMEDIATE ASSIGNMENT REJECT message.

Maximum Duration of Test:

40 s.

Expected Sequence:

Step	Direction	Message	Comments

1	SS		The Rf level of cell B is

lowered until the MS selects

cell A.

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMM ASSIGNMENT

4	MS -> SS	LOC UPD REQ	The mobile identity IE specifies

the TMSI of the MS.

5	SS -> MS	LOC UPD ACC	See below.

6	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

7	SS		The SS waits at least 20

seconds verifying that the MS does not send

any messages on the CCCH of cell A or B.

8	SS -> MS	PAG REQ TYPE 1	Sent in cell A.

The mobile identity IE specifies

the TMSI used by the MS in

step 5.

9	MS -> SS	CHANNEL REQUEST	Establishment cause indicates

"answer to paging".

10	SS -> MS	IMM ASS REJ

Specific Message Contents:

LOCATION UPDATING ACCEPT

Information element			value/remark

Location area identification		LAI of cell A

Unknown IEI			0110 1111

length			1

unknown IE contents		xxxx xxxx (arbitrary octet)

26.5.6.1.2	Unknown IE, comprehension not required / MM / IE unknown in the message

[Dissimilarities to 11.10 phase 1 - editorial notes :

- New test. Is identical to II.5.3.5.6.1.1 except for the unknown IEI in the LOCATION UPDATING message.

]

Purpose

To verify that the MS on receipt of an MM message containing an IE unknown in the message, but known in the MM protocol, which is not encoded as "comprehension required" ignores that IE; this in the special case of the MM message being a LOCATION UPDATING ACCEPT responding to a LOCATION UPDATING REQUEST from the MS.

Conformance requirement(s):

The MS shall ignore all IEs unknown in a message which are not encoded as "comprehension required".

Reference(s):

TS GSM 04.08, section 8.6.1.

Related PICS/PIXIT statement(s):

-

Initial conditions:

System Simulator:

The SS simulates two cells, A and B, belonging to different location areas, default parameters.

Mobile Station:

The MS is in the MM-state 'idle, updated' listening to the BCCH/CCCH of cell B. It has a valid TMSI.

Foreseen final state of the MS:

The MS is in the MM-state 'idle, updated' listening to the BCCH/CCCH of cell A. It has a valid TMSI.

Method of test:

The RF level of cell B is lowered until the MS selects cell A. The SS verifies that the MS establishes an RR connection and performs the normal location updating procedure using its TMSI. The SS responds to the location updating request with the LOCATION UPDATING ACCEPT message containing an optional information element not coded as 'comprehension required' the IEI of which is unknown in the message but used for a location area identification IE in other messages of the MM protocol. (The LOCATION UPDATING ACCEPT message does not contain a mobile identity IE.) Then the SS initiates the channel release procedure. After at least 20 seconds the SS pages the MS with the TMSI that the MS used in the LOCATION UPDATING REQUEST message. The SS verifies that the MS sends a CHANNEL REQUEST message on the RACH of cell A and responds with an IMMEDIATE ASSIGNMENT REJECT message.

Maximum duration of test:

40 s.

Expected sequence:

Step	Direction	Message	Comments

1	SS		The Rf level of cell B is

lowered until the MS selects

cell A.

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMM ASSIGNMENT

4	MS -> SS	LOC UPD REQ	The mobile identity IE specifies

the TMSI of the MS.

5	SS -> MS	LOC UPD ACC	See below.

6	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

7	SS		The SS waits at least 20

seconds verifying that the MS

does not send any message on

the CCCH of cell A or B.

8	SS -> MS	PAG REQ TYPE 1	The mobile identity IE specifies

the TMSI used by the MS in

step 5.

9	MS -> SS	CHANNEL REQUEST	Establishment cause indicates

"answer to paging".

10	SS -> MS	IMM ASS REJ

Specific message contents:

LOCATION UPDATING ACCEPT

Information element				value/remark

Location area identification			LAI of cell A

Unknown IEI				0001 0011

length				1

unknown IE contents			xxxx xxxx (arbitrary octet)

26.5.6.2	Unknown information elements in the non-imperative message part / CC

26.5.6.2.1	Unknown information elements in the non-imperative message part / CC / Call establishment

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was II.3.5.6.3, proc/req 1 in ph.1.

- The behaviour of the MS has been changed: no more STATUS message.

- The ph. 1 test must be corrected so that the unknown IE has bit 8 = 0 and is undefined in 04.08. For phase 2 it is sufficient that the IE has bit 8 = 0 and is undefined in the message.

]

Purpose

To verify that an MS supporting the CC protocol for at least one BC receiving a CC message containing an IE unknown in the message which is not encoded as "comprehension required" ignores that IE; this in the special case of the CC message being a CALL PROCEEDING message received by the MS in state U1.

Conformance Requirement(s):

The MS shall ignore all IEs unknown in a message which are not encoded as "comprehension required".

Reference(s):

TS GSM 04.08, section 8.6.1.

Related PICS/PIXIT Statement(s):

MS supports call control protocol for at least one BC (Y/N).

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a call control entity in CC state U1.

Foreseen Final State of the MS:

The MS has a call control entity in CC state U3.

Method of Test:

The SS sends a CALL PROCEEDING message containing an optional information element not coded as 'comprehension required' the IEI of which is unknown in the message, but used for a called party BCD number IE in other messages of the protocol. The SS verifies by use of the status enquiry procedure that the MS did not change the state.

Maximum Duration of Test:

30 s.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	CALL PROCEEDING	See below.

2	SS -> MS	STATUS ENQUIRY	TI refers to the call in

progress.

3	MS -> SS	STATUS	TI refers to the active call;

Cause IE indicates cause value

#30.

Call state IE indicates state

U3.

Specific Message Contents:

CALL PROCEEDING

Information element			value/remark

Unknown IEI			0101 1110

length			1

unknown IE contents		xxxx xxxx (arbitrary octet)

26.5.6.2.2	Unknown information elements in the non-imperative message part / CC / disconnect

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was II.3.5.6.3, proc/req 3 in ph.1.

- The behaviour of the MS has been changed: no more cause #99 required.

- The ph. 1 test must be corrected so that the unknown IE has bit 8 = 0 and is undefined in 04.08. For phase 2 it is sufficient that the IE has bit 8 = 0 and is undefined in the message.

]

Purpose

To verify that an MS supporting the CC protocol for at least one BC receiving a CC message containing an IE unknown in the message which is not encoded as "comprehension required" ignores that IE; this in the special case of a DISCONNECT message received by the MS in state U10.

Conformance Requirement(s):

The MS shall ignore all IEs unknown in a message which are not encoded as "comprehension required".

Reference(s):

TS GSM 04.08, section 8.6.1.

Related PICS/PIXIT Statement(s):

MS supports call control protocol for at least one BC (Y/N).

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a call control entity in CC state U10.

Foreseen Final State of the MS:

The MS has a call control entity in CC state U19.

Method of Test:

The SS sends a DISCONNECT message containing an optional information element not coded as 'comprehension required' the IEI of which is unknown in the message, but used for a connected number IE in other messages of the protocol. The SS verifies that the MS responds with a RELEASE message; the SS verifies by use of the status enquiry procedure that the MS has entered state U19.

Maximum Duration of Test:

5 s.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	See below.

2	MS -> SS	RELEASE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	Cause IE indicates cause value

#30.

Call state IE indicates state

U19.

Specific Message Contents:

DISCONNECT

Information element			value/remark

Unknown IEI			0100 1100

length			1

unknown IE contents		xxxx xxxx (arbitrary octet)

26.5.6.2.3	Unknown information elements in the non-imperative message part / CC / release

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was II.3.5.6.3, proc/req 4 in ph.1.

- The behaviour of the MS has been changed: no more cause #99 required.

- The ph. 1 test must be corrected so that the unknown IE has bit 8 = 0 and is undefined in 04.08. For phase 2 it is sufficient that the IE has bit 8 = 0 and is undefined in the message.

]

Purpose

To verify that an MS supporting the CC protocol for at least one BC receiving a CC message containing an IE unknown in the message which is not encoded as "comprehension required" ignores that IE; this in the special case of a RELEASE message received by the MS having sent in state U10 a DISCONNECT message.

Conformance Requirement(s):

The MS shall ignore all IEs unknown in a message which are not encoded as "comprehension required".

Reference(s):

TS GSM 04.08, section 8.6.1.

Related PICS/PIXIT Statement(s):

MS supports call control protocol for at least one BC (Y/N).

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a call control entity in CC state U10.

Foreseen Final State of the MS:

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Method of Test:

The MS is made to send a DISCONNECT message. The SS responds with a RELEASE message containing an optional information element not coded as 'comprehension required' the IEI of which is unknown in the message, but used for a high layer compatability IE in other messages of the protocol. The SS verifies that the MS responds with a RELEASE COMPLETE message; the SS then releases the RR connection.

Maximum Duration of Test:

10 s.

Expected Sequence:

Step	Direction	Message	Comments

1	MS		The MS is made to initiate call

clearing.

2	MS -> SS	DISCONNECT

3	SS -> MS	RELEASE	See below.

4	MS -> SS	RELEASE COMPLETE

5	SS -> MS	CHANNEL RELEASE	The RR connection is released.

Specific Message Contents:

RELEASE

Information element			value/remark

Unknown IEI			0111 1101

length			1

unknown IE contents		1 arbitrary octet

26.5.6.2.4	Unknown information elements in the non-imperative message part / CC / release complete

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was II.3.5.6.3, proc/req 5 in ph.1.

- The ph. 1 test must be corrected so that the unknown IE has bit 8 = 0 and is undefined in 04.08. For phase 2 it is sufficient that the IE has bit 8 = 0 and is undefined in the message.

]

Purpose

To verify that an MS supporting the CC protocol for at least one BC receiving a CC message containing an IE unknown in the message which is not encoded as "comprehension required" ignores that IE; this in the special case of a RELEASE COMPLETE message received by the MS in state U19.

Conformance Requirement(s):

The MS shall ignore all IEs unknown in a message which are not encoded as "comprehension required".

Reference(s):

TS GSM 04.08, section 8.6.1.

Related PICS/PIXIT Statement(s):

MS supports call control protocol for at least one BC (Y/N).

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a call control entity in CC state U10.

Foreseen Final State of the MS:

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Method of Test:

The SS sends a DISCONNECT message. The SS verifies that the MS responds with a RELEASE message. The SS answers with a RELEASE COMPLETE  message containing an optional information element not coded as 'comprehension required' the IEI of which is unknown in the message, but used for an auxiliary states IE in other messages of the protocol. The SS verifies that the MS releases the link after some time.

Maximum Duration of Test:

20 s.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT

3	MS -> SS	RELEASE

4	SS -> MS	RELEASE COMPLETE	See below.

5	MS		The MS aborts the RR connection

(it initiates release of L2 on

SAPI 0)

Specific Message Contents:

RELEASE COMPLETE

Information element			value/remark

Unknown IEI			0010 0100

length			1

unknown IE contents		1 arbitrary octet

26.5.7	Handling of unknown, unforeseen, and erroneous protocol data, and of parallel transactions / spare bits

26.5.7.1	Spare bits / RR

26.5.7.1.1	Spare bits / RR / paging channel

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The test was II.5.3.6.1.1 in ph.1.

- The postamble now brings the MS back to idle mode.

- In ph 1 the corresponding test should be modified so that the unknown IEI begins with 0 and is undefined in 04.08 ph1. A choice would be 0011 1111. In ph2 an IEI beginning with 0 but used in RR is more interesting, eg. 01100100.

- The test in ph 2 is modified to send at least one non-default octet in the P1 rest octets IE.

]

Purpose

To verify that the MS in the MM-state "idle, updated" and in RR-idle mode ignores the value of spare bits in the special case of the spare bits occurring in the P1 rest octets IE of a PAGING REQUEST TYPE 1 message. That the spare bits are ignored is checked by adressing the MS in that PAGING REQUEST message and verifying that the MS responds to that paging.

Conformance Requirement(s):

The MS shall ignore the value of spare bits.

Reference(s):

TS GSM 04.08, section 10.5.

Related PICS/PIXIT Statement(s):

None.

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Foreseen Final State of the MS:

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Method of Test:

The SS sends a PAGING REQUEST TYPE 1 message containing at least one octet in the P1 rest octets IE that is different from 0010 1011.

Maximum Duration of Test:

10 s.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	See below.

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMM ASS REJ

Specific Message Contents:

PAGING REQUEST TYPE 1

Information element			Value/remark

L2 pseudo length			k+3 where k is the sum of the length of

the mobile identity 1 IE

Page Mode				Normal paging

Channels needed for Mobiles 1 and 2

Channel (first)			Any channel

Channel (second)		(spare)

Mobile identity 1			IMSI or TMSI of MS under test

Mobile identity 2			Omitted

P1 rest octets			not all octets are "0010 1011"

26.5.7.1.2	Spare bits / RR / BCCH

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The postamble now brings the MS back to idle mode.

- The ph 1 test should be modified: only one octet but in both SI3 and 4!! (Only in SI 4 is not enough. Leading 1 in 1111 1111 means type 1 or 2.) Still better: delete test in ph. 1 and replace by a test where L2 fill frames are sent on the BCCH.

- The ph 2 test is based on the "rest octet scheme".

]

Purpose

To verify that the MS in the MM-state "idle, updated" and in RR-idle mode ignores the value of spare bits in the special case where these spare bits are contained in SI3 rest octets IE and SI4 rest octets IE. That the MS ignores the value of the spare bits is checked by changing the LAI in those message and observing the MS initiating a location update though the spare bits do not all have the default value.

Conformance Requirement(s):

The MS shall ignore the value of spare bits.

Reference(s):

TS GSM 04.08, section 10.5.

Related PICS/PIXIT Statement(s):

None.

Initial Conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Foreseen Final State of the MS:

The MS is in the MM-state "idle, updated" and in RR-idle mode.

Method of Test:

The SS simulates a BCCH where continuously for at least 30 seconds at least one octet of the SI3 rest octets IE in all SYSTEM INFORMATION TYPE 3 messages and at least one octet of the SI4 rest octets IE in all SYSTEM INFORMATION TYPE 4 messages is different from 0010 1011 and the location area identification IE denotes a location area different from the current location area held by the MS. The SS verifies that the MS sends a CHANNEL REQUEST message on the RACH including the establishment cause "location updating". The SS responds with an IMMEDIATE ASSIGNMENT REJECT message.

Maximum Duration of Test:

10 s.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS		The SS starts sending modified

SYSTEM INFORMATION TYPE 3 and

SYSTEM INFORMATION TYPE 4

messages (as defined

below) continuously for at least

30 seconds on the BCCH.

2	MS -> SS	CHANNEL REQUEST	Establishment cause = "location

updating (SDCCH needed). This message may be

received during the 30 seconds.

3	SS -> MS	IMM ASS REJ

Specific Message Contents:

SYSTEM INFORMATION TYPE 3

Information element			value/remark

L2 pseudo length			18

cell identity				as required

location area identifification		denoting a new location area

control channel description		as required

cell options				as required

cell selection parameters		as required

RACH control parameters		as required

SI3 rest octets			at least one octet is different from "0010 1011"

SYSTEM INFORMATION TYPE 4

Information element			value/remark

L2 pseudo length			13

cell selection parameters		as required

RACH control parameters		as required

SI4 rest octets			at least one octet is different from "0010 1011"

26.5.8	Default contents of messages

CHANNEL RELEASE

Information element			Value/remark

RR cause				Normal event

CHANNEL REQUEST

DISCONNECT (SS -> MS)

Information element			Value/remark

Cause

Coding standard		Standard defined for the GSM PLMNS

Location			user

Cause value				#16

IDENTITY REQUEST

Information element			Value/remark

Identity type				Depending on test

Spare half octet			0000

IMMEDIATE ASSIGNMENT

Information element			Value/remark

L2 pseudo length   			n, where n is the L2 pseudo length of the message

Page mode				arbitrary

Spare half octet			0000

Channel description			a valid description of an SDCCH + SACCH

Request reference			Corresponding to the last CHANNEL REQUEST

received from the MS

Timing advance			arbitrary

Mobile allocation			chosen so that, together with the channel description

IE, it describes a valid SDCCH + SACCH

Starting time			Omitted

IA rest octets			m octets, each coded as H'2B, where m = 22 - n

IMMEDIATE ASSIGNMENT REJECT

Information element			Value/remark

L2 pseudo length   			19

Page mode				arbitrary

Spare half octet			0000

Request reference 1			corresponding to the last CHANNEL REQUEST

received from the MS

Wait indication 1			0 seconds

Request reference 2			arbitrary

Wait indication 2			0 seconds

Request reference 3			arbitrary

Wait indication 3			0 seconds

Request reference 4			arbitrary

Wait indication 4			0 seconds

IA rest octets			3 octets, each coded as H'2B

PAGING REQUEST TYPE 1

Information element			Value/remark

L2 pseudo length   			n where n is the sum of the mobile

identity 1 IE and 3

Page Mode				Normal paging

Channels needed for Mobiles 1 and 2

Channel (first)			Any channel

Channel (second)		(spare)

Mobile identity 1			IMSI or TMSI of MS under test

Mobile identity 2			Omitted

P1 rest octets			m octets, each coded as H'2B, where m = 22 - n

PAGING RESPONSE

RELEASE COMPLETE (MS -> SS)

Information element			Value/remark

Cause

Length			length of cause IE

Coding standard		Standard defined for the GSM PLMNS

Location			user

Cause value			#16

RELEASE COMPLETE (SS -> MS)

Information element			Value/remark

Cause

Coding standard		Standard defined for the GSM PLMNS

Location			user

Cause value			#16

STATUS (MS -> SS)

Information element			Value/remark

Cause

Length			length of cause IE

Coding standard		Standard defined for the GSM PLMNS

Location			user

Cause value			as defined in test

Call state				as defined in test

STATUS ENQUIRY (SS -> MS)

Information element			Value/remark

Transaction identifier		relating to the active call