26.7	Elementary procedures of mobility management

The tests are based on GSM 04.08, GSM 03.03 and GSM 03.20.

26.7.1	TMSI reallocation

The intention of the TMSI Reallocation procedure is to assign a new temporary identity for the MS. If the message is not understood by the MS, the network could not establish a link to the MS. As this is a common MM procedure, it can be initiated at any time.

[Editorial Note:	- it is not checked that this procedure can be initiated at any time

- the time for the interruption of the power supply has been reduced from 1 minute to 10 seconds

]

Conformance requirement:

1)	A Mobile Station shall acknowledge a new TMSI when explicitly allocated during a location updating procedure or an incoming call.

2)	If the power supply is interrupted the Mobile Station shall store the TMSI in non-volatile memory.

3)	A Mobile Station shall answer paging with this TMSI and includes it in the Paging Response message.

Test purpose:

To verify that the MS is able to receive and acknowledge a new TMSI by means of an explicit TMSI reallocation procedure.

To verify that the MS has stored the TMSI in a non-volatile memory.

The implicit reallocation procedure is tested in section 26.7.4.1.

Reference(s):

GSM 04.08 section 4.3.1, GSM 03.03 section 2, GSM 03.20 section 2.3.4.

Related PICS/PIXIT statement(s):

None.

Initial conditions:

System Simulator:

Two cells A and B, belonging to different location areas a and b, default parameters.

Mobile Station:

The MS has valid TMSI (= TMSI1), CKSN, Kc. It is in the MM-state "idle, updated", updated in cell B and in the RR-idle mode.

Foreseen final state of the MS:

The MS has a valid TMSI (= TMSI1), CKSN, Kc. It is in the MM-state "idle, updated" in cell A and in the RR-idle mode.

Method of test:

The MS is paged in cell B and the ciphering mode is established. An explicit TMSI reallocation procedure is performed. The channel is released. The MS is switched off and its power supply is interrupted for 10 seconds. The power supply is resumed and the MS is switched on. The system simulator checks, by paging, whether the MS has stored the received TMSI.

The MS is made to select cell A. A normal location updating procedure is performed in cell A. An explicit TMSI reallocation procedure is performed and then the location updating procedure is accepted by the SS. The system simulator checks, by paging, whether the MS has stored the received TMSI.

Maximum duration of test:

2 minutes

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1

2	MS -> SS	CHANNEL REQUEST	Establishment Cause: Answer to

paging.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	CIPHERING MODE COMMAND	The SS starts deciphering.

6	MS -> SS	CIPHERING MODE COMPLETE	The SS starts enciphering.

7	SS -> MS	TMSI REALLOCATION CMD	"Mobile identity"= new TMSI

(TMSI2).

8	MS -> SS	TMSI REALLOCATION CMP

9	SS -> MS	CHANNEL RELEASE	After the sending of this message,

the SS waits for the disconnection

of the main signalling link.

10	MS		The MS is switched off and its

power supply is interrupted for

10 seconds.

11	MS		The power supply is resumed and

the MS is switched on again.

12	SS		The SS waits an amount of time

which is enough to guarantee that

the MS is in service (listening to

its paging subchannel)

13	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" contains the

newly distributed TMSI (= TMSI2).

14	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer to

paging.

15	SS -> MS	IMMEDIATE ASSIGNMENT

16	MS -> SS	PAGING RESPONSE	"Mobile identity" IE contains

the newly allocated TMSI (=TMSI2).

17	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

18	SS		The RF level of cell B is lowered

until the MS selects cell A. The RF

level of cell B is set sufficiently

low to ensure that cell B is not

suitable as defined in GSM 05.08

section 6.6.2

19	MS -> SS	CHANNEL REQUEST	Establishment cause: All

other cases.

20	SS -> MS	IMMEDIATE ASSIGNMENT

21	MS -> SS	LOCATION UPDATING REQ	location updating type= normal,

"ciphering key sequence number"=

CKSN, LAI= b, mobile station

classmark 1 and "mobile identity"=

TMSI2.

22	SS -> MS	TMSI REALLOCATION CMD	TMSI = TMSI1

23	MS -> SS	TMSI REALLOCATION CMP

24	SS -> MS	LOCATION UPDATING ACC	This message does not contain the

optional Mobile Identity field.

25	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link. The SS waits an

amount of time which is enough to

guarantee that the MS is in service

(listening to its paging

subchannel)

26	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains

the new TMSI (= TMSI1).

27	MS -> SS	CHANNEL REQUEST	"Establishment cause" :

Answer to paging.

28	SS -> MS	IMMEDIATE ASSIGNMENT

29	MS -> SS	PAGING RESPONSE	"Mobile identity" IE contains

the new TMSI (= TMSI1).

30	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.2	Authentication

The purpose of this procedure is to verify the user identity. A correct response is essential to guarantee the establishment of the connection. If not, the connection will drop.

[Editorial Note:	- the reference to the network timer T3260 has been removed as the mobile station has to answer as quick as possible. If an upper limit is defined for phase 2 then it will be added.

- unnecessary steps have been removed (step f in phase 1 11.10)

- in the old version it was not clear whether the MS was paged with IMSI or TMSI. This new version includes now both.

- it is not checked that the MS, when the flag "authentication failure" is set, only performs default cell selection

- the MS is brought back to the MM-state "idle, updated". This will allow to combine tests and also to check that the LAI has been deleted (this was not checked in the phase 1 version)

- the checking of the "idle no IMSI" state is done in a less complete way than in 26.7.4.2.1. This test has to be aligned with the other one as they refer to the same MM-state.

- in the conformance requirement and purpose, it shall be added that the MS has deleted LAI CKSN and TMSI after authentication rejection.

- support of speech shall be in PICS

]

Conformance requirement:

1)	A Mobile Station shall correctly respond to an Authentication Request message by sending an Authentication Response message with the SRES information field set to the same value as the one produced by the authentication algorithm in the network.

2)	A Mobile Station shall indicate in a Paging Response message the ciphering key sequence number which was allocated to it through the authentication procedure.

3)	After reception of an Authentication Reject message the Mobile Station:

3.1	shall not answer paging

3.2	shall reject any request for a mobile originating call

4)	After reception of an Authentication Reject message the Mobile Station, if it supports speech, shall accept a request for an emergency call by sending a Channel Request message with the establishment cause set to "emergency call".

Test purpose:

To verify that the MS correctly responds to the authentication elementary procedure, correctly manages the ciphering key sequence number which is distributed to it in this procedure and acts correctly upon failure of the procedure.

This test is not intended to do any test of correct handling of authentication algorithm by the SIM.

Reference(s):

GSM 04.08 section 4.3.2, GSM 03.03 section 2, GSM 03.20 sections 3 and 4.

Related PICS/PIXIT statement(s):

SIM detachement possible without powering down Yes/No

Switch off on button Yes/No

Initial conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is provided with a SIM, of which the relation between the incoming RAND and the resulting SRES is known. For the purpose of this test, the algorithm may even be a look-up table, or very simple standard manipulation of the incoming RAND.

The MS is in the MM-state "Idle updated" with valid TMSI, CKSN (referred as CKSN1), Kc and in the RR-idle mode.

Foreseen final state of the MS:

The MS has valid TMSI, CKSN and Kc. It is in the MM-state "idle, updated" and in the RR-idle mode.

Method of test:

The MS is paged. After the MS has sent a PAGING RESPONSE message to the SS, the SS initiates an authentication procedure and checks the value SRES sent by the MS in the AUTHENTICATION RESPONSE message. The channel is released. The MS is paged and the SS checks the value of the ciphering key sequence number sent by the MS in the PAGING RESPONSE message. An authentication procedure is performed. Then the SS sends an AUTHENTICATION REJECT message. The channel is released. The SS checks that the MS is in a state where only emergency calls are permitted.

Maximum duration of test:

1 minute

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1

2	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer

to paging.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	PAGING RESPONSE	CKSN= CKSN1

5	SS -> MS	AUTHENTICATION REQUEST	CKSN= CKSN2

6	MS -> SS	AUTHENTICATION RESPONSE	"Auth.parameter SRES" IE should

be bit exact with the value as

produced by the authentication

algorithm.

7	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link. The SS waits an

amount of time which is enough to

guarantee that the MS is in

service.

8	SS -> MS	PAGING REQUEST TYPE 1

9	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer

to paging.

10	SS -> MS	IMMEDIATE ASSIGNMENT

11	MS -> SS	PAGING RESPONSE	"Ciphering key sequence number"

shall be the same as the value that

was sent in the last AUTHENTICATION

REQUEST message (= CKSN2).

12	SS -> MS	AUTHENTICATION REQUEST

13	MS -> SS	AUTHENTICATION RESPONSE

14	SS -> MS	AUTHENTICATION REJECT

15	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link. The SS waits an

amount of time which is enough to

guarantee that the MS is in

service.

16	SS -> MS	PAGING REQUEST TYPE 1	The MS is paged with its TMSI.

This message is sent at least once.

17	   MS		The MS shall ignore these messages.

18	   MS		A normal call is attempted.

19	   MS		The MS shall refuse the attempt.

This is checked during 3 seconds.

20	   MS		An emergency call is attempted.

21	MS -> SS	CHANNEL REQUEST	Establishment Cause :

Emergency call

22	SS -> MS	IMMEDIATE ASSIGNMENT

23	MS -> SS	CM SERVICE REQUEST	"CM service type": Emergency

call establishment.

"Ciphering key sequence number"=

"no key available".

Mobile Identity= IMSI of the MS

24	SS -> MS	CM SERVICE ACCEPT

25	MS -> SS	EMERGENCY SETUP

26	SS -> MS	RELEASE COMPLETE	Cause IE : "unassigned number".

27	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

28	   MS		If possible (see PICS) SIM

detachment is performed. Otherwise

if possible (see PICS) switch off

is performed. Otherwise the power

is removed.

29	   MS		The MS must not perform an IMSI

detach. This is checked during 3

seconds.

30	MS		Depending on what has been

performed in step 30 the MS is

brought back to service.

31	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

32	SS -> MS	IMMEDIATE ASSIGNMENT

33	MS -> SS	LOCATION UPDATING REQ	location updating type= normal or

IMSI attach, CKSN= no key

available, Mobile Identity= IMSI,

LAI= deleted LAI

34	SS -> MS	AUTHENTICATION REQUEST	CKSN= CKSN1

35	MS -> SS	AUTHENTICATION RESPONSE

36	SS -> MS	LOCATION UPDATING ACC	Mobile Identity= TMSI

37	MS -> SS	TMSI REALLOCATION COMP

38	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.3	Identification

The purpose of this procedure is to check that the MS gives its identity as requested by the network. If this procedure does not work, it will not possible for the network to rely on the identity claimed by the MS.

Conformance requirement:

1)	When requested by the network the Mobile Station shall send its IMSI.

2)	When requested by the network the Mobile Station shall send the TMSI which it was previously allocated.

3)	When requested by the network the Mobile Station shall send its IMEI as stored in the Mobile Equipement.

Test purpose:

1)	To verify that the MS sends identity information as requested by the system in the following cases: IMSI and TMSI are requested in non-ciphered mode, IMEI is requested in ciphered mode.

2)	To verify that the MS sends its IMEI, when requested to do so, in non-ciphered mode.

Reference(s):

GSM 04.08 sections 4.3.3.

Related PICS/PIXIT statement(s):

None.

26.7.3.1	Identification / test 1

Initial conditions:

System Simulator:

1 cell, default values

Mobile Station:

The MS has a valid TMSI. It is in the MM-state "idle, updated" and in the RR-idle mode.

Foreseen final state of the MS:

The MS has a valid TMSI. It is in the MM-state "idle, updated" and in the RR-idle mode.

Method of test:

The SS requests identity information from the MS :

-	IMSI in non ciphering mode,

-	allocated TMSI in non ciphering mode,

-	IMEI in ciphering mode.

Maximum duration of test:

30 seconds

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1

2	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer to

paging.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	IDENTITY REQUEST	"Identity type" IE is IMSI.

6	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies

the IMSI of the MS.

7	SS -> MS	IDENTITY REQUEST	"Identity type" IE is TMSI.

8	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies

the allocated TMSI of the MS.

9	SS -> MS	CIPHERING MODE COMMAND

10	MS -> SS	CIPHERING MODE COMPLETE

11	SS -> MS	IDENTITY REQUEST	"Identity type" IE is IMEI.

12	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies

the IMEI stored in the Mobile

Equipment.

13	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.3.2	Identification / test 2

Initial conditions:

System Simulator:

1 cell, default values

Mobile Station:

The MS has a valid TMSI. It is in the MM-state "idle, updated" and in the RR-idle mode.

Foreseen final state of the MS:

The MS has a valid TMSI. It is in the MM-state "idle, updated" and in the RR-idle mode.

Method of test:

The SS requests identity information from the MS :

-	IMEI in non ciphering mode.

Maximum duration of test:

30 seconds

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1

2	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer to

paging.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	IDENTITY REQUEST	"Identity type" IE is IMEI.

6	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies

the IMEI of the MS.

7	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.4	Location updating

This procedure is used to register the MS in the network. If it is not performed correctly, a Mobile Terminating call can not be established.

[Editorial Note:	MM-states must be checked again after GSM3 has completed the specifications

]

26.7.4.1	Location updating / accepted

[Editorial Note:	- values of ATT and T3212 have no impact on the test. They could be removed from the initial conditions

- the time during which a non response to a paging message is checked has been added in step 31 and is set to 5 seconds

]

Conformance requirement:

1.1	If the network accepts a location updating from the Mobile Station and reallocates a TMSI in the Location Updating Accept message the Mobile Station shall acknowledge the reception of the new TMSI.

1.2	The Mobile Station shall answer to paging with this TMSI and include it in a Paging Response message.

2	If the network accepts a location updating from the Mobile Station and the Location Updating Accept message contains neither TMSI nor IMSI, the Mobile Station shall answer to paging when addressed with the previously allocated TMSI and include it in the Paging Response message.

3.1	If the network accepts a location updating from the Mobile Station by use of a Location Updating Accept message containing the IMSI of the Mobile Station, the Mobile Station shall not answer paging with the previously allocated TMSI.

3.2	The Mobile Station shall still answer paging with IMSI.

Test purpose:

To test the behaviour of the MS if the network accepts the location updating of the MS.

For the network response three different cases are identified:

1)	TMSI is allocated,

2)	Location updating accept contains neither TMSI nor IMSI,

3)	Location updating accept contains IMSI.

Reference(s):

GSM 04.08 section 4.4.4.5.

Related PICS/PIXIT statement(s):

Value of RF power Capability for Mobile Station Classmark 1 information element.

Initial conditions:

System Simulator:

Two cells, A and B, belonging to different location areas with location area identification a and b of the same PLMN.

IMSI attach/detach is allowed in both cells.

The T3212 timeout value is 1/10 hour in both cells.

Mobile Station:

The MS has a valid TMSI (=TMSI1) and CKSN (=CKSN1). It is in the MM-state "idle, updated", updated in cell A and in the RR-idle mode.

Foreseen final state of the MS:

The MS has no valid TMSI. It has valid CKSN and Kc. It is the MM-state "idle, updated", updated in cell B, and in the RR-idle mode.

Method of test:

The MS is made to select cell B. A normal location updating with TMSI reallocation is performed in cell B. The channel is released. The SS checks, by paging, that the MS has stored the newly allocated TMSI. The channel is released. The MS is made to select cell A. A normal location updating is performed in cell A. The LOCATION UPDATING ACCEPT message contains neither IMSI nor TMSI. The SS checks, by paging, that the MS has kept the old TMSI. The channel is released. A normal location updating is performed in cell B. The LOCATION UPDATING ACCEPT message contains an IMSI. The SS checks, by paging, that the MS has deleted its TMSI and responds to paging with IMSI.

Maximum duration of test:

4 minutes

Expected sequence:

Step	Direction	Message	Comments

1	   SS		The RF level of cell A is

lowered until the MS selects

cell B.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,

"CKSN"= CKSN1, "location area

identification"= a, "mobile station

classmark 1" as given by the PICS

and "mobile identity"= TMSI1.

5	SS -> MS	LOCATION UPDATING ACC	"Mobile identity"= new TMSI

(=TMSI2), LAI= b.

6	MS -> SS	TMSI REALLOCATION CMP

7	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link. The SS waits an

amount of time which is enough to

guarantee that the MS is in

service.

8	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains

the new TMSI (= TMSI2).

9	MS -> SS	CHANNEL REQUEST

10	SS -> MS	IMMEDIATE ASSIGNMENT

11	MS -> SS	PAGING RESPONSE	"Mobile identity" IE contains

the new TMSI (= TMSI2).

12	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

13	   SS		The RF level of cell B is

lowered until the MS selects

cell A.

14	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases

15	SS -> MS	IMMEDIATE ASSIGNMENT

16	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,

"CKSN"= CKSN1, "location area

identification"= b, "mobile station

classmark 1" as given by the PICS

and "mobile identity"= TMSI2.

17	SS -> MS	LOCATION UPDATING ACC	"Mobile identity" IE not included

18	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link. The SS waits an

amount of time which is enough to

guarantee that the MS is in

service.

19	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains

the TMSI (= TMSI2).

20	MS -> SS	CHANNEL REQUEST

21	SS -> MS	IMMEDIATE ASSIGNMENT

22	MS -> SS	PAGING RESPONSE	"Mobile identity" IE contains

the TMSI (=TMSI2).

23	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

24	   SS		The RF level of cell A is

lowered until the MS selects

cell B.

25	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

26	SS -> MS	IMMEDIATE ASSIGNMENT

27	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,

"CKSN"= CKSN1, "location area

identification"= a, "mobile station

classmark 1" as given by the PICS

and "mobile identity"= TMSI2.

28	SS -> MS	LOCATION UPDATING ACC	"Mobile identity" IE contains

IMSI.

29	SS -> MS	CHANNEL RELEASE	After the sending of this message,

the SS waits for the disconnection

of the main signalling link. The SS

waits an amount of time which is

enough to guarantee that the MS is

in service.

30	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains

the old TMSI (= TMSI2).

31	   MS		The MS shall ignore this

message. This is checked during 5

seconds.

32	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains

the IMSI.

33	MS -> SS	CHANNEL REQUEST

34	SS -> MS	IMMEDIATE ASSIGNMENT

35	MS -> SS	PAGING RESPONSE	"Mobile identity" IE contains

the IMSI.

36	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.4.2	Location updating / rejected

26.7.4.2.1	Location updating / rejected / IMSI invalid

[Editorial Note: 	- at this point of time, a value of 3 seconds has been put when the SS checks that the MS does not answer to a paging message. This can be updated.

- the MS is brought back to the MM-state "idle, updated". This will allow to combine tests and also to check that the LAI has been deleted (this was not checked in the phase 1 version)

- it is not checked that the MS does not perform a location updating procedure when a new PLMN is entered. This should be checked before the step with IMSI detachement.

- the test with cause "Illegal MS" has been explicitely included as it was not clear for phase 1 if it was tested or not.

]

Conformance requirement:

1)	If the network rejects a location updating from the Mobile Station with the cause "IMSI unknown in HLR" the Mobile Station shall:

1.1	not perform normal location updating

1.2	not perform periodic location updating

1.3	not respond to paging with IMSI

1.4	not respond to paging with TMSI

1.5	reject any request for Mobile Originating call establishment

1.6	not perform IMSI detach if powered down

2)	If the network rejects a location updating from the Mobile Station with the cause "IMSI unknown in HLR" the Mobile Station, if it supports speech, shall accept a request for an emergency call by sending a Channel Request message with the establishment cause set to "emergency call".

Test purpose:

To test the behaviour of the MS if the network rejects the location updating of the MS with the cause "IMSI unknown in HLR" or "illegal MS".

Reference(s):

GSM 04.08 sections 4.4.4.6 and 4.4.4.8.

Related PICS/PIXIT statement(s):

SIM detachement possible without powering down Yes/No

Switch off on button Yes/No

Value of RF power Capability for Mobile Station Classmark 1 information element.

Initial conditions:

System Simulator:

Two cells : A and B, belonging to different location areas a and b.

IMSI attach/detach is allowed in both cells.

The T3212 timeout value is 1/10 hour in both cells.

Mobile Station:

The MS has valid TMSI, CKSN and Kc. It is camped in cell A, in the MM-state "idle, updated", updated in cell A and in the RR-idle state.

Foreseen final state of the MS:

The MS has valid TMSI, CKSN and Kc. It is in the MM-state "idle, updated", updated in cell A and in the RR-idle mode.

Method of test:

The SS rejects a normal location updating with the cause value "IMSI unknown in HLR". The channel is released. The SS checks that the MS has entered the state "idle, no imsi", i.e. does not perform normal location updating, does not perform periodic updating, does not respond to paging, rejects any requests from CM entities except emergency calls and does not perform IMSI detach if powered down.

The test is repeated with cause value "Illegal MS".

Maximum duration of test:

10 minutes

Expected sequence:

The sequence is executed for execution counter k = 1, 2

Step	Direction	Message	Comments

1	   SS		The RF level of cell A is

lowered until the MS selects

cell B.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ

5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "IMSI

unknown in HLR" for k = 1 or

"Illegal MS" for k = 2.

6	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

7	   SS		The RF levels are then changed

again to make the MS reselect

the cell A.

8	   MS		The MS must perform cell

reselection according to

procedure as specified in GSM

05.08 (this however is not checked

until step 18) but shall not

perform location updating.

9	   SS		The SS waits at least 7 minutes

for a possible periodic

updating.

10 	   MS		The MS shall not perform

periodic updating.

11	SS -> MS	PAGING REQUEST TYPE 1	The MS is paged in cell A.

"Mobile identity" IE contains

IMSI.

12	   MS		The MS shall ignore this

message. This is verified

during 3 seconds.

13	SS -> MS	PAGING REQUEST TYPE 1	The MS is paged in cell A.

"Mobile identity" IE contains

TMSI.

14	   MS		The MS shall ignore this

message. This is verified

during 3 seconds.

15	   MS		A normal MO call is attempted.

16	   MS		The MS may refuse the attempt. The

MS shall not produce any RF

transmission. This is checked

during 3 seconds

17	   MS		An emergency call is attempted.

18	MS -> SS	CHANNEL REQUEST	"Establishment cause" :

Emergency call. This message is

sent in cell A

19	SS -> MS	IMMEDIATE ASSIGNMENT

20	MS -> SS	CM SERVICE REQUEST	"CM service type" : Emergency

call establishment.

21	SS -> MS	CM SERVICE ACCEPT

22	MS -> SS	EMERGENCY SETUP

23	SS -> MS	RELEASE COMPLETE	"Cause"= unassigned number.

24	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

25	   MS		If possible (see PICS) SIM

detachment is performed. Otherwise

if possible (see PICS) switch off

is performed. Otherwise the power

is removed.

26	   MS		The MS must not perform an IMSI

detach. This is checked during 3

seconds.

27	MS		Depending on what has been

performed in step 25 the MS is

brought back to operation.

28	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

29	SS -> MS	IMMEDIATE ASSIGNMENT

30	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,

"CKSN"= no key available, "mobile

station classmark 1" as given by

the PICS, "Mobile Identity"= IMSI,

"LAI"= deleted LAI

31	SS -> MS	AUTHENTICATION REQUEST	"CKSN"= CKSN1

32	MS -> SS	AUTHENTICATION RESPONSE

33	SS -> MS	LOCATION UPDATING ACC	"Mobile Identity"= TMSI

32	MS -> SS	TMSI REALLOCATION COMP

33	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.4.2.2	Location updating / rejected / PLMN not allowed

[Editorial Note:

- the switch off/on of the MS when the RF levels are changed (steps 24 and 25) has been suppressed because it is not useful.

- the test where the LAI is changed has been suppressed because it is the same as the test where the RF level are changed (steps 24 and 25).

- two steps in the old version have been exchanged in order to check that the MS has performed default cell reselection and access the right cell.

- it is not checked that

* the MS reject any requests from CM entities (other than emergency call)

* the MS may respond to paging with IMSI.

- CR 252 rev 1 has been included

]

Conformance requirement:

1)	If the network reject a location updating from the Mobile Station with the cause "PLMN not allowed" the Mobile Station shall:

1.1	not perform periodic updating,

1.2	not perform IMSI detach when switched off,

1.3	not perform IMSI attach when switched on in the same location area,

1.4	not perform normal location updating when in the same PLMN.

2)	If the network rejects a location updating from the Mobile Station with the cause "PLMN not allowed" the Mobile Station shall:

2.1	perform normal location updating when a new PLMN is entered

2.2	accept a request for an emergency call, if it supports speech, by sending a Channel Request message with the establishment cause set to "emergency call".

3)	If the network rejects a location updating from the Mobile Station with the cause "PLMN not allowed" and if after that the PLMN from which this rejection was received, is manually selected, the Mobile Station shall perform a normal location updating procedure.

Test purpose:

To test the behaviour of the MS if the network rejects the location updating of the MS with the cause "PLMN not allowed".

Reference(s):

GSM 04.08 section 4.4.4.6

Related PICS/PIXIT statement(s):

SIM detachement possible without powering down Yes/No

Switch off on button Yes/No

26.7.4.2.2.1	Location updating / rejected / PLMN not allowed / test 1

Initial conditions:

System Simulator:

One cell: A, belonging to PLMN1.

Two cells : A and B, belonging to different location areas a and b and belonging to PLMN2.

IMSI attach/detach is allowed in both cells of PLMN2 but not in PLMN1.

The T3212 timeout value is 1/10 hour in both cells in PLMN2.

Mobile Station:

The MS has a valid TMSI. It is camped in cell A of PLMN1 in the MM-state "idle, updated", updated in cell A and in the RR-idle state.

The MS is in manual mode for PLMN selection.

Foreseen final state of the MS:

The MS has a valid TMSI. It is in the MM-state "idle, updated", updated in cell A of PLMN1 and in the RR-idle state.

The MS is in manual mode for PLMN selection.

Method of test:

The SS rejects a normal location updating with the cause value "PLMN not allowed". The channel is released. The SS checks that the MS does not perform periodic updating, does not perform IMSI detach, does not perform IMSI attach if activated in the same location area, accepts a request for an emergency call and performs normal location updating only when a new PLMN is entered.

Maximum duration of test:

12 minutes

Expected sequence:

Step	Direction	Message	Comments

1	   MS		The MS is switched off.

2	   SS		The SS activates cells A and B of

PLMN2.

3	   MS		The MS is switched on. The MS shall

offer the new PLMN as available to

the user. The PLMN is manually

selected.

4	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

5	SS -> MS	IMMEDIATE ASSIGNMENT

6	MS -> SS	LOCATION UPDATING REQ

7	SS -> MS	LOCATION UPDATING REJ	"Reject cause"= PLMN not

allowed.

8	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

9	   SS		The SS waits for a possible

periodic updating for 7

minutes.

10	   MS		The MS shall not perform

periodic updating.

11	   MS		If possible (see PICS) SIM

detachment is performed. Otherwise

if possible (see PICS) switch off

is performed. Otherwise the power

is removed.

12	   MS		The MS must not perform an IMSI

detach. This is checked during 3

seconds.

13	   MS		Depending on what has been

performed in step 11 the MS is

brought back to operation.

14	   MS		The MS shall not perform IMSI

attach. This is checked during 3

seconds.

15	   SS		The RF level of cell B is

lowered to make the MS reselect

cell A in PLMN2.

16	   MS		No access to the network shall

be registered by the SS within

one minute.

17	   MS		An emergency call is attempted.

18	MS -> SS	CHANNEL REQUEST	"Establishment cause" :

Emergency call sent on cell A

(PLMN2).

19	SS -> MS	IMMEDIATE ASSIGNMENT

20	MS -> SS	CM SERVICE REQUEST	"CM service type"= Emergency call

establishment.

21	SS -> MS	CM SERVICE ACCEPT

22	MS -> SS	EMERGENCY SETUP

23	SS -> MS	RELEASE COMPLETE	Cause IE : "unassigned number".

24	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

25	   MS		The MS is switched off.

26	   SS		The SS activates cell A of PLMN1

and deactivates both cells of

PLMN2.

27	   MS		The MS is switched on.

28	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

29	SS -> MS	IMMEDIATE ASSIGNMENT

30	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,

"CKSN"= no key available,

"LAI"= deleted LAI, "mobile

identity"= IMSI

31	SS -> MS	LOCATION UPDATING ACC	"Mobile identity"= TMSI.

32	MS -> SS	TMSI REALLOCATION CMP

33	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.4.2.2.2	Location updating / rejected / PLMN not allowed / test 2

Initial conditions:

System Simulator:

One cell: A, belonging to PLMN1.

Two cells : A and B, belonging to different location areas a and b and belonging to PLMN2.

IMSI attach/detach is allowed in both cells of PLMN2 but not in PLMN1.

The T3212 timeout value is 1/10 hour in both cells in PLMN2.

Mobile Station:

The MS has a valid TMSI. It is camped in cell A of PLMN1 in the MM-state "idle, updated", updated in cell A and in the RR-idle state.

The MS is in manual mode for PLMN selection.

Foreseen final state of the MS:

The MS has a valid TMSI. It is in the MM-state "idle, updated", updated in cell A of PLMN1 and in the RR-idle state.

The MS is in manual mode for PLMN selection.

Method of test:

The SS rejects a normal location updating with the cause value "PLMN not allowed". The channel is released. Then the PLMN from this rejection was received is manually selected and the SS checks that a normal location updating is performed.

Maximum duration of test:

1 minute

Expected sequence:

Step	Direction	Message	Comments

1	   MS		The MS is switched off.

2	   SS		The SS activates cells A and B of

PLMN2.

3	   MS		The MS is switched on. The MS shall

offer the new PLMN as available to

the user. The PLMN is manually

selected.

4	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

5	SS -> MS	IMMEDIATE ASSIGNMENT

6	MS -> SS	LOCATION UPDATING REQ

7	SS -> MS	LOCATION UPDATING REJ	"Reject cause"= PLMN not

allowed.

8	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

9	   MS		The MS is made to search for

PLMNs and the PLMN indicated

by the SS is manually selected

10	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

11	SS -> MS	IMMEDIATE ASSIGNMENT

12	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,

"CKSN"= no key available,

"LAI"= deleted LAI, "mobile

identity"= IMSI

13	SS -> MSCHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

14	   MS		The MS is switched off.

15	   SS		The SS activates cell A of PLMN1

and deactivates both cells of

PLMN2.

16	   MS		The MS is switched on.

17	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

18	SS -> MS	IMMEDIATE ASSIGNMENT

19	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,

"CKSN"= no key available,

"LAI"= deleted LAI, "mobile

identity"= IMSI

20	SS -> MS	LOCATION UPDATING ACC	"Mobile identity"= TMSI.

21	MS -> SS	TMSI REALLOCATION CMP

22	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.4.2.3	Location updating / rejected / location area not allowed

[Editorial Note:

- two steps have been exchanged (steps f and g in phase 1) in order to finish in a "clean" MM-state (to enable to begin immediately an other test)

- the second reject of location updating has been suppressed as it was not useful

- to finish the test with the MS on, we have to check that it does not perform an IMSI attach procedure; This is a requirement in TS 04.08 and was not checked.

]

Conformance requirement:

1)	If the network rejects a location updating from the Mobile Station with the cause "Location Area not allowed" the Mobile Station shall:

1.1	not perform periodic updating,

1.2	not respond to paging with TMSI,

1.3	reject any request for Mobile Originating call establishment,

1.4	not perform IMSI detach.

2)	If the network rejects a location updating from the Mobile Station with the cause "Location Area not allowed" the Mobile Station shall:

2.1	perform normal location updating when a new location area is entered,

2.2	accept a request for an emergency call, if it supports speech, by sending a Channel Request message with the establishment cause set to "emergency call".

Test purpose:

To test the behaviour of the MS if the network rejects the location updating of the MS with the cause "Location Area not allowed".

Reference(s):

GSM 04.08 sections 4.4.4.6

Related PICS/PIXIT statement(s):

SIM detachement possible without powering down Yes/No

Switch off on button Yes/No

Initial conditions:

System Simulator:

Two cells : A and B, belonging to different location areas a and b.

IMSI attach/detach is allowed in both cells.

The T3212 timeout value is 1/10 hour in both cells.

Mobile Station:

The MS has a valid TMSI. It is in the MM-state "idle, updated", updated in cell A and in the RR-idle mode.

Foreseen final state of the MS:

The MS has a valid TMSI. It is in the MM-state "idle, updated", updated in cell A and in the RR-idle mode.

Method of test:

The SS rejects a normal location updating with the cause value "Location Area not allowed". The channel is released. The SS checks that the MS does not perform periodic updating, does not respond to paging with TMSI, responds to paging with IMSI, rejects any requests from CM entities for MM-connections except emergency calls, performs normal location updating when a new location area is entered and does not perform IMSI detach.

Maximum duration of test:

12 minutes

Expected sequence:

Step	Direction	Message	Comments

1	   SS		The RF level of cell A is

lowered until the MS selects

cell B.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ

5	SS -> MS	LOCATION UPDATING REJ	"Reject cause"= "Location

Area not allowed".

6	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

7	   SS		The SS waits for a possible

periodic updating for 7

minutes.

8	   MS		The MS shall not perform

periodic updating.

9	SS -> MS	PAGING REQUEST TYPE 1	The MS is paged in cell B.

"Mobile identity"= TMSI.

10	   MS		The MS shall ignore this message.

This is checked during 3 seconds.

11	   MS		A normal call is attempted.

12	   MS		The MS shall refuse the attempt. The

MS shall not produce any RF

transmission. This is checked

during 3 seconds

13	   MS		An emergency call is attempted.

14	MS -> SS	CHANNEL REQUEST	"Establishment cause" :

Emergency call.

15	SS -> MS	IMMEDIATE ASSIGNMENT

16	MS -> SS	CM SERVICE REQUEST	"CM service type":

Emergency call establishment.

17	SS -> MS	CM SERVICE ACCEPT

18	MS -> SS	EMERGENCY SETUP

19	SS -> MS	RELEASE COMPLETE	Cause: "unassigned number".

20	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

21	   MS		If possible (see PICS) SIM

detachment is performed. Otherwise

if possible (see PICS) switch off

is performed. Otherwise the power

is removed.

22	   MS		The MS must not perform an IMSI

detach. This is checked during 3

seconds.

23	MS		Depending on what has been

performed in step 26 the MS is

brought back to operation.

24	   MS		The MS shall not perform IMSI

attach. This is checked during 3

seconds.

25	   SS		The RF level of cell B is

lowered until the MS selects

cell A.

26	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

27	SS -> MS	IMMEDIATE ASSIGNMENT

28	MS -> SS	LOCATION UPDATING REQ

29	SS -> MS	AUTHENTICATION REQUEST

30	MS -> SS	AUTHENTICATION RESPONSE

31	SS -> MS	LOCATION UPDATING ACC	Mobile identity= TMSI

32	MS -> SS	TMSI REALLOCATION CMP

33	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.4.2.4	Location updating / rejected / national roaming

[Editorial Note:	- is the Location updating type in Requirement 1 3) (now step 14) really periodic?

- the description of the test assumes that there no IMSI attach/detachement procedures but in the default conditions it is defined that IMSI attach/detach are allowed. So the description of the test is not compatible with the initial conditions as it is assumed that IMSI attach/detach is not allowed. If it is really needed it can be changed back to the old value.

- it is enough to wait 7 minutes in order to check that the MS does not perform periodic location updating (Procedure 1, step 7 and Procedure 3, step 46)

- in procedures 2 and 3 cell B should be set at a level sufficient for cell reselection.

- Procedure 3 could be simplified by using the full capacities of the SS and by using 6 cells

- procedure 4 step b refers to a periodic location updating procedure as Requirements 4.1 refers to a normal location updating procedure

- procedure 5 is useless if it is not possible to remove the SIM without powering down

- no conformance requirement

]

Test purpose:

Procedure 1:

To test that on receipt of a rejection using the National Roaming cause code, the MS ceases trying to update on that cell, that this situation continues for at least one periodic location interval period, and that the rejected area list is re-set by powering down the MS (the requirement given in GSM 04.08-DCS is that the list shall be retained for at least 12 hours. This aspect is not formally tested).

Procedure 2:

To test that if no cell is available for National Roaming, emergency calls can still be made.

Procedure 3:

To test that at least 6 entries can be held in the list (the requirement in the core recommendation is to store at least 10 entries. This is not fully tested by the third procedure).

Procedure 4:

To test that if a cell of the Home PLMN is available then the MS returns to it in preference to any other available cell.

Procedure 5:

To test that if the SIM is removed the National Roaming forbidden list is cleared.

Reference(s):

GSM 04.08 section 4.4.4.6

Related PICS/PIXIT statement(s):

SIM detachement possible without powering down Yes/No

Switch off on button Yes/No

Value of RF power Capability for Mobile Station Classmark 1 information element.

Support of DCS 1800 Yes/No

Initial conditions:

The initial conditions shall be met before each of the different procedures.

System Simulator:

For procedures 1, 2, 3 and 5: Two cells A and B, belonging to different location areas a and b. Neither a or b shall have the MNC as the Home PLMN, however both a and b shall have the same MNC. The MCC shall be the same as that of the Home PLMN.

For procedure 4: three cells A, B, C with 3 different location area codes. None of the cells should be part of the Home PLMN. Cells should differ in signal strength by 10 dB with cell A being the strongest and cell C the weakest. There should be a 20 dB range between A and C. A should be set to a level of - 40 dBm.

IMSI attach/detach is allowed in every cell.

The T3212 timeout value is 1/10 hour in every cell.

Mobile Station:

The MS has valid TMSI, CKSN and Kc. It is camped in cell B, in the MM-state "idle, updated", updated in cell B and in the RR-idle state.

The list of "location areas not allowed for national roaming" shall be empty (this may be achieved by either removing the SIM or switching the MS OFF then ON or powering down the MS depending on PICS).

Foreseen final state of the MS:

The MS has valid TMSI, CKSN and Kc. It is in the MM-state "idle, updated", updated in cell A and in the RR-idle mode.

Method of test:

The SS rejects a normal location updating with the cause value "National Roaming not allowed in this area". The channel is released. The SS checks that the MS does not perform periodic location updating procedure. The MS is turned off and then on. The SS checks that the MS has deleted the list of "location areas not allowed in this area". This procedure is performed another time but the deletion of the list is checked while removing the SIM (instead of turning off the MS).

Then the SS rejects a normal location updating with the cause value "National Roaming not allowed in this area". The channel is released. The SS checks that the MS supports an emergency call.

The SS rejects a normal location updating with the cause value "National Roaming not allowed in this area". This is done for 6 different location areas. Then the SS checks that the MS does not attempt to begin a location updating procedure on the non-allowed location areas.

The SS accepts a normal location updating on a cell not belonging to the HPLMN. Then when the MS attemps to perform a periodic location updating to this cell, the SS rejects this location updating with the cause value "National Roaming not allowed in this area". Two cells are then available, one belonging to the HPLMN but with the weakest level. It is checked that the MS returns to its HPLMN.

Maximum duration of test:

Procedures 1 and 5: 12 minutes each

Procedure 2: 5 minutes

Procedure 3: 17 minutes

Procedure 4: 16 minutes

Expected sequence:

This test is performed if the MS supports DSC 1800 (see PICS).

The following procedure is used during the test:

Change_LAI (x):

-	the purpose of this procedure is to change the value of Location Area Identifier x

-	The Location Area Identifier of cell A shall be changed. The code shall be chosen randomly but shall be different from any previously used in this procedure. The code shall have the same MCC as the Home PLMN and shall not have the same MNC as the Home PLMN.

Procedure 1

Step	Direction	Message	Comments

1	SS		The RF level of cell B is

lowered until the MS selects

cell A.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ

5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National

Roaming not allowed in this

location area"

6	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

7	SS		The SS waits at least 10 minutes

for a possible periodic updating on

cell A.

8 	MS		The MS shall not perform

periodic updating on cell A.

9	MS		If possible (see PICS) the MS is

switched off. Otherwise

if possible the power is removed.

10	MS		Depending on what has been

performed in step 9 the MS is

brought back to operation.

11	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message is 							sent on cell A within 2 minutes

after the end of step 10.

12	SS -> MS	IMMEDIATE ASSIGNMENT

13	MS -> SS	LOCATION UPDATING REQ	Location Updating Type= periodic

14	SS -> MS	LOCATION UPDATING ACC	IE Mobile Identity not present

15	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Procedure 2

Step	Direction	Message	Comments

1	SS		The RF level of cell B is

lowered until the MS selects

cell A.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message is sent on

cell A.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ

5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National

Roaming not allowed in this

location area"

6	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

7	SS		The SS waits for a possible

periodic location updating

procedure on both cells A and B for

2 minutes.

8	MS		The MS shall not perform periodic

updating on cell A within 2 minutes

after the end of step 6

9	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message is sent on

cell B within 2 minutes after the end

of step 6.

10	SS -> MS	IMMEDIATE ASSIGNMENT

11	MS -> SS	LOCATION UPDATING REQ

12	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National

Roaming not allowed in this

location area"

13	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

14	SS		The SS waits for a possible

periodic location updating

procedure on both cells A and B for

2 minutes.

15	MS		The MS shall not perform periodic

updating on both cells A and B

within 2 minutes after the end of

step 13

16	MS		An emergency call is attempted.

17	MS -> SS	CHANNEL REQUEST	"Establishment cause" :

Emergency call. This message is

sent in cell A

18	SS -> MS	IMMEDIATE ASSIGNMENT

19	MS -> SS	CM SERVICE REQUEST	"CM service type" : Emergency

call establishment.

20	SS -> MS	CM SERVICE ACCEPT

21	MS -> SS	EMERGENCY SETUP

22	SS -> MS	RELEASE COMPLETE	"Cause"= unassigned number.

23	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Procedure 3

Step	Direction	Message	Comments

1	SS		The RF level of cell B is

lowered until the MS selects

cell A.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message is sent on

cell A.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ

5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National

Roaming not allowed in this

location area"

6	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

7	SS		The SS waits for a possible

periodic location updating

procedure on both cells A and B for

2 minutes.

8	MS		The MS shall not perform periodic

updating on cell A within 2 minutes

after the end of step 6

9	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message is sent on

cell B within 2 minutes after the end

of step 6.

10	SS -> MS	IMMEDIATE ASSIGNMENT

11	MS -> SS	LOCATION UPDATING REQ

12	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National

Roaming not allowed in this

location area"

13	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

14	SS		Change_LAI (A) within 5 seconds

after step 12

15	SS		The SS waits for a possible

periodic location updating

procedure on both cells A and B for

2 minutes.

16	MS		The MS shall not perform periodic

updating on cell B within 2 minutes

after the end of step 13

17	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message is sent on

cell A within 2 minutes after the end

of step 13.

18	SS -> MS	IMMEDIATE ASSIGNMENT

19	MS -> SS	LOCATION UPDATING REQ

20	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National

Roaming not allowed in this

location area"

21	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

22	SS		Change_LAI (B) within 5 seconds

after step 20

23	SS		The SS waits for a possible

periodic location updating

procedure on both cells A and B for

2 minutes.

24	MS		The MS shall not perform periodic

updating on cell A within 2 minutes

after the end of step 21

25	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message is sent on

cell B within 2 minutes after the end

of step 21.

26	SS -> MS	IMMEDIATE ASSIGNMENT

27	MS -> SS	LOCATION UPDATING REQ

28	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National

Roaming not allowed in this

location area"

29	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

30	SS		Change_LAI (A) within 5 seconds

after step 28

31	SS		The SS waits for a possible

periodic location updating

procedure on both cells A and B for

2 minutes.

32	MS		The MS shall not perform periodic

updating on cell B within 2 minutes

after the end of step 29

33	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message is sent on

cell A within 2 minutes after the end

of step 29.

34	SS -> MS	IMMEDIATE ASSIGNMENT

35	MS -> SS	LOCATION UPDATING REQ

36	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National

Roaming not allowed in this

location area"

37	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

38	SS		Change_LAI (B) within 5 seconds

after step 36

39	SS		The SS waits for a possible

periodic location updating

procedure on both cells A and B for

2 minutes.

40	MS		The MS shall not perform periodic

updating on cell A within 2 minutes

after the end of step 37

41	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message is sent on

cell B within 2 minutes after the end

of step 37.

42	SS -> MS	IMMEDIATE ASSIGNMENT

43	MS -> SS	LOCATION UPDATING REQ

44	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National

Roaming not allowed in this

location area"

45	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

46	SS		The SS waits for a possible

periodic location updating

procedure on both cells A and B for

10 minutes.

47	MS		The MS shall not perform periodic

updating on cell A within 10

minutes after the end of step 45

Procedure 4

Step	Direction	Message	Comments

1	SS		The SS waits for a possible

periodic location updating

procedure on cell A for 7 minutes

after the initial conditions have

been established.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ	Location Updating Type= normal

5	SS -> MS	LOCATION UPDATING ACC	IE Mobile Identity not present

6	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

7	SS		The location area identity of cell

C shall be changed to that of a

location area in the Home PLMN

8	SS		The SS waits for a possible

periodic location updating

procedure on cell A for

7 minutes.

9	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message is sent on

cell A within 7 minutes after the end

of step 6.

10	SS -> MS	IMMEDIATE ASSIGNMENT

11	MS -> SS	LOCATION UPDATING REQ

12	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National

Roaming not allowed in this

location area"

13	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

14	SS		The SS waits for a possible

location updating procedure on cell

C for 2 minutes. The uplink

frequency of cell B may be

monitored by using the SS wideband

receiver

15	MS		The MS shall not transmit on the

uplink frequency of cell B

16	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message is sent on

cell C within 2 minutes after the end

of step 13.

17	SS -> MS	IMMEDIATE ASSIGNMENT

18	MS -> SS	LOCATION UPDATING REQ

19	SS -> MS	LOCATION UPDATING ACC	IE Mobile Identity not present

20	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Procedure 5

Step	Direction	Message	Comments

1	SS		The RF level of cell B is

lowered until the MS selects

cell A.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ

5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National

Roaming not allowed in this

location area"

6	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

7	SS		The SS waits at least 10 minutes

for a possible periodic updating on

cell A.

8 	MS		The MS shall not perform

periodic updating on cell A.

9	MS		If possible (see PICS) SIM

detachment is performed. Otherwise

if possible (see PICS) switch off

is performed. Otherwise the power

is removed.

10	MS		Depending on what has been

performed in step 9 the MS is

brought back to operation.

11	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message is sent

on cell A within 2 minutes after

the end of step 10.

12	SS -> MS	IMMEDIATE ASSIGNMENT

13	MS -> SS	LOCATION UPDATING REQ	Location Updating Type= periodic

14	SS -> MS	LOCATION UPDATING ACC	IE Mobile Identity not present

15	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.4.3	Location updating / abnormal cases

26.7.4.3.1	Location updating / abnormal cases / random access fails

Conformance requirement:

If during the RR connection establishment phase of a normal location updating procedure, channel requests are not answered by the network, the Mobile Station shall:

1.	send (Max-Retrans+1) Channel Request messages

2.	not try to establish a connection during a period of T3213

3.	then perform a normal location updating procedure as it is still necessary.

4.	not repeat the complete procedure if the original cause of the location updating procedure has disappeared.

Test purpose:

To verify that when during the RR connection establishment phase of a location updating procedure, channel requests are not answered by the network, after expiry of T3213 (= 4s in Phase 1) the complete procedure is repeated if still necessary.

[Editorial Note:	- the two procedures are chained

]

Reference(s):

GSM 04.08 section 4.4.4.8 and GSM 05.08 section 6.6.2

Related PICS/PIXIT statement(s):

Value of RF power Capability for Mobile Station Classmark 1 information element.

Initial conditions:

System Simulator:

Two cells: A and B, belonging to different location areas a and b.

The RF power level of cell B is higher than the one of cell A.

IMSI attach/detach is not allowed in both cells.

The T3212 timeout value is set to infinite in both cells.

Mobile Station:

The MS has a valid TMSI. It is in the MM-state "Idle, updated", updated in cell B.

Foreseen final state of the MS:

The MS is in the MM-state "Idle, updated" in cell A.

Method of test:

The SS causes a random access failure in the MS during a normal location updating procedure. After the expiry of T3213 and when the cell reselection procedure is finished the MS will try to restart the normal location updating procedure.

The test is repeated but the original cause of the location updating procedure has disappeared. The SS then checks that the MS will not restart the location updating procedure.

Maximum duration of test:

1 minute

Expected sequence:

Step	Direction	Message	Comments

1	MS		The RF level of cell B is lowered

until the MS selects cell A.

The RF level of cell B is set

sufficiently low to ensure that

cell B is not suitable as defined

in GSM 05.08 section 6.6.2.

2	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

This message is sent by the MS

(Max_Retrans + 1) times

3	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

This message is sent after T3213

expiry and when the MS has

reselected cell after the end of

step 2 (a tolerance has to be defined on this time)

4	SS -> MS	IMMEDIATE ASSIGNMENT

5	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = b,

mobile station classmark 1 and

mobile identity = TMSI.

6	SS -> MS	LOCATION UPDATING ACC	Optional IE Mobile Identity not

included

7	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

8	MS		The RF level of cell A is lowered

until the MS selects cell B.

The RF level of cell A is set

sufficiently low to ensure that

cell A is not suitable as defined

in GSM 05.08 section 6.6.2.

9	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

This message is sent by the MS

(Max_Retrans + 1) times

10	SS		About 2 seconds after the end of

step 9 the RF level of cell A is

set sufficiently high to ensure

that cell A is suitable as defined

in GSM 05.08 section 6.6.2

11	MS		The MS shall not produce any RF

output. This is checked during 6

seconds.

Specific message contents:

None.

26.7.4.3.2	Location updating / abnormal cases / attempt counter smaller than 4, LAI different

[Editorial Note:

- it is not checked that the MS does not perform periodic location updating and IMSI attach procedures

- in procedure 2 time is reduced from one minute to 4 seconds for the checking of no response to paging. This aligns this test with the other parts of 11.10

]

Conformance requirement:

1)	When a failure such as cases d), f) and g) of section 4.4.4.8 of Rec GSM 04.08 has occurred during a normal location updating procedure, if the attempt counter is smaller than 4 and after expiry of T3211, the Mobile Station shall resend its Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Upadting type set to "normal location updating".

2)	When a failure such as cases d), f) and g) of section 4.4.4.8 of Rec GSM 04.08 has occurred during a normal location updating procedure the Mobile Station shall:

2.1	not answer to paging with the previously allocated TMSI

2.2	not perform the IMSI detach procedure, when switched off.

3)	When a failure such as case e) of section 4.4.4.8 of Rec GSM 04.08 has occurred during a normal location updating procedure and when an emergency call establishment is requested by the user the Mobile Station, if it supports speech, shall send a CM Service Request message with CM Service Type IE set to "emergency call establishment", CKSN IE set to "no key available" and Mobile Identity IE set to its IMSI and after acceptance by the network it shall send an Emergency Setup message.

4)	When a failure such as cases d), f) and g) of section 4.4.4.8 of Rec GSM 04.08 has occurred during a normal location updating procedure the Mobile Station shall use a request for a Mobile Originating call establishment as a trigger for a normal location updating procedure and shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

5)	When a failure such as cases d), f) and g) of section 4.4.4.8 of Rec GSM 04.08 has occurred during a normal location updating procedure the Mobile Station shall answer to paging with IMSI and shall send a Paging Response message with CKSN IE set to "no key available" and Mobile Identity IE set to its IMSI.

6)	When a failure such as cases d), f) and g) of section 4.4.4.8 of Rec GSM 04.08 has occurred during a normal location updating procedure the Mobile Station shall perform a normal location updating procedure as soon as it enters a new cell.

Test purpose:

To verify that the MS performs normal location updating procedures when its attempt counter is smaller than 4.

To check that the MS does not perform the IMSI detach procedure when in "idle not updated" state.

To verify that in "idle not updated" state the MS can perform an emergency call.

To verify that in "idle not updated" state the MS uses requests from CM layer other than emergency call as triggering of a normal location updating procedure.

To check that in the "idle not updated" state the MS answers to paging with IMSI.

To verify that the MS performs a normal location updating procedure if it enters a new cell while being in the "idle not updated" state.

Reference(s):

GSM 04.08 section 4.4.4.8 and GSM 05.08 section 6.6.2

Related PICS/PIXIT statement(s):

SIM detachement possible without powering down Yes/No

Switch off on button Yes/No

Value of RF power Capability for Mobile Station Classmark 1 information element.

Support of GSM900 Yes/No

Support of DCS1800 Yes/No

Initial conditions:

System Simulator:

Two cells: A and B, belonging to different location areas a and b.

ATT flag shall be set to IMSI attach/detach allowed.

Mobile Station:

The MS is "idle, updated" in cell A. A valid CKSN value is stored in the SIM and is noted "initial CKSN". A TMSI is allocated.

Foreseen final state of the MS:

The MS is "Idle, updated" in cell B with a valid CKSN and a TMSI.

Method of test:

The MS is made to perform a normal location updating procedure. Three types of failure cases are triggered:

-	sending of a Location Updating Reject with cause randomly chosen: if the MS supports GSM900 between all available cause values except 2, 3, 11 and 12 (which trigger a different action) (case g of GSM 04.08 section 4.4.4.8) or if the MS supports DCS1800 between causes #17 and #22,

-	RR-connection failure (case d),

-	sending of a CHANNEL RELEASE message before the normal end of the procedure (case f),

-	T3210 timeout (case e).

As there is no stored LAI or the stored LAI is different from the broadcast LAI, and the attempt counter in the MS shall be lower than 4, the MS enters the "Idle, not updated" state and waits for T3211 seconds before trying again a location updating procedure.

Then the behaviour of the MS in the "Idle, not updated" state is checked, that is:

-	not answer to paging with TMSI,

-	not perform an IMSI detach procedure,

-	support request for emergency call,

-	use requests from CM layer other than emergency call as triggering of a normal location updating procedure,

-	respond to paging with IMSI,

-	perform  normal location updating procedure when a new cell is entered.

Maximum duration of test:

6 minutes

Expected sequence:

Step	Direction	Message	Comments

1	MS		The RF level of cell A is lowered

until the MS selects cell B.

The RF level of cell A is set

sufficiently low to ensure that

cell A is not suitable as defined

in GSM 05.08 section 6.6.2.

2	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = a, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

5	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value

randomly chosen:

* in Table 10.44 of GSM 04.08,

causes #2, #3, #11 and #12 being

excluded, if the MS supports GSM900

(see PICS)

* among #17 and #22,

if the MS supports DCS1800

6	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

7	MS		The MS shall not produce any RF

output during T3211 seconds at

least after the channel release.

8	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

9	SS -> MS	IMMEDIATE ASSIGNMENT

10	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

11	SS		The SS stops any RF transmission on

the dedicated channel. The SS waits

until there is no more SACCH frames

in the uplink direction

12	MS		The MS shall not produce any RF

output during T3211 seconds at

least after the channel release.

13	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

14	SS -> MS	IMMEDIATE ASSIGNMENT

15	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

16	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

17	MS		The MS shall not produce any RF

output during T3211 seconds at

least after the channel release.

18	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

19	SS -> MS	IMMEDIATE ASSIGNMENT

20	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

21	SS -> MS	AUTHENTICATION REQUEST	CKSN = initial CKSN

22	MS -> SS	AUTHENTICATION RESPONSE

23	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI

24	MS -> SS	TMSI REALLOCATION COMPL

25	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

MS is now "idle, updated" in cell B

26	MS		The RF level of cell B is lowered

until the MS selects cell A.

The RF level of cell B is set

sufficiently low to ensure that

cell B is not suitable as defined

in GSM 05.08 section 6.6.2.

27	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

28	SS -> MS	IMMEDIATE ASSIGNMENT

29	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

30	SS		chooses randomly one among the

three following steps: (5 and 6) or

11 or 16

31	SS		performs the chosen step

32	SS -> MS	PAGING REQUEST TYPE 1	Mobile identity = old TMSI of the

MS.

This message is sent continuously

the MS during 8 seconds

33	SS		The SS checks that there is no

answer from the MS during 12

seconds

33a	SS		If during steps 33 and 34 the MS

attempts to perform a location

updating procedure the SS will

perform steps 30 and 31 and then

continue the procedure.

34	MS		If possible (see PICS) SIM

detachment is performed. Otherwise

if possible (see PICS) switch off

is performed. Otherwise the power

is removed.

35	MS		The MS must not perform an IMSI

detach. This is checked during 30

seconds.

36	MS		Depending on what has been

performed in step 34 the MS is

brought back to operation.

37	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

38	SS -> MS	IMMEDIATE ASSIGNMENT

39	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

40	SS -> MS	AUTHENTICATION REQUEST	CKSN = initial CKSN

41	MS -> SS	AUTHENTICATION RESPONSE

42	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI

43	MS -> SS	TMSI REALLOCATION COMPL

44	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

MS is now "idle, updated" in cell A

45	MS		The RF level of cell A is lowered

until the MS selects cell B.

The RF level of cell A is set

sufficiently low to ensure that

cell A is not suitable as defined

in GSM 05.08 section 6.6.2.

46	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

47	SS -> MS	IMMEDIATE ASSIGNMENT

48	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = a, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

49	SS -> MS	AUTHENTICATION REQUEST

50	MS->SS	AUTHENTICATION RESPONSE	steps 49 and 50 are performed N

times. N shall be chosen in such a

way that T3210 expires.

51	SS		The SS checks that there is no more

activity from the MS on the

channel.

52	MS		The MS is made to perform an

emergency call

53	MS -> SS	CHANNEL REQUEST	Establishment cause: Emergency call

54	SS -> MS	IMMEDIATE ASSIGNMENT

55	MS -> SS	CM SERVICE REQUEST	CM service type = Emergency call

establishment; CKSN = no key

available; Mobile Identity = IMSI

56	SS -> MS	CM SERVICE ACCEPT

57	MS -> SS	EMERGENCY SETUP

58	SS -> MS	RELEASE COMPLETE	Cause = unassigned number

59	SS -> MS	CHANNEL RELEASE

60	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases. The SS will wait at most 15

seconds for this message

61	SS -> MS	IMMEDIATE ASSIGNMENT

62	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

63	SS -> MS	AUTHENTICATION REQUEST	CKSN = initial CKSN

64	MS -> SS	AUTHENTICATION RESPONSE

65	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI

66	MS -> SS	TMSI REALLOCATION COMPL

67	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

MS is now "idle, updated" in cell B

68	MS		The RF level of cell B is lowered

until the MS selects cell A.

The RF level of cell B is set

sufficiently low to ensure that

cell B is not suitable as defined

in GSM 05.08 section 6.6.2.

69	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

70	SS -> MS	IMMEDIATE ASSIGNMENT

71	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

72	SS		chooses randomly one among the

three following steps: (5 and 6),

11 or 16

73	SS		performs the chosen step

74	MS		The MS is made to perform a MO call

before T3211 expiry

75	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

76	SS -> MS	IMMEDIATE ASSIGNMENT

77	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

78	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI

79	MS -> SS	TMSI REALLOCATION COMPL

80	SS -> MS	CHANNEL RELEASE

Steps 81 to 90 are optional as the

MS may have memorized the MO call

attempt

81	MS -> SS	CHANNEL REQUEST	Establishment cause= other

services requested by the user

82	SS -> MS	IMMEDIATE ASSIGNMENT

83	MS -> SS	CM SERVICE REQUEST	Service type= Mobile originating

call establishment, CKSN=

no key available,

Mobile identity= TMSI

84	SS -> MS	AUTHENTICATION REQUEST	CKSN = initial CKSN

85	MS -> SS	AUTHENTICATION RESPONSE

86	SS -> MS	CIPHERING MODE COMMAND

87	MS -> SS	CIPHERING MODE COMPLETE

88	MS -> SS	SETUP

89	SS -> MS	RELEASE COMPLETE	Cause = unassigned number

90	SS -> MS	CHANNEL RELEASE	After the sending of this message,

the SS waits for the disconnection of

the main signalling link.

MS is now "idle, updated" in cell A

91	MS		The RF level of cell A is lowered

until the MS selects cell B.

The RF level of cell A is set

sufficiently low to ensure that

cell A is not suitable as defined

in GSM 05.08 section 6.6.2.

92	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

93	SS -> MS	IMMEDIATE ASSIGNMENT

94	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = a, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

95	SS		chooses randomly one among the

three following steps: (5 and 6),

11 or 16

96	SS		performs the chosen step and lets

enough time for the MS to be

pageable

97	SS -> MS	PAGING REQUEST TYPE 1	IE mobile identity = IMSI.

98	MS -> SS	CHANNEL REQUEST	Establishment cause: Answer to

paging

99	SS -> MS	IMMEDIATE ASSIGNMENT

100	MS -> SS	PAGING RESPONSE	CKSN = no key available;

Mobile Identity = IMSI

101	SS -> MS	CHANNEL RELEASE

102	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases. The SS will wait at most 15

seconds for this message

103	SS -> MS	IMMEDIATE ASSIGNMENT

104	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

105	SS -> MS	AUTHENTICATION REQUEST	CKSN = initial CKSN

106	MS -> SS	AUTHENTICATION RESPONSE

107	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI

108	MS -> SS	TMSI REALLOCATION COMPL

109	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

MS is now "idle, updated" in cell B

110	MS		The RF level of cell B is lowered

until the MS selects cell A.

The RF level of cell B is set

sufficiently low to ensure that

cell B is not suitable as defined

in GSM 05.08 section 6.6.2.

111	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

112	SS -> MS	IMMEDIATE ASSIGNMENT

113	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

114	SS		chooses randomly one among the

three following steps: (5 and 6)

or 11 or 16

115	SS		performs the chosen step and notes

the time when the CHANNEL RELEASE

message is sent

116	MS		The RF level of cell A is lowered

until the MS selects cell B.

The RF level of cell A is set

sufficiently low to ensure that

cell A is not suitable as defined

in GSM 05.08 section 6.6.2.

117	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

The time interval between the last

CHANNEL RELEASE message and this

message shall be less than

0.8*T3211

118	SS -> MS	IMMEDIATE ASSIGNMENT

119	MS -> SS	LOCATION UPDATING REQ	location updating type= normal,

CKSN= no key available ,

LAI = deleted LAI, mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

120	SS -> MS	AUTHENTICATION REQUEST	CKSN = initial CKSN

121	MS -> SS	AUTHENTICATION RESPONSE

122	SS -> MS	LOCATION UPDATING ACC	Mobile identity= TMSI

123	MS -> SS	TMSI REALLOCATION COMPL

124	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

MS is now "Idle, updated" in cell B

Specific message contents:

None.

CR for phase 2:

CR N18: from my point of view this CR is not relevant. The channel release has a global meaning for different actions: sending of a CHANNEL RELEASE message or deactivation of the SACCH in the uplink direction (this shows that the MS has released the channels). Furthermore we cannot check exactly T3211: that why I have included "T3211 at least".

CR N19: this CR is taken into account

CR N20: This CR has been taken into account but not for the indicated reason. In fact, the MS has started T3211. So when this timer will expire, the MS will start a location updating procedure. The test has to be performed before T3211 expiry. T3211 is equal to 15 seconds, we propose to wait a time T which is enough for the MS to be pageable, to send paging message, wait for a response during 3 seconds (with the following constraint T + 3 < 15) and then to switch off the MS and check it will not perform an IMSI detach procedure.

26.7.4.3.3	Location updating / abnormal cases / attempt counter equal to 4

[Editorial Note:

- it is not tested that the MS does not answer to paging with TMSI

- in phase 1 Procedure 2 ended with "the MS is switched off" and it did not appear in the requirement that the MS would send an IMSI detach indication: it has been added and now the MS is brought back to operation at the end of the test

]

Conformance requirement:

1)	When four failures such as cases d) to g) of section 4.4.4.8 of Rec GSM 04.08 have occurred during a normal location updating procedure the Mobile Station shall:

1.1	not perform normal location updating procedures.

1.2	perform periodic location updating after T3212 expiry by sending a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type set to "periodic 	updating".

1.3	if the periodic location updating was unsuccessful, after T3211 expiry the Mobile Station shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

2)	When four failures such as cases d), f), g) of section 4.4.4.8 of Rec GSM 04.08 have occurred during a normal location updating procedure the Mobile Station, if it supports speech, shall be able to perform an emergency call i.e. the Mobile Station is able to send a CM Service Request message with the CM Service Type IE set to "emergency call establishment", CKSN IE set to "no key is available" and Mobile Identity IE set to its IMSI and then send an Emergency Setup message.

3)	When four failures such as cases d), f), g) of section 4.4.4.8 of Rec GSM 04.08 have occurred during a normal location updating procedure

3.1	the Mobile Station shall use a request for a Mobile Originating call establishment as a trigger for a normal location updating procedure and shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

3.2	after a location updating triggered by a request from the CM layer which was .unsuccessful, after T3211 expiry the Mobile Station shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

4)	When four failures such as cases d), f), g) of section 4.4.4.8 of Rec GSM 04.08 have occurred during a normal location updating procedure the Mobile Station shall answer to paging with IMSI i.e. send a Channel Request message with the Establishment Cause IE set to "answer to paging" and then send a Paging Response message with the Mobile Identity IE set to its IMSI and CKSN IE set to "no key is available".

5)	When four failures such as cases d), f), g) of section 4.4.4.8 of Rec GSM 04.08 have occurred during a normal location updating procedure

5.1	the Mobile Station shall perform a normal location updating procedure if it enters a new cell.

5.2	if this location updating is unsuccessful, after T3211 expiry the Mobile Station shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

Test purpose:

To verify that the MS performs only periodic location updating procedures, that it does not perform normal location updating procedures when its attempt counter has reached value 4 and that the MS reset its attempt counter after a timer T3212 expiry.

To verify that the MS still follows the "idle not updated" state requirements after its attempt counter has reached value 4.

To verify that the attempt counter is reset in the cases where it has to be done.

Reference(s):

GSM 04.08 section 4.4.4.8 and GSM 05.08 section 6.6.2

Related PICS/PIXIT statement(s):

SIM detachement possible without powering down Yes/No

Switch off on button Yes/No

Value of RF power Capability for Mobile Station Classmark 1 information element.

Support of GSM 900 Yes/No

Support of DCS 1800 Yes/No

Initial conditions:

System Simulator:

Two cells: A and B, belonging to different location areas a and b.

IMSI attach/detach is allowed in both cells.

T3212 is set to 6 minutes.

Mobile Station:

The MS is "Idle, updated" in cell B with a valid CKSN and a TMSI.

Foreseen final state of the MS:

The MS is "Idle, updated" in cell B with a valid CKSN and a TMSI.

Method of test:

The MS is made to perform a normal location updating. The SS triggers a failure in this procedure. After T3211 expiry the MS will try again the location updating procedure. The SS triggers again a failure. This is done again 2 times. At this point the attempt counter shall be equal to 4. It is then checked that T3212 has been started and that at its expiry the MS will try a periodic location updating procedure. It is verified that the MS has reset its attempt counter after timer T3212 expiry.

Then it is checked that, when the attempt counter has reached the value of 4, the MS is in the "idle, updated" state, that is:

-	not perform an IMSI detach procedure,

-	support request for emergency call,

-	use requests from CM layer other than emergency call as triggering of a normal location updating procedure,

-	respond to paging with IMSI,

-	perform  normal location updating procedure when a new cell is entered.

Maximum duration of test:

20 minutes

Expected sequence:

Step	Direction	Message	Comments

1	MS		The RF level of cell B is lowered

until the MS selects cell A.

The RF level of cell B is set

sufficiently low to ensure that

cell B is not suitable as defined

in GSM 05.08 section 6.6.2.

2	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

5	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value

randomly chosen:

* in Table 10.44 of GSM 04.08,

causes #2, #3, #11 and #12 being

excluded, if the MS supports GSM900

(see PICS)

* among #17 and #22,

if the MS supports DCS1800

6	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

7	MS		The MS shall not produce any RF

output during T3211 seconds at

least after the channel release.

8	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

9	SS -> MS	IMMEDIATE ASSIGNMENT

10	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

11	SS		The SS stops any RF transmission on

the dedicated channel and waits

until there is no more SACCH in the

uplink

12	MS		The SS shall not produce any RF

output during T3211 seconds at

least after the channel release

13	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

14	SS -> MS	IMMEDIATE ASSIGNMENT

15	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

16	SS -> MS	AUTHENTICATION REQUEST

17	MS -> SS	AUTHENTICATION RESPONSE	these steps (15 and 16) are

performed N times.

N shall be chosen in such a way

that T3210 expires.

18	MS		The MS shall not produce any RF

output during T3211 seconds at

least after the channel release.

19	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

20	SS -> MS	IMMEDIATE ASSIGNMENT

21	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

22	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

23	MS		The MS shall not produce any RF

output during T3212 (tolerance

[-15s; 45s]) at least after the

channel release.

24	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

25	SS -> MS	IMMEDIATE ASSIGNMENT

26	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic,

CKSN = no key available,

LAI = deleted LAI, mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

27	SS -> MS	LOCATION UPDATING REJ	IE Reject cause = #17 "network

failure"

28	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

29	MS		The MS shall not produce any RF

output during T3211 seconds at

least after the channel release.

30	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

31	SS -> MS	IMMEDIATE ASSIGNMENT

32	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI, mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

33	SS -> MS	AUTHENTICATION REQUEST	CKSN = initial CKSN

34	MS -> SS	AUTHENTICATION RESPONSE

35	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI

36	MS -> SS	TMSI REALLOCATION COMPL

37	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

MS is now "idle, updated" in cell A

38	MS		The RF level of cell A is lowered

until the MS selects cell B.

The RF level of cell A is set

sufficiently low to ensure that

cell A is not suitable as defined

in GSM 05.08 section 6.6.2.

39	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

40	SS -> MS	IMMEDIATE ASSIGNMENT

41	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = a, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

42	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value

randomly chosen:

* in Table 10.44 of GSM 04.08,

causes #2, #3, #11 and #12 being

excluded, if the MS supports GSM900

(see PICS)

* among #17 and #22,

if the MS supports DCS1800

43	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

44	MS		The MS shall not produce any RF

output during T3211 seconds at

least after the channel release.

45	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

46	SS -> MS	IMMEDIATE ASSIGNMENT

47	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

48	SS		The SS stops any RF transmission on

the dedicated channel and waits

until there is no more SACCH in the

uplink

48a	MS		The MS shall not produce any RF

output during T3211 seconds at

least after the channel release.

49	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

50	SS -> MS	IMMEDIATE ASSIGNMENT

51	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

52	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

53	MS		The MS shall not produce any RF

output during T3211 seconds at

least after the channel release.

54	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

55	SS -> MS	IMMEDIATE ASSIGNMENT

56	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI, mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

57	SS		chooses randomly one among the

three following steps: (39 and

40) or 45 or 49

58	SS		performs the chosen step

59	MS		The MS is made to perform an

emergency call

60	MS -> SS	CHANNEL REQUEST	Establishment cause: Emergency call

61	SS -> MS	IMMEDIATE ASSIGNMENT

62	MS -> SS	CM SERVICE REQUEST	CM service type = Emergency call

establishment; CKSN = no key

available; Mobile Identity = IMSI

63	SS -> MS	CM SERVICE ACCEPT

64	MS -> SS	EMERGENCY SETUP

65	SS -> MS	RELEASE COMPLETE	Cause = unassigned number

66	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

67	MS		If possible (see PICS) SIM

detachment is performed. Otherwise

if possible (see PICS) switch off

is performed. Otherwise the power

is removed.

68	MS		The MS must not perform an IMSI

detach. This is checked during 3

seconds.

69	MS		Depending on what has been

performed in step 67 the MS is

brought back to operation.

70	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

71	SS -> MS	IMMEDIATE ASSIGNMENT

72	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

73	SS -> MS	AUTHENTICATION REQUEST	CKSN = initial CKSN

74	MS -> SS	AUTHENTICATION RESPONSE

75	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI

76	MS -> SS	TMSI REALLOCATION COMPL

77	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

MS is now "idle, updated" in cell B

78	MS		The RF level of cell B is lowered

until the MS selects cell A.

The RF level of cell B is set

sufficiently low to ensure that

cell B is not suitable as defined

in GSM 05.08 section 6.6.2.

79	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

80	SS -> MS	IMMEDIATE ASSIGNMENT

81	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

82	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value

randomly chosen:

* in Table 10.44 of GSM 04.08,

causes #2, #3, #11 and #12 being

excluded, if the MS supports GSM900

(see PICS)

* among #17 and #22,

if the MS supports DCS1800

83	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

84	MS		The MS shall not produce any RF

output during T3211 seconds at

least after the channel release.

85	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

86	SS -> MS	IMMEDIATE ASSIGNMENT

87	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

88	SS		The SS stops any RF transmission on

the dedicated channel and waits

until there is no more SACCH in the

uplink

88a	MS		The MS shall not produce any RF

output during T3211 seconds at

least after the channel release.

89	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

90	SS -> MS	IMMEDIATE ASSIGNMENT

91	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

92	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

93	MS		The MS shall not produce any RF

output during T3211 seconds at

least after the channel release.

94	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

95	SS -> MS	IMMEDIATE ASSIGNMENT

96	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI, mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

97	SS		chooses randomly one among the

three following steps: (39 and

40) or 45 or 49

98	SS		performs the chosen step

99	MS		The MS is made to perform a MO call

100	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

101	SS -> MS	IMMEDIATE ASSIGNMENT

102	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

103	SS		chooses randomly one among the

three following steps: (39 and

40) or 45 or 49

104	SS		performs the chosen step

105	MS		The MS shall not produce any RF

output during T3211 seconds at

least after the channel release.

106	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

107	SS -> MS	IMMEDIATE ASSIGNMENT

108	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

109	SS -> MS	AUTHENTICATION REQUEST	CKSN = initial CKSN

110	MS -> SS	AUTHENTICATION RESPONSE

111	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI

112	MS -> SS	TMSI REALLOCATION COMPL

113	SS -> MS	CHANNEL RELEASE	MS is now "idle, updated" in cell A

The MS may have memorized the MO

call request

114	MS -> SS	CHANNEL REQUEST	Establishment cause= other

services requested by the user

115	SS -> MS	IMMEDIATE ASSIGNMENT

116	MS -> SS	CM SERVICE REQUEST	Service type= Mobile originating

call establishment, CKSN=

CKSN= initial value,

Mobile identity= TMSI

117	SS -> MS	CIPHERING MODE COMMAND

118	MS -> SS	CIPHERING MODE COMPLETE

119	MS -> SS	SETUP

120	SS -> MS	RELEASE COMPLETE	Cause = unassigned number

121	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

122	MS		The RF level of cell A is lowered

until the MS selects cell B.

The RF level of cell A is set

sufficiently low to ensure that

cell A is not suitable as defined

in GSM 05.08 section 6.6.2.

123	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

124	SS -> MS	IMMEDIATE ASSIGNMENT

125	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = a, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

126	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value

randomly chosen:

* in Table 10.44 of GSM 04.08,

causes #2, #3, #11 and #12 being

excluded, if the MS supports GSM900

(see PICS)

* among #17 and #22,

if the MS supports DCS1800

127	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

128	MS		The MS shall not produce any RF

output during T3211 seconds at

least after the channel release.

129	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

130	SS -> MS	IMMEDIATE ASSIGNMENT

131	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

132	SS		The SS stops any RF transmission on

the dedicated channel and waits

until there is no more SACCH in the

uplink

132a	MS		The MS shall not produce any RF

output during T3211 seconds at

least after the channel release.

133	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

134	SS -> MS	IMMEDIATE ASSIGNMENT

135	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

136	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

137	MS		The MS shall not produce any RF

output during T3211 seconds at

least after the channel release.

138	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

139	SS -> MS	IMMEDIATE ASSIGNMENT

140	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI, mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

141	SS		chooses randomly one among the

three following steps: (39 and

40) or 45 or 49

142	SS		performs the chosen step and then

waits enough time for the MS to be

pageable

143	SS -> MS	PAGING REQUEST TYPE 1	IE mobile identity = IMSI.

144	MS -> SS	CHANNEL REQUEST	Establishment cause: Answer to

paging

145	SS -> MS	IMMEDIATE ASSIGNMENT

146	MS -> SS	PAGING RESPONSE	CKSN = no key available;

Mobile Identity = IMSI

147	SS -> MS	CHANNEL RELEASE

148	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

149	SS -> MS	IMMEDIATE ASSIGNMENT

150	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

151	SS -> MS	AUTHENTICATION REQUEST	CKSN = initial CKSN

152	MS -> SS	AUTHENTICATION RESPONSE

153	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI

154	MS -> SS	TMSI REALLOCATION COMPL

155	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

MS is now "idle, updated" in cell B

156	MS		The RF level of cell B is lowered

until the MS selects cell A.

The RF level of cell B is set

sufficiently low to ensure that

cell B is not suitable as defined

in GSM 05.08 section 6.6.2.

157	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

158	SS -> MS	IMMEDIATE ASSIGNMENT

159	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

160	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value

randomly chosen:

* in Table 10.44 of GSM 04.08,

causes #2, #3, #11 and #12 being

excluded, if the MS supports GSM900

(see PICS)

* among #17 and #22,

if the MS supports DCS1800

161	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

162	MS		The MS shall not produce any RF

output during T3211 seconds at

least after the channel release.

163	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

164	SS -> MS	IMMEDIATE ASSIGNMENT

165	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

166	SS		The SS stops any RF transmission on

the dedicated channel and waits

until there is no more SACCH in the

uplink

166a	MS		The MS shall not produce any RF

output during T3211 seconds at

least after the channel release.

167	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

168	SS -> MS	IMMEDIATE ASSIGNMENT

169	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

170	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

171	MS		The MS shall not produce any RF

output during T3211 seconds at

least after the channel release.

172	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

173	SS -> MS	IMMEDIATE ASSIGNMENT

174	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI, mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

175	SS		chooses randomly one among the

three following steps: (39 and

40) or 45 or 49

176	SS		performs the chosen step

177	MS		The RF level of cell A is lowered

until the MS selects cell B.

The RF level of cell A is set

sufficiently low to ensure that

cell A is not suitable as defined

in GSM 05.08 section 6.6.2.

178	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

179	SS -> MS	IMMEDIATE ASSIGNMENT

180	MS -> SS	LOCATION UPDATING REQ	location updating type= normal,

CKSN= no key available ,

LAI = deleted LAI, mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

181	SS		chooses randomly one among the

three following steps: (39 and

40) or 45 or 49

182	SS		performs the chosen step

183	MS		The MS shall not produce any RF

output during T3211 seconds at

least after the channel release.

184	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

185	SS -> MS	IMMEDIATE ASSIGNMENT

186	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI,

Mobile Identity =IMSI.

187	SS -> MS	AUTHENTICATION REQUEST	CKSN = initial CKSN

188	MS -> SS	AUTHENTICATION RESPONSE

189	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI

190	MS -> SS	TMSI REALLOCATION COMPL

191	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

MS is now "idle, updated" in cell B

Specific message contents:

None.

CR for phase 2:

CR N07 rev 1: taken into account

CR N18 and N19: same comment as in 26.7.4.3.2

CR N23: no more valid

CR N25 and N26: included

26.7.4.3.4	Location updating / abnormal cases / attempt counter smaller 4, stored LAI equal to broadcast LAI

[Editorial Note:

- In steps 14 and 34 time to check there is no location updating procedure is reduced from 0.5*T3212 (= 3 minutes) to 2*T3211 (=30 seconds). This time is put to check that T3211 has been stopped with the successful MM establishment. This will reduce the test time.

]

Conformance requirement:

1)	When a failure such as cases d), f) and g) of section 4.4.4.8 of Rec GSM 04.08 has occurred during a periodic location updating procedure (the broadcast LAI is equal to the stored LAI),

1.1	the Mobile Station shall be able to perform a mobile originated call i.e. send a Channel Request with the establishment cause IE set to "Other services requested by the user" and then a CM Service Request message with the CM Service Type IE set to "Mobile Originating call establishment", CKSN and LAI set to those which have been allocated to the Mobile Station, Mobile Identity IE set to the TMSI which has been allocated to the Mobile Station

1.2	then the Mobile Station shall not attempt a location updating procedure.

2)	When a failure such as cases d), f) and g) of section 4.4.4.8 of Rec GSM 04.08 has occurred during an IMSI attach procedure (the broadcast LAI is equal to the stored LAI).

2.1	the Mobile Station shall be able to perform a mobile originated call i.e. send a Channel Request with the establishment cause IE set to "Other services requested by the user" and then a CM Service Request message with the CM Service Type IE set to "Mobile Originating call establishment", CKSN and LAI set to those which have been allocated to the Mobile Station, Mobile Identity IE set to the TMSI which has been allocated to the Mobile Station

2.2	then the Mobile Station shall not attempt a location updating procedure.

3)	When a failure such as cases d), f) and g) of section 4.4.4.8 of Rec GSM 04.08 has occurred during a periodic location updating procedure and the attempt counter is smaller than 4 the Mobile Station shall send, after T3211 expiry, a Location Updating Request message with the Mobile Identity IE set to the TMSI which has been allocated to the Mobile Station, CKSN IE and LAI set to those which have been allocated to the Mobile Station and the Location Updating type set to "periodic updating".

3.2	and when the Mobile Station's attempt counter reaches the value 4 (four failures such as cases d), f) and g) of section 4.4.4.8 of Rec GSM 04.08 have occurred during a periodic location updating procedure) after T3212 expiry it shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type set to "periodic updating".

4)	When the Mobile Station's attempt counter reaches the value 4 (four failures such as cases d), f) and g) of section 4.4.4.8 of Rec GSM 04.08 have occurred during a periodic location updating procedure) it shall use a request for a Mobile Originating call as a trigger for a location updating procedure.

5)	When a failure such as cases d), f) and g) of section 4.4.4.8 of Rec GSM 04.08 has occurred during an IMSI attach procedure and the attempt counter is smaller than 4 the Mobile Station shall send, after T3211 expiry, a Location Updating Request message with the Mobile Identity IE set to the TMSI which has been allocated to the Mobile Station, CKSN IE and LAI set to those which have been allocated to the Mobile Station and the Location Updating type set to "IMSI attach".

5.2	and when the Mobile Station's attempt counter reaches the value 4 (four failures such as cases d), f) and g) of section 4.4.4.8 of Rec GSM 04.08 have occurred during an IMSI attach procedure) after T3212 expiry it shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type set to "periodic updating".

6)	When the Mobile Station's attempt counter reaches the value 4 (four failures such as cases d), f) and g) of section 4.4.4.8 of Rec GSM 04.08 have occurred during an IMSI attach procedure) it shall use a request for a Mobile Originating call as a trigger for a location updating procedure.

Test purpose:

To verify that in the case when the attempt counter is smaller than 4 and the broadcast LAI is equal to the stored LAI, the MS is in the "idle updated" state. To verify that timer T3211 is stopped after a MM connection establishment.

To verify that the MS uses the T3211 timer. and that it enters the "idle not updated" state when its attempt counter reaches value 4 even in the case where the stored LAI is equal to the broadcast LAI.

Reference(s):

GSM 04.08 section 4.4.4.8

Related PICS/PIXIT statement(s):

SIM detachement possible without powering down Yes/No

Switch off on button Yes/No

Value of RF power Capability for Mobile Station Classmark 1 information element.

Support of GSM 900 Yes/No

Support of DCS 1800 Yes/No

Initial conditions:

System Simulator:

One cell: B, belonging to location area b.

IMSI attach/detach is allowed.

T3212 is set to 6 minutes.

Mobile Station:

The MS is "Idle, updated" in cell B with a valid CKSN and a TMSI.

Foreseen final state of the MS:

The MS is "idle, updated" in cell B with a valid CKSN and a TMSI.

Method of test:

A failure during the periodic location updating is triggered: as the broadcast LAI is equal to the stored LAI, the MS is still in the "idle, updated" state and timer T3211 is started. A mobile originating call is attempted. It is checked that this is possible and that T3211 is stopped. Same test is performed with a failure during an IMSI attach procedure.

Then failures are triggered during the periodic location updating to let the attempt counter to reach the value of 4. The MS shall enter the "idle, not updated" state and delete any TMSI, stored LAI, ciphering key sequence number and ciphering key. When the attempt counter reaches the value of 4, timer T3212 shall be started. At timer T3212 expiry a location updating procedure is started. A request of a mobile originating call shall trigger a location updating procedure.

Same tests are performed when the failures are triggered during an IMSI attach procedure.

Maximum duration of test:

40 minutes

Expected sequence:

Step	Direction	Message	Comments

1	SS		The SS shall wait at most T3212

2	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

5	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

6	SS		performs the chosen step

7	MS		The MS is made to perform a MO call

8	MS -> SS	CHANNEL REQUEST	Establishment cause= other

services requested by the user

9	SS -> MS	IMMEDIATE ASSIGNMENT

10	MS -> SS	CM SERVICE REQUEST	Service type= Mobile originating

call establishment, CKSN=

initial CKSN, Mobile Identity= TMSI

11	SS -> MS	CM SERVICE ACCEPT

12	MS -> SS	SETUP

13	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

14	SS		The MS shall not attempt to perform

a location updating procedure. This

is checked during 2*T3211

15	MS		If possible (see PICS) SIM

detachment is performed. Otherwise

if possible (see PICS) switch off

is performed. Otherwise the power

is removed.

16	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

17	SS -> MS	IMMEDIATE ASSIGNMENT

18	MS -> SS	IMSI DETACH INDICATION

19	SS -> MS	CHANNEL RELEASE

20	MS		Depending on what has been

performed in step 15 the MS is

brought back to operation.

21	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

22	SS -> MS	IMMEDIATE ASSIGNMENT

23	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI

attach, CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

24	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

25	SS		performs the chosen step

26	MS		The MS is made to perform a MO call

27	MS -> SS	CHANNEL REQUEST	Establishment cause= other

services requested by the user

28	SS -> MS	IMMEDIATE ASSIGNMENT

29	MS -> SS	CM SERVICE REQUEST	Service type= Mobile originating

call establishment, CKSN=

initial CKSN, Mobile Identity= TMSI

30	SS -> MS	CIPHERING MODE COMMAND

31	MS -> SS	CIPHERING MODE COMPLETE

32	MS -> SS	SETUP

33	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

34	SS		The MS shall not attempt to perform

a location updating procedure. This

is checked during 2*T3211

MS is "idle, updated" in cell B.

35	SS		The SS shall wait at most T3212

36	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

37	SS -> MS	IMMEDIATE ASSIGNMENT

38	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

39	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

40	SS		performs the chosen step

41	MS		The MS shall not produce any RF

output during T3211 at

least after the channel release.

42	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

43	SS -> MS	IMMEDIATE ASSIGNMENT

44	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

45	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

46	SS		performs the chosen step

47	MS		The MS shall not produce any RF

output during T3211 at

least after the channel release.

48	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

49	SS -> MS	IMMEDIATE ASSIGNMENT

50	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

51	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

52	SS		performs the chosen step

53	MS		The MS shall not produce any RF

output during T3211 at

least after the channel release.

54	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

55	SS -> MS	IMMEDIATE ASSIGNMENT

56	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

57	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

58	SS		performs the chosen step

59	MS		The MS shall not produce any RF

output during T3212 at

least after the channel release.

60	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

61	SS -> MS	IMMEDIATE ASSIGNMENT

62	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic

or normal (see Note 1),

CKSN = no key available,

LAI = deleted LAI, mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

63	SS -> MS	AUTHENTICATION REQUEST

64	MS -> SS	AUTHENTICATION RESPONSE

65	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity= TMSI

66	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

67	SS		The SS shall wait at most T3212

68	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

69	SS -> MS	IMMEDIATE ASSIGNMENT

70	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

71	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

72	SS		performs the chosen step

73	MS		The MS shall not produce any RF

output during T3211 at

least after the channel release.

74	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

75	SS -> MS	IMMEDIATE ASSIGNMENT

76	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

77	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

78	SS		performs the chosen step

79	MS		The MS shall not produce any RF

output during T3211 at

least after the channel release.

80	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

81	SS -> MS	IMMEDIATE ASSIGNMENT

82	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

83	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

84	SS		performs the chosen step

85	MS		The MS shall not produce any RF

output during T3211 at

least after the channel release.

86	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

87	SS -> MS	IMMEDIATE ASSIGNMENT

88	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

89	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

90	SS		performs the chosen step

91	MS		The MS is made to perform a MO call

92	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

93	SS -> MS	IMMEDIATE ASSIGNMENT

94	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI, mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

95	SS -> MS	LOCATION UPDATING ACC	IE mobile identity = TMSI

96	SS -> MS	CHANNEL RELEASE

97	MS -> SS	CHANNEL REQUEST	Establishment cause= other

services requested by the user

98	SS -> MS	IMMEDIATE ASSIGNMENT

99	MS -> SS	CM SERVICE REQUEST	Service type= Mobile originating

call establishment, CKSN=

no key available,

Mobile identity= TMSI

100	SS -> MS	AUTHENTICATION REQUEST	CKSN = initial CKSN

101	MS -> SS	AUTHENTICATION RESPONSE

102	SS -> MS	CIPHERING MODE COMMAND

103	MS -> SS	CIPHERING MODE COMPLETE

104	MS -> SS	SETUP

105	SS -> MS	RELEASE COMPLETE	Cause = unassigned number

106	SS -> MS	CHANNEL RELEASE

107	MS		If possible (see PICS) SIM

detachment is performed. Otherwise

if possible (see PICS) switch off

is performed. Otherwise the power

is removed.

108	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

109	SS -> MS	IMMEDIATE ASSIGNMENT

110	MS -> SS	IMSI DETACH INDICATION

111	SS -> MS	CHANNEL RELEASE

112	MS		Depending on what has been

performed in step 107 the MS is

brought back to operation.

113	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

114	SS -> MS	IMMEDIATE ASSIGNMENT

115	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI

attach, CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

116	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

117	SS		performs the chosen step

118	MS		The MS shall not produce any RF

output during T3211 at

least after the channel release.

119	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

120	SS -> MS	IMMEDIATE ASSIGNMENT

121	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI

attach, CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

122	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

123	SS		performs the chosen step

124	MS		The MS shall not produce any RF

output during T3211 at

least after the channel release.

125	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

126	SS -> MS	IMMEDIATE ASSIGNMENT

127	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI

attach, CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

128	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

129	SS		performs the chosen step

130	MS		The MS shall not produce any RF

output during T3211 at

least after the channel release.

131	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

132	SS -> MS	IMMEDIATE ASSIGNMENT

133	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI

attach, CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

134	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

135	SS		performs the chosen step

136	MS		The MS shall not produce any RF

output during T3212 at

least after the channel release.

137	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

138	SS -> MS	IMMEDIATE ASSIGNMENT

139	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic

or normal or IMSI attach (see

Note 2), CKSN = no key available,

LAI = deleted LAI, mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

140	SS -> MS	AUTHENTICATION REQUEST

141	MS -> SS	AUTHENTICATION RESPONSE

142	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity= TMSI

142a	MS -> SS	TMSI REALLOCATION COMPL

143	SS -> MS	CHANNEL RELEASE

144	MS		If possible (see PICS) SIM

detachment is performed. Otherwise

if possible (see PICS) switch off

is performed. Otherwise the power

is removed.

145	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

146	SS -> MS	IMMEDIATE ASSIGNMENT

147	MS -> SS	IMSI DETACH INDICATION

148	SS -> MS	CHANNEL RELEASE

149	MS		Depending on what has been

performed in step 144 the MS is

brought back to operation.

150	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

151	SS -> MS	IMMEDIATE ASSIGNMENT

152	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI

attach, CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

153	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

154	SS		performs the chosen step

155	MS		The MS shall not produce any RF

output during T3211 at

least after the channel release.

156	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

157	SS -> MS	IMMEDIATE ASSIGNMENT

158	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI

attach, CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

159	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

160	SS		performs the chosen step

161	MS		The MS shall not produce any RF

output during T3211 at

least after the channel release.

162	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

163	SS -> MS	IMMEDIATE ASSIGNMENT

164	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI

attach, CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

165	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

166	SS		performs the chosen step

167	MS		The MS shall not produce any RF

output during T3211 at

least after the channel release.

168	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

169	SS -> MS	IMMEDIATE ASSIGNMENT

170	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI

attach, CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

171	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

172	SS		performs the chosen step

173	MS		The MS is made to perform a MO call

174	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

175	SS -> MS	IMMEDIATE ASSIGNMENT

176	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI, mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

177	SS -> MS	AUTHENTICATION REQUEST

178	MS -> SS	AUTHENTICATION RESPONSE

179	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity= TMSI

179a	MS -> SS	TMSI REALLOCATION COMPL

180	SS -> MS	CHANNEL RELEASE

181	MS -> SS	CHANNEL REQUEST	Establishment cause= other

services requested by the user

182	SS -> MS	IMMEDIATE ASSIGNMENT

183	MS -> SS	CM SERVICE REQUEST	Service type= Mobile originating

call establishment,

CKSN = initial value,

Mobile identity= TMSI

184	SS -> MS	CM SERVICE ACCEPT

185	MS -> SS	SETUP

186	SS -> MS	RELEASE COMPLETE	Cause = unassigned number

187	SS -> MS	CHANNEL RELEASE

Note 1:	the MS can include both types of Location updating. As T3212 expires it can be a periodic location updating procedure and as there is no stored LAI it can be a normal one.

Note 2:	same problem as in note 1. Three types of location updating procedures should be allowed.

Specific message contents:

None.

26.7.4.4	Location updating / release

[ editorial notes :

- no conformance requirement

]

Test purpose:

To verify that the MS aborts the RR-connection at the expiry of timer T3240.

Reference(s):

GSM 04.08 section 4.4.4.7.

Related PICS/PIXIT statement(s):

None.

Initial conditions:

System Simulator:

Two cells : A and B, belonging to different location areas a and b.

Mobile Station:

The MS has a valid TMSI. It is in the MM-state "idle, updated", updated in cell A and in the RR-idle state.

Foreseen final state of the MS:

The MS is in the MM-state "idle, updated", updated in the cell B and in the RR-idle state.

Method of test:

A normal location updating procedure is performed. The RR-connection is not released by the SS within the timer T3240. It is checked that the MS aborts the RR-connection.

Maximum duration of test:

1 minute

Expected sequence:

Step	Direction	Message	Comments

1	   SS		The RF level of cell A is

lowered until the MS selects

cell B.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ

5	SS -> MS	LOCATION UPDATING ACC

6	   SS		The SS waits T3240 expiry.

7	   MS		The MS shall abort the RR

connection (no more activity on

the SDCCH).

Specific message contents:

None.

26.7.4.5	Location updating / periodic

[Editorial Note:

- this section will be completely revised for phase 2 as TS GSM 04.08 will change

]

26.7.4.5.1	Location updating / periodic spread

Conformance requirement:

The Mobile Stations shall perform spreading of the time before performing a location updating when the location updating timer value is reduced.

Test purpose:

Apart from the test procedures stated here below, there is a requirement for which no unique test could be defined.

When the location updating timer value is reduced, MSs, of which the last location updating has taken place longer ago than the new timer value indicates, shall spread their reaction time before performing a location updating, to prevent a collision of many location updatings from all those MSs.

There is no exact requirement on how this spread should be achieved, and various options are open to the manufacturer.

It shall be demonstrated to the test house that the MSs perform spreading of the time before performing a location updating in such situations.

The test house reacts in an adaptive way, to verify that the mechanism for spreading the time, which is installed according to the manufacturer, is present and functions correctly.

A mechanism shall be present in the MS to achieve spreading in the time before location updating takes place for the situation as described above. The mechanism shall be satisfactory, and it shall function correctly, as observed by the test house.

Reference(s):

GSM 04.08 section 4.4.2.

26.7.4.5.2	Location updating / periodic normal

[ editorial notes :

- storage of T3212 on the SIM is not mandatory in phase 2, therefore the test must be amended

]

Conformance requirement:

1	The Mobile Station shall reset the timer T3212 of the periodic location updating procedure when a mobile originated call has been performed.

2	The Mobile Station shall reset the timer T3212 of the periodic location updating procedure when the Mobile Station has responded to paging.

3	When an IMSI attach procedure is started, the Mobile Station shall perform a periodic location updating after T3212 expiry.

Test purpose:

To verify that the MS resets the timer T3212 of the periodic location updating procedure when :

-	the first MM-message is received in the case of MM-connection establishment,

-	the MS has responded to paging.

To verify that the MS stores the timer T3212 value in the SIM when it is switched off.

To verify that the MS resets the timer T3212 of the periodic location updating procedure when :

-	IMSI attach is started.

Reference(s):

GSM 04.08 section 4.4.2.

Related PICS/PIXIT statement(s):

SIM detachement possible without powering down Yes/No

Switch off on button Yes/No

Initial conditions:

System Simulator:

2 cells, default parameters.

IMSI attach/detach is not allowed in both cells.

The T3212 timeout value is 1/10 hour in both cells for k = 1 and 2/10 hour in both cells for k = 2.

Mobile Station:

The MS has a valid TMSI. It is in the MM-state "idle, updated", updated in cell A and in the RR-idle state.

Foreseen final state of the MS:

The MS has a valid TMSI. It is in the MM-state "idle, updated", updated in cell B and in the RR-idle state.

Method of test:

T3212 is set to 6 minutes. A normal location updating is performed. The channel is released. One minute later, an MS originated call is established and cleared. The channel is released. It is checked that the MS performs a periodic location updating 6 minutes after the last release of the channel.

One minute after the periodic location updating, the MS is paged, it sends a CHANNEL REQUEST message and the SS responds with an IMMEDIATE ASSIGNMENT message and then releases the link. It is checked that the MS performs a periodic location updating 6 minutes after the release of the link.

One minute after the periodic location updating, the MS is switched off for one minute and then switched on again. It is checked that the MS performs a periodic location updating 5 minutes after the MS has been switched on.

The same test is performed with a different value of T3212 (12 minutes).

Then, the IMSI attach/detach is allowed. T3212 is set to 6 minutes.

The SIM is removed. The MS sends an IMSI DETACH INDICATION message. The SIM is re-inserted. It is checked that the MS performs an IMSI attach and a periodic location updating 6 minutes after the IMSI attach.

Maximum duration of test:

for k=1, 33 minutes; for k=2, 51 minutes

Expected sequence:

Step	Direction	Message	Comments

1	   SS		The RF level of cell A is

lowered until the MS selects

cell B.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= normal

5	SS -> MS	LOCATION UPDATING ACC

6	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

7	   SS		The SS waits 1 minute.

8	   MS		A normal call is attempted.

9	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Other

services requested by the

mobile user.

10	SS -> MS	IMMEDIATE ASSIGNMENT

11	MS -> SS	CM SERVICE REQUEST

12	SS -> MS	CM SERVICE ACCEPT

13	MS -> SS	SETUP

14	SS -> MS	RELEASE COMPLETE	"Cause" IE : "unassigned

number".

15	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

16	   SS		The SS waits until the periodic

location updating.

17	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message shall

arrive over 6 minutes for k = 1

and 12 minutes for k = 2 after

the last release of the RR

connection by the SS.

18	SS -> MS	IMMEDIATE ASSIGNMENT

19	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= periodic

20	SS -> MS	LOCATION UPDATING ACC

21	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

22	   SS		The SS waits 1 minute.

23	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity"= IMSI.

24	MS -> SS	CHANNEL REQUEST	"Establishment cause" :

Answer to paging.

25	SS -> MS	IMMEDIATE ASSIGNMENT

26	MS -> SS	PAGING RESPONSE

27	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

28	   SS		The SS waits until the periodic

location updating.

29	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message shall

arrive over 6 minutes for k = 1

and 12 minutes for k = 2 after

the last release of the RR

connection by the SS.

30	SS -> MS	IMMEDIATE ASSIGNMENT

31	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= periodic

32	SS -> MS	LOCATION UPDATING ACC

33	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

34	   SS		The SS waits 1 minute.

35	   MS		If possible (see PICS) switch off

is performed. Otherwise the power

is removed. This is done during 1

minute.

36	   MS		Depending on what has been

performed in step 35 the MS is

brought back to operation.

37	   SS		The SS waits until the periodic

location updating.

38	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message shall

arrive over 7 minutes for k = 1

and 13 minutes for k = 2 after

the last periodic updating.

39	SS -> MS	IMMEDIATE ASSIGNMENT

40	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= periodic

41	SS -> MS	LOCATION UPDATING ACC

42	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

43	   SS		T3212 is set to 6 minutes.

44	   SS		Imsi attach/detach is allowed.

45	   MS		If possible (see PICS) SIM

detachment is performed. Otherwise

if possible (see PICS) switch off

is performed. Otherwise the power

is removed.

46	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

47	SS -> MS	IMMEDIATE ASSIGNMENT

48	MS -> SS	IMSI DETACH INDICATION

49	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

50	   MS		Depending on what has been

performed in step 45 the MS is

brought back to operation.

51	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

52	SS -> MS	IMMEDIATE ASSIGNMENT

52a	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= IMSI

attach.

53	SS -> MS	LOCATION UPDATING ACC

54	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

55	   SS		The SS waits until the periodic

location updating.

56	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message shall

arrive over 6 minutes after

the IMSI attach.

57	SS -> MS	IMMEDIATE ASSIGNMENT

58	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= periodic

59	SS -> MS	LOCATION UPDATING ACC

60	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.5	MM connection

26.7.5.1	Introduction

[ Editorial note : Is an introduction needed ? ]

26.7.5.2	MM connection / establishment with cipher

Conformance requirement:

1)	The Mobile Station shall be able to correctly set up an MM connection in a Mobile Originating call and send a CM Service Request message with CM Service Type information element set to "MO call establishment", CKSN information element as stored in the SIM and Mobile Identity information element set to the TMSI

2)	The Mobile Station shall be able to interpret cipher mode setting as acceptance of its CM service request i.e. send its Setup message.

Test purpose:

To verify that the MS can correctly set up an MM connection in an origination and interpret cipher mode setting as acceptance of its CM service request.

Reference(s):

GSM 04.08 sections 4.5.1.1.

Related PICS/PIXIT statement(s):

None.

Initial conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is in the MM-state "idle, updated" and in the RR-idle state.

Foreseen final state of the MS:

The MS has valid TMSI, CKSN. It is in the MM-state "idle, updated" and in the RR-idle state.

Method of test:

A mobile originating call is initiated. After the MS has sent the CM SERVICE REQUEST message to the SS, an authentication procedure and a ciphering mode setting procedure are performed. Then, the MS sends a SETUP message and the SS clears the call and releases the channel.

Maximum duration of test:

One minute

Expected sequence:

Step	Direction	Message	Comments

1	   MS		A normal call is attempted.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Other

services requested by the

mobile user.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	AUTHENTICATION REQUEST

6	MS -> SS	AUTHENTICATION RESPONSE

7	SS -> MS	CIPHERING MODE COMMAND	The SS starts deciphering.

8	MS -> SS	CIPHERING MODE COMPLETE	The SS starts enciphering.

9	MS -> SS	SETUP

10	SS -> MS	RELEASE COMPLETE	"Cause" IE : "unassigned

number".

11	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.5.3	MM connection / establishment without cipher

[ editorial notes :

- no conformance requirement

]

Test purpose:

To verify that the MS can correctly set up an MM connection in an origination when ciphering mode setting is not required.

Reference(s):

GSM 04.08 sections 4.5.1.1.

Related PICS/PIXIT statement(s):

None.

Initial conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is in the MM-state "idle, updated" and in the RR-idle state.

Foreseen final state of the MS:

The MS has a valid TMSI. It is in the MM-state "idle, updated" and in the RR-idle state.

Method of test:

A mobile originating call is initiated. The MM-connection is established without invoking the ciphering mode setting procedure.

Then, the MS sends a SETUP message and the SS clears the call and releases the channel.

Maximum duration of test:

one minute

Expected sequence:

Step	Direction	Message	Comments

1	   MS		A normal call is attempted.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Other

services requested by the

mobile user.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	CM SERVICE ACCEPT

6	MS -> SS	SETUP

7	SS -> MS	RELEASE COMPLETE	"Cause" IE : "unassigned

number".

8	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.5.4	MM connection / establishment rejected

[ editorial notes :

- no conformance requirement

]

Test purpose:

To verify that the MS does not send a SETUP message when the service request is rejected by the SS.

[Editorial Note:	- the checking of step 6 (the MS does not send its SETUP message) is now clearly defined

]

Reference(s):

GSM 04.08 sections 4.5.1.1.

Related PICS/PIXIT statement(s):

None.

Initial conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is in the MM-state "idle, updated" and in the RR-idle state.

Foreseen final state of the MS:

The MS has a valid TMSI; It is in the MM-state "idle, updated" and in the RR-idle state.

Method of test:

A mobile originating call is initiated. After the MS has sent the CM SERVICE REQUEST message to the SS, the SS responds with a CM SERVICE REJECT message with reject cause "requested service option not subscribed". It is checked that the MS does not send the SETUP message.

Maximum duration of test:

1 minute

Expected sequence:

Step	Direction	Message	Comments

1	   MS		A normal call is attempted.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Other

services requested by the

mobile user.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	CM SERVICE REJECT	"Reject cause" IE : "requested

service option not subscribed".

6	   SS		The MS shall not send a SETUP

message. This is checked during

5 seconds.

7	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.5.5	MM connection / establishment rejected cause 4

Conformance requirement:

1)	The Mobile Station shall be able to correctly set up an MM connection in a Mobile Originating call and send a CM Service Request message with CM Service Type information element set to "MO call establishment", CKSN information element as stored in the SIM and Mobile Identity information element set to the TMSI

2)	The Mobile Station, when receiving a CM SERVICE REJECT message with reject cause "IMSI unknown in VLR" shall wait for the network to release the RR connection

3)	The Mobile Station shall then be able to perform a location updating procedure.

Test purpose:

To verify that the MS can correctly accept a CM SERVICE REJECT message with reject cause "IMSI unknown in VLR".

Reference(s):

GSM 04.08 sections 4.5.1.1.

Related PICS/PIXIT statement(s):

None.

Initial conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is in the MM-state "idle, updated" and in the RR-idle state.

Foreseen final state of the MS:

The MS has valid TMSI, CKSN. It is in the MM-state "idle, updated" and in the RR-idle state.

Method of test:

A mobile originating call is initiated. After the MS has sent the CM SERVICE REQUEST message to the SS, the SS responds with a CM SERVICE REJECT message with reject cause "IMSI unknown in VLR". On receipt of this message, the MS shall delete any TMSI, LAI, cipher key and cipher key sequence number. The channel is released. It is checked that the MS performs a normal location updating procedure.

Maximum duration of test:

One minute

Expected sequence:

Step	Direction	Message	Comments

1	   MS		A normal call is attempted.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Other

services requested by the

mobile user.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	CM SERVICE REJECT	"Reject cause"= "IMSI

unknown in VLR"

6	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

7	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

8	SS -> MS	IMMEDIATE ASSIGNMENT

9	MS -> SS	LOCATION UPDATING REQ	"Ciphering key sequence number"=

"No key is available".

"Mobile identity"= IMSI.

"Location area identification"=

deleted LAI.

11	SS -> MS	AUTHENTICATION REQUEST

12	MS -> SS	AUTHENTICATION RESPONSE

13	SS -> MS	LOCATION UPDATING ACC	"Mobile identity"= new TMSI.

14	MS -> SS	TMSI REALLOCATION CMP

15	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.5.6	MM connection / expiry T3230

[ editorial notes :

- no conformance requirement

]

Test purpose:

To verify that at T3230 expiry, the MS aborts the MM-connection establishment.

Reference(s):

GSM 04.08 section 4.5.1.2.

Related PICS/PIXIT statement(s):

None.

Initial conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is in the MM-state "idle, updated" and in the RR-idle state.

Foreseen final state of the MS:

The MS has a valid TMSI. It is in the MM-state "idle, updated" and in the RR-idle state.

Method of test:

A mobile originating call is initiated. After the MS has sent the CM SERVICE REQUEST message to the SS, the SS waits for expiry of timer T3230. It is checked that the MS does not send a SETUP message but waits for the release of the RR-connection.

Maximum duration of test:

1 minute

Expected sequence:

Step	Direction	Message	Comments

1	   MS		A normal call is attempted.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Other

services requested by the

mobile user.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	   SS		The SS waits for expiry of

timer T3230.

6	SS -> MS	CM SERVICE ACCEPT

7	MS -> SS	MM-STATUS	"Reject cause " IE is "message

not compatible with the call

state or not implemented".

8	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.6	Location updating / release

[ editorial notes :

- no conformance requirement

]

Test purpose:

The MM connection release testing comes basically down to testing the functioning of timer T3240. Testing of that timer is sufficiently covered by the tests on location updating. No separate test on MM connection release is foreseen.

26.7.7	Paging / normal / type 1 no roaming

[these tests will be moved into 26.7. They have to be modified for phase 2, because the core specification has changed a lot.]

(* complete testing does not seem to be very useful - a single test will verify the deletion of the TMSI. *)

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered II.5.3.6.2.1.9 in phase 1 version.

-	the paging parameters are chosen randomly by the SS.

-	this test does already include the update from v 3.5.0 to 3.6.0]

Purpose:

To test that the MS, when in the "idle, roaming not allowed" state, does not answer to PAGING REQUEST TYPE 1 messages which addresses the MS by its former TMSI.

(* To test that when the mobile receives a LOCATION UPDATING REJECT message with cause 'PLMN not allowed', the TMSI is deleted. *)

Conformance Requirement:

An MS, in the "idle, roaming not allowed" state (location updating procedure rejected by the network with cause #11 : PLMN not allowed), shall not answer to paging messages which addresses the MS by its former TMSI.

Reference(s):

GSM 04.08 sections 3.3.2 and 4.4.4.6.

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen randomly by the SS.

Mobile Station:

The MS is in the "idle, roaming not allowed" state.

This state can e.g. be achieved in the following way : First, the MS is allowed to camp on the cell. The LAI of the cell is then changed to an arbitrarily new value. This makes the MS perform a LOCATION UPDATING REQUEST, to which the SS responds with a LOCATION UPDATING REJECT with cause "PLMN not allowed". The MS will now be in the "idle, roaming not allowed" state.

Foreseen Final State of the MS:

"Idle, roaming not allowed".

Method of Test:

The SS pages the MS with a PAGING REQUEST TYPE 1 messages on the paging subchannel which corresponds to the MS's IMSI.

Since the MS is being addressed by its old TMSI (which shall have been deleted) it shall ignore the request. The SS waits 1 second to verify this.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains the MS's

old TMSI 2nd Mobile Ident contains

IMSI of another MS.

2	--------	--------	During 1 second, the SS checks

that the MS does not produce any

RF-transmission.

Specific Message Contents:

None.

26.7.8	Default contents of messages

AUTHENTICATION RESPONSE

Information element	Value/remark

Authentication parameter SRES	As applicable

AUTHENTICATION REJECT

Information element	Value/remark

None but message head

CHANNEL RELEASE

Information element	Value/remark

RR cause	Normal release

CIPHERING MODE COMMAND

Information element	Value/remark

Cipher mode setting	Start ciphering

Cipher Response	IMEI must not be included

CM SERVICE ACCEPT

Information element	Value/remark

None but message head

CM SERVICE REQUEST

Information element	Value/remark

CM service type	Mobile originating call establishment

Ciphering key sequence number	According to SIM contents

Mobile station classmark 2	See PICS/PIXIT

Mobile identity	TMSI of the MS

CM SERVICE REJECT

Information element	Value/remark

Reject cause

IDENTITY REQUEST

Information element	Value/remark

Identity type	Depending on test

Spare half octet	0000

IDENTITY RESPONSE

Information element	Value/remark

Mobile identity	Depending on test

IMMEDIATE ASSIGNMENT

Information element	Value/remark

L2 pseudo length

Page mode	Normal Paging

Spare half octet	0000

Channel description

- Channel type and TDMA offset	SDCCH/4 or SDCCH/8

- Time slot number	Arbitrary legal value

- Subsequent fields of the Channel description IE are specified in section 26.1.1

Request reference

- Random access information	As received from MS

- N51,N32,N26	Corresponding to the frame in which the Channel Request was sent

Timing advance	0

Mobile allocation	Empty (L=0)

Starting time	Omitted

IA rest octets	all bits set to spare

IMSI DETACH INDICATION

Information element	Value/remark

Mobile station classmark 1	See PICS/PIXIT

Mobile identity	TMSI of the MS

LOCATION UPDATING ACCEPT

Information element	Value/remark

Location area identification	As in section 26.1.1

Mobile identity	Depending on test

Follow on proceed	Omitted

PAGING REQUEST TYPE 1

Information element	Value/remark

L2 pseudo length

Page Mode	Normal paging

Channels needed

- mobile 1	"any channel"

- mobile 2	spare

Mobile identity 1	TMSI of MS under test

Mobile identity 2	Omitted

P1 rest octets	All bits set to spare

PAGING RESPONSE

Information element	Value/remark

Ciphering key sequence number	According to SIM contents

Spare half octet	0000

Mobile station classmark 2	See PICS/PIXIT

Mobile identity	TMSI of the MS

TMSI REALLOCATION COMMAND

Information element	Value/remark

Location area identification	As in section 26.1.1

Mobile identity	TMSI

TMSI REALLOCATION COMPLETE

Information element	Value/remark

None but message head