26.8	Tests related to circuit switched call control

26.8.1	Circuit switched Call Control (CC) state machine verification

26.8.1.1	General on Call Control (CC) state machine verification

The principle of checking the call control functions consists in the validation of each call control identified state.

State U0 as an initial state is not verified in the tests of 26.8.1.2 (establishment of an outgoing call).

State U0.1 is never verified.

The steps to be followed within each performed test are:

-	bring the MS into the required state,

-	trigger the tested event,

-	check the MS response and new state.

In sections 26.8.1.2 and 26.8.1.3 different tables are defined to bring the MS into the required initial state. The exact table to be chosen is specified individually in section "Initial conditions" of "Method of test" for each test case.

For each test, unless otherwise specified, a bearer capability among those supported by the MS must be chosen randomly, and the test must be performed according to that bearer capability.

The initial states are to be checked through STATUS ENQUIRY messages sent by the SS, when feasible). This is not explicitly stated in the tables of expected sequences of signalling messages. The checking of final states are explicitly included into the expected sequences of signalling messages.

The following postamble may be used by the SS to bring MS back to idle mode in those test cases, in which it is not already included into expected sequence of signalling messages:

Table 26.8.1.1/1: A postamble to bring the MS back to idle mode.

Step	Direction	Message	Comments

n	SS -> MS	CHANNEL RELEASE

n+1	MS		the MS shall release the main

signalling link (DISC/UA)

The postamble has not been included into the all of the tests in order to leave an option to concatenate the procedures in the future by using a final state of a test case as an initial state to another one.

For the special case of U0, the state is checked by sending STATUS ENQUIRY message with all possible values of transaction identifier (seven values) as U0 is the only state in which for every TI the MS will answer with release complete with cause #81. If U0 is to be verified when no RR connection exists, first a mobile terminating radio connection must be established.

The MS responses are either call management messages received by the SS or lower layers functions activated within the MS or MMI actions (e.g. the buzzing of an alerting tone).

A timeout within the MS is triggered by the SS when it does not answer back an MS expected response.

The test sequences may be split in 3 main groups:

-	establishment and release of an outgoing call;

-	establishment and release of an incoming call;

-	in-call functions.

Remark on verification of transient states:

The following states may be transient, depending on implementation and previous messages:

State U6:

State U6 is always transient. Then there is an internal transition:

CALL CONF

U6 	> U9

State U9:

State U9 is not transient if

-	the implementation does not support immediate connect and

-	an appropriate TCH is not yet assigned and

-	the signalling element has not been present in the SETUP.

If the implementation supports immediate connect, there is an internal transition:

CONN

U9 	> U8

If the appropriate TCH is available or the signalling element was present in SETUP, there is an internal transition:

ALERTING

U9 	> U7

State U7:

If the implementation allows for automatic connect after an implementation specific time T, then there is an internal transition:

after T, CONN

U7 	> U8

State U12:

U12 is a stable state, if the traffic channel is in speech mode, progress indicator is present in DISCONNECT and it indicates the availability of in-band announcements. Otherwise there is an internal transition:

REL

U12 	> U19

A test ending in a transient state (this fact may depend on the implementation) can be executed by verifying that the SS receives the message that accompanies the (next) internal transition (see above).

If a test starts in a transient state, then the test is executed without verification of the starting state.

26.8.1.1.1	Message definition

messages	fields provided

all messages	protocol discriminator

transaction identifier

message type

ALERTING	none

CALL PROCEEDING	none

CONNECT	none

CONNECT ACK	none

DISCONNECT	cause

progress indicator

MODIFY	bearer capability

MODIFY COMPLETE	bearer capability

MODIFY REJECT	cause

bearer capability

NOTIFY	notification indicator

PROGRESS	progress indicator = #8

RELEASE	none

RELEASE COMPLETE	none

SETUP	signal

up to 2 bearer capabilities

progress indicator

START DTMF	keypad facility

START DTMF ACK	keypad facility

START DTMF REJECT	cause

STOP DTMF	none

STOP DTMF ACK	none

STATUS ENQUIRY	none

STATUS	cause

call state

Fields provided in the downlink messages

26.8.1.1.2	List of abbreviations

The following abbreviations are used throughout the section 26.8.1:

AUTH REQUEST		= AUTHENTICATION REQUEST

AUTH RESPONSE		= AUTHENTICATION RESPONSE

CHANNEL MODE MOD ACK	= CHANNEL MODE MODIFY ACKNOWLEDGE

CIPH MODE COMMAND	= CIPHERING MODE COMMAND

CIPH MODE COMPLETE	= CIPHERING MODE COMPLETE

START DTMF ACK		= START DTMF ACKNOWLEDGE

26.8.1.2	Establishment of an outgoing call

Initial conditions

As a minimum requirement the MS is updated and has been given a TMSI, a ciphering key and cipher key sequence number, and the layer 2, RR and MM functionalities have been verified.

There are as many CM initial conditions as states to be checked.

The tables below describe message exchanges which bring the MS in the requested initial states.

A state may be taken as initial only when all the states which lead to this initial states have been validated. The order followed in the test procedure will be U0, U0.1, U1, U3, U4, U10, U12, U19, U11 as seen in the table underneath.

The MS is brought again in the initial state starting with U0 at each new test performed.

Table 26.8.1.2/1: Establishment of an outgoing call, procedure 1 (late assignment)

Step	Direction	Message	Comments/actions/next state

1	MS -> SS	CHANNEL REQUEST	initiate outgoing call

2	SS -> MS	IMMEDIATE ASSIGNMENT	SDCCH, U0

3	MS -> SS	CM-SERVICE REQUEST	U0.1

4	SS -> MS	CIPH MODE COMMAND

5	MS -> SS	CIPH MODE COMPLETE

6	MS -> SS	SETUP	U1

7	SS -> MS	CALL PROCEEDING	U3

8	SS -> MS	ALERTING	U4

9	SS -> MS	ASSIGNMENT COMMAND	TCH

10	MS -> SS	ASSIGNMENT COMPLETE

11	SS -> MS	CONN

12	MS -> SS	CONNECT ACKNOWLEDGE	U10

A13	SS -> MS	DISCONNECT 1)	U12

B13	SS -> MS	DISCONNECT 2)	U12

B14	MS -> SS	RELEASE	U19

C13			MMI action, terminate call

C14	MS -> SS	DISCONNECT	U11

Note 1)	The cause value #8 in band information or appropriate pattern now available.

Note 2)	The cause value is different from #8 in band information or appropriate pattern now available.

Table 26.8.1.2/2: Establishment of an outgoing call, procedure 2

Step	Direction	Message	Comments/actions/next state

1	MS -> SS	CHANNEL REQUEST	initiate outgoing call

2	SS -> MS	IMMEDIATE ASSIGNMENT	TCH, U0

3	MS -> SS	CM-SERVICE REQUEST	U0.1

4	SS -> MS	CHANNEL MODE MODIFY 1)

5	MS -> SS	CHANNEL MODE MOD ACK

6	SS -> MS	CIPH MODE COMMAND

7	MS -> SS	CIPH MODE COMPLETE

8	MS -> SS	SETUP	U1

9	SS -> MS	CALL PROCEEDING	U3

10	SS -> SS	ALERTING	U4

11	SS -> MS	CONN

12	MS -> SS	CONNECT ACKNOWLEDGE	U10

A13	SS -> MS	DISCONNECT 2)	U12

B13	SS -> MS	DISCONNECT 3)	U12

B14	MS -> SS	RELEASE	U19

C13			MMI action, terminate call

C14	MS -> SS	DISCONNECT	U11

Note 1)	Assigned channel is appropriate for the chosen bearer capability (see 26.8.1).

Note 2)	The cause value #8 in band information or appropriate pattern now available.

Note 3)	The cause value is different from #8 in band information or appropriate pattern now available.

Table 26.8.1.2/3: Establishment of an outgoing call, procedure 3

Step	Direction	Message	Comments/actions/next state

1	MS -> SS	CHANNEL REQUEST	initiate outgoing call

2	SS -> MS	IMMEDIATE ASSIGNMENT	SDCCH, U0

3	MS -> SS	CM-SERVICE REQUEST	U0.1

4	SS -> MS	CIPH MODE COMMAND

5	MS -> SS	CIPH MODE COMPLETE

6	MS -> SS	SETUP	U1

7	SS -> MS	AUTH REQUEST

8	MS -> SS	AUTH RESPONSE

9	SS -> MS	CALL PROCEEDING	U3

10	SS -> MS	ASSIGNMENT COMMAND	TCH

11	MS -> SS	ASSIGNMENT COMPLETE

12	SS -> SS	ALERTING	U4

13	SS -> MS	CONN

14	MS -> SS	CONNECT ACKNOWLEDGE	U10

A15	SS -> MS	DISCONNECT 1)	U12

B15	SS -> MS	DISCONNECT 2)	U12

B16	MS -> SS	RELEASE	U19

C15			MMI action, terminate call

C16	MS -> SS	DISCONNECT	U11

Note 1)	The cause value #8 in band information or appropriate pattern now available.

Note 2)	The cause value is different from #8 in band information or appropriate pattern now available.

Table 26.8.1.2/4: Establishment of an outgoing call, procedure 4

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	initiate outgoing call

2	SS -> MS	IMMEDIATE ASSIGNMENT	TCH, U0

3	MS -> SS	CM-SERVICE REQUEST	U0.1

4	SS -> MS	IDENTITY REQUEST

5	MS -> SS	IDENTITY RESPONSE

6	SS -> MS	CIPH MODE COMMAND

7	MS -> SS	CIPH MODE COMPLETE

8	MS -> SS	SETUP	U1

9	SS -> MS	CHANNEL MODE MODIFY 1)

10	MS -> SS	CHANNEL MODE MOD ACK

11	SS -> MS	CALL PROCEEDING	U3

12	SS -> MS	ALERTING	U4

13	SS -> MS	CONN

14	MS -> SS	CONNECT ACKNOWLEDGE	U10

A15	SS -> MS	DISCONNECT 2)	U12

B15	SS -> MS	DISCONNECT 3)	U12

B16	MS -> SS	RELEASE	U19

C15			MMI action, terminate call

C16	MS -> SS	DISCONNECT	U11

Note 1)	Assigned channel is appropriate for the chosen bearer capability (see 26.8.1).

Note 2)	The cause value #8 in band information or appropriate pattern now available.

Note 3)	The cause value is different from #8 in band information or appropriate pattern now available.

26.8.1.2.1	Outgoing call / U0 null state

26.8.1.2.1.1	Outgoing call / U0 null state / MM connection requested

26.8.1.2.1.1.1	Definition and applicability

The call control entity of the Mobile Station requests the MM-sublayer to establish a mobile originating MM-connection. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.1.1.2	Conformance requirement

1)	Upon initiation of an outgoing basic call by user the MS shall initiate establishment of an MM connection, using as first MM message a CM-SERVICE REQUEST message with CM service type "Mobile originating call establishment or packet mode connection establishment".

Reference(s):

TS GSM 04.08 section 5.2.1.1, TS GSM 04.08 section 4.5.1.1, TS GSM 04.08 section 3.3.1.

26.8.1.2.1.1.3	Test purpose

To verify that upon initiation of an outgoing basic call by user the MS initiates establishment of an MM connection, using as first MM message a CM-SERVICE REQUEST message with CM service type "Mobile originating call establishment or packet mode connection establishment".

26.8.1.2.1.1.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS:

U0, null.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. When the SS receives CM-SERVICE REQUEST, the contents of it shall be checked, and also the state of the call control entity shall be checked by STATUS ENQUIRY with all the relevant transaction identifiers.

Maximum duration of test:

30 s

Expected sequence:

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	initiate outgoing call

2	SS -> MS	IMMEDIATE ASSIGNMENT	TCH

3	MS -> SS	CM-SERVICE REQUEST	verify the type of call which is asked

for "basic" or "emergency by the MS

4	SS -> MS	STATUS ENQUIRY

5	MS -> SS	RELEASE COMPLETE	with cause 81# (invalid TI value)

6	SS		repeat steps 4-5 to cover all the

transaction identifiers from 000 ...110

7	SS -> MS	CHANNEL RELEASE

8	MS		the MS shall release the main signalling

link (DISC/UA)

Specific message contents:

None.

26.8.1.2.1.1.5	Requirements

The MS shall respond as indicated in the expected sequence above.

26.8.1.2.2	Outgoing call / U0.1 MM connection pending

26.8.1.2.2.1	Outgoing call / U0.1 MM connection pending / CM service rejected

26.8.1.2.2.1.1	Definition and applicability

A request for MM connection is rejected by the SS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.2.1.2	Conformance requirement

1)	Upon receiving indication of an MM-connection establishment being rejected, CC entity should inform upper layer of this rejection.

Reference(s):

TS GSM 04.08, section 4.5.1.1 , TS GSM 04.07, section 3.2.

26.8.1.2.2.1.3	Test purpose

To verify that a CC entity of the MS in CC-state U0.1, "MM-connection pending", upon the MS receiving a CM SERVICE REJECT message, returns to CC state U0, "Null".

26.8.1.2.2.1.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U0.1 by using table 26.8.1.2/1.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. When the SS receives CM-SERVICE REQUEST, the contents of it shall be checked. The SS rejects it by CM-SERVICE REJECT. Then the SS will check the state of the MS by using STATUS ENQUIRY with all the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	CM-SERVICE REJECT

2	SS -> MS	STATUS ENQUIRY

3	MS -> SS	RELEASE COMPLETE	cause shall be 81# (invalid TI value)

4	SS		repeat steps 4-5 to cover all the

transaction identifiers from 000 ...110

7	SS -> MS	CHANNEL RELEASE

8	MS		the MS shall release the main signalling 						link (DISC/UA)

Specific message contents:

None.

26.8.1.2.2.1.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.2.2	Outgoing call / U0.1 MM connection pending / CM service accepted

26.8.1.2.2.2.1	Definition and applicability

A CM request is accepted for the MM-connection by the SS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.2.2.2	Conformance requirement

1)	A CC entity of the MS in CC-state U0.1, "MM-connection pending", upon the MS receiving a CM SERVICE ACCEPT message, shall send a SETUP message specifying the Called party BCD number that was entered into the MS and then enter CC state U1, "Call initiated".

Reference(s):

TS GSM 04.08, section 4.5.1.1, TS GSM 04.08, section 5.2.1.1.1.

26.8.1.2.2.2.3	Test purpose

To verify that a CC entity of the MS in CC-state U0.1, "MM-connection pending", upon the MS receiving a CM SERVICE ACCEPT message, sends a SETUP message specifying the Called party BCD number that was entered into the MS and then enters CC state U1, "Call initiated".

26.8.1.2.2.2.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U0.1 by using table 26.8.1.2/1.

Foreseen final state of the MS:

U1, call initiated.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. When the MS is requesting a MM-connection, the SS will indicate acceptance by sending a CM-SERVICE ACCEPT message. The MS shall respond with SETUP. Then the SS will check the state of the call control entity by STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	CM-SERVICE ACCEPT

2	MS -> SS	SETUP	with called party BCD number

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause shall be 30# (response to enq.)

and state U1 call initiated

Specific message contents:

None.

26.8.1.2.2.2.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.2.3	Outgoing call / U0.1 MM connection pending / lower layer failure

26.8.1.2.2.3.1	Definition and applicability

The call control entity of the MS being in the state, U0.1, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.2.3.2	Conformance requirement

1)	Upon a lower layer failure the MS releases the MM connection in progress and returns to idle mode. In that state no call exists, and the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

Reference(s):

TS GSM 04.08, section 4.5.1.2, TS GSM 04.08, section 5.1.2.1., TS GSM 04.08 section 5.5.3.2. and TS GSM 04.08 section 8.3.

26.8.1.2.2.3.3	Test purpose

To verify that after the MS with a CC entity in state U0.1, "MM-connection pending", has detected a lower layer failure and has returned to idle mode, the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

26.8.1.2.2.3.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U0.1 by using table 26.8.1.2/1.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

1 min.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. When the MS has sent a CM-SERVICE REQUEST message, the SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1		SS	SS generates lower layer failure

2		SS	SS waits 20 s for the MS to return to

listening to paging

3	SS -> MS	PAGING REQUEST

4	MS -> SS	CHANNEL REQUEST

5	SS -> MS	IMMEDIATE ASSIGNMENT

6	MS -> SS	PAGING RESPONSE

7	SS -> MS	STATUS ENQUIRY

8	MS -> SS	RELEASE COMPLETE	cause shall be 81# (invalid TI value)

9	SS		repeat steps 4-5 to cover all the

transaction identifiers from 000 ...110

10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA).

Specific message contents:

None.

26.8.1.2.2.3.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.3	Outgoing call / U1 call initiated

26.8.1.2.3.1	Outgoing call / U1 call initiated / receiving CALL PROCEEDING

26.8.1.2.3.1.1	Definition and applicability

The call control entity of the MS being in the state, U1, a CALL PRECEEDING message is sent by the SS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.3.1.2	Conformance requirement

1)	A CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a CALL PROCEEDING message, shall enter CC state U3, "Mobile originating call proceeding".

Reference(s):

TS GSM 04.08, section 5.2.1.1.1, 5.2.1.1.2, 5.2.1.1.3.

26.8.1.2.3.1.3	Test purpose

To verify that a CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a CALL PROCEEDING message, enters CC state U3, "Mobile originating call proceeding".

26.8.1.2.3.1.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U1 by using table 26.8.1.2/2.

Foreseen final state of the MS:

U3, Mobile originating call proceeding.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U1. The SS sends a CALL PROCEEDING message to the MS. The SS checks by using the status enquiry procedure that the CC entity has entered the state U3.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	CALL PROCEEDING	tone generation not mandatory

2	SS -> MS	STATUS ENQUIRY

3	MS -> SS	STATUS	cause 30#, state U3

Specific message contents:

None.

26.8.1.2.3.1.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.3.2	Outgoing call / U1 call initiated / rejecting with RELEASE COMPLETE

26.8.1.2.3.2.1	Definition and applicability

The call contrl entity of the MS being in the state, U1, the call is rejected by a RELEASE COMPLETE message sent by the SS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.3.2.2	Conformance requirement

1)	A CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a RELEASE COMPLETE message with valid cause value, shall enter CC state U0, "Null".

2)	On returning to idle mode, the CC entities relating to the seven mobile originating transaction identifiers shall be in state U0, "Null".

3)	On releasing the MM-connection, the MS shall wait for  MM layer release initiated by the network.

Reference(s):

Conformance requirement 1:	TS GSM 04.08, section 5.4.2, TS GSM 04.08, section 5.4.4.

Conformance requirement 2:	TS GSM 04.08, section 5.5.3.2.

Conformance requirement 3:	TS GSM 04.08, section 5.4.4.3, TS GSM 04.08, section 4.5.3, TS GSM 04.08, section 3.5.1.

26.8.1.2.3.2.3	Test purpose

1)	To verify that a CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a RELEASE COMPLETE message with valid cause value, enters CC state U0, "Null".

2)	To verify that in returning to idle mode, the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

3)	To verify that in releasing the MM-connection, the MS shall wait for MM layer release initiated by SS.

26.8.1.2.3.2.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U1 by using table 26.8.1.2/2.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U1. The SS sends a RELEASE COMPLETE message to the MS. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	RELEASE COMPLETE	1)

2	SS -> MS	STATUS ENQUIRY

3	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

4	SS		repeat steps 2-3 to cover all the

transaction identifiers from 000...110

5	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA).

Specific message contents:

1) With a valid cause value among:

related to numbering,

#1 unallocated number

#3 no route to destination

#22 number changed

#28 invalid number format

related to bearer capabilities,

(Editorial note, #8 added)

#8 operator determined barring

#57 bearer capability not authorized

#58 bearer capability not presently available

#63 service or option not available

#65 bearer service not implemented

#34 no circuit/channel available (call queueing).

26.8.1.2.3.2.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.3.3	Outgoing call / U1 call initiated / T303 expiry

26.8.1.2.3.3.1	Definition and applicability

The call contrl entity of the MS being in the state, U1, if no response is then received from the SS, timer T303 expires at the MS side. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.3.3.2	Conformance requirement

1)	A CC entity of the MS in CC-state U1, "Call initiated", upon expiry of T303 (accuracy +/-20% between reception of CM-SERVICE REQUEST and DISCONNECT by SS) shall send a DISCONNECT message to its peer entity and enter state U11, "Disconnect request".

Reference(s):

TS GSM 04.08, section 5.2.1.1, TS GSM 04.08, section 5.4.

26.8.1.2.3.3.3	Test purpose

1)	To verify that a CC entity of the MS in CC-state U1, "Call initiated", upon expiry of T303 (accuracy +/-20% between reception of CM-SERVICE REQUEST and DISCONNECT by SS) sends a DISCONNECT message to its peer entity and enters state U11, "Disconnect request".

26.8.1.2.3.3.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U1 by using table 26.8.1.2/2.

Foreseen final state of the MS:

U11, disconnect request.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U1. The T303 expiries at the MS and the MS shall send DISCONNECT. The SS checks by using the status enquiry procedure that the CC entity has entered the state U11, disconnect request.

Expected sequence:

Step	Direction	Message	Comments

1	SS		SS waits for T303 expiry

2	MS -> SS	DISCONNECT

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, status U11

Specific message contents:

None.

26.8.1.2.3.3.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.3.4	Outgoing call / U1 call initiated / lower layer failure

26.8.1.2.3.4.1	Definition and applicability

The call control entity of the MS being in the state, U1, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.3.4.2	Conformance requirement

1)	Upon a lower layer failure MM informs the relevant CM entities that the MM connection has been interrupted. As call re-establishment is not allowed, the CC entity must perform a local release. The MS returns to idle mode. In that state no call exists, and the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

Reference(s):

TS GSM 04.08, section 4.5.2.3, TS GSM 04.08, section 5.1.2.1, TS GSM 04.08 section 5.5.3.2.

26.8.1.2.3.4.3	Test purpose

To verify that after the MS with a CC entity in state U1 "Call initiated", has detected a lower layer failure and has returned to idle mode, the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

26.8.1.2.3.4.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U1 by using table 26.8.1.2/4.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

1 min.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The MS is brought to the state U1. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure

2	SS		SS waits 20 s for the MS to return to

listening to paging

3	SS -> MS	PAGING REQUEST

4	MS -> SS	CHANNEL REQUEST

5	SS -> MS	IMMEDIATE ASSIGNMENT

6	MS -> SS	PAGING RESPONSE

7	SS -> MS	STATUS ENQUIRY

8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

9	SS		repeat steps 7-8 to cover all the

transaction identifiers from 000...110

10	SS -> MS	CHANNEL RELEASE

11	MS		the MS shall release the main signalling

link (DISC/UA)

Specific message contents:

None.

26.8.1.2.3.4.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.3.5	Outgoing call / U1 call initiated / receiving ALERTING

26.8.1.2.3.5.1	Definition and applicability

The call control entity of the MS being in the state, U1, an ALERTING message is sent to the MS as a indication that a call is being alerted at a called end. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.3.5.2	Conformance requirement

1)	A CC entity of the MS in CC-state U1, "Call initiated", upon receipt of an ALERTING message, shall enter CC state U4, "Call delivered".

Reference(s):

TS GSM 04.08, section 5.2.1.1.

26.8.1.2.3.5.3	Test purpose

To verify that a CC entity of the MS in CC-state U1, "Call initiated", upon receipt of an ALERTING message, enters CC state U4, "Call delivered".

26.8.1.2.3.5.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U1 by using table 26.8.1.2/4.

Foreseen final state of the MS:

U4, call delivered.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U1. The SS sends an ALERTING message to the MS. The SS checks by using the status enquiry procedure that the CC entity has entered the state U4, call delivered.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	ALERTING	alerting indication not mandatory

2	SS -> MS	STATUS ENQUIRY

3	MS -> SS	STATUS	cause 30#, state U4

Specific message contents:

None.

26.8.1.2.3.5.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.3.6	Outgoing call / U1 call initiated / entering state U10

26.8.1.2.3.6.1	Definition and applicability

The call control entity of the MS being in the state, U1, a CONNECT message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.3.6.2	Conformance requirement

1)	A CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a CONNECT message, shall send a CONNECT ACKNOWLEDGE message to its peer entity and enter CC state U10, "Active".

Reference(s):

TS GSM 04.08, section 5.2.1.1.1, TS GSM 04.08, section 5.2.1.1.6.

26.8.1.2.3.6.3	Test purpose

To verify that a CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a CONNECT message, sends a CONNECT ACKNOWLEDGE message to its peer entity and enters CC state U10, "Active".

26.8.1.2.3.6.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U1 by using table 26.8.1.2/4.

Foreseen final state of the MS:

U10, call active.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U1. The SS sends a CONNECT message to the MS. The MS shall respond by sending a CONNECT ACKNOWLEDGE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U10, active.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	CONNECT

2	MS -> SS	CONNECT ACKNOWLEDGE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

26.8.1.2.3.6.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.3.7	Outgoing call / U1 call initiated / unknown message received

26.8.1.2.3.7.1	Definition and applicability

The call control entity of the MS being in the state, U1, an unknown message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.3.7.2	Conformance requirement

1)	A CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a message with message type not defined for the protocol discriminator from its peer entity shall return a STATUS message.

Reference(s):

TS GSM 04.08 section 8.4.

26.8.1.2.3.7.3	Test purpose

To verify that a CC entity of the MS in CC-state U1, "Call initiated", upon receipt of a message with message type not defined for the protocol discriminator unknown message from its peer entity returns a STATUS message.

26.8.1.2.3.7.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U1 by using table 26.8.1.2/1.

Foreseen final state of the MS:

U1, call initiated.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U1. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD

2	MS -> SS	STATUS	cause 97#, state U1

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U1

Specific message contents:

None.

26.8.1.2.3.7.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.4	Outgoing call / U3 MS originating call proceeding

26.8.1.2.4.1	Outgoing call / U3 MS originating call proceeding / ALERTING received

26.8.1.2.4.1.1	Definition and applicability

The call control entity of the MS being in the state, U3, an ALERTING message is sent to the MS as a indication that a call is being alerted at a called end. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.4.1.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a ALERTING message shall enter CC-state U4, "Call Delivered".

Reference(s):

TS GSM 04.08 section 5.2.1.1.5.

26.8.1.2.4.1.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a ALERTING message enters CC-state U4, "Call Delivered".

26.8.1.2.4.1.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/2.

Foreseen final state of the MS:

U4, call delivered.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends an ALERTING message to the MS. The SS checks by using the status enquiry procedure that the CC entity has entered the state U4, call delivered.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	ALERTING	alert tone generation not mandatory

2	SS -> MS	STATUS ENQUIRY

3	MS -> SS	STATUS	cause 30#, state U4

Specific message contents:

None.

26.8.1.2.4.1.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.4.2	Outgoing call / U3 MS originating call proceeding / CONNECT received

26.8.1.2.4.2.1	Definition and applicability

The call control entity of the MS being in the state, U3, a CONNECT message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.4.2.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a CONNECT message shall return a "CONNECT ACKNOWLEDGE" message to its peer entity and enter the CC state U10, "Active".

2)	The MS shall then stop any locally generated indication.

Reference(s):

Conformance requirement 1:	TS GSM 04.08 section 5.2.1.1.6.

Conformance requirement 2:	TS GSM 04.08 section 5.2.1.1.6.

26.8.1.2.4.2.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a CONNECT message returns a "CONNECT ACKNOWLEDGE" message to its peer entity and enters the CC state U10, "Active".

2)	To verify that the MS stops locally generated indication, if any.

26.8.1.2.4.2.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/2.

Foreseen final state of the MS:

U10, active.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends a CONNECT message to the MS. The MS shall respond by sending a CONNECT ACKNOWLEDGE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U10, active.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	CONNECT	the MS shall stop tone generation, if any

2	MS -> SS	CONNECT ACKNOWLEDGE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

26.8.1.2.4.2.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.4.3	Outgoing call / U3 MS originating call proceeding / PROGRESS received without in band information

26.8.1.2.4.3.1	Definition and applicability

The call control entity of the MS being in the state, U3, a PROGRESS message is received by the MS. The PROGRESS message does not contain indication of in-band information availability. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.4.3.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a PROGRESS message with valid cause values shall stay in CC-state U3.

2)	After receipt of the PROGRESS message timer T310 shall be stopped.

Reference(s):

Conformance requirement 1:	TS GSM 04.08 section 5.2.1.1.4.

Conformance requirement 2:	2.1 TS GSM 04.08 section 11.3.

26.8.1.2.4.3.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a PROGRESS message with valid cause values stays in CC-state U3.

2)	To verify that after receipt of the PROGRESS message timer T310 is stopped.

26.8.1.2.4.3.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/2.

Foreseen final state of the MS:

U3, mobile originating call proceeding.

Maximum duration of test:

1 min.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends a PROGRESS message not containing indication of in-band information availability to the MS. The SS checks that the MS has stopped T310, ie at T310 timeout no DISCONNECT message is sent by the MS. Then the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	PROGRESS 1)	the MS shall stop all the CC timers

2	SS -> MS	STATUS ENQUIRY

3	MS -> SS	STATUS	cause 30#, state U3

4	SS		SS waits at least T310 and checks

no DISCONNECT is sent by the MS

5	SS -> MS	STATUS ENQUIRY

6	MS -> SS	STATUS	cause 30#, state U3

Specific message contents:

Note 1)	Tested with a valid cause value among:

#1 call is not end to end PLMN/ISDN

#2 destination address is not PLMN/ISDN

#4 call has returned to PLMN/ISDN

26.8.1.2.4.3.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.4.4	Outgoing call / U3 MS originating call proceeding / PROGRESS with in band information

26.8.1.2.4.4.1	Definition and applicability

The call control entity of the MS being in the state, U3, a PROGRESS message indicating availability of in band information is received by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.4.4.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a PROGRESS message indicating in-band announcement shall through-connect the traffic channel  for speech, if TCH is in a speech mode. If TCH is not in speech mode, the MS shall not through-connect the TCH.

Reference(s):

TS GSM 04.08 section 3.4.3., TS GSM 04.08 section 5.1.3, TS GSM 04.08 section 5.2.1.1.4., TS GSM 04.08 section 5.2.1.1.9.,TS GSM 04.08 section 5.5.1., TS GSM 04.08 section 11.3.

26.8.1.2.4.4.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a PROGRESS message indicating in-band announcement through-connects the traffic channel for speech, if TCH is in speech mode. If TCH is not in a speech mode, the MS does not through-connect the TCH.

26.8.1.2.4.4.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/2.

Foreseen final state of the MS:

U3, mobile originating call proceeding.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends a PROGRESS message containing indication of in-band information availability to the MS. The SS checks that if channel mode is speech, the TCH shall be through connected. If channel mode is not speech, the TCH shall not be through connected. Also the SS checks that the MS has stopped T310, ie at T310 timeout no DISCONNECT message is sent by the MS. Then the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	PROGRESS 1)	the MS shall stop all the CC timers,

if channel mode is speech, the TCH shall be

through connected. If channel mode is not

speech, the TCH shall not be through

connected.

2	SS -> MS	STATUS ENQUIRY

3	MS -> SS	STATUS	cause 30#, state U3

4	SS		SS waits at least T310 and checks

no DISC is sent by the MS

5	SS -> MS	STATUS ENQUIRY

6	MS -> SS	STATUS	cause 30#, state U3

7	SS		the SS will check that the user connection

for speech is attached

(both downlink and uplink)

Specific message contents:

Note 1)	the cause value:

#8 in band information or appropriate pattern now available.

26.8.1.2.4.4.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.4.5	Outgoing call / U3 MS originating call proceeding / DISCONNECT with in band tones

26.8.1.2.4.5.1	Definition and applicability

The call control entity of the MS being in the state, U3, a DISCONNECT message indicating availability of in band information is received by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.4.5.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a DISCONNECT with progress indicator #8, shall through-connect the speech channel to make in-band announcements available, if traffic channel is in speech mode. If TCH is not in speech mode, the MS shall send a RELEASE message.

Reference(s):

TS GSM 04.08 section 5.2.1.1.4., TS GSM 04.08 section 5.4.4.

26.8.1.2.4.5.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a DISCONNECT with progress indicator #8 through-connects the speech channel to make in-band announcements available, if traffic channel is in speech mode. If TCH is not in speech mode, the MS sends a RELEASE message.

26.8.1.2.4.5.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/2.

Foreseen final state of the MS:

U12, disconnect indication.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends a DISCONNECT message containing indication of in-band information availability to the MS. The SS checks that if channel mode is speech, the TCH shall be through connected and the MS enters state U12, disconnect indication. If channel mode is not speech, the TCH shall not be through connected and the MS shall enter state U19, release request.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	1)

TCH in speech mode:

B2	SS		the SS will check that the audio path

for in band tones is attached.

B3	SS -> MS	STATUS ENQUIRY

B4	MS -> SS	STATUS	cause 30#, state U12

TCH is not in speech mode:

C2	MS -> SS	RELEASE

C3	SS -> MS	STATUS ENQUIRY

C4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

Note 1)	the cause value:

#8 in band information or appropriate pattern now available.

26.8.1.2.4.5.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.4.6	Outgoing call / U3 MS originating call proceeding / DISCONNECT without in band tones

26.8.1.2.4.6.1	Definition and applicability

The call control entity of the MS being in the state, U3, a DISCONNECT message is received by the MS. The DISCONNECT message does not contain indication of in-band information availability. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.4.6.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a DISCONNECT without progress indicator shall return a RELEASE message and enter the CC-state U19, "Release Request"

Reference(s):

TS GSM 04.08 section 5.4.4.

26.8.1.2.4.6.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a DISCONNECT without progress indicator returns a RELEASE message and enters the CC-state U19, "Release Request".

26.8.1.2.4.6.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/2.

Foreseen final state of the MS:

U19, release request.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends a DISCONNECT message not containing indication of in-band information availability to the MS. The MS shall respond with a RELEASE message. The SS checks by using the status enquiry procedure that the CC entity of the MS has entered the state U19, release request.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT

2	MS -> SS	RELEASE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

None.

26.8.1.2.4.6.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.4.7	Outgoing call / U3 MS originating call proceeding / RELEASE received

26.8.1.2.4.7.1	Definition and applicability

The call control entity of the MS being in the state, U3, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.4.7.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a RELEASE will return a RELEASE COMPLETEPLETE and enter the CC-state U0, "Null".

2)	The MS on returning to the idle mode shall release the MM-connection and the CC-entities relating to the seven mobile originating transaction identifiers shall be in CC-state U0, "Null".

3)	On releasing the MM-connection, the MS shall wait for  MM layer release initiated by the network.

Reference(s):

Conformance requirement 1:	TS GSM 04.08 section 5.4.2., TS GSM 04.08 section 5.4.4.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2.

Conformance requirement 3:	TS GSM 04.08, section 5.4.4.3, TS GSM 04.08, section 4.5.3, TS GSM 04.08, section 3.5.1.

26.8.1.2.4.7.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon receipt of a RELEASE will return a RELEASE COMPLETEPLETE and enter the CC-state U0, "Null".

2)	To verify that the MS on returning to the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile originating transaction identifiers are in CC-state U0, "Null".

3)	To verify that in releasing the MM-connection, the MS shall wait for  MM layer release initiated by SS.

26.8.1.2.4.7.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/2.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

1 min 30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends a RELEASE message to the MS. The MS shall respond with a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	RELEASE

2	MS -> SS	RELEASE COMPLETE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

5	SS		repeat steps 3-4 to cover all the

transaction identifiers from 000...110

6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA).

Specific message contents:

None.

26.8.1.2.4.7.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.4.8	Outgoing call / U3 MS originating call proceeding / termination requested by the user

26.8.1.2.4.8.1	Definition and applicability

The call contrl entity of the MS being in the state, U3, the user requests to terminate the call. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.4.8.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon request by the user to terminate will send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

Reference(s):

TS GSM 04.07 section 3.2.2., TS GSM 04.08 section 5.4.3.

26.8.1.2.4.8.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", upon request by the user to terminate will send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

26.8.1.2.4.8.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/3.

Foreseen final state of the MS:

U11, disconnect request.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The user requests termination of the call. The MS shall send a DISCONNECT message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U11, disconnect request.

Expected sequence:

Step	Direction	Message	Comments

1			MMI action, terminate call

2	MS -> SS	DISCONNECT	U11

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

26.8.1.2.4.8.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.4.9	Outgoing call / U3 MS originating call proceeding / traffic channel allocation

26.8.1.2.4.9.1	Definition and applicability

The call control entity of the MS being in the state, U3, a traffic channel assignment procedure is performed. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.4.9.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH without changing the state of the call in progress.

Reference(s):

TS GSM 04.08 section 3.4.3., TS GSM 04.08 section 5.2.1.1.9.

26.8.1.2.4.9.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding", when allocated a traffic channel by the network performing the assignment procedure, performs a layer 2 establishment on the FACCH without changing the state of the call in progress.

26.8.1.2.4.9.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/3.

Foreseen final state of the MS:

U3, mobile originating call proceeding.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends an ASSIGNMENT COMMAND for traffic channel to the MS. The MS shall establish layer 2 link on the newly allocated channel and respond with an ASSIGNMENT COMPLETE message. The SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	ASSIGNMENT COMMAND (TCH)	the MS shall perform L2 establishment

on the FACCH

2	MS -> SS	ASSIGNMENT COMPLETE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U3

Specific message contents:

None.

26.8.1.2.4.9.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.4.10	Outgoing call / U3 MS originating call proceeding / timer T310 timeout

26.8.1.2.4.10.1	Definition and applicability

The call contrl entity of the MS being in the state, U3, if no response is then received from the SS, timer T310 expires at the MS side. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.4.10.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding" shall, upon expiry of timer T310, initiate call release by sending DISCONNECT and enter the CC-state U11, "Disconnect Request".

Reference(s):

TS GSM 04.08 section 5.2.1.1.3./Abnormal case, TS GSM 04.08 section 5.4.3, TS GSM 04.08 section 11.3.

26.8.1.2.4.10.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding" will, upon expiry of timer T310, initiate call release by sending DISCONNECT and enter the CC-state U11, "Disconnect Request".

26.8.1.2.4.10.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/3.

Foreseen final state of the MS:

U11, disconnect request.

Maximum duration of test:

1 min.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The T310 expiries at the MS and the MS shall send DISCONNECT. The SS checks timer T310 accuracy and that the CC entity has entered the state U11, disconnect request.

Expected sequence:

Step	Direction	Message	Comments

1	SS		the SS waits for T310 timeout

2	MS -> SS	DISCONNECT	check the timer T310 accuracy

from CALL PROCEEDING emission to

DISCONNECT reception

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

26.8.1.2.4.10.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.4.11	Outgoing call / U3 MS originating call proceeding / lower layer failure

26.8.1.2.4.11.1	Definition and applicability

The call control entity of the MS being in the state, U3, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.4.11.2	Conformance requirement

1)	If a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding" has detected a lower layer failure and has returned to idle mode, the CC entities relating to the seven mobile originating transaction identifiers shall be in state U0, "Null".

Reference(s):

TS GSM 04.08 section 4.5.2.3., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 3.5.2.1.

26.8.1.2.4.11.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding" having detected a lower layer failure and having returned to idle mode, the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

26.8.1.2.4.11.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/4.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

1 min 30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The MS is brought to the state U3. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure

2	SS		SS waits 20 s for the MS to return to

listening to paging

3	SS -> MS	PAGING REQUEST

4	MS -> SS	CHANNEL REQUEST

5	SS -> MS	IMMEDIATE ASSIGNMENT

6	MS -> SS	PAGING RESPONSE

7	SS -> MS	STATUS ENQUIRY

8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

9	SS		repeat steps 18-19 to cover all the

transaction identifiers from 000...110

10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA).

Specific message contents:

None.

26.8.1.2.4.11.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.4.12	Outgoing call / U3 MS originating call proceeding / unknown message received

26.8.1.2.4.12.1	Definition and applicability

The call control entity of the MS being in the state, U3, an unknown message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.4.12.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding" having received an unknown message from its peer entity shall return a STATUS message.

Reference(s):

TS GSM 04.08 section 8.5.

26.8.1.2.4.12.3	Test purpose

To verify that a CC-entity of the MS in CC-state U3, "Mobile Originating Call Proceeding" having received an unknown message from its peer entity returns a STATUS message.

26.8.1.2.4.12.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U3 by using table 26.8.1.2/1.

Foreseen final state of the MS:

U3, mobile originating call proceeding.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U3. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD

2	MS -> SS	STATUS	cause 97#, state U3

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U3

Specific message contents:

None.

26.8.1.2.4.12.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.5	Outgoing call / U4 call delivered

26.8.1.2.5.1	Outgoing call / U4 call delivered / CONNECT received

26.8.1.2.5.1.1	Definition and applicability

The call control entity of the MS being in the state, U4, a CONNECT message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.5.1.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U4, "Call Delivered", upon receipt of the CONNECT message shall return a CONNECT ACKNOWLEDGENOWLEDGE to its peer entity and enter the CC-state U10, "Active".

Reference(s):

TS GSM 04.08 section 5.2.1.1.6.

26.8.1.2.5.1.3	Test purpose

To verify that a CC-entity of the MS in CC-state U4, "Call Delivered", upon receipt of the CONNECT message returns a CONNECT ACKNOWLEDGENOWLEDGE to its peer entity and enters the CC-state U10, "Active".

26.8.1.2.5.1.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/3.

Foreseen final state of the MS:

U10, active.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U4. The SS sends a CONNECT message to the MS. The MS shall respond by sending a CONNECT ACKNOWLEDGE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U10, active.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	CONNECT

2	MS -> SS	CONNECT ACKNOWLEDGE	MS stops alerting, if applicable

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

26.8.1.2.5.1.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.5.2	Outgoing call / U4 call delivered / termination requested by the user

26.8.1.2.5.2.1	Definition and applicability

The call contrl entity of the MS being in the state, U4, the user requests to terminate the call. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.5.2.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U4, "Call Delivered", upon request by the user to terminate shall send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

Reference(s):

TS GSM 04.07 section 3.2.2.,TS GSM 04.08 section 5.4.3.

26.8.1.2.5.2.3	Test purpose

To verify that a CC-entity of the MS in CC-state U4, "Call Delivered", upon request by the user to terminate will send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

26.8.1.2.5.2.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/3.

Foreseen final state of the MS:

U11, disconnect request.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U4. The user requests termination of the call. The MS shall send a DISCONNECT message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U11, disconnect request.

Expected sequence:

Step	Direction	Message	Comments

1			MMI action, terminate call

2	MS -> SS	DISCONNECT

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

26.8.1.2.5.2.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.5.3	Outgoing call / U4 call delivered / DISCONNECT with in band tones

26.8.1.2.5.3.1	Definition and applicability

The call control entity of the MS being in the state, U4, a DISCONNECT message indicating availability of in band information is received by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.5.3.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U4, "Call Delivered" shall, upon receipt of a DISCONNECT with a progress indicator indicating in-band information, shall through-connect the speech channel to make in-band announcements available, if traffic channel is in speech mode. If TCH is not in speech mode, the MS shall send a RELEASE message.

Reference(s):

TS GSM 04.08 section 3.4.3.1., TS GSM 04.08 section 5.1.3., TS GSM 04.08 section 5.4.4.1., TS GSM 04.08 section 5.5.1., TS GSM 04.08 section 5.2.1.1.9.

26.8.1.2.5.3.3	Test purpose

To verify that a CC-entity of the MS in CC-state U4, "Call Delivered", upon receipt of a DISCONNECT with a progress indicator indicating in-band information, through-connects the speech channel to make in-band announcements available, if traffic channel is in speech mode. If TCH is not in speech mode, the MS shall send a RELEASE message.

26.8.1.2.5.3.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/2.

Foreseen final state of the MS:

U12, disconnect indication.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U4. The SS sends a DISCONNECT message containing indication of in-band information availability to the MS. The SS checks that if channel mode is speech, the TCH shall be through connected and the MS enters state U12, disconnect indication. If channel mode is not speech, the TCH shall not be through connected and the MS shall enter state U19, release request.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	1)

TCH in speech mode:

A2	SS		the SS will check that the audio path

for in band tones is attached.

A3	SS -> MS	STATUS ENQUIRY

A4	MS -> SS	STATUS	cause 30#, state U12

TCH is not in speech mode:

B2	MS -> SS	RELEASE

B3	SS -> MS	STATUS ENQUIRY

B4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

Note 1:	the cause value:

#8 in band information or appropriate pattern now available.

26.8.1.2.5.3.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.5.4	Outgoing call / U4 call delivered / DISCONNECT without in band tones

26.8.1.2.5.4.1	Definition and applicability

The call control entity of the MS being in the state, U4, a DISCONNECT message is received by the MS. The DISCONNECT message does not contain indication of in-band information availability. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.5.4.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U4, "Call Delivered", upon receipt of a DISCONNECT without progress indicator, shall return a RELEASE message and enter the CC-state U19, "Release Request".

Reference(s):

TS GSM 04.08 section 5.4.4.

26.8.1.2.5.4.3	Test purpose

To verify that a CC-entity of the MS in CC-state U4, "Call Delivered", upon receipt of a DISCONNECT without progress indicator, returns a RELEASE message and enters the CC-state U19, "Release Request".

26.8.1.2.5.4.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/2.

Foreseen final state of the MS:

U19, release request.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U4. The SS sends a DISCONNECT message not containing indication of in-band information availability to the MS. The MS shall respond with a RELEASE message. The SS checks by using the status enquiry procedure that the CC entity of the MS has entered the state U19, release request.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT

2	MS -> SS	RELEASE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

None.

26.8.1.2.5.4.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.5.5	Outgoing call / U4 call delivered / RELEASE received

26.8.1.2.5.5.1	Definition and applicability

The call control entity of the MS being in the state, U4, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.5.5.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U4, "Call Delivered", upon receipt of the RELEASE message shall respond with the RELEASE COMPLETEPLETE message and enter the CC-state U0, "Null".

2)	The MS on returning to idle mode shall release the MM-connection and the CC-entities relating to the seven mobile originating transaction identifiers shall be in CC-state U0, "Null".

Reference(s):

Conformance requirement 1:	TS GSM 04.08 section 5.4.2., TS GSM 04.08 section 5.4.4.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2.

26.8.1.2.5.5.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U4, "Call Delivered", upon receipt of the RELEASE message will respond with the RELEASE COMPLETEPLETE message and enter the CC-state U0, "Null"

2)	To verify that the MS on returning the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile originating transaction identifiers are in CC-state U0, "Null".

26.8.1.2.5.5.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/2.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

1 min 30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U4. The SS sends a RELEASE message to the MS. The MS shall respond with a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	RELEASE

2	MS -> SS	RELEASE COMPLETE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

5	SS		repeat steps 19-20 to cover all the

transaction identifiers from 000...110

6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA).

Specific message contents:

None.

26.8.1.2.5.5.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.5.6	Outgoing call / U4 call delivered / lower layer failure

26.8.1.2.5.6.1	Definition and applicability

The call control entity of the MS being in the state, U4, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.5.6.2	Conformance requirement

1)	When CC-entity of the MS in CC-state U4, "Call Delivered" has detected a lower layer failure and has returned to idle mode, the CC-entities relating to the seven mobile originating transaction identifiers shall be in CC-state U0, "Null".

Reference(s):

TS GSM 04.08 section 4.5.2.3., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 3.5.2.1.

26.8.1.2.5.6.3	Test purpose

To verify that a CC-entity of the MS in CC-state U4, "Call Delivered" having detected a lower layer failure and has returned to idle mode, the CC-entities relating to the seven mobile originating transaction identifiers are in CC-state U0, "Null".

26.8.1.2.5.6.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/2.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

1 min 30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The MS is brought to the state U4. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure

2	SS		SS waits 20 s for the MS to return to

listening to paging

3	SS -> MS	PAGING REQUEST

4	MS -> SS	CHANNEL REQUEST

5	SS -> MS	IMMEDIATE ASSIGNMENT

6	MS -> SS	PAGING RESPONSE

7	SS -> MS	STATUS ENQUIRY

8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

9	SS		repeat steps 19-20 to cover all the

transaction identifiers from 000...110

10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA).

Specific message contents:

None.

26.8.1.2.5.6.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.5.7	Outgoing call / U4 call delivered / traffic channel allocation

26.8.1.2.5.7.1	Definition and applicability

The call control entity of the MS being in the state, U4, a traffic channel assignment procedure is performed. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.5.7.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U4, "Call Delivered", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH without changing the state of the call in progress.

Reference(s):

TS GSM 04.08 section 3.4.3., TS GSM 04.08 section 5.2.1.1.9.

26.8.1.2.5.7.3	Test purpose

To verify that a CC-entity of the MS in CC-state U4, "Call Delivered", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH without changing the state of the call in progress.

26.8.1.2.5.7.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/3.

Foreseen final state of the MS:

U4, call delivered.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U4. The SS sends an ASSIGNMENT COMMAND for traffic channel to the MS. The MS shall establish layer 2 link on the newly allocated channel and respond with an ASSIGNMENT COMPLETE message. The SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	ASSIGNMENT COMMAND	TCH, the MS shall perform L2 establishment

on the FACCH

2	MS -> SS	ASSIGNMENT COMPLETE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U4

Specific message contents:

None.

26.8.1.2.5.7.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.5.8	Outgoing call / U4 call delivered / unknown message received

26.8.1.2.5.8.1	Definition and applicability

The call control entity of the MS being in the state, U4, an unknown message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.5.8.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U4, "Call Delivered", having received an unknown message from its peer entity shall return a STATUS message.

Reference(s):

TS GSM 04.08 section 8.4.

26.8.1.2.5.8.3	Test purpose

To verify that a CC-entity of the MS in CC-state U4, "Call Delivered", having received an unknown message from its peer entity returns a STATUS message.

26.8.1.2.5.8.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U4 by using table 26.8.1.2/4.

Foreseen final state of the MS:

U4, call delivered.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U4. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD

2	MS -> SS	STATUS	cause 97#, state U4

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U4

Specific message contents:

None.

26.8.1.2.5.8.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.6	U10 call active

26.8.1.2.6.1	U10 call active / termination requested by the user

26.8.1.2.6.1.1	Definition and applicability

The call contrl entity of the MS being in the state, U10, the user requests to terminate the call. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.6.1.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U10, "Call Active", upon request by the user to terminate shall send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

Reference(s):

TS GSM 04.07 section 3.2.2., TS GSM 04.08 section 5.4.3.

26.8.1.2.6.1.3	Test purpose

To verify that the a CC-entity of the MS in CC-state U10, "Call Active", upon request by the user to terminate will send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

26.8.1.2.6.1.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U10 by using table 26.8.1.2/1.

Foreseen final state of the MS:

U11, disconnect request.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U10. The user requests termination of the call. The MS shall send a DISCONNECT message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U11, disconnect request.

Expected sequence:

Step	Direction	Message	Comments

1			MMI action, terminate call

2	MS -> SS	DISCONNECT	U11

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

26.8.1.2.6.1.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.6.2	U10 call active / RELEASE received

26.8.1.2.6.2.1	Definition and applicability

The call control entity of the MS being in the state, U10, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.6.2.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U10, "Call Active", upon receipt of the RELEASE shall respond with the RELEASE COMPLETEPLETE message and enter the CC-state U0, "Null"

2)	When the MS returns to the idle mode it shall release the MM-connection and the CC-entities relating to the seven mobile originating transaction identifiers shall be in CC-state U0, "Null"

Reference(s):

Conformance requirement 1:	TS GSM 04.08 section 5.4.2., TS GSM 04.08 section 5.4.4.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2.

26.8.1.2.6.2.3	Test purpose

1)	To verify that the a CC-entity of the MS in CC-state U10, "Call Active", upon receive of the RELEASE will respond with the RELEASE COMPLETEPLETE message and enter the CC-state U0, "Null"

2)	To verify that the MS on returning to the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile originating transaction identifiers are in CC-state U0, "Null"

26.8.1.2.6.2.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U10 by using table 26.8.1.2/1.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

1 min 30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U10. The SS sends a RELEASE message to the MS. The MS shall respond with a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	RELEASE

2	MS -> SS	RELEASE COMPLETE	the MS starts T3240

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

5	SS		repeat steps 22-23 to cover all the

transaction identifiers from 000...110

6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA)

Specific message contents:

None.

26.8.1.2.6.2.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.6.3	U10 call active / DISCONNECT with in band tones

26.8.1.2.6.3.1	Definition and applicability

The call control entity of the MS being in the state, U10, a DISCONNECT message indicating availability of in band information is received by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.6.3.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U10, "Call Active", upon receipt of a DISCONNECT message with a Progress Indicator indicating in-band information, shall through-connect the speech channel to make in-band announcements available, if traffic channel is in speech mode. If TCH is not in speech mode, the MS shall send a RELEASE message.

Reference(s):

TS GSM 04.08 section 5.4.4.1., TS GSM 04.08 section 5.5.1.

26.8.1.2.6.3.3	Test purpose

To verify that a CC-entity of the MS in CC-state U10, "Call Active", upon receipt of a DISCONNECT message with a Progress Indicator indicating in-band information, through-connects the speech channel to make in-band announcements available, if traffic channel is in speech mode. If TCH is not in speech mode, the MS sends a RELEASE message.

26.8.1.2.6.3.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U10 by using table 26.8.1.2/2.

Foreseen final state of the MS:

U12, disconnect indication.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U10. The SS sends a DISCONNECT message containing indication of in-band information availability to the MS. The SS checks that if channel mode is speech, the TCH shall be through connected and the MS enters state U12, disconnect indication. If channel mode is not speech, the TCH shall not be through connected and the MS enters state U19, release request.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	1)

TCH in speech mode:

A2	SS		the SS will check that the audio path

for in band tones is attached.

A3	SS -> MS	STATUS ENQUIRY

A4	MS -> SS	STATUS	cause 30#, state U12

TCH is not in speech mode:

B2	MS -> SS	RELEASE

B3	SS -> MS	STATUS ENQUIRY

B4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

Note 1:	the cause value:

#8 in band information or appropriate pattern now available.

26.8.1.2.6.3.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.6.4	U10 call active / DISCONNECT without in band tones

26.8.1.2.6.4.1	Definition and applicability

The call control entity of the MS being in the state, U4, a DISCONNECT message is received by the MS. The DISCONNECT message does not contain indication of in-band information availability. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.6.4.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U10, "Call Active", upon receipt of a DISCONNECT message without progress indicator, shall return a RELEASE message and enter the CC-state U19, "Release Request".

Reference(s):

TS GSM 04.08 section 5.4.4.

26.8.1.2.6.4.3	Test purpose

To verify that the a CC-entity of the MS in CC-state U10, "Call Active", upon receipt of a DISCONNECT message without progress indicator, returns a RELEASE message and enters the CC-state U19, "Release Request".

26.8.1.2.6.4.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U10 by using table 26.8.1.2/2.

Foreseen final state of the MS:

U19, release request.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U10. The SS sends a DISCONNECT message not containing indication of in-band information availability to the MS. The MS shall respond with a RELEASE message. The SS checks by using the status enquiry procedure that the CC entity of the MS has entered the state U19, release request.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT

2	MS -> SS	RELEASE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

None.

26.8.1.2.6.4.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.7	U11 disconnect request

26.8.1.2.7.1	U11 disconnect request / clear collision

26.8.1.2.7.1.1	Definition and applicability

The call control entity of the MS being in the state, U11, a DISCONNECT message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.7.1.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U11, "Disconnect Request", upon receipt of a DISCONNECT message, shall return to its peer entity the RELEASE message and enter the CC-state U19, "Release Request".

Reference(s):

TS GSM 04.08 section 5.4.5.

26.8.1.2.7.1.3	Test purpose

To verify that the a CC-entity of the MS in CC-state U11, "Disconnect Request", upon receipt of a DISCONNECT message, returns to its peer entity the RELEASE message and enters the CC-state U19, "Release Request".

26.8.1.2.7.1.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U11 by using table 26.8.1.2/3.

Foreseen final state of the MS:

U19, release request.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U11. The SS sends a DISCONNECT message to the MS. The MS shall respond with a RELEASE message. The SS checks by using the status enquiry procedure that the CC entity of the MS has entered the state U19, release request.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT

2	MS -> SS	RELEASE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

None.

26.8.1.2.7.1.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.7.2	U11 disconnect request / RELEASE received

26.8.1.2.7.2.1	Definition and applicability

The call control entity of the MS being in the state, U11, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.7.2.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U11, "Disconnect Request", upon receipt of the RELEASE message shall return RELEASE COMPLETEPLETE and enter the CC-state U0, "Null".

2)	On returning to the idle mode the MS shall release the MM-connection and the CC-entities relating to the seven mobile originating transaction identifiers shall be in CC-state U0, "Null".

Reference(s):

Conformance requirement 1:	TS GSM 04.08 section 5.4.3.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 8.3.

26.8.1.2.7.2.3	Test purpose

1)	To verify that the a CC-entity of the MS in CC-state U11, "Disconnect Request", upon receipt of the RELEASE message shall return RELEASE COMPLETEPLETE and enter the CC-state U0, "Null".

2)	To verify that the MS on returning to the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile originating transaction identifiers are in CC-state U0, "Null".

26.8.1.2.7.2.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U11 by using table 26.8.1.2/3.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

1 min 30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U11. The SS sends a RELEASE message to the MS. The MS shall respond with a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	RELEASE

2	MS -> SS	RELEASE COMPLETE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

5	SS		repeat steps 3-4 to cover all the

transaction identifiers from 000...110

6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA).

Specific message contents:

None.

26.8.1.2.7.2.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.7.3	U11 disconnect request / timer T305 timeout

26.8.1.2.7.3.1	Definition and applicability

The call contrl entity of the MS being in the state, U11, if no response is then received from the SS, timer T305 expires at the MS side. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.7.3.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U11, "Disconnect Request" shall on expiry of T305, proceed with the connection release procedure by sending the RELEASE message to its peer entity and shall enter the CC-state U19, "Release Request".

Reference(s):

TS GSM 04.08 section 5.4.3., TS GSM 04.08 section 11.3.

26.8.1.2.7.3.3	Test purpose

To verify that the a CC-entity of the MS in CC-state U11, "Disconnect Request" shall on expiry of T305, proceeds ahead with the connection release procedure by sending the RELEASE message to its peer entity and enters the CC-state U19, "Release Request".

26.8.1.2.7.3.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U11 by using table 26.8.1.2/3.

Foreseen final state of the MS:

U19, release request.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U11. The T305 expiries at the MS and the MS shall send a RELEASE message. The SS checks timer T305 accuracy and that the CC entity has entered the state U19, release request.

Expected sequence:

Step	Direction	Message	Comments

1	SS		SS waits until T305 expires at the MS

2	MS -> SS	RELEASE	SS checks the time between DISCONNECT

and RELEASE, 1)

(T305 +/- 10%)

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

Note 1: With the same cause value as originally contained in the DISCONNECT message. An additional cause information element (#102 recovery on timer expiry) may be included.

26.8.1.2.7.3.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.7.4	U11 disconnect request / lower layer failure

26.8.1.2.7.4.1	Definition and applicability

The call control entity of the MS being in the state, U11, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.7.4.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U11, "Disconnect Request" having detected a lower layer failure shall return to the idle mode. The CC entities relating to the seven mobile originating transaction identifiers shall be in state U0, "Null".

Reference(s):

TS GSM 04.08 section 4.5.2.3., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 3.5.2.1., TS GSM 04.08 section 8.3.

26.8.1.2.7.4.3	Test purpose

To verify that the a CC-entity of the MS in CC-state U11, "Disconnect Request" having detected a lower layer failure returns to the idle mode. The CC entities relating to the seven mobile originating transaction identifiers are thus in state U0, "Null".

26.8.1.2.7.4.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U11 by using table 26.8.1.2/4.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

1 min 30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The MS is brought to the state U11. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure

2	SS		SS waits 20 s for the MS to return to

listening to paging

3	SS -> MS	PAGING REQUEST

4	MS -> SS	CHANNEL REQUEST

5	SS -> MS	IMMEDIATE ASSIGNMENT

6	MS -> SS	PAGING RESPONSE

7	SS -> MS	STATUS ENQUIRY

8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

9	SS		repeat steps 7-8 to cover all the

transaction identifiers from 000...110

10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA).

Specific message contents:

None.

26.8.1.2.7.4.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.7.5	U11 disconnect request / unknown message received

26.8.1.2.7.5.1	Definition and applicability

The call control entity of the MS being in the state, U4, an unknown message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.7.5.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U11, "Disconnect Request" shall on expiry of T305, proceed with the connection release procedure by sending the RELEASE message to its peer entity and shall enter the CC-state U19, "Release Request".

Reference(s):

TS GSM 04.08 section 5.4.3., TS GSM 04.08 section 11.3.

26.8.1.2.7.5.3	Test purpose

To verify that the a CC-entity of the MS in CC-state U11, "Disconnect Request" shall on expiry of T305, proceeds ahead with the connection release procedure by sending the RELEASE message to its peer entity and enters the CC-state U19, "Release Request".

26.8.1.2.7.5.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U11 by using table 26.8.1.2/4.

Foreseen final state of the MS:

U11, disconnect request.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U11. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD

2	MS -> SS	STATUS	cause 97#, state U11

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

26.8.1.2.7.5.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.8	U12 disconnect indication

26.8.1.2.8.1	U12 disconnect indication / call releasing requested by the user

26.8.1.2.8.1.1	Definition and applicability

The call control entity of the MS being in the state, U12, the user requests to terminate the call. This test is applicable only for mobile stations supporting bearer capability for speech.

26.8.1.2.8.1.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U12, "Disconnect Indication" being in network initiated call release phase, shall, upon receiving a call release request from the user send a RELEASE to its peer entity and enter CC-state U19, "Release Request".

Reference(s):

TS GSM 04.07 section 3.2.2., TS GSM 04.08 section 5.4.4.

26.8.1.2.8.1.3	Test purpose

To verify that a CC-entity of the MS in CC-state U12, "Disconnect Indication" being in network initiated call release phase, shall, upon receiving a call release request from the user sends a RELEASE to its peer entity and enters CC-state U19, "Release Request"

26.8.1.2.8.1.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U12 by using table 26.8.1.2/1.

Foreseen final state of the MS:

U19, release request.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U12. The user requests termination of the call. The MS shall send a RELEASE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U19, release request.

Expected sequence:

Step	Direction	Message	Comments

1			MMI action, "on hook"

2	MS -> SS	RELEASE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

None.

26.8.1.2.8.1.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.8.2	U12 disconnect indication / RELEASE received

26.8.1.2.8.2.1	Definition and applicability

The call control entity of the MS being in the state, U12, a RELEASE message is received by the MS. This test is applicable only for mobile stations supporting bearer capability for speech.

26.8.1.2.8.2.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U12, "Disconnect Indication", upon receipt of a RELEASE message shall return to its peer entity the RELEASE COMPLETEPLETE message and enter the CC-state U0, "Null".

2)	On returning to the idle mode the MS shall release the MM-connection and the CC-entities relating to the seven mobile originating transaction identifiers shall be in CC-state U0, "Null".

Reference(s):

Conformance requirement 1:	TS GSM 04.08 section 5.4.2

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 8.3.

26.8.1.2.8.2.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U12, "Disconnect Indication", upon receipt of a RELEASE message returns to its peer entity the RELEASE COMPLETEPLETE message and enters the CC-state U0, "Null"

2)	To verify that the MS on returning to the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile originating transaction identifiers are in CC-state U0, "Null".

26.8.1.2.8.2.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U12 by using table 26.8.1.2/1.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

1 min.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U12. The SS sends a RELEASE message to the MS. The MS shall respond with a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	RELEASE

2	MS -> SS	RELEASE COMPLETE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

5	SS		repeat steps 22-23 to cover all the

transaction identifiers from 000...110

6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA).

Specific message contents:

None.

26.8.1.2.8.2.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.8.3	U12 disconnect indication / lower layer failure

26.8.1.2.8.3.1	Definition and applicability

The call control entity of the MS being in the state, U12, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable only for mobile stations supporting bearer capability for speech.

26.8.1.2.8.3.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U12, "Disconnect Indication" having detected a lower layer failure shall return to idle mode. The CC-entities relating to the seven mobile originating transaction identifiers shall be in state U0, "Null".

Reference(s):

TS GSM 04.08 section 4.5.3.2., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 3.5.2.1., TS GSM 04.08 section 8.3.

26.8.1.2.8.3.3	Test purpose

To verify that a CC-entity of the MS in CC-state U12, "Disconnect Indication" having detected a lower layer failure returns to idle mode. The CC-entities relating to the seven mobile originating transaction identifiers are thus in state U0, "Null".

26.8.1.2.8.3.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U12 by using table 26.8.1.2/2.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

1 min 30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The MS is brought to the state U12. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure

2	SS		SS waits 20 s for the MS to return to

listening to paging

3	SS -> MS	PAGING REQUEST

4	MS -> SS	CHANNEL REQUEST

5	SS -> MS	IMMEDIATE ASSIGNMENT

6	MS -> SS	PAGING RESPONSE

7	SS -> MS	STATUS ENQUIRY

8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

9	SS		repeat steps 7-8 to cover all the

transaction identifiers from 000...110

10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA).

Specific message contents:

None.

26.8.1.2.8.3.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.8.4	U12 disconnect indication / unknown message received

26.8.1.2.8.4.1	Definition and applicability

The call control entity of the MS being in the state, U4, an unknown message is received by the MS. This test is applicable only for mobile stations supporting bearer capability for speech.

26.8.1.2.8.4.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U12, "Disconnect Indication" having received an unknown message from its peer entity shall return a STATUS message.

Reference(s):

TS GSM 04.08 section 8.4.

26.8.1.2.8.4.3	Test purpose

To verify that a CC-entity of the MS in CC-state U12, "Disconnect Indication" having received an unknown message from its peer entity returns a STATUS message.

26.8.1.2.8.4.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U12 by using table 26.8.1.2/3.

Foreseen final state of the MS:

U12, disconnect indication.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U12. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD

2	MS -> SS	STATUS	cause 97#, state U12

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U12

Specific message contents:

None.

26.8.1.2.8.4.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.9	Outgoing call / U19 release request

26.8.1.2.9.1	Outgoing call / U19 release request / timer T308 timeout

26.8.1.2.9.1.1	Definition and applicability

The call control entity of the MS being in the state, U19, if no response is then received from the SS, timer T308 expires at the MS side. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.9.1.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U19, "Release Request" will, upon the first expiry of timer T308 send the RELEASE message to its peer entity and remain in the CC-state U19.

Reference(s):

TS GSM 04.08 section 5.4.4.3.1, TS GSM 04.08 section 11.3.

26.8.1.2.9.1.3	Test purpose

To verify that a CC-entity of the MS in CC-state U19, "Release Request" will, upon the first expiry of timer T308 send the RELEASE message to its peer entity and remain in the CC-state U19.

26.8.1.2.9.1.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U19 by using table 26.8.1.2/4.

Foreseen final state of the MS:

U19, release request.

Maximum duration of test:

1 min.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U19. The T308 expiries at the MS and the MS shall send a RELEASE message. The SS checks timer T308 accuracy and that the state of the CC entity has remained unchanged.

Expected sequence:

Step	Direction	Message	Comments

1	SS		SS waits until T308 at the MS

2	MS -> SS	RELEASE	SS checks the time between the two

RELEASE messages (T308 +/- 10%)

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

None.

26.8.1.2.9.1.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.9.2	Outgoing call / U19 release request / 2nd timer T308 timeout

26.8.1.2.9.2.1	Definition and applicability

The call control entity of the MS being in the state, U19, if no response is then received after timer T308 has expired two times in success at the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.9.2.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U19, "Release Request", upon the 2nd expiry of the timer T308, shall enter the CC-state U0, "Null".

2)	Subsequently the MS shall proceed with releasing the MM-connection and enter the idle mode with the CC entities relating to the seven mobile originating transaction identifiers in state U0, "Null".

Reference(s):

Conformance requirement 1:	TS GSM 04.08 section 5.4.4.3.1., TS GSM 04.08 section 11.3.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2.

26.8.1.2.9.2.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U19, "Release Request", upon the 2nd expiry of the timer T308, enters the CC-state U0, "Null".

2)	To verify that subsequently the MS proceeds with releasing the MM-connection and enters the idle mode with the CC entities relating to the seven mobile originating transaction identifiers in state U0, "Null".

26.8.1.2.9.2.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U19 by using table 26.8.1.2/4.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

2 min 30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U19. The SS allows T308 expiry at the MS, and the MS shall repeat sending the RELEASE message and start timer T308 again. The SS allows again T308 expiry at the MS. The MS shall abort the RR-connection (DISC/UA). The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS		SS waits until T308 expiry at the MS

2	MS -> SS	RELEASE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U19

5	SS		SS waits until the second T308 expiry

at the MS

6	SS		SS waits T3240 expiry at the MS

7	MS		the main signalling link shall be

released by the MS (L2: DISC/UA).

8	SS		SS waits 10 s for the MS to return to

listening to paging

9	SS -> MS	PAGING REQUEST

10	MS -> SS	CHANNEL REQUEST

11	SS -> MS	IMMEDIATE ASSIGNMENT

12	MS -> SS	PAGING RESPONSE

13	SS -> MS	STATUS ENQUIRY

14	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

15	SS		repeat steps 13-14 to cover all the

transaction identifiers from 000...110

16	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA).

Specific message contents:

None.

26.8.1.2.9.2.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.9.3	Outgoing call / U19 release request / RELEASE received

26.8.1.2.9.3.1	Definition and applicability

The call control entity of the MS being in the state, U19, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.9.3.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U19, "Release Request", upon receipt of a RELEASE, shall release the MM-connection and enter the CC-state U0, "Null" with the CC entities relating to the seven mobile originating transaction identifiers in state U0, "Null".

Reference(s):

TS GSM 04.08 section 5.4.5., TS GSM 04.08 section 11.3, TS GSM 04.08 section 5.5.3.2.

26.8.1.2.9.3.3	Test purpose

To verify that a CC-entity of the MS in CC-state U19, "Release Request", upon receipt of a RELEASE, shall release the MM-connection and enters the CC-state U0, "Null" with the CC entities relating to the seven mobile originating transaction identifiers in state U0, "Null".

26.8.1.2.9.3.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U19 by using table 26.8.1.2/4.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

1 min 30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U19. The SS sends a RELEASE message to the MS. The MS shall release the MM-connection. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	RELEASE 1)

2	SS -> MS	STATUS ENQUIRY

3	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

4	SS		repeat steps 25-26 to cover all the

transaction identifiers from 000...110

5	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA).

Specific message contents:

Note 1)	With the same cause number as originally contained in DISC and optional cause #102 recovery on timer expiry.

26.8.1.2.9.3.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.9.4	Outgoing call / U19 release request / RELEASE COMPLETE received

26.8.1.2.9.4.1	Definition and applicability

The call control entity of the MS being in the state, U19, a RELEASE COMPLETE message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.9.4.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U19, "Release Request", upon receipt of a RELEASE COMPLETEPLETE, shall release the MM-connection and enter the CC-state U0, "Null" with the CC entities relating to the seven mobile originating transaction identifiers in state U0, "Null".

Reference(s):

TS GSM 04.08 section 5.4.4.3., TS GSM 04.08 section 4.5.3, TS GSM 04.08 section 8.3.

26.8.1.2.9.4.3	Test purpose

To verify that a CC-entity of the MS in CC-state U19, "Release Request", upon receipt of a RELEASE COMPLETEPLETE, shall release the MM-connection and enters the CC-state U0, "Null" with the CC entities relating to the seven mobile originating transaction identifiers in state U0, "Null".

26.8.1.2.9.4.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U19 by using table 26.8.1.2/1.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

1 min 30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U19. The SS sends a RELEASE COMPLETE message to the MS. The MS shall release the MM-connection. The SS checks by using the status enquiry procedure that the CC entity has entered the state U0 with all the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	RELEASE COMPLETE

2	SS -> MS	STATUS ENQUIRY

3	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

4	SS		repeat steps 25-26 to cover all the

transaction identifiers from 000...110

5	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA).

Specific message contents:

None.

26.8.1.2.9.4.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.2.9.5	Outgoing call /  U19 release request / lower layer failure

26.8.1.2.9.5.1	Definition and applicability

The call control entity of the MS being in the state, U19, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.2.9.5.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U19, "Release Request", having detected a lower layer failure, shall return to the idle mode, the CC entities relating to the seven mobile originating transaction identifiers shall be in state U0, "Null".

Reference(s):

TS GSM 04.08 section 4.5.2.3., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 8.3.

26.8.1.2.9.5.3	Test purpose

To verify that a CC-entity of the MS in CC-state U19, "Release Request", having detected a lower layer failure, returns to the idle mode, the CC entities relating to the seven mobile originating transaction identifiers are in state U0, "Null".

26.8.1.2.9.5.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U19 by using table 26.8.1.2/1.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

1 min 30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The MS is brought to the state U19. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure

2	SS		SS waits 20 s for the MS to return to

listening to paging

3	SS -> MS	PAGING REQUEST

4	MS -> SS	CHANNEL REQUEST

5	SS -> MS	IMMEDIATE ASSIGNMENT

6	MS -> SS	PAGING RESPONSE

7	SS -> MS	STATUS ENQUIRY

8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

9	SS		repeat steps 7-8 to cover all the

transaction identifiers from 000...110

10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA).

Specific message contents:

None.

26.8.1.2.9.5.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3	Establishment of an incoming call / Initial conditions

The tables below describe message exchanges which bring the MS in the requested initial states in case of an incoming call.

A state may be taken as initial only when all the states which lead to this initial states have been validated. The order will be U0, U6, U9, U7, U8, U10, U26 etc. as in the following tables.

Table 26.8.1.3/1: Establishment of an incoming call, procedure 1

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT	U0, SDCCH

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	AUTH REQUEST

6	MS -> SS	AUTH RESPONSE

7	SS -> MS	CIPH MODE COMMAND

8	MS -> SS	CIPH MODE COMPLETE

9	SS -> MS	SETUP	U6, 1)

10	MS -> SS	CALL CONFIRMED	U9

A11	MS -> SS	CONNECT	U8, p = Y 2)

B11	MS -> SS	ALERTING	U7, p = N 2)

B12	MS		3)

B13	MS -> SS	CONNECT	U8

14	SS -> MS	ASSIGNMENT COMMAND	TCH

15	MS -> SS	ASSIGNMENT COMPLETE

16	SS -> MS	CONNECT ACKNOWLEDGE	U10

Note 1:	With signal information included in the SETUP message.

Note 2:	The MS is supporting immeditate connect (p = Y/N). See PICS/PIXIT statement.

Note 3:	If necessary (see PICS/PIXIT statement), the MS is made to accept the call in the way described in a PICS/PIXIT statement.

Table 26.8.1.3/2: Establishment of an incoming call, procedure 2

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT	U0, SDCCH

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	CIPH MODE COMMAND

6	MS -> SS	CIPH MODE COMPLETE

7	SS -> MS	SETUP	U6, 1)

8	MS -> SS	CALL CONFIRMED	U9

A9	MS -> SS	CONNECT	U8, p = Y 2)

A10	SS -> MS	ASSIGNMENT COMMAND	TCH

A11	MS -> SS	ASSIGNMENT COMPLETE

B9	MS -> SS	ALERTING	U7, p = N 2)

B10	MS		3)

B11	SS -> MS	ASSIGNMENT COMMAND	TCH

B12	MS -> SS	ASSIGNMENT COMPLETE

B13	MS -> SS	CONNECT	U8

14	SS -> MS	AUTH REQUEST

15	MS -> SS	AUTH RESPONSE

16	SS -> MS	CONNECT ACKNOWLEDGE	U10

Note 1:	With signal information included in the SETUP message.

Note 2:	The MS is supporting immeditate connect (p = Y/N). See PICS/PIXIT statement.

Note 3:	If necessary (see PICS/PIXIT statement), the MS is made to accept the call in the way described in a PICS/PIXIT statement.

Table 26.8.1.3/3: Establishment of an incoming call, procedure 3

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT	U0, TCH

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	AUTH REQUEST

6	MS -> SS	AUTH RESPONSE

7	SS -> MS	CIPH MODE COMMAND

8	MS -> SS	CIPH MODE COMPLETE

9	SS -> MS	CHANNEL MODE MODIFY	1)

10	MS -> SS	CHANNEL MODE MOD ACK

11	SS -> MS	SETUP	U6, 2)

12	MS -> SS	CALL CONFIRMED	U9

A13	MS -> SS	CONNECT	U8, p = Y 3)

B13	MS -> SS	ALERTING	U7, p = N 3)

B14	MS		4)

B15	MS -> SS	CONNECT	U8

16	SS -> MS	CONNECT ACKNOWLEDGE	U10

Note 1:	Assigned channel is appropriate for the chosen bearer capability.

Note 2:	With signal information included in the SETUP message.

Note 3:	The MS is supporting immeditate connect (p = Y/N). See PICS/PIXIT statement.

Note 4:	If necessary (see PICS/PIXIT statement), the MS is made to accept the call in the way described in a PICS/PIXIT statement.

Table 26.8.1.3/4: Establishment of an incoming call, procedure 4

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT	U0, SDCCH

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	CIPH MODE COMMAND

6	MS -> SS	CIPH MODE COMPLETE

7	SS -> MS	SETUP	U6, 1)

8	MS -> SS	CALL CONFIRMED	U9

9	SS -> MS	ASSIGNMENT COMMAND	TCH

10	MS -> SS	ASSIGNMENT COMPLETE

A11	MS -> SS	CONNECT	U8, p = Y 2)

B11	MS -> SS	ALERTING	U7, p = N 2)

B12	MS		3)

B13	MS -> SS	CONNECT	U8

14	SS -> MS	AUTH REQUEST

15	MS -> SS	AUTH RESPONSE

16	SS -> MS	CONNECT ACKNOWLEDGE	U10

Note 1:	The signal information element is not included in the SETUP message.

Note 2:	The MS is supporting immeditate connect (p = Y/N). See PICS/PIXIT statement.

Note 3:	If necessary (see PICS/PIXIT statement), the MS is made to accept the call in the way described in a PICS/PIXIT statement.

26.8.1.3.1	Incoming call / U0 null state

26.8.1.3.1.1	Incoming call / U0 null state / SETUP received with a non supported bearer capability

26.8.1.3.1.1.1	Definition and applicability

The call control entity of the MS being in the state, U19, a SETUP message is received with only one bearer capability not supported by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.3.1.1.2	Conformance requirement

1)	A CC entity of the MS, upon receipt of SETUP containing non supported bearer capability, shall return a RELEASE COMPLETE with correct cause value to its peer entity.

Reference(s):

TS GSM 04.08 section 5.2.2.2., TS GSM 04.08 Annex B.

26.8.1.3.1.1.3	Test purpose

To verify that a CC entity of the MS, upon receipt of SETUP containing non supported bearer capability, returns a RELEASE COMPLETE with correct cause value to its peer entity.

26.8.1.3.1.1.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The MS receives a SETUP message that contains a bearer capability not supported by the MS. The SS checks by using the status enquiry procedure that the CC entity is still in the state U0 with all the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST	SS sends paging

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT (SDCCH)

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	AUTH REQUEST

6	MS -> SS	AUTH RESPONSE

7	SS -> MS	CIPH MODE COMMAND

8	MS -> SS	CIPH MODE COMPLETE

9	SS -> MS	SETUP	1)

10	MS -> SS	RELEASE COMPLETE	2)

Specific message contents:

Note 1:	With a bearer capability not supported by the MS

Note 2:	With one of the following cause field values:

#57 bearer capability not authorized

#58 bearer capability not presently available

#65 bearer service not implemented

#88 incompatible destination

26.8.1.3.1.1.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.2.	Incoming call / U6 call present

26.8.1.3.2.1	Incoming call / U6 call present / automatic call rejection

26.8.1.3.2.1.1	Definition and applicability

Although the state U6 is transient, the ability to refuse a call (automatically) in this state is tested, if it is implemented at the MS. The test is applicable for those equipments described above supporting at least one bearer capability.

26.8.1.3.2.1.2	Conformance requirement

1)	A CC entity of the MS in CC-state U6, "Call Present", shall upon receipt of a rejection indication of the incoming call from the user, send RELEASE COMPLETEPLETE with the appropriate cause value to its peer entity and enter the CC-state U0, "Null". The CC entities relating to the seven mobile terminating transaction identifiers shall be in state U0, "Null".

Reference(s):

TS GSM 11.10, Annex B (for PICS/PIXIT statement), TS GSM 04.07 section 3.2.2., TS GSM 04.08 section 5.2.2.3.1., TS GSM 04.08 section 5.5.3.2, TS GSM 04.08 section 8.3.

26.8.1.3.2.1.3	Test purpose

To verify that a CC entity of the MS in CC-state U6, "Call Present", shall upon receipt of a rejection indication of the incoming call from the user, shall send RELEASE COMPLETEPLETE with the appropriate cause value to its peer entity and enter the CC-state U0, "Null". The CC entities relating to the seven mobile terminating transaction identifiers are then in state U0, "Null".

26.8.1.3.2.1.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	the MS supports an ability to refuse a call after receipt of a SETUP message

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U6 by using table 26.8.1.3/2.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

1 min 30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The call control entiry of the MS is brought to the state U6 (Note ! The state U6 is not checked, since it is not stable). The MS is made to refuse the call (the refusal may require some preliminary preparations in order to achieve refusal at this point). The MS shall send a RELEASE COMPLETE message and enter a call control state U0. The SS checks by using the status enquiry procedure that the CC entity has entered the state state U0 with all the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1			the MS is made to refuse the call

2	MS -> SS	RELEASE COMPLETE	1)

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

5	SS		repeat steps 3-4 to cover all the

transaction identifiers from 000...110

6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA).

Specific message contents:

Note 1:	With cause value #21 call rejected.

26.8.1.3.2.1.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.3	Incoming call / U9 mobile terminating call confirmed

26.8.1.3.3.1	Incoming call / U9 mobile terminating call confirmed / alerting or immediate connecting

26.8.1.3.3.1.1	Definition and applicability

The call control entity of the MS having entered the state, U9, with signal information received in the preceeding SETUP message, the subsequent behaviour of the MS is tested. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.3.3.1.2	Conformance requirement

1)	A CC entity in CC-state U9, "MS Terminating Call Confirmed", (if signalled by the network in previous SETUP message that it may alert) shall either send a ALERTING message to its peer entity and enter state U7, or send a CONNECT message to its peer entity and enter U8.

Reference(s):

TS GSM 04.08 section 5.2.2.3.2., TS GSM 11.10, Annex 3 - section 2.2.

26.8.1.3.3.1.3	Test purpose

To verify that a CC entity in CC-state U9, "MS Terminating Call Confirmed", (if signalled by the network in previous SETUP message that it may alert) will either send a ALERTING message to its peer entity and enter state U7, or send a CONNECT message to its peer entity and enter U8.

26.8.1.3.3.1.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	the MS is supporting immediate connect (p = Y/N)

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U9 by using table 26.8.1.3/2.

Foreseen final state of the MS:

-	U8, connect request, if the MS supports immediate connect

-	otherwise U7, call received

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U9 by using a SETUP message containing signalling information element. (The state U9 is not a stable state in this case, and consequently it is not checked as an initial state.) If the MS supports immediate connect, it sends a CONNECT message and enters the state U8, connect request. Otherwise the MS sends an ALERTING message and enters the state U7, call receiving. The SS checks by using the status enquiry procedure that the CC entity has entered its state as described.

Expected sequence:

Step	Direction	Message	Comments

A11	MS -> SS	CONNECT	p = Y

A12	SS -> MS	STATUS ENQUIRY

A13	MS -> SS	STATUS	cause 30#, state U8

B11	MS -> SS	ALERTING	p = N

B12	SS -> MS	STATUS ENQUIRY

B13	MS -> SS	STATUS	cause 30#, state U7

Specific message contents:

None.

26.8.1.3.3.1.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.3.2	Incoming call / U9 mobile terminating call confirmed / TCH assignment

26.8.1.3.3.2.1	Definition and applicability

The call control entity of the MS being in the state, U9, an assignment procedure is performed for traffic channel. This test is applicable for any equipment supporting at least one bearer capability, except for those supporting immediate connect.

26.8.1.3.3.2.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH, send a ALERTING message and enter state U7.

Reference(s):

TS GSM 04.08 section 3.4.3., TS GSM 04.08 section 5.2.2.7., TS GSM 04.08 section 5.2.2.3.2.

26.8.1.3.3.2.3	Test purpose

To verify that A CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", when allocated a traffic channel by the network performing the assignment procedure, performs a layer 2 establishment on the FACCH, sends a ALERTING message and enters state U7.

26.8.1.3.3.2.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	not applicable for the MSs supporting immediate connect

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U9 by using table 26.8.1.3/4.

Foreseen final state of the MS:

U9, mobile terminating call confirmed.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U9 (by using a SETUP message not containing signalling information element). The SS sends an ASSIGNMENT COMMAND for traffic channel to the MS. The MS shall establish layer 2 link on the newly allocated channel and respond with an ASSIGNMENT COMPLETE message. The MS sends an ALERTING message and enters state U7, call received. The SS verifies by using the status enquiry procedure that the MS has entered the correct state.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	ASSIGNMENT COMMAND	TCH with speech

2	MS		the MS shall establish L2 link

3	MS -> SS	ASSIGNMENT COMPLETE

4	MS -> SS	ALERTING

5	SS -> MS	STATUS ENQUIRY

6	MS -> SS	STATUS	cause 30#, state U7

Specific message contents:

None.

26.8.1.3.3.2.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.3.3	Incoming call / U9 mobile terminating call confirmed / termination requested by the user

26.8.1.3.3.3.1	Definition and applicability

The call control entity of the MS being in the state, U9, th user requests for releasing of the call. This test is applicable for any equipment supporting at least one bearer capability and, addition to this, the facility to send a DISCONNECT message in state U9, except for those supporting immediate connect.

26.8.1.3.3.3.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", upon request by the user to terminate shall send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

Reference(s):

TS GSM 04.07 section 3.2.2., TS GSM 04.08 section 5.4.3.

26.8.1.3.3.3.3	Test purpose

To verify that a CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", upon request by the user to terminate will send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

26.8.1.3.3.3.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	not applicable for those MSs supporting immediate connect

-	the MS supports user requested call clearing in the state U9

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U9 by using table 26.8.1.3/4.

Foreseen final state of the MS:

U11, disconnect request.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U9 (by using a SETUP message not containing signalling information element). Then the user requests termination of the call, if possible. The MS sends a DISCONNECT message and enters state U11, disconnect request. The SS verifies by using the status enquiry procedure that the MS has entered the correct state.

Expected sequence:

Step	Direction	Message	Comments

1			the MS is made to clear the call

2	MS -> SS	DISCONNECT

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

26.8.1.3.3.3.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.3.4	Incoming call / U9 mobile terminating call confirmed / DISCONNECT received

26.8.1.3.3.4.1	Definition and applicability

The call control entity of the MS being in the state, U9, a DISCONNECT message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability, except for those supporting immediate connect.

26.8.1.3.3.4.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", upon receipt of a DISCONNECT shall return a RELEASE message and enter the CC-state U19, "Release Request".

Reference(s):

TS GSM 04.08 section 5.4.4.

26.8.1.3.3.4.3	Test purpose

To verify that a CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", upon receipt of a DISCONNECT returns a RELEASE message and enters the CC-state U19, "Release Request".

26.8.1.3.3.4.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	not applicable for those MSs supporting immediate connect

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U9 by using table 26.8.1.3/4.

Foreseen final state of the MS:

U19, release request.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U9. The SS sends a DISCONNECT message to the MS. The MS responds by sending a RELEASE message and enters state U19, release request. The SS verifies by using the status enquiry procedure that the MS has entered the correct state.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT

2	MS -> SS	RELEASE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

None.

26.8.1.3.3.4.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.3.5	Incoming call / U9 mobile terminating call confirmed / RELEASE received

26.8.1.3.3.5.1	Definition and applicability

The call control entity of the MS being in the state, U9, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability, except for those supporting immediate connect.

26.8.1.3.3.5.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", upon receipt of a RELEASE shall return a RELEASE COMPLETEPLETE and enter the CC-state U0, "Null".

TS GSM 04.08 section 5.4.4.

2)	On returning to the idle mode the MS shall release the MM-connection and the CC-entities relating to the seven mobile terminating transaction identifiers shall be in CC-state U0, "Null".

Reference(s):

Conformance requirement 1:	TS GSM 04.08 section 5.4.4.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 8.3.

26.8.1.3.3.5.3	Test purpose

1)	To verify that a CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed", upon receipt of a RELEASE will return a RELEASE COMPLETEPLETE and enter the CC-state U0, "Null".

2)	To verify that the MS on returning to the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile terminating transaction identifiers are in CC-state U0, "Null".

26.8.1.3.3.5.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	not applicable for those MSs supporting immediate connect

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U9 by using table 26.8.1.3/4.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

1 min 30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U9. The SS sends a RELEASE message to the MS. The MS responds by sending a RELEASE COMPLETE message and enters state U0, null. The SS verifies by using the status enquiry procedure that the MS has entered the correct state with the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	RELEASE

2	MS -> SS	RELEASE COMPLETE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

5	SS		repeat steps 3-4 to cover all the

transaction identifiers from 000...110

6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA).

Specific message contents:

None.

26.8.1.3.3.5.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.3.6	Incoming call / U9 mobile terminating call confirmed / lower layer failure

26.8.1.3.3.6.1	Definition and applicability

The call control entity of the MS being in the state, U9, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one bearer capability, except for those supporting immediate connect.

26.8.1.3.3.6.2	Conformance requirement

1)	A CC entity of the MS in CC-state U9, "MS Terminating Call Confirmed", having detected a lower layer failure shall return to idle mode with the CC entities relating to the seven mobile terminating transaction identifiers in CC-state U0, "Null".

Reference(s):

TS GSM 04.08 section 4.5.2.3., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 8.3

26.8.1.3.3.6.3	Test purpose

To verify that a CC entity of the MS in CC-state U9, "MS Terminating Call Confirmed", having detected a lower layer failure returns to idle mode with the CC entities relating to the seven mobile terminating transaction identifiers in CC-state U0, "Null".

26.8.1.3.3.6.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	not applicable for those MSs supporting immediate connect

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U9 by using table 26.8.1.3/4.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

1 min 30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The MS is brought to the state U9. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure

2	SS		SS waits 20 s for the MS to return to

listening to paging

3	SS -> MS	PAGING REQUEST

4	MS -> SS	CHANNEL REQUEST

5	SS -> MS	IMMEDIATE ASSIGNMENT

6	MS -> SS	PAGING RESPONSE

7	SS -> MS	STATUS ENQUIRY

8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

9	SS		repeat steps 7-8 to cover all the

transaction identifiers from 000...110

10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA).

Specific message contents:

None.

26.8.1.3.3.6.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.3.7	Incoming call / U9 mobile terminating call confirmed / unknown message received

26.8.1.3.3.7.1	Definition and applicability

The call control entity of the MS being in the state, U9, an unknown message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability, except for those supporting immediate connect.

26.8.1.3.3.7.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed" having received an unknown message from its peer entity shall return a STATUS message.

Reference(s):

TS GSM 04.08 section 8.4.

26.8.1.3.3.7.3	Test purpose

To verify that a CC-entity of the MS in CC-state U9, "MS Terminating Call Confirmed" having received an unknown message from its peer entity returns a STATUS message.

26.8.1.3.3.7.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	not applicable for those MSs supporting immediate connect

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U9 by using table 26.8.1.3/4.

Foreseen final state of the MS:

U9, mobile terminating call proceeding.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U9. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD

2	MS -> SS	STATUS	cause 97#, state U9

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U9

Specific message contents:

None.

26.8.1.3.3.7.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.4.	Incoming call / U7 call received

26.8.1.3.4.1	Incoming call / U7 call received / call accepted

26.8.1.3.4.1.1	Definition and applicability

The call control entity of the MS being in the state, U7, a user accepts the incoming call. This test is applicable for any equipment supporting at least one bearer capability, except for those supporting immediate connect.

26.8.1.3.4.1.2	Conformance requirement

1)	A CC entity of a MS in CC-state U7, "Call Received", upon a user accepting the incoming call, shall send a CONNECT message to its peer entity and enter the CC-state U8, "Connect Request".

Reference(s):

TS GSM 04.07 section 3.2.2., TS GSM 04.08 section 5.2.2.5.

26.8.1.3.4.1.3	Test purpose

To verify that a CC entity of a MS in CC-state U7, "Call Received", upon a user accepting the incoming call, shall send a CONNECT message to its peer entity and enter the CC-state U8, "Connect Request"

26.8.1.3.4.1.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	the MS is supporting immediate connect

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/3.

Foreseen final state of the MS:

U8, connect request.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U7. The user accepts the incoming call. The MS sends a CONNECT message. The SS checks by using the status enquiry procedure that the CC entity has entered state U8, connect request.

Expected sequence:

Step	Direction	Message	Comments

1			the MS is made to accept the call by the user

2	MS -> SS	CONNECT

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U8

Specific message contents:

None.

26.8.1.3.4.1.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.4.2	Incoming call / U7 call received / termination requested by the user

26.8.1.3.4.2.1	Definition and applicability

The call control entity of the MS being in the state, U7, a user requests to terminate incoming call. This test is applicable for any equipment supporting at least one bearer capability, except for those supporting immediate connect.

26.8.1.3.4.2.2	Conformance requirement

1)	A CC entity of a MS in CC-state U7, "Call Received", upon request by the user to terminate shall send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

Reference(s):

TS GSM 04.07 section 3.2.2., TS GSM 04.08 section 5.4.3.

26.8.1.3.4.2.3	Test purpose

To verify that a CC entity of a MS in CC-state U7, "Call Received", upon request by the user to terminate will send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

26.8.1.3.4.2.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	the MS is supporting immediate connect

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/3.

Foreseen final state of the MS:

U11, disconnect request.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U7. The user initiates clearing the incoming call. The MS sends a DISCONNECT message. The SS checks by using the status enquiry procedure that the CC entity has entered state U11, disconnect request.

Expected sequence:

Step	Direction	Message	Comments

1			the MS is made to terminate/reject the call

2	MS -> SS	DISCONNECT

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

26.8.1.3.4.2.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.4.3	Incoming call / U7 call received / DISCONNECT received

26.8.1.3.4.3.1	Definition and applicability

The call control entity of the MS being in the state, U7, a DISCONNECT message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability, except for those supporting immediate connect.

26.8.1.3.4.3.2	Conformance requirement

1)	A CC entity of a MS in CC-state U7, "Call Received", upon receipt of a DISCONNECT with a progress indicator indicating in-band information from network, if a TCH was not assigned, shall return a RELEASE message and enter the CC-state U19, "Release Request".

Reference(s):

TS GSM 04.08 section 5.4.4.

26.8.1.3.4.3.3	Test purpose

To verify that a CC entity of a MS in CC-state U7, "Call Received", upon receipt of a DISCONNECT with a progress indicator indicating in-band information from network, if a TCH was not assigned, returns a RELEASE message and enters the CC-state U19, "Release Request".

26.8.1.3.4.3.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	the MS is supporting immediate connect

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/1.

Foreseen final state of the MS:

U19, release request.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U7. The SS sends a DISCONNECT message. The MS responds with a RELEASE message. The SS checks by using the status enquiry procedure that the CC entity has entered state U19, release request.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	1)

2	MS -> SS	RELEASE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

Note 1:	With a progress indicator indicating in-band information; the cause value #8.

26.8.1.3.4.3.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.4.4	Incoming call / U7 call received / RELEASE received

26.8.1.3.4.4.1	Definition and applicability

The call control entity of the MS being in the state, U7, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability, except for those supporting immediate connect.

26.8.1.3.4.4.2	Conformance requirement

1)	A CC entity of a MS in CC-state U7, "Call Received", upon receipt of a RELEASE shall return a RELEASE COMPLETEPLETE and enter the CC-state U0, "Null".

2)	On returning to the idle mode the MS shall release the MM-connection and the CC-entities relating to the seven mobile terminating transaction identifiers shall be in CC-state U0, "Null".

Reference(s):

Conformance requirement 1:	TS GSM 04.08 section 5.4.4.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2.

26.8.1.3.4.4.3	Test purpose

1)	To verify that a CC entity of a MS in CC-state U7, "Call Received", upon receipt of a RELEASE will return a RELEASE COMPLETEPLETE and enter the CC-state U0, "Null".

2)	To verify that the MS on returning to the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile terminating transaction identifiers are in CC-state U0, "Null".

26.8.1.3.4.4.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	the MS is supporting immediate connect

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/1.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

1 min.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U7. The SS sends a RELEASE message. The MS responds with a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity has entered state U0, null, with the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	RELEASE

2	MS -> SS	RELEASE COMPLETE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

5	SS		repeat steps 3-4 to cover all the

transaction identifiers from 000...110

6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA).

Specific message contents:

None.

26.8.1.3.4.4.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.4.5	Incoming call / U7 call received / lower layer failure

26.8.1.3.4.5.1	Definition and applicability

The call control entity of the MS being in the state, U7, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one bearer capability, except for those supporting immediate connect.

26.8.1.3.4.5.2	Conformance requirement

1)	A CC entity of a MS in CC-state U7, "Call Received", having detected a lower layer failure shall return to idle mode with the CC entities relating to the seven mobile terminating transaction identifiers in CC-state U0, "Null".

Reference(s):

TS GSM 04.08 section 4.5.2.3., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2., TS GSM 04.08 section 8.3.

26.8.1.3.4.5.3	Test purpose

To verify that a CC entity of a MS in CC-state U7, "Call Received", having detected a lower layer failure returns to idle mode with the CC entities relating to the seven mobile terminating transaction identifiers in CC-state U0, "Null".

26.8.1.3.4.5.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	the MS is supporting immediate connect

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/2.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

1 min 30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The MS is brought to the state U7. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure

2	SS		SS waits 20 s for the MS to return to

listening to paging

3	SS -> MS	PAGING REQUEST

4	MS -> SS	CHANNEL REQUEST

5	SS -> MS	IMMEDIATE ASSIGNMENT

6	MS -> SS	PAGING RESPONSE

7	SS -> MS	STATUS ENQUIRY

8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

9	SS		repeat steps 7-8 to cover all the

transaction identifiers from 000...110

10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA).

Specific message contents:

None.

26.8.1.3.4.5.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.4.6	Incoming call / U7 call received / unknown message received

26.8.1.3.4.6.1	Definition and applicability

The call control entity of the MS being in the state, U7, an unknown message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability, except for those supporting immediate connect.

26.8.1.3.4.6.2	Conformance requirement

1)	A CC entity of a MS in CC-state U7, "Call Received", having received an unknown message from its peer entity shall return a STATUS message.

Reference(s):

TS GSM 04.08 section 8.4.

26.8.1.3.4.6.3	Test purpose

To verify that a CC entity of a MS in CC-state U7, "Call Received", having received an unknown message from its peer entity returns a STATUS message.

26.8.1.3.4.6.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	the MS is supporting immediate connect

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/3.

Foreseen final state of the MS:

U7, call received.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U7. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD

2	MS -> SS	STATUS	cause 97#, state U7

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U7

Specific message contents:

None.

26.8.1.3.4.6.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.4.7	Incoming call / U7 call received / TCH assignment

26.8.1.3.4.7.1	Definition and applicability

The call control entity of the MS being in the state, U7, an assignment procedure is performed for traffic channel. This test is applicable for any equipment supporting at least one bearer capability, except for those supporting immediate connect.

26.8.1.3.4.7.2	Conformance requirement

1)	A CC entity of a MS in CC-state U7, "Call Received", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH without changing the state of the call in progress.

Reference(s):

TS GSM 04.08 section 3.4.3., TS GSM 04.08 section 5.2.2.7.

26.8.1.3.4.7.3	Test purpose

To verify that a CC entity of a MS in CC-state U7, "Call Received", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH without changing the state of the call in progress.

26.8.1.3.4.7.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	the MS is supporting immediate connect

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U7 by using table 26.8.1.3/1.

Foreseen final state of the MS:

U7, call received.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U7. The SS sends an ASSIGNMENT COMMAND for traffic channel to the MS. The MS shall establish layer 2 link on the newly allocated channel and respond with an ASSIGNMENT COMPLETE message. The SS verifies by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	ASSIGNMENT COMMAND	TCH

2	MS		the MS shall establish L2 link

3	MS -> SS	ASSIGNMENT COMPLETE

4	SS -> MS	STATUS ENQUIRY

5	MS -> SS	STATUS	cause 30#, state U7

Specific message contents:

None.

26.8.1.3.4.7.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.5.	Incoming call / U8 connect request

26.8.1.3.5.1	Incoming call / U8 connect request / CONNECT acknowledged

26.8.1.3.5.1.1	Definition and applicability

The call control entity of the MS being in the state, U8, a CONNECT ACKNOWLEDGE message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.3.5.1.2	Conformance requirement

1)	A CC entity of a MS in CC-state U8, "Connect Request", upon receipt of CONNECT REQUEST shall enter the CC-state U10, "Call Active".

Reference(s):

TS GSM 04.08 section 5.2.2.6.

26.8.1.3.5.1.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", upon receipt of CONNECT REQUEST shall enter the CC-state U10, "Call Active".

26.8.1.3.5.1.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	the MS is supporting immediate connect (p = Y/N)

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/2.

Foreseen final state of the MS:

U10, active.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U8. The SS sends a CONNECT ACKNOWLEDGE message. The SS checks by using the status enquiry procedure that the CC entity of the MS has entered state U10, active.

Expected sequence:

Step	Direction	Message	Comments

A1	SS -> MS	ASSIGNMENT COMMAND	p = Y

A2	MS -> SS	ASSIGNMENT COMPLETE

3	SS -> MS	CONNECT ACKNOWLEDGE

4	SS -> MS	STATUS ENQUIRY

5	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

26.8.1.3.5.1.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.5.2	Incoming call / U8 connect request / timer T313 timeout

26.8.1.3.5.2.1	Definition and applicability

The call control entity of the MS being in the state, U8, if no response is then received from the SS, timer T308 expires at the MS side. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.3.5.2.2	Conformance requirement

1)	A CC entity of a MS in CC-state U8, "Connect Request", having waited for a reasonable length of time (expiry of timer T313) without receiving the appropriate protocol message to complete the incoming call, shall initiate the clearing of that incoming call by sending the CC message DISCONNECT and enter the CC-state U11, "Disconnect Request".

Reference(s):

TS GSM 04.08 section 5.2.2.6., TS GSM 04.08 section 5.4.3.

26.8.1.3.5.2.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", having waited for a reasonable length of time (expiry of timer T313) without receiving the appropriate protocol message to complete the incoming call, shall initiate the clearing of that incoming call by sending the CC message DISCONNECT and enter the CC-state U11, "Disconnect Request"

26.8.1.3.5.2.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	the MS is supporting immediate connect

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/2.

Foreseen final state of the MS:

U11, disconnect request.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U10. The T313 expiries at the MS and the MS sends a DISCONNECT message and enters state U11, disconnect request. The SS checks by using the status enquiry procedure that the MS has entered the correct state.

Expected sequence:

Step	Direction	Message	Comments

A1	SS -> MS	ASSIGNMENT COMMAND	p = Y

A2	MS -> SS	ASSIGNMENT COMPLETE

3	SS		SS waits until timer T313

expiry at the MS

4	MS -> SS	DISCONNECT

5	SS -> MS	STATUS ENQUIRY

6	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

26.8.1.3.5.2.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.5.3	Incoming call / U8 connect request / termination requested by the user

26.8.1.3.5.3.1	Definition and applicability

The call control entity of the MS being in the state, U10, the user requests for releasing of the call. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.3.5.3.2	Conformance requirement

1)	A CC entity of a MS in CC-state U8, "Connect Request", upon request by the user to terminate shall send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

Reference(s):

TS GSM 04.07 section 3.2.2., TS GSM 04.08 section 5.4.3., TS GSM 11.10, Annex 3 - section 2.2.

26.8.1.3.5.3.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", upon request by the user to terminate will send a DISCONNECT message and enter the CC-state U11, "Disconnect Request".

26.8.1.3.5.3.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	the MS is supporting immediate connect

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/2.

Foreseen final state of the MS:

U11, disconnect request.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U10. Then the user requests termination of the call. The MS sends a DISCONNECT message and enters state U11, disconnect request. The SS verifies by using the status enquiry procedure that the MS has entered the correct state.

Expected sequence:

Step	Direction	Message	Comments

A1	SS -> MS	ASSIGNMENT COMMAND	p = Y

A2	MS -> SS	ASSIGNMENT COMPLETE

3			the user requests to clear the call

4	MS -> SS	DISCONNECT

5	SS -> MS	STATUS ENQUIRY

6	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

26.8.1.3.5.3.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.5.4	Incoming call / U8 connect request / DISCONNECT received with in-band information

26.8.1.3.5.4.1	Definition and applicability

The call control entity of the MS being in the state, U8, a DISCONNECT message indicating availability of in band information is received by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.3.5.4.2	Conformance requirement

1)	A CC entity of a MS in CC-state U8, "Connect Request", upon receipt of a DISCONNECT with progress indicator #8, shall through-connect the speech channel to make in-band announcements available and enter CC-state U12, if traffic channel is in speech mode. If TCH is not in speech mode, the MS shall send a RELEASE message and enter CC-state U19.

Reference(s):

TS GSM 04.08 section 3.4.3.1., TS GSM 04.08 section 5.1.3. - note, TS GSM 04.08 section 5.2.2.7., TS GSM 04.08 section 5.4.4., TS GSM 04.08 section 5.5.1.

26.8.1.3.5.4.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", upon receipt of a DISCONNECT with progress indicator #8, through-connects the speech channel to make in-band announcements available and enters CC-state U12, if traffic channel is in speech mode. If TCH is not in speech mode, the MS senda a RELEASE message and enters CC-state U19.

26.8.1.3.5.4.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	the MS is supporting immediate connect (p = Y/N)

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station:		The MS is in MM-state "idle, updated" with valid TMSI and CKSN. The MS is brought into the state U8 by using table 26.8.1.3/3 with the following amendment when the TCH is allocated: if the MS has a bearer capability for speech, TCH for speech is allocated. Otherwise any other mode appropriate for the bearer capabilities of the MS is selected for TCH.

Foreseen final state of the MS:

U12, disconnect indication or U19 depending on the bearer capabilities.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U8. The SS sends a DISCONNECT message containing indication of in-band information availability to the MS. The SS checks that if channel mode is speech, the TCH shall be through connected and the MS enters state U12, disconnect indication. If channel mode is not speech, the TCH shall not be through connected and the MS enters state U19, release request.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	1)

TCH in speech mode:

A2	SS		the SS will check that the audio path

for in band tones is attached.

A3	SS -> MS	STATUS ENQUIRY

A4	MS -> SS	STATUS	cause 30#, state U12

TCH is not in speech mode:

B2	MS -> SS	RELEASE

B3	SS -> MS	STATUS ENQUIRY

B4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

Note 1:	With a progress indicator indicating in-band information; the cause value #8.

26.8.1.3.5.4.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.5.5	Incoming call / U8 connect request / DISCONNECT received without in-band information

26.8.1.3.5.5.1	Definition and applicability

The call control entity of the MS being in the state, U8, a DISCONNECT message is received by the MS. The DISCONNECT message does not contain indication of in-band information availability. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.3.5.5.2	Conformance requirement

1)	A CC entity of a MS in CC-state U8, "Connect Request", upon receipt of a DISCONNECT without progress indicator, shall return a RELEASE message and enter the CC-state U19, "Release Request".

Reference(s):

TS GSM 04.08 section 5.4.4., TS GSM 04.08 section 5.4.4.2.

26.8.1.3.5.5.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", upon receipt of a DISCONNECT without progress indicator, returns a RELEASE message and enters the CC-state U19, "Release Request".

26.8.1.3.5.5.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	the MS is supporting immediate connect

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/3.

Foreseen final state of the MS:

U19, release request.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then, the MS is made to initiate a call. The CC entity of the MS is brought to the state U8. The SS sends a DISCONNECT message not containing indication of in-band information availability to the MS. The MS shall respond with a RELEASE message. The SS checks by using the status enquiry procedure that the CC entity of the MS has entered the state U19, release request.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	DISCONNECT	1)

2	MS -> SS	RELEASE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U19

Specific message contents:

Note 1:	Without a progress indicator indicating in-band information

26.8.1.3.5.5.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.5.6	Incoming call / U8 connect request / RELEASE received

26.8.1.3.5.6.1	Definition and applicability

The call control entity of the MS being in the state, U8, a RELEASE message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability, except for those supporting immediate connect.

26.8.1.3.5.6.2	Conformance requirement

1)	A CC entity of a MS in CC-state U8, "Connect Request", upon receipt of a RELEASE shall return a RELEASE COMPLETEPLETE and enter the CC-state U0, "Null".

2)	On returning to the idle mode the MS shall release the MM-connection and the CC-entities relating to the seven mobile terminating transaction identifiers shall be in CC-state U0, "Null".

Reference(s):

Conformance requirement 1:	TS GSM 04.08 section 5.4.4.

Conformance requirement 2:	TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2.

26.8.1.3.5.6.3	Test purpose

1)	To verify that a CC entity of a MS in CC-state U8, "Connect Request", upon receipt of a RELEASE will return a RELEASE COMPLETEPLETE and enter the CC-state U0, "Null".

2)	To verify that the MS on returning to the idle mode releases the MM-connection and that the CC-entities relating to the seven mobile terminating transaction identifiers are in CC-state U0, "Null".

26.8.1.3.5.6.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	the MS is supporting immediate connect

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/3.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

1 min.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U8. The SS sends a RELEASE message. The MS responds with a RELEASE COMPLETE message. The SS checks by using the status enquiry procedure that the CC entity has entered state U0, null, with the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	RELEASE

2	MS -> SS	RELEASE COMPLETE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

5	SS		repeat steps 3-4 to cover all the

transaction identifiers from 000...110

6	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA).

Specific message contents:

None.

26.8.1.3.5.6.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.5.7	Incoming call / U8 connect request / lower layer failure

26.8.1.3.5.7.1	Definition and applicability

The call control entity of the MS being in the state, U8, a lower layer failure is accomplished at the MS and consequently, communication at layer 3 level with the peer entity is terminated. This test is applicable for any equipment supporting at least one bearer capability, except for those supporting immediate connect.

26.8.1.3.5.7.2	Conformance requirement

1)	A CC entity of a MS in CC-state U8, "Connect Request", having detected a lower layer failure shall return to idle mode with the CC entities relating to the seven mobile terminating transaction identifiers in CC-state U0, "Null".

Reference(s):

TS GSM 04.08 section 4.5.2.3., TS GSM 04.08 section 4.5.3., TS GSM 04.08 section 5.5.3.2.

26.8.1.3.5.7.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", having detected a lower layer failure returns to idle mode with the CC entities relating to the seven mobile terminating transaction identifiers in CC-state U0, "Null".

26.8.1.3.5.7.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	the MS is supporting immediate connect

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/1.

Foreseen final state of the MS:

U0, null.

Maximum duration of test:

1 min 30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The MS is brought to the state U8. The SS generates a lower layer failure at the MS. The SS waits long enough to enable the MS to return to idle state listening to paging, and then pages MS to create RR-connection. Finally, the SS will check the state of the MS by using STATUS ENQUIRY with the relevant transaction identifiers.

Expected sequence:

Step	Direction	Message	Comments

1	SS		SS generates lower layer failure

2	SS		SS waits 20 s for the MS to return to

listening to paging

3	SS -> MS	PAGING REQUEST

4	MS -> SS	CHANNEL REQUEST

5	SS -> MS	IMMEDIATE ASSIGNMENT

6	MS -> SS	PAGING RESPONSE

7	SS -> MS	STATUS ENQUIRY

8	MS -> SS	RELEASE COMPLETE	cause 81# (invalid TI value)

9	SS		repeat steps 7-8 to cover all the

transaction identifiers from 000...110

10	SS -> MS	CHANNEL RELEASE	the main signalling link shall be

released by the MS (L2: DISC/UA).

Specific message contents:

None.

26.8.1.3.5.7.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.5.8	Incoming call / U8 connect request / TCH assignment

26.8.1.3.5.8.1	Definition and applicability

The call control entity of the MS being in the state, U8, an assignment procedure is performed for traffic channel. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.3.5.8.2	Conformance requirement

1)	A CC entity of a MS in CC-state U8, "Connect Request", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH without changing the state of the call in progress.

Reference(s):

TS GSM 04.08 section 3.4.3., TS GSM 04.08 section 5.2.2.7.

26.8.1.3.5.8.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", when allocated a traffic channel by the network performing the assignment procedure, shall perform a layer 2 establishment on the FACCH without changing the state of the call in progress.

26.8.1.3.5.8.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	the MS is supporting immediate connect

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/1.

Foreseen final state of the MS:

U8, connect request.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U8. The SS sends an ASSIGNMENT COMMAND for traffic channel to the MS. The MS shall establish layer 2 link on the newly allocated channel and respond with an ASSIGNMENT COMPLETE message. The SS verifies by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	ASSIGNMENT COMMAND	TCH

2	MS -> SS	ASSIGNMENT COMPLETE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U8

Specific message contents:

None.

26.8.1.3.5.8.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.3.5.9	Incoming call / U8 connect request / unknown message received

26.8.1.3.5.9.1	Definition and applicability

The call control entity of the MS being in the state, U8, an unknown message is received by the MS. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.3.5.9.2	Conformance requirement

1)	A CC entity of a MS in CC-state U8, "Connect Request", having received an unknown message from its peer entity shall return a STATUS message.

Reference(s):

TS GSM 04.08 section 8.4.

26.8.1.3.5.9.3	Test purpose

To verify that a CC entity of a MS in CC-state U8, "Connect Request", having received an unknown message from its peer entity returns a STATUS message.

26.8.1.3.5.9.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	the MS is supporting immediate connect

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

The MS is brought into the state U8 by using table 26.8.1.3/1.

Foreseen final state of the MS:

U8, connect request.

Maximum duration of test:

30 s.

Procedure:

A teleservice is selected that is supported by the MS; if the MS supports speech, the selected teleservice is speech. If necessary, the MS is configured for that teleservice. Then a mobile terminated call is initiated. The CC entity of the MS is brought to the state U8. The SS sends a message with message type not defined for the protocol discriminator to the MS. The MS shall respond with a STATUS message, and finally the SS checks by using the status enquiry procedure that the state of the CC entity has remained unchanged.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	unknown message	message type not defined for PD

2	MS -> SS	STATUS	cause 97#, state U8

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U8

Specific message contents:

None.

26.8.1.3.5.9.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.4	In call fuctions

26.8.1.4.1	In-call functions / DTMF information transfer

26.8.1.4.1.1	In-call functions / DTMF information transfer / basic procedures

26.8.1.4.1.1.1	Definition and applicability

Dual Tone Multi Frequency (DTMF) is an inband one out of four plus one out of four signalling system primarily used from terminal instruments in telecommunication networks.

The support of DTMF is only permitted when a bearer capability for speech is in use or during the speech phase of alternate speech/data and alternate speech/facsimile teleservices.

26.8.1.4.1.1.2	Conformance requirement

1)	An MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active": when made to send a DTMF tone, shall send a START DTMF message on the correct DCCH.

Reference(s):

TS GSM 04.08, 5.3.3

2)	An MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active": when made to send a DTMF tone (the corresponding IA5 character being selected from among the ones supported), shall send a START DTMF message specifying the correct IA5 character in the "keypad information" field of the keypad facility information element.

2.1	TS GSM 04.08, 5.3.3

26.8.1.4.1.1.3	Test purpose

1)	To verify that an MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active": when made to send a DTMF tone, sends a START DTMF message on the correct DCCH.

2)	To verify that an MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active": when made to send a DTMF tone (the corresponding IA5 character being selected from among the ones supported), sends a START DTMF message specifying the correct IA5 character in the "keypad information" field of the keypad facility information element.

26.8.1.4.1.1.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	supported character set (e.g. 0-9, #, *, A, B, C, D)

-	if and how DTMF tone is indicated to the user

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in CC-state "active".

Foreseen final state of the MS:

CC-state "active".

Maximum duration of test:

1 min.

Procedure:

The MS being in the call active state, a user causes a DTMF tone to be generated e.g. by depression of a key in the MS. A DTMF digit corresponding to the digit indicated by the user is sent in a START DTMF message by the MS. The SS will return a START DTMF ACK message to the MS. This acknowledgement may be used in the MS to generate an indication as a feedback for a successful transmission. Then the user indicates that the DTMF sending should cease e.g. by releasing the key. The MS will send a STOP DTMF message to the network which is acknowledged with STOP DTMF ACKNOWLEDGE by the SS.

The sequence described above is repeated for each of the applicable characters 0-9, #, *, A, B, C, and D.

Then a case of rejecting a DTMF tone is tested and the state of the MS is verified.

Expected sequence:

Step	Direction	Message	Comments

1	MS -> SS	START DTMF	the user causes DTMF tone to be generated

SS		the SS will verify that the transmitted

information corresponds to the digit pressed

2	SS -> MS	START DTMF ACK	possible indication of a DTMF tone depending

the PICS/PIXIT statements

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U10

5	MS -> SS	STOP DTMF	the user causes DTMF tone to be ceased at a

point in time greater than or equal to [Ts]

after receiving START DTMF ACK

6	SS -> MS	STOP DTMF ACKNOWLEDGE	the DTMF tone indication shall be stopped

7			the steps 1-4 shall be repeated for each of the

applicable characters 0-9, #, *, A, B, C, D.

The time between characters (from STOP

DTMF ACK message to the next START DTMF

message) shall be greater than [To].

8	SS -> MS	STATUS ENQUIRY

9	MS -> SS	STATUS	cause 30#, state U10

10	MS -> SS	START DTMF

11	SS -> MS	START DTMF REJECT

12	SS -> MS	STATUS ENQUIRY

13	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

26.8.1.4.1.1.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.4.1.2	In-call functions / DTMF information transfer / timing requirements

26.8.1.4.1.2.1	Definition and applicability

There are certain timing requirements regarding the minimum length of a tone and the interval between two successive DTMF tones as seen from the network point of view regardless in what way the user indicates activation and ceasing of tones. Those timing requirements are tested in this test.

The support of DTMF is only permitted when a bearer capability for speech is in use or during the speech phase of alternate speech/data and alternate speech/facsimile teleservices.

26.8.1.4.1.2.2	Conformance requirement

1a)	An MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active": when made to transmit characters so that the time between characters is less than the duration of timer value "To", shall keep the time between consecutive STOP DTMF message and START DTMF message be greater than the duration of timer value "To".

1b)	An MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active": when made to transmit different arbitary characters, so that the time between characters is less than the duration of timer value "To", shall transmit the correct characters in sequence.

2)	An MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active" and made to trigger stopping of tranmission of the DTMF tone at a point in time less than "Ts" after START DTMF, shall send a STOP DTMF message at least "Ts" after the corresponding START DTMF ACK message.

Reference(s):

Conformance requirement 1a:	TS GSM 04.08, 5.3.3

Conformance requirement 1b:	TS GSM 04.08, 5.3.3

Conformance requirement 2:	TS GSM 04.08, 5.3.3

26.8.1.4.1.2.3	Test purpose

1a)	To verify that an MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active": when the procedure of test purpose 1 is performed twice so that the time between characters is less than the duration of timer value "To", the time between consecutive STOP DTMF message and START DTMF message is still greater than the duration of timer value "To".

1b)	To verify that an MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active": when the procedure of test purpose 1 is performed twice, with different arbitrary characters, so that the time between characters is less than the duration of timer value "To", the correct characters are transmitted in sequence.

2)	To verify that an MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active": is made to trigger stopping of tranmission of the DTMF tone at a point in time less than "Ts" after START DTMF, it sends a STOP DTMF message at least "Ts" after the corresponding START DTMF ACK message.

26.8.1.4.1.2.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	supported character set (e.g. 0-9, #, *, A, B, C, D)

-	if and how DTMF tone is indicated to the user

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in CC-state "active".

Foreseen final state of the MS:

CC-state "active".

Maximum duration of test:

1 min.

Procedure:

The MS being in the call active state, a user causes two successive DTMF tones to be generated in such a way that the time between characters is less than the duration of timer value "To", the time between consecutive STOP DTMF message and START DTMF message.

Then the test is repeated in such a way that the arbitary chosen characters are different from each others.

Then a user causes a "short" character by triggering stopping of transmission of the DTMF tone at a point in time less than "Ts" after START DTMF, "Ts" being the minimum time between START DTMF and STOP DTMF messages.

Then two procedures are repeated after one another in such a way that two different, arbitrary, characters are transmitted. However the "sending time" (from START DTMF message to the STOP DTMF message) shall be less than [Ts] and the time between characters (from the STOP DTMF ACK message to the next START DTMF message) shall be less than [To].

Expected sequence:

Step	Direction	Message	Comments

1	MS -> SS	START DTMF	the user causes a DTMF tone to be generated

SS		the SS will verify that the transmitted

information corresponds to the digit pressed

2	SS -> MS	START DTMF ACK	possible indication of a DTMF tone depending

the PICS/PIXIT statements

3	MS -> SS	STOP DTMF	the user causes the DTMF tone to be ceased

4	SS -> MS	STOP DTMF ACKNOWLEDGE	the DTMF tone indication shall be stopped

5	MS -> SS	START DTMF	the user initiates another DTFM tone

by pressing the same character

"immediately" after ceasing the first one

6	SS		the SS will verify: To > To min

7	SS -> MS	START DTMF ACK	possible indication of a DTMF tone depending

the PICS/PIXIT statements

8	MS -> SS	STOP DTMF	the user causes DTMF tone to be ceased

9	SS -> MS	STOP DTMF ACKNOWLEDGE	the DTMF tone indication shall be stopped

10	SS -> MS	STATUS ENQUIRY

11	MS -> SS	STATUS	cause 30#, state U10

12	MS -> SS	START DTMF	the user causes DTMF tone to be generated

SS		the SS will verify that the transmitted

information corresponds to the digit pressed

13	SS -> MS	START DTMF ACK	possible indication of a DTMF tone depending

the PICS/PIXIT statements

14	MS -> SS	STOP DTMF	the user causes DTMF tone to be ceased

15	SS -> MS	STOP DTMF ACKNOWLEDGE	the DTMF tone indication shall be stopped

16	MS -> SS	START DTMF	the user initiates another DTFM tone

by pressing a different character

"immediately" after ceasing the first one

17	SS		the SS will verify: To > To min

18	SS -> MS	START DTMF ACK	possible indication of a DTMF tone depending

the PICS/PIXIT statements

19	MS -> SS	STOP DTMF	the user causes DTMF tone to be ceased

20	SS -> MS	STOP DTMF ACKNOWLEDGE	the DTMF tone indication shall be stopped

21	SS -> MS	STATUS ENQUIRY

22	MS -> SS	STATUS	cause 30#, state U10

23	MS -> SS	START DTMF	the user causes DTMF tone to be generated

SS		the SS will verify that the transmitted

information corresponds to the digit pressed

24	SS -> MS	START DTMF ACK	possible indication of a DTMF tone depending

the PICS/PIXIT statements

25	MS -> SS	STOP DTMF	the user causes DTMF tone to be ceased

"immediately" after having initiated

the generation of a DTMF tone

26	SS		the SS will verify: Ts > Ts min

27	SS -> MS	STOP DTMF ACKNOWLEDGE	the DTMF tone indication shall be stopped

28	SS -> MS	STATUS ENQUIRY

29	MS -> SS	STATUS	cause 30#, state U10

30	MS -> SS	START DTMF	the user causes DTMF tone to be generated

SS		the SS will verify that the transmitted

information corresponds to the digit pressed

31	SS -> MS	START DTMF ACK	possible indication of a DTMF tone depending

the PICS/PIXIT statements

32	MS -> SS	STOP DTMF	the user causes DTMF tone to be ceased

"immediately" after having initiated

the generation of a DTMF tone

33	SS		the SS will verify: Ts > Ts min

34	SS -> MS	STOP DTMF ACKNOWLEDGE	the DTMF tone indication shall be stopped

35	MS -> SS	START DTMF	the user initiates another DTFM tone

by pressing a different character

"immediately" after ceasing the first one

36	SS		the SS will verify: To > To min

37	SS -> MS	START DTMF ACK	possible indication of a DTMF tone depending

the PICS/PIXIT statements

38	MS -> SS	STOP DTMF	the user causes DTMF tone to be ceased

"immediately" after having initiated

the generation of the second DTMF tone

39	SS		the SS will verify: Ts > Ts min

40	SS -> MS	STOP DTMF ACKNOWLEDGE	the DTMF tone indication shall be stopped

41	SS -> MS	STATUS ENQUIRY

42	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

26.8.1.4.1.2.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.4.1.3	In-call functions / DTMF information transfer / downlink DTMF

26.8.1.4.1.3.1	Definition and applicability

Dual Tone Multi Frequency (DTMF) is an inband one out of four plus one out of four signalling system primarily used from terminal instruments in telecommunication networks.

The support of DTMF is only permitted when the speech teleservice is being used or during the speech phase of alternate speech/data and alternate speech/facsimile teleservices.

This test is a case for downlink DTMF to ensure future compatibility if/when downlink DTMF is introduced.

26.8.1.4.1.3.2	Conformance requirement

1)	An MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active": when receiving a START DTMF message, shall not send a START DTMF ACK message and shall stay in state U10, "Active".

2)	An MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active": when receiving a STOP DTMF message, shall not send a STOP DTMF ACK message and shall stay in state U10, "Active".

Reference(s):

Conformance requirement 1:	TS GSM 04.08, 5.3.3

Conformance requirement 2:	TS GSM 04.08, 5.3.3

26.8.1.4.1.3.3	Test purpose

1)	To verify that an MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active": when receiving a START DTMF message, does not send a START DTMF ACK message and stays in state U10, "Active".

1.1	TS GSM 04.08, 5.3.3

2)	To verify that an MS supporting the Mobile originating DTMF protocol control procedure, having a CC entity for speech in state U10, "Active": when receiving a STOP DTMF message, does not send a STOP DTMF ACK message and stays in state U10, "Active".

2.1	TS GSM 04.08, 5.3.3

26.8.1.4.1.3.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in CC-state "active".

Foreseen final state of the MS:

CC-state "active".

Maximum duration of test:

10 s.

Procedure:

The MS being in the call active state, the SS sends START DTMF message to the MS. The SS checks that the MS does not respond and does not change its state. Then the SS sends STOP DTMF message to the MS. The SS checks that the MS does not respond and does not change its state.

Expected sequence:

Step	Direction	Message	Comments

1	MS -> SS	START DTMF	the user causes DTMF tone to be generated

SS		the SS will verify that the transmitted

information corresponds to the digit pressed

2	SS -> MS	START DTMF ACK	possible indication of a DTMF tone depending

the PICS/PIXIT statements

3	MS -> SS	STOP DTMF	the user causes DTMF tone to be ceased

4	SS -> MS	STOP DTMF ACKNOWLEDGE	the DTMF tone indication shall be stopped

5			the steps 1-4 shall be repeated for each of the

applicable characters 0-9, #, *, A, B, C, D.

6	MS -> SS	START DTMF

7	SS -> MS	START DTMF REJECT

8	SS -> MS	STATUS ENQUIRY

9	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

26.8.1.4.1.3.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.4.2	In-call functions / User notification

User notification procedure allows the network to notify a MS of any call-related event during the "active" state of a call. It also may allow a MS to notify the remote user of any appropriate call-related event during the "active" state of a call by sending a NOTIFY message containing a notification indicator to the network. No state change occurs at any of the interface sides during this procedure.

26.8.1.4.2.1	In-call functions / User notification / MS terminated

26.8.1.4.2.1.1	Definition and applicability

This is a case for testing user notification procedure terminated by the mobile station. The test is applicable for those equipments supporting at least one bearer capability for speech declaring also having user notification feature implemented.

26.8.1.4.2.1.2	Conformance requirement

1)	A CC entity of a MS in CC-state U10, "active", upon receiving of a NOTIFY message shall remain in the active state.

Reference(s):

TS GSM 04.08, 5.3.1.

26.8.1.4.2.1.3	Test purpose

To verify that a CC entity of a MS in CC-state U10, "active", upon receiving of a NOTIFY message remains in the active state.

26.8.1.4.2.1.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	support of user notification procedure

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in CC-state "active".

Foreseen final state of the MS:

CC-state "active".

Maximum duration of test:

10 s.

Procedure:

The MS being in the call active state, the SS will send a NOTIFY message to the MS. The state of the MS is checked after that.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	NOTIFY

2	SS -> MS	STATUS ENQUIRY

3	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

26.8.1.4.2.1.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.4.3	In-call functions / Channel changes

The two following test cases are for testing some elementary radio resource level procedures during an active state of a call to ensure call maintenance also during physical channel changes.

26.8.1.4.3.1	In-call functions / Channel changes / A successful channel change in active state

26.8.1.4.3.1.1	Definition and applicability

This is a case to test a change of a physical channel during active state of a call. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.4.3.1.2	Conformance requirement

1)	The MS being in the call active state after having successful completed a channel assignment or channel mode modify shall remain in the call active state.

Reference(s):

TS GSM 04.08, section 5.3.4.3.2, TS GSM 04.08, section 3.4.6.

26.8.1.4.3.1.3	Test purpose

To verify that the MS being in the call active state after having successful completed a channel assignment or channel mode modify remains in the call active state.

26.8.1.4.3.1.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in CC-state "active".

Foreseen final state of the MS:

CC-state "active".

Maximum duration of test:

10 s.

Procedure:

The MS being in the call active state, the SS initiated channel assignment procedure causing an intracell change of channel by sending ASSIGNMENT COMMAND message to the MS. The MS performs channel assignment procedure and after the main signalling link is successfully established, the MS returns an ASSIGNMENT COMPLETE message. The state of the MS is then checked.

Then the SS initiates channel mode change procedure by sending a CHANNEL MODE MODIFY message to the MS containing the reference of the channel and the new mode to use. The MS changes the mode for the indicated channel and replies by a CHANNEL MODE MOD ACKNOWLEDGE message indicating the new channel mode. The state of the MS is then checked.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	ASSIGNMENT COMMAND

2	MS -> SS	ASSIGNMENT COMPLETE

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U10

5	SS -> MS	CHANNEL MODE MODIFY

6	MS -> SS	CHANNEL MODE MOD ACK

7	SS -> MS	STATUS ENQUIRY

8	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

26.8.1.4.3.1.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.4.3.2	In-call functions / Channel changes / An unsuccessful channel change in active mode

26.8.1.4.3.2.1	Definition and applicability

This is a case to test an unsuccessful change of a physical channel during active state of a call. This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.4.3.2.2	Conformance requirement

1)	The MS, when returning to the old channel after handover failure and having established the link, shall remain in the call active state.

Reference(s):

TS GSM 04.08, section 5.3.4.3.

26.8.1.4.3.2.3	Test purpose

To verify that the MS, when returning to the old channel after handover failure and establishing correctly the link, will remain in the call active state.

26.8.1.4.3.2.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in CC-state "active".

Foreseen final state of the MS:

CC-state "active".

Maximum duration of test:

10 s.

Procedure:

The MS being in the call active state, the SS initiates handover procedure. When the MS tries to establish main signalling link, it is prohibited by the SS. Then the MS shall return back to the old channel and re-establish correctly the link. The state of the MS is then checked.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND

2	MS -> SS	HANDOVER ACCESS	the SS does not respond

3	MS -> SS	HANDOVER FAILURE	after the MS has re-established the main

signalling link in the old channel

4	SS -> MS	STATUS ENQUIRY

5	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

26.8.1.4.3.2.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.4.4	In-call functions / MS terminated in-call modification

26.8.1.4.4.1	In-call functions / MS terminated in-call modification / Modify when new mode is not supported

26.8.1.4.4.1.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which the new mode is not supported (and consequently nor one of those negotiated and agreed during the establishment phase of the call). This test is applicable for any equipment supporting at least one bearer capability.

26.8.1.4.4.1.2	Conformance requirement

1)	The MS after having received a MODIFY message with a new mode which is not the actual one and cannot be supported by the MS shall reject it by sending a MODIFY REJECT message or a STATUS message.

Reference(s):

TS GSM 04.08, sections 5.3.4.3.4.2 and 5.3.4.4.

26.8.1.4.4.1.3	Test purpose

To verify that the MS after having received a MODIFY message with a new mode which is not the actual one and cannot be supported by the MS rejects it by sending a MODIFY REJECT message or a STATUS message.

26.8.1.4.4.1.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in CC-state "active".

Foreseen final state of the MS:

CC-state "active".

Maximum duration of test:

10 s.

Procedure:

The MS being in the call active state, the SS initiatiates in-call modification procedure by sending a MODIFY message with new mode different from actual mode and one of those not supported by the MS. The MS either returns a MODIFY REJECT message with the old bearer capability or a STATUS message with reject cause #97. The state of the MS is then checked.

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	MODIFY	with new mode different from actual one

2a	MS -> SS	MODIFY REJECT	with the old call mode included OR

2b	MS -> SS	STATUS	cause #97, state U10

3	SS -> MS	STATUS ENQUIRY

4	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

26.8.1.4.4.1.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.4.5	In-call functions / MS originated in-call modification

26.8.1.4.5.1	In-call functions / MS originated in-call modification / A successful case of modifying

26.8.1.4.5.1.1	Definition and applicability

This test is to test a successful case of in-call modification, which is triggered by the calling tone identification (CNG) received by the MS. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.1.2	Conformance requirement

1)	The procedure shall be initiated by the MS in the "active" state of the call. It shall send a MODIFY message including the new mode to be changed to; and enter the "mobile originating modify" state. The new mode given in the MODIFY message shall be one of those already negotiated and agreed during the establishment phase of the call. The MS shall stop sending Bm-channel information according to the old mode and enter the state U26 "Mobile Originating Modify".

2)	Upon receipt of the MODIFY COMPLETE message the MS shall start sending channel information according to the new call mode and enter the "active" state.

Reference(s):

Conformance requirement 1:	TS GSM 04.08, section 5.3.4.3.1.

Conformance requirement 2:	TS GSM 04.08, section 5.3.4.3.2.

26.8.1.4.5.1.3	Test purpose

1)	To verify that the procedure is initiated by the MS in the "active" state of the call. It sends a MODIFY message including the new mode to be changed to; and enters the "mobile originating modify" state. The new mode given in the MODIFY message is one of those already negotiated and agreed during the establishment phase of the call. The MODIFY originating side stops sending Bm-channel information.

2)	To verify that upon receipt of the MODIFY COMPLETE message the MS starts sending channel information according to the new call mode and enters the "active" state.

26.8.1.4.5.1.4	Method of test

Related PICS/PIXIT statement(s):

-	a way to activate a dual mode call

-	a way to activate in-call modification

-	support of dual bearer capability services

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS:

CC-state "active".

Maximum duration of test:

10 s.

Procedure:

The MS being in the call active state, in-call modification procedure is initiated from the MS side. The MS shall send a MODIFY message with new mode to the SS. The SS returns a MODIFY COMPLETE message. The state of the MS is then checked.

Expected sequence:

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call

2	SS -> MS	IMMEDIATE ASSIGNMENT	SDCCH

3	MS -> SS	CM-SERVICE REQUEST

4	SS -> MS	CIPH MODE COMMAND

5	MS -> SS	CIPH MODE COMPLETE

6	MS -> SS	SETUP	containing bearer capabilities for dual mode

call

7	SS -> MS	AUTH REQUEST

8	MS -> SS	AUTH RESPONSE

9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode

call

10	SS -> MS	ASSIGNMENT COMMAND	TCH

11	MS -> SS	ASSIGNMENT COMPLETE

12	SS -> SS	ALERTING

13	SS -> MS	CONNECT

14	MS -> SS	CONNECT ACKNOWLEDGE

15	SS		the SS generates calling tone indication (CNG)

16	MS -> SS	MODIFY	MMI action to change the mode or

by automatically detecting the CNG, verify that

the bearer capability is one of those already

given in the initial SETUP message

17	SS -> MS	STATUS ENQUIRY

18	MS -> SS	STATUS	cause 30#, state U26

19	SS -> MS	MODIFY COMPLETE	contains the new mode as bearer capability

20	SS		allow at least 2 seconds for the MS to adapt

for the new mode

21	SS -> MS	STATUS ENQUIRY

22	MS -> SS	STATUS	cause 30#, state U10

23	SS		verify that the MS stops sending Bm channel

information according to the old mode

Specific message contents:

None.

26.8.1.4.5.1.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.4.5.2	In-call functions / MS originated in-call modification / Modify rejected

26.8.1.4.5.2.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which the in-call modification is rejected. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.2.2	Conformance requirement

1)	Upon receipt of the MODIFY REJECT message with the old bearer capability the MS shall: resume sending Bm-channel information according to the present call mode; resume interpreting received Bm-channel information according to the present call mode; and enter the "active" state.

Reference(s):

TS GSM 04.08, section 5.3.4.3.4.1.

26.8.1.4.5.2.3	Test purpose

To verify that upon receipt of the MODIFY REJECT message with the old bearer capability the MS resumes sending Bm-channel information according to the present call mode; resumes interpreting received Bm-channel information according to the present call mode; and enters the "active" state.

26.8.1.4.5.2.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	support of dual bearer capability services

-	support of dual bearer capability services

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS:

CC-state "active".

Maximum duration of test:

10 s.

Procedure:

The MS being in the call active state, in-call modification procedure is initiated from the MS side. The MS shall send a MODIFY message with new mode to the SS. The SS returns a MODIFY REJECT message. The state of the MS is then checked.

Expected sequence:

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call

2	SS -> MS	IMMEDIATE ASSIGNMENT 	SDCCH

3	MS -> SS	CM-SERVICE REQUEST

4	SS -> MS	CIPH MODE COMMAND

5	MS -> SS	CIPH MODE COMPLETE

6	MS -> SS	SETUP	containing bearer capabilities for dual mode

call

7	SS -> MS	AUTH REQUEST

8	MS -> SS	AUTH RESPONSE

9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode

call

10	SS -> MS	ASSIGNMENT COMMAND	TCH

11	MS -> SS	ASSIGNMENT COMPLETE

12	SS -> SS	ALERTING

13	SS -> MS	CONNECT

14	MS -> SS	CONNECT ACKNOWLEDGE

15	MS -> SS	MODIFY	MMI action to change the mode

16	SS -> MS	MODIFY REJECT	with cause #58 bearer capability not

available and with old bearer capabilities

17	SS -> MS	STATUS ENQUIRY

18	MS -> SS	STATUS	cause 30#, state U10

Specific message contents:

None.

26.8.1.4.5.2.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.4.5.3	In-call functions / MS originated in-call modification / An abnormal case of acceptance

26.8.1.4.5.3.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which the in-call modification is accepted incorrectly. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.3.2	Conformance requirement

1)	Upon receipt of the MODIFY COMPLETE message indicating a call mode which does not correspond to the requested one the MS shall discard it and take no action.

Reference(s):

TS GSM 04.08, section 5.3.4.4.

26.8.1.4.5.3.3	Test purpose

To verify that upon receipt of the MODIFY COMPLETE message indicating a call mode which does not correspond to the requested one the MS discards it and takes no action.

26.8.1.4.5.3.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	support of dual bearer capability services

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS:

CC-state U26 "Mobile Originating Modify".

Maximum duration of test:

10 s.

Procedure:

The MS being in the call active state, in-call modification procedure is initiated from the MS side. The MS shall send a MODIFY message with new mode to the SS. The SS returns a MODIFY COMPLETE message specifying a mode that does not correspond to the requested one. It will be verified then that the MS shall not take any action and the state of the MS will be checked.

Expected sequence:

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call

2	SS -> MS	IMMEDIATE ASSIGNMENT 	SDCCH

3	MS -> SS	CM-SERVICE REQUEST

4	SS -> MS	CIPH MODE COMMAND

5	MS -> SS	CIPH MODE COMPLETE

6	MS -> SS	SETUP	containing bearer capabilities for dual mode

call

7	SS -> MS	AUTH REQUEST

8	MS -> SS	AUTH RESPONSE

9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode

call

10	SS -> MS	ASSIGNMENT COMMAND	TCH

11	MS -> SS	ASSIGNMENT COMPLETE

12	SS -> SS	ALERTING

13	SS -> MS	CONNECT

14	MS -> SS	CONNECT ACKNOWLEDGE

15	MS -> SS	MODIFY	MMI action to change the mode

16	SS -> MS	MODIFY COMPLETE	with a mode that does not correspond

to the requested one

17	SS -> MS	STATUS ENQUIRY

18	MS -> SS	STATUS	cause 30#, state U26

Specific message contents:

None.

26.8.1.4.5.3.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.4.5.4	In-call functions / MS originated in-call modification / An abnormal case of rejection

26.8.1.4.5.4.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which the in-call modification is rejected incorrectly. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.4.2	Conformance requirement

1)	Upon receipt of the MODIFY REJECT message indicating a call mode which does not correspond to the actual one the MS shall discard it and take no action.

Reference(s):

TS GSM 04.08, section 5.3.4.4.

26.8.1.4.5.4.3	Test purpose

To verify that upon receipt of the MODIFY REJECT message indicating a call mode which does not correspond to the actual one the MS discards it and takes no action.

26.8.1.4.5.4.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	support of dual bearer capability services

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS:

CC-state U26 "Mobile Originating Modify".

Maximum duration of test:

10 s.

Procedure:

The MS being in the call active state, in-call modification procedure is initiated from the MS side. The MS shall send a MODIFY message with new mode to the SS. The SS returns a MODIFY REJECT message specifying a mode that does not correspond to the actual one. The state of the MS is then checked.

Expected sequence:

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call

2	SS -> MS	IMMEDIATE ASSIGNMENT 	SDCCH

3	MS -> SS	CM-SERVICE REQUEST

4	SS -> MS	CIPH MODE COMMAND

5	MS -> SS	CIPH MODE COMPLETE

6	MS -> SS	SETUP	containing bearer capabilities for dual mode

call

7	SS -> MS	AUTH REQUEST

8	MS -> SS	AUTH RESPONSE

9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode

call

10	SS -> MS	ASSIGNMENT COMMAND	TCH

11	MS -> SS	ASSIGNMENT COMPLETE

12	SS -> SS	ALERTING

13	SS -> MS	CONNECT

14	MS -> SS	CONNECT ACKNOWLEDGE

15	MS -> SS	MODIFY	MMI action to change the mode

16	SS -> MS	MODIFY REJECT	with a mode that does not correspond

to the actual one

17	SS -> MS	STATUS ENQUIRY

18	MS -> SS	STATUS	cause 30#, state U26

Specific message contents:

None.

26.8.1.4.5.4.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.4.5.5	In-call functions / MS originated in-call modification / timeout of timer T323

26.8.1.4.5.5.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which timer T323 expires in state U26, mobile originating modify. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.5.2	Conformance requirement

1)	Upon expiration of T323 the MS shall initiate the procedures for call clearing with cause #102 "recovery on timer expiry".

Reference(s):

TS GSM 04.08, section 5.3.4.3.4.3.

26.8.1.4.5.5.3	Test purpose

To verify that upon expiration of T323 the MS shall initiate the procedures for call clearing with cause #102 "recovery on timer expiry".

26.8.1.4.5.5.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	support of dual bearer capability services

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS:

CC-state U11 "disconnect request".

Maximum duration of test:

10 s.

Procedure:

The MS being in the call active state, in-call modification procedure is initiated from the MS side. The MS shall send a MODIFY message with new mode to the SS. The SS does not respond until timer T323 expires at the MS. The MS is expected to respond with a DISCONNECT message. The SS checks timer T323 accuracy between emission of MODIFY and reception of DISCONNECT messages, the state of the MS and a cause value from the DISCONNECT message.

Expected sequence:

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call

2	SS -> MS	IMMEDIATE ASSIGNMENT 	SDCCH

3	MS -> SS	CM-SERVICE REQUEST

4	SS -> MS	CIPH MODE COMMAND

5	MS -> SS	CIPH MODE COMPLETE

6	MS -> SS	SETUP	containing bearer capabilities for dual mode

call

7	SS -> MS	AUTH REQUEST

8	MS -> SS	AUTH RESPONSE

9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode

call

10	SS -> MS	ASSIGNMENT COMMAND	TCH

11	MS -> SS	ASSIGNMENT COMPLETE

12	SS -> SS	ALERTING

13	SS -> MS	CONNECT

14	MS -> SS	CONNECT ACKNOWLEDGE

15	MS -> SS	MODIFY	MMI action to change the mode

16	SS		the SS waits for the timer

T323 expiry

17	MS -> SS	DISCONNECT	cause value #102, the SS checks timer T323

accuracy between emission of MODIFY and

reception of DISCONNECT messages

18	SS -> MS	STATUS ENQUIRY

19	MS -> SS	STATUS	cause 30#, state U11

Specific message contents:

None.

26.8.1.4.5.5.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.4.5.6	In-call functions / MS originated in-call modification / A successful channel change in state mobile originating modify

26.8.1.4.5.6.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which a change of a physical channel occurs in state U26, mobile originating modify. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.6.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U26, "Mobile Originating Modify", after successful completion of a channel assignment procedure or channel mode modify procedure shall remain in the call state U26.

Reference(s):

TS GSM 04.08, section 5.3.4.3.2, TS GSM 04.08, section 3.4.6.

26.8.1.4.5.6.3	Test purpose

To verify that a CC-entity of the MS in CC-state U26, "Mobile Originating Modify", after successful completion of a channel assignment procedure or channel mode modify procedure remains in the call state U26.

26.8.1.4.5.6.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	support of dual bearer capability services

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS:

CC-state U26, mobile originating modify.

Maximum duration of test:

10 s.

Procedure:

The MS being in the call active state, in-call modification procedure is initiated from the MS side. The MS shall send a MODIFY message with a new mode to the SS. The SS does not respond immediately, but performs channel assignment procedure. Once it has been completed, a channel mode modify procedure is performed. The state of the MS is checked finally.

Expected sequence:

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call

2	SS -> MS	IMMEDIATE ASSIGNMENT 	SDCCH

3	MS -> SS	CM-SERVICE REQUEST

4	SS -> MS	CIPH MODE COMMAND

5	MS -> SS	CIPH MODE COMPLETE

6	MS -> SS	SETUP	containing bearer capabilities for dual mode

call

7	SS -> MS	AUTH REQUEST

8	MS -> SS	AUTH RESPONSE

9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode

call

10	SS -> MS	ASSIGNMENT COMMAND	TCH

11	MS -> SS	ASSIGNMENT COMPLETE

12	SS -> SS	ALERTING

13	SS -> MS	CONNECT

14	MS -> SS	CONNECT ACKNOWLEDGE

15	MS -> SS	MODIFY	MMI action to change the mode

16	SS -> MS	ASSIGNMENT COMMAND

17	MS -> SS	ASSIGNMENT COMPLETE

18	SS -> MS	STATUS ENQUIRY

19	MS -> SS	STATUS	cause 30#, state U26

20	SS -> MS	CHANNEL MODE MODIFY

21	MS -> SS	CHANNEL MODE MOD ACK

22	SS -> MS	STATUS ENQUIRY

23	MS -> SS	STATUS	cause 30#, state U26

Specific message contents:

None.

26.8.1.4.5.6.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.4.5.7	In-call functions / MS originated in-call modification / An unsuccessful channel change in state mobile originating modify

26.8.1.4.5.7.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which an unsuccessful change of a physical channel occurs in state U26, mobile originating modify. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.7.2	Conformance requirement

1)	A CC-entity of the MS in CC-state U26, "Mobile Originating Modify", when returning to the old channel after handover failure and having established the link, shall remain in the call state U26.

Reference(s):

TS GSM 04.08, section 5.3.4.3.2.

26.8.1.4.5.7.3	Test purpose

To verify that a CC-entity of the MS in CC-state U26, "Mobile Originating Modify", when returning to the old channel after handover failure and having established the link, remains in the call state U26.

26.8.1.4.5.7.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	support of dual bearer capability services

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS:

CC-state U26, mobile originating modify.

Maximum duration of test:

10 s.

Procedure:

The MS being in the call active state, in-call modification procedure is initiated from the MS side. The MS shall send a MODIFY message with a new mode to the SS. The SS initiates handover procedure. When the MS tries to establish the main signalling link, it is prohibited by the SS. Then the MS shall return back to the old channel and re-establish correctly the link. The state of the MS is then checked.

Expected sequence:

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call

2	SS -> MS	IMMEDIATE ASSIGNMENT 	SDCCH

3	MS -> SS	CM-SERVICE REQUEST

4	SS -> MS	CIPH MODE COMMAND

5	MS -> SS	CIPH MODE COMPLETE

6	MS -> SS	SETUP	containing bearer capabilities for dual mode

call

7	SS -> MS	AUTH REQUEST

8	MS -> SS	AUTH RESPONSE

9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode

call

10	SS -> MS	ASSIGNMENT COMMAND	TCH

11	MS -> SS	ASSIGNMENT COMPLETE

12	SS -> SS	ALERTING

13	SS -> MS	CONNECT

14	MS -> SS	CONNECT ACKNOWLEDGE

15	MS -> SS	MODIFY	MMI action to change the mode

16	SS -> MS	HANDOVER COMMAND

17	MS -> SS	HANDOVER ACCESS	the SS does not respond

18	MS -> SS	HANDOVER FAILURE	after the MS has re-established the main

signalling link in the old channel

19	SS -> MS	STATUS ENQUIRY

20	MS -> SS	STATUS	cause 30#, state U26

Specific message contents:

None.

26.8.1.4.5.7.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.1.4.5.8	In-call functions / MS originated in-call modification / unknown message received

26.8.1.4.5.8.1	Definition and applicability

This is to test a special case of a in-call modification procedure, in which an unknown message is received in state U26, mobile originating modify. This test is applicable for any equipment supporting any dual mode bearer capability service (BS61 - Alternate Speech/Data, BS81 - Speech followed by Data, Teleservice 61 - Alternate Speech/Group 3 fax).

26.8.1.4.5.8.2	Conformance requirement

1)	A CC entity of a MS in CC-state U26, "Mobile Originating Modify", having received an unknown message from its peer entity shall return a STATUS message.

Reference(s):

TS GSM 04.08, section 8.4

26.8.1.4.5.8.3	Test purpose

To verify that a CC entity of a MS in CC-state U26, "Mobile Originating Modify", having received an unknown message from its peer entity returns a STATUS message.

26.8.1.4.5.8.4	Method of test

Related PICS/PIXIT statement(s):

-	supported teleservices

-	support of dual bearer capability services

Initial conditions:

System Simulator:	1 cell, default parameters.

Mobile Station: 	The MS is in MM-state "idle, updated" with valid TMSI and CKSN.

Foreseen final state of the MS:

CC-state U26, mobile originating modify.

Maximum duration of test:

10 s.

Procedure:

The MS being in the call active state, in-call modification procedure is initiated from the MS side. The MS shall send a MODIFY message with a new mode to the SS. The SS sends a message with message type not defined for the protocol discriminator. The state of the MS is then checked.

Expected sequence:

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	MMI action to initiate a dual mode call

2	SS -> MS	IMMEDIATE ASSIGNMENT 	SDCCH

3	MS -> SS	CM-SERVICE REQUEST

4	SS -> MS	CIPH MODE COMMAND

5	MS -> SS	CIPH MODE COMPLETE

6	MS -> SS	SETUP	containing bearer capabilities for dual mode

call

7	SS -> MS	AUTH REQUEST

8	MS -> SS	AUTH RESPONSE

9	SS -> MS	CALL PROCEEDING	agreeing bearer capabilities for dual mode

call

10	SS -> MS	ASSIGNMENT COMMAND	TCH

11	MS -> SS	ASSIGNMENT COMPLETE

12	SS -> SS	ALERTING

13	SS -> MS	CONNECT

14	MS -> SS	CONNECT ACKNOWLEDGE

15	MS -> SS	MODIFY	MMI action to change the mode

16	SS -> MS	unknown message	message type not defined for PD

17	MS -> SS	STATUS	cause 30#, state U26

Specific message contents:

None.

26.8.1.4.5.8.5	Requirements

The MS shall respond as indicated in the steps of expected sequence.

26.8.2	Call Re-establishment

26.8.2.1	Introduction

The call re-establishment procedure is used to re-instate a call which has failed due to a lower layer failure. It is described in GSM 04.08, sections 4.5.1.6 and 5.5.4.

This test only applies to MS supporting speech and to non-speech MS supporting call re-establishment (see PIXIT).

26.8.2.2	Purpose of the test

The purpose of these test is to verify that the MS can correctly perform a call re-establishment and that re-establishment attempts are carried out only when this is allowed.

26.8.2.3	Call present, re-establishment allowed

26.8.2.3.1	Initial conditions

The SS simulates cells A and B which have equal LAI. The conditions of cells A and B shall allow call re-establishment:

Cell B is not barred, the RACH control parameters information element sent in SYSTEM INFORMATION TYPE 1 to 4 messages of cell B specifies 'call reestablishment allowed in the cell', the NCC of cell B is indicated as permitted in the PLMN permitted information element of SYSTEM INFORMATION TYPE 2 and 6 messages of cell A.

26.8.2.3.2	Procedure

The MS is brought into the idle, updated state in cell A, and a call is established according to section "structured procedures, mobile originating call, early assignment, method of test", paragraphs a) to k).

The SS then stops transmission on the TCH/SACCH. The RF level of cell A is lowered in order to force the MS to select cell B.

In the required re-establishment of the call, after the radio link time-out, the SS applies the following procedure on cell B:

a)	The SS accepts the CHANNEL REQUEST message and returns the IMMEDIATE ASSIGNMENT message.

b)	The SS accepts the CM-REESTABLISHMENT REQUEST message from the MS via the data link establishment procedure.

c)	The SS sends message CIPH MODE COMMAND and starts deciphering.

d)	After reception of the CIPHERING MODE COMPLETE message the SS starts enciphering.

e)	The SS then sends message ASSIGNMENT COMMAND.

f)	Having received message ASSIGNMENT COMPLETE, the SS enters conversation state.

The contents of the messages shall be according to section "structured procedures, mobile originating call, early assignment, method of test".

26.8.2.3.3	Requirements

After expiry of the radio link timeout the MS shall re-establish the call according to the following procedure:

1)	Before step a) the MS shall initiate immediate assignment by sending the CHANNEL REQUEST message with establishment cause set to call re-establishment.

2)	After step a) the MS shall send a layer 2 SABM frame containing the CM REESTABLISHMENT REQUEST message to the SS.

3)	After step c) the MS shall activate ciphering and send the message CIPHERING MODE COMPLETE.

4)	After step e) the MS shall send the message ASSIGNMENT COMPLETE on the correct channel.

The MS shall then through connect the speech path in both directions.

The contents of the messages shall be according to section "structured procedures, mobile originating call, early assignment, requirements" with the following change/addition:

In message CHANNEL REQUEST the establishment cause shall be call re-establishment.

Message:	CM-REESTABLISHMENT REQUEST	(GSM 04.08, 9.2.4) to the SS:

Information element	Comment	Value

protocol discriminator	MM	0101

transaction identifier	not used	0000

message type		0x10 1000

ciphering key seq.numberas applic.from loc.upd	xxx

MS classmark 2

- revision level	as applicable	xxx

- encryption algorithm	A5	00

- RF power capability	as applicable	xxx

- short message capability	as applicable	x

- frequency band	band no 0	000

mobile identity

- odd/even no of digits	as applicable	x

- type of identity	TMSI	100

- identity digits	as applicable

location area identification	Cell B

26.8.2.4	Call present, re-establishment not allowed

26.8.2.4.1	Initial conditions

The RE field of the RACH control parameters information element broadcast in messages SYSTEM INFORMATION TYPE 1, 2, 3 and 4 of the serving cell is set to "call reestablishment not allowed in the cell".

26.8.2.4.2	Procedure

a)	The MS is brought into the idle, updated state and a call is established according to section "structured procedures, mobile originating call, early assignment, method of test", paragraphs a) to k).

b)	The SS then stops transmission on the TCH/SACCH.

26.8.2.4.3	Requirements

1)	The MS shall not try to re-establish the call, but return to the idle, updated state.

26.8.2.5	Call under establishment, re-establishment allowed

26.8.2.5.1	Initial conditions

The default conditions of the serving cell shall allow call re-establishment (bit RE is set to 0).

26.8.2.5.2	Procedure

a)	The MS is brought into the idle, updated state.

b)	Call establishment is started according to the procedures of section "structured procedures, mobile originating call, early assignment, method of test", paragraphs a) to h).

c)	The SS then stops transmission on the SDCCH/SACCH.

26.8.2.5.3	Requirements

1)	The MS shall not try to re-establish the call, but return to the idle, updated state.

26.8.3	user to user signalling

26.8.3.1	Purpose of the test

The purpose of this test is to verify that inclusion of the 'user-user' information element in a 'call-control' message causes no adverse effects on MS operation.

The 'user to user' information element is used to convey information between the mobile user and a remote ISDN user.

Note:	There is no test for an MS originating call including a 'user-user' information element since it is not a mandatory MS feature.

26.8.3.2	Method of test

26.8.3.2.1	Initial conditions.

1)	The SS shall simulate a BSS with CCCH and BCCH.

2)	The MS shall be placed in the MM-state "idle,updated" (refer location updating accepted 26.6.4.1).

26.8.3.2.2	Procedure 1

a)	The SS shall page the MS and establish a normal call.

b)	The SETUP message from the SS shall contain a 'user-user' information element.

Message:	SETUP	(GSM 04.08, 9.3.15)

Information element	Comment	Value

message type	setup

bearer capabilities	(see note 2) B.C. iei

length

full rate speech

user-user	user-user iei

length

PD (see note 1)

user-user(see note 1)

Note 1:	The codings above are for example only. For the case of an MS which supports 'user-user' signalling it may be necessary to add meaning to the data fields.

Note 2:	An applicable bearer capability shall be used, see PICS/PIXIT statement.

26.8.3.2.3	Procedure 2

a)	The SS shall initiate call cleardown and include in the subsequent DISCONNECT message a 'user-user' information element.

Message:	DISCONNECT	(GSM 04.08, 9.4.16)

Information element	Comment	Value

protocol discriminator	call-control	0011

transaction identifier	originating	1xxx

message type	disconnect	0x10 0101

cause	(MV)

length (4 bytes)	0000 0010

coding std(gsm)	0110 0000

cause (normal)	1001 0000

user-user	user-user iei	0111 1110

length (4 bytes)	0000 0010

PD (see note)	0000 0000

user-user(see note)	0000 0000

Note:	The coding above is for example only. For the case of an MS which supports 'user-user' signalling it may be necessary to add meaning to the data fields.

26.8.3.3	Requirements

26.8.3.3.1	Requirement to procedure 1

1)	The MS shall respond to paging.

2)	The MS shall not be adversely affected by the inclusion of the information element and shall successfully establish a normal call.

26.8.3.3.2	Requirements to procedure 2

1)	The MS shall not respond adversely to the inclusion of the 'user-user' information element in the DISCONNECT message and shall continue to clear the call normally.