6.	Reference test methods.

6.1	General

[Annex 1] (normative) gives reference test conditions to be used throughout this ETS, unless otherwise specified. It consists of a part on general conditions, and a part on normal and extreme test conditions.

Unless otherwise specified tests are run using the normal test conditions.

If a test is to be run using the extreme test conditions then this is identified in the test description.

For all tests, the MS is connected to the SS. This connection, unless otherwise specified, is to the permanent antenna connector for a MS which is equipped with one, or via the temporary antenna connector defined in [Annex 1, GC7], for a MS with an integral antenna, and not normally having a means of connecting an external antenna.

During the tests the measuring equipment shall be arranged as described in [Annex 4]. This arrangement of measuring equipment is generally referred to as the System Simulator (SS).

6.2	Choice of frequencies in the frequency hopping mode

For the tests using frequency hopping, 38 frequencies are used over

P-GSM 900:		a 21 MHz band

E-GSM 900:		a [??] MHz band

DCS 1800:		a 75 MHz band

Table 6.1: Hopping frequencies

ARFCN

P-GSM 900	10, 14, 17, 18, 22, 24, 26, 30, 31, 34, 38, 42, 45, 46, 50, 52, 54, 58,

59, 62, 66, 70, 73, 74, 78, 80, 82, 86, 87, 90, 94, 98, 101, 102, 106,

108, 110, 114

E-GSM 900

DCS 1800	522, 539, 543, 556, 564, 573, 585, 590, 606, 607, 624, 627, 641, 648,

658, 669, 675, 690, 692, 709, 711, 726, 732, 743, 753, 760, 774, 777,

794, 795, 811, 816, 828, 837, 845, 858, 862, 879

Note:	The range of frequencies available during tests under simulated fading conditions is restricted by the fading simulator bandwidth.

6.3	"Ideal" radio conditions

In this ETS the following conditions are referenced by the term "ideal" radio conditions:

No multipath conditions

MS Power control level:

GSM 900:	7

DCS 1800:	3

RF level to MS:	63 dBmVemf( )	(Not tests in section [II.4 or II.6.1.4])

RF level to MS:	20 dB above reference sensitivity level ( )	[II.4])

RF level to MS:	28 dBmVemf( )	(Tests in section [II.6.1.4])

6.4	Standard test signals.

The standard test signals C0, C1, I0, I1 and I2 as used in this ETS, are defined in GSM 11.40.