26.6	Test of the elementary procedures for radio resource management

Note:	For SS implementor : if tests are concatenated, it is important that unused fields in IMMEDIATE ASSIGNMENT REJECT messages do not use Request References that relate to CHANNEL REQUEST messages recently transmitted by the MS.

26.6.1	Immediate Assignment

The immediate assignment procedure is used by the network to establish a dedicated control channel for the MS and network to communicate the detail of the service requested. If the Mobile Station does not implement the procedure correctly, radio resources can be wasted as the Mobile Station might use the wrong channels.

26.6.1.1	Immediate Assignment / SDDCH or TCH assignment

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.1.3 in phase 1 version.

-	Timing Advance is no longer specified]

Purpose:

To verify that the MS can correctly set up a dedicated SDCCH control channel and that the MS can correctly set up a dedicated TCH/FACCH control channel.

Conformance Requirement:

1.	Following a PAGING REQUEST message, the MS shall correctly set up an RR connection on the SDCCH/8 described in the IMMEDIATE ASSIGNMENT message.

2.	Following a PAGING REQUEST message, the MS shall correctly set up an RR connection on the TCH/FACCH described in the IMMEDIATE ASSIGNMENT message.

Reference(s):

GSM 04.08 section 3.3.1.2.1

Related PICS/PIXIT Statement(s):

Support rate(s) of TCH: TCH/F and/or TCH/H.

Initial Conditions:

System Simulator:

1 cell, except that CCCH-CONF is set to '1 basic physical channel used for CCCH not combined with SDCCHs'.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST message the SS assigns an SDCCH. The MS shall go to the correct channel and send a PAGING RESPONSE message. Then the SS initiates RR-release by sending a CHANNEL RELEASE message.

If TCH/F is supported by the MS, the test is repeated with the SS assigning a TCH/F.

If TCH/H is supported by the MS, the test is repeated with the SS assigning a TCH/H.

Maximum Duration of Test:

6 seconds per value of the execution timer

Expected Sequence:

This sequence is performed for execution counter, K = 1, 2, 3 (unless the TCH is not supported).

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMM ASSIGNMENT	Channel Type : see below

4	MS -> SS	PAGING RESPONSE	Shall be sent on the correct channel

5	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

IMMEDIATE ASSIGNMENT

K=1, SDCCH test : Channel Type = SDCCH/8

K=2, TCH/F test : Channel Type = Bm + ACCHs

K=3, TCH/H test : Channel Type = Lm + ACCHs, subchannel randomly drawn

26.6.1.2	Immediate Assignment / extended assignment

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.1.5 in phase 1 version.

-	two procedures have been concatenated to one test.

-	in the last part of the test the MS shall continue to send CHANNEL REQUESTs]

-

]

Note:	In these tests the SS must send the immediate assignment messages in due time to allow for the MS to receive them and send a PAGING RESPONSE rather than another random access. This applies to the whole of section 26.

Purpose:

To verify that the MS can correctly identify its own assignment in either the Request Reference 1 or the Request Reference 2 information element in an extended assignment message.

To verify that the MS only reacts to an Immediate Assignment which references one of the last 3 CHANNEL REQUEST messages from the MS.

Conformance Requirement:

1:	The MS shall go to the allocated SDCCH/4 and send a PAGING RESPONSE message containing its identity and its classmark.

2:	The MS shall go to the allocated SDCCH/8 and send a PAGING RESPONSE message containing its identity and its classmark.

3:	Same as test purpose

4:	Same as test purpose

Reference(s):

GSM 04.08 section 3.3.

Related PICS/PIXIT Statement(s):

None.

Initial Conditions:

System Simulator:

1 cell, Max-retrans is set to 7.

Mobile Station:

The MS is in the "idle, updated" state. with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with a TMSI allocated.

Method of Test:

In the first part of the test, the SS pages the MS, which shall react by sending CHANNEL REQUEST messages. Immediately after reception of the n-th CHANNEL REQUEST message (n being randomly chosen by the SS from the set {1, 2 ... 8}) the SS sends an IMMEDIATE ASSIGNMENT EXTENDED message, which references one of the last 3 CHANNEL REQUEST messages from the MS. The MS shall then go to the correct channel and send a PAGING RESPONSE message. The SS will then release the channel.

In the second part of the test, the SS again pages the MS, which shall react by sending CHANNEL REQUEST messages. Immediately after reception of the k-th CHANNEL REQUEST message (k being randomly chosen by the SS from the set {4, 5 ... 8}) the SS sends an IMMEDIATE ASSIGNMENT EXTENDED message which, instead of referencing one of the last 3 CHANNEL REQUEST messages from the MS, references an earlier CHANNEL REQUEST message. The MS shall then ignore the IMMEDIATE ASSIGNMENT EXTENDED message and continue to send CHANNEL REQUEST messages until the Max-Retrans value has been reached. Then a period of 7 seconds shall elapse in order to allow the MS to perform cell reselection (this allows for the time between the last CHANNEL REQUEST message and the beginning of cell reselection).

In the third part of the test, the CCCH_CONF of the SS is set to non-combined and the SS pages the MS, which shall react by sending CHANNEL REQUEST messages. Immediately after reception of the r-th CHANNEL REQUEST message (r being randomly chosen by the SS from the set {4, 5 ... 8}) the SS sends an IMMEDIATE ASSIGNMENT EXTENDED message which, in the second request reference, references one of the last 3 CHANNEL REQUEST messages from the MS. The associated Channel Description allocates SDCCH(S) (S being randomly chosen by the SS from the set {0,1 ... 7}). The MS shall then go to the correct channel and send a PAGING RESPONSE message. The SS will then release the channel.

Maximum Duration of Test:

90 seconds.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST	n CHANNEL REQUESTs (n being

randomly drawn from {1.... 8}

are sent, all with Establ. Cause =

"Answer to paging"

3	MS -> SS	CHANNEL REQUEST

4	SS -> MS	IMM ASSIGN EXT 	see note 1

5	MS -> SS	PAGING RESPONSE

6	SS -> MS	CHANNEL RELEASE

7	--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

8	SS -> MS	PAG REQ TYPE 1

9	MS -> SS	CHANNEL REQUEST	k CHANNEL REQUESTs (k being

randomly drawn from the set {4, 5,

8}) are sent all with

10	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

11	SS -> MS	IMM ASSIGN EXT	see note 2

12	MS -> SS	CHANNEL REQUEST	8-k CHANNEL REQUESTs are sent, all

with Establ. Cause = "Answer to

13	MS -> SS	CHANNEL REQUEST	paging"

14	SS		The SS verifies that the MS does not

transmit any Layer 2 frames for at least

3 seconds.

15	 SS		The SS sets CCCH_CONF to

non-combined

16	 SS		The SS waits 40 seconds to

allow the MS to perform

cell reselection and to

read the BCCH information

17	SS -> MS	PAG REQ TYPE 1

18	MS -> SS	CHANNEL REQUEST	r CHANNEL REQUESTs (r being

randomly drawn from {4, 5... 8}

are sent, all with Establ. Cause =

"Answer to paging"

19	MS -> SS	CHANNEL REQUEST

20	SS -> MS	IMM ASSIGN EXT 	see note 3

21	MS -> SS	PAGING RESPONSE

22	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

Note 1 : 	The first Request Reference is the one which pertains to the i-th CHANNEL REQUEST sent by the MS, where i is an integer in the set {max (1,n-2) ... n}, its value being randomly drawn by the SS. The second Request Reference shall be different from any Request Reference the MS has generated in this test.

Note 2 : 	The first Request Reference is the one which pertains to the i-th CHANNEL REQUEST sent by the MS, where i is an integer in the set {1 ... k-3}, its value being randomly drawn by the SS. The second Request Reference shall be different from any Request Reference the MS has generated in this test.

Note 3 : 	The second Request Reference is the one which pertains to the i-th CHANNEL REQUEST sent by the MS, where i is an integer in the set {r-2, r-1, r}, its value being randomly drawn by the SS. The first Request Reference shall be different from any Request Reference the MS has generated in this test.

26.6.1.3	Immediate Assignment / assignment rejection

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.1.6 in phase 1 version.

-	two procedures have been concatenated to one test.

-	there is an IMMEDIATE ASSIGNMENT REJECT added at the end of the test to avoid a cell reselection.]

Purpose:

To verify that the MS can accept an Immediate Assignment Rejection.

To verify that the MS can respond to paging  after an Immediate Assignment Rejection is received on a different cell.

Conformance Requirement:

1:	The MS shall respond to the Paging Request message by sending a Channel Request message with establishment cause set to "Answer to Paging". After the reception of IMMEDIATE ASSIGNMENT REJECT, the MS shall not transmit during the time indicated in the "Wait Indication" field of the IMMEDIATE ASSIGNMENT REJECT message, and then it shall answer to the new paging requests.

2:	After an assignment rejection, the MS shall perform a cell reselection (idle mode operation) and the MS shall not transmit unless a different cell is selected.

Reference(s):

GSM 04.08, section 3.3.1.2.2.

Related PICS/PIXIT Statement(s):

None.

Initial Conditions:

System Simulator:

2 cells with the same LAI, Max-Retrans is 7.

Mobile Station:

The MS is camped on cell A and is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

The MS is camped on cell B and is in the "idle, updated" state, with a TMSI allocated.

Method of Test:

The SS pages the MS, which shall react by sending CHANNEL REQUESTs. Immediately after reception of the n-th CHANNEL REQUEST (n being an integer from the set {1, 2 ... 8}, randomly chosen by the SS) the SS sends an IMMEDIATE ASSIGNMENT REJECT message, which references one of the last 3 CHANNEL REQUESTs from the MS, and with the Wait Indication set to x seconds (x being an integer from the set {5, 6 ... 255}, randomly chosen by the SS). The SS continues to send paging messages for that mobile station in every block of the mobile station's paging subgroup for x+2 seconds. The MS shall only answer with a CHANNEL REQUEST after x seconds have elapsed.

The SS responds to this CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message with the Wait Indication set to 255 seconds.

Immediately afterwards the SS changes the power levels so the MS selects cell B. After 12 seconds have elapsed the SS pages the MS in cell B and the MS shall answer to this page. In order to avoid another cell reselection the SS then sends another IMMEDIATE ASSIGNMENT REJECT.

Maximum Duration of Test:

5 minutes.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST	n CHANNEL REQUESTs (n being

.	.	.	randomly drawn from the set {1, 2

.	.	.	 ... 8}) are sent, all with

1+n	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

2+n	SS -> MS	IMM ASSIGN REJ	1st, 3rd and 4th Request

References are different to all

n Request References received from

the MS under test.

2nd Request Reference : see note 1.

2nd Wait Indication = x seconds (x

being randomly drawn from the set

{5 ,6 ... 255}

3+n	SS -> MS	PAG REQ TYPE 1	Sent for more than x seconds

.	.	.

.	.	.

k	SS -> MS	PAG REQ TYPE 1	between x and x+2 seconds (note 2)

k+1	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

k+2	SS -> MS	IMM ASSIGN REJ	1st, 2nd and 4th Request

References are different to all

n Request References received from

the MS.

The 3rd Request Reference pertains

to the last CHANNEL REQUEST

sent by the MS.

The 3rd Wait Indication is 255

seconds

k+3	--------	----------	Raise power level of cell B, lower

power level of cell A until the MS

selects cell B

k+4	SS -> MS	PAG REQ TYPE 1	Sent once, 12 seconds after the change

of levels

k+5	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

k+6	SS -> MS	IMM ASSIGN REJ

Note 1 :	The Request Reference is the one which pertains to the i-th CHANNEL REQUEST sent by the MS, where i is an integer from the set {max(1,n-2) ... n}, its value being randomly drawn by the SS.

Note 2 :	the value of k is not important in this test.

Specific Message Contents:

None.

26.6.1.5	Immediate Assignment / ignore assignment

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.1.7 in phase 1 version.

-	two tests have been concatenated to one.

(* it is not clear that there were originally two tests : it is possible to do as one test. How have R+S implemented the FTA? *)

-	an IMMEDIATE ASSIGNMENT REJECT has been added to the end of the test in order to avoid a cell reselection

-	Timing Advance is no longer included]

Purpose:

To verify that the MS ignores an assignment for another MS while it is waiting for an assignment of its own.

Conformance Requirement:

1.	An MS waiting for a response from the network, following the sending of a CHANNEL REQUEST, shall ignore an IMMEDIATE ASSIGNMENT message with a request reference containing a wrong frame number.

2.	An MS is waiting for an assignment of its own, shall ignore an IMMEDIATE ASSIGNMENT message with a request reference containing a wrong random access information.

Reference(s):

GSM 04.08 section 3.3.1.2.1

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

1 cell, max_retrans = 7

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

[The SS pages the MS, which reacts with CHANNEL REQUESTs. The SS responds to the first CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT containing a wrong Request Reference (in the first run of the test the frame number is wrong, in the repetition it is the random access info that is wrong). It is verified for 2 seconds that the MS does not start signalling on the SDCCH. The MS shall ignore the assignment and send another CHANNEL REQUEST message. In order to avoid cell reselection the SS now answers with a correct IMMEDIATE ASSIGNMENT REJECT and repeats the test once.

Note:	The test can be repeated before the 2 seconds have passed. In this case the SS shall continue to monitor the SDCCH assigned in step 3 while at the same time try to establish a new SDCCH.]

The SS pages the MS, which reacts with CHANNEL REQUESTs. The SS responds to the first CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT containing a slightly different Request Reference (only the frame number is wrong). This IMMEDIATE ASSIGNMENT shall be ignored by the MS. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT containing a slightly different Request Reference (only the random access information is wrong). This IMMEDIATE ASSIGNMENT shall be ignored by the MS. The MS sends a third CHANNEL REQUEST message. In order to avoid cell reselection the SS now answers with a correct IMMEDIATE ASSIGNMENT REJECT. It is checked throughout the test that the MS does not start signalling on any SDCCH.

Maximum Duration of Test:

6 seconds.

Expected Sequence:

(*

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

3	SS -> MS	IMM ASSIGNMENT	Frame number in Request Reference

is 2 too high. The MS shall not

start signalling on the assigned

SDCCH. This is verified for a

period of 2 seconds

4	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

5	SS -> MS	IMM ASSIGN REJ

6	SS -> MS	PAG REQ TYPE 1

7	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

8	SS -> MS	IMM ASSIGNMENT	Random access info in Request

Reference is wrong. The MS shall

not start signalling on the

assigned SDCCH. This is verified

for a period of 2 seconds

9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

10	SS -> MS	IMM ASSIGN REJ

*)

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to

paging", Random Reference = X,

sent in frame number, FN

3	SS -> MS	IMM ASSIGNMENT	The Random Access Information is

that included in the CHANNEL

REQUEST message but the frame

number is FN+2. The MS shall not

start signalling on any SDCCH.

4	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to

paging", Random Reference = X,

sent in frame number, FN

5	SS -> MS	IMM ASSIGNMENT	The frame number is FN but the

Random Access Information in

the Request Reference is not that

which was sent in the CHANNEL

REQUEST. The MS shall not

start signalling on any SDCCH.

6	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

7	SS -> MS	IMM ASSIGN REJ	Note

(* may need to allow for more channel requests : must also change the default value of max retrans *)

Note:	throughout the test, and for at least 2 seconds after step 5, it shall be checked that the MS does not signal on any SDCCH.

Specific Message Contents:

None.

26.6.2	Test of paging

The Paging procedure is used by the network to cause the Mobile Station to establish a radio connection. Normally the Mobile Station listens to its paging subchannel, but this can be modified by the use of different page modes. The correct implementation of the paging procedure in the Mobile Station is essential for the basic establishment of a connection.

26.6.2.1	Normal paging

26.6.2.1.1	Paging / normal / type 1

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.2.1.3 in phase 1 version.

-	6 procedures have been concatenated to one test.

-	It is stated more clearly that the paging parameters are chosen randomly by the SS.

]

(* step 5 has been changed for phase 2 imsi response to imsi paging.*)

(* steps 25 and 26 may be deleted as ignoring IMEI pages is difficult to 'justify'.*)

Purpose:

To test that the MS is able to determine its CCCH group and paging group correctly and that the MS responds correctly to various PAGING REQUEST TYPE 1 messages when the page mode is set to normal paging. All valid ways of addressing the MS are tested. It is tested that the MS responds with the same type of identity that is used in the PAGING REQUEST TYPE 1 message. It is tested that the MS ignores fill paging.

Conformance Requirement:

1.	The MS shall respond correctly to various PAGING REQUEST TYPE 1 messages, when the page mode is set to normal paging, in the following cases:

1.1	The MS is addressed with its IMSI in the first Mobile Identity field. The optional Mobile Identity field is not present.

1.2.	The MS is addressed with its TMSI in the first Mobile Identity field. The optional Mobile Identity field specifies an IMSI different from that of the MS.

1.3.	The first Mobile Identity field specifies a TMSI different from that of the MS. The optional Mobile Identity field addresses the MS by its IMSI.

1.4	The first Mobile Identity field specifies a TMSI different from that of the MS. The optional Mobile Identity field contains the correct TMSI of the MS.

2.	An MS shall ignore PAGING REQUEST TYPE 1 messages with incorrect information, when the page mode is set to normal paging, in the following case:

2.1	The MS is addressed with its TMSI in the first Mobile Identity field, but the type of identity in this field is set to "No Identity". The optional Mobile Identity field is not present.

Reference(s):

GSM 04.08 section 3.3.2, GSM 05.02 section 6.5.

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The SS pages the MS 5 times with different PAGING REQUEST TYPE 1 messages on the paging subchannel which corresponds to the MS's IMSI.

In the first 4 cases, where the MS is addressed by its IMSI or its TMSI, the MS shall answer to the paging by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST by assigning a channel, and the MS shall then send a correct PAGING RESPONSE. The SS then releases the channel.

In the last case, it is tested that the MS does not answer to paging that does not address it.

Maximum Duration of Test:

2 min

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains IMSI of

MS, 2nd Mobile Ident not present.

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

3	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

4	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 3

5	MS -> SS	PAGING RESPONSE	Mobile Ident : IMSI.

6	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

7	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains TMSI of

MS, 2nd Mobile Ident contains IMSI

of another MS.

8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

10	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 9

11	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.

12	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

13	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains TMSI of

another MS, 2nd Mobile Ident

contains IMSI of MS.

14	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

15	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

16	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 15

17	MS -> SS	PAGING RESPONSE	Mobile Ident : IMSI.

18	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

19	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains TMSI of

another MS, 2nd Mobile Ident

contains TMSI of MS.

20	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

21	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

22	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 21

23	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.

24	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

25	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains TMSI of

MS but with type of identity set

to "no identity", second Mobile

Ident not present.

26	--------	--------	During 1 second, the SS checks

that the MS does not produce any

Layer 2 frames.

(* In steps 26 and 28, it may be sufficient to check for no layer 3 messages rather than checking for no RF-transmission.*)

Specific Message Contents:

None.

26.6.2.1.2	Paging / normal / type 2

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.2.1.5 in phase 1 version.

-	6 procedures have been concatenated to one test.

-	It is stated more clearly that the paging parameters are chosen randomly by the SS.]

(* imsi response to imsi paging is included.*)

(* imei paging test may not be 'justified' *)

Purpose:

To test that the MS is able to determine its CCCH group and paging group correctly and that the MS responds correctly to various PAGING REQUEST TYPE 2 messages when the page mode is set to normal paging. All valid ways of addressing the MS are tested. It is tested that the MS responds with the same type of identity that is used in the PAGING REQUEST TYPE 2 message. It is tested that the MS ignores a PAGING REQUEST TYPE 2 message that does not address it.

Conformance Requirement:

1.	The MS shall respond correctly (by sending CHANNEL REQUEST messages with an Establishment Cause set to "Answer to Paging") to various PAGING REQUEST TYPE 2 messages, when the page mode is set to normal paging, in the following cases:

1.1	The MS is addressed in the first TMSI field.

1.2	The MS is addressed in the second TMSI field.

1.3	The MS is addressed in the optional Mobile Identity field with its TMSI.

1.4	The MS is addressed in the optional Mobile Identity field with its IMSI.

2.	The MS shall ignore PAGING REQUEST TYPE 2 messages with incorrect information, when the page mode is set to normal paging, in the following case:

2.1	The MS is addressed in the optional Mobile Identity field with its TMSI, but the type of identity in this field is set to "No Identity".

Reference(s):

GSM 04.08 section 3.3.2.

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The SS pages the MS 5 times with different PAGING REQUEST TYPE 2 messages on the paging subchannel which corresponds to the MS's IMSI.

In the first 4 cases, where the MS is addressed by its IMSI or by its TMSI, the MS shall answer to the paging by sending CHANNEL REQUESTs. The SS responds to the second request by assigning a channel, and the MS shall then send a correct PAGING RESPONSE. The SS then releases the channel.

In the last case, it is tested that the MS does not answer to paging that does not address it.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 2	1st TMSI addresses MS, 2nd TMSI

addresses another MS, Mobile

Identity IE not present

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

3	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

4	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 3

5	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.

6	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

7	SS -> MS	PAG REQ TYPE 2	1st TMSI addresses another MS, 2nd

TMSI addresses MS, Mobile Identity

IE not present.

8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

10	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 9

11	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.

12	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

13	SS -> MS	PAG REQ TYPE 2	1st TMSI addresses another MS, 2nd

TMSI addresses another MS, Mobile

Identity IE contains TMSI of MS.

14	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

15	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

16	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 15

17	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.

18	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

19	SS -> MS	PAG REQ TYPE 2	1st TMSI addresses another MS, 2nd

TMSI addresses another MS, Mobile

Identity IE contains IMSI of MS.

20	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

21	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

22	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 21

23	MS -> SS	PAGING RESPONSE	Mobile Ident : IMSI.

24	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

25	SS -> MS	PAG REQ TYPE 2	1st TMSI addresses another MS, 2nd

TMSI addresses another MS, Mobile

Identity IE contains TMSI of MS

but with type of identity set to

"no identity".

26	--------	--------	During 1 second, the SS checks

that the MS does not produce any

Layer 2 frames.

(* RF transmission ==> L3 messages ?*)

Specific Message Contents:

None.

26.6.2.1.3	Paging / normal / type 3

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.2.1.7 in phase 1 version.

-	4 procedures have been concatenated to one test.

-	It is stated more clearly that the paging parameters are chosen randomly by the SS.]

Purpose:

To test that the MS is able to determine its CCCH group and paging group correctly and that the MS responds correctly to various PAGING REQUEST TYPE 3 messages when the page mode is set to normal paging. All valid ways of addressing the MS are tested.

Conformance Requirement:

An MS shall respond correctly to various PAGING REQUEST TYPE 3 messages, when the page mode is set to normal paging. The MS shall send CHANNEL REQUEST messages, with an Establishment Cause set to "Answer to Paging", until the network answers. The number of CHANNEL REQUEST messages shall be limited by the parameter Max-retrans. After the assignment procedure, the MS shall send a PAGING RESPONSE message on the channel assigned by the network.

Reference(s):

GSM 04.08 section 3.3.2.

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The SS pages the MS 4 times with different PAGING REQUEST TYPE 3 messages on the paging subchannel which corresponds to the MS's IMSI.

In all the cases the MS shall answer to the paging by sending CHANNEL REQUESTs. The SS responds to the second request by assigning a channel, and the MS shall then send a correct PAGING RESPONSE. The SS then releases the channel.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 3	1st TMSI addresses MS; 2nd, 3rd

and 4th TMSIs address other MSs.

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

3	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

4	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 3

5	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.

6	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

7	SS -> MS	PAG REQ TYPE 3	2nd TMSI addresses MS; 1st, 3rd

and 4th TMSIs address other MSs.

8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

10	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 9

11	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.

12	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

13	SS -> MS	PAG REQ TYPE 3	3rd TMSI addresses MS; 1st, 2nd

and 4th TMSIs address other MSs.

14	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

15	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

16	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 15

17	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.

18	SS -> MS	CHANNEL RELEASE

--------	--------	The SS waits 12 seconds to allow

the MS to perform cell reselection

19	SS -> MS	PAG REQ TYPE 3	4th TMSI addresses MS; 1st, 2nd

and 3rd TMSIs address other MSs.

20	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

21	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

22	SS -> MS	IMM ASSIGNMENT	Request Reference = pertaining to

the message received in step 21

23	MS -> SS	PAGING RESPONSE	Mobile Ident : TMSI.

24	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

None.

26.6.2.2	Paging / extended

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	two procedures have been concatenated to one test.

-	It is stated more clearly that the paging parameters are chosen randomly by the SS.

-	Instead of specifying a specific page mode , the mode is now chosen arbitrarily.*)

-	It is specified that in step 6 the random reference is not one used by the MS.]

Purpose:

To test that the MS is operating in the extended page mode when this is ordered by the SS in either a PAGING REQUEST message or an IMMEDIATE ASSIGNMENT message.

Conformance Requirement:

1.	The MS shall operate in the extended page mode when this is ordered by the network in a PAGING REQUEST message not addressing the MS but on the paging subchannel which corresponds to the MS's identity.

2.	The MS shall operate in the extended page mode when this is ordered by the network in an IMMEDIATE ASSIGNMENT message on the paging subchannel which corresponds to the MS's identity.

Reference(s):

GSM 04.08 section 3.3.2.1; GSM 05.02, section 6.5

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The SS sends a PAGING REQUEST TYPE 1 message not addressing the MS under test but on the paging subchannel which corresponds to the MS's identity. The page mode is set to "extended paging". In the next but one paging subblock on the same CCCH the SS sends a PAGING REQUEST TYPE 1 message specifying an arbitrarily chosen page mode and addressing the MS by its TMSI. The MS shall respond to the last page by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message.

The SS then sends an IMMEDIATE ASSIGNMENT on the paging subchannel which corresponds to the MS's identity. The random reference is different to those used by the Mobile Station in the last two CHANNEL REQUEST messages. (Phase 2 requires a Mobile Station to react on an IMMEDIATE ASSIGNMENT after a rejection.) The page mode is again set to "extended paging". In the next but one paging subblock on the same CCCH the SS sends a PAGING REQUEST TYPE 1 message specifying an arbitrarily chosen page mode and addressing the MS by its TMSI. The MS shall respond with CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT message.

[editorial note : If it is an essential feature that the MS ignores paging on other subchannels, later, a test shall be elaborated, which tests that the MS does not take into account the paging reorganisation (only page modes on the MS's own paging subchannel shall be taken into account). However it seems not reasonable to forbid the MS listening to e.g. all subchannels.]

Maximum Duration of Test:

10 sec

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	Mobile Ident : IMSI of another MS,

page mode = "extended paging"

2	SS -> MS	PAG REQ TYPE 1	Sent in the next but one paging

subblock. Page mode

is arbitrarily chosen

Mobile Ident : TMSI of the MS.

3	MS -> SS	CHANNEL REQUEST

4	MS -> SS	CHANNEL REQUEST

5	SS -> MS	IMM ASSIGN REJ	Wait indication = 0 seconds

6	SS -> MS	IMM ASSIGNMENT	Sent in the paging subblock of

MS under test. Page mode =

"extended paging", Request

reference chosen arbitrarily by

the SS, but different from all

references used earlier in this

test sequence

7	SS -> MS	PAG REQ TYPE 1	Sent in the next but one paging

subblock. Page mode is arbitrarily

chosen. Mobile Ident : TMSI of

the MS.

8	MS -> SS	CHANNEL REQUEST

9	MS -> SS	CHANNEL REQUEST

10	SS -> MS	IMM ASSIGN REJ

Specific Message Contents:

None.

26.6.2.2.1	Paging / reorganisation / procedure 1

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.2.3 in phase 1 version.

-	the paging parameters are chosen   arbitrarily by the SS

-	type of identity in the first PAGING REQUEST TYPE 1 is no longer specified as IMSI.]

(* half of this test can probably be deleted -- however as these are the 2nd and 4th quarters it would disrupt the SS !!! *)

Purpose:

To test that the MS correctly determines its new paging subchannel when the CCCH structure is changed from non-combined to combined.

Conformance Requirement:

1.	An MS, after reception of a message with page mode set to "paging reorganisation", shall answer to paging messages (with page mode set to "normal paging") sent on its old CCCH in paging blocks which do not belong to the MS's paging sub-channel.

2.	When the network changes the paging group of the MS by modifying BCCH parameters (CCCH-CONF set to "1 basic physical channel used for CCCH combined with SDCCH", and BS-AG-BLKS-RES set to "2 blocks reserved for access grant"), the MS shall calculate its new paging group and answer to paging messages on its new paging subchannel.

3.	When the network changes the paging group of the MS by modifying BCCH parameters (CCCH-CONF set to "2 basic physical channels used for CCCH, not combined with SDCCHs" and BS-AG-BLKS-RES set to "2 blocks reserved for access grant"), the MS shall calculate its new paging group and answer to paging messages on its new paging subchannel.

Reference(s):

GSM 04.08 section 3.3.2.1; GSM 05.02, section 6.5.

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS, with the exceptions that the CCCH-CONF shall not be set to combined and that BS-PA-MFRMS shall not be set to 9..

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated. The IMSI of the MS is from a defined/default range that ensures its paging channel changes when the broadcast parameters are changed.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The SS sends an IMMEDIATE ASSIGNMENT EXTENDED message on the MS's paging subchannel, with the page mode element set to "paging reorganisation" and Request References that do not pertain to the MS. Before the MS's original paging subchannel re-occurs, the SS pages it on the CCCH corresponding to the Mobile Station's IMSI with a PAGING REQUEST TYPE 2 message (page mode = normal paging) containing the MS's TMSI in some paging block which does not belong to the Mobile Station's paging subchannel. The MS shall respond by sending CHANNEL REQUESTs.The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection) on an arbitrarily selected paging subchannel.

Then the SS starts sending messages with page mode set to "paging reorganization" on all paging subchannels.

The SS pages the MS with its TMSI on an arbitrarily selected paging subchannel. The MS shall respond by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection).

The SS changes the paging parameters.

Then the SS sets the page mode to "normal paging".

The SS then waits for the duration of five 51-TDMA multiframes (4 to allow the MS to read all the system information type 1, 2, 3, and 4 messages on the BCCH, and one to calculate the new paging group). Then the MS is paged with a PAGING REQUEST TYPE 1 on its new paging subchannel. The MS shall respond by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection).

Then the MS is paged with a PAGING REQUEST TYPE 2 on its new paging subchannel. The MS shall respond by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection).

Maximum Duration of Test:

10 sec

Expected Sequence:

This sequence is performed for execution counter, K =1, 2.

Step	Direction	Message	Comments

1	SS -> MS	IMM ASSIGN EXT	Sent on the MS's paging

channel. Page mode set to "paging

reorganisation". Request Reference

not pertaining to the MS.

2	SS -> MS	PAG REQ TYPE 2	Sent before the MS's original

paging subchannel re-occurs, but

later than the next paging

block of that CCCH.

3	MS -> SS	CHANNEL REQUEST

4	MS -> SS	CHANNEL REQUEST

5	SS -> MS	IMM ASSIGN REJ	Wait indication = 0 seconds.

6	--------	--------	All L3 messages sent on any paging

subchannel are paging fill

frames specify "paging re

organization.

7	SS -> MS	PAG REQ TYPE 2	Sent on an arbitrarily

selected paging subchannel

Page mode "paging reorganisation"

8	MS -> SS	CHANNEL REQUEST

9	MS -> SS	CHANNEL REQUEST

10	SS -> MS	IMM ASSIGN REJ	Wait indication = 0 seconds.

11			Change of paging

parameters in SYS INFO 3 and 4 as

described below for K=1, 2

12			The SS waits until it has

sent all system information

messages (page mode is still

paging reorganisation).

13	--------	--------	All L3 messages sent on any paging

subchannel specify "normal paging

14	--------	--------	Wait 3 seconds

15	SS -> MS	PAG REQ TYPE 1	Sent on the new paging subchannel							of the MS.

16	MS -> SS	CHANNEL REQUEST

17	MS -> SS	CHANNEL REQUEST

18	SS -> MS	IMM ASSIGN REJ	Wait indication = 0 seconds.

19	SS -> MS	PAG REQ TYPE 2	Sent on the new paging subchannel					of the MS.

20	MS -> SS	CHANNEL REQUEST

21	MS -> SS	CHANNEL REQUEST

22	SS -> MS	IMM ASSIGN REJ

Specific Message Contents:

For execution counter K :

K=1:

SYSTEM INFORMATION TYPEs 3 and 4 shall have the Control Channel Description IE changed to:

CCCH-CONF	"1 basic physical channel used for CCCH, combined with SDCCHs"

BS-AG-BLKS-RES	2

BS-PA-MFRMS	9

K=2:

SYSTEM INFORMATION TYPEs 3 and 4 shall have the Control Channel Description IE changed to:

CCCH-CONF	"2 basic physical channel used for CCCH, not combined with SDCCHs"

BS-AG-BLKS-RES	2

BS-PA-MFRMS	9

26.6.2.2.2	Paging / reorganisation / procedure 2

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.2.3 in phase 1 version.

-	It is stated clearly that the paging parameters are chosen randomly by the SS.]

Purpose:

To test that the MS is operating in the "paging reorganisation" page mode when this is ordered by the SS and the MS is paged in its former access grant channel.

Conformance requirement

An MS, after reception of a message with page mode set to "paging reorganisation", shall answer to paging messages (with page mode set to "normal paging") sent in a former Access Grant block.

Reference(s):

GSM 04.08 section 3.3.2.1.

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

1 cell, Max-Retrans = 1, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated. The IMSI of the MS is from a defined\default range that ensures its paging channel changes when the broadcast parameters are changed.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The SS sends an IMMEDIATE ASSIGNMENT EXTENDED on the MS's paging subchannel, with the page mode element set to "paging reorganisation". The MS is then paged immediately in a former Access Grant block with a PAGING REQUEST TYPE 2 message. The MS shall respond by sending CHANNEL REQUESTs.The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection).

Maximum Duration of Test:

5 sec

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	IMM ASSIGN EXT	Page mode set to "paging

reorganisation"

2	SS -> MS	PAG REQ TYPE 2	Sent in a former access grant

block.

3	MS -> SS	CHANNEL REQUEST

4	MS -> SS	CHANNEL REQUEST

5	SS -> MS	IMM ASSIGN REJ

Specific Message Contents:

None.

26.6.2.3	Paging / same as before

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	the paging parameters are chosen randomly by the SS.]

Purpose:

To test that the MS remembers the page mode from the previous paging request message.

Conformance Requirement:

An MS, after first receiving a message with page mode set to "extended paging" and then another message with page mode set to "same as before", shall remember the page mode from the previous message and answer to paging messages in the next but one paging sub block.

Reference(s):

GSM 04.08 section 3.3.2.1;  GSM 05.02, section 6.5.

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

1 cell, Max-Retrans = 2, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily by the SS.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The SS sends an IMMEDIATE ASSIGNMENT REJECT on the MS's paging subchannel, with the page mode element set to "extended paging". In the next but one subblock on the same CCCH, nothing addresses the MS. When the MS's specific paging subchannel reoccurs, a PAGING REQUEST TYPE 3 is sent, not addressing the MS under test and with page mode set to "same as before". In the next but one subblock on the same CCCH the SS sends a PAGING REQUEST TYPE 1 message specifying paging reorganisation and addressing the MS. The MS shall respond by sending CHANNEL REQUESTs. The SS responds to the second CHANNEL REQUEST with an IMMEDIATE ASSIGNMENT REJECT (in order to avoid a cell reselection).

Maximum Duration of Test:

10 sec

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	IMM ASSIGN REJ	Page mode set to "extended paging"

2	SS -> MS	XXXX	In the next but one subblock

nothing addresses the MS

3	SS -> MS	PAG REQ TYPE 3	This is sent in the next paging

subblock on the MS's specific

paging subchannel. The page mode

is set to "same as before", and the MS

under test is not addressed.

4	SS -> MS	PAG REQ TYPE 1	The MS is addressed in this 'next

but one subblock'. Page mode set

to "paging reorganisation"

5	MS -> SS	CHANNEL REQUEST

6	MS -> SS	CHANNEL REQUEST

7	SS -> MS	IMM ASSIGN REJ

Specific Message Contents:

None.

26.6.2.4	Paging / multislot CCCH

26.6.2.4.1	Purpose:

1)	To test that the MS is able to determine its CCCH group and paging group correctly in the case of a CCCH configuration on more than one timeslot when it is paged on a timeslot other than 0. The MS is addressed with a PAGING REQUEST TYPE 1 message when the page mode is set to normal paging. The MS is paged with its IMSI in the 1st Mobile Identity field, the optional Mobile Identity field being not present, is the only way of addressing tested.

2)	To test that in such conditions the MS answers to the paging message on the timeslot on which the paging message was sent.

26.6.2.4.2	Conformance Requirement:

The MS shall respond correctly to a PAGING REQUEST TYPE 1 message, when the page mode is set to normal paging, when a multislot CCCH is used and the MS is addressed with its IMSI in the first Mobile Identity field, the optional Mobile Identity field being not present.

26.6.2.4.3	Reference(s):

GSM 04.08 section 3.3.2, GSM 05.02 section 6.5.

26.6.2.4.4	Related PICS/PIXIT Statement(s):

None.

26.6.2.4.5	Initial Condition(s):

System Simulator:

1 cell, a legal combination of CCCH-CONF, BS-AG-BLKS-RES and BS-PA-MFRMS is chosen arbitrarily under the following constraint :

CCCH-CONF is in the set :

-	2 basic physical channels used for CCCH, not combined with SDCCHs

-	3 basic physical channels used for CCCH, not combined with SDCCHs

-	4 basic physical channels used for CCCH, not combined with SDCCHs

Mobile Station :

The IMSI last 3 digits are so that the CCCH-GROUP of the MS under test is other than 0. According to section 6.5.2 of recommendation 05.02, this means that :

(IMSI mod 1000) mod (BS_CC_CHANS X N) is greater or equal to N, where N = BS_PA_MFRMS X (9 - BS_AG_BLKS_RES).

The MS is in the "idle, updated" state.

26.6.2.4.6	Foreseen Final State of the MS:

"Idle updated".

26.6.2.4.7	Test procedure:

The SS pages the MS once with a PAGING REQUEST TYPE 1 message on the timeslot and paging subchannel which correspond to the MS's IMSI.

The MS shall send the CHANNEL REQUEST on the same timeslot as the paging message.

The SS sends an IMMEDIATE ASSIGNMENT on the same timeslot as the paging message.

Maximum Duration of Test:

10 s

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1	1st Mobile Ident contains IMSI of

MS, 2nd Mobile Ident not present.

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging".

on the same timeslot as the paging

message

3	SS -> MS	IMM ASSIGNMENT	on the same timeslot as the paging

message

4	MS -> SS	PAGING RESPONSE	Mobile Ident : IMSI.

5	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

None.

26.6.3	Test of measurement report

When an RR-connection exists, the MS shall send measurement reports. These reports contain reception characteristics from serving and neighbouring cells. The measurement report procedure is described in section 3.4.1.2 of 04.08.

Note:	The capability to calculate RxLev and RxQual is tested in sections 15 and 16. In this test only the signalling aspect is verified.

[ previous Editor's note : In phase 2, the MS shall be provoked to return the "Neighbour cell information not available" value in the NO-NCELL-M field. It should also be considered whether to use different RF-levels and ARFCN's in the 4 tests]

[editorial note : this section has not been corrected since the last signalling meeting ]

26.6.3.1	Measurement / no neighbours

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.3.3 in phase 1 version

-	one test has been splitted into 4]

Purpose:

To test that, when the SS gives absolutely no information about neighbouring cells, the MS does not report on neighbouring cells.

Conformance Requirement:

The MS shall continuously send MEASUREMENT REPORT messages on every SACCH block and the measurement valid indication shall be set to valid (0) within the second block at the latest.

Reference(s):

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

Related PICS/PIXIT Statement(s):

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

8 cells with the following settings:

Transmitter		Level	NCC	BCC	ARFCN	Cell Identity

Serving, S1		-60	1	3	002	0001H

Neighbour, N1	-85	1	5	008	0002H

Neighbour, N2	-80	1	7	014	0003H

Neighbour, N3	-75	1	1	020	0004H

Neighbour, N4	-55	1	3	026	0005H

Neighbour, N5	-50	1	5	032	0006H

Neighbour, N6	-45	1	7	038	0007H

Neighbour, N7	-40	1	1	044	0008H

With the exception of the Cell Allocation, the rest of the parameters for all eight cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

[ Editor's note : the default parameters of all 8 cells shall be defined somewhere]

Mobile Station:

The MS is in the active state of a call (U10).

[Editor's note : is this the same as "a call in progress" ?]

Foreseen Final State of the MS:

Active state of a call (U10).

Method of Test:

With the MS having a call in progress, the SS sends SYSTEM INFORMATION TYPE 5 & 6 on the SACCH. The BA is indicated as empty. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that no measurement results have been obtained.

Maximum Duration of Test:

-

Expected Sequence:

Since SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT are sent continously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents:

SYSTEM INFORMATION TYPE 5:

Information Element				value/remark

Neighbour Cells Description

Format Identifier			bit map 0

BCCH Allocation Sequence number	1

BCCH Allocation ARFCN			No channels belong to the BCCH

allocation

MEASUREMENT REPORT:

Information Element				value/remark

Protocol Discriminator				RR Management

Transaction Identifier				0000

Message Type				MEASUREMENT REPORT

Measurement Results

BA-used				1

DTX-used				DTX was not used

RXLEV-FULL-SERVING-CELL		See note 1

RXLEV-SUB-SERVING-CELL		See note 1

MEAS-VALID				See note 2

RXQUAL-FULL-SERVING-CELL		See note 1

RXQUAL-SUB-SERVING-CELL		See note 1

NO-NCELL-M				No neighbour cell measurement result, or

Neighbour cell information not available

for serving cell

RXLEV-NCELL-1			00 0000

BCCH-FREQ-NCELL-1			 0 0000

BSIC-NCELL-1				00 0000

.     .     .				.       .

.     .     .				.       .

RXLEV-NCELL-6			00 0000

BCCH-FREQ-NCELL-6			 0 0000

BSIC-NCELL-6				00 0000

[Editor's note : is the term "No result" unambigous ? - now coded as sent]

Note 1:	The actual values are not checked.

Note 2:	The Measurement Valid Indication shall be set to valid within the second SACCH block at the latest.

[ Editor's note : this timing requirement is not applicable, since it is assumed that the initial conditions are stable -- once the MS is in U10 then this can be checked immediately.]

26.6.3.2	Measurement / all neighbours present

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.3.3 in phase 1 version

-	one test has been splitted into 4]

Purpose:

To test that, when the SS gives information about neighbouring cells, the MS reports appropriate results.

Conformance Requirement:

The MS shall continuously send MEASUREMENT REPORT messages on every SACCH blocks and the measurement valid indication shall be set to valid (0) within the second block at the latest. After 20 seconds the order of values in the MEASUREMENT REPORT message shall contain measurement results for the 6 strongest BCCH carriers among those monitored by the MS.

Reference(s):

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

8 cells with the following settings:

Transmitter		Level	NCC	BSCC	ARFCN	Cell identity

Serving, S1		-60	1	3	002	0001H

Neighbour, N1	-85	1	5	008	0002H

Neighbour, N2	-80	1	7	014	0003H

Neighbour, N3	-75	1	1	020	0004H

Neighbour, N4	-55	1	3	026	0005H

Neighbour, N5	-50	1	5	032	0006H

Neighbour, N6	-45	1	7	038	0007H

Neighbour, N7	-40	1	1	044	0008H

With the exception of the Cell Allocation, the rest of the parameters for all eight cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

[ Editor's note : the default parameters of all 8 cells shall be defined somewhere]

Mobile Station:

The MS is in the active state of a call (U10).

Foreseen Final State of the MS:

Active state of a call (U10).

Method of Test:

With the MS having a call in progress, the SS sends SYSTEM INFORMATION TYPE 5 & 6 on the SACCH. All 8 of the BCCHs "on air" are indicated in the BA. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that measurement results for the 6 strongest carriers have been obtained.

Maximum Duration of Test:

-

Expected Sequence:

Since SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT are sent continously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents:

SYSTEM INFORMATION TYPE 5:

Information Element				value/remark

Neighbour Cells Description

Format Identifier			bit map 0

BCCH Allocation Sequence		1

BCCH Allocation ARFCN			Only channel numbers 2, 8, 14, 20, 26, 32,

38, and 44 belong to the BCCH allocation.

MEASUREMENT REPORT:

Information Element				value/remark

Protocol Discriminator				RR Management

Transaction Identifier				0000

Message Type				MEASUREMENT REPORT

Measurement Results

BA-used				1

DTX-used				DTX was not used

RXLEV-FULL-SERVING-CELL		See note 1

RXLEV-SUB-SERVING-CELL		See note 1

MEAS-VALID				See note 2

RXQUAL-FULL-SERVING-CELL		See note 1

RXQUAL-SUB-SERVING-CELL		See note 1

NO-NCELL-M				6 neighbour cell measurement results

RXLEV-NCELL-1			See note 1

BCCH-FREQ-NCELL-1			Shall not correspond to N1 or N2

BSIC-NCELL-1				Corresponds to that of BCCH-FREQ-NCELL-1

RXLEV-NCELL-2			See note 1

BCCH-FREQ-NCELL-2			Shall not correspond to N1 or N2

BSIC-NCELL-2				Corresponds to that of BCCH-FREQ-NCELL-2

RXLEV-NCELL-3			See note 1

BCCH-FREQ-NCELL-3			Shall not correspond to N1 or N2

BSIC-NCELL-3				Corresponds to that of BCCH-FREQ-NCELL-3

RXLEV-NCELL-4			See note 1

BCCH-FREQ-NCELL-4			Shall not correspond to N1 or N2

BSIC-NCELL-4				Corresponds to that of BCCH-FREQ-NCELL-4

RXLEV-NCELL-5			See note 1

BCCH-FREQ-NCELL-5			Shall not correspond to N1 or N2

BSIC-NCELL-5				Corresponds to that of BCCH-FREQ-NCELL-5

RXLEV-NCELL-6			See note 1

BCCH-FREQ-NCELL-6			Shall not correspond to N1 or N2

BSIC-NCELL-6				Corresponds to that of BCCH-FREQ-NCELL-6

Note 1:	These actual values are not checked.

Note 2:	The Measurement Valid Indication shall be set to valid within the second SACCH block at the latest.

[ Editor's note : this timing requirement is not applicable, since it is assumed that the initial conditions are stable.]

26.6.3.3	Measurement / barred cells and non-permitted NCCs

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	one test has been splitted into 4]

Purpose:

To test that, when a combination of normal neighbours, barred cells and non-permitted NCCs is "on air", the MS reports only on normal neighbours.

Conformance Requirement:

The MS shall continuously send MEASUREMENT REPORTs on every SACCH blocks and the measurement valid indication shall be set to valid (0) within the second block at the latest. After 20 seconds the messages shall contain measurement results only for the 4 BCCH carriers on which the MS is allowed to report.

Reference(s):

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

8 cells with the following settings:

Transmitter		Level	NCC	BCC	ARFCN	Cell identity

Serving, S1		-60	1	3	002	0001H

Neighbour, N1	-85	1	5	008	0002H

Neighbour, N2	-80	1	7	014	0003H

Neighbour, N3	-75	2	1	020	0004H

Neighbour, N4	-55	3	3	026	0005H

Neighbour, N5	-50	4	5	032	0006H

Neighbour, N6	-45	1	7	038	0007H

Neighbour, N7	-40	1	1	044	0008H

With the exception of the Cell Allocation, the rest of the parameters for all eight cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

[ Editor's note : the default parameters of all 8 cells shall be defined somewhere]

Note:	The BA sent in SYSTEM INFORMATION TYPE 5 does not include N1, N4 and N5. N1 may be the case of a barred cell, N3 simulates the case where another operator is transmitting on the same frequency (e.g. in border areas), N4 & N5 simulate the case where other operators are transmitting on other frequencies.

Mobile Station:

The MS is in the active state of a call (U10).

Foreseen Final State of the MS:

Active state of a call (U10).

Method of Test:

With the MS having a call in progress, the SS sends SYSTEM INFORMATION TYPE 5 & 6 on the SACCH. 5 of the 8 BCCHs "on air" are indicated in the BA. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that measurement results for the 4 strongest permitted carriers have been obtained (one of the carriers in the BA belongs to a non-permitted NCC).

Maximum Duration of Test:

-

Expected Sequence:

Since SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT messages are sent continously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents:

SYSTEM INFORMATION TYPE 5:

Information Element				value/remark

Neighbour Cells Description

Format Identifier			bit map 0

BCCH Allocation Sequence		1

BCCH Allocation ARFCN			only channel numbers 2, 14, 20, 38, and

44 belong to the BCCH allocation.

MEASUREMENT REPORT:

Information Element				value/remark

Protocol Discriminator				RR Management

Transaction Identifier				0000

Message Type				MEASUREMENT REPORT

Measurement Results

BA-used				1

DTX-used				DTX was not used

RXLEV-FULL-SERVING-CELL		See note 1

RXLEV-SUB-SERVING-CELL		See note 1

MEAS-VALID				See note 2

RXQUAL-FULL-SERVING-CELL		See note 1

RXQUAL-SUB-SERVING-CELL		See note 1

NO-NCELL-M				4 neighbour cell measurement results

RXLEV-NCELL-1			See note 1

BCCH-FREQ-NCELL-1			Corresponds to one of N7, N6, S1 or N2

BSIC-NCELL-1				Corresponds to that of BCCH-FREQ-NCELL-1

RXLEV-NCELL-2			See note 1

BCCH-FREQ-NCELL-2			Corresponds to one of N7, N6, S1 or N2

BSIC-NCELL-2				Corresponds to that of BCCH-FREQ-NCELL-2

RXLEV-NCELL-3			See note 1

BCCH-FREQ-NCELL-3			Corresponds to one of N7, N6, S1 or N2

BSIC-NCELL-3				Corresponds to that of BCCH-FREQ-NCELL-3

RXLEV-NCELL-4			See note 1

BCCH-FREQ-NCELL-4			Corresponds to one of N7, N6, S1 or N2

BSIC-NCELL-4				Corresponds to that of BCCH-FREQ-NCELL-4

RXLEV-NCELL-5			00 0000

BCCH-FREQ-NCELL-5			 0 0000

BSIC-NCELL-5				00 0000

RXLEV-NCELL-6			00 0000

BCCH-FREQ-NCELL-6			 0 0000

BSIC-NCELL-6				00 0000

Note 1:	These actual values are not checked.

Note 2:	The Measurement Valid Indication shall be set to valid within the second SACCH block at the latest.

[ Editor's note : this timing requirement is not applicable, since it is assumed that the initial conditions are stable.]

26.6.3.4	Measurement / DTX

[ dissimilarities to 11.10 phase 1 - editorial notes :

-	this section was numbered 26.6.3.3 in phase 1 version

-	one test have been splitted into 4

-	Handover is no more specified]

Purpose:

To test that, in the case of the MS using DTX and the SS indicating that power control is in use, the MS reports appropriate results.

Conformance Requirement:

After the sending of the HANDOVER COMPLETE, the MS shall continuously send measurement reports in every SACCH blocks, the measurement valid indication shall be set to valid (0) within the second block at the latest. After 20 seconds the order of values in the MEASUREMENT REPORT message shall contain measurement results for the 6 strongest BCCH carriers among those monitored by the MS. Further, in a quiet environment, the DTX-USED field shall be set by the MS to "DTX used".

Reference(s):

GSM 04.08 section 3.4.1.2, GSM 05.08 section 8.4.

Related PICS/PIXIT Statement(s):

Support for state U10 of the Call Control protocol.

Support for transparent data services only: yes/no.

Initial Condition(s):

System Simulator:

8 cells with the following settings:

Transmitter		Level	NCC	BCC	ARFCN	Cell identity

Serving, S1		-60	1	3	002	0001H

Neighbour, N1	-85	1	5	008	0002H

Neighbour, N2	-80	1	7	014	0003H

Neighbour, N3	-75	1	1	020	0004H

Neighbour, N4	-55	1	3	026	0005H

Neighbour, N5	-50	1	5	032	0006H

Neighbour, N6	-45	1	7	038	0007H

Neighbour, N7	-40	1	1	044	0008H

In the serving cell, the DTX indicator is set to "MS shall use discontinous transmission"

With the exception of the Cell Allocation, the rest of the parameters for all eight cells are the same as the default settings and default SYSTEM INFORMATION TYPE 1 to 4 message contents for cell A. The Cell Allocation for the serving cell is the same as the default setting for cell A. The Cell Allocations for the neighbour cells need have only one entry, consisting of the ARFCN of that cell's BCCH.

[ Editor's note : the default parameters of all 8 cells shall be defined somewhere]

Mobile Station:

The MS is in the active state of a call (U10).

The MS has just completed a handover into the serving cell, S1.

Foreseen Final State of the MS:

Active state of a call (U10).

Method of Test:

With the MS having a call in progress on an arbitrary cell, the MS is handed over to cell S1. On cell S1, the SS sends SYSTEM INFORMATION TYPE 5 & 6 on the SACCH with all 8 of the BCCHs "on air" indicated in the BA. Cell S1 also indicates that DTX shall be used. The MS shall send MEASUREMENT REPORTs back to the SS, and it shall be indicated in these that measurement results for the 6 strongest carriers have been obtained and that DTX has been used. (The MS is positioned in an environment free from acoustic noise.)

Maximum Duration of Test:

-

Expected Sequence:

Since SYSTEM INFORMATION TYPE 5, SYSTEM INFORMATION TYPE 6 and MEASUREMENT REPORT messages are sent continously, a table is not applicable in this test. The interval between 2 successive Layer 2 frames containing MEASUREMENT REPORTs shall not exceed one Layer 2 frame.

Specific Message Contents:

SYSTEM INFORMATION TYPE 5:

Neighbour Cells Description

- Format Identifier	bit map 0

- BCCH Allocation Sequence	1

- BCCH Allocation ARFCN	only channel numbers 2, 8, 14, 20, 26, 32,

38, and 44 belong to the BCCH allocation.

SYSTEM INFORMATION TYPE 6:

Cell Options

- Power Control Indicator	Power Control Indicator is set

- DTX Indicator		MS shall use DTX

- Radio-Link-Timeout	8

MEASUREMENT REPORT:

Protocol Discriminator		RR Management

Transaction Identifier		0000

Message Type		MEASUREMENT REPORT

Measurement Results

- BA-used		0

- DTX-used		DTX was used (note 4)

- RXLEV-FULL-SERVING-CELL	See note 1

- RXLEV-SUB-SERVING-CELL	See note 1

- MEAS-VALID		See note 2

- RXQUAL-FULL-SERVING-CELL	See note 1

- RXQUAL-SUB-SERVING-CELL	See note 1

- NO-NCELL-M		6 neighbour cell measurement results

- RXLEV-NCELL-1		See note 1

- BCCH-FREQ-NCELL-1	Shall not correspond to N1 or N2

- BSIC-NCELL-1		Corresponds to that of BCCH-FREQ-NCELL-1

- RXLEV-NCELL-2		See note 1

- BCCH-FREQ-NCELL-2	Shall not correspond to N1 or N2

- BSIC-NCELL-2		Corresponds to that of BCCH-FREQ-NCELL-2

- RXLEV-NCELL-3		See note 1

- BCCH-FREQ-NCELL-3	Shall not correspond to N1 or N2

- BSIC-NCELL-3		Corresponds to that of BCCH-FREQ-NCELL-3

- RXLEV-NCELL-4		See note 1

- BCCH-FREQ-NCELL-4	Shall not correspond to N1 or N2

- BSIC-NCELL-4		Corresponds to that of BCCH-FREQ-NCELL-4

- RXLEV-NCELL-5		See note 1

- BCCH-FREQ-NCELL-5	Shall not correspond to N1 or N2

- BSIC-NCELL-5		Corresponds to that of BCCH-FREQ_NCELL-5

- RXLEV-NCELL-6		See note 1

- BCCH-FREQ-NCELL-6	Shall not correspond to N1 or N2

- BSIC-NCELL-6		Corresponds to that of BCCH-FREQ-NCELL-6

Note 1:	These actual values are not checked.

Note 2:	The Measurement Valid Indication shall be set to valid within the second SACCH block after the HANDOVER COMPLETE message at the latest.

[ Editor's note : this timing requirement is not applicable, since it is assumed that the initial conditions are stable.]

Note 3:	For an MS that only supports transparent data services, the value of DTX-used shall not be checked.

26.6.4	Test of the channel assignment procedure

An intracell change of channel can be requested by upper layers in order to change the channel type, or it may be initiated by the RR-sublayer, e.g. for an intra cell handover. This change is performed using the channel assignment procedure. If the procedure is incorrectly implemented in the MS, the establishment and maintenance of connections is endangered. This applies for the succesful case and for the assignment failure: the MS's correct return to the old channel after assignment failure is a necessary part of the GSM system design.

[Dissimilarities to 11.10 phase 1 - editorial notes :

- The phase 2 CR from France Telecom is not yet included.??

- The succesful procedure is not tested for SDCCH-only MSes.

- Mobile allocation needs further description; was the cell allocation present in SYSTEM INFO? (for 6.4.1)

]

26.6.4.1	Dedicated assignment / Successful case

[Dissimilarities to 11.10 phase 1 - editorial notes :

- This section was part of II.5.3.6.4.2 in phase 1 version; for non-halfrate MSes the test was the same with the exceptions noted below; for F+H-MSes, the test is different, because the TCH/H cases are tested at the end, not in the middle, and both hopping and non-hopping are checked.

-	N(SD) is used instead of V(SD)

- In phase 1 it was required that the MS uses the commanded power level on the assigned channels; in phase 2 it is checked that the MS reports that level.

- A postamble has been added.

]

This test is only applicable to an MS supporting a TCH.

Purpose:

1.	To verify that upon receipt of an ASSIGNMENT COMMAND, the MS switches to the channel defined in the ASSIGNMENT COMMAND, establishes the link and sends an ASSIGNMENT COMPLETE message. This is tested for an MS supporting TCH in the special cases of a transition

1.1	from non-hopping SDCCH to hopping TCH/F using a different timeslot

1.2	from hopping TCH/F to non-hopping TCH/F using a different timeslot

1.3	from non-hopping TCH/F to non-hopping TCH/F using a different timeslot

1.4	from non-hopping TCH/F to hopping TCH/H using a different timeslot; this test purpose is only applicable if the MS supports TCH/H

1.5	from hopping TCH/H to non-hopping TCH/H using a different timeslot; this test purpose is only applicable if the MS supports TCH/H

1.6	from non-hopping TCH/H to hopping TCH/F using a different timeslot; this test purpose is only applicable if the MS supports TCH/H.

2.	To verify that an MS supporting TCH, having sent an MM- or CM message which was not acknowledged on L2 before the channel assignment procedure was initiated and before the MS has left the old channel, repeats that message after completion of the assignment procedure without incrementing N(SD). This is tested in the special case of MM message AUTHENICATION RESPONSE.

3.	To verify that, if an MS supporting TCH has received an ASSIGNMENT COMMAND message which contains only the description of a channel to be used after the starting time, and if the starting time has not already elapsed, the mobile station shall wait up to the starting time before accessing the channel.

4.	To verify that an MS supporting TCH, having received an ASSIGNMENT COMMAND, having sent an SABM frame to establish the main signalling link on the assigned channel, reports the power level specified in the ASSIGNMENT COMMAND message, in the uplink SACCH L1 header of the SACCH message sent in the SACCH period following the transmission of the SABM frame.

Conformance Requirement:

1.	Upon receipt of the ASSIGNMENT COMMAND message, the mobile station initiates a local end release of link layer connections, disconnects the physical channels, commands the switching to the assigned channels and initiates the establishment of lower layer connections (this includes the activation of the channels, their connection and the establishment of the main signalling links).

2.	MM-messages and CM-messages using SAPI=0 sent from the mobile station to the network can be duplicated by the data link layer in the following case:

a channel change of dedicated channels is required (assignment or handover procedure) and the last layer 2 frame has not been acknowledged by the peer data link layer before the mobile station leaves the old channel.

In this case, the mobile station does not know whether the network has received the message correctly. Therefore, the mobile station has to send the message again after the new dedicated channel is established.

3.	An ASSIGNMENT COMMAND message may indicate a frequency change in progress, with a starting time and possibly alternative channel descriptions.

In the case of the reception of an ASSIGNMENT COMMAND message which contains only the description of a channel to be used after the starting time, and if the starting time has not already elapsed,the mobile station shall wait up to the starting time before accessing the channel.

4.	The MS shall establish the link with the power level specified in the ASSIGNMENT COMMAND message.

The MS shall confirm the power control level that it is currently employing in the uplink SACCH L1 header. The indicated value shall be the power control level actually used by the mobile for the last burst of the previous SACCH period.

Reference(s):

1, 3:	GSM 04.08, subclause 3.4.3.

2:	GSM 04.08, subclause 3.1.4.3.

4:	GSM 04.08, subclause 3.4.3; GSM 05.08, subclause 4.2.

Related PICS/PIXIT Statement(s):

-	TCH supported (Y/N)

-	Supported rate(s) of TCH: (F/F+H).

-	The supported channel mode(s) need to be declared.

-	(*The frequency band that is supported needs to be supplied *)

Initial Condition(s):

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The SS pages the MS and allocates an SDCCH. Then 2 different channels are assigned with ASSIGNMENT COMMANDs. Each time the MS shall switch to the assigned channel, establish the link and send an ASSIGNMENT COMPLETE message.

Then the SS sends a AUTHENTICATION REQUEST message. The MS shall answer with an AUTHENTICATION RESPONSE message, which is not acknowledged on L2 by the SS. Immediately after the AUTHENTICATION RESPONSE message is received, the SS sends an ASSIGNMENT COMMAND. The MS shall switch to the assigned channel, establish the link with the commanded power level and send as ASSIGNMENT COMPLETE message. Then MS shall repeat the AUTHENTICATION RESPONSE message, with the same N(SD) value.

Then the SS sends an ASSIGNMENT COMMAND, which includes a Starting Time IE. The MS shall react as specified above, but this shall be done at the time specified in Starting Time IE.

For an MS not supporting TCH/H, the SS initiates the channel release procedure and the test ends here. For an MS supporting TCH/H, the channel assignment procedure is performed another three times, with half rate channels involved, and again it is checked that the MS correctly completes the procedures, before the SS initiates the channel release procedure.

Maximum Duration of Test:

30 s

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGN	Channel Type = SDCCH/8, FH is not

used, Timeslot Number = n.

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/F, FH is used,

Timeslot Number = (n+1) mod 8

6	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel after

establishment of the main signalling link.

7	SS		The SS checks that the MS

reports the requested power

level in the layer 1 header

of the SACCH message

that is sent in the first SACCH

muliframe following the SABM.

8	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/F, FH is not

used, Timeslot Number = (n+3)mod 8

9	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel after

establishment of the main signalling link.

10	SS -> MS	AUTHENT REQUEST

11	MS -> SS	AUTHENT RESPONSE	This message is not L2

acknowledged by the SS

12	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/F, FH is used,

Timeslot Number = (n+4) mod 8

13	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel after

establishment of the main signalling link.

14	MS -> SS	AUTHENT RESPONSE	N(SD) shall be the same as in

step 10

15	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/F, FH is not

used, Timeslot Number = (n+5) mod 8,

Starting Time = (current frame

number + 100 frames) mod 42432

16	MS -> SS	SABM	Sent on the assigned channel.

The frames containing the SABM shall

have frame numbers x with

0 <=  (x - starting time) mod 42432 <= 17

17	SS -> MS	UA

18	MS -> SS	ASSIGN COMPLETE

A			This test part is performed

if the MS doesn't support

TCH/H (see PICS/PIXIT)

A19	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

B			This test part is performed

if the MS supports

TCH/H (see PICS/PIXIT)

B19	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/H,

FH is used,

Timeslot Number = (n+6)mod 8

B20	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel

after establishment of the

main signalling link.

B21 	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/H,

FH is not used,

Timeslot Number = (n+7)mod 8

B22	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel

after establishment of the

main signalling link.

B23	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/F,

FH is used,

Timeslot Number = (n+1)mod 8

B24	MS -> SS	ASSIGN COMPLETE	Sent on the correct channel

after establishment of the

main signalling link.

B25	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

ASSIGNMENT COMMAND:

Channel Description

- Channel Type and TDMA offset	As specified in table

- Timeslot Number	As specified in table

- Training Sequence Code	Chosen randomly by the SS

- Hopping	As specified in table

- ARFCN	Within the range supported by the

MS

Power Command

- Power level	Chosen randomly by the SS but with a changed value each time.

Channel Mode

- Mode	Arbitrarily selected from the full

rate capabilities declared for the

MS

Mobile Allocation	If present, chosen randomly by the testhouse

Starting Time	Only included in step 15

26.6.4.2	Dedicated assignment / failure

26.6.4.2.1	Dedicated assignment / failure / failure during active state

This test is only applicable to an MS supporting the call control protocol.

[ dissimilarities to 11.10 phase 1 - editorial notes :

- For an MS supporting TCH/H, the test is repeated for TCH/H.

- Instead of checking the power level, it is checked which power level the MS specifies in the L1 header.

]

Purpose:

To test that, when the MS fails to seize the new channel, the MS reactivates the old channel, reporting use of the last power level used on the old channel.

This is tested in the special cases of a transition:

- from TCH/F to hopping TCH/F in state U10 if the MS supports TCH/F and call control

- from TCH/H to hopping TCH/H in state U10 if the MS supports TCH/H and call control

Conformance Requirement:

On the mobile station side, if a lower layer failure happens on the new channel before the ASSIGNMENT COMPLETE message has been sent, the mobile station deactivates the new channels, reactivates the old channels, reconnects the TCHs if any and triggers the establishment of the main signalling link. It then sends a ASSIGNMENT FAILURE message, cause "protocol error unspecified" on the main DCCH and resumes the normal operation, as if no assignment attempt had occurred. The operational parameters (e.g. ciphering mode) when returning on the old channel are those applied before the procedure.

Reference(s):

GSM 04.08 section 3.4.3.3.

Related PICS/PIXIT Statement(s):

- Support for TCH/F.

- Support for TCH/H.

- Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

1 cell, default parameters. The SS orders the MS to use a power level P.

Mobile Station:

The MS is in the active state (U10) of a mobile terminated call.

Foreseen Final State of the MS:

The active state (U10) of a mobile terminated call.

Method of Test:

The MS is in the active state (U10) of a mobile terminated call. The SS sends an ASSIGNMENT COMMAND allocating a new TCH/F, but does not activate the new channel. It is checked that the MS  triggers the establishment of the main signalling link on the old channel and then sends an ASSIGNMENT FAILURE.

Maximum Duration of Test:

30 s

Expected Sequence:

The test is repeated for execution counter k = 1, 2 if the MS supports TCH/H.

Step	Direction	Message	Comments

1	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/F, if k = 1,

Channel Type = TCH/H, if k = 2.

Power level specified in power command

is different to P.

The MS attempts (and fails) to establish a

signalling link on the new channel.

2			The MS reestablishes the signalling link on

the old channel.

3	MS -> SS	ASSIGN FAILURE	RR cause value ="protocol error unspecified"

4	SS		The SS checks that the MS

reports power level P

in the L1 header of the SACCH message

that is sent in the first SACCH

muliframe following the SABM.

Specific Message Contents:

None

26.6.4.2.2	Dedicated assignment / failure / general case

[ dissimilarities to 11.10 phase 1 - editorial notes :

New test.

]

Purpose:

To test that, when the MS fails to seize the new channel, the MS reactivates the old channel.

This is tested in the special cases of a transition:

-	from SDCCH to hopping TCH/F; this test part is only applicable if the MS supports TCH/F.

-	from non-hopping SDCCH to hopping TCH/H; this test part is only applicable if the MS supports TCH/H.

-	from hopping TCH/F to hopping TCH/H; this test part is only applicable if the MS supports TCH/H.

Note:	26.6.8.4 contains the case of an assignment failure SDCCH -> SDCCH.

Conformance Requirement:

On the mobile station side, if a lower layer failure happens on the new channel before the ASSIGNMENT COMPLETE message has been sent, the mobile station deactivates the new channels, reactivates the old channels, reconnects the TCHs if any and triggers the establishment of the main signalling link. It then sends a ASSIGNMENT FAILURE message, cause "protocol error unspecified" on the main DCCH and resumes the normal operation, as if no assignment attempt had occurred. The operational parameters (e.g. ciphering mode) when returning on the old channel are those applied before the procedure.

Reference(s):

GSM 04.08 section 3.4.3.3.

Related PICS/PIXIT Statement(s):

-	Support for TCH/F.

-	Support for TCH/H.

Initial Condition(s):

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is "idle updated".

Foreseen Final State of the MS:

The MS is "idle updated".

Method of Test:

A mobile terminated RR connection is established on an SDCCH. The SS sends an ASSIGNMENT COMMAND message allocating a hopping TCH/F, but does not activate the assigned channels. The MS shall try to activate the new channel (this is not verified) and shall then reactivate the old channel and trigger the establishment of the main signalling link on the old channel. Then the MS shall send an ASSIGNMENT FAILURE.

For an MS not supporting TCH/H, the SS initiates the channel release procedure and the test ends here. For an MS supporting TCH/H, the test sequence is repeated another two times, with half rate channels involved, and again it is checked that the MS correctly returns to the old channels, before the SS initiates the channel release procedure.

Maximum Duration of Test:

30 s

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMM ASSIGNMENT	Channel Type : SDCCH

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/F, hopping.

The MS attempts (and fails) to establish a

signalling link on the new channel.

6			The MS reestablishes the signalling link on

the old channel.

7	MS -> SS	ASSIGN FAILURE	RR cause value ="protocol error unspecified"

A			This test part is performed

if the MS does not support TCH/H

A8	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

B			This test part is performed

if the MS supports TCH/H.

B8	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/H, hopping.

The MS attempts (and fails) to establish a

signalling link on the new channel.

B9			The MS reestablishes the signalling link on

the old channel.

B10	MS -> SS	ASSIGN FAILURE	RR cause value ="protocol error unspecified"

B11	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/F, hopping.

B12	MS -> SS	ASSIGN COMPLETE	Sent on the assigned channel after

establishment of the main signalling link.

B13	SS -> MS	ASSIGN COMMAND	Channel Type = TCH/H, hopping.

The MS attempts (and fails) to establish a

signalling link on the new channel.

B14			The MS reestablishes the signalling link on

the old channel.

B15	MS -> SS	ASSIGN FAILURE	RR cause value ="protocol error unspecified"

B16	SS -> MS	CHANNEL RELEASE	The main signalling link is released.

Specific Message Contents:

None.

26.6.5	Test of handover

With the Handover procedure, it is possible to completely alter the channels allocated to a MS. This makes it possible in particular to switch a call in progress from one cell to another. The procedure is always initiated by the network, with the MS in a dedicated mode.

Sections 26.6.5.1 - 26.6.5.4 contain generic test procedures to be used for executing successfull Handover tests. Table 1 contains a list of the different combinations of parameters which have to be tested, together with a reference to the appropriate generic test procedure. If a test uses a channel rate which the MS under test does not support, the test shall be skipped. An MS supporting only full rate speech shall consequently pass [17] tests, while an MS supporting both full and half rate speech shall pass [22] tests.

Table 1

From	To	Timing	Start	Syn	State	Section

Adv.	Time	?	of call	26.

TCH/F, no FH	TCH/F, no FH	20	none	no	U10	6.5.1

TCH/F, no FH	TCH/F, no FH	50	none	no	U10	6.5.1

TCH/F, no FH	TCH/F, no FH	20	1.1s	no	U10	6.5.1

TCH/H, no FH	TCH/H, FH	20	none	no	U10	6.5.1

TCH/H, no FH	TCH/H, FH	50	none	no	U10	6.5.1

TCH/H, no FH	TCH/H, FH	20	1.1s	no	U10	6.5.1

SDCCH/8, no FH	SDCCH/8, FH	20	none	no	estab	6.5.2

SDCCH/8, no FH	SDCCH/8, FH	20	1.1s	no	estab	6.5.2

TCH/F, FH	TCH/F, FH	20	none	no	estab	6.5.2

TCH/F, FH	TCH/F, FH	20	1.1s	no	estab	6.5.2

SDCCH/8, FH	TCH/F, no FH	20	none	no	estab	6.5.2

SDCCH/8, FH	TCH/F, no FH	20	1.1s	no	estab	6.5.2

TCH/F, FH	TCH/F, no FH	2k+y	none	yes	U10	6.5.3

TCH/F, FH	TCH/F, no FH	2k+y	1.1s	yes	U10	6.5.3

TCH/H, FH	TCH/H, no FH	2k+y	none	yes	U10	6.5.3

TCH/H, FH	TCH/H, no FH	2k+y	1.1s	yes	U10	6.5.3

SDCCH/8, FH	SDCCH/8, FH	2k+y	none	yes	estab	6.5.4

SDCCH/8, FH	SDCCH/8, FH	2k+y	1.1s	yes	estab	6.5.4

TCH/F, no FH	TCH/F, FH	2k+y	none	yes	estab	6.5.4

TCH/F, no FH	TCH/F, FH	2k+y	1.1s	yes	estab	6.5.4

SDCCH/8, no FH	TCH/F, no FH	2k+y	none	yes	estab	6.5.4

SDCCH/8, no FH	TCH/F, no FH	2k+y	1.1s	yes	estab	6.5.4

Table 2

TCH/FS	TCH/HS	SDCCH

n	10-20	5-10	2-5

In addition to the successfull case of Handover, 2 unsuccessfull cases shall be tested. These tests are described in 26.6.5.8 and 26.6.5.9.

26.6.5.1	Handover / successful / active call / non-synchronized

Purpose:

To test that when the MS is ordered to make a non-synchronized handover it continuously sends access bursts on the main DCCH until it receives a PHYSICAL INFORMATION message from the SS. To test that the MS correctly handles the values of any Starting Time IE in the HANDOVER COMMAND message and the Timing Advance IE in the PHYSICAL INFORMATION message. To test that the MS activates the new channel correctly.

Conformance Requirement:

The MS shall correctly apply the handover procedure in the non-synchronized case when a call is in progress and when handover is performed from a TCH/F without frequency hopping towards a TCH/F without frequency hopping.

The MS shall correctly apply the handover procedure in the non-synchronized case when a call is in progress and when handover is performed from a TCH/H without frequency hopping to a TCH/H with frequency hopping. This does not apply to MSs not supporting TCH/H.

Reference(s):

GSM 04.08 section 3.4.4.

Related PICS/PIXIT Statement(s):

Supported rate(s) of TCH: TCH/FS and/or TCH/HS.

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

2 cells with same LAI, default parameters.

Mobile Station:

The MS is in the active state (U10) of a call on cell A.

[editorial note : since not all MSs support such high power levels, it might be better to choose e.g. power level 10]

Foreseen Final State of the MS:

The active state (U10) of a call on cell B.

Method of Test:

The MS is in the active state (U10) of a call on cell A. The SS sends a HANDOVER COMMAND on the main DCCH. The MS shall (at the time specified in the Starting Time information element, if included in the message) begin to send access bursts at the commanded power level on the new DCCH to cell B. The SS observes the access bursts and after receiving n (n being randomly drawn between values according to table 2 of section 26.6.5) access bursts, the SS sends PHYSICAL INFORMATION with a Timing Advance as specified in table 1 of section 26.6.5. The MS shall activate the channel in sending and receiving mode, and connect the channel in the range of (0..3) seconds after the transmission time of the PHYSICAL INFORMATION. It shall do so with the correct timing advance. Then the MS shall establish a signalling link and send a HANDOVER COMPLETE message. The SS shall receive this message within T3103 seconds after sending the HANDOVER COMMAND, but not before a UA frame has been sent by the SS.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	Channel description : as specified in

table 1 of section 26.6.5.

Power Command : 5. Synchronisation

Indication : not synchronised.

Cell Channel Description : appears

if hopping is used on target cell.

Starting Time : as specified in

table 1 of section 26.6.5.

(if no Starting Time is specified then

this IE is not included).

2	MS -> SS	HANDOVER ACCESS	Repeated until reception of PHYSICAL

INFORMATION. Handover Reference as

included in the HANDOVER COMMAND

3	SS -> MS	PHYSICAL INFO	Sent after reception of n HANDOVER

ACCESS. Timing Advance : as

specified in table 1 of section

26.6.5.

4	MS -> SS	SABM	Sent without information field

5	SS -> MS	UA

6	MS -> SS	HANDOVER COMPLETE

Specific Message Contents:

None

26.6.5.2	Handover / successful / cell under establishment / non-synchronized

Purpose:

To test that when the MS is ordered to make a non-synchronized handover, it continuously sends access bursts on the main DCCH until it receives a PHYSICAL INFORMATION from the SS. To test that the MS correctly takes the values of the Timing Advance and Starting Time information elements into account. To test that the MS activates the new channel correctly. To test that the MS correctly retransmits Layer 3 MM or CC messages, that were not acknowledged by Layer 2 before the Handover, after completion of the Handover.

Conformance Requirement:

The MS shall correctly apply the handover procedure from SDCCH without frequency hopping to SDCCH with frequency hopping in the non-synchronized case during call establishment. If during call establishment a Layer 3 MM or CC message just sent by the MS is not Layer 2 acknowledged before the channel change caused by the HANDOVER COMMAND message, the MS shall send the Layer 3 message to the new cell, using the same value in the N(SD) field, after the handover procedure.

The MS shall correctly apply the handover procedure from TCH/F with frequency hopping to TCH/F with frequency hopping in the non-synchronized case during call establishment. If during call establishment a Layer 3 MM or CC message just sent by the MS is not Layer 2 acknowledged before the channel change caused by the HANDOVER COMMAND message, the MS shall send the Layer 3 message to the new cell, using the same value in the N(SD) field, after the handover procedure.

The MS shall correctly apply the handover procedure from SDCCH with frequency hopping to TCH/F without frequency hopping in the non-synchronized case, during call establishment. If during call establishment a Layer 3 MM or CC message just sent by the MS is not Layer 2 acknowledged before the channel change caused by the HANDOVER COMMAND message, the MS shall send the Layer 3 message to the new cell, using the same value in the N(SD) field, after the handover procedure.

Reference(s):

GSM 04.08, section 3.4.4

GSM 04.08, section 3.1.4.2

Related PICS/PIXIT Statement(s):

Supported rate(s) of TCH: TCH/FS and/or TCH/HS.

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

2 cells with same LAI, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and camped on cell A.

Foreseen Final State of the MS:

"Idle, updated" with TMSI allocated and camped on cell B.

Method of Test:

A Mobile Originating Call is initiated. After the MS has sent the SETUP message (and before the last L2 frame carrying the SETUP message is acknowledged by the SS) the SS sends a HANDOVER COMMAND, ordering the MS to switch to cell B. The MS shall then (at the time specified in the Starting Time information element if it is included) begin to send access bursts at the commanded power level on the new DCCH to cell B. The SS observes the access bursts and after receiving n (n being randomly drawn between values according to table 2 of section 26.6.5) access bursts, the SS sends PHYSICAL INFORMATION with a Timing Advance as specified in table 1 of section 26.6.5. The MS shall activate the channel in sending and receiving mode, and connect the channel in the range of (0..3) seconds after the transmission time of the PHYSICAL INFORMATION. It shall do so with the correct timing advance. Then the MS shall establish a signalling link and send a HANDOVER COMPLETE message. The SS shall receive this message within T3103 seconds after sending the HANDOVER COMMAND, but not before a UA frame has been sent by the SS. The MS shall then again send the SETUP message to the SS, using the same value in the N(SD) field. Finally the SS sends a CHANNEL RELEASE to end the test.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	--------	--------	A MO call is initiated

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating call,

NECI not set to 1

3	SS -> MS	IMM ASSIGNMENT	Channel Description and Mobile

Allocation : as specified in table

1 of section 26.6.5.

4	MS -> SS	CM SERVICE REQUEST	CM Service Type = Mobile Origina-

ting Call Establishment.

5	SS -> MS	CIPH MODE COMMAND

6	MS -> SS	CIPH MODE COMPLETE

7	MS -> SS	SETUP	Last L2 frame not acknowledged by

the SS.

8	SS -> MS	HANDOVER COMMAND	Channel description : as specified

in table 1 of section 26.6.5.

Synchronisation

Indication : not synchronized.

Cell Channel Description : appears

if hopping is used on target cell.

Starting Time : as specified in

table 1 of section 26.6.5. (if no

Starting Time is specified then this IE is

not included)

9	MS -> SS	HANDOVER ACCESS	Repeated until reception of PHYSICAL

INFORMATION. Handover Reference as

included in the HANDOVER COMMAND

10	SS -> MS	PHYSICAL INFO	Sent after reception of n HANDOVER

ACCESS. Timing Advance : as

specified in table 1 of section

26.6.5.

11	MS -> SS	SABM	Sent without information field

12	SS -> MS	UA

13	MS -> SS	HANDOVER COMPLETE

14	MS -> SS	SETUP	Same N(SD) as in step 7.

15	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

None

26.6.5.3	Handover / successful / active call / finely synchronized

Purpose:

To test that when the MS is ordered to make a finely synchronized handover to a synchronized cell, it sends 4 access bursts on the main DCCH and then activates the channel correctly, taking into account the value of any Starting Time information element and correctly calculating the timing advance to use.

Conformance Requirement:

The MS shall correctly apply the handover procedure from TCH/F with frequency hopping to TCH/F without frequency hopping in the synchronized case when a call is in progress.

The MS shall correctly apply the handover procedure from TCH/H with frequency hopping to TCH/H without frequency hopping in the synchronized case when a call is in progress. This requirement does not apply to MSs not supporting TCH/H.

Reference(s):

GSM 04.08 section 3.4.4.

GSM 05.10, section 6.6

Related PICS/PIXIT Statement(s):

Supported rate(s) of TCH: TCH/FS and/or TCH/HS.

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

2 cells, A and B, with same LAI, default parameters.

The BCCH of cell A is sent k bit periods before the BCCH of cell B. The timing advance in cell A sent to the MS is y [bit periods].

Mobile Station:

The MS is in the active state (U10) of a call (on cell A). Used power level is 3.

[editorial note : since not all MSs support such high power levels, it might be better to choose e.g. power level 10]

Foreseen Final State of the MS:

The active state (U10) of a mobile call (on cell B). Used power level is 5.

Method of Test:

The MS is in the active state (U10) of a call on cell A. The SS sends a HANDOVER COMMAND on the main DCCH. The MS shall (at the time specified in the Starting Time information element, if included) send 4 access bursts, at the commanded power level, in 4 successive slots on the new DCCH to cell B. Then the MS shall establish a signalling link using the correct Timing Advance and send a HANDOVER COMPLETE message. The SS shall receive this message within T3103 seconds after sending the HANDOVER COMMAND, but not before a UA frame has been sent by the SS.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	Channel description : as specified

in table 1 of section 26.6.5.

Synchronisation

Indication : Synchronized.

Cell

Channel Description : appears if

hopping is used on target cell.

Starting Time : as specified in

table 1 of section 26.6.5.

If no Starting Time is specified then

this IE is not included.

2	MS -> SS	HANDOVER ACCESS	Handover Reference as included in

3	MS -> SS	HANDOVER ACCESS	the HANDOVER COMMAND.

4	MS -> SS	HANDOVER ACCESS

5	MS -> SS	HANDOVER ACCESS

6	MS -> SS	SABM	Sent without information field

7	SS -> MS	UA

8	MS -> SS	HANDOVER COMPLETE

Specific Message Contents:

None

26.6.5.4	Handover / successful / call under establishment / finely synchronized

Purpose:

To test that when the MS is ordered to make a finely synchronized handover to a synchronized cell, it sends 4 access bursts on the main DCCH and then activates the channel correctly, taking into account the value of any Starting Time information element and correctly calculating the timing advance to use. To test that the MS correctly retransmits Layer 3 MM or CC messages that were not acknowledged by Layer 2 before the Handover, after completion of the Handover.

Conformance Requirement:

The MS shall correctly apply the handover procedure from SDCCH with frequency hopping to SDCCH with frequency hopping in the finely synchronized case, during call establishment.

If during call establishment a Layer 3 MM or CC message just sent by the MS is not Layer 2 acknowledged before the channel change caused by the HANDOVER COMMAND message, the MS shall send the Layer 3 message to the new cell, using the same value in the N(SD) field, after the handover procedure.

The MS shall correctly apply the handover procedure from TCH/F without frequency hopping to TCH/F with frequency hopping in the synchronized case during call establishment. If during call establishment a Layer 3 MM or CC message just sent by the MS is not Layer 2 acknowledged before the channel change caused by the HANDOVER COMMAND message, the MS shall send the Layer 3 message to the new cell, using the same value in the N(SD) field, after the handover procedure.

The MS shall correctly apply the handover procedure from SDCCH without frequency hopping to TCH/F without frequency hopping in the synchronized case, during call establishment. If during call establishment a Layer 3 MM or CC message just sent by the MS is not Layer 2 acknowledged before the channel change caused by the HANDOVER COMMAND message, the MS shall send the Layer 3 message to the new cell, using the same value in the N(SD) field, after the handover procedure.

Reference(s):

GSM 05.10, section 6.6

GSM 04.08, section 3.4.4

GSM 04.08, section 3.1.4.2

Related PICS/PIXIT Statement(s):

Supported rate(s) of TCH: TCH/FS.

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

2 cells, A and B, with same LAI, default parameters.

The BCCH of cell A is sent k bit periods before the BCCH of cell B. The timing advance in cell A sent to the MS is y [bit periods].

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and camped on cell A.

Foreseen Final State of the MS:

"Idle, updated" with TMSI allocated and camped on cell B.

Method of Test:

A Mobile Originating Call is initiated. After the MS has sent the SETUP message (and before the last L2 frame carrying the SETUP message is acknowledged by the SS) the SS sends a HANDOVER COMMAND, ordering the MS to switch to cell B. The MS shall then (at the time specified in the Starting Time information element, if included) send 4 access bursts, at the commanded power level, in 4 successive slots on the new DCCH to cell B. Then the MS shall establish a signalling link using the correct timing advance and send a HANDOVER COMPLETE message. The SS shall receive this message within T3103 seconds after sending the HANDOVER COMMAND, but not before a UA frame has been sent by the SS. The MS shall then again send the SETUP message to the SS, using the same value in the N(SD) field. Finally the SS sends a CHANNEL RELEASE to end the test.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	--------	--------	A MO call is initiated

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating call,

NECI not set to 1"

3	SS -> MS	IMM ASSIGNMENT	Channel Description and Mobile

Allocation : as specified in table

1 of section 26.6.5.

4	MS -> SS	CM SERVICE REQUEST	CM Service Type = Mobile Origina-

ting Call Establishment.

5	SS -> MS	CIPH MODE COMMAND

6	MS -> SS	CIPH MODE COMPLETE

7	MS -> SS	SETUP	Last L2 frame not acknowledged by

the SS.

8	SS -> MS	HANDOVER COMMAND	Channel description : as specified

in table 1 of section 26.6.5.

Synchronisation

Indication : synchronized. Cell

Channel Description : appears if

hopping is used on target cell.

Starting Time : as specified in

table 1 of section 26.6.5.

(If no Starting Time is specified then

this IE is not included)

9	MS -> SS	HANDOVER ACCESS	Handover Reference as included in

10	MS -> SS	HANDOVER ACCESS	the HANDOVER COMMAND

11	MS -> SS	HANDOVER ACCESS

12	MS -> SS	HANDOVER ACCESS

13	MS -> SS	SABM	Sent without information field

14	SS -> MS	UA

15	MS -> SS	HANDOVER COMPLETE

16	MS -> SS	SETUP	Same N(SD) as in step 7

17	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

None

26.6.5.5	Pre-synchronized Handovers

26.6.5.5.1	Handover / successful / active call / pre-synchronized / Timing Advance IE not included

If an MS does not implement the pre-synchronized handover procedure correctly then calls may fail.

Purpose:

To verify that when the MS is ordered to make a pre-synchronized handover to another cell, it sends 4 access bursts on the main DCCH and then activates the channel correctly and correctly calculates the time to transmit.

Conformance Requirement:

1	The MS shall correctly apply the handover procedure from TCH/F without frequency hopping to TCH/F without frequency hopping in the pre-synchronized case when a call is active.

2	When the Timing Advance information element is not included in the HANDOVER COMMAND, the MS shall access the new cell with the default timing advance of 1 bit period.

3	The MS shall be ready to transmit the HANDOVER COMPLETE message within 650 ms of the end of the HANDOVER COMMAND message.

Reference(s):

Conformance requirement 1: GSM 04.08 section 3.4.4.

Conformance requirement 2: GSM 05.10, section 6.6.

Conformance requirement 3: GSM 04.13 [v1.2.0, section 5.2.6.1].

Related PICS/PIXIT Statement(s):

Supported rate(s) of TCH: TCH/FS.

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

2 cells, A and B, with same LAI, default parameters.

The BCCH of cell A is sent k bit periods before the BCCH of cell B. k is arbitrarily selected by the test house.

Mobile Station:

The MS is in the active state (U10) of a call (on cell A) using a full rate TCH in non-hopping mode.

Foreseen Final State of the MS:

The active state (U10) of a mobile call (on cell B) using a full rate TCH in non-hopping mode.

Method of Test:

The MS is in the active state (U10) of a call on cell A. The SS sends a HANDOVER COMMAND for a pre-synchronised handover without the Timing Advance IE on the main DCCH. The MS shall send 4 access bursts, at the commanded power level, in 4 successive slots of the new DCCH to cell B with a Timing Advance of zero. Then the MS shall establish a signalling link using a Timing Advance of one and send a HANDOVER COMPLETE message. The MS shall be ready to transmit the HANDOVER COMPLETE message before 650 ms after the end of the HANDOVER COMMAND message, but not before a UA frame has been sent by the SS.

The term 'ready to transmit' is defined in GSM 04.13.

Maximum Duration of Test:

5 seconds.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	See specific message contents below.

2	MS -> SS	HANDOVER ACCESS	Handover Reference as included in the

3	MS -> SS	HANDOVER ACCESS	HANDOVER COMMAND.

4	MS -> SS	HANDOVER ACCESS

5	MS -> SS	HANDOVER ACCESS

6	MS -> SS	SABM	Sent without information field. Sent with a

timing advance of 1 bit period.

7	SS -> MS	UA

8	MS -> SS	HANDOVER COMPLETE	This message shall be ready to be transmitted

before 650 ms after the completion of step 1.

Specific Message Contents:

HANDOVER COMMAND

As default message contents, except:

Synchronisation Indication			pre-synchronized; ROT=0; NCI=0.

26.6.5.5.2	Handover / successful / call being established / pre-synchronized / Timing Advance IE is included / reporting of observed time difference requested.

If an MS does not implement the pre-synchronized handover procedure correctly then calls may fail.

If an MS does not report the observed time difference between cells correctly then pseudo synchronised handovers might not be possible for any MS.

Purpose:

To test that when the MS is ordered to make a pre-synchronized handover to another cell, it sends 4 access bursts on the main DCCH and then activates the channel correctly and correctly calculates the time to transmit. To test that the MS correctly retransmits Layer 3 MM or CC messages that were not acknowledged by Layer 2 before the Handover, after completion of the Handover. To test that the MS correctly reports on the time difference between the cells.

Conformance Requirement:

1	The MS shall correctly apply the handover procedure from an SDCCH/4 to a TCH/F without frequency hopping in the pre-synchronized case while a call is being established.

2	If during call establishment a Layer 3 MM or CC message just sent by the MS is not Layer 2 acknowledged before the channel change caused by the HANDOVER COMMAND message, the MS shall send the Layer 3 message to the new cell, using the same value in the N(SD) field, after the handover procedure.

3	When the Timing Advance information element is included in the HANDOVER COMMAND, the MS shall access the new cell with the timing advance included in the Timing Advance IE.

4	The MS shall be ready to transmit the HANDOVER COMPLETE message within 650 ms of the end of the HANDOVER COMMAND message.

5	When requested to do so in the HANDOVER COMMAND message, the MS shall return the Mobile Time Difference IE in the HANDOVER COMPLETE message indicating the sum of the observed time difference between the cells and the timing advance used on the old cell.

Reference(s):

Conformance requirement 1: GSM 04.08, section 3.4.4.

Conformance requirement 2: GSM 04.08, section 3.1.4.3.

Conformance requirement 3: GSM 05.10, section 6.6.

Conformance requirement 4: GSM 04.13 [v1.2.0, section 5.2.6.1].

Conformance requirement 5: GSM 04.08, section 10.5.2.39.

Related PICS/PIXIT Statement(s):

Supported rate(s) of TCH: TCH/FS.

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

2 cells, A and B, with same LAI, default parameters.

The BCCH of cell A is sent k bit periods before the BCCH of cell B.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and camped on cell A.

Foreseen Final State of the MS:

"Idle, updated" with TMSI allocated and camped on cell B.

Method of Test:

A Mobile Originating Call is initiated. The SS sends an IMMEDIATE ASSIGNMENT message allocating an SDCCH/4. The MS is commanded to use a timing advance of y bit periods on cell A. After the MS has sent the SETUP message (and before the last L2 frame carrying the SETUP message is acknowledged by the SS) the SS sends a HANDOVER COMMAND, ordering the MS to switch to cell B. The MS shall then send 4 access bursts, at the commanded power level, in 4 successive slots of the new DCCH to cell B. Then the MS shall establish a signalling link using the correct timing advance and send a HANDOVER COMPLETE message. The MS shall be ready to transmit the HANDOVER COMPLETE message before 650 ms after the end of the HANDOVER COMMAND message, but not before a UA frame has been sent by the SS.

The term 'ready to transmit' is defined in GSM 04.13.

The MS shall then again send the SETUP message to the SS, using the same value in the N(SD) field. Finally the SS sends a CHANNEL RELEASE to end the test.

Maximum Duration of Test:

20 seconds.

Expected Sequence:

Step	Direction	Message	Comments

1	--------	--------	A MO call is initiated

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMM ASSIGNMENT	to an SDCCH/4.

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	CIPH MODE COMMAND

6	MS -> SS	CIPH MODE COMPLETE

7	MS -> SS	SETUP	Last L2 frame not acknowledged by the SS.

8	SS -> MS	HANDOVER COMMAND	See specific message contents below.

9	MS -> SS	HANDOVER ACCESS	Handover Reference as included in the

10	MS -> SS	HANDOVER ACCESS	HANDOVER COMMAND

11	MS -> SS	HANDOVER ACCESS

12	MS -> SS	HANDOVER ACCESS

13	MS -> SS	SABM	Sent without information field. Sent with a

timing advance of 9 bit periods.

14	SS -> MS	UA

15	MS -> SS	HANDOVER COMPLETE	This message shall be ready to be transmitted

before 650 ms after the completion of step 8.

Shall include the Mobile Time Difference IE

with value (2k+y) mod 127 500 half bit

periods. A tolerance of ±2 half bit periods is

allowed.

16	MS -> SS	SETUP	Same N(SD) as in step 7

17	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

HANDOVER COMMAND

As default message contents, except:

Synchronisation Indication			pre-synchronized; ROT=1; NCI=0.

Timing Advance					9 bit periods.

26.6.5.6	Handover / successful / active call / pseudo synchronized

This test only applies to MSs that claim to support the pseudo synchronized handover procedure. If MSs that claim to support this procedure do not correctly implement it, then calls may fail.

Purpose:

To test that when the MS is ordered to make a pseudo synchronized handover to another cell, it sends 4 access bursts on the main DCCH and then activates the channel correctly and correctly calculates the time to transmit. To test that the MS correctly reports the time difference between the cells.

Conformance Requirement:

1	The MS shall correctly apply the handover procedure from TCH/F without frequency hopping to TCH/F without frequency hopping in the pseudo synchronized case when a call is in progress.

2	The MS shall access the new cell with the correct timing advance.

3	The MS shall be ready to transmit the HANDOVER COMPLETE message within 650 ms of the end of the HANDOVER COMMAND message.

4	When requested to do so in the HANDOVER COMMAND message, the MS shall return the Mobile Time Difference IE in the HANDOVER COMPLETE message indicating the sum of the observed time difference between the cells and the timing advance used on the old cell.

Reference(s):

Conformance requirement 1: GSM 04.08 section 3.4.4.

Conformance requirement 2: GSM 05.10, section 6.6.

Conformance requirement 3: GSM 04.13 [v1.2.0, section 5.2.6.1].

Conformance requirement 4: GSM 04.08, section 10.5.2.39.

Related PICS/PIXIT Statement(s):

Supported rate(s) of TCH: TCH/FS.

Support for state U10 of the Call Control protocol.

Support for pseudo synchronisation.

Initial Condition(s):

System Simulator:

2 cells, A and B, with same LAI, default parameters.

The BCCH of cell A is sent k bit periods before the BCCH of cell B. k is arbitrarily selected by the test house.

The MS is being commanded to use a timing advance of y bit periods on cell A, where y is arbitrarily selected from the set {11, 12, ... ,62}.

Mobile Station:

The MS is in the active state (U10) of a call (on cell A) using a full rate TCH in non-hopping mode.

Foreseen Final State of the MS:

The active state (U10) of a mobile call (on cell B) using a full rate TCH in non-hopping mode.

Method of Test:

The MS is in the active state (U10) of a call on cell A. The SS sends a HANDOVER COMMAND for a pseudo-synchronised handover with the Real Time Difference IE included. The Time Difference value is set to (2k+10) modulo 256. The MS shall send 4 access bursts, at the commanded power level, in 4 successive slots of the new DCCH to cell B with a Timing Advance of zero. Then the MS shall establish a signalling link using a Timing Advance of (y-10) bit periods and send a HANDOVER COMPLETE message. The MS shall be ready to transmit the HANDOVER COMPLETE message before 650 ms after the end of the HANDOVER COMMAND message, but not before a UA frame has been sent by the SS.

(* observed time difference	= 2k half bit periods.

Ta		= y bit periods

"real time difference"	= 2k+10 half bit periods

==> Tb	= 2k-(2k+10)+y = y-10		*)

The term 'ready to transmit' is defined in GSM 04.13.

Maximum Duration of Test:

5 seconds.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	See specific message contents below.

2	MS -> SS	HANDOVER ACCESS	Handover Reference as included in the

3	MS -> SS	HANDOVER ACCESS	HANDOVER COMMAND.

4	MS -> SS	HANDOVER ACCESS

5	MS -> SS	HANDOVER ACCESS

6	MS -> SS	SABM	Sent without information field; sent with

timing advance of (y-10) bit periods

7	SS -> MS	UA

8	MS -> SS	HANDOVER COMPLETE	This message shall be ready to be transmitted

before 650 ms after the completion of step 1.

Shall include the Mobile Time Difference IE with

value (2k+y) mod 127 500 half bit periods. A

tolerance of ±2 half bit periods is allowed.

Specific Message Contents:

HANDOVER COMMAND

As default message contents, except:

Synchronisation Indication			pseudo-synchronized; ROT=1; NCI=0.

Time Difference				(2k+10) mod 256.

26.6.5.7	Handover / successful / active call / non-synchronized / reporting of observed time difference requested.

If an MS does not report the observed time difference between cells correctly then pseudo synchronised handovers might not be possible for any MS.

Purpose:

To verify that when the MS is ordered to make a non-synchronized handover to another cell and is ordered to report on the time difference between the cells, that it does so correctly.

Conformance Requirement:

1	The MS shall correctly apply the handover procedure from a TCH/F without frequency hopping to a TCH/F without frequency hopping in the non-synchronized case while a call is active.

2	When requested to do so in the HANDOVER COMMAND message, the MS shall return the Mobile Time Difference IE in the HANDOVER COMPLETE message indicating the sum of the observed time difference between the cells and the timing advance used on the old cell.

Reference(s):

Conformance requirement 1: GSM 04.08, section 3.4.4.

Conformance requirement 2: GSM 04.08, section 10.5.2.39.

Related PICS/PIXIT Statement(s):

Supported rate(s) of TCH: TCH/FS.

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

2 cells, A and B, with same LAI, default parameters.

The BCCH of cell A is sent k bit periods before the BCCH of cell B.

The MS is commanded to use a timing advance of y bit periods on cell A.

Mobile Station:

The MS is in the active state (U10) of a call (on cell A) using a full rate TCH in non-hopping mode.

Foreseen Final State of the MS:

The active state (U10) of a mobile call (on cell B) using a full rate TCH in non-hopping mode.

Method of Test:

The MS is in the active state (U10) of a call on cell A. The SS sends a HANDOVER COMMAND on the main DCCH. The HANDOVER COMMAND includes a Synchronisation Indication IE that instructs the MS to supply the observed time difference between the cells. The MS shall begin to send access bursts on the new DCCH to cell B and the SS sends one PHYSICAL INFORMATION message. The MS shall activate the channel in sending and receiving mode and establish a signalling link using the correct timing advance. The MS shall transmit a HANDOVER COMPLETE message containing the Mobile Time Difference IE with a correct value.

Maximum Duration of Test:

5 seconds.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	See specific message contents below.

2	MS -> SS	HANDOVER ACCESS	Repeated on every burst of the uplink main

DCCH until reception of PHYSICAL

INFORMATION. Handover Reference as

included in the HANDOVER COMMAND.

3	SS -> MS	PHYSICAL INFORMATION

4	MS -> SS	SABM	Sent without information field

5	SS -> MS	UA

6	MS -> SS	HANDOVER COMPLETE	Shall include the Mobile Time Difference IE

with value (2k+y) mod 127 500 half bit

periods. A tolerance of ±2 half bit periods is

allowed.

Specific Message Contents:

HANDOVER COMMAND

as default message contents, except:

Synchronisation Indication			'not synchronised'; ROT=1; NCI=0.

26.6.5.8	Handover / L3-failure

Purpose:

To verify the function of timer T3124 and the contents in the message HANDOVER FAILURE

Conformance Requirement:

The MS shall return to the old channel in the case of an handover failure caused by the non reception of the PHYSICAL INFORMATION message. On the old channel the MS shall use the Power Level that it was previously using on that channel.

Reference(s):

GSM 04.08 section 3.4.4.

Related PICS/PIXIT Statement(s):

Supported rate(s) of TCH: TCH/FS.

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

2 cells with same LAI, default parameters.

Mobile Station:

The MS is in the active state (U10) of a call (on cell A). Used power level is the maximum supported by the MS.

[editorial note : since not all MSs support such high power levels, it might be better to choose e.g. power level 10]

Foreseen Final State of the MS:

The active state (U10) of a mobile call (on cell A). Used power level is the maximum supported by the MS.

Method of Test:

The MS is in the active state (U10) of a call on cell A. The SS sends a HANDOVER COMMAND with Power Command set to 8 on the main DCCH. The MS shall begin to send access bursts at the commanded power level on the new DCCH to cell B. The SS activates the SACCH, but does not send PHYSICAL INFORMATION (thus causing a timeout of T3124). The MS shall re-establish the old link on cell A and send a HANDOVER FAILURE within 3 seconds from the transmission of HANDOVER COMMAND, using the old power level.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	Channel description : non-hopping,

full rate

Power Command : 8. Synchronisation

Indication : non synchronized.

2	MS -> SS	HANDOVER ACCESS	Several messages are sent, all

with correct Handover References.

3	MS -> SS	HANDOVER FAILURE	Sent on old channel,

RR cause value ="Abnormal release,

unspecified", "Abnormal release,

channel unacceptable", "Abnormal

release, timer expired" or

"Abnormal release, no activity on

the radio path". Layer 1 header

has the same power level as the

layer 1 header in step 1.

Shall be sent

within 3 seconds from the trans-

mission of HANDOVER COMMAND.

Specific Message Contents:

None

26.6.5.9	Handover / L1-failure

Purpose:

To verify the function of timer T3124 and the contents in the message HANDOVER FAILURE

Conformance Requirement:

The MS shalll return to the old channel in the case of an handover failure caused by a layer 1 failure on the target cell. On the old channel the MS shall use the Power Level that it was previously using on that channel.

Reference(s):

GSM 04.08 section 3.4.4.

Related PICS/PIXIT Statement(s):

Supported rate(s) of TCH: TCH/FS.

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

2 cells with same LAI, default parameters.

Mobile Station:

The MS is in the active state (U10) of a call (on cell A). Used power level is the maximum supported by the MS.

[editorial note : since not all MSs support such high power levels, it might be better to choose e.g. power level 10]

Foreseen Final State of the MS:

The active state (U10) of a mobile call (on cell A). Used power level is the maximum supported by the MS.

Method of Test:

The MS is in the active state (u10) of a call on cell A. The SS sends a HANDOVER COMMAND on the main DCCH. The MS shall begin to send access bursts at the commanded power level on the new DCCH to cell B. With the exception of normal BCCH signalling, the SS does not transmit anything on cell B (thus causing a timeout of T3124). The MS shall re-establish the old link on cell A and send a HANDOVER FAILURE within 3 seconds from the transmission of HANDOVER COMMAND, using the old power level.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	HANDOVER COMMAND	Channel description : non-hopping,

full rate. Synchronisation

Indication : non synchronized.

2	MS -> SS	HANDOVER ACCESS	Several messages are sent, all

with correct Handover References.

3	MS -> SS	HANDOVER FAILURE	Sent on old channel,

RR cause value ="Abnormal release,

unspecified", "Abnormal release,

channel unacceptable", "Abnormal

release, timer expired" or

"Abnormal release, no activity on

the radio path". Layer 1 header

has the same power level as the

layer 1 header in step 1.

Shall be sent

within 3 seconds from the trans-

mission of HANDOVER COMMAND.

Specific Message Contents:

None

26.6.6	Test of frequency redefinition

The Frequency Redefinition procedure is used by the network to change the frequencies and hopping sequences of the allocated channels.

26.6.6.1	Frequency redefinition

Purpose:

To verify that the MS, after receiving a Frequency Redefinition message, starts using the new frequencies and hopping sequence at the time indicated in the message.

Conformance Requirement:

An MS, after receiving a FREQUENCY REDEFINITION message, shall start using the new frequencies and hopping sequence in the correct time slot when the MS is allocated a dedicated channel.

The behaviour described in the test purpose is applied for each combination of the value T(k) (k = 1,2,3) and for each supported dedicated channel type.

Reference(s):

GSM 04.08 section 3.4.5.

Related PICS/PIXIT Statement(s):

Supported rate(s) of TCH: TCH/F and/or TCH/H.

Initial Condition(s):

System Simulator:

1 cell, CCCH_CONF set to 1 basic physical channel used for CCCH, not combined with SDCCHs. The cell allocation is set to CA(1) before each execution of this test.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

Test parameters:

ca(1) is set to 64.

A random subset CA(1) of the set {1,...,124} containing ca(1) elements is drawn.

An element B of the set CA(1) is randomly drawn.

A random value ca(2) in the range 20,...,63 is drawn.

A random subset CA(2) of the set {1,...,124} with ca(2) elements and containing B is drawn.

A random value ca(3) in the range 4,...,19 is drawn.

A random subset CA(3) of the set {1,...,124} with ca(3) elements and containing B is drawn.

For j = 1,2,3, values ma(j) in the range j,...,ca(j)-1 and values MAIO(j) in the range 0,...,ma(j)-1 are randomly drawn.

Subsets MA(j) of CA(j) not containing B and having ma(j) elements are randomly drawn.

Let T(1) = 91, T(2) = 42000.

A random value T(3) in the range 92,...,29999 is drawn.

The System Simulator pages the MS and after the MS has responded with a CHANNEL REQUEST, the simulator assigns a dedicated channel (TCH/F, TCH/H or SDCCH, as defined by the execution counter R). Then the SS sends a FREQUENCY REDEFINITION message, which modifies the frequencies/hopping sequence to be used by the MS. The MS shall then, at the TDMA frame defined by the contents of the "Starting Time" information element , use the new frequencies/hopping sequence. (The value of T(2) ensures that the MS believes the Starting Time has passed and so the MS shall start transmitting immediately. The range for T(3) ensures that the MS has to wait until the designated frame before starting transmission on the new frequencies.)

The verification is performed at the RF burst level. The MS transmits the standard test signal C1 as defined in GSM 11.40, and for the TCH case, the SS checks the received pattern with the expected pattern. For the SDCCH case the MS transmits fill frames, and the SS checks for each burst whether the burst is transmitted at the right frequency.

[editorial note : how is the MS made to transmit the C1 signal ? By means of loop-back ? ]

Maximum Duration of Test:

3 * (number of supported channels * T(3) + 7)

Expected Sequence:

This sequence is performed for every combination of execution counters K = 1,2,3 and R = 1,2,3:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMM ASSIGNMENT

4	MS -> SS	PAGING RESPONSE

5	--------	--------	The SS checks that the MS is

transmitting on the correct

frequencies without delay.

6	SS -> MS	FREQUENCY REDEF	See description 1 below.

7	--------	--------	The SS checks that the MS is

transmitting on the correct

frequencies and that the transmissions

started in the correct frame.

8	SS -> MS	FREQUENCY REDEF	See description 2 below

9	--------	--------	The SS checks that the MS is

transmitting on the correct

frequencies and that the transmissions

started in the correct frame.

10	SS -> MS	CHANNEL RELEASE

For	K=1	T(K) = T(1);

K=2	T(K) = T(2);

K=3	T(K) = T(3).

Specific Message Contents:

IMMEDIATE ASSIGNMENT

L2 pseudo length					value dependent on the length of the

Mobile Allocation and thus on the number

of channels in CA(1).

Channel Description

Channel type				SDCCH/8 arbitrary offset, for R=1

and TDMA offset				Bm + ACCHs for R=2

Lm + ACCHs arbitrary offset, for R=3

Timeslot number				arbitrarily selected by test house

TSC						arbitrarily selected by test house

Hopping channel				RF hopping channel

MAIO						MAIO(1)

HSN						0

Request reference				corresponds to the Channel Request

Timing advance					30 bit periods

Mobile Allocation					corresponds to set MA(1)

Starting Time					not present

IA rest octets					all bits are set to spare

FREQUENCY REDEFINITION (Description 1)

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101011

Channel Description

Channel type				SDCCH/8 offset not changed, for R=1

and TDMA offset				Bm + ACCHs for R=2

Lm + ACCHs offset not changed, for R=3

Timeslot number				not changed

TSC						not changed

Hopping channel				RF hopping channel

MAIO						MAIO(2)

HSN						0

Mobile Allocation					corresponds to set MA(2)

Starting Time					The last burst of the first L2 frame

containing the beginning of this message is

transmitted in frame number X. The starting

time is set to frame number (X plus T(k)

modulo 42432).

Cell Channel Description

Information element identifier			62H

contents					corresponds to set CA(2) with 'Format ID'

set to "bit map 0"

FREQUENCY REDEFINITION (Description 2)

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101011

Channel Description

Channel type				SDCCH/8 offset not changed, for R=1

and TDMA offset				Bm + ACCHs, for R=2

Lm + ACCHs offset not changed, for R=3

Timeslot number				not changed

TSC						not changed

Hopping channel				RF hopping channel

MAIO						MAIO(3)

HSN						0

Mobile Allocation					corresponds to set MA(3)

Starting Time					The last burst of the first L2 frame

containing the beginning of this message is

transmitted in frame number X. The starting

time is set to frame number (X plus T(k)

modulo 42432).

Cell Channel Description

Information element identifier			62H

contents					corresponds to set CA(3) with 'Format ID'

set to "bit map 0"

26.6.7	Test of the channel mode modify procedure

The channel mode modify procedure allows the network to request the MS to change the channel mode for one channel. If the mobile station doesn't correctly respond to the CHANNEL MODE MODIFY message (with a positive acknowledgement if the new channel mode is supported, with a negative acknowledgement if the new channel mode is not supported), the network may try to repeat the procedure, release the connection, or continue to wait for the acknowledgement (the maximum time resulting from layer two re-transmissions and MS reaction time being around 5 seconds).

26.6.7.1	Test of the channel mode modify procedure / full rate

This test is only applicable to an MS supporting TCH/F.

26.6.7.1.1	Conformance Requirement

When the MS has received the CHANNEL MODE MODIFY message, the mobile station changes the mode for the indicated channel and then replies by a CHANNEL MODE MODIFY ACKNOWLEDGE message indicating the new channel mode.

If the mobile station does not support the indicated mode, it shall retain the old mode and return the associated channel mode information in the CHANNEL MODE MODIFY ACKNOWLEDGE message.

26.6.7.1.2	Purpose:

To verify that the MS, having a call control entity in state U10, "active" and a TCH/F being assigned, acknowledges a CHANNEL MODE MODIFY message by sending a CHANNEL MODE MODIFY ACKNOWLEDGEMENT message specifying

-	the new mode if that mode is supported

-	the old mode if the new mode is not supported.

This shall be verified for the channel modes

-	signalling only

-	speech full rate

-	data 9.6 Kb/s

-	data 4.8 Kb/s full rate

-	data 2.4 Kb/s full rate.

26.6.7.1.3	Reference(s):

TS GSM 04.08, subclauses 3.4.6.2 and 3.4.6.3

26.6.7.1.4	Related PICS/PIXIT Statement(s):

-	The MS supports TCH/F (Y/N)

-	The MS supports the call control protocol for at least one BC (Y/N)

-	Bearer Capabilities supported by the MS

-	Channel modes supported by the MS:

*	MS supports speech full rate (p1 = Y/N)

*	MS supports data 9.6 Kb/s (p2 = Y/N)

*	MS supports data 4.8 Kb/s full rate (p3 = Y/N)

*	MS supports data 2.4 Kb/s full rate (p4 = Y/N)

26.6.7.1.5	Initial Condition(s):

System Simulator:

1 cells, default parameters.

Mobile Station:

The MS is in the active state (U10) of a call established for a supported BC(u10). The channel mode of the dedicated channel is suitable for that BC. (This channel mode is considered to be "the channel mode in use" at the beginning of the test.)

26.6.7.1.6	Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

26.6.7.1.7	Method of TestTest procedure:

The SS sends a series of CHANNEL MODE MODIFY messages to the MS. Each time it is checked that the MS responds with a CHANNEL MODE MODIFY ACKNOWLEDGE message specifying

-	the channel mode that has been specified in the CHANNEL MODE MODIFY message, if the MS supports that mode (this mode then becomes the "channel mode in use")

-	the channel mode that was in use when the CHANNEL MODE MODIFY message has been received, if the MS does not support the channel mode specified in the CHANNEL MODE MODIFY message.

Maximum Duration of Test:

30 seconds.

Expected Sequence:

Step	Direction	Message	Comments

1	SS->MS	CHANNEL MODE MODIFY

2	MS->SS	CHAN MODE MOD ACK

3	SS->MS	CHANNEL MODE MODIFY

4	MS->SS	CHAN MODE MOD ACK

5	SS->MS	CHANNEL MODE MODIFY

6	MS->SS	CHAN MODE MOD ACK

7	SS->MS	CHANNEL MODE MODIFY

8	MS->SS	CHAN MODE MOD ACK

9	SS->MS	CHANNEL MODE MODIFY

10	MS->SS	CHAN MODE MOD ACK

11	SS->MS	CHANNEL RELEASE

Specific Message Contents:

CHANNEL MODE MODIFY

Channel description		describes the already assigned dedicated

channel

Channel mode

Mode			in step 1:	signalling only

in step 3:	data 9.6 Kb/s

in step 5:	data 4.8 Kb/s full rate

in step 7:	data 2.4 Kb/s full rate

in step 9:	speech full rate

CHANNEL MODE MODIFY ACKNOWLEDGE

Channel mode

Mode			in step 2:		signalling only

in step 4:		if p2 = Y: data 9.6 Kb/s

if p2 = N: signalling only

in step 6:		if p3 = Y: data 4.8 Kb/s full rate

if p3 = N: same as in step 4

in step 8:		if p4 = Y: data 2.4 Kb/s full rate

if p4 = N: same as in step 6

in step 10:	if p1 = Y: speech full rate

if p1 = N: same as in step 8

26.6.7.2	Test of the channel mode modify procedure / half rate

This test is only applicable to an MS supporting TCH/H.

26.6.7.2.1	Conformance Requirement

When the MS has received the CHANNEL MODE MODIFY message, the mobile station changes the mode for the indicated channel and then replies by a CHANNEL MODE MODIFY ACKNOWLEDGE message indicating the new channel mode.

If the mobile station does not support the indicated mode, it shall retain the old mode and return the associated channel mode information in the CHANNEL MODE MODIFY ACKNOWLEDGE message.

26.6.7.2.2	Purpose:

To verify that the MS, having a call control entity in state U10, "active" and a TCH/H being assigned, acknowledges a CHANNEL MODE MODIFY message by sending a CHANNEL MODE MODIFY ACKNOWLEDGEMENT message specifying

-	the new mode if that mode is supported

-	the old mode if the new mode is not supported.

This shall be verified for the channel modes

-	signalling only

-	speech half rate

-	data 4.8 Kb/s half rate

-	data 2.4 Kb/s half rate

26.6.7.2.3	Reference(s):

TS GSM 04.08, subclauses 3.4.6.2 and 3.4.6.3

26.6.7.2.4	Related PICS/PIXIT Statement(s):

-	The MS supports TCH/H (Y/N)

-	The MS supports the call control protocol for at least one BC (Y/N)

-	Bearer Capabilities supported by the MS

-	Channel modes supported by the MS:

*	MS supports speech half rate (q1 = Y/N)

*	MS supports data 4.8 Kb/s half rate (q2 = Y/N)

*	MS supports data 2.4 Kb/s half rate (q3 = Y/N)

26.6.7.2.5	Initial Condition(s):

System Simulator:

1 cells, default parameters.

Mobile Station:

The MS is in the active state (U10) of a call established for a supported BC. The channel mode of the dedicated channel is suitable for that BC. (This channel mode is considered to be "the channel mode in use" at the beginning of the test.)

26.6.7.2.6	Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

26.6.7.2.7	Test procedure:

The SS sends a series of CHANNEL MODE MODIFY messages to the MS. Each time it is checked that the MS  responds with a CHANNEL MODE MODIFY ACKNOWLEDGE message specifying

-	the channel mode that has been specified in the CHANNEL MODE MODIFY message, if the MS supports that mode (this mode then becomes the "channel mode in use")

-	the channel mode that was in use when the CHANNEL MODE MODIFY message has been received, if the MS does not support the channel mode specified in the CHANNEL MODE MODIFY message.

Maximum Duration of Test:

30 seconds.

Expected Sequence:

Step	Direction	Message	Comments

1	SS->MS	CHANNEL MODE MODIFY

2	MS->SS	CHAN MODE MOD ACK

3	SS->MS	CHANNEL MODE MODIFY

4	MS->SS	CHAN MODE MOD ACK

5	SS->MS	CHANNEL MODE MODIFY

6	MS->SS	CHAN MODE MOD ACK

7	SS->MS	CHANNEL MODE MODIFY

8	MS->SS	CHAN MODE MOD ACK

9	SS->MS	CHANNEL MODE MODIFY

10	MS->SS	CHAN MODE MOD ACK

11	SS->MS	CHANNEL RELEASE

Specific Message Contents:

CHANNEL MODE MODIFY

Channel description			describes the already assigned

dedicated channel

Channel mode

Mode				in step 1: signalling only

in step 3: data 4.8 Kb/s half rate

in step 5: data 2.4 Kb/s half rate

in step 7: speech half rate

CHANNEL MODE MODIFY ACKNOWLEDGE

Channel mode

Mode				in step 2:		signalling only

in step 4:		if q2 = Y: data 4.8 Kb/s half rate

if q2 = N: signalling only

in step 6:		if q3 = Y: data 2.4 Kb/s half rate

if q3 = N: same as in step 4

in step 8:		if q1 = Y: speech half rate

if q1 = N: same as in step 6.

26.6.8	Test of ciphering mode setting

The Ciphering Mode Setting Procedure can be used by the network to trigger the start and stop of stream ciphering.

26.6.8.1	Ciphering mode / start ciphering

Purpose:

To verify that the MS starts ciphering when it receives a CIPHERING MODE COMMAND message with Cipher Mode Setting = "Start Ciphering". To verify that it continues to use the old cipher key after it receives an AUTHENTICATION REQUEST whilst in ciphered mode.

Conformance Requirement:

1:	When the MS receives the CIPHERING MODE COMMAND message with Ciphering Mode Setting information element set to "start ciphering", the MS starts ciphering and deciphering with the algorithm indicated by the "algorithm identifier" field;

-	the MS responds with a CIPHERING MODE COMPLETE message in ciphered mode;

-	the ciphering uses the cipher key determined during the authentication procedure.

2:	The MS responds to the AUTHENTICATION REQUEST message with an AUTHENTICATION RESPONSE message and continues to use the ciphering key obtained from the previous authentication procedure.

Reference(s):

GSM 04.08 section 3.4.7.

Related PICS/PIXIT Statement(s):

Support for state U10 of the Call Control protocol.

Supported encryption algorithms: A5/1 and/or A5/2

Initial Condition(s):

System Simulator:

1 cells, Radio-Link-Timeout set to 64.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The MS is made to originate a call. It shall send at least one CHANNEL REQUEST message. The SS sends an IMMEDIATE ASSIGNMENT and the MS shall answer with a CM SERVICE REQUEST. The SS sends an AUTHENTICATION REQUEST and the MS shall answer with AUTHENTICATION RESPONSE. Then the SS sends a CIPHERING MODE COMMAND, ordering the MS to start ciphering with an algorithm supported by the MS. After transmission of this command the SS starts deciphering. The MS shall respond with a CIPHERING MODE COMPLETE message in ciphered mode using the cipher key determined during the authentication procedure, and continue to establish the call with a SETUP message. After reception of the CIPHERING MODE COMPLETE the SS starts enciphering.

The SS then sends another AUTHENTICATION REQUEST and the MS shall respond with an AUTHENTICATION RESPONSE. The MS shall continue to use the old cipher key.

Finally the SS sends a CHANNEL RELEASE to end the test.

[editorial note : In the old 11.10 it was required that the MS should store the new key, but that was not tested. Should it be tested in phase 2 ? ]

Maximum Duration of Test:

-

Expected Sequence:

This sequence is performed for execution counter, K=1, 2.

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating Call"

NECI not set to 1

2	SS -> MS	IMM ASSIGNMENT

3	MS -> SS	CM SERVICE REQUEST

4	SS -> MS	AUTHENT REQUEST

5	MS -> SS	AUTHENT RESPONSE

6	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "Start

Ciphering; cipher with algorithm A5/K".

The SS starts deciphering.

7	MS -> SS	CIPH MODE COMPLETE	Sent in ciphered mode using

the cipher key determined in between

steps 4&5. The SS start enciphering

8	MS -> SS	SETUP

9	SS -> MS	AUTHENT REQUEST	Determines a new cipher key

10	MS -> SS	AUTHENT RESPONSE	Sent in ciphered mode using the

cipher key determined in between steps

4&5.

11	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

CIPHERING MODE COMMAND

For	k = 1, A5/k = A5/1

For	k = 2, A5/k = A5/2

26.6.8.2	Ciphering mode / no ciphering

Purpose:

To verify that the MS does not start ciphering when it receives a CIPHERING MODE COMMAND message with Cipher Mode Setting = "No Ciphering".

Conformance Requirement:

When the MS receives a CIPHERING MODE COMMAND message with Ciphering Mode Setting information element set to "no ciphering" the MS shall respond in non ciphered mode with a CIPHERING MODE COMPLETE message.

When the CIPHERING MODE COMMAND with Ciphering Mode Setting information element set to "no ciphering" is received as a response to a CM SERVICE REQUEST, the MS shall continue the establishment of the CM service.

Reference(s):

GSM 04.08 section 3.4.7.

Related PICS/PIXIT Statement(s):

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

1 cells, Radio-Link-Timeout set to 64.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The MS is made to originate a call. It shall send at least one CHANNEL REQUEST message. The SS sends an IMMEDIATE ASSIGNMENT and the MS shall answer with a CM SERVICE REQUEST. The SS sends an AUTHENTICATION REQUEST and the MS shall answer with an AUTHENTICATION RESPONSE. Then the SS sends a CIPHERING MODE COMMAND, ordering the MS not to start ciphering. The MS shall respond with a CIPHERING MODE COMPLETE message in non-ciphered mode and continue to establish the call with a SETUP message.

Finally the SS sends a CHANNEL RELEASE to end the test.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating call;

NECI not equal to 1

2	SS -> MS	IMM ASSIGNMENT

3	MS -> SS	CM SERVICE REQUEST

4	SS -> MS	AUTHENT REQUEST

5	MS -> SS	AUTHENT RESPONSE

6	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "No

Ciphering"

7	MS -> SS	CIPH MODE COMPLETE	Sent in non-ciphered mode

8	MS -> SS	SETUP

11	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

None

26.6.8.3	Ciphering mode / old cipher key

Purpose:

To verify that the MS uses the stored cipher key when it receives a CIPHERING MODE COMMAND without a preceding authentication procedure.

Conformance Requirement:

When the MS receives the CIPHERING MODE COMMAND message with Ciphering Mode Setting information element set to "start ciphering", the MS starts ciphering and deciphering with the algorithm indicated by the "algorithm identifier" field. Also;

-	the MS responds with a CIPHERING MODE COMPLETE message in the correct ciphered mode;

-	the ciphering shall use the previously stored cipher key

-	in the case of a mobile originating speech caal, the MS shall send a SETUP message after the completion of the ciphering procedure.

Reference(s):

GSM 04.08 section 3.4.7.

Related PICS/PIXIT Statement(s):

Support for state U10 of the Call Control protocol.

Supported encyrption algorithms: A5/1 and/or A5/2.

Initial Condition(s):

System Simulator:

1 cells, Radio-Link-Timeout = 64.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and a known cipher key stored.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The MS is made to originate a call. It shall send at least one CHANNEL REQUEST message. The SS sends an IMMEDIATE ASSIGNMENT and the MS shall answer with a CM SERVICE REQUEST. The SS sends a CIPHERING MODE COMMAND, ordering the MS to start ciphering with a supported algorithm. After transmission of this command the SS starts deciphering. The MS shall respond with a CIPHERING MODE COMPLETE message in the commanded ciphered mode using the stored cipher key and continue to establish the call with a SETUP message. After reception of the CIPHERING MODE COMPLETE the SS starts enciphering.

Finally the SS sends a CHANNEL RELEASE to end the test.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating Call:

NECI not equal to 1

2	SS -> MS	IMM ASSIGNMENT

3	MS -> SS	CM SERVICE REQUEST

4	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "Start

Ciphering", algorithm arbitrarily selected

from those supported by the MS. The SS starts

deciphering.

5	MS -> SS	CIPH MODE COMPLETE	Sent in commanded ciphered mode with

the stored cipher key. The SS

starts enciphering.

6	MS -> SS	SETUP

7	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

None

26.6.8.4	Ciphering mode / Change of mode, algorithm and key

Networks can be implemented that do not have the same ciphering algorithms on all base stations. In such networks changes of algorithms and ciphering mode may occur and calls will fail if MSs incorrectly handle commands or use an incorrect cipher key.

Purpose:

1	To verify that when the MS is in the "not ciphered" mode and receives the CIPHERING MODE COMMAND message with Ciphering Mode Setting information element set to "start ciphering", the MS uses the cipher key stored in the SIM to start ciphering and deciphering with the algorithm indicated by the "algorithm identifier" field and that the MS responds with a CIPHERING MODE COMPLETE message.

2	To verify that the MS is ready to transmit the CIPHERING MODE COMPLETE message before 500ms after the end of the CIPHERING MODE COMMAND message.

3	To verify that when the MS receives an ASSIGNMENT COMMAND message containing a Cipher Mode Setting IE after receipt of a CIPHERING MODE COMMAND message, the MS shall perform the assignment, use the commanded mode and/or algorithm on the new channel, and not change the ciphering key.

4	To verify that when the MS receives a HANDOVER COMMAND message containing a Cipher Mode Setting IE after receipt of a CIPHERING MODE COMMAND message, the MS shall perform the handover, use the commanded mode and/or algorithm on the new channel, and not change the ciphering key

5	To verify that when the MS is in the "ciphered" mode and receives the CIPHERING MODE COMMAND message with Cipher Mode Setting IE set to "no ciphering", the MS loads the cipher key stored in the SIM into the ME, stops ciphering and deciphering and, responds with a CIPHERING MODE COMPLETE message.

6	To verify that the MS responds to an AUTHENTICATION REQUEST message with an AUTHENTICATION RESPONSE message and continues to use the cipher key obtained from the previous authentication procedure.

7	To verify that when the MS is in the "not ciphered" mode and receives the CIPHERING MODE COMMAND message with Ciphering Mode Setting information element set to "no ciphering", the does not start ciphering or deciphering, but does respond with a CIPHERING MODE COMPLETE message.

8	To verify that when the MS receives a HANDOVER COMMAND message and the handover fails, the MS sends a HANDOVER FAILURE message on the old channel using the old ciphering mode and (if ciphered) the old algorithm and old key.

9	To verify that when the MS receives an ASSIGNMENT COMMAND message and the assignment fails, the MS sends an ASSIGNMENT FAILURE message on the old channel using the old ciphering mode and (if ciphered) the old algorithm and old key.

Conformance Requirement:

1	When the MS in the "not ciphered" mode receives the CIPHERING MODE COMMAND message with Ciphering Mode Setting information element set to "start ciphering", the MS shall load the cipher key stored in the SIM into the ME, use this key to start ciphering and deciphering with the algorithm indicated by the "algorithm identifier" field and, respond with a CIPHERING MODE COMPLETE message.

2	If the last timeslot of the message block containing a CIPHERING MODE COMMAND message occurs at time T, then the MS shall be ready to transmit the CIPHERING MODE COMPLETE message before T+500ms.

3	When the MS receives an ASSIGNMENT COMMAND message containing a Cipher Mode Setting IE after receipt of a CIPHERING MODE COMMAND message, the MS shall perform the assignment, use the commanded mode and/or algorithm on the new channel, and not change the ciphering key.

4	When the MS receives a HANDOVER COMMAND message containing a Cipher Mode Setting IE after receipt of a CIPHERING MODE COMMAND message, the MS shall perform the handover, use the commanded mode and/or algorithm on the new channel, and not change the ciphering key

5	When the MS in the ciphered mode receives the CIPHERING MODE COMMAND message with Cipher Mode Setting IE set to "no ciphering", the MS shall load the cipher key stored in the SIM into the ME, stop ciphering and deciphering and, respond with a CIPHERING MODE COMPLETE message.

6	When the MS receives an AUTHENTICATION REQUEST message it shall process the challenge information and send back an AUTHENTICATION RESPONSE message to the network. The new ciphering key calculated from the challenge information shall overwrite the previous one and be stored on the SIM before the AUTHENTICATION RESPONSE message is transmitted. The ciphering key stored in the SIM shall be loaded in to the ME when any valid CIPHERING MODE COMMAND is received.

7	When the MS in the non-ciphered mode receives the CIPHERING MODE COMMAND message with Cipher Mode Setting IE set to "no ciphering", the MS shall stop ciphering and deciphering and, respond with a CIPHERING MODE COMPLETE message.

8	If a handover fails then the operational parameters used when returning to the old channel are those applied before the HANDOVER COMMAND message was received.

9	If an assignment fails then the operational parameters used when returning to the old channel are those applied before the ASSIGNMENT COMMAND message was received.

Reference(s):

Conformance requirement 1: GSM 04.08 sections 3.4.7.2 and 4.3.2.2.

Conformance requirement 2: GSM 04.13 [version 1.2.0, section 5.2.7.]

Conformance requirement 3: GSM 04.08 section 3.4.3.1.

Conformance requirement 4: GSM 04.08 section 3.4.4.1.

Conformance requirement 5: GSM 04.08 sections 3.4.7.2 and 4.3.2.2.

Conformance requirement 5: GSM 04.08 sections 3.4.7.2 and 4.3.2.2.

Conformance requirement 6: GSM 04.08 section 4.3.2.2.

Conformance requirement 8: GSM 04.08 section 3.4.4.4.

Conformance requirement 9: GSM 04.08 section 3.4.3.3.

Related PICS/PIXIT Statement(s):

Supported ciphering algorithms: A5/1 and/or A5/2.

Initial Condition(s):

System Simulator:

1 cell.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and a known cipher key, K, stored.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The MS is paged. The MS shall send at least one CHANNEL REQUEST message. The SS sends an IMMEDIATE ASSIGNMENT and the MS shall answer with a PAGING RESPONSE message.

The SS sends a CIPHERING MODE COMMAND, ordering the MS to start ciphering with a supported algorithm. After transmission of this command the SS starts deciphering. The MS shall respond with a CIPHERING MODE COMPLETE message in the commanded ciphered mode using the cipher key K. After reception of the CIPHERING MODE COMPLETE the SS starts enciphering. The MS shall be ready to transmit the CIPHERING MODE COMPLETE message before 500 ms after the end of the CIPHERING MODE COMMAND message.

The term 'ready to transmit' is defined in GSM 04.13.

The SS sends an AUTHENTICATION REQUEST message to the MS. Cipher key L is calculated. The MS shall send an AUTHENTICATION RESPONSE message to the SS.

The SS sends a HANDOVER COMMAND (for a finely synchronised intra-BTS handover) with the Cipher Mode Setting IE set to "no ciphering". The MS shall transmit the HANDOVER COMPLETE on the commanded channel in non ciphered mode.

The SS sends an ASSIGNMENT COMMAND message containing a Cipher Mode Setting IE set to "start ciphering". The MS shall start transmitting on the commanded channel using the commanded algorithm and cipher key K. The MS shall transmit the ASSIGNMENT COMPLETE message.

The SS sends a CIPHERING MODE COMMAND, ordering the MS to stop ciphering. After transmission of this command the SS stops deciphering. The MS shall respond with a CIPHERING MODE COMPLETE message in non ciphered mode. After reception of the CIPHERING MODE COMPLETE the SS stops enciphering.

The SS sends a HANDOVER COMMAND (for a finely synchronised intra-BTS handover) with the Cipher Mode Setting IE set to "start ciphering". The MS shall transmit the HANDOVER COMPLETE on the commanded channel in ciphered mode using cipher key L.

The SS sends a HANDOVER COMMAND (for a finely synchronised intra-BTS handover) with the Cipher Mode Setting IE set to "start ciphering" and the algorithm identifier indicating the algorithm currently in use. The MS shall transmit the HANDOVER COMPLETE on the commanded channel in ciphered mode using the same algorithm as before the handover.

The SS sends an ASSIGNMENT COMMAND message containing a Cipher Mode Setting IE set to "no ciphering". The MS shall start transmitting on the commanded channel in non-ciphered mode. The MS shall transmit the ASSIGNMENT COMPLETE message.

The SS sends a CIPHERING MODE COMMAND, containing a Cipher Mode Setting IE set to "no ciphering". The MS shall respond with a CIPHERING MODE COMPLETE message.

The SS sends an AUTHENTICATION REQUEST message to the MS. Cipher key M is calculated. The MS shall send an AUTHENTICATION RESPONSE message to the SS.

The SS sends a HANDOVER COMMAND (for a finely synchronised intra-BTS handover) with the Cipher Mode Setting IE set to "start ciphering". The MS shall transmit the HANDOVER COMPLETE on the commanded channel using the commanded algorithm and cipher key L.

The SS sends a HANDOVER COMMAND (for a finely synchronised intra-BTS handover) with the Cipher Mode Setting IE set to "no ciphering". The SS does not activate the commanded channel. The MS's transmissions on the new channel need not be monitored. The MS shall transmit the HANDOVER FAILURE message on the 'old' channel using the 'old' algorithm and cipher key L.

The SS sends an ASSIGNMENT COMMAND message containing a Cipher Mode Setting IE set to "start ciphering". The SS does not activate the commanded channel. The MS's transmissions on the new channel need not be monitored. The MS shall transmit the ASSIGNMENT FAILURE message on the 'old' channel using the 'old' algorithm and cipher key L.

If the MS only supports one ciphering algorithm then the SS sends a CHANNEL RELEASE message.

If the MS supports more than one ciphering algorithm then the following steps are performed:

The SS sends an ASSIGNMENT COMMAND message containing a Cipher Mode Setting IE set to "start ciphering" and the Algorithm Identifier indicating a different supported algorithm to the one in use. The MS shall start transmitting on the commanded channel using the commanded algorithm. The MS shall transmit the ASSIGNMENT COMPLETE message.

The SS sends a HANDOVER COMMAND (for a finely synchronised intra-BTS handover) containing a Cipher Mode Setting IE set to "start ciphering" and the Algorithm Identifier indicating a different supported algorithm to the one in use. The MS shall transmit the HANDOVER COMPLETE on the commanded channel using the commanded algorithm.

The SS sends a CHANNEL RELEASE to end the test.

Maximum Duration of Test:

3 minutes.

Expected Sequence:

For MSs that only support one ciphering algorithm, the SS shall use step 57A. For MSs that support more than one ciphering algorithm, the SS shall use step 57B and the subsequent steps.

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMM ASSIGNMENT

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "Start Ciphering",

algorithm arbitrarily selected from those

supported by the MS. The SS starts

deciphering with the selected algorithm.

6	MS -> SS	CIPH MODE COMPLETE	Sent in ciphered mode using key 'K', the

stored cipher key, and the commanded

algorithm. This message shall be ready to

be transmitted before 500 ms after the

completion of step 5.

7	SS	-	The SS starts enciphering using key 'K'.

8	SS -> MS	AUTHENT REQUEST	Contains a new Ciphering Key Sequence

Number which is associated with the new

cipher key, 'L'.

9	MS -> SS	AUTHENT RESPONSE

10	SS -> MS	HANDOVER COMMAND	Includes Cipher Mode Setting IE set to

"No Ciphering".

11	SS		The SS activates the new channel without

ciphering.

12	MS -> SS	HANDOVER ACCESS	These four HANDOVER ACCESS messages are

13	MS -> SS	HANDOVER ACCESS	sent on the new channel in non ciphered

14	MS -> SS	HANDOVER ACCESS	mode.

15	MS -> SS	HANDOVER ACCESS

16	MS -> SS	HANDOVER COMPLETE	Sent in non ciphered mode on the new

channel.

17	SS -> MS	ASSIGNMENT COMMAND	Includes Cipher Mode Setting IE set to

"Start Ciphering", with algorithm arbitrarily

selected from those supported by the MS.

18	SS		The SS activates the new channel with

enciphering and deciphering enabled and using

cipher key 'K'.

19	MS -> SS	ASSIGNMENT COMPLETE	Sent on the new channel in ciphered mode

using key 'K' and the commanded algorithm.

20	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "No Ciphering". The

SS starts receiving in non ciphered mode.

21	MS -> SS	CIPH MODE COMPLETE	Sent in non ciphered mode.

22	SS	-	The SS starts transmitting in non ciphered

mode.

23	SS -> MS	HANDOVER COMMAND	Includes Cipher Mode Setting IE set to

"Start Ciphering", with algorithm 'X' arbitrarily

selected from those supported by the MS.

24	SS		The SS activates the new channel with

enciphering and deciphering enabled and using

cipher key 'L'.

25	MS -> SS	HANDOVER ACCESS	These four HANDOVER ACCESS messages are

26	MS -> SS	HANDOVER ACCESS	sent on the new channel in the non ciphered

27	MS -> SS	HANDOVER ACCESS	mode.

28	MS -> SS	HANDOVER ACCESS

29	MS -> SS	HANDOVER COMPLETE	Sent on the new channel in ciphered mode

using key 'L' and algorithm 'X'.

30	SS -> MS	HANDOVER COMMAND	Includes Cipher Mode Setting IE set to

"Start Ciphering", with algorithm identifier

set to 'X'.

31	SS		The SS activates the new channel with

enciphering and deciphering enabled.

32	MS -> SS	HANDOVER ACCESS	These four HANDOVER ACCESS messages are

33	MS -> SS	HANDOVER ACCESS	sent on the new channel in the non ciphered

34	MS -> SS	HANDOVER ACCESS	mode.

35	MS -> SS	HANDOVER ACCESS

36	MS -> SS	HANDOVER COMPLETE	Sent on the new channel in ciphered mode

using algorithm 'X'.

37	SS -> MS	ASSIGNMENT COMMAND	Includes Cipher Mode Setting IE set to

"No Ciphering".

38	SS		The SS activates the new channel without

ciphering.

39	MS -> SS	ASSIGNMENT COMPLETE	Sent in non-ciphered mode on the new

channel.

40	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "No Ciphering".

41	MS -> SS	CIPH MODE COMPLETE	Sent in non ciphered mode.

42	SS -> MS	AUTHENT REQUEST	Contains a new Ciphering Key Sequence

Number which is associated with the new

cipher key, 'M'.

43	MS -> SS	AUTHENT RESPONSE

44	SS -> MS	HANDOVER COMMAND	Includes Cipher Mode Setting IE set to

"Start Ciphering", with algorithm 'Y' arbitrarily

selected from those supported by the MS.

45	SS		The SS activates the new channel with

enciphering and deciphering enabled and using

cipher key 'L'.

46	MS -> SS	HANDOVER ACCESS	These four HANDOVER ACCESS messages are

47	MS -> SS	HANDOVER ACCESS	sent on the new channel in the non ciphered

48	MS -> SS	HANDOVER ACCESS	mode.

49	MS -> SS	HANDOVER ACCESS

50	MS -> SS	HANDOVER COMPLETE	Sent on the new channel in ciphered mode

using key 'L'

51	SS -> MS	HANDOVER COMMAND	Includes Cipher Mode Setting IE set to

"No Ciphering".

52	SS, MS		The SS does not activate the new channel

The MS's transmissions on the new channel

need not be monitored.

53	MS -> SS	HANDOVER FAILURE	sent on old channel using algorithm 'Y' and

key 'L'.

54	SS -> MS	ASSIGNMENT COMMAND	Includes Cipher Mode Setting IE set to

"Start Ciphering".

55	SS, MS		The SS does not activate the new channel

The MS's transmissions on the new channel

need not be monitored.

56	MS -> SS	ASSIGNMENT FAILURE	sent on old channel using algorithm 'Y' and

key 'L'.

57A	SS -> MS	CHANNEL RELEASE

57B	SS -> MS	ASSIGNMENT COMMAND	Includes Cipher Mode Setting IE set to

"Start Ciphering", with algorithm 'Z' arbitrarily

selected from those supported by the MS but

different to algorithm 'Y'.

58B	SS		The SS activates the new channel with

enciphering and deciphering enabled.

59B	MS -> SS	ASSIGNMENT COMPLETE	Sent on the new channel in ciphered mode

using key 'L' and algorithm 'Z'.

60B	SS -> MS	HANDOVER COMMAND	Includes Cipher Mode Setting IE set to

"Start Ciphering" and the algorithm identifier

set to 'Y'.

61B	SS		The SS activates the new channel with

enciphering and deciphering enabled.

62B	MS -> SS	HANDOVER ACCESS	These four HANDOVER ACCESS messages are

63B	MS -> SS	HANDOVER ACCESS	sent on the new channel in the non ciphered

64B	MS -> SS	HANDOVER ACCESS	mode.

65B	MS -> SS	HANDOVER ACCESS

66B	MS -> SS	HANDOVER COMPLETE	Sent on the new channel in ciphered mode

using key 'L' and algorithm 'Y'.

67B	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

Contents of ASSIGNMENT COMMAND message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101110

Channel Description

-	Channel Type				SDCCH/4

-		TDMA offset			Chosen arbitrarily, but different to the one in use.

-	Timeslot Number			Timeslot zero

-	Training Sequence Code			5 (same as the BCC)

-	Hopping				Single RF channel

-	ARFCN				Channel number 30

Power Command

-	Power level				Chosen arbitrarily by the test house

Cipher Mode Setting				As specified above.

All other information elements:			Not present

Contents of HANDOVER COMMAND message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101011

Cell Description

-	Network Colour Code			1

-	Base station Colour Code		5

-	BCCH Carrier Number			Channel Number 30

Channel Description

-	Channel Type				SDCCH/4

-		TDMA offset			Chosen arbitrarily, but different to the one in use.

-	Timeslot Number			Timeslot zero

-	Training Sequence Code			5 (same as the BCC)

-	Hopping				Single RF channel

-	ARFCN				Channel Number 30

Handover Reference

-	Handover Reference Value		Chosen arbitrarily by the test house.

Power Command

-	Power level				Chosen arbitrarily by the test house

Synchronisation Indication

-	Synchronisation indication		synchronized

-	Report observed time difference		not included

-	Normal cell indication			out of range ignored

Cipher Mode setting				As specified above.

All other information elements:			Not present

26.6.8.5	Ciphering mode / IMEI request

If the MS does not supply the IMEI when requested, the network will not know whether or not the MS is type approved, ie. whether or not it has passed any tests.

If the MS supplies its IMEI when not requested, this may cause calls to systematically fail.

Purpose:

To verify that the MS supplies its IMEI in the CIPHERING MODE COMPLETE message when it receives a CIPHERING MODE COMMAND message with a Cipher Response bit set to 'IMEI shall be included'.

To verify that the MS does not supply any Mobile Identity IE in the CIPHERING MODE COMPLETE message when it receives a CIPHERING MODE COMMAND message with a Cipher Response bit set to 'IMEI shall not be included'.

Conformance Requirement:

1:	When the MS receives the CIPHERING MODE COMMAND message with Cipher Response bit set to 'IMEI shall be included', the MS shall include the IMEI in the Mobile Identity IE in the CIPHERING MODE COMPLETE message.

2:	When the MS receives the CIPHERING MODE COMMAND message with Cipher Response bit set to 'IMEI shall not be included', the MS shall not include the Mobile Identity IE in the CIPHERING MODE COMPLETE message.

Reference(s):

Conformance requirement 1: GSM 04.08 sections 3.4.7.2 and 9.1.10.1.

Conformance requirement 2: GSM 04.08 section 9.1.10.1.

Related PICS/PIXIT Statement(s):

-

Initial Condition(s):

System Simulator:

1 cell.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The MS is paged. It shall send at least one CHANNEL REQUEST message. The SS sends an IMMEDIATE ASSIGNMENT and the MS shall answer with a PAGING RESPONSE. Then the SS sends a CIPHERING MODE COMMAND indicating 'No ciphering' and with the Cipher Response bit set to 'IMEI shall not be included'. The MS shall respond with a CIPHERING MODE COMPLETE message that does not include the Mobile Identity IE.

Then the SS sends a CIPHERING MODE COMMAND indicating 'No ciphering' and with the Cipher Response bit set to 'IMEI shall be included'. The MS shall respond with a CIPHERING MODE COMPLETE message that carries the IMEI in the Mobile Identity IE.

Finally the SS sends a CHANNEL RELEASE to end the test.

Maximum Duration of Test:

10 seconds.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMM ASSIGNMENT

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "No Ciphering".

Cipher Response = "IMEI shall not be

included".

6	MS -> SS	CIPH MODE COMPLETE	Shall not include Mobile Identity IE.

7	SS -> MS	CIPH MODE COMMAND	Cipher Mode Setting = "No Ciphering".

Cipher Response = "IMEI shall be included".

8	MS -> SS	CIPH MODE COMPLETE	Shall include one Mobile Identity IE carrying

the MS's IMEI.

9	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

None.

26.6.9	Test of additional assignment

The Additional Assignment procedure is used to change an MS's channel configuration from Lm + ACCH to Lm + Lm + ACCH. It is therefore only relevant to those mobiles which perform such an operation.

No tests are specified at the moment.

26.6.10	Test of partial release

The Partial Release procedure is used to change an MS's channel configuration from Lm + Lm + ACCH to Lm + ACCH. It is therefore only relevant to those mobiles which perform such an operation.

No tests are specified at the moment.

26.6.11	Test of classmark

26.6.11.1	Classmark change

This procedure allows the MS to indicate to the network that a change in the classmark (e.g. due to addition of power amplification) has taken place.

Purpose:

To verify that if the RF power capability or any other capability indicated in a Classmark IE of the MS is changed during a call, the change is communicated on the DCCH to the network.

To verify that if the RF power capability or any other capability indicated in a Classmark IE of the MS is changed in idle mode, the out of date capabilities are not communicated to the network during RR connection establishment.

Conformance Requirement:

If the RF power capability of the MS is changed during a call, this change shall be signalled to the network.

If the RF power capability of the MS is changed in idle mode, the up to date RF power capability shall be signalled to the network during RR connection establishment.

Reference(s):

GSM 04.08 section 3.4.10.

Related PICS/PIXIT Statement(s):

Support for more than one RF power class: yes/no.

Support for state U10 of the Call Control protocol.

Initial Condition(s):

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated and with no additional power amplification applied.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated and with no additional power amplification applied.

Method of Test:

With the MS in idle mode, the RF power capability shall be changed by the addition of power amplification, after which the MS is made to originate a call. The new RF power capability shall be included in the CM SERVICE REQUEST message. After the call has reached the Call Control state U10, the RF power capability of the MS is changed by removal of the additional power amplification. The MS shall send a CLASSMARK CHANGE message indicating the new RF power capability. The RF power capability is then changed by adding the power amplification. The MS shall again send a CLASSMARK CHANGE message indicating the new RF power capability. The call is then released by the SS.

With the MS in idle mode, the power amplification is removed. The SS then pages the MS, which in the PAGING RESPONSE message shall indicate the correct RF power capability.

Finally the SS transmits a CHANNEL RELEASE to end the test.

Maximum Duration of Test:

-

Expected Sequence:

Step	Direction	Message	Comments

1	--------	--------	Add power amplification

2	--------	--------	The MS shall be made to

originate a call

3	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Originating call"

NECI not equal to one

4	SS -> MS	IMM ASSIGNMENT

5	MS -> SS	CM SERVICE REQUEST	The "Mobile Station Classmark 2"

IE shall indicate the new RF power

capability

6	SS -> MS	CM SERVICE ACCEPT

7	MS -> SS	SETUP

8	SS -> MS	CALL PROCEEDING

9	SS -> MS	ALERT

10	SS -> MS	ASSIGNMENT COMMAND

11	MS -> SS	ASSIGNMENT COMPLETE

12	SS -> MS	CONNECT

13	MS -> SS	CONNECT ACKNOWLEDGE

14	--------	--------	Remove the power amplification

15	MS -> SS	CLASSMARK CHANGE	The "Mobile Station Classmark 2"

IE shall indicate the new power

capability

16	--------	--------	Add power amplification

17	MS -> SS	CLASSMARK CHANGE	The "Mobile Station Classmark 2"

IE shall indicate the new power

capability

18	SS -> MS	CHANNEL RELEASE

19	--------	--------	Remove the power amplification

--------	--------	The SS waits 12 seconds to allow the

MS to perform cell reselection

20	SS -> MS	PAG REQ TYPE 1

21	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

22	SS -> MS	IMM ASSIGNMENT

23	MS -> SS	PAGING RESPONSE	The "Mobile Station Classmark 2"

IE shall indicate the new power

capability

24	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

None

26.6.11.2	Classmark interrogation

This procedure allows the network to request the MS to supply all its classmark information to the network.

Networks may systematically use this procedure (eg. during location updating) and, it if is incorrectly implemented in the MS, the basic connection establishment procedure may systematically fail.

Purpose:

To verify that if the network requests the MS to supply all its classmark information then this information is communicated on the DCCH to the network.

Conformance Requirement:

On receipt of a CLASSMARK ENQUIRY message, the MS sends a CLASSMARK CHANGE message to the network containing the Mobile Station Classmark 2 information element and depending upon the contents of this information element, possibly the Mobile Station Classmark 3 information element.

Reference(s):

GSM 04.08 sections 3.4.11 and 9.1.11.1.

GSM 04.13 [version 1.2.0, section 5.2.9]

Related PICS/PIXIT Statement(s):

Contents of Mobile Station Classmark 2 information element.

Existence of Mobile Station Classmark 3 information element: yes/no.

Contents of Mobile Station Classmark 3 information element.

Switch off button: yes/no.

Initial Condition(s):

System Simulator:

1 cell, default parameters.

Mobile Station:

"Idle, updated", with TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The MS is switched off (or has its power removed).

The SS then sets the IMSI attach-detach flag in the SYSTEM INFORMATION messages so that the MS shall perform a location update when switched on.

The MS is switched on (or its power is re-applied). The MS then initiates a location update attempt. After the mobile has sent the LOCATION UPDATING REQUEST message, the SS transmits a CLASSMARK ENQUIRY message. The MS shall be ready to transmit the CLASSMARK CHANGE message before 300 ms after the end of the CLASSMARK ENQUIRY message.

The term 'ready to transmit' is defined in GSM 04.13.

Then the SS transmits a LOCATION UPDATING ACCEPT message that does not contain a Mobile Identity IE.

Maximum Duration of Test:

2 minutes.

Expected Sequence:

Step	Direction	Message	Comments

1	MS		The MS is switched off (or has its power

removed).

2	SS		IMSI attach-detach flag changed.

3	MS		The MS is switched on (or its power

is re-applied).

4	MS -> SS	CHANNEL REQUEST

5	SS -> MS	IMMEDIATE ASSIGNMENT

6	MS -> SS	LOCATION UPDATING REQ

7	SS -> MS	CLASSMARK ENQUIRY

8	MS -> SS	CLASSMARK CHANGE	Contents as defined for default message.

This message shall be ready to be transmitted

before 300 ms after the completion of step 7.

9	SS -> MS	LOCATION UPDATING ACC

10	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

None

Default Message Contents

Contents of LOCATION UPDATING ACCEPT message:

Protocol Discriminator				MM message

Skip Indicator					0000

Message Type					00000010

Location Area Identification

-	Mobile Country Code			001 decimal

-	Mobile Network Code			01 decimal

-	Location Area Code			0001H

Mobile Identity					Not present

Follow on proceed					Not present

26.6.12	Test of channel release

The purpose of this procedure is to deactivate the dedicated channels in use. When the channels are released, the MS returns to the CCCH configuration, idle mode.

26.6.12.1	Channel release / SDCCH

Purpose:

To verify that the MS is able to correctly release an SDCCH after having received a CHANNEL RELEASE message.

Conformance Requirement:

After the acknowledgement of the Layer 2 disconnection by the network, the MS shall not produce any further RF-transmission.

Reference(s):

GSM 04.08 section 3.5.1.

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The MS is paged and allocated a dedicated channel and the Layer 2 signalling link is established. The SS then sends a CHANNEL RELEASE message, after which the MS shall initiate a Layer 2 disconnection process on the main signalling link. After the acknowledgement of the Layer 2 disconnection by the SS, the MS shall stop transmission of Layer 2 messages. This is verified for 3 seconds. The MS shall return to the idle state, which is verified through the paging procedure to which the MS shall respond.

Maximum Duration of Test:

20 seconds

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

3	SS -> MS	IMM ASSIGNMENT	Channel Type = SDCCH/8

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	CHANNEL RELEASE	With a valid RR cause value.	6	MS -> SS	DISC

7	SS -> MS	UA

--------	--------	The SS verifies for 3 seconds that the

MS does not produce any Layer 2 messages

--------	--------	The SS waits 12 seconds to allow the

MS to perform cell reselection

8	SS -> MS	PAG REQ TYPE 1

9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

10	SS -> MS	IMM ASSIGNMENT

11	MS -> SS	PAGING RESPONSE

12	SS -> MS	CHANNEL RELEASE

[Editorial note : A timelimit of 3 seconds for checking the RF-transmissions has been added in all the channel release tests ]

Specific Message Contents:

None

26.6.12.2	Channel release / SDCCH - no L2 ACK

Purpose:

To verify that the MS is able to correctly release a SDCCH after having received a CHANNEL RELEASE message, even if the SS does not L2 acknowledge the L2 DISC frame.

Conformance Requirement:

After the expiry of timer T3110 the MS shall not produce any further RF-transmission.

Reference(s):

GSM 04.08 section 3.5.1.

Related PICS/PIXIT Statement(s):

None.

Initial Condition(s):

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The MS is paged and allocated a dedicated channel and the Layer 2 signalling link is established. The SS then sends a CHANNEL RELEASE message, after which the MS shall send at least 2 L2 DISC frames. The SS does not acknowledge any of the L2 DISC frames. After 2 seconds, the SS verifies for 3 seconds that the MS has stopped transmission of Layer 2 messages. The MS shall return to the idle state, which is verified through the paging procedure to which the MS shall respond.

Maximum Duration of Test:

25 seconds.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

3	SS -> MS	IMM ASSIGNMENT	Channel Type = SDCCH/8

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	CHANNEL RELEASE	With a valid RR cause value.

6	MS - > SS	DISC	The MS shall send at least 2 L2

DISC frames, to which the SS does

not respond. After a period of 2

seconds, the SS verifies for 3

seconds that the MS does not

produce any further Layer 2 messages

--------	--------	The SS waits 12 seconds to allow the

MS to perform cell reselection

7	SS -> MS	PAG REQ TYPE 1

8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

9	SS -> MS	IMM ASSIGNMENT

10	MS -> SS	PAGING RESPONSE

11	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

None.

26.6.12.3	Channel release / TCH-F

Purpose:

To verify that the MS is able to correctly release a full-rate TCH after having received a CHANNEL RELEASE message.

Conformance Requirement:

After the acknowledgement of the Layer 2 disconnection by the network the MS shall not produce any further RF-transmission.

Reference(s):

GSM 04.08 section 3.5.1.

Related PICS/PIXIT Statement(s):

Supported rate of TCH: TCH/F.

Initial Condition(s):

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The MS is paged and allocated a dedicated channel and the Layer 2 signalling link is established. The SS then sends a CHANNEL RELEASE message, after which the MS shall initiate a Layer 2 disconnection process on the main signalling link. After the acknowledgement of the Layer 2 disconnection by the SS, the MS shall stop transmission of Layer 2 messages. This is verified for 3 seconds. The MS shall return to the idle state, which is verified through the paging procedure to which the MS shall respond.

Maximum Duration of Test:

20 seconds

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

3	SS -> MS	IMM ASSIGNMENT	Channel Type = "Bm + ACCHs"

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	CHANNEL RELEASE	With a valid RR cause value.

6	MS -> SS	DISC

7	SS -> MS	UA

--------	--------	The SS verifies for 3 seconds that the

MS does not produce any Layer 2 messages

--------	--------	The SS waits 12 seconds to allow the

MS to perform cell reselection

8	SS -> MS	PAG REQ TYPE 1

9	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

10	SS -> MS	IMM ASSIGNMENT

11	MS -> SS	PAGING RESPONSE

12	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

None

26.6.12.4	Channel release / TCH-F - no L2 ACK

Purpose:

To verify that the MS is able to correctly release a TCH/F after having received a CHANNEL RELEASE message, even if the SS does not L2 acknowledge the L2 DISC frame.

Conformance Requirement:

After the expiry of timer T3110 the MS shall not produce any further RF-transmission.

Reference(s):

GSM 04.08 section 3.5.1.

Related PICS/PIXIT Statement(s):

Supported rate of TCH: TCH/F.

Initial Condition(s):

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in the "idle, updated" state, with a TMSI allocated.

Foreseen Final State of the MS:

"Idle, updated", with TMSI allocated.

Method of Test:

The MS is paged and allocated a dedicated channel and the Layer 2 signalling link is established. The SS then sends a CHANNEL RELEASE message (with cause "abnormal release, unspecified"), after which the MS shall send at least 2 L2 DISC frames. The SS does not acknowledge any of the L2 DISC frames. After 2 seconds, the SS verifies for 3 seconds that the MS has stopped transmission of Layer 2 messages. The MS shall return to the idle state, which is verified through the paging procedure to which the MS shall respond.

Maximum Duration of Test:

25 seconds.

Expected Sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAG REQ TYPE 1

2	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

3	SS -> MS	IMM ASSIGNMENT	Channel Type = "Bm + ACCHs"

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	CHANNEL RELEASE	Cause value = "Abnormal release,

unspecified".

6	MS -> SS	DISC	The MS shall sned at

least 2 L2 DISC frames, to which

the SS does not respond. After a

period of 2 seconds, the SS

verifies for 3 seconds that the MS

does not produce any further

Layer 2 messages

--------	--------	The SS waits 12 seconds to allow the

MS to perform cell reselection

7	SS -> MS	PAG REQ TYPE 1

8	MS -> SS	CHANNEL REQUEST	Establ. Cause = "Answer to paging"

9	SS -> MS	IMM ASSIGNMENT	Channel Type = SDCCH/8

10	MS -> SS	PAGING RESPONSE

11	SS -> MS	CHANNEL RELEASE

Specific Message Contents:

None.

26.6.13	Default contents of GSM 900 layer 3 messages for RR tests

This section contains the default values of L3 messages, which unless indicated otherwise in section 26.6 shall be transmitted by the system simulator and which are required to be received from the MS under test. These values are used in order to be consistent with the phase 2 version of 26.6.

The neccessary L3 messages are listed in alphabetic order, with the exception of the SYSTEM INFORMATION messages, where it is the information elements which are listed in alphabetic order (this is because some information elements occur in several SYSTEM INFORMATION types).

In this section, decimal values are normally used. However, sometimes a hexadecimal value, indicated by an 'H', or a binary value, indicated by a 'B' is used.

Default SYSTEM INFORMATION:

Note:	SYSTEM INFORMATION 2 bis, SYSTEM INFORMATION 5 bis, SYSTEM INFORMATION 7, and SYSTEM INFORMATION 8 messages are not used.

Cell A

Contents of information elements in SYSTEM INFORMATION TYPE 1 to 6 messages for cell A.

(CBCH) Channel Description			Not present

(CBCH) Mobile Allocation				Not present

Cell Channel Description

-	Format identifier			bit map 0

-	Cell Allocation ARFCN			Channel Numbers 20, 30, 50 and 70.

Cell Identity

-	Cell Identity Value			0001H

Cell Options

-	Power Control Indicator			Power Control Indicator is not set

-	DTX Indicator				MS shall not use DTX

-	Radio-Link-Timeout			8 SACCH blocks

Cell Selection Parameters

-	Cell-Reselect-Hysteresis			12 dB

-	MX-TXPWR-MAX-CCH			Minimum level

-	ACS					No addition cell parameters are present in

SYSTEM INFORMATION messages 7 and 8.

-	NECI					New establishment causes not supported

-	RXLEV-ACCESS-MIN			Minimum level

Control Channel Description

-	Attach-Detach allowed			No Attach/Detach

-	BS-AG-BLKS-RES			0 blocks reserved for access grant

-	CCCH-CONF				1 basic physical channel used for

CCCH, combined with SDCCHs

-	BS-PA-MFRMS				5 multiframe periods for transmission of

paging messages

-	T3212 Timeout value			Infinite

L2 pseudo length

-	System information 1			21

-	System information 2			22

-	System information 3			18

-	System information 4			12

Location Area Identification

-	Mobile Country Code			001 decimal

-	Mobile Network Code			01 decimal

-	Location Area Code			0001H

Message Type

-	System information 1			00011001

-	System information 2			00011010

-	System information 3			00011011

-	System information 4			00011100

-	System information 5			00011101

-	System information 6			00011110

Neighbour Cells Description

-	Format identifier			bit map 0

-	BCCH Allocation Sequence		0

-	BCCH Allocation ARFCN			Channels numbers 10, 20, 80, 90,

100, 110 and 120

-	EXT-IND				This IE carries the complete BA

NCC Permitted					0000 0010

RACH Control Parameters

-	Max Retrans				Max 1 retrans

-	Tx-integer				5 slots used

-	Cell Barred for Access			Cell is not barred

-	Call Reestablishment Allowed		Not allowed

-	Access Control Class			Access is not barred

-	Emergency Call allowed			Yes

SI 1 rest octets					Not used (all bits are set to spare)

SI 2 rest octets					Not used (all bits are set to spare)

SI 3 rest octets					Not used (all bits are set to spare)

SI 4 rest octets					Not used (all bits are set to spare)

Default settings for cell A:

Downlink input level				63 dBmicroVolt emf

Uplink output power				minimum supported by the MS's power

class

Propagation profile					static

BCCH/CCCH carrier number			20

Cell B

The contents of SYSTEM INFORMATION TYPE 1 to 6 messages for cell B are identical to those of cell A with the following exceptions:

Cell Channel Description

-	Format Identifier			Bit map 0

-	Cell Allocation ARFCN			Channel Number 10

Note:	This IE needs modification when used in handover tests which command the MS to go to a frequency hopping channel in cell B.

Cell Identity

-	Cell Identity Value			0002H

Default settings for cell B:

Downlink input level				53 dBmicroVolt emf

Uplink output power				minimum supported by the MS's power

class

Propagation profile					static

BCCH/CCCH carrier number			10

Contents of ALERTING message (SS to MS):

Protocol Discriminator				Call Control,

Transaction Identifier

TI value					As used in the SETUP message

TI flag						1 (destination side)

Message Type					00000001

All other information elements			Not present

Contents of ASSIGNMENT COMMAND message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101110

Channel Description

-	Channel Type and TDMA offset		Bm + ACCHs

-	Timeslot Number			Chosen randomly by the test house

-	Training Sequence Code			Chosen randomly by the test house

-	Hopping				Single RF channel

-	ARFCN				Channel number 30

Power Command

-	Power level				Chosen arbitrarily by the test house

All other information elements			Not present

Contents of ASSIGNMENT COMPLETE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101001

RR Cause

-	RR Cause Value			Normal event

Contents of ASSIGNMENT FAILURE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101111

RR Cause

-	RR Cause Value			Depending on test

Contents of AUTHENTICATION REQUEST message:

Protocol Discriminator				MM message

Skip Indicator					0000

Message Type					00010010

Ciphering Key Sequence Number

-	Key Sequence				Chosen randomly by the test house

from the range 0 to 6

Authentication Parameter RAND

-	RAND value				Chosen arbitrarily by the test house

Contents of AUTHENTICATION RESPONSE message:

Protocol Discriminator				MM message

Skip Indicator					0000

Message Type					0X010100

Other information element(s)				Not checked

Contents of CALL PROCEEDING message:

Protocol Discriminator				Call Control

Transaction Identifier

TI value					As used in the SETUP message

TI flag						1 (destination side)

Message Type					00000010

All other information elements			Not present

Contents of CHANNEL MODE MODIFY message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00010000

Channel Description

-	Channel Type and TDMA offset		Depending on test

-	Timeslot Number			Chosen randomly by the test house

-	Training Sequence Code			Chosen randomly by the test house

-	Hopping				Single RF channel

-	ARFCN				Channel number 30

Channel Mode

-	Mode					Depending on test

Contents of CHANNEL MODE MODIFY ACKNOWLEDGE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00010111

Channel Description

-	Channel Type and TDMA offset		Depending on test

-	Timeslot Number			Same as in the CHANNEL MODE MODIFY

message

-	Training Sequence Code			Same as in the CHANNEL MODE MODIFY

message

-	Hopping				Single RF channel

-	Frequency Band			Band number 0

-	ARFCN				Channel number 30

Channel Mode

-	Mode					Same as in the CHANNEL MODE MODIFY

message

Contents of CHANNEL RELEASE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00001101

RR Cause

-	RR Cause Value			Normal event

Contents of CHANNEL REQUEST message

Establishment Cause				Not checked

Random Reference				Not checked

Contents of CIPHERING MODE COMMAND message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00110101

Cipher Mode Setting

-	algorithm identifier			cipher with A5/1

-	SC					Start ciphering

Cipher Response					IMEI shall not be included

Contents of CIPHERING MODE COMPLETE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00110010

Mobile Identity					Not present

Contents of the CLASSMARK CHANGE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					000100110

Mobile Station Classmark 2				See PICS/PIXIT

Mobile Station Classmark 3				For presence and contents see PICS/PIXIT

Contents of CM SERVICE ACCEPT message:

Protocol Discriminator				Mobility Management

Skip Indicator					0000

Message Type					00100001

Contents of CM SERVICE REQUEST message

Protocol Discriminator				Mobility Management

Skip Indicator					0000

Message Type					0X100100

Other information elements				Not checked

Contents of CONNECT message (SS to MS)

Protocol Discriminator				Call Control

Transaction Identifier

TI value					As used in the SETUP message

TI flag						1

Message Type					00000111

All other information elements			Not present

Contents of CONNECT ACKNOWLEDGE message (MS to SS)

Protocol Discriminator				Call Control

Transaction Identifier

TI value					As used in the SETUP message

TI flag						0

Message Type					0X001111

Contents of HANDOVER ACCESS message:

Handover Reference				Equal to the value included in

the Handover Command message

Contents of HANDOVER COMMAND message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101011

Cell Description

-	Network Colour Code			1

-	Base station Colour Code		5

-	BCCH Carrier Number			Set to the BCCH carrier number of cell B.

(one of 10, 20, 80, 90, 100, 110 or 120)

Channel Description

-	Channel Type and TDMA offset		Bm + ACCHs

-	Timeslot Number			Chosen randomly by the test house

-	Training Sequence Code			Chosen randomly by the test house

-	Hopping				Single RF channel

-	ARFCN				Chosen randomly by the test house from those

supported on the target cell

Handover Reference

-	Handover Reference Value		Chosen arbitrarily by the test house.

Power Command

-	Power level				Chosen arbitrarily by the test house

All other information elements			Not present

Contents of HANDOVER COMPLETE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101100

RR cause					Normal event

Time difference					Not present

Contents of HANDOVER FAILURE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101000

RR cause					Dependent on the test

Contents of IMMEDIATE ASSIGNMENT message:

L2 pseudo length					This is the sum of the lengths of all the

information elements present in the

message except for the IA rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 11.

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00111111

Page Mode

-	Page Mode				Normal Paging

Channel Description

-	Channel Type and TDMA offset		For non-combined CCCH/SDCCH (see initial

conditions), SDCCH/8, with subchannel

chosen randomly by

the testhouse

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, with subchannel

chosen randomly by the test house;

-	Timeslot Number			For non-combined CCCH/SDCCH (see initial

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, timselot zero.

-	Training Sequence Code			For non-combined CCCH/SDCCH (see initial

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, TSC=5 (same as

the BCC).

-	Hopping				Single RF channel

-	ARFCN				For non-combined CCCH/SDCCH (see initial

conditions), Channel number 30;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, Channel number 20.

Request Reference				Pertaining to last Channel Request

sent by the MS

Timing Advance

-	Timing advance value			30 bit periods

Mobile Allocation

-	Length				0

Starting Time					Not present

IA rest octets					Not used (all bits set to spare)

Contents of IMMEDIATE ASSIGNMENT EXTENDED message:

L2 pseudo length					This is the sum of the lengths of all the

information elements present in the

message except for the IAX rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 18.

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00111001

Page Mode

-	Page Mode				Normal Paging

Channel Description 1

- Channel Type and TDMA offset		For non-combined CCCH/SDCCH (see test

conditions), SDCCH/8, with subchannel

chosen randomly by the test house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, with subchannel

chosen randomly by the test house;

- Timeslot Number			For non-combined CCCH/SDCCH (see test

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, timselot zero.

- Training Sequence Code		For non-combined CCCH/SDCCH (see initial

conditions), chosen randomly by the test

house;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, TSC=5 (same as the

BCC).

- Hopping				Single RF channel

- ARFCN				For non-combined CCCH/SDCCH (see initial

conditions),Channel number 30;

For combined CCCH/SDCCH (default SS

conditions), SDCCH/4, Channel number 20.

Request Reference 1				Pertaining to last Channel Request sent

by the MS

Timing Advance 1

-	Timing advance value			Chosen arbitrarily by the test house

Channel Description 2

-	Channel Type and TDMA offset		Same channel type as in Channel

Description 1, but different TDMA offset to that

in Channel Description 1.

-	Timeslot Number			equal to the value in Channel Description 1.

-	Training Sequence Code			equal to the value in Channel Description 1.

-	Hopping				Single RF channel

-	ARFCN				equal to the value in Channel Description 1.

Request Reference 2				Not pertaining to any Channel Requests sent

by the MS.

Timing Advance 2

-	Timing advance value			Chosen arbitrarily by the test house

Mobile Allocation

-	Length				0

Starting Time					Not present

IAX rest octets					Not used (all bits set to spare)

Contents of IMMEDIATE ASSIGNMENT REJECT message:

L2 pseudo length					19

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00111010

Page Mode

-	Page Mode				Normal Paging

Request Reference				Pertaining to last Channel Request sent

by the MS

Wait Indication					0 seconds

Request Reference				Not pertaining to the MS under test

Wait Indication					0 seconds

Request Reference				Not pertaining to the MS under test

Wait Indication					0 seconds

Request Reference				Not pertaining to the MS under test

Wait Indication					0 seconds

IAR rest octets					Not used (all bits set to spare)

Contents of LOCATION UPDATING REQUEST message:

Protocol Discriminator				MM message

Skip Indicator					0000

Message Type					0X001000

Other information elements		Not checked

Contents of PAGING REQUEST TYPE 1 message:

L2 pseudo length					This is the sum of the lengths of all the

information elements present in the

message except for the P1 rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 9.

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00100001

Page Mode

-	Page Mode				Normal Paging

Channels needed

-	first channel				any channel

-	second channel				any channel

Mobile Identity 1

-	odd/even indication			Even

-	Type of Identity				TMSI

-	Identity Digits				TMSI previously allocated to MS

Mobile Identity 2					Not present

P1 rest octets					Not used (all bits set to spare)

Contents of PAGING REQUEST TYPE 2 message:

L2 pseudo length					This is the sum of the lengths of all the

information elements present in the

message except for the P2 rest octets and

L2 pseudo length IEs. For the default

message the L2 pseudo length is 11.

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00100010

Page Mode

-	Page Mode				Normal Paging

Channels needed

-	first channel				any channel

-	second channel				any channel

Mobile Identity 1

-	TMSI value				TMSI previously allocated to MS

Mobile Identity 2

-	TMSI value				TMSI not allocated to MS

Mobile Identity					Not present

P2 rest octets					Not used (all bits set to spare)

Contents of PAGING REQUEST TYPE 3 message:

L2 pseudo length					19

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00100100

Page Mode

-	Page Mode				Normal Paging

Channels needed

-	first channel				any channel

-	second channel				any channel

Mobile identity 1

-	TMSI value				TMSI previously allocated to MS

Mobile identity 2

-	TMSI value				TMSI not allocated to MS

Mobile identity 3

-	TMSI value				TMSI not allocated to MS

Mobile identity 4

-	TMSI value				TMSI not allocated to MS

P3 rest octets					Not used (all bits set to spare)

Contents of PAGING RESPONSE message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00100111

Ciphering Key Sequence Number

-	Key Sequence				Key sequence number previously allocated

to MS, or '111' if no key is available.

Mobile Station Classmark 2

Mobile Identity

-	odd/even indication			Even

-	Type of identity				TMSI

-	Identity Digits				TMSI previously allocated to MS

Contents of PHYSICAL INFORMATION message:

Protocol Discriminator				RR Management

Skip Indicator					0000

Message Type					00101101

Timing advance					20 bit periods

Contents of SETUP message; (MS to SS; for full rate speech)

Protocol Discriminator				Call Control

Transaction Identifier

TI value					any value from the set {0, ..., 6}

TI flag						0

Message Type					0X000101

Other information elements				Not checked

Contents of SETUP message; (SS to MS; for full rate speech)

(* This message is not used in this direction in the RR tests- yet! It is put into hidden text in case a new test needs it.*)

Protocol Discriminator				Call Control

Transaction Identifier

TI value					chosen randomly by the test house from the

set {0, ..., 6}

TI flag						0

Message Type					00000101

BC repeat indicator				Not present

Bearer capability 1

IEI						04h

length						01h

information transfer capability		speech

transfer mode				circuit mode

coding standard				GSM standardized coding

radio channel requirement			full rate channel

Signal							[Not present]

All other information elements			Not present