26.7	Elementary procedures of mobility management

The tests are based on GSM 04.08 and GSM 03.03.

In this section, when the expected sequence require that "a mobile originated CM connection is attempted", it shall be for a service other than emergency call.

In this section, a initial CM message is either a SETUP message, a REGISTER message or a CP-DATA message (in that case the acknowledged mode of operation on SAPI 3 will have be established and this message will be sent on SAPI 3).

26.7.1	TMSI reallocation

The intention of the TMSI Reallocation procedure is to assign a new temporary identity for the MS. If the message is not understood by the MS, the network could not establish a link to the MS. As this is a common MM procedure, it can be initiated at any time.

[Editorial Note:	- it is not checked that this procedure can be initiated at any time

]

Conformance requirement:

1)	A Mobile Station shall acknowledge a new TMSI when explicitly allocated during a location updating procedure or an incoming call.

2)	If the power supply is interrupted the Mobile Station shall store the TMSI in non-volatile memory.

3)	A Mobile Station shall answer paging with this TMSI and includes it in the Paging Response message.

Test purpose:

To verify that the MS is able to receive and acknowledge a new TMSI by means of an explicit TMSI reallocation procedure.

To verify that the MS has stored the TMSI in a non-volatile memory.

The implicit reallocation procedure is tested in section 26.7.4.1.

Reference(s):

GSM 04.08 section 4.3.1, GSM 03.03 section 2.

Related PICS/PIXIT statement(s):

Switch off button Yes/No

Way to bring the MS into service

Initial conditions:

System Simulator:

Two cells A and B, belonging to different location areas a and b, default parameters.

Mobile Station:

The MS has valid TMSI (= TMSI1), CKSN, Kc. It is "idle updated" on cell B.

Foreseen final state of the MS:

The MS has a valid TMSI (= TMSI1), CKSN, Kc. It is "idle updated" on cell A.

Method of test:

The MS is paged in cell B and the ciphering mode is established. An explicit TMSI reallocation procedure is performed. The channel is released. The MS is switched off and its power supply is interrupted for 10 seconds. The power supply is resumed and the MS is switched on. The system simulator checks, by paging, whether the MS has stored the received TMSI.

The MS is made to select cell A. A normal location updating procedure is performed in cell A. An explicit TMSI reallocation procedure is performed and then the location updating procedure is accepted by the SS. The system simulator checks, by paging, whether the MS has stored the allocated TMSI.

Maximum duration of test:

2 minutes

Expected sequence:

Step	Direction	Message	Comments

The following messages are sent and shall be

received on cell B

1	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" = TMSI1

2	MS -> SS	CHANNEL REQUEST	Establishment Cause: Answer to paging.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	CIPH MODE COMMAND	The SS starts deciphering.

6	MS -> SS	CIPH MODE COMPLETE	The SS starts enciphering.

7	SS -> MS	TMSI REALLOCATION CMD	"Mobile identity"= new TMSI

(TMSI2) different from TMSI 1.

8	MS -> SS	TMSI REALLOCATION CMP

9	SS -> MS	CHANNEL RELEASE	After the sending of this message,

the SS waits for the disconnection

of the main signalling link.

10	MS		If possible (see PICS), the MS is switched off and its

power supply is interrupted for

10 seconds.

11	MS		The MS is brought into service

12	SS		The SS waits an amount of time

which is enough to guarantee that

the MS is in service (listening to

its paging subchannel)

13	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" = TMSI2.

14	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer to

paging.

15	SS -> MS	IMMEDIATE ASSIGNMENT

16	MS -> SS	PAGING RESPONSE	"Mobile identity" =TMSI2.

17	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

The following messsages are sent and shall be

received on cell A

18	SS		The RF level of cell B is lowered

until the MS selects cell A. The RF

level of cell B is set sufficiently

low to ensure that cell B is not

suitable as defined in GSM 05.08

section 6.6.2

19	MS -> SS	CHANNEL REQUEST	Establishment cause: All

other cases.

20	SS -> MS	IMMEDIATE ASSIGNMENT

21	MS -> SS	LOCATION UPDATING REQ	location updating type= normal,

"ciphering key sequence number"=

CKSN, LAI= b, "mobile identity"=

TMSI2.

22	SS -> MS	TMSI REALLOCATION CMD	TMSI = TMSI1

23	MS -> SS	TMSI REALLOCATION CMP

24	SS -> MS	LOCATION UPDATING ACC	This message does not contain the

optional Mobile Identity field.

25	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link. The SS waits an

amount of time which is enough to

guarantee that the MS is "idle updated" on cell A

26	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains

the new TMSI (= TMSI1).

27	MS -> SS	CHANNEL REQUEST	"Establishment cause" :

Answer to paging.

28	SS -> MS	IMMEDIATE ASSIGNMENT

29	MS -> SS	PAGING RESPONSE	"Mobile identity" IE contains

the new TMSI (= TMSI1).

30	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.2	Authentication

The purpose of this procedure is to verify the user identity. A correct response is essential to guarantee the establishment of the connection. If not, the connection will drop.

[Editorial Note:	- the reference to the network timer T3260 has been removed as the mobile station has to answer as quick as possible. If an upper limit is defined for phase 2 then it will be added.

- in the old version it was not clear whether the MS was paged with IMSI or TMSI. This new version includes now both.

- it is not checked that the MS, when the flag "authentication failure" is set, only performs default cell selection such a test may be in section 20

- authentication and authentication rejection have been seperated in two tests. the rejection test has been completed.

]

26.7.2.1	Authentication accepted

26.7.2.1.1	Conformance requirement:

1)	A Mobile Station shall correctly respond to an Authentication Request message by sending an Authentication Response message with the SRES information field set to the same value as the one produced by the authentication algorithm in the network.

2)	A Mobile Station shall indicate in a Paging Response message the ciphering key sequence number which was allocated to it through the authentication procedure.

26.7.2.1.2	Test purpose:

1)	To check that a Mobile Station correctly responds to an Authentication Request message by sending an Authentication Response message with the SRES information field set to the same value as the one produced by the authentication algorithm in the network.

2)	To check that a Mobile Station indicates in a Paging Response message the ciphering key sequence number which was allocated to it through the authentication procedure.

26.7.2.1.3	Reference(s):

GSM 04.08 section 4.3.2, GSM 03.03 section 2

26.7.2.1.4	Related PICS/PIXIT statement(s):

none

26.7.2.1.5	Initial conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has valid TMSI, CKSN (CKSN1), Kc. It is "idle updated" on the cell.

26.7.2.1.6	Foreseen final state of the MS:

The MS has valid TMSI, CKSN and Kc. It is "idle updated" on the cell.

26.7.2.1.7	Test Procedure

The MS is paged. After the MS has sent a PAGING RESPONSE message to the SS, the SS initiates an authentication procedure and checks the value SRES sent by the MS in the AUTHENTICATION RESPONSE message. The channel is released. The MS is paged and the SS checks the value of the ciphering key sequence number sent by the MS in the PAGING RESPONSE message.

Maximum duration of test:

1 minute

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1

2	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer

to paging.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	PAGING RESPONSE	CKSN= CKSN1

5	SS -> MS	AUTHENT REQUEST	The SS initiates authentication

with CKSN2 different from CKSN1

6	MS -> SS	AUTHENT RESPONSE	"Auth.parameter SRES" IE shall

be bit exact with the value as

produced by the authentication

algorithm.

7	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link. The SS waits an

amount of time which is enough to

guarantee that the MS is in

service.

8	SS -> MS	PAGING REQUEST TYPE 1

9	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer

to paging.

10	SS -> MS	IMMEDIATE ASSIGNMENT

11	MS -> SS	PAGING RESPONSE	"Ciphering key sequence number"

shall be the same as the value that

was sent in the last AUTHENTICATION

REQUEST message (= CKSN2).

12	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.2.2	Authentication rejected

26.7.2.2.1	Conformance requirement:

1)	After reception of an Authentication Reject message the Mobile Station shall:

1.1	not perform normal location updating

1.2	not perform periodic location updating

1.3	not respond to paging with TMSI

1.4	reject any request from CM entity for MM connection except for emergency call

1.5	not perform IMSI detach if deactivated

2)	After reception of an Authentication Reject message the Mobile Station, if it supports speech, shall accept a request for an emergency call by sending a CHANNEL REQUEST message with the establishment cause set to "emergency call" and include an IMEI as mobile identity in the CM SERVICE REQUEST message.

3)	After reception of an Authentication Reject message the Mobile Station shall delete the stored LAI, CKSN and TMSI.

26.7.2.2.2	Test purpose:

1)	To check that ,after reception of an Authentication Reject message, the Mobile Station :

1.1	does not perform normal location updating

1.2	does not perform periodic location updating

1.3	does not respond to paging with TMSI

1.4	rejects any request from CM entity for MM connection except for emergency call

1.5	does not perform IMSI detach if deactivated

2)	To check that, after reception of an Authentication Reject message the Mobile Station, if it supports speech, accepts a request for an emergency call by sending a CHANNEL REQUEST message with the establishment cause set to "emergency call" and includes an IMEI as mobile identity in the CM SERVICE REQUEST message.

3)	To check that, after reception of an Authentication Reject message and after having been deactivated and reactivated, the MS performs location updating using its IMSI as mobile identity and indicates deleted LAI and CKSN.

26.7.2.2.3	Reference(s):

GSM 04.08 sections 4.3.2.5.

26.7.2.2.4	Related PICS/PIXIT statement(s):

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Support of speech Yes/No

26.7.2.2.5	Initial conditions:

System Simulator:

Two cells : A and B, belonging to different location areas a and b.

IMSI attach/detach is allowed in both cells.

The T3212 timeout value is 1/10 hour in both cells.

Mobile Station:

The MS has valid TMSI, CKSN and Kc. It is "idle updated" on cell B.

26.7.2.2.6	Foreseen final state of the MS:

The MS has valid TMSI, CKSN and Kc. It is "idle updated" on cell A.

26.7.2.2.7	Test procedure:

The SS rejects an authentication. The channel is released. The SS checks that the MS has entered the state MM-IDLE substate NO IMSI, i.e. does not perform normal location updating, does not perform periodic updating, does not respond to paging, rejects any requests from CM entities except emergency calls and does not perform IMSI detach if powered down.

Maximum duration of test:

10 minutes

Expected sequence:

Step	Direction	Message	Comments

The following messages are sent and shall be received on cell B

1	SS -> MS	PAGING REQUEST TYPE 1

2	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer

to paging.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	PAGING RESPONSE	"Ciphering key sequence number"

shall be the same as the value that

was sent in the last AUTHENTICATION

REQUEST message (= CKSN2).

5	SS -> MS	AUTHENT REQUEST

6	MS -> SS	AUTHENT RESPONSE

7	SS -> MS	AUTHENT REJECT

8	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

9	SS -> MS	PAGING REQUEST TYPE 1	The MS is paged in cell A.

"Mobile identity" IE contains

TMSI.

10	   MS		The MS shall ignore this

message. This is verified

during 3 seconds.

11	   SS		The SS waits for at least for 15 s

12	   MS		A MO CM connection is attempted.

13	   MS		The MS shall not initiate an RR

connection establishment on cell A or

cell B. This is checked during 3 seconds

14	   MS		If the MS supports speech (see PICS),

an emergency call is attempted.

15	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Emergency call.

16	SS -> MS	IMMEDIATE ASSIGNMENT

17	MS -> SS	CM SERVICE REQUEST	"CM service type" : Emergency call

establishment. "Mobile identity" : type

of identity is set to IMEI

18	SS -> MS	CM SERVICE ACCEPT

19	MS -> SS	EMERGENCY SETUP

20	SS -> MS	RELEASE COMPLETE	"Cause"= unassigned number.

21	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

The following messages are sent and shall be received on cell A

22	   SS		The RF levels are changed

to make the MS reselect

the cell A.

23	   MS		The MS performs cell

reselection according to

procedure as specified in GSM

05.08 (this however is not checked

until step 29). The MS shall not initiate

an RR connection establishment on cell A

or on cell B.

24	   SS		The SS waits at least 7 minutes

for a possible periodic

updating.

25 	   MS		The MS shall initiate an RR connection

establishment on cell A or on cell B.

26	   MS		If possible (see PICS) SIM

detachment is performed. Otherwise

if possible (see PICS) switch off

is performed. Otherwise the power

is removed.

27	   MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B.

This is checked during 3 seconds.

28	MS		Depending on what has been

performed in step 26 the MS is

brought back to operation.

29	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

30	SS -> MS	IMMEDIATE ASSIGNMENT

31	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,

"CKSN"= no key available,

"Mobile Identity"= IMSI,

"LAI"= deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE)

32	SS -> MS	AUTHENT REQUEST	"CKSN"= CKSN1

33	MS -> SS	AUTHENT RESPONSE

34	SS -> MS	LOCATION UPDATING ACC	"Mobile Identity"= TMSI

35	MS -> SS	TMSI REALLOCATION CMP

36	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

[editorial note : other methods could be used in step 18, e.g. a CM SERVICE REJECT]

Specific message contents:

None.

26.7.3	Identification

The purpose of this procedure is to check that the MS gives its identity as requested by the network. If this procedure does not work, it will not be possible for the network to rely on the identity claimed by the MS.

Conformance requirement:

1)	When requested by the network the Mobile Station shall send its IMSI.

2)	When requested by the network the Mobile Station shall send the TMSI which it was previously allocated.

3)	When requested by the network the Mobile Station shall send its IMEI as stored in the Mobile Equipement.

Test purpose:

1)	To verify that the MS sends identity information as requested by the system in the following cases: IMSI and TMSI are requested in non-ciphered mode, IMEI is requested in ciphered mode.

2)	To verify that the MS sends its IMEI, when requested to do so, in non-ciphered mode.

Reference(s):

GSM 04.08 sections 4.3.3.

Related PICS/PIXIT statement(s):

IMEI of the ME

26.7.3.1	Identification / test 1

Initial conditions:

System Simulator:

1 cell, default values

Mobile Station:

The MS has a valid TMSI. It is "idle updated" on the cell.

Foreseen final state of the MS:

The MS has a valid TMSI. It is "idle updated" on the cell.

Method of test:

The SS requests identity information from the MS :

-	IMSI in non ciphering mode,

-	allocated TMSI in non ciphering mode,

-	IMEI in ciphering mode.

Maximum duration of test:

30 seconds

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1

2	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer to

paging.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	IDENTITY REQUEST	"Identity type" IE is IMSI.

6	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies

the IMSI of the MS.

7	SS -> MS	IDENTITY REQUEST	"Identity type" IE is TMSI.

8	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies

the allocated TMSI of the MS.

9	SS -> MS	CIPH MODE COMMAND

10	MS -> SS	CIPH MODE COMPLETE

11	SS -> MS	IDENTITY REQUEST	"Identity type" IE is IMEI.

12	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies

the IMEI stored in the Mobile

Equipment.

13	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.3.2	Identification / test 2

[editorial note : it is still not clear that the MS is allowed to send its IMEI in non ciphered mode. 04.08 says that "for the presentation of the IMEI the requirememts of 09.02 apply. CR to 02.09 will be prepared]

Initial conditions:

System Simulator:

1 cell, default values

Mobile Station:

The MS has a valid TMSI. It is in "idle updated".

Foreseen final state of the MS:

The MS has a valid TMSI. It is "idle updated".

Method of test:

The SS requests identity information from the MS :

-	IMEI in non ciphering mode.

Maximum duration of test:

30 seconds

Expected sequence:

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1

2	MS -> SS	CHANNEL REQUEST	Establishment Cause : Answer to

paging.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	PAGING RESPONSE

5	SS -> MS	IDENTITY REQUEST	"Identity type" IE is IMEI.

6	MS -> SS	IDENTITY RESPONSE	"Mobile identity" IE specifies

the IMEI of the MS.

7	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.3.3	Handling of IMSI shorter than the maximum length

26.7.3.3.1	Conformance requirement

The MS shall be capable of handling an IMSI that is not of the maximum length.

26.7.3.3.2	Test purpose

To check that the MS behaves correctly when activated with an IMSI of length less than the maximum length.

In this condition, the MS shall:

-	perform location updating

-	answer to paging with IMSI

-	give the correct IMSI when asked by an IDENTITY REQUEST

-	attempt CM connection establishment when requested to

-	attempt call re-establishment when needed

-	attempt IMSI detach when needed

-	erase its TMSI when the IMSI is sent by the network in a LOCATION UPDATING ACCEPT or a TMSI REALLOCATION COMMAND message.

26.7.3.3.3	Reference(s)

GSM 04.08 clause 10.5.1.4

26.7.3.3.4	Related PICS/PIXIT statement(s)

On/Off switch - Yes/No

26.7.3.3.5	Initial conditions

System Simulator:

1 cell, default values

Mobile Station:

The MS has no valid TMSI.

It is "idle updated".

The IMSI has the value 001011234.

26.7.3.3.6	Foreseen final state of MS

The MS has a valid TMSI. It is in "idle, updated".

26.7.3.3.7	Test Procedure

The MS is paged with its IMSI. The MS shall answer to paging and include the correct IMSI in the PAGING RESPONSE message. During call establishment, the SS asks for the IMSI of the MS. The MS shall answer by an IDENTITY RESPONSE message including the correct IMSI. During the active phase of the call, the SS stops sending valid SACCH frames. The MS performs call re-establishment. The MS shall include the correct IMSI in the CM RE-ESTABLISHMENT message. a TMSI REALLOCATION COMMAND including a TMSI is sent to the MS. The MS acknowledges this message. The call is release.

The MS is paged with its IMSI. The MS shall answer to paging and includes its TMSI in the PAGING RESPONSE message. During call establishment, the SS sends a TMSI REALLOCATION COMMAND including the IMSI to the MS. The MS shall acknowledge this message. The MS shall erase its TMSI. The call is released.

The MS is switched off or powered down. The MS performs IMSI detach. The MS shall include the correct IMSI in the IMSI DETACH message.

The MS is switched on or powered on. The MS performs IMSI attach. The MS shall include the correct IMSI in the LOCATION UPDATING REQUEST message. A TMSI is allocated to the MS.

The LAC of the cell is changed. The MS performs location updating. The SS includes the IMSI in the LOCATION UPDATING ACCEPT message.

A mobile originated CM connection is attempted. The MS shall include the correct IMSI in the CM SERVICE REQUEST message.

Maximum duration of test

5 minutes.

Expected sequence

Step	Direction	Message	Comments

1	SS -> MS	PAGING REQUEST TYPE 1	"mobile identity 1" contains IMSI of MS

2	MS -> SS	CHANNEL REQUEST	Establishment cause: Answer to paging

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	PAGING RESPONSE	"mobile identity" contains the IMSI of the MS

5	SS -> MS	IDENTITY REQUEST	"identity type" IE is IMSI

6	MS -> SS	IDENTITY RESPONSE	"mobile identity" IE contains the IMSI of the MS

7			The call is established using the sequence of the

generic terminating call set-up procedure.

8	SS		The SS stops sending valid SACCH frames

9	MS -> SS	CHANNEL REQUEST

10	SS -> MS	IMMEDIATE ASSIGNMENT

11	MS -> SS	CM RE-ESTAB REQUEST	"mobile identity" IE contains IMSI of the MS

12	SS -> MS	TMSI REALLOC COMMAND	"mobile identity" contains a TMSI

13	MS -> SS	TMSI REALLOC COMPLETE

14	SS -> MS	CHANNEL RELEASE

15	SS -> MS	CHANNEL RELEASE	After sending this message, the SS waits for the

disconnection of the main signalling link.

16	SS -> MS	PAGING REQUEST TYPE 1	"mobile identity 1" contains TMSI of MS

17	MS -> SS	CHANNEL REQUEST	Establishment cause: Answer to paging

18	SS -> MS	IMMEDIATE ASSIGNMENT

19	MS -> SS	PAGING RESPONSE	"mobile identity" contains the TMSI of the MS

20	SS -> MS	AUTHENTICATION REQUEST

21	MS -> SS	AUTHENTICATION RESPONSE

22	SS -> MS	TMSI REALLOC COMMAND	"mobile identity" contains a IMSI of MS

23	MS -> SS	TMSI REALLOC COMPLETE

24	SS -> MS	CHANNEL RELEASE

25	MS		If possible (see PICS) the MS is switched off,

otherwise the MS is powered down.

26	MS -> SS	CHANNEL REQUEST	If the MS was switched off it

performs IMSI detach

27	SS -> MS	IMMEDIATE ASSIGNMENT

28	MS -> SS	IMSI DETACH	"mobile identity" contains IMSI of MS

29	SS -> MS	CHANNEL RELEASE

30	MS		The MS is activated

31	MS -> SS	CHANNEL REQUEST

32	SS -> MS	IMMEDIATE ASSIGNMENT

33	MS -> SS	LOCATION UPDATING REQ	"mobile identity" contains IMSI of MS

34	SS -> MS	LOCATION UPDATING ACC	"mobile identity" contains a TMSI

35	MS -> SS	TMSI REALLOC COMMAND

36	SS -> MS	CHANNEL RELEASE

37	SS		The SS changes the LAC of the cell

38	SS		The SS waits for at least 30 seconds

39	MS -> SS	CHANNEL REQUEST

40	SS -> MS	IMMEDIATE ASSIGNMENT

41	MS -> SS	LOCATION UPDATING REQ	"mobile identity" contains TMSI of the MS

42	SS -> MS	LOCATION UPDATING ACC	"mobile identity" contains IMSI of the MS

43	SS -> MS	CHANNEL RELEASE

44	MS		a mobile originated CM connection is attempted

45	MS -> SS	CHANNEL REQUEST

46	SS -> MS	IMMEDIATE ASSIGNMENT

47	MS -> SS	CM SERVICE REQUEST	"mobile identity" contains IMSI of the MS

Specific message contents

None

26.7.4	Location updating

This procedure is used to register the MS in the network. If it is not performed correctly, no call can be established.

[Editorial Note:	MM-states must be checked again after GSM3 has completed the specifications

]

26.7.4.1	Location updating / accepted

[Editorial Note:	- values of ATT and T3212 have no impact on the test. They could be removed from the initial conditions

- the time during which a non response to a paging message is checked has been added in step 31 and is set to 5 seconds

]

Conformance requirement:

1.1	If the network accepts a location updating from the Mobile Station and reallocates a TMSI in the Location Updating Accept message the Mobile Station shall acknowledge the reception of the new TMSI.

1.2	The Mobile Station shall answer to paging with this TMSI and include it in a Paging Response message.

2	If the network accepts a location updating from the Mobile Station and the Location Updating Accept message contains neither TMSI nor IMSI, the Mobile Station shall answer to paging when addressed with the last allocated TMSI and include it in the Paging Response message.

3.1	If the network accepts a location updating from the Mobile Station by use of a Location Updating Accept message containing the IMSI of the Mobile Station, the Mobile Station shall not answer paging with the last allocated TMSI.

3.2	The Mobile Station shall still answer paging with IMSI.

Test purpose:

To test the behaviour of the MS if the network accepts the location updating of the MS.

For the network response three different cases are identified:

1)	TMSI is allocated,

2)	Location updating accept contains neither TMSI nor IMSI,

3)	Location updating accept contains IMSI.

Reference(s):

GSM 04.08 section 4.4.4.6.

Related PICS/PIXIT statement(s):

None.

Initial conditions:

System Simulator:

Two cells, A and B, belonging to different location areas with location area identification a and b of the same PLMN.

IMSI attach/detach is allowed in both cells.

The T3212 timeout value is 1/10 hour in both cells.

Mobile Station:

The MS has a valid TMSI (=TMSI1) and CKSN (=CKSN1). It is "idle updated" on cell A.

Foreseen final state of the MS:

The MS has no valid TMSI. It has valid CKSN and Kc. It is "idle, updated" on cell B.

Method of test:

The MS is made to select cell B. A normal location updating with TMSI reallocation is performed in cell B. The channel is released. The SS checks, by paging, that the MS has stored the newly allocated TMSI. The channel is released. The MS is made to select cell A. A normal location updating is performed in cell A. The LOCATION UPDATING ACCEPT message contains neither IMSI nor TMSI. The SS checks, by paging, that the MS has kept the old TMSI. The channel is released. The MS is made to select cell B. A normal location updating is performed in cell B. The LOCATION UPDATING ACCEPT message contains an IMSI. The SS checks, by paging, that the MS has deleted its TMSI and responds to paging with IMSI.

Maximum duration of test:

4 minutes

Expected sequence:

Step	Direction	Message	Comments

1	   SS		The RF level of cell A is

lowered until the MS selects cell B.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All other cases.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,

"CKSN"= CKSN1, "location area

identification"= a, "mobile station

classmark 1" as given by the PICS

and "mobile identity"= TMSI1.

5	SS -> MS	LOCATION UPDATING ACC	"Mobile identity"= new TMSI

(=TMSI2), LAI= b.

6	MS -> SS	TMSI REALLOCATION CMP

7	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link. The SS waits an

amount of time which is enough to

guarantee that the MS is in service.

8	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains

the new TMSI (= TMSI2).

9	MS -> SS	CHANNEL REQUEST

10	SS -> MS	IMMEDIATE ASSIGNMENT

11	MS -> SS	PAGING RESPONSE	"Mobile identity" IE contains

the new TMSI (= TMSI2).

12	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main signalling link.

13	   SS		The RF level of cell B is

lowered until the MS selects

cell A.

14	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases

15	SS -> MS	IMMEDIATE ASSIGNMENT

16	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,

"CKSN"= CKSN1, "location area

identification"= b, "mobile station

classmark 1" as given by the PICS

and "mobile identity"= TMSI2.

17	SS -> MS	LOCATION UPDATING ACC	"Mobile identity" IE not included

18	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link. The SS waits an

amount of time which is enough to

guarantee that the MS is in

service.

19	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains

the TMSI (= TMSI2).

20	MS -> SS	CHANNEL REQUEST

21	SS -> MS	IMMEDIATE ASSIGNMENT

22	MS -> SS	PAGING RESPONSE	"Mobile identity" IE contains

the TMSI (=TMSI2).

23	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

24	   SS		The RF level of cell A is

lowered until the MS selects

cell B.

25	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

26	SS -> MS	IMMEDIATE ASSIGNMENT

27	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,

"CKSN"= CKSN1, "location area

identification"= a, "mobile station

classmark 1" as given by the PICS

and "mobile identity"= TMSI2.

28	SS -> MS	LOCATION UPDATING ACC	"Mobile identity" IE contains

IMSI.

29	SS -> MS	CHANNEL RELEASE	After the sending of this message,

the SS waits for the disconnection

of the main signalling link. The SS

waits an amount of time which is

enough to guarantee that the MS is

in service.

30	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains

the old TMSI (= TMSI2).

31	   MS		The MS shall ignore this

message. This is checked during 5

seconds.

32	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains

the IMSI.

33	MS -> SS	CHANNEL REQUEST

34	SS -> MS	IMMEDIATE ASSIGNMENT

35	MS -> SS	PAGING RESPONSE	"Mobile identity" IE contains

the IMSI.

36	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.4.2	Location updating / rejected

26.7.4.2.1	Location updating / rejected / IMSI invalid

[Editorial Note: 	- at this point of time, a value of 3 seconds has been put when the SS checks that the MS does not answer to a paging message. This can be updated.

- the MS is brought back to the MM-state "idle, updated". This will allow to combine tests and also to check that the LAI has been deleted (this was not checked in the phase 1 version)

- the test with cause "Illegal MS" has been explicitely included as it was not clear for phase 1 if it was tested or not.]

Conformance requirement:

1)	If the network rejects a location updating from the Mobile Station with the cause "IMSI unknown in HLR", "Illegal MS" or "Illegal ME" the Mobile Station shall:

1.1	not perform normal location updating

1.2	not perform periodic location updating

1.3	not respond to paging with IMSI

1.4	not respond to paging with TMSI

1.5	reject any request from CM entity for MM connection other than for emergency call

1.6	not perform IMSI detach if powered down

2)	If the network rejects a location updating from the Mobile Station with the cause "IMSI unknown in HLR", "Illegal MS" or "Illegal ME" the Mobile Station, if it supports speech, shall accept a request for an emergency call by sending a Channel Request message with the establishment cause set to "emergency call" and include an IMEI as mobile identity in the CM SERVICE REQUEST message.

3)	If the network rejects a location updating from the Mobile Station with the cause "IMSI unknown in HLR", "Illegal MS" or "Illegal ME" the Mobile Station shall delete the stored LAI, CKSN and TMSI.

Test purpose:

To test the behaviour of the MS if the network rejects the location updating of the MS with the cause "IMSI unknown in HLR", "illegal MS" or "Illegal ME".

Reference(s):

GSM 04.08 sections 4.4.4.7.

Related PICS/PIXIT statement(s):

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Initial conditions:

System Simulator:

Two cells : A and B, belonging to different location areas of the same PLMN.

IMSI attach/detach is allowed in both cells.

The T3212 timeout value is 1/10 hour in both cells.

Mobile Station:

The MS has valid TMSI, CKSN and Kc. It is "idle updated" on cell A.

Foreseen final state of the MS:

The MS has valid TMSI, CKSN and Kc. It is "idle updated" on cell A.

Method of test:

The SS rejects a normal location updating with the cause value "IMSI unknown in HLR". The channel is released. The SS checks that the MS has entered the state MM-IDLE and the substate NO IMSI, i.e. does not perform normal location updating when a new cell of the same or another PLMN is entered, does not perform periodic updating, does not respond to paging, rejects any requests from CM entities except emergency calls and does not perform IMSI detach if powered down.

The test is repeated with cause value "Illegal MS" and with cause value "Illegal ME".

Maximum duration of test:

15 minutes

Expected sequence:

The sequence is executed for execution counter k = 1, 2, 3

Step	Direction	Message	Comments

The following messages are sent and shall be

received on cell B.

1	SS		The RF level of cell A is

lowered until the MS selects

cell B.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ

5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "IMSI

unknown in HLR" for k = 1,

"Illegal MS" for k = 2,

"Illegal ME" for k = 3.

6	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

The following messages are sent and shall be

received on cell A.

7	SS		The RF levels are then changed

again to make the MS reselect

the cell A.

8	MS		The MS performs cell

reselection according to

procedure as specified in GSM

05.08 (this however is not checked

until step 18). The MS shall not initiate

an RR connection establishment on cell A

or on cell B.

9	   SS		The SS waits at least 7 minutes

for a possible periodic

updating.

10 	   MS		The MS shall not perform

periodic updating.

11	SS -> MS	PAGING REQUEST TYPE 1	The MS is paged in cell A.

"Mobile identity" IE contains

IMSI.

12	   MS		The MS shall ignore this

message. This is verified

during 3 seconds.

13	SS -> MS	PAGING REQUEST TYPE 1	The MS is paged in cell A.

"Mobile identity" IE contains

TMSI.

14	   MS		The MS shall ignore this

message. This is verified

during 3 seconds.

15	   MS		A MO CM connection is attempted.

16	   MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B.

This is checked during 3 seconds

17	   MS		An emergency call is attempted.

18	MS -> SS	CHANNEL REQUEST	"Establishment cause" :

Emergency call. This message is

sent in cell A

19	SS -> MS	IMMEDIATE ASSIGNMENT

20	MS -> SS	CM SERVICE REQUEST	"CM service type" : Emergency

call establishment. "Mobile identity" : type

of identity is set to IMEI

21	SS -> MS	CM SERVICE ACCEPT

22	MS -> SS	EMERGENCY SETUP

23	SS -> MS	RELEASE COMPLETE	"Cause"= unassigned number.

24	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

25	   MS		If possible (see PICS) SIM

detachment is performed. Otherwise

if possible (see PICS) switch off

is performed. Otherwise the power

is removed.

26	   MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B.

This is checked during 3 seconds.

27	MS		Depending on what has been

performed in step 25 the MS is

brought back to operation.

28	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

29	SS -> MS	IMMEDIATE ASSIGNMENT

30	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,

"CKSN"= no key available, "mobile

station classmark 1" as given by

the PICS, "Mobile Identity"= IMSI,

"LAI"= deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE)

31	SS -> MS	AUTHENT REQUEST	"CKSN"= CKSN1

32	MS -> SS	AUTHENT RESPONSE

33	SS -> MS	LOCATION UPDATING ACC	"Mobile Identity"= TMSI

32	MS -> SS	TMSI REALLOCATION CMP

33	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.4.2.2	Location updating / rejected / PLMN not allowed

[Editorial Note:

- the switch off/on of the MS when the RF levels are changed (steps 24 and 25) has been suppressed because it is not useful.

- the test where the LAI is changed has been suppressed because it is the same as the test where the RF level are changed (steps 24 and 25).

- two steps in the old version have been exchanged in order to check that the MS has performed default cell reselection and access the right cell.

- It is not checked that the mobile identity in the emergency call is IMSI

- CR 252 rev 1 has been included

]

Conformance requirement:

1)	If the network reject a location updating from the Mobile Station with the cause "PLMN not allowed" the Mobile Station shall:

1.1	not perform periodic updating,

1.2	not perform IMSI detach when switched off,

1.3	not perform IMSI attach when switched on in the same location area,

1.4	not perform normal location updating when in the same PLMN and when that PLMN is not selected manually.

1.5	reject any request from CM entity for MM connection other than for emergency call

2)	If the network rejects a location updating from the Mobile Station with the cause "PLMN not allowed" the Mobile Station shall:

2.1	perform normal location updating when a new PLMN is entered

2.2	accept a request for an emergency call, if it supports speech, by sending a Channel Request message with the establishment cause set to "emergency call".

3)	If the network rejects a location updating from the Mobile Station with the cause "PLMN not allowed" and if after that the PLMN from which this rejection was received, is manually selected, the Mobile Station shall perform a normal location updating procedure.

Test purpose:

To test the behaviour of the MS if the network rejects the location updating of the MS with the cause "PLMN not allowed".

Reference(s):

GSM 04.08 section 4.4.4.6

Related PICS/PIXIT statement(s):

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

The MS is automatically in automatic mode after switch on Yes/No

26.7.4.2.2.1	Location updating / rejected / PLMN not allowed / test 1

Initial conditions:

System Simulator:

One cell: C, belonging to PLMN1.

Two cells : A and B, belonging to different location areas a and b and belonging to PLMN2. PLMN2 is different from HPLMN and from PLMN1.

IMSI attach/detach is allowed in cells A and B but not in cell C.

The T3212 timeout value is 1/10 hour in cells A and B.

Mobile Station:

The MS has a valid TMSI. It is "idle updated" on cell C.

The MS is in manual mode for PLMN selection.

Foreseen final state of the MS:

The MS has a valid TMSI. It is "idle updated" on cell C.The MS is in manual mode for PLMN selection.

Method of test:

The SS rejects a normal location updating with the cause value "PLMN not allowed". The channel is released. The SS checks that the MS does not perform periodic updating, does not perform IMSI detach, does not perform IMSI attach if activated in the same location area, rejects any request for CM connection establishment other than emergency call, accepts a request for an emergency call and performs normal location updating only when a new PLMN is entered.

Maximum duration of test:

12 minutes

Expected sequence:

Step	Direction	Message	Comments

The following messages are sent and shall be

received on cell B.

1	MS		The MS is switched off.

2	SS		The SS activates cells A and B and

deactivates cell C.

Cell B has a level higher by

at least 5 dB than cell A

3	MS		The MS is switched on.If necessary

the MS is put in manual selection

mode. The MS shall

offer the new PLMN as available to

the user. The PLMN is manually selected.

4	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

5	SS -> MS	IMMEDIATE ASSIGNMENT

6	MS -> SS	LOCATION UPDATING REQ

7	SS -> MS	LOCATION UPDATING REJ	"Reject cause"= PLMN not allowed.

8	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

9	   SS		The SS waits for a possible

periodic updating for 7 minutes.

10	   MS		The MS shall not perform periodic updating.

11	   MS		If possible (see PICS) SIM

detachment is performed. Otherwise

if possible (see PICS) switch off

is performed. Otherwise the power

is removed.

12	   MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B.

This is checked during 3 seconds.

13	   MS		Depending on what has been

performed in step 11 the MS is

brought back to operation. The MS is

not made to select PLMN 2.

14	   MS		The MS shall not initiate an RR

connection establishment. This is checked

during 3 seconds.

The following message are sent and shall be

received on cell A.

15	   SS		The RF level of cell B is

lowered to make the MS reselect cell A.

16	   MS		No access to the network shall

be registered by the SS within one minute.

17	   MS		An emergency call is attempted.

18	MS -> SS	CHANNEL REQUEST	"Establishment cause" :

Emergency call

19	SS -> MS	IMMEDIATE ASSIGNMENT

20	MS -> SS	CM SERVICE REQUEST	"CM service type"= Emergency call

establishment.

21	SS -> MS	CM SERVICE ACCEPT

22	MS -> SS	EMERGENCY SETUP

23	SS -> MS	RELEASE COMPLETE	Cause IE : "unassigned number".

24	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

25	   MS		A MO CM connection is attempted.

26	   MS		The MS shall not initiate an RR connection

establishment. This is checked

during 3 seconds

The following messages are sent and shall be

received on cell C

27	   MS		The MS is switched off.

28	   SS		The SS activates cell C

and deactivates cells A and B

29	   MS		The MS is switched on. If necessary

the MS is placed into the automatic mode.

30	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

31	SS -> MS	IMMEDIATE ASSIGNMENT

32	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,

"CKSN"= no key available,

"LAI"= deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE)

"mobile identity"= IMSI

33	SS -> MS	LOCATION UPDATING ACC	"Mobile identity"= TMSI.

34	MS -> SS	TMSI REALLOCATION CMP

35	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.4.2.2.2	Location updating / rejected / PLMN not allowed / test 2

Initial conditions:

System Simulator:

One cell C, belonging to PLMN1.

Two cells A and B, belonging to different location areas a and b and belonging to PLMN2. PLMN2 is different from HPLMN.

IMSI attach/detach is allowed in cells A and B but not in cell C.

The T3212 timeout value is 1/10 hour in cells A and B.

Mobile Station:

The MS has a valid TMSI. It is "idle updated" on cell C.

The MS is in manual mode for PLMN selection.

Foreseen final state of the MS:

The MS has a valid TMSI. It is "idle, updated"on cell C.

The MS is in manual mode for PLMN selection.

Method of test:

The SS rejects a normal location updating with the cause value "PLMN not allowed". The channel is released. Then the PLMN from this rejection was received is manually selected and the SS checks that a normal location updating is performed.

Maximum duration of test:

1 minute

Expected sequence:

Step	Direction	Message	Comments

The following messages are sent and shall be

received on cell B

1	MS		The MS is switched off.

2	SS		The SS activates cells A and B and

deactivates cell C..

Cell B has a level higher by at least

5 dB than cell A

3	MS		The MS is switched on. If the MS is

in manual mode, it shall

offer the new PLMN as available to

the user. In this casethe PLMN is manually

selected.

4	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

5	SS -> MS	IMMEDIATE ASSIGNMENT

6	MS -> SS	LOCATION UPDATING REQ

7	SS -> MS	LOCATION UPDATING REJ	"Reject cause"= PLMN not

allowed.

8	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

9	   MS		The MS is made to search for

PLMNs and the PLMN indicated

by the SS is manually selected

10	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

11	SS -> MS	IMMEDIATE ASSIGNMENT

12	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,

"CKSN"= no key available,

"LAI"= deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE)

"mobile identity"= IMSI

13	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

The following messages are sent and shall be

received on cell C.

14	   MS		The MS is switched off.

15	   SS		The SS activates cell C

and deactivates cells A and B

16	   MS		The MS is switched on.

If necessary, the MS is put

into the automatic mode.

17	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

18	SS -> MS	IMMEDIATE ASSIGNMENT

19	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,

"CKSN"= no key available,

"LAI"= deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE)

"mobile identity"= IMSI

20	SS -> MS	LOCATION UPDATING ACC	"Mobile identity"= TMSI.

21	MS -> SS	TMSI REALLOCATION CMP

22	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.4.2.3	Location updating / rejected / location area not allowed

[Editorial Note:

- two steps have been exchanged (steps f and g in phase 1) in order to finish in a "clean" MM-state (to enable to begin immediately an other test)

- the second reject of location updating has been suppressed as it was not useful

- to finish the test with the MS on, we have to check that it does not perform an IMSI attach procedure; This is a requirement in TS 04.08 and was not checked.

- the test with cause LA not allowed tests less than the test with cause national roaming not allowed. Since the behaviour of the MS in the 2 cases is the same (except for the return to HPLMN timer) they could be merged in one test.

]

Conformance requirement:

1)	If the network rejects a location updating from the Mobile Station with the cause "Location Area not allowed" the Mobile Station shall:

1.1	not perform periodic updating,

1.2	not respond to paging with TMSI,

1.3	reject any request from CM entity for MM connection other than for emergency call,

1.4	not perform IMSI detach.

2)	If the network rejects a location updating from the Mobile Station with the cause "Location Area not allowed" the Mobile Station shall:

2.1	perform normal location updating when a new location area is entered,

2.2	accept a request for an emergency call, if it supports speech, by sending a Channel Request message with the establishment cause set to "emergency call".

Test purpose:

To test the behaviour of the MS if the network rejects the location updating of the MS with the cause "Location Area not allowed".

Reference(s):

GSM 04.08 sections 4.4.4.7

Related PICS/PIXIT statement(s):

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Method to clear the list of forbidden location areas periodically.

Initial conditions:

System Simulator:

Two cells : A and B, belonging to different location areas a and b.

IMSI attach/detach is allowed in both cells.

The T3212 timeout value is 1/10 hour in both cells.

Mobile Station:

The MS has a valid TMSI. It is "idle updated" on cell A.

Foreseen final state of the MS:

The MS has a valid TMSI. It is "idle updated" on cell A.

Method of test:

The SS rejects a normal location updating with the cause value "Location Area not allowed". The channel is released. The SS checks that the MS does not perform periodic updating, does not respond to paging with TMSI, responds to paging with IMSI, rejects any requests from CM entities for MM-connections except emergency calls, performs normal location updating when a new location area is entered and does not perform IMSI detach.

Different types of MS may use different methods to periodically clear the list of forbidden location areas (eg. every day at 12am). If the list is cleared while the test is being run, it may be necessary to re-run the test.

Maximum duration of test:

12 minutes

Expected sequence:

Step	Direction	Message	Comments

The following messages are sent and shall be

received on cell B

1	   SS		The RF level of cell A is

lowered so that cell B is selected, while keeping

the C1 and C2 of cell A greater than 10.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ

5	SS -> MS	LOCATION UPDATING REJ	"Reject cause"= "Location

Area not allowed".

6	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

7	   SS		The SS waits for a possible

location updating for 7 minutes.

8	   MS		The MS shall not initiate an

RR-connection establishment either on

cell A or cell B.

9	SS -> MS	PAGING REQUEST TYPE 1	The MS is paged in cell B.

"Mobile identity"= TMSI.

10	   MS		The MS shall ignore this message.

This is checked during 3 seconds.

11	   MS		A MO CM connection is attempted.

12	   MS		The MS shall not initiate an RR 	connection

establishment on cell A or cell B.

This is checked during 3 seconds

13	   MS		An emergency call is attempted.

14	MS -> SS	CHANNEL REQUEST	"Establishment cause" :

Emergency call.

15	SS -> MS	IMMEDIATE ASSIGNMENT

16	MS -> SS	CM SERVICE REQUEST	"CM service type":

Emergency call establishment.

17	SS -> MS	CM SERVICE ACCEPT

18	MS -> SS	EMERGENCY SETUP

19	SS -> MS	RELEASE COMPLETE	Cause: "unassigned number".

20	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

21	   MS		If possible (see PICS) SIM

detachment is performed. Otherwise

if possible (see PICS) switch off

is performed. Otherwise the power

is removed.

22	   MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

This is checked during 3 seconds.

23	MS		Depending on what has been

performed in step 21 the MS is

brought back to operation.

24	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

25	SS -> MS	IMMEDIATE ASSIGNMENT

26	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,

"CKSN"= no key available,

"LAI"= deleted LAI, "mobile

identity"= IMSI

27	SS -> MS	LOCATION UPDATING REJ	"Reject cause"= "Location

Area not allowed".

28	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

The following messages are sent and shall be

received on cell A

29	   SS		The RF level of cell B is

lowered until the MS selects

cell A.

30	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

31	SS -> MS	IMMEDIATE ASSIGNMENT

32	MS -> SS	LOCATION UPDATING REQ

33	SS -> MS	AUTHENT REQUEST

34	MS -> SS	AUTHENT RESPONSE

35	SS -> MS	LOCATION UPDATING ACC	Mobile identity= TMSI

36	MS -> SS	TMSI REALLOCATION CMP

37	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.4.2.4	Location updating / rejected / national roaming

[Editorial Note:		- Procedure 3 could be simplified by using the full capacities of the SS and by using 6 cells

]

Conformance requirement:

1)	If the network rejects a location updating from the Mobile Station with the cause "National roaming not allowed in this area" the Mobile Station shall:

1.1	not perform periodic updating,

1.2	not respond to paging with TMSI,

1.3	reject any request from CM entity for MM connection other than for emergency call,

1.4	not perform IMSI detach.

2)	If the network rejects a location updating from the Mobile Station with the cause "National roaming not allowed in this area" the Mobile Station shall:

2.1	perform normal location updating when a new location area is entered,

2.2	accept a request for an emergency call, if it supports speech, by sending a Channel Request message with the establishment cause set to "emergency call".

2.3	periodically search for its HPLMN. [editorial note : no test purpose checks that]

3)	The mobile station shall reset the list of "forbidden location areas for national roaming" when powered down or when the SIM is removed.

4)	The MS shall be capable of storing at leat 6 entries in the list of "forbidden forbidden location areas for national roaming".

Test purpose:

Test purpose 1

To test that on receipt of a rejection using the National Roaming cause code, the MS ceases trying to update on that cell, that this situation continues for at least one periodic location interval period, and that the corresponding list is re-set by powering down the MS (the requirement in TS GSM 04.08 is that the list shall be retained for at least 12 hours. This aspect is not formally tested).

Test purpose 2

To test that if no cell is available, the MS does not answer to paging with TMSI, rejects a request from CM entity except other than emergency calls.

Test purpose 3

To test that at least 6 entries can be held in the list of "forbidden location areas for national roaming" (the requirement in TS GSM 04.08 is to store at least 10 entries. This is not fully tested by the third procedure).

Test purpose 4

To test that if a cell of the Home PLMN is available then the MS returns to it in preference to any other available cell.

Test purpose 5

To test that if the SIM is removed the list of "forbidden location areas for national roaming"is cleared.

Reference(s):

GSM 04.08 section 4.4.4.6

Related PICS/PIXIT statement(s):

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Support of speech Yes/No

Method to clear the list of location areas for national roaming periodically.

The MS is automatically in automatic mode after switch on Yes/No

Initial conditions:

The initial conditions shall be met before each of the different procedures.

System Simulator:

For procedures 1, 2, 3 and 5: Two cells A and B, belonging to different location areas of the same PLMN with LAI a and b. The MCC of that PLMN is the same as that of the HPLMN. The MNC of that PLMN is different from that of the HPLMN.

For procedure 4: three cells A, B, C of the same PLMN which is not the HPLMN with 3 different location area codes. Cells should differ in signal strength by 10 dB with cell A being the strongest and cell C the weakest. There should be a 20 dB range between A and C. A should be set to a level of - 40 dBm.

IMSI attach/detach is allowed in every cell.

The T3212 timeout value is 1/10 hour in every cell.

Mobile Station:

Procedures 1, 2, 3 and 5 : The MS has valid TMSI, CKSN and Kc. It is "idle updated" on cell B.

Procedure 4 : The MS has valid TMSI, CKSN and Kc. It is "idle updated" on cell A.

The list of "forbidden location areas for national roaming" shall be empty (this may be achieved by either removing the SIM or switching the MS OFF then ON or powering down the MS depending on PICS).

Foreseen final state of the MS:

Procedures 1, 2, 3 and 5 : The MS has valid TMSI, CKSN and Kc. It is "idle updated" on cell A.

Procedure 4 : The MS has valid TMSI, CKSN and Kc. It is "idle updated" on cell C.

Method of test:

Procedure 1 :

The SS rejects a normal location updating with the cause value "National Roaming not allowed in this area". The channel is released. The SS checks that the MS does not perform periodic location updating procedure. The MS is turned off and then on. The SS checks that the MS performs location updating on the cell on which its location update request had been rwejected (this checks that the LA is not the forbidden list after switch on). This procedure is performed another time but the deletion of the list is checked while removing the SIM (instead of turning off the MS).

Procedure 2 :

The SS rejects a normal location updating with the cause value "National Roaming not allowed in this area". The channel is released. The SS checks that the MS does not answer to a paging message with TMSI, rejects a request from CM entity but supports an emergency call.

Procedure 3 :

The SS rejects a normal location updating with the cause value "National Roaming not allowed in this area". This is done for 6 different location areas. Then the SS checks that the MS does not attempt to begin a location updating procedure on the non-allowed location areas.

Procedure 4 :

The SS accepts a normal location updating on a cell not belonging to the HPLMN. Then when the MS attemps to perform a periodic location updating to this cell, the SS rejects this location updating with the cause value "National Roaming not allowed in this area". Two cells are then available, one belonging to the HPLMN but with the weakest level. It is checked that the MS returns to its HPLMN.

Procedure 5 : If SIM removal is possible without powering down :

The SS rejects a normal location updating with the cause value "National Roaming not allowed in this area". The channel is released. The SS checks that the MS does not perform periodic location updating procedure. The SIM is removed and inserted in the MS. The SS checks that the MS performs location updating on the cell on which its location update request had been rwejected (this checks that the LA is not the forbidden list after switch on).

Different types of MS may use different methods to periodically clear the list of forbidden areas (eg. every day at 12am) for national roaming. If the list is cleared while the test is being run, it may be necessary to re-run the test.

Maximum duration of test:

Procedures 1 and 5: 12 minutes each

Procedure 2: 6 minutes

Procedure 3: 17 minutes

Procedure 4: 16 minutes

Expected sequence:

The following procedure is used during the test:

Change_LAI (x):

-	the purpose of this procedure is to change the value of Location Area Identifier of cell x

-	The Location Area Identifier of cell x shall be changed. The code shall be chosen arbitrarily but shall be different from any previously used in this procedure. The code shall have the same MCC as the Home PLMN and shall not have the same MNC as the Home PLMN.

Procedure 1

Step	Direction	Message	Comments

The following messages are sent and shall be

received on cell A.

1	SS		The RF level of cell B is

lowered until cell B is no

more suitable and the the MS selects cell A.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ

5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National

Roaming not allowed in this

location area"

6	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

7	SS		The SS waits at least 7 minutes

for a possible location updating

8 	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B.

9	MS		If possible (see PICS) the MS is

switched off. Otherwise

if possible the power is removed.

10	MS		Depending on what has been

performed in step 9 the MS is

brought back to operation and placed

in a automatic mode.

11	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

12	SS -> MS	IMMEDIATE ASSIGNMENT

13	MS -> SS	LOCATION UPDATING REQ	Location Updating Type= normal

14	SS -> MS	LOCATION UPDATING ACC	IE Mobile Identity not present

15	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main signalling link.

Procedure 2

Step	Direction	Message	Comments

The following messages are sent and shall be

received on cell A

1	SS		The RF level of cell B is

lowered until the MS selects

cell A. The level of cell B

shall be such that cell B is

suitable for cell selection.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message is sent on

cell A.

3	SS -> MS	IMDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ

5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National ┘E

Roaming not allowed in this

location area"

6	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

The following messages are sent and shall be

received on cell B

7	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All other cases.

8	SS -> MS	IMMEDIATE ASSIGNMENT

9	MS -> SS	LOCATION UPDATING REQ

10	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National

Roaming not allowed in this

location area"

11	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

12	SS		The SS waits for a possible

location updating

procedure on both cells A and B for

2 minutes.

13	MS		The MS shall not initiate an RR connection

establishment on  cell A or on cell B

within 2 minutes after the end of step 11

14	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" = TMSI.This message is

sent on cell A and on cell B

15	  MS		The MS shall not initiate an RR connection

on cell A or on cell B. This is checked

during 3 seconds.

16	   MS		A MO CM connection is attempted.

17	   MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B.

This is checked during 3 seconds.

The following messages are sent and shall be

received on cell A

Steps 20 to 27 are performed if the MS

supports speech.

18	MS		An emergency call is attempted.

19	MS -> SS	CHANNEL REQUEST	"Establishment cause" :

Emergency call.

20	SS -> MS	IMMEDIATE ASSIGNMENT

21	MS -> SS	CM SERVICE REQUEST	"CM service type" : Emergency

call establishment.

22	SS -> MS	CM SERVICE ACCEPT

23	MS -> SS	EMERGENCY SETUP

24	SS -> MS	RELEASE COMPLETE	"Cause"= unassigned number.

25	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Procedure 3

Step	Direction	Message	Comments

The following messages are sent and shall be

received on cell A

1	SS		The RF level of cell B is

lowered until the MS selects

cell A. The level of cell B

shall be such that cell B is

suitable for cell selection.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ

5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National

Roaming not allowed in this

location area"

6	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

The following messages are sent and shall be

received on cell B

9	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

10	SS -> MS	IMMEDIATE ASSIGNMENT

11	MS -> SS	LOCATION UPDATING REQ

12	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National

Roaming not allowed in this

location area"

13	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

14	SS		Change_LAI (A) within 5 seconds

after step 12

The following messages are sent and shall be

received on cell A

17	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

18	SS -> MS	IMMEDIATE ASSIGNMENT

19	MS -> SS	LOCATION UPDATING REQ

20	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National

Roaming not allowed in this

location area"

21	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

22	SS		Change_LAI (B) within 5 seconds

after step 20

The following messages are sent and shall be

received on cell B

25	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

26	SS -> MS	IMMEDIATE ASSIGNMENT

27	MS -> SS	LOCATION UPDATING REQ

28	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National

Roaming not allowed in this

location area"

29	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

30	SS		Change_LAI (A) within 5 seconds

after step 28

The following messages are sent and shall be

received on cell A

33	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

34	SS -> MS	IMMEDIATE ASSIGNMENT

35	MS -> SS	LOCATION UPDATING REQ

36	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National

Roaming not allowed in this

location area"

37	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

38	SS		Change_LAI (B) within 5 seconds

after step 36

The following messages are sent and shall be

received on cell B

41	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

42	SS -> MS	IMMEDIATE ASSIGNMENT

43	MS -> SS	LOCATION UPDATING REQ

44	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National

Roaming not allowed in this

location area"

45	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

46	SS		The SS waits for a possible

location updating

procedure on both cells A and B for

7 minutes.

47	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

within 7 minutes after the end of step 45

Procedure 4

Step	Direction	Message	Comments

The following messages are sent and shall be

received on cell A

1	SS		The SS waits for a

periodic location updating

procedure on cell A for 7 minutes

after the initial conditions have

been established.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ	Location Updating Type= periodic

5	SS -> MS	LOCATION UPDATING ACC	IE Mobile Identity not present

6	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

7	SS		The location area identity of cell

C shall be changed to that of a

location area in the Home PLMN

8	SS		The SS waits for a

periodic location updating

procedure on cell A for

7 minutes.

9	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message is sent on

cell A within 7 minutes after the end

of step 6.

10	SS -> MS	IMMEDIATE ASSIGNMENT

11	MS -> SS	LOCATION UPDATING REQ	"Location updating type" = periodic

12	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National

Roaming not allowed in this

location area"

13	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

The following messages are sent and shall be

received on cell C

16	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

17	SS -> MS	IMMEDIATE ASSIGNMENT

18	MS -> SS	LOCATION UPDATING REQ

19	SS -> MS	LOCATION UPDATING ACC	IE Mobile Identity not present

20	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Procedure 5

Step	Direction	Message	Comments

The following messages are sent and shall be

received on cell A

1	SS		The RF level of cell B is

lowered until the MS selects cell A.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ

5	SS -> MS	LOCATION UPDATING REJ	"Reject cause" IE is "National

Roaming not allowed in this

location area"

6	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

7	SS		The SS waits at least 7 minutes

for a possible location updating

8 	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B.

9	MS		The SIM is removed

10	MS		The SIM is inserted into the ME.

11	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

12	SS -> MS	IMMEDIATE ASSIGNMENT

13	MS -> SS	LOCATION UPDATING REQ	Location Updating Type= normal

14	SS -> MS	LOCATION UPDATING ACC	IE Mobile Identity not present

15	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.4.3	Location updating / abnormal cases

26.7.4.3.1	Location updating / abnormal cases / random access fails

Conformance requirement:

If during the RR connection establishment phase of a normal location updating procedure, channel requests are not answered by the network, the Mobile Station shall:

1.	send (Max-Retrans+1) Channel Request messages

2.	not try to establish a connection during a period of T3213

3.	then perform a normal location updating procedure as it is still necessary.

4.	not repeat the complete procedure if the original cause of the location updating procedure has disappeared.

Test purpose:

To verify that when during the RR connection establishment phase of a location updating procedure, channel requests are not answered by the network, after expiry of T3213 (= 4s in Phase 1) the complete procedure is repeated if still necessary.

[Editorial Note:	- the two procedures are chained

]

Reference(s):

GSM 04.08 section 4.4.4.9 and GSM 05.08 section 6.6.2

Related PICS/PIXIT statement(s):

None

Initial conditions:

System Simulator:

Two cells: A and B of the same PLMN, belonging to different location areas with LAI a and b.

The RF power level of cell B is higher than the one of cell A.

IMSI attach/detach is not allowed in both cells.

The T3212 timeout value is set to infinite in both cells.

Mobile Station:

The MS has a valid TMSI. It is "Idle updated" on cell B.

Foreseen final state of the MS:

The MS is "Idle updated" on cell A.

Method of test:

The SS causes a random access failure in the MS during a normal location updating procedure. After the expiry of T3213 and when the cell reselection procedure is finished the MS will try to restart the normal location updating procedure.

The test is repeated but the original cause of the location updating procedure has disappeared. The SS then checks that the MS will not restart the location updating procedure.

Maximum duration of test:

1 minute

Expected sequence:

Step	Direction	Message	Comments

The following messages are sent and shall be

received on cell A

1	MS		The RF level of cell B is lowered

until the MS selects cell A.

The RF level of cell B is set

sufficiently low to ensure that

cell B is not suitable as defined

in GSM 05.08 section 6.6.2.

2	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

This message is sent by the MS

(Max_Retrans + 1) times

3	   SS		The SS waits for 3 seconds

4	   MS		The MS shall not send any layer 3

message during this time.

5	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

The time difference between this message

and the last CHANNEL REQUEST sent in

step 2 shall be in the range 4 s - 6 s.

6	SS -> MS	IMMEDIATE ASSIGNMENT

7	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = b,

mobile station classmark 1 and

mobile identity = TMSI.

8	SS -> MS	LOCATION UPDATING ACC	Optional IE Mobile Identity not

included

9	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

The following messages are sent and shall be

received on cell B

10	MS		The RF level of cell A is lowered

until the MS selects cell B.

The RF level of cell A is set

sufficiently low to ensure that

cell A is not suitable as defined

in GSM 05.08 section 6.6.2.

11	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

This message is sent by the MS

(Max_Retrans + 1) times

12	SS		About 2 seconds after the end of

step 11 the RF level of cell A is

set sufficiently high to ensure

that cell A is suitable as defined

in GSM 05.08 section 6.6.2

13	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B.

This is checked during 6 seconds.

Specific message contents:

None.

26.7.4.3.2	Location updating / abnormal cases / attempt counter less or equal to 4, LAI different

[Editorial Note:

- it is not checked that the MS does not perform periodic location updating and IMSI attach procedures

]

Conformance requirement:

1)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure, if the attempt counter is smaller than 4 and after expiry of T3211, the Mobile Station shall resend its Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Upadting type set to "normal location updating".

2)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure the Mobile Station shall:

2.1	not answer to paging with the previously allocated TMSI

2.2	not perform the IMSI detach procedure, when switched off.

3)	When a failure such as case e) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure and when an emergency call establishment is requested by the user the Mobile Station, if it supports speech, shall send a CM Service Request message with CM Service Type IE set to "emergency call establishment", CKSN IE set to "no key available" and Mobile Identity IE set to its IMSI and after acceptance by the network it shall send an Emergency Setup message.

4)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure the Mobile Station shall use a request from CM entity other than emergency call as a trigger for a normal location updating procedure and shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

5)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure the Mobile Station shall answer to paging with IMSI and shall send a Paging Response message with CKSN IE set to "no key available" and Mobile Identity IE set to its IMSI.

6)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a normal location updating procedure the Mobile Station shall perform a normal location updating procedure as soon as it enters a new cell.

Test purpose:

To verify that the MS performs normal location updating procedures when its attempt counter is smaller than 4.

To check that the MS does not perform the IMSI detach procedure when "idle not updated".

To verify that when "idle not updated" the MS can perform an emergency call.

To verify that when "idle not updated" the MS uses requests from CM layer other than emergency call as triggering of a normal location updating procedure.

To check that when "idle not updated" the MS answers to paging with IMSI.

To verify that the MS performs a normal location updating procedure if it enters a new cell while being "idle not updated".

Reference(s):

GSM 04.08 section 4.4.4.9 and GSM 05.08 section 6.6.2

Related PICS/PIXIT statement(s):

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Initial conditions:

System Simulator:

Two cells: A and B of the same PLMN, belonging to different location areas with LAI a and b.

ATT flag shall be set to IMSI attach/detach allowed.

Mobile Station:

The MS is "idle updated" on cell A. A valid CKSN value is stored in the SIM and is noted "initial CKSN". A TMSI is allocated.

Foreseen final state of the MS:

The MS is "Idle updated" on cell B with a valid CKSN and a TMSI.

Method of test:

The MS is made to perform a normal location updating procedure. Three types of failure cases are triggered:

-	sending of a Location Updating Reject with cause randomly chosen between all defined cause values except 2, 3, 6, 11, 12 and 13 (which trigger a different action) (case g of GSM 04.08 section 4.4.4.9),

-	RR-connection failure (case d),

-	sending of a CHANNEL RELEASE message before the normal end of the procedure (case f),

-	T3210 timeout (case e).

As there is no stored LAI or the stored LAI is different from the broadcast LAI, and the attempt counter in the MS shall be lower than 4, the MS enters the state MM-IDLE and substate LIMITED SERVICE and waits for T3211 seconds before trying again a location updating procedure.

Then the behaviour of the MS in the MM-IDLE LIMITED SERVICE state is checked, that is:

-	not answer to paging with TMSI,

-	not perform an IMSI detach procedure,

-	support request for emergency call,

-	use requests from CM layer other than emergency call as triggering of a normal location updating procedure,

-	perform normal location updating procedure when a new cell is entered.

Maximum duration of test:

6 minutes

Expected sequence:

Step	Direction	Message	Comments

The following messages are sent and shall be received on cell B

1	MS		The RF level of cell A is lowered

until the MS selects cell B.

The RF level of cell A is set

sufficiently low to ensure that

cell A is not suitable as defined

in GSM 05.08 section 6.6.2.

2	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = a, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

5	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value

randomly chosen:

* in Table 10.44 of GSM 04.08,

causes #2, #3, #6, #11, #12 and #13 being

excluded

6	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

7	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

during T3211 seconds at least after

the channel release.

8	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

9	SS -> MS	IMMEDIATE ASSIGNMENT

10	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

11	SS		The SS stops any RF transmission on

the dedicated channel. The SS waits

until there is no more SACCH frames

in the uplink direction

12	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

during T3211+RadioLinkTimeout seconds at

least after the channel release.

13	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

14	SS -> MS	IMMEDIATE ASSIGNMENT

15	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

16	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

17	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

during T3211 seconds at least after

the channel release.

18	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

19	SS -> MS	IMMEDIATE ASSIGNMENT

20	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

21	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN

22	MS -> SS	AUTHENT RESPONSE

23	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI

24	MS -> SS	TMSI REALLOCATION CMP

25	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

MS is now "idle updated" in cell B

The following messages are sent and shall be received on cell A

26	MS		The RF level of cell B is lowered

until the MS selects cell A.

The RF level of cell B is set

sufficiently low to ensure that

cell B is not suitable as defined

in GSM 05.08 section 6.6.2.

27	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

28	SS -> MS	IMMEDIATE ASSIGNMENT

29	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

30	SS		chooses randomly one among the

three following steps: (5 and 6) or

11 or 16

31	SS		performs the chosen step

32	SS -> MS	PAGING REQUEST TYPE 1	Mobile identity = old TMSI of the

MS.

This message is sent continuously to

the MS during 8 seconds

33	SS		The SS checks that there is no

answer from the MS during 12

seconds

33a	SS		If during steps 33 and 34 the MS

attempts to perform a location

updating procedure the SS will

perform steps 30 and 31 and then

continue the procedure.

34	MS		If possible (see PICS) SIM

detachment is performed. Otherwise

if possible (see PICS) mobile switch off

is performed. Otherwise the power

is removed.

35	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B.

This is checked during 30 seconds.

36	MS		Depending on what has been

performed in step 34 the MS is

brought back to operation.

37	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

38	SS -> MS	IMMEDIATE ASSIGNMENT

39	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

40	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN

41	MS -> SS	AUTHENT RESPONSE

42	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI

43	MS -> SS	TMSI REALLOCATION CMP

44	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

MS is now "idle updated" in cell A

45	MS		The RF level of cell A is lowered

until the MS selects cell B.

The RF level of cell A is set

sufficiently low to ensure that

cell A is not suitable as defined

in GSM 05.08 section 6.6.2.

46	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

47	SS -> MS	IMMEDIATE ASSIGNMENT

48	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = a, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

49	SS -> MS	AUTHENT REQUEST

50	MS->SS	AUTHENT RESPONSE	steps 49 and 50 are performed N

times. N shall be chosen in such a

way that T3210 expires.

51	SS		The SS checks that there is no more

activity from the MS on the

channel.

52	MS		The MS is made to perform an

emergency call

53	MS -> SS	CHANNEL REQUEST	Establishment cause: Emergency call

54	SS -> MS	IMMEDIATE ASSIGNMENT

55	MS -> SS	CM SERVICE REQUEST	CM service type = Emergency call

establishment; CKSN = no key

available; Mobile Identity = IMSI

56	SS -> MS	CM SERVICE ACCEPT

57	MS -> SS	EMERGENCY SETUP

58	SS -> MS	RELEASE COMPLETE	Cause = unassigned number

59	SS -> MS	CHANNEL RELEASE

60	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases. The SS will wait at most 15

seconds for this message

61	SS -> MS	IMMEDIATE ASSIGNMENT

62	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

63	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN

64	MS -> SS	AUTHENT RESPONSE

65	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI

66	MS -> SS	TMSI REALLOCATION CMP

67	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

MS is now "idle updated" in cell B

68	MS		The RF level of cell B is lowered

until the MS selects cell A.

The RF level of cell B is set

sufficiently low to ensure that

cell B is not suitable as defined

in GSM 05.08 section 6.6.2.

69	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

70	SS -> MS	IMMEDIATE ASSIGNMENT

71	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

72	SS		chooses randomly one among the

three following steps: (5 and 6),

11 or 16

73	SS		performs the chosen step

74	MS		A MO CM connection is attempted

before T3211 expiry

75	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

76	SS -> MS	IMMEDIATE ASSIGNMENT

77	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

78	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI

79	MS -> SS	TMSI REALLOCATION CMP

80	SS -> MS	CHANNEL RELEASE

Steps 81 to 90 are optional as the

MS may have memorized the request for

CM connection attempt

81	MS -> SS	CHANNEL REQUEST	Establishment cause= other

services requested by the user

82	SS -> MS	IMMEDIATE ASSIGNMENT

83	MS -> SS	CM SERVICE REQUEST	CKSN=

no key available,

Mobile identity= TMSI

84	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN

85	MS -> SS	AUTHENT RESPONSE

86	SS -> MS	CIPH MODE COMMAND

87	MS -> SS	CIPH MODE COMPLETE

88	MS -> SS	An initial CM message

89	SS -> MS	RELEASE COMPLETE	Cause = unassigned number

90	SS -> MS	CHANNEL RELEASE	After the sending of this message,

the SS waits for the disconnection of

the main signalling link.

MS is now "idle updated" in cell A

91	MS		The RF level of cell A is lowered

until the MS selects cell B.

The RF level of cell A is set

sufficiently low to ensure that

cell A is not suitable as defined

in GSM 05.08 section 6.6.2.

92	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

93	SS -> MS	IMMEDIATE ASSIGNMENT

94	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = a, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

95	SS		chooses randomly one among the

three following steps: (5 and 6),

11 or 16

96	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases. The SS will wait at most 15

seconds for this message

97	SS -> MS	IMMEDIATE ASSIGNMENT

98	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

99	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN

100	MS -> SS	AUTHENT RESPONSE

101	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI

102	MS -> SS	TMSI REALLOCATION CMP

103	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

MS is now "idle, updated" in cell B

104	MS		The RF level of cell B is lowered

until the MS selects cell A.

The RF level of cell B is set

sufficiently low to ensure that

cell B is not suitable as defined

in GSM 05.08 section 6.6.2.

105	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

106	SS -> MS	IMMEDIATE ASSIGNMENT

107	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

108	SS		chooses randomly one among the

three following steps: (5 and 6)

or 11 or 16

109	SS		performs the chosen step and notes

the time when the CHANNEL RELEASE

message is sent

110	MS		The RF level of cell A is lowered

until the MS selects cell B.

The RF level of cell A is set

sufficiently low to ensure that

cell A is not suitable as defined

in GSM 05.08 section 6.6.2.

111	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

The time interval between the last

CHANNEL RELEASE message and this

message shall be less than

0.8*T3211

112	SS -> MS	IMMEDIATE ASSIGNMENT

113	MS -> SS	LOCATION UPDATING REQ	location updating type= normal,

CKSN= no key available ,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

114	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN

115	MS -> SS	AUTHENT RESPONSE

116	SS -> MS	LOCATION UPDATING ACC	Mobile identity= TMSI

117	MS -> SS	TMSI REALLOCATION CMP

118	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

MS is now "Idle, updated" in cell B

Specific message contents:

None.

CR for phase 2:

CR N18: from my point of view this CR is not relevant. The channel release has a global meaning for different actions: sending of a CHANNEL RELEASE message or deactivation of the SACCH in the uplink direction (this shows that the MS has released the channels). Furthermore we cannot check exactly T3211: that why I have included "T3211 at least".

CR N19: this CR is taken into account

CR N20: This CR has been taken into account but not for the indicated reason. In fact, the MS has started T3211. So when this timer will expire, the MS will start a location updating procedure. The test has to be performed before T3211 expiry. T3211 is equal to 15 seconds, we propose to wait a time T which is enough for the MS to be pageable, to send paging message, wait for a response during 3 seconds (with the following constraint T + 3 < 15) and then to switch off the MS and check it will not perform an IMSI detach procedure.

26.7.4.3.3	Location updating / abnormal cases / attempt counter equal to 4

[Editorial Note:

- it is not tested that the MS does not answer to paging with TMSI

- in phase 1 Procedure 2 ended with "the MS is switched off" and it did not appear in the requirement that the MS would send an IMSI detach indication: it has been added and now the MS is brought back to operation at the end of the test

]

Conformance requirement:

1)	When four failures such as cases d) to g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a normal location updating procedure the Mobile Station shall:

1.1	not perform normal location updating procedures.

1.2	perform periodic location updating after T3212 expiry by sending a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type set to "periodic updating".

1.3	if the periodic location updating was unsuccessful, after T3211 expiry the Mobile Station shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

2)	When four failures such as cases d), f), g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a normal location updating procedure the Mobile Station, if it supports speech, shall be able to perform an emergency call i.e. the Mobile Station is able to send a CM Service Request message with the CM Service Type IE set to "emergency call establishment", CKSN IE set to "no key is available" and Mobile Identity IE set to its IMSI and then send an Emergency Setup message.

3)	When four failures such as cases d), f), g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a normal location updating procedure

3.1	the Mobile Station shall use a request from CM entity for MM connection for a service other than emergency call as a trigger for a normal location updating procedure and shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

3.2	after a location updating triggered by a request from the CM layer which was .unsuccessful, after T3211 expiry the Mobile Station shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

4)	When four failures such as cases d), f), g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a normal location updating procedure

4.1	the Mobile Station shall perform a normal location updating procedure if it enters a new cell.

4.2	if this location updating is unsuccessful, after T3211 expiry the Mobile Station shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type IE set to "normal location updating".

Test purpose:

To verify that the MS performs only periodic location updating procedures, that it does not perform normal location updating procedures when its attempt counter has reached value 4 and that the MS reset its attempt counter after a timer T3212 expiry.

To verify that the MS still follows the MM-IDLE LIMITED SERVICE state requirements after its attempt counter has reached value 4.

To verify that the attempt counter is reset in the cases where it has to be done.

Reference(s):

GSM 04.08 section 4.4.4.9 and GSM 05.08 section 6.6.2

Related PICS/PIXIT statement(s):

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Support of speech Yes/No

Initial conditions:

System Simulator:

Two cells: A and B, belonging to different location areas a and b.

IMSI attach/detach is allowed in both cells.

T3212 is set to 6 minutes.

Mobile Station:

The MS is "Idle updated" on cell B with a valid CKSN and a TMSI.

Foreseen final state of the MS:

The MS is "Idle updated" on cell B with a valid CKSN and a TMSI.

Method of test:

The MS is made to perform a normal location updating. The SS triggers a failure in this procedure. After T3211 expiry the MS will try again the location updating procedure. The SS triggers again a failure. This is done again 2 times. At this point the attempt counter shall be equal to 4. It is then checked that T3212 has been started and that at its expiry the MS will try a periodic location updating procedure. It is verified that the MS has reset its attempt counter after timer T3212 expiry.

Then it is checked that, when the attempt counter has reached the value of 4, the MS is in the MM-IDLE state and LIMITED SERVICE substate, that is:

-	not perform an IMSI detach procedure,

-	support request for emergency call,

-	use requests from CM layer other than emergency call as triggering of a normal location updating procedure,

-	perform normal location updating procedure when a new cell is entered.

Maximum duration of test:

20 minutes

Expected sequence:

Step	Direction	Message	Comments

The following messages are sent and shall be

received on cell A

1	MS		The RF level of cell B is lowered

until the MS selec⌠its cell A.

The RF level of cell B is set

sufficiently low to ensure that

cell B is not suitable as defined

in GSM 05.08 section 6.6.2.

2	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

5	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value

randomly chosen:

* in Table 10.44 of GSM 04.08,

causes #2, #3, #6, #11, #12 and #13 being

excluded

6	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

7	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

during T3211 seconds at least after the

channel release.

8	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

9	SS -> MS	IMMEDIATE ASSIGNMENT

10	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

11	SS		The SS stops any RF transmission on

the dedicated channel and waits

until there is no more SACCH in the

uplink

12	MS		The MS shall notinitiate an RR connection

establishment on cell A or on cell B

during T3211 seconds at least after the

channel release

13	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

14	SS -> MS	IMMEDIATE ASSIGNMENT

15	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

16	SS -> MS	AUTHENT REQUEST

17	MS -> SS	AUTHENT RESPONSE	these steps (15 and 16) are

performed N times.

N shall be chosen in such a way

that T3210 expires.

18	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

during T3211 seconds at least after the

channel release.

19	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

20	SS -> MS	IMMEDIATE ASSIGNMENT

21	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

22	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

23	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

during T3212 (tolerance

[-15s; 45s]) at

least after the channel release.

24	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

25	SS -> MS	IMMEDIATE ASSIGNMENT

26	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE)

mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

27	SS -> MS	LOCATION UPDATING REJ	IE Reject cause = #17 "network

failure"

28	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

29	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

during T3211 seconds at

least after the channel release.

30	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

31	SS -> MS	IMMEDIATE ASSIGNMENT

32	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE)

mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

33	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN

34	MS -> SS	AUTHENT RESPONSE

35	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI

36	MS -> SS	TMSI REALLOCATION CMP

37	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

MS is now "idle, updated" in cell A

38	MS		The RF level of cell A is lowered

until the MS selects cell B.

The RF level of cell A is set

sufficiently low to ensure that

cell A is not suitable as defined

in GSM 05.08 section 6.6.2.

39	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

40	SS -> MS	IMMEDIATE ASSIGNMENT

41	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = a, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

42	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value

randomly chosen:

* in Table 10.44 of GSM 04.08,

causes #2, #3, #6, #11, #12 and #13 being

excluded

43	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

44	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

during T3211 seconds at

least after the channel release.

45	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

46	SS -> MS	IMMEDIATE ASSIGNMENT

47	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

48	SS		The SS stops any RF transmission on

the dedicated channel and waits

until there is no more SACCH in the

uplink

48a	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

during T3211 seconds at

least after the channel release.

49	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

50	SS -> MS	IMMEDIATE ASSIGNMENT

51	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

52	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

53	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

during T3211 seconds at

least after the channel release.

54	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

55	SS -> MS	IMMEDIATE ASSIGNMENT

56	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE)

mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

57	SS		chooses randomly one among the

three following steps: (39 and

40) or 45 or 49

58	SS		performs the chosen step

59	MS		If the MS supports speech, it is made to perform an

emergency call

60	MS -> SS	CHANNEL REQUEST	Establishment cause: Emergency call

61	SS -> MS	IMMEDIATE ASSIGNMENT

62	MS -> SS	CM SERVICE REQUEST	CM service type = Emergency call

establishment; CKSN = no key

available; Mobile Identity = IMSI

63	SS -> MS	CM SERVICE ACCEPT

64	MS -> SS	EMERGENCY SETUP

65	SS -> MS	RELEASE COMPLETE	Cause = unassigned number

66	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

67	MS		If possible (see PICS) SIM

detachment is performed. Otherwise

if possible (see PICS) switch off

is performed. Otherwise the power

is removed.

68	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B.

This is checked during 3 seconds.

69	MS		Depending on what has been

performed in step 67 the MS is

brought back to operation.

70	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

71	SS -> MS	IMMEDIATE ASSIGNMENT

72	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

73	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN

74	MS -> SS	AUTHENT RESPONSE

75	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI

76	MS -> SS	TMSI REALLOCATION CMP

77	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

MS is now "idle, updated" in cell B

78	MS		The RF level of cell B is lowered

until the MS selects cell A.

The RF level of cell B is set

sufficiently low to ensure that

cell B is not suitable as defined

in GSM 05.08 section 6.6.2.

79	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

80	SS -> MS	IMMEDIATE ASSIGNMENT

81	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

82	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value

randomly chosen:

* in Table 10.44 of GSM 04.08,

causes #2, #3, #6, #11 #12, and #13 being

excluded

83	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

84	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

during T3211 seconds at

least after the channel release.

85	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

86	SS -> MS	IMMEDIATE ASSIGNMENT

87	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

88	SS		The SS stops any RF transmission on

the dedicated channel and waits

until there is no more SACCH in the

uplink

88a	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

during T3211 seconds at

least after the channel release.

89	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

90	SS -> MS	IMMEDIATE ASSIGNMENT

91	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

92	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

93	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

during T3211 seconds at

least after the channel release.

94	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

95	SS -> MS	IMMEDIATE ASSIGNMENT

96	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE)

mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

97	SS		chooses randomly one among the

three following steps: (39 and

40) or 45 or 49

98	SS		performs the chosen step

99	MS		A MO CM connection is attempted

100	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

101	SS -> MS	IMMEDIATE ASSIGNMENT

102	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

103	SS		chooses randomly one among the

three following steps: (39 and

40) or 45 or 49

104	SS		performs the chosen step

105	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

during T3211 seconds at

least after the channel release.

106	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

107	SS -> MS	IMMEDIATE ASSIGNMENT

108	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

109	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN

110	MS -> SS	AUTHENT RESPONSE

111	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI

112	MS -> SS	TMSI REALLOCATION CMP

113	SS -> MS	CHANNEL RELEASE	MS is now "idle, updated" in cell A

The MS may have memorized the request

for CM connection

114	MS -> SS	CHANNEL REQUEST

115	SS -> MS	IMMEDIATE ASSIGNMENT

116	MS -> SS	CM SERVICE REQUEST	CKSN= initial value,

Mobile identity= TMSI

117	SS -> MS	CIPH MODE COMMAND

118	MS -> SS	CIPH MODE COMPLETE

119	MS -> SS	An initial CM message

120	SS -> MS	RELEASE COMPLETE	Cause = unassigned number

121	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

122	MS		The RF level of cell A is lowered

until the MS selects cell B.

The RF level of cell A is set

sufficiently low to ensure that

cell A is not suitable as defined

in GSM 05.08 section 6.6.2.

123	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

124	SS -> MS	IMMEDIATE ASSIGNMENT

125	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = a, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

126	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value

randomly chosen:

* in Table 10.44 of GSM 04.08,

causes #2, #3, #6, #11, #12 and #13 being

excluded

127	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

128	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

during T3211 seconds at

least after the channel release.

129	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

130	SS -> MS	IMMEDIATE ASSIGNMENT

131	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

132	SS		The SS stops any RF transmission on

the dedicated channel and waits

until there is no more SACCH in the

uplink

132a	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

during T3211 seconds at

least after the channel release.

133	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

134	SS -> MS	IMMEDIATE ASSIGNMENT

135	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

136	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

137	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

during T3211 seconds at

least after the channel release.

138	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

139	SS -> MS	IMMEDIATE ASSIGNMENT

140	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE)

mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

141	SS		chooses randomly one among the

three following steps: (39 and

40) or 45 or 49

142	SS		performs the chosen step

143	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

144	SS -> MS	IMMEDIATE ASSIGNMENT

145	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

146	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN

147	MS -> SS	AUTHENT RESPONSE

148	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI

149	MS -> SS	TMSI REALLOCATION CMP

150	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

MS is now "idle, updated" in cell B

151	MS		The RF level of cell B is lowered

until the MS selects cell A.

The RF level of cell B is set

sufficiently low to ensure that

cell B is not suitable as defined

in GSM 05.08 section 6.6.2.

152	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

153	SS -> MS	IMMEDIATE ASSIGNMENT

154	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

155	SS -> MS	LOCATION UPDATING REJ	IE Reject cause is set to a value

randomly chosen:

* in Table 10.44 of GSM 04.08,

causes #2, #3, #6, #11, #12 and #13 being

excluded

156	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

157	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

during T3211 seconds at

least after the channel release.

158	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

159	SS -> MS	IMMEDIATE ASSIGNMENT

160	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

161	SS		The SS stops any RF transmission on

the dedicated channel and waits

until there is no more SACCH in the

uplink

162	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

during T3211 seconds at

least after the channel release.

163	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

164	SS -> MS	IMMEDIATE ASSIGNMENT

165	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

1660	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

167	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

during T3211 seconds at

least after the channel release.

168	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

169	SS -> MS	IMMEDIATE ASSIGNMENT

170	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE)

mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

171	SS		chooses randomly one among the

three following steps: (39 and

40) or 45 or 49

172	SS		performs the chosen step

173	MS		The RF level of cell A is lowered

until the MS selects cell B.

The RF level of cell A is set

sufficiently low to ensure that

cell A is not suitable as defined

in GSM 05.08 section 6.6.2.

174	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

175	SS -> MS	IMMEDIATE ASSIGNMENT

176	MS -> SS	LOCATION UPDATING REQ	location updating type= normal,

CKSN= no key available ,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE)

mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

177	SS		chooses randomly one among the

three following steps: (39 and

40) or 45 or 49

178	SS		performs the chosen step

179	MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B

during T3211 seconds at

least after the channel release.

180	MS -> SS	CHANNEL REQUEST	Establishment cause= All other

cases

181	SS -> MS	IMMEDIATE ASSIGNMENT

182	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE),

Mobile Identity =IMSI.

183	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN

184	MS -> SS	AUTHENT RESPONSE

185	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity = new TMSI

186	MS -> SS	TMSI REALLOCATION CMP

187	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

MS is now "idle, updated" in cell B

Specific message contents:

None.

CR for phase 2:

CR N07 rev 1: taken into account

CR N18 and N19: same comment as in 26.7.4.3.2

CR N23: no more valid

CR N25 and N26: included

26.7.4.3.4	Location updating / abnormal cases / attempt counter less or equal to 4, stored LAI equal to broadcast LAI

[Editorial Note:

- In steps 14 and 34 time to check there is no location updating procedure is reduced from 0.5*T3212 (= 3 minutes) to 2*T3211 (=30 seconds). This time is put to check that T3211 has been stopped with the successful MM establishment. This will reduce the test time.

]

Conformance requirement:

1)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a periodic location updating procedure (the broadcast LAI is equal to the stored LAI),

1.1	the Mobile Station shall be able to establish an MM connection i.e. send a Channel Request and then a CM Service Request message, CKSN and LAI set to those which have been allocated to the Mobile Station, Mobile Identity IE set to the TMSI which has been allocated to the Mobile Station

1.2	then the Mobile Station shall not attempt a location updating procedure.

2)	When a failure such as cases d), f) and g) of section 4.4.4.8 of TS GSM 04.08 has occurred during an IMSI attach procedure (the broadcast LAI is equal to the stored LAI).

2.1	the Mobile Station shall be able to establish an MM connection i.e. send a Channel Request and then a CM Service Request message, CKSN and LAI set to those which have been allocated to the Mobile Station, Mobile Identity IE set to the TMSI which has been allocated to the Mobile Station

2.2	then the Mobile Station shall not attempt a location updating procedure.

3)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during a periodic location updating procedure and the attempt counter is smaller than 4 the Mobile Station shall send, after T3211 expiry, a Location Updating Request message with the Mobile Identity IE set to the TMSI which has been allocated to the Mobile Station, CKSN IE and LAI set to those which have been allocated to the Mobile Station and the Location Updating type set to "periodic updating".

3.2	and when the Mobile Station's attempt counter reaches the value 4 (four failures such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a periodic location updating procedure) after T3212 expiry it shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type set to "periodic updating".

4)	When the Mobile Station's attempt counter reaches the value 4 (four failures such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 have occurred during a periodic location updating procedure) it shall use a request for a CM connection other than emergency call as a trigger for a location updating procedure.

5)	When a failure such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 has occurred during an IMSI attach procedure and the attempt counter is smaller than 4 the Mobile Station shall send, after T3211 expiry, a Location Updating Request message with the Mobile Identity IE set to the TMSI which has been allocated to the Mobile Station, CKSN IE and LAI set to those which have been allocated to the Mobile Station and the Location Updating type set to "IMSI attach".

5.2	and when the Mobile Station's attempt counter reaches the value 4 (four failures such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 have occurred during an IMSI attach procedure) after T3212 expiry it shall send a Location Updating Request message with the Mobile Identity IE set to its IMSI, CKSN IE set to "no key is available" and the Location Updating type set to "periodic updating".

6)	When the Mobile Station's attempt counter reaches the value 4 (four failures such as cases d), f) and g) of section 4.4.4.9 of TS GSM 04.08 have occurred during an IMSI attach procedure) it shall use a request for a CM connection other than emergency call as a trigger for a location updating procedure.

Test purpose:

To verify that in the case when the attempt counter is smaller than 4 and the broadcast LAI is equal to the stored LAI, the MS is in the MM-IDLE state and NORMAL SERVICE substate. To verify that timer T3211 is stopped after a MM connection establishment.

To verify that the MS uses the T3211 timer. and that it enters the MM-IDLE state and NORMAL SERVICE substate when its attempt counter reaches value 4 even in the case where the stored LAI is equal to the broadcast LAI.

Reference(s):

GSM 04.08 section 4.4.4.9

Related PICS/PIXIT statement(s):

SIM removal possible without powering down Yes/No

Switch off on button Yes/No

Initial conditions:

System Simulator:

One cell: B, belonging to location area b.

IMSI attach/detach is allowed.

T3212 is set to 6 minutes.

Mobile Station:

The MS is "Idle updated" on cell B with a valid CKSN and a TMSI.

Foreseen final state of the MS:

The MS is "idle updated" on cell B with a valid CKSN and a TMSI.

Method of test:

A failure during the periodic location updating is triggered: as the broadcast LAI is equal to the stored LAI, the MS is still in the MM-IDLE state and NORMAL SERVICE substate and timer T3211 is started. A CM connection other than for emergency call is attempted. It is checked that this is possible and that T3211 is stopped. Same test is performed with a failure during an IMSI attach procedure.

Then failures are triggered during the periodic location updating to let the attempt counter to reach the value of 4. The MS shall enter the MM-IDLE LIMITED SERVICE state and delete any TMSI, stored LAI, ciphering key sequence number and ciphering key. When the attempt counter reaches the value of 4, timer T3212 shall be started. At timer T3212 expiry a location updating procedure is started. A request for CM connection other for than emergency call shall trigger a location updating procedure.

Same tests are performed when the failures are triggered during an IMSI attach procedure.

Maximum duration of test:

40 minutes

Expected sequence:

Step	Direction	Message	Comments

1	SS		The SS shall wait at most T3212

2	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

5	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

6	SS		performs the chosen step

7	MS		A MO CM connection is attempted

8	MS -> SS	CHANNEL REQUEST

9	SS -> MS	IMMEDIATE ASSIGNMENT

10	MS -> SS	CM SERVICE REQUEST	CKSN= initial CKSN, Mobile Identity= TMSI

11	SS -> MS	CM SERVICE ACCEPT

12	MS -> SS	An initial CM message

13	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

14	SS		The MS shall not initiate an RR connection

establishment. This is checked during 2*T3211

15	MS		If possible (see PICS) SIM

detachment is performed. Otherwise

if possible (see PICS) switch off

is performed. Otherwise the power

is removed.

16	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

17	SS -> MS	IMMEDIATE ASSIGNMENT

18	MS -> SS	IMSI DETACH INDICATION

19	SS -> MS	CHANNEL RELEASE

20	MS		Depending on what has been

performed in step 15 the MS is

brought back to operation.

21	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

22	SS -> MS	IMMEDIATE ASSIGNMENT

23	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI

attach, CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

24	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

25	SS		performs the chosen step

26	MS		A MO CM connection is attempted

27	MS -> SS	CHANNEL REQUEST

28	SS -> MS	IMMEDIATE ASSIGNMENT

29	MS -> SS	CM SERVICE REQUEST	CKSN= initial CKSN, Mobile Identity= TMSI

30	SS -> MS	CIPH MODE COMMAND

31	MS -> SS	CIPH MODE COMPLETE

32	MS -> SS	An initial CM message

33	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

34	SS		The MS shall not initiate an RR connection

establishment. This is checked during 2*T3211

MS is "idle, updated" in cell B.

35	SS		The SS shall wait at most T3212

36	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

37	SS -> MS	IMMEDIATE ASSIGNMENT

38	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

39	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

40	SS		performs the chosen step

41	MS		The MS shall not initiate an RR connection

establishment during T3211 at

least after the channel release.

42	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

43	SS -> MS	IMMEDIATE ASSIGNMENT

44	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

45	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

46	SS		performs the chosen step

47	MS		The MS shall not initiate an RR connection

establishment during T3211 at

least after the channel release.

48	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

49	SS -> MS	IMMEDIATE ASSIGNMENT

50	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

51	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

52	SS		performs the chosen step

53	MS		The MS shall not initiate an RR connection

establishment during T3211 at

least after the channel release.

54	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

55	SS -> MS	IMMEDIATE ASSIGNMENT

56	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

57	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

58	SS		performs the chosen step

59	MS		The MS shall not initiate an RR connection

establishment during T3212 at

least after the channel release.

60	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

61	SS -> MS	IMMEDIATE ASSIGNMENT

62	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic

or normal (see Note 1),

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE)

mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

63	SS -> MS	AUTHENT REQUEST

64	MS -> SS	AUTHENT RESPONSE

65	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity= TMSI

66	SS -> MS	CHANNEL RELEASE	The SS waits for the disconnection

of the main signalling link

67	MS		The MS shall no initiate an RR connection

establishment earlier than T3212 after the

transmission of the CHANNEL RELEASE

in step 66

68	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

69	SS -> MS	IMMEDIATE ASSIGNMENT

70	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

71	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

72	SS		performs the chosen step

73	MS		The MS shall not initiate an RR connection

establishment during T3211 at

least after the channel release.

74	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

75	SS -> MS	IMMEDIATE ASSIGNMENT

76	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

77	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

78	SS		performs the chosen step

79	MS		The MS shall not initiate an RR connection

establishment during T3211 at

least after the channel release.

80	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

81	SS -> MS	IMMEDIATE ASSIGNMENT

82	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

83	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

84	SS		performs the chosen step

85	MS		The MS shall not initiate an RR connection

establishment during T3211 at

least after the channel release.

86	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

87	SS -> MS	IMMEDIATE ASSIGNMENT

88	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic,

CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

89	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

90	SS		performs the chosen step

91	MS		A MO CM connection is attempted

92	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

93	SS -> MS	IMMEDIATE ASSIGNMENT

94	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE)

mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

95	SS -> MS	LOCATION UPDATING ACC	IE mobile identity = TMSI

96	SS -> MS	CHANNEL RELEASE

97	MS -> SS	CHANNEL REQUEST

98	SS -> MS	IMMEDIATE ASSIGNMENT

99	MS -> SS	CM SERVICE REQUEST	CKSN = no key available,

Mobile identity= TMSI

100	SS -> MS	AUTHENT REQUEST	CKSN = initial CKSN

101	MS -> SS	AUTHENT RESPONSE

102	SS -> MS	CIPH MODE COMMAND

103	MS -> SS	CIPH MODE COMPLETE

104	MS -> SS	An initial CM message

105	SS -> MS	RELEASE COMPLETE	Cause = unassigned number

106	SS -> MS	CHANNEL RELEASE

107	MS		If possible (see PICS) SIM

detachment is performed. Otherwise

if possible (see PICS) switch off

is performed. Otherwise the power

is removed.

108	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

109	SS -> MS	IMMEDIATE ASSIGNMENT

110	MS -> SS	IMSI DETACH INDICATION

111	SS -> MS	CHANNEL RELEASE

112	MS		Depending on what has been

performed in step 107 the MS is

brought back to operation.

113	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

114	SS -> MS	IMMEDIATE ASSIGNMENT

115	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI

attach, CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

116	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

117	SS		performs the chosen step

118	MS		The MS shall not initiate an RR connection

establishment during T3211 at

least after the channel release.

119	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

120	SS -> MS	IMMEDIATE ASSIGNMENT

121	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI

attach, CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

122	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

123	SS		performs the chosen step

124	MS		The MS shall not initiate an RR connection

establishment during T3211 at

least after the channel release.

125	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

126	SS -> MS	IMMEDIATE ASSIGNMENT

127	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI

attach, CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

128	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

129	SS		performs the chosen step

130	MS		The MS shall not initiate an RR connection

establishment during T3211 at

least after the channel release.

131	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

132	SS -> MS	IMMEDIATE ASSIGNMENT

133	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI

attach, CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

134	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

135	SS		performs the chosen step

136	MS		The MS shall not initiate an RR connection

establishment during T3212 at

least after the channel release.

137	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

138	SS -> MS	IMMEDIATE ASSIGNMENT

139	MS -> SS	LOCATION UPDATING REQ	location updating type = periodic

or normal or IMSI attach (see

Note 2), CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE)

mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

140	SS -> MS	AUTHENT REQUEST

141	MS -> SS	AUTHENT RESPONSE

142	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity= TMSI

142a	MS -> SS	TMSI REALLOCATION CMP

143	SS -> MS	CHANNEL RELEASE

144	MS		If possible (see PICS) SIM

detachment is performed. Otherwise

if possible (see PICS) switch off

is performed. Otherwise the power

is removed.

145	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

146	SS -> MS	IMMEDIATE ASSIGNMENT

147	MS -> SS	IMSI DETACH INDICATION

148	SS -> MS	CHANNEL RELEASE

149	MS		Depending on what has been

performed in step 144 the MS is

brought back to operation.

150	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

151	SS -> MS	IMMEDIATE ASSIGNMENT

152	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI

attach, CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

153	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

154	SS		performs the chosen step

155	MS		The MS shall not initiate an RR connection

establishment during T3211 at

least after the channel release.

156	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

157	SS -> MS	IMMEDIATE ASSIGNMENT

158	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI

attach, CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

159	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

160	SS		performs the chosen step

161	MS		The MS shall not initiate an RR connection

establishment during T3211 at

least after the channel release.

162	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

163	SS -> MS	IMMEDIATE ASSIGNMENT

164	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI

attach, CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

165	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

166	SS		performs the chosen step

167	MS		The MS shall not initiae an RR connection

establishment during T3211 at

least after the channel release.

168	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

169	SS -> MS	IMMEDIATE ASSIGNMENT

170	MS -> SS	LOCATION UPDATING REQ	location updating type = IMSI

attach, CKSN = initial value,

LAI = b, mobile station classmark 1

as given by the PICS and

mobile identity = TMSI.

171	SS		chooses randomly one among the

three following choices: (5 and 6)

or 11 or 16 of 26.7.4.3.2

172	SS		performs the chosen step

173	MS		The MS is made to perform a MO call

174	MS -> SS	CHANNEL REQUEST	Establishment cause: All other

cases

175	SS -> MS	IMMEDIATE ASSIGNMENT

176	MS -> SS	LOCATION UPDATING REQ	location updating type = normal,

CKSN = no key available,

LAI = deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE)

mobile station

classmark 1 as given by the PICS

and mobile identity = IMSI.

177	SS -> MS	AUTHENT REQUEST

178	MS -> SS	AUTHENT RESPONSE

179	SS -> MS	LOCATION UPDATING ACC	IE mobile Identity= TMSI

179a	MS -> SS	TMSI REALLOCATION CMP

180	SS -> MS	CHANNEL RELEASE

181	MS	A MO CM connection is atempted

182	MS -> SS	CHANNEL REQUEST

183	SS -> MS	IMMEDIATE ASSIGNMENT

184	MS -> SS	CM SERVICE REQUEST	CKSN = initial value,

Mobile identity= TMSI

185	SS -> MS	CM SERVICE ACCEPT

186	MS -> SS	An initial CM message

187	SS -> MS	RELEASE COMPLETE	Cause = unassigned number

188	SS -> MS	CHANNEL RELEASE

Note 1:	the MS can include both types of Location updating. As T3212 expires it can be a periodic location updating procedure and as there is no stored LAI it can be a normal one.

Note 2:	same problem as in note 1. Three types of location updating procedures should be allowed.

Specific message contents:

None.

26.7.4.4	Location updating / release / expiry of T3240

[ editorial notes :

- no conformance requirement

- new tests may be needed]

Test purpose:

To verify that the MS aborts the RR-connection at the expiry of timer T3240.

Reference(s):

GSM 04.08 section 4.4.4.7.

Related PICS/PIXIT statement(s):

None.

Initial conditions:

System Simulator:

Two cells : A and B, belonging to different location areas a and b.

Mobile Station:

The MS has a valid TMSI. It is "idle updated" on cell A.

Foreseen final state of the MS:

The MS is "idle updated" on cell B

Method of test:

A normal location updating procedure is performed. The RR-connection is not released by the SS within the timer T3240. It is checked that the MS aborts the RR-connection.

Maximum duration of test:

1 minute

Expected sequence:

Step	Direction	Message	Comments

1	   SS		The RF level of cell A is

lowered until the MS selects

cell B.

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ

5	SS -> MS	LOCATION UPDATING ACC

6	   SS		The SS waits T3240 expiry.

7	   MS		The MS shall abort the RR

connection (disconnection of layer 2).

Specific message contents:

None.

26.7.4.5	Location updating / periodic

26.7.4.5.1	Location updating / periodic spread

26.7.4.5.1.1	Conformance requirement:

1)	The Mobile Stations shall perform spreading of the time before performing a periodic location updating when the location updating timer value is reduced.

2)	The Mobile Station shall reset timer T3212 when the Mobile Station is deactivated, and shall start with a value between zero and the broadcasted value when reactivated in the same cell, IMSI attach being forbidden.

3)	When activated the Mobile Station shall start timer T3212 with a value randomly drawn in the allowed range.

Note :	This conformance requirement is not covered by a test purpose. It is intended to be covered by a manufacturer declaration.

26.7.4.5.1.2	Test purpose:

1)	To check that when the location updating timer is reduced, the timer running in the MS is started with a value depending on the current timer value and the new broadcasted T3212 value.

2)	To verify that when the MS is reactivated in the same cell (as the one in which it was deactivated), IMSI attach being forbidden, the MS starts the timer T3212 with a value between zero and the broadcasted value

Note:	It is not tested that the value is random.

26.7.4.5.1.3	Reference(s):

GSM 04.08 section 4.4.2.

26.7.4.5.1.4	Related PICS/PIXIT statement(s):

None.

26.7.4.5.1.5	Initial conditions:

System Simulator:

One cell, T3212 is set to 30 minutes.

IMSI attach is allowed in the cell.

Mobile Station:

The MS is deactivated. The stored MCC, MNC and LAC correspond to the broacasted values. The stored update status is "updated".

26.7.4.5.1.6	Foreseen final state of the MS:

The MS has a valid TMSI. It is "idle updated".

26.7.4.5.1.7	Test procedure:

The MS is activated. It performs IMSI attach. 3 minutes after the end of the IMSI attach procedure, the value of T3212 is set to 6 minutes. The MS shall perform periodic location updating 6 minutes after the end of the IMSI attach procedure.

Then, the IMSI attach/detach is forbidden. T3212 is still set to 6 minutes.

The MS is deactivated. The MS is reactivated. It is checked that the MS performs a periodic location updating during the 6 minutes following activation.

Maximum duration of test:

20 minutes

Expected sequence:

Step	Direction	Message	Comments

1	   MS		The MS is activated

2	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ	"location updating type": IMSI attach

5	SS -> MS	LOCATION UPDATING ACC

6	SS -> MS	CHANNEL RELEASE	After the sending of this message,

the SS waits for the disconnection

of the main signalling link.

7	   SS		3 minutes after step 6 the value of T3212

is set to 6 minutes

8	   SS		the SS waits for 4 minutes

9	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

This message shall be sent

by the MS between 5minutes 45s and

6minutes 15s after step 6.

10	SS -> MS	IMMEDIATE ASSIGNMENT

11	MS -> SS	LOCATION UPDATING REQ

"location updating type": periodic updating

12	SS -> MS	LOCATION UPDATING ACC

13	SS -> MS	CHANNEL RELEASE

After the sending of this message,

the SS waits for the disconnection

of the main signalling link.

14	   SS		IMSI attach/detach is not allowed.

15	   MS		The MS is deactivated

16	   MS		The MS is activated

17	   SS		The SS waits until the periodic

location updating.

18	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message shall

arrive during the 7 minutes

following the MS activation.

19	SS -> MS	IMMEDIATE ASSIGNMENT

20	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= periodic

21	SS -> MS	LOCATION UPDATING ACC

22	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.4.5.2	Location updating / periodic normal / test 1

26.7.4.5.2.1	Conformance requirement:

1	The Mobile Station shall reset the timer T3212 of the periodic location updating procedure when the first MM message is received or ciphering mode setting is completed in the case of MM connection establishment.

2	The Mobile Station shall reset the timer T3212 of the periodic location updating procedure when the Mobile Station has responded to paging and thereafter has received the first correct L3 message.

26.7.4.5.2.2	Test purpose:

To verify that the MS resets the timer T3212 of the periodic location updating procedure when :

-	the first MM-message is received in the case of MM-connection establishment, ciphering mode being not set,

-	the MS has responded to paging and the first L3 message is received.

Note :	T3212 is stopped when the MM-idle state is left and restarted when the MM sublayer returns to that state, substate NORMAL SERVICE or ATTEMPTING TO UPDATE. As a consequence, the exact time when T3212 is reset between those two events cannot be tested.

26.7.4.5.2.3	Reference(s):

GSM 04.08 section 4.4.2.

26.7.4.5.2.4	Related PICS/PIXIT statement(s):

None.

26.7.4.5.2.5	Initial conditions:

System Simulator:

1 cell, default parameters.

IMSI attach/detach is not allowed.

The T3212 timeout value is 2/10 hour .

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

26.7.4.5.2.6	Foreseen final state of the MS:

The MS has a valid TMSI. It is "idle updated".

26.7.4.5.2.7	Test procedure:

An MS originated MM connection is established and cleared. The channel is released. It is checked that the MS performs a periodic location updating 12 minutes after the release of the channel.

One minute after the periodic location updating, the MS is paged, it sends a CHANNEL REQUEST message and the SS responds with an IMMEDIATE ASSIGNMENT message, a call is established and then cleared. It is checked that the MS performs a periodic location updating 12 minutes after the release of the link.

Maximum duration of test:

30 minutes.

Expected sequence:

Step	Direction	Message	Comments

1	   MS		A MO CM connection is attempted

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	CM SERVICE ACCEPT

6	MS -> SS	An initial CM message

7	SS -> MS	RELEASE COMPLETE	"Cause" IE : "unassigned

number".

8	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

9	   SS		The SS waits until the periodic

location updating.

10	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message shall

arrive between11 minutes 45 s and

12 minutes 15 s after

the last release of the RR

connection by the SS.

11	SS -> MS	IMMEDIATE ASSIGNMENT

12	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= periodic

13	SS -> MS	LOCATION UPDATING ACC

14	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

15	   SS		The SS waits 1 minute.

16	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity"= IMSI.

17	MS -> SS	CHANNEL REQUEST	"Establishment cause" :

Answer to paging.

18	SS -> MS	IMMEDIATE ASSIGNMENT

19	MS -> SS	PAGING RESPONSE

20	SS -> MS	AUTHENTICATION REQ

21	MS -> SS	AUTHENTIC RESPONSE

22	SS - MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

23	   SS		The SS waits until the periodic

location updating.

24	MS -> SS	CHANNEL REQUEST

"Establishment cause" : All

other cases. This message shall

arrive between 11 minutes 45 s and

12 minutes 15 s after

the last release of the RR

connection by the SS.

25	SS -> MS	IMMEDIATE ASSIGNMENT

26	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= periodic

27	SS -> MS	LOCATION UPDATING ACC

28	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.4.5.3	Location updating / periodic normal / test 2

26.7.4.5.3.1	Conformance requirement:

When a generic location updating or an IMSI attach procedure is started, the Mobile Station shall perform a periodic location updating after T3212 expiry.

26.7.4.5.3.2	Test purpose:

To verify that the MS resets the timer T3212 of the periodic location updating procedure when :

-	a generic location updating is started,

-	IMSI attach is started.

Note :	T3212 is stopped when the MM-idle state is left and restarted when the MM sublayer returns to that state, substate NORMAL SERVICE or ATTEMPTING TO UPDATE. As a consequence, the exact time when T3212 is reset between those two events cannot be tested.

26.7.4.5.3.3	Reference(s):

GSM 04.08 section 4.4.2.

26.7.4.5.3.4	Related PICS/PIXIT statement(s):

SIM removal possible without powering down Yes/No

Switch off on button yes/No

26.7.4.5.3.5	Initial conditions:

System Simulator:

2 cells, IMSI attach/detach is allowed in both cells.

T3212 is set to 6 minutes.

Mobile Station:

The MS has a valid TMSI. It is "idle updated"an cell A.

26.7.4.5.3.6	Foreseen final state of the MS:

The MS has a valid TMSI. It is "idle updated" on cell B.

26.7.4.5.3.7	Test procedure

A normal location updating is performed. The channel is released. One minute later, the MS is deactivated, then reactivated in the same cell. It is checked that the MS performs an IMSI attach and a periodic location updating 6 minutes after the IMSI attach.

Maximum duration of test:

20 minutes

Expected sequence:

Step	Direction	Message	Comments

The following messages are sent and shall be

received on cell B.

1	SS		The RF level of cell A is

lowered until the MS selects

cell B.

2	MS -> SS	CHANNEL REQUEST	"establishment cause" : all other cases

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal

5	SS -> MS	LOCATION UPDATING ACC

6	SS ->MS	CHANNEL RELEASE	After the sending of this message,

the SS waits for the disconnection

of the main signalling link.

7	   SS		The SS waits until the periodic

location updating.

8	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message shall

arrive between 5 minutes 45 s and 6 minutes 15 s

after the last release of the RR

connection by the SS.

9	SS -> MS	IMMEDIATE ASSIGNMENT

10	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= periodic

11	SS -> MS	LOCATION UPDATING ACC

12	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

13	   MS		If possible (see PICS) SIM

removal is performed. Otherwise

if possible (see PICS) switch off

is performed. Otherwise the power

is removed.

steps 14 to 17 may be performed or not

depending on the action made in step 13

14	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

15	SS -> MS	IMMEDIATE ASSIGNMENT

16	MS -> SS	IMSI DETACH INDICATION

17	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

18	   MS		Depending on what has been

performed in step 13 the MS is

brought back to operation.

19	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

20	SS -> MS	IMMEDIATE ASSIGNMENT

21	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= IMSI

attach.

22	SS -> MS	LOCATION UPDATING ACC

23	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

24	   SS		The SS waits until the periodic

location updating.

25	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases. This message shall

arrive between 5 minutes 45 s and 6 minutes 15 s

after the last release of the RR

connection by the SS.

26	SS -> MS	IMMEDIATE ASSIGNMENT

27	MS -> SS	LOCATION UPDATING REQ	"Location updating type"= periodic

28	SS -> MS	LOCATION UPDATING ACC

29	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.5	MM connection

26.7.5.1	Introduction

26.7.5.2	MM connection / establishment with cipher

Conformance requirement:

1)	The Mobile Station shall be able to correctly set up an MM connection in a Mobile Originating CM connection attemptand send a CM Service Request message with CKSN information element as stored in the SIM and Mobile Identity information element set to the TMSI

2)	The Mobile Station shall be able to interpret cipher mode setting as acceptance of its CM service request i.e. send a CM message.

Test purpose:

To verify that the MS can correctly set up an MM connection in an origination and interpret cipher mode setting as acceptance of its CM service request.

Reference(s):

GSM 04.08 sections 4.5.1.1.

Related PICS/PIXIT statement(s):

Support of CC protocol Yes/No.

Initial conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Foreseen final state of the MS:

The MS has valid TMSI, CKSN. It is "idle updated".

Method of test:

A mobile originating call is initiated. After the MS has sent the CM SERVICE REQUEST message to the SS, an authentication procedure and a ciphering mode setting procedure are performed. Then, the MS sends a CM message and the SS clears the call and releases the channel.

Maximum duration of test:

One minute

Expected sequence:

Step	Direction	Message	Comments

1	   MS		A MO CM onnection is attempted

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	AUTHENT REQUEST

6	MS -> SS	AUTHENT RESPONSE

7	SS -> MS	CIPH MODE COMMAND	The SS starts deciphering.

8	MS -> SS	CIPH MODE COMPLETE	The SS starts enciphering.

9	MS -> SS	An initial CM message

10	SS -> MS	RELEASE COMPLETE	"Cause" IE : "unassigned

number".

11	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.5.3	MM connection / establishment without cipher

Conformance requirement:

1)	Upon reception of the CM SERVICE ACCEPT message, the MS shall send a CM message.

Test purpose:

To verify that the MS can correctly set up an MM connection in an originating CM connection establishment when ciphering mode setting is not required.

Reference(s):

GSM 04.08 sections 4.5.1.1.

Related PICS/PIXIT statement(s):

None.

Initial conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Foreseen final state of the MS:

The MS has a valid TMSI. It is "idle updated".

Method of test:

A mobile originating CM connection is attempted. The MM-connection is established without invoking the ciphering mode setting procedure.

Then, the MS sends a CM message and the SS releases the channel.

Maximum duration of test:

one minute

Expected sequence:

Step	Direction	Message	Comments

1	   MS		A MO CM connection is attempted

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	CM SERVICE ACCEPT

6	MS -> SS	An initial CM message

7	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.5.4	MM connection / establishment rejected

Conformance requirement

Upon reception of a CM SERVICE REJECT message, the MS shall not send any layer 3 message, start timer T3240 and enter the "wait for network command" state.

Test purpose:

To verify that the MS does not send a layer 3 message when the service request is rejected by the SS.

Reference(s):

GSM 04.08 sections 4.5.1.1.

Related PICS/PIXIT statement(s):

None.

Initial conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Foreseen final state of the MS:

The MS has a valid TMSI; It is "idle updated".

Method of test:

A mobile originating CM connection is attempted. After the MS has sent the CM SERVICE REQUEST message to the SS, the SS responds with a CM SERVICE REJECT message with reject cause "requested service option not subscribed". It is checked that the MS does not send a layer 3 message.

Maximum duration of test:

1 minute

Expected sequence:

Step	Direction	Message	Comments

1	   MS		A MO CM connection is attemped

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	CM SERVICE REJECT	"Reject cause" IE : "requested

service option not subscribed".

6	   SS		The MS shall not send a layer 3

message. This is checked during

5 seconds.

7	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.5.5	MM connection / establishment rejected cause 4

Conformance requirement:

1)	The Mobile Station shall be able to correctly set up an MM connection in a Mobile Originating CM connection attempt and send a CM Service Request message with CKSN information element as stored in the SIM and Mobile Identity information element set to the TMSI

2)	The Mobile Station, when receiving a CM SERVICE REJECT message with reject cause "IMSI unknown in VLR" shall wait for the network to release the RR connection.

3)	The Mobile Station shall then be able to perform a location updating procedure.

Test purpose:

To verify that the MS can correctly accept a CM SERVICE REJECT message with reject cause "IMSI unknown in VLR".

Reference(s):

GSM 04.08 sections 4.5.1.1.

Related PICS/PIXIT statement(s):

None.

Initial conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Foreseen final state of the MS:

The MS has valid TMSI, CKSN. It is "idle updated".

Method of test:

A mobile originating CM connection is attempted. After the MS has sent the CM SERVICE REQUEST message to the SS, the SS responds with a CM SERVICE REJECT message with reject cause "IMSI unknown in VLR". On receipt of this message, the MS shall delete any TMSI, LAI, cipher key and cipher key sequence number. The channel is released. It is checked that the MS performs a normal location updating procedure.

Maximum duration of test:

One minute

Expected sequence:

Step	Direction	Message	Comments

1	   MS		A MO CM connection is attempted

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	CM SERVICE REJECT	"Reject cause"= "IMSI

unknown in VLR"

6	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

7	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

8	SS -> MS	IMMEDIATE ASSIGNMENT

9	MS -> SS	LOCATION UPDATING REQ	"Ciphering key sequence number"=

"No key is available".

"Mobile identity"= IMSI.

"Location area identification"=

deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE).

11	SS -> MS	AUTHENT REQUEST

12	MS -> SS	AUTHENT RESPONSE

13	SS -> MS	LOCATION UPDATING ACC	"Mobile identity"= new TMSI.

14	MS -> SS	TMSI REALLOCATION CMP

15	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.5.6	MM connection / expiry T3230

[ editorial notes :

- no conformance requirement

]

Test purpose:

To verify that at T3230 expiry, the MS aborts the MM-connection establishment.

Reference(s):

GSM 04.08 section 4.5.1.2.

Related PICS/PIXIT statement(s):

None.

Initial conditions:

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS has a valid TMSI. It is "idle updated".

Foreseen final state of the MS:

The MS has a valid TMSI. It is "idle updated".

Method of test:

A mobile originating CM connection is attempted. After the MS has sent the CM SERVICE REQUEST message to the SS, the SS waits for expiry of timer T3230. It is checked that the MS does not send a layer 3 message but waits for the release of the RR-connection.

Maximum duration of test:

1 minute

Expected sequence:

Step	Direction	Message	Comments

1	   MS		A MO CM connection is attempted.

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	   SS		The SS waits for expiry of

timer T3230.

6	SS -> MS	CM SERVICE ACCEPT

7	MS -> SS	MM-STATUS	"Reject cause " IE is "message

not compatible with the call

state or not implemented".

8	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.5.7	MM connection / abortion by the network

26.7.5.7.1 MM connection / abortion by the network / cause #6

26.7.5.7.1.1	Conformance Requirement

1)	Upon reception of an ABORT message, the MS shall release any ongoing MM connection and enter the "wait for network command" state.

2)	If the cause in the ABORT message was cause #6, the Mobile Station shall:

2.1	not perform normal location updating

2.2	not perform periodic location updating

2.3	not respond to paging with TMSI

2.4	reject any request for Mobile Originating call establishment except Emergency call

2.5	not perform IMSI detach if deactivated

3)	After reception of an ABORT message with cause #6, the Mobile Station, if it supports speech, shall accept a request for an emergency call by sending a Channel Request message with the establishment cause set to "emergency call".

4)	After reception of an ABORT message with cause #6, the Mobile Station shall delete the stored LAI, CKSN and TMSI.

26.7.5.7.1.2	Test Purpose

To check that upon reception of an ABORT message with cause #6 during call establishment :

-	the MS does not send any layer 3 message.

-	after reception of an ABORT message and after having been deactivated and reactivated, the MS performs location updating using its IMSI as mobile identity and indicates deleted LAI and CKSN.

-	the MS does not perform location updating, does not answer to paging with TMSI, rejects any request for mobile originating call except emergency call, does not perform IMSI detach.

-	the MS accepts a request for emergency call.

26.7.5.7.1.3	Reference(s)

GSM 04.08 section 4.3.5

26.7.5.7.1.4	Related PICS/PIXIT Statement(s)

SIM removal possible without powering down Yes/No.

Switch off on button Yes/No

Support of speech Yes/No

26.7.5.7.1.5	Initial Conditions

System Simulator:

2 cells, default parameters.

Mobile Station:

The MS has a valid TMSI, CKSN and Kc. It is "idle updated" on cell B.

26.7.5.7.1.6	Foreseen final state of the MS:

The MS has a valid TMSI. It is "idle updated" on cell A.

26.7.5.7.1.7	Test procedure:

A mobile originating call is attempted. Upon reception of the AUTHENTICATION RESPONSE message, the SS sends an ABORT message with cause #6. The SS waits for 5 seconds. The MS shall not send any layer 3 message. The SS releases the RR connection.

The SS checks that the MS has entered the state MM-IDLE substate NO IMSI, i.e. does not perform normal location updating, does not perform periodic updating, does not respond to paging, rejects any requests from CM entities except emergency calls and does not perform IMSI detach if deactivated.

Maximum Duration Of Test

Expected Sequence

Step	Direction	Message	Comments

The following messages are sent and shall be received on cell B

1	MS		A mobile originating call is attempted

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	CM SERVICE REQUEST

5	SS -> MS	AUTHENT REQUEST

6	MS -> SS	AUTHENT RESPONSE

7	SS -> MS	ABORT	"reject cause" = #6

8	SS		The SS waits for 5 seconds

9	MS		The MS shall not send any layer 3 message

during that time

10	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

The following messages are sent and shall be received on cell A

11	   SS		The RF levels are changed

to make the MS reselect cell A.

12	   MS		The MS performs cell reselection

according to procedure as specified in GSM

05.08 (this however is not checked

until step 22). The MS shall not initiate

an RR connection establishment on

cell A or on cell B.

13	   SS		The SS waits at least 7 minutes

for a possible periodic

updating.

14 	   MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B.

15	SS -> MS	PAGING REQUEST TYPE 1	"Mobile identity" IE contains

TMSI.

16	   MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B.

This is verified during 3 seconds.

17	   MS		A MO CM connection is attempted.

18	   MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B.

This is checked during 3 seconds

19	   MS		If the MS supports speech (see PICS),

an emergency call is attempted.

20	MS -> SS	CHANNEL REQUEST	"Establishment cause" : Emergency call.

21	SS -> MS	IMMEDIATE ASSIGNMENT

22	MS -> SS	CM SERVICE REQUEST	"CM service type" : Emergency

call establishment.

23	SS -> MS	CM SERVICE ACCEPT

24	MS -> SS	EMERGENCY SETUP

25	SS -> MS	RELEASE COMPLETE	"Cause"= unassigned number.

26	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

27	   MS		If possible (see PICS) SIM

detachment is performed. Otherwise

if possible (see PICS) switch off

is performed. Otherwise the power

is removed.

28	   MS		The MS shall not initiate an RR connection

establishment on cell A or on cell B.

This is checked during 3 seconds.

29	MS		Depending on what has been

performed in step 29 the MS is

brought back to operation.

30	MS -> SS	CHANNEL REQUEST	"Establishment cause" : All

other cases.

31	SS -> MS	IMMEDIATE ASSIGNMENT

32	MS -> SS	LOCATION UPDATING REQ	"location updating type"= normal,

"CKSN"= no key available,

"Mobile Identity"= IMSI,

"LAI"= deleted LAI (the MCC and MNC hold

the previous values, the LAC is coded FFFE)

33	SS -> MS	AUTHENT REQUEST	"CKSN"= CKSN1

34	MS -> SS	AUTHENT RESPONSE

35	SS -> MS	LOCATION UPDATING ACC	"Mobile Identity"= TMSI

36	MS -> SS	TMSI REALLOCATION CMP

37	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.5.7.2 MM connection / abortion by the network / cause #6

26.7.5.7.2.1	Conformance Requirement

Upon reception of an ABORT message, the MS shall release any ongoing MM connection and enter the "wait for network command" state.

26.7.5.7.2.2	Test Purpose

To check that when multiple MM connections are established, the MS releases all MM connections upon reception of an ABORT message, in the case when the two MM connections are established for a mobile terminating call and a non call related supplementary service operation.

26.7.5.7.2.3	Reference(s)

GSM 04.08 section 4.3.5

26.7.5.7.2.4	Related PICS/PIXIT Statement(s)

The MS supports a non call related supplementary service operation during an active call Yes/No.

26.7.5.7.2.5	Initial Conditions

System Simulator:

1 cell, default parameters.

Mobile Station:

The MS is in state U10 of a mobile terminating call

26.7.5.7.2.6	Foreseen final state of the MS:

The MS has a valid TMSI. It is "idle updated"

26.7.5.7.2.7	Test procedure:

A non call related supplementary service operation is attempted at the MS. Upon reception of the REGISTER message, the SS sends an ABORT message with cause # 17. The SS sends a DISCONNECT using the TI of the mobile terminating call. The MS shall send a RELEASE COMPLETE message with the PD and TI of the DISCONNECT message and with cause #81. The SS releases the RR connection.

Maximum Duration Of Test

Expected Sequence

This procedure is performed if the MS supports non call related supplementary service operation

Step	Direction	Message	Comments

1	MS		A non call related supplementary

service operation is attempted

at the MS

2	MS -> SS	CM SERVICE REQUEST

3	SS -> MS	CM SERVICE ACCEPT

4	MS -> SS	REGISTER

5	SS -> MS	ABORT	"reject cause" = #17

6	SS -> MS	DISCONNECT	with the TI of the

mobile terminating call

7	MS -> SS	RELEASE COMPLETE	"cause"= #81. Same PD and TI

as the DISCONNECT message

8	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.5.8	MM connection / follow-on request pending

26.7.5.8.1	MM connection / follow-on request pending / test 1

26.7.5.8.1.1	Conformance Requirement

The MS shall not attempt to establish a new MM connection after location updating on the same RR connection if not allowed by the network.

26.7.5.8.1.2	Test Purpose

To check that when the network does not include the follow on proceed IE in a LOCATION UPDATING ACCEPT message, a MS that has a CM application request pending does not attempt to establish a new MM connection on that RR connection.

26.7.5.8.1.3	Reference(s)

GSM 04.08 section 4.4.4.6

26.7.5.8.1.4	Related PICS/PIXIT Statement(s)

none

26.7.5.8.1.5	Initial Conditions

System Simulator:

1 cell, ATT flag is set to "MSs in the cell shall apply IMSI attach and detach procedure".

Mobile Station:

The MS is deactivated.

26.7.5.8.1.6	Foreseen final state of the MS:

The MS has a valid TMSI. It is "idle updated".

26.7.5.8.1.7	Test procedure

The MS is activated and a CM connection is immediately attempted. The MS performs location updating. The SS does not include the follow on proceed information element in the LOCATION UPDATING ACCEPT message. The SS waits for at least 8 seconds. The MS shall not send any layer 3 message for 8 seconds.

Maximum Duration of Test

20 s

Expected Sequence

Step	Direction	Message	Comments

1	MS		The MS is activated and CM connection

is immediately attempted.

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

3	MS -> SS	LOCATION UPDATING REQ

4	SS -> MS	LOCATION UPDATING ACC	folllow on proceed IE not included

5	   SS		The SS wait for at least 8 seconds

6	   MS		The MS shall not send any layer 3

message for 8 seconds after reception

of the LOCATION UPDATING ACCEPT

message.

9	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

None.

26.7.5.8.2	MM connection / follow-on request pending / test 2

26.7.5.8.2.1	Conformance Requirement

A MS supporting the follow-on request procedure and having a CM connection request pending shall correctly establish an MM connection following a location update when allowed by the network.

26.7.5.8.2.2	Test Purpose

To check that when the network includes the follow on proceed IE in a LOCATION UPDATING ACCEPT message, a MS that supports the follow on request procedure and that has a CM application request pending establishes successfully a new MM connection on that RR connection.

26.7.5.8.2.3	Reference(s)

GSM 04.08 section 4.4.4.6

26.7.5.8.2.4	Related PICS/PIXIT Statement(s)

MS supports the follow on request procedure   Yes/No

26.7.5.8.2.5	Initial Conditions

System Simulator:

1 cell, ATT flag is set to "MSs in the cell shall apply IMSI attach and detach procedure".

Mobile Station:

The MS is deactivated.

26.7.5.8.2.6	Foreseen final state of the MS:

The MS has a valid TMSI. It is "idle updated".

26.7.5.8.2.7	Test procedure

The MS is activated and a CM connection is immediately attempted. The MS performs location updating. The SS includes the follow on proceed information element in the LOCATION UPDATING ACCEPT message. The SS waits for at least 8 seconds.

If the MS supports the follow on request procedure :

The MS shall send a CM SERVICE REQUEST. Upon reception of that message, the SS sends a CM SERVICE ACCEPT message. The MS shall send an initial CM message. Upon reception of that message, the SS releases the RR connection.

If the MS does not support the follow on request procedure :

The MS shall not send any layer 3 message for 8 seconds.

Maximum Duration of Test

20 s

Expected Sequence

Step	Direction	Message	Comments

1	MS		The MS is activated and an CM connection

is immediately attempted.

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ

5	SS -> MS	LOCATION UPDATING ACC	folllow on proceed IE included

If the MS supports the follow on

request procedure (see PICS) steps A6 to A8

are performed, otherwise steps B6 to B7

B8 are performed

A6	MS -> SS	CM-SERVICE REQUEST

A7	SS ->MS	CM-SERVICE ACCEPT

A8	MS -> SS	An initial CM message

B6	   SS		The SS wait for at least 8 seconds

B7	   MS		The MS shall not send any layer 3

message for 8 seconds after reception

of the LOCATION UPDATING ACCEPT

message.

9	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

none

26.7.5.8.3	MM connection / follow-on request pending / test 3

26.7.5.8.3.1	Conformance Requirement

1)	The MS shall not set the follow on request bit in a LOCATION UPDATING REQUEST message if no MM connection request is pending.

2)	When the network includes the follow on proceed IE in a LOCATION UPDATING ACCEPT message, a MS that has no CM application request pending shall not attempt to establish a new MM connection on that RR connection.

3)	The MS shall correctly handle a CM connection established by the network on the RR connection that was used for the location updating procedure.

26.7.5.8.3.2	Test Purpose

1)	To check that a MS that has no CM application request pending sets the Follow-On-Request bit to No follow-on request pending in a LOCATION UPDATING REQUEST message.

2)	To check that when the network includes the follow on proceed IE in a LOCATION UPDATING ACCEPT message, a MS that has no CM application request pending does not attempt to establish a new MM connection on that RR connection.

3)	To check that the MS accepts establishment by the network of a new MM connection on the existing RR connection.

26.7.5.8.3.3	Reference(s)

GSM 04.08 section 4.4.4.6

26.7.5.8.3.4	Related PICS/PIXIT Statement(s)

Supported services on TCH

26.7.5.8.3.5	Initial Conditions

System Simulator:

1 cell, ATT flag is set to "MSs in the cell shall apply IMSI attach and detach procedure".

Mobile Station:

The MS is deactivated.

26.7.5.8.3.6	Foreseen final state of the MS:

The MS has a valid TMSI. It is "idle updated".

26.7.5.8.3.7	Test procedure

The MS is activated. The MS performs location updating. The MS shall set the FOR bit to No follow-on request pending in the LOCATION UPDATING REQUEST message. The SS includes the follow on proceed information element in the LOCATION UPDATING ACCEPT message. The SS waits for 5 seconds. The MS shall not send any layer 3 message for 5 seconds. The SS sends a SETUP message to the MS requesting a basic service supported by the MS. The MS shall send either a CALL CONFIRMED message if it supports a service on TCH or a RELEASE COMPLETE with cause #88.

Maximum Duration of Test

20 s

Expected Sequence

Step	Direction	Message	Comments

1	MS		The MS is activated

2	MS -> SS	CHANNEL REQUEST

3	SS -> MS	IMMEDIATE ASSIGNMENT

4	MS -> SS	LOCATION UPDATING REQ	"Location updating type" : the FOR bit

is set to No follow-on request pending

5	SS -> MS	LOCATION UPDATING ACC	follow on proceed IE is included

6	   SS		The SS wait for 5 seconds

7	   MS		The MS shall not send any layer 3

message for 5 seconds after reception

of the LOCATION UPDATING ACCEPT

message.

8	SS -> MS	SETUP

If the MS supports a basic service on TCH

A9	MS -> SS	CALL CONFIRMED

If the MS does not support any basic

service on TCH

B9	MS -> SS	RELEASE COMPLETE	cause #88

10	SS -> MS	CHANNEL RELEASE	After the sending of this

message, the SS waits for the

disconnection of the main

signalling link.

Specific message contents:

none

26.7.6	Default contents of messages

AUTHENTICATION RESPONSE

Information element	Value/remark

Authentication parameter SRES	As applicable

AUTHENTICATION REJECT

Information element	Value/remark

None but message head

CHANNEL RELEASE

Information element	Value/remark

RR cause	Normal release

CIPHERING MODE COMMAND

Information element	Value/remark

Cipher mode setting	Start ciphering

Cipher Response	IMEI must not be included

CM SERVICE ACCEPT

Information element	Value/remark

None but message head

CM SERVICE REQUEST

Information element	Value/remark

CM service type	Mobile originating call establishment

Ciphering key sequence number	According to SIM contents

Mobile station classmark 2	See PICS/PIXIT

Mobile identity	TMSI of the MS

CM SERVICE REJECT

Information element	Value/remark

Reject cause

IDENTITY REQUEST

Information element	Value/remark

Identity type	Depending on test

Spare half octet	0000

IDENTITY RESPONSE

Information element	Value/remark

Mobile identity	Depending on test

IMMEDIATE ASSIGNMENT

Information element	Value/remark

L2 pseudo length

Page mode	Normal Paging

Spare half octet	0000

Channel description

- Channel type and TDMA offset	SDCCH/4 or SDCCH/8

- Time slot number	Arbitrary legal value

- Subsequent fields of the Channel description IE are specified in section 26.1.1

Request reference

- Random access information	As received from MS

- N51,N32,N26	Corresponding to the frame in which the Channel Request was sent

Timing advance	0

Mobile allocation	Empty (L=0)

Starting time	Omitted

IA rest octets	all bits set to spare

IMSI DETACH INDICATION

Information element	Value/remark

Mobile station classmark 1	See PICS/PIXIT

Mobile identity	TMSI of the MS

LOCATION UPDATING ACCEPT

Information element	Value/remark

Location area identification	As in section 26.1.1

Mobile identity	Depending on test

Follow on proceed	Omitted

PAGING REQUEST TYPE 1

Information element	Value/remark

L2 pseudo length

Page Mode	Normal paging

Channels needed

- mobile 1	"any channel"

- mobile 2	spare

Mobile identity 1	TMSI of MS under test

Mobile identity 2	Omitted

P1 rest octets	All bits set to spare

PAGING RESPONSE

Information element	Value/remark

Ciphering key sequence number	According to SIM contents

Spare half octet	0000

Mobile station classmark 2	See PICS/PIXIT

Mobile identity	TMSI of the MS

TMSI REALLOCATION COMMAND

Information element	Value/remark

Location area identification	As in section 26.1.1

Mobile identity	TMSI

TMSI REALLOCATION COMPLETE

Information element	Value/remark

None but message head